3GPP TSG-SA5 Meeting #161	S5-252578

Fukuoka, Japan, 19 - 23 May 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.104 | CR | 0222 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.104 Update the use case for failure prediction |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA5 |  |  |  |  |  |  |  |  |  |
| Work item code: | eMDAS_Ph3 |  |  |  |  |  | Date: |  |  | 2025-05-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In current use case, it states that threshold crossing statistics information can help the MDAS consumer to know whether the NF is in the overloaded state. However, it is not stated how this information and state are relevant for failure prediction. Therefore, an example is added to make this relationship clear. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the use case to improve the description. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The description in use case is unclear and difficult to understand. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.2.3.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

1st Change

7.2.3	MDA assisted fault management

7.2.3.1	Failure prediction

7.2.3.1.1	Description

This MDA capability is for failure prediction.

7.2.3.1.2	Use case

There are multiple sources of faults which may cause the 5G system to fail to provide the expected service. These faults and the associated failures need extensive troubleshooting. In order to reduce network and service failure time and performance degradation, it is necessary to supervise the status of various network functions and resources, and predict the running trend of network and potential failures to intervene in advance. These predictions can be used by the management system to autonomously maintain the health of the network, e.g. speedy recovery actions on a network function related to the predicted potential failure.

Due to the fact that failure prediction could depend on the existing alarm incidents and relevant historical and real-time data (performance measurement information, configuration data, network topology information, etc.), there is a possibility for MDA to be used in conjunction with AI/ML technologies and model training to predict potential failures.

In order to avoid the occurrence of failures and abnormal network status, it is necessary for consumers of analytics to obtain the required details of potential failure and the corresponding degradation trend (abnormal KPI, performance measurement information, possible alarm type, fault root cause, etc.). Therefore, MDA, may in conjunction with AI/ML technology, be required to obtain basic health maintenance knowledge (e.g. the relationship between the failures or potential failures and the related maintenance actions) through predefined expertise or model training, so as to effectively predict potential failures. The basic health maintenance knowledge could be updated with feedback. The consumer of analytics could provide the performance crossing threshold information and resource usage threshold information to MDAS producer for calculating and collecting the statistics result, e.g. percentage of time when the performance KPI exceeds the threshold, and percentage of time when resource usage of the NF is beyond the threshold. Such statistics information helps the MDAS consumer to know whether the NF is in the overloaded state which is likely to cause network failure. Furthermore, the statistics information can be combined with historical data and ML model for MDAS producer to perform failure prediction of the NF (e.g. to predict potential NF impact from a signalling storm).

Besides the MDA capability to obtain basic health maintenance relationships between the service failure and related potential failures at network levels, the MDA capability for failure prediction may take role of coordination in cross domain. When MDA capability takes role of coordination in cross domain, the MDA capability for failure prediction can collect analytics output of failure prediction from single domain management and provide recommendation actions accordingly.

Along with the predicted fault, the information such as the trend of the predicted fault and the duration of the predicted fault are helpful for the consumers to seamlessly correlate the predicted failure and the actual failure when it occurs in the system.

If necessary, MDA could also provide corresponding recommended actions for failure prevention. In case the producer of MDA MnS is unable to recommend an action for failure prevention, it may be useful to recommend collecting additional management data.

The comprehensive collection of all potentially related management data from deployed field entities is time-consuming and resource intensive. Therefore, it is advisable to collect more relevant management data pre-emptively, enabling effective debugging and root cause analysis to mitigate future failure occurrences. To support robust root cause analysis, it is essential to collect specific management data that facilitates the identification and resolution of predicted failures.

End of Change