3GPP TSG-SA5 Meeting #162	S5-253225

# Stor-Göteborg, Sweden, 25th Aug 2025 - 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.541 | CR | 1557 | rev | - | Current version: | 18.12.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-18 CR 28.541 YANG stage-3 corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson Hungary Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI18 |  |  |  |  |  | Date: |  |  | 2025-07-21 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | YANG SS is not matching the approved stage 2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update YANG code to match existing stage 2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 2 and Stage 3 mismatch; interoperability problems. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Forge only |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1791 at commit c3b3318b30d64b1c126707f5b0f06b81103bcc0c |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

Forge MR link: https://forge.3gpp.org/rep/sa5/MnS/-/merge_requests/1791 at commit c3b3318b30d64b1c126707f5b0f06b81103bcc0c

*** START OF CHANGE 1 ***

*** yang-models/_3gpp-5g-common-yang-types.yang ***

<CODE BEGINS>

module _3gpp-5g-common-yang-types {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-5g-common-yang-types";

prefix "types5g3gpp";

import _3gpp-common-yang-types { prefix types3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "The model defines common types for 5G networks and

network slicing.

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541";

revision 2025-08-18 { reference CR-1557; }

revision 2024-10-06 { reference CR-1389; }

revision 2024-05-24 { reference CR-1273 ; }

revision 2023-09-18 { reference CR-1043 ; }

revision 2023-05-10 { reference CR-0916; }

revision 2021-08-05 { reference S5-214053/CR-0518; }

revision 2020-11-05 { reference CR-0412 ; }

revision 2019-10-20 { reference "Initial version."; }

grouping NcgiGrp {

description "Represents the Ncgi datatype";

list plmnId {

description "This attribute represents a PLMN Identity.";

min-elements 1;

max-elements 1;

key "idx";

leaf idx { type uint32 ; }

uses types3gpp:PLMNId ;

}

leaf nrCellId {

type string;

mandatory true;

description "This attribute represents NR Cell Identity.

It's a 36-bit string identifying an NR Cell Id as specified in

clause 9.3.1.7 of TS 38.413, in hexadecimal representation. Each

character in the string shall take a value of

'0' to '9', 'a' to 'f' or 'A' to 'F' and shall represent 4 bits.

The most significant character representing the 4 most significant

bits of the Cell Id shall appear first in the string, and the

character representing the 4 least significant bit of the

Cell Id shall appear last in the string.

Pattern: '^[A-Fa-f0-9]{9}$'

Example:

An NR Cell Id 0x225BD6007 shall be encoded as '225BD6007'.";

}

leaf nId {

type string;

mandatory true;

description "Network Identity; Shall be present if PlmnIdNid identifies

an SNPN (see clauses 5.30.2.3, 5.30.2.9, 6.3.4, and 6.3.8 in

3GPP TS 23.501.";

}

}

typedef NRTAC {

type string;

description "This holds the identity of the common Tracking Area Code

for the PLMNs.

allowedValues:

a) It is the TAC or Extended-TAC.

b) A cell can only broadcast one TAC or Extended-TAC. See TS 36.300,

subclause 10.1.7 (PLMNID and TAC relation).

c) TAC is defined in subclause 19.4.2.3 of 3GPP TS 23.003

and Extended-TAC is defined in subclause 9.3.1.29 of 3GPP TS 38.473.

d) For a 5G SA (Stand Alone), it has a non-null value.";

}

grouping PlmnIdNid {

description "Represents the SCP domain specific information as defined

in TS 29.510 ";

uses types3gpp:PLMNId;

leaf nid {

type string;

description "This attribute represents network Identity;

Shall be present if PlmnIdNid identifies an SNPN.

(see clauses 5.30.2.3, 5.30.2.9, 6.3.4, and 6.3.8 in TS 23.501";

}

}

grouping PLMNInfo {

description "The PLMNInfo data type define a S-NSSAI member in a specific

PLMNId, and it have two attributes PLMNId and S-NSSAI (PLMNId, S-NSSAI).

The PLMNId represents a data type that is comprised of mcc

(mobile country code) and mnc (mobile network code), (See TS 23.003

subclause 2.2 and 12.1) and S-NSSAI represents an data type, that is

comprised of an SST (Slice/Service type) and an optional

SD (Slice Differentiator) field";

uses types3gpp:PLMNId;

uses types3gpp:SNssai;

}

typedef CommModelType {

reference "3GPP TS 23501";

type enumeration {

enum DIRECT_COMMUNICATION_WO_NRF {

value 0;

description "Directly communicate to other pre-configured NF service.";

}

enum DIRECT_COMMUNICATION_WITH_NRF {

value 1;

description "Directly communicate to other NF service discovered

by NRF.";

}

enum INDIRECT_COMMUNICATION_WO_DEDICATED_DISCOVERY {

value 2;

description "Communicate to pre-configured other NF service through

SCP as a proxy.";

}

enum INDIRECT_COMMUNICATION_WITH_DEDICATED_DISCOVERY {

value 3;

description "Communication to NF service discovered by NRF through SCP

as a proxy.";

}

}

}

grouping CommModel {

leaf groupId {

type uint16;

}

leaf commModelType {

type CommModelType;

}

leaf-list targetNFServiceList {

type types3gpp:DistinguishedName;

}

leaf commModelConfiguration {

type string;

}

}

grouping SupportedFunc {

leaf function {

type string;

}

leaf policy {

type string;

}

}

typedef EnergySavingLoadThresholdT {

type uint32 {

range 0..10000;

}

units 1/10000;

}

typedef EnergySavingTimeDurationT {

type uint32 {

range 0..900;

}

units seconds;

}

typedef PhysCellID {

type uint32 {

range "0..1007";

}

reference "clause 7.4.2 of TS 38.211";

}

typedef UTC24TimeOfDayT {

description "Time of day in HH:MM or H:MM 24-hour format per UTC

time zone.";

type string {

pattern "(([01]?[0-9])|(2[0-3])):([0-5][0-9])";

}

}

typedef DayOfWeekT {

type enumeration {

enum Monday;

enum Tuesday;

enum Wednesday;

enum Thursday;

enum Friday;

enum Saturday;

enum Sunday;

}

}

}

<CODE ENDS>

*** END OF CHANGE 1 ***

*** START OF CHANGE 2 ***

*** yang-models/_3gpp-5gc-nrm-GtpUPathQoSMonitoringControl.yang ***

<CODE BEGINS>

module _3gpp-5gc-nrm-GtpUPathQoSMonitoringControl {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-GtpUPathQoSMonitoringControl;

prefix gtpUPathQMCtrl3gpp;

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-5gc-nrm-smffunction { prefix smf3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the capabilities and properties for control

of GTP-U path QoS monitoring defined in 3GPP TS 23.501.

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541";

revision 2025-08-18 { reference CR-1557 ; }

revision 2023-09-18 { reference CR-1043 ; }

revision 2020-11-05 { reference CR-0412 ; }

revision 2020-09-30 { reference "CR-0393"; }

revision 2020-08-03 { reference "CR-0321"; }

revision 2020-04-10 { reference "S5-202103"; }

grouping GtpUPathDelayThresholdsType {

description "Thresholds for reporting the packet delay for GTP-U path QoS

monitoring ";

reference "3GPP TS 29.244";

leaf n3AveragePacketDelayThreshold {

mandatory true;

type uint32;

}

leaf n3MinPacketDelayThreshold {

mandatory true;

type uint32;

}

leaf n3MaxPacketDelayThreshold {

mandatory true;

type uint32;

}

leaf n9AveragePacketDelayThreshold {

mandatory true;

type uint32;

}

leaf n9MinPacketDelayThreshold {

mandatory true;

type uint32;

}

leaf n9MaxPacketDelayThreshold {

mandatory true;

type uint32;

}

}

grouping GtpUPathQoSMonitoringControlGrp {

description "Represents the GtpUPathQoSMonitoringControl IOC.";

leaf gtpUPathQoSMonitoringState {

description "The state of GTP-U path QoS monitoring.";

mandatory true;

type enumeration {

enum ENABLED;

enum DISABLED;

}

}

list gtpUPathMonitoredSNSSAIs {

key "sd sst";

description "The S-NSSAIs for which the the GTP-U path QoS monitoring is

to be performed.";

reference "3GPP TS 23.003";

uses types3gpp:SNssai;

}

leaf-list monitoredDSCPs {

description "The DSCPs for which the GTP-U path QoS monitoring is to be

performed.";

reference "3GPP TS 29.244";

type uint32;

}

leaf isEventTriggeredGtpUPathMonitoringSupported {

description "It indicates whether the event triggered GTP-U path QoS

monitoring reporting based on thresholds is supported.";

mandatory true;

reference "3GPP TS 29.244";

type boolean;

}

leaf isPeriodicGtpUMonitoringSupported {

description "It indicates whether the periodic GTP-U path QoS monitoring

reporting is supported.";

mandatory true;

reference "3GPP TS 29.244";

type boolean;

}

leaf isImmediateGtpUMonitoringSupported {

description "It indicates whether the immediate GTP-U path QoS monitoring

reporting is supported.";

mandatory true;

reference "3GPP TS 29.244";

type boolean;

}

list gtpUPathDelayThresholds {

key n3AveragePacketDelayThreshold;

// if max-elements is increased later, the key may need to be modified

min-elements 1;

max-elements 1;

description "It specifies the thresholds for reporting the packet delay

for the GTO-U path QoS monitoring.";

uses GtpUPathDelayThresholdsType;

}

leaf gtpUPathMinimumWaitTime {

description "It specifies the minimum waiting time (in seconds) between

two consecutive reports for event triggered GTP-U path QoS monitoring

reporting.";

type uint32;

}

leaf gtpUPathMeasurementPeriod {

description "It specifies the period (in seconds) for reporting the packet

delay for GTP-U path QoS monitoring.";

type uint32;

}

}

augment "/me3gpp:ManagedElement/smf3gpp:SMFFunction" {

list GtpUPathQoSMonitoringControl {

description "Specifies the capabilities and properties for control of

GTP-U path QoS monitoring. For more information about the GTP-U path

QoS monitoring.";

reference "3GPP TS 23.501";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses GtpUPathQoSMonitoringControlGrp;

}

}

}

}

<CODE ENDS>

*** END OF CHANGE 2 ***

*** START OF CHANGE 3 ***

*** yang-models/_3gpp-5gc-nrm-QFQoSMonitoringControl.yang ***

<CODE BEGINS>

module _3gpp-5gc-nrm-QFQoSMonitoringControl {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-QFQoSMonitoringControl;

prefix qFQMCtrl3gpp;

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-5gc-nrm-smffunction { prefix smf3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the capabilities and properties for control

of QoS monitoring per QoS flow per UE for URLLC service defined

in 3GPP TS 23.501.

Copyright 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541";

revision 2025-08-18 { reference CR-1557 ; }

revision 2023-09-24 { reference CR-0916; }

revision 2020-11-08 { reference CR-0412 ; }

revision 2020-08-03 { reference "CR-0321"; }

revision 2020-04-10 { reference "S5-202101";   }

grouping QFPacketDelayThresholdsTypeGrp {

description "Represents the QFPacketDelayThresholdsType";

leaf thresholdDl {

type uint32;

units milliseconds;

description "Downlink threshold";

}

leaf thresholdUl {

type uint32;

units milliseconds;

description "Uplink threshold";

}

leaf thresholdRtt {

type uint32;

units milliseconds;

description "Round trip threshold";

}

}

grouping QFQoSMonitoringControlGrp {

description "Represents the QFQoSMonitoringControl IOC.";

reference "3GPP TS 28.541";

leaf qFQoSMonitoringState {

description "The state of QoS monitoring per QoS flow per UE.";

mandatory true;

type enumeration {

enum ENABLED;

enum DISABLED;

}

}

list qFMonitoredSNSSAIs {

description "The S-NSSAIs for which the QoS monitoring per QoS flow

per UE is to be performed.";

reference "3GPP TS 23.003";

key "sd sst";

uses types3gpp:SNssai;}

leaf-list qFMonitored5QIs {

description "The 5QIs for which the QoS monitoring per QoS flow

per UE is to be performed.";

reference "3GPP TS 23.501";

type uint32 {

range "0..255";

}

}

leaf isEventTriggeredQFMonitoringSupported {

description "It indicates whether the event based QoS monitoring

reporting per QoS flow per UE is supported.";

mandatory true;

reference "3GPP TS 29.244";

type boolean;

}

leaf isPeriodicQFMonitoringSupported {

description "It indicates whether the periodic QoS monitoring reporting

per QoS flow per UE is supported.";

mandatory true;

reference "3GPP TS 29.244";

type boolean;

}

leaf isSessionReleasedQFMonitoringSupported {

description "It indicates whether the session release based QoS monitoring

reporting  per QoS flow per UE is supported.";

mandatory true;

reference "3GPP TS 29.244";

type boolean;

}

list qFPacketDelayThresholds {

key "idx";

min-elements 1;

max-elements 1;

description "It specifies the thresholds for reporting the packet delay

between PSA and UE for QoS monitoring per QoS flow per UE.";

leaf idx { type uint32 ; }

uses QFPacketDelayThresholdsTypeGrp;

}

leaf qFMinimumWaitTime {

description "It specifies the minimum waiting time (in seconds) between

two consecutive reports for event triggered QoS monitoring reporting

per QoS flow per UE.";

type uint32;

}

leaf qFMeasurementPeriod {

description "It specifies the period (in seconds) for reporting the

packet delay for QoS monitoring per QoS flow per UE.";

type uint32;

}

list qFMonitoredSatelliteBackhaulCategories {

description "The satellite backhaul categories for which the QoS

monitoring per QoS flow per UE is to be performed.";

key satellitebackhaulcat;

leaf satellitebackhaulcat{

description "Specifies the satellite backhaul.";

reference "3GPP TS 23.501";

type enumeration {

enum DYNAMIC_GEO;

enum DYNAMIC_MEO;

enum DYNAMIC_LEO;

enum DYNAMIC_OTHER_SAT;

}

}

}

}

augment "/me3gpp:ManagedElement/smf3gpp:SMFFunction" {

list QFQoSMonitoringControl {

description "Represents the QFQoSMonitoringControl IOC.";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses QFQoSMonitoringControlGrp;

}

}

}

}

<CODE ENDS>

*** END OF CHANGE 3 ***

*** START OF CHANGE 4 ***

*** yang-models/_3gpp-5gc-nrm-amffunction.yang ***

<CODE BEGINS>

module _3gpp-5gc-nrm-amffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-amffunction;

prefix amf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import ietf-inet-types { prefix inet; }

import ietf-yang-types { prefix yang; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-nr-nrm-gnbcuupfunction { prefix gnbcuup3gpp; }

import _3gpp-5gc-nrm-nfprofile { prefix nfp3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "AMFFunction derived from basic ManagedFunction.

Copyright 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2025-08-18 { reference CR-1557 ; }

revision 2024-10-06 { reference CR-1389; }

revision 2024-04-04 { reference CR-1139; }

revision 2023-09-18 { reference CR-1043 ; }

revision 2022-01-07 { reference CR-0643; }

revision 2020-11-06 { reference CR-0412 ; }

revision 2019-10-25 { reference "S5-194457 S5-193518"; }

revision 2019-05-31 { reference "Ericsson refactoring."; }

revision 2018-08-07 { reference "Initial revision"; }

grouping NTNGlobalRanNodeIDGrp{

list pLMNId {

description "The PLMN Identifier is composed of

a Mobile Country Code (MCC) and

a Mobile Network Code (MNC).";

min-elements 1;

max-elements 1;

key "mcc mnc";

uses types3gpp:PLMNId;

}

choice GlobalRanNodeID{

description "Unique identifier of an NG-RAN node.";

reference "3GPP TS 38.413";

mandatory true;

case gNB {

leaf GlobalgNBID{

description "Unique identifier of a gNB.";

reference "3GPP TS 38.413";

type int64;

mandatory true;

}

}

case ng-eNB{

leaf Globalng-eNBID{

type string{

pattern '^(MacroNGeNB-[A-Fa-f0-9]{5}|LMacroNGeNB-

[A-Fa-f0-9]{6}|SMacroNGeNB

-[A-Fa-f0-9]{5})$';

}

description "This represents the identifier of

the ng-eNB ID as specified in

clause 9.3.1.8 of 3GPP TS 38.413.";

mandatory true;

}

}

case N3IWF{

leaf N3IWFID{

type string{

pattern '^[A-Fa-f0-9]+$';

}

description "This IE shall contain the N3IWF

identifier received over NGAP and

shall be encoded as a string of

hexadecimal characters.";

mandatory true;

}

}

case TNGF{

leaf TNGFID{

type string{

pattern '^[A-Fa-f0-9]+$';

}

description "This represents the identifier of

the TNGF ID as specified in clause

9.3.1.161 of 3GPP TS 38.413 in hexadecimal

representation.";

mandatory true;

}

}

case TWIF{

leaf TWIFID{

type string;

description "This represents the TWIF identification

as specified in clause

9.3.1.163 of TS 38.413";

mandatory true;

}

}

case W-AGF{

leaf W-AGFID{

type string{

pattern '^[A-Fa-f0-9]+$';

}

description "This represents the identifier of

the W-AGF ID as specified in

clause 9.3.1.162 of 3GPP TS 38.413

in hexadecimal representation.";

mandatory true;

}

}

}

}

grouping SatelliteBackhaulInfoGrp{

description "Defines information related to satellite backhaul category

and corresponding information of gNB.";

reference "3GPP TS 23.501";

list nTNGlobalRanNodeID{

description "Unique identifier of an NG-RAN node.";

min-elements 1;

max-elements 1;

leaf GlobalRanNodeIDValue{

type string;

description "this relies on the choice of GlobalRanNodeID";

}

key "GlobalRanNodeIDValue";

uses NTNGlobalRanNodeIDGrp;

}

leaf satelliteBackhaulCategory{

description "Satellite backhaul category refers to the type of the

satellite used in the backhaul. Only a single backhaul category

can be indicated.";

reference "3GPP TS 29.571";

type enumeration{

enum GEO;

enum MEO;

enum LEO;

enum OTHER_SAT;

enum DYNAMIC_GEO;

enum DYNAMIC_MEO;

enum DYNAMIC_LEO;

enum DYNAMIC_OTHER_SAT;

enum NON_SATELLITE;

}

mandatory true;

}

leaf geoSatelliteId{

description "Unique identifier of a GEO satellite.";

reference "3GPP TS 29.571";

type string;

//condition present only if the UPF is deployed on the satellite.

}

}

grouping GUAMInfoGrp {

description "Represents the GUAMInfo datatype.";

list pLMNId {

description "This attribute represents a PLMN Identity.";

min-elements 1;

max-elements 1;

key "idx";

leaf idx { type uint32 ; }

uses types3gpp:PLMNId ;

}

leaf aMFIdentifier  {

type int64;

description "The AMFI is constructed from an AMF Region ID, an

AMF Set ID and an AMF Pointer. The AMF Region ID identifies the region,

the AMF Set ID uniquely identifies the AMF Set within the AMF Region,

and the AMF Pointer uniquely identifies the AMF within the AMF Set.

(Ref. 3GPP TS 23.003)";

}

}

grouping N2InterfaceAmfInfoGrp {

description "Represents the N2InterfaceAmfInfo datatype.";

leaf-list ipv4EndpointAddress {

type inet:ipv4-address;

min-elements 1;

description "This attribute represents available AMF endpoint

IPv4 address(es) for N2.";

}

leaf-list ipv6EndpointAddress {

type inet:ipv6-address;

min-elements 1;

description "This attribute represents available AMF endpoint

IPv6 address(es) for N2.";

}

leaf amfName {

type inet:host-name;

description "This attribute represents AMF Name FQDN as defined in

clause 28.3.2.5 of TS 23.003.";

}

}

grouping BlockedLocationInfoGrp {

description "Repreasents the datatype BlockedLocationInfo";

list blockedLocation {

description "This provides the geographical location at which the PLMN

are not allowed in case of NTN.";

min-elements 1;

max-elements 1;

key "mnc mcc";

uses types3gpp:PLMNId ;

}

list blockedDurWindow {

description "This provides the time durations for which the PLMN are

not allowed at a given location in case of NTN";

key idx;

leaf idx { type uint32 ; }

uses types3gpp:TimeWindowGrp;

}

list blockedSlice {

description "This provides the slice for which the access is

not allowed at a given location in case of NTN. ";

max-elements 1;

key idx;

unique "sst sd";

leaf idx { type uint32; }

uses types3gpp:SNssai;

}

}

grouping NTNPLMNRestrictionsInfoGrp {

description "Represents the datatype NTNPLMNRestrictionsInfo";

list pLMNId {

description "This attribute represents a PLMN Identity.";

min-elements 1;

max-elements 1;

key "mnc mcc";

uses types3gpp:PLMNId ;

}

list blockedLocationInfoList {

description "This defines the information related with the location

for which the access restrictions are to be applied in case of NTN.";

key "idx";

leaf idx { type uint32 ; }

uses BlockedLocationInfoGrp;

}

}

grouping NtnLocationInfoGrp {

description  "Represents the NtnLocationInfo datatype.";

list location {

description "This defines the Location (geographical area) under

consideration to which the satellite coverage info belongs";

key idx;

leaf idx { type uint32 ; }

uses types3gpp:GeoAreaGrp;

}

list availabilityWindows  {

description "This attribute defines the list of time windows at which

the satellite coverage will be available for this location.

Either availabilityWindows or nonAvailabilityWindows shall be present.";

key idx;

leaf idx { type uint32 ; }

uses types3gpp:TimeWindowGrp;

}

list nonAvailabilityWindows {

description "This attribute defines the list of time windows at which

the satellite coverage will not be available for this location.

Either availabilityWindows or nonAvailabilityWindows shall be present.";

key idx;

leaf idx { type uint32 ; }

uses types3gpp:TimeWindowGrp;

}

}

grouping SatelliteCoverageInfoGrp {

description "Represents the datatype SatelliteCoverageInfo";

leaf nRSatelliteRATtype {

type enumeration {

enum NRLEO;

enum NRMEO;

enum NRGEO;

enum NROTHERSAT;

}

description "This attribute defines the RAT Type for NR satellite

access.";

}

list locationInfo {

description "This attribute defines the information about location and

corresponding time windows for which the satellite coverage will be

available or unavailable.";

key idx;

leaf idx { type uint32 ; }

uses NtnLocationInfoGrp;

}

}

grouping SliceExpiryInfoGrp {

description "Represents the datatype SliceExpiryInfo";

list pLMNInfo {

min-elements 1;  // ???

config false;

description "It defines the PLMN(s) of a Network Function. ";

key idx;

leaf idx { type uint32 ; }

uses types5g3gpp:PLMNInfo;

}

leaf expiryTime {

type yang:date-and-time;

config false;

description "This attribute provides information about the time at

which the slice is scheduled to be expired as it is not required

anymore.

This attribute will be set based on the sliceAvailability coming as

part of ServiceProfile.";

}

}

grouping MappedCellIdInfoGrp {

description "Represents the datatype MappedCellIdInfo";

list ntnGeoArea {

description "This attribute indicates a specific geographical location

mapped to Mapped Cell ID(s).";

min-elements 1;

max-elements 1;

key "idx";

leaf idx { type uint32 ; }

uses types3gpp:GeoAreaGrp;

}

list mappedCellId {

description "This attribute is in format of NCGI to indicate a fixed

geographical area (See subclause 16.14.5 in TS 38.300";

min-elements 1;

max-elements 1;

key "idx";

leaf idx { type uint32 ; }

uses types5g3gpp:NcgiGrp;

}

}

grouping AmfInfoGrp {

description "Represents the data type AmfInfo.";

leaf amfRegionId {

type int64;

mandatory true;

description "It represents the AMF Region ID, which identifies the

region. AllowedValues: defined in subclause 2.10.1 of 3GPP TS 23.003 ";

}

leaf amfSetId {

type int64;

mandatory true;

description "It represents the AMF Set ID, which is uniquely identifies

the AMF Set within the AMF Region.

allowedValues: defined in subclause 2.10.1 of 3GPP TS 23.003";

}

list taiList {

min-elements 1;

description "The list of TAIs.";

key "idx";

leaf idx { type uint32 ; }

uses gnbcuup3gpp:TAIGrp;

}

list taiRangeList {

min-elements 1;

description "The range of TAIs.";

key "idx";

leaf idx { type uint32 ; }

uses nfp3gpp:TaiRangeGrp;

}

list gUAMIdList {

description "List of supported Globally Unique AMF Ids (GUAMIs).";

config false;

min-elements 1;

key "idx";

leaf idx { type uint32 ; }

uses GUAMInfoGrp;

}

list backupInfoAmfFailure {

description "List of GUAMIs for which the AMF acts as a backup for AMF

failure.";

min-elements 1;

key "idx";

leaf idx { type uint32 ; }

uses GUAMInfoGrp;

}

list backupInfoAmfRemoval {

description "List of GUAMIs for which the AMF acts as a backup for

planned AMF removal.";

min-elements 1;

key "idx";

leaf idx { type uint32 ; }

uses GUAMInfoGrp;

}

list n2InterfaceAmfInfo {

description "This attribute represents the N2 interface information of

the AMF. ";

max-elements 1;

key "idx";

leaf idx { type uint32 ; }

uses N2InterfaceAmfInfoGrp;

}

leaf amfOnboardingCapability {

type boolean;

default false;

description "This attribute indicates the AMF supports SNPN Onboarding

capability. This is used for the case of Onboarding of UEs for SNPNs

(see TS 23.501, clause 5.30.2.10).

- FALSE: AMF does not support SNPN Onboarding;

- TRUE: AMF supports SNPN Onboarding.";

}

leaf highLatencyCom {

type boolean;

description "This attribute indicates whether the AMF supports High

Latency communication (e.g. for NR RedCap UE). This is used for CP NF

to discover AMF supporting High Latency communication (see TS 23.501,

clause 6.3.5).

- FALSE: AMF does not support High Latency communication e.g. for

NR RedCap UE;

- TRUE: AMF supports High Latency communication e.g. for NR RedCap UE;";

}

}

grouping AMFFunctionGrp {

description "Represents the AMFFunction IOC";

uses mf3gpp:ManagedFunctionGrp;

list pLMNInfoList {

min-elements 1;

description "It defines the PLMN(s) of a Network Function. ";

key idx;

leaf idx { type uint32 ; }

uses types5g3gpp:PLMNInfo;

}

container aMFIdentifier {

description "An AMF identifier, comprising an AMF Region ID, an

AMF Set ID and an AMF Pointer.";

uses types3gpp:AmfIdentifier;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the

service-based interface.";

type inet:domain-name;

}

leaf-list cNSIIdList {

type string;

description "It is a set of NSI ID. NSI ID is an identifier for

identifying the Core Network part of a Network Slice instance when

multiple Network Slice instances of the same Network Slice are deployed,

and there is a need to differentiate between them in the 5GC.

See NSI ID definition in clause 3.1 of TS 23.501 and

subclause 6.1.6.2.7 of  TS 29.531";

}

list managedNFProfile {

key idx;

min-elements 1;

max-elements 1;

uses types3gpp:ManagedNFProfile;

description "";

}

list commModelList {

min-elements 1;

key "groupId";

description "Specifies a list of commModel. It can be used by NF and

NF services to interact with each other in 5G Core network ";

reference "3GPP TS 23.501";

uses types5g3gpp:CommModel;

}

list amfInfo {

max-elements 1;

description "This attribute represents information of an AMF

NF Instance.";

key "idx";

leaf idx { type uint32 ; }

uses AmfInfoGrp;

}

list nTNPLMNRestrictionsList {

description "This attribute defines the location restrictions per PLMN

that relates to non-terrestrial network access.";

key "idx";

leaf idx { type uint32 ; }

uses NTNPLMNRestrictionsInfoGrp;

}

list satelliteCoverageInfoList {

description "This attribute defines the information related to NR

Satellite RAT type and corresponding information of satellite coverage";

key "idx";

leaf idx { type uint32 ; }

uses SatelliteCoverageInfoGrp;

}

list sliceExpiryInfo {

description "This provides information related to a network slice

validity.";

key "idx";

leaf idx { type uint32 ; }

uses SliceExpiryInfoGrp;

}

list satelliteBackhaulInfoList{

description "Specifies a list of satellite backhaul information. It can be

used by NF and NF services.";

leaf GlobalRanNodeIDValue{

type string;

description "this relies on the choice of GlobalRanNodeID";

}

key "GlobalRanNodeIDValue";

min-elements 1;

uses SatelliteBackhaulInfoGrp;

}

leaf-list mdtUserConsentReqList {

type enumeration {

enum M1;

enum M2;

enum M3;

enum M4;

enum M5;

enum M6;

enum M7;

enum M8;

enum M9;

enum MDT_UE_LOCATION;

}

description "represents a list of MDT measurement names that are

subject to user consent at MDT activation.

Any MDT measurement, whose name is not specified in this list, is not

subject to user consent at MDT activation.";

}

list mappedCellIdInfoList {

description "It provides the list of mapping between GEO area and

Mapped Cell ID.";

key "idx";

leaf idx { type uint32 ; }

uses MappedCellIdInfoGrp;

}

leaf aMFSetRef {

type types3gpp:DistinguishedName;

description "This is the DN of AMFSet";

}

}

augment "/me3gpp:ManagedElement" {

list AMFFunction {

description "This IOC represents the AMF functionality in 5GC.

For more information about the AMF, see 3GPP TS 23.501.

The attribute sliceExpiryInfo is used when the validity information of

a network slice need to be configured. The attribute

sliceExpiryInfo.pLMNInfo indicates the network slice to which the

validity information applies.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses AMFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

<CODE ENDS>

*** END OF CHANGE 4 ***

*** START OF CHANGE 5 ***

*** yang-models/_3gpp-5gc-nrm-amfregion.yang ***

<CODE BEGINS>

module _3gpp-5gc-nrm-amfregion {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-amfregion;

prefix amfr3gpp;

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the AMF Region which consists one or

multiple AMF Sets.

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2025-08-18 { reference CR-1557 ; }

revision 2024-05-24 { reference CR-1273 ; }

revision 2023-09-18 { reference CR-1043 ; }

revision 2022-01-07 { reference CR-0643; }

revision 2020-11-06 { reference CR-0412 ; }

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-11 { reference ""; }

grouping AMFRegionGrp {

description "Represents the AMFRegion IOC";

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at

least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC)

and a Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf-list nRTACList {

description "List of Tracking Area Codes (legacy TAC or extended TAC)

where the represented management function is serving.";

reference "TS 38.413 clause 9.3.3.10";

min-elements 1;

config false;

type types5g3gpp:NRTAC;

}

list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

//conditional support only if the network slicing feature is supported.

reference "3GPP TS 23.003";

key "sd sst";

uses types3gpp:SNssai;

}

leaf aMFRegionId {

description "Represents the AMF Region ID, which identifies the region.";

mandatory true;

type types3gpp:AmfRegionId;

}

leaf-list aMFSet {

description "The AMFSet that the AFMRegion is associated with.";

min-elements 1;

type instance-identifier;

}

}

augment "/subnet3gpp:SubNetwork" {

list AMFRegion {

description "5G Core AMFRegion IOC";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses AMFRegionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

<CODE ENDS>

*** END OF CHANGE 5 ***

*** START OF CHANGE 6 ***

*** yang-models/_3gpp-5gc-nrm-amfset.yang ***

<CODE BEGINS>

module _3gpp-5gc-nrm-amfset {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-amfset;

prefix amfset3gpp;

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the AMF Set which consists of some AMFs

that serve a given area and Network Slice.

Copyright 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2025-08-18 { reference CR-1557 ; }

revision 2024-05-24 { reference CR-1273 ; }

revision 2023-09-18 { reference CR-1043 ; }

revision 2022-01-07 { reference CR-0643; }

revision 2020-11-06 { reference CR-0412 ; }

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-11 { reference "Ericsson refactoring."; }

grouping AMFSetGrp {

description "Represents the AMFSet IOC";

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at

least one (the primary PLMN Id). The PLMN Identifier is composed

of a Mobile Country Code (MCC) and a Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf-list nRTACList {

description "List of Tracking Area Codes (legacy TAC or extended TAC)

where the represented management function is serving.";

reference "TS 38.413 clause 9.3.3.10";

min-elements 1;

config false;

type types5g3gpp:NRTAC;

}

list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

//conditional support only if the network slicing feature is supported.

reference "3GPP TS 23.003";

key "sd sst";

uses types3gpp:SNssai;

}

leaf aMFRegion {

description "The AMFRegion that the AFMSet is associated with.";

type instance-identifier;

}

leaf-list aMFSetMemberList {

description "List of DNs of AMFFunction instances of the AMFSet.";

min-elements 1;

max-elements 1;

type types3gpp:DistinguishedName;

}

}

augment "/subnet3gpp:SubNetwork" {

list AMFSet {

description "5G Core AMFSet IOC";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses AMFSetGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

<CODE ENDS>

*** END OF CHANGE 6 ***

*** START OF CHANGE 7 ***

*** yang-models/_3gpp-5gc-nrm-neffunction.yang ***

<CODE BEGINS>

module _3gpp-5gc-nrm-neffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-neffunction;

prefix nef3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-yang-extensions { prefix yext3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-5gc-nrm-nfprofile { prefix nfp3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the NEF function in 5GC. For more

information about the NEF, see 3GPP TS 23.501.

Copyright 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541";

revision 2025-08-18 { reference CR-1557 ; }

revision 2024-05-18 { reference CR-1273 ; }

revision 2024-04-12 { reference CR-1218 ; }

revision 2023-09-18 { reference CR-1103 ; }

revision 2023-09-18 { reference CR-1043 ; }

revision 2023-04-26 { reference CR-0916; }

revision 2022-01-07 { reference CR-0643; }

revision 2020-11-06 { reference CR-0412 ; }

revision 2019-10-20 { reference "initial revision"; }

grouping DnnInfoItemGrp {

description "This data type represents set of parameters supported by NF

for a given S-NSSAI. (See clause 6.1.6.2.97 of TS 29.510)";

leaf dnn {

type string;

mandatory true;

description "It represents supported DNN or Wildcard DNN if the NF

supports all DNNs for the related S-NSSAI. The DNN shall contain the

Network Identifier and it may additionally contain an Operator

Identifier. If the Operator Identifier is not included, the DNN is

supported for all the PLMNs in the plmnList of the NF Profile.";

}

}

grouping SnssaiInfoItemGrp {

description "This data type represents set of parameters supported by NF

for a given S-NSSAI. (See clause 6.1.6.2.97 of TS 29.510)";

list sNssai {

description "It represents the S-NSSAI the NetworkSlice managed object

is supporting. The S-NSSAI is defined in TS 23.003.";

min-elements 1;

key idx;

leaf idx { type uint32 ; }

uses types3gpp:SNssai;

}

list dnnInfoList{

description "It represents list of parameters supported by the

NF per DNN.";

min-elements 1;

key idx;

leaf idx { type uint32 ; }

uses DnnInfoItemGrp;

}

}

grouping AfEventExposureDataGrp {

description "This data type represents the AF Event Exposure data managed

by a given NEF Instance. (See clause 6.1.6.2.50 TS 29.510";

leaf-list afEvents {  // stage 2 double defined

type string;

min-elements 1;

description "It represents AF Event(s) exposed by the NEF after

registration of the AF(s) at the NEF.";

}

leaf-list afIds {

type string;

min-elements 1;

description "It represents list of application function identifiers of

the managed PFDs.";

}

leaf-list appIds {  // stage 2 shaky

type string;

min-elements 1;

description "It represents list of internal application identifiers.";

}

}

grouping PfdDataGrp {

description "This data type represents the list of Application IDs

and/or AF IDs managed by a given NEF Instance.

(See clause 6.1.6.2.49 TS 29.510)";

leaf-list appIds {  // stage 2 shaky

type string;

min-elements 1;

description "It represents list of internal application identifiers.";

}

leaf-list afIds {

type string;

min-elements 1;

description "It represents list of application function identifiers of

the managed PFDs.";

}

}

grouping UnTrustAfInfoGrp {

description "This data type represents information of an untrusted AF

Instance. (See clause 6.1.6.2.95 TS 29.510)";

leaf afId {

type string;

mandatory true;

description "It represents associated AF id.";

}

list sNssaiInfoList {

description "It represents S-NSSAIs and DNNs supported by the AF.";

min-elements 1;

key idx;

leaf idx { type uint32 ; }

uses SnssaiInfoItemGrp;

}

leaf mappingInd {

type boolean;

yext3gpp:initial-value false;

description "When present, this attribute indicates whether the

AF supports mapping between UE IP address (IPv4 address or IPv6 prefix)

and UE ID (i.e. GPSI).

True: the AF supports mapping between UE IP address and UE ID;

False: the AF does not support mapping between UE IP address

and UE ID.";

}

}

grouping NefInfoGrp {

description "This data type represents information of an NEF Instance.

(See clause 6.1.6.2.48 TS 29.510).";

list taiList {

config false;

description "List of TAIs";

yext3gpp:inVariant;

yext3gpp:notNotifyable;

key idx;

leaf idx { type uint32 ; }

uses types3gpp:TaiGrp;

}

list taiRangelist {

config false;

description "Range of TAIs";

yext3gpp:inVariant;

yext3gpp:notNotifyable;

key idx;

leaf idx { type uint32 ; }

uses nfp3gpp:TaiRangeGrp;

}

leaf nefId {

type string;

mandatory true;

description "It represents the NEF ID.

(see clause 6.1.6.3.2 of TS 29.510)";

}

list pfdData {

config false;

description "It represents PFD data, containing the list of internal

application identifiers and/or the list of application function

identifiers for which the PFDs can be provided.

Absence of this attribute indicates that the PFDs for any internal

application identifier and for any application function identifier

can be provided.";

min-elements 1;

key idx;

leaf idx { type uint32 ; }

uses PfdDataGrp;

}

list afEeData {

config false;

description "It represents the AF provided event exposure data. The NEF

registers such information in the NRF on behalf of the AF.";

min-elements 1;

key idx;

leaf idx { type uint32 ; }

uses AfEventExposureDataGrp;

}

list gpsiRanges {

description "It represents list of ranges of GPSIs whose profile data

is available.";

min-elements 1;

key idx;

leaf idx { type uint32 ; }

uses nfp3gpp:IdentityRange;

}

list externalGroupIdentifiersRanges {

description "It represents list of ranges of external groups whose

profile data is available.";  // stage 2 definition is shaky

min-elements 1;

key idx;

leaf idx { type uint32 ; }

uses nfp3gpp:IdentityRange;

}

leaf-list servedFqdnList {

type string;

min-elements 1;

description "It represents pattern (regular expression according to

the ECMA-262 dialect [75]) representing the Domain names served by

the NEF.";

}

leaf-list dnaiList {

type string;

min-elements 1;

description "It represents list of Data network access identifiers

supported by the NEF. The absence of this attribute indicates that

the NEF can be selected for any DNAI.";

}

list unTrustAfInfoList {

description "It represents list of information corresponding to the AFs.";

min-elements 1;

key idx;

leaf idx { type uint32 ; }

uses UnTrustAfInfoGrp;

}

leaf uasNfFunctionalityInd {

type boolean;

yext3gpp:initial-value false;

description "When present, this attribute shall indicate whether the ,

NEF supports UAS NF functionality:

- True: UAS NF functionality is supported by the NEF.

- False (default): UAS NF functionality is not supported by the NEF";

}

leaf multiMemAfSessQosInd {

type boolean;

default false;

description "This attribute indicates whether the NEF supports

Multi-member AF session with required QoS functionality:

- TRUE: Multi-member AF session with required QoS functionality is

supported by the NEF

- FALSE (default): Multi-member AF session with required QoS

functionality is not supported by the NEF.";

}

leaf memberUESelAssistInd {

type boolean;

default false;

description "This attribute indicates whether the NEF supports member

UE selection assistance functionality:

- TRUE: member UE selection assistance functionality is supported by

the NEF

- FALSE (default): member UE selection assistance functionality is

not supported by the NEF.";

}

}

grouping NEFFunctionGrp {

description "Represents the NEFFunction IOC";

uses mf3gpp:ManagedFunctionGrp;

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the

service-based interface.";

type inet:domain-name;

}

list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

key "sd sst";

uses types3gpp:SNssai;

}

list managedNFProfile {

description "This parameter defines profile for managed NF(See TS 23.501)";

min-elements 1;

max-elements 1;

key idx;

uses types3gpp:ManagedNFProfile;

}

leaf-list capabilityList {

description "List of supported capabilities of the NEF.";

reference "3GPP TS 23.003";

type string;

}

leaf isCAPIFSup {

type boolean;

yext3gpp:inVariant;

}

list nefInfo {

description "This attribute represents information of an NEF NF Instance.";

key idx;

leaf idx { type uint32 ; }

uses NefInfoGrp;

}

}

augment "/me3gpp:ManagedElement" {

list NEFFunction {

description "5G Core NEF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NEFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

<CODE ENDS>

*** END OF CHANGE 7 ***

*** START OF CHANGE 8 ***

*** yang-models/_3gpp-5gc-nrm-nfservice.yang ***

<CODE BEGINS>

module _3gpp-5gc-nrm-nfservice {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-nfservice;

prefix nfs3gpp;

import _3gpp-common-yang-types { prefix types3gpp; }

import ietf-yang-types { prefix yang; }

import ietf-inet-types { prefix inet; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "NF service class.

Copyright 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 29.510";

revision 2025-08-18 { reference CR-1557; }

revision 2023-09-18 { reference CR-1043 ; }

revision 2021-01-25 { reference CR-0454 ; }

revision 2020-11-05 { reference CR-0412 ; }

revision 2019-06-17 { reference "initial revision"; }

grouping NFServiceGrp {

description "Represents the NFService IOC";

leaf serviceInstanceID {

description

"Unique ID of the service instance within a given NF Instance.";

mandatory true;

type string;

}

leaf serviceName {

description "Name of the service instance (e.g. 'nudm-sdm').";

mandatory true;

type ServiceName;

}

list versions { //check in review if key is ok (unique)

description "API versions supported by the NF Service and if available,

the corresponding retirement date of the NF Service.";

min-elements 1;

key "apiVersionInUri apiFullVersion";

uses NFServiceVersion;

}

leaf scheme {

description "URI scheme (e.g. 'http', 'https').";

mandatory true;

type UriScheme;

}

leaf nfServiceStatus {

description "Status of the NF Service Instance.";

mandatory true;

type NFServiceStatus;

}

leaf fqdn {

description "FQDN of the NF Service Instance.";

//optional support

type inet:domain-name;

}

leaf interPlmnFqdn {

description "If the NF service needs to be discoverable by other NFs in a

different PLMN, then an FQDN that is used for inter PLMN routing.";

//optional support

type inet:domain-name;

}

list ipEndPoints {

description "IP address(es) and port information of the Network Function

(including IPv4 and/or IPv6 address)where the service is listening

for incoming service requests.";

//optional support

key idx;

leaf idx {

type string;

}

min-elements 1;

uses ipEndPoint;

}

leaf apiPrefix {

description "Optional path segment(s) used to construct the {apiRoot}

variable of the different API URIs.";

//optional support

type string;

}

list defaultNotificationSubscriptions {

description "Notification endpoints for different notification types.";

key notificationType;

//optional support

min-elements 1;

uses types3gpp:DefaultNotificationSubscription;

}

list allowedPlmns {

description "PLMNs allowed to access the service instance.

The absence of this attribute indicates that any PLMN is allowed to

access the service instance.";

min-elements 1;

//optional support

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf-list allowedNfTypes {

description "Type of the NFs allowed to access the service instance.

The absence of this attribute indicates that any NF type is allowed

to access the service instance.";

min-elements 1;

//optional support

type types3gpp:NfType;

}

leaf-list allowedNfDomains {

description "Pattern representing the NF domain names allowed to access the service instance.";

//optional support

min-elements 1;

type string;

}

list allowedNssais {

description "S-NSSAI of the allowed slices to access the service instance.

The absence of this attribute indicates that any slice is allowed to

access the service instance.";

min-elements 1;

//optional support

key "sd sst";

uses types3gpp:SNssai;

}

leaf priority {

description "Priority (relative to other services of the same type)

in the range of 0-65535, to be used for NF Service selection; lower

values indicate a higher priority.";

//optional support

type uint16;

}

leaf capacity {

description "Static capacity information in the range of 0-65535,

expressed as a weight relative to other services of the same type.";

//optional support

type uint16;

}

leaf load {

description "Dynamic load information, ranged from 0 to 100, indicates

the current load percentage of the NF Service.";

//optional support

type types3gpp:Load;

}

leaf recoveryTime {

description "Timestamp when the NF was (re)started.";

//optional support

type yang:date-and-time;

}

list chfServiceInfo { //is the key unique

description "Specific data for a CHF service instance.";

//optional support

max-elements 1;

key "primaryChfServiceInstance secondaryChfServiceInstance";

uses ChfServiceInfo;

}

leaf supportedFeatures {

description "Supported Features of the NF Service instance.";

//optional support

type SupportedFeatures;

}

}

typedef SupportedFeatures {

type string {

pattern '[A-Fa-f0-9]*';

}

}

grouping ipEndPoint {

choice address {

leaf ipv4Address {

type inet:ipv4-address;

}

leaf ipv6Address {

type inet:ipv6-address;

}

leaf ipv6Prefix {

type inet:ipv6-prefix;

}

}

leaf transport {

type TransportProtocol;

}

leaf port {

type uint16;

}

}

typedef TransportProtocol {

type enumeration {

enum TCP;

enum STCP;

enum UDP;

}

}

grouping NFServiceVersion {

leaf apiVersionInUri {

mandatory true;

type string;

}

leaf apiFullVersion {

mandatory true;

type string;

}

leaf expiry {

//optional to support

type yang:date-and-time;

}

}

typedef ServiceName {

type enumeration {

enum NNRF_NFM;

enum NNRF_DISC;

enum NUDM_SDM;

enum NUDM_UECM;

enum NUDM_UEAU;

enum NUDM_EE;

enum NUDM_PP;

enum NAMF_COMM;

enum NAMF_EVTS;

enum NAMF_MT;

enum NAMF_LOC;

enum NSMF_PDUSESSION;

enum NSMF_EVENT-EXPOSURE;

enum NAUSF_AUTH;

enum NAUSF_SORPROTECTION;

enum NNEF_PFDMANAGEMENT;

enum NPCF_AM-POLICY-CONTROL;

enum NPCF_SMPOLICYCONTROL;

enum NPCF_POLICYAUTHORIZATION;

enum NPCF_BDTPOLICYCONTROL;

enum NPCF_EVENTEXPOSURE;

enum NPCF_UE_POLICY_CONTROL;

enum NSMSF_SMS;

enum NNSSF_NSSELECTION;

enum NNSSF_NSSAIAVAILABILITY;

enum NUDR_DR;

enum NLMF_LOC;

enum N5G_EIR_EIC;

enum NBSF_MANAGEMENT;

enum NCHF_SPENDINGLIMITCONTROL;

enum NCHF_CONVERGEDCHARGING;

enum NNWDAF_EVENTSSUBSCRIPTION;

enum NNWDAF_ANALYTICSINFO;

}

}

typedef UriScheme {

type enumeration {

enum HTTP;

enum HTTPS;

}

}

typedef NFServiceStatus {

type enumeration {

enum REGISTERED;

enum SUSPENDED;

enum UNDISCOVERABLE;

}

}

grouping ChfServiceInfo {

leaf primaryChfServiceInstance {

description "Shall be present if the CHF service instance serves as a

secondary CHF instance of another primary CHF service instance.";

//conditional to support

type string;

}

leaf secondaryChfServiceInstance {

description "Shall be present if the CHF service instance serves as a

primary CHF instance of another secondary CHF service instance.";

//conditional to support

type string;

}

}

}

<CODE ENDS>

*** END OF CHANGE 8 ***

*** START OF CHANGE 9 ***

*** yang-models/_3gpp-5gc-nrm-ngeirfunction.yang ***

<CODE BEGINS>

module _3gpp-5gc-nrm-ngeirfunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-ngeirfunction;

prefix ngeir3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the 5G-EIR function in 5GC. For more

information about the 5G-EIR, see 3GPP TS 23.501.

Copyright 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541";

revision 2023-09-18 { reference CR-1043 ; }

revision 2022-01-07 { reference CR-0643; }

revision 2020-11-06 { reference CR-0412 ; }

revision 2019-10-25 { reference "S5-194457 S5-195427 S5-193518"; }

revision 2019-05-15 {reference "initial revision"; }

grouping NGEIRFunctionGrp {

description "Represents the NGEIRFunction IOC";

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at

least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and

a Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

//optional support

reference "3GPP TS 23.003";

key "sd sst";

uses types3gpp:SNssai;

}

list managedNFProfile {

key idx;

min-elements 1;

max-elements 1;

uses types3gpp:ManagedNFProfile;

}

list commModelList {

min-elements 1;

key "groupId";

description "Specifies a list of commModel. It can be used by NF and

NF services to interact with each other in 5G Core network ";

reference "3GPP TS 23.501";

uses types5g3gpp:CommModel;

}

}

augment "/me3gpp:ManagedElement" {

list NGEIRFunction {

description "5G Core NGEIR Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NGEIRFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

<CODE ENDS>

*** END OF CHANGE 9 ***

*** START OF CHANGE 10 ***

*** yang-models/_3gpp-5gc-nrm-nrffunction.yang ***

<CODE BEGINS>

module _3gpp-5gc-nrm-nrffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-nrffunction;

prefix nrf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5gc-nrm-nfprofile { prefix nfp3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the NRF function in 5GC.

For more information about the NRF, see 3GPP TS 23.501 [2].

Copyright 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541";

revision 2025-08-18 { reference CR-1557; }

revision 2023-09-18 { reference CR-1043 ; }

revision 2023-04-26 { reference CR-0916; }

revision 2020-11-08 { reference CR-0412 ; }

revision 2020-08-03 { reference "CR-0321"; }

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-05-15 { reference "initial revision"; }

grouping NRFFunctionGrp {

description "Represents the NRFFunction IOC";

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at

least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a

Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the service-based

interface.";

type inet:domain-name;

}

leaf-list cNSIIdList {

description "NSI ID. NSI ID is an identifier for identifying the Core

Network part of a Network Slice instance when multiple Network Slice

instances of the same Network Slice are deployed, and there is a need

to differentiate between them in the 5GC, see clause 3.1 of TS 23.501

and subclause 6.1.6.2.7 of 3GPP TS 29.531";

type string;

}

list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

//optional support

reference "3GPP TS 23.003";

key "sd sst";

uses types3gpp:SNssai;

}

list nFProfileList {

description "Set of NFProfile(s) to be registered in the NRF instance.";

//optional support

key nfInstanceID;

uses nfp3gpp:NFProfileGrp;

}

}

augment "/me3gpp:ManagedElement" {

list NRFFunction {

description "5G Core NRF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NRFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

<CODE ENDS>

*** END OF CHANGE 10 ***

*** START OF CHANGE 11 ***

*** yang-models/_3gpp-5gc-nrm-nssffunction.yang ***

<CODE BEGINS>

module _3gpp-5gc-nrm-nssffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-nssffunction;

prefix nssf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the NSSF function in 5GC. For more

information about the NSSF, see 3GPP TS 23.501.

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541";

revision 2025-08-18 { reference CR-1557; }

revision 2023-09-18 { reference CR-1043 ; }

revision 2023-04-26 { reference CR-0916; }

revision 2020-11-08 { reference CR-0412 ; }

revision 2020-08-03 { reference "CR-0321"; }

revision 2019-10-25 { reference "S5-194457 S5-195427 S5-193518"; }

revision 2019-05-15 { reference "initial revision"; }

grouping NSSFFunctionGrp {

description "Represents the NSSFFunction IOC";

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at least

one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a

Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the service-based

interface.";

type inet:domain-name;

}

list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

reference "3GPP TS 23.003";

key "sd sst";

uses types3gpp:SNssai;

}

leaf-list cNSIIdList {

description "NSI ID. NSI ID is an identifier for identifying the Core

Network part of a Network Slice instance when multiple Network Slice

instances of the same Network Slice are deployed, and there is a need

to differentiate between them in the 5GC, see clause 3.1 of TS 23.501

and subclause 6.1.6.2.7 of 3GPP TS 29.531";

type string;

}

list managedNFProfile {

key idx;

min-elements 1;

max-elements 1;

description "Profile definition of a Managed NF (See TS 23.501)";

uses types3gpp:ManagedNFProfile;

}

}

augment "/me3gpp:ManagedElement" {

list NSSFFunction {

description "5G Core NSSF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NSSFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

<CODE ENDS>

*** END OF CHANGE 11 ***

*** START OF CHANGE 12 ***

*** yang-models/_3gpp-5gc-nrm-nwdaffunction.yang ***

<CODE BEGINS>

module _3gpp-5gc-nrm-nwdaffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-nwdaffunction;

prefix nwdaf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-yang-extensions { prefix yext3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-common-top { prefix top3gpp; }

import _3gpp-5gc-nrm-nfprofile { prefix nfp3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the NWDAF function in 5GC. For more

information about the NWDAF, see 3GPP TS 23.501.

Copyright 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541";

revision 2025-08-18 { reference CR-1557; }

revision 2024-08-06 { reference CR-1130; }

revision 2024-04-12 { reference CR-1218; }

revision 2023-09-18 { reference CR-1043; }

revision 2023-04-26 { reference CR-0916; }

revision 2020-11-08 { reference CR-0412; }

revision 2019-10-25 { reference "S5-194457 S5-195427 S5-193518"; }

revision 2019-05-15 {reference "initial revision"; }

typedef NwdafEvent {

description "The detailed ENUM value for NwdafEvent see the

Table 5.1.6.3.4-1 in TS 29.520";

type union {

type enumeration {

enum SLICE_LOAD_LEVEL;

enum NETWORK_PERFORMANCE;

enum NF_LOAD;

enum SERVICE_EXPERIENCE;

enum UE_MOBILITY;

enum UE_COMMUNICATION;

enum QOS_SUSTAINABILITY;

enum ABNORMAL_BEHAVIOUR;

enum USER_DATA_CONGESTION;

enum NSI_LOAD_LEVEL;

enum DN_PERFORMANCE;

enum DISPERSION;

enum RED_TRANS_EXP;

enum WLAN_PERFORMANCE;

enum SM_CONGESTION;

}

type string;

}

}

grouping NwdafCapabilityGrp {

description "This data type represents the capability supported by the

NWDAF.";

reference "TS 29.510";

leaf analyticsAggregation {

type boolean ;

default false;

description "It indicates whether the NWDAF supports analytics

aggregation";

}

leaf analyticsMetadataProvisioning {

type boolean ;

default false;

description "It indicate whether the NWDAF supports analytics metadata

provisioning:";

}

}

grouping MlAnalyticsInfoGrp {

description "This data type represents ML Analytics Filter information

supported by the Nnwdaf_MLModelProvision service.";

reference "TS 29.510";

leaf-list mlAnalyticsIds {

type NwdafEvent;

ordered-by user;

description "This attribute represents the Analytic functionalities

(identified by nwdafEvent defined in TS 29.520 [85]) of the NWDAF

instance. MnS consumer can configure this attribute to specify

which Analytic functionalities (identified by nwdafEvent) can be

performed the NWDAF instance. If the value of this attribute is not

present, the NWDAF instance can perform any NWDAFEvents

Analytics Id(s) supported by the Nnwdaf_MLModelProvision service,

if none are provided the NWDAF can serve any mlAnalyticsId.";

}

list sNSSAIList {

min-elements 1;

description "List of S-NSSAIs the managed object is capable of

supporting.";

reference "3GPP TS 23.003";

key "sd sst";

uses types3gpp:SNssai;

}

list trackingAreaList {

description "This attribute represents area of Interest of the ML model,

if none are provided the ML model for the analytics can apply to any

TAIs.

If present, it represents the list of TAIs, it may contain one or

more non-3GPP access TAIs.";

key idx ;

leaf idx { type uint32; }

uses types3gpp:TaiGrp;

}

leaf-list mlModelInterInfo {

type string {

pattern "[0-9]{6}";

}

description "This attribute defines the list of NWDAF vendors that are

allowed to retrieve ML models from the NWDAF containing MTLF. The

absence of this attribute indicates that none of the NWDAF vendors

can retrieve the ML models.

allowedValues: 6 decimal digits; if the SMI code has less than

6 digits, it shall be padded with leading digits '0' to complete a

6-digit string value.";

}

leaf flCapabilityType {

type enumeration {

enum FL_SERVER {

description "NWDAF containing MTLF as Federated Learning Server";

}

enum FL_CLIENT {

description "NWDAF containing MTLF as Federated Learning Client";

}

enum FL_SERVER_AND_CLIENT {

description "NWDAF containing MTLF as Federated Learning Server and

Client.";

}

}

description "This attribute defines the federated learning capability

type supported by NWDAF containing MTLF.";

}

list flTimeInterval {

key idx ;

leaf idx { type uint32; }

description "This attribute defines the time window at which the

indicated flCapabilityType supported by NWDAF MTLF is available.

This attribute shall be present only if flCapabilityType attribute

is present.";

uses types3gpp:TimeWindowGrp;

}

}

grouping NwdafInfoGrp {

description "This data type represents specific data for the NWDAF.";

reference "TS 29.510";

leaf-list nwdafEvents {

type NwdafEvent;

ordered-by user;

description "This attribute represents the Analytic functionalities

(identified by nwdafEvent defined in TS 29.520) of the NWDAF instance.

MnS consumer can configure this attribute to specify which Analytic

functionalities (identified by nwdafEvent) can be performed the NWDAF

instance. If the value of this attribute is not present, the NWDAF

instance can perform any NWDAFEvents.

The detailed ENUM value for NwdafEvent see the Table 5.1.6.3.4-1

in TS 29.520";

}

leaf-list eventIds {

type string ;

min-elements 1;

description "It represents the EventId(s) supported by the

Nnwdaf_AnalyticsInfo service, if none are provided the NWDAF can serve

any eventId. (see clause TS 29.520)";

}

list taiList {

description "The list of TAIs. ";

min-elements 1;

key idx ;

leaf idx { type uint32; }

uses types3gpp:TaiGrp;

}

list taiRangeList {

min-elements 1;

key idx ;

leaf idx { type uint32; }

description "The range of TAIs.";

uses nfp3gpp:TaiRangeGrp;

}

list nwdafCapability {

max-elements 1;

key idx ;

leaf idx { type uint32; }

description "This attribute indicates the capability of the NWDAF.

If not present, the NWDAF shall be regarded with no capability.";

uses NwdafCapabilityGrp;

}

leaf analyticsDelay {

type int32 ;

units seconds;

description "It represents the supported Analytics Delay related to the

eventIds and nwdafEvents.

It is an unsigned integer identifying a period of time in units of

seconds.(see clause 5.2.2 TS 29.571).";

}

leaf-list servingNfTypeList {

type types3gpp:NfType;

min-elements 1;

description "It contains the list of NF type(s) from which the NWDAF NF

can collect data. The absence of this attribute indicates that the

NWDAF can collect data from any NF type.";

}

leaf-list servingNfSetIdList {

type string ;

min-elements 1;

description "It contains the list of NF type(s) from which the NWDAF NF

can collect data. The absence of this attribute indicates that the

NWDAF can collect data from any NF type. (see clause 5.4.2 NfSetId

in TS 29.571)";

}

list mlAnalyticsList {

min-elements 1;

key idx ;

leaf idx { type uint32; }

description "It represents ML Analytics Filter information supported by

the Nnwdaf_MLModelProvision service.";

uses MlAnalyticsInfoGrp;

}

}

grouping NetworkSliceInfoGrp {

description "Represents information of network slice when the NWDAF is

authorized to collect the management data of the network slice. ";

list sNSSAI {

description "It represents the S-NSSAI the NetworkSlice managed object

is supporting. The S-NSSAI is defined in TS 23.003 ";

key idx;

min-elements 1;

max-elements 1;

leaf idx {

type string;

}

uses types3gpp:SNssai;

}

leaf-list cNSIId {

type string;

description "It represents NSI ID which is an identifier for identifying

the Core Network part of a Network Slice instance when multiple

Network Slice instances of the same Network Slice are deployed,

and there is a need to differentiate between them in the 5GC.

See NSI ID definition in clause 3.1 of TS 23.501 and

subclause 6.1.6.2.7 of TS 29.531.";

}

leaf networkSliceRef {

type types3gpp:DistinguishedName;

mandatory true;

description "This holds a DN of the NetworkSlice managed object relating

to the NetworkSlice instance differentiated by sNSSAI and optional

cNSIId.";

}

}

grouping NWDAFFunctionGrp {

description "The NWDAFFunction IOC includes attributes inherited from

ManagedFunction IOC (defined in TS 28.622)";

uses mf3gpp:ManagedFunctionGrp;

list pLMNInfoList {

description "It defines the PLMN(s) of a Network Function.";

min-elements 1;

key "idx";

leaf idx { type uint32; }

uses types3gpp:PLMNId;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the service-based

interface.";

type inet:domain-name;

}

list managedNFProfile {

key idx;

min-elements 1;

max-elements 1;

description "Profile definition of a Managed NF (See TS 23.501)";

uses types3gpp:ManagedNFProfile;

}

list commModelList {

min-elements 1;

key "groupId";

description "Specifies a list of commModel. It can be used by NF and

NF services to interact with each other in 5G Core network ";

reference "3GPP TS 23.501";

uses types5g3gpp:CommModel;

}

list networkSliceInfoList {

key idx;

min-elements 1;

description "The attribute specifies a list of NetworkSliceInfo which

is defined as a datatype (see clause 5.3.95). It is used by an

authorized consumer, e.g. NWDAF to facilitate the data collection

from OAM.";

leaf idx {

type string;

}

uses NetworkSliceInfoGrp;

yext3gpp:inVariant;

}

list nwdafInfo {

min-elements 1;

max-elements 1;

key idx;

leaf idx { type uint32; }

description "It represents specific data for the NWDAF.";

uses NwdafInfoGrp;

}

leaf administrativeState {

type types3gpp:BasicAdministrativeState ;

mandatory true;

description "This attribute determines whether the NWDAF is enabled or

disabled. MnS consumer can configure this attribute to activate or

de-activate the analytic functionalities (identified by nwdafEvent

defined in TS 29.520) of the NWDAF instance.";

}

leaf nwdafLogicalFuncSupported {

type enumeration {

enum NWDAF_WITH_ANLF {

description "Indicates the NWDAF containing Analytics logical

function (AnLF)";

}

enum NWDAF_WITH_MTLF {

description "Indicates the NWDAF containing Model Training logical

function (MTLF).";

}

enum NWDAF_WITH_ANLF_MTLF {

description "Indicates the NWDAF containing both Analytics logical

function (AnLF) and Model Training logical function (MTLF).";

}

}

config false;

description "It represents the logical functions supported by the NWDAF.

If not present, the NWDAF shall be regarded with no logical

decomposition, in that case the NWDAF only supports the analytics

services.";

}

}

augment "/me3gpp:ManagedElement" {

list NWDAFFunction {

description "5G Core NWDAF Function.

For more information about the NWDAF, see TS 23.501. Several

attributes (including 'nwdafInfo', 'administrativeState' and

'ManagedNFProfile.servingScope') are used to control the functionalities

(identified by nwdafEvent defined in TS 29.520) of the NWDAF instance.

The attribute 'ManagedNFProfile.servingScope' is used to represent

specified certain geographical area(s) can be served by the NWDAF

instance.

The attribute 'NwdafInfo.taiList' and 'NwdafInfo.taiRangeList' is used

to represent specified certain tracking area(s) can be served by the

NWDAF instance.";

reference "3GPP TS 28.541 3GPP TS 23.501";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NWDAFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

<CODE ENDS>

*** END OF CHANGE 12 ***

*** START OF CHANGE 13 ***

*** yang-models/_3gpp-5gc-nrm-pcffunction.yang ***

<CODE BEGINS>

module _3gpp-5gc-nrm-pcffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-pcffunction;

prefix pcf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the PCF function in 5GC. For more

information about the PCF, see 3GPP TS 23.501.

Copyright 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541";

revision 2025-08-18 { reference CR-1557; }

revision 2024-05-24 { reference CR-1273 ; }

revision 2023-09-18 { reference CR-1043 ; }

revision 2023-04-26 { reference CR-0916; }

revision 2020-11-08 { reference CR-0412 ; }

revision 2020-08-06 { reference "CR-0333"; }

revision 2020-08-06 { reference "CR-0331"; }

revision 2019-10-25 { reference "S5-194457 S5-193518"; }

revision 2019-05-22 { reference "initial revision"; }

grouping PCFFunctionGrp {

description "Represents the PCFFunction IOC";

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at

least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a

Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the service-based

interface.";

type inet:domain-name;

}

list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

//optional support

reference "3GPP TS 23.003";

key "sd sst";

uses types3gpp:SNssai;

}

list managedNFProfile {

key idx;

min-elements 1;

max-elements 1;

description "Profile definition of a Managed NF (See TS 23.501)";

uses types3gpp:ManagedNFProfile;

}

list commModelList {

min-elements 1;

key "groupId";

description "Specifies a list of commModel. It can be used by NF and

NF services to interact with each other in 5G Core network ";

reference "3GPP TS 23.501";

uses types5g3gpp:CommModel;

}

leaf dynamic5QISetRef {

type types3gpp:DistinguishedName;

description "DN of the Dynamic5QISet that the PCFFunction supports

(is associated to).";

}

leaf configurable5QISetRef {

type types3gpp:DistinguishedName;

description "DN of the Configurable5QISet that the PCFFunction supports

(is associated to).";

}

}

augment "/me3gpp:ManagedElement" {

list PCFFunction {

description "5G Core PCF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses PCFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

<CODE ENDS>

*** END OF CHANGE 13 ***

*** START OF CHANGE 14 ***

*** yang-models/_3gpp-5gc-nrm-smffunction.yang ***

<CODE BEGINS>

module _3gpp-5gc-nrm-smffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-smffunction;

prefix smf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "SMFFunction derived from basic ManagedFunction.

Copyright 2024, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541";

revision 2025-08-18 { reference CR-1557; }

revision 2024-05-24 { reference CR-1273 ; }

revision 2023-09-18 { reference CR-1043 ; }

revision 2023-04-26 { reference CR-0916; }

revision 2020-11-08 { reference CR-0412 ; }

revision 2020-08-06 { reference "CR-0333"; }

revision 2020-06-03 { reference "CR-0286"; }

revision 2019-10-25 { reference "S5-194457 S5-193518"; }

revision 2019-05-31 {reference "Ericsson refactoring."; }

revision 2018-08-07 { reference "Initial revision";}

grouping SMFFunctionGrp {

description "Represents the SMFFuntion IOC";

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

min-elements 1;

description "A list of PLMN identifiers (Mobile Country Code and Mobile

Network Code).";

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf-list nRTACList {

description "List of Tracking Area Codes (legacy TAC or extended TAC)

where the represented management function is serving.";

reference "TS 38.413 clause 9.3.3.10";

min-elements 1;

config false;

type types5g3gpp:NRTAC;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the service-based

interface.";

type inet:domain-name;

}

list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

reference "3GPP TS 23.003";

key "sd sst";

uses types3gpp:SNssai;

}

list managedNFProfile {

key idx;

min-elements 1;

max-elements 1;

description "Profile definition of a Managed NF (See TS 23.501)";

uses types3gpp:ManagedNFProfile;

}

list commModelList {

min-elements 1;

key "groupId";

description "Specifies a list of commModel. It can be used by NF and

NF services to interact with each other in 5G Core network ";

reference "3GPP TS 23.501";

uses types5g3gpp:CommModel;

}

leaf configurable5QISetRef {

type types3gpp:DistinguishedName;

description "DN of the Configurable5QISet that the SMFFunction supports

(is associated to).";

}

leaf dynamic5QISetRef {

type types3gpp:DistinguishedName;

description "DN of the Dynamic5QISet that the SMFFunction supports

(is associated to).";

}

list dnaiSatelliteMappingList {

description "List of the mapping relationship between

satellite ID and at least one DNAI.";

min-elements 1;

key "satelliteid";

uses dnaiSatelliteMapping;

}

}

grouping dnaiSatelliteMapping {

leaf-list dnaiList {

description "List of Data network access identifiers.";

min-elements 1;

type string;

}

leaf satelliteid {

description "Unique identifier of a GEO satellite.";

type string;

}

}

augment "/me3gpp:ManagedElement" {

list SMFFunction {

description "5G Core SMF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses SMFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

<CODE ENDS>

*** END OF CHANGE 14 ***

*** START OF CHANGE 15 ***

*** yang-models/_3gpp-5gc-nrm-udmfunction.yang ***

<CODE BEGINS>

module _3gpp-5gc-nrm-udmfunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-udmfunction;

prefix udm3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the UDM function in 5GC. For more

information about the UDM, see 3GPP TS 23.501.

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541";

revision 2025-08-18 { reference CR-1557; }

revision 2024-04-04 { reference CR-1139; }

revision 2023-09-18 { reference CR-1043 ; }

revision 2023-04-26 { reference CR-0916; }

revision 2020-11-08 { reference CR-0412 ; }

revision 2019-10-25 { reference "S5-194457 S5-195427 S5-193518"; }

revision 2019-05-22 { reference "initial revision";}

grouping UDMFuntionGrp {

description "Represents the UDMFuntion IOC";

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at

least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a

Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the service-based

interface.";

type inet:domain-name;

}

list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

//optional support

reference "3GPP TS 23.003";

key "sd sst";

uses types3gpp:SNssai;

}

list managedNFProfile {

key idx;

min-elements 1;

max-elements 1;

description "Profile definition of a Managed NF (See TS 23.501)";

uses types3gpp:ManagedNFProfile;

}

list commModelList {

min-elements 1;

key "groupId";

description "Specifies a list of commModel. It can be used by NF and

NF services to interact with each other in 5G Core network ";

reference "3GPP TS 23.501";

uses types5g3gpp:CommModel;

}

leaf-list mdtUserConsentReqList {

type enumeration {

enum M1;

enum M2;

enum M3;

enum M4;

enum M5;

enum M6;

enum M7;

enum M8;

enum M9;

enum MDT_UE_LOCATION;

}

description "represents a list of MDT measurement names that are

subject to user consent at MDT activation.

Any MDT measurement, whose name is not specified in this list, is not

subject to user consent at MDT activation.";

}

}

augment "/me3gpp:ManagedElement" {

list UDMFunction {

description "5G Core UDM Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses UDMFuntionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

<CODE ENDS>

*** END OF CHANGE 15 ***

*** START OF CHANGE 16 ***

*** yang-models/_3gpp-5gc-nrm-udrfunction.yang ***

<CODE BEGINS>

module _3gpp-5gc-nrm-udrfunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-udrfunction;

prefix udr3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the UDR function in 5GC. For more information

about the UDR, see 3GPP TS 23.501.

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541";

revision 2025-08-18 { reference CR-1557; }

revision 2023-09-18 { reference CR-1043 ; }

revision 2023-04-26 { reference CR-0916; }

revision 2020-11-08 { reference CR-0412 ; }

revision 2019-10-25 { reference "S5-194457 S5-195427 S5-193518"; }

revision 2019-05-22 {reference "initial revision"; }

grouping UDRFuntionGrp {

description "Representse the UDRFuntion IOC";

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at

least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a

Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the service-based

interface.";

type inet:domain-name;

}

list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

//optional support

reference "3GPP TS 23.003";

key "sd sst";

uses types3gpp:SNssai;

}

list managedNFProfile {

key idx;

min-elements 1;

max-elements 1;

description "Profile definition of a Managed NF (See TS 23.501)";

uses types3gpp:ManagedNFProfile;

}

}

augment "/me3gpp:ManagedElement" {

list UDRFunction {

description "5G Core UDR Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses UDRFuntionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

<CODE ENDS>

*** END OF CHANGE 16 ***

*** START OF CHANGE 17 ***

*** yang-models/_3gpp-5gc-nrm-udsffunction.yang ***

<CODE BEGINS>

module _3gpp-5gc-nrm-udsffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-udsffunction;

prefix udsf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import ietf-inet-types { prefix inet; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3gpp SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "This IOC represents the UDSF function which can be interacted

with any other 5GC NF defined in 3GPP TS 23.501.

Copyright 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541";

revision 2025-08-18 { reference CR-1557; }

revision 2023-09-18 { reference CR-1043 ; }

revision 2023-04-26 { reference CR-0916; }

revision 2020-11-08 { reference CR-0412 ; }

revision 2019-10-25 { reference "S5-194457 S5-195427 S5-193518"; }

revision 2019-05-22 { reference "initial revision"; }

grouping UDSFFuntionGrp {

description "Represents the UDSFFuntion IOC";

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at

least one (the primary PLMN Id).

The PLMN Identifier is composed of a Mobile Country Code (MCC) and a

Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf sBIFQDN {

description "The FQDN of the registered NF instance in the

service-based interface.";

type inet:domain-name;

}

list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

//optional support

reference "3GPP TS 23.003";

key "sd sst";

uses types3gpp:SNssai;

}

list managedNFProfile {

key idx;

min-elements 1;

max-elements 1;

description "Managed Network Function profile";

reference "3GPP TS 23.501";

uses types3gpp:ManagedNFProfile;

}

}

augment "/me3gpp:ManagedElement" {

list UDSFFunction {

description "5G Core UDSF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses UDSFFuntionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;    }

}

}

<CODE ENDS>

*** END OF CHANGE 17 ***

*** START OF CHANGE 18 ***

*** yang-models/_3gpp-5gc-nrm-upffunction.yang ***

<CODE BEGINS>

module _3gpp-5gc-nrm-upffunction {

yang-version 1.1;

namespace urn:3gpp:sa5:_3gpp-5gc-nrm-upffunction;

prefix upf3gpp;

import _3gpp-common-managed-function { prefix mf3gpp; }

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-5g-common-yang-types { prefix types5g3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "UPFFunction derived from basic ManagedFunction.

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2025-08-18 { reference CR-1557; }

revision 2024-05-24 { reference CR-1273 ; }

revision 2023-09-18 { reference CR-1043 ; }

revision 2023-04-26 { reference CR-0916; }

revision 2020-11-08 { reference CR-0412 ; }

revision 2019-10-25 { reference "S5-194457 S5-193518"; }

revision 2019-05-31 { reference "Ericsson refactoring."; }

revision 2018-08-07 { reference "Initial revision"; }

grouping UPFFunctionGrp {

description "Represents the UPFFunction IOC";

uses mf3gpp:ManagedFunctionGrp;

list pLMNIdList {

description "A list of PLMN identifiers (Mobile Country Code and Mobile

Network Code).";

min-elements 1;

key "mcc mnc";

uses types3gpp:PLMNId;

}

leaf-list nRTACList {

description "List of Tracking Area Codes (legacy TAC or extended TAC)

where the represented management function is serving.";

reference "TS 38.413 clause 9.3.3.10";

min-elements 1;

config false;

type types5g3gpp:NRTAC;

}

list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of supporting.

(Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

reference "3GPP TS 23.003";

key "sd sst";

uses types3gpp:SNssai;

}

list managedNFProfile {

key idx;

min-elements 1;

max-elements 1;

description "Profile definition of a Managed NF (See TS 23.501)";

uses types3gpp:ManagedNFProfile;

}

leaf-list supportedBMOList {

type string;

description "List of supported BMOs (Bridge Managed Objects) required

for integration with TSN system.";

}

}

augment /me3gpp:ManagedElement {

list UPFFunction {

description "5G Core UPF Function";

reference "3GPP TS 28.541";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses UPFFunctionGrp;

}

uses mf3gpp:ManagedFunctionContainedClasses;

}

}

}

<CODE ENDS>

*** END OF CHANGE 18 ***

*** START OF CHANGE 23 ***

*** yang-models/_3gpp-nr-nrm-eutranfrequency.yang ***

<CODE BEGINS>

module _3gpp-nr-nrm-eutranfrequency {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-eutranfrequency";

prefix "eutraneteutranfreq3gpp";

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-nr-nrm-gnbcucpfunction { prefix gnbcucp3gpp; }

import _3gpp-nr-nrm-eutranetwork { prefix eutranet3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the EUtranFrequency Information

Object Class (IOC), that is part of the NR Network Resource Model (NRM).

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541 5G Network Resource Model (NRM),

3GPP TS 28.658 (E-UTRAN) Network Resource Model (NRM)";

revision 2025-04-25 { reference "CR-1526 CR-1527" ; } // common for r18, r19

revision 2023-09-18 { reference CR-1043 ; }

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-17 { reference "Initial revision"; }

grouping EUtranFrequencyGrp {

description "Represents the EUtranFrequency IOC.";

reference "3GPP TS 28.541";

leaf earfcnDL {

description "Specifies the channel number for the central DL frequency.";

reference "3GPP TS 36.101";

mandatory true;

type uint32 { range "0..262143"; }

}

leaf-list multiBandInfoListEutra {

description "List of additional frequency bands the frequency belongs to.";

config false;

min-elements 0;

type uint16 { range "1..256"; }

}

}

grouping EUtranFrequencyWrapper {

list EUtranFrequency {

description "Represents certain E-UTRAN frequency properties.";

reference "3GPP TS 28.658";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses EUtranFrequencyGrp;

}

}

}

feature EUtranFrequencyUnderSubNetwork {

description "The EUtranFrequency shall be contained under SubNetwork";

}

augment "/subnet3gpp:SubNetwork" {

if-feature EUtranFrequencyUnderSubNetwork ;

uses EUtranFrequencyWrapper;

}

feature EUtranFrequencyUnderEUtraNetwork {

description "The EUtranFrequency shall be contained under EUtraNetwork";

}

augment "/me3gpp:ManagedElement/gnbcucp3gpp:GNBCUCPFunction/"

+ "eutranet3gpp:EUtraNetwork" {

if-feature EUtranFrequencyUnderEUtraNetwork;

uses EUtranFrequencyWrapper;

}

}

<CODE ENDS>

*** END OF CHANGE 23 ***

*** START OF CHANGE 24 ***

*** yang-models/_3gpp-nr-nrm-nrfrequency.yang ***

<CODE BEGINS>

module _3gpp-nr-nrm-nrfrequency {

yang-version 1.1;

namespace "urn:3gpp:sa5:_3gpp-nr-nrm-nrfrequency";

prefix "nrfreq3gpp";

import _3gpp-common-managed-element { prefix me3gpp; }

import _3gpp-nr-nrm-gnbcucpfunction { prefix gnbcucp3gpp; }

import _3gpp-nr-nrm-nrnetwork { prefix nrnet3gpp; }

import _3gpp-common-subnetwork { prefix subnet3gpp; }

import _3gpp-common-top { prefix top3gpp; }

organization "3GPP SA5";

contact "https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Defines the YANG mapping of the NRFrequency Information Object

Class (IOC) that is part of the NR Network Resource Model (NRM).

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541 5G Network Resource Model (NRM)";

revision 2025-04-25 { reference "CR-1526 CR-1527"; } // common for r18, r19

revision 2023-09-18 { reference CR-1043 ; }

revision 2019-10-28 { reference S5-193518 ; }

revision 2019-06-17 { reference "Initial version"; }

grouping NRFrequencyGrp {

description "Represents the NRFrequency IOC.";

leaf absoluteFrequencySSB {

description "The absolute frequency applicable for a downlink NR carrier

frequency associated with the SSB, in terms of NR-ARFCN.";

mandatory true;

type uint32 { range "0.. 3279165"; }

}

leaf ssBSubCarrierSpacing {

description "Sub-carrier spacing of the SSB.

Note that the allowed values of SSB used for representing data,

by e.g. a BWP, are: 15, 30, 60 and 120 in units of kHz.";

mandatory true;

type uint8 { range "15 | 30 | 60 | 120 | 240"; }

units "kHz";

}

leaf multiFrequencyBandListNR {

description "List of additional frequency bands the frequency belongs to.

The list is automatically set by the gNB.";

mandatory true;

config false;

type uint16 { range "1..256"; }

}

}

grouping NRFrequencyWrapper {

list NRFrequency {

description "Represents certain NR frequency properties.";

key id;

uses top3gpp:Top_Grp;

container attributes {

uses NRFrequencyGrp;

}

}

}

feature NRFrequencyUnderNRNetwork {

description "NRFrequency shall be contained under NRNetwork";

}

augment "/me3gpp:ManagedElement/gnbcucp3gpp:GNBCUCPFunction/"

+ "nrnet3gpp:NRNetwork" {

if-feature NRFrequencyUnderNRNetwork;

uses NRFrequencyWrapper;

}

feature NRFrequencyUnderSubNetwork {

description "NRFrequency shall be contained under SubNetwork";

}

augment "/subnet3gpp:SubNetwork" {

if-feature NRFrequencyUnderSubNetwork;

uses NRFrequencyWrapper;

}

}

<CODE ENDS>

*** END OF CHANGE 24 ***

*** START OF CHANGE 25 ***

*** yang-models/_3gpp-ns-nrm-serviceprofile.yang ***

<CODE BEGINS>

submodule _3gpp-ns-nrm-serviceprofile {

yang-version 1.1;

belongs-to _3gpp-ns-nrm-networkslice { prefix ns3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

import _3gpp-ns-nrm-common { prefix ns3cmn; }

organization "3GPP SA5";

contact

"https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "A network slice instance in a 5G network.

Copyright 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541

Management and orchestration;

5G Network Resource Model (NRM);

Information model definitions for network slice NRM (chapter 6)

";

revision 2025-08-18 { reference CR-1557; }

revision 2023-09-18 { reference CR-1043 ; }

revision 2020-06-02 {

reference "CR-0485, CR-0508";

}

revision 2020-02-19 {

description "Introduction of YANG definitions for network slice NRM";

reference "CR-0458";

}

revision 2019-06-23 {

description "Initial revision";

reference "3GPP TS 28.541 V15.X.XX";

}

typedef availability-percentage {

description "

Percentage value of the amount of time the end-to-end communication

service is delivered according to an agreed QoS, divided by the amount

of time the system is expected to deliver the end-to-end service

according to the specification in a specific area.";

reference "3GPP TS 22.261 3.1";

type decimal64 {

fraction-digits 4; // E.g. 99.9999

range 0..100;

}

}

typedef V2XMode-enum {

type enumeration {

enum NOT_SUPPORTED;

enum SUPPORTED_BY_NR;

}

}

grouping ServiceProfileGrp {

leaf serviceProfileId {

description "Service profile identifier.";

type types3gpp:DistinguishedName;

}

list sNSSAIList {

description "The S-NSSAI list to be supported by the new NSI to be

created or the existing NSI to be re-used.";

min-elements 1;

key idx;

unique "sst sd";

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses types3gpp:SNssai;

}

list pLMNIdList {

description "List of PLMN IDs.";

min-elements 1;

key "mcc mnc";

ordered-by user;

uses types3gpp:PLMNId;

}

leaf maxNumberofUEs {

description "The maximum number of UEs that may simultaneously

access the network slice instance.";

mandatory true;

type uint64;

}

leaf-list coverageArea {

min-elements 1;

description "A list of TrackingAreas where the NSI can be selected.";

type types3gpp:Tac;

}

leaf latency {

description "The packet transmission latency (milliseconds) through

the RAN, CN, and TN part of 5G network, used to evaluate utilization

performance of the end-to-end network slice instance.";

reference "3GPP TS 28.554 clause 6.3.1";

mandatory true;

type uint16;

units milliseconds;

}

leaf uEMobilityLevel {

description "The mobility level of UE accessing the network slice

instance.";

reference "3GPP TS 22.261 clause 6.2.1";

type types3gpp:UeMobilityLevel;

}

leaf resourceSharingLevel {

description "Specifies whether the resources to be allocated to the

network slice instance may be shared with another network slice

instance(s).";

type types3gpp:ResourceSharingLevel;

}

//Stage2 issue: The sNSSAIList above specifies one or potentially

//              several sST objects for the service profile.

//              How do they relate?

leaf sST {

description "Specifies the slice/service type. See 3GPP TS 23.501

for defined values.";

mandatory true;

type uint32;

reference "3GPP TS 23.501 5.15.2.2";

}

leaf availability {

description "The availability requirement for a network slice

instance, expressed as a percentage.";

type availability-percentage;

}

list delayTolerance {

description "An attribute specifies the properties of service delivery

flexibility, especially for the vertical services that are not

chasing a high system performance.";

reference "TS 22.104 clause 4.3";

config false;

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

list servAttrCom {

description "This list represents the common properties of service

requirement related attributes.";

reference "GSMA NG.116 corresponding to Attribute categories,

tagging and exposure";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ns3cmn:ServAttrComGrp;

}

leaf support {

description "An attribute specifies whether or not the network

slice supports service delivery flexibility, especially for the

vertical services that are not chasing a high system performance.";

type ns3cmn:Support-enum;

}

}

list deterministicComm {

//Stage2 issue: deterministicComm is not defined in 28.541 chapter 6,

//              but I guess deterministicComm is meant

description "This list represents the properties of the deterministic

communication for periodic user traffic. Periodic traffic refers to the

type of traffic with periodic transmissions.";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

list servAttrCom {

description "This list represents the common properties of service

requirement related attributes.";

reference "GSMA NG.116 corresponding to Attribute categories,

tagging and exposure";

config false;

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ns3cmn:ServAttrComGrp;

}

leaf availability {

//Stage2 issue: Defined differently in 28.541 chapter 6, but XML

//              uses DeterministicCommAvailability

config false;

type ns3cmn:DeterminCommAvailability;

}

leaf periodicityList {

//Stage2 issue: Not defined in 28.541 chapter 6. XML and YAML

//              says "string".

type string;

}

}

list dLThptPerSlice {

description "This attribute defines achievable data rate of the

network slice in downlink that is available ubiquitously across

the coverage area of the slice";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ns3cmn:XLThptGrp;

}

list dLThptPerUE {

description "This attribute defines data rate supported by the network

slice per UE";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ns3cmn:XLThptGrp;

}

list uLThptPerSlice {

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

description "This attribute defines achievable data rate of the

network slice in uplink that is available ubiquitously across

the coverage area of the slice";

uses ns3cmn:XLThptGrp;

}

list uLThptPerUE {

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

description "This attribute defines data rate supported by the

network slice per UE";

uses ns3cmn:XLThptGrp;

}

list maxPktSize {

config false;

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

description "This parameter specifies the maximum packet size

supported by the network slice";

list servAttrCom {

description "This list represents the common properties of service

requirement related attributes.";

reference "GSMA NG.116 corresponding to Attribute categories,

tagging and exposure";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ns3cmn:ServAttrComGrp;

}

leaf maxSize {

//Stage2 issue: Not defined in 28.541, guessing integer bytes

type uint32;

units bytes;

}

}

list maxNumberofPDUSessions {

description "Represents the maximum number of

concurrent PDU sessions supported by the network slice";

config false;

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

list servAttrCom {

description "This list represents the common properties of service

requirement related attributes.";

reference "GSMA NG.116 corresponding to Attribute categories,

tagging and exposure";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ns3cmn:ServAttrComGrp;

}

leaf nOofPDUSessions {

//Stage2 issue: Not defined in 28.541, guessing integer

type uint32;

}

}

list kPIMonitoring {

description "Represents performance monitoring";

config false;

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

list servAttrCom {

description "This list represents the common properties of service

requirement related attributes.";

reference "GSMA NG.116 corresponding to Attribute categories,

tagging and exposure";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ns3cmn:ServAttrComGrp;

}

leaf kPIList {

//Stage2 issue: Data format not specified, low interoperability

description "An attribute specifies the name list of KQIs and KPIs

available for performance monitoring";

type string;

}

}

list userMgmtOpen {

description "An attribute specifies whether or not the network slice

supports the capability for the NSC to manage their users or groups

of users' network services and corresponding requirements.";

config false;

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

list servAttrCom {

description "This list represents the common properties of service

requirement related attributes.";

reference "GSMA NG.116 corresponding to Attribute categories,

tagging and exposure";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ns3cmn:ServAttrComGrp;

}

leaf support {

type ns3cmn:Support-enum;

}

}

list v2XCommModels {

description "An attribute specifies whether or not the V2X

communication mode is supported by the network slice.";

config false;

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

list servAttrCom {

description "This list represents the common properties of service

requirement related attributes.";

reference "GSMA NG.116 corresponding to Attribute categories,

tagging and exposure";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ns3cmn:ServAttrComGrp;

}

leaf v2XMode {

type V2XMode-enum;

}

}

list termDensity {

description "An attribute specifies the overall user density over

the coverage area of the network slice";

config false;

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

list servAttrCom {

description "This list represents the common properties of service

requirement related attributes.";

reference "GSMA NG.116 corresponding to Attribute categories,

tagging and exposure";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ns3cmn:ServAttrComGrp;

}

leaf density {

type uint32;

units users/km2;

}

}

leaf activityFactor {

//Stage2 issue: This is modeled as writable/config true in 28.542,

//              but that does not appear to match the description

description "An attribute specifies the percentage value of the

amount of simultaneous active UEs to the total number of UEs where

active means the UEs are exchanging data with the network";

reference "TS 22.261 Table 7.1-1";

type decimal64 {

fraction-digits 1;

}

}

leaf uESpeed {

//Stage2 issue: This is modeled as writable/config true in 28.542,

//              but that does not appear to match the description

description "An attribute specifies the maximum speed (in km/hour)

supported by the network slice at which a defined QoS can be

achieved";

type uint32;

units km/h;

}

leaf jitter {

//Stage2 issue: This is modeled as writable/config true in 28.542,

//              but that does not appear to match the description

description "An attribute specifies the deviation from the desired

value to the actual value when assessing time parameters";

reference "TS 22.104 clause C.4.1";

type uint32;

units microseconds;

}

leaf survivalTime {

description "An attribute specifies the time that an application

consuming a communication service may continue without an

anticipated message.";

reference "TS 22.104 clause 5";

type string;

}

leaf reliability {

description "An attribute specifies in the context of network layer

packet transmissions, percentage value of the amount of sent

network layer packets successfully delivered to a given system

entity within the time constraint required by the targeted service,

divided by the total number of sent network layer packets.";

reference "TS 22.261, TS 22.104";

type string;

}

leaf maxDLDataVolume {

//Stage2 issue: Not defined in 28.541. XML and YAML says "string"

type string;

}

leaf maxULDataVolume {

//Stage2 issue: Not defined in 28.541. XML and YAML says "string"

type string;

}

list nBIoT {

description "An attribute specifies whether NB-IoT is supported in

the RAN in the network slice";

config false;

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

list servAttrCom {

description "This list represents the common properties of service

requirement related attributes.";

reference "GSMA NG.116 corresponding to Attribute categories,

tagging and exposure";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ns3cmn:ServAttrComGrp;

}

leaf support {

description "An attribute specifies whether NB-IoT is supported

in the RAN in the network slice";

type ns3cmn:Support-enum;

}

}

}

}

<CODE ENDS>

*** END OF CHANGE 25 ***

*** START OF CHANGE 26 ***

*** yang-models/_3gpp-ns-nrm-sliceprofile.yang ***

<CODE BEGINS>

submodule _3gpp-ns-nrm-sliceprofile {

yang-version 1.1;

belongs-to _3gpp-ns-nrm-networkslicesubnet { prefix nss3gpp; }

import _3gpp-common-yang-types { prefix types3gpp; }

// import _3gpp-ns-nrm-networkslice { prefix ns3gpp; }

import _3gpp-ns-nrm-common { prefix ns3cmn3gpp; }

organization "3GPP SA5";

contact

"https://www.3gpp.org/DynaReport/TSG-WG--S5--officials.htm?Itemid=464";

description "Represents the properties of network slice subnet related

requirement that should be supported by the network slice subnet

instance in a 5G network.

Copyright 2025 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI,

TTA, TTC). All rights reserved.";

reference "3GPP TS 28.541

Management and orchestration;

5G Network Resource Model (NRM);

Information model definitions for network slice NRM (chapter 6)

";

revision 2025-08-18 { reference CR-1557; }

revision 2023-11-04 { reference CR-1129; }

revision 2023-09-18 { reference CR-1043 ; }

revision 2023-04-26 { reference CR-0916; }

revision 2021-07-20 { reference CR-0566; }

revision 2021-05-05 { reference CR-0485; }

revision 2020-02-19 {

description "Introduction of YANG definitions for network slice NRM";

reference "CR-0458";

}

revision 2019-05-27 {

description "initial revision.";

reference "Based on

3GPP TS 28.541 V15.X.XX";

}

typedef SliceSimultaneousUse-enum {

type enumeration {

enum ZERO;

enum ONE;

enum TWO;

enum THREE;

enum FOUR;

}

}

typedef ServiceType-enum {

type enumeration {

enum eMBB;

enum URLLC;

enum MIoT;

enum V2X;

}

}

grouping PositioningGrp {

description "Represents positioning support.";

reference "Clause 3.4.20 of GSMA NG.116 ";

uses ns3cmn3gpp:ServAttrComGrp ;

leaf-list availability {

type enumeration {

enum CIDE_CID ;

enum OTDOA;

enum RF_FINGERPRINTING;

enum AECID;

enum HYBRID_POSITIONING;

enum NET_RTK;

}

min-elements 1;

config false;

description "Specifies if this attribute is provided by the RAN domain

of the network slice and contains a list of positioning methods

provided by the RAN domain. If the list is empty this attribute is

not available in the RAN domain and the other parameters might be

ignored, see NG.116. Values allowed: are

CIDE-CID (LTE and NR), OTDOA (LTE and NR), RF fingerprinting, AECID,

Hybrid positioning, NET-RTK.";

}

leaf predictionfrequency {

type enumeration {

enum PERSEC;

enum PERMIN;

enum PERHOUR;

}

mandatory true;

description "Specifies how often location information is provided.

This parameter simply defines how often the customer is allowed to

request location information. This is not related to the time it

takes to determine the location, which is a characteristic of the

positioning method.

If leaf-list availability is empty, the value has no meaning.";

reference "NG.116";

}

leaf accuracy {

type decimal64 {

fraction-digits 2;

}

units meter;

mandatory true;

description "Specifies the accuracy of the location information.

Accuracy depends on the respective positioning solution applied in the

RAN domain of the network slice.";

reference "NG.116";

}

}

grouping TopSliceSubnetProfileGrp {

leaf latency {

description "The packet transmission latency (milliseconds) through

the RAN, CN, and TN part of 5G network, used to evaluate

utilization performance of the end-to-end network slice instance.";

reference "3GPP TS 28.554 clause 6.3.1";

//optional support

mandatory true;

type uint16;

units milliseconds;

}

leaf maxNumberofUEs {

description "Specifies the maximum number of UEs may simultaneously

access the network slice instance.";

//optional support

mandatory true;

type uint64;

}

list dLThptPerSliceSubnet {

description "This attribute defines achievable data rate of the

network slice subnet in downlink that is available ubiquitously

across the coverage area of the slice";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ns3cmn3gpp:XLThptGrp;

}

list dLThptPerUE {

description "This attribute defines data rate supported by the

network slice per UE, refer NG.116.";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ns3cmn3gpp:XLThptGrp;

}

list uLThptPerSliceSubnet {

description "This attribute defines achievable data rate of the

network slice subnet in uplink that is available ubiquitously

across the coverage area of the slice";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ns3cmn3gpp:XLThptGrp;

}

list uLThptPerUE {

description "This attribute defines data rate supported by the

network slice per UE, refer NG.116";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ns3cmn3gpp:XLThptGrp;

}

list maxPktSize {

config false;

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

description "This parameter specifies the maximum packet size

supported by the network slice";

list servAttrCom {

description "This list represents the common properties of service

requirement related attributes.";

reference "GSMA NG.116 corresponding to Attribute categories,

tagging and exposure";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ns3cmn3gpp:ServAttrComGrp;

}

leaf maxSize {

//Stage2 issue: Not defined in 28.541, guessing integer bytes

type uint32;

units bytes;

}

}

list maxNumberofPDUSessions {

description "Represents the maximum number of

concurrent PDU sessions supported by the network slice";

config false;

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

list servAttrCom {

description "This list represents the common properties of service

requirement related attributes.";

reference "GSMA NG.116 corresponding to Attribute categories,

tagging and exposure";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ns3cmn3gpp:ServAttrComGrp;

}

leaf nOofPDUSessions {

//Stage2 issue: Not defined in 28.541, guessing integer

type uint32;

}

}

leaf sliceSimultaneousUse {

description "This attribute describes whether a network slice

can be simultaneously used by a device together with other

network slices and if so, with which other classes of network slices.";

type SliceSimultaneousUse-enum;

}

list delayTolerance {

description "An attribute specifies the properties of service delivery

flexibility, especially for the vertical services that are not

chasing a high system performance.";

reference "TS 22.104 clause 4.3";

config false;

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

list servAttrCom {

description "This list represents the common properties of service

requirement related attributes.";

reference "GSMA NG.116 corresponding to Attribute categories,

tagging and exposure";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ns3cmn3gpp:ServAttrComGrp;

}

leaf support {

description "An attribute specifies whether or not the network

slice supports service delivery flexibility, especially for the

vertical services that are not chasing a high system performance.";

type ns3cmn3gpp:Support-enum;

}

}

list termDensity {

description "An attribute specifies the overall user density over

the coverage area of the network slice";

config false;

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

list servAttrCom {

description "This list represents the common properties of service

requirement related attributes.";

reference "GSMA NG.116 corresponding to Attribute categories,

tagging and exposure";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ns3cmn3gpp:ServAttrComGrp;

}

leaf density {

type uint32;

units users/km2;

}

}

leaf activityFactor {

//Stage2 issue: This is modeled as writable/config true in 28.542,

//              but that does not appear to match the description

description "An attribute specifies the percentage value of the

amount of simultaneous active UEs to the total number of UEs where

active means the UEs are exchanging data with the network";

reference "TS 22.261 Table 7.1-1";

type decimal64 {

fraction-digits 1;

}

}

list coverageAreaTAList {

description "A list of TrackingAreas where the NSI can be selected.";

key idx; //no obvious leaf to use as a key

leaf idx { type uint32; }

//optional support

min-elements 1;

uses types3gpp:TaiGrp;

}

leaf uEMobilityLevel {

description "The mobility level of UE accessing the network slice

instance.";

//optional support

type types3gpp:UeMobilityLevel;

}

leaf resourceSharingLevel {

description "Specifies whether the resources to be allocated to the

network slice subnet instance may be shared with another network

slice subnet instance(s).";

//optional support

type types3gpp:ResourceSharingLevel;

}

leaf uESpeed {

//Stage2 issue: This is modeled as writable/config true in 28.542,

//              but that does not appear to match the description

description "An attribute specifies the maximum speed (in km/hour)

supported by the network slice at which a defined QoS can be

achieved";

type uint32;

units km/h;

}

leaf reliability {

description "An attribute specifies in the context of network layer

packet transmissions, percentage value of the amount of sent

network layer packets successfully delivered to a given system

entity within the time constraint required by the targeted service,

divided by the total number of sent network layer packets.";

reference "TS 22.261, TS 22.104";

type string;

}

leaf serviceType {

description "An attribute specifies the standardized network slice type.

allowedValues: eMBB, URLLC, MIoT, V2X.";

type ServiceType-enum;

}

list deterministicComm {

//Stage2 issue: deterministicComm is not defined in 28.541 chapter 6,

//              but I guess determinComm is meant

description "This list represents the properties of the deterministic

communication for periodic user traffic. Periodic traffic refers to the

type of traffic with periodic transmissions.";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

list servAttrCom {

description "This list represents the common properties of service

requirement related attributes.";

reference "GSMA NG.116 corresponding to Attribute categories,

tagging and exposure";

config false;

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ns3cmn3gpp:ServAttrComGrp;

}

leaf availability {

//Stage2 issue: Defined differently in 28.541 chapter 6, but XML

//              uses DeterminCommAvailability

config false;

type ns3cmn3gpp:DeterminCommAvailability;

}

leaf periodicityList {

//Stage2 issue: Not defined in 28.541 chapter 6. XML and YAML

//              says "string".

type string;

}

}

leaf survivalTime {

description "An attribute specifies the time that an application

consuming a communication service may continue without an

anticipated message.";

reference "TS 22.104 clause 5";

type string;

}

list positioning {

key predictionfrequency;

min-elements 1;

max-elements 1;

description "Specifies whether the network slice provides

geo-localization methods or supporting methods";

reference "Clause 3.4.20 of NG.116";

uses PositioningGrp;

}

}

grouping CNSliceSubnetProfileGrp {

leaf latency {

description "The packet transmission latency (milliseconds) through

the RAN, CN, and TN part of 5G network, used to evaluate

utilization performance of the end-to-end network slice instance.";

reference "3GPP TS 28.554 clause 6.3.1";

//optional support

mandatory true;

type uint16;

units milliseconds;

}

leaf maxNumberofUEs {

description "Specifies the maximum number of UEs may simultaneously

access the network slice instance.";

//optional support

mandatory true;

type uint64;

}

list dLThptPerSliceSubnet {

description "This attribute defines achievable data rate of the

network slice subnet in downlink that is available ubiquitously

across the coverage area of the slice";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ns3cmn3gpp:XLThptGrp;

}

list dLThptPerUE {

description "This attribute defines data rate supported by the

network slice per UE, refer NG.116.";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ns3cmn3gpp:XLThptGrp;

}

list uLThptPerSliceSubnet {

description "This attribute defines achievable data rate of the

network slice subnet in uplink that is available ubiquitously

across the coverage area of the slice";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ns3cmn3gpp:XLThptGrp;

}

list uLThptPerUE {

description "This attribute defines data rate supported by the

network slice per UE, refer NG.116";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ns3cmn3gpp:XLThptGrp;

}

list maxPktSize {

config false;

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

description "This parameter specifies the maximum packet size

supported by the network slice";

list servAttrCom {

description "This list represents the common properties of service

requirement related attributes.";

reference "GSMA NG.116 corresponding to Attribute categories,

tagging and exposure";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ns3cmn3gpp:ServAttrComGrp;

}

leaf maxSize {

//Stage2 issue: Not defined in 28.541, guessing integer bytes

type uint32;

units bytes;

}

}

list maxNumberofPDUSessions {

description "Represents the maximum number of

concurrent PDU sessions supported by the network slice";

config false;

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

list servAttrCom {

description "This list represents the common properties of service

requirement related attributes.";

reference "GSMA NG.116 corresponding to Attribute categories,

tagging and exposure";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ns3cmn3gpp:ServAttrComGrp;

}

leaf nOofPDUSessions {

//Stage2 issue: Not defined in 28.541, guessing integer

type uint32;

}

}

leaf sliceSimultaneousUse {

description "This attribute describes whether a network slice

can be simultaneously used by a device together with other

network slices and if so, with which other classes of network slices.";

type SliceSimultaneousUse-enum;

}

list delayTolerance {

description "An attribute specifies the properties of service delivery

flexibility, especially for the vertical services that are not

chasing a high system performance.";

reference "TS 22.104 clause 4.3";

config false;

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

list servAttrCom {

description "This list represents the common properties of service

requirement related attributes.";

reference "GSMA NG.116 corresponding to Attribute categories,

tagging and exposure";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ns3cmn3gpp:ServAttrComGrp;

}

leaf support {

description "An attribute specifies whether or not the network

slice supports service delivery flexibility, especially for the

vertical services that are not chasing a high system performance.";

type ns3cmn3gpp:Support-enum;

}

}

list coverageAreaTAList {

description "A list of TrackingAreas where the NSI can be selected.";

key idx; //no obvious leaf to use as a key

leaf idx { type uint32; }

//optional support

min-elements 1;

uses types3gpp:TaiGrp;

}

leaf resourceSharingLevel {

description "Specifies whether the resources to be allocated to the

network slice subnet instance may be shared with another network

slice subnet instance(s).";

//optional support

type types3gpp:ResourceSharingLevel;

}

list deterministicComm {

//Stage2 issue: deterministicComm is not defined in 28.541 chapter 6,

//              but I guess determinComm is meant

description "This list represents the properties of the deterministic

communication for periodic user traffic. Periodic traffic refers to the

type of traffic with periodic transmissions.";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

list servAttrCom {

description "This list represents the common properties of service

requirement related attributes.";

reference "GSMA NG.116 corresponding to Attribute categories,

tagging and exposure";

config false;

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ns3cmn3gpp:ServAttrComGrp;

}

leaf availability {

//Stage2 issue: Defined differently in 28.541 chapter 6, but XML

//              uses DeterminCommAvailability

config false;

type ns3cmn3gpp:DeterminCommAvailability;

}

leaf periodicityList {

//Stage2 issue: Not defined in 28.541 chapter 6. XML and YAML

//              says "string".

type string;

}

}

}

grouping PositioningRANSubnetGrp {

description "Represents positioning support in RAN domain";

leaf-list availability {

type enumeration {

enum CIDE_CID ;

enum OTDOA;

enum RF_FINGERPRINTING;

enum AECID;

enum HYBRID_POSITIONING;

enum NET_RTK;

}

config false;

description "Specifies if this attribute is provided by the RAN domain

of the network slice and contains a list of positioning methods

provided by the RAN domain. If the list is empty this attribute is

not available in the RAN domain and the other parameters might be

ignored, see NG.116. Values allowed: are

CIDE-CID (LTE and NR), OTDOA (LTE and NR), RF fingerprinting, AECID,

Hybrid positioning, NET-RTK.";

}

leaf predictionfrequency {

type enumeration {

enum PERSEC;

enum PERMIN;

enum PERHOUR;

}

mandatory true;

description "Specifies how often location information is provided.

This parameter simply defines how often the customer is allowed to

request location information. This is not related to the time it

takes to determine the location, which is a characteristic of the

positioning method.

If leaf-list availability is empty, the value has no meaning.";

reference "NG.116";

}

leaf accuracy {

type decimal64 {

fraction-digits 2;

}

units meter;

mandatory true;

description "Specifies the accuracy of the location information.

Accuracy depends on the respective positioning solution applied in the

RAN domain of the network slice.";

reference "NG.116";

}

}

grouping RANSliceSubnetProfileGrp {

description "Represents the RANSliceSubnetProfile datatype";

leaf latency {

description "The packet transmission latency (milliseconds) through

the RAN, CN, and TN part of 5G network, used to evaluate

utilization performance of the end-to-end network slice instance.";

reference "3GPP TS 28.554 clause 6.3.1";

//optional support

mandatory true;

type uint16;

units milliseconds;

}

leaf maxNumberofUEs {

description "Specifies the maximum number of UEs may simultaneously

access the network slice instance.";

//optional support

mandatory true;

type uint64;

}

list dLThptPerSliceSubnet {

description "This attribute defines achievable data rate of the

network slice subnet in downlink that is available ubiquitously

across the coverage area of the slice";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ns3cmn3gpp:XLThptGrp;

}

list dLThptPerUE {

description "This attribute defines data rate supported by the

network slice per UE, refer NG.116.";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ns3cmn3gpp:XLThptGrp;

}

list uLThptPerSliceSubnet {

description "This attribute defines achievable data rate of the

network slice subnet in uplink that is available ubiquitously

across the coverage area of the slice";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ns3cmn3gpp:XLThptGrp;

}

list uLThptPerUE {

description "This attribute defines data rate supported by the

network slice per UE, refer NG.116";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ns3cmn3gpp:XLThptGrp;

}

list maxPktSize {

config false;

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

description "This parameter specifies the maximum packet size

supported by the network slice";

list servAttrCom {

description "This list represents the common properties of service

requirement related attributes.";

reference "GSMA NG.116 corresponding to Attribute categories,

tagging and exposure";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ns3cmn3gpp:ServAttrComGrp;

}

leaf maxSize {

//Stage2 issue: Not defined in 28.541, guessing integer bytes

type uint32;

units bytes;

}

}

list delayTolerance {

description "An attribute specifies the properties of service delivery

flexibility, especially for the vertical services that are not

chasing a high system performance.";

reference "TS 22.104 clause 4.3";

config false;

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

list servAttrCom {

description "This list represents the common properties of service

requirement related attributes.";

reference "GSMA NG.116 corresponding to Attribute categories,

tagging and exposure";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ns3cmn3gpp:ServAttrComGrp;

}

leaf support {

description "An attribute specifies whether or not the network

slice supports service delivery flexibility, especially for the

vertical services that are not chasing a high system performance.";

type ns3cmn3gpp:Support-enum;

}

}

leaf sliceSimultaneousUse {

description "This attribute describes whether a network slice

can be simultaneously used by a device together with other

network slices and if so, with which other classes of network slices.";

type SliceSimultaneousUse-enum;

}

list termDensity {

description "An attribute specifies the overall user density over

the coverage area of the network slice";

config false;

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

list servAttrCom {

description "This list represents the common properties of service

requirement related attributes.";

reference "GSMA NG.116 corresponding to Attribute categories,

tagging and exposure";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ns3cmn3gpp:ServAttrComGrp;

}

leaf density {

type uint32;

units users/km2;

}

}

leaf activityFactor {

//Stage2 issue: This is modeled as writable/config true in 28.542,

//              but that does not appear to match the description

description "An attribute specifies the percentage value of the

amount of simultaneous active UEs to the total number of UEs where

active means the UEs are exchanging data with the network";

reference "TS 22.261 Table 7.1-1";

type decimal64 {

fraction-digits 1;

}

}

list coverageAreaTAList {

description "A list of TrackingAreas where the NSI can be selected.";

key idx; //no obvious leaf to use as a key

leaf idx { type uint32; }

//optional support

min-elements 1;

uses types3gpp:TaiGrp;

}

leaf uEMobilityLevel {

description "The mobility level of UE accessing the network slice

instance.";

//optional support

type types3gpp:UeMobilityLevel;

}

leaf resourceSharingLevel {

description "Specifies whether the resources to be allocated to the

network slice subnet instance may be shared with another network

slice subnet instance(s).";

//optional support

type types3gpp:ResourceSharingLevel;

}

leaf uESpeed {

//Stage2 issue: This is modeled as writable/config true in 28.542,

//              but that does not appear to match the description

description "An attribute specifies the maximum speed (in km/hour)

supported by the network slice at which a defined QoS can be

achieved";

type uint32;

units km/h;

}

leaf reliability {

description "An attribute specifies in the context of network layer

packet transmissions, percentage value of the amount of sent

network layer packets successfully delivered to a given system

entity within the time constraint required by the targeted service,

divided by the total number of sent network layer packets.";

reference "TS 22.261, TS 22.104";

type string;

}

leaf serviceType {

description "An attribute specifies the standardized network slice type.

allowedValues: eMBB, URLLC, MIoT, V2X.";

type ServiceType-enum;

}

list deterministicComm {

//Stage2 issue: deterministicComm is not defined in 28.541 chapter 6,

//              but I guess determinComm is meant

description "This list represents the properties of the deterministic

communication for periodic user traffic. Periodic traffic refers to the

type of traffic with periodic transmissions.";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

list servAttrCom {

description "This list represents the common properties of service

requirement related attributes.";

reference "GSMA NG.116 corresponding to Attribute categories,

tagging and exposure";

config false;

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses ns3cmn3gpp:ServAttrComGrp;

}

leaf availability {

//Stage2 issue: Defined differently in 28.541 chapter 6, but XML

//              uses DeterminCommAvailability

config false;

type ns3cmn3gpp:DeterminCommAvailability;

}

leaf periodicityList {

//Stage2 issue: Not defined in 28.541 chapter 6. XML and YAML

//              says "string".

type string;

}

}

leaf survivalTime {

description "An attribute specifies the time that an application

consuming a communication service may continue without an

anticipated message.";

reference "TS 22.104 clause 5";

type string;

}

list positioning {

min-elements 1;

max-elements 1;

description "Specifies whether the RAN domain of the network slice

provides geo-localization methods or supporting methods.";

reference "Clause 3.4.20 of NG.116 [50].";

uses PositioningRANSubnetGrp;

}

}

grouping SliceProfileGrp {

leaf sliceProfileId {

description "A unique identifier of the property of network slice

subnet related requirement should be supported by the network

slice subnet instance.";

type types3gpp:DistinguishedName;

}

list sNSSAIList {

description "List of S-NSSAIs the managed object is capable of

supporting. (Single Network Slice Selection Assistance Information)

An S-NSSAI has an SST (Slice/Service type) and an optional SD

(Slice Differentiator) field.";

key idx;

unique "sst sd";

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses types3gpp:SNssai;

}

list pLMNIdList {

description "List of at most six entries of PLMN Identifiers, but at

least one (the primary PLMN Id).  The PLMN Identifier is composed

of a Mobile Country Code (MCC) and a Mobile Network Code (MNC).";

min-elements 1;

max-elements 6;

key "mcc mnc";

ordered-by user;

uses types3gpp:PLMNId;

}

leaf maxNumberofUEs {

description "Specifies the maximum number of UEs may simultaneously

access the network slice instance.";

//optional support

mandatory true;

type uint64;

}

list coverageAreaTAList {

description "A list of TrackingAreas where the NSI can be selected.";

key idx; //no obvious leaf to use as a key

leaf idx { type uint32; }

//optional support

min-elements 1;

uses types3gpp:TaiGrp;

}

leaf latency {

description "The packet transmission latency (milliseconds) through

the RAN, CN, and TN part of 5G network, used to evaluate

utilization performance of the end-to-end network slice instance.";

reference "3GPP TS 28.554 clause 6.3.1";

//optional support

mandatory true;

type uint16;

units milliseconds;

}

leaf uEMobilityLevel {

description "The mobility level of UE accessing the network slice

instance.";

//optional support

type types3gpp:UeMobilityLevel;

}

leaf resourceSharingLevel {

description "Specifies whether the resources to be allocated to the

network slice subnet  instance may be shared with another network

slice subnet instance(s).";

//optional support

type types3gpp:ResourceSharingLevel;

}

list CNSliceSubnetProfile {

description " This represents the requirements for the top slice associated with the

network slice. ";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses TopSliceSubnetProfileGrp;

}

list RANSliceSubnetProfile {

description " This represents the requirements for the top slice associated with the

network slice. ";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses TopSliceSubnetProfileGrp;

}

list TopSliceSubnetProfile {

description " This represents the requirements for the top slice associated with the

network slice. ";

key idx;

max-elements 1;

leaf idx {

description "Synthetic index for the element.";

type uint32;

}

uses TopSliceSubnetProfileGrp;

}

}

}

<CODE ENDS>

*** END OF CHANGE 26 ***