3GPP TSG-SA5 Meeting #162 	S5-253236

Stor-Göteborg, Sweden, 25th August 2025 - 29th August 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 28.405 | CR | 0043 | rev | - | Current version: | 19.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Rel-19 CR TS 28.405 Corrections on Deactivation procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TraceQoE_OAM |  |  |  |  |  | Date: |  |  | 2025-08-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In the current Deactivation procedure, there are a few errors with the reference |  |  |  |  |  |  |  |  |
| Summary of change: |  | Corrections on the incorrect references |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unable to find the correct references |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.3.1, 4.3.3, 4.2.3.1, 4.4.4, 4.5.3.1, 4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** START OF NEXT CHANGE ***

#### 4.1.3.1	Forced deactivation in UTRAN

If the operator technician or the management application wants to deactivate a measurement collection job before the pre-set time has expired, the management system sends the deactivateQMCJob operation TS 28.308 [3] to the RNC. The RNC sets the network request session to ended, but does not delete the QoE Reference Id and the QoE Collection Entity Address parameters, as the UE still may send reports which shall be send to the collection centre. For UE request sessions which have reported that a recording session is started, the RNC sends the MeasurementControl message (as TS 25.331 [4]) with the DeactivateJob request to relevant UEs. The Access stratum sends +CAPPLEVMC AT command (as TS 27.007 [5]) to the application with the DeactivateJob request. The application stops the recording session and stops recording of the requested information. The QoE Reference Id and the QoE Collection Entity Address parameters in the RNC are deleted when the UE request session is ended.

*** START OF NEXT CHANGE ***

### 4.3.3	Deactivation of measurement collection job in UTRAN

#### 4.3.3.1	Pre-set time has elapsed in UTRAN

When the pre-set time is elapsed, the HSS or SGSN and the RNC sets the network request session to ended, but do not delete the QoE Reference Id and the QoE Collection Entity Address parameters, due to that reports still can come from the UE that has to be transferred to the collection centre. The QoE Reference Id and the QoE Collection Entity Address parameters are deleted when the session is ended.

#### 4.33.2	Forced deactivation in UTRAN

If the operator technician or the management application wants to deactivate a measurement collection job, the management system sends the deactivateQoEJob operation TS 28.308 [3] to the HSS or SGSN, that propagate it to the RNC. The RNC sets the network request session to ended, but does not delete the QoE Reference Id and the QoE Collection Entity Address parameters, as the UE still may send reports which shall be send to the collection centre. For UE request sessions which have reported that a recording session is started, the RNC sends the MeasurementControl message (as TS 25.331 [4]) with the DeactivateJob request to relevant UEs. The Access stratum sends +CAPPLEVMC AT command (as TS 27.007 [5]) to the application with the DeactivateJob request. The application stops the recording session and stops recording of the requested information. The QoE Reference Id and the QoE Collection Entity Address parameters in the RNC are deleted when the UE request session is ended.

#### 4.3.3.3	Deactivation of recording session in UTRAN

See clause 4.1.3.3.

*** START OF NEXT CHANGE ***

#### 4.2.3.1	Forced deactivation

When the operator technician or the management application wants to deactivate a measurement collection job, the management system sends the deactivateQMCJob operation TS 28.308 [3] to the eNB. The eNB sets the network request session to ended, but does not delete the QoE Reference Id and the QoE Collection Entity Address parameters, as the UE still may send reports which shall be send to the collection centre. For UE request sessions which have reported that a recording session is started, the eNB sends the RRCConnectionReconfiguration message (as TS 36.331 [8]) to relevant UEs. The RRCConnectionReconfiguration message is including measConfigAppLayer set to discard application layer measurement report information in otherConfig (as TS 36.331 [8]). The Access stratum sends +CAPPLEVMC AT command (as TS 27.007 [5]) to the application with the discard request. The application stops the recording session and stops recording of the requested information. The QoE Reference Id and the QoE Collection Entity Address parameters in the eNB are deleted when the UE request session is ended.

*** START OF NEXT CHANGE ***

### 4.4.4	Deactivation of measurement collection for a UE in LTE

#### 4.4.4.1	Pre-set time has elapsed in LTE

When the pre-set time is elapsed, the MME and the eNB sets the network request session to ended, but do not delete the QoE Reference Id and the QoE Collection Entity Address parameters, due to that reports still can come from the UE that has to be transferred to the collection centre. The QoE Reference Id and the QoE Collection Entity Address parameters are deleted when the session is ended.

#### 4.4.4.2	Forced deactivation in LTE

When the operator technician or the management application wants to deactivate a measurement collection job, the management system sends the deactivateQMCJob operation TS 28.308 [3] to the MME that propagates to the eNB . The eNB sets the network request session to ended, but does not delete the QoE Reference Id and the QoE Collection Entity Address parameters, as the UE still may send reports which shall be send to the collection centre. For UE request sessions which have reported that a recording session is started, the eNB sends the RRCConnectionReconfiguration message (as TS 36.331 [8]) to relevant UEs. The RRCConnectionReconfiguration message is including measConfigAppLayer set to discard application layer measurement report information in otherConfig (as TS 36.331 [8]). The Access stratum sends +CAPPLEVMC AT command (as TS 27.007 [5]) to the application with the discard request. The application stops the recording session and stops recording of the requested information. The QoE Reference Id and the QoE Collection Entity Address parameters in the eNB are deleted when the UE request session is ended.

#### 4.4.4.3	Deactivation of recording session in LTE

See clause 4.2.3.2.

*** START OF NEXT CHANGE ***

#### 4.5.3.1	Forced deactivation

When the operator technician or the management application wants to deactivate a measurement collection job, the management system sends the delete QMC job request  to the gNB. The gNB sets the network request session to ended, but does not delete the QoE Reference Id and the QoE Collection Entity Address parameters, as the UE still may send reports which shall be sent to the collection centre. For UE request sessions which have reported that a recording session is started, the gNB sends the RRCReconfiguration message (as TS 38.331 [11]) to relevant UEs. The RRCReconfiguration message is including measConfigAppLayerId set to discard application layer measurement report information in appLayerMeaConfig (as TS 38.331 [11]). The Access Stratum sends +CAPPLEVMCNR AT command (as TS 27.007 [5]) to the application with the discard request. The application stops the recording session and stops recording of the requested information. The QoE Reference Id and the QoE Collection Entity Address parameters in the gNB are deleted when the UE request session is ended.

*** START OF NEXT CHANGE ***

### 4.6.3	Deactivation of measurement collection for a UE in NR

#### 4.6.3.1	Pre-set time has elapsed in NR

When the pre-set time is elapsed, the gNB sets the network request session to ended, but do not delete the QoE Reference Id and the QoE Collection Entity Address parameters, due to that reports still can come from the UE that has to be transferred to the collection centre. The QoE Reference Id and the QoE Collection Entity Address parameters are deleted when the session is ended.

#### 4.6.3.2	Forced deactivation in NR

When the operator technician or the management application wants to deactivate a measurement collection job, the management system sends the delete QMC job request to the UDM that propgates to the AMF/gNB . The gNB sets the network request session to ended, but does not delete QoE Reference Id and the QoE Collection Entity Address parameters, as the UE still may send reports which shall be send to the collection centre. For UE request sessions which have reported that a recording session is started, the gNB sends the RRCReconfiguration message (as TS 38.331 [11]) to relevant UEs. The RRCReconfiguration message is including measConfigAppLayer set to discard application layer measurement report information in otherConfig (as TS 38.331 [11]). The Access stratum sends +CAPPLEVMC AT command (as TS 27.007 [5]) to the application with the discard request. The application stops the recording session and stops recording of the requested information. QoE Reference Id and the QoE Collection Entity Address  parameters in the gNB are deleted when the UE request session is ended.

#### 4.6.3.3	Deactivation of recording session in NR

Regardless of whether the pre-set time has elapsed or not, the recording session continues to be active until the session for the application is ended.

*** END OF CHANGE ***