3GPP TSG-SA5 (Telecom Management) 	Tdoc S5-070159

Meeting SA5#51, 22 - 26 Jan 2007, Seville, ES

| CR-Form-v9.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.260 | CR | 0041 |  rev | - |  Current version: | 7.1.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Introduce IMS charging implications of VCC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Orange (alain.bibas@orange-ftgroup.com) |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | VCC-CH |  |  |  |  |  |  | Date:  |  | 26/01/2007 |
| Category:  | B |  |  |  |  |  |  | Release:  |  | Rel-7 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) |  |
| Reason for change:  |  | Charging aspects for VCC are not covered yet and need to be completed |  |  |  |  |  |  |  |  |
| Summary of change:  |  | A detailed scenario that illustrates these principles is added. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | Specific charging functionality for VCC is not available which may lead to double billing of IMS session and CS call when VCC is enabled. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 2, 3.2 and 5.2.2.1.12A (new), Annex B |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | x | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
|  |  | x |  | O&M Specifications |  |  | 32.250 |  |  |  |
| Other comments:  |  | Related R7 CR 32.250 in S5-070120 |  |  |  |  |  |  |  |  |

1st Modified Section

# 2	References

The following documents contain provisions, which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.240: "Telecommunication management; Charging management; Charging architecture and principles".

[2]	3GPP TS 32.250: "Telecommunication management; Charging management; Circuit Switched (CS) domain charging".

[3]- [10]	Void.

[11]	3GPP TS 32.251: "Telecommunication management; Charging management; Packet Switched (PS) domain charging".

[12]- [49]	Void.

[50]	3GPP TS 32.299: "Telecommunication management; Charging management; Diameter charging application

[51]	3GPP TS 32.298: "Telecommunication management; Charging management; Charging Data Record (CDR) parameter description

[52]	3GPP TS 32.297: "Telecommunication management; Charging management; Charging Data Records (CDR) file format and transfer

[53]	Void.

[54]	3GPP TS 32.295: "Telecommunication management; Charging management; Charging Data Record (CDR) transfer".

[55]- [99]	Void.

[100]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[101]- [102]	Void.

[103]	3GPP TS 23.002: "Network Architecture".

[104]- [199]	Void.

[200]	3GPP TS 22.228: "IMS Stage 1".

[201]	3GPP TS 23.228: "Functional stage 2 description of IMS".

[202]	3GPP TS 24.228: "Signalling flows for the IP multimedia call control based on SIP and SDP, Stage 3”

[203]	3GPP TS 23.218: "IP Multimedia (IM) session handling; IM call model; Stage 2".

[204]	3GPP TS 24.229: "Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[205]	3GPP TS 29.229: "Cx and Dx Interfaces based on the Diameter protocol; Protocol Details".

[206]- [299]	Void.

[300]- [399]	Void.

[400]	Void.

[401]	IETF RFC 3588 (2003): “diameter base protocol".

[402]	IETF RFC 4006:  “Diameter Credit Control Application".

[403]	IETF RFC 2806: "URLs for Telephone Calls".

[404]	IETF RFC 3261: "SIP: Session Initiation Protocol".

[405]	IETF RFC 2486: "The Network Access Identifier".

[406]	RFC 3455 (January 2003): "Private Header (P-Header) Extensions to the Session Initiation Protocol (SIP) for the 3rd-Generation Partnership Project (3GPP)".

Next Modified Section

## 3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply:

ABNF	Augmented Backus-Naur Form

ACA	Accounting Answer

ACR	Accounting Request

AS	Application Server

AVP	Attribute Value Pair

B2BUA	Back-to-Back User Agent

BGCF	Breakout Gateway Control Function

BS	Billing System

CCA	Credit Control Answer

CCF	Charging Collection Function

CCR	Credit Control Request

CDF	Charging Data Function

CDR	Charging Data Record

CGF	Charging Gateway Function

CPCF	Content Provider Charging Function

ECF	Event Charging Function

ECUR	Event Charging with Unit Reservation

CSCF	Call Session Control Function (I-Interrogating; P-Proxy; and S-Serving)

IEC	Immediate Event Charging

IMS	IP Multimedia Subsystem

IMS-GWF	IMS Gateway Function

ISC	IMS Service Control

MGCF	Media Gateway Control Function

MRFC	Media Resource Function Controller

MRFP	Multimedia Resource Function Processor

OCS	Online Charging System

SCCF	Subscriber Content Charging Function

SCUR	Session Charging with Unit Reservation

SDP	Session Description Protocol

SIP	Session Initiation Protocol

UA	User Agent

UE	User Equipment

VCC	Voice Call Continuity

VDN	VCC Domain transfer Number

Next Modified Section

##### 5.2.2.1.12A	AS Related Procedures - AS Acting as a VCC AS

##### 5.2.2.1.12A.1 VCC UE originating call from IMS and Domain Transfer IMS to CS

The following figure shows the case where an Application Server acts as a Voice Call Continuity AS (VCC AS). It illustrates the Diameter transactions that are required for VCC UE originating call from IMS (A.) follwed by a Domain Transfer from IMS to CS (B.)

A. VCC UE Originating call from IMS and routed to IMS

- Figure: Message Sequence Chart VCC UE originating call from IMS and Domain Transfer IMS to CS

1.	The VCC session is initiated

2.	The S-CSCF validates the service profile, and invokes any appropriate service logic required for this user.

3.	The S-CSCF forwards the INVITE request message to the VCC AS, according to the service origination logic defined by initial Filter Criteria (iFC) in the subscriber profile of the HSS.

4.	The VCC call is anchored at the VCC AS in the home IMS Domain upon reception of the SIP Invite (Call Id#1)

5 and 6.               The S-CSCF forwards the INVITE request message to the terminating network (Call Id#2)

7.                        The S-CSCF in the terminating network performs call control and service triggering for the

incoming call

- The session is conveyed to the UE-2 in the terminating network

- The destination party answers and a final response 200 OK is received.

10. 	Upon reception of the final response, the S-CSCF in the terminating network sends an Accounting-Request with
Accounting-Record-Type indicating START_RECORD to record start of a user session and start of a media component in the S-CSCF CDR.

11. and 12	The CDF from the terminating network opens an S-CSCF CDR and acknowledges the reception of the data

13.	The final response 200 OK is transmitted to the S-CSCF in the Originating network

14. 	Upon reception of the final response, the S-CSCF in the originating network sends an Accounting-Request with Accounting-Record-Type indicating START_RECORD to record VCC call routing and start of a user session/media component in the S-CSCF CDR.

15. and 16	The CDF from the Originating network opens an S-CSCF CDR related to the Remote leg and acknowledges the reception of the data.

17.	Same as 13. but for VCC AS (Remote leg)

18.	Same as 14. but for the VCC AS (Remote leg)

19. and 20.	Same as 15. and 16. but opening a VCC AS CDR related to the Remote leg

21.	Same as 13. but for VCC AS (Access leg)

22.	Same as 14. but for the VCC AS (Access leg)

23. and 24.	Same as 15. and 16. but opening a VCC AS (Access leg)

25.	Same as 14. but for the S-CSCF (Access leg)

26. and 27.	Same as 15. and 16. but opening a VCC AS (Access leg)

28.	The final response to SIP Invite (Call Id#1) is received by the UE-1

B. Domain Transfer from IMS to CS

- Figure: Message Sequence Chart VCC UE originating call from IMS and Domain Transfer IMS to CS (Contn'd)

- The VCC User (UE-1) originates a voice call in the CS domain using the VCC Domain Transfer Number (VDN) of the VCC function to establish an Access Leg via the CS domain and request Domain Transfer of the active IMS session to CS Domain.

- The originating call is processed in the CS network according to CS origination procedures. The user profil is retrieved from the HLR with the CAMEL Detection points for VCC.

- A CAP Initial DP message is sent to the VCC AS to get instructions on how to process the call

- The VCC application detects that the user is in IMS call

- It responds with a CAP continue

- The MSC opens a MOC Record as defined in 3GPP TS 32.250 [2]

- The VMSC routes the call towards the user’s home IMS network via an MGCF in the home network

- The VCC AS PSI DN is resolved using the VDN at the CS network

- The MGCF initiates an INVITE towards the VCC AS in the home IMS of the originating VCC user. This message is routed based on one of the standard procedures specified for PSI based Application Server Termination.

38, 39, 40, 41 and 42. The VCC AS completes the establishment of the Access Leg using normal CS to IMS

procedures.

43.Upon reception of the final response, the MGCF sends an Accounting-Request with

Accounting-Record-Type indicating START_RECORD to record start of a user session in the MGCF CDR.

44 and 45.The CDF from the terminating network opens an S-CSCF CDR and acknowledges the reception of the

data

46.and 47.  The VCC AS performs the Domain Transfer by updating the Remote Leg with the connection

information of the newly established Access Leg using the Access Leg Update procedure toward remote end. It

sends an UPDATE message. The new SDP in the UPDATE message indicates that the IP address towards

the VCC UE is changed: the IP address that was previously the IP address of the VCC UE is changed into the IP

address of the MGW.The new bearer path goes now from the VCC UE to the MSC, the IM-MGW and then to

the other end-point network.

48 and 49. The 200 OK responses to the SIP Update are received.

50. At Access Leg update the VCC AS sends Accounting-Request with Accounting-Record-Type indicating

INTERIM_RECORD to record update of a session in the VCC AS CDR.

51. and 52. The CDF closes the S-CSCF CDR in the terminating network and acknowledges the reception of the

data

53. Same as 50, but for S-CSCF.

54 and 55. Same as 51 updating the S-CSCF CDR.

56. The source Access Leg which is the Access leg previously established over IMS is subsequently released

57. At session termination the VCC AS sends Accounting-Request with Accounting-Record-Type indicating

STOP_RECORD to record stop of a session and stop of a media component in the VCC AS CDR. This CDR

may be generated with special handling. One example of special handling is to zero rate the IMS resource usage

for the Access leg establishment. This can be performed using the "Service Specific Data" parameter in the VCC

AS CDR indicating  the anchoring of a VCC call.

58. and 59. The CDF closes the AS CDR for Access leg and acknowledges the reception of the data

60. Same as 51, but for S-CSCF.

61 and 62. Same as 52 closing the S-CSCF CDR related to the Access leg. This CDR may be generated with special

handling. One example of special handling is to zero rate the IMS resource usage for Access leg establishment.

This can be performed using the correlation mechanism with the VCC AS CDR for Access leg.

Next Modified Section

Annex B (informative):
Bibliography

a)	The 3GPP charging specifications

-	3GPP TS 32.252: "Telecommunication management; Charging management; Wireless Local Area Network (WLAN) charging".

-	3GPP TS 32.270: "Telecommunication management; Charging management; Multimedia Messaging Service (MMS) charging".

-	3GPP TS 32.271: "Telecommunication management; Charging management; Location Services (LCS) charging".

-	3GPP TS 32.296: "Telecommunication management; Charging management; Online Charging System (OCS) applications and interfaces".

b)	Common 3GPP specifications

-	3GPP TS 22.101: "Service aspects; Service Principles".

-	3GPP TS 22.115 "Service aspects; Charging and Billing".

-	3GPP TS 23.003: "Numbering, addressing and identification".

-	3GPP TS 27.001: "General on Terminal Adaptation Functions (TAF) for Mobile Stations (MS)".

c)	other Domain and Service specific 3GPP / ETSI specifications

-

d)	Relevant ITU Recommendations

-	ITU-T Recommendation D.93: "Charging and accounting in the international land mobile telephone service (provided via cellular radio systems)".

-	ITU-T Recommendation E.164: "The international public telecommunication numbering plan".

-	ITU-T Recommendation Q.767: "Application of the ISDN user part of CCITT signalling System No.7 for international ISDN interconnections".

-	ITU-T Recommendation X.25: "Interface between Data Terminal Equipment (DTE) and Data Circuit-terminating Equipment (DCE) for terminals operating in the packet mode and connected to public data networks by dedicated circuit".

-	ITU-T Recommendation X.121: "International numbering plan for public data networks".

e)	Relevant IETF RFCs

-	IETF RFC 959 (1985): "File Transfer Protocol".

End of modifications