3GPP TSG-SA5 (Telecom Management)	 S5-070244

- Meeting SA5#51, 22 - 26 Jan 2007, Seville, SPAIN

| CR-Form-v9.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.405 | CR | 0024 |  rev | - |  Current version: | 7.3.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Add measurements related to number of HSDPA users |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | China Mobile (liangshuangchun@sjy.chinamobile.com, liyewen@chinamobile.com) |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | OAM7-PM |  |  |  |  |  |  | Date:  |  | 26/01/2007 |
| Category:  | B |  |  |  |  |  |  | Release:  |  | Rel-7 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) |  |
| Reason for change:  |  | With the number of HSDPA user of a serving HS-DSCH cell measurement, the operator will learn how many users use HSDPA. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Add measurement about HSDPA.MeanNbrUser |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  |  |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | New clauses 4.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | x | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | x | Test specifications |  |  |  |  |  |  |
|  |  |  | x | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

## 4.x	Mean number of HSDPA users in a serving HS-DSCH cell

- This measurement provides the mean number of simultaneous HSDPA users in a serving HS-DSCH cell.

- SI.

- This measurement is obtained by sampling at a pre-defined interval the number of simultaneous users in a serving HS-DSCH cell and then taking the arithmetic mean.

- A single integer value.

- HSDPA.MeanNbrUser

- UtranCell.

- Valid for packet switched traffic.

- UMTS.

End of modifications

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Cat | Old | New |
| Mar 2006 | SA_31 | SP-060108 | -- | -- | Split of 32.403-710 into four new TSs 32.404, 32.405, 32.406, 32.408. Submitted to TSG SA #31 for Approval. | -- | 1.0.0 | 7.0.0 |
| Jun 2006 | SA_32 | SP-060260 | 0001 | -- | Add TDD Counters to Measure Mean Code Resources Used | B | 7.0.0 | 7.1.0 |
| Jun 2006 | SA_32 | SP-060260 | 0002 | -- | Add ATM layer performance measurements | B | 7.0.0 | 7.1.0 |
| Jun 2006 | SA_32 | SP-060260 | 0003 | -- | Add Power Counters for radio channel monitoring | B | 7.0.0 | 7.1.0 |
| Jun 2006 | SA_32 | SP-060260 | 0004 | -- | Add measurements about Radio Access Bearer (RAB) related to Iu release request | B | 7.0.0 | 7.1.0 |
| Jun 2006 | SA_32 | SP-060260 | 0005 | -- | Add paging counters for UTRAN | B | 7.0.0 | 7.1.0 |
| Jun 2006 | SA_32 | SP-060260 | 0006 | -- | Correction for wrong reference to TS 25.331 | F | 7.0.0 | 7.1.0 |
| Jun 2006 | SA_32 | SP-060260 | 0007 | -- | Correction for TDD Counters to Measure Max Code Resources Used | F | 7.0.0 | 7.1.0 |
| Jun 2006 | SA_32 | SP-060260 | 0008 | -- | Add measurements about SHO radio link performance | B | 7.0.0 | 7.1.0 |
| Sep 2006 | SA_33 | SP-060663 | 0009 | 1 | Add RRC Connection Usage Measurements to fully gauge the usage of the UTRAN | B | 7.1.0 | 7.2.0 |
| Sep 2006 | SA_33 | SP-060663 | 0010 | 1 | Correction to TDD measurements for Received Total Wideband Power and Transmitted Carrier Power | F | 7.1.0 | 7.2.0 |
| Sep 2006 | SA_33 | SP-060550 | 0011 | -- | Add measurements about RAB with different data rates of conversational service - Align with RAN2's 25.993 | B | 7.1.0 | 7.2.0 |
| Sep 2006 | SA_33 | SP-060550 | 0012 | -- | Add measurements about the non-HS carrier power - Align with RAN3's 25.433 | B | 7.1.0 | 7.2.0 |
| Sep 2006 | SA_33 | SP-060550 | 0013 | -- | Add Uplink ISCP measurements in PM for TDD | B | 7.1.0 | 7.2.0 |
| Sep 2006 | SA_33 | SP-060550 | 0014 | -- | Add measurements about the non-HS carrier power for TDD | B | 7.1.0 | 7.2.0 |
| Sep 2006 | SA_33 | SP-060550 | 0015 | -- | Constrain the applicability of performance measurements related to Iur interface | C | 7.1.0 | 7.2.0 |
| Sep 2006 | SA_33 | SP-060550 | 0016 | -- | Add measurements about call duration in utran cell for CS domain | B | 7.1.0 | 7.2.0 |
| Sep 2006 | SA_33 | SP-060550 | 0017 | -- | Add measurements related to channel switches for HSDPA | B | 7.1.0 | 7.2.0 |
| Sep 2006 | SA_33 | SP-060550 | 0018 | -- | Add measurements about the HSDPA mobility management | B | 7.1.0 | 7.2.0 |
| Sep 2006 | SA_33 | SP-060550 | 0019 | -- | Add HSDPA setup related measurements | B | 7.1.0 | 7.2.0 |
| Dec 2006 | SA_34 | SP-060726 | 0020 | -- | Correction of the HHO measurements | C | 7.2.0 | 7.3.0 |
| Dec 2006 | SA_34 | SP-060726 | 0021 | -- | Add Measurements related to MAC-hs | B | 7.2.0 | 7.3.0 |
| Dec 2006 | SA_34 | SP-060726 | 0022 | -- | Correct the measurements related to RAB release request by CN - Align with 25.413 | F | 7.2.0 | 7.3.0 |