3GPP TSG-SA5 (Telecom Management)	S5-070257

- Meeting SA5#51, 22 - 26 Jan 2007, Seville, SPAIN

| CR-Form-v9.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.299 | CR | 0176 |  rev | - |  Current version: | 7.4.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Correction to the definition of SCUR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Nokia, Siemens (benni.alexander@nokia.com, gerald.goermer@siemens.com) |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | CH7 |  |  |  |  |  |  | Date:  |  | 26/01/2007 |
| Category:  | F |  |  |  |  |  |  | Release:  |  | Rel-7 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R97 (Release 1997) R98 (Release 1998) R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) |  |
| Reason for change:  |  | The ambiguous definition of SCUR would lead to different and conflicting implementations. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | The definition of SCUR is corrected. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | Vendors would end up implementing SCUR differently, which will lead to compatibilty and charging errors. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 5.2.2.3.1, 5.2.2.3.2, 5.2.2.3.3, 5.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

1st Modified Section

##### 5.2.2.3.1	Decentralized Unit Determination and Centralized Rating

In the following scenario, the CTF requests the reservation of units prior to session supervision. An account debit operation is carried out following the conclusion of session termination.

Figure 5.2.2.3.1: Session Charging with Reservation / Decentralized Unit Determination and Centralized Rating

- Request for resource usage: The UE-A requests session establishment from the CTF.

- Units Determination: depending on the requested type of the session the CTF determines the number of units accordingly.

- Reserve Units Request: the CTF requests the OCF to reserve the number of units determined in item 2

- Rating Control: assisted by the rating entity the OCF calculates the number of monetary units that represents the price for the number of units determined in item 2.

- Account Control: the OCF checks whether the user's account balance is sufficient for the requested reservation.

- Reservation Control: if the user's account balance is sufficient then the corresponding reservation is made.

- Reserve Units Response: the OCF informs the CTF of the reserved number of units.

- Reserved Units Supervision: simultaneously with the ongoing session, the CTF monitors the consumption of the reserved units.

- Session ongoing: the CTF maintains the session. One or more debit and reserve operations may be performed when the session is ongoing.

- Session Release: the session is released

- Debit Units Request: the CTF requests the OCF to assure the deduction of an amount corresponding to the consumed number of units from the subscriber's account.

- Rating Control: assisted by the rating entity the OCF calculates the number of monetary units to deduct from the subscriber's account.

- Account Control: the OCF triggers the deduction of the calculated amount from the subscriber's account.

- Debit Units Response: the OCF informs the CTF of the actually deducted units.

Next Modified Section

##### 5.2.2.3.2	Centralized Unit Determination and Centralized Rating

In the following scenario, the CTF requests the OCF to reserve units based on the session identifiers specified by the CTF. An account debit operation is carried out following the conclusion of session.

Figure 5.2.2.3.2: Session Charging with Reservation / Centralized Unit Determination and Centralized Rating

- Request for resource usage: The UE-A requests the session establishment from the CTF.

- Reserve Units Request: depending on the requested type of the session by the UE-A, the CTF selects the service identifier and forwards the Reserve Units Request to the OCF.

- Units Determination: the OCF determines the number of non-monetary units needed for the content/service delivery, based on the received service key.

- Rating Control: assisted by the rating entity the OCF calculates the number of monetary units that represent the price for the number of units determined in item 3.

- Account Control: the OCF checks whether the user's account balance is sufficient for the requested reservation.

- Reservation Control: if the user's account balance is sufficient, then the corresponding reservation is made.

- Reserve Units Response: the OCF informs the CTF of the reserved number of units. This includes the case where the number of units reserved indicates the permission to render the service that was identified by the received service key.

- Granted Units Supervision: simultaneously with the ongoing session, the CTF monitors the consumption of the reserved units.

- Session ongoing: the CTF maintains the session. One or more debit and reserve operations may be performed when the session is ongoing.

- Session Released: the session is released.

- Debit Units Request: the CTF requests the OCF to assure the deduction of an amount corresponding to the consumed number of units from the subscriber’s account

- Rating Control: assisted by the rating entity the OCF calculates the number of monetary units to deduct from the subscriber's account.

- Account Control: the OCF triggers the deduction of the calculated amount from the subscriber's account.

- Debit Units Response: the OCF informs the CTF of the actually deducted units.

Next Modified Section

##### 5.2.2.3.3	Decentralized Unit Determination and Decentralized Rating

In the following scenario, the CTF request the OCF to assure the reservation of an amount of the specified number of monetary units from the subscriber's account. An account debit operation that triggers the deduction the amount from the subscriber's account is carried out following the conclusion of session establishment.

Figure 5.2.2.3.3: Session Charging with Reservation / Decentralized Unit Determination and Decentralized Rating

- Request for resource usage: The UE-A requests the session establishment from the CTF.

- Units Determination: depending on the requested type of the session by the UE-A, the CTF determines the number of units accordingly.

- Rating Control: the CTF calculates the number of monetary units that represent the price for the number of units determined in item 2.

- Reserve Units Request: the CTF requests the OCF to assure the reservation of an amount corresponding to the calculated number of monetary units from the subscriber's account.

- Account Control: the OCF checks whether the user's account balance is sufficient for the requested reservation.

- Reservation Control: if the user's credit balance is sufficient, then the corresponding reservation is made.

- Reserve Units Response: the OCF informs the CTF of the reserved number of monetary units.

- Budget Control: simultaneously with the ongoing session, the CTF monitors the consumption of the granted amount.

- Session ongoing: the CTF maintains the session. One or more debit and reserve operations may be performed when the session is ongoing.

- Session Released: the session is released.

- Debit Units Request: the CTF requests the OCF to assure the deduction of an amount corresponding to the consumed number of monetary units from the subscriber's account.

- Account Control: the OCF triggers the deduction of the consumed amount from the subscriber's account.

- Debit Units Response: the OCF indicates to the CTF the number of deducted monetary units.

Next Modified Section

### 5.2.3	Basic Operations

On-line credit control uses two basic logical operations: Debit Units and Reserve Units.

- "Debit Units Request"; sent from CTF  OCF
After receiving a service request from the subscriber, the CTF sends a Debit Units Request to the OCF. The CTF may either specify a service identifier (centralised unit determination) or the number of units requested (decentralised unit determination).

- "Debit Units Response"; sent from OCF  CTF 
The OCF replies with a Debit Units Response, which informs the CTF of the number of units granted as a result of the Debit Units Request. This includes the case where the number of units granted indicates the permission to render the requested service.

- "Reserve Units Request"; sent from CTF  OCF
Request to reserve a number of units for the service to be provided by an CTF. In case of centralised unit determination, the CTF specifies a service identifier in the Reserve Unit Request, and the OCF determines the number of units requested. In case of decentralised unit determination, the number of units requested is specified by the CTF.

- "Reserve Units Response"; sent from OCF  CTF 
Response from the OCF which informs the CTF of the number of units that were reserved as a result of the "Reserve Units Request".

IEC uses the Direct Debiting One Time Event procedure specified in RFC 4006 and therefore is performed by the use of the logical "Debit Units" operation, as specified in section 6.3.3.

SCUR and ECUR use both the "Debit Units" and "Reserve Units" operations.

SCUR uses the Session Based Credit Control procedure specified in RFC 4006. In session charging with unit reservation, when the "Debit Units" and "Reserve Units" operations are both needed, they shall be combined in one message, as specified in section 6.3.5.

For SCUR and ECUR the consumed units are deducted from the subscriber's account after service delivery. Thus, the reserved and consumed units are not necessarily the same. Using this operation, it is also possible for the CTF to modify the current reservation, including the return of previously reserved units.

Table 5.2.3.1 and table 5.2.3.2 describe the content of these operations.

Table 5.2.3.1: Debit and Reserve Units Request Content

| Debit and Reserve Units Request | Category | Description |
| --- | --- | --- |
| Session Identifier | M | This field identifies the operation session. |
| Originator Host | M | This field contains the identification of the source point of the operation. |
| Originator Domain | M | This field contains the realm of the operation originator. |
| Destination Domain | M | This field contains the realm of the operation destination. |
| Operation Identifier | M | This field is a unique operation identifier. |
| Operation Token | M | This field contains the service identifier. |
| Operation Type | M | This field defines the transfer type: event for event based charging and start, interim, stop for session based charging. |
| Operation Number | M | This field contains the sequence number of the transferred messages. |
| Destination Host | OC | This field contains the identification of the destination point of the operation. |
| User Name | OC | This field contains the identification of the user. |
| Origination State | OC | Tbd. |
| Origination Timestamp | OC | This field contains the time when the operation is requested. |
| Subscriber Identifier | OM | This field contains the identification of the mobile subscriber (i.e. MSISDN) that uses the requested service. |
| Termination Cause | OC | This field contains the termination reason of the service. |
| Requested Action | OC | This field contains the requested action. |
| Multiple Operation | OM | This field indicate the occurrence of multiple operations. |
| Multiple Unit Operation | OM | This field contains the parameter for the quota management. |
| Subscriber Equipment Number | OC | This field contains the identification of the mobile device (i.e. IMEI) that uses the subscriber. |
| Proxy Information | OC | This field contains the parameter of the proxy. |
| Route Information | OC | This field contains the parameter of the route. |
| Service Information | OM | This parameter holds the individual service specific parameters as defined in the corresponding ‘middle tier’ TS. |

Table 5.2.3.2: Debit and Reserve Units Response Content

| Debit and Reserve Units Response | Category | Description |
| --- | --- | --- |
| Session Identifier | M | This field identifies the operation session. |
| Operation Result | M | This field identifies the result of the operation. |
| Originator Host | M | This field contains the identification of the source point of the operation. |
| Originator Domain | M | This field contains the realm of the operation originator. |
| Operation Identifier | M | This field is a unique operation identifier. |
| Operation Type | M | This field defines the transfer type: event for event based charging and start, interim, stop for session based charging. |
| Operation Number | M | This field contains the sequence number of the transferred messages. |
| Operation Failover | OC | This field contains an indication to the CTF whether or not a failover handling is to be used when necessary. |
| Multiple Unit Operation | OM | This field contains the parameter for the quota management. |
| Operation Failure Action | OC | For credit control sessions the content of this field enables the credit-control client to decide what to do if sending credit-control messages to the credit-control server has been temporarily prevented. |
| Operation Event Failure Action | OC | For one time event direct debiting the content of this field enables the credit-control client to decide what to do if sending credit-control messages to the credit-control server has been temporarily prevented. |
| Redirection Host | OC | Tbd. |
| Redirection Host Usage | OC | Tbd. |
| Redirection Cache Time | OC | Tbd. |
| Proxy Information | OC | This field contains the parameter of the proxy. |
| Route Information | OC | This field contains the parameter of the route. |
| Failed parameter | OC | This field contains missing and/or unsupported parameter that caused the failure. |
| Service Information | OC | This parameter holds the individual service specific parameters as defined in the corresponding ‘middle tier’ TS. |

End of modifications