3GPP TSG-SA5 (Telecom Management)	S5-102907

SA5#74-CH, 8 - 10 Nov 2010, Issy-les-Moulineaux, France	revision of S5-102862

| CR-Form-v9.6 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.296 | CR | 0013 |  rev | 2 |  Current version: | 9.1.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Correction of price and tariff request scenarios |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Huawei |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | CH9 |  |  |  |  |  |  | Date:  |  | 10/11/2010 |
| Category:  | F |  |  |  |  |  |  | Release:  |  | Rel-9 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) |  |
| Reason for change:  |  | Steps in the description of the sequence are not aligned with the diagram itself. No node is avaliable |  |  |  |  |  |  |  |  |
| Summary of change:  |  | EBCF is corrected to SBCF Removal of the reference to the node |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | Wrong description of the sequences |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 6.2.1.1.2, 6.2.1.2.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

First change

##### 6.2.1.1.2	PriceRequest scenario with Unit Reservation

The following figure describes the case where the EBCF invokes the PriceRequest method in an event charging with unit reservation (ECUR) scenario.

Figure : Price Request Method with unit reservation

Step 1:	The EBCF receives an online charging request for a certain event/service.

Step 2:	The EBCF requests account and counter information for the subscriber from the Account Balance Management Function.

Step 2' (Optional):In case there is no existing subscriber's context, the EBCF creates a new subscriber's context information. Otherwise, the EBCF updates the existing subscriber's context and gets the list of active services for the given subscriber. This Step only applies when correlation is enabled.

Step 3:	Upon receipt of this data, the EBCF sends a price request to the Rating Function in order to determine the price of the desired service. Note that this scenario assumes that the EBCF has not received any service cost information in the online charging request.

Step 4:	The Rating Function calculates the price and counter updates for the given service according to the service and subscriber specific information included in the request.

Step 5:	The calculated price and counter updates are returned to the EBCF.

Step 6:	The EBCF continues event charging and performs account and counter control (i.e. it makes reservations for the requested service).

Step 7:	The EBCF sends the appropriate online charging response (i.e. it grants or denies service delivery).

Step 8:	After service delivery, the serving network node sends an indication of successful (or unsuccessful) service delivery to the EBCF.

Step 9:	The EBCF performs account and counter control (i.e. it adjusts the account and counter values accordingly).

Step 10:	Finally, the EBCF sends an acknowledgment message to the serving network node in order to terminate the online charging dialogue.

If the second online charging request (step 8) contains updated information about the consumed service, a second Price Retrieval Operation may occur between step 8 and step 9, i.e. the EBCF may send a second PriceRequest containing the updated information to the Rating Function. The Rating Function will send an appropriate PriceResponse, and the information contained in this second PriceResponse may be used by the EBCF for account and counter control (step 9).

If the second online charging request (step 8) contains information about the release of the ongoing service, the EBCF updates the existing subscriber's context to remove the entry related to this service and to get the list of remaining active services. A second Price Retrieval Operation may occur between step 8 and step 9, i.e. the EBCF may send a second PriceRequest containing the updated information to the Rating Function. The Rating Function will send an appropriate PriceResponse containing the updated price for the remaining ongoing services and the information contained in this second PriceResponse may be used by the EBCF for account and counter control (step 9).

Next change

##### 6.2.1.2.1	Basic TariffRequest scenario

The following figure shows a basic message flow for the tariff request method.

Figure : Basic Tariff request method

Step 1:	The SBCF receives an online charging request referring to an MS's bearer/session resource usage.

Step 2:	The SBCF requests account and counter information for the subscriber from the Account Balance Management Function.

Step 2' (Optional):In case there is no existing subscriber's context, the SBCF creates a new subscriber's context information. Otherwise, the SBCF updates the existing subscriber's context and gets the list of active services for the given subscriber. This Step only applies when correlation is enabled.

Step 3:	Upon receipt of this data, the SBCF requests tariff information applicable for this bearer/session.

Step 4:	The Rating Function retrieves the appropriate tariff to be applied for the bearer/session.

Step 5:	The Rating Function returns the tariff information to the SBCF.

Step 6:	Based on the received tariff information, the SBCF determines the granted units and the price.

Step 7:	The SBCF continues bearer/session charging and performs account and counters control (i.e. it makes reservations).

Step 8:	Assuming successful account control, the SBCF returns the granted units to the requesting network element. Service usage can start immediately or later.

Step 9:	When the granted units have been used, a new request is send from the serving network element to the SBCF.

Step 10:	This time the SBCF can directly determine the granted units and the price.

Step 11:	The SBCF performs account and counter control (i.e. it makes reservations).

Step 12:	Again, assuming successful account control, a positive acknowledgment (including granted units) is returned to the network entity.

Step 13:	The MS terminates bearer/session usage. The used units are sent to the SBCF.

Step 14:	The SBCF performs final rating for the consumed bearer/session resources.

Step 15:	The SBCF adjusts the account and counters accordingly.

Step 16:	Finally, the SBCF sends an acknowledgment message to the serving network node in order to terminate the online charging dialogue.

In this basic scenario only one request to the Rating Function is needed during the whole online charging session.

The message flow in the previous figure is an example of an online charging session to illustrate the usage of the basic Tariff Request method on the Re interface. The sequence of steps 9-12 is optional or can be repeated multiple times.

If the SBCF detects that the account balance is not sufficient for a minimum number of granted units (in step 6 in the example), there will be no reservation (step 7 will be skipped), and the SBCF will deny the online charging request (in step 8), i.e. the service usage request will be rejected by the mobile network. The same handling applies if the account reservation fails (in step 7, 11). Note that this can only occur, if the account is modified independently of the current online charging session, e.g. due to several parallel online charging sessions. This handling of insufficient account balances applies for all class "A" TariffRequest scenarios, i.e. also for the following subsections.

This basic scenario assumes, that the tariff information is valid for the whole online charging session (i.e. the tariff is not affected by the amount of resources used, the time of day, etc.).

In addition to the tariff information, the TariffResponse may also include "Basic Price", that is deducted directly from the subscriber's account balance. This may be used e.g. to realize a basic fee, that is applicable once per day/week/month, if a certain service is used. Note that the Basic Price may be service specific. The SBCF has to include the timestamp of the last charging of the Basic Price in the TariffRequest, if the Basic Price is applicable. This handling applies for all class "A" TariffRequest scenarios, i.e. also for the following subsections.

End of changes