3GPP TSG-SA5 (Telecom Management)	S5-102915

Meeting SA5#74-CH, 08 – 10 Nov 2010, Issy-les-Moulineaux, FRANCE	revision of S5-102831

| CR-Form-v9.6 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.298 | CR | 0163 |  rev | 2 |  Current version: | 10.2.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Add missing Charging Data Record (CDR) tag for MMTelRecord |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | ZTE, China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | CH8 |  |  |  |  |  |  | Date:  |  | 10/11/2010 |
| Category:  | A |  |  |  |  |  |  | Release:  |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) |  |
| Reason for change:  |  | The tag for MMTelRecord is missing |  |  |  |  |  |  |  |  |
| Summary of change:  |  | Add missing CDR tag for MMTelRecord |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | Billing Systems can not decode MMTelRecord |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 5.2.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

First change

#### 5.2.4.5	MMTel CDRs

This subclause contains the abstract syntax definitions that are specific to the CDR types defined in TS 32.275 [35].

MMTelChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) mMTelChargingDataTypes (9) asn1Module (0) version1 (0)}

DEFINITIONS IMPLICIT TAGS	::=

- BEGIN

-- EXPORTS everything

- IMPORTS

RecordType, IMSI, IMEI, IPAddress, LocalSequenceNumber, ManagementExtensions, MSISDN, NodeAddress, SubscriptionID, TimeStamp, ServiceContextID

FROM GenericChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) genericChargingDataTypes (0) asn1Module (0) version1 (0)}

CarrierSelectRouting, CauseForRecordClosing, Early-Media-Components-List, IMS-Charging-Identifier, IMSCommunicationServiceIdentifier, Incomplete-CDR-Indication, InterOperatorIdentifiers, InterOperatorIdentifierlist, InvolvedParty, ListOfInvolvedParties, MessageBody,

Media-Components-List, NumberPortabilityRouting, Service-Id, Session-Id, SessionPriority, SIP-Method

FROM IMSChargingDataTypes {itu-t (0) identified-organization (4) etsi(0) mobileDomain (0) charging (5) imsChargingDataTypes (4) asn1Module (0) version1 (0)}

;

------------------------------------------------------------------------------

--

--  MMTel RECORDS

--

------------------------------------------------------------------------------

MMTelServiceRecord	::= CHOICE

--

-- Record values 83 are MMTel specific

--

{

mMTelRecord			[83] MMTelRecord

}

MMTelRecord 	::= SET

{

recordType							[0] RecordType,

retransmission						[1] NULL OPTIONAL,

sIP-Method							[2] SIP-Method OPTIONAL,

role-of-Node						[3] Role-of-Node OPTIONAL,

nodeAddress							[4] NodeAddress OPTIONAL,

session-Id							[5] Session-Id OPTIONAL,

list-Of-Calling-Party-Address		[6] ListOfInvolvedParties OPTIONAL,

called-Party-Address				[7] InvolvedParty OPTIONAL,

serviceRequestTimeStamp				[9] TimeStamp OPTIONAL,

serviceDeliveryStartTimeStamp		[10] TimeStamp OPTIONAL,

serviceDeliveryEndTimeStamp			[11] TimeStamp OPTIONAL,

recordOpeningTime					[12] TimeStamp OPTIONAL,

recordClosureTime					[13] TimeStamp OPTIONAL,

interOperatorIdentifiers			[14] InterOperatorIdentifierlist OPTIONAL,

localRecordSequenceNumber			[15] LocalSequenceNumber OPTIONAL,

recordSequenceNumber				[16] INTEGER OPTIONAL,

causeForRecordClosing				[17] CauseForRecordClosing OPTIONAL,

incomplete-CDR-Indication			[18] Incomplete-CDR-Indication OPTIONAL,

iMS-Charging-Identifier				[19] IMS-Charging-Identifier OPTIONAL,

list-Of-SDP-Media-Components		[21] SEQUENCE OF Media-Components-List OPTIONAL,

gGSNaddress							[22] NodeAddress OPTIONAL,

serviceReasonReturnCode				[23] UTF8String OPTIONAL,

list-Of-Message-Bodies				[24] SEQUENCE OF MessageBody OPTIONAL,

recordExtensions					[25] ManagementExtensions OPTIONAL,

expiresInformation					[26] INTEGER OPTIONAL,

event								[28] UTF8String OPTIONAL,

accessNetworkInformation			[29] OCTET STRING OPTIONAL,

serviceContextID					[30] ServiceContextID OPTIONAL,

list-of-subscription-ID				[31] SEQUENCE OF SubscriptionID OPTIONAL,

list-Of-Early-SDP-Media-Components 	[32] SEQUENCE OF Early-Media-Components-List OPTIONAL,

iMSCommunicationServiceIdentifier	[33] IMSCommunicationServiceIdentifier OPTIONAL,

numberPortabilityRouting			[34] NumberPortabilityRouting OPTIONAL,

carrierSelectRouting				[35] CarrierSelectRouting OPTIONAL,

sessionPriority						[36] SessionPriority OPTIONAL,

requested-Party-Address				[101] InvolvedParty OPTIONAL,

list-Of-Called-Asserted-Identity 	[102] ListOfInvolvedParties OPTIONAL,

mMTelInformation					[110] MMTelInformation	OPTIONAL

}

------------------------------------------------------------------------------

--

--  MMTel DATA TYPES

--

------------------------------------------------------------------------------

MMTelInformation			::= SET

{

ListOfSupplServices	    [0] SEQUENCE OF SupplService OPTIONAL,

subscriberRole			[1] SubscriberRole OPTIONAL

}

- ParticipantActionType ::= ENUMERATED

{

CREATE_CONF         (0),

JOIN_CONF           (1),

INVITED_INTO_CONF   (2),

QUIT_CONF           (3)

}

- SupplService := SET

{

serviceType				    [0] ServiceType,

serviceMode				    [1] ServiceMode OPTIONAL,

numberOfDiversions		    [2] INTEGER OPTIONAL,

associated-Party-Address	[3] InvolvedParty OPTIONAL,

serviceId		            [4] Service-Id OPTIONAL,

changeTime                  [5] TimeStamp,

numberOfParticipants        [6] INTEGER OPTIONAL,

participantActionType       [7] ParticipantActionType OPTIONAL,

cUGInformation				[8]	OCTET STRING OPTIONAL

}

- ServiceType ::= INTEGER

--

-- Values   1024 are reserved for specific Network/Manufacturer variants

--

{

oIPresentation		(0),

oIRestriction		(1),

tIPresentation		(2),

tIRestriction  		(3),

hOLD				(4),

cBarring			(5),

cDIVersion			(6),

cDIVersionNotif		(7),

cWaiting			(8),

mWaitingIndic		(9),

cONF				(10),

fLexibleAlerting	(11),

cCBS				(12),

cCNR				(13),

mCID				(14),

cAT					(15),

cUG					(16) ,

pNM					(17),

cRS					(18)

}

- ServiceMode ::= INTEGER

--

-- Values   1024 are reserved for specific Network/Manufacturer variants

--

{	cFunCond			(0),

cFbusy				(1),

cFnoReply 			(2),

cFnotLogged			(3),

deflection			(4),

notReach			(5),

iCBarring			(6),

oCBarring			(7),

aCRejection 		(8),

eCTBlind			(9),

eCTConsultative		(10),

threePTY			(11)

}

- SubscriberRole ::= ENUMERATED

{

originating			(0),

terminating			(1)

}

- END

End of changes