3GPP TSG-SA5 (Telecom Management)	S5-102928

SA5#74-CH, 8 - 10 Nov 2010, Issy-les-Moulineaux, FRANCE	revision of S5-102860

| CR-Form-v9.6 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 32.252 | CR | 0006 |  rev | 2 |  Current version: | 9.0.0 |  |
| For HELP on using this form look at the pop-up text over the  symbols. Comprehensive instructions on how to use this form can be found at http://www.3gpp.org/specs/CR.htm. |  |  |  |  |  |  |  |  |

| Proposed change affects:  | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title:  | Add Packet Data Gateway (PDG) charging when re-using GGSN functionality – Align with SA2 TS 23.234 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG:  | Orange |  |  |  |  |  |  |  |  |  |
| Source to TSG:  | S5 |  |  |  |  |  |  |  |  |  |
| Work item code:  | eIWLAN_Mob |  |  |  |  |  |  | Date:  |  | 10/11/2010 |
| Category:  | B |  |  |  |  |  |  | Release:  |  | Rel-10 |
|  | Use one of the following categories: F (correction) A (corresponds to a correction in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: R99 (Release 1999) Rel-4 (Release 4) Rel-5 (Release 5) Rel-6 (Release 6) Rel-7 (Release 7) Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) |  |
| Reason for change:  |  | The charging architecture for IWLAN charging when PDG is implemented by re-using GGSN functionalities is not covered. |  |  |  |  |  |  |  |  |
| Summary of change:  |  | A new annex is proposed to cover this scenario The list of references and Abbreviations are updated. |  |  |  |  |  |  |  |  |
| Consequences if  not approved: |  | PDG charging when re-using GGSN functionality is not covered. |  |  |  |  |  |  |  |  |
| Clauses affected:  |  | 2, 3.3, Annex X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs  |  |  | X | Other core specifications  |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
|  |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments:  |  |  |  |  |  |  |  |  |  |  |

First change

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.240: "Telecommunication management; Charging management; Charging architecture and principles".

[2]-[10]	void.

[11]	3GPP TS 32.251: "Telecommunication management; Charging management; Packet Switched (PS) domain charging".

[12]-[49]	Void.

[50]	3GPP TS 32.299: "Telecommunication management; Charging management; Diameter charging applications".

[51]	3GPP TS 32.298: "Telecommunication management; Charging management; Charging Data Record (CDR) parameter description".

[52]	3GPP TS 32.297: "Telecommunication management; Charging management; Charging Data Records (CDR) file format and transfer".

[53]	Void

[54]	3GPP TS 32.295: "Telecommunication management; Charging management; Charging Data Record (CDR) transfer".

[55]-[70]	Void.

[71]	3GPP TS 23.125: "Overall high level functionality and architecture impacts of flow based charging; Stage 2".

[72]-[99]	Void.

[100]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[101]-[199]	Void.

[200]	3GPP TS 22.934: "Feasibility study on 3GPP system to Wireless Local Area Network (WLAN) interworking".

[201]	3GPP TS 23.234: "3GPP system to Wireless Local Area Network (WLAN) interworking; System description".

[202]	3GPP TS 23.327: "Mobility between 3GPP-Wireless Local Area Network (WLAN) interworking and 3GPP systems".

[203]	Void

[204]	3GPP TS 29.234: "3GPP system to Wireless Local Area Network (WLAN) interworking; Stage 3".

[205]-[400]	Void.

[401]	IETF RFC 3588: "Diameter Base Protocol".

[402]	IETF RFC 4006: "Diameter Credit-Control Application".

[403]	Void

[404]	IETF RFC 4005: "Diameter Network Access Server Application".

[405]	IETF draft-ietf-geopriv-radius-lo-04: "Carrying Location Objects in RADIUS".

Next change

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations defined in 3GPP TR 21.905 [50], 3GPP TS 22.934 [200], 3GPP TS 23.234 [201], 3GPP TS 23.934 (see bibliography), 3GPP TS 32.240 [1] and the following apply:

3GPP	3rd Generation Partnership Project

BD	Billing Domain

CDR	Charging Data Record

EPS	Evolved Packet Systems

GGSN	Gateway GPRS Support Node

GTP	GPRS Tunnelling Protocol

I-WLAN	Interworking WLAN

IP	Internet Protocol

LAN	Local Area Network

PS	Packet Switched

PDG	Packet Data Gateway

PDP	Packet Data Protocol, e.g. IP

SGSN	Serving GPRS Support Node

TTG	Tunnel Termination Gateway

TS	Technical Specification

UMTS	Universal Mobile Telecommunications System

WLAN	Wireless LAN

Next change

Annex X (normative):
Re-using the GGSN charging functions to implement the PDG charging functions

This annex does not introduce new charging requirements for the Packet Data Gateway (PDG).

# X.1	General

3GPP TS 23.234 [201] depicts a PDG implementation that re-uses GGSN functionality. Existing GGSN deployments may be re-used to implement the PDG functionality via using a subset of the Gn reference point (Gn’). A Tunnel Termination Gateway (TTG) is used to cover all aspects of the PDG that are not covered by the GGSN. In such implementation, the GGSN charging gateway interfaces are used to provide charging gateway interfaces of the PDG. The following figure depicts a PDG implementation that re-uses GGSN charging functionality. The Ga reference point between the GGSN and the CGF is used to transfer offline charging records from the PDG CDF to the CGF (Wz reference point).

Figure X.1: PDG implementation re-using GGSN charging functionality

# X.2	No re-use of policy control in the GGSN

The end-to-end tunnel between the WLAN UE and the PDG is setup according to the procedure described in 3GPP TS 23.234 [208]. In a configuration when the Gn' reference point is used, the end-to-end tunnel setup between the WLAN UE and the PDG is terminated by the TTG of the PDG, and the setup of GTP tunnel(s) is triggered towards the GGSN part of the PDG. In such configuration, end-to-end tunnel is mapped one-to-one to a GTP tunnel, as illustrated in the figure X.2.

Figure X.2: Mapping between end-to-end tunnel and single GTP tunnel

NOTE:	When the policy control in the GGSN is used, the mapping between end-to-end and GTP tunnel may not be one-to-one. This configuration is FFS.

## X.2.1	Offline charging

In this configuration, the GGSN collects charging data related to the PDP contexts in the G-CDR, as described in the Rel-7 GGSN charging (3GPP TS 32.251 [2]). Charging information for WLAN 3GPP IP access (WLAN CDR) shall be collected by the GGSN in the G-CDR according to the following mapping table:

Table X.2.1.1 : Mapping between WLAN CDR and G-CDR fields

| G-CDR field | G-CDR Category | WLAN-CDR mapping guidance |
| --- | --- | --- |
| Record Type | M | Record Type |
| Network initiated PDP context | OC | Not applicable in this 3GPP relase |
| Served IMSI | M | Served IMSI |
| Served IMEISV | OC | Not applicable in this 3GPP 3GPP relase |
| GGSN Address used | M | PDG Address used |
| Charging ID | M | Charging ID |
| SGSN Address | M | Serving WAG address |
| Access Point Name Network Identifier | OM | Access Point Name Network Identifier |
| PDP Type | OM | Not applicable in this relase |
| Served PDP Address | OC | WLAN UE remote IP address |
| Dynamic Address Flag | OC | Not applicable in this 3GPP relase |
| List of Traffic Data Volumes | OM | Can be used to record the data volume of the PDP context, which should be equivalent to the volume of the end-to-end tunnel. NOTE: In case of I-WLAN mobility architecture and HA co-located with GGSN, the volume shall be collected as if it was collected by the HA. |
| Record Opening Time | M | Record Opening Time |
| MS Time Zone | OC | Not applicable in this 3GPP relase |
| Duration | M | Duration |
| Cause for Record Closing | M | Cause for Record Closing |
| Diagnostics | OM | Diagnostics |
| Record Sequence Number | C | Record Sequence Number |
| Node ID | OM | Node ID |
| Record Extensions | OC | Record Extensions |
| Local Record Sequence Number | OM | Local Record Sequence Number |
| APN Selection Mode | OM | APN Selection Mode |
| Served MSISDN | OM | Served MSISDN |
| User Location Information | OC | Not applicable in this 3GPP relase |
| Charging Characteristics | M | Charging Characteristics |
| Charging Characteristics Selection Mode | OM | Charging Characteristics Selection Mode |
| IMS Signalling Context | OC | Not applicable in this 3GPP relase |
| External Charging Identifier | OC | Not applicable in this 3GPP relase |
| SGSN PLMN Identifier | OM | WAG PLMN identifier |
| CAMEL Information | OC | Not applicable |
| RAT Type | OC | Can be used to differentiate G-CDR used for 3GPP access from G-CDR for WLAN access. |

NOTE: 	The “Serving AAA server/Proxy address” and “WLAN session ID” fields are not covered by the G-CDR.

## X.2.2	Online charging

The online charging is FFS.

End of changes