3GPP TS 32.409 V10.2.0 (2010-12)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Telecommunication management;

Performance Management (PM);

Performance measurements;
 IP Multimedia Subsystem (IMS)

(Release 10)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. 
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

GSM, UMTS, management, performance

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2010, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI currently being registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

# Contents

Foreword	9

Introduction	9

1	Scope	10

2	References	10

3	Measurement family and abbreviations	13

3.1	Measurement family	13

3.2	Abbreviations	13

4	Functionality related measurements	16

4.1	Measurements related to CSCF	16

4.1.1	Registration related measurements	16

4.1.1.1	Initial registration	16

4.1.1.1.1	Attempted initial registration procedures	16

4.1.1.1.2	Successful initial registration procedures	16

4.1.1.1.3	Failed initial registration procedures	17

4.1.1.1.4	IMS initial registration set-up time (Mean)	17

4.1.1.1.5	Attempted initial registration procedures per access technology	18

4.1.1.1.6	Successful initial registration procedures per access technology	18

4.1.1.2	Re-registration	18

4.1.1.2.1	Attempted re-registration procedures	18

4.1.1.2.2	Successful re-registration procedures	19

4.1.1.2.3	Failed re-registration procedures	19

4.1.1.2.4	Attempted re-registration procedures per access technology	20

4.1.1.2.5	Successful re-registration procedures per access technology	20

4.1.1.3	De-registration	21

4.1.1.3.1	De-registrations initiated by UE	21

4.1.1.3.1.4	Attempted de-registrations initiated by UE per access technology	22

4.1.1.3.1.5	Successful de-registrations initiated by UE per access technology	22

4.1.1.3.2	De-registration initiated by HSS	22

4.1.1.3.3	De-registration initiated by service platform	24

4.1.1.4	Third party registration	25

4.1.1.4.1	Attempted third party registration procedures	25

4.1.1.4.2	Successful third party registration procedures	25

4.1.1.4.3	Failed third party registration procedures	25

4.1.1.5	User registration status query	26

4.1.1.5.1	Attempted user registration status query procedures	26

4.1.1.5.2	Successful user registration status query procedures	26

4.1.1.5.3	Failed user registration status query procedures	26

4.1.1.6	S-CSCF registration/deregistration notification	27

4.1.1.6.1	Attempted S-CSCF registration/deregistration notification procedures	27

4.1.1.6.2	Successful S-CSCF registration/deregistration notification procedures	27

4.1.1.6.3	Failed S-CSCF registration/deregistration notification procedures	28

4.1.2	Session control related measurements	29

4.1.2.1	Attempted session establishments	29

4.1.2.2	Successful session establishments	29

4.1.2.3	Answered session establishments	29

4.1.2.4	Failed session establishments	30

4.1.2.5	Simultaneous online and answered sessions (Maximum)	30

4.1.2.6	Successful session establishment time (Mean)	30

4.1.2.7	Simultaneous online and answered sessions (Mean)	31

4.1.3	User location query related measurements	32

4.1.3.1	Attempted user location query procedures	32

4.1.3.2	Successful user location query procedures	32

4.1.3.3	Failed user location query procedures	32

4.1.4	Inter-network domain calling related measurements	34

4.1.4.1	Attempted session establishments from users of other network domains	34

4.1.4.2	403 (Forbidden) generated for sessions from users of other network domains	34

4.1.4.3	Attempted session establishments to users of other network domains	35

4.1.4.4	403 (Forbidden) received for sessions to users of other network domains	35

4.1.4.5	Successful session establishments from users of other network domains	36

4.1.4.6	Successful session establishments to users of other network domains	36

4.1.5	Roaming users related measurements	37

4.1.5.1	Initial registrations of visiting users from other IMS network domains	37

4.1.5.2	Number of SIP 403 (Forbidden) messages for the roamers	37

4.1.5.3	Roaming users to other IMS network domains	38

4.1.5.4	Number of 200 OK (Successful) messages for the roamers	38

4.1.6	Authentication related measurements	39

4.1.6.1	Attempted Authentication procedures	39

4.1.6.2	Successful Authentication procedures	39

4.1.6.3	Failed Authentication procedures	39

4.1.7	HSS initiated update of User Profile related measurements	41

4.1.7.1	Attempted HSS initiated User Profile Update procedures	41

4.1.7.2	Successful HSS initiated User Profile Update procedures	41

4.1.7.3	Failed HSS initiated user profile update procedures	41

4.1.8	Subscription procedure related measurements	43

4.1.8.1	Number of attempted Subscription procedures	43

4.1.8.2	Number of successful Subscription procedures	43

4.1.8.3	Number of failed Subscription procedures	43

4.1.9	Notify procedure related measurements	45

4.1.9.1	Number of emitted Notify procedures	45

4.1.9.2	Number of successful Notify procedures	45

4.1.9.3	Number of failed Notify procedures	45

4.1.10	 Immediate Messaging related measurements	46

4.1.10.1	Attempted Immediate Messaging procedures	46

4.1.10.2	Successful Immediate Messaging procedures	46

4.1.10.3	Failed Immediate Messaging procedures	46

4.1.11	 Emergency session procedure related measurements	47

4.1.11.1	Attempted emergency session procedures	47

4.1.11.2	Successful emergency session procedures	47

4.1.11.3	Failed emergency session procedures	47

4.2	Measurements related to HSS	48

4.2.1	HSS database usage related measurements	48

4.2.1.1	Provisioning related measurements	48

4.2.1.1.1	Number of provisioned IMS subscriptions currently stored in a HSS	48

4.2.1.1.2	Number of provisioned Private User Identity stored in a HSS	48

4.2.1.1.3	Number of provisioned Public User Identities with SIP URI format stored in a HSS	48

4.2.1.1.4	Number of provisioned Public User Identities with TEL URI format stored in a HSS	49

4.2.1.1.5	Number of Private Service Identities stored in a HSS	49

4.2.1.1.6	Number of Public Service Identities with SIP URI format stored in a HSS	49

4.2.1.1.7	Number of Public Service Identities with TEL URI format stored in a HSS	50

4.2.1.2	Registration Status related measurements	50

4.2.1.2.1	Number of Public User Identities in registered state	50

4.2.1.2.2	Number of Public User Identities in unregistered state	50

4.2.1.2.3	Number of Private User Identities in registered state	51

4.2.2	User registration status query related measurements	52

4.2.2.1	Attempted user registration status queries	52

4.2.2.2	Successful user registration status queries	52

4.2.2.3	Failed user registration status queries	53

4.2.3	S-CSCF registration/de-registration notification related measurements	54

4.2.3.1	Attempted S-CSCF registration/de-registration notifications	54

4.2.3.2	Successful S-CSCF registration/de-registration notifications	54

4.2.3.3	Failed S-CSCF registration/de-registration notifications	54

4.2.4	Network initiated de-registration by the HSS related measurements	56

4.2.4.1	Attempted network initiated de-registrations by HSS	56

4.2.4.2	Successful network initiated de-registrations by HSS	56

4.2.4.3	Failed network initiated de-registrations by HSS	56

4.2.5	User location query related measurements	58

4.2.5.1	Attempted user location queries	58

4.2.5.2	Successful user location queries	58

4.2.5.3	Failed user location queries	58

4.2.6	HSS initiated update of user profile related measurements	60

4.2.6.1	Attempted HSS initiated updates of user profile	60

4.2.6.2	Successful HSS initiated updates of user profile	60

4.2.6.3	Failed HSS initiated updates of user profile	60

4.2.7	Authentication procedure related measurements	62

4.2.7.1	Attempted authentication procedures	62

4.2.7.2	Successful authentication procedures	62

4.2.7.3	Failed authentication procedures	62

4.2.8	Data read (Sh-Pull) procedure related measurements	64

4.2.8.1	Attempted data read (Sh-Pull) procedures	64

4.2.8.2	Successful data read (Sh-Pull) procedures	64

4.2.8.3	Failed data read (Sh-Pull) procedures	64

4.2.9	Data update (Sh-Update) procedure related measurements	66

4.2.9.1	Attempted data update (Sh-Update) procedures	66

4.2.9.2	Successful data update (Sh-Update) procedures	66

4.2.9.3	Failed data update (Sh-Update) procedures	66

4.2.10	Subscription to notifications (Sh-Subs-Notif) procedure related measurements	68

4.2.10.1	Attempted subscription to notifications (Sh-Subs-Notif) procedures	68

4.2.10.2	Successful subscription to notifications (Sh-Subs-Notif) procedures	68

4.2.10.3	Failed subscription to notifications (Sh-Subs-Notif) procedures	68

4.2.11	Notification (Sh-Notif) procedure related measurements	70

4.2.11.1	Attempted notification (Sh-Notif) procedures	70

4.2.11.2	Successful notification (Sh-Notif) procedures	70

4.2.11.3	Failed notification (Sh-Notif) procedures	70

4.3	Measurements related to BGCF	72

4.3.1	Session Control related measurements	72

4.3.1.1	Overview	72

4.3.1.2	Attempted sessions at BGCF	72

4.3.1.3	Attempted sessions forwarded to MGCF	72

4.3.1.4	Attempted sessions forwarded to BGCF	73

4.3.1.6	Successful sessions forwarded to BGCF	74

4.4	Measurements related to MGCF	76

4.4.1	Call control related measurements	76

4.4.1.1	Call control related measurements, CS network originated	76

4.4.1.1.1	Overview	76

4.4.1.1.2	Attempted calls, CS network originated	77

4.4.1.1.3	Successful calls, CS network originated	77

4.4.1.1.4	Answered calls, CS network originated	77

4.4.1.1.5	Failed calls, CS network originated	77

4.4.1.1.6	Call set-up time (Mean), CS network originated	78

4.4.1.2	Call control related measurements, IM CN originated	78

4.4.1.2.1	Overview	78

4.4.1.2.2	Attempted calls, IM CN originated	79

4.4.1.2.3	Successful calls, IM CN originated	79

4.4.1.2.4	Answered calls, IM CN originated	80

4.4.1.2.5	Failed calls, IM CN originated	80

4.4.1.2.6	Call set-up time (Mean), IM CN originated	80

4.4.1.3	Call control related measurements, call release	81

4.4.1.3.1	Overview	81

4.4.1.3.2	Call release initiated by CS network	81

4.4.1.3.3	Call release initiated by IM CN subsystem	81

4.4.1.3.4	Call release initiated by MGCF	82

4.4.1.3.5	Call release initiated by MGW	83

4.4.1.4	Number of simultaneous online and answered calls (Maximum)	83

4.4.1.5	Number of simultaneous online and answered calls (mean)	83

4.5	Measurements related to IM-MGW	84

4.6	Measurements related to MRFC	84

4.6.1	Session control related measurements	84

4.6.1.1	Attempted session establishments	84

4.6.1.2	Successful session establishments	84

4.6.1.3	Answered session establishments	85

4.6.1.4	Failed session establishments	85

4.6.2	Multi-party sessions related measurements	85

4.6.2.1	Attempted session establishments for multi-party sessions	85

4.6.2.2	Successful session establishments for multi-party sessions	86

4.6.2.3	Answered session establishments for multi-party sessions	86

4.6.2.4	Failed session establishments for multi-party sessions	86

4.6.2.5	Attempted event subscription procedures for multi-party sessions	87

4.6.2.6	Successful event subscription procedures for multi-party sessions	87

4.6.2.7	Failed event subscription procedures for multi-party sessions	88

4.6.3	Announcement service related measurements	88

4.6.3.1	Attempted session establishments for announcements	88

4.6.3.2	Successful session establishments for announcements	88

4.6.3.3	Answered session establishments for announcements	89

4.6.3.4	Failed session establishments for announcements	89

4.6.4	Transcoding service related measurements	90

4.6.4.1	Attempted session establishments for transcoding service	90

4.6.4.2	Successful session establishments for transcoding service	90

4.6.4.3	Answered session establishments for transcoding service	91

4.6.4.4	Failed session establishments for transcoding service	91

4.7	Measurements related to MRFP	92

4.7.1	RTP related measurements	92

4.7.1.1	Number of incoming RTP packets	92

4.7.1.2	Number of outgoing RTP packets	92

4.7.1.3	Number of octets of incoming RTP packets	92

4.7.1.4	Number of octets of outgoing RTP packets	93

4.7.1.5	Number of lost incoming RTP packets	93

4.8	Measurements related to PCRF	94

4.8.1	Authorization of QoS resources related measurements	94

4.8.1.1	Overview	94

4.8.1.2	Attempted resource authorization procedures at session establishment	95

4.8.1.3	Attempted resource authorization procedures at session modification	95

4.8.1.4	Successful resource authorization procedures at session establishment	95

4.8.1.5	Successful resource authorization procedures at session modification	96

4.8.1.6	Failed resource authorization procedures	96

4.8.2	Resource reservation related measurements	97

4.8.2.1	Void	97

4.8.2.2	Attempted resource reservation procedures	97

4.8.2.3	Successful resource reservation procedures	97

4.8.2.4	Failed resource reservation procedures by PDF	97

4.8.2.5	Failed resource reservation procedures by GGSN	98

4.8.3	IP-CAN session modification related measurements	99

4.8.3.1	Attempted IP-CAN session modification	99

4.8.3.2	Successful IP-CAN session modification	99

4.8.3.3	Failed IP-CAN session modification	99

4.8.3	PDP context modification related measurements	101

4.8.3.1	Attempted authorization of PDP context modification	101

4.8.3.2	Successful authorization of PDP context modification	101

4.8.3.3	Failed authorization of PDP context modification	101

4.8.3.4	Indication of PDP context modification	102

4.8.5	Gateway Control session establishment related measurements	102

4.8.5.1	Attempted Gateway Control session establishment	102

4.8.5.2	Successful Gateway Control session establishment	102

4.8.5.3	Failed Gateway Control session establishment	103

4.9	Measurements related to SLF	104

4.9.1	All routing information interrogations related measurements	104

4.9.1.1	Attempted routing information interrogation procedures	104

4.9.1.2	Successful routing information interrogation procedures	104

4.9.2	Routing information interrogation from CSCF related measurements	105

4.9.2.1	Overview	105

4.9.2.2	Attempted routing information interrogation procedures from CSCF	105

4.9.2.3	Successful routing information interrogation procedures from CSCF	106

4.9.3	Routing information interrogation from AS related measurements	107

4.9.3.1	Overview	107

4.9.3.2	Attempted routing information interrogation procedures from AS	107

4.9.3.3	Successful routing information interrogation procedures from AS	107

4.10	Measurements related to AS	109

4.10.1	Session control related measurements	109

4.10.1.1	IM CN originated session establishments	109

4.10.1.1.1	Attempted IM CN originated session establishments	109

4.10.1.1.2	Successful IM CN originated session establishments	109

4.10.1.1.3	Answered IM CN originated session establishments	109

4.10.1.1.4	Failed IM CN originated session establishments	110

4.10.1.2	AS originated session establishments	110

4.10.1.2.1	Attempted AS originated session establishments	110

4.10.1.2.2	Successful AS originated session establishments	111

4.10.2	Messaging service related measurements	111

4.10.2.1	Message related measurements, Page-Mode	111

4.10.2.1.1	Attempted incoming messages, Page-Mode	111

4.10.2.1.2	Successful incoming messages, Page-Mode	111

4.10.2.1.3	Attempted outgoing messages, Page-Mode	112

4.10.2.1.4	Successful outgoing messages, Page-Mode	112

4.10.2.2	Message related measurements, Session-Mode	112

4.10.2.2.1	Overview	112

4.10.2.2.2	Attempted messaging session establishments, Session Mode	113

4.10.2.2.3	Successful messaging session establishments, Session-Mode	114

4.10.3	Conference service related measurements	114

4.10.3.1	Overview	114

4.10.3.2	Attempted creations of conference	122

4.10.3.3	Successful creations of conference	122

4.10.3.4	Attempted joining of conference	123

4.10.3.5	Successful joining of conference	123

4.10.3.6	Attempted invitations for another user to join conference	123

4.10.3.7	Successful invitations for another user to join conference	124

4.10.3.8	Attempted subscriptions for the conference event	124

4.10.3.9	Successful subscriptions for the conference event	124

4.10.3.10	Number of simultaneous online users (Maximum)	125

4.10.3.11	Number of simultaneous online conferences (Maximum)	125

4.10.3.12	Number of simultaneous online users (Mean)	125

4.10.3.13	Number of simultaneous online conferences (Mean)	125

4.10.4	Presence service related measurements	127

4.10.4.1	Number of simultaneous online watchers (Maximum)	127

4.10.4.2	Number of simultaneous online presentities (Maximum)	127

4.10.4.3	Number of simultaneous online watchers (Mean)	127

4.10.4.4	Number of simultaneous online presentities (Mean)	128

4.10.4.5	Attempted subscriptions for presence	128

4.10.4.6	Successful subscriptions for presence	128

4.10.4.7	Attempted notifications for presence	129

4.10.4.8	Successful notifications for presence	129

4.10.4.9	Attempted publications for presence	129

4.10.4.10	Successful publications for presence	129

4.10.5	PoC service related measurements	131

4.10.5.1	Attempted creations of PoC session	131

4.10.5.2	Successful creations of PoC session	131

4.10.5.3	Attempted joining of PoC session	131

4.10.5.4	Successful joining of PoC session	131

4.10.5.5	Attempted invitations for another user to join PoC session	132

4.10.5.6	Successful invitations for another user to join PoC session	132

4.10.5.7	Number of simultaneous online users (Maximum)	132

4.10.5.8	Number of simultaneous online PoC session (Maximum)	133

4.10.6	Measurements related to XDM Enabler	134

4.10.6.1	XCAP operation related measurements	134

4.10.6.1.1	Overview	134

4.10.6.1.2	Attempted HTTP PUT procedures	134

4.10.6.1.3	Successful HTTP PUT procedures	135

4.10.6.1.4	Attempted HTTP GET procedures	135

4.10.6.1.5	Successful HTTP GET procedures	135

4.10.6.1.6	Attempted HTTP DELETE procedures	136

4.10.6.1.7	Successful HTTP DELETE procedures	136

4.10.6.2	SIP related measurements	136

4.10.6.2.1	Overview	136

4.10.6.2.2	Attempted Subscription procedures	137

4.10.6.2.3	Successful Subscription procedures	137

4.10.6.2.4	Attempted Notification procedures	138

4.10.6.2.5	Successful Notification procedures	138

4.10.7	IMS supplementary services related measurements	138

4.10.7.1  Communication Diversion (CDIV) service establishments	138

4.10.7.1.1  Attempted Communication Diversion (CDIV) service procedure	138

4.10.7.1.2  Successful Communication Diversion (CDIV) service procedure	139

4.10.7.2	Originating Identification Presentation (OIP) service establishments	139

4.10.7.2.1	Attempted Originating Identification Presentation (OIP) procedure	139

4.10.7.2.2	Successful Originating Identification Presentation (OIP) procedure	139

4.11	Measurements related to IBCF	141

4.11.1	Attempted session establishments from users of other network domains	141

4.11.2	403 (Forbidden) generated for sessions from users of other network domains	141

4.11.3	Attempted session establishments to users of other network domains	142

4.11.4	403 (Forbidden) received for sessions to users of other network domains	142

5	Equipment related measurements	143

5.1	Processor usage	143

5.1.1	Mean processor usage	143

5.1.2	Peak processor usage	143

Annex A (normative):	 Use cases for performance measurements defintion	144

A.1	Use case of Immediate Messaging procedure related measurements at CSCF	144

A.2	Use case of IP-CAN Session Establishment related measurements	144

A.3	Use case of emergency session related measurements	144

A.4	Use case of CDIV service related measurements	144

A.5	Use case of OIP and OIR service related measurements	144

A.6	Use case of successful session establishments of inter-network domains	145

A.7	Use case of Gateway Control Session Establishment related measurements	145

Annex B (informative):	Void	146

Annex C (informative):	Use case of successful session establishment mean time	147

Annex D (informative):	 Use case of Number of 200 OK (Successful) messages for the roamers	148

Annex E (informative):	Change history	149

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The present document is part of a TS-family covering the 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Telecommunication Management; as identified below:

TS 32.401:	Performance Management (PM); Concept and requirements

TS 52.402:	Performance Management (PM); Performance measurements - GSM

TS 32.404:	Performance Management (PM); Performance measurements - Definitions and template

TS 32.405:	Performance Management (PM); Performance measurements UTRAN

TS 32.406:	Performance Management (PM); Performance measurements Core Network (CN) PS domain

TS 32.407:	Performance Management (PM); Performance measurements Core Network (CN) CS domain

TS 32.408:	Performance Management (PM); Performance measurements Teleservice

TS 32.409:	Performance Management (PM); Performance measurements IMS

The present document is part of a set of specifications, which describe the requirements and information model necessary for the standardised Operation, Administration and Maintenance (OA&M) of a multi-vendor 3G-system.

During the lifetime of a 3G network, its logical and physical configuration will undergo changes of varying degrees and frequencies in order to optimise the utilisation of the network resources. These changes will be executed through network configuration management activities and/or network engineering, see 3GPP TS 32.600 [3].

Many of the activities involved in the daily operation and future network planning of a 3G network require data on which to base decisions. This data refers to the load carried by the network and the grade of service offered. In order to produce this data performance measurements are executed in the NEs, which comprise the network. The data can then be transferred to an external system, e.g. an Operations System (OS) in TMN terminology, for further evaluation. The purpose of the present document is to describe the mechanisms involved in the collection of the data and the definition of the data itself.

# 1	Scope

The present document describes the measurements of IMS.

3GPP TS 32.401 [5] describes Performance Management concepts and requirements.

The present document is valid for all measurement types provided by an implementation of an IMS network. These may be measurement types defined within the present document, measurements defined within other standards bodies, or vendor specific measurement types.

Only measurement types that are specific to IMS network are defined within the present documents. I.e. vendor specific measurement types and measurements related to "external" technologies used in IMS network, such as ATM or IP, are not covered. Instead, these could be applied as described by the other, "external" standards bodies (e.g. ITU-T or IETF) or according to manufacturer's documentation.

The definition of the standard measurements is intended to result in comparability of measurement data produced in a multi-vendor network, for those measurement types that can be standardised across all vendors' implementations.

The structure of the present document is as follows:

-	Header 1: Network Element (e.g. measurements related to CSCF);

-	Header 2: Measurement function (e.g. Registration related measurements);

-	Header 3: Measurements.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication and/or edition number or version number) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.101: "Telecommunication management; Principles and high level requirements".

[2]	3GPP TS 32.102: "Telecommunication management; Architecture".

[3]	3GPP TS 32.600: "Telecommunication management; Configuration Management (CM); Concept and high-level requirements".

[4]	3GPP TS 32.632: "Telecommunication management; Configuration Management (CM); Core Network Resources Integration Reference Point (IRP): Network Resource Model (NRM)".

[5]	3GPP TS 32.401: "Telecommunication management; Performance Management (PM); Concept and requirements".

[6]	3GPP TS 23.003: "Numbering, addressing and identification".

[7]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[8]	3GPP TS 24.229: "Internet Protocol (IP) multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[9]	3GPP TS 29.228: "IP Multimedia (IM) Subsystem Cx and Dx Interfaces; Signalling flows and message".

[10]	3GPP TS 29.229: "Cx and Dx interfaces based on the Diameter protocol; Protocol details".

[11]	3GPP TS 29.163: "Interworking between the IP Multimedia (IM) Core Network (CN) subsystem and Circuit Switched (CS) networks".

[12]	3GPP TS 32.404: "Performance Management (PM); Performance measurements - Definitions and template".

[13]	3GPP TS 29.207: "Policy control over Go interface".

[14]	3GPP TS 29.208: "End-to-end Quality of Service (QoS) signalling flows".

[15]	IETF RFC 3588: "Diameter Base Protocol".

[16]	3GPP TS 23.008: "Organization of subscriber data".

[17]	3GPP TS 29.328: "IP Multimedia Subsystem (IMS) Sh interface; Signalling flows and message contents".

[18]	3GPP TS 29.329: "Sh interface based on the Diameter protocol; Protocol details".

[19]	ITU-T Recommendation Q.850: "Usage of cause and location in the Digital Subscriber Signalling System No. 1 and the Signalling System No. 7 ISDN User Part".

[20]	ITU-T Recommendation H.248.1: "Gateway control protocol: Version 3".

[21]	ITU-T Recommendation H.248.8: "Gateway control protocol: Error code and service change reason description".

[22]	IETF RFC 3265: "Session Initiation Protocol (SIP)-Specific Event Notification".

[23]	IETF RFC 3550: "RTP: A Transport Protocol for Real-Time Applications".

[24]	3GPP TS 24.147: "Conferencing using the IP Multimedia (IM) Core Network (CN) subsystem; Stage 3".

[25]	3GPP TS 23.141: "Presence service; Architecture and functional description; Stage 2".

[26]	3GPP TR 23.979: "3GPP enablers for Open Mobile Alliance (OMA); Push-to-talk over Cellular (PoC) services; Stage 2".

[27]	OMA-TS-XDM_Core: "XML Document Management Specification".

[28]	3GPP TS 33.203: "3G security; Access security for IP-based services".

[29]	3GPP TS 32.407: "Performance Management (PM); Performance measurements Core Network (CN) Circuit Switched (CS) domain".

[30]	3GPP TS 32.732: "Telecommunication management; IP Multimedia Subsystem (IMS) Network Resource Model (NRM) Integration Reference Point (IRP): Information Service (IS)".

[31]	3GPP TS 23.203: "Policy and charging control architecture".

[32]	3GPP TS 29.212: "Policy and charging control over Gx reference point".

[33]	3GPP TS 29.213: "Policy and charging control signalling flows and Quality of Service (QoS) parameter mapping".

[34]	IETF RFC 3261: "SIP: Session Initiation Protocol".

[35]	3GPP TS 23.167: "IP Multimedia Subsystem (IMS) emergency sessions".

[36]	3GPP TS 24.604: "Communication Diversion (CDIV) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".

[37]	3GPP TS 24.607: "Originating Identification Presentation (OIP) and Originating Identification Restriction (OIR) using IP Multimedia (IM) Core Network (CN) subsystem; Protocol specification".

# 3	Measurement family and abbreviations

## 3.1	Measurement family

The measurement names defined in the present document are all beginning with a prefix containing the measurement family name (e.g. UR.AttInitReg, SC.AttOrigSession). This family name identifies all measurements which relate to a given functionality and it may be used for measurement administration (see 3GPP TS 32.401 [5]).

The list of families currently used in the present document is as follows:

-	CC (measurements related to Call Control).

-	CONF (measurements related to conference service).

-	DBU (	measurements related to Database Usage).

-	DTR (measurements related to Data Read).

-	DTU (measurements related to Data Update).

-	EQPT (measurements related to Equipment).

-	IDS (measurements related to Inter-network Domain Session).

-	LIQ (measurements related to Location Information Query).

-	MA (measurements related Multimedia Authentication).

-	NOTIF (measurements related to Notification).

-	PoC (measurements related to PoC service).

-	PRES (measurements related to Present service).

-	QoS (measurements related to Quality of Service).

-	RII (measurements related to Routing Information Interrogation).

-	RU (measurements related to Roaming Users).

-	SC (measurements related to Session Control).

-	SUB (Measurements related to Subscription to notifications).

-	UP (measurements related to User Profile).

-	UR (measurements related to UE registration).

-	XDM (measurements related to XDM enabler).

## 3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply:

3G	3rd Generation

3GPP	3G Partnership Project

BGCF	Breakout Gateway Control Function

CS	Circuit switched

CN	Core Network

CSCF	Call Session Control Function

HSS	Home Subscriber Server

IM	IP Multimedia

IMS	IP Multimedia Subsystem

IBCF	Interconnection Border Control Function

I-CSCF	Interrogating CSCF

NE	Network Element

NM	Network Manager

OA&M	Operation, Administration and Maintenance

OS	Operations System (EM, NM)

PCC	Policy and Charging Control

PCEF	Policy and Charging Enforcement Function

PCRF	Policy and Charging Rule Function

PM	Performance Management

P-CSCF	Proxy CSCF

QoS	Quality of Service

S-CSCF	Serving CSCF

UMTS	Universal Mobile Telecommunications System

You can find below a list of abbreviations used within the measurement types for field E of the measurement template (see 3GPP TS 32.404 [12]).

Ans	Answer(ed)

Att	Attempted

Auth	Authorization

CCR	Credit-Control-Request

CCA	Credit-Control-Answer

Cs	Circuit switched

DER	Discrete Event Registration

DeReg	De-Registration

Dmn	Domain

Estab	Establish(ment)

Fail	Failed(/Failure)

Fwd	Forwared(ed)

Glb	Global

Info	Information

Im	IP Multimedia

Inc	Incoming

Ind	Indication

Init	Initial

Interop	Interoperator

LIA	Location-Info-Answer

LIR	Location-Info-Request

MAA	Multimedia-Authentication-Answer

MAR	Multimedia-Authentication-Request

Max	Maximum

Modf	Modify/Modification

Nbr	Number

Ntwk	Network

Orig	Originated

Outg	Outgoing

PM	Page-Mode

PNA	Push-Notification -Answer

PNR	Push-Notification-Request

PPA	Push-Profile-Answer

PPR	Push-Profile-Request

Pri	Private

PUA	Profile-Update-Answer

Pub	Public

PUR	Profile-Update-Request

Reg	Registration

Rej	Rejected

Req	Request

Rel	Release

ReReg	Re-Registration

Res	Resource

Rsv	Reserve/Reservation

Rmg	Roaming

RTA	Registration-Termination-Answer

RTR	Registration-Termination-Request

SAA	Server-Assignment-Answer

SAR	Server-Assignment-Request

SNA	Subscribe-Notifications-Answer

SNR	Subscribe-Notifications-Request

Simul	Simultaneous

SM	Session-Mode

Srv	Service

Succ	Successful

Term	Terminated

UAA	User-Authorization-Answer

UAR	User-Authorization-Request

UDA	User-Data-Answer

UDR	User-Data-Request

# 4	Functionality related measurements

The measurements defined in this clause are related to the functionality aspect performance. The detailed measurements for each function are defined in the following subclauses.

## 4.1	Measurements related to CSCF

Editor's note: with regards to each measurement defined for the CSCF, the functionality (e.g. P-CSCF or S-CSCF) which the measurent is applicable for is illustrated in bullet a) by the words "This measurement is only valid for the CSCF with *** functionality", if there is no such kind of illustration in bullet a), it means that this measurement is applicable for all of the P-CSCF, S-CSCF and I-CSCF functionalities.

### 4.1.1	Registration related measurements

#### 4.1.1.1	Initial registration

The measurements defined in subclauses 4.1.1.1.1, 4.1.1.1.2 and 4.1.1.1.3 are subject to the "2 out of 3 approach".

##### 4.1.1.1.1	Attempted initial registration procedures

- This measurement provides the number of attempted initial registrations at the CSCF. This measurement is only valid for the CSCF with P-CSCF and/or S-CSCF functionality.

- CC.

- On receipt by the CSCF of a SIP_REGISTER message for the initial registration (see 3GPP TS 23.228 [7]).

- A single integer value.

- UR.AttInitReg.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

##### 4.1.1.1.2	Successful initial registration procedures

- This measurement provides the number of successful initial registrations at the CSCF. This measurement is only valid for the CSCF with P-CSCF and/or S-CSCF functionality.

- CC.

- On transmission by the CSCF of a SIP_200_OK message for the initial registration (see 3GPP TS 23.228 [7]).

- A single integer value.

- UR.SuccInitReg.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

##### 4.1.1.1.3	Failed initial registration procedures

- This measurement provides the number of failed initial registration procedures at the CSCF. The measurement is split into subcounters per response status codes. This measurement is only valid for the CSCF with P-CSCF and/or S-CSCF functionality.

- CC.

- On transmission by the CSCF of a response message (e.g. SIP_4xx) indicating a failed initial registration, each failed initial registration procedure is added to the relevant measurement according to the status code. Possible status codes are included in 3GPP TS 29.229 [10]. The sum of all supported per status code measurements shall equal the total number of failed initial registration procedures. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of measured status codes plus a possible sum measurement identified by the .sum suffix.

- The measurement name has the form UR.FailInitReg.Cause
where Cause identifies the status code.

- CscfFunction

- Valid for packet switched traffic.

- IMS

##### 4.1.1.1.4	IMS initial registration set-up time (Mean)

- This measurement provides the mean setup time of the successful IMS initial registrations. This measurement is only valid for the CSCF with P-CSCF functionality.

- DER (n=1).

- This measurement is obtained by accumulating the time intervals for every successful IMS initial registration between the receipt by the P-CSCF from the UE of a SIP_REGISTER message and the corresponding SIP_200_OK message for the initial registration over a granularity period using DER. The end value of this time will then be divided by the number of successful IMS initial registrations observed in the granularity period to give the arithmetic mean, the accumulator shall be reinitialised at the beginning of each granularity period. See 3GPP TS 23.228 [7].

Figure 1: UE registration procedure

- A single integer value. (in milliseconds).

- UR.MeanInitRegSetupTime.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

- This measurement is mainly dedicated to Vendor Performance Modelling, Operator Traffic Engineering and Vendor Development Engineering communities.

##### 4.1.1.1.5	Attempted initial registration procedures per access technology

- This measurement provides the number of attempted initial registrations from the UE belonging to a given access technology. (Note: The P-Access-Network-Info header is extended to include specific information relating to particular access technologies. The UE shall populate the P-Access-Network-Info header, with the following access-type tokens: "3GPP-GERAN","3GPP-UTRAN-FDD", "3GPP-UTRAN-TDD", "3GPP2-1X", "IEEE-802.11a" or "IEEE-802.11b", etc., as appropriate to the access technology in use.). This measurement is only valid for the CSCF with P-CSCF functionality.

- CC.

- On receipt by the CSCF of a SIP_REGISTER message for the initial registration. Each Register message with different access-type token in the P-Access-Network-Info header for the initial registration is added to the relevant measurement according to access technology (see 3GPP TS 24.229 [8]).

- Each measurement is an integer value.

- The measurement name has the form UR.AttInitReg.AT
where AT identifies the access technology the UE is belonging to.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

##### 4.1.1.1.6	Successful initial registration procedures per access technology

- This measurement provides the number of successful initial registrations from the UE belonging to a given access technology. This measurement is only valid for the CSCF with P-CSCF functionality.

- CC.

- On transmission by the CSCF of a SIP_200_OK message for the initial registration, corresponding to the SIP_REGISTER message with different access-type token in the P-Access-Network-Info header for the initial registration, each successful initial registration is added to the relevant measurement according to access technology (see 3GPP TS 24.229 [8]).

- A single integer value.

- The measurement name has the form UR.SuccInitReg.AT
where AT identifies the access technology the UE is belonging to.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.1.1.2	Re-registration

The measurements defined in subclauses 4.1.1.2.1, 4.1.1.2.2 and 4.1.1.2.3 are subject to the "2 out of 3 approach".

##### 4.1.1.2.1	Attempted re-registration procedures

- This measurement provides the number of attempted re-registrations at the CSCF. This measurement is only valid for the CSCF with P-CSCF and/or S-CSCF functionality.

- CC.

- On receipt by the CSCF of a SIP_REGISTER message for the re-registration (see 3GPP TS 23.228 [7]).

- A single integer value.

- UR.AttReReg.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

##### 4.1.1.2.2	Successful re-registration procedures

- This measurement provides the number of successful re-registrations at the CSCF. This measurement is only valid for the CSCF with P-CSCF and/or S-CSCF functionality.

- CC.

- On transmission by the CSCF of a SIP_200_OK message for the re-registration (see 3GPP TS 23.228 [7]).

- A single integer value.

- UR.SuccReReg.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

##### 4.1.1.2.3	Failed re-registration procedures

- This measurement provides the number of failed re-registration procedures at the CSCF. The measurement is split into subcounters per response status codes. This measurement is only valid for the CSCF with P-CSCF and/or S-CSCF functionality.

- CC.

- On transmission by the CSCF of a response message(e.g. SIP_4xx) indicating a failed re-registration, each failed re-registration procedure is added to the relevant measurement according to the status code. Possible status codes are included in 3GPP TS 29.229 [10]. The sum of all supported per status code measurements shall equal the total number of failed re-registration procedures. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of measured per status code plus a possible sum measurement identified by the .sum suffix.

- The measurement name has the form UR.FailReReg.Cause
where Cause identifies the status code.

- CscfFunction.

- Valid for packet switched traffic.

- IMS

##### 4.1.1.2.4	Attempted re-registration procedures per access technology

- This measurement provides the number of attempted re-registrations from the UE belonging to a given access technology. (Note: The P-Access-Network-Info header is extended to include specific information relating to particular access technologies. The UE shall populate the P-Access-Network-Info header, with the following access-type tokens: "3GPP-GERAN","3GPP-UTRAN-FDD", "3GPP-UTRAN-TDD", "3GPP2-1X", "IEEE-802.11a" or "IEEE-802.11b", etc., as appropriate to the access technology in use.). This measurement is only valid for the CSCF with P-CSCF functionality.

- CC.

- On receipt by the CSCF of a SIP_REGISTER message for the re-registration. Each Register message with different access-type token in the P-Access-Network-Info header for the re-registration is added to the relevant measurement according to access technology (see 3GPP TS 24.229 [8]).

- Each measurement is an integer value.

- The measurement name has the form UR.AttReReg.AT
where AT identifies the access technology the UE is belonging to.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

##### 4.1.1.2.5	Successful re-registration procedures per access technology

- This measurement provides the number of successful re-registrations from the UE belonging to a given access technology. This measurement is only valid for the CSCF with P-CSCF functionality.

- CC.

- On transmission by the CSCF of a SIP_200_OK message for the re-registration, corresponding to the SIP_REGISTER message with different access-type token in the P-Access-Network-Info header for the re-registration, each successful re-registration is added to the relevant measurement according to access technology (see 3GPP TS 24.229 [8]).

- A single integer value.

- The measurement name has the form UR.SuccReReg.AT
where AT identifies the access technology the UE is belonging to.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.1.1.3	De-registration

##### 4.1.1.3.1	De-registrations initiated by UE

The measurements defined in subclauses 4.1.1.3.1.1, 4.1.1.3.1.2 and 4.1.1.3.1.3 are subject to the "2 out of 3 approach".

4.1.1.3.1.1	Attempted de-registration procedures initiated by UE

- This measurement provides the number of attempted de-registrations initiated by UE at the CSCF. 
This measurement is only valid for the CSCF with P-CSCF and/or S-CSCF functionality.

- CC.

- On receipt by the CSCF of a SIP_REGISTER message for the de-registration (see 3GPP TS 23.228 [7]).

- A single integer value.

- UR.AttDeRegUe.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

4.1.1.3.1.2	Successful de-registration procedures initiated by UE

- This measurement provides the number of successful de-registrations initiated by UE at the CSCF. 
This measurement is only valid for the CSCF with P-CSCF and/or S-CSCF functionality.

- CC.

- On transmission by the CSCF of a SIP_200_OK message corresponding to SIP_REGISTER message for the de-registration (see 3GPP TS 23.228 [7]).

- A single integer value.

- UR.SuccDeRegUe.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

4.1.1.3.1.3	Failed de-registration procedures initiated by UE

- This measurement provides the number of failed de-registration procedures initiated by UE at the CSCF. The measurement is split into subcounters per response status codes. 
This measurement is only valid for the CSCF with P-CSCF and/or S-CSCF functionality.

- CC.

- On transmission by the CSCF of a response message (e.g. SIP_4xx) indicating a failed de-registration procedure initiated by UE, each failed de-registration procedure initiated by UE is added to the relevant measurement according to the status code. Possible status codes are included in 3GPP TS 29.229 [10]. The sum of all supported per status code measurements shall equal the total number of failed de-registration procedures initiated by UE. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of measured status codes plus a possible sum measurement identified by the .sum suffix.

- The measurement name has the form UR.FailDeRegUe.Cause
where Cause identifies the status code.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

###### 4.1.1.3.1.4	Attempted de-registrations initiated by UE per access technology

- This measurement provides the number of attempted de-registrations from the UE belonging to a given access technology. (Note: The P-Access-Network-Info header is extended to include specific information relating to particular access technologies. The UE shall populate the P-Access-Network-Info header, with the following access-type tokens: "3GPP-GERAN","3GPP-UTRAN-FDD", "3GPP-UTRAN-TDD", "3GPP2-1X", "IEEE-802.11a" or "IEEE-802.11b", etc., as appropriate to the access technology in use.). This measurement is only valid for the CSCF with P-CSCF functionality.

- CC.

- On receipt by the CSCF of a SIP_REGISTER message for the de-registration. Each Register message with different access-type token in the P-Access-Network-Info header for the de-registration is added to the relevant measurement according to access technology (see 3GPP TS 24.229 [8]).

- Each measurement is an integer value.

- The measurement name has the form UR.AttDeRegUe.AT
where AT identifies the access technology the UE is belonging to.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

###### 4.1.1.3.1.5	Successful de-registrations initiated by UE per access technology

- This measurement provides the number of successful de-registrations from the UE belonging to a given access technology. This measurement is only valid for the CSCF with P-CSCF functionality.

- CC.

- On transmission by the CSCF of a SIP_200_OK message for the de-registration, corresponding to the SIP_REGISTER message with different access-type token in the P-Access-Network-Info header for the de-registration, each successful de-registration is added to the relevant measurement according to access technology (see 3GPP TS 24.229 [8]).

- A single integer value.

- The measurement name has the form UR.SuccDeRegUe.AT
where AT identifies the access technology the UE is belonging to.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

##### 4.1.1.3.2	De-registration initiated by HSS

The measurements defined in this subclause are subject to the "2 out of 3 approach".

4.1.1.3.2.1	Attempted de-registration procedures initiated by HSS

- This measurement provides the number of attempted de-registrations initiated by HSS at the CSCF. This measurement is only valid for the CSCF with S-CSCF functionality.

- CC.

- On receipt by the S-CSCF of a Cx-Deregister message for the de-registration initiated by HSS (see 3GPP TS 29.228 [9]).

- A single integer value.

- UR.AttDeRegHss.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

4.1.1.3.2.2	Successful de-registration procedures initiated by HSS

- This measurement provides the number of successful de-registrations initiated by HSS at the CSCF. This measurement is only valid for the CSCF with S-CSCF functionality.

- CC.

- On transmission by the S-CSCF of a "Cx-Deregister Resp" message indicating a successful de-registration initiated by HSS (see 3GPP TS 29.228 [9]).

- A single integer value.

- UR.SuccDeRegHss.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

4.1.1.3.2.3	Failed de-registration procedures initiated by HSS

- This measurement provides the number of failed de-registration procedures initiated by HSS at the CSCF. The measurement is split into subcounters per diameter result codes. This measurement is only valid for the CSCF with S-CSCF functionality.

- CC.

- On transmission by the CSCF of a response message(e.g. SIP_4xx) indicating a failed de-registration procedure initiated by HSS, each failed de-registration procedure initiated by HSS is added to the relevant measurement according to the status code. Possible status codes are included in 3GPP TS 29.229 [10]. The sum of all supported per status code measurements shall equal the total number of failed de-registration procedures initiated by HSS. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of measured result code plus a possible sum measurement identified by the .sum suffix.

- The measurement name has the form UR.FailDeRegHss.Cause
where Cause identifies the Diameter result code.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

##### 4.1.1.3.3	De-registration initiated by service platform

The measurements defined in this subclause are subject to the "2 out of 3 approach".

4.1.1.3.3.1	Attempted de-registration procedures initiated by service platform

- This measurement provides the number attempted of de-registrations initiated by service platform at the CSCF. This measurement is only valid for the CSCF with S-CSCF functionality.

- CC.

- On transmission by the S-CSCF of a SIP_NOTIFY message indicating a de-registration initiated by service platform (see 3GPP TS 24.229 [8]).

- A single integer value.

- UR.AttDeRegSrvPlatform.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

4.1.1.3.3.2	Successful de-registration procedures initiated by service platform

- This measurement provides the number of successful de-registrations initiated by service platform at the CSCF. This measurement is only valid for the CSCF with S-CSCF functionality.

- CC.

- On receipt by the S-CSCF of a SIP_200_OK message corresponding to the SIP_NOTIFY message indicating a de-registration initiated by service platform (see 3GPP TS 24.229 [8]).

- A single integer value.

- UR.SuccDeRegSrvPlatform.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

4.1.1.3.3.3	Failed de-registration procedures initiated by service platform

- This measurement provides the number of failed de-registration procedures initiated by service platform at the CSCF. The measurement is split into subcounters per response status codes. This measurement is only valid for the CSCF with S-CSCF functionality.

- CC.

- On receipt by the CSCF of a response message(e.g. SIP_4xx) indicating a failed de-registration procedure initiated by service platform, each failed de-registration procedure initiated by service platform is added to the relevant measurement according to the status code. Possible status codes are included in 3GPP TS 29.229 [10]. The sum of all supported per status code measurements shall equal the total number of failed de-registration procedures initiated by service platform. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of the measured status codes plus a possible sum measurement identified by the .sum suffix.

- The measurement name has the form UR.FailDeRegSrvPlatform.Cause
where Cause identifies the status code.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.1.1.4	Third party registration

The measurements defined in this subclause are subject to the "2 out of 3 approach".

##### 4.1.1.4.1	Attempted third party registration procedures

- This measurement provides the number of attempted third-party registrations triggered by CSCF. This measurement is only valid for the CSCF with S-CSCF functionality.

- CC.

- On transmission by the S-CSCF of a SIP_REGISTER message for the third party registration (see 3GPP TS 24.229 [8]).

- A single integer value.

- UR.Att3rdPartyReg.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

##### 4.1.1.4.2	Successful third party registration procedures

- This measurement provides the number of successful third-party registrations triggered by CSCF. This measurement is only valid for the CSCF with S-CSCF functionality.

- CC.

- On receipt by the S-CSCF of a SIP_200_OK message for the third party registration (see 3GPP TS 24.229 [8]).

- A single integer value.

- UR.Succ3rdPartyReg.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

##### 4.1.1.4.3	Failed third party registration procedures

- This measurement provides the number of failed third-party registrations triggered by CSCF.

- CC.

- On receipt by the CSCF of a response message(e.g. SIP_4xx) indicating a failed third party registration procedure, each failed third party registration procedure is added to the relevant measurement according to the status code. Possible status codes are included in 3GPP TS 29.229 [10]. The sum of all supported per status code measurements shall equal the total number of failed third party registration procedures. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of the measured status codes plus a possible sum measurement identified by the .sum suffix.

- A single integer value. The number of measurements is equal to the number of the measured status codes plus a possible sum measurement identified by the .sum suffix.

- The measurement name has the form UR.Fail3rdPartyReg.Cause.
where Cause identifies the status code.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.1.1.5	User registration status query

The measurements defined in this subclause are subject to the "2 out of 3 approach".

##### 4.1.1.5.1	Attempted user registration status query procedures

- This measurement provides number of attempted User registration status query procedures at the CSCF. This measurement is only valid for the CSCF with I-CSCF functionality.

- CC.

- On transmission by the I-CSCF of a User-Authorization-Request Diameter command to the HSS (see 3GPP TS 29.229 [9]).

- A single integer value.

-	UR.AttUAR.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

##### 4.1.1.5.2	Successful user registration status query procedures

- This measurement provides number of successful user registration status query procedures at the CSCF. This measurement is only valid for the CSCF with I-CSCF functionality.

- CC.

- On receipt by the I-CSCF of a User-Authorization-Answer Diameter command from HSS indicating that the User Authorization Request is successful with the specific result code, each successful user registration status query procedure is added to the relevant measurement according to the successful result code. Possible successful result codes are included in 3GPP TS 29.229 [10]. The sum of all supported per result code measurements shall equal the total number of successful User registration status query procedures. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of result codes plus a possible sum value identified by the .sum suffix.

- The measurement name has the form UR.SuccUAA.ResultCode
where ResultCode identifies the successful Result Code.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

##### 4.1.1.5.3	Failed user registration status query procedures

- This measurement provides number of user registration status query procedures rejected by HSS for the specific causes. The measurement is split into subcounters per failure cause. This measurement is only valid for the CSCF with I-CSCF functionality.

- CC.

- On receipt by the I-CSCF of a User-Authorization-Answer Diameter command from HSS indicating that the User Authorization Request is rejected for the specific cause, each failed user registration status query procedure is added to the relevant measurement according to the failure cause. Possible failure causes are included in 3GPP TS 29.229 [10]. The sum of all supported per cause measurements shall equal the total number of failed User registration status query procedures. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes plus a possible sum value identified by the .sum suffix.

- The measurement name has the form UR.FailUAA.Cause
where Cause identifies the failure cause. The following subcounters with specific failure causes (see 3GPP TS 29.229 [10]) are required:

-	UR.NbrFailUAA.5002	Result-Code: DIAMETER_ERROR_IDENTITIES_DONT_MATCH;

-	UR.NbrFailUAA.5004	Result-Code: DIAMETER_ERROR_ROAMING_NOT_ALLOWED;

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.1.1.6	S-CSCF registration/deregistration notification

The measurements defined in this subclause are subject to the "2 out of 3 approach".

##### 4.1.1.6.1	Attempted S-CSCF registration/deregistration notification procedures

- This measurement provides number of attempted S-CSCF registration/deregistration notification procedures. This measurement is only valid for the CSCF with S-CSCF functionality.

- CC.

- On transmission by the S-CSCF of a Server-Assignment-Request Diameter command to the HSS (see 3GPP TS 29.228 [9]).

- A single integer value.

- UR.AttSAR.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

##### 4.1.1.6.2	Successful S-CSCF registration/deregistration notification procedures

- This measurement provides number of successful S-CSCF registration/deregistration notification procedures. This measurement is only valid for the CSCF with S-CSCF functionality.

- CC.

- On receipt by the S-CSCF of a Server-Assignment-Answer Diameter command from the HSS indicating a successful S-CSCF registration/deregistration notification procedure (see 3GPP TS 29.228 [9]).

- A single integer value.

- UR.SuccSAA.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

##### 4.1.1.6.3	Failed S-CSCF registration/deregistration notification procedures

- This measurement provides number of failed S-CSCF registration/deregistration notification procedures. The measurement is split into subcounters per failure cause. This measurement is only valid for the CSCF with S-CSCF functionality.

- CC.

- On receipt by the S-CSCF of a Server-Assignment-Answer Diameter command from HSS indicating the S-CSCF registration/deregistration notification procedure is failed for the specific cause, each failed S-CSCF registration/deregistration notification procedure is added to the relevant measurement according to the failure cause. Possible failure causes are included in TS 29.229 [10]. The sum of all supported per cause measurements shall equal the total number of failed S-CSCF registration/deregistration notification procedures. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes plus a possible sum value identified by the .sum suffix.

- The measurement name has the form UR.FailSAA.Cause
where Cause identifies the failure cause.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

### 4.1.2	Session control related measurements

The measurements defined in subclauses 4.1.2.1, 4.1.2.2 and 4.1.2.4 are subject to the "2 out of 3 approach".

#### 4.1.2.1	Attempted session establishments

- This measurement provides the number of attempted session establishments at CSCF.

- CC.

- On receipt by the CSCF of an initial SIP_INVITE message (see 3GPP TS 23.228 [7]).

- A single integer value.

- SC.AttSession.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.1.2.2	Successful session establishments

- This measurement provides the number of successful session establishments at the CSCF. The measurement is split into subcounters per message type (e.g. SIP_180) indicating the successful session establishments.

- CC.

- On transmission by the CSCF of the first response message (e.g. SIP_180) indicating a successful session establishment corresponding to the initial SIP_INVITE message (see 3GPP TS 23.228 [7]), each successful session establishment is added to the relevant measurement according to the message type, the possible response message types are identified in 3GPP TS 24.229 [8]. The sum of all supported per message type measurements shall equal the total number of successful session establishments. In case only a subset of per message type measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of message types plus a possible sum value identified by the .sum suffix.

- The measurement name has the form SC.SuccSession.type
where type identifies the message type.

The following subcounters with specific message types are the samples of this measurement:

-	SC.SuccSession.180	Message type: SIP_180;

-	SC.SuccSession.200	Message type: SIP_200_OK only if the SIP_180 has not been sent for this 	session.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.1.2.3	Answered session establishments

- This measurement provides the number of answered session establishments at the CSCF.

- CC.

- On transmission by the CSCF of a SIP_200_OK response message corresponding to the initial SIP_INVITE messages (see 3GPP TS 23.228 [7]).

- A single integer value.

- SC.AnsSession.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.1.2.4	Failed session establishments

- This measurement provides the number of failed session establishments at the CSCF. The measurement is split into subcounters per response status codes.

- CC.

- On transmission by the CSCF of a response message (e.g. SIP_430) indicating a failed session establishment corresponding to the initial SIP_INVITE messages (see 3GPP TS 23.228 [7]), each failed session establishment is added to the relevant measurement according to the status code. The sum of all supported per status code measurements shall equal the total number of failed session establishments. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes plus a possible sum value identified by the .sum suffix.

- The measurement name has the form SC.FailSession.Cause
where Cause identifies the status code.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.1.2.5	Simultaneous online and answered sessions (Maximum)

- This measurement provides the maximum number of simultaneous online sessions which have been answered at the CSCF.

- SI.

- This measurement is obtained by sampling at a pre-defined interval the number of simultaneous online sessions which have been answered and then taking the maximum.

- A single integer value.

- SC.NbrSimulAnsSessionMax.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.1.2.6	Successful session establishment time (Mean)

- This measurement provides the mean establishment time of the successful sessions at the CSCF. The measurement is split into subcounters per meadia type. The measurement is only valid for P-CSCF and S-CSCF.

- DER (n=1).

- This measurement is obtained by accumulating the time intervals for every successful session between the receipt of an initial SIP_INVITE message by the CSCF (see 3GPP TS 23.228 [7]) and the transmission of the first corresponding response message (which may be SIP_180 or SIP_2XX) indicating a successful session establishment over a granularity period. The end value of this time will then be divided by the number of successful session establishements observed in the granularity period to give the arithmetic mean, the accumulator shall be reinitialised at the beginning of each granularity period. The measurement is split into subcounters per meadia type, and the possible media type can be seen in TS 24.229[8].

- A single integer value(in milliseconds)

- SC.SessionEstabTimeMean.MediaName

- CscfFunction.

- Valid for packet switched traffic.

- IMS

#### 4.1.2.7	Simultaneous online and answered sessions (Mean)

- This measurement provides the mean number of simultaneous online and answered sessions at the P-CSCF and S-CSCF.

- SI.

- This measurement is obtained by sampling at a pre-defined interval the number of simultaneous online and answered sessions which are in the sate between receipt by the P-CSCF of SIP_200_OK message and receipt by the P-CSCF of message SIP_BYE, or between receipt by the S-CSCF of message SIP_200_OK and receipt or transmission by the S-CSCF of the message SIP_BYE, and then taking the arithmetic mean.

- A single integer value.

- SC.NbrSimulAnsSessionMean

- PCSCFFunction
SCSCFFunction

- Valid for packet switched traffic.

- IMS.

### 4.1.3	User location query related measurements

The measurements defined in this subclause are subject to the "2 out of 3 approach".

#### 4.1.3.1	Attempted user location query procedures

- This measurement provides number of attempted user location query procedures. This measurement is only valid for the CSCF with I-CSCF functionality.

- CC.

- On transmission by the I-CSCF of a Location-Info-Request Diameter command to the HSS (see 3GPP TS 29.228 [9]).

- A single integer value.

- LIQ.AttLIR.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.1.3.2	Successful user location query procedures

- This measurement provides number of successful user location query procedures. This measurement is only valid for the CSCF with I-CSCF functionality.

- CC.

- On receipt by the I-CSCF of a Location-Info-Answer Diameter command from the HSS indicating a successful location information query procedure (see 3GPP TS 29.228 [9]).

- A single integer value.

- LIQ.SuccLIA.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.1.3.3	Failed user location query procedures

- This measurement provides number of failed user location query procedures. The measurement is split into subcounters per failure cause. This measurement is only valid for the CSCF with I-CSCF functionality.

- CC.

- On receipt by the I-CSCF of a Location-Info-Answer Diameter command from HSS indicating the user location query is failed for the specific cause, each failed user location query procedure is added to the relevant measurement according to the failure cause. Possible failure causes are included in 3GPP TS 29.229 [10]. The sum of all supported per cause measurements shall equal the total number of failed user location query procedures. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes plus a possible sum value identified by the .sum suffix.

- The measurement name has the form LIQ.FailLIA.Cause
where Cause identifies the failure cause.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

### 4.1.4	Inter-network domain calling related measurements

#### 4.1.4.1	Attempted session establishments from users of other network domains

- This measurement provides the number of attempted session establishments originated from the users (regardless of whether the user is roaming or not) who are belonging to other network domains at the CSCF in the terminating network. The measurement is split into subcounters per network domain. This measurement is only valid for the CSCF with S-CSCF and/or I-CSCF functionality.

- CC.

- On receipt by the CSCF of a SIP_ INVITE message, and the calling party is belonging to other network domains (i.e. the domain name information of the calling party retrieved from the SIP_INVITE message is different from the domain of this CSCF). Each session whose calling party recognized by the CSCF as belonging to a given network domain is added to the relevant per network domain measurement. The sum of all supported per network domain measurements shall equal the total number of attempted session establishments from users of other network domains. In case only a subset of per network measurements is supported, a sum subcounter will be provided first (see 3GPP TS 23.228 [7]).

- Each measurement is an integer value. The number of measurements is equal to the number of network domain plus a possible sum value identified by the .sum suffix.

- The measurement name has the form IC.AttSessionFromOtherNtwkDmn.Domain
where Domain identifies the network domain, and it is in the format of the domain name information in the SIP messages.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.1.4.2	403 (Forbidden) generated for sessions from users of other network domains

- This measurement provides the number of SIP_403 (Forbidden) response messages at the CSCF in the terminating network generated for the sessions from users of other network domains. The measurement is split into subcounters per network domain. This measurement is only valid for the CSCF with S-CSCF and/or I-CSCF functionality.

- CC.

- On transmission by the CSCF of SIP_403 message corresponding to the SIP_ INVITE message of which the calling party is belonging to other network domains received by the CSCF. Each SIP_403 message generated by the CSCF for the session whose calling party is belonging to a given network domain is added to the relevant per network domain measurement. The sum of all supported per network domain measurements shall equal the total number of 403 (Forbidden) generated for sessions from users of other network domains. In case only a subset of per network measurements is supported, a sum subcounter will be provided first (see 3GPP TS 23.228 [7]).

- Each measurement is an integer value. The number of measurements is equal to the number of network domain plus a possible sum value identified by the .sum suffix.

- The measurement name has the form IC.403SessionFromOtherNtwkDmn.Domain
where Domain identifies the network domain, and it is in the format of the domain name information in the SIP messages.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.1.4.3	Attempted session establishments to users of other network domains

- This measurement provides the number of attempted session establishments towards users (regardless of whether the user is roaming or not) who are belonging to other network domains at the CSCF in the originating network. The measurement is split into subcounters per network domain. This measurement is only valid for the CSCF with S-CSCF and/or I-CSCF functionality.

- CC.

- On transmission by the CSCF of a SIP_ INVITE messages, and the called party is belonging to other network domains (i.e. the domain name information of the called party retrieved from the SIP_INVITE message is different from the domain of this CSCF). Each session whose called party recognized by the CSCF as belonging to a given network domain is added to the relevant per network domain measurement. The sum of all supported per network measurements shall equal the total number of attempted session establishments to users of other network domains. In case only a subset of per network measurements is supported, a sum subcounter will be provided first (see 3GPP TS 23.228 [7]).

- Each measurement is an integer value. The number of measurements is equal to the number of network domain plus a possible sum value identified by the .sum suffix.

- The measurement name has the form IC.AttSessionToOtherNtwkDmn.Domain
where Domain identifies the network domain, and it is in the format of the domain name information in the SIP messages.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.1.4.4	403 (Forbidden) received for sessions to users of other network domains

- This measurement provides the number of SIP_403 (Forbidden) response messages received for sessions to users of other network domains at the CSCF in the originating network. The measurement is split into subcounters per network domain. This measurement is only valid for the CSCF with S-CSCF and/or I-CSCF functionality.

- CC.

- On receipt by the CSCF of SIP_403 message corresponding to the SIP_ INVITE message corresponding to the SIP_ INVITE message of which the called party is belonging to other network domains transmitted by the CSCF. Each SIP_403 message received by the CSCF for the session whose called party is belonging to a given network domain is added to the relevant per network domain measurement. The sum of all supported per network domain measurements shall equal the total number of 403 (Forbidden) received for sessions to users of other network domains. In case only a subset of per network measurements is supported, a sum subcounter will be provided first (see 3GPP TS 23.228 [7]).

- Each measurement is an integer value. The number of measurements is equal to the number of network domain plus a possible sum value identified by the .sum suffix.

- The measurement name has the form IC.403SessionToOtherNtwkDmn.Domain
where Domain identifies the network domain, and it is in the format of the domain name information in the SIP messages.

- CscfFunction

- Valid for packet switched traffic.

- IMS.

#### 4.1.4.5	Successful session establishments from users of other network domains

- This measurement provides the number of  sucessful session establishments from the users (regardless of whether the user is roaming or not) who are belonging to other network domains at the CSCF in the originating network. The measurement is split into subcounters per network domain. This measurement is only valid for the CSCF with P-CSCF and/or S-CSCF functionality.

- CC.

- On transmission by the CSCF of the first response message (which may be SIP_180 or SIP_2XX) indicating a successful session establishment corresponding to the initial SIP_INVITE message (see 3GPP TS 23.228 [7]), and the called party is belonging to other network domains. Each session whose called party recognized by the CSCF as belonging to a given network domain is added to the relevant per network domain measurement. The sum of all supported per network measurements shall equal the total number of successful session establishments to users of other network domains. In case only a subset of per network measurements is supported, a sum subcounter will be provided first (see 3GPP TS 23.228 [7]).

- Each measurement is an integer value. The number of measurements is equal to the number of  other  network domain plus a possible sum value identified by the .sum suffix.

- The measurement name has the form IC.SuccSessionFromOtherNtwkDmn.Domain
where Domain identifies the other network domain, and it is in the format of the domain name information in the SIP messages.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.1.4.6	Successful session establishments to users of other network domains

- This measurement provides the number of  sucessful session establishments towards users (regardless of whether the user is roaming or not) who are belonging to other network domains at the CSCF in the originating network. The measurement is split into subcounters per network domain. This measurement is only valid for the CSCF with P-CSCF and/or S-CSCF functionality.

- CC.

- On transmission by the CSCF of the first response message (which may be SIP_180 or SIP_2XX) indicating a successful session establishment corresponding to the initial SIP_INVITE message (see 3GPP TS 23.228 [7]), and the calling party is belonging to other network domains received by the CSCF. Each session whose called party recognized by the CSCF as belonging to a given network domain is added to the relevant per network domain measurement. The sum of all supported per network measurements shall equal the total number of successful session establishments to users of other network domains. In case only a subset of per network measurements is supported, a sum subcounter will be provided first (see 3GPP TS 23.228 [7]).

- Each measurement is an integer value. The number of measurements is equal to the number of  other network domain plus a possible sum value identified by the .sum suffix.

- The measurement name has the form IC.SuccSessionToOtherNtwkDmn.Domain
where Domain identifies the other network domain, and it is in the format of the domain name information in the SIP messages.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

### 4.1.5	Roaming users related measurements

#### 4.1.5.1	Initial registrations of visiting users from other IMS network domains

- This measurement provides number of initial registrations of users accessing the visited IMS network but are belonging other IMS network domains. The measurement is split into subcounters per IMS network domain. This measurement is only valid for the CSCF with P-CSCF functionality.

- CC.

- On receipt by the P-CSCF of the SIP_REGISTER message from the UE who is belonging to other network domains (i.e. the domain name information of the roamer retrieved from the SIP_REGISTER message is different from the domain of this CSCF) for the initial registration. Each roamer recognized by the P-CSCF as belonging to a given IMS network domain is added to the relevant per IMS network domain measurement. The sum of all supported per IMS network domain measurements shall equal the total number of visiting users. In case only a subset of per IMS network domain measurements is supported, a sum subcounter will be provided first (see 3GPP TS 23.228 [7]).

- Each measurement is an integer value. The number of measurements is equal to the number of IMS network domain plus a possible sum value identified by the .sum suffix.

- The measurement name has the form RU.AttInitRegOfVisitUsers.Domain
where Domain identifies the network domain, and it is in the format of the domain name information in the SIP messages.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.1.5.2	Number of SIP 403 (Forbidden) messages for the roamers

- This measurement provides the number of 403-Forbidden response messages generated to UEs who are belonging to other IMS network domains with whom there are roaming agreements for the registration procedures. The measurement is split into subcounters per network domain. This measurement is only valid for the CSCF with P-CSCF functionality.

- CC.

- On transmission by the P-CSCF of a SIP_403 (Forbidden) response message to the UE who is belonging to other IMS networks for the initial registration procedure.  Each SIP_403 message transmitted by the P-CSCF for the roamer who is belonging to a given network domain is added to the relevant per network domain measurement. The sum of all supported per network domain measurements shall equal the total number of 403 (Forbidden) generated for the roamers who are belonging to other IMS network domains. In case only a subset of per network measurements is supported, a sum subcounter will be provided first (see 3GPP TS 23.228 [7]).

- Each measurement is an integer value. The number of measurements is equal to the number of IMS network domain plus a possible sum value identified by the .sum suffix.

- The measurement name has the form RU.Nbr403InitRegOfVisitUsers.Domain
where Domain identifies the network domain, and it is in the format of the domain name information in the SIP messages.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.1.5.3	Roaming users to other IMS network domains

- This measurement provides the number of the roamers visiting other IMS network domains. This measurement is only valid for the CSCF with S-CSCF functionality.

- CC.

- On receipt by the S-CSCF of a SIP_REGISTER message from the P-CSCF which is belonging to other IMS network domains.

- A single integer value.

- RU.RmgUsersOut.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.1.5.4	Number of 200 OK (Successful) messages for the roamers

- This measurement provides the number of 200 OK (Successful) response messages generated to UEs who are belonging to other IMS network domains with whom there are roaming agreements for the registration procedures. The measurement is split into subcounters per network domain. This measurement is only valid for the CSCF with P-CSCF functionality.

- CC.

- On transmission by the P-CSCF of a 200 OK (Successful) response message to the UE who is belonging to other IMS networks for the initial registration procedure.  Each 200 OK message transmitted by the P-CSCF for the roamer who is belonging to a given network domain is added to the relevant per network domain measurement. The sum of all supported per network domain measurements shall equal the total number of 200 OK (Successful) generated for the roamers who are belonging to other IMS network domains. In case only a subset of per network measurements is supported, a sum subcounter will be provided first (see 3GPP TS 23.228 [7]).

- Each measurement is an integer value. The number of measurements is equal to the number of IMS network domain plus a possible sum value identified by the .sum suffix.

- The measurement name has the form RU.Nbr200InitRegOfVisitUsers.Domain
where Domain identifies the network domain, and it is in the format of the domain name information in the SIP messages.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

### 4.1.6	Authentication related measurements

The measurements defined in this subclause are subject to the "2 out of 3 approach".

#### 4.1.6.1	Attempted Authentication procedures

- This measurement provides number of attempted Authentication procedures. This measurement is only valid for the CSCF with S-CSCF functionality.

- CC.

- On transmission by the S-CSCF of a Multimedia-Authentication-Request Diameter command to the HSS (see 3GPP TS 29.228 [9]).

- A single integer value.

- MA.AttMAR.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.1.6.2	Successful Authentication procedures

- This measurement provides number of successful Authentication procedures. This measurement is only valid for the CSCF with S-CSCF functionality.

- CC.

- On receipt by the S-CSCF of a Multimedia-Authentication-Answer Diameter command from the HSS indicating a successful authentication procedure (see 3GPP TS 29.228 [9]).

- A single integer value.

- MA.SuccMAA.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.1.6.3	Failed Authentication procedures

- This measurement provides number of failed Authentication procedures. The measurement is split into subcounters per failure cause. This measurement is only valid for the CSCF with S-CSCF functionality.

- CC.

- On receipt by the S-CSCF of a Multimedia-Authentication-Answer Diameter command from HSS indicating the authentication procedure is failed for the specific cause, each failed authentication procedure is added to the relevant measurement according to the failure cause. Possible failure causes are included in 3GPP TS 29.229 [10].
And each expected Multimedia-Authentication-Answer Diameter command not received by the S-CSCF is added into failure cause 'No Reply' (not specified in 3GPP TS 29.229 [10]).
The sum of all supported per cause measurements shall equal the total number of failed authentication procedures. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes plus a possible sum value identified by the .sum suffix.

- The measurement name has the form MA.FailMAA.Cause
where Cause identifies the failure cause.
The cause 'No Reply' is identified by the .NoReply suffix.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

### 4.1.7	HSS initiated update of User Profile related measurements

The measurements defined in this subclause are subject to the "2 out of 3 approach".

#### 4.1.7.1	Attempted HSS initiated User Profile Update procedures

- This measurement provides number of attempted HSS initiated user profile update procedures. This measurement is only valid for the CSCF with S-CSCF functionality.

- CC.

- On receipt by the S-CSCF of a Push-Profile-Request Diameter command from the HSS (see 3GPP TS 29.228 [9]).

- A single integer value.

- UP.AttPPR.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.1.7.2	Successful HSS initiated User Profile Update procedures

- This measurement provides number of successful HSS initiated user profile update procedures. This measurement is only valid for the CSCF with S-CSCF functionality.

- CC.

- On transmission by the S-CSCF to the HSS of a Push-Profile-Answer Diameter command indicating a successful user profile update procedure (see 3GPP TS 29.228 [9]).

- A single integer value.

- UP.SuccPPA.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.1.7.3	Failed HSS initiated user profile update procedures

- This measurement provides number of failed HSS initiated user profile update procedures. The measurement is split into subcounters per failure cause. This measurement is only valid for the CSCF with S-CSCF functionality.

- CC.

- On transmission by the S-CSCF to the HSS of a Push-Profile-Answer Diameter command indicating the user profile update procedure is failed for the specific cause, each failed user profile update procedure is added to the relevant measurement according to the failure cause. Possible failure causes are included in 3GPP TS 29.229 [10]. The sum of all supported per cause measurements shall equal the total number of failed user profile update procedures. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes plus a possible sum value identified by the .sum suffix.

- The measurement name has the form UP.FailPPA.Cause
where Cause identifies the failure cause.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

### 4.1.8	Subscription procedure related measurements

The measurements defined in this subclause are subject to the "2 out of 3 approach".

#### 4.1.8.1	Number of attempted Subscription procedures

- This measurement provides the number of initial subscriptions attempted at the CSCF with S-CSCF functionality.

- CC.

- On receipt by the CSCF of an initial SIP_SUBSCRIBE message (see TS 24.229 [8]).

- A single integer value.

- SUB.AttSubscribe.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.1.8.2	Number of successful Subscription procedures

- This measurement provides the number of successful subscription establishments received at the CSCF with S-CSCF functionality.

- CC.

- On transmission by the CSCF of a response message with a response code (e.g. SIP_200_OK) indicating a successful subscription setup corresponding to the initial SIP_SUBSCRIBE message (see TS 24.229 [8]).

- A single integer value.

- SUB.SuccSubscribe.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.1.8.3	Number of failed Subscription procedures

- This measurement provides the number of failed subscription establishments at the CSCF. The measurement is split into subcounters per response codes.

- CC.

- On transmission by the CSCF of a response message with a response code (e.g. SIP_430) indicating a failed subscription establishment corresponding to the initial SIP_SUBSCRIBE message (see TS 24.229 [8]), each failed subscription establishment trial is added to the relevant measurement according to the message type. In case only a subset of per message type measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of message types plus a possible sum value identified by the .sum suffix.

- The measurement name has the form SUB.FailedSubscribe.Cause
where Cause identifies the response code.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

### 4.1.9	Notify procedure related measurements

The measurements defined in this subclause are subject to the "2 out of 3 approach".

#### 4.1.9.1	Number of emitted Notify procedures

- This measurement provides the number of emitted Notify SIP messages at the CSCF with S-CSCF functionality.

- CC.

- On transmission by the CSCF of a SIP_NOTIFY message (see TS 24.229 [8]).

- A single integer value.

- NOTIF.EmittedNotify.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.1.9.2	Number of successful Notify procedures

- This measurement provides the number of successful Notify messages at the CSCF with S-CSCF functionality.

- CC.

- On receipt by the CSCF of a response message with response code (e.g. SIP_200_OK) indicating a successful response corresponding to the emited SIP_NOTIFY message (see TS 24.229 [8]).

- A single integer value.

- NOTIF.SuccNotify.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.1.9.3	Number of failed Notify procedures

- This measurement provides the number of failed Notify messages at the CSCF with S-CSCF functionality. The measurement is split into subcounters per response codes.

- CC.

- On receipt by the CSCF of a response message with response code (e.g. SIP_430) indicating a failed response corresponding to the SIP_NOTIFY message (see TS 24.229 [8]), each failed Notify message trial is added to the relevant measurement according to the message type. In case only a subset of per message type measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of message types plus a possible sum value identified by the .sum suffix.

- The measurement name has the form NOTIF.FailedNotify.Cause
where Cause identifies the response code.

- CscfFunction.

- Valid for packet switched traffic.

- IMS.

### 4.1.10	 Immediate Messaging related measurements

The three measure types defined in this subclause are subject to the “2 out of 3 approach”.

#### 4.1.10.1	Attempted Immediate Messaging procedures

- This measurement provides the number of attempted immediate messaging procedures at CSCF.

- CC.

- On receipt by the CSCF of a SIP_MESSAGE request message for the Immediate Messaging (see 3GPP TS 23.228 [7]).

- A single integer value.

- SC. AttImMsg.

- CscfFunction.

- Valid for packet switching.

- IMS.

#### 4.1.10.2	Successful Immediate Messaging procedures

- This measurement provides the number of successful immediate messaging procedures at CSCF.

- CC.

- On transmission by the CSCF of a response message with a response code (e.g. SIP_200_OK) indicating a successful response corresponding to the SIP_MESSAGE request message (see TS 24.229 [8]).

- A single integer value.

- SC. SuccImMsg.

- CscfFunction.

- Valid for packet switching.

- IMS.

#### 4.1.10.3	Failed Immediate Messaging procedures

- This measurement provides the number of failed immediate messaging procedures at CSCF.

- CC

- On transmission by the CSCF of a response message with response code (e.g. SIP_430) indicating a failed response corresponding to the SIP_MESSAGE request message (see TS 24.229 [8]), each failed SIP_MESSAGE request message trial is added to the relevant measurement according to the response message type. In case only a subset of per message type measurements is supported, a sum subcounter will be provided first.

- Each measurement (as defined in e) is an integer value. The number of measurements is equal to the number of causes supported plus a possible sum value identified by the .sum suffix.

- SC.FailImMsg.Cause	
where Cause identifies the cause of failure.

- CscfFunction

- Valid for packet switching.

- IMS.

### 4.1.11	 Emergency session procedure related measurements

The three measurements defined in this subclause are subject to the “2 out of 3 approach”.

#### 4.1.11.1	Attempted emergency session procedures

- This measurement provides the number of attempted emergency session procedures at CSCF. This measurement is valid for P-CSCF.

- CC.

- On receipt by the CSCF of a SIP initial request message except SIP_REGISTER for the emergency session (see 3GPP TS 23.167 [35], TS 24.229 [8]).

- A single integer value.

- SC.AttEmgSess.

- CscfFunction.

- Valid for packet switching.

- IMS.

#### 4.1.11.2	Successful emergency session procedures

- This measurement provides the number of successful emergency session procedures at CSCF. This measurement is valid for P-CSCF.

- CC.

- On transmission by the CSCF of a SIP_200_OK response message corresponding to the SIP initial request message except SIP_REGISTER for the emergency session (see 3GPP TS 23.167 [35], TS 24.229 [8]).

- A single integer value.

- SC.SuccEmgSess.

- CscfFunction.

- Valid for packet switching.

- IMS.

#### 4.1.11.3	Failed emergency session procedures

- This measurement provides the number of failed emergency session procedures at CSCF. The measurement is split into subcounters per response status codes. This measurement is valid for P-CSCF.

- CC

- On transmission by the CSCF of a response message with response code (e.g. SIP_380) indicating a failed response corresponding to the SIP initial request message except SIP_REGISTE for the emergency session (see 3GPP TS 23.167 [35], TS 24.229 [8]). Each failed session establishment is added to the relevant measurement according to the status code. The sum of all supported per status code measurements shall equal the total number of failed session establishments. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes plus a possible sum value identified by the .sum suffix.

- The measurement name has the form SC. FailEmgSess.Cause
where Cause identifies the status code.

- CscfFunction

- Valid for packet switching.

- IMS.

## 4.2	Measurements related to HSSBGCF

### 4.2.1	HSS database usage related measurements

#### 4.2.1.1	Provisioning related measurements

##### 4.2.1.1.1	Number of provisioned IMS subscriptions currently stored in a HSS

- This measurement provides the number of provisioned IMS subscriptions currently stored in a HSS.

- Gauge.

- Incremented when a  IMS subscription (3GPP TS 23.228 [7]) is provisioned in the HSS and decremented when a subscription is  removed from the HSS.

- A single integer value.

- DBU.NbrImsSubscription.

- HssFunction.

- Valid for Circuit switching traffic and packet switching traffic.

- IMS.

##### 4.2.1.1.2	Number of provisioned Private User Identity stored in a HSS

- This measurement provides the number of provisioned Private User Identity currently stored in a HSS.

- Gauge.

- Incremented when a Private User Identity is provisioned in the HSS and decremented when a Private User Identity is removed from the HSS (3GPP TS 23.008 [16]).

- A single integer value.

- DBU.NbrPriUserId.

- HssFunction.

- Valid for Circuit switching traffic and packet switching traffic.

- IMS.

##### 4.2.1.1.3	Number of provisioned Public User Identities with SIP URI format stored in a HSS

- This measurement provides the number of provisioned Public User Identities with SIP URI format stored in a HSS.

- Gauge.

- Incremented when a Public User Identity with SIP URI format (3GPP TS 23.008 [16]) is provisioned in the HSS and decremented when a Public User Identity with SIP URI format is  removed from the HSS.

- A single integer value.

- DBU.NbrSipPubUserId.

- HssFunction.

- Valid for Circuit switching traffic and packet switching traffic.

- IMS.

##### 4.2.1.1.4	Number of provisioned Public User Identities with TEL URI format stored in a HSS

- This measurement provides the number of provisioned Public User Identities with TEL URI format stored in a HSS.

- Gauge.

- Incremented when a Public User Identity with TEL URI format (3GPP TS 23.008 [16]) is provisioned in the HSS and decremented when a Public User identity with TEL URI format is removed from the HSS.

- A single integer value.

- DBU.NbrTelPubUserId.

- HssFunction.

- Valid for Circuit switching traffic and packet switching traffic.

- IMS.

##### 4.2.1.1.5	Number of Private Service Identities stored in a HSS

- This measurement provides the number of Private Service Identities currently stored in a HSS.

- Gauge.

- Incremented when a Private Service Identity (3GPP TS 23.008 [16]) is provisioned in the HSS and decremented when a Private Service Identity is removed from the HSS.

- A single integer value.

- DBU.NbrPriSrvId.

- HssFunction.

- Valid for Circuit switching traffic and packet switching traffic.

- IMS.

##### 4.2.1.1.6	Number of Public Service Identities with SIP URI format stored in a HSS

- This measurement provides the number of Public Service Identities with SIP URI format stored in a HSS.

- Gauge.

- Incremented when a Public Service Identity with SIP URI format (3GPP TS 23.008 [16]) is provisioned in the HSS and decremented when a Public Service Identity with SIP URI format is removed from the HSS.

- A single integer value.

- DBU.NbrSipPubSrvId.

- HssFunction.

- Valid for Circuit switching traffic and packet switching traffic.

- IMS.

##### 4.2.1.1.7	Number of Public Service Identities with TEL URI format stored in a HSS

- This measurement provides the number of Public Service Identities with TEL URI format stored in a HSS.

- Gauge.

- Incremented when a  Public Service Identity with TEL URI format (3GPP TS 23.008 [16]) is provisioned in the HSS and decremented when a Public Service Identity with TEL URI format is removed from the HSS.

- A single integer value.

- DBU.NbrTelPubSrvId.

- HssFunction.

- Valid for Circuit switching traffic and packet switching traffic.

- IMS.

#### 4.2.1.2	Registration Status related measurements

##### 4.2.1.2.1	Number of Public User Identities in registered state

- This measurement provides the number of Public User Identities in registered state stored in a HSS. The measurement is split into subcounters per visited network domain.

- Gauge.

- Incremented when a Public User Identity's Registration Status changes to "registered", and decremented when a Public User Identity's Registration Status changes from "registered".

- An integer value.

- DBU.NbrRegPubUserId.RoamingDomain
where RoamingDomain identifies  the Domain Name of the visited network that a registration is initiated from.

- HssFunction.

- Valid for Circuit switching traffic and packet switching traffic.

- IMS.

##### 4.2.1.2.2	Number of Public User Identities in unregistered state

- This measurement provides the number of Public User Identities in unregistered state stored in a HSS.

- Gauge.

- Incremented when a Public User Identity's Registration Status changes to "unregistered" and decremented when a Public User Identity's Registration Status changes from "unregistered".

- An integer value.

- DBU.NbrUnregPubUserId.

- HssFunction.

- Valid for Circuit switching traffic and packet switching traffic.

- IMS.

##### 4.2.1.2.3	Number of Private User Identities in registered state

- This measurement provides the number of Private User Identities that have been used in the registration of one or more currently registered Public User Identities stored in a HSS.

- Gauge.

- Incremented on the emergence of the first registered Public User Identity associated with a Private User Identity and  decremented on the deregistration of the last registered Public User Identity associated with a Private User Identity.

- An integer value.

- DBU.NbrRegPriUsrId.

- HssFunction.

- Valid for Circuit switching traffic and packet switching traffic.

- IMS.

### 4.2.2	User registration status query related measurements

The measurements defined in this subclause are subject to the “2 out of 3 approach”.

Figure 2

#### 4.2.2.1	Attempted user registration status queries

- This measurement provides the number of attempted user registration status queries received by a HSS from the I-CSCF.

- CC.

- On receipt of a Diameter command User-Authorization-Request (UAR) by the HSS from the I-CSCF. (3GPP TS 29.228 [9]).

- A single integer value.

- UR.AttUAR.

- HssFunction.

- Valid for Circuit Switching traffic and packet switching traffic.

- IMS.

#### 4.2.2.2	Successful user registration status queries

- This measurement provides the number of successfully performed user registration status queries by a HSS. The measurement can be split into subcounters per the success cause.

- CC.

- On transmission by HSS of Diameter command User-Authorization-Answer (UAA) with Result Code of either DIAMETER_SUCCESS, DIAMETER_SUBSEQUENT_REGISTRATION or DIAMETER_FIRST_REGISTRATION to the I-CSCF (3GPP TS 29.228 [9], 3GPP TS 29.229 [10]), each failed user registration status query procedure is added to the relevant measurement according to the success cause. The sum of all supported per cause measurements shall equal the total number of successful User registration status query procedures. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- A single integer value per measurement type defined in e).

- The measurement name has the form UR.SuccUAA.Cause
where Cause identifies the successful cause. The Cause shall be the cause code of one of the three success causes defined in bullet c) when constructing the measurement name,

- HssFunction.

- Valid for Circuit Switching traffic and packet switching traffic.

- IMS.

#### 4.2.2.3	Failed user registration status queries

- This measurement provides the number of failed user registration status query procedures processed by a HSS. The measurement is split into subcounters per the failure cause.

- CC

- On transmission by the HSS of Diameter command User-Authorization-Answer (UAA) with a Result Code other than DIAMETER_SUCCESS, DIAMETER_SUBSEQUENT_REGISTRATION or DIAMETER_FIRST_REGISTRATION (3GPP TS 29.228 [9]) to the I-CSCF, each failed user registration status query procedure is added to the relevant measurement according to the failure cause. Possible failure causes are included in 3GPP TS 29.229 [10]. The sum of all supported per cause measurements shall equal the total number of failed User registration status query procedures. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement (as defined in e) is an integer value. The number of measurements is equal to the number of causes supported plus a possible sum value identified by the .sum suffix.

- The measurement name has the form UR.FailUAA.Cause
where Cause identifies the failure.cause.

- HssFunction.

- Valid for Circuit Switching traffic and packet switching traffic.

- IMS.

### 4.2.3	S-CSCF registration/de-registration notification related measurements

The measurements defined in this subclause are subject to the “2 out of 3 approach”.

Figure 3

#### 4.2.3.1	Attempted S-CSCF registration/de-registration notifications

- This measurement provides the number of attempted S-CSCF registration/deregistration notifications received by a HSS.

- CC.

- On receipt of Diameter command Server-Assignment-Request (SAR) by the HSS from the S-CSCF (3GPP TS 29.228 [9]).

- A single integer value.

- UR.AttSAR.

- HssFunction.

- Valid for circuit switching traffic and packet switching traffic.

- IMS.

#### 4.2.3.2	Successful S-CSCF registration/de-registration notifications

- This measurement provides the number of successfully performed S-CSCF registration/deregistration notifications by a HSS.

- CC.

- On transmission by HSS of Diameter command Server-Assignment-Answer (UAA) with Result Code of DIAMETER_SUCCESS (3GPP TS 29.228 [9]) to the S-CSCF.

- A single integer value.

- UR.SuccSAA.

- HssFunction.

- Valid for circuit switching traffic and packet switching traffic.

- IMS.

#### 4.2.3.3	Failed S-CSCF registration/de-registration notifications

- This measurement provides the number of failed S-CSCF registration/deregistration notification procedures by a HSS with the S-CSCF. The measurement is split into subcounters per the failure cause.

- CC.

- On transmission by the HSS of Diameter command Server-Assignment-Answer (UAA) with a Result Code other than DIAMETER_SUCCESS (3GPP TS 29.228 [9]) to the S-CSCF, each S-CSCF Registration/Deregistration notification procedure is added to the relevant measurement according to the failure cause. Possible failure causes are included in 3GPP TS 29.229 [10]. The sum of all supported per cause measurements shall equal the total number of failed S-CSCF Registration/Deregistration notification procedures. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement (as defined in e) is an integer value. The number of measurements is equal to the number of causes supported plus a possible sum value identified by the .sum suffix.

- The measurement name has the form UR.FailSAA.Cause
where Cause identifies the failure cause.

- HssFunction.

- Valid for circuit switching traffic and packet switching traffic.

- IMS.

### 4.2.4	Network initiated de-registration by the HSS related measurements

The measurements defined in this subclause are subject to the  “2 out of 3 approach”.

Figure 4

#### 4.2.4.1	Attempted network initiated de-registrations by HSS

- This measurement provides the number of attempted network initiated de-registration procedures by a HSS.

- CC.

- On transmission of Diameter command Registration-Termination-Request (RTR) by the HSS to the S-CSCF (3GPP TS 29.228 [9]).

- A single integer value.

- UR.AttRTR.

- HssFunction.

- Valid for circuit switching traffic and packet switching traffic.

- IMS.

#### 4.2.4.2	Successful network initiated de-registrations by HSS

- This measurement provides the number of successfully performed network initiated de-registration by a HSS.

- CC.

- On receipt by HSS of Diameter command Registration-Termination-Anser (RTA) with Result Code of DIAMETER_SUCCESS (3GPP TS 29.228 [9]) from the S-CSCF.

- A single integer value.

- UR.SuccRTA.

- HssFunction.

- Valid for Circuit switching traffic and packet switching traffic.

- IMS.

#### 4.2.4.3	Failed network initiated de-registrations by HSS

- This measurement provides the number of failed network initiated de-registration procedures perceived by a HSS from the S-CSCF. The measurement is split into subcounters per the failure cause.

- CC.

- On receipt by the HSS of Diameter command Registration-Termination-Answer (RTA) with Result Code other than DIAMETER_SUCCESS (3GPP TS 29.228 [9]) from the S-CSCF, each failed network initiated de-registration by HSS is added to the relevant measurement according to the failure cause. Possible failure causes are included in 3GPP TS 29.229 [10].
And each expected Diameter command Registration-Termination-Aswer (RTA) not received by the HSS is added into failure cause 'No Reply' (not specified in 3GPP TS 29.229 [10]).
The sum of all supported per cause measurements shall equal the total number of failed network initiated de-registrations. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement (as defined in e) is an integer value. The number of measurements is equal to the number of causes supported plus a possible sum value identified by the .sum suffix.

- The measurement name has the form UR.FailRTA.Cause:
where Cause identifies the failure cause.
The cause 'No Reply' is identified by the .NoReply suffix.

- HssFunction.

- Valid for Circuit switching traffic and packet switching traffic.

- IMS.

### 4.2.5	User location query related measurements

The measurements defined in this subclause are subject to the "2 out of 3 approach".

Figure 5

#### 4.2.5.1	Attempted user location queries

- This measurement provides the number of attempted user location queries received by a HSS from the I-CSCF.

- CC.

- On receipt of Diameter command Location-Info-Request (LIR) by the HSS from the I-CSCF (3GPP TS 29.228 [9]).

- A single integer value.

- LIQ.AttLIR.

- HssFunction.

- Valid for Circuit switching traffic and packet switching traffic.

- IMS.

#### 4.2.5.2	Successful user location queries

- This measurement provides the number of successfully performed user location queries by a HSS from the I-CSCF.

- CC.

- On transmission by HSS of Diameter command Location-Info-Answer (LIA) with Result Code of DIAMETER_SUCCESS (3GPP TS 29.228 [9]) to the I-CSCF.

- A single integer value.

- LIQ.SuccLIA.

- HssFunction.

- Valid for Circuit switching traffic and packet switching traffic.

- IMS.

#### 4.2.5.3	Failed user location queries

- This measurement provides the number of failed user location queries procedures by a HSS with the I-CSCF. The measurement is split into subcounters per the failure cause.

- CC.

- On transmission by the HSS of Diameter command Location-Info-Answer (LIA) with a Result Code other than DIAMETER_SUCCESS (3GPP TS 29.228 [9]) to the I-CSCF, each failed user location query procedure is added to the relevant measurement according to the failure cause. Possible failure causes are included in 3GPP TS 29.229 [10]. The sum of all supported per cause measurements shall equal the total number of failed user location query procedures. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement (as defined in e) is an integer value. The number of measurements is equal to the number of causes supported plus a possible sum value identified by the .sum suffix.

- The measurement name has the form LIQ.FailLIA.Cause
where Cause identifies the failure cause.

- HssFunction

- Valid for Circuit switching traffic and packet switching traffic

- IMS

### 4.2.6	HSS initiated update of user profile related measurements

The measurements defined in this subclause are subject to the "2 out of 3 approach".

Figure 6

#### 4.2.6.1	Attempted HSS initiated updates of user profile

- This measurement provides the number of attempted HSS initiated update of User Profile by a HSS to the S-CSCF.

- CC.

- On transmission of Diameter command Push-Profile-Request (PPR) by the HSS to the S-CSCF (3GPP TS 29.228 [9]).

- A single integer value.

- UP.AttPPR.

- HssFunction.

- Valid for Circuit switching traffic and packet switching traffic.

- IMS.

#### 4.2.6.2	Successful HSS initiated updates of user profile

- This measurement provides the number of successfully performed HSS initiated update of User Profile by a HSS to the S-CSCF.

- CC.

- On receipt by HSS of Diameter command Push-Profile-Answer (PPA) with Result Code of DIAMETER_SUCCESS (3GPP TS 29.228 [9]) from the S-CSCF.

- A single integer value.

- UP.SuccPPA.

- HssFunction.

- Valid for Circuit switching traffic and packet switching traffic.

- IMS.

#### 4.2.6.3	Failed HSS initiated updates of user profile

- This measurement provides the number of failed HSS initiated update of User Profile procedures by a HSS with the S-CSCF. The measurement is split into subcounters per the failure cause.

- CC.

- On receipt by the HSS of Diameter command Push-Profile-Answer (PPA) with a Result Code other than DIAMETER_SUCCESS (3GPP TS 29.228 [9]) from the S-CSCF, each failed HSS initiated updates of User Profile procedure is added to the relevant measurement according to the failure cause. Possible failure causes are included in 3GPP TS 29.229 [10].
And each expected Diameter command Push-Profile-Answer (PPA) not received by the HSS is added into failure cause 'No Reply' (not specified in 3GPP TS 29.229 [10]).
The sum of all supported per cause measurements shall equal the total number of failed HSS initiated updates of User Profile procedures. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement (as defined in e) is an integer value. The number of measurements is equal to the number of causes supported plus a possible sum value identified by the .sum suffix.

- The measurement name has the form UP.FailPPA.Cause
where Cause identifies the failure cause.
The cause 'No Reply' is identified by the .NoReply suffix.

- HssFunction.

- Valid for Circuit switching traffic and packet switching traffic.

- IMS.

### 4.2.7	Authentication procedure related measurements

The measurements defined in this subclause are subject to the "2 out of 3 approach".

Figure 7

#### 4.2.7.1	Attempted authentication procedures

- This measurement provides the number of attempted authentication procedures received by a HSS from the S-CSCF.

- CC.

- On receipt of Diameter command Multimedia-Auth-Request (MAR) by the HSS from the S-CSCF (3GPP TS 29.228 [9]).

- A single integer value.

- MA.AttMAR.

- HssFunction.

- Valid for Circuit switching traffic and packet switching traffic.

- IMS.

#### 4.2.7.2	Successful authentication procedures

- This measurement provides the number of successfully performed authentication procedures by a HSS with the S-CSCF (3GPP TS 33.203 [28]).

- CC.

- On transmission by HSS of Diameter command Multimedia-Auth-Answer (MAA) with Result Code of DIAMETER_SUCCESS (3GPP TS 29.228 [9]) to the S-CSCF.

- A single integer value.

- MA.SuccMAA.

- HssFunction.

- Valid for Circuit switching traffic and packet switching traffic.

- IMS.

#### 4.2.7.3	Failed authentication procedures

- This measurement provides the number of failed authentication procedures by a HSS with the S-CSCF (3GPP TS 33.203 [28]). The measurement is split into subcounters per the failure cause.

- CC.

- On transmission by the HSS of Diameter command Multimedia-Auth-Answer (MAA) with a Result Code other than DIAMETER_SUCCESS (3GPP TS 29.228 [9]) to the S-CSCF, each failed authentication procedure is added to the relevant measurement according to the failure cause. Possible failure causes are included in 3GPP TS 29.229 [10]. The sum of all supported per cause measurements shall equal the total number of failed authentication procedures. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement (as defined in e) is an integer value. The number of measurements is equal to the number of causes supported plus a sum value identified by the .sum suffix.

- The measurement name has the form MA.FailMAA.Cause
where Cause identifies the failure cause.

- HssFunction.

- Valid for Circuit switching traffic and packet switching traffic.

- IMS.

### 4.2.8	Data read (Sh-Pull) procedure related measurements

The measurements defined in this subclause are subject to the "2 out of 3 approach".

Figure 8

#### 4.2.8.1	Attempted data read (Sh-Pull) procedures

- This measurement provides the number of attempted Data read (Sh-Pull) procedures by a HSS from the AS.

- CC.

- On receipt of Diameter command User-Data-Request (UDR) by the HSS from the AS. (3GPP TS 29.328 [17]).

- A single integer value.

- DTR.AttUDR.

- HssFunction.

- Valid for Circuit switching traffic and packet switching traffic.

- IMS.

#### 4.2.8.2	Successful data read (Sh-Pull) procedures

- This measurement provides the number of successfully Data read (Sh-Pull) procedures by a HSS with the AS.

- CC.

- On transmission by HSS of Diameter command User-Data-Answer (UDA) to the AS with the result code indicating successful execution of a Data read (Sh-Pull) procedure (3GPP TS 29.329 [18]).

- A single integer value.

- DTR.SuccUDA.

- HssFunction.

- Valid for Circuit switching traffic and packet switching traffic.

- IMS.

#### 4.2.8.3	Failed data read (Sh-Pull) procedures

- This measurement provides the number of failed Data read (Sh-Pull) procedures by a HSS with the AS. The measurement is split into subcounters per the failure cause.

- CC.

- On transmission by the HSS of Diameter command User-Data-Answer (UDA) to the AS with the result code indicating a failure (3GPP TS 29.328 [17]), each failed data read procedure is added to the relevant measurement according to the failure cause. Possible failure causes are included in 3GPP TS 29.329 [18]. The sum of all supported per cause measurements shall equal the total number of failed data read procedures. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes supported plus a possible sum value identified by the .sum suffix.

- The measurement name has the form DTR.FailUDA.Cause
where Cause identifies the failure cause.

- HssFunction.

- Valid for Circuit switching traffic and packet switching traffic.

- IMS.

### 4.2.9	Data update (Sh-Update) procedure related measurements

The measurements defined in this subclause are subject to the "2 out of 3 approach".

Figure 9

#### 4.2.9.1	Attempted data update (Sh-Update) procedures

- This measurement provides the number of attempted Data update (Sh-Update) procedures by a HSS from the AS.

- CC.

- On receipt of Diameter command Profile-Update-Request (PUR) by the HSS from the AS (3GPP TS 29.328 [17]).

- A single integer value.

- DTU.AttPUR.

- HssFunction.

- Valid for Circuit switching traffic and packet switching traffic.

- IMS.

#### 4.2.9.2	Successful data update (Sh-Update) procedures

- This measurement provides the number of successfully Data update (Sh-Update) procedures by a HSS with the AS.

- CC.

- On transmission by HSS of Diameter command Profile-Update-Answer (PUA) to the AS with the result code indicating successful execution of a data update (Sh-Update) procedure (3GPP TS 29.329 [18]).

- A single integer value.

- DTU.SuccPUA.

- HssFunction.

- Valid for Circuit switching traffic and packet switching traffic.

- IMS.

#### 4.2.9.3	Failed data update (Sh-Update) procedures

- This measurement provides the number of failed data update (Sh-Update) procedures by a HSS with the AS. The measurement is split into subcounters per the failure cause.

- CC.

- On transmission by the HSS of Diameter command Profile-Update-Answer (PUA) to the AS with the result code indicating a failure (3GPP TS 29.328 [17]), each failed data update procedure is added to the relevant measurement according to the failure cause. Possible failure causes are included in 3GPP TS 29.329 [18]. The sum of all supported per cause measurements shall equal the total number of failed data update procedures. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes supported plus a possible sum value identified by the .sum suffix.

- The measurement name has the form DTU.FailPUA.Cause
where Cause identifies the failure cause.

- HssFunction.

- Valid for Circuit switching traffic and packet switching traffic.

- IMS.

### 4.2.10	Subscription to notifications (Sh-Subs-Notif) procedure related measurements

The measurements defined in this subclause are subject to the "2 out of 3 approach".

Figure 10

#### 4.2.10.1	Attempted subscription to notifications (Sh-Subs-Notif) procedures

- This measurement provides the number of attempted subscription to notifications (Sh-Subs-Notif) procedures by a HSS from the AS.

- CC.

- On receipt of Diameter command Subscribe-Notifications-Request (SNR) by the HSS from the AS (3GPP TS 29.328 [17]).

- A single integer value.

- SUB.AttSNR.

- HssFunction.

- Valid for Circuit switching traffic and packet switching traffic.

- IMS.

#### 4.2.10.2	Successful subscription to notifications (Sh-Subs-Notif) procedures

- This measurement provides the number of successfully subscription to notifications (Sh-Subs-Notif) procedures by a HSS with the AS.

- CC.

- On transmission by HSS of Diameter Subscribe-Notifications-Answer (SNA) to the AS with the result code indicating successful execution of a subscription to notifications (Sh-Subs-Notif) procedure (3GPP TS 29.329 [18]).

- A single integer value.

- SUB.SuccSNA.

- HssFunction.

- Valid for Circuit switching traffic and packet switching traffic.

- IMS.

#### 4.2.10.3	Failed subscription to notifications (Sh-Subs-Notif) procedures

- This measurement provides the number of failed subscription to notifications (Sh-Subs-Notif) procedures by a HSS with the AS. The measurement is split into subcounters per the failure cause.

- CC.

- On transmission by the HSS of Diameter command Subscribe-Notifications-Answer (SNA) to the AS with the result code indicating a failure (3GPP TS 29.328 [17]), each failed subscription to notifications procedure is added to the relevant measurement according to the failure cause. Possible failure causes are included in 3GPP TS 29.329 [18]. The sum of all supported per cause measurements shall equal the total number of failed subscription to notifications procedures. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement (as defined in e) is an integer value. The number of measurements is equal to the number of causes supported plus a possible sum value identified by the .sum suffix.

- The measurement name has the form SUB.FailSNA.Cause
where Cause identifies the failure cause.

- HssFunction.

- Valid for Circuit switching traffic and packet switching traffic.

- IMS.

### 4.2.11	Notification (Sh-Notif) procedure related measurements

The measurements defined in this subclause are subject to the "2 out of 3 approach".

Figure 11

#### 4.2.11.1	Attempted notification (Sh-Notif) procedures

- This measurement provides the number of attempted notification (Sh-Notif) procedures by a HSS to the AS.

- CC.

- On transmission of Diameter command Push-Notification-Request (PNR) by the HSS to the AS (3GPP TS 29.328 [17]).

- A single integer value.

- NOTIF.AttPNR.

- HssFunction.

- Valid for Circuit switching traffic and packet switching traffic.

- IMS.

#### 4.2.11.2	Successful notification (Sh-Notif) procedures

- This measurement provides the number of successfully performed Notification (Sh-Notif) procedures by a HSS to the AS.

- CC.

- On receipt by HSS of Diameter command Push-Notification -Answer (PNA) with the result code indicating successful execution of a notification (Sh-Notif) procedure (3GPP TS 29.329 [18]) from the AS.

- A single integer value.

- NOTIF.SuccPNA.

- HssFunction.

- Valid for Circuit switching traffic and packet switching traffic.

- IMS.

#### 4.2.11.3	Failed notification (Sh-Notif) procedures

- This measurement provides the number of failed notification (Sh-Notif) procedures by a HSS with the AS. The measurement is split into subcounters per the reject cause failure cause.

- CC.

- On receipt by the HSS of Diameter command Push-Notification-Answer (PNA) with the result code indicating a failure (3GPP TS 29.328 [17]) from the AS, each failed notification procedure is added to the relevant measurement according to the failure cause. Possible failure causes are included in TS 29.329 [18].
And each expected Diameter command Push-Notification-Answer (PNA) not received by the HSS is added into failure cause 'No Reply' (not specified in 3GPP TS 29.329 [18]).
The sum of all supported per cause measurements shall equal the total number of failed notification procedures. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement (as defined in e) is an integer value. The number of measurements is equal to the number of causes supported plus a possible sum value identified by the .sum suffix.

- The measurement name has the form NOTIF.FailPNA.Cause:
where Cause identifies the failure cause.
The cause 'No Reply' is identified by the .NoReply suffix.

- HssFunction.

- Valid for Circuit switching traffic and packet switching traffic.

- IMS.

## 4.3	Measurements related to BGCF

### 4.3.1	Session Control related measurements

#### 4.3.1.1	Overview

The performance measurements defined in subclause 4.3.1 are related to the procedures that BGCF forwards session to the selected network.

Based on the procedures described in 3GPP TS 23.228 [7] and 3GPP TS 24.229 [8], when the BGCF receives a request, the BGCF shall forward the request:

-	to an MGCF within its own network for calls routing to PSTN/CS domain in the same network; or

-	to another BGCF in the selected network for calls routing to PSTN/CS domain in another network; or

-	to an I-CSCF in the selected network for sessions routing to another IMS network ; or

-	to an IBCF in its own network for sessions routing to another IMS network or other SIP-based multimedia networks, if local policy requires IBCF capabilities towards another network.

#### 4.3.1.2	Attempted sessions at BGCF

- This measurement provides the number of attempted sessions at the BGCF when network breakout is to occur.

- CC.

- On receipt by the BGCF of an initial SIP_INVITE message (see 3GPP TS 23.228 [7]).

- A single integer value.

- SC.AttSession.

- BgcfFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.3.1.3	Attempted sessions forwarded to MGCF

- This measurement provides the number of attempted sessions forwarded to the MGCF within the same network at the BGCF for calls routing to PSTN/CS domain in the same network.

- CC.

- On transmission by the BGCF of an initial SIP_INVITE message to the MGCF within the same network (see 3GPP TS 23.228 [7]).

- A single integer value.

- SC.AttSessionFwdToMGCF.

- BgcfFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.3.1.4	Attempted sessions forwarded to BGCF

- This measurement provides the number of attempted sessions forwarded to the BGCF in  the selected network for calls routing to PSTN/CS domain in another network.

- CC.

- On transmission by the BGCF of an initial SIP_INVITE message to the BGCF in a different network (see 3GPP TS 23.228 [7]).

- A single integer value.

- SC.AttSessionFwdToBGCF.

- BgcfFunction.

- Valid for packet switched traffic.

- IMS.

- 4.3.1.4a	Attempted sessions forwarded to I-CSCF

- This measurement provides the number of attempted sessions forwarded to the I-CSCF in the selected network for sessions routing to another IMS network.

- CC.

- On transmission by the BGCF of an initial SIP_INVITE message to the I-CSCF in a different network (see 3GPP TS 23.228 [7]).

- A single integer value.

- SC.AttSessionFwdToCSCF.

- BgcfFunction.

- Valid for packet switched traffic.

- IMS.

- 4.3.1.4b	Attempted sessions forwarded to IBCF

- This measurement provides the number of attempted sessions forwarded to the IBCF in its own network for sessions routing to another IMS network or other SIP-based multimedia networks, if local policy requires IBCF capabilities towards another network.

- CC.

- On transmission by the BGCF of an initial SIP_INVITE message to the IBCF in a different network (see 3GPP TS 23.228 [7]).

- A single integer value.

- SC.AttSessionFwdToIBCF.

- BgcfFunction.

- Valid for packet switched traffic.

- IMS.

- 4.3.1.5	Successful sessions forwarded to MGCF

- This measurement provides the number of successful sessions forwarded to the MGCF at the BGCF for calls routing to PSTN or CS domain in the same network.

- CC.

- On receipt by the BGCF of a response message (e.g. SIP_180 or SIP_200) indicating a successful session from the MGCF in the same network, corresponding to the transmitted initial SIP_INVITE message (see 3GPP TS 23.228 [7]). Each successful session forwarded to MGCF is added to the relevant measurement according to the message type. The sum of all supported per message type measurements shall equal the total number of successful sessions forwarded to MGCF. In case only a subset of per message type measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of message types plus a possible sum value identified by the .sum suffix.

- SC.SuccSessionFwdToMGCF.type
where type identifies the message type.

- The following subcounters  with specific message types are the samples of this measurement:
SC.SuccSessionFwdToMGCF.180	Message type: SIP_180
SC.SuccSessionFwdToMGCF.200	Message type: SIP_200_OK only if the SIP_180 has not been received 	for this session.

- BgcfFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.3.1.6	Successful sessions forwarded to BGCF

- This measurement provides the number of successful forwarded sessions to the BGCF in the selected network for calls routing to PSTN/CS domain in another network.

- CC.

- On receipt by the BGCF of a response message (e.g.SIP_180 or SIP_200）indicating a successful session from the BGCF in another network, corresponding to the transmitted initial SIP_INVITE message (see 3GPP TS 23.228 [2]). Each successful session forwarded to BGCF is added to the relevant measurement according to the message type. The sum of all supported per message type measurements shall equal the total number of successful sessions forwarded to BGCF. In case only a subset of per message type measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of message types plus a possible sum value identified by the .sum suffix.

- SC.SuccSessionFwdToBGCF.type
where type identifies the message type

- The following subcounters  with specific message types are the samples of this measurement:
SC.SuccSessionFwdToBGCF.180	Message type: SIP_180
SC.SuccSessionFwdToBGCF.200	Message type: SIP_200_OK only if the SIP_180 has not been received 	for this session.

- BgcfFunction.

- Valid for packet switched traffic.

- IMS.

- 4.3.1.7	Successful sessions forwarded to I-CSCF

  - This measurement provides the number of successful sessions forwarded to the I-CSCF in the selected network for sessions routing to another IMS network.

  - CC.

  - On receipt by the BGCF of a response message (e.g. SIP_180 or SIP_200) indicating a successful session from the I-CSCF in a different network, corresponding to the transmitted initial SIP_INVITE message (see 3GPP TS 23.228 [7]). Each successful session forwarded to I-CSCF is added to the relevant measurement according to the message type. The sum of all supported per message type measurements shall equal the total number of successful sessions forwarded to I-CSCF. In case only a subset of per message type measurements is supported, a sum subcounter will be provided first.

  - Each measurement is an integer value. The number of measurements is equal to the number of message types plus a possible sum value identified by the .sum suffix.

  - SC.SuccSessionFwdToCSCF.type
where type identifies the message type.

  - The following subcounters  with specific message types are the samples of this measurement:
SC.SuccSessionFwdToCSCF.180	Message type: SIP_180
SC.SuccSessionFwdToCSCF.200	Message type: SIP_200_OK only if the SIP_180 has not been received 	for this session.

  - BgcfFunction.

  - Valid for packet switched traffic.

  - IMS.

- 4.3.1.8	Successful sessions forwarded to IBCF

- This measurement provides the number of successful forwarded sessions to the IBCF in its own network for sessions routing to another IMS network or other SIP-based multimedia networks, if local policy requires IBCF capabilities towards another network..

- CC.

- On receipt by the IBCF of a response message (e.g.SIP_180 or SIP_200）indicating a successful session from the IBCF in another network, corresponding to the transmitted initial SIP_INVITE message (see 3GPP TS 23.228 [7]). Each successful session forwarded to IBCF is added to the relevant measurement according to the message type. The sum of all supported per message type measurements shall equal the total number of successful sessions forwarded to IBCF. In case only a subset of per message type measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of message types plus a possible sum value identified by the .sum suffix.

- SC.SuccSessionFwdToIBCF.type
where type identifies the message type

- The following subcounters  with specific message types are the samples of this measurement:
SC.SuccSessionFwdToIBCF.180	Message type: SIP_180
SC.SuccSessionFwdToIBCF.200	Message type: SIP_200_OK only if the SIP_180 has not been received 	for this session.

- BgcfFunction.

- Valid for packet switched traffic.

- IMS.

## 4.4	Measurements related to MGCF

### 4.4.1	Call control related measurements

#### 4.4.1.1	Call control related measurements, CS network originated

The measurements defined in subclauses 4.4.1.1.2, 4.4.1.1.3 and 4.4.1.1.5 are subject to the "2 out of 3 approach".

##### 4.4.1.1.1	Overview

The performance measurements defined in this subclause are related to the CS network originated call that is intentionally routed to IM CN subsystem.

Figure 14: CS network originated, IM CN terminated call procedure

##### 4.4.1.1.2	Attempted calls, CS network originated

- This measurement provides the number of attempted calls from CS network.

- CC.

- On receipt by the MGCF of an "IAM" message(see 3GPP TS 29.163 [11]).

- A single integer value.

- CC.AttCallCsOrig.

- MgcfFunction.

- Valid for circuit switched traffic.

- IMS.

##### 4.4.1.1.3	Successful calls, CS network originated

- This measurement provides the number of successful calls from CS network.

- CC.

- On transmission by the MGCF of an "ACM" or "CON" message (see 3GPP TS 29.163 [11]).

NOTE:	If ACM message is not sent, the "CON" message shall be the trigger.

- A single integer value.

- CC.SuccCallCsOrig.

- MgcfFunction.

- Valid for circuit switched traffic.

- IMS.

##### 4.4.1.1.4	Answered calls, CS network originated

- This measurement provides the number of answered calls from CS network.

- CC.

- On transmission by the MGCF of an "ANM" or "CON" message (see 3GPP TS 29.163 [11]).

- A single integer value.

- CC.AnsCallCsOrig.

- MgcfFunction.

- Valid for circuit switched traffic.

- IMS.

##### 4.4.1.1.5	Failed calls, CS network originated

  - This measurement provides number of failed calls (the calls are originated from CS network).

  - CC.

  - Before MGCF sends “ACM" or "CON" message (see 3GPP TS 29.163 [11]), the call is released..

  - A single integer value.

  - CC.FailCallCsOrig.

  - MgcfFunction.

  - Valid for circuit switched traffic.

  - IMS.

##### 4.4.1.1.6	Call set-up time (Mean), CS network originated

- This measurement provides the mean setup time of the successful CS network originated calls.

- DER (n=1).

- This measurement is obtained by accumulating the time intervals for every successful CS network originated calls between the receipt of "IAM" message and the transmission of  the corresponding "ACM" message or "CON" message if the "ACM" message is not sent over a granularity period using DER (see 3GPP TS 29.163 [11]). The end value of this time will then be divided by the number of successful CS network originated calls observed in the granularity period to give the arithmetic mean, the accumulator shall be reinitialised at the beginning of each granularity period.

- A single integer value (in milliseconds).

- CC.SetupTimeCsOrigMean.

- MgcfFunction.

- Valid for circuit switched traffic.

- IMS.

#### 4.4.1.2	Call control related measurements, IM CN originated

The measurements defined in subclauses 4.4.1.2.2, 4.4.1.2.3 and 4.4.1.2.5 are subject to the "2 out of 3 approach".

##### 4.4.1.2.1	Overview

The performance measurements defined in this subclause is related to the IM CN subsystem originated call that is intentionally routed to CS network.

Figure 15: IM CN originated, CS network terminated call procedure

##### 4.4.1.2.2	Attempted calls, IM CN originated

- This measurement provides the number of attempted calls from IM CN subsystem.

- CC.

- On receipt by the MGCF of an initial SIP_INVITE message (see 3GPP TS 29.163 [11]).

- A single integer value.

- CC.AttCallImOrig.

- MgcfFunction.

- Valid for packet switched traffic.

- IMS.

##### 4.4.1.2.3	Successful calls, IM CN originated

- This measurement provides the number of successful calls from IM CN subsystem. The measurement is split into subcounters per message type (e.g. SIP_180) indicating the successful IM CN originated call.

- CC.

- On transmission by the MGCF of a response message (e.g. SIP_180) indicating a successful IM CN subsystem originated call corresponding to the initial SIP_INVITE message (see 3GPP TS 29.163 [11]), each successful IM CN originated call is added to the relevant measurement according to the message type, the possible response message types are identified in 3GPP TS 24.229 [8]. The sum of all supported per message type measurements shall equal the total number of successful IM CN subsystem originated calls. In case only a subset of per message type measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of message types plus a possible sum value identified by the .sum suffix.

- The measurement name has the form CC.SuccCallImOrig.type
where type identifies the message type.

- The following subcounters with specific message types are the samples of this measurement:
CC.SuccCallImorig.180	Message type: SIP_180. MGCF sends SIP_180.
CC.SuccCallImorig.200	Message type: SIP_200. MGCF sends SIP_200_OK after receiving CON 	message instead of ACM message.

- MgcfFunction.

- Valid for packet switched traffic.

- IMS.

##### 4.4.1.2.4	Answered calls, IM CN originated

- This measurement provides the number of answered calls from IM CN subsystem.

- CC.

- On transmission by the MGCF of a SIP_200_OK message corresponding to an initial SIP_INVITE message. (see 3GPP TS 29.163 [11]).

- A single integer value.

- CC.AnsCallImOrig.

- MgcfFunction.

- Valid for packet switched traffic.

- IMS.

##### 4.4.1.2.5	Failed calls, IM CN originated

- This measurement provides number of failed calls (the calls are originated from IM CN subsystem).

- CC.

- Before MGCF sends SIP 180 or SIP 200 without 180 (see 3GPP TS 29.163 [11]), the call is released..

- A single integer value.

- CC.FailCallImOrig.

- MgcfFunction.

- Valid for packet switched traffic.

- IMS.

##### 4.4.1.2.6	Call set-up time (Mean), IM CN originated

- This measurement provides the mean setup time of the successful IM CN subsystem originated calls.

- DER (n=1).

- This measurement is obtained by accumulating the time intervals for every successful IM CN subsystem originated calls between the receipt of an initial SIP_INVITE message and the transmission of corresponding SIP_180 or SIP_200_OK without SIP_180 message over a granularity period using DER (see 3GPP TS 29.163 [11]). The end value of this time will then be divided by the number of successful IM CN subsystem originated calls observed in the granularity period to give the arithmetic mean, the accumulator shall be reinitialised at the beginning of each granularity period.

- A single integer value (in milliseconds).

- CC.SetupTimeImOrigMean.

- MgcfFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.4.1.3	Call control related measurements, call release

##### 4.4.1.3.1	Overview

The performance measurements defined in this subclause are related to the call release initiated by CS network, IM CN subsystem, MGCF itself and MGW respectively.

##### 4.4.1.3.2	Call release initiated by CS network

- This measurement provides the number of call release initiated by CS network with "REL" or "RSC" or "GRS" message. The measurement is split into subcounters per release message type, and per cause for the REL message.

- CC.

- On receipt by the MGCF of an ISUP/BICC "REL" or "RSC" or "GRS" message (see 3GPP TS 29.163 [11]).

- Each measurement is an integer value. The number of measurements is equal to the number of supported causes in ISUP/BICC "REL" message, plus two values for ISUP/BICC "RSC" and "GRS" message respectively and a possible sum value identified by the .sum suffix.

- CC.RelCallCsInit.CauseOrMsg.

- Where CauseOrMsg identifies the cause in ISUP/BICC "REL" message if the call is released by ISUP/BICC "REL" message (see ITU-T Recommendation Q.850 [19]), or the message type ISUP/BICC "RSC" or "GRS" respectively if the call is released by ISUP/BICC "RSC" or "GRS" message. If the call is released by ISUP/BICC "RSC" or "GRS" message, the CauseOrMsg shall be the constant string "RSC" or "GRS" respectively when constructing the measurement name.

- MgcfFunction.

- Valid for circuit switched traffic.

- IMS.

##### 4.4.1.3.3	Call release initiated by IM CN subsystem

- This measurement provides the number of call release initiated by IM CN subsystem. The measurement is split into subcounters per message type.

- CC.

- On receipt by the MGCF of a SIP_BYE or SIP_CANCEL or SIP_3XX or SIP_4XX or SIP_5XX or SIP_6XX message (see 3GPP TS 29.163 [11]).

- 6 single integer values.

- CC.RelCallsImInit.BYE
CC.RelCallsCsInit.CANCEL
CC.RelCallsCsInit.3XX
CC.RelCallsCsInit.4XX
CC.RelCallsCsInit.5XX
CC.RelCallsCsInit.6XX

- MgcfFunction.

- Valid for packet switched traffic.

- IMS.

##### 4.4.1.3.4	Call release initiated by MGCF

- This measurement provides the number of autonomous call release initiated by MGCF itself. The measurement is split into subcounters per message type.

- CC.

- On transmission of call release message by MGCF due to autonomous call releases (see 3GPP TS 29.163 [11]).

NOTE:	MGCF sends call release message to CS network and IM CN subsystem respectively for one call, just count once.

- 8 single integer values.

- CC.RelCallsMgcfInit.484Imgcf
CC.RelCallsMgcfInit.480Imgcf
CC.RelCallsMgcfInit.488Imgcf
CC.RelCallsMgcfInit.500Imgcf
CC.RelCallsMgcfInit.CotOmgcf
CC.RelCallsMgcfInit.47Omgcf
CC.RelCallsMgcfInit.IsupBiccOmgcf
CC.RelCallsMgcfInit.SipOmgcf.

- The corresponding scenario for these 2 measurements are:

- CC.RelCallsMgcfInit.484Imgcf	I-MGCF initiates call release by sending SIP 484 Address Incomplete.
CC.RelCallsMgcfInit.480Imgcf	I-MGCF initiates call release by sending SIP 480 Temporarily 	Unavailable.
CC.RelCallsMgcfInit.488Imgcf	I-MGCF initiates call release by sending SIP 488 Not Acceptable Here.
CC.RelCallsMgcfInit.500Imgcf	I-MGCF initiates call release by sending SIP 500 Server Internal Error.
CC.RelCallsMgcfInit.CotOmgcf	O-MGCF initiates call release on reception of COT with the Continuity 	Indicators parameter set to "continuity check failed" or expiry of 	ISUP/BICC T8.
CC.RelCallsMgcfInit.47Omgcf	O-MGCF initiates call release by sending ISUP/BICC "REL" with 	cause value (resource unavailable or unspecified).
CC.RelCallsMgcfInit.IsupBiccOmgcf	O-MGCF initiates call release by ISUP/BICC procedure.
CC.RelCallsMgcfInit.SipOmgcf	O-MGCF initiates call release by SIP procedure.

- MgcfFunction.

- Valid for circuit switched and packet switched traffic.

- IMS.

##### 4.4.1.3.5	Call release initiated by MGW

- This measurement provides the number of call release initiated by MGW. The measurement is split into subcounters per H248 error code and H248 service change reason.

- CC.

- On receipt by the MGCF of a H248 message which triggers call release (see ITU-T Recommendation H.248.1 [20]).

- Each measurement is an integer value. The number of measurements is equal to the number of causes plus a possible sum value identified by the .sum suffix.

- CC.RelCallsMgwInit.Cause.
Where Cause identifies the H248 error code or H248 service change reason.(see ITU-T Recommendation H.248.8 [21]).

- MgcfFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.4.1.4	Number of simultaneous online and answered calls (Maximum)

- This measurement provides the maximum number of simultaneous online calls which have been answered in the MGCF.

- SI.

- This measurement is obtained by sampling at a pre-defined interval the number of simultaneous online calls which have been answered and then taking the maximum.

- A single integer value.

- CC.NbrAnsCallMax.

- MgcfFunction.

- Valid for circuit switched traffic.

- IMS.

#### 4.4.1.5	Number of simultaneous online and answered calls (mean)

- This measurement provides the mean number of simultaneous online calls which have been answered in the MGCF.

- SI.

- This measurement is obtained by sampling at a pre-defined interval the number of simultaneous online calls which have been answered and then taking the arithmetic mean.

- A single integer value.

- CC.NbrAnsCallMean.

- MgcfFunction.

- Valid for circuit switched traffic.

- IMS.

## 4.5	Measurements related to IM-MGW

The measurements related to IM-MGW is the same with CS MGW defined in 3GPP TS 32.407[29].

## 4.6	Measurements related to MRFC

### 4.6.1	Session control related measurements

The measurements defined in subclauses 4.6.1.1, 4.6.1.2 and 4.6.1.4 are subject to the "2 out of 3 approach".

#### 4.6.1.1	Attempted session establishments

- This measurement provides the number of attempted session establishments at MRFC.

- CC.

- On receipt by the MRFC of an initial SIP_INVITE message (see 3GPP TS 23.228 [7]).

- A single integer value.

- SC.AttSession.

- MrfcFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.6.1.2	Successful session establishments

- This measurement provides the number of successful session establishments at the MRFC. The measurement is split into subcounters per message type (e.g. SIP_180) indicating the successful session establishments.

- CC.

- On transmission by the MRFC of a response message (e.g. SIP_180) indicating a successful session establishment corresponding to the initial SIP_INVITE message (see 3GPP TS 23.228 [7]), each successful session is added to the relevant measurement according to the message type, the possible response message types are identified in 3GPP TS 24.229 [8]. The sum of all supported per message type measurements shall equal the total number of successful session establishments. In case only a subset of per message type measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of message types plus a possible sum value identified by the .sum suffix.

- SC.SuccSession.type
where type identifies the message type.

- The following subcounters with specific message types are the samples of this measurement:
SC.SuccSession.180	Message type: SIP_180
SC.SuccSession.200	Message type: SIP_200_OK only if the SIP_180 has not been sent for this session.

- MrfcFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.6.1.3	Answered session establishments

- This measurement provides the number of answered session establishments at the MRFC.

- CC.

- On transmission by the MRFC of a SIP_200_OK response message corresponding to the initial SIP_INVITE messages (see 3GPP TS 23.228 [7]).

- A single integer value.

- SC.AnsSession.

- MrfcFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.6.1.4	Failed session establishments

- This measurement provides the number of failed session establishments at the MRFC. The measurement is split into subcounters per response status codes.

- CC.

- On transmission by the MRFC of a response message (e.g. SIP_430) indicating a failed session establishment corresponding to the initial SIP_INVITE messages (see 3GPP TS 23.228 [7]), each failed session establishment is added to the relevant measurement according to the status code. The sum of all supported per status code measurements shall equal the total number of failed session establishments. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of status codes plus a possible sum value identified by the .sum suffix.

- The measurement name has the form SC.FailSession.Cause
where Cause identifies the status code.

- MrfcFunction.

- Valid for packet switched traffic.

- IMS.

### 4.6.2	Multi-party sessions related measurements

The measurements defined in subclauses 4.6.2.1, 4.6.2.2 and 4.6.2.4 are subject to the "2 out of 3 approach".

The measurements defined in subclauses 4.6.2.5, 4.6.2.6 and 4.6.2.7 are subject to the "2 out of 3 approach".

#### 4.6.2.1	Attempted session establishments for multi-party sessions

- This measurement provides the number of attempted session establishments for multi-party sessions at MRFC.

- CC.

- On receipt by the MRFC of an initial SIP_INVITE message for the multi-party sessions service (see 3GPP TS 23.228 [7]).

- A single integer value.

- SC.AttMultipartySession.

- MrfcFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.6.2.2	Successful session establishments for multi-party sessions

- This measurement provides the number of successful session establishments for multi-party sessions at the MRFC. The measurement is split into subcounters per message type (e.g. SIP_180) indicating the successful session establishments for multi-party sessions.

- CC.

- On transmission by the MRFC of a response message (e.g. SIP_180) indicating a successful session establishment corresponding to the initial SIP_INVITE message for the multi-party sessions service (see 3GPP TS 23.228 [7]), each successful session is added to the relevant measurement according to the message type, the possible response message types are identified in 3GPP TS 24.229 [8]. The sum of all supported per message type measurements shall equal the total number of successful session establishments for multi-party sessions. In case only a subset of per message type measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of message types plus a possible sum value identified by the .sum suffix.

- SC.SuccMultipartySession.type
where type identifies the message type.

- The following subcounters with specific message types are the samples of this measurement:
SC.SuccSession.180	Message type: SIP_180
SC.SuccSession.200	Message type: SIP_200_OK only if the SIP_180 has not been sent for this session.

- MrfcFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.6.2.3	Answered session establishments for multi-party sessions

- This measurement provides the number of answered session establishments for multi-party sessions at the MRFC.

- CC.

- On transmission by the MRFC of a SIP_200_OK response message corresponding to the initial SIP_INVITE messages for the multi-party sessions service (see 3GPP TS 23.228 [7]).

- A single integer value.

- SC.AnsMultipartySession.

- MrfcFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.6.2.4	Failed session establishments for multi-party sessions

- This measurement provides the number of failed session establishments for multi-party sessions at the MRFC. The measurement is split into subcounters per response status codes.

- CC.

- On transmission by the MRFC of a response message (e.g. SIP_430) indicating a failed session establishment corresponding to the initial SIP_INVITE message for the multi-party session service (see 3GPP TS 23.228 [7]), each failed session establishment is added to the relevant measurement according to the status code. The sum of all supported per status code measurements shall equal the total number of failed session establishments for the multi-party sessions. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of status codes plus a possible sum value identified by the .sum suffix.

- The measurement name has the form SC.FailMultipartySession.Cause
where Cause identifies the status code.

- MrfcFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.6.2.5	Attempted event subscription procedures for multi-party sessions

- This measurement provides the number of attempted event subscription procedures for multi-party sessions at MRFC.

- CC.

- On receipt by the MRFC of a SIP_SUBSCRIBE message for the multi-party sessions service (see 3GPP TS 23.228 [7] and RFC3265[22]).

- A single integer value.

- SC.AttSubscribe.

- MrfcFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.6.2.6	Successful event subscription procedures for multi-party sessions

- This measurement provides the number of successful event subscription procedures for multi-party sessions at MRFC.

- CC.

- On transmission by the MRFC of a SIP_200_OK message corresponding to the SIP_SUBSCRIBE message for the multi-party sessions service (see 3GPP TS 23.228 [7] and RFC3265[22]).

- A single integer value.

- SC.SuccSubscribe.

- MrfcFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.6.2.7	Failed event subscription procedures for multi-party sessions

- This measurement provides the number of failed event subscription procedures for multi-party sessions at MRFC. The measurement is split into subcounters per response status codes.

- CC.

- On transmission by the MRFC of a response message (e.g. SIP_430) indicating a failed event subscription procedure corresponding to the initial SIP_SUBSCRIBE message for the multi-party session service (see 3GPP TS 23.228 [7] and RFC3265[22]), each failed event subscription procedures is added to the relevant measurement according to the status code. The sum of all supported per status code measurements shall equal the total number of failed event subscription procedures for the multi-party sessions. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of status codes plus a possible sum value identified by the .sum suffix.

- The measurement name has the form SC.FailSubscribe.Cause
where Cause identifies the status code.

- MrfcFunction.

- Valid for packet switched traffic.

- IMS.

### 4.6.3	Announcement service related measurements

The measurements defined in subclauses 4.6.3.1, 4.6.3.2 and 4.6.3.4 are subject to the "2 out of 3 approach".

#### 4.6.3.1	Attempted session establishments for announcements

- This measurement provides the number of attempted session establishments for announcements at MRFC.

- CC.

- On receipt by the MRFC of an initial SIP_INVITE message for the announcement service (see 3GPP TS 23.228 [7]).

- A single integer value.

- SC.AttAnnounce.

- MrfcFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.6.3.2	Successful session establishments for announcements

- This measurement provides the number of successful session establishments for announcements at the MRFC. The measurement is split into subcounters per message type (e.g. SIP_180) indicating the successful session establishments for announcements.

- CC.

- On transmission by the MRFC of a response message (e.g. SIP_180) indicating a successful session establishment corresponding to the initial SIP_INVITE message for the announcement service (see 3GPP TS 23.228 [7]), each successful session is added to the relevant measurement according to the message type, the possible response message types are identified in 3GPP TS 24.229 [8]. The sum of all supported per message type measurements shall equal the total number of successful session establishments for announcements. In case only a subset of per message type measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of message types plus a possible sum value identified by the .sum suffix.

- SC.SuccAnnounce.type
where type identifies the message type.

- The following subcounters with specific message types are the samples of this measurement:
SC.SuccSession.180	Message type: SIP_180
SC.SuccSession.200	Message type: SIP_200_OK only if the SIP_180 has not been sent for this session.

- MrfcFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.6.3.3	Answered session establishments for announcements

- This measurement provides the number of answered session establishments for announcements at the MRFC.

- CC.

- On transmission by the MRFC of a SIP_200_OK response message corresponding to the initial SIP_INVITE messages for the announcement service (see 3GPP TS 23.228 [7]).

- A single integer value.

- SC.AnsAnnounce.

- MrfcFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.6.3.4	Failed session establishments for announcements

- This measurement provides the number of failed session establishments for announcements at the MRFC. The measurement is split into subcounters per response status codes.

- CC.

- On transmission by the MRFC of a response message (e.g. SIP_430) indicating a failed session establishment corresponding to the initial SIP_INVITE message for the announcement service (see 3GPP TS 23.228 [7]), each failed session establishment is added to the relevant measurement according to the status code. The sum of all supported per status code measurements shall equal the total number of failed session establishments for the announcements. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of status codes plus a possible sum value identified by the .sum suffix.

- The measurement name has the form SC.FailAnnounce.Cause
where Cause identifies the status code.

- MrfcFunction.

- Valid for packet switched traffic.

- IMS.

### 4.6.4	Transcoding service related measurements

The measurements defined in subclauses 4.6.4.1, 4.6.4.2 and 4.6.4.4 are subject to the "2 out of 3 approach".

#### 4.6.4.1	Attempted session establishments for transcoding service

- This measurement provides the number of attempted session establishments for transcoding service at MRFC.

- CC.

- On receipt by the MRFC of an initial SIP_INVITE message for the transcoding service (see 3GPP TS 23.228 [7]).

- A single integer value.

- SC.AttTranscoding.

- MrfcFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.6.4.2	Successful session establishments for transcoding service

- This measurement provides the number of successful session establishments for transcoding service at the MRFC. The measurement is split into subcounters per message type (e.g. SIP_180) indicating the successful session establishments for transcoding service.

- CC.

- On transmission by the MRFC of a response message (e.g. SIP_180) indicating a successful session establishment corresponding to the initial SIP_INVITE message for the transcoding service (see 3GPP TS 23.228 [7]), each successful session is added to the relevant measurement according to the message type, the possible response message types are identified in 3GPP TS 24.229 [8]. The sum of all supported per message type measurements shall equal the total number of successful session establishments for transcoding service. In case only a subset of per message type measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of message types plus a possible sum value identified by the .sum suffix.

- SC.SuccTranscoding.type
where type identifies the message type.

- The following subcounters with specific message types are the samples of this measurement:
SC.SuccSession.180	Message type: SIP_180
SC.SuccSession.200	Message type: SIP_200_OK only if the SIP_180 has not been sent for this session.

- MrfcFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.6.4.3	Answered session establishments for transcoding service

- This measurement provides the number of answered session establishments for transcoding service at the MRFC.

- CC.

- On transmission by the MRFC of a SIP_200_OK response message corresponding to the initial SIP_INVITE messages for the transcoding service (see 3GPP TS 23.228 [7]).

- A single integer value.

- SC.AnsTranscoding.

- MrfcFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.6.4.4	Failed session establishments for transcoding service

- This measurement provides the number of failed session establishments for transcoding service at the MRFC. The measurement is split into subcounters per response status codes.

- CC.

- On transmission by the MRFC of a response message (e.g. SIP_430) indicating a failed session establishment corresponding to the initial SIP_INVITE message for the transcoding service (see 3GPP TS 23.228 [7]), each failed session establishment is added to the relevant measurement according to the status code. The sum of all supported per status code measurements shall equal the total number of failed session establishments for the transcoding service. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of status codes plus a possible sum value identified by the .sum suffix.

- The measurement name has the form SC.FailTranscoding.Cause
where Cause identifies the status code.

- MrfcFunction.

- Valid for packet switched traffic.

- IMS.

## 4.7	Measurements related to MRFP

### 4.7.1	RTP related measurements

Performance Measurement definitions in this subclause are only valid if the RTP protocol is used at the MRFP.

#### 4.7.1.1	Number of incoming RTP packets

- This measurement provides the number of incoming RTP packets at MRFP.

- CC.

- On receipt by the MRFP of a RTP packet (see RFC 3550 [23]).

- A single integer value.

- RTP.IncPacket.

- MrfpFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.7.1.2	Number of outgoing RTP packets

- This measurement provides the number of outgoing RTP packets at MRFP.

- CC.

- On transmission by the MRFP of a RTP packet (see RFC 3550 [23]).

- A single integer value.

- RTP.OutgPacket.

- MrfpFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.7.1.3	Number of octets of incoming RTP packets

- This measurement provides the number of octets of incoming RTP packets at MRFP.

- DER (n=1).

- This measurement is obtained by accumulating the number of incoming RTP packet octets (bytes) on receipt by the MRFP of a RTP packet (see RFC 3550 [23]).

- A single integer value.

- RTP.IncOctet.

- MrfpFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.7.1.4	Number of octets of outgoing RTP packets

- This measurement provides the number of octets of outgoing RTP packets at MRFP.

- DER (n=1).

- This measurement is obtained by accumulating the number of outgoing RTP packet octets (bytes) on transmission by the MRFP of a RTP packet (see RFC 3550 [23]).

- A single integer value.

- RTP.OutgOctet.

- MrfpFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.7.1.5	Number of lost incoming RTP packets

- This measurement provides the number of lost incoming RTP packets.

- CC.

- On detection of loss of an incoming RTP packet by the MRFP (see RFC 3550 [23]).

- A single integer value.

- RTP.LostIncPacket.

- MrfpFunction.

- Valid for packet switched traffic.

- IMS.

## 4.8	Measurements related to PCRF

The measurements related to PDF are restricted to R6 network architecture.

Editor's note: PcrfFunction used in following text is a subject for further study of IMS NRM modelling for R7 requirements and working 3GPP/3GPP2 IMS NRM alignment.

### 4.8.1	Authorization of QoS resources related measurements

The five measurement types defined in this subclause are subject to the "4 out of 5 approach".

#### 4.8.1.1	Overview

The measurements defined in subclause 4.8.1 are related to procedures of QoS resource authorization. AF initiates a Diameter transaction to PCRF for authorizing the resources in the session establishment or modification. PCRF authorizes the resources based on binding mechanism and service information.

The procedure is shown in figure 16.

EMBED Word.Picture.8

Figure 16: Resource authorization procedure

#### 4.8.1.2	Attempted resource authorization procedures at session establishment

- This measurement provides the number of attempted resource authorizations initiated by AF to PCRF when establishing new sessions.

- CC.

- On receipt by the PCRF of an AAR Diameter command from AF with a new session ID (see 3GPP TS 29.213[33]).

- A single integer value.

- QoS.EstabAttAARSession.

- PcrfFunction.

- Valid for packet switched traffic.

- IMS

#### 4.8.1.3	Attempted resource authorization procedures at session modification

- This measurement provides the number of attempted resource authorizations initiated by AF to PCRF when modifying the existing sessions.

- CC.

- On receipt by the PCRF of an AAR Diameter command from AF to modify an existing session (see 3GPP TS 29.213 [33]).

- A single integer value.

- QoS.ModfAttAARSession.

- PcrfFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.8.1.4	Successful resource authorization procedures at session establishment

- This measurement provides the number of successful resource authorizations initiated by AF to PCRF when establishing new sessions.

- CC.

- On transmission by the PCRF of an AAA Diameter command to AF with a new stored session ID and "2xxxx" result-code (see 3GPP TS 29.213[33], RFC 3588 [15]).

- A single integer value.

- QoS.EstabSuccAAASession.

- PcrfFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.8.1.5	Successful resource authorization procedures at session modification

- This measurement provides the number of successful resource authorizations initiated by AF to PCRF when modifying the existing sessions.

- CC.

- On transmission by the PCRF of an AAA Diameter command to AF with an existing session ID and "2xxxx" result-code (see 3GPP TS 29.213[33], RFC 3588 [15]).

- A single integer value.

- QoS.ModfSuccAAASession.

- PcrfFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.8.1.6	Failed resource authorization procedures

- This measurement provides the number of failed resource authorizations initiated by AF to PCRF.

- CC.

- On transmission by the PCRF of an AAA Diameter command to AF with non-2xxx result-code (see 3GPP TS 29.213 [33], RFC 3588 [15]). Each failed resource authorization procedure will be added to the relevant measurement according to the failure cause. Possible failure causes are included in RFC 3588 [15]. The sum of all supported per cause measurements shall equal the total number of failed resource authorization procedures. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes plus a possible sum value identified by the .sum suffix.

- The measurement name has the form QoS.FailAAA.Cause
where Cause identifies the failure cause.

- PcrfFunction.

- Valid for packet switched traffic.

- IMS.

### 4.8.2	Resource reservation related measurements

#### 4.8.2.1	Void

#### 4.8.2.2	Attempted resource reservation procedures

- This measurement provides the number of attempted resource reservation procedures initiated by GGSN to PDF.

- CC.

- On receipt by the PDF of COPS REQ message from GGSN (see 3GPP TS 29.207[13]).

- A single integer value.

- QoS.AttResRsv.

- PdfFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.8.2.3	Successful resource reservation procedures

- This measurement provides the number of successful resource reservation procedures initiated by GGSN to PDF.

- CC.

- On receipt by the PDF of a COPS RPT message from GGSN indicating a successful resource reservation procedure (see 3GPP TS 29.207[13]).

- A single integer value.

- QoS.SuccResRsv.

- PdfFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.8.2.4	Failed resource reservation procedures by PDF

- This measurement provides the number of resource reservation failures caused by the authorization failure in PDF. This measurement is split into subcounters per failure cause.

- CC.

- On transmission by the PDF of a COPS DEC message containing both an INSTALL and a REMOVE decisions to GGSN (see 3GPP TS 29.207[13]).

- Three integer values.

- QoS.FailResRsvPdf.1	Failure cause "NoCorrespondingSession".

- QoS.FailResRsvPdf.2	Failure cause "InvalidBundling".

QoS.FailResRsvPdf.3	Failure cause "AuthorisationFailure".

- PdfFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.8.2.5	Failed resource reservation procedures by GGSN

- This measurement provides the number of resource reservation failures caused by the enforcement failure in GGSN.

- CC.

- On receipt by the PDF of a COPS RPT message with a failure result from GGSN, corresponding to the transmission of the COPS DEC message containing only an INSTALL decision to GGSN (see 3GPP TS 29.207 [13]).

- A single integer value.

- QoS.FailResRsvGgsn.

- PdfFunction.

- Valid for packet switched traffic.

- IMS.

### 4.8.3	IP-CAN session modification related measurements

The measurements defined in subclauses 4.8.3.1, 4.8.3.2 and 4.8.3.3 are subject to the "2 out of 3 approach".

#### 4.8.3.1	Attempted IP-CAN session modification

- This measurement provides the number of attempted IP-CAN session modifications procedures initiated by UE.

- CC.

- On receipt by the PCRF of a Credit-Control-Request(CCR) Diameter command from PCEF to modify an existing  IP-CAN session initiated by UE (PCC Rule Provisioning in PULL Mode) (see 3GPP TS 29.213 [33], 3GPP TS 29.212[32]).

- A single integer value.

- QoS.ModfAttCCR.

- PcrfFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.8.3.2	Successful IP-CAN session modification

- This measurement provides the number of successful IP-CAN session modifications procedures initiated by UE.

- CC.

- On transmission by the PCRF of a Credit-Control-Answer(CCA) Diameter command to PCEF with an existing session ID and "2xxx" result-code in the UE-Initiated IP-CAN session modification procedures(PCC Rule Provisioning in PULL Mode) (see 3GPP TS 29.212[32], 3GPP TS 29.213[33], RFC 3588 [15]).

- A single integer value.

- QoS.ModfSuccCCA.

- PcrfFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.8.3.3	Failed IP-CAN session modification

- This measurement provides the number of failed IP-CAN session modifications procedures initiated by UE. The measurement is split into subcounters per the reject cause.

- CC.

- On transmission by the PCRF of a Credit-Control-Answer(CCA) Diameter command to PCEF with non-2xxx result-code in the UE-Initiated IP-CAN session modification procedures (PCC Rule Provisioning in PULL Mode) (see 3GPP TS 29.212 [32], 3GPP TS 29.213[33], RFC 3588 [15]). Each failed IP-CAN session modification will be added to the relevant measurement according to the failure cause. Possible failure causes are included in RFC 3588 [15]. The sum of all supported per cause measurements shall equal the total number of failed resource authorization procedures. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes supported plus a possible sum value identified by the .sum suffix.

- QoS.ModfFailCCA.Cause

- where Cause identifies the failure cause.

- PcrfFunction.

- Valid for packet switched traffic.

- IMS.

### 4.8.3	PDP context modification related measurements

The measurements defined in subclauses 4.8.3.1, 4.8.3.2 and 4.8.3.3 are subject to the "2 out of 3 approach".

#### 4.8.3.1	Attempted authorization of PDP context modification

- This measurement provides the number of attempted authorizations of PDP context modifications initiated by GGSN.

- CC.

- On receipt by the PDF of a COPS REQ message from GGSN to authorize a PDP context modification request (see 3GPP TS 29.207 [13]).

- A single integer value.

- QoS.AttPdpContextModfAuth.

- PdfFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.8.3.2	Successful authorization of PDP context modification

- This measurement provides the number of successful authorizations of PDP context modifications initiated by GGSN.

- CC.

- On transmission by the PDF of a COPS DEC message containing only an INSTALL decision to GGSN, in response to a PDP context modification request from GGSN, indicating a successful authorization of PDP context modification (see 3GPP TS 29.207 [13]).

- A single integer value.

- QoS.SuccPdpContextModfAuth.

- PdfFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.8.3.3	Failed authorization of PDP context modification

- This measurement provides the number of failed authorizations of PDP context modifications initiated by GGSN.

- CC.

- On transmission by the PDF of a COPS DEC message containing an INSTALL decision and a REMOVE decision to GGSN, corresponding to the receipt of the PDP context modification request from GGSN (see 3GPP TS 29.207 [13]).

- A single integer value.

- QoS.FailPDPContextModfAuth.

- PdfFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.8.3.4	Indication of PDP context modification

- This measurement provides the number of PDP context modifications to/from 0kbps initiated by GGSN.

- CC.

- On receipt by the PDF of a COPS RPT message indicating the PDP context modification to/from 0kbps from GGSN (see 3GPP TS 29.207 [13]).

- A single integer value.

- QoS.IndPdpContextModf.

- PdfFunction.

- Valid for packet switched traffic.

- IMS.

### 4.8.5	Gateway Control session establishment related measurements

The measurements defined in subclauses 4.8.5.1, 4.8.5.2 and 4.8.5.3 are subject to the "2 out of 3 approach".

#### 4.8.5.1	Attempted Gateway Control session establishment

- This measurement provides the number of attempted session establishments initiated by BBERF to PCRF.

- CC.

- For the non-roaming case, On receipt by the H-PCRF of a Credit-Control-Request(CCR) Diameter command from BBERF; For the roaming case, On receipt by the V-PCRF of a Credit-Control-Request(CCR) Diameter command from BBERF (see 3GPP TS 29.213 [33]).

- A single integer value.

- QoS.EstabfGCAttCCR.

- PcrfFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.8.5.2	Successful Gateway Control session establishment

- This measurement provides the number of successful session establishments initiated by BBERF to PCRF.

- CC.

- For the non-roaming case, On transmission by the H-PCRF of an CCA Diameter command to BBERF; For the roaming case, On transmission by the H-PCRF of an CCA Diameter command to V-PCRF or by the V-PCRF of an CCA Diameter command to BBERF. (see 3GPP TS 29.213[33]).

- A single integer value.

- QoS.EstabfGCSuccCCA.

- PcrfFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.8.5.3	Failed Gateway Control session establishment

- This measurement provides the number of failed session establishments initiated by BBERF to PCRF.

- CC.

- On transmission by the PCRF of a Credit-Control-Answer (CCA) Diameter command to BBERF in the Gateway Control session establishments procedures (3GPP TS 29.213[33]). Each failed Gateway Control session establishments will be added to the relevant measurement according to the failure cause. Possible failure causes are included in RFC 3588 [15]. The sum of all supported per cause measurements shall equal the total number of failed resource authorization procedures. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes supported plus a possible sum value identified by the .sum suffix.

- QoS.EstabfGCFailCCA.Cause

- where Cause identifies the failure cause.

- PcrfFunction.

- Valid for packet switched traffic.

- IMS.

## 4.9	Measurements related to SLF

### 4.9.1	All routing information interrogations related measurements

#### 4.9.1.1	Attempted routing information interrogation procedures

- This measurement provides the number of all routing information interrogation requests received by the SLF via all diameter application interfaces. The diameter interfaces can be Dx interface, Dh interface or Dw interface.

- CC.

- On receipt by the SLF of a routing information interrogation request diameter message from all diameter interfaces (see 3GPP TS 23.228 [7] and RFC 3588[15]).

- A single integer value.

- RII.AttAll.

- SlfFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.9.1.2	Successful routing information interrogation procedures

- This measurement provides the number of all successful routing information interrogation responses sent by the SLF via all diameter application interfaces. The diameter interfaces can be Dx interface, Dh interface or Dw interface.

- CC.

- On transmission by the SLF of a routing information interrogation response diameter message with the Result-Code AVP set to DIAMETER_REDIRECT_INDICATION via all diameter interfaces, indicating a successful routing information interrogation (see 3GPP TS 23.228 [7] and RFC 3588 [15]).

- A single integer value.

- RII.SuccAll

- SlfFunction.

- Valid for packet switched traffic.

- IMS.

### 4.9.2	Routing information interrogation from CSCF related measurements

#### 4.9.2.1	Overview

The performance measurements defined in subclause 4.9.2 are related to routing information interrogations at the SLF from CSCF.

Figure 18: Routing information interrogation procedure from CSCF

#### 4.9.2.2	Attempted routing information interrogation procedures from CSCF

- This measurement provides the number of routing information interrogation requests received from CSCF by the SLF.

- CC.

- On receipt by the SLF of a DX_SLF_QUERY message from CSCF via Dx interface (see 3GPP TS 23.228 [7]).

- A single integer value.

- RII.AttFromCSCF.

- SlfFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.9.2.3	Successful routing information interrogation procedures from CSCF

- This measurement provides the number of successful routing information interrogation responses sent to CSCF by the SLF.

- CC.

- On transmission by the SLF of a DX_SLF_RESP message with the Result-Code AVP set to DIAMETER_REDIRECT_INDICATION to CSCF via Dx interface (see 3GPP TS 23.228 [7] and RFC 3588[15]).

- A single integer value.

- RII.SuccFromCSCF.

- SlfFunction.

- Valid for packet switched traffic.

- IMS.

### 4.9.3	Routing information interrogation from AS related measurements

#### 4.9.3.1	Overview

The performance measurements defined in subclause 4.9.3 are related to routing information interrogations at the SLF from AS.

Figure 19: Routing information interrogation procedure from AS

#### 4.9.3.2	Attempted routing information interrogation procedures from AS

- This measurement provides the number of routing information interrogations requests received from AS by the SLF.

- CC.

- On receipt by the SLF of a DH_SLF_QUERY message from AS via Dh interface (see 3GPP TS 23.228 [7]).

- A single integer value.

- RII.AttFromAS.

- SlfFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.9.3.3	Successful routing information interrogation procedures from AS

- This measurement provides the number of successful routing information interrogation responses sent to AS by the SLF.

- CC.

- On transmission by the SLF of a DH_SLF_RESP message with the Result-Code AVP set to DIAMETER_REDIRECT_INDICATION to AS via Dh interface (see 3GPP TS 23.228 [7] and RFC 3588[15]).

- A single integer value.

- RII.SuccFromAS.

- SlfFunction.

- Valid for packet switched traffic.

- IMS.

## 4.10	Measurements related to AS

### 4.10.1	Session control related measurements

#### 4.10.1.1	IM CN originated session establishments

For the IM CN originated session establishments AS may act as terminating UA, redirect server, a SIP proxy, routeing B2BUA or initiating B2BUA. See 3GPP TS 24.229 [8] or 3GPP TS 23.228 [7]. The measurements defined in subclauses 4.10.1.1.1, 4.10.1.1.2 and 4.10.1.1.4 are subject to the "2 out of 3 approach".

##### 4.10.1.1.1	Attempted IM CN originated session establishments

- This measurement provides the number of attempted session establishments originated from the IM CN.

- CC.

- On receipt by the AS of an initial SIP_INVITE message from the IM CN (see 3GPP TS 23.228 [7]).

- A single integer value.

- SC.AttSessionImOrig.

- ASFunction.

- Valid for packet switched traffic.

- IMS.

##### 4.10.1.1.2	Successful IM CN originated session establishments

- This measurement provides the number of successful session establishments originated from the IM CN.

- CC.

- On transmission by the AS of the first response message (e.g. SIP_180, SIP_200 or SIP_302 etc. relating with the role of the AS) indicating a successful IM CN originated session establishment corresponding to the initial SIP_INVITE message, each successful session establishment is added to the relevant measurement according to the message type, the possible response message types are identified in 3GPP TS 24.229 [8]. The sum of all supported per message type measurements shall equal the total number of successful IM CN originated session establishments.

- Each measurement is an integer value. The number of measurements is equal to the number of message types plus a possible sum value identified by the .sum suffix.

- SC.SuccSessionImOrig.type
where type identifies the message type.

- The following subcounters with specific message types are the samples of this measurement:
SC.SuccSession.180	Message type: SIP_180
SC.SuccSession.200	Message type: SIP_200_OK only if the SIP_180 has not been sent for this session.

- ASFunction.

- Valid for packet switched traffic.

- IMS.

##### 4.10.1.1.3	Answered IM CN originated session establishments

- This measurement provides the number of answered session establishments originated from the IM CN.

- CC.

- On transmission by the AS of a SIP_200_OK response message indicating an answered IM CN originated session establishment corresponding to the initial SIP_INVITE messages (see 3GPP TS 23.228 [7]).

- A single integer value.

- SC.AnsSessionImOrig.

- ASFunction.

- Valid for packet switched traffic

- IMS.

##### 4.10.1.1.4	Failed IM CN originated session establishments

- This measurement provides the number of failed session establishments originated from the IM CN. The measurement is split into subcounters per response status codes.

- CC.

- On transmission by the AS of a response message (e.g. SIP_430) indicating a failed session establishment originated from the IM CN corresponding to the initial SIP_INVITE messages (see 3GPP TS 23.228 [7]), each failed session establishment originated from the IM CN is added to the relevant measurement according to the status code. The sum of all supported per status code measurements shall equal the total number of failed session establishments. In case only a subset of per cause measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of causes plus a possible sum value identified by the .sum suffix.

- The measurement name has the form SC.FailSessionImOrig.Cause
where Cause identifies the status code.  (see 3GPP TS 24.229 [8] and RFC3261 [34])

- ASFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.10.1.2	AS originated session establishments

For the AS originated session establishments, AS may act as originating UA, routeing B2BUA or initiating B2BUA. See 3GPP TS 24.229 [8] or 3GPP TS 23.228[7]. The measurements defined in subclauses 4.10.1.2.1, 4.10.1.2.2 and 4.10.1.2.4 are subject to the "2 out of 3 approach".

##### 4.10.1.2.1	Attempted AS originated session establishments

- This measurement provides the number of attempted session establishments originated from the AS.

- CC.

- On transmission by the AS of an initial SIP_INVITE message indicating the AS originated session establishment, each initial SIP_INVITE message originated by the AS shall be counted as one attempted AS originated session establishment.
If the AS acts as originating UA, it initiates a session by sending a SIP_INVITE message.
If the AS acts as routeing B2BUA, it terminates the request from IM CN and initiates a new session by sending a SIP_INVITE message.
If the AS acts as initiating B2BUA, it may initiates two new sessions by sending two SIP_INVITE messages, or initiates one new session in the case of receiving the request from the IM CN and the new session is logically connected but unrelated to the incoming request (see 3GPP TS 24.229 [8]).

- A single integer value.

- SC.AttSessionAsOrig.

- ASFunction.

- Valid for packet switched traffic.

- IMS.

##### 4.10.1.2.2	Successful AS originated session establishments

- This measurement provides the number of successful session establishments originated from the AS.

- CC.

- On receipt of the first response message (e.g. SIP_180, SIP_200, etc.) indicating a successful AS originated session establishment corresponding to the initial SIP_INVITE message originated by the AS, each successful session establishment is added to the relevant measurement according to the message type, the possible response message types are identified in 3GPP TS 24.229 [8]. The sum of all supported per message type measurements shall equal the total number of successful AS originated session establishments.

- Each measurement is an integer value. The number of measurements is equal to the number of message types plus a possible sum value identified by the .sum suffix.

- SC.SuccSessionAsOrig.type
where type identifies the message type.

- The following subcounters with specific message types are the samples of this measurement:
SC.SuccSession.180	Message type: SIP_180
SC.SuccSession.200	Message type: SIP_200_OK only if the SIP_180 has not been received for this session.

- AsFunction.

- Valid for packet switched traffic.

- IMS.

### 4.10.2	Messaging service related measurements

#### 4.10.2.1	Message related measurements, Page-Mode

##### 4.10.2.1.1	Attempted incoming messages, Page-Mode

  - This measurement provides number of attempted incoming messages received by the messaging enabler with page-mode at the AS.

  - CC.

  - On receipt by the AS (messaging enabler) of an initial SIP_MESSAGE message (see 3GPP TS 23.228 [7]).

  - A single integer value.

  - MSG.AttIncMessagePM.

  - AsFunction.

  - Valid for packet switched traffic.

  - IMS.

##### 4.10.2.1.2	Successful incoming messages, Page-Mode

- This measurement provides number of successful incoming messages with page-mode at the AS.

- CC.

- On transmission by the AS (messaging enabler) of a response SIP_2XX message (see 3GPP TS 24.229 [8]) indicating the incoming message is successfully forwarded.

- A single integer value.

- MSG.SuccIncMessagePM.

- AsFunction.

- Valid for packet switched traffic.

- IMS.

##### 4.10.2.1.3	Attempted outgoing messages, Page-Mode

- This measurement provides number of attempted outgoing messages transmitted by the messaging enabler with page-mode at the AS.

- CC.

- On transmission by the AS (messaging enabler) of a SIP_MESSAGE message (see 3GPP TS 23.228 [7]).

- A single integer value.

- MSG.AttOutMessagePM.

- AsFunction.

- Valid for packet switched traffic.

- IMS.

##### 4.10.2.1.4	Successful outgoing messages, Page-Mode

- This measurement provides number of successful outgoing messages transmitted by the messaging enabler with page-mode at the AS.

- CC.

- On receipt by the AS (messaging enabler) of a response SIP_2XX message (see 3GPP TS 24.229 [8]) indicating the message transmitted by the AS is successfully processed.

- A single integer value.

- MSG.SuccOutMessagePM.

- AsFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.10.2.2	Message related measurements, Session-Mode

##### 4.10.2.2.1	Overview

The performance measurements defined in this subclause are related to message sending with session mode. The following procedure shows the session based messaging with an intermediate AS node. See 3GPP TS 23.228 [7].

Figure 20: Session based messaging with an intermediate AS node scenario

##### 4.10.2.2.2	Attempted messaging session establishments, Session Mode

- This measurement provides number of attempted messaging session establishments for the two participants received by the messaging enabler with session-mode at the AS.

- CC.

- On receipt by the AS (messaging enabler) of an initial SIP_INVITE message initiated by a participant which requests to establish sessions just for the two participants for the messaging service (see 3GPP TS 29.228 [9]).

- A single integer value.

- MSG.AttSesssionSM.

- AsFunction.

- Valid for packet switched traffic.

- IMS.

##### 4.10.2.2.3	Successful messaging session establishments, Session-Mode

- This measurement provides number of successful messaging session establishments by the messaging enabler for the two participants with session-mode at the AS.

- CC.

- On transmission by the AS (messaging enabler) of a SIP_200_OK message indicating a successful messaging session establishment for the two participants of Messaging Service (see 3GPP TS 29.228 [9]).

- A single integer value.

- MSG.SuccSessionSM.

- AsFunction.

- Valid for packet switched traffic.

- IMS.

### 4.10.3	Conference service related measurements

#### 4.10.3.1	Overview

The performance measurements defined in this subclause are related to the conference service (see 3GPP TS 24.147 [24]).

The following procedures are of interest for this purpose:

-	User creating a conference;

-	User joining a conference;

-	AS inviting a user to a conference;

-	User subscribing to the conference event package.

Figure 21: User automatically creating a conference - MRFC/AS is located in user's home network

Figure 22: User automatically creating a conference - MRFC/AS is not located in user's home network

Figure 23: User calling into a conference - network MRFC/AS is not located in user's home network - conference URI resolved by the terminating home network

Figure 24: User calling into a conference - MRFC/AS is not located in user's home network - conference URI can be resolved by the originating home network

Figure 25: User inviting another user to a conference by sending a REFER request to MRFC/AS

Figure 26: MRFC/AS inviting a user to a conference - MRFC/AS routes directly to I-CSCF

Figure 27: MRFC/AS inviting another user to a conference by sending a REFER request to UE#2

Figure 28: User subscribing to conference event package -
MRFC/AS is not located in user's home network

#### 4.10.3.2	Attempted creations of conference

- This measurement provides the number of attempted creations of conference.

- CC.

- On receipt by the Conference AS of a SIP_INVITE message to create the conference (see 3GPP TS 24.147 [24]).

- A single integer value.

- CONF.AttCreation.

- AsFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.10.3.3	Successful creations of conference

- This measurement provides the number of successful creations of conference.

- CC.

- On transmission by the Conference AS of a SIP_200_OK message indicating the conference is successfully created (see 3GPP TS 24.147 [24]).

- A single integer value.

- CONF.SuccCreation.

- AsFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.10.3.4	Attempted joining of conference

- This measurement provides the number of attempted joining of conference.

- CC.

- On receipt by the Conference AS of a SIP_INVITE message indicating the requests by the user to join the conference (see 3GPP TS 24.147 [24]).

- A single integer value.

- CONF.AttJoining.

- AsFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.10.3.5	Successful joining of conference

- This measurement provides the number of successful joining of conference.

- CC.

- On transmission by the Conference AS of a SIP_200_OK message indicating the user has successfully joined the conference (see 3GPP TS 24.147 [24]).

- A single integer value.

- CONF.SuccJoining.

- AsFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.10.3.6	Attempted invitations for another user to join conference

- This measurement provides the number of attempted invitations for another user to join conference.

- CC.

- On transmission by the Conference AS of a SIP_INVITE message to invite a user to join a conference (see 3GPP TS 24.147 [24]).

- A single integer value.

- CONF.AttInvitation.

- AsFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.10.3.7	Successful invitations for another user to join conference

- This measurement provides the number of successful invitations for another user to join conference.

- CC.

- On receipt by the Conference AS of a response message (e.g. SIP_183 or SIP_200_OK) indicating the invited user has successfully joined the conference (see 3GPP TS 24.147 [24]), each successful invitation is added to the relevant measurement according to the message type, the possible response message types are identified in 3GPP TS 24.229 [8]. The sum of all supported per message type measurements shall equal the total number of successful invitations. In case only a subset of per message type measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of message types plus a possible sum value identified by the .sum suffix.

- CONF.SuccInviJoinConf.type
where type identifies the message type.

- The following subcounters with specific message types are the samples of this measurement:
CONF.SuccInvitation.183	Message type: SIP_183
CONF.SuccInvitation.200	Message type: SIP_200_OK only if the SIP_183 has not been received for this 	session.

- AsFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.10.3.8	Attempted subscriptions for the conference event

- This measurement provides the number of attempted subscriptions for the conference event.

- CC.

- On receipt by the Conference AS of SIP_SUBSCRIBE message initiated by the conference participant (see 3GPP TS 24.147 [24]).

- A single integer value.

- CONF.AttSubscription

- AsFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.10.3.9	Successful subscriptions for the conference event

- This measurement provides the number of successful subscriptions for the conference event.

- CC.

- On transmission by the AS of a SIP_200_OK message indicating the successful subscription for the conference event, corresponding the SIP_SUBCRIBE request message initiated by the conference participant (see 3GPP TS 24.147 [24]).

- A single integer value.

- CONF.SuccSubscription.

- AsFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.10.3.10	Number of simultaneous online users (Maximum)

- This measurement provides the maximum number of simultaneous online conference users.

- SI.

- This measurement is obtained by sampling at a pre-defined interval the number of simultaneous online users on the conference and then taking the maximum.

- A single integer value.

- CONF.OnlineUserMax.

- AsFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.10.3.11	Number of simultaneous online conferences (Maximum)

- This measurement provides the maximum number of simultaneous online conferences.

- SI.

- This measurement is obtained by sampling at a pre-defined interval the number of simultaneous online conferences and then taking the maximum.

- A single integer value.

- CONF.OnlineConfMax.

- AsFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.10.3.12	Number of simultaneous online users (Mean)

- This measurement provides the mean number of simultaneous online users.

- SI.

- This measurement is obtained by sampling at a pre-defined interval the number of simultaneous online users on the conference and then taking the arithmetic mean.

- A single integer value.

- CONF.OnlineUserMean.

- AsFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.10.3.13	Number of simultaneous online conferences (Mean)

- This measurement provides the mean number of simultaneous online conferences.

- SI.

- This measurement is obtained by sampling at a pre-defined interval the number of simultaneous online conferences and then taking the arithmetic mean.

- A single integer value.

- CONF.OnlineConfMean.

- AsFunction.

- Valid for packet switched traffic.

- IMS.

### 4.10.4	Presence service related measurements

#### 4.10.4.1	Number of simultaneous online watchers (Maximum)

- This measurement provides the maximum number of simultaneous online watchers at the presence enabler/AS.

- SI.

- This measurement is obtained by sampling at a pre-defined interval the number of simultaneous online watchers which have been registered and then taking the maximum.

- A single integer value.

- PRES.OnlineWatcherMax.

- AsFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.10.4.2	Number of simultaneous online presentities (Maximum)

- This measurement provides the maximum number of simultaneous online presentities in the presence enabler/AS.

- SI.

- This measurement is obtained by sampling at a pre-defined interval the number of simultaneous online presentities which have been registered and then taking the maximum.

- A single integer value.

- PRES.OnlinePresentityMax.

- AsFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.10.4.3	Number of simultaneous online watchers (Mean)

- This measurement provides the number of mean number of simultaneous online watchers in the presence enabler/AS.

- SI.

- This measurement is obtained by sampling at a pre-defined interval the number of simultaneous online watchers which have been registered and then taking the arithmetic mean.

- A single integer value.

- PRES.MeanOnlineWatcher.

- AsFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.10.4.4	Number of simultaneous online presentities (Mean)

- This measurement provides the number of mean number of simultaneous online presentities in the presence enabler/AS.

- SI.

- This measurement is obtained by sampling at a pre-defined interval the number of simultaneous online presentities which have been registered and then taking the arithmetic mean.

- A single integer value.

- PRES.MeanOnlinePresentity.

- AsFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.10.4.5	Attempted subscriptions for presence

- This measurement provides the number of attempted subscriptions received by the presence enabler/AS.

- CC.

- On receipt by the AS (presence enabler) of a SIP_SUBSCRIBE message (see 3GPP TS 23.141[25]).

- A single integer value.

- PRES.AttSubscribe.

- AsFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.10.4.6	Successful subscriptions for presence

- This measurement provides the number of successful subscriptions at the presence enabler/AS.

- CC.

- On transmission by the AS (presence enabler) of a response message (e.g. SIP_200_OK or SIP_202_Accepted) for the subscription request from the IM CN (See 3GPP TS 23.141 [25]), each successful subscription is added to the relevant measurement according to the message type, the possible response message types are identified in 3GPP TS 24.229 [8]. The sum of all supported per message type measurements shall equal the total number of successful subscriptions. In case only a subset of per message type measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of message types plus a possible sum value identified by the .sum suffix.

- PRES.SuccSubscribe. type
where type identifies the message type.

- The following subcounters with specific message types are the samples of this measurement:
SC.SuccSession.200	Message type: SIP_200_OK
SC.SuccSession.202	Message type: SIP_202 only if the SIP_200_OK has not been sent for this session.

- AsFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.10.4.7	Attempted notifications for presence

- This measurement provides the number of attempted notifications sent by the presence enabler/AS.

- CC.

- On transmission by the AS (presence enabler) of a SIP_NOTIFY message to the IM CN (see 3GPP TS 23.141 [25]).

- A single integer value.

- PRES.AttNotify.

- AsFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.10.4.8	Successful notifications for presence

- This measurement provides the number of successful notifications send by the presence enabler/AS.

- CC.

- On receipt by the AS (presence enabler) of a SIP_200_OK message indicating the successful notification corresponding to the SIP_NOTIFY message sent by the AS (see 3GPP TS 23.141 [25]).

- A single integer value.

- PRES.SuccNotify.

- AsFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.10.4.9	Attempted publications for presence

- This measurement provides the number of attempted publications received by the presence enabler/AS.

- CC.

- On receipt by the AS (presence enabler) of a SIP_PUBLISH message (see 3GPP TS 23.141 [25]).

- A single integer value.

- PRES.AttPublish.

- AsFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.10.4.10	Successful publications for presence

- This measurement provides the number of successful publications at the presence enabler/AS.

- CC.

- On transmission by the AS (presence enabler) of a SIP_200_OK message indicating the successful publication, corresponding to received SIP_PUBLISH message (see 3GPP TS 23.141 [25]).

- A single integer value.

- PRES.SuccPub.

- ASFunction.

- Valid for packet switched traffic.

- IMS.

### 4.10.5	PoC service related measurements

#### 4.10.5.1	Attempted creations of PoC session

- This measurement provides the number of attempted creations of PoC session.

- CC.

- On receipt by the PoC AS of SIP_INVITE message for creation of PoC session (see 3GPP TR 23.979 [26]).

- A single integer value.

- PoC.AttCreation.

- AsFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.10.5.2	Successful creations of PoC session

- This measurement provides the number of successful creation ofPoC session.

- CC.

- On transmission by the PoC AS of a SIP_200_OK message indicating the PoC session is successfully created (see 3GPP TR 23.979 [26]).

- A single integer value.

- PoC.SuccCreation.

- AsFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.10.5.3	Attempted joining of PoC session

- This measurement provides the number of attempted joining PoC session requests which means that the user calls into the PoC session.

- CC.

- On receipt by the PoC AS of PoC session SIP_INVITE message indicating the requests by the user to join the PoC session (see 3GPP TR 23.979 [26]).

- A single integer value.

- PoC.AttJoining.

- AsFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.10.5.4	Successful joining of PoC session

- This measurement provides the number of successful joining of PoC session.

- CC.

- On transmission by the PoC AS of a SIP_200_OK message indicating the user has successfully joined the PoC session (see 3GPP TR 23.979 [26]).

- A single integer value.

- PoC.SuccJoining.

- AsFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.10.5.5	Attempted invitations for another user to join PoC session

- This measurement provides the number of attempted invitations for another user to join PoC session.

- CC.

- On transmission by the PoC AS of a SIP_INVITE message to invite a user to join a PoC session (see 3GPP TR 23.979 [26]).

- A single integer value.

- PoC.AttInvitation.

- AsFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.10.5.6	Successful invitations for another user to join PoC session

- This measurement provides the number of successful invitations for another user to join PoC session.

- CC.

- On receipt by the PoC AS of a SIP_200_OK message indicating the invited user has successfully joined the PoC session (see 3GPP TR 23.979 [26]).

- A single integer value.

- PoC.SuccInvitation.

- AsFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.10.5.7	Number of simultaneous online users (Maximum)

- This measurement provides the maximum number of simultaneous online PoC users.

- SI.

- This measurement is obtained by sampling at a pre-defined interval the number of simultaneous online users that have been joined and then taking the maximum.

- A single integer value.

- PoC.OnlineUserMax.

- AsFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.10.5.8	Number of simultaneous online PoC session (Maximum)

- This measurement provides the maximum number of simultaneous online PoC sessions.

- SI.

- This measurement is obtained by sampling at a pre-defined interval the number of simultaneous online PoC sessions and then taking the maximum.

- A single integer value.

- PoC.OnlineSessionMax.

- AsFunction.

- Valid for packet switched traffic.

- IMS.

### 4.10.6	Measurements related to XDM Enabler

#### 4.10.6.1	XCAP operation related measurements

##### 4.10.6.1.1	Overview

The performance measurements defined in this subclause are related to the XCAP operation request from the XDMC for document manipulation (e.g. retrieving/adding/deleting elements/attributes, etc.) stored in XDMS. See "XML Document Management (XDM) Specification", OMA-TS-XDM_Core-V1_0 [27].

The procedures are shown in figure 29.

Figure 29: XCAP operation request procedures

##### 4.10.6.1.2	Attempted HTTP PUT procedures

- This measurement provides the number of attempted HTTP PUT procedures.

- CC.

- On receipt by the XDMS of a HTTP_PUT request from the aggregation proxy (see OMA-TS-XDM_Core-V1_0 [27]).

- A single integer value.

- XDM.AttHttpPut.

- AsFunction.

- Valid for packet switched traffic.

- IMS.

##### 4.10.6.1.3	Successful HTTP PUT procedures

- This measurement provides the number of successful HTTP PUT procedures.

- CC.

- On transmission by the XDMS of a response message (e.g. HTTP_200_OK) indicating a successful HTTP PUT procedure, each successful HTTP PUT procedure is added to the relevant measurement according to the message type, the possible response message types are identified in 3GPP TS 24.229 [8]. The sum of all supported per message type measurements shall equal the total number of successful HTTP PUT procedures. In case only a subset of per message type measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of message types plus a possible sum value identified by the .sum suffix.

- XDM.SuccHttpPut.type
where type identifies the message type.

- The following subcounters with specific message types are the samples of this measurement:
XDM.SuccHttpPut.200	Message type: SIP_200_OK.

- AsFunction.

- Valid for packet switched traffic.

- IMS.

##### 4.10.6.1.4	Attempted HTTP GET procedures

- This measurement provides the number of attempted HTTP GET procedures.

- CC.

- On receipt by the XDMS of a HTTP_GET request from the aggregation proxy (See OMA-TS-XDM_Core-V1_0 [27]).

- A single integer value.

- XDM.AttHttpGet.

- AsFunction.

- Valid for packet switched traffic.

- IMS.

##### 4.10.6.1.5	Successful HTTP GET procedures

- This measurement provides the number of successful HTTP GET procedures.

- CC.

- On transmission by the XDMS of a response message (e.g. HTTP_200_OK) indicating a successful HTTP GET procedure, each successful HTTP GET procedure is added to the relevant measurement according to the message type, the possible response message types are identified in 3GPP TS 24.229 [8]. The sum of all supported per message type measurements shall equal the total number of successful HTTP GET procedures. In case only a subset of per message type measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of message types plus a possible sum value identified by the .sum suffix.

- XDM.SuccHttpGet.type
where type identifies the message type.

- The following subcounters with specific message types are the samples of this measurement:
XDM.SuccHttpGet.200	Message type: SIP_200_OK.

- AsFunction.

- Valid for packet switched traffic.

- IMS.

##### 4.10.6.1.6	Attempted HTTP DELETE procedures

- This measurement provides the number of attempted HTTP DELETE procedures.

- CC.

- On receipt by the XDMS of a HTTP_ DELETE request from the aggregation proxy (see OMA-TS-XDM_Core-V1_0 [27]).

- A single integer value.

- XDM.AttHttpDel.

- AsFunction.

- Valid for packet switched traffic.

- IMS.

##### 4.10.6.1.7	Successful HTTP DELETE procedures

- This measurement provides the number of successful HTTP DELETE procedures.

- CC.

- On transmission by the XDMS of a response message (e.g. HTTP_200_OK) indicating a successful HTTP DELETE procedure, each successful HTTP DELETE procedure is added to the relevant measurement according to the message type, the possible response message types are identified in 3GPP TS 24.229 [8]. The sum of all supported per message type measurements shall equal the total number of successful HTTP DELETE procedures. In case only a subset of per message type measurements is supported, a sum subcounter will be provided first.

- Each measurement is an integer value. The number of measurements is equal to the number of message types plus a possible sum value identified by the .sum suffix.

- XDM.SuccHttpDel.type
where type identifies the message type.

- The following subcounters with specific message types are the samples of this measurement:
XDM.SuccHttpDel.200	Message type: SIP_200_OK.

- AsFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.10.6.2	SIP related measurements

##### 4.10.6.2.1	Overview

- The performance measurements defined in this subclause are related to SIP procedures for subscribing and notifying the changes in the XML documents stored in the XDMS. See "XML Document Management (XDM) Specification", OMA-TS-XDM_Core-V1_0 [27].

The procedures are shown in figure 30.

Figure 30: XDM Client subscribes to changes in XDMS Procedures

##### 4.10.6.2.2	Attempted Subscription procedures

- This measurement provides the number of attempted subscription procedures.

- CC.

- On receipt by the XDMS of a SIP_SUBSCRIBE message (See OMA-TS-XDM_Core-V1_0 [27]).

- A single integer value.

- XDM.AttSubscibe.

- AsFunction.

- Valid for packet switched traffic.

- IMS.

##### 4.10.6.2.3	Successful Subscription procedures

- This measurement provides the number of successful subscription procedures.

- CC.

- On transmission by the XDMS of a response SIP_200_OK message indicating a successful subscription procedure (see OMA-TS-XDM_Core-V1_0 [27]).

- A single integer value.

- XDM.SuccSubscribe.

- AsFunction.

- Valid for packet switched traffic.

- IMS.

##### 4.10.6.2.4	Attempted Notification procedures

- This measurement provides the number of attempted notification procedures.

- CC.

- On sending by the XDMS of a SIP_NOTIFY message (see OMA-TS-XDM_Core-V1_0 [27]).

- A single integer value.

- XDM.AttNotify.

- AsFunction.

- Valid for packet switched traffic.

- IMS.

##### 4.10.6.2.5	Successful Notification procedures

- This measurement provides the number of successful notification procedures.

- CC.

- On receipt by the XDMS of a response SIP_200_OK message indicating the successful XDMS originated notification procedures.

- A single integer value.

- XDM.SuccNotify.

- AsFunction.

- Valid for packet switched traffic.

- IMS.

### 4.10.7	IMS supplementary services related measurements

#### 4.10.7.1  Communication Diversion (CDIV) service establishments

##### 4.10.7.1.1  Attempted Communication Diversion (CDIV) service procedure

- This measurement provides the number of attempted Communication Diversion (CDIV) service procedure.

- CC.

- On receipt by the AS of a SIP_INVITE message to create the Communication Diversion (CDIV) service (see 3GPP TS 24.604 [36]).

- Each measurement is an integer value. The number of measurements is equal to the number of services plus a possible sum value identified by the .sum suffix.

- The measurement name has the form SC.AttCdiv.Subservice
where Subservice identifies the specific service. The following subcounters with specific services(see 3GPP TS 24.604 [36]) are required:

-	SC.AttCdiv.Cfu	Communication Forwarding Unconditional (CFU) service;

- -	SC.AttCdiv.Cfb	                Communication Forwarding on Busy user (CFB) service;

- -	SC.AttCdiv.Cfnr	                Communication Forwarding on no Reply (CFNR) service;

- -	SC.AttCdiv.Cfnrc	           Communication Forwarding on Subscriber Not Reachable (CFNRc) service;

- ASFunction.

- Valid for packet switched traffic.

- IMS.

##### 4.10.7.1.2  Successful Communication Diversion (CDIV) service procedure

- This measurement provides the number of successful Communication Diversion (CDIV) service procedure.

- CC.

- On transmission by the AS of a SIP_200_OK message indicating the Communication Diversion (CDIV) service is successfully created (see 3GPP TS 24.604 [36]).

- Each measurement is an integer value. The number of measurements is equal to the number of services plus a possible sum value identified by the .sum suffix.

- The measurement name has the form SC.SuccCdiv.Subservice
where Subservice identifies the specific service. The following subcounters with specific services(see 3GPP TS 24.604 [36]) are required:

-	SC.SuccCdiv.Cfu	Communication Forwarding Unconditional (CFU) service;

- -	SC.SuccCdiv.Cfb	           Communication Forwarding on Busy user (CFB) service;

- -	SC.SuccCdiv.Cfnr	           Communication Forwarding on no Reply (CFNR) service;

- -	SC.SuccCdiv.Cfnrc	           Communication Forwarding on Subscriber Not Reachable (CFNRc) service;

- ASFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.10.7.2	Originating Identification Presentation (OIP) service establishments

##### 4.10.7.2.1	Attempted Originating Identification Presentation (OIP) procedure

- This measurement provides the number of attempted Originating Identification Presentation (OIP) service procedure.

- CC.

- On receipt by the AS of a SIP_INVITE message to create the Originating Identification Presentation (OIP) service (see 3GPP TS 24.607 [37]).

- A single integer value.

- SC.AttOip.

- ASFunction.

- Valid for packet switched traffic.

- IMS.

##### 4.10.7.2.2	Successful Originating Identification Presentation (OIP) procedure

- This measurement provides the number of successful Originating Identification Presentation (OIP) service procedure.

- CC.

- On transmission by the AS of a SIP_200_OK message indicating the Originating Identification Presentation (OIP) service is successfully created (see 3GPP TS 24.607 [37]).

- A single integer value.

- SC.SuccOip.

- ASFunction.

- Valid for packet switched traffic.

- IMS.

### 4.11	Measurements related to IBCF

Editor's note: IbcfFunction used in following text is a subject for further study of IMS NRM modelling for R8 requirements and will align with IMS NRM defined in 3GPP TS 32.732 [30].

#### 4.11.1	Attempted session establishments from users of other network domains

- This measurement provides the number of attempted session establishments originated from the users (regardless of whether the user is roaming or not) who are belonging to other network domains at the IBCF in the terminating network. The measurement is split into subcounters per network domain.

- CC.

- On receipt by the IBCF of a SIP_ INVITE message, and the calling party is belonging to other network domains (i.e. the domain name information of the calling party retrieved from the SIP_INVITE message is different from the domain of this IBCF). Each session whose calling party recognized by the IBCF as belonging to a given network domain is added to the relevant per network domain measurement. The sum of all supported per network domain measurements shall equal the total number of attempted session establishments from users of other network domains. In case only a subset of per network measurements is supported, a sum subcounter will be provided first (see 3GPP TS 23.228 [7]).

- Each measurement is an integer value. The number of measurements is equal to the number of network domain plus a possible sum value identified by the .sum suffix.

- The measurement name has the form IDS.AttSessionFromOtherNtwkDmn.Domain
where Domain identifies the network domain, and it is in the format of the domain name information in the SIP messages.

- IbcfFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.11.2	403 (Forbidden) generated for sessions from users of other network domains

- This measurement provides the number of SIP_403 (Forbidden) response messages at the IBCF in the terminating network generated for the sessions from users of other network domains. The measurement is split into subcounters per network domain.

- CC.

- On transmission by the IBCF of SIP_403 message corresponding to the SIP_ INVITE message of which the calling party is belonging to other network domains received by the IBCF. Each SIP_403 message generated by the IBCF for the session whose calling party is belonging to a given network domain is added to the relevant per network domain measurement. The sum of all supported per network domain measurements shall equal the total number of 403 (Forbidden) generated for sessions from users of other network domains. In case only a subset of per network measurements is supported, a sum subcounter will be provided first (see 3GPP TS 23.228 [7]).

- Each measurement is an integer value. The number of measurements is equal to the number of network domain plus a possible sum value identified by the .sum suffix.

- The measurement name has the form IDS.403SessionFromOtherNtwkDmn.Domain
where Domain identifies the network domain, and it is in the format of the domain name information in the SIP messages.

- IbcfFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.11.3	Attempted session establishments to users of other network domains

- This measurement provides the number of attempted session establishments towards users (regardless of whether the user is roaming or not) who are belonging to other network domains at the IBCF in the originating network. The measurement is split into subcounters per network domain.

- CC.

- On transmission by the IBCF of a SIP_ INVITE messages, and the called party is belonging to other network domains (i.e. the domain name information of the called party retrieved from the SIP_INVITE message is different from the domain of this IBCF). Each session whose called party recognized by the IBCF as belonging to a given network domain is added to the relevant per network domain measurement. The sum of all supported per network measurements shall equal the total number of attempted session establishments to users of other network domains. In case only a subset of per network measurements is supported, a sum subcounter will be provided first (see 3GPP TS 23.228 [7]).

- Each measurement is an integer value. The number of measurements is equal to the number of network domain plus a possible sum value identified by the .sum suffix.

- The measurement name has the form IDS.AttSessionToOtherNtwkDmn.Domain
where Domain identifies the network domain, and it is in the format of the domain name information in the SIP messages.

- IbcfFunction.

- Valid for packet switched traffic.

- IMS.

#### 4.11.4	403 (Forbidden) received for sessions to users of other network domains

- This measurement provides the number of SIP_403 (Forbidden) response messages received for sessions to users of other network domains at the IBCF in the originating network. The measurement is split into subcounters per network domain.

- CC.

- On receipt by the IBCF of SIP_403 message corresponding to the SIP_ INVITE message of which the called party is belonging to other network domains transmitted by the IBCF. Each SIP_403 message received by the IBCF for the session whose called party is belonging to a given network domain is added to the relevant per network domain measurement. The sum of all supported per network domain measurements shall equal the total number of 403 (Forbidden) received for sessions to users of other network domains. In case only a subset of per network measurements is supported, a sum subcounter will be provided first (see 3GPP TS 23.228 [7]).

- Each measurement is an integer value. The number of measurements is equal to the number of network domain plus a possible sum value identified by the .sum suffix.

- The measurement name has the form IDS.403SessionToOtherNtwkDmn.Domain
where Domain identifies the network domain, and it is in the format of the domain name information in the SIP messages.

- IbcfFunction

- Valid for packet switched traffic.

- IMS.

# 5	Equipment related measurements

The measurements defined in this subclause are related to the equipment aspect performance. The measurements defined in the following subclauses are common for all of the equipments in IMS and shall be applied to all of the network elements in IMS.

## 5.1	Processor usage

### 5.1.1	Mean processor usage

- This measurement provides the mean usage of each key processor during the granularity period. Each equipment may have more than one key processors, the measurement is split into subcounters per key processor.

- SI.

- This measurement is obtained by sampling at a pre-defined interval the usage of the processor and then taking the arithmetic mean for each key processor.

- Each measurement is an integer value (Unit: %).

- The measurement name has the form EQPT.MeanProcessorUsage.ProcessorID
where ProcessorID identifies the key processor of this equipment, the format of ProcessorID is vendor specific.

- ManagedElement.

- Valid for circuit switched and packet switched traffic.

- IMS.

### 5.1.2	Peak processor usage

- This measurement provides the peak usage of each key processor during the granularity period. Each equipment may have more than one key processors, the measurement is split into subcounters per key processor.

- SI.

- This measurement is obtained by sampling at a pre-defined interval the usage of the processor and then taking the maximum for each key processor.

- Each measurement is an integer value (Unit: %).

- The measurement name has the form EQPT.PeakProcessorUsage.ProcessorID
where ProcessorID identifies the key processor of this equipment, the format of ProcessorID is vendor specific.

- ManagedElement.

- Valid for circuit switched and packet switched traffic.

- IMS.

Annex A (normative): 
Use cases for performance measurements defintion

The present Annex provides the concrete use cases for the IMS performance measurements defined in the present specification.

# A.1	Use case of Immediate Messaging procedure related measurements at CSCF

IMS users shall be able to exchange immediate messages with each other by using the Immediate Messaging procedure. CSCF may delivery successfully the immediate messages, also may refuse the immediate messages or respond to the sender with fail indications.If the Immediate Messaging procedure at CSCF is fail, the users’ subjective feel to the network will be bad.So it is necessary to define Immediate Messaging procedure related measurements for operator to evaluate the rate of success and failure at CSCF in IMS network.

# A.2	Use case of IP-CAN Session Establishment related measurements

IP-CAN session is a main procedure in policy and charging control architecture (see 3GPP TS 29.213 [33]). If IP-CAN session establishment is failed, it means that PCEF cannot get PCC rules from PCRF and PCEF shall reject the attempt for UE-initiated resource request. So it is necessary to define IP-CAN session establishment related measurements to monitor or evaluate session performance of PCC procedures.

# A.3	Use case of emergency session related measurements

Emergency session function is an important function at P-CSCF. The emergency session related measurements can help operator to evaluate the emergency session processing capacity of IMS network entity, analyze the establishement failure reasons of emergency session and solve the problems. The emergency session establishment procedure may not comply with the eme emergency configuration policy, and these measurements can be used to check the effectiveness of the policy.

# A.4	Use case of CDIV service related measurements

The Communications Diversion (CDIV) service is an important supplementary service. It can divert user to another destination in the communication.The CDIV services mainly include Communication Forwarding Unconditional (CFU), Communication Forwarding on Busy user (CFB), Communication Forwarding on no Reply (CFNR) and Communication Forwarding on Subscriber Not Reachable (CFNRc). The Communications Diversion (CDIV) service related measurements at AS can reflect the performance of the CDIV service processing and the status of user using the CDIV service.

# A.5	Use case of OIP and OIR service related measurements

The Originating Identification Presentation (OIP) is an important supplementary service. It provides the terminating user with the possibility of receiving the identity information to identify the originating user. The OIP service related measurements at AS can reflect the performance of the OIP service processing and the status of user using the OIP service.

# A.6	Use case of successful session establishments of inter-network domains

When inter-network domain calling occurred,not only the number of attempted session establishment but  also successful session establishment is essential. If the successful session has not been established, it means that the attempt for inter-network calling has been rejected.This measurement can be used to caculate the rate of successful seesion, it will reflect the connectivity status of both network of inter-network domain calling, and also can infulence the feeling of users directly. So it is necessary to define successful session establishments to monitor or evaluate session performance of inter-network calling.

# A.7	Use case of Gateway Control Session Establishment related measurements

Gateway Control session is a main procedure in policy and charging control architecture (see 3GPP TS 29.213 [33]). If Gateway Control session establishment is failed, it means that BBERF cannot get PCC rules from PCRF and BBERF shall reject the attempt for UE-initiated resource request, the whole procedure will terminate. So it is necessary to define Gateway Control session establishment related measurements to evaluate session performance of PCC procedures and help the trouble shooting.

Annex B (informative):
Void

Annex C (informative):
Use case of successful session establishment mean time

The establishment mean time of successful session can reflect the response speed which is an important aspect of IMS response ability. It can be used to evaluate the users’ subjective feeling, and if the time is too long, naturally the users will feel they are having a poor network.

Annex D (informative): 
Use case of Number of 200 OK (Successful) messages for the roamers

The number of successful initial registrations of roamers can be a valuable reference to decide whether the network expansion or network configuration adjustment is needed. This measurement can also be used to evaluate the user requirements more accurately, and then the operator can customize the approriate value added services according to the user requirements of different IMS domains, for example, the preferential policies of roaming charging for the IMS domain with greater user requirements.

Annex E (informative):
Change history

| Change history |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Cat | Old | New |
| Mar 2007 | SP-35 | SP-070067 | -- | -- | Submitted to SA#35 for for Information | -- | -- | 1.0.0 |
| Mar 2007 | -- | -- | -- | -- | Editorial clean-up | -- | 1.0.0 | 1.0.1 |
| May 2007 | SP-36 | SP-070287 | -- | -- | Submitted to SA#36 for Approval | -- | 2.0.0 | 7.0.0 |
| Sep 2007 | SP-37 | SP-070613 | 0001 | -- | Update IMS Performance Measurements With Missing Granularity And Incorrect CC Performance Measurement Types | F | 7.0.0 | 7.1.0 |
| Sep 2007 | SP-37 | SP-070614 | 0002 | -- | Add performance measurements on SIP subscription and notification | B | 7.1.0 | 8.0.0 |
| Mar 2008 | SP-39 | SP-080058 | 0006 | -- | Correction to the registration measurements in CSCF | A | 8.0.0 | 8.1.0 |
| Mar 2008 | SP-39 | SP-080069 | 0003 | -- | Correction of BGCF related measurements - Align with 23.228 | F | 8.0.0 | 8.1.0 |
| Mar 2008 | SP-39 | SP-080069 | 0004 | -- | Add registration related measurement per access technology | B | 8.0.0 | 8.1.0 |
| Dec 2008 | SP-42 | SP-080845 | 0008 | -- | Correction of de-registration related measurements | A | 8.1.0 | 8.2.0 |
| Dec 2008 | SP-42 | SP-080846 | 0009 | -- | Add IBCF related measurements | B | 8.1.0 | 8.2.0 |
| Sep 2009 | SP-45 | SP-090534 | 0010 | -- | Correction of measurement names for RTP related measurements | F | 8.2.0 | 8.3.0 |
| Sep 2009 | SP-45 | SP-090534 | 0013 | -- | Correction measurements related to PDF - align with 29.213 | F | 8.2.0 | 8.3.0 |
| Sep 2009 | SP-45 | SP-090627 | 0011 | -- | Addition of Immediate Messaging related measurements | B | 8.3.0 | 9.0.0 |
| Sep 2009 | SP-45 | SP-090627 | 0012 | -- | Addition of PCRF IP-CAN Session Establishment related measurements | B | 8.3.0 | 9.0.0 |
| Dec 2009 | SP-46 | SP-090719 | 0014 | -- | Addition of AS Session control related measurements | B | 9.0.0 | 9.1.0 |
| Apr 2010 | -- | -- | -- | -- | Addition of missing change history | -- | 9.1.0 | 9.1.1 |
| Jun 2010 | SP-48 | SP-100264 | 0016 | - | Add the measurement of number of 200 OK (Successful) messages for the roamers | B | 9.1.1 | 10.0.0 |
| Jun 2010 | SP-48 | SP-100264 | 0017 | - | Add emergency session related measurements | B | 9.1.1 | 10.0.0 |
| Jun 2010 | SP-48 | SP-100264 | 0015 | - | Add the measurement of successful session establishment mean time | B | 9.1.1 | 10.0.0 |
| Sep 2010 | SP-49 | SP-100489 | 0018 | - | Addition of CDIV services related measurements | B | 10.0.0 | 10.1.0 |
| Sep 2010 | SP-49 | SP-100489 | 0019 | - | Addition of OIP service related measurements | B | 10.0.0 | 10.1.0 |
| Sep 2010 | SP-49 | SP-100489 | 0020 | - | Addition of successful session establishment of inter-network domain calling related measurement | B | 10.0.0 | 10.1.0 |
| Dec 2010 | SP-50 | SP-100764 | 0021 | - | Adding Gateway Control session establishment related measurements | B | 10.1.0 | 10.2.0 |
| Dec 2010 | SP-50 | SP-100752 | 0025 | - | Adding mean number of simultaneous online and answered sessions | B | 10.1.0 | 10.2.0 |