3GPP TS 28.655 V11.2.0 (2014-09)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Services and System Aspects;

Telecommunication management;

GSM/EDGE Radio Access Network (GERAN)

Network Resource Model (NRM)

Integration Reference Point (IRP);

Information Service (IS)

(Release 11)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. 
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.	 
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

GSM, EDGE, GERAN, NRM, IRP, Converged Management

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2014, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI currently being registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Forward	4

Introduction	4

1	Scope	5

2	References	5

3	Definitions and abbreviations	6

3.1	Definitions	6

3.2	Abbreviations	6

4	Model	6

4.1	Imported information entities and local labels	6

4.2	Class diagram	7

4.2.1	Relationships	7

4.2.2	Inheritance	8

4.3	Class definitions	8

4.3.1	BSSFunction	8

4.3.1.1	Definition	8

4.3.1.2	Attributes	9

4.3.1.3	Attribute constraints	9

4.3.1.4	Notifications	9

4.3.2	BTSSiteMgr	9

4.3.2.1	Definition	9

4.3.2.2	Attributes	9

4.3.2.3	Attribute constraints	9

4.3.2.4	Notifications	9

4.3.3	GSMCell	9

4.3.3.1	Definition	9

4.3.3.2	Attributes	10

4.3.3.3	Attribute constraints	10

4.3.3.4	Notifications	10

4.3.4	GSMRelation	10

4.3.4.1	Definition	10

4.3.4.2	Attributes	11

4.3.4.3	Attribute constraints	11

4.3.4.4	Notifications	11

4.3.5	ExternalGSMCell	11

4.3.5.1	Definition	11

4.3.5.2	Attributes	12

4.3.5.3	Attribute constraints	12

4.3.5.4	Notifications	12

4.3.6	ExternalBSSFunction	12

4.3.6.1	Definition	12

4.3.6.2	Attributes	12

4.3.6.3	Attribute constraints	12

4.3.6.4	Notifications	12

4.4	Attribute definitions	13

4.4.1	Attribute properties	13

4.4.2	Constraints	17

4.5	Common notifications	17

4.5.1	Alarm notifications	17

4.5.2	Configuration notifications	17

Annex A (informative):	Change history	18

# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# Introduction

The present document is part of a TS-family covering the 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Telecommunication management; as identified below:

28.654:	GSM/EDGE Radio Access Network (GERAN) Network Resource Model (NRM) Integration Reference Point (IRP); Requirements

28.655:	GSM/EDGE Radio Access Network (GERAN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)

28.656:	GSM/EDGE Radio Access Network (GERAN) Network Resource Model (NRM) Integration Reference Point (IRP); Solution Set (SS) definitions

# 1	Scope

The present document specifies the GERAN Network Resource Model (NRM) that can be communicated between an IRPAgent and IRPManagers for telecommunication network management purposes, including management of converged networks.

This document specifies the semantics and behaviour of class attributes and relations visible across the reference point in a protocol and technology neutral way.  It does not define their syntax and encoding.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies.  In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TS 32.101: "Telecommunication management; Principles and high level requirements".

[2]	3GPP TS 32.102: "Telecommunication management; Architecture".

[3]	3GPP TS 24.008: "Mobile radio interface Layer 3 specification; Core network protocols; Stage 3".

[4]	3GPP TS 44.018: "Mobile radio interface layer 3 specification; Radio Resource Control (RRC) protocol".

[5]	3GPP TS 45.008: "Radio subsystem link control".

[6]	3GPP TS 45.002: "Multiplexing and multiple access on the radio path".

[7]	3GPP TS 23.002: "Network architecture".

[8]	3GPP TS 23.003: "Numbering, Addressing and Identification".

[9]	3GPP TS 28.652: "Telecommunication management; Configuration Management (CM); UTRAN Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[10]	3GPP TS 28.658: "Telecommunication management; Evolved Universal Terrestrial Radio Access Network (E-UTRAN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS)".

[11]	3GPP TS 32.111-2: "Telecommunication management; Fault Management (FM); Part 2: Alarm Integration Reference Point (IRP): Information Service (IS)".

[12]	3GPP TS 28.662: "Telecommunication management; Generic Radio Access Network (RAN) Network Resource Model (NRM) Integration Reference Point (IRP); Information Service (IS) ".

[13]	3GPP TS 32.300: "Telecommunication management; Configuration Management (CM); Name convention for Managed Objects".

[14]	3GPP TS 32.600: "Telecommunication management; Configuration Management (CM); Concept and high-level requirements".

[15]	3GPP TS 32.302: "Telecommunication management; Configuration Management (CM); Notification Integration Reference Point (IRP): Information Service (IS)".

[16]	3GPP TS 28.622: "Telecommunication management; Generic Network Resource Model (NRM) Integration Reference Point (IRP): Information Service (IS)".

[17]	3GPP TS 32.662: "Telecommunication management; Configuration Management (CM); Kernel CM; Information service (IS)".

[18]	3GPP TS 28.625: "Telecommunication Management; State Management Data Definition Integration Reference Point (IRP): Information Service (IS)".

# 3	Definitions and abbreviations

## 3.1	Definitions

For the purposes of the present document, the following terms and definitions apply. For terms and definitions not found here, please refer to 3GPP TS 32.101 [1], 3GPP TS 32.102 [2] and 3GPP TS 32.600 [14].

Association: See definition in TS 28.622 [16].

Managed Element (ME): See definition in TS 28.622 [16].

Network Resource Model (NRM): See definition in TS 28.622 [16].

## 3.2	Abbreviations

For the purposes of the present document, the following abbreviations apply:

DN	Distinguished Name (see 3GPP TS 32.300 [13])

EM	Element Manager

GERAN	GSM-EDGE Radio Access Network

GPRS	General Packet Radio System

IOC	Information Object Class

IRP	Integration Reference Point

ME	Managed Element

NE	Network Element

NR	Neighbour cell Relation

NRM	Network Resource Model

RDN	Relative Distinguished Name (see 3GPP TS 32.300 [13])

RNC	Radio Network Controller

UML	Unified Modelling Language

# 4	Model

## 4.1	Imported information entities and local labels

| Label reference | Local label |
| --- | --- |
| 3GPP TS 28.622 [16], information object class, ManagedElement | ManagedElement |
| 3GPP TS 28.622 [16], information object class, ManagedFunction | ManagedFunction |
| 3GPP TS 28.622 [16], information object class, SubNetwork | SubNetwork |
| 3GPP TS 28.622 [16], information object class, Top | Top |
| 3GPP TS 28.622 [16], information object class, VsDataContainer | VsDataContainer |
| 3GPP TS 28.652 [9], information object class, UTRANRelation | UTRANRelation |
| 3GPP TS 28.658 [10], information object class, EUTRANRelation | EUTRANRelation |
| 3GPP TS 28.625 [18], attribute, operationalState | operationalState |

## 4.2	Class diagram

### 4.2.1	Relationships

This clause depicts the set of classes that encapsulate information relevant for this service. This clause provides the overview of all classes in UML.  Subsequent clauses provide more detailed specification of various aspects of these classes.

The figures below show the containment/naming hierarchy and the associations of the GERAN NRM.

NOTE 1:	The listed cardinality numbers represent transient as well as steady-state numbers, and reflect all managed object creation and deletion scenarios.

NOTE 2:	The ExternalBSSFunction is used in the Core Network NRM.

Figure 4.2.1-1: GERAN NRM Containment/Naming and Association diagram

Each Managed Object is identified with a Distinguished Name (DN) according to 3GPP TS 32.300 [13] that expresses its containment hierarchy. As an example, the DN of an IOC representing a cell could have a format like:

SubNetwork =Sweden, MeContext =MEC-Gbg-1, ManagedElement =RNC-Gbg-1, BSSFunction=BSS1.

NOTE 1:	The listed cardinality numbers represent transient as well as steady-state numbers, and reflect all managed object creation and deletion scenarios.

NOTE 2:	Each instance of the VsDataContainer shall only be contained under one IOC. The VsDataContainer can be contained under IOCs defined in other NRMs.

Figure 4.2.1-2: GERAN NRM Containment/Naming and Association diagram

The VsDataContainer is only used for the Bulk CM IRP.

### 4.2.2	Inheritance

This clause depicts the inheritance relationships that exist between IOCs.

Figure 4.2.2-1 shows the inheritance hierarchy for the GERAN NRM.

Figure 4.2.2-1: GERAN NRM Inheritance Hierarchy

## 4.3	Class definitions

### 4.3.1	BSSFunction

#### 4.3.1.1	Definition

This IOC represents BSS functionality. For more information about the BSS, see Ref 3GPP TS  23.002 [7].

#### 4.3.1.2	Attributes

None.

#### Attribute constraints

None.

#### 4.3.1.4	Notifications

The common notifications defined in subclause 4.5.1 are valid for this IOC, without exceptions or additions.

### 4.3.2	BTSSiteMgr

#### 4.3.2.1	Definition

This IOC contains site specific information for a BTS site.

#### 4.3.2.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| latitude | O | M | M | - | O |
| longitude | O | M | M | - | O |
| operationalState (see NOTE 1) | CM | M | - | - | M (see NOTE 2) |
| Note 1: No state propagation shall be implied. Note 2: The attribute value change is conveyed by the notifyStateChange notification. |  |  |  |  |  |

#### 4.3.2.3	Attribute constraints

| Name | Definition |
| --- | --- |
| operationalState CM support qualifier | The State Management IRP is supported. |

#### 4.3.2.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC In addition, the following set of notification, defined in 3GPP TS 32.662 [17], is also valid.

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyStateChange | O |  |

### 4.3.3	GSMCell

#### 4.3.3.1	Definition

This IOC represents the GSM radio cell. The applicability of instantiation of this class is depending on the ME type. It may only be instantiated under ME of type BSC.

#### 4.3.3.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| cellIdentity | M | M | M | - | M |
| cellAllocation | M | M | M | - | M |
| ncc | M | M | M | - | M |
| bcc | M | M | M | - | M |
| lac | M | M | M | - | M |
| mcc | M | M | M | - | M |
| mnc | M | M | M | - | M |
| rac | CM | M | M | - | M |
| racc | CM | M | M | - | M |
| tsc | CM | M | M | - | M |
| rxLevAccessMin | M | M | M | - | M |
| msTxPwrMaxCCH | M | M | M | - | M |
| rfHoppingEnabled | M | M | M | - | M |
| hoppingSequenceList | M | M | M | - | M |
| plmnPermitted | M | M | M | - | M |

#### 4.3.3.3	Attribute constraints

| Name | Definition |
| --- | --- |
| rac CM support qualifier | GPRS is supported in the cell. |
| racc CM support qualifier | GPRS is supported in the cell. |
| tsc CM support qualifier | RET, TMA etc. are not modelled according to Ref 3GPP TS 32.792 [12]. |

#### 4.3.3.4	Notifications

The common notifications defined in subclause 4.5 are valid for this IOC, without exceptions or additions.

### 4.3.4	GSMRelation

#### 4.3.4.1	Definition

This IOC contains a Neighbour cell Relation (NR) from a source cell to a target cell, where the target cell is a GSMCell or ExternalGSMCell instance.

NOTE:	In handover relation terms, the cell containing the GSMRelation object is the source cell for the handover. The cell referred to in the GSMRelation object is the target cell for the handover. This defines a one-way handover relation where the direction is from source cell to target cell.

The source cell can be a GSMCell instance. This is the case for an Intra-GERAN NR.

The source cell can be a UTRANGenericCell instance. This is the case for Inter-RAT NR from UTRAN to GERAN. See 3GPP TS 28.652 [9].

The source cell can be an EUTRANGenericCell instance. This is the case for Inter-RAT NR from E-UTRAN to GERAN. See 3GPP TS 28.658 [10].

#### 4.3.4.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| bcchFrequency | CM | M | - | - | M |
| ncc | CM | M | - | - | M |
| bcc | CM | M | - | - | M |
| lac | CM | M | - | - | M |
| isHOAllowed | CM | M | M | - | M |
| isRemoveAllowed | CM | M | M | - | M |
| isESCoveredBy | CM | M | M | - | M |
| Attribute related to role |  |  |  |  |  |
| adjacentCell | CM | M | - | - | M |

#### 4.3.4.3	Attribute constraints

| Name | Definition |
| --- | --- |
| bcchFrequency CM support qualifier | The EM does not guarantee consistency between the cell definition and what is broadcast on system information and RET, TMA etc. are not modelled according to Ref 3GPP TS 28.662 [12]. |
| ncc CM support qualifier | The EM does not guarantee consistency between the cell definition and what is broadcast on system information. |
| bcc CM support qualifier | The EM does not guarantee consistency between the cell definition and what is broadcast on system information. |
| lac CM support qualifier | The EM does not guarantee consistency between the cell definition and what is broadcast on system information. |
| isRemoveAllowed Support Qualifier | The condition is "Inter-RAT ANR function is supported in the source cell, and the source cell is an EUTRANGenericCell or a UTRANGenericCell ". |
| isHOAllowed Support Qualifier | The condition is "Inter-RAT ANR function is supported in the source cell, and the source cell is an EUTRANGenericCell or a UTRANGenericCell ". |
| isESCoveredBy Support Qualifier | The condition is "The source cell is an E-UTRAN or UTRAN cell which supports Inter-RAT Energy Saving". |
| adjacentCell Support Qualifier | The conditions are: “The target cell and the serving cell (name-containing this GSMRelation) are managed by different IRPAgent” or “the target cell and the serving cell (name-containing this GSMRelation) are managed by the same IRPAgent”. When former condition is true, the role-attribute holds the DN of an ExternalGSMCell instance. When the latter condition is true, the role-attribute holds the DN of a GSMCell instance. |

#### 4.3.4.4	Notifications

The common notifications defined in subclause 4.5.2 are valid for this IOC, without exceptions or additions.

### 4.3.5	ExternalGSMCell

#### 4.3.5.1	Definition

This IOC represents a radio cell controlled by another IRPAgent. This IOC has necessary attributes for inter-system handover.  It contains a subset of the attributes of related IOCs controlled by another IRPAgent. To maintain the consistency between the attribute values of these two IOCs is outside the scope of this document.

#### 4.3.5.2	Attributes

| Attribute name | Support Qualifier | isReadable | isWritable | isInvariant | isNotifyable |
| --- | --- | --- | --- | --- | --- |
| cellIdentity | M | M | M | - | M |
| bcchFrequency | M | M | M | - | M |
| ncc | M | M | M | - | M |
| bcc | M | M | M | - | M |
| lac | M | M | M | - | M |
| mcc | M | M | M | - | M |
| mnc | M | M | M | - | M |
| rac | CM | M | M | - | M |
| racc | CM | M | M | - | M |

#### 4.3.5.3	Attribute constraints

| Name | Definition |
| --- | --- |
| rac CM support qualifier | GPRS is supported in the cell. |
| racc CM support qualifier | GPRS is supported in the cell. |

#### 4.3.5.4	Notifications

The common notifications defined in subclause 4.5.2 are valid for this IOC, without exceptions or additions.

### 4.3.6	ExternalBSSFunction

#### 4.3.6.1	Definition

This IOC represents a BSSFunction controlled by another IRPAgent. It contains a subset of the attributes of related IOCs controlled by another IRPAgent. To maintain the consistency between the attribute values of these two IOCs is outside the scope of the present document.

#### 4.3.6.2	Attributes

None.

#### 4.3.6.3	Attribute constraints

None.

#### 4.3.6.4	Notifications

The common notifications defined in subclause 4.5.2 are valid for this IOC, without exceptions or additions.

## 4.4	Attribute definitions

### 4.4.1	Attribute properties

The following table defines the properties of attributes specified in the present document.

Table 4.4.1: Attributes

| Attribute Name | Documentation and Allowed values | Properties |
| --- | --- | --- |
| GSMCell.bcc | Base station colour code, BCC (part of BSIC). Ref 3GPP TS 44.018 [4]. See Ref 3GPP TS 44.018 [4]. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ExternalGSMCell.bcc | Base station colour code, BCC (part of BSIC). Ref 3GPP TS 44.018 [4]. See Ref 3GPP TS 44.018 [4]. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| GSMRelation.bcc | Base station colour code, BCC (part of BSIC. Ref 3GPP TS 44.018 [4]) for another GSM cell or the external GSM cell, that is broadcast in System Information in the Cell. See Ref 3GPP TS 44.018 [4]. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ExternalGsmCell.bcchFrequency | This attribute contains the absolute radio frequency channel number of the BCCH channel of the GSM cell. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| GSMRelation.bcchFrequency | This attribute contains the absolute radio frequency channel number of the BCCH channel of another GSM cell or the external GSM cell, that is broadcast in System Information in the Cell. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| cellAllocation | This attribute defines the set of radio frequencies allocated and available to a cell, the first element sets the BCCH frequency, Ref 3GPP TS 44.018 [4]. See Ref 3GPP TS 44.018 [4]. allowedValues: N/A | type: Integer multiplicity: 0..* isOrdered: False isUnique: True defaultValue: None isNullable: False |
| cellIdentity | Cell Identity (Ref 3GPP TS 24.008 [3]). See Ref 3GPP TS 24.008 [3]. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| hoppingSequenceList | This attribute defines a sequence of structures. Each structure has two elements. One element carries the MA as defined in 44.018 [4] and the other element carries the HSN as defined in 45.502 [6]. allowedValues: N/A | type: <<datatype>> multiplicity: 1 isOrdered: False isUnique: True defaultValue: None isNullable: False |
| isESCoveredBy | The value of the attribute is configured by the IRPManager and is not changed by the IRPAgent. It indicates whether the adjacentCell according to this planning provides no, partial or full Inter-RAT coverage for the cell which name-contains the GSMRelation instance. Adjacent cells with this attribute equal to “yes” are recommended to be considered as candidate cells to take over the coverage when the original cell is about to be transferred to energySaving state. The entirety of adjacent cells with this property equal to “partial” are recommended to be considered as entirety of candidate cells to take over the coverage when the original cell is about to be transferred to energySaving state. allowedValues: No, partial, yes | type: ENUM multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| GSMCell.lac | Location Area Code (LAC). Ref 3GPP TS 24.008 [3]. See Ref 3GPP TS 24.008 [3]. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| GSMRelation.lac | Location Area Code, LAC (Ref 3GPP TS 24.008 [3]) for another GSM cell or the external GSM cell, that is broadcast in System Information in the Cell. See Ref 3GPP TS 24.008 [3]. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| latitude | Used for geographical positioning of the site manager. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| longitude | Used for geographical positioning of the site manager. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| mcc | Mobile Country Code, MCC (part of the PLMN Id, Ref. 3GPP TS 23.003 [8]). See Ref 3GPP TS 24.008 [3]. | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None allowedValues: N/A isNullable: False |
| mnc | Mobile Network Code, MNC (part of the PLMN Id, Ref. 3GPP TS 23.003 [8]). See Ref 3GPP TS 24.008 [3]. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| msTxPwrMaxCCH | Maximum Transmission Power for a Mobile Station on a CCH. Attribute description Ref 3GPP TS 45.008 [5] (MS_TXPWR_MAX_CCH). See Ref 3GPP TS 45.008 [5]. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| GSMCell.ncc | Network Colour Code, NCC (part of BSIC). Ref 3GPP TS 44.018 [4]. See Ref 3GPP TS 44.018 [4]. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| ExternalGSMCell.ncc | Network Colour Code, NCC (part of BSIC. Ref 3GPP TS 44.018 [4]) for another GSM cell or the external GSM cell, that is broadcast in System Information in the Cell. See Ref 3GPP TS 44.018 [4]. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| plmnPermitted | Network Colour Code Permitted. Attribute description reference 3GPP TS 45.008 [5] (NCC_PERMITTED). See Ref 3GPP TS 45.008 [5]. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rac | Routing Area Code, RAC. Ref 3GPP TS 44.018 [4]. See Ref 3GPP TS 44.018 [4]. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| racc | Routing Area Colour Code, RACC. Ref 3GPP TS 44.018 [4]. See Ref 3GPP TS 44.018 [4]. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rfHoppingEnabled | Indicates if frequency hopping is enabled. Boolean value false represents ‘disabled’; true represents ‘enabled’. allowedValues: False, True. | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| rxLevAccessMin | Minimum Access Level. Attribute description Ref 3GPP TS 45.008 [5] (RXLEV_ACCESS_MIN) See Ref 3GPP TS 45.008 [5]. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| tsc | Training Sequence Code, an attribute of the class channel in Ref 3GPP TS 44.018 [4]. See Ref 3GPP TS 44.018 [4]. allowedValues: N/A | type: Integer multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| isHOAllowed | This indicates if HO is allowed or prohibited. If true, handover is allowed from source cell to target cell. The source cell is identified by the name-containing UTRANGenericCell or EUTRANGenericCell of the GSMRelation that has the isHOAllowed. The target cell is referenced by the GSMRelation that has this isHOAllowed. If false, handover shall not be allowed. allowedValues: False, True. | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| isRemoveAllowed | This indicates if the subject GSMRelation can be removed (deleted) or not. If true, the subject GSMRelation instance can be removed (deleted). If false, the subject GSMRelation instance shall not be removed (deleted) by any entity but an IRPManager. allowedValues: False, True. | type: Boolean multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |
| Attribute related to role |  |  |
| adjacentCell | This holds the DN of GSMCell or ExternalGSMCell. allowedValues: N/A | type: DN multiplicity: 1 isOrdered: N/A isUnique: N/A defaultValue: None isNullable: False |

### 4.4.2	Constraints

None.

## 4.5	Common notifications

### 4.5.1	Alarm notifications

This clause presents a list of notifications, defined in 3GPP TS 32.111-2 [11], that IRPManager can receive. The notification header attribute objectClass/objectInstance, defined in 3GPP TS 32.302 [15], would capture the DN of an instance of an IOC defined in this IRP specification.

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyAckStateChanged | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyChangedAlarm | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyClearedAlarm | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyNewAlarm | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyComments | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyAlarmListRebuilt | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |
| notifyPotentialFaultyAlarmList | See Alarm IRP (3GPP TS 32.111-2 [11]) |  |

### Configuration notifications

This clause presents a list of notifications, defined in 3GPP TS 32.662 [17], that IRPManager can receive. The notification header attribute objectClass/objectInstance, defined in 3GPP TS 32.302 [15], would capture the DN of an instance of an IOC defined in this IRP specification.

| Name | Qualifier | Notes |
| --- | --- | --- |
| notifyAttributeValueChange | O |  |
| notifyObjectCreation | O |  |
| notifyObjectDeletion | O |  |

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 2014-06 | SA#64 | SP-140332 | 001 | 1 | Correct hopping sequence list data type | 11.0.0 | 11.1.0 |
|  |  | SP-140359 | 002 | - | remove the feature support statements |  |  |
| 2014-09 | SA#65 | SP-140558 | 004 | - | Correction of operationalState attribute definitions | 11.1.0 | 11.2.0 |