3GPP TSG-SA WG6 Meeting #5	S6-150575

Seoul, South Korea, 6-10 July 2015	(revision of S6-15xxxx)

Title:	Editorial Improvement to 7.5.3.3 of TS 23.179

Agenda Item:	7.4

Source:	OTD

Contact:	Selvam Rengasami, selvam@trideaworks.com

Abstract: Provides editorial changes to improve the text of 7.5.3.3 of TS 23.179

#### 7.5.3.3	Reference point SIP-2 (between the SIP core and the MCPTT server)

The SIP-2 reference point exists between the SIP core and the MCPTT server for establishing a session in support of MCPTT and shall use the 3GPP ISC interface. The SIP-2 reference point is used for:

-	notification to the MCPTT server of SIP registration by the MCPTT UE;

-	authentication and security to the service layer;

-	event subscription and notification; and

-	session management and media negotiation.