3GPP TSG-SA WG6 Meeting #42-e	S6-210530

e-meeting, 1st – 9th March 2021	(revision of S6-21xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.286 | CR | 0042 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Session oriented services |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XAPP |  |  |  |  |  | Date: |  |  | 2021-02-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | V2X application specific server initiated session oriented service and the VAE layer mechanism for session-oriented service are introduced. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Procedures are added for V2X application specific server initiated session oriented service and VAE layer mechanism for session-oriented service as studied in TR 23.764 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Session oriented services will not be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.x.4 (new), 9.x.4.1 (new), 9.x.4.2 (new), 9.x.4.3 (new), 9.x.4.4 (new), 9.x.5 (new), 9.x.5.1 (new), 9.x.5.2 (new), 9.x.5.3 (new), 9.x.5.4 (new), |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 9.x.4	V2X application specific server triggered session-oriented service

#### 9.x.4.1	General

The VAE layer provides support at the application layer for V2X application specific server triggered session-oriented services to support tele-operated driving by establishing, updating and terminating sessions.

#### 9.x.4.2	Triggering of session-oriented service establishment

Pre-conditions:

-	The V2X application-specific server has decided to trigger the session-oriented service.

NOTE: The reasons that the V2X application-specific server decides to trigger the tele-operated driving session are out of scope of the present document.

Figure 9.x.4.2-1: Triggering of session-oriented service establishment

1.	The V2X application-specific server triggers the VAE server to initiate a session-oriented service to the VAE client. This request can include information such as the identity of the VAE client, the identity of the V2X application specific server, the identity of the service session, the V2X service ID and the "type" or "QoS requirements" of the session.

2.	The VAE server provides a session-oriented service trigger response to the V2X application-specific server indicating the ability or inability of VAE server to initiate a session-oriented service with the VAE client.

NOTE:	The VAE server performs the procedure specified in clause 9.x.5.2 to establish session oriented service with VAE client (e.g. remote vehicle).

#### 9.x.4.3	Triggering of session-oriented service update

Pre-conditions:

-	The VAE server has established a session-oriented service with the VAE client as described in clause 9.x.5.2.

-	The V2X application-specific server needs to update parameters of the session.

Figure 9.x.4.3-1: Triggering of session-oriented service update

1.	The V2X application-specific server provides the VAE server with the identity of the session and the update of session parameters, e.g. change of QoS requirements of the session.

2.	The VAE server provides a session-oriented service update response to the V2X application specific server including the ability or inability to continue the session-oriented service with the VAE client with the updated conditions.

NOTE:	The VAE server performs the procedure specified in clause 9.x.5.3 to change session oriented service with VAE client (e.g. remote vehicle).

#### 9.x.4.4	Triggering of session-oriented service termination

Pre-conditions:

-	The VAE server has established a session-oriented service with the VAE client as described in clause 9.x.5.2.

-	The V2X application specific server has decided to terminate the session-oriented service.

Figure 9.x.4.4-1: Triggering of session-oriented service termination

1.	The V2X application-specific server provides the VAE server with a request to terminate the session-oriented service with the VAE client. It may include the identity of the service session.

2.	The VAE server provides a session-oriented service termination response to the V2X application-specific server that the session can be terminated.

NOTE:	The VAE server performs the procedure specified in clause 9.x.5.4 to terminate session oriented service with VAE client (e.g. remote vehicle).

### 9.x.5	Session-oriented service

#### 9.x.5.1	General

The VAE layer provides support at the application layer for session-oriented services to support tele-operated driving by establishing, updating and terminating sessions.

#### 9.x.5.2	Session-oriented service establishment

Pre-conditions:

-	The V2X UE has been authenticated for using session-oriented services and has connected to the V2X application server.

-	The VAE server has agreed to establish a session upon the request from the V2X application specific server or from the VAE client.

-	The V2X application specific client for the terminating V2X UE has been informed about the already negotiated session oriented service to VAE client.

Figure 9.x.5.2-1: Procedure for establishing a session-oriented service between the VAE server and the VAE client

1.	The VAE server sends a request to one or more VAE clients to establish a session-oriented service including identity of the VAE client, identity of the session, V2X service ID and reporting configuration.

2.	The VAE client sends a session-oriented service response to the VAE server indicating approval of session establishment request by the VAE server.

3.	If V2X application specific server has triggered then, the VAE server sends a session-oriented service establish notification to the V2X application-specific server.

4.	The VAE client sends a session-oriented service establish notification to the V2X application-specific client.

#### 9.x.5.3	Session-oriented service update

Pre-condition:

-	The VAE server has established a session-oriented service with the VAE client.

-	The VAE server has received a session-oriented service update from the V2X application-specific server or from the VAE client or a change in QoS and network conditions from 3GPP network system.

-	The VAE server has agreed to continue the session-oriented service with the VAE client with the updated conditions upon the request from the V2X application specific server or from the VAE client.

Figure 9.x.5.3-1: Procedure for updating a session-oriented service between the VAE server and the VAE client

1.	The VAE server sends a change request to one or more VAE clients including the identity of the session, updates requirements (e.g. change of network/QoS requirements), or server information (e.g. change of VAE server).

2.	The VAE client sends a change response to the VAE server indicating acceptance of the change request by the VAE server.

3.	If V2X application specific server has triggered then, the VAE server sends a session-oriented service change notification to the V2X application-specific server.

4.	The VAE client sends a session-oriented service change notification to the V2X application-specific client.

#### 9.x.5.4	Session-oriented service termination

Pre-condition:

-	The VAE server has established a session-oriented service with the VAE client.

-	The VAE server has received a session-oriented service termination request from the V2X application-specific server or from the VAE client or a change in QoS or network conditions from 3GPP network system that prevents the VAE server from continuing the session-oriented service.

-	The VAE server has indicated to terminate the session-oriented service with the VAE client upon the request from the V2X application specific server or from the VAE client.

Figure 9.x.5.4-1: Procedure for terminating a session-oriented service between the VAE server and the VAE client

1.	The VAE server sends a session-oriented termination request to the VAE client to terminate the session (e.g. end of session with VAE server or requirements for session-oriented service not satisfied) including the identity of the session.

2.	The VAE client sends a session-oriented termination response to the VAE server indicating acceptance of the termination request by the VAE server.

3.	If V2X application specific server has triggered then, the VAE server sends a session-oriented service termination notification to the V2X application-specific server.

4.	The VAE client sends a session-oriented service termination notification to the V2X application-specific client.