3GPP TSG-SA WG6 Meeting #44	S6-211641

e-meeting, 12th – 20th July 2021	(revision of S6-21xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0078 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to Location based Group |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL |  |  |  |  |  | Date: |  |  | 07-07-2021 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1) Resolving following EN: “Editor's Note: Group updates due to Users or UEs moving in or out of the location is FFS.” 2) Clause 9.3.10 “Obtaining UE(s) information at a location” has been added recently to obtain list of UEs from specifc area. > Step #2 to 4 of clause 10.3.7 can be achieved using Clause 9.3.10. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1) To resolve the EN – added procedure in location management service where VAL server (or any SEAL server) can subscribe to location area of interest to received notification of all UEs within the area or UEs moving in/out of the area. 2) Step#2 to 4 has been replaced with clause 9.3.10. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1) EN will remain unresolved in Rel-17. 2) LMS will support duplicate procedures |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.3.7, 9.3.2.a (NEW), 9.3.2.b (NEW), 9.3.2.c (NEW), 9.3.2.d (NEW), 9.3.2.e (NEW), 9.3.2.f (NEW), 9.3.2.g (NEW), 9.3.x (New), 9.3.x.1 (New), 9.3.x.2 (New), 9.3.x.3 (New), 9.3.x.4 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 10.3.7	Location-based group creation

Figure 10.3.7-1 below illustrates the location-based group creation.

Pre-conditions:

1.	The group management client, group management server, VAL server, location management server and the VAL group members belong to the same VAL system.

2.	The authorized VAL user/UE/administrator is not aware of the users' or UE identities which will be combined to form the VAL group.

Figure 10.3.7-1: Location-based group creation

1.	The group management client or the VAL server requests location-based group create operation to the group management server. The location criteria for determining the identities of the users or UEs to be combined shall be included in this message. The location information criteria may include the geographic location information where the UEs moving in or moving out to be monitored, or it may include reference UE information where in the UEs moving in or moving out of given proximity range from the reference UE to be monitored.

2.	The group management server requests the location management server for obtaining the users or UEs corresponding to the location information as specified in clause 9.3.10 or subscribes to the list of UEs moving in or out of the area as specified in clause 9.3.x.

3.	During the group creation, the group management server creates and stores the information of the location-based group. The group management server performs the check on the policies e.g. maximum limit of the total number of VAL group members for the VAL group(s). If an external group identifier, identifying the member UEs of the VAL group at the 3GPP core network is available, then the external group ID is stored in the newly created VAL group’s configuration information.

NOTE:	The exact policies are out of scope of the present document.

4.	The group management server provides a location-based group creation response to the group management client or the VAL server.

* * * Next Change * * * *

#### 9.3.2.a	Location area subscription request

Table 9.3.2.a-1 describes the information flow from the VAL server to the location management server for location area subscription request.

Table 9.3.2.a-1: Location information subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Location Information criteria | M | Location information (e.g. current location of the VAL UE with proximity range, geographic area, etc) |
| >> Reference UE | O | VAL UE which is the reference UE for the proximity range |
| >> Proximity range | O | Proximity range within which the UEs will be monitored |
| >> Geographic area | O | Geographic area to be monitored |
| Time between consecutive reports | M | It indicates the interval time between consecutive reports |
| Triggering events | O | Identifies when the server will send the notification (e.g. distance travelled, age of the location, etc) |

#### 9.3.2.b	Location area subscription response

Table 9.3.2.b-1 describes the information flow from the location management server to the VAL server for location area subscription response.

Table 9.3.2.b-1: Location area subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Subscription status | M | It indicates the subscription result |
| Subscription identity | O | If subscription is successful, identifies the subscription |

#### 9.3.2.c	Location area information notification

Table 9.3.2.c-1 describes the information flow from the location management server to the VAL server.

Table 9.3.2.c-1: Location area information notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription identity | M | Identity of the subscription |
| UEs currently present (NOTE) | O | List of the identities of all VAL UEs who are currently present in the given location area |
| UEs moved in/out (NOTE) | O | List of UEs either moved in to the location area or moved out of the location area |
| >> UEs moved in | O | List of the identities of the VAL UEs who moved in to the given location area since previous notification. |
| >> UEs moved out | O | List of the identities of the VAL UEs who moved out of the given location area since previous notification. |
| Triggering event | O | Identity of the event that triggered the sending of the notification |
| NOTE: Any one of these shall be present. |  |  |

#### 9.3.2.d	Location area subscription modify request

Table 9.3.2.d-1 describes the information flow from the VAL server to the location management server for location area subscription modify request.

Table 9.3.2.d-1: Location information subscription modify request

| Information element | Status | Description |
| --- | --- | --- |
| Subscription identity | M | Identifies the subscription |
| Location Information criteria | M | Location information (e.g. current location of the VAL UE with proximity range, geographic area, etc) |
| >> Reference UE | O | VAL UE which is the reference UE for the proximity range |
| >> Proximity range | O | Proximity range within which the UEs will be monitored |
| >> Geographic area | O | Geographic area to be monitored |
| Time between consecutive reports | M | It indicates the interval time between consecutive reports |
| Triggering events | O | Identifies when the server will send the notification (e.g. distance travelled, age of the location, etc) |

#### 9.3.2.e	Location area subscription modify response

Table 9.3.2.e-1 describes the information flow from the location management server to the VAL server for location area subscription modify response.

Table 9.3.2.e-1: Location area subscription modify response

| Information element | Status | Description |
| --- | --- | --- |
| Subscription status | M | It indicates the request result |

#### 9.3.2.f	Location area unsubscribe request

Table 9.3.2.f-1 describes the information flow from the VAL server to the location management server for location area unsubscribe request.

Table 9.3.2.f-1: Location information unsubscribe request

| Information element | Status | Description |
| --- | --- | --- |
| Subscription identity | M | Identifies the subscription |

#### 9.3.2.g	Location area unsubscribe response

Table 9.3.2.g-1 describes the information flow from the location management server to the VAL server for location area unsubscribe response.

Table 9.3.2.g-1: Location area unsubscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Subscription status | M | It indicates the request result |

* * * Next Change * * * *

### 9.3.x	Obtaining location area information procedure

#### 9.3.x.1	Location area subscribe procedure

Figure 9.3.x.1-1 illustrates the high level procedure of location area subscription request. The same procedure can be applied for location management client and other SEAL servers that would like to subscribe to the list of UEs moving in or moving out of the specific location area. The subscribe request can be for own reference UE or any other reference UE for which the subscriber is authorized to monitor location information.

Figure 9.3.x.1-1: Location area information subscription procedure

1.	The VAL server sends a location area subscription request to the location management server to subscribe to the list of UEs moving in or moving out of the specific location area. In the request message, the VAL server includes the information as specified in Table 9.3.2.a-1. The location information criteria may include the geographic location information where the UEs moving in or moving out to be monitored, or it may include reference UE information where in the UEs moving in or moving out of given proximity range from the reference UE (target UE) to be monitored. The reference UE information may include VAL UE ID, External Identifier of the UE, GPSI, MSISDN, like so.

2.	The location management server shall check if the VAL server is authorized to initiate the location area subscription request.

3.	The location management server replies with a location area subscription response indicating the subscription status. In the response message, the location management server includes the information as specified in Table 9.3.2.b-1.

#### 9.3.x.2	Location area subscribe modify procedure

Figure 9.3.x.2-1 illustrates the high level procedure of location area subscribe modify request. The same procedure can be applied for location management client and other SEAL servers that would like to modify the subscription to the list of UEs moving in or moving out of the specific location area. The subscribe modify request can be for own reference UE or any other reference UE for which the subscriber is authorized to monitor location information.

Figure 9.3.x.2-1: Location area information subscription modify procedure

1.	The VAL server sends a location area subscription modify request to the location management server to modify the subscription to the list of UEs moving in or moving out of the specific location area. In the request message, the VAL server includes the information as specified in Table 9.3.2.d-1.

2.	The location management server shall check if the VAL server is authorized to initiate the location area subscription modification request.

3.	The location management server replies with a location area subscription modify response indicating the subscription status. In the response message, the location management server includes the information as specified in Table 9.3.2.e-1.

#### 9.3.x.3	Location area unsubscribe procedure

Figure 9.3.x.3-1 illustrates the high level procedure of location area unsubscribe request. The same procedure can be applied for location management client and other SEAL servers that would like to unsubscribe to the list of UEs moving in or moving out of the specific location area. The unsubscribe request can be for own reference UE or any other reference UE for which the subscriber is authorized to monitor location information.

Figure 9.3.x.3-1: Location area information unsubscribe procedure

1.	The VAL server sends a location area subscription modify request to the location management server to modify the subscription to the list of UEs moving in or moving out of the specific location area. In the request message, the VAL server includes the information as specified in Table 9.3.2.f-1.

2.	The location management server replies with a location area subscription response indicating the subscription status. In the response message, the location management server includes the information as specified in Table 9.3.2.g-1.

#### 9.3.x.4	Location area notification procedure

Figure 9.3.x.4-1 illustrates the high level procedure of location area information notification. The same procedure can be applied for location management client and other SEAL servers who have subscribe to the list of the UEs moving in or moving out of the specific location area.

Figure 9.3.x.4-1: Location are information notification procedure

1.	One of the event occurs at the location management server as specified in the subscribe request. The location management server identifies the UEs which are moved into the area or moved out of the area based on the location data and time stamp of the location. The location management server also determines that the age of the location of the UEs is not more than the age value specified in the subscribe request, if available. The LMS may report the list of all UEs in the given location or UEs moved in and moved out from the last reported UEs list.

2.	The location management server sends a location area information notification to the VAL server. In the notification message, the location management server includes the information as specified in Table 9.3.2.c-1.

* * * End of Change * * * *