3GPP TSG-SA WG6 Meeting #45-bis-e	S6-212256

e-meeting, 11th – 19th October 2021	(revision of S6-21xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0299 | rev | - | Current version: | 17.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of subclauses to capture MC gateway UE function details |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung, Nokia, Nokia Shanghai Bell, BDBOS |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCGWUE |  |  |  |  |  | Date: |  |  | 2021-10-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This CR introduces sub clauses to capture the details of functionalities and procedures for supporting MC gateway UE function. It provides the skeletal outline for this new section so that details can be filled. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New section 11 and sub clauses are introduced to capture the relevant details. Added descriptive text to Section 11.1 General. Editor’s Note is added to specify that details are to be filled. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * * * * FIRST CHANGE * * * * * * *

# 11	MC gateway UE

# Editor's Note:	This clause captures the sub clauses required for MC gateway UE function. Further sub clauses can be introduced on need basis.

## 11.1	General

An MC gateway UE has the functionality of providing service access with the MC service system for multiple MC service clients. The MC gateway UE enables MC service access for those MC service clients operating on devices that have no 3GPP access and also enables MC service access for the devices which cannot host MC service clients.

The MC gateway UE service may provide the following necessary functions:

-	Allowing an MC service client to connect to a MC gateway UE.

-	Allowing to serve an MC service user using an MC service client on a device which is connected to the MC gateway UE via non-3GPP transport.

-	Allowing an MC service client to choose between multiple MC gateway UEs for the required MC service capability.

-	Support of MC service continuation while the MC service client changes the MC gateway UE association.

-	Ensure that the content of communications between the MC system and MC service client connected to an MC gateway UE is unaltered.

-	Ensure that the communication traffic attributes, e.g. priority and QoS, of an MC service client connected to an MC gateway UE remains independent from other MC service clients concurrently connected to the same MC gateway UE.

-	Independent MC service client identification, authorisation and service profile association using the MC gateway UE functionality.

-	Sharing of an MC gateway UE transport resources with multiple connected MC service clients.

-	Sending an indication to the connected MC serving clients when the MC gateway UE cannot serve the connected MC service clients (any longer).

## 11.2	Functional Model

### 11.2.1	General

Editor's Note:	This clause will provide the general description for functional model.

## 11.3	Using IMS identities behind the MC gateway UE

### 11.3.1	General

Editor's Note:	This clause will provide the general description of using IMS identities for identification of MC service users behind an MC gateway UE residing on non-3GPP devices.

## 11.4	Routing of data and signalling by the MC gateway UE

### 11.4.1	General

Editor's Note:	This clause will capture the details of how the MC gateway UE handles the routing of data and signalling traffic.

## 11.5	Procedures and information flows

Editor's Note:	This clause will capture the procedures and information flows for MC gateway UE function.

### 11.5.1	Connection authorisation mechanisms

Editor's Note:	This clause will capture the procedures and information flows for connection authorisation.

### 11.5.2	3GPP access network related location information management

Editor's Note:	This clause will capture the procedures and information flows for 3GPP related location information management.

### 11.5.3	MBMS support for MC clients residing on non-3GPP devices

Editor's Note:	This clause will capture the procedures and information flows for MBMS support.

* * * * * * * END CHANGES * * * * * * *