3GPP TSG-SA WG6 Meeting #49-e	S6-221038

16th – 25th May 2022, Online	(revision of S6-220xxx)

Source:	vivo

Title:	Solution for in KI#1 – PIN Profile

Spec:	3GPP TR 23.700-78

Agenda item:	9.13

Document for:	Approval

Contact:	Huazhang Lv (Huazhang.lv@vivo.com)

1. Introduction

This pCR proposes a solution for KI#1.

2. Reason for Change

This solution addresses aspects of Key Issue #1.

For an established PIN, the PIN should have the profile or configuration information in PIN server, PEMC and PEGC.

In this paper, it mainly discusses the PIN Profile and configuration information related to PIN.

3. Conclusions

N/A

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-78 v 0.2.0.

* * * First Change, all new text * * * *

## 7.X	Solution #X: PIN Profile

### 7.X.1	Architecture enhancements

The architecture of PIN is referred to the solution 1 in clause 7.

### 7.X.2	Solution description

#### 7.X.2.1	General

This solution addresses aspects of Key Issue #1.

For an established PIN, the PIN should have the profile or configuration information in PIN server, PEMC and PEGC.

#### 7.X.2.2	PIN Profile in a PIN

Table 7.X.2.2.1-1: PIN Profile in PIN server, PEMC, PEGC and PINE

| Parameter Name | Parameter Description | PIN Server | PEMC | PEGC | PINE |
| --- | --- | --- | --- | --- | --- |
| PIN ID | The identifier of the PIN | Y | Y | Y | Y |
| PIN Description | Human-readable description of the PIN, for example, the company name, location or the type of service. | Y | Y | Y | Y |
| PEMC ID list | The list of identifiers of the PIN elements which can be allowed to take the role as PEMC (e.g.: PIN client ID, UE GPSI etc.,) | Y | Y | Y | Y |
| PEMC Endpoint | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the PEMC. | Y | N | Y | Y |
| PEGC ID list | The list of identifiers of the PIN elements which can be allowed to take the role as PEGC (e.g.: PIN client ID, UE GPSI etc.,) | Y | Y | Y | Y |
| PEGC Endpoint | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the PEGC. | Y | Y | N | Y |
| PIN Server ID | The identifier of the PIN server that serves the PIN | N | Y | Y | Y |
| PIN server Endpoint | Endpoint information (e.g. URI, FQDN, IP address) used to communicate with the PIN server. | N | Y | Y | Y |
| PIN Elements List (see NOTE) | List of PIN elements currently registered/joined the PIN > PIN element ID | Y | Y | Y | N |

Editor’s note: Other parameters will be defined of updated in PIN in future.

* * * * End-of Changes * * * *