3GPP TSG-SA WG6 Meeting #49-e	S6-221337

16th – 25th May 2022, Online	(revision of S6-220xxx)

Source:	vivo

Title:	Update for Solution 1 in KI#1 – PIN architecture update

Spec:	3GPP TR 23.700-78

Agenda item:	9.13

Document for:	Approval

Contact:	Huazhang Lv (Huazhang.lv@vivo.com)

1. Introduction

This pCR proposes a solution update for solution 1.

2. Reason for Change

For the PIN architecture, the PIN element in PIN can also interact with other PIN elements outside of PIN and the application layer. So, another PIN architecture is listed.

And the PIN client, application client is also should be described in the text.

3. Conclusions

N/A

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-78 v 0.2.0.

* * * First Change * * * *

## 7.2	Solution #1: PINAPP architecture

Editor's note:	Further updates of PINAPP architecture is needed.

### 7.2.1	Architecture assumption

The Figure 7.2.1-1 shows the application architecture for enabling PINAPP.

Figure 7.2.1-1: PINAPP architecture

NOTE 1:	It is possible that an application client on PIN elements can communicate with application server directly via 5GS or indirectly via PEGC.

NOTE 2:	It is possible that an application client can communicate with other application client in the same PIN directly or via PEGC.

NOTE 3:	It is possible that an application client can communicate with other application client in another PIN via PEGC.

Editor’s Note: Whether the suggested architecture for PINAPP is sufficient to meet the PIN related requirements or not is FFS

### 7.2.2	Functional elements

The functional entities of the application architecture for enabling PINAPP are described in this clause.

PEMC: A PIN Element with Management Capability is a PIN Element that provides a means for an authorised administrator to configure and manage a PIN.

PIN server: Deployed by operator in network, and provisioning of configuration information to UE (PINAPP). Authorization of the Creation request of PIN, and arranges the PEGC information about access control to PIN.

PEGC: A PIN Element with Gateway Capability is a PIN Element that provide any 5G services to any UE or PINAPP via a PEGC connected.

PIN client: The PIN enabler layer deployed in PIN elements to enable the management procedure to PEMC and routing control procedure to PEGC.

Application client: AC is the application resident in the PIN elements performing the client function.

### 7.2.3	Reference Points

Editor's note:	The supports of each reference point is FFS.

PIN - 1: The interactions related to enabling PINAPP, between the Application client and the PIN client.

PIN - 2: The interactions related to enabling PINAPP, between the PIN client and PEGC.

PIN - 3: The interactions related to enabling PINAPP, between the PIN client and PEMC.

PIN - 4: The interactions related to enabling PINAPP, between the PEGC and PEMC.

PIN - 5: The interactions related to enabling PINAPP, between the one PIN client and another PIN client.

PIN - 6: The interactions related to enabling PINAPP, between the one PEMC and PIN server.

PIN - 7: The interactions related to enabling PINAPP, between the one PEGC and PIN server.

PIN - 8: The interactions related to enabling PINAPP, between the one PIN server and 3GPP core network.

PIN - 9: The interactions related to enabling PINAPP, between the application server and PIN server.

PIN – 10: The interactions related to enabling PINAPP, between the PIN client in PIN element and PIN server.

### 7.2.4	Functional Entity Cardinality

#### 7.2.4.1	PEMC

The following cardinality rules apply for PEMC:

a)	one or multiple PEMCs per PIN.

#### 7.2.4.2	PEGC

The following cardinality rules apply for PEGC:

a)	one or multiple PEGCs per PIN.

#### 7.2.4.3	PIN server

The following cardinality rules apply for PIN server:

a)	one PIN server per PIN.

b)	Multiple PINs per PIN server.

NOTE:	There are multiple PIN server deployed in a PLMN.

#### 7.2.4.4	PIN client

The following cardinality rules apply for PIN client:

a)	one or more PIN clients per PIN.

b)	one PIN clients per PIN element.

#### 7.2.4.5	Application client

The following cardinality rules apply for application client:

a)	one or more application clients per PIN client.

* * * * End-of Changes * * * *