3GPP TSG-SA WG6 Meeting #51-e	S6-222799

e-meeting, 10th – 19th October 2022	(revision of S6-22xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0133 | rev | - | Current version: | 18.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | EES determines the selected ACR scenario |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph2 |  |  |  |  |  | Date: |  |  | 2022-09-28 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In current specification, multiple entities can detect ACR event and perform ACR procedure, so it may happen AC(s)/EEC has different capabilities with EAS(s)/EES, and select different approaches. Firstly, the EES has all the information from UE side (AC profile) and EAS side regarding the ACR scenarios supported by the respective entities. Secondly, not all ACR scenarios supported by ACs may be deployed by the EES. So, EES is the best entity to determine which ACR approach can be enabled for the AC and EAS considering the detailed information (e.g EAS features, KPIs) of both the AC and the EAS and thus also avoiding the overlap of ACR initiations by multiple entities which introduce unnecessary signalling to resolve co-existence issues. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The EES will decide selected ACR scenario for the AC. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Solution for EES determining the selected ACR scenario will remain under specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.x (new), 8.x.1 (new), 8.x.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

## 8.X	ACR scenario determination

### 8.X.1	General

There are multiple ACR scenarios defined in clause 8.8.2, and different entity (e.g. AC, EEC, EES, and EAS) may supported different ACR scenarios, requiring support from the enabling layer to determine the selected ACR scenario for certain AC.

This clause describes the features that the ACR scenario can be determined by the EEL to avoid the case that to avoid the case that different entity will choose different ACR scenario for the service continuity causing the ACR will not be triggered

### 8.X.2	EES determines the selected ACR scenario

Figure 8.x.2-1: EES determines the selected ACR scenario

1.	The EAS sends an ACR scenario selection subscription request to the EES.

2.	The EES sends an ACR scenario selection subscription response to the EAS with the subscription result.

3.	The EEC sends ACR scenario selection request to the EES, the request contains the AC service continuity support and EEC service continuity support indicating which ACR scenarios are supported by the AC and the EEC.

NOTE：The step 3 can be the enhancement on the EAS discovery request or the ACR scenario selection request which is up to stage3 to define.

4.	Upon the EES obtains the AC service continuity support and EEC service continuity support from UE and EAS service continuity support, then the EES determines one suitable ACR scenario for the AC based on the ACR scenarios supported by AC, EEC, EES and EAS. The EES can select an appropriate ACR scenario for the AC from the intersection of the ACR scenarios supported by AC, EEC, EES and EAS.

NOTE 1:	The ACR scenario supported by the EAS is available in the EAS service continuity support of the EAS profile.

Editor's note: It is FFS how the EES can know the supported service continuity scenarios of the T-EES and T-EAS.

5.	The EES sends the ACR scenario selection notification to the EAS, the notification contains the selected ACR scenario and related ACID.

6.	The EES sends the ACR scenario selection response to the EEC including the selected ACR scenario and related ACID, then the EEC may notify the AC with the selected ACR scenario.

NOTE 2:	Using multiple ACR scenario can detect ACR timely.

* * * END of Change * * * *