3GPP TSG-SA WG6 Meeting #52-Bis-e meeting	S6-230122

e-meeting, 11th – 20th January 2023

Source:	vivo, Samsung

Title:	PINAPP architeture

Spec:	3GPP TS 23.542 v0.0.0

Agenda item:	8.19

Document for:	Approval

Contact:	Huazhang Lv, huazhang.lv@vivo.com

1. Introduction

Add the the PINAPP architeture to section 6.2.1 of TS.

2. Reason for Change

There should be the reference point representation of the architeture for PINAPP. According to the conclusion of TR 23.700-78, the architeture is the same as in solution 1.

3. Conclusions

N/A

4. Proposal

It is proposed to agree the following changes to 3GPP TS 23.542 V0.0.0

* * * Start of Change 1 * * *

## 6.2	Architecture

### 6.2.1	General Architecture

Figure 6.2.1-1 illustrates the reference point representation of the architecture for PINAPP.

Figure 6.2.1-1: PINAPP architecture

The PIN elements contains PIN client and/or application clients. The PIN Element with gateway capability (PEGC) performs the role of an entity supporting gateway capability for PIN. The PIN Element with management capability (PEMC) performs the role of an entity supporting management capability for PIN.

NOTE 1:	It is possible that an application client on PIN elements can communicate with application server directly via 5GS or indirectly via PEGC.

NOTE 2:	It is possible that an application client can communicate with other application client in the same PIN directly or via PEGC.

NOTE 3:	It is possible that an application client can communicate with other application client in another PIN via PEGC.

* * * Start of Change 2 * * *

### 6.2.2	Architecture of user accessing services provided by PIN Element from outside the PIN

The Figure 6.2.2-1 shows the application architecture to enable authorized user to access services provided by PIN element behind the PEGC. For simplicity, not all functional elements of Figure 6.2.2-1 are shown in below Figure 6.2.2-1.

Figure 6.2.2-1: PINAPP architecture of User accessing services provided by PIN Element from outside the PIN

The interactions between PEGC and PIN client of the authorized user are supported over interface PIN-11. The interactions between PEMC and PIN client of the authorized user are supported over interface PIN-12. The authorized user uses PIN-12 to configure the policies in a PIN.

NOTE1: The authorized user is allowed to manage a PIN due to authorized user has PIN-12 interface to communicate with PEMC.

NOTE2: The reference point of PIN 11 and PIN 12 utilizes Uu reference point as described in 3GPP TS 23.401 and 3GPP TS 23.501.

* * * End of Change * * *