3GPP TSG-SA WG6 Meeting #53	S6-230523

Athens, Greece 27th February – 3rd March 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.554 | CR | 0106 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Device Triggering should not be restricted to the AS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | one2many |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMARCH_Ph2 |  |  |  |  |  | Date: |  |  | 2023-0214 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The MSGin5G Server triggering a recipient that has become unavailable when a message needs to be delivered to that recipient is restricted to messages send by an Application Server. There seems to be no reason for that restriction. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Device Triggering is performed by the MSGin5G Server for messages received from both an Application Server and MSGin5G UEs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Restricted functionality for which there seems to be no good reason. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.9.3.1, 8.9.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 8.9.3	MSGin5G device triggering

#### 8.9.3.1	General

MSGin5G device triggering is the means by which an MSGin5G Server receives an MSGin5G message and the 3GPP network device triggering capabilities, exposed via T8 /N33 reference point, are leveraged.  When the MSGin5G Server receives an MSGin5G message request, but the target MSGin5G UE is not reachable, the MSGin5G Server may use the 3GPP network device triggering mechanism to wake up the device and then deliver the payload to the destination.

#### 8.9.3.2	Procedure

Figure 8.9.3.2-1 shows the MSGin5G device triggering procedure.

Pre-conditions:

1.	The target UE is an MSGin5G UE.

2.	The MSGin5G Client is registered with the MSGin5G Server.

3.	At a later time, after the registration is completed, the MSGin5G UE becomes unreachable by the MSGin5G Server.

Figure 8.9.3.2-1: MSGin5G Triggering Procedure

1.	The MSGin5G Server receives an API request for sending an MSGin5G message, the API request includes the IEs as detailed in clause 9.1.2.1.

2.	If the MSGin5G Server determines that the recipient MSGin5G Client is not reachable, it initiates a device trigger request via the SCEF/NEF.

To determine the reachability of the target MSGin5G UE, the MSGin5G Server may use the UE reachability status monitoring procedure in clause 8.9.2. The MSGin5G Server may also use availability information provided by the MSGin5G Client at registration in the MSGin5G Client Communication Availability IE, as detailed in Table 8.2.1-1.

NOTE 1:	How the MSGin5G Server uses the MSGin5G Client Communication Availability IE, the UE reachability status monitoring procedure, or a combination thereof to make this determination is implementation specific.

NOTE 2:	If the recipient MSGin5G Client is reachable then the trigger request is not required, the MSGin5G Server sends the MSGin5G message as detailed in clause 8.3.3 and the rest of the steps in this procedure are skipped.

3.	The MSGin5G Server sends a request for Device Triggering via SCEF/NEF and determines the flow as detailed in clause 8.9.3.3.2. The Device Triggering request uses the UE Identifier, port number(s) and associated protocol information provided by the MSGin5G Client at registration in the MSGin5G Client Triggering Information IE.

The MSGin5G Server may use MSGin5G Client Communication Availability and/or pre-configured information to determine the timing of the Device Triggering request, e.g. the trigger may be sent to ensure that the target UE is reachable prior to resuming MSGin5G communications.

4.	The MSGin5G Server receives a response from SCEF/NEF indicating the success or failure status of the request, as detailed in clause 8.9.3.3.

5.	The device trigger is delivered to the target via SCEF/NEF and the Core Network. The targeted MSGin5G Client or Application Client receives the device trigger request.  The targeted MSGin5G Client or Application Client parses the payload of the trigger request and determines the device trigger purpose. The target UE becomes reachable, and the MSGin5G Client or Application Client becomes available for further MSGin5G communications.

6.	The MSGin5G Server receives a Device Triggering delivery status report from SCEF/NEF indicating the success of the delivery, as detailed in clause 8.9.3.3.

7.	The MSGin5G Server send a Device Triggering delivery status report response to SCEF/NEF to acknowledge the delivery status report, as detailed in clause 8.9.3.3.

8.	The MSGin5G Server responds to the request received in step 1.

Based on the trigger purpose derived from the payload, the targeted MSGin5G Client performs the corresponding actions (e.g. establish access network connectivity, contact the Application Server etc).