3GPP TSG-SA WG6 Meeting #55	S6-231760

Berlin, Germany 22nd – 26th May 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.554 | CR | 0130 | rev | - | Current version: | 18.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | update of clause 8.2.8 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMARCH_Ph2 |  |  |  |  |  | Date: |  |  | 2023-05-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The architecture of constrained device and UE is updated an agreed in S6-231645. This CR is proposed to update clause 8.2.8 addressing the selection of MSGin5G Gateway UE |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update the MSGin5G UE bulk registration procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MSGin5G UE bulk registration procedure is not aligned with the updated architecture. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8.2.8	Constrained device with MSGin5G Client selecting MSGin5G Gateway UE

The signalling flow for MSGin5G Client-2 in the MSGin5G UE-2 (which is a constrained device) to request other MSGin5G UEs supporting MSGin5G Gateway service functionality to provide their configuration is illustrated in figure 8.2.8-1.

Pre-condition:

1)	MSGin5G UE-2 is not able to communicate with MSGin5G Server directly;

2)	MSGin5G UE-2 discovers nearby MSGin5G UEs using ProSe direct discovery procedure as specified in 3GPP TS 23.304 [18].

3)	An MSGin5G UE-1a supports MSGin5G Gateway service functionality (i.e. MSGin5G UE-1a is an MSGin5G Gateway UE) and an MSGin5G UE-1b does not support MSGin5G Gateway service functionality (i.e. MSGin5G UE-1b is not an MSGin5G Gateway UE).

Figure 8.2.8-1: Constrained UE sending message to request configuration of the MSGin5G Gateway UE

1)	The MSGin5G Client-2 in MSGin5G UE-2 (which is a constrained UE) sends message to all surrounding MSGin5G UEs (using ProSe Direction communication as specified in 3GPP TS 23.304 [18]) to provide their gateway configuration if MSGin5G Gateway service functionality is supported by the MSGin5G UE, i.e. the MSGin5G UE is a MSGin5G Gateway UE. The request message includes information elements as specified in Table 8.2.8-1.

Table 8.2.8-1: Information elements for Request for gateway UE configuration

| Information element | Status | Description |
| --- | --- | --- |
| Layer-2 ID | M | Layer-2 identity of UE-2 |
| UE service ID | M | UE service identifier of the UE-2 |
| MSGin5G UE ID | O | MSGin5G device identifier of the UE-2 |
| Required packet size | M | Maximum allowed packet size of the UE-2 |

2)	Upon receiving the request from the MSGin5G Client of MSGin5G UE-2, the MSGin5G UE-1a supports the MSGin5G Gateway service functionality sends the response for the gateway configuration. The response message includes information elements as specified in Table 8.2.8-2. The MSGin5G UE-1b does not support the MSGin5G Gateway service functionality ignores the request for gateway UE configuration, i.e. should not send a corresponding response.

Table 8.2.8-2: Information elements for Response for gateway UE configuration

| Information element | Status | Description |
| --- | --- | --- |
| Layer-2 ID | M | Layer-2 identity of UE-1 |
| UE service ID | M | UE service identifier of the UE-1 |
| MSGin5G UE ID | O | MSGin5G device identifier of the UE-1 |
| Allowed packet size | M | Maximum allowed packet size of the UE-2 |

3)	If the MSGin5G UE-2 receives responses from multiple different MSGin5G Gateway UEs, then the MSGin5G Client of the MSGin5G UE-2 selects a MSGin5G Gateway UE whose value of the Allowed packet size is equal or more than the value of the Required packet size of the MSGin5G UE-2. If value of the Allowed packet size is less than the value of the Required packet size for all MSGin5G Gateway UEs, then the MSGin5G Client of the MSGin5G UE-2 selects the MSGin5G Gateway UE with the maximum value of the Allowed packet size among all MSGin5G Gateway UEs and further sets the maximum allowed packet size of the constrained UE to the value of the Allowed packet size of the selected MSGin5G Gateway UE.

Once MSGin5G UE selects the MSGin5G Gateway UE, the MSGin5G UE indicates the selected MSGin5G Gateway UE by registering to use the gateway service. The signalling flow for MSGin5G Client in the MSGin5G UE (which is a constrained device) to register itself to use gateway service of the MSGin5G Gateway UE is illustrated in figure 8.2.8-2.

Figure 8.2.8-2: Constrained UE registering to use MSGin5G Gateway UE

1)	Upon selecting the MSGin5G Gateway UE, the MSGin5G Client of the MSGin5G UE sends request message to register with the selected MSGin5G Gateway UE to indicate it about the use of gateway service by the MSGin5G UE (i.e. constrained device). The request message may also include the other required parameters like UE service ID and MSGin5G UE ID of the MSGin5G UE (i.e. constrained UE) and the time till when constrained device is intended to use the gateway service from the selected MSGin5G Gateway UE.

2)	Upon receiving the request from the MSGin5G Client of the MSGin5G UE, the MSGin5G Gateway Client in MSGin5G Gateway UE checks whether the MSGin5G UE is authorized to use the gateway service or not. And if the MSGin5G UE is authorized, the MSGin5G Gateway Client in MSGin5G Gateway UE sends the response message with the status of the registration. If the registration is success, the response message also includes other parameters like the accepted time till when constrained device is allowed to use the gateway service from the selected MSGin5G Gateway UE.