3GPP TSG-SA WG6 Meeting #55	S6-232429

Berlin, 22th – 26th May 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.222 | CR | 0118 | rev |  | Current version: | 17.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Revoking notification to other AEFs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eCAPIF |  |  |  |  |  | Date: |  |  | 2023-08-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | 18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The revocation of API invoker authorization is per service API granularity as indicated in the clause 8.23.2.1 of 23.222. #### 8.23.2.1 Revoke API invoker authorization request Table 8.23.2.1-1 describes the information flow revoke API invoker authorization request from the API exposing function to the CAPIF core function or from the CAPIF core function to the API exposing function. Table 8.23.2.1-1: Revoke API invoker authorization request Information element; Status; Description. API invoker identity information; M; The information that determines the identity of the API invoker. Service API identification; M; The identification information of the service API for which the authorization is revoked.. Cause; M; The cause for revoking the API invoker authorization. Now in some cases (e.g.,the distributed deployment case), multiple AEFs may have the same capability and provide the same API. The clause 8.2.4.2.2 of 29.222 defines the servcieAPIDescriptions allow one API to have multiple AEF profiles. ##### 8.2.4.2.2 Type: ServiceAPIDescription Table 8.2.4.2.2-1: Definition of type ServiceAPIDescription Attribute name; Data type; P; Cardinality; Description; Applicability. apiName; string; M; 1; API name, it is set as {apiName} part of the URI structure as defined in subclause 4.4 of 3GPP TS 29.501 [18].. apiId; string; O; 0..1; API identifier assigned by the CAPIF core function to the published service API. Shall not be present in the HTTP POST request from the API publishing function to the CAPIF core function. Shall be present in the HTTP POST response from the CAPIF core function to the API publishing function and in the HTTP GET response from the CAPIF core function to the API invoker (discovery API).. aefProfiles; array(AefProfile); C; 1..N; AEF profile information, which includes the exposed API details (e.g. protocol). For CAPIF-4/4e interface, API publishing function shall provide this attribute to the CAPIF core function in service API publishing. For CAPIF-1/1e interface, the CAPIF core function shall provide this attribute to the API Invoker during service API discovery. (NOTE 2). description; string; O; 0..1; Text description of the API. …; …; .; …; …; …. In this case, when a certain service API invocation is revoked by one of those AEFs, the other ones shall be notfied. However, this is missing in TS 23.222. |
| Summary of change: |  | 1) Update the clause 8.23.3 to notify other AEFs who provide the same API 2) Upate the clause 8.23.4 to reflect the case that multiple AEFs may have the same capability and provide the same API. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Access control of the API invocation at certain AEF(s) may be compromised when multiple AEF provides the same service API. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.23.3, 8.23.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is revised from S6-231832: update the Reason for change to refer corresponding clause in 23.222 and 29.222 which indicate that one service API may have multiple AEF profiles, and the revocation of API invocation is per service API. |  |  |  |  |  |  |  |  |

# /******************** First Changes ********************/

### 8.23.3	Procedure for CAPIF revoking API invoker authorization initiated by AEF

Figure 8.23.3-1 illustrates the procedure for revoking API invoker authorization to access service API initiated by the AEF.

Pre-conditions:

1.	The API invoker is authenticated and authorized to use the service API.

2.	The AEF in the CAPIF is configured with the access policy to be applied to the service API invocation corresponding to the API invoker and the service API.

3.	Authorization details of the AEF are available with the CAPIF core function.

API invoker

CAPIF Core function

AEF

2. Revoke API invoker authorization request

1. Trigger for revoking API invoker authorization

3. Invalidate the authorization of the API invoker for service API

4. Revoke API invoker authorization response

6. Revoke API invoker authorization notify

5. Invalidate the authorization of the API invoker for service API

7. Notify other AEFs to invalidate the API invoker authorization corresponding to the service API as described in step 2~4 of clause 8.23.4

Figure 8.23.3-1: Procedure for revoking API invoker authorization initiated by AEF

1.	The AEF triggers the revocation of the API invoker authorization.

2.	The AEF sends revoke API invoker authorization request to the CAPIF core function with the details of the API invoker and the service API.

3.	Upon receiving the information to revoke the API invoker's authorization for service API invocation, the CAPIF core function invalidates the API invoker authorization corresponding to the service API.

4.	The CAPIF core function sends a revoke API invoker authorization response to the AEF.

5.	Upon successful revocation of API invoker authorization corresponding to the service API at the CAPIF core function, the AEF invalidates the API invoker authorization corresponding to the service API.

6.	The CAPIF core function sends a revoke API invoker authorization notify to the API invoker whose authorization to access the service API has been revoked.

7.	Optionally, if multiple AEFs provides the same service API is provided by multiple AEFs, the CAPIF core function based on the cause from step 2, may notify the other AEFs which provides the same service APIs to invalidate the API invoker authorization corresponding to the service API as described in step 2 to step 4 of clause 8.23.4.

NOTE:	Only the AEFs which provides the service API and being discovered by the API invoker needs to be notified.

### 8.23.4	Procedure for CAPIF revoking API invoker authorization initiated by CAPIF core function

Figure 8.23.4-1 illustrates the procedure for revoking API invoker authorization to access service API initiated by the CAPIF core function.

Pre-conditions:

1.	The API invoker is authenticated and authorized to use the service API.

2.	The AEF in the CAPIF is configured with the access policy to be applied to the service API invocation corresponding to the API invoker and the service API.

Figure 8.23.4-1: Procedure for revoking API invoker authorization initiated by CAPIF core function

1.	The CAPIF core function triggers the revocation of the API invoker authorization.

2.	The CAPIF core function sends revoke API invoker authorization request to the AEF with the details of the API invoker and the service API.

NOTE:	The step 2 can be sent to multiple AEFs if applicable.

3.	Upon receiving the information to revoke the API invoker's authorization for service API invocation, the AEF invalidates the API invoker authorization corresponding to the service API.

4.	The AEF sends a revoke API invoker authorization response to the CAPIF core function.

5.	The CAPIF core function invalidates the API invoker authorization corresponding to the service API.

6.	The CAPIF core function sends a revoke API invoker authorization notify to the API invoker whose authorization to access the service API has been revoked.