3GPP TSG-SA WG6 Meeting #56	S6-232613

Goteborg, Sweden, 21st – 25th August 2023	(revision of S6-232270 and 2550)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.554 | CR | 0148 | rev | 2 | Current version: | 18.4.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections to MSGin5G UE bulk (de-)registration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | one2many |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GMARCH_Ph2 |  |  |  |  |  | Date: |  |  | 2023-08-24 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | If the MSGin5G Gateway UE receives an urgent registration request from an MSGin5G UE and a queue with at least one registration request for bulk-handling already exists, the MSGin5G Gateway UE should have the option to add the urgent registration request to the existing queue and subsequently send the bulk registration request to the MSGin5G Server. Under the current specification this implementation choice is prohibited. The description for the Registration request expiration time IE in table 8.2.7-1: "Indicates the maximum processing time of the registration request allowed by the MSGin5G Client which sent the registration request." However, processing of the request is not only done by the MSGin5G Client, but also by the MSGin5G Server. The MSGin5G Client has no control over the maximum processing time of the request. The text of step 2b is already included in the text of the common part of step 2. Furthermore, it should implementation specific if the MSGin5G Client-1 is going to check for each and every registration request if bulk-registration is supported at all. The same reasons for change apply to both clause 8.2.7 and 8.2.11. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The text is modified to allow the urgent registration request not to bypass the existing queue, but to be added to the queue and for the queue to be sent immediately afterwards to the MSGin5G Server. The description of the Registration request expiration time IE should indicate the maximum wait time allowed by the MSGin5G Client before which the MSGin5G Client shall send the registration request to the MSGin5G Server. Removal of step 2b. Remove "application provider" from note 2. The same changes are applied to clause 8.2.7 and 8.2.11. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification will contain unnecessary implementation requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.7, 8.2.11 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Merged S6-232365 into Revision 1. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 8.2.7	MSGin5G UE bulk registration over MSGin5G-6 reference point

When MSGin5G UE-1 and UE-2, which support an MSGin5G Client, perform registration via MSGin5G Gateway UE to contact the MSGin5G Server, the MSGin5G Gateway service functionality in the MSGin5G Gateway UE may decide to use bulk registration procedure specified in this clause based on the registration request sent from MSGin5G Clients in MSGin5G UE-1 and MSGin5G UE-2, if allowed by service policy. The procedure for MSGin5G UE bulk registration is illustrated in figure 8.2.7-1.

Pre-conditions:

1.	The MSGin5G Gateway UE has registered to the MSGin5G Server successfully.

2.	MSGin5G UE-1 and MSGin5G UE-2 have discovered and selected an MSGin5G Gateway UE as specified in clause 8.2.8 and connected to the serving network via MSGin5G Gateway UE successfully.

3.	UE Service IDs and the MSGin5G Server addresses have been configured on the MSGin5G Gateway UE, MSGin5G UE-1 and MSGin5G UE-2. All these MSGin5G UEs are served by a same MSGin5G Server.

4.	Both the MSGin5G Ues and MSGin5G Server have been configured with the necessary credentials to enable authenticating one another.

Figure 8.2.7-1: MSGin5G Client bulk registration

- The MSGin5G Client-1 sends an MSGin5G UE registration request to the MSGin5G Server as specified in clause 8.2.1, In addition to the Information Elements specified in table 8.2.1-1, the MSGin5G Client-1 may also add the Information Elements specified in table 8.2.7-1 in the MSGin5G UE registration request.

Table 8.2.7-1: MSGin5G UE registration request

| Information element | Status | Description |
| --- | --- | --- |
| Registration urgent degree | O | Indicates whether the registration request is urgent or not. |
| Wait time | O | Indicates the maximum wait time of the registration request allowed by the MSGin5G Client which sent the registration request before the bulk registration request is sent to the MSGin5G Server. |

2.	Upon receiving the MSGin5G UE registration request, the MSGin5G Gateway Client checks whether this request can be queued for bulkhandling based on the service policy and the information included in the MSGin5G UE registration request, e.g.

whether the MSGin5G UE registration request is urgent.

NOTE 1:	Whether bulk registration is supported can be configured in the MSGin5G Gateway Client.

NOTE 2:	The bulk registration conditions are specified by the application provider or MSGin5G Service provider and are out of scope of this document.

3.	Based on step 2:

a)	if the MSGin5G UE registration request is urgent, MSGin5G Gateway Client forwards the registration request to MSGin5G Server, as part of a bulk registration request if other registration requests were already in the queue for bulk handling; or

b)	if the MSGin5G UE registration request can be bulk-handled, MSGin5G Gateway Client sends a bulk registration notification to MSGin5G Client-1 and caches/stores the MSGin5G UE registration request until the bulk registration conditions can be fulfilled. The Information Elements specified in table 8.2.7-2 are included in the bulk registration notification. The steps 4 and 5 are skipped.

Table 8.2.7-2: bulk registration notification

| Information element | Status | Description |
| --- | --- | --- |
| UE Service ID | M | UE service identifier assigned to the requesting MSGin5G UE. |
| Expected registration time | O | The expected time when the bulk registration can be handled and the MSGin5G UE registration Response can be received. |

4.	The MSGin5G Server handles the MSGin5G UE registration request as specified in clause 8.2.1 and sends the corresponding MSGin5G UE registration response to MSGin5G Gateway Client.

5.	The MSGin5G Gateway Client sends the MSGin5G UE registration response to MSGin5G Client-1. The MSGin5G Client-1 handles the MSGin5G UE registration response as specified in clause 8.2.1 and skips all the remaining steps in the present clause.

6.	MSGin5G Client-2 sends an MSGin5G UE registration request and the MSGin5G UE registration request is handed as in steps 2-5.

7.	The MSGin5G Gateway Client finds that the conditions of sending a bulk registration request can be fulfilled, e.g.

a)	the registration request expiration time in the MSGin5G UE registration request sent from MSGin5G Client-1 or MSGin5G Client-2 is reached; or

b)	the periodic bulk registration interval is reached; or

c)	the maximum MSGin5G UE registration request number in a MSGin5G UE bulk registration request is reached.

NOTE 3:	The maximum registration time, periodic bulk registration interval and maximum MSGin5G UE registration request number are implementation specific and out of the scope of the current specification.

8.	The MSGin5G Gateway Client includes all cached/stored MSGin5G UE registration requests in an MSGin5G UE bulk registration request and sends it to the MSGin5G Server. The Information Elements specified in table 8.2.7-3 are included in the MSGin5G UE bulk registration request.

Table 8.2.7-3: MSGin5G UE bulk registration request

| Information element | Status | Description |
| --- | --- | --- |
| Number of individual MSGin5G UE registration requests | M | Indicates total number of MSGin5G UE registration requests which are bulked in this MSGin5G UE bulk registration request |
| List of individual MSGin5G UE registration request | M | Each element in this list contains information as specified in Table 8.2.1-1. |

9.	Upon receiving the MSGin5G UE bulk registration request, the MSGin5G Server splits the MSGin5G UE bulk registration request, executes the authentication, authorization and registration procedures for each MSGin5G Client individually as specified in clause 8.2.1.

Editor's note:	Security aspects of bulk registrationis the responsibility of SA3.

10.	The MSGin5G Server includes all corresponding MSGin5G UE registration responses in an MSGin5G UE bulk registration response and sends it to MSGin5G Gateway Client. The Information Elements specified in table 8.2.7-4 are included in the MSGin5G UE bulk registration response.

Table 8.2.7-4: MSGin5G UE bulk registration response

| Information element | Status | Description |
| --- | --- | --- |
| Number of individual MSGin5G UE registration responses | M | Indicates total number of MSGin5G UE registration responses which are bulked in this MSGin5G UE bulk registration response |
| List of individual MSGin5G UE registration response | M | Each element in this list contains information as specified in Table 8.2.1-4. |

11.	Upon receiving the MSGin5G UE bulk registration response, the MSGin5G Gateway Client splits the MSGin5G UE bulk registration response into multiple individual MSGin5G UE registration responses. Each individual MSGin5G UE registration response contains information as specified in Table 8.2.1-4.

12.	MSGin5G Gateway Client sends the MSGin5G UE registration Response 1 to MSGin5G Client-1.

13.	MSGin5G Gateway Client sends the MSGin5G UE registration Response 2 to MSGin5G Client-2.

* * * Next Change * * * *

### 8.2.11	MSGin5G UE bulk de-registration over MSGin5G-6 reference point

The MSGin5G Gateway UE may decide to use bulk de-registration procedure specified in this clause based on the de-registration request sent from MSGin5G Clients in MSGin5G Ues or, if allowed by service policy. The procedure for MSGin5G UE bulk de-registration is illustrated in figure 8.2.11-1.

Pre-conditions:

1.	The MSGin5G Gateway UE has registered to the MSGin5G Server successfully.

2.	MSGin5G UE-1 and MSGin5G UE-2 have registered to the MSGin5G Server via MSGin5G Gateway UE successfully as specified in clause 8.2.8.

Figure 8.2.11-1: MSGin5G UE bulk de-registration

1.	The MSGin5G Client-1 sends an MSGin5G UE de-registration request to the MSGin5G Server as specified in clause 8.2.2, In addition to the Information Elements specified in table 8.2.2-1, the MSGin5G Client-1 may also add the Information Elements specified in table 8.2.11-1 in the MSGin5G UE de-registration request.

Table 8.2.11-1: MSGin5G UE de-registration request

| Information element | Status | Description |
| --- | --- | --- |
| De-registration urgent degree | O | Indicates whether the de-registration request is urgent or not. |
| Wait time | O | Indicates the maximum wait time of the de-registration request allowed by the MSGin5G Client-2 before the bulk de-registration request is sent to the MSGin5G Server. |

2.	Upon receiving the MSGin5G UE de-registration request, the MSGin5G Gateway Client checks whether this request can be queued for bulk-handling based on the service policy and the information included in the MSGin5G UE registration request, e.g.

whether the MSGin5G UE de-registration request is urgent.

NOTE 1:	Whether bulk de-registration is supported can be configured in the MSGin5G Gateway Client.

NOTE 2:	The bulk de-registration conditions are specified by theapplication provider or MSGin5G Service provider and are out of scope of this document.

3.	Based on step 2:

a)	if the MSGin5G UE de-registration request is urgent, the MSGin5G Gateway Client may also include other registration requests which were already in the queue for bulk handling; or

b)	if the MSGin5G UE de-registration request can be bulked, MSGin5G Client-1 sends a bulk de-registration notification to MSGin5G Client-1 and caches/stores the MSGin5G UE de-registration request until the bulk de-registration conditions can be fulfilled. The Information Elements specified in table 8.2.11-2 are included in the bulk de-registration notification. The steps 4 and 5 are skipped.

Table 8.2.11-2: bulk de-registration notification

| Information element | Status | Description |
| --- | --- | --- |
| UE Service ID | M | UE service identifier assigned to the requesting MSGin5G UE. |
| Expected de-registration time | O | The expected time when the bulk de-registration can be handled and the MSGin5G UE de-registration response can be received. |

4.	The MSGin5G Server handles the MSGin5G UE de-registration request as specified in clause 8.2.2 and sends the corresponding MSGin5G UE de-registration response to MSGin5G Gateway Client.

5.	The MSGin5G Gateway Client sends the MSGin5G UE de-registration response to MSGin5G Client-1. The MSGin5G Client-1 handles the MSGin5G UE de-registration response as specified in clause 8.2.2 and skips all the remaining steps in the present clause.

6.	MSGin5G Client-2 sends an MSGin5G UE de-registration request and the MSGin5G UE de-registration request is handled as in steps 2-5.

7.	The MSGin5G Gateway Client finds that the conditions of sending a bulk de-registration request can be fulfilled, e.g.

a)	the de-registration request expiration time in the MSGin5G UE de-registration request sent from MSGin5G Client-1 or MSGin5G Client-2 is reached; or

b)	the periodic bulk de-registration interval is reached; or

c)	the maximum MSGin5G UE de-registration request number in a MSGin5G UE bulk de-registration request is reached.

NOTE 3:	The maximum de-registration time, periodic bulk de-registration interval and maximum MSGin5G UE de-registration request number are implementation specific and out of the scope of the current specification.

8.	The MSGin5G Gateway Client includes all cached/stored MSGin5G UE de-registration requests in an MSGin5G UE bulk de-registration request and sends it to the MSGin5G Server. The Information Elements specified in table 8.2.11-3 are included in the MSGin5G UE bulk de-registration request.

Table 8.2.11-3: MSGin5G UE bulk de-registration request

| Information element | Status | Description |
| --- | --- | --- |
| Number of individual MSGin5G UE de-registration requests | M | Indicates total number of MSGin5G UE de-registration requests which are bulked in this MSGin5G UE bulk de-registration request |
| List of individual MSGin5G UE de-registration request | M | Each element in this list contains information as specified in Table 8.2.2-1. |

9.	Upon receiving the MSGin5G UE bulk de-registration request, the MSGin5G Server processes each of the MSGin5G UE de-registration from the list individually as specified in clause 8.2.2.

Editor's note:	Security aspects of MSGin5G UE bulk de-registration is the responsibility of SA3.

10.	The MSGin5G Server includes all corresponding MSGin5G UE de-registration responses in an MSGin5G UE bulk de-registration response and sends it to MSGin5G Gateway Client. The Information Elements specified in table 8.2.11-4 are included in the MSGin5G UE bulk de-registration response.

Table 8.2.11-4: MSGin5G UE bulk de-registration response

| Information element | Status | Description |
| --- | --- | --- |
| Number of individual MSGin5G UE de-registration responses | M | Indicates total number of MSGin5G UE de-registration responses which are bulked in this MSGin5G UE bulk de-registration response |
| List of individual MSGin5G UE de-registration response | M | Each element in this list contains information as specified in Table 8.2.2-2 |

11.	Upon receiving the MSGin5G UE bulk de-registration response, the MSGin5G Gateway Client splits the MSGin5G UE bulk de-registration response into multiple individual MSGin5G UE de-registration responses. Each individual MSGin5G UE de-registration response contains information as specified in Table 8.2.2-2.

12.	MSGin5G Gateway Client sends the MSGin5G UE de-registration Response 1 to MSGin5G Client-1.

13.	MSGin5G Gateway Client sends the MSGin5G UE de-registration Response 2 to MSGin5G Client-2.