3GPP TSG-SA WG6 Meeting #58	S6-233601

Chicago, USA, 13th – 17th November 2023	(revision of S6-23xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0267 | rev | - | Current version: | 18.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Group configuration data subscription update |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEAL_Ph3 |  |  |  |  |  | Date: |  |  | 2023-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Currently, subscriptions for group configuration data modification can only be created. The issue with this approach is that when a group management client wants to update a subscription, e.g. Minimum time between consecutive notifications or Notification Target URI (new IE), it is not possible to allow this operation. The CRUD (Create-Read-Update-Delete) paradigm used in the RESTful mapping in stage 3 does require to allow an update operation. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Enable a group configuration subscription update procedure triggered by a group management client. - Additionally, enable providing a custom notification target URI and a subscription duration. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete stage 2 specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.3.2.22, 10.3.2.23, 10.3.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * * First Change * * * *

#### 10.3.2.22	Subscribe group configuration request

Table 10.3.2.22-1 describes the information flow from the group management client to the group management server for subscribe group configuration request and group configuration subscription update request .

Table 10.3.2.22-1: Subscribe group configuration request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity (see NOTE 2) | M | The identity of the group management client performing the request. |
| VAL group ID | M | VAL group ID of the group |
| VAL services requested (see NOTE 1) | O | Service(s) for which group configuration is requested |
| Notification target URI | M | Target URI where the group management client performing the request will receive the notifications. |
| Subscription duration | O | It indicates the subscription duration. |
| Minimum time between consecutive notifications | O | Defaults to 0 if not provided, otherwise indicates the minimum time interval between consecutive notifications |
| NOTE 1: If 'VAL services requested' is not present, group configuration is requested for all services defined for the VAL group. NOTE 2: This information element shall not be updated via group information subscription update procedure in clause 10.3.6.3. |  |  |

* * * * Next Change * * * *

#### 10.3.2.23	Subscribe group configuration response

Table 10.3.2.23-1 describes the information flow from the group management server to the group management client  for subscribe group configuration response and group configuration subscription update response.

Table 10.3.2.23-1: Subscribe group configuration response

| Information element | Status | Description |
| --- | --- | --- |
| VAL group ID | M | VAL group ID of the group |
| Result | M | Indicates the success or failure for the result |

* * * * Next Change * * * *

#### 10.3.6.3	Subscription and notification for group configuration data

The procedure for subscription for group configuration data as described in figure 10.3.6.3-1 is used by the group management client to indicate to the group management server that it wishes to receive updates of group configuration data for groups for which it is authorized.

Pre-conditions:

-	The group management server has some group configurations stored.

Figure 10.3.6.3-1: Subscription for group configurations

1.	The group management client subscribes to the group configuration information stored at the group management server using the subscribe group configuration request.

2.	The group management server provides a subscribe group configuration response to the group management client indicating success or failure of the request.

The procedure for notification of group configuration data as described in figure 10.3.6.3-2 is used by the group management server to inform the group management client that new group configuration data is available. It can also be used by the group management server to provide new group related key material to the group management client.

Pre-conditions:

-	The group management client has subscribed to the group configuration information

-	The group management server has received and stored new group configuration information, or the group management server has generated and stored new key material, or both of these have occurred.

Figure 10.3.6.3-2: Notification of group configurations

1.	The group management server provides the notification to the group management client, who previously subscribed for the group configuration information. Optionally, the notify group configuration request may contain group related key material for the group management client.

2.	The group management client provides a notify group configuration response to the group management server.

3.	If the group management server had provided group related key material to the group management client, the group management client stores the key material.

If the group management server has notified the group management client about new group configuration information through this procedure, the group management client may then follow the procedure described in subclause 10.3.6.2 in order to retrieve that group configuration information.

The information flow for updating an existing subscription for group configuration data is presented in figure 10.3.6.3-X. This procedure is used by the group management client to update some parameters related to an existing subscription, for instance the notification target URI, the subscription duration, or the minimum time between consecutive notifications.

Pre-conditions:

-	The group management client has an active subscription to group configuration data changes.

Figure 10.3.6.3-X: Update an existing subscription for group configurations

1.	The group management client decides to update an active subscription and sends a group configuration subscription update request to the group management server including the information specified in table 10.3.2.22-1.

2.	The group management server shall check if the group management client is authorized to update the subscription. If so, the group management server updates the subscription and provides a group configuration subscription update response to the group management including the information specified in table 10.3.2.23-1.

* * * * End of Changes * * * *