3GPP TSG-SA WG6 Meeting #58	S6-234031

Chicago, USA, 13th – 17th November 2023	(revision of S6-233992, S6-233510)

Source:	Samsung, Apple

Title:	Pseudo-CR on Removing EN related to EDGE-10 mapping with ETSI MEC

Spec:	3GPP TR 23.958 v1.1.1

Agenda item:	7.9

Document for:	Approval

Contact:	Sapan Shah

1. Introduction

This pCR proposes to remove EN related to EDGE-10 mapping with ETSI MEC

2. Reason for Change

Following EN exists in clause 5.1:

Editor’s Note: Mapping of EDGE-10 reference point with ETSI MEC architecture is FFS.

EDGE-10 is defined in TS 23.558 to exchange EDN information between ECSs/ECS-ERs for federation.

Mff reference point is defined as follows (in MEC 003):

Mff: The Mff reference point between MEFs within the MEC federation is used for sharing information (e.g. MEC system information).

Based on MEC 0040, the information registration happens between MEO (MEC Orchestrator) and MEF (MEC Federator). And the information exchange happens between MEFs (information related to MEO). As per Table 6.2.X-1 of MEC 040, the system information consists of system ID, system name, system provider and end point information.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.958 v1.1.1.

* * * First Change * * * *

## 5.1	General

Clause 5 provides the mapping of EAS Profile and AppInfo to allow application registration across the platforms.

* * * Next Change * * * *

## 5.x	Support for Federation

### 5.x.1	EDGEAPP

EDGE-9 and EDGE-10 reference points are used to support federation functionalities as per clause 8.17 and clause 8.18 of 3GPP TS 23.558 [2]. EDGE-9 reference point in EDGEAPP architecture is used to discover EAS from the EES of the partner ECSP for edge node sharing. EDGE-10 reference point in EDGEAPP architecture is used for ECS registration, ECS discovery via ECS-ER and Service provisioning information retrieval from ECS-ER in order to provide support for roaming, federation and edge node sharing. EDN information is exchanged between ECSs/ECS-ERs using EDGE-10 reference point.

Further, application life cycle management for federation is specified in 3GPP TS 28.538 [r28538]. Once application server is instantiated, it registeres with EES, which in turn registers with ECS. The EDN information contains EDN connection information, list of EES along with EASIDs, provider’s ID, endpoint information and other required details.

### 5.x.2	ETSI MEC

The MEC federator (MEF) as described in clause 7.1.9 of ETSI GS MEC 003 [4] enables a MEC federation between MEC systems. Mff reference point is defined between MEFs within the MEC federation for sharing information, such as MEC system information that includes MEC system ID, MEC system name, MEC system provider and MEF endpoint information. The federation enablement service is summarised in clause 5.2.1 of ETSI GS MEC 040 [6] and enables the shared usage of MEC services and applications across different systems.

* * * Next Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.558: "Architecture for enabling Edge Applications".

[3]		GSMA PRD OPG.02 - “Operator Platform Telco Edge Requirements Version 5.0”, https://www.gsma.com/futurenetworks/wp-content/uploads/2023/07/OPG.02-v5.0-Operator-Platform-Requirements-and-Architecture.pdf.

[4]	ETSI GS MEC 003 (V3.1.1), "Multi-access Edge Computing (MEC); Framework and Reference Architecture".

[5]	ETSI GS MEC 011 (V3.1.1), “Multi-access Edge Computing (MEC); Edge Platform Application Enablement”.

[6]	ETSI GS MEC 040 (V3.1.1), “Multi-access Edge Computing (MEC); Federation enablement APIs”.

[7]	ETSI GS MEC 021 V2.2.1, "Multi-access Edge Computing (MEC); Application Mobility Service API".

[8]	3GPP TS 28.538: "Management and orchestration; Edge Computing Management".

[9]	ETSI GS MEC 010-2 (V2.2.1), "Multi-access Edge Computing (MEC); MEC Management; Part 2: Application lifecycle, rules and requirements management".

[10]	3GPP TS 23.222: “Common API Framework for 3GPP Northbound APIs”.

[11]	ETSI GR MEC 031 (V3.1.1), “Multi-access Edge Computing (MEC); MEC 5G Integration”.

[12]	GSMA PRD OPG.03 - “Southbound Interface Network Resources APIs Version 3.0”, https://www.gsma.com/futurenetworks/wp-content/uploads/2023/07/OPG.03-v3.0-Southbound-Interface-Network-Resources-APIs.pdf.

[13]	GSMA PRD OPG.04 - “East-Westbound Interface APIs Version 3.0”, https://www.gsma.com/futurenetworks/wp-content/uploads/2023/07/OPG.04-v3.0-GSMA-Operator-Platform-Group-East-Westbound-Interface-APIs-Version-3.0.pdf.

[14]	GSMA PRD OPG.05 - “User-Network Interface APIs Version 1.0”, https://www.gsma.com/futurenetworks/wp-content/uploads/2023/03/GSMA-Operator-Platform-Group-User-Network-Interface-APIs-v1.pdf.

[r28538]	3GPP TS 28.538: "Management and orchestration; Edge Computing Management".

* * * End of Change * * * *