3GPP TSG-SA WG6 Meeting #59	S6-240207

Athens, Greece 26th February – 1st March 2024	(revision of S6-24xxxx)

Source:	Lenovo

Title:	Solution on FL event notifications

Spec:	3GPP TR 23.700-82 v0.2.0

Agenda item:	8.3

Document for:	Approval

Contact:	Emmanouil Pateromichelakis <epateromiche@lenovo.com>

1. Introduction

This paper proposes a solution for KI #3.

2. Reason for Change

This solution addresses the issue related to how to allow external consumers/subscribers or involved entities of the AI/ML services to be aware and get notified on the availability and capabilities of the registered FL members (who can be heterogeneous and may belong to different domains) which can be potentially used for the AI/ML model training and/or inference. Here heterogeneous refers to both external and network entities, whereas external can application at the telco edge/cloud the end devices / UEs or 3rd party premises.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-82 v0.2.0.

* * * First Change * * * *

## 8.x	Solution #x: Support for FL event notifications

### 8.x.1	Solution description

This solution enables a subscriber/consumer (who can be the coordinator FL server or API invoker entity) for subscribing for AI/ML model related events and getting notified on changes on the availability of the FL members which are to be used for ML model training.

This solution consists of the following procedures 1) the subscription for the FL related events in 8.x.1.1 and 2) the event notification procedure in 8.x.1.2. Also, 8.1.1.3 describes a list of the events.

#### 8.x.1.1 Procedure on subscription for FL related events

This procedure describes the subscription for events related to FL member availability.

Pre-conditions:

1.	The subscribing entity has the authorization to subscribe for the FL-related events.

Figure 8.x.1.1-1: Procedure for FL-related event subscription

1.	The subscribing entity sends an event subscription request to the AI/ML Service Registry in order to receive notification of events related to the AI/ML model lifecycle changes and in particular the availability of FL member for a target service or area or UEs. The FL-related event subscription request includes the following parameters as in Table 8.x.1.1-1.

Table 8.x.1.1-1 Event subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Identity information | M | The information to determine the identity of the subscribing entity |
| ML model ID or profile | M | The identity of the ML model for which the subscription is needed. This identity can be also in terms of a profile for the ML model. |
| Target area information | M | The target service area for which the subscription applies. This area can be in form of topological area (list of cells/TAs) or set of DNAI/DNN or geographical coordinates for the coverage of the subscription. |
| Time of validity | M | The time validity for the subscription. |
| FL member type | O | The type of FL member which can be an FL server or FL client. |
| FL member ID | O | The ID of the entity serving as FL member (if known). This can be used for events where the FL member is known and possibly selected, and the availability is to be monitored. |
| List of UE IDs /type | O | The list of UEs (and types) for which the subscription for event applies |
| PLMN ID | O | The PLMN for which the event subscription will apply (in case of multi-PLMN support by the AIML service registry) |
| Service ID | M | The application service identity or profile for which the event subscription applies. |
| Service API criteria | M | The API related criteria include information for the service API status which are used at the ML model lifecycle. This can be the failure API invocation event, new API available event, API version change event, API location change event, etc and other query information like service API identifier, service API name, etc. |
| Event type | M | The event type requested, based on the List of FL related events (shown in Table 8.x.1.3-1). |
| Notification reception information | M | The information of the subscribing entity for receiving the notifications for the event. |
| ML model / vendor interoperability information | O | The ML model or vendor interoperability indication. |

2.	Upon receiving the event subscription request from the subscribing entity, the AI/ML Service Registry checks for the relevant authorization for the event subscription.

3.	If the authorization is successful, the AI/ML Service Registry stores the subscription information.

4.	The AI/ML Service Registry sends an event subscription response indicating successful operation. The FL-related event subscription response includes the parameters in Table 8.x.1.1-2.

Table 8.x.1.1-2 Event subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates the success or failure of the event subscription operation |
| Subscription identifier | M | The unique identifier for the event subscription. |

#### 8.x.1.2 Procedure on FL related event notification

This procedure describes the event notification procedure for the FL member availability. This can be triggered based on a change at the availability / capabilities of the candidate or selected FL members.

Pre-conditions:

1.	The subscription procedure is performed by the subscribing entity.

Figure 8.x.1.2-1: Procedure for FL-related event notifications

1.	 The AI/ML Service Registry receives a trigger notification from a Candidate or Selected FL member, who has already registered (based on Solution #9). This trigger can be the registration of a new FL member at the AI/ML Service registry which may act as a trigger in this procedure. Such trigger notification message includes the parameters as in 8.x.1.2-1.

Table 8.x.1.2-1 Trigger notification

| Information element | Status | Description |
| --- | --- | --- |
| ML model ID or profile | M | The identity of the ML model for which the subscription is needed. |
| FL member ID | M (NOTE) | The ID of the entity serving as FL member (if known). |
| List of UE IDs | M (NOTE) | The list of UEs (and types) for which the trigger applies |
| Service ID | M (NOTE) | The application service identity or profile for which the trigger applies. |
| Trigger report | M | The trigger report which comprises the change of availability or of the capabilities related to the FL member. This report can also include measurements or notification (based on triggers) of a change in the load/energy status for the FL member. |
| API change indication | O | The expected change of service API endpoint for the FL member (e.g., due to FL member mobility to different service area). |
| Cause | O | The cause for the trigger notification for unavailability of an FL member |
| Predicted timer | O | Optionally, a time window in which the FL member is going to be unavailable or available in the future (e.g., in case of predicted mobility/unavailability). |
| ML model / vendor interoperability information | O | The ML model or vendor interoperability indication. |
| NOTE: At least one of these IEs shall be present. |  |  |

2.	The AI/ML Service Registry generates events to be consumed by the subscribing entity(s), based on the trigger notification, or based on a received registration from a new FL member (based on Solution #9).

3.	For the generated event, the AI/ML Service Registry retrieves the list of corresponding subscriptions.

4.	The AI/ML Service Registry sends FL-related event notifications to all the subscribing entity(s) that have subscribed for the event matching the criteria. If a notification reception information is available as part of the subscribing entity event subscription, then the notification reception information is used by the AI/ML Service Registry to send event notifications to the subscribing entity. Such FL-related Event notifications include one or the following parameters as in Table 8.x.1.2-2.

Table 8.x.1.2-2: FL related Event notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription identifier | M | The unique identifier of the event subscription |
| Event identifier | M | The unique identifier for the event. For the list of events, refer to the table 8.x.1.3-1 with the list of FL related events. |
| Event related information | M | The event related information (e.g., time at which the event originated or is planned to be executed, location of event). |
| Event content | M | The content of the event information. |
| > List of FL member ID /address | M | The list of the FL member IDs and their addresses. |
| >> FL member enter / leave | M (NOTE) | The indication of an FL member entering or leaving the available list. |
| >> FL member updated capability | M (NOTE) | The update of capabilities for the FL members. |
| >> FL member load status | M (NOTE) | The load information if the event is about the load monitoring. |
| >> FL member energy status | M (NOTE) | The energy information if the event is about the energy monitoring. |
| NOTE: At least one of these shall be present. |  |  |

#### 8.x.1.3 List of FL-related Events

The list of the FL related events is provided in Table 8.x.1.3-1.

Table 8.x.1.3-1: List of FL related events

| Events | Events Description |
| --- | --- |
| Availability/Unavailability of candidate FL member(s) | The event type relates to the availability or unavailability indication of a candidate FL member or a set of members, which can be a registered entity as a candidate FL server or FL client. Such availability can be for a given service area, or VAL UE, or for a given ML model ID/profile. |
| Availability changes of selected FL member | The event type relates to the availability change of a selected FL member, which can be a selected entity as a FL server or FL client. Such availability change can be for a given service area, or VAL UE, or for a given ML model ID/profile. |
| FL service API update | The event relates to the API update related to the FL service. This can be due to the relocation of an FL member to a different DN/EDN, and the need to provide the updated northbound API. |
| Monitoring FL member load | This event relates to the monitoring of the computational load for the requested FL member. Such load can be the estimated (based on measurements) or expected/predicted. |
| Monitoring FL member energy | This event relates to the monitoring of the energy consumption for the requested FL member. Such energy consumption can be the estimated (based on measurements) or expected/predicted. |
| FL model monitoring info | Time schedule and latency for the FL training. Latency considering the target latency, i.e., when the FL model training shall be completed. |
| FL member capability change | An FL member may change its capability. Especially for UE or AF, the capability for performing some FL related tasks (e.g., training, reporting, data collection) may be changed due to the energy / load, processing capabilities change, connectivity change / congestion/interference. |
| System related events | Alarm events providing fault information for the FL service. |
| Performance related events | Events related to system load conditions. |

### 8.x.2	Architecture Impacts

Editor's note:	This clause provides the architecture impacts of the solution and possible new SA6 capabilities and interfaces.

### 8.x.3	Corresponding APIs

Editor's note:	This clause provides the corresponding APIs for supporting the solution.

### 8.x.4	Solution evaluation

Editor's note:	This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

* * * End of Change * * * *

## 8.0	Mapping of solutions to key issues

Table 8.1-1: Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI #5 | KI #6 | KI #7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sol #1 |  |  |  |  |  |  |  |
| Sol #2 |  |  |  |  |  |  |  |
| Sol #3 |  | X |  |  |  |  |  |
| Sol #4 |  | X |  |  |  |  |  |
| Sol #5 |  | X |  |  |  |  |  |
| Sol #6 |  |  | X |  |  |  |  |
| Sol #7 |  |  | X |  |  |  |  |
| Sol #8 |  |  | X |  |  |  |  |
| Sol #9 |  |  | X |  |  |  |  |
| Sol #10 |  |  |  |  |  |  |  |
| Sol #X |  |  | X |  |  |  |  |