3GPP TSG-SA WG6 Meeting #65	S6-250184

Athens, Greece 17th – 21st February 2025	(revision of S6-250xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.437 | CR | 0009 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to requestory identity in information flows |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Metaverse_App |  |  |  |  |  | Date: |  |  | 2025-02-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | TS 23.434 defined these entities : User ID, VAL user ID, VAL UE ID, VAL service ID. All the information flows from SEAL client to SEAL server uses the VAL user identity or VAL UE identity. Also the spatial anchor creation response in 8.3.1.3.2 do not have co-relation b/w the spatial anchor ID and its corresponind data in the request. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated the requestor identity in the information flows to clarify that it could be one of VAL server or VAL user or VAL UE identity. Added a NOTE in 8.3.1.3.2 to clarify that stage 3 shall take care of mapping the spatial anchor ID and its corresponding data given in the request. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.3.1.3.1, 9.3.2.3.1, 9.3.3.3.1, 9.3.4.3.1, 9.3.5.3.1, 9.4.3.1, 9.5.2.3.1, 9.5.3.3.1, 9.5.4.3.1, 8.3.1.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 9.3.1.3.1	Create spatial map request

Table 9.3.1.3.1-1 descibes the information elements from VAL server (or SEAL SM client) to SM server for create spatial map request.

Table 9.3.1.3.1-1: Create spatial map request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or VAL User or VAL UE) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Application service ID | M | ID of the requesting VAL application service |
| Area of interest | M | Three-dimensional area information to produce the spatial map |
| Notification Target Address | O | Notification target address (e.g. URL, URI, IP) where the notifications should be sent. |
| Information to include | O | Information to be included in the spatial map |
| > Access control rules | O | Access control rules defining which entities are permitted to discover and access the spatial map |
| > List of spatial map layers | O | List of spatial map layer information. Each element includes the information described below. |
| >> Spatial map layer information | O | Information to specify the corresponding spatial map layer specific information |
| > Augmented layer information | O | List of required augmented layer information. This IE can contain multiple values. Possible values are: - VAL_USERS; - SPATIAL_ANCHORS. |

NOTE 1:	For the current release, the spatial map layer information is implementation specific and out of scope.

Editor's note: Details of Access control rules along with new terminology is FFS.

* * * Next Change * * * *

##### 9.3.2.3.1	Discover spatial map request

Table 9.3.2.3.1-1 descibes the information elements from VAL server (or SEAL SM client) to SEAL SM server for discover spatial map request.

Table 9.3.2.3.1-1: Discover spatial map request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or VAL User or VAL UE) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Application service ID | M | ID of the requesting VAL application service |
| Filtering criteria | M | Criteria to get spatial map(s) |
| > Area of interest | O (NOTE) | Three-dimensional area information to discover spatial maps |
| > Location information | O (NOTE) | Location, orientation and pose information that is used to identify relevant spatial maps. |
| > Optional discovery filters | O | A set of characteristics to search the matching spatial maps |
| >> List of spatial map layers | O | The list of spatial map layers to discover spatial map(s) with matching layers. Each element includes the information described below. |
| >>> Spatial map layer information | O | Information to specify the corresponding spatial map layer specific information. |
| NOTE: At least one of the information elements shall be provided. |  |  |

* * * Next Change * * * *

##### 9.3.3.3.1	Get spatial map information request

Table 9.3.3.3.1-1 descibes the information elements for get spatial map request from the VAL server or SEAL SM client to the SEAL SM server.

Table 9.3.3.3.1-1: Get spatial map information request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or VAL User or VAL UE) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Application service ID | M | ID of the requesting VAL application service |
| Spatial map ID | M | ID of the spatial map to get information on it |
| > List of spatial map layers | O | A list of spatial map layers for which information is requested. |

* * * Next Change * * * *

##### 9.3.4.3.1	Update spatial map request

Table 9.3.4.3.1-1 descibes the information elements for update spatial map request from VAL server or SEAL SM client to SEAL SM server.

Table 9.3.4.3.1-1: Update spatial map request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or VAL User or VAL UE) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Application service ID | M | ID of the requesting VAL application service |
| Spatial map ID | M | ID of the spatial map to update |
| Spatial map update information | M | The spatial map Information to update as described in described in Table 7.3.3.1-1. Only IEs which need to be updated are included. |
| > Augmented layer information | O | List of required augmented layer information. This IE can contain multiple values. Possible values are: - VAL_UE; - SPATIAL_ANCHORS. |
| NOTE: At least one of the information elements shall be provided. |  |  |

* * * Next Change * * * *

##### 9.3.5.3.1	Delete spatial map request

Table 9.3.5.3.1-1 descibes the information elements for delete spatial map request from VAL server or SEAL SM client to SEAL SM server.

Table 9.3.5.3.1-1: Delete spatial map request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or VAL User or VAL UE) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Spatial map ID | M | Spatial map ID to delete |

* * * Next Change * * * *

#### 9.4.3.1	Spatial localization service request

Table 9.4.3.1-1 describes information elements for the spatial localization service request from the SM client or VAL server to the SM server.

Table 9.4.3.1-1: Spatial localization service request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL User or VAL UE, VAL server) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Spatial Map Id | M | Identity of the spatial map |
| Area of interest | O | Contains three dimensional area within spatial map |
| Target identities | O | List of identity of the users or objects to localize |

* * * Next Change * * * *

##### 9.5.2.3.1	SM data source registration request

Table 9.5.2.3.1-1 describes information elements in the SM data source registration request from the SEAL SM client to the SEAL SM server.

Table 9.5.2.3.1-1: SM data source registration request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | The identity of the requestor (e.g., VAL User or VAL UE) |
| Requestor security credentials | M | The security credentials of the requestor |
| Proposed expiration time | O | Proposed expiration time for the registration |
| UE ID | O | The identifier of the hosting UE (i.e., GPSI) |
| VAL client ID | O | The identifier of the VAL client |
| Notification endpoint | M | The endpoint for notifying the SM data source about a SM information request. |
| SM data source profile | M | The SM data source profile of the registering SM data source as described in Table 7.3.3.2-1. |

* * * Next Change * * * *

##### 9.5.3.3.1	SM data source registration update request

Table 9.5.3.3.1-1: SM data source registration update request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | The identity of the requestor (e.g., VAL User or VAL UE) |
| Requestor security credentials | M | The security credentials of the requestor |
| Registration ID | M | Identifier of the registration |
| Proposed expiration time (NOTE) | O | Proposed expiration time for the registration |
| Updated SM data source profile (NOTE) | O | The SM data source profile, as described in Table 7.3.3.2-1, with updated SM information details. Included only if there is an update in SM information details. |
| NOTE: At least one of the IEs is included. |  |  |

* * * Next Change * * * *

##### 9.5.4.3.1	SM data source deregistration request

Table 9.5.4.3.1-1 describes information elements in the SM data source deregistration request from the SEAL SM client to the SEAL SM server.

Table 9.5.4.3.1-1: SM data source deregistration request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor ID | M | The identity of the requestor (e.g., VAL User or VAL UE) |
| Requestor security credentials | M | The security credentials of the requestor |
| Registration ID | M | Identifier of the registration |

* * * Next Change * * * *

##### 8.3.1.3.2	Spatial anchor create response

Table 8.3.1.3.2-1 describes information elements for the spatial anchor create response from the SAn service.

Table 8.3.1.3.2-1: Spatial anchor create response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O | Indicates that the spatial anchor create request was successful. |
| > List of spatial anchor ID (NOTE) | O | The list of created spatial anchor identifiers. |
| Failure response | O | Indicates that the create spatial anchor request has failed. |
| > Cause | O | The cause for the request failure. |
| NOTE: Association of spatial anchor ID and the corresponding spatial anchor provided in Spatial anchor create request can be provided in stage 3. |  |  |

* * * End Changes * * * *