3GPP TSG-SA WG6 Meeting #67	S6-252107

Fukuoka, Japan, 19th – 23rd May 2025

Source:	InterDigital

Title:	Solution on Management of test information for ML model performance and correctness evaluation

Spec:	3GPP TR 23.700-83 v0.1.0

Agenda item:	9.4

Document for:	Approval

Contact:	michel.roy@interdigital.com

1. Introduction

This contribution proposes a solution to KI#2.

2. Reason for Change

This contribution proposes a solution to Key Issue #2: support enhancements for evaluation and determination of ML model performance and correctness within the AIML Enablement Layer.

The solution proposes procedures for management of test information needed to perform ML model performance and correctness evaluation.

3. Conclusions

NA

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-83 v0.1.0.

* * * First Change * * * *

## 7.x	Solution #x: Management of test information for ML model performance and correctness evaluation.

### 7.x.1	Solution description

#### 7.x.1.1 Introduction

This solution proposes enhancements to address Key Issue #2 – Open Issue #2 and provides procedures required to enable evaluation of ML model performance and correctness considering application specific ML models.

This solution proposes to:

- define new procedures and information flows to enable management (e.g., storage, retrieval, removal) of ML model test information in the ML model repository;

- update information flows to link the ML model test information with ML model(s).

ML models that can be stored and managed in the AIML enablement layer are application specific ML models. Such ML models can be retrieved by the AIMLE client or VAL server to perform inferencing at the VAL layer.

ML model specific test information is required for a consumer of a ML model to perform evaluation of a ML model performance and correctness. Thus, this solution proposes to enable storage and retrieval of ML model test information in the ML model repository.

#### 7.x.1.2 ML model test information storage

Figure 7.x.1.2-1 describes procedures for storing ML model test information in the ML model repository.

Figure 7.x.1.2-1. ML test information storage

1.	The requestor (e.g., AIMLE client or VAL server) sends an ML test information storage request to the AIMLE server. The request includes the requestor identifier along with security credentials and ML test information.

2.	Upon receiving the request from the requestor, the AIMLE server validates if the requestor is authorized for the request. If the requestor is authorized, the AIMLE server sends an ML test information storage request to store the ML test information in the ML repository. The request includes the requestor identifier along with security credentials and ML test information.

3.	Upon receiving the request from the AIMLE server, the ML model repository validates if the requestor is authorized for the request.  If the requestor is authorized, the ML model repository stores the ML test information, creates an ML test profile including a ML test profile identifier and sends an ML test information storage response to the AIMLE server. The response includes an indication of success for the request and the ML test profile identifier if the request was successful.

4.	Upon receiving the response from the ML model repository, the AIMLE server sends an ML test information storage response to the AIMLE client or VAL server. The response includes an indication of success for the request and the ML test profile identifier if the request was successful.

#### 7.x.1.3 ML model test information retrieval

Figure 7.x.1.3-1 describes procedures for retrieving ML model test information from the ML model repository.

Figure 7.x.1.3-1. ML test information retrieval

1.	The requestor (e.g., AIMLE client or VAL server) sends an ML test information retrieval request to the AIMLE server. The request includes the requestor identifier along with security credentials and ML test information retrieval filters.

2.	Upon receiving the request from the requestor, the AIMLE server validates if the requestor is authorized for the request. If the requestor is authorized, the AIMLE server sends an ML test information retrieval request to obtain the ML test information from the ML repository. The request includes the requestor identifier along with security credentials and ML test information retrieval filters.

3.	Upon receiving the request from the AIMLE server, the ML model repository validates if the requestor is authorized for the request.  If the requestor is authorized, the ML model repository identifies ML test profiles based on the ML test information retrieval filters and sends a ML test information retrieval response to the requestor. The response includes an indication of success for the request and the identified ML test profiles if the request was successful.

4.	Upon receiving the response from the ML model repository, the AIMLE server sends an ML test information retrieval response to the AIMLE client or VAL server. The response includes an indication of success for the request and the ML test profile(s) if the request was successful.

#### 7.x.1.4 ML model test information removal

Figure 7.x.1.4-1 describes procedures for removal of ML model test information from the ML model repository.

Figure 7.x.1.4-1. ML test information removal

1.	The requestor (e.g., AIMLE client or VAL server) sends an ML test information removal request to the AIMLE server. The request includes the requestor identifier along with security credentials and ML test information identifier(s) to be removed.

2.	Upon receiving the request from the requestor, the AIMLE server validates if the requestor is authorized for the request. If the requestor is authorized, the AIMLE server sends an ML test information removal request to remove the ML test information from the ML repository. The request includes the requestor identifier along with security credentials and ML test information identifier(s).

3.	Upon receiving the request from the AIMLE server, the ML model repository validates if the requestor is authorized for the request.  If the requestor is authorized, the ML model repository removes the ML test information based on the ML test information identifier(s) and sends a ML test information removal response to the requestor. The response includes an indication of success for the request and the ML test information identifiers that were removed if the request was successful.

4.	Upon receiving the response from the ML model repository, the AIMLE server sends an ML test information removal response to the AIMLE client or VAL server. The response includes an indication of success for the request and the ML test information identifiers that were removed if the request was successful.

### 7.x.2	Architecture Impacts

No architecture impact has been identified.

### 7.x.3	Corresponding APIs

#### 7.x.3.1	ML Test Profile

Table 7.x.3.1-1 describes the ML test profile and Table 7.x.3.1-2 describes the ML test which is used in the ML test information management procedures between the AIMLE client, VAL server, AIMLE server and ML model repository.

Table 7.x.3.1-1: ML Test Profile

| Information element | Status | Description |
| --- | --- | --- |
| ML test profile identifier | M | The identifier of the ML test profile. |
| AIMLE server identifier | M | The identifier of the AIMLE server that stored the ML test information. |
| ML repository identifier | M | The identifier of the ML repository where the ML test information is stored. |
| List of ML tests | M | The list of ML test information as defined in Table 7.x.3.1-2. |

Table 7.x.3.1-2: ML Test Information

| Information element | Status | Description |
| --- | --- | --- |
| ML test identifier | M | The identifier of the test. |
| List of ML models identifiers | M | The list of ML models identifiers to which the ML test applies. |
| ML test source identifier | O | The identifier of the ML test source (e.g., VAL server ID, VAL client ID) that stored the ML test in the ML repository. |
| ML test description | M | The description of the ML test. |
| ML test metric information | M | The metrics measured by the ML test. |
| ML test input information | M | Represents the ML test input information which is aligned with the associated ML model input parameters. The encoding, format, and value of ML test input information is not specified since it is specific to the ML model(s) to which this test information applies. |
| ML test output information | M | Represents the expected ML test output information which is expected as a result of processing the ML test input through the ML model. The encoding, format, and value of ML test output information is not specified since it is specific to the ML model(s) to which this test information applies. |
| ML test grading information | M | The information for evaluating and grading the performance and correctness of a ML test (e.g., grading the obtained ML test output based on the expected ML model test output). |
| ML test execution information | O | The ML test execution information which is required to evaluate performance and correctness (e.g., whether the test is executed once, repeatedly in a time window, has its output averaged, etc.). If not included, the test is evaluated and graded once. |

#### 7.x.3.2	ML Test Information Storage

Table 7.x.3.2-1 describes the request from the AIMLE client or VAL server to the AIMLE server or from the AIMLE server to ML model repository for storing ML test information in the ML repository.

Table 7.x.3.2-1: ML Test Information Storage Request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Security credentials | M | The security credentials of the requestor. |
| List of ML test information | M | The list of ML test information to be stored in the ML repository as described in Table 7.x.3.1-2. |

Table 7.x.3.2-2 describes the response sent by the AIMLE server or sent by the ML model repository in response to processing ML test information storage request.

Table 7.x.3.2-2: ML Test Information Storage Response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the request was successful. |
| > ML test profile identifier | M | The ML test profile identifier stored in the ML model repository. |
| Failure response | O (NOTE) | Indicates that the request was failed. |
| > Cause | O | The failure cause |
| NOTE: Only one of these information elements shall be provided. |  |  |

#### 7.x.3.3	ML Test Information Retrieval

Table 7.x.3.3-1 describes the request from the AIMLE client or VAL server to the AIMLE server or from the AIMLE server to ML model repository for retrieving ML test information from the ML repository.

Table 7.x.3.3-1: ML Test Information Retrieval Request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Security credentials | M | The security credentials of the requestor. |
| ML test information filters | M | The test information filter used to identify the retrieved ML test information. |
| > ML test profile identifier | O | Represent the filtering criteria which is a ML test profile identifier. |
| > ML test information filter | O | Represents the filtering criteria, which can be any of the ML test information as defined in Table 7.x.3.1-2. |

Table 7.x.3.3-2 describes the response sent by the AIMLE server or sent by the ML model repository in response to processing ML test information retrieval request.

Table 7.x.3.3-2: ML Test Information Retrieval Response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the request was successful. |
| > List of ML test profiles | M | The list of retrieved ML test profiles stored in the ML model repository. |
| Failure response | O (NOTE) | Indicates that the request was failed. |
| > Cause | O | The failure cause |
| NOTE: Only one of these information elements shall be provided. |  |  |

#### 7.x.3.4	ML Test Information Removal

Table 7.x.3.4-1 describes the request from the AIMLE client or VAL server to the AIMLE server or from the AIMLE server to ML model repository for removing ML test information from the ML repository.

Table 7.x.3.4-1: ML Test Information Removal Request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Security credentials | M | The security credentials of the requestor. |
| List of ML test profile identifiers | M | The list of test profile identifiers to be removed from the ML model repository. |

Table 7.x.3.4-2 describes the response sent by the AIMLE server or sent by the ML model repository in response to processing ML test information removal request.

Table 7.x.3.4-2: ML Test Information Removal Response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | Indicates that the request was successful. |
| > List of ML test profile identifiers | M | The list of ML test profile identifiers removed from the ML model repository. |
| Failure response | O (NOTE) | Indicates that the request was failed. |
| > Cause | O | The failure cause |
| NOTE: Only one of these information elements shall be provided. |  |  |

### 7.x.4	Solution evaluation

Editor's Note:		The solution evaluation is FFS.

* * * End of Changes * * * *