3GPP TSG-SA WG6 Meeting #67	S6-252197

Fukuoka, Japan, 19th – 23rd May 2025	(revision of S6-252xxx)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.437 | CR | 0048 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Subscription to Spatial Map Data Source(s) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | Metaverse_Ph2-APP |  |  |  |  |  | Date: |  |  | 2025-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Currently SM data source(s) registers with SM server regarding their availability and VAL server can discover the list of SM data source(s) available in a particular area of interest. But this is a one time fetch and the SM data source(s) availability keeps changing and SM data source profile can get updated. Hence there is a need for subscription mechanism to get notified about the SM data source availability changes and SM Data source profile changes. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Subscription mechanism for the SM data source(s) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete functionality |  |  |  |  |  |  |  |  |
| Clauses affected: |  | (All new) 9.5.x, 9.5.x.1, 9.5.x.2, 9.5.x.2.1,9.5.x.2.2, 9.5.x.3, 9.5.x.3.1, 9.5.x.3.2, 9.5.x.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 9.5.x	SM data source subscription

#### 9.5.x.1	General

SM data source subscription enables VAL server or SEAL server to subscribe to the SM server to get the updates of changes in SM data sources and data source profiles.

#### 9.5.x.2	Procedures

##### 9.5.x.2.1	Subscribe spatial map event

Figure 9.5.x.2.1-1 depicts the procedure for the authorized VAL server or SEAL server to subscribe to the SM server for the the SM data source specific event. VAL server or SEAL server can get notification information when the the SM server detects subscribed event.

Pre-conditions:

1.	The VAL server or SEAL server has received information (e.g. URI, IP address) related to the SEAL SM server;

2.	The VAL server or SEAL server has received security credentials authorizing it to communicate with the SEAL SM server.

Figure 9.5.x.2.1-1: SM data source subscription

1.	The requestor (e.g., VAL server or SEAL server) sends a SM data source subscription request to the SM server. The request includes requestor ID, security credentials, a notification endpoint, event to subscribe which triggers the notification etc as specified in Table 9.5.x.3.1-1.

2.	Upon receiving the request from the requestor, the SM server validates if the requestor is authorized for the request. If the requestor is authorized, the SM server creates the subscription and stores the subscription information.

a. 	If the subscribed event is for "change of SM Data source(s)", the SM server starts monitoring for the registration and de-registration of the SM data source(s) in the given area of interest or based on the list of SM data source(s) provided in the subscription request.

b.	If the subscribed event is for "changes in SM data source(s) profiles", the SM server starts monitoring for any updates to the SM data source profile(s) of the SM data source(s) provided in the subscription request.

3.	The SM server sends a SM data source subscription response to the requestor with the indication of success or failure as specified in Table 9.5.x.3.2-1.

##### 9.5.x.2.2	Notify SM data source event

Figure 9.5.x.2.2-1 depicts the notify spatial map event operation between the authorized SM data source consumer (VAL server or SEAL server) and SEAL SM server.

Figure 9.5.x.2.2-1: SM data source notification

1.	The SEAL SM server detects an event that triggers the notification to the subscribers for the corresponding event. SM server determines whether the detected event matches triggering criteria and notifies subscribers accordingly.

a)	If the subscribed event is "changes of SM data source(s)", the SM server detects if any SM data source(s) becomes available or unavailable based on the list of SM data source(s) provided in the subscription request or based on the area of interest provided in the subscription request.

b)	If the subscribed event is "changes of SM data source(s) profile", the SM server detects if any SM data source(s) provided in the subscription request modifies or updates its data source profile.

2.	The SEAL SM server notifies the VAL servers or SEAL servers which subscribed the corresponding event. The notification includes a subscription ID and event-specific information. If the event is "changes of SM data source(s)", SM data source notification notification message includes the information on the detected triggering event, e.g. SM data source(s) IDs and if it becomes available or unavailable. If the event is "changes of SM data source(s) profile", the notification message includes the SM data source(s) IDs and its updates SM data source profile information.

* * * Next Change * * * *

#### 9.5.x.3	Information flows

##### 9.5.x.3.1	Subscribe SM data source request

Table 9.5.x.3.1-1 shows the subscribe SM data source request sent by VAL server or SEAL server to SEAL SM server to subscribe for SM data source event(s).

Table 9.5.x.3.1-1: Subscribe SM data source request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., VAL server or SEAL server) |
| Security credentials | M | The security credentials of the requestor. |
| Application service ID | M | ID of the requesting VAL application service |
| Notification Target Address | O | Notification target address (e.g. URL) where the notifications should be sent. |
| Expiry time | O | Proposed subscription expiry time |
| Target SM data source(s) | M | SM data source(s) to subscribe |
| > List of SM data source IDs | O | ID(s) of the SM data source(s) to subscribe |
| > Area of interest | O | Three-dimensional area information to discover SM data source(s) to subscribe |
| List of events | M | Events to subscribe. Each element includes the information described below. |
| > Event type | O | List of events to subscribed. Possible values are: - Change of SM Data source(s) - Change of SM Data source(s) profile |
| > Triggering criteria | O | Specifies the condition to determine whether the event is occurred and to notify the requestor |
| >> Changes in SM data source(s) | O (NOTE) | Triggers notification if any or specified SM data source(s) becomes available or unavailable. |
| >> Changes in SM data source(s) profiles | O (NOTE) | Triggers notification if any or specified SM data source(s) profile as specified in Clause 7.3.3.2 gets updated. |
| > Notification intervals | O | Specifies when to notify the detected event, Possible values are: - Event based (e.g., promptly); Or - Periodically (with minimum time between consecutive notifications) - intervals (i.e. minimum time between consecutive notifications) |
| > interval time | O | Time between consecutive notifications (applicable only if notification intervals is set to intervals. |
| NOTE: At least one of the sub-elements shall be provided. |  |  |

##### 9.5.x.3.2	Subscribe SM data source response

Table 9.5.x.3.2-1 shows the subscribe SM data source response sent by SEAL SM server to VAL server or SEAL server.

Table 9.5.x.3.2-1: Subscribe SM data source response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Result of the subscribe SM data source event request |
| Subscription Id (NOTE) | O | Identity of the subscription |
| Expire time | O | Subscription expire time |
| Failure cause | O | Indicates reason for the failure. |
| NOTE: This IE is mandatory when Result IE indicates success. |  |  |

##### 9.5.x.3.3	Notify SM data source event

Table 9.5.x.3.3-1 shows the SM data source notification sent by SEAL SM server to VAL server or SEAL server to notify about SM data source event(s).

Table 9.5.x.3.3-1: SM data source event notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Identifier of the subscription request |
| Notification information | M | Specifies what matches the triggering notification condition, including related information |
| > Event type | M | Type of event detected. One of the event as specified in Table 9.5.x.3.1-1. |
| >> Changes of SM data source(s) | O (NOTE) | Information on the SM data source which triggered the notification |
| >>> SM data source ID | O | ID of the SM data source |
| >>> Type of change | O | Specifies the detected type of change (whether it became available or unavailable) |
| >> Changes of SM data source(s) profile | O (NOTE) | Information on the SM data source which triggered the notification |
| >>> SM data source ID | O | Identifier of the SM data source |
| >>> SM Data source profile | O | Updated data source profile of the SM data source as specified in clause 7.3.3.2. |
| NOTE: This IE includes the list of SM data source ID and the corresponding information if the changes are detected for more than one SM data source. |  |  |

* * * End Change * * * *