3GPP TSG-SA WG6 Meeting #67	S6-252316

Fukuoka, Japan 19th – 23rd May 2025	(revision of S6-252118)

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.281 | CR | 0246 | rev | 1 | Current version: | 19.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Ambient viewing alignments |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC_Ph2-MC |  |  |  |  |  | Date: |  |  | 2025-05-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | CR 0433 of Rel-19 has clarified for ambient listening the state whether an ambient listening session can be initiated when the ambient target user is already engaged in an MCPTT call. The same concept is considered in ambient viewing with this CR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Adding new IE in the ambient viewing call response 2. updating clauses 7.6.3.1 and 7.6.3.2 to realize that ambient viewing session occur when the MCVideo user is not currently engaged in an ongoing private or group communication. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Inconsistency between ambient viewing and ambient listening concepts, and stage 1 requirements on ambient listening/viewing are not met. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.6.2.2, 7.6.3.1 and 7.6.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 7.6.2.2	Ambient viewing call response

Table 7.6.2.2-1 describes the information flow ambient viewing call response from the MCVideo client to the MCVideo server and MCVideo server to the MCVideo client.

Table 7.6.2.2-1: Ambient viewing call response

| Information Element | Status | Description |
| --- | --- | --- |
| MCVideo ID | M | The MCVideo ID of the "viewing" user |
| MCVideo ID | M | The MCVideo ID of the "viewed to" user |
| SDP answer (NOTE 1) | O | Media parameters |
| Ambient viewing type (NOTE 1) | O | The ambient viewing type indicates remotely initiated ambient viewing call or locally initiated ambient viewing call. |
| Result | O | It indicates the success or failure of the ambient viewing request |
| Reason (NOTE 2) | O | It indicates the reason of failure of ambient viewing request, e.g., due to the engagement of the "viewed to" user in MCVideo communication. |
| NOTE 1: This information element is included when the Result information element is success. NOTE 2: This information element is included when the Result information element is failure. |  |  |

# * * * * Second change * * * *

#### 7.6.3.1	Remotely initiated ambient viewing call setup procedure

The MCVideo service provides the capability for an authorised user to initiate a remotely initiated ambient viewing call.

Figure 7.6.3.1-1 illustrates the high level procedure of remotely initiated ambient viewing call setup procedure.

Pre-conditions:

-	MCVideo client 1 is the client of the authorized user who is authorized to invoke a remotely initiated ambience viewing call with MCVideo client 2.

-	MCVideo user 1 is the "viewing" user at MCVideo client 1, and MCVideo user 2 is the "viewed to" user at MCVideo client 2.

Figure 7.6.3.1-1: Remotely initiated ambient viewing call setup

1.	MCVideo client 1 initiates a remotely initiated ambient viewing call to MCVideo client 2 by sending an ambient viewing request to MCVideo server. The remotely initiated ambient viewing call type is included.

2.	The MCVideo server performs an authorization check for the authorized user 1 for the remotely initiated ambient viewing call. If authorization fails, the MCVideo server provides a failure response to MCVideo client 1 in step 2a and the procedure ends.

Furthermore, the MCVideo server performs a check whether MCVideo client 2 is engaged in an MCVideo private or group communication. If MCVideo client 2 is engaged in a communication, an ambient viewing call response is sent to MCVideo client 1 in step 2a and the procedure ends.

3.	The MCVideo server sends the ambient viewing call request to MCVideo client 2.

4.	MCVideo client 2 returns the ambient viewing call response to the MCVideo server.

5.	MCVideo server provides an ambient viewing call response to MCVideo client 1, indicating that whether the remotely initiated ambient viewing call is set up successfully or not.

6.	The transmission control server of the MCVideo server then sends a media transmit granted message to MCVideo client 2 according to the ambient viewing type received in step 1.

7.	After receiving the media transmit granted message, the MCVideo client to be viewed to transmit video to the MCVideo client 1 for viewing.

NOTE:	MCVideo client 2 does not provide any indication of the ambient viewing call release to its user.

#### 7.6.3.2	Locally initiated ambient viewing call setup procedure

The MCVideo service provides the capability for an authorised user to initiate a locally initiated ambient viewing call.

Figure 7.6.3.2-1 illustrates the high level procedure of locally initiated ambient viewing call setup procedure.

Pre-conditions:

-	MCVideo client 2 is the client of the authorized user who is authorized to invoke a locally initiated ambient viewing call from MCVideo client 2.

-	MCVideo user 1 is the "viewing" user at MCVideo client 1, and MCVideo user 2 is the "viewed to" user at MCVideo client 2.

-	MCVideo client 2 is not engaged in an ongoing MCVideo private or group communication.

Figure 7.6.3.2-1: Locally initiated ambient viewing call setup

1.	MCVideo client 2 initiates a locally initiated ambient viewing call to MCVideo client 1 by sending an ambient viewing request to MCVideo server. The locally initiated ambient viewing call type is included.

2.	The MCVideo server performs an authorization check for the authorized user 2 for the locally initiated ambient viewing call. If authorization fails, the MCVideo server provides a failure response to MCVideo client 2.

3.	The MCVideo server sends the ambient viewing call request to MCVideo client 1.

4.	MCVideo client 1 returns the ambient viewing call response to the MCVideo server.

5.	MCVideo server provides an ambient viewing call response to MCVideo client 2, indicating that whether the locally initiated ambient viewing call is set up successfully or not.

6.	The transmission control server of the MCVideo server then sends a media transmit granted message to MCVideo client 2 according to the ambient viewing type received in step 1.

7.	After receiving the media transmit granted message, the MCVideo client to be viewed to, transmits video to the MCVideo client 1 for viewing.

NOTE:	MCVideo client 2 does not provide any indication of the ambient viewing call release to its user.

# * * * * End of changes * * * *