3GPP TSG-SA6 Meeting #67	S6-252327

# Fukuoka, Japan, 19th May 2025 - 23th May 2025 	                   (revision of S6-252030)

Source:	Airbus

Title:	Key Issue - Recording and audit of e2e encrypted group communication

Spec:	3GPP TR 23.700-39 V0.1.0

Agenda item:	9.2

Document for:	Approval

Contact:	Jukka Vialen (jukka.vialen@airbus.com)

1. Introduction

This contribution proposes a key issue for recording and audit of e2e encrypted group communication.

2. Reason for Change

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-39 V0.1.0.

* * * First Change * * * *

### 5.x	Key Issue x – Recording and replay of e2e encrypted group communication

Figure 5.x-1: Functional elements relevant for recording and replay of e2e encrypted group communication

This Key issue focuses on the case when group communication is e2e encrypted.

A basic assumption is that the Group Master Key (GMK) is known only to the group members and, at some point of time, also to the Recording server. To ensure e2e confidentiality, as required in [R-6.15.4-005] of TS 22.280 [2], the GMK shall never be revealed to any external entity, including the Replay clients.

There are two alternatives to record e2e encrypted group media:

- Decrypt the media in the Recording server before storage. This option requires that the media, before storage, is re-encrypted with some implementation specific mechanisms.

- Store the recorded media as e2e encrypted. This option requires that, when a replay is requested by an auditor at some later point of time, the Recording server shall be able to fetch/generate the used GMK and decrypt the media.

In both alternatives, for the replay of recordings, a secure connection between the Recording server and the Replay client is required (REC-1), but this connection and the security mechanisms used there are outside the scope of 3GPP.

Issues:

- Study clause 5.7 of 3GPP TS 33.180 [x]. Is there something that should be referred to in 3GPP TS 23.280 [2], 3GPP TS 23.281 [4], 3GPP TS 23.282 [5] or 3GPP TS 23.379 [3]?

- How does the Recording server get the GMK? Study both options 1 and 2 (see above) and define – if needed - the required procedures, information flows and parameters

NOTE: This KI will require co-operation with SA3 and the solutions may require changes also to 3GPP TS 33.180 [x].

* * * Next Change * * * *

# Annex B: Mapping of stage 1 requirements to solutions

| Requirement | Scenarios | Key Issues | Solutions | NOTE |
| --- | --- | --- | --- | --- |
| [R-6.15.4-003] [R-6.15.4-005] [R-6.15.4-011] | All group comm scenarios. | x |  |  |

* * * End of changes * * * *