3GPP TSG-SA WG6 Meeting #67	S6-252466

Fukuoka, Japan, 19th – 23rd May 2025	(revision of S6-252154)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0395 | rev | 1 | Current version: | 19.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct the S&F events to align with SA2 exposure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GSAT_Ph3_App |  |  |  |  |  | Date: |  |  | 2025-05-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As the satellite S&F events subscription and notification depends on exposure from SA2, to align with the SA2’s specification, the S&F events in subscription and notification procedures and information flow should be corrected to keep align with the definition/ terminology in SA2 TS 23.682. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Correct the DL estimated delivery time to Estimated S&F DL Delivery Time to keep consistent with SA2 terminology. - Add Feeder link availability period in satellite S&F events related information flow to align with SA2 specification in TS 23.682. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The satellite S&F events subscription and notification information flow cannot be performed correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 21.3.2.1, 21.3.3.1,21.3.3.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

21.3.2.1	NRM Server exposing the S&F events to the VAL server procedure

Pre-condition:

-	The NRM client, the NRM server and the VAL server all are deployed on the ground.

-	The PGW-U in 3GPP CN is deployed on the satellite.

-	The VAL UE has registered to the 3GPP network via satellite access as defined in clause 5.3.2.1 of 3GPP TS 23.401 [9] and supports the S&F mode.

Figure 21.3.2.1-1: NRM Server exposing the satellite S&F events to the VAL server procedure

1.	The VAL server sends the satellite S&F events subscription request to the NRM server in which the subscribed events may include the UE status in S&F mode, the DL estimated delivery time, the trigger conditions, etc.

NOTE:	Step 1 is also applicable to the update procedure when the VAL server requires to update the satellite S&F events subscription request.

2.	The NRM server authorizes the subscription request from the VAL server.

3.	If the subscription request is authorized, the NRM server sends the satellite S&F events subscription request to the NRM client and/or the 3GPP CN respectively.

4.	The NRM client and/or 3GPP CN acknowledge the NRM server with the satellite S&F events subscription response respectively.

5.	The NRM server responds to the VAL server with the satellite S&F events subscription response.

6.	If the trigger condition in the subscription request is met, the NRM client and/or 3GPP CN may report the requested satellite S&F events as specified in table 21.3.3.3 to the NRM server via the satellite S&F events subscription notification.

7.	The NRM server notifies the VAL server for the received satellite S&F events in step 6. The VAL server may take them into consideration and adjust the application behaviour (e.g., pending the DL data transition when the UE is registered in S&F Mode, re-transmit the pending DL data when the feeder link is available, adjust the DL traffic volume according to the Estimated S&F DL Delivery Time, etc.) to minimize the service interruption.

8.	The VAL server sends the satellite S&F events unsubscribe request to the NRM server to unsubscribe the subscription to monitor the satellite S&F events.

9.	The NRM server relays the satellite S&F events unsubscribe request to the NRM client and/or 3GPP CN. And receives the response from the NRM client and/or 3GPP CN respectively.

10.	The NRM server replies to the VAL server with the satellite S&F events unsubscribe response.

* * * Next Change * * * *

21.3.3.1	Satellite S&F events subscription request

Table 21.3.3.1-1 describes the information flow from VAL server to NRM server or from the NRM server to NRM client for satellite S&F events subscription request.

Table 21.3.3.1-1: Satellite S&F status events subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL server/VAL users or VAL UEs or NRM server. |
| Satellite S&F events | O | Lists of the satellite S&F events that to be monitored for one or more UEs (e.g. UE status in S&F mode, the Estimated S&F DL Delivery Time, Feeder link availability period, etc). |
| Triggering conditions | M | Identifies the conditions that triggered NRM client sending the notification report. |

* * * Next Change * * * *

21.3.3.3	Satellite S&F events notification

Table 21.3.3.3-1 describes the information flow from NRM server to VAL server or from the NRM client to NRM server to notify the subscribed satellite S&F events.

Table 21.3.3.3-1: Satellite S&F status events notification

| Information element | Status | Description |
| --- | --- | --- |
| Identity | M | Identity of the requesting VAL server/VAL users or VAL UEs or NRM server. |
| Satellite S&F events | O | List the notification for satellite S&F events that to be monitored for one or more UE(s). |
| > UE status in S&F mode | O (NOTE) | Indicates the UE is registered in S&F mode or moving from S&F mode to not registered in S&F mode. |
| > Estimated S&F DL Delivery Time | O (NOTE) | Indicates the estimated/expected time required to deliver the data to the UE from the time data has been received in the network. |
| > Feeder link availability period | O (NOTE) | Indicates the availability period when the feeder link is available. |
| Triggering events | M | Identifies the events that triggered NRM client sending the report. |
| Timestamp | O | Timestamp of the notification report. |
| NOTE: The definition refers to the clause 5.6.3.10 of 3GPP TS 23.682 [13]. |  |  |

* * * End of Changes * * * *