3GPP TSG-SA WG6 Meeting #68	S6-253609

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253126)

Source:	China Mobile

Title:	Solution evaluation of sol#3

Spec:	3GPP TR 23.700-51

Agenda item:	9.5

Document for:	Approval

Contact:	Tangqing Liu, liutangqing@chinamobile.com

1. Introduction

This paper proposes to add solution evaluation for sol#3.

2. Reason for Change

Add solution evaluation for sol#3.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-51.

* * * First Change * * * *

5.4	Solution #3: QoS information setting and adjustment based on available bitrate

5.4.1	Architecture Impacts

This solution is based on architecture of SEALDD as described in 3GPP TS 23.433 [6].

5.4.2	Solution description

This solution addresses on application enabler layer enhancement to support for QoS information setting and adjustment based on available bitrate exposed from PCF.

In Release 18 and Release 19, SA2 has defined that 5GS and XR/media services cooperate to provide a better user experience using External Network Exposure. The 5GS can expose available bitrate described in clause 5.37.4 of TS 23.501[3]. The available bitrate refers to the bitrate that NG-RAN is able to provide for a QoS Flow within a duration. The duration is based on RAN implementation [3]. During this duration, NG-RAN can not provide a better bitrate, so SEALDD transmission bitrate is suggested to keep align with NG-RAN. If the available bitrate is changed, SEALDD can adjust QoS to keep align with new value of NG-RAN.

5.4.2.1	Impact to existing SEALDD procedures

The SEALDD procedures and information flows in 3GPP TS 23.433 [6] can be enhanced (highlighted in bold italics) as follows.

9.12.2.1.2 SEALDD enabled multi-modal data transmission establishment Figure 9.12.2.1.2-1 illustrate the procedure for establishing XR data transmission connection, and the SEALDD facilitates the multi-modal application to transmit its data between the VAL client and VAL server with RTP packetilization and PDU set inclusion. Pre-condition: - The VAL server can discover and select the SEALDD server by CAPIF functions. Figure 9.12.2.1.2-1: SEALDD enabled multi-modal data transmission connection establishment procedure 1. The VAL server decides to use SEALDD service for multi-modal traffic transfer and allocates address/port as SEALDD-S Data transmission connection information for receiving the data packets from SEALDD server. The VAL server sends Sdd_multi-modalTransmission request to the SEALDD server discovered by CAPIF. The service request includes UE ID/address, VAL server ID, VAL service ID, SEALDD-S Data transmission connection information of the VAL server side, and optionally, the protocol description, the QoS information for the application traffic, e.g. QoS requirements. 2. Same as step 2 of clause 9.2.2.2. For application multi-modal service, the SEALDD server derives PDU Set related assistance information based on received VAL service ID and/or VAL server ID, and protocol description for interacting with NEF/PCF. SEALDD subscribes to 5GC for QoS monitoring as defined in clause 5.2.6.9 in 3GPP TS 23.502 [4]. If UL and/or DL available bitrate is provided in the QoS monitoring report, SEALDD server will adjust transmission bitrate to align with available bitrate. 3-5. Same as step 3-5 of clause 9.2.2.2. 6. The SEALDD client establishes multi-modal transmission connection with the SEALDD server. The request includes the SEALDD client ID, VAL user/UE ID, VAL server ID, VAL service ID, SEALDD-UU flow IDs, and traffic descriptors for the multiple flows from the SEALDD client side. The SEALDD server sends the SEALDD traffic descriptors for multiple flows from SEALDD server side (e.g. address/port for multiple SEALDD-UU flow) to the SEALDD client, and may send the protocol description (UL related info) received from the VAL server to the SEALDD client in the multi-modal transmission connection response. The response also includes a multi-modal SEALDD-UU flow ID which the SEALDD server allocates. 7-8. Same as step 8-9 of clause 9.2.2.2. The multi-modal application traffic is exchanged between VAL client and VAL server via SEALDD layer as described in clause 9.2.2.2. If packetization indication indicates that SEALDD layer needs to perform packetization, the SEALDD server performs packetization and sends streaming data (e.g. RTP packet) via SEALDD-UU user plane (e.g. SEALDD/UDP/IP) to the SEALDD client for downlink application traffic. Similarly, the SEALDD client performs packetization and sends streaming data (e.g. RTP packet) via SEALDD-UU user plane (e.g. SEALDD/UDP/IP) to the SEALDD server for uplink application traffic. The SEALDD server and client also perform PDU Set inclusion (e.g. in RTP extension as defined in 3GPP TS 26.522 [TS26522]), and if needed, stream session and transport management (e.g. RTCP, RTSP).

5.4.3	Solution evaluation

This solution addresses KI #3 to support:

-	How the SEALDD server can be enhanced to support to traffic data rate adaptation based on the available bitrate from 5GS.

This solution enable SEALDD server subscribes to 5GC for QoS monitoring and adjust transmission bitrate based on available bitrate exposed from PCF. This solution has no architecture imapct.

NOTE: Potential additional impacts, e.g., impact to SEALDD client, due to the data rate adaptation performed by the SEALDD server will be solved during normative phase.

* * * End Change * * * *