3GPP TSG-SA WG6 Meeting #68	S6-253611

Gothenburg, Sweden 25th – 29th August 2025	(revision of S6-253203,

S6-252521)

Source:	Nokia

Title:	Pseudo-CR on New solution for KI#1

Spec:	3GPP TR 23.700-51 (v0.2.0)

Agenda item:	9.5

Document for:	Approval

Contact:	Sapan Shah (Sapan.shah@nokia.com)

1. Introduction

This pCR provides solution to KI#1.

2. Reason for Change

KI#1 states following:

“Additionally, many policies are defined in TS 23.433, such as Multi-modal flows alignment policy, Synchronization policy, UE-to-UE policy are defined, but the VAL server has no way to know whether and when these policies are enforced, therefore the work on policy enforcement notification to VAL server needs to be studied.”

And has following open issue:

How could application enabling layer be enhanced to support for policy (Multi-modal flows alignment policy, Synchronization policy, UE-to-UE policy, expedited transfer indication, policy enforcement notification, etc.).

Although SEALDD client supports feature to hold the flow packets which arrived early till all other flows arrived, it may be possible that the flows are arriving at different time due to issues while transporting the flows. The 5G core or the SEALDD server is not aware of the fact that flows are arriving at different times (even if they arrive within threshold limits) and so they are not able to perform course corrections to make sure that going forward flows arrived at almost same time. If the information about the flow alignment is not provided to the 5G core or SEALDD server then the problem may occur frequently and at some point, flow packets may not arrive to the SEALDD client (i.e. UE) at all.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-51 (v0.2.0).

* * * First Change * * * *

## 5.1	Mapping of solutions to key issues

Table 5.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 |
| --- | --- | --- | --- | --- |
| Sol #1 | x |  |  |  |
| Sol #2 |  | x |  |  |
| Sol #3 |  |  | x |  |
| Sol #4 |  |  |  | x |
| Sol #x | X |  |  |  |

* * * Next Change * * * *

## 5.x	Solution #x: Enhancement to flow alignment notification

### 5.x.1	Architecture Impacts

There is no architecture impacts identified based on this solution.

### 5.x.2	Solution description

Although SEALDD client supports feature to hold the flow packets which arrived early till all other flows arrived, it may be possible that the flows are arriving at different time due to issues while transporting the flows. The 5G core or the SEALDD server is not aware of the fact that flows are arriving at different times (even if they arrive within threshold limits) and so they are not able to perform course corrections to make sure that going forward flows arrived at almost same time. If the information about the flow alignment is not provided to the 5G core or SEALDD server then the problem may occur frequently and at some point, flow packets may not arrive to the SEALDD client (i.e. UE) at all. The solution enhances the clause 9.7.2.1 of 3GPP TS 23.433 [6] as follows.

NOTE:	During normative work, whether the solution requires to merge with other existing procedure will be considered.

#### 9.7.2.1 Data transmission quality measurement Figure 9.7.2-1 illustrate the procedure for SEALDD enabled data transmission quality measurement. The SEALDD client and SEALDD server is enhanced to carry out the data transmission quality measurement. Pre-conditions: 1. The SEALDD server and SEALDD client are synchronized to the time source provided by 5GS as specified in 3GPP TS 23.501 [5]. 2. The VAL server discovers and selects the SEALDD server by CAPIF functions. Figure 9.7.2.1-1: SEALDD enabled data transmission quality measurement procedure 1. An on-going regular data transmission connection is established according to clause 9.2.2.2. 2. The VAL server sends a SEALDD transmission quality measurement subscription request to the SEALDD server. The request includes the identifiers of the application traffic (e.g. VAL service ID, VAL server ID), requirement of transmission quality measurement (e.g. latency, jitter, bitrate, packet loss rate, multi model flow delay differences) and measurement target UE (e.g. a single UE, a group of UEs or all UEs), flow(s) traffic descriptor(s) (e.g. multi-modal SEALDD flow IDs) and may also include reporting criteria, reporting frequency, spatial condition and temporal condition. NOTE 1: The spatial and/or temporal condition can be used by SEALDD server to apply when and where the measurement is performed. For instance, the measurement is expected to be done for a group of VAL UEs with a scheduled route (from city A to city B via highway A2 and A3), from 9:00 a.m. to 11:00 a.m. on Tuesday and from 1:00 p.m. to 5:00 p.m. on Thursday. 3. Upon receiving the request, the SEALDD server performs an authorization check. If authorization is successful, the SEALDD server sends a response to the VAL server with the subscription ID, expiration time. 4-5. The SEALDD server sends a SEALDD transmission quality measurement subscription request to the SEALDD client and the SEALDD client responds to the SEALDD server. 6. If the transmission quality measurement requirement list provided by VAL server in step 2, indicates that the latency is needed to be measured, the SEALDD server initiates the DL packet delay measurement. The SEALDD server encapsulates the DL monitoring packet (i.e. DL SEALDD packet with SEALDD DL monitoring header and VAL traffic as payload, or dummy DL SEALDD packet generated for data transmission quality monitoring) with local time T1 when the SEALDD server sends out the DL monitoring packets. The SEALDD server considers the spatial and/or temporal conditions when starting/resuming the transmission quality measurement. If the conditions are not satisfied, the SEALDD server stops/suspends the transmission quality measurement. NOTE: For other metrics in transmission quality measurement requirement list (e.g. bitrate), the transmission quality result can be obtained by performance detection on the SEALDD server within a period of time. 7. The SEALDD client receives the DL monitoring packet and records the local time T2. For multi model flows, the SEALDD Client measures the flow delay differences as per clause 9.7.2.3 (Step-10). If the endpoint for the SEALDD traffic is located at the tethered device, the SEALDD client in 5G UE measures the QoS between the SEALDD client in 5G UE and the SEALDD client (i.e. for architecture that has the SEALDD client on the tethered device) or the VAL client (i.e. for architecture that without the SEALDD client on the tethered device) in tethered device, e.g., the SEALDD client may measure the tethered link delay using the ICMP ping protocol as defined in IETF RFC792 [RFC792]). The detailed procedures of Tethering link measurement and provisioning is defined in clause 9.12.2.3. NOTE 2: The tethered link measurement between SEALDD client and VAL client is based on assumption that there is business agreement between the VAL provider and the SEALDD provider, which request the VAL client to respond to ICMP Ping packet. NOTE 3: Only RTT can be measured for the link between SEALDD client and the tethered device. 8. The SEALDD client notifies the transmission quality measurement report including measured multi modal flow delay differences measurements to SEALDD server. 9. Similarly, the SEALDD client encapsulates the UL monitoring packet (i.e. UL SEALDD packet with SEALDD UL monitoring header and VAL traffic as payload, or dummy UL SEALDD packet generated for data transmission quality monitoring) with local time T2 recorded in step 5 and local time T3 when the SEALDD client sends out the UL monitoring packet. If the endpoint for the VAL traffic is located at the tethered device, the measurement delay in the tethered link (step 5) is considered in T2 and T3 timestamps, e.g., T2 timestamp includes the half of the measurement delay and T3 timestamp excludes the half of the measurement delay. NOTE 4: With ICMP ping over tethered link, it is assumed that the UL packet delay and the DL packet delay are the same between the tethering UE and the tethered device. 10. The SEALDD server records the local time T4 when the SEALDD server receives the UL monitoring packet and calculates the latency with T1, T2, T3, T4. The SEALDD server can also calculate the bitrate, jitter and packet loss rate over a certain period over a specific SEALDD connection by recording the status of the SEALDD packets carrying VAL traffic or dummy SEALDD packets generated for transmission quality measurement reports. The SEAL DD server also evaluates the reporting criteria if present in the SEALDD transmission quality measurement subscription request in order to generate the transmission quality measurement report. Based on the received multi model flow delay differences between flows measured at SEALDD client (received in step-8) and list of flow-ids who reached SEALDD client after the threshold, the SEALDD server takes corrective action (i.e. to adjust the QoS, advance or delay the transmission of the DL packets). 11. The SEALDD server reports the data transmission quality measurement results (e.g. latency, jitter, bitrate, packet loss rate, multi modal flow delay differences) to the VAL server via the notification message. When a VAL group ID or a list of VAL UE IDs or all VAL UEs indication is received in step 2, step 4 to step 7 is repeated for VAL UEs in the group/list or for all VAL UEs. The SEALDD server maps the VAL UE group ID to a list of VAL UE IDs if a VAL group ID is received. The SEALDD server identifies SEALDD connections corresponding to the desired VAL UE(s) or flow(s) traffic descriptor(s) to trigger measurement. And depending on the reporting requirement for multiple UEs, the SEALDD server calculates the needed report for the VAL server. When the VAL server decides to update or unsubscribe transmission quality measurement subscription after performing step 2 and step 3, the VAL server can send data transmission quality measurement subscription update request and data transmission quality measurement unsubscribe request to SEALDD server, as specified in Table 9.7.3.9-1 and Table 9.7.3.11-1, respectively. #### 9.7.3.1 SEALDD enabled data transmission quality measurement subscription request Table 9.7.3.1-1 describes the information flow from the VAL server to the SEALDD server for subscribing to the data transmission measurement service. Table 9.7.3.1-1: SEALDD transmission quality measurement subscription request Information element; Status; Description. Application traffic identifiers; M; Identify of the application traffic (e.g. VAL server ID, VAL service ID). Identity; O (See NOTE); Identifier of the VAL UE or VAL user for which measurements need to be provided.. VAL UE/user group ID; O (See NOTE); Identifier of a specific VAL UE/user group, as defined in clause 7.5 of 3GPP TS 23.434 [4].. Identity list; O (See NOTE); Identifies a list of VAL UEs, e.g. the list of UE ID, or a list of VAL users.. All VAL UEs or VAL users Indication; O (See NOTE); Indicates all VAL UEs or VAL users of the application identified by application traffic identifiers.. Flow(s) traffic descriptor(s); O (See NOTE); Indicates the flow(s) traffic descriptor(s) (Address/port or URL) for the measurement.. Crossflow measurement information; O (see NOTE); Represents the crossflow measurement information, e.g., list of pairs traffic descriptor with traffic direction (UL or DL). Example of the crossflow measurement information: {[traffic descriptor 1, UL]; [traffic descriptor 2, DL]}.. Measurement conditions; O; Indicates the temporal and/or spatial conditions.. Transmission quality measurement requirements list; M; The measurement requirement information. > Measurement ID; M; Measurement identifiers, e.g. UL/DL/E2E latency, UL/DL bitrate, UL/DL/E2E packet loss rate, UL/DL/E2E jitter, crossflow identifier (e.g., crossflow latency, crossflow bitrate, crossflow packet loss rate, crossflow jitter), and Multi modal flow delay differences.. > Reporting frequency; O; The reporting frequency of measurement results (e.g. periodic reporting). If not present, it implies periodic reporting.. > Reporting periodicity; O; If the reporting frequency is periodic, the reporting periodicity shall be provided. For multiple UEs/users, it is recommended to give sufficient time to allow report aggregation.. > Reporting granularity; O; The reporting granularity indicates whether the measurement report is for individual VAL UE/user or for VAL UE/user group or for all VAL UEs/users, if VAL UE/user group or all VAL UEs/users is the measurement target.. > Measurement period window; O; Indicates the measurement period window for transmission quality measurements. > Measurement expiration time; O; Indicates the measurement expiration time. > Reporting criteria; O; Indicates the criteria for reporting measurement results, e.g. if the latency or bitrate reaches below or above a certain value. It also includes a unique identifier for each criteria of more than one criteria is specified.. NOTE: One of them shall be present as the measurement target UE or traffic descriptor.. #### 9.7.3.6 Transmission quality measurement subscription request Table 9.7.3.6-1 describes the information flow from the SEALDD server to the SEALDD client for data transmission measurement subscription. Table 9.7.3.6-1: Transmission quality measurement subscription request Information element; Status; Description. SEALDD-UU flow ID; O (see NOTE); Identifier of the SEALDD-UU flow.. Multi-modal SEALDD flow ID; O (see NOTE); Identity of the multi-modal SEALDD flows associated with the SEALDD multi-modal connection for the measurement.. Measurement conditions; O; Indicates the temporal and/or spatial conditions.. Transmission quality measurement requirements list; M; The measurement requirement information. > Measurement ID; M; Measurement identifiers, e.g. latency, bitrate, jitter, Multi modal flow delay difference, signal strength (e.g. RSRP, SINR, RSRQ, RSSI). > Reporting frequency; O; The reporting frequency of measurement results (e.g. periodic reporting). If not present, it implies periodic reporting.. > Reporting periodicity; O; If the reporting frequency is periodic, the reporting periodicity shall be provided.. > Measurement period window; O; Indicates the measurement period window for transmission quality measurements. > Measurement expiration time; O; Indicates the measurement expiration time. > Reporting criteria; O; Indicates the criteria for reporting measurement results, e.g. if the latency or bitrate reaches below or above a certain value. It also includes a unique identifier for each criteria if more than one criteria is specified.. > Delay difference indicator; O; Indicates the delay difference needs to be measured.. > Flow alignment reporting criteria; O; Indicates the criteria for reporting flow alignment measurement results, e.g. if the flow alignment buffering reaches below or above a certain value for a certain amount of time. It also includes a unique identifier for each criteria if more than one criteria is specified.. > SEALDD policy; O; Specifies quality guarantee policies associated with the SEALDD connection. >> Quality guarantee policy; M; Indicates the event (e.g. measurement threshold) to be measured for, the quality guarantee.. >> Non-3GPP access measurement information; O; Indicates the non-3GPP access(like WLAN) measurement policy (e.g. WLAN SSIDs, BSSID, location-based measurement). NOTE: Only one of the information element is present.. #### 9.7.3.3 SEALDD enabled data transmission quality measurement notification Table 9.7.3.3-1 describes the information flow from the SEALDD server to the VAL server for notifying the transmission quality measurement reports. Table 9.7.3.3-1: SEALDD transmission quality measurement notification Information element; Status; Description. Subscription ID; M; Subscription identifier corresponding to the subscription.. Transmission quality measurement reports list; M; The generated transmission quality results in SEALDD server or SEALDD client, as specified in Table 9.7.3.3-2.. Table 9.7.3.3-2 describes the information elements for the transmission quality measurement reports list, provided by the SEALDD server after performing transmission quality measurement. Table 9.7.3.3-2: SEALDD transmission quality measurement reports list Information element; Status; Description. > Measurement ID; M; Measurement identifiers, (e.g. UL/DL/E2E latency, UL/DL bitrate, UL/DL/E2E packet loss rate, UL/DL/E2E jitter) and crossflow identifiers (e.g., crossflow latency, crossflow bitrate, crossflow packet loss rate, crossflow jitter), Multi modal flow delay differenc.. > VAL UE/user ID(s) or flow(s)traffic descriptor(s); O (see NOTE); It indicates the VAL UE(s) or VAL user(s) under SEALDD measurement. For a single VAL UE/user or flow(s)traffic descriptor(s), it can be omitted and the associated measurement values are for the single VAL UE/user. For multiple VAL UEs/users or flow(s)traffic descriptor(s) with reporting granularity set to individual UE/user or flow traffic descriptor, the associated measurement values are for individual VAL UE/user or flow traffic descriptor as indicated in this IE. For multiple VAL UEs/users with reporting granularity set to VAL UE/user group/list or all VAL UEs/users, the associated measurement values are aggregation for all VAL UEs/users or the VAL UE/user group/list and this IE includes the measured VAL UEs/users.. > Crossflow measurement information; O (see NOTE); Represents the crossflow measurement information, e.g., list of pairs traffic descriptor with traffic direction (UL or DL). Example of the crossflow measurement information: {[traffic descriptor 1, UL]; [traffic descriptor 2, DL]}.. > Average measurement value; O; The average measurement value of measurement results. > Minimum measurement value; O; The minimum measurement value of measurement results. > maximum measurement value; O; The maximum measurement value of measurement results. > Standard deviation measurement value; O; Standard deviation measurement value of measurement results. > kPercentile measurement value; O; Indicates the kpercentile measurement value of measurement results. > Measurement period; O; Indicates the measurement period. > Timestamp; O; Indicates the timestamp of measurement results. NOTE: These IEs are mutually exclusive..

### 5.x.3	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

* * * End of Change * * * *