3GPP TSG-SA WG6 Meeting #69	S6-254049

Wuhan, China 13th – 17th October 2025

Source:	Huawei, Hisilicon

Title:	Pseudo-CR on <Solution for Key issue#2: sensing based DAA service >

Spec:	3GPP TR 23.700-15 V0.1.0

Agenda item:	9.13

Document for:	Approval

Contact:	Lei Chen(chenlei79@huawei.com)

1. Introduction

This contribution proposes a solution of sensing based DAA service for key issue#2.

2. Reason for Change

In TR 23.700-15, there are the following open issues for Key issue #2: Enhance UAV service utilizing sensing results:

-	How to enhance the UAS application enabler to support enhanced trajectory tracking for UAVs utilizing the sensing results.

-	How to enhance the UAS application enabler to support enhanced DAA service/functionalities for UAVs utilizing the sensing results.

This pCR is to handle the second open issue.

3. Conclusions

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-15.

* * * First Change * * * *

6.x	Solution #x: Sensing based DAA service

6.x.1	Solution description

This solution intends to solve the second open issue of key issue #2, i.e. enhance DAA service by utilizing sensing results. This solution is based on the procedures in clause 7.7.2 of 3GPP TS 23.255.

6.x.2	Procedures

#### 6.x.2.1	Configuration of sensing based DAA policies to the UAE layer

##### 6.x.2.1.1	Management of sensing based DAA support configuration

Figure 6.x.2.1-1 illustrates the sensing based DAA support management procedure where the UAE server receives an application request for managing the sensing based DAA application policy from the UAS application specific server.

Pre-condition:

1.	The UAV has received its UAS ID from the UAS application specific server.

2.	The UAV has performed the UAS UE registration procedure.

Figure6.x.2.1-1: Sensing based DAA support management procedure

1.	The UAS application specific server sends to the UAE server a sensing based DAA support management request. The request includes the UAV (UAE client) identifier and the sensing based DAA application policy. The sensing based DAA application policy includes parameter for the control of sensing based DAA.

2.	The UAE server shall send to the UAS application specific server a sensing based DAA support management response with a positive or negative acknowledgement of the request.

3.	The UAE server shall timestamp and store the sensing based DAA application policy and execute the sensing based DAA configuration according to clause 6.x.2.1.1.

4.	After execution of sensing based DAA configuration, the UAE server shall send a sensing based DAA support management complete to the UAS application specific server.

##### 6.x.2.1.2	Sensing based DAA support configuration procedure

Figure 6.x.2.1.2-1 illustrates the sensing based DAA support configuration procedure. This procedure enables the configuration of the UAE server, based on a request from UAS application specific server to configure the sensing based DAA application policy.

Pre-conditions:

1.	UAE server has performed the DAA support management procedure according to clause 6.x.2.1.1.

2.	UAE server is configured with NEF 1..N information of their supported region of operation.

Figure 6.x.2.1.2-1: sensing based DAA support configuration procedure

1.	The UAE server obtains and initiates tracking the UAV location from the location management server as speicified in 3GPP TS 23.434.

2.	The UAE server determines the dynamic area for sensing based on the location of the UAV and the sensing based DAA application policy, and then deteremines the NEF(s) operating in the dynamic area.

3.	The UAE server sends subscribe sensing results of the dynamic area request to the NEF who is responsidble for the sensing result expose of the corresponding area.

4. 	The NEF stores the subscription.

5.	The NEF sends subscription response to UAE server, in which the sensing result of requested dynamic area is included.

6.	The NEF detects UAVs in proximity based on the obtained sensing results and sensing based DAA application policy.

Editor Note: the detail information of request message depends on SA2

Editor Note: the detail of subscription response (i.e. IEs, periodical or event based response) denpends on SA2

#### 6.x.2.2	Sensing based DAA event reporting

Figure 6.x.2.2-1 illustrates the procedure of sensing based DAA event reporting.

Pre-conditions:

1.	UAE server has detected UAVs in proximity as specified in 6.x.2.1.2.

Figure 6.x.2.2 -1: Sensing based DAA event reporting

1.	The UAE client shall send a sensing based DAA event information to the UAS application specific server with information about one or more UAVs in proximity as detected and the corresponding timestamp.

2.	The UAS application specific server provides a sensing based DAA event information acknowledge to the UAE server.