3GPP TSG-SA WG6 Meeting #69	S6-254513

Wuhan, P.R.China, 13th – 17th October 2025	revision of S6-254226

Source:	Ericsson

Title:	New Solution for KI#1 on New Service on Energy Saving Assistance

Spec:	3GPP TR 23.700-44 v0.1.0

Agenda item:	9.9

Document for:	Approval

Contact:	Jing Yue <jing.yue@ericsson.com>

1. Introduction

As described in Key Issue #1 (TR 23.700-44 clause 5.1), the study aspects on enhance application enablement layer architecture and services to support energy saving include:

- Whether enhancements to existing application enablement layer services or a new application enabler service is required to support energy savings operations in the application enablement layer.

According to the use cases and comment functions identified in S6-254224, service on energy saving assistance is needed for supporting consumer on energy saving. This pCR proposes new Energy Saving Enablement (ESE) service to assist consumer on energy saving.

2. Reason for Change

This pCR proposes a new solution for Key Issue #1 by introducing a new Energy Saving Enablement (ESE) service to assist consumer on energy saving.

3. Conclusions

This paper proposes solution on new Energy Assistance and Management Enablement (ESE) service to assist consumer on energy saving.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-44 v0.1.0.

* * * First Change * * * *

## 6.0	Mapping of solutions to key issues

Table 6.0-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI#5 | KI#6 |
| --- | --- | --- | --- | --- | --- | --- |
| Sol #1 |  |  |  |  |  |  |
| Sol #2 |  |  |  |  |  |  |
| Sol #X2 | X |  |  |  |  |  |

* * * Next Change * * * *

## 6.X2	Solution #X2: New Service on Energy Saving Assistance

### 6.X2.1	Solution Description

#### 6.X2.1.0	General

The following clauses specify the procedure and information flow of ESE (Energy Saving Enablement) service on energy saving assistance.

Editor’s Note:	Whether a standalone architecture for energy saving enablement or integrate energy saving functionality with the existing SEAL enablers will be decided during solution evaluation and conclusion stage of the study phase.

#### 6.X2.1.1	Procedure

Figure 6.X2.1.1-1: Energy saving assistance

- A consumer (e.g., VAL server, SEAL server/client) sends energy saving assistance request to ESE server. The request includes information as described in Table 6.X2.1.2.2-1.

Editor’s Note:	Whether the ESE server is a standalone energy saving enablement server or integrate energy saving functionality with the existing SEAL enablement server will be decided during solution evaluation and conclusion stage of the study phase.

- The ESE server authenticates and authorizes the request. If authorized, the ESE server determines the next actions (e.g. collect energy-related data, monitor energy-related information/status) for assisting energy saving. The ESE server can consolidate similar requests from Consumers (e.g., data collection from same source into single southbound requests, avoid re-fetching data that it already has if the request parameters allow it such as time frame matches, etc.).

- The ESE sends energy-related data collection response to the consumer. The response includes information as described in Table 6.X2.1.2.3-1.

4.	The ESE server performs operations for the energy saving request.

4a.	The ESE server may collect energy-related data.

4b.	The ESE server may monitor the energy-related information/status.

Editor’s Note:	Whether energy-related data collection via new energy saving enablement service/functionality or via existing enablement service (e.g. A-DCCF) is FFS.

5.	The ESE server generates assistance information for energy saving based on the data/information obtained in step 4.

6.	The ESE server sends energy saving assistance notify to the consumer with the assistance information. The notify includes information as described in Table 6.X2.1.2.4-1.

#### 6.X2.1.2	Information Flows

##### 6.X2.1.2.1	General

The following information flows are specified for energy saving assistance.

##### 6.X2.1.2.2	Energy saving assistance request

Table 6.X2.1.2.2-1 shows the request sent by an ESE service consumer to an ESE server for energy saving assistance procedure.

Table 6.X2.1.2.2-1: Energy saving assistance request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| VAL service ID | M | The identity of the VAL service for which the request applies. |
| Energy saving target(s) | M | The identity(ies) of the target energy saving entity(ies) (e.g. VAL UE ID(s), SEAL server ID(s)) |
| Assistance requirements | M | Identifies the requirement for the assistance on energy saving. |
| >Assistance information type | M | Identifies the assistance information type required for energy saving (e.g. information on entity which consume the most/least energy for performing an operation, assistance information for selection of entity(ies)). |
| >Operation information | O | Identifies the information of the operation. The IE shall be provided if the Assistance information type is information on entity which consume the most/least energy for performing an operation. |
| >Energy requirements | O | Identifies the requirements on energy for the selection of entity(ies). The IE shall be provided if the Assistance information type is assistance information for selection of entity(ies). |
| Area of Interest | O | The geographical or service area for which the request applies. |
| Reporting requirements | O | It describes the requirements for reporting. This requirement may include e.g. the type of reporting (periodic or one-time report), the reporting periodicity in case of periodic. |

##### 6.X2.1.2.3	Energy saving assistance response

Table 6.X2.1.2.3-1 shows the response sent by the ESE server to the consumer for energy saving assistance procedure.

Table 6.X2.1.2.3-1: Energy saving assistance response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the request (positive or negative acknowledgement). |
| Subscription ID | O | Identifier of the subscription. It shall be provided if the request if the Result is positive acknowledgement. |

##### 6.X2.1.2.4	Energy saving assistance notify

Table 6.X2.1.2.4-1 shows the notification sent by the ESE server to the consumer for energy saving assistance procedure.

Table 6.X2.1.2.4-1: Energy saving assistance notify

| Information element | Status | Description |
| --- | --- | --- |
| Success status | O (NOTE) | Indicates that energy saving assistance was successful. |
| >Assistance information | M | Provides energy saving assistance information. |
| Failure response | O (NOTE) | Indicates that energy saving assistance was failure. |
| >Cause | M | Reason for the failure. |
| NOTE: One of the information elements shall be provided in the output. |  |  |

### 6.X2.2	Architecture impacts

Editor’s Note:	The architecture impacts of the solution is FFS.

### 6.X2.3	Solution evaluation

Editor’s Note:	The evaluation of the solution is FFS.

# *** End of Changes ***