3GPP TSG-SA6 Meeting #70	S6-255059

# Dallas, USA, 17th - 21st Nov 2025			                                         (revision of S6-xxxxxx)

Source:	Airbus

Title:		Discussion paper on Location management stage2-stage3 alignment

Spec:	3GPP TS 23.280, TS 24.283

Agenda item:	8.2

Document for:	Discussion and agreement

Contact:	Jukka Vialen (jukka.vialen@airbus.com)

1. Introduction

CT1 has introduced a new (Rel-19) TS 24.283 Mission Critical Location Management (MCLoc); Protocol specification. The LMC-LMS interface (CSC-14 reference point) in this new protocol specification utilizes HTTP only. Same change is applicable also for CSC-22 (LMS-LMS) and CSC-23 (LMS-gateway server). In the CSC-15 (LMS-MCS server), SIP is used only for group dynamic data subscribe/notify.

In addition to the reference points, also the location management information flows and procedures in TS 23.280 are not aligned with TS 24.283.

The required changes to the reference points in TS 23.280 are proposed in CR0698 (S6-255060) with Rel-20 mirror in CR0699 (S6-255061).

The potential changes to information flows and procedures are discussed in this document.

2. Proposal

Since the amount of changes is quite big, one CR to cover everything would be impractical and difficult to handle.

The proposal is to first agree the changes to the reference points (S6-255060 and S6-255061), including an EN indicating that further changes are expected to information flows and procedures.

Furthermore, an LS to CT1 may be necessary.

### Review of TS 23.280 location management related information flows against TS 24.283:

| TS 23.280 (Rel-19) | TS 24.283 (Rel-19) |
| --- | --- |
| Not in TS 23.280, needs to be added. | 6.2.2.2 RegistrationRequest 6.2.2.2.2 LMC register to LMS using RegistrationRequest operation -The reason for LM specific client registration is that, unlike other CSC servers, LMS may be the first to contact LMC (request location from the LMC). |
| Not in TS 23.280, needs to be added, after CT1 clarifies why this procedure is needed. | 6.2.2.3 RegistrationUpdate 6.2.2.3.2 LMC updating registration using RegistrationUpdate operation -why is this needed? |
| Not in TS 23.280, needs to be added, after CT1 clarifies why this procedure is needed. | 6.2.2.4 RegistrationDelete 6.2.2.4.2 LMC deregistering using RegistrationDelete operation -why is this needed? |
| 10.9.2.1 Location reporting configuration Also, the location history reporting functionality must be reviewed and aligned. Clauses 10.9.2.11-16 and 10.9.3.9. It seems that stage3 has added ‘locationHistory’ attribute to the ConfigurationNotify to command LMC to start storing location history. In stage2, the location history recording is implicitely always on (?) whenever LMC location feature is activated with location reporting configuration. | 6.2.2.5 ConfigurationNotify 6.2.2.5.2 LMS send configuration using ConfigurationNotify operation -see especially the “locationHistory attribute” |
| Not present in TS 23.280, needs to be added. | 6.2.2.6 ConfigurationRequest 6.2.2.6.2 LMC request the location configuration using ConfigurationRequest operation -“The LMC may at any time request to download the location configuration from the LMS” |
| 10.9.2.3 Location information request In stage2, this message can be sent: LMS->LMC, LMS->LMS and LMC->LMS | 6.2.2.7 LocationReport 6.2.2.7.2 LMS send location request using LocationReportRequest - The LMS may at any time determine to send a location report request to a registered LMC LMS -> LMC #### 6.3.2.2 LocationRequest #### 6.3.2.2.2 Client request location information to LMS using LocationRequest operation -can be initiated by LMC, MC service server, IWF, LMS |
| 10.9.2.2 Location information report In stage2, this message can be sent: LMC->LMS, LMS->LMC, LMS->MCS server, LMS->LMS 10.9.2.7 Location information notification LMS->MCS server LMS->LMC LMS->LMS | 6.2.2.8 LocationReportPublish 6.2.2.8.2 LMC reports to LMS using LocationReportPublish LMC -> LMS 6.3.2.4 LocationInfoNotify 6.3.2.4.2 LMS Notify clients using LocationInfoNotify operation - The LMS use this procedure to notify an authorized user about other users' location information -Can ‘notify’ also be used in LMS->LMS and LMS->MCS server? |
| 10.9.2.5 Location information subscription request (10.9.2.6 Location information subscription response) LMC->LMS, MCS server -> LMS, LMS->LMS | 6.3.2.3 LocationInfoSubscribe 6.3.2.3.2 Client request subscription using LocationInfoSubscribe operation - This service operation is used by the LMC to subscribe to location information at the LMS -can be initiated by LMC, MC service server, IWF, LMS |
| 10.9.2.8 Location information cancel subscription request LMC->LMS, MCS server -> LMS, LMS->LMS 10.9.2.9 Location information cancel subscription response | 6.3.2.6 LocationInfoSubscribeDelete 6.3.2.6.2 Client requests unsubscribe using LMS_LocationInfoSubscribeDelete operation Can be initiated by: LMC, MC service server, IWF, LMS |
| No dedicated infoflow/procedure in stage2. For alignment, should add one sentence (and a parameter?) to the: 10.9.2.5 Location information subscription request Alternatively, add a dedicated procedure/information flow. | 6.3.2.5 LocationInfoSubscribeUpdate 6.3.2.5.2 Client requests update of subscription using LocationInfoSubscribeUpdate operation - To update an existing subscription -can be initiated by LMC, MC service server, IWF, LMS |
| 10.9.2.4 Location reporting trigger - LMC->LMS - can request multiple targets in one messages: “MC service ID list” or “MC service group ID list” | 6.3.2.7 LocationInfoConfigRequest 6.3.2.7.2 Client request location configuration change for another client from LMS using LocationInfoConfigRequest operation -can be initiated by LMC, MC service server, IWF, LMS -only one target client is possible -note also the ‘locationHistory’ attribute, not included in the stage2 infoflow |
| 10.9.2.4a Location reporting trigger cancel | ? |
| 10.9.2.11 Location information history status request 10.9.2.12 Location information history status report 10.9.2.13 Location information history request 10.9.2.14 Location information history report 10.9.2.15 Location information history cancel request 10.9.2.16 Location information history cancel response Stage3 does not have dedicated messages for location history, but a ‘locationHistory’ attribute in request/report messages. Stage2 information flows should be aligned with stage3. See also 10.9.2.1 above. | 6.2.2.5.2 LMS send configuration using ConfigurationNotify operation 6.2.2.7.2 LMS send location request using LocationReportRequest 6.2.2.8.2 LMC reports to LMS using LocationReportPublish 6.3.2.2.2 Client request location information to LMS using LocationRequest operation 6.3.2.7.2 Client request location configuration change for another client from LMS using LocationInfoConfigRequest operation |
| 10.9.2.17 Location reporting temporary configuration request 10.9.2.18 Location reporting temporary configuration response - These could be removed, the same functionaltiy can be implemented with loc-trigger and loc-trigger-cancel | Not possible to specify the criteria in detail -> this would be implementation specific functionality |
| 10.9.2.19 Restrict location information dissemination request 10.9.2.20 Restrict location information dissemination response 10.9.2.21 Restrict location information dissemination notification - ???? |  |
| 10.9.2.22 Location reporting trigger override indication 10.9.2.23 Location reporting cancel trigger override indication - ??? |  |