3GPP TSG-SA WG6 Meeting #70	S6-255085

Dallas, United States 17th – 21st November 2025	(revision of S6-255xxx)

Source:	Netherlands Police

Title:	Pseudo-CR on DiMo scenarios for location service

Spec:	3GPP TR 23700-37-040

Agenda item:	9.1

Document for:	Approval

Contact:	keesdotverweijatpolitiedotnl

1. Introduction

This pCR introduces DiMo scenarios for location service.

2. Reason for Change

This scenarios should be added to the study.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23700-37 v 0.4.0.

* * * First Change * * * *

## 4.x	Scenario x: Discreet monitoring of location service

### 4.x.1	Scenario x.1: Target user reporting location

This scenario describes the case where authorized MC user A requests discreet monitoring for communications involving target MC user B where MC user A and MC user B are served by the same MC system, MC system A and target MC service user B is within the authority of authorized MC user A. The scenario is illustrated in figure 4.x.1-1 below.

Figure 4.x.1-1: Discreet monitoring of target user reporting location

The scenario consists of the following aspects:

-	Authorized MC user A identifies MC user B as the target for discreet monitoring. MC system A verifies that MC user A is authorized to perform discreet monitoring on MC user B. This aspect takes place before any communications involving MC user B can be subject to discreet monitoring.

-	Triggered by an event, target MC user B reports location to MC system A.

-	MC system A provides the location report of target MC user B to authorized MC user A.

### 4.x.2	Scenario x.2: Target user sending a location reporting trigger

This scenario describes the case where authorized MC user A requests discreet monitoring for communications involving target MC user B where MC user A and MC user B are served by the same MC system, MC system A and target MC service user B is within the authority of authorized MC user A. The scenario is illustrated in figure 4.x.2-1 below.

Figure 4.x.2-1: Discreet monitoring of target user sending location trigger

The scenario consists of the following aspects:

-	Authorized MC user A identifies MC user B as the target for discreet monitoring. MC system A verifies that MC user A is authorized to perform discreet monitoring on MC user B. This aspect takes place before any communications involving MC user B can be subject to discreet monitoring.

-	Target MC user B sends a location trigger configuration to MC system A for another MC user of MC system A.

-	MC system A provides the location trigger to authorized MC user A.

### 4.x.3	Scenario x.3: Target user receiving a location report

This scenario describes the case where authorized MC user A requests discreet monitoring for communications involving target MC user B where MC user A and MC user B are served by the same MC system, MC system A and target MC service user B is within the authority of authorized MC user A. The scenario is illustrated in figure 4.x.3-1 below.

Figure 4.x.3-1: Discreet monitoring of target user reporting location

The scenario consists of the following aspects:

-	Authorized MC user A identifies MC user B as the target for discreet monitoring. MC system A verifies that MC user A is authorized to perform discreet monitoring on MC user B. This aspect takes place before any communications involving MC user B can be subject to discreet monitoring.

-	Target MC user B has subscribed to location reports from an MC user of MC system A.

-	Target MC user B receives a location report from MC system A.

-	MC system A provides the location report to authorized MC user A.

* * * End of Changes * * * *