3GPP TSG-SA WG6 Meeting #70	S6-255115

Dallas, United States 17th – 21st November 2025	(revision of S6-25xxxx)

Source:	Telefónica, Apple, Fogus, UMA

Title:	TR 23.947 Example Application Flows: CAPIF + SEAL

Spec:	3GPP TR 23.947

Agenda item:	9.14

Document for:	Approval

Contact:	w_featherstone@apple.com

1. Introduction

Add CAPIF+SEAL Application Flow clause within the overall Example Application Flows clause.

2. Reason for Change

Complete clause 6.3.2 and add other headings.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.947, version 0.2.0.

* * * First Change * * * *

## 6.3	Example Application Flows

The following flows will be considered “CAPIF + EDGEAPP”, “CAPIF + SEAL” and “CAPIF + EDGEAPP + SEAL”

### 6.3.1 CAPIF+EDGEAPP Application Flow

### 6.3.2 CAPIF+SEAL Application Flow

This subsection illustrates the logical flow of interactions between the SEAL Server, VAL Server, and VAL Client when CAPIF is used to enable secure API exposure, discovery, and invocation.

Figure 6.3.2-1: CAPIF+SEAL Flow

In this configuration, the SEAL Server is onboarded and registered in CAPIF as an API Provider, while the VAL Server acts as an API Invoker that consumes SEAL Enabler APIs through CAPIF-issued authorization tokens. The VAL Client, running on the user equipment (UE), accesses the SEAL capabilities directly through APIs exposed by the VAL Server.

#### 6.3.2.1 SEAL Server Onboarding

The onboarding procedure allows the SEAL Server to become a trusted entity within the CAPIF ecosystem. By performing this process, the SEAL Server obtains its official provider identity, operator-signed certificates, and the authorization to later publish its SEAL Enabler APIs (e.g., Group, Configuration, or Location).

This procedure establishes the initial trust relationship between the SEAL Server and the CCF, ensuring that subsequent CAPIF operations occur over authenticated and authorized interfaces.

The message flow for this onboarding is depicted in Figure 6.3.2.1-1.

Figure 6.3.2.1-1: SEAL Server onboarding as CAPIF API Provider

1.	The SEAL Server establishes a TLS session with the CCF using the CAPIF Root CA provisioned during bootstrap. This step authenticates both parties and guarantees confidentiality and integrity for subsequent control messages.

2.	After establishing the secure channel, the SEAL Server sends a provider onboarding request to the CCF using the endpoint
POST /api-provider-management/v1/registrations.

The HTTP request includes:

  - Header:
Content-Type: application/json

  - Body:
A csrSet structure containing Certificate Signing Requests (CSRs) for the SEAL Server’s internal exposure functions:

    - AMF – Application Management Function

    - APF – Access Provisioning Function

    - AEF – API Exposure Function

Each CSR allows the CCF to issue corresponding provider-side certificates that will later be used to authenticate CAPIF interfaces.

Upon validating the request and checking the SEAL Server’s credentials, the CCF issues an HTTP 201 (Created) response. The response body includes:

  - apiProvDomId – a globally unique identifier representing the SEAL Server’s provider domain within CAPIF.

  - issuedCerts – the set of signed certificates corresponding to the submitted CSRs (AMF, APF, AEF).

This response completes the onboarding exchange and establishes the SEAL Server as a recognized API Provider under the operator’s CAPIF domain.

3.	Upon receiving the onboarding response, the SEAL Server stores the assigned apiProvDomId and the newly issued certificates in its local configuration repository.
This information will be used to authenticate future CAPIF interactions (such as API publication or discovery support).

It is now a trusted CAPIF API Provider, capable of publishing SEAL APIs and being discovered by VAL entities through the CAPIF catalogue in subsequent procedures.

#### 6.3.2.2 SEAL Server Publish APIs

#### 6.3.2.3 VAL Server Onboarding

#### 6.3.2.4 VAL Server Discover

#### 6.3.2.5 VAL Server Security Context

#### 6.3.2.6 VAL Server requesting token

#### 6.3.2.7 VAL Client Requesting VAL Server Service

### 6.3.3 CAPIF+EDGEAPP+SEAL Application Flow

* * * End of Changes * * * *