3GPP TSG-SA WG6 Meeting #70	S6-255148

Dallas, USA, 17th – 21st November 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.437 | CR | 0095 | rev | - | Current version: | 20.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update PINE profile visibility for discovery by other PINE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI20, Metaverse_App |  |  |  |  |  | Date: |  |  | 2025-11-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-20 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | S6-245736 introduced the PINE discovery procedure in the specification based on conclusion of the KI#6 of the TR 23.700-21, solution#12. Current specifications support the use case where for example - PINE A can send the request to PEMC to discover the PINE B to host the application traffic. After the determination, the PEMC sends the endpoint information to PINE A and the PINE A can offload the traffic either directly to PINE B or via PEGC. However, in the above case if PINE B does not have resource available to host the application traffic of PINE A or not interested to perform offloading task due to various conditions, then this results in unnecessary message exchanges. How to avoid this unnecessary signalling and further optimizing the operation is addressed in this CR. |  |  |  |  |  |  |  |  |
| Summary of change: |  | PINE either not willing or willing to participate in offloading of application traffic from other PINE on temporary basis can update its visibility status accordingly with the PEMC, so that PEMC could either include the PINE or not when other PINE request for PINE discovery for offload purpose. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Results in signalling overhead and lot of failures due to requested PINE not able to offload application traffic from the requesting PINE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.8.2.x (new), 8.5.8.3.1, 8.5.8.3.x (new), 8.5.8.3.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

##### 8.5.8.2.x	PINE Profile update within PIN

Figure 8.5.8.2.x-1 illustrates procedure of PIN Element (PINE) to update its profile with the PEMC to indicate its willingness to support in PINE discovery procedure by another PINE.

Pre-conditions:

1.	The UE (PINE) has already joined the PIN as specified in clause 8.5.8.2.

3.	The PINE has been authorized to communicate with the PEMC and already has application layer connection with PEMC;

4.	Profile information about the PIN is available at the PEMC and PIN server.

Figure 8.5.8.2.x-1:  PIN Element Profile update procedure

1.	The PINE client sends request to the PIN element with Management capability (PEMC) client to update its profile. The request includes PIN element identifier, the security credentials of the PINE client received during authorization procedure and may include the UE identifier (such as GPSI), PIN client ID, UE location, PIN ID as an identifier of the PIN and update in its services/capabilities which also includes its willingness to participate in future PINE discovery by another PINE or not with visibility status.

2.	The PEMC authorizes the PINE client and updates the PINE profile based on the provided parameters in the request and on information of PIN Element.

3.	The PEMC client sends response back to the PINE client acknowledging the update in the PINE profile.

* * * Next Change * * * *

##### 8.5.8.3.1	General

The following information flows are specified for PIN creation:

-	PIN Management PINE join into PIN request and response;

-	PIN Management PINE leave from PIN request and response;

-	PIN Management PINE discovery request and response;

-	PIN Management PINE Profile update request and response;

* * * Next Change * * * *

##### 8.5.8.3.x	PIN Management PINE Profile update request

Table 8.5.8.3.x-1 shows the request sent by a PINE client to a PEMC client for the PINE profile update.

Table 8.5.8.3.x-1: PIN Management PINE Profile update request

| Parameter Name | Status | Parameter Description |
| --- | --- | --- |
| PIN ID | M | The identifier of the PIN to which the PIN element wants to join |
| UE identifier | O | PIN Element or UE identifier |
| PIN client ID | M | The unique identity of the PIN client within PIN |
| PINE role | O | Role of PINE: PEMC, PEGC, other |
| Application List | O | List of application identities |
| > Application Identity | O | Identity of the application |
| > Application schedule | O | Operational schedules of each application (e.g., time windows) |
| > Application KPIs | O | Minimum KPIs required by each application to operate effectively within the PIN (e.g., PIN bandwidth, PIN request rate, PIN response time) |
| capabilities | O | capabilities of the PIN client like whether it can be assigned with the role of PEMC or PEGC etc. |
| Visibility status | O | This status indicates if the PIN client is willing to be discovered by the other PINE. (for e.g. the value is set to “True” if allowed to be discoverable by other PINE and “False” otherwise). |
| access type | O | Access type supported for the communication |
| Supported services | O | Identifies the list of services the PINE is providing. |
| Port number | O | Port number of PIN client on PINE to support PIN enabler layer communication. NOTE: The port number is the port used by a PINE to expose a service within the PIN. |
| UE location | O | The location information of the UE. The UE location is described in clause 7.2.7. |

* * * Next Change * * * *

##### 8.5.8.3.y	PIN Management PINE Profile update response

Table 8.5.8.3.y-1 shows the response sent by the PEMC to the PINE.

Table 8.5.8.3.y-1: PIN Management PINE Profile update response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the request (e.g., success or fail), including failure reason if needed. |
| > PIN client profile | O | Updated PIN client profile |

* * * End of Change * * * *