3GPP TSG-SA WG6 Meeting #70	S6-255200

Dallas, USA, 17th – 21th November 2025	(revision of S6-25xxxx)

Source:	Ericsson

Title:	Update to Solution #3 on Evaluation

Spec:	3GPP TR 23.700-83 v1.1.0

Agenda item:	9.4

Document for:	Approval

Contact:	Jing Yue <jing.yue@ericsson.com>

1. Introduction

This pCR proposes the inclusion of solution evaluation of the solution #3 on New AIMLE Service on ML Model Maintenance. Furthermore, the content for architecture impacts and the corresponding APIs section input are added.

2. Reason for Change

The solution evaluation, the corresponding APIs, and the architecture impacts clauses of the solution #3 are missing.

3. Conclusions

This paper proposes to add content for the solution evaluation, the corresponding APIs, and the architectural impacts claues of solution #3 on new AIMLE service on ML model maintenance.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-83 v1.1.0.

* * * First Change * * * *

### 7.3.2	Architecture Impacts

There were no identified architectural impacts in this solution.

### 7.3.3	Corresponding APIs

This subclause provides a summary of the corresponding API for solution #3.

-	ML model maintenance subscription API (subscribe / notify model; API provider: AIMLE Server; known consumer: AIMLE service consumer; corresponding to step 1, 3, and 7 of clause 7.3.1).

### 7.3.4	Solution evaluation

This solution supports ML model maintenance by addressing Key Issue #2: support enhancement for evaluation and determination of ML model performance and correctness within AIML Enablement Layer.

The AIMLE service consumers contact the AIMLE server to obtain ML model maintenance information, and the AIMLE server returns this information by retrieving it using the ML Evaluation Profile ID. In cases where the performance of the ML model is not as expected, the AIMLE server determines the factors to be considered in the ML model maintenance evaluation. Based on this information, the AIMLE server triggers actions such as model replacement or retraining, if necessary.

This solution does not impact the current AIMLE architecture.

* * * End of Change * * * *