3GPP TSG-SA WG6 Meeting #70	S6-255229

Dallas, United States 17th – 21st November 2025	(revision of S6-255xxx)

Source:	China Mobile

Title:	Pseudo-CR on Solution evaluation and interim conclusion of KI#4

Spec:	3GPP TR 23.700-57 v0.4.0

Agenda item:	9.6

Document for:	Approval

Contact:	liuyueyjy@chinamobile.com

1. Introduction

This pCR provides the solution evaluation and interim conclusion of KI#4.

2. Reason for Change

This pCR provides the solution evaluation and interim conclusion of KI#4.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-57 v0.4.0.

* * * First Change * * * *

7.3.4	Key Issue #4: Northbound interface of DCAR

The open issues of this KI are as follows:

1.	Study the northbound interface of DCAR, which is used to manage the DC applications in the DCAR by application providers.

2.	Whether the MMTel-3 reference point specified in 3GPP TS 23.392 [2] can be reused as the northbound interface of DCAR, or whether enhancement or modification is needed.

To address the open issues the following solution has been proposed:

-	Solution #2: Northbound interface of DCAR.

In this solution, a new reference point, MMTel-5, is introduced between the MMTel Enabler Server and the DCAR. The interactions via the MMTel-5 reference point, including uploading, updating, and deleting DC applications and profiles, are needed to be supported by both the MMTel Enabler Server and the DCAR.

* * * Next Change * * * *

8.4	Interim conclusion of KI#4

For KI#4 on "Northbound interface of DCAR" the following interim conclusions are agreed:

a)	a new MMTel-5 reference point is added between the MMTel Enabler Server and the DCAR;

b)	the MMTel Enabler server can upload, update, and delete DC applications and profiles on DCAR.

The Solution#2 will be used for the architecture and interaction on Northbound interface of DCAR. This solution will be considered as the basis for technical specification with necessary enhancements and additions.