3GPP TSG-SA WG6 Meeting #70	S6-255271

Dallas, USA, 17th – 21st November 2025	revision of S6-25xxxx

Source:	Ericsson

Title:	New Solution for KI#1 on Sensing Requests Process and Sensing Result Reuse

Spec:	3GPP TR 23.700-15 v0.2.0

Agenda item:	9.13

Document for:	Approval

Contact:	Jing Yue <jing.yue@ericsson.com>

1. Introduction

In Key Issue #1, there is study aspect on whether and how to resolve conflicts in sensing service requirements among different vertical applications or under resource limitations.

Multiple sensing requests may from different consumers to the SEAL sensing server. These requests on sensing result can be processed (e.g. merged, split, or grouped) and form request(s) to the sensing result source (5GC NF (NEF)), to remove the conflict in the requests (e.g., requirements on collection of sensing result on area are the same but time periods are different), and saving singling to 5GC NF (NEF). In addition, cached sensing result can be reused if fulfil the new request on sensing result, to save resource on sensing result collection.

This pCR proposes solution for KI#1 on sensing requests process and sensing result reuse.

2. Reason for Change

This pCR proposes solution for KI#1 on sensing requests process and sensing result reuse.

3. Conclusions

This paper proposes solution on sensing requests process and sensing result reuse.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-15 v0.2.0.

* * * First Change * * * *

## 6.1	Mapping of solutions to key issues

Table 6.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 |
| --- | --- | --- | --- | --- |
| Sol #1 | x |  |  |  |
| Sol #2 | x |  |  |  |
| Sol #3 | x |  |  |  |
| Sol #4 |  | x |  |  |
| Sol #5 |  | x |  |  |
| Sol #6 |  | x |  |  |
| Sol #7 |  | x |  |  |
| Sol #8 |  |  | x |  |
| Sol #9 |  |  | x |  |
| Sol #10 |  |  | x |  |
| Sol #11 |  |  |  | x |
| Sol X | x |  |  |  |

* * * Next Change * * * *

## 6.X	Solution #X: Sensing Requests Process and Sensing Result Reuse

### 6.X.1	Solution Description

#### 6.X.1.1	General

The following clauses specify SEAL Sensing Server service on support processing sensing requests from different consumers to remove conflict in the request, and reuse of sensing result to save resource for sensing result collection.

#### 6.X.1.2	Procedure

Pre-condition:

- The requests from different consumers are received at the SEAL sensing server in the same time period, so that the SEAL sensing server can process these requests.

Figure 6.X.1.2-1: Procedure on support sensing requests process and sensing result reuse

- Multiple consumers (e.g. VAL server, SEAL server, UAE server, VAE server) send sensing requests separately to the SEAL sensing server. The request contains the requirements on sensing result collection (e.g. target time period for collection sensing result, target area).

- Upon receiving the request(s), the SEAL sensing server checks for the relevant authorization for the request(s). If the authorization is successful, the SEAL sensing server determines next action on processing of the request(s).

- The SEAL sensing server processes the requests from different consumers to remove conflict, e.g., merge requests into one request for sensing collection, split one request into multiple ones, or grouped the requests into smaller number of requests.

- The SEAL sensing server checks the cached sensing result and determines whether the required sensing result are available. If sensing result is available, go to step 5.

- If the required sensing result is not available, the SEAL sensing server interacts with the 5GC NF (NEF) for collecting sensing result.

- The SEAL Sensing Server caches the received sensing result, and processes the collected sensing result based on the request in step 1.

- The SEAL Sensing Server sends sensing responses to the consumers.

#### 6.X.1.3	Information Flows

Editor’s Note:	The information flows of the solution is FFS.

### 6.X.2	Architecture impacts

Editor’s Note:	The architecture impacts of the solution is FFS.

### 6.X.3	Solution evaluation

Editor’s Note:	The evaluation of the solution is FFS.

# *** End of Changes ***