3GPP TSG-SA WG6 Meeting #70	S6-255280

Dallas, United States 17th – 21st November 2025	(revision of S6-255xxx)

Source:	Samsung

Title:	Pseudo-CR on Updates to Solution #26

Spec:	3GPP TR 23.700-83 v1.1.0

Agenda item:	9.4

Document for:	Approval

Contact:	Narendranath Durga Tangudu (n.tangudu@samsung.com)

1. Introduction

This pCR proposes various udpates to solution #26 (AIMLE server discovery).

2. Reason for Change

This pCR proposes various updates to solution #26. The udpates are related to Architecture impacts, APIs and solution evaluation. The clause 10.1.1 is also updated with solution #26 as new API/procedure.

The EN (Editor’s Note: The relation with CAPIF discovery is FFS) is clarified with a note. AIMLE server discovery is not API discovery. AIMLE server discovery is to identify another AIMLE server that can match the AIMLE service consumer requirements. This solution does not involve or impact the discovery of APIs as specified in CAPIF.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-83 v1.1.0.

* * * First Change * * * *

#### 7.26.1.2 	Procedure

Assumptions:

- AIMLE server is aware of the other AIMLE servers’ information, for example, AIMLE servers may have registered their capabilities with ML repository.

- There are various deployments that include multiple AIMLE servers, for example, hierarchichal, Edge deployments etc.

Pre-conditions:

- Each AIMLE client or VAL server is pre-configured with a default AIMLE server URI. The default AIMLE server exposes the discovery API that returns a list of AIMLE servers matching given criteria.

Figure 7.26.1.2-1: AIMLE server discovery procedure

1.	An AIMLE service consumer (VAL server, AIMLE client) sends an AIMLE server discovery request message to discover the list of suitable AIMLE servers. The request message includes AIMLE server requirements to discover suitable servers based on discovery criteria like geographical proximity etc. The AIMLE server discovery request includes information as described in Table 7.25.1.3.1-1.

2.	The AIMLE server performs authentication and authorization checks to determine if the requestor can discover AIMLE servers. If the requestor is authorized, the AIMLE server performs the discovery based on the discovery criteria in the request message. The server utilizes the provided criteria to filter and identify suitable AIMLE servers. If there are multiple AIMLE servers that fulfil the criteria, then the default AIMLE server selects the suitable servers, ensuring that the returned list is optimized for the requestor's needs, improving performance and reliability.

3.	The default AIMLE servers send the list of the suitable AIMLE servers to the requestor. The information is included in Table 7.26.1.3.2-1. The available list may have an availability duration, and the requestor can communicate with any AIMLE server based on availability duration information. This duration informs the requestor about the time frame within which the servers are guaranteed to be available, allowing for better planning and resource allocation.

The discovery request needs to be initiated again by the AIMLE service consumer if the availability duration information is expired. This ensures that the information remains current and relevant, preventing issues related to outdated server availability.

NOTE: AIMLE server discovery is to identify another AIMLE server that can match the AIMLE service consumer requirements. This procedure does not involve or impact the discovery of APIs as specified in CAPIF.

* * * Next Change * * * *

### 7.26.2 	Architecture Impacts

This solution proposes new procedure for AIMLE discovery. The service consumer is AIMLE service consumer (e.g., AIMLE client, VAL server) and the service producer is AIMLE server. There are no new functional elements or reference points/interfaces introduced in this solution. Hence, there are no architecture impacts.

### 7.26.3	 Corresponding APIs

A new API is needed for the discovery of AIMLE server. The API details are shown below.

### A AIMLE server discovery API #### A.1 General Table A.1-1 illustrates the API for AIMLE server discovery. This API enables an AIMLE service consumer (e.g., AIMLE client, VAL server) to discover the AIMLE server information from an AIMLE server. Table 9.2.16.1-1: Aimles_ServerDiscover API API Name; API Operations; Operation Semantics; Consumer(s). Aimles_ServerDiscover; Request; Request/Response; AIMLE client, VAL server. #### A.2 Aimles_ServerDiscover_Request operation API operation name: Aimles_ServerDiscover_Request Description: The consumer requests discovery of AIMLE server to an AIMLE server. Inputs: See clause 7.26.1.3.1. Outputs: See clause 7.26.1.3.1. See clause 7.26.1.2 for details of usage of this operation.

### 7.26.4	 Solution Evaluation

This solution addresses the open issue #5 in Key issue#6. This solution enables a AIMLE service consumer to discover an alternate AIMLE server, when there is a need for AIMLE service consumer to avail services from another AIMLE server due to various conditions such as change in AIMLE task requirement, mobility requirements etc. Such information is captured in the discovery request and the receiving AIMLE server can further fetch the AIMLE server that can meet the requirements.

* * * Next Change * * * *

### 10.1.1	Summary of new AIMLE services

This clause provides a summary of the new AIMLE and ML repository capabilities based on the Solutions.

Table 10.1.1-1 is listing solutions proposing new capabilities related to AIMLE:

Table 10.1.1-1: Solutions proposing new or enhanced AIMLE capabilities

| Solution | Proposed Capabilities | New or Enhancement |
| --- | --- | --- |
| #1 | ML model inference | New |
| #2 | AIMLE data management assistance | Enhancement |
| #3 | ML Model Maintenance | New |
| #4 | ML Model Evaluation | New |
| #6 | ML model Perf monitoring | Enhancement |
| #7 | AIMLE client discovery in cross-PLMN scenarios | Enhancement |
| #8 | Multi-client split operation pipeline | Enhancement / New |
| #9 | AIMLE server registration in hierarchical AIMLE deployment | New |
| #10 | Support of AIMLE services for roaming UEs | Enhancement |
| #12 | ML model split learning using relays | New and Enhancement |
| #13 | Edge ML model training support | New |
| #14 | AIMLE Client Discovery | Enhancement |
| #15 | Sample alignment for VFL | Enhancement |
| #16 | AIMLE service migration support in UE roaming scenarios | New |
| #17 | ML model collaborative inference | New |
| #18 | Support for federated AI services | New |
| #19 | Cross-PLMN/Domain AIMLE client discovery, selection, monitoring | New |
| #20 | ML Model Performance Evaluation with information collection from ML model consumer | New |
| #21 | Split operation node registration involving multiple AIMLE clients | Enhancement |
| #22 | Sample Alignment Enablement for VAL Servers in Vertical Federated Learning | New |
| #26 | AIMLE server discovery | New |

Table 10.1.1-2 is listing the solutions proposing new ML repository capabilities:

Table 10.1.1-2: Solutions proposing new or enhanced ML repository capabilities

| Solution | Proposed Capabilities | New or Enhanced |
| --- | --- | --- |
| #11 | FL member registration in multi-operator scenarios | Enhanced |
| #12 | ML model split learning using relays | Enhancement |
| #18 | Support for federated AI services | Enhancement |

* * * End of Changes * * * *