| 3GPP TR 23.700-26 V0.3.0 (2025-11) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on Application Enablement for Ambient IoT Services; Phase 2; (Release 20) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

Introduction	7

1	Scope	8

2	References	8

3	Definitions of terms, symbols and abbreviations	8

3.1	Terms	8

3.2	Symbols	9

3.3	Abbreviations	9

4	Architectural Assumptions and Requirements 	9

4.1	Architectural Assumption	9

4.2	Architectural Requirements	9

5	Architecture for Ambient IoT service enablement	9

6	Key issues	10

6.1	Key issue #1: Enhance Application enablement layer for Ambient IoT services	10

6.1.1	Description	10

6.1.2	Open Issues	10

6.2	Key issue #2: Exposing the value-added information of AIoT devices to the consumer	10

6.2.1	Description	10

6.2.2	Open Issues	11

6.3	Key issue #3: Key issue on provisioning and monitoring AIoT device presence	11

6.3.1	Description	11

6.3.2	Open Issues	12

6.4	Key issue #4: AIoT Data Management	12

6.4.1	Description	12

6.4.2	Open Issues	12

7	Solutions	13

7.0	Mapping of solutions to key issues	13

7.1	Solution #1: New architecture and functional model for AIoT services	13

7.1.1	Solution description	13

7.1.1.1	On-network Functional model for AIoT services	13

7.1.1.2	Functional entities description	14

7.1.1.3	Reference points description	15

7.1.2	Architecture Impacts	15

7.1.3	Corresponding APIs	15

7.1.4	Solution evaluation	15

7.2	Solution #2: Enhance application enablement layer for Ambient IoT services	16

7.2.1	Solution Architecture	16

7.2.2	On-network functional model for AIoTApp Service with new application enabler	16

7.2.2.1	Reference points	17

7.2.3	Off-network functional model for AIoTApp Service	17

7.2.4	Solution evaluation	17

7.3	Solution #3: AIoT application work flow using AIoT enabler layer	17

7.3.1	Solution description	17

7.3.2	Architecture Impacts	18

7.3.3	Corresponding APIs	18

7.3.4	Solution evaluation	18

7.4	Solution #4: Support of AIoT device presence information subscription	19

7.4.1	Solution description	19

7.4.1.1 	Introduction	19

7.4.1.2 	AIoT presence information subscription	19

7.4.1.3 	AIoT presence information notification	20

7.4.2	Architecture Impacts	21

7.4.3	Corresponding APIs	21

7.4.3.1	AIoT Presence Information Subscription request	21

7.4.3.2	AIoT Presence Information Subscription response	21

7.4.3.3	AIoT Presence Information notification	21

7.5	Solution#5: Services on Application AIoT Discovery and Monitoring	22

7.5.1	Solution Description	22

7.5.1.1	General	22

7.5.1.2	Procedure for Application AIoT Discovery	23

7.5.1.3	Procedure for Application AIoT Monitoring	24

7.5.1.4	Information Flows	25

7.5.2	Architecture impacts	25

7.5.3	Solution evaluation	25

7.6	Solution #6: Solution for AIoT service provision	25

7.6.1	Solution description	25

7.6.1.1	General	25

7.6.1.2	Procedures	25

7.6.1.3	Information flows	27

7.6.1.3.1 	AIoT service data provisioning request	27

7.6.1.3.2 	AIoT service data provisioning response	29

7.6.2	Architecture Impacts	29

7.6.3	Corresponding APIs	29

7.6.4	Solution evaluation	29

7.7	Solution #7: Application AIoT Task execution	30

7.7.1	Solution description	30

7.7.1.1	General	30

7.7.1.2	Procedures	30

7.7.1.3	Information flows	31

7.7.1.3.1	AIoT task initiate request	31

7.7.1.3.2	AIoT task initiate response	32

7.7.2	Architecture Impacts	32

7.7.3	Corresponding APIs	32

7.7.4	Solution evaluation	32

7.8	Solution #8: Exposing the value-added information of AIoT devices to the consumer	32

7.8.1	Solution Architecture	32

7.8.2	Solution description	32

7.8.3	Procedure	33

7.8.3.1	Exposure of value-added information via AIoT application enabler	33

7.8.3.2	Information flows	34

7.8.3.2.1	AIoT App Service request	34

7.8.3.2.2	AIoT App service response	35

7.8.3.2.3	AIoT App service notification	35

7.8.4	Solution evaluation	35

7.9	Solution #9: Support of periodical and event-triggered AIoT service operations	36

7.9.1	Solution description	36

7.9.1.1	General	36

7.9.1.2	Procedure of AIoT inventory service requests	36

7.9.2	Architecture Impacts	38

7.9.3	Corresponding APIs	38

7.9.4	Solution evaluation	38

7.10	Solution #10: Support of querying history data for AIoT devices	38

7.10.1	Solution description	38

7.10.1.1	General	38

7.10.1.2	Procedure of querying history data for AIoT devices	38

7.10.2	Architecture Impacts	39

7.10.3	Corresponding APIs	40

7.10.4	Solution evaluation	40

7.11	Solution #11: Provision and monitor AIoT device presence	40

7.11.1	Solution Architecture	40

7.11.2	Solution description	40

7.11.3	Procedure	41

7.11.3.1	Provision and monitor AIoT device presence	41

7.11.3.2	Information flows	42

7.11.3.2.1	AIoT App Service request	43

7.11.3.2.2	AIoT App service response	43

7.11.3.2.3	AIoT App service notification	43

7.11.4	Corresponding APIs	43

7.11.5	Solution evaluation	44

7.12	Solution #12: Support of monitoring requests for AIoT devices	44

7.12.1	Solution description	44

7.12.1.1	General	44

7.12.1.2	Procedure of Monitoring requests for AIoT devices	44

7.12.2	Architecture Impacts	46

7.12.3	Corresponding APIs	46

7.12.4	Solution evaluation	46

7.13	Solution #13: AIoT Data Processing management	46

7.13.1	Solution description	46

7.13.1.1	General	46

7.13.1.2	Procedures	46

7.13.1.3	Information flows	48

7.13.2	Architecture Impacts	48

7.13.3	Corresponding APIs	48

7.13.4	Solution evaluation	48

7.14	Solution #14: New functional model for AIoT services	48

7.14.1	Solution description	48

7.14.1.1	On-network Functional model for AIoT services	48

7.14.1.2	Functional entities description	49

7.14.2	Architecture Impacts	49

7.14.3	Corresponding APIs	49

7.14.4	Solution evaluation	49

7.15	 Solution #15: Solution for AIoT information provisioning	50

7.15.1	Solution description	50

7.15.1.1 	Introduction	50

7.15.1.2 	AIoT information provisioning	50

7.15.1.3 	AIoT information provisioning update	51

7.15.1.4 	AIoT information provisioning delete	51

7.15.2	Architecture Impacts	52

7.15.3	Corresponding APIs	52

7.15.3.1	AIoT Information Provisioning request	52

7.15.3.2	AIoT Information Provisioning response	53

7.15.3.3	AIoT Information Provisioning Update request	53

7.15.3.4	AIoT Information Provisioning Update response	53

7.15.3.5	AIoT Information Provisioning Delete request	54

7.15.3.6	AIoT Information Provisioning Delete response	54

7.15.4	Solution evaluation	54

8	Overall evaluation	55

8.1	General	55

8.2	Overall evaluation of key issue #1	55

8.3	Overall evaluation of key issue #2	55

8.4	Overall evaluation of key issue #3	55

8.5	Overall evaluation of key issue #4	55

9	Conclusions	56

9.1	General conclusions	56

9.2	Conclusions of key issue #x	56

Annex A (informative): Change history	57

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

Ambient IoT has the potential to benefit a large number of vertical industries, so it is important to study how to support the Ambient IoT services in the application enabled layer, aligned with SA1 requirements in 3GPP TS 22.369 [2] and SA2 architecture defined in 3GPP TS 23.369 [3] in Rel-19.

This technical report identifies the key issues, corresponding application architecture requirements and related solutions with recommendations for the normative work.

# 1	Scope

The present document is a technical report which identifies the architecture requirements and related functionalities to support the Ambient IoT services in the application enabled layer.

The aspects of the study mainly include enriching the processing of the collected Ambient IoT devices data exposed from other working group (e.g. SA2), providing the value-added Ambient IoT device information to the consumers and monitoring the Ambient IoT devices to minimize the service interruption (e.g. inform the application server the AIoT device has been disabled in time).

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.369: "Service requirements for ambient power-enabled IoT, Stage 1".

[3]	3GPP TS 23.369: "Architecture support for Ambient power-enabled Internet of Things, Stage 2".

[4]	3GPP TR 23.700-30: "Study on Architecture support of Ambient power-enabled Internet of Things (AIoT); Phase 2”.

[5]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[6]	3GPP TS 23.542: "Application layer support for Personal IoT Network".

[7]	3GPP TS 23.436: "Procedures for Application Data Analytics Enablement Service".

[8]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

example: text used to clarify abstract rules by applying them literally.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

<ABBREVIATION>	<Expansion>

AIoT	Ambient IoT

# 4	Architectural Assumptions and Requirements

## 4.1	Architectural Assumption

[AA-4.1-a] The architecture for the AIoT services in the application enablement layer should align with the  5GS architecture for  AIoT service which is specified in the clause 4 of 3GPP TS 23.369 [3].

[AA-4.1-b] The architecture for the AIoT services in the application enablement layer should take the API framework as specified in 3GPP TS 23.222 [8] as a baseline.

## 4.2	Architectural Requirements

[AR-4.2-a] The AIoT enabler layer shall support the provisioning of AIoT related application information by AIoT service consumers.

[AR-4.2-b] The AIoT enabler layer shall support AIoT service event report from the AIoT enabler layer to the AIoT application layer.

[AR-4.2-c] The AIoT enabler layer shall support AIoT event-triggered  and/or periodic event reporting.

[AR-4.2-d] The AIoT enabler layer shall support authentication of the AIoT application layer consumer.

[AR-4.2-e] The AIoT enabler layer shall support authorization of access and operation request from AIoT application layer consumer.

[AR-4.2-f] The AIoT enabler layer shall support exposing the value-added AIoT information to the application layer consumer.

[AR-4.2-g] The AIoT enabler layer shall support history data query request for AIoT devices from the application layer consumer.

[AR-4.2-h] The AIoT enabler layer shall support monitoring of AIoT devices.

[AR-4.2-i] The AIoT enabler layer shall support comsumption of the inventory and command services as described in 3GPP TS 23.369 [3].

# 5	Architecture for Ambient IoT service enablement

This clause will document any new architecture or the architectural enhancement for the existing application enablers.

# 6	Key issues

## 6.1	Key issue #1: Enhance Application enablement layer for Ambient IoT services

### 6.1.1	Description

Ambient power-enabled IoT devices or AIoT devices are of low complexity, small size, lower power consumption, having limited memory capacity, less compute power, and lacking advanced capabilities, which make AIoT devices different from previously defined 3GPP IoT devices. Therefore, AIoT devices cannot accommodate and run directly the VAL functionality component (i.e., VAL client) to support AIoT services, nor can they support the application enablement layer client entity (e.g., SEAL client).

As shown in the TS 23.434[5], clauses 6.1 & 6.2, for the generic on-network functional model for SEAL, while there is neither VAL client nor SEAL client providing the service enabler layer support functions from the client side, there should necessarily exist the VAL server (i.e., AIoT application server or the 3rd-party AS/AF) and the SEAL server. The SEAL server communicates with the VAL server. As explained above, due to the lack of the corresponding client-side functionality components (e.g., VAL client, SEAL-client), a SEAL server may only communicate with the underlying 3GPP network systems using the respective 3GPP interfaces specified by the 3GPP network system, i.e., either the N33 via NEF (for an untrusted AF), or the new interface off the SA2-defined AIOT NF (i.e., AIOTF) directly providing the AIoT service operations to a trusted AF (please reference the SA2 TS 23.369[3]).

There are already some IoT services/devices that have been studied in FS_ACE_IOT and PINAPP which the related normative work has been specified in 3GPP TS 23.434[5] and 3GPP TS 23.542[6]. Especially for PINAPP, a new application enabler (i.e. PIN enabler) has been developed to support the PIN service and personal IoT devices. As the AIoT devices are the new type of IoT devices, whether a new architecture/functional model or the existing IoT related application enablers could support the AIoT devices should be studied further.

### 6.1.2	Open Issues

This key issue is proposed to study the enhancement of the application enablement architecture for AIoT service in application enabler layer:

-	Study whether and how to enhance the application enablement architecture /functional model or new application enabler to support the Ambient IoT services (i.e., AIoT inventory and command services).

## 6.2	Key issue #2: Exposing the value-added information of AIoT devices to the consumer

### 6.2.1	Description

At present, 5G core network in Rel-19 could provide the requested AIoT Devices ID(s) to the AF as specified in 3GPP TS 23.369[3]. However, these AIoT Devices ID(s) are huge and may not fulfill the service requirements from the 3rd parties (e.g. AS/AF). The following are some examples to show the gaps between the service requirements and the current exposure capabilities.

For example, a 3rd-party retail App-server (i.e., VAL server) may issue periodic inventory service requests to check the volume of some specific types of AIoT devices in a certain area. When the number of the available AIoT devices of a type drops below a certain threshold, the retail store might run into service interruption of not having sufficient goods for customers. In this scenario, the corresponding retailer is better off replenishing the stock in advance by adding more devices of the type. If the retail store asks for periodic inventory service, it’s expected to be informed when and which type of devices will be increased or decreased. However, based on current exposure results from 5G core network as specified in 3GPP TS 23.369[3], only the AIoT devices ID will be provided for the inventory request, how to identify the AIoT devices in service granularity and how to indicate when one type of device will reach the pre-defined threshold to perform the increase or decrease actions should be studied further in application enablement layer.

In another scenario, if AF sends continuously a batch of 'inventory service’ requests covering different areas (with different area-IDs), though these areas belonging to a same (large) targeted area whose identity could be a geographical civic address. Then, the AIoT application enabler may aggregate these requests and send only one combined 'inventory service’ request to the 5G core network instead of sending several 'inventory service’ requests continuously.

Besides, some application servers (e.g. a logistics company) may need to regularly track the location of the AIoT devices (e.g. the goods in the truck) and identify the trajectory or the direction for the AIoT devices depending on their locations, and then send warnings to the AIoT devices in case they deviate the right trajectory/direction. However, based on current exposure results from 5G core network, only the device ID with RAN reader ID may be exposed, but how to map the RAN ID to the location information (e.g. civic address) and track the devices’ location continually as requested should be studied further in application enablement layer.

In a word, it would be of great advantage to AIoT services consumers, if they received enriched results from the enabler layer. Hence, how to provide value-added services to the consumers based on the AIoT devices information retrieved from the core network should be studied at the application enablement layer.

### 6.2.2	Open Issues

Based on the above analysis, the following open issues need to be studied in the application enabled layer:

1.	How to enrich the processing (e.g. filtering, categorization, consolidation) of AIoT service requests and the collected Ambient IoT device data;

2.	How to utilize the exposure from other working group (e.g. SA2) and provide the value-added AIoT devices information to the consumer (e.g. VAL server) based on open issue #1.

## 6.3	Key issue #3: Key issue on provisioning and monitoring AIoT device presence

### 6.3.1	Description

The 5G core network (5GC) defines the “inventory” and the “command” AIoT service operations in 3GPP TS 23.369 [3]. While architecture and framework for supporting AIoT services in the 5GC have been defined, there exist unique scenarios for which the application enablement layer can enhance the AIoT service operations.

For example, a 3rd-party retail application server (i.e., VAL server) may issue periodic inventory service requests to check the volume of some specific types of AIoT devices in a certain area. When the number of available AIoT devices of a type drops below a certain threshold, product shortage may happen. In this scenario, a retailer may desire to replenish product stock in advance to avoid shortage. An AIoT application enabler may provide a service enabling threshold-driven inventory checks for monitoring the changing volume of AIoT devices. An AIoT application enabler can provide unform and efficient monitoring for different 3rd party retail application servers (i.e., VAL servers).

In another example scenario, a 3rd-party application server (e.g., VAL server) may collect AIoT device information via inventory service command for an AIoT device originally located in a first target area. This AIoT device may be moved to a second target area. The 3rd-party application server may issue inventory service command to the AIoT device for the first target area, thus generating a failure to retrieve the AIoT device information. For this scenario, an AIoT application enabler may provide a remediation mechanism by searching within alternate targeted area(s) so as to pinpoint the up-to-date information of the AIoT device. The 5GC does not provide such remediation mechanisms.

Further, 3GPP TS 22.369 [2] mentions that “Subject to user consent, operator’s policy and 3rd party request, the 5G system shall provide information about an Ambient IoT device or a group of Ambient IoT devices (e.g. position) to the trusted 3rd party via the 5G network.”. The 5GC support for AIoT service operations does not meet that requirement.

KI #3 focuses on providing enhanced means and information realted to monitoring the presence of provisioned AIoT devices by the application enablement layer. Monitoring of AIoT device presence may need to be performed according to different schedules and according to different presence monitoring criteria, for example for a group of AIoT devies associated with a  product or with an object.

The exposure of AIoT devices by the core network (e.g., via the inventory or command service operations) offers limited reporting options; thus, it may be beneficial for an AIoT information consumer (e.g., VAL server) to have access to enhanced capabilities offered by the application enablement layer for configuring and monitoring the presence of AIoT devices.

### 6.3.2	Open Issues

This key issue will study the following aspects

1.	How to provision information about AIoT devices requiring monitoring in the application enablement layer.

2.	How concurrent AIoT device monitoring operations can be managed in the application enablement layer considering existing CN AIoT exposure capabilities.

3.	Whether and how enhanced AIoT device monitoring information, such as status, location or trajectory of Ambient IoT devices, can be provided by the application enablement layer.

## 6.4	Key issue #4: AIoT Data Management

### 6.4.1	Description

AIoT inventory service provides the inventory resultdata only including device ID. However, the upper-layer applications require high-level, semantic-rich information such as "which application layer object, at what time, in which logical location, experienced what meaningful application layer event, and associated with which application layer attributes". For example, typical AIoT inventory result data is about "Device ID list: ID1, ID2, ID3…… ", while application requirement is about "the Product SKU123 has just been received through the Gate 1 at Warehouse 1".

So, the AIoT data and application requement has significant gaps in semantics, granularity and application layer context:

1)	Physical location vs logical location: Currently the external target area for inventory is normally e.g., a civic address or shapes, but the application needs a logically significant application layer position, such as warehouse zone A, workstation 3, entry checkpoint1.

2)	Device id vs application layer object: AIoT inventory service provided by SA2 only identifies the tag(EPC) attached to the item, but the application needs to get the information of the item(product, asset, pallet).

3)	Network inventory event vs application layer event: The 5GS AIoT inventory services provide inventory result only with Device ID list, but the application usually needs to aggregate device ID and other information like target service area, timestamp, and even aggreate the inventory result from multiple times inventory to generate a meaningful event, for example:

-	application layer event: A packet containing 5 items has been shipped. (Confirm that all five product tags are read at a specific location and time and are associated with the same shipping order.)

-	Counting: There are 25 items on the shelf. (Summarize the number of unique tags which is associated to some item, read within a specific target area.)

-	State: Pallet a is moved to production line. (Check the tag’s physical location is accurately changed from target area A to target area B)

For the downlink datastream, the 5G core network provides inventory service API. It requires the upper layer to specify the network resources (service area) for executing inventory instructions. However, the application layer is only interested in the applicatoin layer items and application layer area information. There is a semantic gap between the application layer area information and the physical area information on the network side. Direct integration and use the network reported data require a significant amount of adaptation development work for the AIoT application, which is not conducive to promoting the 3GPP AIoT marketing adoptation.

### 6.4.2	Open Issues

Open issues:

1.	How to align the inventory result data acording to the desired application semantics and formats.

2.	How to configure, manage and expose the conversion of inventory result data according to the vertical application requirements, such as ERP(enterprise resource planning system), MES(Management Execution System) or WMS(warehousing management system).

# 7	Solutions

## 7.0	Mapping of solutions to key issues

Table 6.0-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 |
| --- | --- | --- | --- | --- |
| Sol #1 | x |  |  |  |
| Sol #2 | x |  |  |  |
| Sol #3 | x | x | x | x |
| Sol #4 |  |  | x |  |
| Sol #5 |  | x | x | x |
| Sol #6 |  | x |  | x |
| Sol #7 |  | x | x | x |
| Sol #8 |  | x |  |  |
| Sol #9 |  | x |  |  |
| Sol #10 |  | x |  |  |
| Sol #11 |  |  | x |  |
| Sol #12 |  | x | x |  |
| Sol #13 |  | x |  | x |
| Sol #14 | x |  |  |  |
| Sol #15 |  |  | x |  |

## 7.1	Solution #1: New architecture and functional model for AIoT services

### 7.1.1	Solution description

This solution addresses the KI#1. The following procedures introduce the on-network functional model for AIoT services, also mention the functional entities and reference points in the new architecture. This solution aligns with the 5GS architecture for AIoT services which is specified in the clause 4.2 of 3GPP TS 23.369 [3].

#### 7.1.1.1	On-network Functional model for AIoT services

Figure 7.1.1.1-1 illustrates the generic on-network functional model for AIoT service.

Figure 7.1.1.1-1: On-network functional model for AIoT services

The AIoT management server, acting as an AF, could obtain AIoT data information from the 3GPP core network via NEF (over N33 reference point) which is defined in clause 4.5.4 of 3GPP TS 23.369 [3]. If the AIoT management server is a trusted AF, it may interact with AIOTF directly for AIoT related service as defined in clause 4.5.5 of 3GPP TS 23.369 [3]. The AIoT management server may communicate with the SEAL server(s) over AM-X reference point and utilize the related SEAL services as specified in 3GPP TS 23.434 [5].

NOTE 1:	The Network interface is either the N33 via NEF (for an untrusted AF), or the interface off the AIOT NF (i.e., AIOTF) directly providing the AIoT service operations to a trusted AF as defined in the clause 4.5.3 of 3GPP TS 23.369 [3].

The VAL server(s) could communicate with the AIoT management server over AM-S reference point for requesting the AIoT services and receiving the AIoT data information.

The AIoT management client, acting as a consumer, could also request the AIoT services to the AIoT management server over AM-UU reference point, and provide the support for AIoT related functions to the VAL client(s) over AM-C reference point.

NOTE 2:	In this release, the AIoT management client is not an UE reader, but only acts as an AIoT service consumer.

The VAL client communicates with the VAL server over the VAL-UU reference point which is outside the scope of this document.

#### 7.1.1.2	Functional entities description

AMS (AIoT Management Server):

The AMS performs the following functionalities to support AIoT services:

-	Interact with 3GPP core network via NEF to obtain the AIoT device data, acting as an untrusted AF;

-	Interact with AIOTF in 3GPP core network directly to obtain the AIoT device data, acting as a trusted AF;

-	Enrich the processing of AIoT service requests and the collected Ambient IoT related data;

-	Provide the value-added information of AIoT devices to the AIoT service consumers (e.g. VAL servers, VAL Clients);

-	Provision and monitoring the AIoT devices;

-	Expose the AIoT device monitoring information to the AIoT service consumers (e.g.  VAL servers, VAL Clients);

-	Acting as CAPIF's API exposing function as specified in 3GPP TS 23.222 [8].

NOTE:	The functionalities for the AMS may update according to the approved solution and final conclusions.

AMC (AIoT Management Client):

The AMC performs the following functionalities to support AIoT services:

-	Acting as the application client for consuming the AIoT services;

NOTE:	The specific functionalities for the AMC may update according to the approved solutions and final conclusions.

#### 7.1.1.3	Reference points description

AM-S

The interactions related to AIoT functions between the VAL server(s) and the AIoT management servers are supported by AM-S reference point.

AM-S reference point is used by the VAL server to request and receive the AIoT device related information from AIoT management server.

AM-S reference point is an instance of CAPIF-2 reference point as specified in 3GPP TS 23.222 [8].

AM-X

The interactions related to AIoT functions between the AIoT management servers and the SEAL servers (i.e. ADAES) are supported by AM-X reference point.

AM-X reference point is used by the AIoT management server to request and receive the AIoT device related information.

AM-C:

The interactions related to AIoT functions between the VAL client(s) and the AIoT management clients within a VAL UE are supported by AM-C reference point.

AM-UU:

The interactions related to AIoT functions between the AIoT management clients and the AIoT management servers are supported by AM-UU reference point.

### 7.1.2	Architecture Impacts

This solution proposes a new architecture and functional mode for AIoT services.

### 7.1.3	Corresponding APIs

This clause provides the corresponding APIs for supporting the solution.

### 7.1.4	Solution evaluation

This solution addresses the KI#1 to provide a new architecture and functional model in the application enablement layer to support the AIoT services. Specifically, this solution introduces the on-network functional model, the relevant functional entities and the reference points included in the new architecture for the AIoT services.

In this solution, the AIoT management server could obtain AIoT data information from the 3GPP core network acting as a trusted or untrusted AF. And it may also communicate with the SEAL serve (i.e. ADAES) and utilize the related SEAL services as specified in 3GPP TS 23.434 [5].

To align with the 5GS architecture for AIoT services specified in clause 4.2 of 3GPP TS 23.369 [3], the AIoT management client in the new architecture for the application enablement layer only acts as a service consumer which could request the AIoT services from the AIoT management server over AM-UU reference point, but it will not act as a UE reader to obtain and report the AIoT device information.

## 7.2	Solution #2: Enhance application enablement layer for Ambient IoT services

### 7.2.1	Solution Architecture

This solution proposes to define an independent architecture, based on which a new application enabler AIoTApp is defined and acts as a SEAL layer function entity.

### 7.2.2	On-network functional model for AIoTApp Service with new application enabler

Ambient power-enabled IoT devices or AIoT devices are of low complexity, small size, lower power consumption, having limited memory capacity, less compute power, and lacking advanced capabilities, which results in AIoT devices not being able to accommodate and run directly the VAL functionality component (i.e., VAL client) to support AIoT services, nor can they support the application enablement layer client entity (e.g., SEAL client).

For AmbientIoT_App service, upon viewing the generic on-network functional model for SEAL as in the 3GPP TS 23.434[5] clauses 6.2, while there is neither VAL client nor SEAL client providing the service enabler layer support functions from the client side, there should necessarily exist the VAL server, i.e., AIoT application server or the 3rd-party AS/AF, and the SEAL layer enabler (or server). The SEAL server communicates with the VAL server via the SEAL-S reference point. However, due to the lack of the corresponding client-side service enabler layer functional entities (e.g., VAL client, SEAL-client), a SEAL server may only communicate with the underlying 3GPP network systems using the respective 3GPP interfaces specified by the 3GPP network system, i.e., either the N33 via NEF (for an untrusted AF), or the interface off the SA2-defined AIOT NF (i.e., AIOTF) directly providing the AIoT service operations to a trusted AF (as in the 3GPP TS 23.369[3] clause 4.5.3).

Figure 7.2.2-1: On-network functional model for AIoTApp service

As shown in the Fig. 7.2.2-1, for the unique (one-side) on-network functional model, the application enablement architecture layer can only be enhanced at the server side (i.e., the right-hand side of the picture) of the application enabler for Ambient IoT service. A new application enabler, i.e., AIoTApp, is introduced to act as a SEAL layer function entity. The AIoTApp enabler function provides services to VAL server over the AIoTApp-S reference point. The detailed 5GC architecture to support the AIoT service is shown in the TS 23.369 [3].

NOTE:	The network interface represents either the N33 interface via NEF (for an untrusted AF), or the interface off the AIOT NF (i.e., AIOTF) directly providing the AIoT service operations to a trusted AF as defined in the 3GPP TS 23.369 [3] clause 4.5.3.

#### 7.2.2.1	Reference points

AIoTApp-S

The functional interactions of the AIoT services between the AIoT application enabler and the VAL server(s) are supported by the AIoTApp-S reference point.

The AIoTApp-S reference point is used by the VAL server to request and receive the AIoT device related information from the AIoT App enabler.

### 7.2.3	Off-network functional model for AIoTApp Service

The off-network functional model for AmbientIoT application enablement architecture is not supported in this release.

### 7.2.4	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

## 7.3	Solution #3: AIoT application work flow using AIoT enabler layer

### 7.3.1	Solution description

This solution addresses key issue#1, 2, 3 and 4 to introduce a AIoT application work flow using AIoT enabler layer based on the AIoT enabler functional model defined in Sol#2.

The AIoT network provides fundamental tag inventory service with typical output AIoT data is about "Device ID list: ID1, ID2, ID3…… ". However, the upper-layer applications require high-level, semantic-rich information such as "which business object, at what time, in which logical location, experienced what meaningful business event, and associated with which business attributes". To ensure better adaptation and integration between the AIoT network and the application layer, it is necessary to build AIoT-enabled service capabilities to bridge the gap between AIoT inventory data obtained from the 3GPP network and AIoT application layer data, as well as the AIoT task and the 3GPP network service APIs.

In addition to the function model defined in Sol#2, this solution assmuption the enabler layer has the following

The functional model introduce a new functionality AIoT enabler server. It supports the following functionalities:

- AIoT service provisioning: it supports the AIoT application layer to provision its AIoT application tag data, application object data, AIoT application service region, and AIoT service scenario associated with each region, interested AIoT service events. Via this service provisioning, the AIoT enabler server allows each AIoT service provider to customize their own AIoT task, region, event.

- AIoT task control: It supports the dynamic conversion between the AIoT application layer data and the AIoT service API or data reported from 3GPP system, including:

-	translate the AIoT task (i.e., inventory check of all goods in warehouse A) to the network AIoT services (i.e., Inventory and command service). A AIoT task is AIoT service oriented and normally contains several key information, e.g., service region(s), service objects, scenario.

-	collects the network reported AIoT result data, derives the corresponding AIoT service events and AIoT service layer data, and notifies the AIoT application server.

The end-to-end data processing workflow is illustrated in the following figure 7.3.1-1:

Figure 7.3.1-1: Overview of AIoT enabler workflow

1.	Perform the initial and basic AIoT service provisioning including the batch registration of AIoT tags, AIoT regions, AIoT application objects, and AIoT application service regions, the association between AIoT tags and business objects, the association between AIoT application service region and target service area, and also the AIoT service scenario as well as the interested AIoT service events. After the basic configuration phase, the application layer can invoke AIoT services using its configued task, scenario, region.

2.	AIoT Inventory Task Generation and Distribution. The AIoT enabler server receives the request with AIoT task  from the application layer. The AIoT enabler server transforms these AIoT tasks into AIoT inventory/command towards the 3GPP system. For example, the application layer AIoT task "Inventory check of all goods in warehouse A", whose application objects are goods, application service region is warehouse A, service scenario is stock-taking, is translated into the AIoT inventory API invocation " inventory command with service area 1 and AIoT device ID filter information for goods."

3.	The 3GPP network begins inventorying and reports the results to the AIoT Enabled Server.

4.	The AIoT-enabler server receives the AIoT inventory results and, based on the AIoT service configuration informations set in step 1 and scenario-processing models specified in step 2, applies the scenario-based processing on the AIoT inventory results to form AIoT service events.

For example, the AIoT inventory result "Tag x, y and zwere seen at AIoT service area a (specified in the inventory service requests)" is translated into "Goods X, Y and Z  have completed inventory at warehouse A." Depend on the different requirements at the application layer, the AIoT enabler server can directly report the AIoT service event to the AIoT application server or further process it in conjunction with AIoT service logic to form another AIoT service event, such as "The receipt task at Checkpoint 1 has been successfully completed," and then report it.

### 7.3.2	Architecture Impacts

This solution has no architecture impact.

### 7.3.3	Corresponding APIs

No service API is introduced.

### 7.3.4	Solution evaluation

## 7.4	Solution #4: Support of AIoT device presence information subscription

### 7.4.1	Solution description

#### 7.4.1.1 	Introduction

This solution addresses Key Issue #3 on provisioning information in the application enablement layer and enables presence monitoring of AIoT devices.

This solution proposes that an AIoT Enablement Server (AIOTES) offers a service to authorized consumers (e.g., VAL Servers, VAL clients) for subscribing to AIoT device presence information.

This solution proposes to:

- define a new procedure and information flows to enable provisioning of AIoT presence information subscription in the AIOTES for presence reporting;

- define a new procedure and information flow for detecting events and reporting presence of AIoT devices.

#### 7.4.1.2 	AIoT presence information subscription

Figure 7.4.1.2-1 describes a procedure for AIoT presence information subscription.

Figure 7.4.1.2-1: AIoT Presence Information Subscription

1.	The requestor (e.g., VAL server or AIOTEC) sends an AIoT presence information subscription request to the AIOTES. The request includes the requestor identifier, information about AIoT device(s) [device identifier, area information] and information about the presence reporting configuration [notification endpoint, frequency, schedule, location indication, change only indication].

2.	Upon receiving the request from the requestor, the AIOTES validates if the requestor is authorized for the request. If the requestor is authorized, the AIOTES allocates a presence reporting subscription identifier, and stores the associated AIoT device information and presence reporting configuration.

The AIOTES initiates the presence monitoring by sending the Nnef_AIoT_Inventory Request as described in 3GPP TS 23.369 [3]. Based on the AIoT devices information and reporting configuration, the AIOTES includes the AIOTES identifier, the AIOTES notification endpoint, the AIoT device identifiers and may include Area Information, Location information indication and number of AIoT devices as input parameters for Nnef_AIoT_Inventory Request. If the reporting configuration includes schedule, the AIOTES sends the Nnef_AIoT_Inventory Request according to the time schedule (e.g., delays the request based on reporting time schedule).

The AIOTES configures the reporting based on the presence reporting configuration. If the reporting configuration includes frequency, the AIOTES schedules periodic reporting of the monitored presence to the notification endpoint according to frequency. If the reporting configuration includes change only indication, the AIOTES sends the notification only if a change in the presence information has happened since the previous notification.

If the AIOTES sends the Nnef_AIoT_Inventory Request and the request is successful, the AIOTES stores the returned transaction identifier.

3.	The AIOTES sends an AIoT Information Provisioning response to the requestor. If the request was authorized, the response includes an indication of success for the request, the presence reporting session identifier and an indication that the presence monitoring has started. Otherwise, the response includes an indication of failure and may include a failure cause.

#### 7.4.1.3 	AIoT presence information notification

Figure 7.4.1.3-1 describes a procedure for AIoT presence information notification.

Figure 7.4.1.3-1. AIoT Presence Information Notification

1.	The AIOTES detects a presence reporting event. The presence reporting event may be one of receiving a Nnef_AIoT_Notify message or detecting a reporting event based on the reporting configuration.

If the AIOTES receives a Nnef_AIoT_Notify message as described in 3GPP TS 23.369 [3], the AIOTES identifies retrieves the subscription information using the AF transaction identifier. The AIOTES stores or updates the received AIoT device presence information locally and prepares the information to be included in the AIoT Presence Information notification. If the change only indication is present in the reporting configuration, the AIOTES determines, for each device, if the received presence information has changed and the AIoT Presence Information notification is sent only if the AIOTES has determined a change. If the Nnef_AIoT_Notify message includes a last report indication, the AIOTES may send a Nnef_AIoT_Inventory Request as described in step 2 of clause 7.4.1.2 to continue receiving inventory notifications.

If the AIOTES detects a reporting event which is the periodic reporting frequency timer expiration, the AIOTES prepares the information to be included in the AIoT Presence Information notification based on the AIoT device presence information locally available.

If the AIOTES detects a reporting event which is a scheduled reporting time event, if the reporting event is to start reporting, the AIOTES starts inventory command by sending the Nnef_AIoT_Inventory Request as described in step 2 of clause 7.x.1.2. If the detected reporting event is to stop reporting, the AIOTES sends a last notification to the requestor to indicate that reporting is stopped and stops sending notifications to the requestor.

2.	The AIOTES sends an AIoT presence information notification to the subscriber (e.g., VAL server or AIOTEC) configured in the presence reporting configuration.

The notification includes a presence report including the AIoT device identifiers, the presence status of the AIoT devices and may include the location information (e.g., as indicated in Nnef_AIoT_Notify), a duration since the last presence status change. The presence report includes a presence reporting status (e.g., started or stopped) if the detected event scheduled reporting time event.

### 7.4.2	Architecture Impacts

This solution proposes that a new functional entity, as an AIoT Enablement Server (AIOTES), is supported in the application enablement layer as described in Solution #1 and Solution #2.

### 7.4.3	Corresponding APIs

#### 7.4.3.1	AIoT Presence Information Subscription request

Table 7.4.3.1-1 describes the AIoT Presence Information Subscription request.

Table 7.4.3.1-1: AIoT Presence Information Subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| AIoT device information | M | The information about AIoT devices for the subscription. |
| > AIoT device identifiers | M | The list of device identifiers for which the presence is monitored. |
| > Area information | O | The area information where the presence is monitored. |
| Reporting configuration | M | The configuration for presence reporting. |
| > Notification endpoint | M | The endpoint information (e.g., URL, URI, IP address) where presence reports should be sent. |
| > Frequency | O | The frequency at which the presence is reported to the requestor. |
| > Schedule | O | The time schedule (e.g., duration, time of day, period) at which presence reporting is required. |
| > Location information indication | O | The indication that location information is requested in the presence report. |
| > Change only indication | O | The indication that reporting only includes device for which the presence status has changed. |

#### 7.4.3.2	AIoT Presence Information Subscription response

Table 7.4.3.2-1 describes the AIoT Presence Information Subscription response.

Table 7.4.3.2-1: AIoT Presence Information Subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | The indication that the request was successful. |
| > Subscription identifier | M | The identifier for the subscription. |
| Failure response | O (NOTE) | The indication that the request failed. |
| > Cause | O | The failure cause |
| NOTE: Only one of these information elements shall be provided. |  |  |

#### 7.4.3.3	AIoT Presence Information notification

Table 7.4.3.3-1 describes the AIoT Presence Information notification.

Table 7.4.3.3-1: AIoT Presence Information notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription identifier | M | The identifier for the subscription. |
| Presence report | M | The presence report information. |
| > AIoT device identifiers | M | The list of device identifiers for which presence is reported |
| > Presence status | M | The status of presence which is one of present or absent or unknown. |
| > Location information | O | The location information of the AIoT device. |
| > Presence status duration | O | The duration since the last presence status change. |
| > Presence reporting status | O | The status of presence reporting which may be an indication that presence reporting has started or stopped. |

### 7.4.4	Solution evaluation

This solution addresses Key Issue #3 on provisioning information in the application enablement layer and enables presence monitoring of AIoT devices. It provides procedures for enabling a consumer (e.g., VAL server or AIOTEC) of monitoring AIoT device presence via a subscription mechanism.

The solution uses the capabilities of inventory operation as described in 3GPP TS 23.369 [3] for obtaining the AIoT device presence and allows monitoring AIoT devices via their identifiers and/or area information. The monitoring reporting allows fine grained control of the monitoring via a reporting configuration.

The solution is compatible with architectures described in Solution #1 and Solution #2.

## 7.5	Solution#5: Services on Application AIoT Discovery and Monitoring

### 7.5.1	Solution Description

#### 7.5.1.1	General

The existing services on AIoT inventory and AIoT command cannot satisfy the application requirements on the aspects listed in the above key issues. For example, based on current exposure results from 5G core network as specified in 3GPP TS 23.369 [3], only the AIoT devices ID will be provided for the inventory request, and cannot identify the AIoT devices in service granularity and indicate when one type of device will reach the pre-defined threshold to perform the increase or decrease.

The following clauses specify AIoT discovery and monitoring services for supporting AIoT applications.

#### 7.5.1.2	Procedure for Application AIoT Discovery

Figure 7.5.1.2-1: Application AIoT Discovery

1.	A consumer (e.g., VAL server) sends application AIoT discovery request to AIoT App server. The request information includes VAL service ID, Area of interest (e.g., physical location, logical location (e.g., warehouse, factory, supper market)), Application AIoT device information (e.g., list of application layer objects (e.g., goods in a warehouse) which correspond to a list of application AIoT devices, type(s) of the objects), Number of AIoT devices, Requirements on categorization (e.g., location-based, object type-based), Requirements on discovery (e.g., count number of objects in specific location, count number of specific objects), and Time interval.

2.	The AIoT App server authenticates and authorizes the request.

3.	If authorized, the AIoT App server performs categorization of the AIoT devices in the list provided in step 1 according to the requirements on categorization and determines the AIoT inventory request to 5GC NF (i.e. NEF/AIOTF) for each of the categories. Based on the list of AIoT devices in a category, the AIoT App determines the Filtering Information to be used in the inventory request.

4.	The AIoT App server sends the AIoT inventory requests according to the determination in step 3 to the 5GC NF (NEF or AIOTF), for obtaining the AIoT device IDs and location of each AIoT device for each of the category.

5.	The AIoT App server maps the received AIoT device IDs into objects for each category and processes the received information according to the requirements on discovery, e.g., count the number of objects in specific location, count the number of specific object type.

6.	The AIoT App server sends application AIoT discovery response to the consumer, with the required discovery information.

#### 7.5.1.3	Procedure for Application AIoT Monitoring

Figure 7.5.1.3-1: Application AIoT Monitoring

- A consumer (e.g., VAL server) sends application AIoT monitoring request to AIoT App server. The request information include VAL service ID, Area of interest (e.g., physical location, logical location (e.g., warehouse, factory, supper market)), Application AIoT device information (e.g., list of application layer objects (e.g., goods in a warehouse) which correspond to a list of application AIoT devices), Number of AIoT devices, Requirements on categorization (e.g., location-based), Requirements on monitoring (e.g., monitor the number of objects in specific location (e.g., number of goods in a warehouse), the expired time of the objects (e.g., the expired time of the goods in a supermarket)), Time interval for monitored data processing, and Reporting requirements.

Reporting requirements could be Periodic reporting (frequency of the reporting), or Event-triggered reporting (report when a threshold is exceeded or change occur).

For threshold-based report, Threshold(s) (e.g., number(s) of objects in specific locations, time threshold(s) for specific objects) is included in the request.

For flexible report (i.e., change occur and report), the event could be, for example, number of specific type object change compared with previous check (e.g., increase or decrease in given location (e.g., Goods with the given category are in or out the given location)) or status of object change (e.g., expired time is approached/reached (e.g., Goods are close to be expired with given time range per goods category)). Frequency for check the changes of object (e.g., number, status (may align with the time interval for monitored data processing)) are included in the request.

2.	The AIoT App server authenticates and authorizes the request.

3.	The AIoT App server responds to the application AIoT monitoring request, with subscription ID.

4.	If authorized, the AIoT App server performs categorization of the AIoT devices in the list provided in step 1 according to the requirements on categorization, determines the operations according to the requirements on monitoring in step 1 (e.g., AIoT inventory for monitoring the number changes of objects, AIoT command-read for monitoring the status of object), and schedules periodic AIoT inventory and/or command-read.

5.	Based on the determinations in step 4, the AIoT App server performs corresponding operations for monitoring.

5a.	The AIoT App server may send AIoT inventory request(s) to 5GC NF (NEF/AIoTF) to obtain the AIoT device IDs in given location.

5b.	The AIoT App server may send AIoT command request(s) to 5GC NF (NEF/AIoTF) to read the AIoT data and location information.

6.	Based on the received information in step 5 and requirements in step 1, the AIoT App server processes the data, and determines whether need to send notification to the consumer after each round of monitored data processing, e.g., report periodically, report the changes of object number, report the changes of object status (e.g., expired time approached/reached).

7.	The AIoT App server sends application AIoT monitoring notification to the consumer, with the required monitoring information.

#### 7.5.1.4	Information Flows

Editor’s Note:	The information flows of the solution is FFS.

### 7.5.2	Architecture impacts

Editor’s Note:	The architecture impacts of the solution is FFS.

### 7.5.3	Solution evaluation

Editor’s Note:	The evaluation of the solution is FFS.

## 7.6	Solution #6: Solution for AIoT service provision

### 7.6.1	Solution description

#### 7.6.1.1	General

This solution address key issue#2, and #4 to introduce a solution for AIoT service provision.

AIoT network service can provide the information of what (AIoT device IDs) and where (AF ID and the external Target Area information), as shown in the TS 23.369[7]. However, these network exposured data can’t be directly consumed by the 3rd party appication layer and needs to be correlated with application information. The core idea is to establish a mapping relationship for network layer information and the application data.

#### 7.6.1.2	Procedures

The figure 7.6.1.2-1 show the high-level procedures of AIoT service provisioning.

Figure 7.6.1.2-1: Service data provisioning

1.	The AIoT application layer consumer invokes the service provisioning service of AIoT enabler server by sending the AIoT service data provisioning request. The request contains the following information:

a.	Mapping of AIoT devece IDs and specific application objects (such as a product, a pallet, a tool, or a person) ID. This is the premise of all associations. When attaching AIoT devices to objects of the ASP, it is necessary to create a "AIoT devece ID and application object" association in theAIoT enabler server to record this binding relationship.

b.	The application object information including the object ID, name, type, other attributes, etc.

c.	AIoT application service regions and its associated target area information, the associated AIoT service scenarios.

d.	AIoT service scenario and its cooresponding service requriements and interested AIoT service events

The AIoT service scenario is tightly correlated with the application service meanings. The cooresponding service requirements and the interested AIoT service event will be used be later inforced during the invocation of AIoT network services and the processing of network exposured AIoT data before providing enriched information to the AIoT application layer.

The AIoT service scenario categorized based on the application requirements, such as inbound/outbound, inventory, monitor and tag command, and each type may need different kinds of 3GPP AIoT network service. For different scenarios, the interested AIoT service events may be different, which should be set according to the requirements of the application layer. For example, the warehouse storage inventory scenario may need the inventory result about what objects are present or what objects are absent or the number of objects.

An example of scenario type is listed as table 7.6.1.2-1.

Table 7.6.1.2-1: an example of scenario type, service requirement, interested service event

| AIoT service scenario Type | AIoT network services requements | interested AIoT service events |
| --- | --- | --- |
| Inbond/Outbound | Rapid inventory and report with time interval X for result aggregation | Inbond/Outbound event/ Alarm Event |
| Storage Inventory | non-rapid inventory and report time interval Y for result aggregation | Inventory/Alarm/ Counting Event |
| Monitor | periodic inventory | Moving/Alarm Event |
| Tag command | Read/Write/Disable service | Read/Write/Disable result event |

2.	The AIoT enabler layer performs the authorization check. If authorized, it stores the information received in the request for futher usage during the AIoT task execution and notification phase.

3.	The AIoT enabler layer returns a AIoT service data provisioning response indicating the success of service provisioning.

#### 7.6.1.3	Information flows

##### 7.6.1.3.1 AIoT service data provisioning request

Table 7.6.1.3.1 describes the AIoT service data provisioning request from the consumer (VAL server) and the AIoT enabler server.

Table 7.6.1.3.1-1: AIoT service data provisioning request

| Information element | Status | Description |
| --- | --- | --- |
| Requester identifier | M | The identifier of the requester |
| List of AIoT device mapping info | O (NOTE) | The list of mapping of AIoT devece ID and the application service data |
| > AIoT device mapping information | M | The mapping of AIoT devece ID and the application service data as described in table 7.6.1.3.1-2 |
| List of application objects | O(NOTE) | The list of application object information which describes the attributes of an application object |
| > Application object information | M | The attrbiutes of an application object as described in table 7.6.1.3.1-3 |
| List of application regions | O(NOTE) | The list of application region information which describes the attributes of an application layer region |
| > Application region information | M | The attributes of an application layer region as described in table 7.6.1.3.1-4 |
| List of service scenario | O(NOTE) | The list of application region information which describes the attributes of a service scenario |
| > Service scenario information | M | The attributes of a service scenario as described in table 7.6.1.3.1-5 |
| NOTE: At least one of them shall be present. |  |  |

Table 7.6.1.3.1-2: AIoT device mapping information

| Information element | Status | Description |
| --- | --- | --- |
| AIoT Device ID | M | The device ID of the tags. |
| Active state | M | Indicates whether the tag is activated at the application layer. |
| Application objects ID | M | The identifier of the application object. |
| Application region ID | O | The application region associated with the tag |
| Service scenario ID | O | The service scenario associated with the tag |
| VAL server information | O | The endpoint or identifier of the VAL server to receive the information related with this tag |

Table 7.6.1.3.1-3: Application object information

| Information element | Status | Description |
| --- | --- | --- |
| Application object ID | M | The unique identifier of the application object. |
| Application Object Type | M | The type of the application objects. |
| Application Object Name | O | The specific name of the application objects. |
| Other Application Attributes | O | Other attributes of the application objects at the application layer. |

Table 7.6.1.3.1-4: Application region informaiton

| Information element | Status | Description |
| --- | --- | --- |
| AIoT application region ID | M | The unique identifier of the AIoT application region. |
| AIoT application region name | O | The name of the AIoT application region. |
| Target service area info | O | The target service area information mapped to the AIoT application region |
| Service scenario ID | M | The AIoT service scenario that the region area belongs to (e.g., inbound/outbound, Storage Inventory, Monitor, Tag command) |

Table 7.6.1.3.1-5: Service scenario information

| Information element | Status | Description |
| --- | --- | --- |
| AIoT service scenario ID | M | The unique identifier of AIoT service scenario |
| AIoT service scenario name | O | The name of AIoT service scenario |
| AIoT network services requirements | M | AIoT network services requirements |
| >AIoT service operation type | M | Inventory/Read/Write/Disable service |
| >IS periodic | M | Single or periodic. |
| >Period | O | If periodic, the period of the AIoT service operation. |
| >device ID information | O | Device ID information (e.g., device id filter information, estimated quantity) |
| Interested AIoT service events | M | The information that the AIoT application services interests. |
| >Inbound event | O (NOTE) | Which application objects have been arriving at which AIoT application service region. |
| >Outbound event |  | Which application objects have been leaving at which AIoT application service region. |
| >Alarm event | O(NOTE) | What application objects are not present at which AIoT application service area. |
| > Inventory event | O(NOTE) | What application objects have been inventoried at which AIoT application service area. |
| >Counting event | O(NOTE) | The counting result of the devices that are inventoried. |
| >Moving event | O(NOTE) | What application objects have been moved from one AIoT application service area to another. |
| >service result event | O(NOTE) | The result of the AIoT read/write/disable service. |
| NOTE: At least one of them shall be present. |  |  |

NOTE:	The Application objects ID, Application region ID, Service scenario ID are unique within a specific application domain.

##### 7.6.1.3.2 AIoT service data provisioning response

Table 7.6.1.3.2-1 describes the AIoT service data provisioning response from the AIoT enabler server and the consumer (VAL server).

Table 7.6.1.3.2-1: AIoT service data provisioning response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | The indication that the request was successful. |
| Failure response | O (NOTE) | The indication that the request failed. |
| > Cause | O | The failure cause. |
| NOTE: The IE is only present if the Result is failure. |  |  |

### 7.6.2	Architecture Impacts

This solution introduces the AIoT service provisioning capability to the AIoT enabler layer.

### 7.6.3	Corresponding APIs

A new AIoT service provisioning service is introduced.

### 7.6.4	Solution evaluation

## 7.7	Solution #7: Application AIoT Task execution

### 7.7.1	Solution description

#### 7.7.1.1	General

This solution address key issue#2, 3 and 4 to introduce a solution for application AIoT task execution. The AIoT tasks from the application layer cannot be directly aligned with the AIoT NEF interface. It is necessary to translate AIoT tasks into AIoT inventory and/or command towards the 3GPP core network.

#### 7.7.1.2	Procedures

The figure 7.7.1.2-1 shows a high-level description of the AIoT task execution initiated by AIoT application layer entity (e.g., VAL server) towards the AIoT enabler server.

Pre-conditions:

-	The VAL consumer (e.g., VAL server) has provisioned the application layer related information as described in sol#6.

Figure 7.7.1.2-1: AIoT Task initiation

1.	AIoT task is generated in the AIoT application layer according to the service requirements. Usually, the AIoT task is described with AIoT service scenario with the information of application object and AIoT application service region as described in table 7.7.1.2-1. The AIoT task is initiated and sent to AIoT enabler server with AIoT task information.

Table 7.7.1.2-1: example of Application AIoT task description

Application AIoT Tasks information

Application Object ID(s)

AIoT application region ID

AIoT service scenario ID

2.	The AIoT enabler server determines to invoke the network Inventory and/or Command service API towards the 3GPP core network. The value of input parameters of the Inventory and/or Command service API is determined based on the received AIoT task information and the provisioned service data as described in sol#6 in clause 7.6.

For example, for the inbound task in warehouse A at check point X, the AIoT enable layer needs to determine the target area information, which is associated to “warehouse A check point X” from table 7.6.1.3.1-4. Due to the AIoT service scenario ID indicating it is “inbound” scenario, then a rapid periodic AIoT inventory service should be performed with a time interval of result aggregation being set Xms. The AIoT inventory towards the 3GPP network may be performance multiple times as instructed by the AIoT network services requirements as described in table 7.6.1.3.1-5.

In another example, for the monitor task in warehouse A, the AIoT enable server needs determines the corresponding AIoT network services requirements and the Interested AIoT service events as described in table 7.6.1.3.1-5. Then the AIoT enabler server performs periodically inventory towards target service area corresponding to the service region area of warehouse A (the mapping is provisioned in table 7.6.1.3.1-4) with a time interval of result aggregation being set Yms.

3.	The final AIoT service API invocation(s) is initiated towards the 3GPP core network.

4.	The 3GPP core network starts the AIoT services operations

5.	The AIoT enabler server sends a AIoT task initiate response information to the application layer to notify whether the AIoT task is initiate successfully or not, then the application layer decides whether to wait for the result or republish the AIoT task.

NOTE 1:	The step 5 can be performed before step 4.

NOTE 2:	The step 1 and step 5 can use the subscribe/notification model.

#### 7.7.1.3	Information flows

##### 7.7.1.3.1	AIoT task initiate request

Table 7.7.3.1-1 describes the AIoT task initiate request from the AIoT application layer consumer (e.g., VAL server) towards the AIoT enabler server.

Table 7.7.1.3.1-1: AIoT task initiate request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identifier | M | The identifier of the requester. |
| Notification endpoint | M | The endpoint to receive the reported AIoT task execution result. |
| AIoT Service scenario ID | M | Represent the AIoT service scenario that the AIoT task initiate request is targeting for. |
| Request target Information | M | The information associated with the AIoT request. |
| > Application Object ID(s) | M | The target object of the AIoT task initiate request is targeting for. |
| AIoT application region ID | O | The application region where AIoT task initiate request is targeting for. |
| Reporting Interval | M | Time interval for AIoT enabler server to report results to AIoT application layer. |

NOTE:	The Application objects ID, Application region ID, Service scenario ID are unique within a specific application domain.

##### 7.7.1.3.2	AIoT task initiate response

Table 7.7.1.3.2-1 describes the AIoT task initiate response from the AIoT enabler server to the AIoT application layer consumer (e.g., VAL server).

Table 7.7.1.3.2-1: AIoT task initiate response

| Information element | Status | Description |
| --- | --- | --- |
| Success | M (NOTE) | Indicate the request is successfully processed. |
| > CorrelationID | M | The identifier of the AIoT task execution which will be used further to correlate the notifications sent from the AIoT enabler server. |
| Failed | M (NOTE) | The AIoT task initiate request execution failed. |
| > Failure Cause | O | The Reasons for AIoT task initiate request execution failure. |
| NOTE: Only one of them shall present. |  |  |

### 7.7.2	Architecture Impacts

This solution introduces the AIoT task transformation capability to the AIoT enabler server.

### 7.7.3	Corresponding APIs

This solution introduces a new AIoT task execution service.

### 7.7.4	Solution evaluation

## 7.8	Solution #8: Exposing the value-added information of AIoT devices to the consumer

### 7.8.1	Solution Architecture

This solution is based on an independent architecture that is newly defined in the Sol#2. In the architecture, an AIoTApp application enabler is defined and acts as a SEAL layer function entity. This function entity can be deployed and implemented by integrating with other related service enabler functions (e.g., SEALDD server, NSCE server) for the support of the Ambient IoT application services.

### 7.8.2	Solution description

For AIoT service operation requests, the collected AIoT Devices ID(s) could be huge, less categorized and may not fulfill the service requirements from the 3rd parties (e.g. AS/AF).

For example, a 3rd-party retail App-server (i.e., VAL server) may issue periodic inventory service requests to check the volume of some specific types of AIoT devices in a (large) area. When the number of the available AIoT devices of a type drops below a certain threshold, the retail store might run into service interruption of not having sufficient goods for customers. In this scenario, the corresponding retailer is better off replenishing the stock in advance by adding more devices of the type. The current exposure information from the 5G core network as specified in 3GPP TS 23.369[3] lacks the categorization for the AIoT devices.

In another scenario, if a VAL server (AF) sends an 'inventory service’ request covering a large area, or send continuously a batch of 'inventory service’ requests covering different (small) areas, though belonging to the same (large) targeted area whose identity could be a geographical civic address, then, the AIoT application enabler may aggregate these requests to optimize the efficiency by sending only one combined 'inventory service’ request, or sending multiple requests to multiple AIOTF in 5GC.

Besides, some VAL servers may need to regularly track the location of the AIoT devices (e.g. the goods in the truck) and identify the trajectory or the direction for the AIoT devices depending on their locations. Alert messages may be sent to certain AIoT devices in case they deviate from the right trajectory/direction. How to leverage the 5GC exposure capability of mapping the RAN ID to the location information (e.g. civic address) and accordingly tracking the devices’ location continually as requested should be studied further in application enablement layer.

In a word, it would be of great advantage to AIoT services consumers, i.e., the VAL server, if they can receive enriched results from the Ambient IoT App enabler.

Based on the functional model for AIoTApp service as in the Sol#2, the application enablement architecture layer is proposed to enhance at the server side. The VAL server communicates with the AIoTApp enabler via the AIoTApp-S reference point, which further communicates with the underlying 3GPP network systems. For any additional processing of value-add services, e.g., more effective batched processing of service requests, aggregation of captured AIoT device information, exposure of location info. of AIoT devices, etc.,   the application enabler AIoTApp can provide the enriched services.

### 7.8.3	Procedure

#### 7.8.3.1	Exposure of value-added information via AIoT application enabler

Figure 7.8.3.1-1 illustrates the procedure for the AIoT application enabler to interact with the 5GS for the support of the Ambient IoT App service that leverages the corresponding 5GC NF services and APIs as specified in the TS 23.369 [3]. Upon receiving the requested AIoT device information, the AIoTApp enabler conducts the value-added information processing before exposing the information to the VAL server (i.e., AIoT service consumer).

Pre-condition:

-	The VAL server discovers and selects the AIoTApp enabler by CAPIF functions.

-	Multiple AIOTFs may be discovered and selected in the 5GS.

Figure 7.8.3.1-1: Exposure of value-added info via AIoT App enabler

1.	The VAL server decides to use the AIoT application enabler (AIoTApp) to collect the AIoT device information and allocates address/port as AIoTApp-S Data transmission connection information for receiving the AIoT data packets from the AIoTApp enabler. The VAL server sends the AIoT_app_service request to the AIoTApp enabler. The type of AIoT service is Inventory request. The service request includes VAL server ID, VAL service ID, AIoTApp-S Data transmission connection information of the VAL server side, and the AIoT service related parameters (e.g., AF ID, [External Target Area information], [information about the target AIoT Device(s)], [Approximate number of AIoT Devices], [time interval], etc.). The details of these service parameters are described in 3GPP TS 23.369 [3].

As specified in the clause 5.8 of TS 23.369 [3], Information about the target AIoT Device(s) may include Filtering Information that might be constructed by one or multiple components (i.e. ID Type, PLMN Identifier, NID, third party identifier and Identification Information). These components can be used by the AIoTApp enabler to enrich the processing.

2.	Upon receiving the request, the AIoTApp enabler performs an authorization check. If authorization is successful, the AIoTApp enabler allocates the AIoTApp-S data transmission connection information (e.g., address/port) of AIoTApp enabler side to receive the AIoT data packets from the VAL server to be delivered to the 5GC. The AIoTApp enabler responds with the AIoT_app_service response.

NOTE 1:	The AIoTApp-S data transmission connection information of the AIoTApp enabler side is optional, if the AIoTApp enabler uses the downlink pull mode to fetch the AIoT data from the address provided by the VAL server in step 1, and uses the uplink push mode to send the AIoT data to the address provided by VAL server.

3.	After the AIoTApp enabler receives from the VAL server the AIoT service related parameters, depending on the local configuration, the AIoTApp enabler may apply enriched and optimized processing, e.g.:

-	If the received request covers a large area, the AIoTApp may send multiple requests to multiple discovered and selected AIOTF NFs in 5GC. This is applicable to the trusted AF case.

-	If the AIoTApp receives batched requests continuously with each one spanning a small area, the AIoTApp may either aggregate the batched requests and send only one combined service request or send multiple requests concurrently to multiple AIOTF NFs in 5GC.

4.	The AIoTApp sends service operation requests towards the (selected) AIOTF(s), as in 3GPP TS 23.369 [3] clause 6.2. There could be multiple AIOTF NFs selected from the step-3.

NOTE 2:	All the invoked service operations, together with the service parameters, conform to what have been specified in TS 23.369 [3]. E.g., depending on the trustiness of the AF (i.e., AIoTApp), AIoTApp sends either External Target Area information (to NEF as an untrusted AF) or Target Area information (to AIOTF as a trusted AF).

The 5GS processes the AIoT service request(s) and collects the AIoT device information based on the provided service parameters, or local configuration. There could be multiple AIoTF(s) sending collected AIoT device information to AIoTApp (or via NEF). Details are in TS 23.369 [3].

5.	The AIoTApp enabler apply value-added processing to the received AIoT device information, e.g.

-	If AIoTApp receives AIoT device information for a large target area from multiple AIOTF NFs, then it can consolidate the information before send to the VAL server.

-	Based on the components in the Filtering Information received in the step-1, e.g., ID Type, the AIoTApp enabler may consolidate and categorize AIoT devices and derive the number of the available AIoT devices of a type in a target area. The AIoTApp then exposes the value-added information to the VAL server for further processing.

6,	The AIoTApp enabler sends the processed AIoT device information via the AIoT_app_service notification to the VAL server.

#### 7.8.3.2	Information flows

Editor’s Note: The details of Information Flows are FFS.

##### 7.8.3.2.1	AIoT App Service request

Table 7.8.3.2.1-1 describes the information flow from the VAL server to the AIoTApp enabler for the support of the AIoT App service.

Table 7.8.3.2.1-1: AIoT App service request

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | Identity of the VAL server |
| VAL service ID | O | Identity of the VAL service |
| AIoTApp-S endpoint information | M | Address/port and/or URL of the VAL server to receive the AIoT data packets from the AIoTApp. |
| AIoT service parameters | M (Note 1) | AIoT service parameters (e.g., AF ID, (external) target area info, etc.). |
| DL AIoT data delivery status subscription indication | O | Indicates the VAL server expected to receive the DL AIoT delivery status notification |
| NOTE: The AIoT service parameters are specified in the 3GPP TS 23.369 [3]. |  |  |

##### 7.8.3.2.2	AIoT App service response

Table 7.8.3.2.2-1 describes the information flow from the AIoTApp enabler to the VAL server to respond to the AIoT App service request.

Table 7.8.3.2.2-1: AIoT App service response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure. |
| AIoTApp-S enpoint information | O | Address/port and/or URL of the AIoTApp enabler to receive the AIoT data packets from the VAL server to be delivered to 5GC. |
| Cause | O (see NOTE) | Indicates the reason for the failure, e.g., AF ID not supported, Target Area not allowed. |
| NOTE: The IE is only present if the Result is failure. |  |  |

##### 7.8.3.2.3	AIoT App service notification

Table 7..8.3.2.3-1 describes the information flow from the AIoTApp enabler to the VAL server to notify the events related to the AIoT App service.

Table 7.8.3.2.3-1: AIoT App service notification

| Information element | Status | Description |
| --- | --- | --- |
| Event ID | O (see NOTE) | Identifies event of the AIoTApp enabler interaction status with 5GC for AIoT App services, e.g., enriched processing results, no collected AIoT device info., etc. |
| VAL service ID | O | Identity of the VAL service. |
| DL AIoT data delivery instructions | O (see NOTE) | Indicates the instructions to the VAL server regarding the DL AIoT App service |
| > Collected AIoT device info. | O | Indicates the collected raw AIoT device information. |
| > Enrich-processed AIoT info. | O | Indicates enriched processing results of AIoT info (e.g., categorization, consolidation, filtering, etc.) |
| NOTE: Either Event ID IE or the DL AIoT data delivery instruction ID is present. |  |  |

### 7.8.4	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

## 7.9	Solution #9: Support of periodical and event-triggered AIoT service operations

### 7.9.1	Solution description

#### 7.9.1.1	General

This solution addresses the KI#2. Just as the example described in the KI#2, a 3rd-party retail App-server (i.e., VAL server) may issue periodic or event-triggered inventory service requests to check the changed volume of some specific types of AIoT devices in a certain area. When the number of the available AIoT devices of a type drops below a certain threshold, the retail store might run into service interruption of not having sufficient goods for customers. In this scenario, the corresponding retailer is better off replenishing the stock in advance by adding more devices of the type. If the retail store asks for periodic or event-triggered inventory service, it’s expected to be informed when and which type of devices will be increased or decreased.

This solution is proposed to fulfil the above scenario and mainly support the following service requests:

-	Periodic inventory service request;

-	Event-triggered inventory service request;

-	Report changed volume of some specific types of AIoT devices in a certain area;

-	Report when and which type of devices will be increased or decreased.

Clause 7.9.1.2 illustrates the high-level procedure of periodic and event-triggered AIoT inventory service requests.

#### 7.9.1.2	Procedure of AIoT inventory service requests

Figure 7.9.1.2-1 illustrates the high-level procedures of periodic and event-triggered inventory service request triggered by the VAL server(s).

Pre-condition:

-	The AIoT enabler is the new application enabler in the application enablement layer which supports the AIoT related operations and services. For the functionalities of new AIoT enabler, please check the Sol#1 for KI#1 for more details.

Figure 7.9.1.2-1: Procedure of AIoT inventory service requests

1.	The VAL server sends an AIoT Inventory subscription request to the AIoT enabler, including the service type indication (e.g., inventory), service ID, target area information, target AIoT device information , triggering conditions (e.g., periodic or event-triggered reporting), time interval for periodic reporting, trigger events for event-triggered reporting (e.g. when the number of one type of AIoT devices is going to reach the pre-defined threshold), only reporting the changed volume indicator for AIoT devices, etc.

The information about the target AIoT device(s) may include the Filtering Information (e.g. ID Type, PLMN Identifier, NID, third party identifier and Identification Information, etc.) as described in clause 5.8 of 3GPP TS 23.369 [3].

The indication of only reporting the changed volume for AIoT devices means only the changed volume compared to last report for the specific AIoT device will be reported instead of all of AIoT device IDs.

2.	The AIoT enabler checks whether the VAL server is authorized to request the Inventory subscription request, may be based on e.g., the pre-configurations or the operator policies.

3.	If the request is authorized, the AIoT enabler sends an AIoT Inventory subscription response to the VAL server.

4.	Then, the AIoT enabler invokes Nnef_AIoT_Inventory request to the 3GPP CN to obtain the AIoT device data as specified in clause 6.2.2 of 3GPP TS 23.369 [3] according to the service requests (e.g. time interval for periodic reporting)  in step 1. And the AIoT enabler will set the time interval for the periodical Nnef_AIoT_Inventory request less than or equals to the time interval received in Step 1 for the periodic reporting request.

5.	The AIoT enabler stores the received AIoT device information and analyse them to generate the related analysis information for AIoT devices based on the AIoT Inventory subscription service request. The AIoT enabler may classify the AIoT devices based on the requested AIoT Device(s) filtered information (e.g., ID type ), count the number for each type of AIoT devices per different granularity (e.g. maximum, minimum and average number), compare the number for two sequential times for the same target AIoT device in the same target area, and then calculate the corresponding changed information including e.g., the change is increase/decrease, the changed number/volume, the specific AIoT device type corresponding to the changes, etc.

Or the AIoT enabler may compare the number of AIoT devices with pre-defined threshold received in step 1 if the trigger condition is event trigger. If the number of certain AIoT devices are going to reach the pre-defined threshold, the AIoT enabler may immediately notify the VAL server to indicate the number of specific AIoT devices will reach the pre-defined threshold, with the AIoT device ID, the total device numbers, the trigger events, the suggested actions (e.g. increase/decrease the number of specific AIoT devices), etc.

6.	The AIoT enabler will report the AIoT device analysis information to the VAL server based on the trigger condition received in step 1. The report may include the AIoT device ID, the number of AIoT devices which may be all devices or the devices filtered based on different criteria (e.g. PLMN, third-party ID,  etc.), the changed information compared to the previous report (e.g. the change is increase/decrease, the number/volume of change, etc.), the trigger events and the suggested actions (e.g. increase/decrease the number of specific AIoT devices), the time point of the report, the last report indication, etc.

NOTE 1:	Whether the AIoT enabler reports the changed information depends on the indication of only reporting the changed volume for AIoT devices in Step 1 is included or not.

NOTE 2:	Reporting of changed information is only applicable to the second and subsequent notification messages.

### 7.9.2	Architecture Impacts

This solution proposes a new architecture (i.e., new AIoT enabler) to support AIoT services.

For the new architecture, please check the Sol#1for KI#1 for more details.

### 7.9.3	Corresponding APIs

This clause provides the corresponding APIs for supporting the solution.

### 7.9.4	Solution evaluation

This solution addresses the KI#2 to support the periodic and the event-triggered AIoT inventory service requests from the VAL servers.

This solution assumes there is a new application enabler (i.e. AIoT enabler) in the application enablement layer to support the AIoT services. The new AIoT enabler supports enriching the processing of AIoT device ID (including classification, aggregation, comparation, calculation, etc.) and reporting the generated value-added AIoT device information and suggested actions to the VAL server.

## 7.10	Solution #10: Support of querying history data for AIoT devices

### 7.10.1	Solution description

#### 7.10.1.1	General

This solution addresses the KI#2. Some application servers (e.g., a super market) may need to check the history data for some specific type of AIoT devices (e.g., check the sales volume of umbrellas from last year) to predict the trends for this year and beyond. So it’s necessary for application servers to query the history data for the AIoT devices in the application enabled layer.

This solution is proposed to fulfil the above scenario and mainly support the following service requests:

-	Query the history data (e.g., device number, device location info, etc.) for AIoT devices;

-	Report the history data to the application server.

#### 7.10.1.2	Procedure of querying history data for AIoT devices

Figure 7.10.1.2-1 illustrates the high-level procedure of querying history data for AIoT devices.

Pre-condition:

-	The AIoT enabler is the new application enabler in the application enablement layer which supports the AIoT related operations and services.

-	The AIoT enabler support to store the history data for requested AIoT devices.

-	The VAL server doesn’t have the requested AIoT history data.

Figure 7.10.1.2-1: Procedure of querying history data for AIoT devices

1.	The AIoT enabler may interact with 3GPP CN to query the AIoT devices data periodically as described in solution Sol#9 and store the obtained AIoT device data.

2. 	The VAL server sends an AIoT history data request to the AIoT enabler, including the query service type (e.g., device location, device number), service ID, target area information, target AIoT device information, and target time point/period, etc.

Information about the target AIoT Device(s) may include Filtering Information as described in clause 5.8 of 3GPP TS 23.369 [3].

3.	The AIoT enabler checks whether the VAL server is authorized to request the AIoT history data request may be based on e.g. the pre-configurations or operator policies.

4.	If the request is authorized, the AIoT enabler checks if the stored sensing data in the local storage could meet the service requirements received in step 1.  If yes, the AIoT enabler may report the requested history data and perform the statistical analysis for the retrieved information. E.g., if the query type is device number, the report may include the total number for the specific AIoT devices as well as the maximum/minimum/average number. If there is no local storage or the local storage cannot fulfil the service requirements, the AIoT enabler may response with the failure cause.

5.	The AIoT enabler reports the history data for the requested AIoT device to the VAL server via sending AIoT history data response message. The report may include the AIoT device ID, the retrieved history data for the AIoT device, and related statistical analysis for the retrieved data, etc.

### 7.10.2	Architecture Impacts

This solution proposes a new architecture (i.e., new AIoT enabler) to support AIoT services.

For the new architecture, please check the Sol#1for KI#1 for more details.

### 7.10.3	Corresponding APIs

This clause provides the corresponding APIs for supporting the solution.

### 7.10.4	Solution evaluation

This solution addresses the KI#2 to support the AIoT devices history information query request from the VAL server.

This solution assumes there is a new application enabler (i.e. AIoT enabler) in the application enablement layer to support the AIoT services. And the AIoT enabler supports to store the history data for the AIoT devices.

The solution applies when the VAL server doesn’t have the requested AIoT devices history data and the AIoT enabler enables could retrieve the data from the local storage internally and report them with the statistical analysis (e.g. maximum, minimum, average number, etc.).

## 7.11	Solution #11: Provision and monitor AIoT device presence

### 7.11.1	Solution Architecture

This solution is based on an independent architecture that is newly defined in the Sol#2. In the architecture, an AIoTApp application enabler is defined and acts as a SEAL layer function entity. This function entity can be deployed and implemented by integrating with other related service enabler functions for the support of the Ambient IoT application services.

### 7.11.2	Solution description

For the AIoT service operation requests defined in 3GPP TS 23.369 [3], i.e., Inventory and Command, there exist certain scenarios for which the AIoT application enabler can enhance the operations.

For example, a 3rd-party application server (e.g., VAL server) may collect the AIoT device information (e.g., location information) via inventory service request for an AIoT device originally located in a first target area. This AIoT device may be moved to a second target area for any reason. When the 3rd-party application server issues inventory service request for the same AIoT device to the first target area, the failure to retrieve the AIoT device information will be generated. For this scenario, the AIoTApp application enabler may provide a remediation mechanism by searching within alternate targeted area(s) so as to pinpoint the up-to-date information of the AIoT device. The 5GC does not provide such remediation mechanisms.

In another scenario, some application servers (e.g. a logistics company) may need to regularly track the locations of AIoT devices (e.g. the goods in the truck) and identify the (moving) path of the AIoT devices, based on which warnings could be sent to the AIoT devices in case they deviate the right/expected track. Recently, the SA2 WG has reached agreement to support the exposure of AIoT device location information to AF as in 3GPP TS 23.369 [3].

In a 3rd scenario, when concurrent monitoring operations need to be performed on multiple AIoT devices, repeatedly issuing the same commands can significantly reduce system scheduling efficiency. Since base stations typically only support serial operations, meaning they can handle only one task at a time, the application enablement layer must coordinate and aggregate concurrent requests from multiple applications. For instance, if multiple operations target different AIoT devices within the same area, they can be consolidated into a single command and uniformly dispatched to the 5GC, thereby avoiding redundant scheduling and enhancing processing efficiency. Also, the application enablement layer can intelligently filter based on device identifiers, for instance, selecting devices that match specific code segments. Drawing from the masking mechanism of RFID, these devices can be abstracted into a logical group and described through a unified masking rule. This involves filtering target devices according to identifier rules (such as those with similar ID segments) and then configuring them into a mask group. Subsequently, the application enablement layer only needs to issue a single monitoring command to the 5GC and specify this mask to achieve batch operations on all devices within the group.

Therefore, it would be of great advantage if the Ambient IoT App enabler could help monitor the presence as well as track the path of AIoT devices in more efficient schemes.

Based on the functional model for AIoTApp service as in the Sol#2, the application enablement architecture layer is proposed to enhance at the server side. The VAL server communicates with the AIoTApp enabler via the AIoTApp-S reference point, which further communicates with the underlying 3GPP network systems via either the N33 (via NEF for an untrusted AF), or the interface off the 5GC AIOT NF (i.e., AIOTF) directly providing the AIoT service operations to a trusted AF. The enhanced provisioning and monitoring operations will be handled by the AIoTApp enabler.

A VAL server provides to the App enabler AIoTApp the parameter ‘location information requested’, along with other parameters. Then, AIoTApp interacts with the 5GS NF AIOTF to retrieve the location information of an AIoT device. Accordingly, AIoTApp can track the presence of the device as well as its trajectory. AIoTApp enabler may also request location information based on its local configuration.

AIoTApp can send concurrent service requests to multiple AIOTFs in 5GC to optimize the operations.

### 7.11.3	Procedure

#### 7.11.3.1	Provision and monitor AIoT device presence

Figure 7.11.3.1-1 illustrates the procedure for the AIoT application enabler to interact with the 5GS for the support of the Ambient IoT App service that leverages the corresponding 5GC NF services and APIs as specified in the TS 23.369 [3]. The VAL server sends the ‘location information request’ indication to AIoTApp, along with the potential monitoring schedule. The AIoTApp sends the received request, or based on local configuration, to the 5GC. Once the requested AIoT device information (e.g., location information) is received from the 5GC, the AIoTApp enabler can do more enhanced information processing (e.g., presence verification, location tracking) before sending the information to the VAL server (i.e., AIoT service consumer).

Pre-condition:

-	The VAL server discovers and selects the AIoTApp enabler by CAPIF functions.

-	Multiple AIOTFs may be discovered and selected in the 5GS.

-	AIOTFs have determined to provide the AIoT device location information to AF based on operator policy.

Figure 7.11.3.1-1: Provision and monitor AIoT device presence

1.	The VAL server decides to use the AIoT application enabler (AIoTApp) to collect the AIoT device information and allocates address/port as AIoTApp-S Data transmission connection information for receiving the AIoT data packets from the AIoTApp enabler. The VAL server sends the AIoT_app_service request to the AIoTApp enabler. The type of AIoT service request is Inventory. The service request includes VAL server ID, VAL service ID, and the AIoT service related parameters (e.g., AF ID, [External Target Area information], [information about the target AIoT Device(s)], [Approximate number of AIoT Devices], [time interval], location information requested). The VAL server may also send AIoT device provisioning and monitoring parameters (e.g., device location tracking request, monitoring schedule (e.g., every hour for 24 hours), device-id/a group of device-IDs). The details of AIoT service related parameters are described in 3GPP TS 23.369 [3].

As specified in the clause 5.8 of 3GPP TS 23.369 [3], Information about the target AIoT Device(s) may include Filtering Information that might be constructed by one or multiple components (i.e. ID Type, PLMN Identifier, NID, third party identifier and Identification Information). These components can be used by the AIoTApp enabler to identify a particular AIoT device or a group of devices.

2.	Upon receiving the request, the AIoTApp enabler performs an authorization check. If authorization is successful, the AIoTApp enabler responds with the AIoT_app_service response.

3.	After the AIoTApp enabler receives from the VAL server both the AIoT service related parameters and AIoT device provisioning and monitoring parameters, depending on the local configuration, the AIoTApp enabler sends AIoT service requests to 5GC, with the indication ‘location information requested’.

The AIoTApp may send concurrently the requests for a group of AIoT device-IDs to multiple discovered and selected AIOTF NFs in 5GC.

4.	The AIoTApp sends Inventory service operations towards the (selected) AIOTF(s), as in 3GPP TS 23.369 clause 6.2.

There could be multiple AIOTF NFs selected from the step-3 for concurrent AIoT device monitoring operations.

If the AIoTApp enabler has received multiple AIoT Inventory service requests, and the target area parameters of these multiple requests point to the same network target service area, the AIoT App enabler aggregates those AIoT service requests and initiate a single Inventory service API invocation (with the parameter being the network target area) towards the 3GPP network. If multiple service requests under the same network target service area include different AIoT device IDs which are covered by the same filter(s), the AIoT App enabler further processes those device IDs using a masking mechanism and then sends a single Inventory service API invocation with the filter(s) towards the 3GPP network.

NOTE 2:	All the invoked service operations, together with the service parameters, conform to what have been specified in TS 23.369. E.g., depending on the trustiness of the AF (i.e., AIoTApp), AIoTApp sends either External Target Area information (to NEF as an untrusted AF) or Target Area information (to AIOTF as a trusted AF).

NOTE 3:	The AF (i.e., AIoTApp) can send the parameter ‘location information requested’ to AIOTF in 5GC to request the AIOTF to report the location information of AIoT devices.

The 5GS processes the AIoT service request(s) and collects the AIoT device information based on the provisioned service parameters, or local configuration. The AIoT device location information can be reported to the AF (i.e., AIoTApp) as described in the TS 23.369 [3]. (Multiple) AIOTF(s) send collected AIoT device information to AIoTApp (or via NEF). This information may include the location information of an AIoT device, depending on operator policy and the requested parameter from the AIoTApp.

5.	The AIoTApp enabler processes the received AIoT device information, e.g.

-	If the VAL server has requested the AIoTApp to monitor the presence of an AIoT device and the AIoTApp receives the device information for the AIoT device, then AIoTApp reports the presence to the VAL server.

-	If the VAL server has requested the AIoTApp to monitor the presence of an AIoT device and the AIoTApp does not receive the device information for the AIoT device from 5GC, then AIoTApp may expand the service request in more Target Areas that have been received in step-1 from the VAL server. The step 4 of the procedure will be repeated to monitor the presence of the AIoT device.

-	If the VAL server has requested the AIoTApp to track the historical location information of an AIoT device, then AIoTApp will record the received location information of the AIoT device. Based on the monitoring schedule received in step-1, the AIoTApp enabler may repeat the step 4 of the procedure to continue tracking the location of the device. When the monitoring schedule is fulfilled, the AIoTApp can send the tracked historical location information of the AIoT device to the VAL server.

6.	The AIoTApp enabler sends the processed AIoT device information via the AIoT_app_service notification to the VAL server.

#### 7.11.3.2	Information flows

Editor’s Note: The details of Information flows are FFS.

##### 7.11.3.2.1	AIoT App Service request

Table 7.11.3.2.1-1 describes the information flow from the VAL server to the AIoTApp enabler for the support of the AIoT App service.

Table 7.11.3.2.1-1: AIoT App service request

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | Identity of the VAL server |
| VAL service ID | O | Identity of the VAL service |
| AIoT service parameters | M (Note 1) | AIoT service parameters (e.g., AF ID, (external) target area info, location information requested, AIoT device filtering info., etc.). |
| AIoT monitoring schedule | O | AIoT service request schedule based on which AIoT service requests are sent to 5GC. |
| DL AIoT service request delivery status subscription indication | O | Indicates the VAL server expected to receive the DL AIoT service request delivery status notification |
| NOTE 1: The AIoT service parameters are specified in the 3GPP TS 23.369 [3]. |  |  |

##### 7.11.3.2.2	AIoT App service response

Table 7.11.3.2.2-1 describes the information flow from the AIoTApp enabler to the VAL server to respond to the AIoT App service request.

Table 7.11.3.2.2-1: AIoT App service response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure. |
| Cause | O (see NOTE) | Indicates the reason for the failure, e.g., AF ID not supported. |
| NOTE: The IE is only present if the Result is failure. |  |  |

##### 7.11.3.2.3	AIoT App service notification

Table 7.11.3.2.3-1 describes the information flow from the AIoTApp enabler to the VAL server to notify the events related to the AIoT App service.

Table 7.11.3.2.3-1: AIoT App service notification

| Information element | Status | Description |
| --- | --- | --- |
| Event ID | O (see NOTE) | Identifies event of the AIoTApp enabler interaction status with 5GC for AIoT App services, e.g., device present, location & trajectory tracking, no collected AIoT device info., etc. |
| VAL service ID | O | Identity of the VAL service. |
| DL AIoT service request delivery instructions | O (see NOTE) | Indicates the instructions to the VAL server regarding the DL AIoT App service request |
| > Collected AIoT device info. | O | Indicates the collected AIoT device information (raw, location, etc.) |
| > AIoT device tracking info. | O | Indicates tracked trajectory, presence monitoring results of AIoT device info |
| NOTE: Either Event ID IE or the DL AIoT service request delivery instruction ID is present. |  |  |

### 7.11.4	Corresponding APIs

This clause provides the corresponding APIs for supporting the solution.

### 7.11.5	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

## 7.12	Solution #12: Support of monitoring requests for AIoT devices

### 7.12.1	Solution description

#### 7.12.1.1	General

This solution addresses the KI#2 and KI#3. Some application servers (e.g. a logistics company) may need to regularly track the location of the AIoT devices (e.g. the goods in the truck) and identify the trajectory or the direction for the AIoT devices depending on their locations, and then send warnings to the AIoT devices in case they deviate the planned  trajectory/direction.

This solution is proposed to fulfil the above scenario and mainly support the following service requests:

-	Monitoring the location of requested AIoT devices;

-	Report the location information for the target AIoT devices;

-	Report to indicate the target AIoT device is deviating or will deviate the planned location trajectory.

#### 7.12.1.2	Procedure of Monitoring requests for AIoT devices

Figure 7.12.1.2-1 illustrates the high-level procedure of monitoring requests for AIoT devices.

Pre-condition:

-	The AIoT enabler is the new application enabler in the application enablement layer which supports the AIoT related operations and services.

Figure 7.12.1.2-1: Procedure of Monitoring requests for AIoT devices

1.	The VAL server sends an AIoT monitoring subscription request to the AIoT enabler, including the monitoring service type (e.g., location), service ID, target area information, target AIoT device information, triggering conditions (e.g., periodic or event triggered reporting), time intervals for periodic reporting, monitoring time duration, events for event-triggered reporting , AIoT device group member deviation indicator, the group ID for a group of AIoT devices and the planned location trajectory if the AIoT device group member deviation indicator is included, etc.

The AIoT device group member deviation indicator indicates whether and which AIoT device (or type of AIoT devices) is deviating (or will deviate) from other similar devices or the planned location trajectory in a group.

The information about the target AIoT Device(s) may include Filtering Information as described in clause 5.8 of 3GPP TS 23.369[3].

2.	The AIoT enabler checks whether the VAL server is authorized to request the AIoT monitoring subscription request, may be based on e.g. the pre-configurations or operator policies.

3.	If the request is authorized, the AIoT enabler sends an AIoT monitoring subscription response to the VAL server.

4.	Then, the AIoT enabler invokes Nnef_AIoT_Command request to the 3GPP CN periodically as specified in clause 6.2.3 of 3GPP TS 23.369 [3] with location information requested IE to obtain the requested AIoT device data including the location information of AIoT devices. The Command type is Read.

5.	Upon received the AIoT device data and related location information, the AIoT enabler stores them based on the AIoT monitoring subscription request. If the subscription request includes the AIoT device group member deviation analytics indication, the AIoT enabler may interact with ADAES to obtain the predicted location information and the deviation analytics for the requested AIoT devices as specified in clause 8.15 of 3GPP TS 23.436 [7].

The AIoT enabler will send the obtained AIoT device data (including related location information) to the ADAE server who doesn’t need to sends a data collection subscription request to the Data Producer (e.g. 5GC). Based on the received AIoT device data, the ADAE server analyses and sends location-related UE group analytics notifications to the AIoT enabler which may include the location information, the predicted location information, the deviation analytics for the group of AIoT devices, etc. The deviation analytics mainly indicate which AIoT device (or type of AIoT devices) is deviating (or will deviate) from other similar devices or the planned location trajectory in a group.

6. 	The AIoT enabler checks if the received location-related analytics in step 5 could meet the service requirements or not. And if not, the step 4 and step 5 may need to be operated repeatedly.

7.	The AIoT enabler reports the monitoring results for the requested AIoT device to the VAL server via sending AIoT monitoring notification message. The report may include the AIoT device ID, the location information for the AIoT device, the predicted location information for the AIoT device, the deviation analytics, the device availability status, etc.

Editor’s note: Whether and how the procedures of AIoT Inventory subscription and AIoT monitoring subscription can be combined or not is FFS.

Editor’s note: Whether and how the ADAE analytics specified in clause 8.15 of 3GPP TS 23.436 can be used or enhanced for the direction and the deviation predictions of the requested AIoT devices is FFS.

### 7.12.2	Architecture Impacts

This solution proposes a new architecture (i.e., new AIoT enabler) to support AIoT services.

For the new architecture, please check the Sol#1 for KI#1 for more details.

### 7.12.3	Corresponding APIs

This clause provides the corresponding APIs for supporting the solution.

### 7.12.4	Solution evaluation

This solution addresses the KI#2 and KI#3 to support the monitoring the location information for the AIoT devices.

This solution assumes there is a new application enabler (i.e. AIoT enabler) in the application enablement layer to support the AIoT services. The AIoT enabler enables to obtain the location information for the requested AIoT devices, and interact with ADAES to identify which AIoT devices are deviating or will deviate the other similar devices or the planned location trajectory based on the location-related UE group analytics.

## 7.13	Solution #13: AIoT Data Processing management

### 7.13.1	Solution description

#### 7.13.1.1	General

This solution address key issue#2, and #4 to introduce a solution for AIoT data processing of the network exposured AIoT inventory/command result.

The inventory results in the notification from 3GPP network is further processed and transformed into the application layer interested service events using the provisioned service data.

#### 7.13.1.2	Procedures

The figure 7.13.1.2-1 shows the high-level procedure of AIoT data processing management and event notification.

Pre-conditions:

1.	The AIoT task initiate service has been invoked by the AIoT application layer consumer, and the AIoT enabler has initiated the AIoT service request to the 3GPP core network as described in sol#6.

Figure  7.13.1.2-1: AIoT Data Processing management

1.	 The AIoT enabler server receives the notification(s) with inventory/command result from the 3GPP core network as described in 3GPP TS 23.369 [3].

2.	The AIoT enabler processes the network expoured data from step 1 and transforms them into service event based on the provisioned service data and the AIoT taks initiate request previously received. The AIoT service event contains the information of “what AIoT device ID(s)”,  “in which area,” and “what happened”. For “what happened”, there are some examples (not limited to) as below:

Type1: AIoT device appearance, the AIoT device is being reported at the service region for the first time;

Type2: AIoT device disappearance, the AIoT device is not being reported during the limited time;

Type3: AIoT device stay, the AIoT device is continuously reported at the same service region;

Type4: AIoT device moving, the AIoT device is reported from different regions.

The reported AIoT device ID is transformed back to the application layer object ID.

Further the AIoT enabler server processes the network expoured data per AIoT service scenario type indicated in the AIoT task initiate service request.

-	Inbound or Outbound type: for all the AIoT devices appearance events during the task execution period, compare the objects ID(s) reported from 3GPP network and the objects ID lists from the AIoT tasks. If the ID(s) can match, then, the application objects can be successfully inbound or outbound; otherwise, it is considered an illegal operation and an alert information is triggered.

-	Inventory type: for all the AIoT devices appearance and stay events, the corresponding objects can be seen as the normal inventory information. However, for the AIoT devices disappearance event, an alert or other method such as specific AIoT device ID inventory should be initiated.

-	Monitor type: for all the AIoT devices appearance, disappearance, moving events, the AIoT device state changed, the AIoT enable layer should record and report.

Editor’s note:	The potential overapping between inbound/outbound type and inventory type and how to resolve it is FFS.

3.	Report the interested AIoT service events to the application layer and the AIoT task is completed.

#### 7.13.1.3	Information flows

Table 7.13.1.3-1 describes the information flow of the AIoT service event notification service (from AIoT enable server to AIoT application layer consumer.

Table 7.13.1.3-1: AIoT service event notification

| Information element | Status | Description |
| --- | --- | --- |
| VAL service ID | O | Identity of the VAL service. |
| Interested AIoT service event | M | The interested AIoT service event being detected. The AIoT service event is defined in table 7.6.1.3.1-5. |
| Task execution output | M | The additional information with the interested AIoT service event. |
| > AIoT task execution result | M | The output from the AIoT task execution corresponding the interested AIoT service event. For example, for the Interested AIoT service event is inbound/outbound/alarm event, it includes e.g., application object ID(s), type(s), name(s), application region(s). For moving event, it includes the missing e.g., application object ID(s), type(s), name(s), application region list where the application object moves along. |
| > AIoT data | O | Indicates the network reported AIoT device information (AIoT device ID list, target service area, etc.) |

### 7.13.2	Architecture Impacts

This solution introduces AIoT service event notification capability to the AIoT enabler server.

### 7.13.3	Corresponding APIs

The AIoT service event notification service is introduced.

### 7.13.4	Solution evaluation

## 7.14	Solution #14: New functional model for AIoT services

### 7.14.1	Solution description

This solution addresses the KI#1. The following procedures introduce the on-network functional model for AIoT services capturing the relevant functional entities and reference points. This solution aligns with the network architecture for AIoT services which specified in clause 4.2 of 3GPP TS 23.369 [3].

#### 7.14.1.1	On-network Functional model for AIoT services

Figure 7.14.1.1-1 illustrates the generic on-network functional model for AIoT service.

Figure 7.14.1.1-1: On-network functional model for AIoT services

The AIoTE server, acting as an AF, could obtain AIoT data information from the 3GPP core network via NEF (over N33 reference point) which is defined in clause 4.5.4 of 3GPP TS 23.369 [3]. If the AIoTE server is a trusted AF, it may interact with AIOTF directly for AIoT related service as defined in clause 4.5.5 of 3GPP TS 23.369 [3]. The AIoTE server may communicate with other SEAL server(s) over SEAL-S reference point.

The VAL server(s) or AIoT application specific server(s) could communicate with the AIoT management server over AIoT-S reference point for requesting the AIoT services and receiving the AIoT data information.

The AIoTE client, acting as a consumer, could also request the AIoT services to the AIoTE server over AIoT-AE reference point, and provide the support for AIoT related functions to the VAL client(s) over AIoT-C reference point.

NOTE:	In this release, the AIoTE client only acts as an AIoT service consumer.

#### 7.14.1.2	Functional entities description

AIoTE server (AIoT Enabler Server): Functionalities offered by the AIoTE server are similar to AMS entity described in clause 7.1.1.2.

AIoTE client (AIoT Enabler Client): This entity is similar to AMC entity described in clause 7.1.1.2.

### 7.14.2	Architecture Impacts

This solution proposes a new architecture and functional mode for AIoT services.

### 7.14.3	Corresponding APIs

None

### 7.14.4	Solution evaluation

This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

## 7.15		Solution #15: Solution for AIoT information provisioning

### 7.15.1	Solution description

#### 7.15.1.1 	Introduction

This solution address key issue#3 to introduce a solution for AIoT information provisioning for the application enablement layer AIoT service.

An AIoT service consumer needs to provision information (application AIoT device profile) in the AIoT Enablement Server (AIOTES), before consuming AIoT services from the AIOTES.

NOTE 1:	 The application AIoT device profile is information known at the application layer consumer (e.g., VAL server or VAL client) about AIoT devices and required by the AIoT Enablement Server (AIOTES) to provide AIoT services to the AIoT service consumer.

NOTE 2:	According to the architecture in Solution #1, the VAL client consumes AIoT services from the AIOTES via the AIoT Enabler Client (AIOTEC).

NOTE 3:	The functionality of the AIOTEC and AIOTES is aligned and avoid conflict with SA2 functionality.

The provisioned application AIoT device profile is stored by the AIoT enablement server (AIOTES) and can be used for requesting operations such as inventory or commands from the network.

This solution defines procedures for:

-	Provisioning application AIoT device profile information in clause 7.15.1.2;

-	Updating provisioned application AIoT device profile information in clause 7.15.1.3

-	Deleting provisioned application AIoT device profile information in clause 7.15.1.4

#### 7.15.1.2 	AIoT information provisioning

The figure 7.15.1.2-1 describes the procedure for provisioning application AIoT device information in the AIOTES.

Figure 7.15.1.2-1: AIoT information provisioning

1.	The requestor (e.g., VAL server or AIOTEC) sends an AIoT information provisioning request to the AIOTES. The request includes the requestor identifier and a list of application AIoT device profiles as described in clause 7.15.3.1.

NOTE:	In this release, the AIOTEC is not a UE reader, only acts as an AIoT service consumer and application AIoT device profile information is provided by the VAL layer.

2.	Upon receiving the request from the requestor, the AIOTES validates if the requestor is authorized for the request. If the requestor is authorized, the AIOTES stores the provided application AIoT device profiles. The application AIoT device profile includes information used by the AIOTES when requesting AIoT operations (e.g., Inventory, Command) from the network as described in 3GPP TS 23.369 [3]. If the application AIoT device profile includes maximum rate, the AIOTES performs AIoT supported operations at a rate that does not exceed the maximum rate when requested by a consumer, and the AIOTES determines to wait if requested to perform operations at a faster rate. If the application AIoT device profile includes maximum transmission size, the AIOTES uses the maximum transmission size to determine the number of interactions required with the AIoT device, and the AIOTES can perform more than one operation if the requested data size exceeds the maximum transmission rate. If the application AIoT device profile includes area information, the AIOTES requests AIoT operations (e.g., Inventory, Command) with the network according to the area information.

3.	The AIOTES sends an AIoT information provisioning response to the requestor as described in clause 7.15.3.2. If the request was authorized, the response includes an indication of success for the request and the identifiers of application AIoT device profiles that were successfully provisioned. Otherwise, the response includes an indication of failure and may include a failure cause.

#### 7.15.1.3 	AIoT information provisioning update

The figure 7.15.1.3-1 describes the procedure for updating application AIoT device profiles in the AIOTES.

Figure 7.15.1.3-1: AIoT information provisioning update

1.	The requestor (e.g., VAL server or AIOTEC) sends an AIoT information provisioning update request to the AIOTES. The request includes the requestor identifier and a list of application AIoT device profiles as described in clause 7.y.3.3.

NOTE:	In this release, the AIoT management client is not an UE reader, only acts as an AIoT service consumer and application AIoT device profile information is provided by the VAL layer.

2.	Upon receiving the request from the requestor, the AIOTES validates if the requestor is authorized for the request. If the requestor is authorized, the AIOTES updates the application AIoT device profiles identified via their identifier.

3.	The AIOTES sends an AIoT information provisioning response to the requestor as described in clause 7.15.3.4. If the request was authorized, the response includes an indication of success for the request and the identifier of application AIoT device profiles that were successfully updated. Otherwise, the response includes an indication of failure and may include a failure cause.

#### 7.15.1.4 	AIoT information provisioning delete

The figure 7.15.1.4-1 describes the procedure for AIoT information provisioning delete.

Figure 7.15.1.3-1: AIoT information provisioning delete

1.	The requestor (e.g., VAL server or AIOTEC) sends an AIoT information provisioning delete request to the AIOTES. The request includes the requestor identifier and a list of identifiers of application AIoT device profiles as described in clause 7.15.3.5.

2.	Upon receiving the request from the requestor, the AIOTES validates if the requestor is authorized for the request. If the requestor is authorized, the AIOTES deletes the application AIoT device profiles information identified via their identifier.

3.	The AIOTES sends an AIoT information provisioning response to the requestor as described in clause 7.15.3.6. If the request was authorized, the response includes an indication of success for the request and the identifier of application AIoT device profiles that were successfully deleted. Otherwise, the response includes an indication of failure and may include a failure cause.

### 7.15.2	Architecture Impacts

This solution proposes a functional entity, as an AIoT Enablement Server (AIOTES), is supported in the application enablement layer as described in Solution #1 and Solution #2.

### 7.15.3	Corresponding APIs

#### 7.15.3.1	AIoT Information Provisioning request

Table 7.15.3.1-1 describes the AIoT information provisioning request.

Table 7.15.3.1-1: AIoT Information Provisioning request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| List of application AIoT device profiles | M | The list of application AIoT device profile to be provisioned as described in Table 7.15.3.1-2. |

Table 7.15.3.1-2: Application AIoT device profile

| Information element | Status | Description |
| --- | --- | --- |
| Device identifier | M | The identifier of an AIoT device which is used for interactions with the network as described in 3GPP TS 23.369 [3]. |
| Device type | M | The type of the AIoT device (e.g., smart-tag, sensor, etc.). |
| Device capabilities | M | The capabilities of the AIoT device. |
| > Supported operations | M | The operation types supported by the AIoT device (e.g., Inventory, Command) as described in 3GPP TS 23.369 [3]. For Command operation, information about the supported command parameters and the command response size is included. |
| > Maximum rate | O | The maximum rate at which operations can be sent to the AIoT device. This information indicates the maximum rate for requesting operations to the AIoT device and may indicate the energy replenishment time between operations. |
| > Maximum transmission size | O | The maximum amount of data that the AIoT device is capable of transmitting in one operation. This information indicates whether multiple operations may be required to obtain information from the AIoT device. |
| Area information | O | The area information associated with the AIoT device. |
| Device status | M | The status (e.g., enabled, disabled) of the AIoT device. |

#### 7.15.3.2	AIoT Information Provisioning response

Table 7.15.3.2-1 describes the AIoT information provisioning response.

Table 7.15.3.2-1: AIoT Information Provisioning response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | The indication that the request was successful. |
| > List of device identifiers | M | The list of device identifiers (application AIoT device profiles) that were provisioned. |
| Failure response | O (NOTE) | The indication that the request failed. |
| > Cause | O | The failure cause |
| NOTE: Only one of these information elements shall be provided. |  |  |

#### 7.15.3.3	AIoT Information Provisioning Update request

Table 7.15.3.3-1 describes the AIoT information provisioning update request.

Table 7.15.3.3-1: AIoT Information Provisioning Update request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| List of application AIoT device profiles | M | The list of application AIoT device profile to be updated as described in Table 7.15.3.1-2. |

#### 7.15.3.4	AIoT Information Provisioning Update response

Table 7.15.3.4-1 describes the AIoT information provisioning update response.

Table 7.15.3.4-1: AIoT Information Provisioning Update response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | The indication that the request was successful. |
| > List of device identifiers | M | The list of device identifiers (application AIoT device profiles) that were updated. |
| Failure response | O (NOTE) | The indication that the request failed. |
| > Cause | O | The failure cause |
| NOTE: Only one of these information elements shall be provided. |  |  |

#### 7.15.3.5	AIoT Information Provisioning Delete request

Table 7.15.3.5-1 describes the AIoT information provisioning delete request.

Table 7.15.3.5-1: AIoT Information Provisioning Delete request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| List of device identifiers | M | The list of device identifiers (application AIoT device profiles) to be deleted. |

#### 7.15.3.6	AIoT Information Provisioning Delete response

Table 7.15.3.6-1 describes the AIoT information provisioning delete response.

Table 7.15.3.6-1: AIoT Information Provisioning Delete response

| Information element | Status | Description |
| --- | --- | --- |
| Successful response | O (NOTE) | The indication that the request was successful. |
| > List of device identifiers | M | The list of device identifiers (application AIoT device profiles) that were deleted. |
| Failure response | O (NOTE) | The indication that the request failed. |
| > Cause | O | The failure cause |
| NOTE: Only one of these information elements shall be provided. |  |  |

### 7.15.4	Solution evaluation

This solution addresses Key Issue #3 on provisioning and managing application AIoT device profiles provided by the VAL layer. The solution allows the VAL layer to provision and manage application AIoT device profiles with the AIoT enablement service (e.g., AIOTES) to enable AIoT operations with the network.

The provisioned information includes application AIoT device profile(s) and enables the AIoT enablement service (e.g., AIOTES) to perform operations (e.g., Inventory, Command) by considering capabilities associated with AIoT devices. The management operations described in the solution proposes procedures for provisioning, updating, and removing application AIoT device profiles.

The solution uses the capabilities of inventory and command operations as described in 3GPP TS 23.369 [3].

The solution is compatible with architectures described in Solution #1 and Solution #2.

# 8	Overall evaluation

## 8.1	General

This clause will provide a general description of the overall evaluation.

## 8.2	Overall evaluation of key issue #1

The KI#1 mainly studies the following open issue:

-	Whether and how to enhance the application enablement architecture /functional model or new application enabler to support the Ambient IoT services (i.e. AIoT inventory and command services).

To address the open issues aforementioned, there are 3 solutions proposed as following:

Solution #1 introduces a new architecture and functional model in the application enablement layer to support the AIoT services. Specifically, this solution introduces the on-network functional model, the relevant functional entities (including the AIoT management server, the AIoT management client, the VAL server, the VAL client, etc.) and the reference points included in the new architecture for the AIoT services.

To align with the system architecture for AIoT services specified in clause 4.2 of 3GPP TS 23.369 [3], the AIoT management client in the new architecture for the application enablement layer only acts as a service consumer which could request the AIoT services to the AIoT management server over AM-UU reference point, but it will not act as a UE reader to obtain and report the AIoT device information. And the off-network functional model for AIoT application enablement architecture is not supported for Sol#1.

Solution #2 also introduces a new architecture and functional model in the application enablement layer to support the AIoT services. The main difference between Sol#1 and Sol#2 is there is neither VAL client nor AIoT client in Sol#2 to provide the application enabler layer support for AIoT services from the client side. And the application enablement architecture layer can only be enhanced for AIoT service at the server side which acts as a SEAL layer function entity. Besides, the off-network functional model for AIoT application enablement architecture is not supported for Sol#2.

Solution #14 also introduces a new architecture and functional model in the application enablement layer to support the AIoT services. The solution highlights the AIoT services entities should be part of Application enabler layer and not the services layer. And it also introduces the on-network functional model, the relevant functional entities (including the AIoTE server, the AIoTE client, the VAL/SEAL server, the VAL/SEAL client, etc.) and the reference points included in the new architecture for the AIoT services.

Editor’s note: The overall evaluation for the solutions may be updated when the relevant solution updates.

## 8.3	Overall evaluation of key issue #2

This clause will provide an overall evaluation for KI#2.

## 8.4	Overall evaluation of key issue #3

This clause will provide an overall evaluation for KI#3.

## 8.5	Overall evaluation of key issue #4

This clause will provide an overall evaluation for KI#4.

# 9	Conclusions

## 9.1	General conclusions

This clause will provide general conclusions for the study.

## 9.2	Conclusions of key issue #x

This clause will provide conclusions for the specific key issue.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-08 | SA6#68 |  |  |  |  | Skeleton | 0.0.0 |
| 2025-08 | SA6#68 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-253714，S6-253636，S6-253751，S6-253638，S6-253639，S6-253752. | 0.1.0 |
| 2025-10 | SA6#69 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-254523，S6-254525，S6-254526，S6-254527，S6-254664，S6-254667, S6-254669, S6-254670, S6-254725, S6-254747, S6-254748, S6-254749, S6-254750, S6-254751, S6-254770. | 0.2.0 |
| 2025-11 | SA6#70 |  |  |  |  | Implementation of the following pCRs approved by SA6: S6-255139，S6-255534, S6-255540, S6-255541, S6-255542, S6-255543, S6-255545, S6-255563, S6-255564, S6-255613, S6-255622, S6-255624, S6-255626, S6-255627, S6-255638, S6-255639, S6-255657, S6-255658. | 0.3.0 |