| 3GPP TR 23.700-42 V0.3.0 (2025-11) |  |
| --- | --- |
| Technical Report |  |
| 3rd Generation Partnership Project; Technical Specification Group Services and System Aspects; Study on Application user consent; (Release 20) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet https://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2025, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	6

Introduction	7

1	Scope	8

2	References	8

3	Definitions of terms, symbols and abbreviations	9

3.1	Terms	9

3.2	Symbols	9

3.3	Abbreviations	9

4	Use Cases	9

4.1 	General	9

4.2	Use Case #1: End-to-end basic scenario	10

4.2.1	Use Case Description	10

4.2.2	Use Case Analysis	10

4.3	Use Case #2: Add app-user consent	11

4.3.1	Use Case Description	11

4.3.2	Use Case Analysis	11

4.4	Use Case #3: Update app-user consent (includes revoke)	11

4.4.1	Use Case Description	11

4.4.2	Use Case Analysis	12

4.5	Use Case #4: App-user consent handled by exposure platform	12

4.5.1	General	12

4.5.1	Use Case Description	12

4.5.2	Use Case Analysis	13

5	Business Relationships	13

5.1	General	13

5.2	Business relationship: Federation model	14

5.2.1	Description	14

5.3 	Business relationship: Aggregator model	15

5.3.1	Description	15

6	Terminology	15

6.1	Background	15

6.2	Related industry specifications	15

6.2.1	General	15

6.2.2	Industry group #1: GSMA OPG	16

6.2.3	Industry group #2: CAMARA	16

6.2.4	Industry group #3: 3GPP	16

6.3	Analysis	17

7	Key issues	18

7.1	Key issue #1:  Application user consent management	18

7.1.1	Description	18

7.1.2	Open issues	18

7.2	Key issue #2:  E2E clarification of the consent needed for applications	18

7.2.1	Description	18

7.2.2	Open issues	19

8	Solutions	19

8.1	Mapping of solutions to key issues	19

8.1	Solution #1: Standalone Application user consent management function	19

8.1.1	General	19

8.1.2	Architecture impacts	20

8.1.3	Solution description	21

8.1.3.1	General	21

8.x.3.2	Information flows	21

8.x.4	Solution evaluation	21

8.2	Solution #2:  End to end consent for applications	21

8.2.1	General	21

8.2.2	Architecture  impacts	22

8.2.3	Solution description	22

9	Overall evaluation	23

9.x	Evaluation of key issue #x	23

10	Conclusions	23

10.1 	Architectural conclusions	23

10.2	Key issues conclusions	23

10.2.x	Conclusions of key issue #x	23

11	Recommended requirements	23

11.1 	General	23

11.2	Description	23

Annex A (informative): Change history	24

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall	indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should	indicates a recommendation to do something

should not	indicates a recommendation not to do something

may	indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can	indicates that something is possible

cannot	indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will	indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not	indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# Introduction

The application user application user consent provides the capability to support application enablement scenarios, aiming, among other objectives, to ensure regulatory compliance across interoperable exposure ecosystems. The key issues, derived from analysis of use cases and existing market models, are analysed in this TR with the aim of proposing requirements and identifying interoperable solutions for handling application user consent considering architecture modularity and alignment with other industry specifications (like GSMA).

# 1	Scope

The present document is a technical report which analyses and proposes solutions for the application user (app-user) consent.

The study work is performed in a phased approach. In the first phase, the study identifies and analyse the use cases, it clarifies the related terminology, analyses related industry solutions (e.g., GSMA OPG, CAMARA), it explores the relevant business relationships with a potential impact on the app-user consent end to end solutions.

In the second phase, the technical report provides an analysis of the app-user consent use cases, identifies the key issues, gaps and overlaps for an end-to-end app-user consent 3GPP solution. It covers different possible scenarios with user service experience impacts (up-front, in-flow, etc.), it provides solutions to the identified key issues, evaluates them and concludes on recommended solutions, as well as proposing requirements.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	GSMA PRD OPG.02, "Operator Platform: Requirements and Architecture".

[3]	CAMARA API "Access and Consent Management", IdentityAndConsentManagement/documentation/CAMARA-API-access-and-user-consent.md at r3.3 · camaraproject/IdentityAndConsentManagement · GitHub.

[4]	3GPP TS 23.501: "System architecture for the 5G System (5GS)"

[5]	3GPP TS 23.502: "Procedures for the 5G System (5GS)"

[6]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS)"

[7]	3GPP TS 23.271: "Functional stage 2 description of Location Services (LCS)"

[8]	3GPP TS 23.288: " Architecture enhancements for 5G System (5GS) to support network data analytics services"

[9]	3GPP TS 33.501: " Security architecture and procedures for 5G system "

[10]	3GPP TS 33.558: "Security aspects of enhancement of support for enabling edge applications"

[11]	3GPP TS 29.503: "5G System; Unified Data Management Services"

[12]	3GPP TS 23.558: "Architecture for enabling Edge Applications"

[13]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs"

[14] 	3GPP TS 33.122: " Security aspects of Common API Framework (CAPIF) for 3GPP northbound APIs"

[15]	“General Data Protection Regulation (GDPR)”, https://gdpr.eu/tag/gdpr/

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".

# 3	Definitions of terms, symbols and abbreviations

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Application user consent: The agreement of a user/subscriber provided to the API Provider, to allow the API Provider to expose the user’s personal data (subjected to regulations) to the application.

NOTE: The subscriber can be the end-user.

Editor Note: The terms used in the definition user/subscriber, personal data need further clarification.

Exposure platform: implementation consisting of a set of functions supporting a controlled exposure to API consumers of certain underlying capabilities offered by API Provider(s).

NOTE: An exposure platform can be developed leveraging a standardized exposure framework (e.g., CAPIF CCF).

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

AEF	API Exposing Function

ASP	Application Service Provider

CSP	Communications Service Provider

E-W	East West

# 4	Use Cases

## 4.1 	General

This clause describes relevant use cases where different aspects of app-user consent need to be considered. 
It is assumed in all cases that an exposure platform provider is aware of the regulation, and therefore it has internal knowledge (via internal policies, provisioning, etc) which applications and possibly also purposes, require checks on app-user consent data, as part of the overall application authorization process.

## 4.2	Use Case #1: End-to-end basic scenario

### 4.2.1	Use Case Description

Preconditions:

- The API provider (PLMN A) is the owner and manager of Alice’s subscription in the API Provider (PLMN A) domain.

- Alice is an app-user (e.g., for Netflix).

- The API provider (PLMN A) has a service agreement with the provider B of the Exposure Platform.

- Netflix application has an agreement with the Exposure Platform provider B,

- Provider A has an app-user consent management solution for the users that have a subscription with the API Provider, including Alice.

Summary:

An application (e.g., Netflix) requests to access some of the app-user data such as for example location, to determine if the app-user Alice, who is accessing the Netflix application, is allowed to use the Netflix service at her current location.

Description:

- The use case implies an app-user consent check for this specific application context: Application: e.g., Netflix, stated purpose(s): e.g., confirm conditions for providing the service, and the app-user data requested: e.g., app-user’s location data from API Provider domain.

- Assuming the basic scenario, i.e., the app-user consent was already provided by Alice for the above application context, then the response from the Consent Manager of API Provider of Alice (PLMN A) to the Exposure Platform is positive.

- The Exposure Platform provides authorization to Netflix application request, which the application uses further in its service request to the API Provider (PLMN A) to obtain app-user’s location data.

- Once the API Provider (PLMN A) receives the Service Request for Alice’s location data, it can validate with the Exposure Platform of provider B the received authorization data from the Netflix application

- The API Provider (PLMN A) processes the request, but as internal requests in PLMN A’s Core Network get performed in view of obtaining the app-user’s location data, it can be determined (via mechanisms outside the scope of this specification) if a check on the PLMN A’s user consent is required in order to provide the location data between NFs of the Core Network.

### 4.2.2	Use Case Analysis

As the Exposure platform and the Consent management entity belong to different domains, the interactions for the app-user consent check between the Exposure platform and the Consent management entity require an interoperable, standard interaction.

The app-user consent enabler needs to expose the capability to perform consent check for a given application, stated purpose and the indicated user data.

There can be different scopes for the user consent: the app-user consent for specific user data exposure to applications, and the user consent intended for internal consumption of data by Core Network NFs.

The application user consent capture is defined under the "Consent Capture" term in GSMA PRD OPG.02 [2] and is not the subject of this study.

## 4.3	Use Case #2: Add app-user consent

### 4.3.1	Use Case Description

Preconditions:

- The API provider (PLMN A) is the owner and manager of Alice’s subscription in the API Provider (PLMN A) domain.

- Alice is an app-user for several applications.

- The API provider (PLMN A) has a service agreement with the provider B of the Exposure Platform.

- The ASPs for the applications used by Alice have an agreement with the Exposure Platform provider B.

- API Provider (PLMN A) has an app-user Consent management solution for the users that have a subscription with the API Provider, including Alice. The Consent management solution can be inside or outside the API Provider (PLMN A) domain.

- There is no app-user consent data for Alice in the Consent management of API Provider (PLMN A) for an application that Alice wants to use.

Summary:

Alice was prompted to provide her app-user consent to the API Provider (PLMN A) and she has provided the data. 
The Consent Management entity receives the app-user consent data for Alice and for the specific application and persists it for further use.

Description:

- The Consent Capture methods are out of scope.

- The app-user consent data can vary depending on the regional regulations which govern the API Provider (PLMN A) and Alice’s subscription.

- Once Alice has provided her app-user consent data, the Consent Management entity receives the request to add the app-user consent data for the specific app.

- The app-user consent is provided by Alice per application, and optionally subject to API Provider’s policies, also some context for the app-user consent, such as purpose of use by the application and per Alice’s user data to be exposed from API Provider (PLMN A) domain.

- Once the Consent Management entity receives the app-user consent data for Alice, it persists it for further use and until Alice decides to change her app-user consent for that application.

### 4.3.2	Use Case Analysis

The Consent Management entity has to have the capability to receive for each app-user, their app-user consent data per application, and additional app-user consent context data such as purpose and the user data that is allowed to be exposed to the application.

Consent Management entity has the capability to persist each user’s app-user consent data, until such time when app-user decides to change their current consent data for an application (update of app-user consent data is handled in clause 4.4).

The application user consent capture is defined under the "Consent Capture" term in GSMA PRD OPG.02 [2] and is not the subject of this study.

## 4.4	Use Case #3: Update app-user consent (includes revoke)

### 4.4.1	Use Case Description

Preconditions:

- The API provider (PLMN A) is the owner and manager of Alice’s subscription in the API Provider (PLMN A) domain.

- Alice is an app-user for several applications.

- The API provider (PLMN A) has a service agreement with the provider B of the Exposure Platform.

- The ASP for the applications used by Alice have an agreement with the Exposure Platform provider B.

- API Provider (PLMN A) has an app-user consent management solution for the users that have a subscription with the API Provider, including Alice.

- The app-user consent data for Alice was provisioned in the Consent management entity of API Provider (PLMN A) for an application X that Alice wants to use.

Summary:

Alice decided to change her app-user consent for application X. 
The Consent Management entity receives the updates to the app-user consent data for Alice and for the specific application X and persists it.

Description:

- The Consent Capture methods are out of scope. 
The app-user consent data can vary depending on the regional regulations which govern the API Provider (PLMN A) and Alice’s subscription.

- Once Alice has provided the updates to her app-user consent data for application X, the Consent Management entity receives the updated data and performs the updates of the app-user consent data for the specific app X accordingly. The updates to Alice’s app-user consent data for application X can change the existing data to either provide access, or to revoke it.

- Once the Consent Management entity receives the updated app-user consent data for Alice, it persists it for further use and until Alice decides to change her app-user consent for that application.

### 4.4.2	Use Case Analysis

The Consent Management enabler has to be able to receive updates of the app-user consent data at any time, whenever the app-user decides to change their current consent data for an application.

The application user consent capture is defined under the “Consent Capture” term in GSMA PRD OPG.02 [2] and is not the subject of this study.

## 4.5	Use Case #4: App-user consent handled by exposure platform

### 4.5.1	General

This section describes use case where the applications require app-user consent and the app-user consent is handled by exposure platform.

### 4.5.1	Use Case Description

This use case illustrates the case of app-user consent data delegated to service providers like exposure platforms. When the API provider is responsible for the user data shared via the exposed APIs, the API provider can delegate the app-user consent capture and management to trusted exposure platforms. The exposure platform provider that exposes the services of the API provider will determine the authorization of the service API invocation based on the app-user consent information available at exposure platform.

An example can be, exposure platforms leveraging CAPIF, can capture and maintain for their API invokers, the app-user consent data for access to specific user data from the API provider, including MSISDN, location data and history, etc.

Certain API providers also expose service APIs where there is dependency on other consents (e.g., from Core network). So, for a given service API exposure, app-user consent and consent from core network might need to be considered for fulfilment of the service API request.

For example, there are multiple such use cases for NEF service exposure, where multiple consents (CAPIF RNAA and consent information from core network) are specified as part of the application authorization flow and then again as part of the API invocation.

An example is the case of EDGEAPP, where for a service invocation (such as UE location information), the EES is expected to check the user consent in UDM for the EDGEAPP purpose. Upon service invocation to NEF, the NEF also is expected to check another value of the user consent in UDM for the NW_CAP_EXPOSURE purpose. If CAPIF RNAA would be used to authorize the access of the EAS or the EES (depending on the case) to the UE location information API exposed by the NEF, then the RO authorization for that EAS/EES is also checked in the Authorization Function (of the CCF) for the specific edge application.

The use case needs to be studied further:

- The flows for the app-user consent checks integrated with exposure platform need to be further clarified, for the different consent scopes and purposes to analyse if any gaps exist as part of the application enablement and exposure platform leveraging flows.

### 4.5.2	Use Case Analysis

Study the gaps, where multiple consent information are required for exposure to applications (e.g., EDGEAPP case) and how this can be addressed with one or both of CAPIF RNAA (using RO authorization for EDGEAPP) and/or with the application user consent management, consent in UDM.

Editor’s Note: The consent in UDM is subject for further coordination with SA3 and SA2.

# 5	Business Relationships

## 5.1	General

The business relationships between main exposure actors impact on the approach and solutions for handling the application user consent (app-user consent) is not determined. Typical business relationships models used by service and capabilities providers that are considered in the app-user consent analysis are:

- Federation model,

- Aggregator model.

Some common aspects of the federation and aggregator models are:

- allowing an Application Service Provider to use one common interface to access capabilities exposed by multiple service and capabilities providers, subject to an agreement between the ASP and the service and capabilities providers involved,

- providing continuation of use, for application purposes, of the services enabled by the service and capabilities providers, when UEs are moving between mobile networks.

The general assumption is that the ownership and management responsibility for the app-user consent data belongs to the API Provider that owns the subscription under which the user consumes services in the API provider domain. 
The business relationships between different exposure partners and the open issues related to their implications on app-user consent are described in the next clauses.

While the exposure platforms and the API Provider can belong to the same or different PLMN domains, the exposure platforms can leverage standardized exposure frameworks (e.g., CAPIF).

Editor’s note: The business models may be updated based on use cases and their main actors.

## 5.2	Business relationship: Federation model

### 5.2.1	Description

The federation business model enables different service and capabilities providers to efficiently leverage exposure platforms from other service and capabilities providers (API providers), establishing an exposure ecosystem partnership.

In the federated model described in GSMA PRD OPG.02 [2], an exposure platform (such as the Operator Platform (OP)):

- facilitates access to the edge networks and other capabilities of an API Provider (PLMN operator), or federation of PLMN operators and exposure partners (per clause 1.1.3 in GSMA PRD OPG.02 [2]),

- allows one exposure partner to access services offered by another exposure partner (subject to a federation agreement between the exposure partners), on behalf of the Application Service Providers that it serves (per clause 2.2.5 in GSMA PRD OPG.02 [2]).

- enables continuation of a UE’s use of the services and capabilities exposed via the exposure platform, such as when moving into a visited PLMN (per clause 2.2.5 in GSMA PRD OPG.02 [2]).

There are different cases of federation, such as:

- federated platforms interworking in an East-West approach (CAPIF interconnect is an example). This case is depicted in figure 4.2-2 below.

Figure 5.2-1: Business relationships in Federation model B (interconnect)

Editor’s Note: Additional federation business relationships to be considered.

## 5.3 	Business relationship: Aggregator model

### 5.3.1	Description

ASPs target simplification not only at API level, but also in eliminating the need to implement variations and customizations to multiple exposure platform (e.g., Operator Platform (OP)) providers and to API providers, as well as to establishing many individual business relationships. 
To simplify their business relationships model, the ASPs establish a business relationship with an aggregator, and in turn the aggregator sets up all the necessary individual business relationships with multiple exposure platform providers and API providers (e.g., PLMN operators). 
In this model, the ASP reaches applications users across many API providers and countries, transparently, via the aggregator. The exposure platform providers need to provide enough information to the aggregator, so that the aggregator can identify the right home PLMN for a user, as well as a target network e.g., when roaming.

Figure 5.3-1: Business relationships in Aggregator model

An exposure platform provider, same as the API provider, can establish business relationships with multiple aggregators to widen their exposure and reach to more ASPs.

# 6	Terminology

## 6.1	Background

## 6.2	Related industry specifications

### 6.2.1	General

Various specifications tackling the consent obtained or provided by a subscriber have used specific terminology to refer to the subscriber consent, provided to a network service provider. This clause collects the different terms used and studies the differences in meaning, with the goal to clarify what the terms refer to in their industry documentation contexts.

### 6.2.2	Industry group #1: GSMA OPG

The GSMA PRD OPG.02 [2] defines the term "Consent" as "The agreement of a Subscriber to allow the usage of their personal data. This agreement can be revoked at any time", with the clarification in Annex F "Privacy Management considerations" that the terms "Consent" and "Application-related Consent" are interchangeable in the PRD.

- In clause 4.3.6.1.3 "Managing Consent" of GSMA PRD OPG.02 [2] the Operator Platform (OP) shall be able to "receive notifications when a Subscriber revokes Consent via NBI". This implies that some capabilities are expected, such as providing subscription and notification mechanisms for app-user consent data changes.

- The process of Consent Capture is also described in GSMA PRD OPG.02 [2], which specifies that the mechanism for interacting with the end-user for consent capture are left to the CSP.

### 6.2.3	Industry group #2: CAMARA

The CAMARA API Access and User Consent Management have been published with following terms in the API documentation:

- "Consent: an explicit opt-in action that the User takes to allow processing of personal data. Consent grants a Legal Entity (e.g. the Operator or ASP) access to a set of Scopes related to the Resource Owner, for a specific Purpose. "

- "Purpose: The reason for which Personal Data will be processed by an Application. For example, an Application might want to create a movie recommendation for an End-User using their Personal Data, such as age or gender. CAMARA defines a standard set of Purposes which can be used by Applications to specify the reason for their intended Personal Data processing. "

- "Scope: the OpenID Connect scope which maps one or more protected resources, some scopes may require processing of Personal Data."

There are several distinct procedure flows for the consent management interactions in CAMARA API "Access and User Consent Management" [3], all describing interface interactions between two entities called "API Explosure Platform" and the "Consent Master" to allow the "API Explosure Platform" to perform a "check consent" with the "Consent Master".

While the term definition for Consent is generically worded and it does not explicitly state its focus on applications, the flows depict always an Application making the authentication and authorization requests.  Based on this, it can be inferred that the scope of the CAMARA API "Access and User Consent Management” apply to the user consent provided for applications.

Based on the flow interactions, the "check consent" is a required operation to be supported.

### 6.2.4	Industry group #3: 3GPP

No term definition for user consent, or for consent, was found in the 3GPP specifications studied.

In SA3, there are two related terms that are used for different purposes:

- User consent:

  - covers Core Network data shared to NFs or to the Edge applications.

  - Requirements and procedures are defined in Annex V of 3GPP TS 33.501 [9]. User consent data is managed and stored in UDM.

  - Requirements are defined on the NFs that are an enforcement point for the user consent in Annex V.3 of [9], as well as additional requirements on NWDAF in Annex X.7 of [9].

  - For edge applications, 3GPP TS 33.558 [10] reuses the same procedures by referencing 3GPP TS 33.501 [9] (Annex V). Either EES or the NEF act as the consent enforcing entity of the user consent for the edge applications.

- RO authorization:

  - has a definition in 3GPP TS 33.122 [14] in context of RNAA:
"Resource owner authorization: The permission provided by the resource owner to allow the API invoker to access the resource owner’s resource via the northbound API."

  - It covers the resource owner (subscriber) agreement, to expose over northbound APIs, specific user data from API Provider domain to applications.

Several SA2 specifications mention user consent, but no definition of the term is provided. In the procedures, the user consent data is in UDM/UDR and is accessed and enforced by Core Network NFs: EIF in 3GPP TS 23.501 [4], NEF in 3GPP TS 23.502 [5], LMF, NWDAF in 3GPP TS 23.288 [8], and Location functions in 3GPP TS 23.273 [6] and 3GPP TS 23.271 [7].

The CT4 specification 3GPP TS 29.503 [11] defined in clause 6.1.6.3.20 “Enumeration: UcPurpose” the user consent ennumeration values that are stored in UDM, which can be consumed by e.g., NFs and trusted AF:

- "ANALYTICS": User consent for analytics

- "MODEL_TRAINING": User consent for model training

- "NW_CAP_EXPOSURE": User consent for network capability exposure.

- "EDGEAPP_UE_LOCATION": User consent for the manipulation of UE information for the purpose of UE Location retrieval by the EDGEAPP EAS entity.

SA6 specifications have introduced user consent aspects in:

- CAPIF 3GPP TS 23.222 [13] under RNAA, which includes the user consent for applications under the term RO authorization defined in 3GPP TS 33.122 [14]. 
Also in CAPIF 3GPP TS 23.222 [13] it is stated in clause 6.2.3. in NOTE 4:
"The authorization information from the resource owner used by CCF (described in 3GPP TS 33.122 "[14] ") is independent from the user consent information used from user subscription data at UDM/UDR (described in Annex V of 3GPP TS 33.501"[9] "). In the current release of 3GPP specifications, no synergy between CCF and UDM is specified."

- EDGEAPP 3GPP TS 23.558 [12] has user consent for edge applications, referencing 3GPP TS 33.558 [10] and 3GPP TS 33.501 [9] (Annex V). A summary of the user consent for edge applications is provided in Table 6.2-2 below.

For reference, a summary of the user consent purpose and specifications indicating consent handling in the above 3GPP SA6 EDGEAPP specification is in Table 6.2-1 below.

Table 6.2-1: SA6 EDGEAPP user consent summary

| User Consent purpose (per 3GPP TS 23.558 [12]) | User Data (subjected to consent) | Enforcer | User Consent management |
| --- | --- | --- | --- |
| Generally per each AF/ app | UE Location - clause 8.14.2.3 | LMF, NEF (via enforcing the OAuth token scope) | UDM (referred in SA3 3GPP TS 33.558 [10] and 3GPP TS 33.501 [9]) |
| EDGEAPP (per app/AF) | GPSI (MSISDN) - clause 7.2.6 | EES (clauses 6.3.2; 8.5.2.2) | UDM |
| EDGEAPP (per app/AF) | UE tunnel e2e info (security) – clause 8.3.3.2.2 | EES (clauses 6.3.2; 8.5.2.2) | UDM |
| EDGEAPP (per app/AF) | AF specific UE identifier (MSISDN) – clause 8.6.5.2 | EES (clauses 6.3.2; 8.5.2.2) | UDM |

## 6.3	Analysis

There are some differences in the terminology used in GSMA PRD OPG.02 [2] captured in clause 6.2.2, CAMARA [3] captured in clause 6.2.3 and in 3GPP captured in clause 6.2.4, and the assumptions used.

While in 3GPP the user consent and RO authorization are considered forms of user authorization, in GSMA OPG and CAMARA the term user consent is considered an agreement between two parties: the subscriber and the Service API provider.

The approaches have also some differences, as in GSMA OPG and CAMARA the Consent Manager/Master is a standalone enabler under the responsibility of the Service provider, while in 3GPP the user consent for applications is inconsistently handled either in UDM/UDR in a global manner (undistinguishing between different applications), or is embedded inside the CAPIF’s Authorization Function (in CAPIF provider’s domain).

In conclusion, the “user consent” in 3GPP does not cover consistently and completely the application purposed user consent. The “RO authorization” term is connected to the authorization procedure over northbound APIs and is only used when CAPIF is used. RO authorization is explicit permission given by the resource owner.

# 7	Key issues

## 7.1	Key issue #1:  Application user consent management

### 7.1.1	Description

This KI brings up the gap of the missing architecture for the application user consent management. Based on the use cases and business relationships studied in the present document, there are certain aspects that need to be studied to fulfil an architecture for the application user consent.

According to different regional regulations, for sharing some of the user data to applications, it is required to obtain the user’s agreement provided to a data controller (e.g., PLMN operator). One of the most commonly referenced, the GDRP [15] indicates that the user consent must be obtained per application targetted for user data sharing, per purpose of the application and per user’s data that is being shared. 
In addition to the regulations, it is acknowledged that there is operational data that needs to be maintained by the data controllers (e.g., PLMN operator). The Annex G.4.2 of the GSMA PRD OPG.02 [2] provides a description of these data records, which can be used as reference. 
Per some of the regional regulations (e.g., GDPR) the application user consent solutions must allow the user to update the application user consent data (change his/her mind) at any time. 
The solution to this KI will need to handle the architecture aspects including the considerations above.

### 7.1.2	Open issues

The architecture for application user consent management needs to be studied, considering aspects derived from the use cases and business relationships. These include gaps and open issues indicated in the use case analysis in clauses 4.2.2, 4.3.2 and 4.4.2.

## 7.2	Key issue #2:  E2E clarification of the consent needed for applications

### 7.2.1	Description

This KI proposes to study the end to end flow that clarifies what user consent is needed for applications.

Based on the use case in clause 4.2 "Use Case #1: End-to-end basic scenario", for an application to be able to access the application user’s data from the API Provider (e.g. PLMN operator), a check on the application user consent may be involved (depending on applicable regulations). In addition, inside the API Provider domain (e.g., PLMN operator) there may be another user consent check for the internal access of user data by Core Network entities. As some of the user consent checked internally in the Core Network is ambiguously connected to potential application related consent, clarity on the end to end flow is needed.

As there can be many types of applications, there is a need for clarification on the applicability of the application user consent:

- applications provided by the network (e.g., MNO, deployed within MNO domain) for the network (e.g., support with network analytics, network optimizations).

- applications that offer services to the end-users, including the case of applications used for addressing legal obligations (e.g., a person’s life protection).

### 7.2.2	Open issues

Clarifications of the end to end aspects related to what application user  consent and user consent checks are needed throughout E2E flow for exposure to applications, such as:

- Whether and how to fulfil the application authorization flow with application user consent check, and how it is enforced upon service invocations from an authorized application, as well as additional user consent checks in the Core Network (CN), for the consumption of the user data by CN Network Functions.

Study to determine criteria/principles for applicability of the application user consent.to different types of applications.

# 8	Solutions

## 8.1	Mapping of solutions to key issues

Table 6.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | … |
| --- | --- | --- | --- |
| Sol #1 | x |  |  |
| Sol #2 |  | x |  |
| Sol #... |  |  |  |

## 8.1	Solution #1: Standalone Application user consent management function

### 8.1.1	General

This solution relates to KI #1 on the architecture of application user consent management. The open issues to be studied from this KI are described in clause 7.1.2.

### 8.1.2	Architecture impacts

This solution proposes a standalone application user consent function. The following aspects, indicated in the use cases analysis in clauses 4.2.2, 4.3.2 and 4.4.2 and in the business relationships, are covered in this solution for the architecture of the application user consent management function:

- The exposure of interfaces to allow the consumers (e.g., Exposure platforms) to check the application user consent for an user, or to subscribe to be notified of application user consent data changes.

- The support for the management of the data needed for the application user consent, covering application user consent per application and additional data (subject to the applicable regional regulation, such as purpose and user data to be made available for user data scope).

  - While additional operational data is expected to be also recorded by the application user consent management, the extent and composition of operational data can vary based on regional and provider specific needs, and therefore is not studied and addressed in the present document. An example for reference is available in Annex G.4.2 of the GSMA PRD OPG.02 [2].

  - As the storage of the application user consent data and of any additional operational data are handled by the application user consent function, no specific interfaces need to be standardized for the actual storage as these are internal to the application user consent function.

- The support for operating immediately application user consent data changes, at any time the user chooses to change his/her application user consent for an application. This can include update/revocation of application user consent for the indicated application and application context (e.g., purposes), notifying the subscribed entities to the application user consent data chnages (so they can apply the change to the authorization result for ongoing and new application traffic for that user).

Figure 8.1.2-1: Standalone Application user consent management function

Editor’s Note: The architecture will be updated to address further use cases and updates to requirements.

The APCOT-1 is exposed by the Application user consent management function and supports application user consent check requests and responses, as well as subscribe/notify to application user consent data changes and queries for consent data. Consumers of APCOT-1 include Exposure platforms.

The APCOT-2 supports application user consent data management operations (application user consent create, updates/revocation, removal). The Consent capture solutions are specific to each data controllers (e.g., PLMN operator) and are not in scope of the study.

Editor’s Note: the security aspects for the APCOT-1 and APCOT-2 are handled by SA3.

### 8.1.3	Solution description

An example of interaction between the consumer of APCOT-1 interface (an Exposure platform) and the application user consent management function is described below.

Figure 8.1.3-1: Application user consent check over APCOT-1

1.	The Consumer (Exposure platform) requests the application user consent check for the specific user and application context information over APCOT-1.

2.	The Application user consent management function checks the available application user consent data for the received request to determine whether the consent was granted or not.

3.	The Application user consent management function provides the check result back to the Exposure platform, indicating if the application user consent was granted or not.

NOTE:     when the Exposure platform and the application user consent management function are collocated, the steps 1 and 3 are internal and do not require use of a standard interface (an example is CAPIF Authorization Function).

### 8.1.4	Solution evaluation

This section provides solution evaluation based on the open issues specified in the related key issue(s).

## 8.2	Solution #2:  End to end consent for applications

### 8.2.1	General

This solution relates to KI #2. on the E2E clarifications on user consent for applications. The open issues to be studied from this KI are described in clause 7.2.2.

### 8.2.2	Architecture impacts

There are no additional architecture impacts for this solution.

### 8.2.3	Solution description

As indicated in the end-to-end use case described in clause 4.2 and in KI#2, the overall consent needs for an application request may involve two different types of checks on user consent. This solution addresses clarifications of the end-to-end aspects related to what application user consent and user consent checks are needed for applications, such as:

- the application user consent check done in an application authorization flow, related to sharing of user data from the API provider (e.g., PLMN operator) to the application.

- upon the service invocation request from an authorized application:

  - part of the CN processing of the received northbound service invocation, additional user consent checks can be done in the Core Network (CN) for the internal consumption of the user data.

As per 3GPP TS 29.503 [11] there are two values defined in the CN (UDM) which appear related to exposure to applications, defined as follows:

- "NW_CAP_EXPOSURE": User consent for network capability exposure.

- "EDGEAPP_UE_LOCATION": User consent for the manipulation of UE information for the purpose of UE. Location retrieval by the EDGEAPP EAS entity.

Any needed application user consent for an application request is checked as part of the application authorization flow, in accordance with applicable regulatory requirements (e.g., per application, purpose, user data, etc). and it is a pre-requisite for a successful service invocation by the application. This aproach is aligned in both CAPIF RNAA and in GSMA OPG.

So once the application authorization including the application user consent check is successful, then when the respective service invocation is received by the CN from the authorized application, in this solution there is no need for a duplicate user consent check in the CN (UDM) for application purposes. In addition, the values for user consent purpose in UDM: "NW_CAP_EXPOSURE" and  "EDGEAPP_UE_LOCATION do not provide any distinction per application, per application purpose and shared user data, so they do not address the needs of the application user consent management. Therefore these two user consent purpose values in UDM are considered insufficient for the purposes of application-related user consent.

Since in the API Provider domain (e.g., PLMN operator) there is a 3GPP user consent solution specified in Annex V of TS 33.501 [9], which is ambiguously connected to potential application related consent for the applications deployed at the edge (EDGEAPP)  in 3GPP TS 33.558 [10], further clarity on the applicability of the EDGEAPP specific user consent solution, as well as on the end to end flow is needed.

This issue was also highlighted in 3GPP TS 23.222 [13] : “The authorization information from the resource owner used by CCF (described in 3GPP TS 33.122) is independent from the user consent information used from user subscription data at UDM/UDR (described in Annex V of 3GPP TS 33.501). In the current release of 3GPP specifications, synergy between CCF and UDM is not specified.”

NOTE: while these user consent purpose values in UDM: "NW_CAP_EXPOSURE" and "EDGEAPP_UE_LOCATION” are not necessary in this solution for application user consent purposes, they might apply in other scenarios and this needs to be assessed by the respective WGs (SA3, CT4).

The application user consent is not required for all applications. In some cases, there is no application user consent required such as the case when the service provided by the application is required by law (e.g., for a person’s life protection), or when it is covered by a general/subscription agreement with the API Provider (PLMN operator) for example apps for network optimizations, measurements. 
As a general principle, the application user consent is needed for an application access to user’s data when that user data and application purposes are subject to regulations.

### 8.x.4	Solution evaluation

This section provides solution evaluation based on the open issues specified in the related key issue(s).

## 8.x	Solution #x: <Solution Title>

### 8.x.1	General

This solution relates to KI #N.

This section describes the high-level principle of the solution.

### 8.x.2	Architecture requirements and impacts

This section describes any architectural requirements and architecture impacts based on the proposed solution.

### 8.x.1	Solution description

#### 8.x.1.1	General

This section describes the solution in detail.

#### 8.x.1.2	Information flows

This section describes the information flow tables required for this solution.

### 8.x.4	Solution evaluation

This section provides solution evaluation based on the open issues specified in the related key issue(s).

# 9	Overall evaluation

## 9.x	Evaluation of key issue #x

This section evaluates all solutions related to key issue #x.

# 10	Conclusions

## 10.1 	Architectural conclusions

This section provides conclusion and suggestion for normative work for the architecture options studied in this TR.

## 10.2	Key issues conclusions

## 10.2.x	Conclusions of key issue #x

This section provides conclusion and suggestion for normative work for the solutions related to key issue #x.

# 11	Recommended requirements

## 11.1 	General

Subject to applicable regional regulations, different type of data is required to be obtained from the user as part of user’s agreement provided to a data controller (e.g., PLMN operator) to share his/her data to applications.

## 11.2	Description

[APCOT-11.2-1] The Application user consent management shall expose an interface to support the application user consent check requests.

[APCOT-11.2-2] The Application user consent management shall expose an interface to support the subscription requests for application user consent data changes and shall send the due notifications to the subscribed entities.

[APCOT-11.2-3] The Application user consent management shall expose an interface to support the application user consent management operations (create, update/revoke, delete) on the application user consent data, at any time the user wishes to change his/her application user consent for an application.

[APCOT-11.2-4] The application user consent management function shall manage the application user consent provided to a data controller (e.g., PLMN operator). The application user consent is per application targeted for user data sharing, per purpose of the application and per user’s data that is being shared.

Editor’s Note: requirements will be updated during study conclusion phase.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | New version |
| 2025-09 | SA6#68 | S6-253383 |  |  |  | FS_APCOT_pCR_TR skeleton | v0.0.0 |
| 2025-09 | SA6#68 | S6-253384, S6-253385, S6-253142, S6-253772, S6-253773, S6-253774 |  |  |  | Incorporates approved pCRs: S6-253384, S6-253385, S6-253142, S6-253772, S6-253773, S6-253774 | v0.1.0 |
| 2025-10 | SA6#69 | S6-254127 S6-254757 S6-254697 S6-254698 S6-254699 S6-254777 S6-254701 S6-254702 S6-254703 S6-254383 |  |  |  | Incorporates approved pCRs: S6-254127, S6-254757, S6-254697, S6-254698, S6-254699, S6-254777, S6-254701, S6-254702, S6-254703, S6-254383 Editorial updates: - Formatting of ENs - Fixed numbering of phase 2 clauses - Replaced curly quotes | v.0.2.0 |
| 2025-10 | SA6#69 | Editorial |  |  |  | Editorial fixes (incorporation of S6-254697) | v.0.2.1 |
| 2025-11 | SA#70 | S6-255659 S6-255663 S6-255572 S6-255690 S6-255691 |  |  |  | Incorporates approved pCRs: S6-255659, S6-255663, S6-255572, S6-255690, S6-255691 | v.0.3.0 |