3GPP TR 29.949 V0.3.11 (2014-022)

Technical Report

3rd Generation Partnership Project;

Technical Specification Group Core Network and Terminals;

Study on Technical aspects on Roaming End-to-end scenarios with VoLTE IMS and other networks;

(Release 12)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP.
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Report is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

<keyword[, keyword]>

MCC selects keywords from stock list.

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2013, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	4

1	Scope	5

2	References	5

3	Definitions, symbols and abbreviations	6

3.1	Definitions	6

3.2	Symbols	8

3.3	Abbreviations	8

4	Overview	9

4.1 	General	9

4.2	3GPP specifications relevant for roaming architecture and procedures	9

4.3	Consideration of SIP header fields and elements	10

4.3.1	Inter Operator Identifier	10

4.3.1.1	Type of IOI 's	10

4.3.1.2	Format of IOI's	10

4.3.1.3	Basic IMS roaming scenario without loopback	11

4.3.1.4	Originating visited network with loopback routeing scenario	12

4.3.2	Naming of URIs	14

4.3.2.1	General	14

4.3.2.2	Home Network Domain	14

4.3.2.3	Private user identity	14

4.3.2.4	Public user identity	14

4.3.2.4	Instance-ID	14

5	Description of Scenarios	15

5.1.1	The used values for the message example	15

5.1.1.1	IOI values	15

5.2.1	General	15

5.2.1.1	Registration via IC SIP proxies	15

5.3.1	General	21

5.3.2	Call flow	21

5.4	Originating roaming with loopback	27

5.4.1	General	27

5.4.2	Call Flow	27

5.5	Routeing when the originating and terminating user reside in the same home network	35

5.6	Routeing from originating home to terminating home network	35

5.7	Routeing from originating visited network to terminating home network	35

5.8	Routeing in terminating home network when user resides in this network	35

5.10	Insertion of service related media in the originating home network	36

5.10.1	General	36

5.10.2	Insertion of service related media in the originating home network with loopback	36

5.10.3	Insertion of service related media in the originating home network without loopback	39

5.11	PS to CS SRVCC access transfer scenarios	40

5.11.1	General	40

5.11.2	PS to CS SRVCC information transfer during registration	41

5.11.3	PS to CS SRVCC access transfer for a call in the active phase without loopback	48

5.11.4	PS to CS SRVCC access transfer for a call in the active phase with loopback	54

5.12	Invocation and configuration of services during roaming in a visited network	54

Annex A: Change history	57

Annex X	57

5.2.1.2	Exampe of an call traversing an IP transport network as transit	57

# Foreword

This Technical Report has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

# 1	Scope

The present document describes end-to-end use cases and signalling flows based on the roaming architecture for voice over IMS with local breakout. It summarizes the contents of existing 3GPP technical specifications which are needed for roaming use cases. The scenarios used cover the following use cases:

-	registration when roaming;

-	routeing from originating visited network to originating home network;

-	loopback routeing from originating home network to originating visited network;

-	routeing when the originating and terminating user reside in the same home network;

-	routeing from the originating home network to the terminating home network;

-	routeing from the originating visited network to the terminating home network;

-	routeing in terminating home network when user resides in this network;

-	routeing from terminating home network to the terminating visited network;

-	insertion of service related media in the originating home network;

-	routeing through transit networks; and

-	access transfer scenarios (SRVCC).

Editor's Note:	It is ffs if the call forwarding service invocation in a Call Deflection case will be considered.

Editor's Note: The call flows in this document are currently in a preliminary version available and needs extensive review.

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.002: "Network architecture".

[3]	3GPP TS 23.228: "IP multimedia subsystem; Stage 2".

[4]	3GPP TS 24.229: "IP multimedia call control protocol based on Session Initiation Protocol (SIP) and Session Description Protocol (SDP); Stage 3".

[5]	3GPP TS 29.079: "Optimal Media Routeing within the IP Multimedia Subsystem; Stage 3".

[6]	3GPP TS 29.165: "Inter-IMS Network to Network Interface (NNI)".

[7]	3GPP TR 23.850: "Study on roaming architecture for voice over IP Multimedia Subsystem (IMS) with local breakout" (Release 11).

[8]	GSMA IR.92: "IMS Profile for Voice and SMS".

[9]	IETF RFC 3261: "SIP: Session Initiation Protocol".

[10]	IETF RFC 1594: "FYI on Questions and Answers to Commonly asked "New Internet User" Questions".

[11]	3GPP TS 23.122: "Non-Access-Stratum (NAS) functions related to Mobile Station (MS) in idle mode".

[12]	3GPP TS 23.218: "IP Multimedia (IM) Session Handling; IM call model".

[13]	IETF RFC 3263: "Session Initiation Protocol (SIP): Locating SIP Servers".

[14]	3GPP TS 29.228: "IP Multimedia (IM) Subsystem Cx and Dx Interfaces; Signalling flows and message contents".

[15]	GSMA IR.88: "LTE Roaming Guidelines"

[16]	3GPP TS

23.003: " Numbering, addressing and identification"

[17]	IETF RFC 4282: "The Network Access Identifier".

[18]	IETF RFC 3966: "The tel URI for Telephone Numbers"

[19]	IETF RFC 4122: "A Universally Unique IDentifier (UUID) URN Namespace".

[20]	3GPP TS 24.237: "IP Multimedia Subsystem (IMS) Service Continuity; Stage 3".

[21]	IETF RFC 6809: "Mechanism to Indicate Support of Features and Capabilities in the Session Initiation Protocol (SIP)".

[22]	3GPP TS 24.628: "Common Basic Communication procedures using IP Multimedia (IM)".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1].

For the purposes of the present document, the following terms and definitions given in 3GPP TR 24.229 [4] apply:

Entry point: In the case that "border control concepts", as specified in 3GPP TS 23.228 [3], are to be applied in an IM CN subsystem, then these are to be provided by capabilities within the IBCF, and the IBCF acts as an entry point for this network (instead of the I-CSCF). In this case the IBCF and the I-CSCF can be co-located as a single physical node. If "border control concepts" are not applied, then the I-CSCF is considered as an entry point of a network. If the P-CSCF is in the home network, then the I-CSCF is considered as an entry point for this document.

Exit point: If operator preference requires the application of "border control concepts" as specified in 3GPP TS 23.228 [3], then these are to be provided by capabilities within the IBCF, and requests sent towards another network are routed via a local network exit point (IBCF), which will then forward the request to the other network (discovering the entry point if necessary).

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.228 [3] subclauses 3.1, 4.3.3.1, 4.3.6, 4.6, 4.13, 5.2, 5.4.12.1 and 5.10 apply:

Border control concepts

Geo-local service number

Home local service number

Implicit registration set

Interconnection Border Control Function (IBCF)

Interrogating-CSCF (I-CSCF)

IMS Application Level Gateway (IMS-ALG)

IMS application reference

IMS Application Reference Identifier (IARI)

IMS communication service

IMS Communication Service Identifier (ICSI)

Local service number

IP-Connectivity Access Network (IP-CAN)

Policy and Charging Rule Function (PCRF)

Private user identity

Proxy-CSCF (P-CSCF)

Public Service Identity (PSI)

Public user identity

Serving-CSCF (S-CSCF)

Statically pre-configured PSI

For the purposes of the present document, the following terms and definitions given in IETF RFC 1594 [10] apply.

Fully-Qualified Domain Name (FQDN)

For the purposes of the present document, the following terms and definitions given in IETF RFC 3261 [9] apply (unless otherwise specified see clause 6).

Client

Dialog

Final response

Header

Header field

Loose routeing

Method

Option-tag

Provisional response

Proxy, proxy server

Recursion

Redirect server

Registrar

Request

Response

Server

Session

(SIP) transaction

Stateful proxy

Stateless proxy

Status-code

Tag

Target Refresh Request

User agent client (UAC)

User agent server (UAS)

User agent (UA)

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.002 [2] subclause 4.1.1.1 and subclause 4a.7 apply:

Breakout Gateway Control Function (BGCF)

Call Session Control Function (CSCF)

Home Subscriber Server (HSS)

Location Retrieval Function (LRF)

Media Gateway Control Function (MGCF)

MSC Server enhanced for IMS centralized services

Multimedia Resource Function Controller (MRFC)

Multimedia Resource Function Processor (MRFP)

Subscription Locator Function (SLF)

For the purposes of the present document, the following terms and definitions given in 3GPP TS 23.218 [12] subclause 3.1 apply:

Filter criteria

Initial filter criteria

Initial request

Standalone transaction

Subsequent request

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

Symbol format (EW)

<symbol>	<Explanation>

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in 3GPP TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in 3GPP TR 21.905 [1].

AS	Application Server

ASAVP	Attribute-Value Pair

ATGW	Access Transfer Gateway

B2BUA	Back-to-Back User Agent

BGCF	Breakout Gateway Control Function

CdPty	Called Party

EATF	Emergency Access Transfer Function

E-CSCF,		Emergency CSCF

ENUM	E.164 telephone NUmber Mapping

FQDN	Fully Qualified Domain Name

GRX	GPRS Roaming eXchange

IARI	IMS Application Reference Identifier

IBCF	Interconnection Border Control Function

IC	Interconnection

ICID	IM CN subsystem Charging Identifier

I-CSCF	Interrogating CSCF

ICSI	IMS Communication Service Identifier

IMS	IP Multimedia core network Subsystem

IMSI	International Mobile Subscriber Identity

IM-SSF		IP Multimedia Service Switching Function

IN-SCP	Interligent Networl Service Control Point

IOI	Inter Operator Identifier

LIA	Location Information Answer

LIR	Location Information Request

LRF	Location Retrieval Function

MCC	Mobile Country Code

MMTEL 		Multimedia Telephony

MNC	Mobile Network Code

MOC	Mobile Originating Call

MRFC	Multimedia Resource Function Controller

MRFP	Multimedia Resource Function Processor

MTC	Mobile Terminating Call

OMR	Optimal Media Routeing

PCRF	Policy and Charging Rules Function

P-CSCF	Proxy CSCF

PS	Packed Switched

PSAP	Public Safety Answering Point

PSI	Public Service Identity

RTCP	Real-time Transport Control Protocol

SLF	Subscription Locator Function

SRVCC	Single Radio Voice Call Continuity

TDM	Time Division Multiplex

TRF	Transit

UA	User Agent

UAC	User Agent Client

UAS	User Agent Server

URN	Uniform Resource Name

VoLTE	Voice over LTE

# 4	Overview

Editor's note: This subclause describes an overview of the motivation and documentation within this TR.

## 4.1 	General

Editor's note: This subclause describes an introduction what the overview will provide.

## 4.2	3GPP specifications relevant for roaming architecture and procedures

This subclause describes which 3GPP specifications which are relevant for the implementation of VoLTE IMS roaming.

- 3GPP TR 23.850 [7] investigated solutions for the provision of voice over IMS in roaming scenarios that facilitate the realization of a charging model that replicates the principles of CS model. The conclusions of this study were baseline for the roaming implementation in the 3GPP specifications. This document has now informational character.

- 3GPP TS 23.002 [2] describes the common architecture and the related interfaces.

- 3GPP TS 23.228 [3] describes the IP multimedia subsystem. The relevant sections which describes the roaming relevant stage 2 requirements are subclause 4.2.3 (support of roaming users), subclause 4.15a (roaming architecture for voice over IMS with local breakout), subclause 5.6.1 ((MO#1) mobile origination, roaming), subclause 5.7.1 ((MT#1) mobile termination, roaming) and subclause 5.7.2a ((MT#3) Mobile termination, CS Domain roaming).

- 3GPP TS 24.229 [4] especially describes the SIP and SDP signalling procedures for the IMS. The whole document distinguishes within the text if the procedures are relevant for roaming or non roaming cases or generally valid. Main sections which are mentioned to be specific to roaming additions are subclause 7.7.1.16 (support of roaming architecture for voice over IMS with local breakout) and annex I (additional routeing capabilities in support of transit, roaming and interconnection traffics in IM CN subsystem).

- 3GPP TS 29.079 [5] describes the optimised media routing. Some roaming scenarios are described in subclause 5.7 (roaming architecture for voice over IMS with local breakout and in annex A different roaming scenarios with OMR are shown. Especially clause A.6 shows roaming case with loopback.

- 3GPP TS 29.165 [6] describes the behaviour at the Inter-IMS Network to Network Interface (II-NNI) consisting of Ici and Izi reference points between IMS networks in order to support end-to-end service interoperability. This interface is essential for roaming.

- GSMA IR.92 [8] describes the GSMA profile between UE and Network based on 3GPP specifications.

## 4.3	Consideration of SIP header fields and elements

### 4.3.1	Inter Operator Identifier

#### 4.3.1.1	Type of IOI 's

There are three types of IOI:

1.	type 1 IOI, between the visited network and the home network:

-	between the P-CSCF and the S-CSCF;;

-	between the S-CSCF and the TRF; ;

-	between the BGCF and the TRF;; and

-	between the MSC server and the I-CSCF;

2.	type 2 IOI, between originating network and the terminating network:

-	between the the S-CSCF and the S-CSCF,

-	between the S-CSCF and the MGCF when a call/session is terminated at the PSTN/PLMN,

-	between the MGCF and the S-CSCF of the home terminating network when a call/session is originated from the PSTN/PLMN

NOTE: This includes all scenarios with a PSI AS when accessed across I-CSCF

-	between the TRF and the S-CSCF.

-	between the E-CSCF and the MGCF or IBCF where the request is routed to a PSAP,

-	between the E-CSCF and S-CSCF or IBCF when receiving an emergency request; and

3.	type 3 IOI:

-	between the S-CSCF or I-CSCF of the home operator network and any AS;;

-	between E-CSCF and LRF;;

-	between E-CSCF and EATF; ;

-	between transit function and AS.

Editor's note:	Which type of IOI that shall be used between SCC AS and the ATCF is FFS.

#### 4.3.1.2	Format of IOI's

According to TS 24.229 [4] subclause 7.2A.5.2.1 the SIP entity generating type 1 and type 3 IOI values express the "orig-ioi" and "term-ioi" header field parameters in the format of a quoted string as specified in RFC 3455 [20] with a specific string prefix being "Type 1" and "Type 3" respectively to indicate the type of IOI. For the type 2 IOI, no string prefix is used. The receiving SIP entity does not perform syntactic checking of the contents of the IOI values (the IOI value is passed unmodified to charging entities).

Editor's Note: It has to be clarified if for transit-ioi also the type should be prefixed as for orig-ioi and term-ioi

Editor's Note: There is an ongoing discussion within SA5 with regard to the type 3 transit ioi. When it is applied and in which cases not. Specifically the AS attached to a transit function is issue to be considered.

The following table shows some examples how to format IOI's

Table 4.3.1.2-1 Examples for the format of IOI's

| Example | Type of IOI | Prefix of IOI value | Example |
| --- | --- | --- | --- |
| P-CSCF – IBCF | Type 1 | "Type 1" | "Type 1visited_A.net" |
| S-CSCF – IBCF (terminating path) | Type 2 | none | "home_A.net" |
| S-CSCF - AS | Type 3 | "Type 3" | "Type 3home_A.net" |
| Orig Path (roaming) IBCF - IBCF | FFS | FFS | FFS |
| Term Path (roaming) IBCF - IBCF | FFS | FFS | FFS |
| originating home network to terminating home network | Type 2 | none | "home_B.net" |

Editor's Note: Further consideration of this table is needed due to the outcome of discussion within SA5.

#### 4.3.1.3	Basic IMS roaming scenario without loopback

Within this basic approach 3 traversal scenarios exist which are shown in table 4.3.1.2-1.

Table 4.3.1.2-1: Traversal scenarios for basic IMS roaming scenario without loopback

| Traversal scenario | Type of IOI |
| --- | --- |
| Originating visited network – originating home network | Type 1 |
| Originating home network – terminating home network | Type 2 |
| Terminating home network – terminating visited network | Type 1 |

Figure 4.3.1.2-1 IOI information exchange for the "Basic IMS" roaming case with home routing of media

In order to meet the requirements of 3GPP TS 24.229 [4] there needs to be an IOI exchange including originating IOI, transit IOIs, and terminating IOI for each of the three signalling paths crossing network boundaries.

#### 4.3.1.4	Originating visited network with loopback routeing scenario

Within the loopback approach 4 traversal scenarios exist which are shown in table 4.3.1.3-1.

Table 4.3.1.3-1 Traversal scenarios for "Originating visited network with loopback routeing scenario"

| traversal scenario | Type of IOI |
| --- | --- |
| Originating visited network – originating home network | Type 1 |
| Originating home network – terminating home network | Type 2 |
| Originating visited network – terminating home network | Type 2 |
| Terminating home network – terminating visited network | Type 1 |

For roaming with loopback the "Originating visited network with loopback  routing scenario" introduces the case where the SIP signalling and media is routed from the originating visited network to the destination and where the SIP signalling is first routed to the originating home network for service execution and then back to the originating visited network to allow the originating visited network to route both SIP signalling and media to the destination.

The complete sequence of IOI exchanges for the IMS roaming scenario with originating visited network routing option is depicted in Figure 4.3.1.3-1.

Editor's note: The figure can not be edited (HPLMN / VPLMN replaced with Home network / Visited network) and . How to fix this is FFS.

Figure 4.3.1.3-1: IOI exchange for IMS roaming with originating visited network with loopback routing

In order to meet the requirements of 3GPP TS 24.229 [4] there needs to be an IOI exchange including originating IOI, transit IOIs, and terminating IOI there are four signalling paths crossing network boundaries.

### 4.3.2	Naming of URIs

#### 4.3.2.1	General

3GPP TS 23.003 [16] defines the principal purpose and use of International Mobile station Equipment Identities (IMEI) within the digital cellular telecommunications system and the 3GPP system.

Within subclause 13 of 3GPP TS 23.003 [16] the numbering, addressing and identification within the IP multimedia core network subsystem is described.

This subclause gives examples of used identities within the roaming use case. The normative statements in 3GPP TS 23.003 [16] apply.

#### 4.3.2.2	Home Network Domain

The home network domain name is in the form of an Internet domain name, e.g. operator.com.

3GPP TS 23.003 [16] subclause 13.2 describes how to built the home network domain, if there is no ISIM application.

EXAMPLE: A home network domain name is: IMSI in use with 234150999999999 results in home network domain = ims.mnc015.mcc234.3gppnetwork.org.

#### 4.3.2.3	Private user identity

3GPP TS 23.003 [16] subclause 13.3 describes the principles how to build a private user identity.

The NAI as specified in clause 2.1 of IETF RFC 4282 [17] for 3GPP systems with ISIM application the private user identity is derived from the IMSI.

EXAMPLE: If the IMSI is 234150999999999 (MCC = 234, MNC = 15), the private user identity then takes the form "234150999999999@ims.mnc015.mcc234.3gppnetwork.org".

#### 4.3.2.4	Public user identity

3GPP TS 23.003 [16] subclause 13.4 describes the principles how to build a public user identity.

The Public User Identity takes the form of either a SIP URI (see IETF RFC 3261 [9]) or a tel URI (see IETF RFC 3966 [18]).

According to 3GPP TS 29.165 [6] a global number as defined in IETF RFC 3966 [18] is used in a tel URI or in the user portion of a SIP URI with the user=phone parameter when conveyed via a non-roaming II-NNI except when agreement exists between the operators to also allow other kinds of numbers. For a roaming II NNI no conventions are defined currently.

EXAMPLE 1: SIP URI: sip: +49892233445@operator.com; user=phone

EXAMPLE 2: SIP URI: sip: +49892233445@ 192.0.2.4; user=phone

EXAMPLE 3: SIP URI: sip: bob@operator.com

EXAMPLE 4: SIP: tel:+4687197378

#### 4.3.2.4	Instance-ID

3GPP TS 23.003 [16] subclause 13.8 describes the principles how to build a public user identity.

When an IMEI is available, the instance-id takes the form of an IMEI URN an example of such an instance-id is as follows:

EXAMPLE 1:	urn:gsma:imei:90420156-025763-0

If no IMEI is available, the instance-id takes the form of a string representation of a UUID as a URN as defined in IETF RFC 4122 [19]. An example of such an instance-id is as follows:

EXAMPLE 2:	urn:uuid:f81d4fae-7dec-11d0-a765-00a0c91e6bf6

For more information on the instance-id and when it is used, see 3GPP TS 24.229 [4].

# 5	Description of Scenarios

## 5.1	General

### 5.1.1	The used values for the message example

#### 5.1.1.1	IOI values

The IOI names used in the message examples of clause 5 are shown in Table 5.1.1.1-1.

Table 5.1.1.1-1: The ioi values used for the message examples

| Network | IOI name (NOTE 1, NOTE 2, NOTE 3) |
| --- | --- |
| Originating visited network | visited-a |
| Originating home network | home-a |
| Terminating visited network | visited-b |
| Terminating home network | home-b |
| IC network A | ICa |
| IC network B | ICb |
| NOTE 1: The values for "orig-ioi" and "term-ioi" header field parameters are encoded in the format of a quoted string. NOTE 2: The value for an entry in "transit-ioi" header field parameter contains an indexed value. NOTE 3: The type 1 and type 3 IOIs are prefixed with specific strings (i.e. "Type1" and "Type3"). |  |

## 5.2	Registration when roaming

### 5.2.1	General

This subclause describes the registration process of a roaming user including the traversal of transit network between the visited and the originating home network.

#### 5.2.1.1	Registration via IC SIP proxies

The registration procedure between the visited network and the home network uses an IC network which provides SIP proxy nodes to relay SIP traffic between these networks.

For the registration the UE addresses an FQDN in the home network domain which reflects the registrar (i.e. the S-CSCF of the home network). With this FQDN the IBCF acting as an exit point in the visited network chooses an IC provider and routes to a SIP proxy of the selected IC provider.

The SIP layer signalling is served by the IC provider’s SIP proxy. The SIP proxy decides on the next SIP proxy towards the home network and routes to the next SIP proxy or to the IBCF acting as an entry point in the home network,  if no further SIP proxy is required.

NOTE:	The call flows just show scenarios with a single IC network and a single SIP proxy.

The user A at UE-A is roaming into a visited network vA. The user A is a subscriber of operator hA.

Figure 5.2.1.1-1: Message flow for Registration IC SIP proxy-

Call flow details:

1.	REGISTER request (UE-A to P-CSCF-vA1) - see example in table 5.2.1.1-1.

The user A at UE-A initiates registration. The UE-A sends an initial REGISTER request to the P-CSCF-A according to 3GPP TS 24.229 [4].

Table 5.2.1.1-1: REGISTER request (UE-A to P-CSCF-vA1)

REGISTER sip:homeA.net SIP/2.0

P-Access-Network-Info:3GPP-E-UTRAN-TDD; utran-cell-id-3gpp=234151D0FCE11

From:<sip:userA_public1@homeA.net>;tag=4fa3

To:<sip:userA_public1@homeA.net>

Contact:<sip:[5555::aaa:bbb:ccc:ddd]>;reg-id=1; +sip.instance="<urn:gsma:imei:90420156-025763-0 >";expires=600000

Via:SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd];branch=z9hG4bKnashds7

CSeq: 1 REGISTER

Supported:path

Other SIP header fields are set according to 3GPP TS 24.229 [4].

Request-URI:	The Request-URI sip:homeA.net (the URI that follows the method name, "REGISTER", in the first line) indicates the destination domain of this REGISTER request which is the home network.

2.	REGISTER request (P-CSCF-vA1 to IBCF-vA1) - see example in table 5.2.1.1-2.

When the P-CSCF-vA1 receives the initial REGISTER request the P-CSCF-vA1 adds the P-Visited-Network-ID header field containing the identifier of the P-CSCF network. This may be the visited network domain name or any other identifier that identifies the visited network at the home network.

The P-CSCF-vA1 needs to be in the path for all terminating requests for this user. To ensure this, the P-CSCF-vA1 adds itself to the Path header field for future requests.

The P-CSCF-vA1 selects an IBCF (IBCF-vA1) to be the IBCF acting as an exit point towards the home network hA of the user A and sends the REGISTER request according to 3GPP TS 24.229 [4] to IBCF-vA1.

Table 5.2.1.1-2: REGISTER request (P-CSCF-vA1 to IBCF-vA1)

REGISTER sip:homeA.net SIP/2.0

P-Access-Network-Info: 3GPP-E-UTRAN-TDD; utran-cell-id-3gpp=234151D0FCE11, 3GPP-E-UTRAN-TDD; utran-cell-id-3gpp=234151D0FCE11;network-provided;local-time-zone="UTC+01:00";daylight-saving-time="01"

From:

To:

Contact:

CSeq:

Supported:

Path: <sip:visit-xyz@pcscf-vA1.visited-A.net:5070;lr>

Route: <sip:reg@ibcf-vA1.visited-A.net;lr>

Route: <sip:reg@ibcf-hA1.visited-A.net;lr> or <sip:reg@i-cscf-vA1.visited-A.net;lr>

P-Visited-Network-ID: "Visited Network Number A"

P-Charging-Vector: icid-value="AyretyU0dm+6O2IrT5tAFrbHLso=023551024";orig-ioi="Type 1visited-a"

Via: SIP/2.0/UDP <visit-xyz@pcscf-A.visited-A.net:5060;branch=z9hG4bKnas56565>,

SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd];branch=z9hG4bKnashds7

P-Access-Network-Info:

...

Other SIP header fields are set according to 3GPP TS 24.229 [4] and SDP for OMR is according to 3GPP TS 29.079 [5].

Route:	IBCF-vA1 URI for originating requests (as either configured in P-CSCF or based on the user's URI, the P-CSCF determines that UE is registering from a visiting domain and performs either DNS queries as specified in RFC 3263 [13] or by provisioned data to locate the I-CSCF (for non topology hiding use cases) or the entry point of the home network. The look up in the DNS is based on the domain name specified in the Request-URI.).
Based on operator policy for topology hiding the address of the IBCF-hA1 could be used in instead of the I-CSCF-hA1 to provide topology hiding.
Thus the content of the Route header field addressing the home network could be e.g. <sip:reg@ibcf-hA1.visited-A.net;lr> or <sip:reg@i-cscf-vA1.visited-A.net;lr>

P-Charging-Vector:	The P-CSCF-vA1 provides an ICID value and adds a type 1 orig-ioi with the IOI value identifying the visited network.

Path: 	This is the address of the P-CSCF-vA1 and is included to inform the S-CSCF where to route terminating requests.

Require:	This header is included to ensure that the recipient correctly handles the Path header field. If the recipient does not support the Path header field, a SIP response will be received with a status code of 420 and an Unsupported header field indicating "path". Such a response indicates a wrong configuration of the routing tables and the request has been routed outside the IM CN subsystem.

P-Access-Network-Info: The P-CSCF-vA1 adds a network provided location, time-zone and daylight saving time. If it doesn't indicate that it is a network provided location, the P-Access-Network-Info header field containing the location provided by the UE-A is kept in the INVITE request.

3.	REGISTER request (IBCF-vA1 to IC-A1) - see example in table 5.2.1.1-3.

The IBCF-vA1 receives the initial REGISTER request and analyses the request line and forwards registration to preferred VoIP IC provider (IC-A1).

The IBCF-vA1 adds its own address to the Path header field.

In case the IBCF-vA1 provides topology hiding the path will contain one hidden Via header field entry and one visible Via header field entry added by the IBCF acting as the entry point.

Table 5.2.1.1-3: REGISTER request (IBCF-vA1 to IC-A1)

REGISTER sip: homeA.net SIP/2.0

P-Access-Network-Info:

From:

To:

Contact:

Path: <sip:visit-abc@ibcf-vA1.visited-A.net:5070;lr>,<sip:visit-xyz@pcscf-vA1.visited-A.net:5070;lr>

Route: <sip:reg@ic-A1.interconnection.net;lr>

Route: <sip:reg@ibcf-hA1.visited-A.net;lr> or <sip:reg@i-cscf-vA1.visited-A.net;lr>

P-Visited-Network-ID:

P-Charging-Vector: icid-value="AyretyU0dm+6O2IrT5tAFrbHLso=023551024";orig-ioi="Type 1visited-a"

Via: SIP/2.0/UDP <reg@ibcf-vA1.visited-A.net;branch=z9hG4bK351g45.1>,

SIP/2.0/UDP <visit-xyz@pcscf-A.visited-A.net:5060;branch=z9hG4bKnas56565>,

SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd];branch=z9hG4bKnashds7

...

Other SIP header fields are set according to 3GPP TS 24.229 [4].

Path:	The address of the IBCF (IBCF-vA1) acting as an exit point will be added.

Via: 	Based on topology hiding of functionality the IBCF-vA1 will either:

-	forwards the Via header field with one visible IBCF-vA1 entry and the remaining collected entries as one hidden entry; or

-	an IBCF-vA1 entry and whole content of already collected entries as visible entries .

4.	REGISTER request (IC-A1 to IBCF-hA1) - see example in table 5.2.1.1-4.

The IC-A1 receives the initial REGISTER request and analyses the request line and forwards registration to the IBCF-hA1 of the home network of user A. The routing could either apply via DSN or any other routeing mechanism.

The IC-A1 adds its own address to the Path header field.

Another possibility is that the IC-A1 does not add any entry to the Path header field.

This is based on bilateral agreement. In case where the transit network is only used based on least cost routeing.

Table 5.2.1.1-4: REGISTER request (IC-A1 to IBCF-hA1)

REGISTER sip:homeA.net SIP/2.0

P-Access-Network-Info:

From:

To:

Contact:

Path: <sip:proxy-abc@ic-A1.interconnection-A.net:5070;lr>,<sip:visit-abc@ibcf-vA1.visited-A.net:5070;lr>,<sip:visit-xyz@pcscf-vA1.visited-A.net:5070;lr>

Route: <sip:reg@ibcf-hA1.hplmn-A1.net;lr> or <sip:reg@i-cscf-vA1.visited-A.net;lr>

P-Visited-Network-ID:

P-Charging-Vector: icid-value="AyretyU0dm+6O2IrT5tAFrbHLso=023551024";orig-ioi="Type 1visited-a"

Via: SIP/2.0/UDP <reg@ic-A1.interconnection-A.net;branch=z9hG4bK351g45.1>,

SIP/2.0/UDP <reg@ibcf-vA1.visited-A.net;branch=z9hG4bK351g45.1>,

SIP/2.0/UDP <visit-xyz@pcscf-vA1.visited-	A.net:5060;branch=z9hG4bKnas56565>,

SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd];branch=z9hG4bKnashds7

...

Other SIP header fields are set according to 3GPP TS 24.229[4].

Path: 	Based on SLA between IC partners and roaming partners the address of the IC-A1 will be added or not added by IC-A1.

Via:	The Via header field is added by each proxy in IC networks, in this example the Via header field is only added by IC-A1.

5.	REGISTER request (IBCF-hA1 to I-CSCF-hA1) - see example in table 5.2.1.1-5.

The IBCF-hA1 receives the initial REGISTER request and analyses the request line and forwards registration to the I-CSCF. The IBCF-hA1 adds a Route header field with the address of the I-CSCF of the home network of user A. for performance reasons this I-CSCF could be used only for registration traffic for roaming user which is due to local policy of the home network operator.

The IBCF-hA1 adds its own address to the Path header field.

Table 5.2.1.1-5: REGISTER request (IBCF-hA1 to I-CSCF-hA1)

REGISTER sip:homeA.net SIP/2.0

P-Access-Network-Info:

To:

Contact:

Path: <sip:home-abc@ibcf-hA1.home-A.net:5070;lr>,<sip:proxy-abc@ic-A1.interconnection-	A.net:5070;lr>,<sip:visit-abc@ibcf-vA1.visited-A.net:5070;lr>,<sip:visit-xyz@pcscf-vA1.visited-	A.net:5070;lr>

Route: <sip:reg@i-cscf-hA1.hplmn-A1.net;lr> or <sip:reg@i-cscf-vA1.visited-A.net;lr>

P-Visited-Network-ID:P-Charging-Vector: icid-value="AyretyU0dm+6O2IrT5tAFrbHLso=023551024";

orig-ioi="Type 1visited- a";transit-ioi="ICa.1"

Via: SIP/2.0/UDP <reg@ibcf-hA1.home-A.net;branch=z9hG4bK351g45.1>

Via: SIP/2.0/UDP <reg@ic-A1.interconnection-A.net;branch=z9hG4bK351g45.1

Via: SIP/2.0/UDP reg@ibcf-vA1.visited-A.net;branch=z9hG4bK351g45.1>

Via: SIP/2.0/UDP <visit-xyz@pcscf-vA1.visited-A.net:5060;branch=z9hG4bKnas56565>

Via: SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd];branch=z9hG4bKnashds7

...

Other SIP header fields are set according to 3GPP TS 24.229 [4].

P-Charging-Vector:	In this example the IBCF-hA1 acting as an entry point in the homeA network adds the identity of the IC-A1 VoIP provider in the "transit-ioi" header field parameter. However, if the REGISTER request is routed via more than one IC VoIP provider network each entry point in a IC VoIP provider receiving the REGISTER request adds a "transit-io" i header field entry containing the previous IC VoIP provider identity.

6.	REGISTER request (I-CSCF-hA1 to S-CSCF-hA1) - see example in table 5.2.1.1-6.

The I-CSCF-hA1 receives the REGISTER request and makes a request for information related to the subscriber registration status by sending the private user identity, public user identity and visited domain name to the HSS. The HSS returns the S-CSCF required capabilities and the I-CSCF-hA1 uses this information to select a suitable S-CSCF.

Due to the fact that the I-CSCF-hA1 only identifies the home S-CSCF which is responsible for the roaming user A, the I-CSCF-hA1 is not needed for future requests to be kept in the path, thus an I-CSCF entry is not needed within the Path header field.

But for the response to the REGISTER message the I-CSCF-hA1 has to be added in the Via header field.

Table 5.2.1.1-6: REGISTER request (I-CSCF-hA1 to S-CSCF-hA1)

REGISTER homeA.net SIP/2.0

P-Access-Network-Info:

From:

To:

Contact:

Path: <sip:home-abc@ibcf-hA1.home-A.net:5070;lr>,<sip:proxy-abc@ic-A1.interconnection-A.net:5070;lr>,<sip:visit-abc@ibcf-vA1.visited-A.net:5070;lr>,<sip:visit-xyz@pcscf-vA1.visited-A.net:5070;lr>

Route: <sip:reg@scscf-hA1.hplmn-A1.net;lr>

Route: <sip:reg@s-cscf-hA1.hplmn-A1.net;lr>

P-Visited-Network-ID:

P-Charging-Vector: icid-value="AyretyU0dm+6O2IrT5tAFrbHLso=023551024";orig-ioi="Type 1visited- a";transit-ioi="ICa.1"

Via: SIP/2.0/UDP <reg@i-cscf-hA1.home-A.net;branch=z9hG4bK351g45.1>,

SIP/2.0/UDP <reg@ibcf-hA1.home-A.net;branch=z9hG4bK351g45.1>,

SIP/2.0/UDP <reg@ic-A1.interconnection-A.net;branch=z9hG4bK351g45.1>,

SIP/2.0/UDP <reg@ibcf-vA1.visited-A.net;branch=z9hG4bK351g45.1>,

SIP/2.0/UDP <visit-xyz@pcscf-vA1.visited-A.net:5060;branch=z9hG4bKnas56565>,

SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd];branch=z9hG4bKnashds7

...

Other SIP header fields are set according to 3GPP TS 24.229 [4].

7.	401 (Unauthorized) response (S-CSCF-hA1 to I-CSCF-hA1) - see example in table 5.2.1.1-7.

As the REGISTER request arrived without integrity protection to the P-CSCF, the S-CSCF shall challenge the request. For this, the S-CSCF requires at least one authentication vector to be used in the challenge to the user. If a valid authentication vector  is not available, then the S-CSCF requests at least one authentication vector  from the HSS.

Editor's Note: Here was text pointing to 3GPP TS 33.203 for detailed description of the authentication vector. Needs to be considered if this hint is needed in this TR.

Table 5.2.1.1-7: 401 (Unauthorized) response (S-CSCF-hA1 to I-CSCF-hA1)

SIP/2.0 401 Unauthorized

Via: SIP/2.0/UDP <reg@i-cscf-hA1.home-A.net;branch=z9hG4bK351g45.1>,

SIP/2.0/UDP <reg@ibcf-hA1.home-A.net;branch=z9hG4bK351g45.1>,

SIP/2.0/UDP <reg@ic-A1.interconnection-A.net;branch=z9hG4bK351g45.1>,

SIP/2.0/UDP <reg@ibcf-vA1.visited-A.net;branch=z9hG4bK351g45.1>,

SIP/2.0/UDP <visit-xyz@pcscf-vA1.visited-A.net:5060;branch=z9hG4bKnas56565>,

SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd];branch=z9hG4bKnashds7

From:

To: <sip:userA_public1@homeA.net>;tag=5ef4

Other SIP header fields are set according to 3GPP TS 24.229 [4].

8-12.	401 (Unauthorized) response (I-CSCF-hA1 to UA-A).

Normal response routing procedure as described within 3GPP TS 24.229 [4] apply. No specific roaming procedures apply.

13-17. 	REGISTER request (UE-A to I-CSCF-hA1).

The authentication challenge response is calculated by the UE-A and then put into the Authorization header field and sent back to the registrar in the REGISTER request. Procedures are described within 3GPP TS 24.229 [4].

UE-A registers a second time with an authentication response.

With the exception of the Authorization header field the REGISTER message will contain the same information and the Path header field will be set as described within steps 1-5 of this call flow.

18.	REGISTER request (I-CSCF-hA1 to S-CSCF-hA1)

I-CSCF-hA1 queries IMS-HSS (LIR/LIA) as described within 3GPP TS 29.228 [14] and routes to the previously assigned S-CSCF-hA1. It is the same procedure as described within step 6 of this call flow.

The Path header field will be set as described within step 6 of this call flow.

19-24.		200 OK response (S-CSCF-hA1 to UA-A) - see example in table 5.2.1.1-19.

S-CSCF-hA1 confirms the registration as described within 3GPP TS 24.229 [4]. There are no roaming specific procedures that apply in addition.

Table 5.2.1.1-19: 200 OK response (S-CSCF-hA1 to UA-A)

SIP/2.0 200 OK

Via: SIP/2.0/UDP <reg@i-cscf-hA1.home-A.net;branch=z9hG4bK351g45.1>,

SIP/2.0/UDP <reg@ibcf-hA1.home-A.net;branch=z9hG4bK351g45.1>,

SIP/2.0/UDP <reg@ic-A1.interconnection-A.net;branch=z9hG4bK351g45.1>,

SIP/2.0/UDP <reg@ibcf-vA1.visited-A.net;branch=z9hG4bK351g45.1>,

SIP/2.0/UDP <visit-xyz@pcscf-vA1.visited-A.net:5060;branch=z9hG4bKnas56565>,

SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd];branch=z9hG4bKnashds7

From:

Service-Route: <sip:home-abc@scscf-hA1.home-A.net;lr>

To: <sip:userA_public1@homeA.net>;tag=5ef4

P-Charging-Vector: icid-value="AyretyU0dm+6O2IrT5tAFrbHLso=023551024";orig-ioi="Type 1visited-a";term-ioi="Type 1home-a"

Path: <sip:home-abc@ibcf-hA1.home-A.net:5070;lr>,

<sip:proxy-abc@ic-A1.interconnection-A.net:5070;lr>,

<sip:visit-abc@ibcf-vA1.visited-A.net:5070;lr>,

<sip:visit-xyz@pcscf-vA1.visited-A.net:5070;lr>

Other SIP header fields are set according to 3GPP TS 24.229 [4].

Path:	List of Path header field received in the REGISTER request will be included. (IBCF-hA1, IC-A1, IBCF-vA1, P-CSCF-vA1)

P-Associated-URI:	Containing the list of the registered distinct public user identity and its associated set of implicitly registered distinct public user identities. The first URI in the list of public user identities supplied by the HSS to the S-CSCF will indicate the default public user identity to be used by the S-CSCF.

Service-Route:	Contains the SIP URI identifying the S-CSCF (S-CSCF-hA1) containing an indication that subsequent requests routed via this service route (i.e. from the P-CSCF to the S-CSCF) was sent by the UE-A using either the contact address of the UE-A. In case network topology hiding is required, a visible SIP URI identifying the IBCF (IBCF-hA1) will be added by the IBCF acting as an entry point as the topmost entry and a hidden SIP URI identifying the S-CSCF (S-CSCF-hA1) in the bottom.

P-Charging-Vector:	The P-Charging-Vector header field containing the "orig-ioi" header field parameter, if received in the REGISTER request and a type 1 "term-ioi" header field parameter. The S-CSCF-hA1 shall set the type 1 "term-ioi" header field parameter to a value that identifies the sending network of the response and the "orig-ioi" header field parameter is set to the previously received value of "orig-ioi" header field parameter
The "icid-value" header field parameter is set to the previously received value of "icid-value" header field parameter in the request.

The other header fields used within the 200 (OK) response to the REGISTER request are included due to normal IMS procedures as described within 3GPP TS 24.229 [4].

## 5.3	Originating roaming without loopback

### 5.3.1	General

This subclause describes routeing from originating visited network to originating home network including the traversal of transit network between the visited and the originating home network.

### 5.3.2	Call flow

In this scenario the originating home network decides to route the media directly to the terminating home network. OMRis not executed.

Preconditions:

-	the user A is registered to the home network hA via the visited operator vA network and the IC network A. The IC network does not store any registration information; and

-	the called user is located in another network either in PSTN or in IMS.

NOTE:	For clarity, the SIP 100 (Trying) messages are not shown in the signalling flow.

Figure 5.3.2-1: MOC without loopback

The steps of the flow are as follows:

1.	INVITE request (UE-A to P-CSCF-vA1) - see example in table A.5.3.2-1.

The user A at UE-A initiates a call. The UE-A sends an initial INVITE request to the P-CSCF-A according to 3GPP TS 24.229 [4].

Table 5.3.2-1: INVITE request (UE-A to P-CSCF-vA1)

INVITE tel:+4687197378 SIP/2.0

From: <sip:userA_public1@home-hA.net>;tag=4fa3

Route: <sip:pcscf-vA1.visited-A.net;lr>,<sip:home-abc@scscf-hA1.home-A.net;lr>

P-Access-Network-Info: 3GPP-E-UTRAN-TDD;utran-cell-id-3gpp=234151D0FCE11

Via: SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd];comp=sigcomp;branch=z9hG4bKnashds7

Other SIP headers and SDP according to 3GPP TS 24.229 [4]

Route:	The UE-A builds a proper preloaded route set for all new dialogs and standalone transactions containing the IP address or the FQDN learnt through the P-CSCF discovery procedures and P-CSCF port based on the security mechanism in use. In addition the values received in the Service-Route header field saved from the 200 (OK) response to the last registration or re-registration of the public user identity with associated contact address are added. The content of the received Service-Route header filed is due to operator policy. In this example only the S-CSCF-hA1 will be included.

2.	INVITE request (P-CSCF-vA1 to IBCF-vA1) - see example in table 5.3.2-2.

When the P-CSCF-vA1 receives the initial INVITE request and, since the user A is a roaming user, based on local policy and roaming agreement the P-CSCF-vA1 adds the Feature-Caps header field with the "g.3gpp.trf" header field parameter with the address to the TRF; and

The P-CSCF-vA1 selects an IBCF (IBCF-vA1) to be the exit IBCF towards the home IMS network hA1 of the user A and sends the INVITE request according to 3GPP TS 24.229 [4] to IBCF-v1A.

The received list of URIs in the Route header field within the INVITE will be verified with the Route header field list constructed form the Service-Route header fields received during the last registration procedure.

Table 5.3.2-2: INVITE request (P-CSCF-vA1 to IBCF-vA1)

INVITE tel:+4687197378 SIP/2.0

Route: <sip:ibcf-vA1.visited-A.net;lr>,<sip:home-abc@scscf-hA1.home-A.net;lr>

Record-Route: <sip:pcscf-vA1.visited-A.net;lr>

Feature-Caps:*;+g.3gpp.trf="<sip:trf-vA1.visited-A.net;lr>"

P-Access-Network-Info:3GPP-E-UTRAN-TDD;utran-cell-id-3gpp=234151D0FCE11, 3GPP-E-UTRAN-TDD; utran-cell-id-3gpp=234151D0FCE11;network-provided;local-time-zone="UTC+01:00";daylight-saving-time="01"

P-Charging-Vector: icid-value="AyretyU0dm+6O2IrT5tAFrbHLso=023551024";orig-ioi="Type 1visited-a"

Via: SIP/2.0/UDP <visit-xyz@pcscf-vA1.visited-A.net:5060;branch=z9hG4bKnas56565>

Via: SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd];comp=sigcomp;branch=z9hG4bKnashds7

Other SIP headers and SDP according to 3GPP TS 24.229 [4] .

3.	INVITE request (IBCF-vA1 to IC-A1) - see example in table 5.3.2-3.

When the IBCF-vA1 receives the initial INVITE request containing the "+g.3gpp.trf" header field parameter in a Feature-Caps header field, the IBCF-vA1 updates the SDP as described within TS 29.079 [5].

The IBCF-vA1 selects, based on local policy, to route the INVITE request via IC-A1 network and sends the INVITE request according to 3GPP TS 24.229 [4] to the IC-A1.

Table 5.3.2-3: INVITE request (IBCF-vA1 to IC-A1)

INVITE tel:+4687197378 SIP/2.0

Route: <sip:ic-A1.interconnection-A.net;lr>,<sip:home-abc@scscf-hA1.home-A.net;lr>

Record-Route: <sip:ibcf-vA1.visited-A.net;lr>,<sip:pcscf-vA1.visited-A.net;lr>

Feature-Caps:*;+g.3gpp.trf="<sip:trf-vA1.visited-A.net;lr>"

P-Access-Network-Info:

P-Charging-Vector: icid-value="AyretyU0dm+6O2IrT5tAFrbHLso=023551024";orig-ioi="Type 1visited-a"

Via: SIP/2.0/UDP <ibcf-vA1.visited-A.net;branch=z9hG4bK351g45.1>

Via: SIP/2.0/UDP <visit-xyz@pcscf-vA1.visited-A.net:5060;branch=z9hG4bKnas56565>

Via: SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd];comp=sigcomp;branch=z9hG4bKnashds7

Other SIP headers and the SDP settings according to 3GPP TS 24.229 [4] and the SDP settings according 3GPP TS 29.079 [5].

Record-Route: 	Based on topology hiding functionality the IBCF-vA1 will either:

-	forwards the Record-Route header field with one visible IBCF-vA1 entry and the remaining collected entries as one hidden entry; or

-	an IBCF-vA1 entry and whole content of already collected entries as visible entries.

Via:	Based on topology hiding functionality the IBCF-vA1 will either:

-	forwards the Via header field with one visible IBCF-vA1 entry and the remaining collected entries as one hidden entry; or

-	an IBCF-vA1 entry and whole content of already collected entries as visible entries.

4.	INVITE request (IC-A1 to IBCF-hA1) - see example in table 5.3.2-4.

When the IC-A1 receives the initial INVITE request and since the IC-A1 according to local policy allows itself to be bypassed by media then the IC-A1 updates the SDP as described within TS 29.079 [5].

The IC-A1 selects an entry point (IBCF-hA1) of the home IMS network hA and sends the INVITE request according to 3GPP TS 24.229 [4] to the IBCF-hA1.

The IC-A1 may add a Route header field entry pointing to the entry point of home network hA. This is shown in example in table 5.3.2-4.

Table 5.3.2-4: INVITE request (IC-A1 to IBCF-hA1)

INVITE tel:+4687197378 SIP/2.0

Route: <sip:ibcf-hA1.home-A.net;lr>,<sip:home-abc@scscf-hA1.home-A.net;lr>

Record-Route: <sip:ic-A1.interconnection-A.net;lr>,<sip:ibcf-vA1.visited-A.net;lr>,

<sip:pcscf-vA1.visited-A.net;lr>

Feature-Caps:*;+g.3gpp.trf="<sip:trf1-vA1.visited-A.net;lr>"

P-Access-Network-Info:

P-Charging-Vector: icid-value="AyretyU0dm+6O2IrT5tAFrbHLso=023551024";orig-ioi="Type 1visited-a";

Via: SIP/2.0/UDP <ic-A1.interconnection-A.net;branch=z9hG4bK351g45.1

Via: SIP/2.0/UDP <ibcf-vA1.visited-A.net;branch=z9hG4bK351g45.1>

Via: SIP/2.0/UDP <visit-xyz@pcscf-vA1.visited-A.net:5060;branch=z9hG4bKnas56565>

Via: SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd];comp=sigcomp;branch=z9hG4bKnashds7

Other SIP headers and the SDP settings according to 3GPP TS 24.229 [4] and

additional SDP settings due to 3GPP TS 24.229 [4] and 3GPP TS 29.079 [5]

5.	INVITE request (IBCF-hA1 to S-CSCF-hA1) - see example in table 5.3.2-5.

When the IBCF-hA1 receives the initial INVITE request containing the "+g.3gpp.trf" header field parameter in a Feature-Caps header field, the IBCF-hA1 updates the SDP as described in TS 29.079 [5].

The IBCF-hA1 sends the INVITE request according to 3GPP TS 24.229 [4] to the S-CSCF-hA1.

Table 5.3.2-5: INVITE request (IBCF-hA1 to S-CSCF-hA1)

INVITE tel:+4687197378 SIP/2.0

Route: <sip:home-abc@scscf-hA1.home-A.net;lr>

Record-Route: <sip:ibcf-hA1.home-A.net:5070;lr>,<sip:ic-A1.interconnection-A.net;lr>,

<sip:ibcf-vA1.visited-A.net;lr>,<sip:pcscf-vA1.visited-A.net;lr>

Feature-Caps:*;+g.3gpp.trf="<sip:trf1-vA1.visited-A.net;lr>"

P-Access-Network-Info:

P-Charging-Vector: icid-value="AyretyU0dm+6O2IrT5tAFrbHLso=023551024";orig-ioi="Type 1visited-a"; transit-ioi="ICa.1"

Via: SIP/2.0/UDP <ibcf-hA1.home-A.net;branch=z9hG4bK351g45.1>

Via: SIP/2.0/UDP <ic-A1.interconnection-A.net;branch=z9hG4bK351g45.1

Via: SIP/2.0/UDP <ibcf-vA1.visited-A.net;branch=z9hG4bK351g45.1>

Via: SIP/2.0/UDP <visit-xyz@pcscf-vA1.visited-A.net:5060;branch=z9hG4bKnas56565>

Via: SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd];comp=sigcomp;branch=z9hG4bKnashds7

Other SIP headers and the SDP settings according to 3GPP TS 24.229 [4] and the SDP settings according 3GPP TS 29.079 [5].

6.	INVITE request (S-CSCF-hA1 to AS-hA1) - see example in table 5.3.2-6.

The S-CSCF-hA1 applies first filter criteria that is no specific procedures due to roaming. The case where an AS needs the involvement of media function is described in subclause 5.10

Editor's note:	The subclause 5.x needs to be specified or the reference removed.

The S-CSCF-hA1 decides according to local policy that loopback routeing can be used and include a Feature-Caps header field with the "+g.3gpp.home-visited" header field parameter according to IETF RFC 6809 [21] with the "+g.3gpp.home-visited" header field parameter set to the identifier of the visited network received in the P-Visited-Network-ID header field in the original registration request.

The S-CSCF-hA1 replaces the received orig-ioi parameter value in the P-Charging-Vector header field with the orig-ioi type 3 value identifying the home IMS network hA ("home-ha.net").

If a service needs to be invoked according to the user A profile, a Route header field with a URI pointing to the AS-hA1 and a URI pointing back to the S-CSCF-hA1 is added.

Editor's Note: The behaviour of the transit-ioi value is currently discussed and needs further consideration how the S-CSCF to AS correlation is describes.

Table 5.3.2-6: INVITE request (S-CSCF-hA1 to AS-hA1)

INVITE tel:+4687197378 SIP/2.0

Route: <sip:as-hA1.home-A.net;lr>, <sip:orig@scscf-hA1.home-A.net;lr>

Record-Route: <sip:scscf-hA1.home-A.net:5070;lr>,<sip:ibcf-hA1.home-A.net:5070;lr>,

<sip:ic-A1.interconnection-A.net;lr>,<sip:ibcf-vA1.visited-A.net;lr>,

<sip:pcscf-vA1.visited-A.net;lr>

Feature-Caps:*;+g.3gpp.trf="<sip:trf1-vA1.visited-A.net;lr>",*;+g.3gpp.home-visited="<visited_A>"

P-Access-Network-Info:

P-Charging-Vector: icid-value="AyretyU0dm+6O2IrT5tAFrbHLso=023551024";orig-ioi="Type 3home-a";

Via: SIP/2.0/UDP <scscf-hA1.home-A.net;branch=z9hG4bK351g45.1>

Via: SIP/2.0/UDP <ibcf-hA1.home-A.net;branch=z9hG4bK351g45.1>

Via: SIP/2.0/UDP <ic-A1.interconnection-A.net;branch=z9hG4bK351g45.1

Via: SIP/2.0/UDP <ibcf-vA1.visited-A.net;branch=z9hG4bK351g45.1>

Via: SIP/2.0/UDP <visit-xyz@pcscf-vA1.visited-A.net:5060;branch=z9hG4bKnas56565>

Via: SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd];comp=sigcomp;branch=z9hG4bKnashds7

Other SIP headers and the SDP settings according to 3GPP TS 24.229 [4] and the SDP settings according 3GPP TS 29.079 [5].

7.	INVITE request (AS-hA1 to S-CSCF-hA1) - see example in table 5.3.2-7.

AS-hA1 applies the service and removes the first Route header field entry.

The AS shall remove the "orig-ioi" header field parameter from the forwarded request and insert a type 3 "orig-ioi" header field parameter. The AS-hA1 shall set the type 3 "orig-ioi" header field parameter to a IOI value that identifies the service provider from which the request is sent. In this example the IOI value is the same as the home network operator identity.

The specifics of the service provided by the AS require access to media and the AS-hA1 removes the "+g.3gpp.home-visited" Feature-Caps header field parameter. This absence of the "+g.3gpp.home-visited" Feature-Caps header field parameter is an indication that loopback routeing is not a viable option.

All other SIP headers are set due to procedures described within 3GPP TS 24.229 [4].

Editor's Note: The behaviour of the transit-ioi value is currently discussed and needs further consideration how the S-CSCF to AS correlation is describes.

Table 5.3.2-7: INVITE request (AS-hA1 to S-CSCF-hA1)

INVITE tel:+4687197378 SIP/2.0

Route: <sip:orig@scscf-hA1.home-A.net;lr>

Record-Route: <sip:as-hA1.home-A.net:5070;lr>,<sip:scscf-hA1.home-A.net:5070;lr>,

<sip:ibcf-hA1.home-A.net:5070;lr>,<sip:ic-A1.interconnection-A.net;lr>,

<sip:ibcf-vA1.visited-A.net;lr>,<sip:pcscf-vA1.visited-A.net;lr>

Feature-Caps:*;+g.3gpp.trf="<sip:trf1-vA1.visited-A.net;lr>"

P-Access-Network-Info:

P-Charging-Vector: icid-value="AyretyU0dm+6O2IrT5tAFrbHLso=023551024";orig-ioi="Type 3home-a"

Via: SIP/2.0/UDP <AS-hA1.home-A.net;branch=z9hG4bK351g45.1>

Via: SIP/2.0/UDP <scscf-hA1.home-A.net;branch=z9hG4bK351g45.1>

Via: SIP/2.0/UDP <ibcf-hA1.home-A.net;branch=z9hG4bK351g45.1>

Via: SIP/2.0/UDP <ic-A1.interconnection-A.net;branch=z9hG4bK351g45.1

Via: SIP/2.0/UDP <ibcf-vA1.visited-A.net;branch=z9hG4bK351g45.1>

Via: SIP/2.0/UDP <visit-xyz@pcscf-vA1.visited-A.net:5060;branch=z9hG4bKnas56565>

Via: SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd];comp=sigcomp;branch=z9hG4bKnashds7

Other SIP headers and the SDP settings according to 3GPP TS 24.229 [4] and the SDP settings according 3GPP TS 29.079 [5].

Editor's Note: Due to service behaviour the AS could also be a server of a third party service provider. Further consideration of AS scenarios is needed.

P-Charging-Vector:	The P-Charging-Vector header field containing the "orig-ioi" header field parameter. The AS-hA1 shall set the type 3 "orig-ioi" header field parameter to a value that identifies the sending network (i.e. home network A). The AS has to set itself a type-3 ioi if the AS is located within another domain (i.e. a 3rd party service provider). If more than one AS is within the Path the orig-ioi values could change based on the relation between S-CSCF-hA1 and the related AS.

Record-Route:	This entries to this header needs to be added based on the related dip's to the AS. The AS could behave as B2BUA in such case the AS could store the entries and setup an new Record-Route only with the AS URI as an entry.

8.	INVITE request (S-CSCF-hA1 to IBCF-hA2) - see example in table 5.3.2-8.

When the S-CSCF-hA1 receives the initial INVITE request back from the AS-hA1 and no further services apply then the S-CSCF-hA1 decides according to local policy if loopback routeing shall be used or not. In this use case loopback will not apply. The S-CSCF-hA1 removes the Feature-Caps header field "*;+g.3gpp.trf" from the INVITE request.

In case the domain of the called user is not the same home domain as from the calling user then the S-CSCF-hA1 has to identify the home network of the called user via ENUM/DNS as provided by local policy of the operator.

In case the ENUM/DNS request fails the call will be forwarded to the BGCF or if number portability applies and a rn parameter must be set up the call will be also forwarded to the BGCF. In the case the FQDN needs to be addressed to MGCF the INVITE request will be forwarded to the BGCF. In this use case the INVITE request is sent directly to IBCF-hA2.

The S-CSCF-hA1 replaces the received "orig-ioi" header field parameter value in the P-Charging-Vector header field with the orig-ioi type 2 value identifying the home network hA ("home-ha.net").

In case a transit-ioi value exist within the P-Charging-Vector header field then the values has to be deleted.

Table 5.3.2-8: INVITE request (S-CSCF-hA1 to IBCF-hA2)

INVITE sip:+4687197378@home-B.net;user=phone SIP/2.0

Route: <sip:ibcf-hA2.home-A.net;lr>

Record-Route: <sip:scscf-hA1.home-A.net:5070;lr>,<sip:as-hA1.home-A.net:5070;lr>,

<sip:scscf-hA1.home-A.net:5070;lr>,<sip:ibcf-hA1.home-A.net:5070;lr>,

<sip:ic-A1.interconnection-A.net;lr>,<sip:ibcf-vA1.visited-A.net;lr>,

<sip:pcscf-vA1.visited-A.net;lr>P-Access-Network-Info:

P-Charging-Vector: icid-value="AyretyU0dm+6O2IrT5tAFrbHLso=023551024";orig-ioi="home-a"

Via: SIP/2.0/UDP <scscf-hA1.home-A.net;branch=z9hG4bK351g45.1>

Via: SIP/2.0/UDP <AS-hA1.home-A.net;branch=z9hG4bK351g45.1>

Via: SIP/2.0/UDP <scscf-hA1.home-A.net;branch=z9hG4bK351g45.1>

Via: SIP/2.0/UDP <ibcf-hA1.home-A.net;branch=z9hG4bK351g45.1>

Via: SIP/2.0/UDP <ic-A1.interconnection-A.net;branch=z9hG4bK351g45.1

Via: SIP/2.0/UDP <ibcf-vA1.visited-A.net;branch=z9hG4bK351g45.1>

Via: SIP/2.0/UDP <visit-xyz@pcscf-vA1.visited-A.net:5060;branch=z9hG4bKnas56565>

Via: SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd];comp=sigcomp;branch=z9hG4bKnashds7

Other SIP headers and the SDP settings according to 3GPP TS 24.229 [4].

9-16.	183 (Session Progress) response (IBCF-hA2 to UE-A) - see example in table 5.3.2-9.

When the IBCF-hA2 receives the 183 (Session Progress) response the IBCF-hA2  sends the 183 (Session Progress) response according to 3GPP TS 24.229 [4] towards the UE-A via the S-CSCF-hA1, AS-hA1, IBCF-hA1, IC-A1, IBCF-vA1, P-CSCF-vA1 path as described within 3GPP TS 24.229 [4].

Table 5.3.2-9: SIP 183 (Session Progress) response (IBCF-hA2 to UE-A) in step 9

# SIP/2.0 183 Session Progress

Via: SIP/2.0/UDP <scscf-hA1.home-A.net;branch=z9hG4bK351g45.1>

Via: SIP/2.0/UDP <AS-hA1.home-A.net;branch=z9hG4bK351g45.1>

Via: SIP/2.0/UDP <scscf-hA1.home-A.net;branch=z9hG4bK351g45.1>

Via: SIP/2.0/UDP <ibcf-hA1.home-A.net;branch=z9hG4bK351g45.1>

Via: SIP/2.0/UDP <ic-A1.interconnection-A.net;branch=z9hG4bK351g45.1

Via: SIP/2.0/UDP <ibcf-vA1.visited-A.net;branch=z9hG4bK351g45.1>

Via: SIP/2.0/UDP <visit-xyz@pcscf-vA1.visited-A.net:5060;branch=z9hG4bKnas56565>

Via: SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd];comp=sigcomp;branch=z9hG4bKnashds7

Record-Route: <sip:ibcf-hA2.home-A.net:5070;lr>,<sip:scscf-hA1.home-A.net:5070;lr>,

<sip:as-hA1.home-A.net:5070;lr>,<sip:scscf-hA1.home-A.net:5070;lr>,

<sip:ibcf-hA1.home-A.net:5070;lr>,<sip:-A1.interconnection-A.net;lr>,

<sip:ibcf-vA1.visited-A.net;lr>,<sip:pcscf-vA1.visited-A.net;lr>

P-Charging-Vector: icid-value="1234bc9876e";icid-generated-at=5555::aaa:bbb:ccc:eeee";orig-ioi="home-a"; transit-ioi="any-IC-network"; term-ioi="home-b"

Other SIP headers and the SDP settings according to 3GPP TS 24.229 [4].

P-Charging-Vector: 	The settings of the ioi values are important:

-	in step 9 the "term-ioi"header field parameter includes a type-2 IOI value which identifies the home network of the terminating user. The "transit-ioi" header field parameter includes one or more values identifying the transit networks passed. The "orig-ioi" is set to the same type-2 IOI value that was included in the initial INVITE request by the home network A towards the terminating user;

-	in step 10 the "term-ioi" includes a type-3 IOI value (prefix "Type 3") which identifies the S-CSCF-hA1 network. The "orig-ioi" is set to the same type-3 IOI value that was included in the initial INVITE request from the AS-hA1 to the S-CSCF;

-	in step 11 the "term-ioi" includes a type-3 IOI value (prefix "Type 3") which identifies the AS-hA1 network. The "orig-ioi" is set to the same type-3 IOI value that was included in the initial INVITE request from the S-CSCF to the AS-hA1; and

-	in steps 12-15 the "term-ioi" header field parameter includes a type-1 IOI value (prefix "Type 1") which identifies the home network A. The "transit-ioi"header field parameter includes one or more values identifying the transit networks passed. The "orig-ioi" is set to the same type-1 IOI value that was included in the initial INVITE request from the visited network A towards the home network A.

## 5.4	Originating roaming with loopback

### 5.4.1	General

This use case shows the loopback procedure that includes routeing from originating visited network to originating home network and loopback routeing from originating home network to originating visited network including the traversal of transit network between the visited and the originating home network.

Signalling is prepared to skip the loopback and route directly to the terminating home network on demand of the originating home network, see the use case provided by subclause 5.3.

### 5.4.2	Call flowf

In this scenario the originating home network decides to use loopback routeing and OMR is used.

Preconditions:

-	the user A is registered to the home network hA via the visited operator vA network and the IC network A. The IC network does not store any registration information;

-	the called user is located in another network either in PSTN or in IMS;

-	P-CSCF-vA1 and S-CSCF-hA1 support loopback routeing;

-	IBCF-vA1, IBCF-hA1 and IC-A1 supports OMR and all networks supports loopback routeing; and

-	no B2BUA manipulating SDP is connected in the home operator hA network.

SDP behaviour for loopback including OMR is described within 3GPP TS 29.079 [5].

Editor's Note: Only INVITE are shown. Further consideration for the responses is needed.

NOTE:	For clarity, the SIP 100 (Trying) messages are not shown in the signalling flow.

Figure 5.4.2-1: MOC with loopback and media home routing

The steps of the flow are as follows:

1.	INVITE request (UE-A to P-CSCF-vA1) - see example in table 5.4.2-1.

The user A at UE-A initiates a call. The UE-A sends an initial INVITE request to the P-CSCF-vA1 according to 3GPP TS 24.229 [4].

Table 5.4.2-1: INVITE request (UE-A to P-CSCF-vA1)

INVITE tel:+4687197378 SIP/2.0

Route: <sip:pcscf-vA1.visited-A.net;lr>,<sip:home-abc@scscf-hA1.home-A.net;lr>

P-Access-Network-Info: 3GPP-E-UTRAN-TDD; utran-cell-id-3gpp=234151D0FCE11

Via: SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd];branch=z9hG4bKnashds7

From: <sip:userA_public1@homeA.net>;tag=4fa3

Other SIP header fields and the SDP are set according to 3GPP TS 24.229 [4].

Route:	The UA builds a proper preloaded route set for all new dialogs and standalone transactions containing the IP address or the FQDN learnt through the P-CSCF discovery procedures and the P-CSCF port based on the security mechanism in use. In addition the values received in the Service-Route header field saved from the 200 (OK) response to the last registration or re-registration of the public user identity with associated contact address are added. The content of the received Service-Route header filed is due to operator policy. In this examplee only the S-CSCF-hA1 will be included.

2.	INVITE request (P-CSCF-vA1 to IBCF-vA1) - see example in table 5.4.2-2.

When the P-CSCF-vA1 receives the initial INVITE request and the P-CSCF:

-	adds the Feature-Caps header field with the "g.3gpp.trf" header field parameter with the address to the TRF; and

-	verifies that the resulting list of Route header fields matches the list of URIs received in the Service-Route header field (during the last successful registration or re-registration). This verification is done on a per URI basis, not as a whole string.

The P-CSCF-vA1 selects an IBCF (IBCF-vA1) to be the exit IBCF towards the home IMS network hA and sends the INVITE request according to 3GPP TS 24.229 [4] to IBCF-vA1.

NOTE 1:	It is implementation dependent as to how the P-CSCF obtains the address of the IBCF exit point.

Table 5.4.2-2: INVITE request (P-CSCF-vA1 to IBCF-vA1)

INVITE tel:+4687197378 SIP/2.0

Route: <sip:ibcf-vA1.visited-A.net;lr>,<sip:home-abc@scscf-hA1.home-A.net;lr>

Record-Route: <sip:pcscf-vA1.visited-A.net;lr>

Feature-Caps:*;+g.3gpp.trf="<sip:trf1-vA1.visited-A.net;lr>"

P-Access-Network-Info:3GPP-E-UTRAN-TDD;utran-cell-id-3gpp=234151D0FCE11,

3GPP-E-UTRAN-TDD; utran-cell-id-3gpp=234151D0FCE11;

network-provided;local-time-zone="UTC+01:00";daylight-saving-time="01"

P-Charging-Vector:icid-value="AyretyU0dm+6O2IrT5tAFrbHLso=023551024";orig-ioi="Type 1visited-a"

Via: SIP/2.0/UDP <visit-xyz@pcscf-A1.visited-A.net:5060;branch=z9hG4bKnas56565>,

Via: SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd];branch=z9hG4bKnashds7

Other SIP header fields and the SDP are set according to 3GPP TS 24.229 [4]

3.	INVITE request (IBCF-vA1 to IC-A1) - see example in table 5.4.2-3.

When the IBCF-vA1 receives the initial INVITE request containing the "+g.3gpp.trf" header field parameter in a Feature-Caps header field, the IBCF-vA1 updates the SDP as described within 3GPP TS 29.079 [5].

The IBCF-vA1selects to route the INVITE request via an IC network and sends the INVITE request according to 3GPP TS 24.229 [4] to the IC-A1.

NOTE 2:	If IBCF processes a request without a pre-defined route (e.g. the subscription to "reg" event package originated by the P-CSCF), the next-hop address can be either obtained as specified in IETF RFC 3263 [13] or be provisioned in the IBCF.

Table 5.4.2-3: INVITE request (IBCF-vA1 to IC-A1)

INVITE tel:+4687197378 SIP/2.0

Route: <sip:ic-A1.interconnection-A.net;lr>,<sip:home-abc@scscf-hA1.home-A.net;lr>

Record-Route: <sip:ibcf-vA1.visited-A.net;lr>,<sip:pcscf-vA1.visited-A.net;lr>

Feature-Caps:*;+g.3gpp.trf="<sip:trf1-vA1.visited-A.net;lr>"

P-Access-Network-Info:

P-Charging-Vector:icid-value="AyretyU0dm+6O2IrT5tAFrbHLso=023551024";orig-ioi="Type 1visited-a"

Via: SIP/2.0/UDP <ibcf-vA1.visited-A.net;branch=z9hG4bK351g45.1>

Via: SIP/2.0/UDP <visit-xyz@pcscf-A.visited-A.net:5060;branch=z9hG4bKnas56565>

Via: SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd];branch=z9hG4bKnashds7

Max-Forwards: 68

Other SIP header fields and the SDP are set according to 3GPP TS 24.229 [4] and SDP for OMR is set according to 3GPP TS 29.079 [5].

Record-Route: 	Based on topology hiding functionality the IBCF-vA1 will either:

-	forward the Record-Route header field with one visible IBCF-vA1 entry and the P-CSCF entry as one hidden entry; or

-	an IBCF-vA1 entry and the P-CSCF entry as a visible entry.

Via:	Based on topology hiding functionality the IBCF-vA1 will either:

-	forward the Via header field with one visible IBCF-vA1 Via header field and the P-CSCF Via header field as a hidden Via header field; or

-	an IBCF-vA1 Via header field and the P-CSCF Via header field.

4.	INVITE request (IC-A1 to IBCF-hA1) - see example in table 5.4.2-4.

When the IC-A1 receives the initial INVITE request and since the IC-A1 according to local policy allows itself to be bypassed by media then the IC-A1 updates the SDP as described within 3GPP TS 29.079 [5].

The IC-A1 selects an entry IBCF-hA1 of the home IMS network hA and sends the INVITE request according to 3GPP TS 24.229 [4] to the IBCF-hA1.

Table 5.4.2-4: INVITE request (IC-A1 to IBCF-hA1)

INVITE tel:+4687197378 SIP/2.0

Route: <sip:ibcf-hA1.home-A.net;lr>,<sip:home-abc@scscf-hA1.home-A.net;lr>

Record-Route: <sip:ic-A1.interconnection-A.net;lr>,<sip:ibcf-vA1.visited-A.net;lr>,

<sip:pcscf-vA1.visited-A.net;lr>

Feature-Caps:*;+g.3gpp.trf="<sip:trf1-vA1.visited-A.net;lr>"

P-Access-Network-Info:

P-Charging-Vector: icid-value="AyretyU0dm+6O2IrT5tAFrbHLso=023551024";orig-ioi="Type 1visited-a"

Via: SIP/2.0/UDP <ic-A1.interconnection-A.net;branch=z9hG4bK351g45.1>

Via: SIP/2.0/UDP <ibcf-vA1.visited-A.net;branch=z9hG4bK351g45.1>

Via: SIP/2.0/UDP <visit-xyz@pcscf-vA1.visited-A.net:5060;branch=z9hG4bKnas56565>

Via: SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd];branch=z9hG4bKnashds7

Other SIP header fields and the SDP are set according to 3GPP TS 24.229 [4] and SDP for OMR is set according to 3GPP TS 29.079 [5].

Record-Route: 	Based on interconnection operator SLA the IC-A1 adds an entry to the Record-Route header field or not.

Editor's note: Improvement needed what IC-A1 is allowed to do and what is based on SLA.

5.	INVITE request (IBCF-hA1 to S-CSCF-hA1) - see example in table 5.4.2-5.

When the IBCF-hA1 receives the initial INVITE request containing the "+g.3gpp.trf" header field parameter in a Feature-Caps header field, the IBCF-hA1 updates the SDP as described within 3GPP TS 29.079 [5].

The IBCF-hA1 sends the INVITE request according to 3GPP TS 24.229 [4] to the S-CSCF.

Table 5.4.2-5: INVITE request (IBCF-hA1 to S-CSCF-hA1)

INVITE tel:+4687197378 SIP/2.0

Route: <sip:home-abc@scscf-hA1.home-A.net;lr>

Record-Route: <sip:ibcf-hA1.home-A.net:5070;lr>,<sip:ic-A1.interconnection-A.net;lr>,

<sip:ibcf-vA1.visited-A.net;lr>,<sip:pcscf-vA1.visited-A.net;lr>

Feature-Caps:*;+g.3gpp.trf="<sip:trf1-vA1.visited-A.net;lr>"

P-Access-Network-Info:

P-Charging-Vector: icid-value="AyretyU0dm+6O2IrT5tAFrbHLso=023551024";orig-ioi="Type 1visited-a";transit-ioi="ICa.1"

Via: SIP/2.0/UDP <ibcf-hA1.home-A.net;branch=z9hG4bK351g45.1>

Via: SIP/2.0/UDP <ic-A1.interconnection-A.net;branch=z9hG4bK351g45.1

Via: SIP/2.0/UDP <ibcf-vA1.visited-A.net;branch=z9hG4bK351g45.1>

Via: SIP/2.0/UDP <visit-xyz@pcscf-vA1.visited-A.net:5060;branch=z9hG4bKnas56565>

Via: SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd];branch=z9hG4bKnashds7

Other SIP header fields and the SDP are set according to 3GPP TS 24.229 [4] and SDP for OMR is set according to 3GPP TS 29.079 [5].

P-Charging-Vector:	The IBCF-hA1 adds the "transit-ioi" header field.

6.	INVITE request (S-CSCF-hA1 to AS-hA1) - see example in table 5.3.2-6.

The S-CSCF-hA1 applies the same procedure as in step 6 in subclause 5.3.2.

7.	INVITE request (AS-hA1 to S-CSCF-hA1) - see example in table 5.4.2-7.

AS-hA1 applies the service of the originating user and removes the first Route header field entry.

The AS-hA1 removes the "orig-ioi" header field parameter from the INVITE request and inserts a type 3 "orig-ioi" header field parameter to a value that identifies the service provider from which the request is sent. In this case the IOI value is the same as the home network operator identity.

The specifics of the service does not require access to media and the "+g.3gpp.home-visited" Feature-Caps header field parameter is kept in the INVITE request. This is an indication to the S-CSCF-hA1 that a loopback is a viable option.

All other SIP headers are set due to procedures described within 3GPP TS 24.229 [4].

Editor's note: The behaviour of the transit-ioi value is currently discussed and needs further consideration how the S-CSCF to AS correlation is describes.

Editor's note: Due to service behaviour the AS could also be a server of a third party service provider. Further consideration of AS scenarios is needed

Table 5.4.2-7: INVITE request (AS-hA1 to S-CSCF-hA1)

INVITE tel:+4687197378 SIP/2.0

Route:<sip:orig@scscf-hA1.home-A.net;lr>

Record-Route:<sip:as-hA1.home-A.net:5070;lr>,<sip:scscf-hA1.home-A.net:5070;lr>,

<sip:ibcf-hA1.home-A.net:5070;lr>,<sip:ic-A1.interconnection-A.net;lr>,

<sip:ibcf-vA1.visited-A.net;lr>,<sip:pcscf-vA1.visited-A.net;lr>

Feature-Caps:*;+g.3gpp.trf="<sip:trf1.visitedA.net>",*;+g.3gpp.home-visited="<visited_A>"

P-Access-Network-Info:3GPP-E-UTRAN-TDD;utran-cell-id-3gpp=234151D0FCE11

P-Charging-Vector: icid-value="AyretyU0dm+6O2IrT5tAFrbHLso=023551024";orig-ioi="Type 3home-hA.net"

Via: SIP/2.0/UDP <AS-hA1.home-A.net;branch=z9hG4bK351g45.1>

Via: SIP/2.0/UDP <scscf-hA1.home-A.net;branch=z9hG4bK351g45.1>

Via: SIP/2.0/UDP <ibcf-hA1.home-A.net;branch=z9hG4bK351g45.1>

Via: SIP/2.0/UDP <ic-A1.interconnection-A.net;branch=z9hG4bK351g45.1

Via: SIP/2.0/UDP <ibcf-vA1.visited-A.net;branch=z9hG4bK351g45.1>

Via: SIP/2.0/UDP <visit-xyz@pcscf-vA1.visited-A.net:5060;branch=z9hG4bKnas56565>

Via: SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd];comp=sigcomp;branch=z9hG4bKnashds7

Other SIP headers and the SDP are set according to 3GPP TS 24.229 [4] and SDP for OMR is set according to 3GPP TS 29.079 [5]

P-Charging-Vector:	The P-Charging-Vector header field containing the "orig-ioi" header field parameter. The AS-hA1 shall set the type 3 "orig-ioi" header field parameter to a value that identifies the sending network (i.e. home network hA). The AS has to set itself a type 3 ioi if the AS is located within another domain (i.e. 3rd Party Service Provider). If more than one AS is within the path the orig-ioi values could change based on the relation between S-CSCF-hA1 and the related AS.

Record-Route:	This entries to this header needs to be added based on the related dip's to the AS. The AS could behave as B2BUA in such case the AS could store the entries and setup a new Record-Route header field with the AS as the only entry.

Editor's Note: The behaviour of the transit-ioi value is currently discussed and needs further consideration how the S-CSCF to AS correlation is describes.

Editor's Note: It has to be clarified if for transit-ioi also the type should be prefixed as for orig-ioi and term-ioi.

8.	INVITE request (S-CSCF-hA1 to IBCF-hA1) - see example in table 5.4.2-8.

When the S-CSCF-hA1 receives the initial INVITE request and the "g.3gpp.home-visited" Feature-Caps header field parameter is still in the request, the S-CSCF-hA1 decides according to local policy that loopback routeing will be used.

The S-CSCF-hA1 includes:

-	the TRF URI included in the "+g.3gpp.trf" Feature-Caps header field parameter in the Route header field;

-	a Feature-Caps header field with the "+g.3gpp.loopback" header field parameter to indicate that loopback routeing is ongoing. The "+g.3gpp.loopback" header field parameter can optionally contain the name of the home network; and

-	the orig-ioi within the P-Charging-Vector header field will be replaced by a type-1 IOI which reflects the home network of user A (Ha.operator-hA1.net). Any transit-ioi for the path P-CSCF-vA1 to S-CSCF-hA1 will not be maintained after this point.

The S-CSCF-hA1 removes the "+g.3gpp.home-visited" Feature-Caps header field parameter.

NOTE 3:	Number normalization and ENUM translation is deferred to the visited network according to local policy.

The INVITE request is sent according to 3GPP TS 24.229 [4] to the IBCF-hA1.

Table 5.4.2-8: INVITE request (S-CSCF-hA1 to IBCF-hA1)

INVITE tel:+4687197378 SIP/2.0

Route: <sip:ibcf-hA1.home-A.net;lr>,<sip:trf-vA1.visited-A.net;lr>

Record-Route: <sip:scscf-hA1.home-A.net:5070;lr>,<sip:as-hA1.home-A.net:5070;lr>,

<sip:scscf-hA1.home-A.net:5070;lr>,<sip:ibcf-hA1.home-A.net:5070;lr>,

<sip:ic-A1.interconnection-A.net;lr>,<sip:ibcf-vA1.visited-A.net;lr>,

<sip:pcscf-vA1.visited-A.net;lr>

Feature-Caps:*;+g.3gpp.loopback="homenetwork_A"

P-Access-Network-Info:

P-Charging-Vector: icid-value="AyretyU0dm+6O2IrT5tAFrbHLso=023551024";orig-ioi="Type 3home-a"

Via: SIP/2.0/UDP <scscf-hA1.home-A.net;branch=z9hG4bK351g45.1>

Via: SIP/2.0/UDP <AS-hA1.home-A.net;branch=z9hG4bK351g45.1>

Via: SIP/2.0/UDP <scscf-hA1.home-A.net;branch=z9hG4bK351g45.1>

Via: SIP/2.0/UDP <ibcf-hA1.home-A.net;branch=z9hG4bK351g45.1>

Via: SIP/2.0/UDP <ic-A1.interconnection-A.net;branch=z9hG4bK351g45.1

Via: SIP/2.0/UDP <ibcf-vA1.visited-A.net;branch=z9hG4bK351g45.1>

Via: SIP/2.0/UDP <visit-xyz@pcscf-vA1.visited-A.net:5060;branch=z9hG4bKnas56565>

Via: SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd];branch=z9hG4bKnashds7

Other SIP header fields and the SDP are set according to 3GPP TS 24.229 [4] and the SDP for OMR is set according to 3GPP TS 29.079 [5].

P-Charging-Vector:	The P-Charging Vector header field containing the "orig-ioi" header field parameter. The S-CSCF shall set the type 1 "orig-ioi" header field parameter to a value that identifies the sending network (i.e. home network A). 
Thus the orig-ioi value is now different compared with the received INVITE at the IBCF-hA1 from the visited network.

9.	INVITE request (IBCF-hA1 to IC-A1) - see example in table 5.4.2-9.

When the IBCF-hA1 receives the initial INVITE request containing the "+g.3gpp.loopback" header field parameter in a Feature-Caps header field, the IBCF-hA1 updates the SDP as defined within 3GPP TS 29.079 [5].

The IBCF-hA1 selects to route the INVITE request via an IC network and sends the INVITE request according to 3GPP TS 24.229 [4] to the IC-A1. From theory it could be also another IC but that is due to SLA and operator policy.

Table 5.4.2-9: INVITE request (IBCF-hA1 to IC-A1)

INVITE tel:+4687197378 SIP/2.0

Route: <sip:ic-A1.interconnection-A.net;lr>,<sip:trf-vA1.visited-A.net;lr>

Record-Route: <sip:ibcf-hA1.home-A.net:5070;lr>,<sip:scscf-hA1.home-A.net:5070;lr>,

<sip:as-hA1.home-A.net:5070;lr>,<sip:scscf-hA1.home-A.net:5070;lr>,

<sip:ibcf-hA1.home-A.net:5070;lr>,<sip:ic-A1.interconnection-A.net;lr>,

<sip:ibcf-vA1.visited-A.net;lr>,<sip:pcscf-vA1.visited-A.net;lr>

Feature-Caps:*;+g.3gpp.loopback="homenetwork_A"

P-Access-Network-Info:

P-Charging-Vector: icid-value="AyretyU0dm+6O2IrT5tAFrbHLso=023551024";orig-ioi="Type 1home-a"

Via: SIP/2.0/UDP <ibcf-hA1.home-A.net;branch=z9hG4bK351g45.1>

Via: SIP/2.0/UDP <scscf-hA1.home-A.net;branch=z9hG4bK351g45.1>

Via: SIP/2.0/UDP <AS-hA1.home-A.net;branch=z9hG4bK351g45.1>

Via: SIP/2.0/UDP <scscf-hA1.home-A.net;branch=z9hG4bK351g45.1>

Via: SIP/2.0/UDP <ibcf-hA1.home-A.net;branch=z9hG4bK351g45.1>

Via: SIP/2.0/UDP <ic-A1.interconnection-A.net;branch=z9hG4bK351g45.1

Via: SIP/2.0/UDP <ibcf-vA1.visited-A.net;branch=z9hG4bK351g45.1>

Via: SIP/2.0/UDP <pcscf-vA1.visited-A.net:5060;branch=z9hG4bKnas56565>

Via: SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd];branch=z9hG4bKnashds7

Other SIP header fields and the SDP are set according to 3GPP TS 24.229 [4] and the SDP for OMR is according to 3GPP TS 29.079 [5].

Record-Route:	The entries to this header field need to be added based on operator policy. The example shows the possible Record-Route header field when no restriction according to the operator policy applies.

Based on topology hiding functionality the IBCF-vA1 will either:

-	forward the Record-Route header field with one visible IBCF-vA1 entry and the remaining collected entries as one hidden entry; or

-	an IBCF-vA1 entry and whole content of already collected entries as visible entries.

Via:	Based on topology hiding functionality the IBCF-vA1 will either:

-	forward the Via header field with one visible IBCF-vA1 Via header field and the remaining collected set of Via header fields as one hidden Via header field; or

-	an IBCF-vA1 Via header field and the whole set of already collected Via header fields.

10.	INVITE request (IC-A1 to IBCF-vA1).

When the IC-A1 receives the initial INVITE request the IC-A1 detects that an optimal media path avoiding media to be sent through the home IMS network hA is possible. The IC-A1 updates the SDP according to 3GPP TS 29.079 [5].

The IC-A1 selects an entry IBCF in the visited network and sends the INVITE request according to 3GPP TS 24.229 [4] to the IBCF-vA1. Seen from principle point of view the IBCF could be any entry point (IBCF-vAx) of the visited network.

The content of the Record-Route header field and the Via header field will be based on local policy and updated accordingly.

11.	INVITE request (IBCF-vA1 to TRF).

When the IBCF-vA1 receives the initial INVITE request containing the "+g.3gpp.loopback" header field parameter in a Feature-Caps header field, the IBCF-vA1 searches for a possible optimal media path and detects that an optimal media path within the visited network is possible. The IBCF-vA1 updates the SDP according to 3GPP TS 29.079 [5].

The IBCF-vA1 sends the INVITE request according to 3GPP TS 24.229 [4] to the TRF-vA1.

The content of the Record-Route header field and the Via header field will be based on local policy and updated accordingly.

12.	INVITE request (TRF to IBCF-vA1).

When the TRF receives the initial INVITE request the TRF:

-	performs number normalization (if used by local policy) and ENUM translation and possibly and updates the request-URI if necessary;

-	replaces an type 2 orig-ioi parameter value in the P-Charging-Vector header field with the value identifying the visited network;

-	possibly adds an additional Route header field pointing to the IBCF-A1 and also to the IC in cases where the IBCF-vA1 has no routeing decision to do; and

-	removes the "+g.3gpp.loopback" header field parameter from the Feature-Caps header field of the outgoing request.

The TRF selects an IBCF to act as an exit point and sends the INVITE request according to 3GPP TS 24.229 [4] to the IBCF-vA1 (which could be also another IBCF e.g. IBCF-vA2).

The content of the Record-Route header field and the Via header field will be based on local policy and updated accordingly.

13.	INVITE request (IBCF-vA1 to IC-B1) - see example in table 5.4.2-13.

When the IBCF-vA1 receives the initial INVITE request the IBCF-vA1 selects the IC-B1 if not already a Route header field exists pointing to the IC-B1.

Record-Route header field and Via header field will be based on local policy and updated accordingly. The example in table 5.4.2-13 shows one example of the content of the headers and the text below the figure explains other possibilities.

Table 5.4.2-13: INVITE request (IBCF-vA1 to IC-B1)

INVITE sip:+4687197378@home-B.net;user=phone SIP/2.0

Route:<sip:ic-B1.interconnection-B.net;lr>

Record-Route: <sip:ibcf-vA1.visited-A.net:5070;lr>,<sip:trf-vA1.visited-A.net:5070;lr>,

<sip:ibcf-vA1.visited-A.net:5070;lr>,<sip:ic-A1.interconnection-A.net;lr>,

<sip:ibcf-hA1.home-A.net:5070;lr>,<sip:scscf-hA1.home-A.net:5070;lr>,

<sip:as-hA1.home-A.net:5070;lr>,<sip:scscf-hA1.home-A.net:5070;lr>,

<sip:ibcf-hA1.home-A.net:5070;lr>,<sip:ic-A1.interconnection-A.net;lr>,

<sip:ibcf-vA1.visited-A.net;lr>,<sip:pcscf-vA1.visited-A.net;lr>

P-Access-Network-Info:

P-Charging-Vector:icid-value="AyretyU0dm+6O2IrT5tAFrbHLso=023551024";orig-ioi="visited-a"

Via: SIP/2.0/UDP <ibcf-hA1.home-A.net;branch=z9hG4bK351g45.1>

Via: SIP/2.0/UDP <trf-hA1.home-A.net;branch=z9hG4bK351g45.1>

Via: SIP/2.0/UDP <ibcf-hA1.home-A.net;branch=z9hG4bK351g45.1>

Via: SIP/2.0/UDP <ic-hA1.home-A.net;branch=z9hG4bK351g45.1>

Via: SIP/2.0/UDP <ibcf-hA1.home-A.net;branch=z9hG4bK351g45.1>

Via: SIP/2.0/UDP <scscf-hA1.home-A.net;branch=z9hG4bK351g45.1>

Via: SIP/2.0/UDP <AS-hA1.home-A.net;branch=z9hG4bK351g45.1>

Via: SIP/2.0/UDP <scscf-hA1.home-A.net;branch=z9hG4bK351g45.1>

Via: SIP/2.0/UDP <ibcf-hA1.home-A.net;branch=z9hG4bK351g45.1>

Via: SIP/2.0/UDP <ic-A1.interconnection-A.net;branch=z9hG4bK351g45.1

Via: SIP/2.0/UDP <ibcf-vA1.visited-A.net;branch=z9hG4bK351g45.1>

Via: SIP/2.0/UDP <pcscf-vA1.visited-A.net:5060;branch=z9hG4bKnas56565>

Via: SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd];branch=z9hG4bKnashds7

Other SIP header fields and the SDP are set according to 3GPP TS 24.229 [4] and SDP for OMR is set according to 3GPP TS 29.079 [5].

Record-Route: 	Based on topology hiding functionality the IBCF-vA1 will either:

-	forwards the Record-Route header field with one visible IBCF-vA1 entry and the remaining collected entries as one hidden entry; or

-	an IBCF-vA1 entry and whole content of already collected entries as visible entries.

Via:	Based on topology hiding functionality the IBCF-vA1 will either:

-	forward the Via header field with one visible IBCF-vA1 Via header field and the remaining collected set of Via header fields as one hidden Via header field; or

-	an IBCF-vA1 Via header field and the whole set of already collected Via header fields.

## 5.5	Routeing when the originating and terminating user reside in the same home network

This subclause describes routeing when the originating and terminating user reside in the same home network.

## 5.6	Routeing from originating home to terminating home network

This subclause describes routeing from the originating home network to the terminating home network including transit network scenarios.

Editor's Note: anything else would be a non-roaming use case which is out of scope of this study.

The IC and routing aspects of this use case do not differ from any other session set-up between any two wire-line or mobile networks. The IC might be established via CS-IC or PS-IC. Routing will be based on the destination given by the request line (Called Party Number in case of CS-IC).

## 5.7	Routeing from originating visited network to terminating home network

This subclause describes routeing from the originating visited network to the terminating home network including transit network scenarios.

The signalling and media path between originating visited network to terminating home network might be based on SIP/PS or CS/TDM interconnection interfaces.

NOTE 1: 	This is different from some other IC scenarios discussed in this paper for which SIP/PS interconnection is a mandatory pre-requisite.

The scenario appears after the "originating roaming with loopback" according to subclause 5.4 has taken place. I.e. the call has already been set-up from the P-CSCF in the originating visited network via IMS in the originating home network back to the TRF in the originating visited network.

From viewpoint of the originating visited network it doesn’t matter whether the call originates from an own user served by an own S-CSCF or from a roaming inbound user served by the TRF. In both cases the originating visited network will set-up an outgoing call leg via an IBCF acting as an exit point (MGCF, if CS IC applies) towards the destination network. Hereby the path might be established via IC networks. The destination (i.e. in this case the address of the terminating home network) used for routing is given by the request line provided by the originating visited network (CdPty in case of CS IC).

From the viewpoint of the IC network and from the viewpoint of the terminating home network the routing and call handling does not differ from calls originated in any other network. I.e. the fact that the call is originated by a roamer is neither visible nor required/used for any proceeding.

NOTE 2: 	Even if a calling party identity (P-Asserted-Identity header field, From header field) is present and not within the number range of the originating visited network this does not indicate that the originating party is an inbound roamer within the originating visited network which sets up the call. Number portability and forwarding services might result in the same effect. In any case a mismatch between the calling party identity and address range of the originating visited network does not result in any deviating routing procedures within IC or terminating home network.

## 5.8	Routeing in terminating home network when user resides in this network

This subclause describes the routeing in terminating home network when user resides in this network.

Editor's Note: It looks like that this is a non roaming use case which is out of scope of this study. The proceeding is straight forward legacy proceeding.

## 5.9	Routeing from terminating home network to the terminating visited network

This subclause describes routeing from terminating home network to the terminating visited network including the traversal of transit network between the terminating home network and the visited network.

Editor's Note: Further details to be worked out. (besides straight forward approach according to given VoLTE concepts an alternative optimisation without need for specific IC-Handling in IC-networks might be useful)

## 5.10	Insertion of service related media in the originating home network

### 5.10.1	General

This subclause describes insertion of service related media in the originating home network (i.e. invocation of MRFC/MRF).

3GPP TS 24.628 [22] describes how to include announcements into the path for IMS. There are a couple of options which have to be considered for Roaming.

The following sections are showing some examples where service related media can be included. The examples will not cover all possible scenarios defined within 3GPP.

### 5.10.2	Insertion of service related media in the originating home network with loopback

Figure 5.10.2-1: Announcement started during the establishment of a communication

The calling party initiates a communication by means of an INVITE request. The INVITE request is forwarded toward the called party.

Along the signalling path, created by the INVITE request, some service logic in an Application Server (AS) wants to send an announcement towards the calling party.

The flow is based on the assumptions that the Supported header field includes the option-tag "100rel".

Editor's Note: Is this use case within the scope or should that not happen?

Editor's Note: the use of preconditions has to be documented in addition

The steps of the signalling flow are as follows:

1.-5.	INVITE request (UE to S-CSCF-hA)

INVITE is set as described within subclause 5.3.2	Originating roaming to home network without loopback.

6.		Evaluation of initial filter criteria

S-CSCF-hA validates the service profile of this subscriber and evaluates the initial filter criteria. For this example, assume an Application Server/MRFC involvement.

7.		INVITE request (S-CSCF-hA to MFRC) - see example in table 5.10-1

8.		Service logic in AS/MRFC

Service logic in the AS decides to send an announcement to the calling party. In such a case the AS acts a B2BUA. Thus an early dialog for the announcement  and two dialogs will be created in the final call state one from UE-A to the AS and one from the AS to the terminating UE which allows end-to-end communication.

9.		MRFC-MRFP interaction

The MRFC interacts with the MRFP in order to reserve resources for the announcement. As part of the interaction with MRFP the AS receives the necessary media parameters e.g. IP address and port numbers and provide the IP address and port number for the calling party to the MRFP.

10.-15 	183 (AS to UE)

The AS sends a 183 (Session progress) response to S-CSCF. The response includes:

a)	an answer to the SDP received in the INVITE request;

b)	a P-Early-Media header field set to "sendonly";in case where only Announcements will be played. Invoking an interactive voice response system a P-Early-Media header field set to "sendrecv";

c)	the Require header field set to "100rel".

16.		 PRACK/200 OK (AS to UE)

17.		MRFC-MRFP interaction

The MRFC interacts with the MRFP in order to start the announcement.

18.		MRFC-MRFP interaction

The MRFP sends the announcement towards the calling party.

19.		MRFC-MRFP interaction

The complete announcement is sent and the MRFP interacts with the AS/MRFC in order to inform that the announcement is terminated.

20.		MRFC-MRFP interaction

The MRFC interacts with the MRFP in order to release the resources used for the announcement.

21.-26.	199 response (AS-hA to UE-A) (optional)

If the originating UE has indicated support of the 199 (Early Dialog Terminated) response code, the AS sends a 199 (Early Dialog Terminated) provisional response towards the originating UE.

27.		INVITE request (AS-hA to S-CSCF)

The AS sends the INVITE request towards the S-CSCF. The INVITE request contains the same information as the INVITE request received in step 7 with the modification done by AS according to rules and procedures of 3GPP TS 24.229 [4].

28.-33.	INVITE request (S-CSCF to IC-B1)

The S-CSCF decides to loopback the INVITE and forward the call to the terminating home network.

The SDP contained within the INVITE will be manipulated due to the example shown within 3GPP TR 29.079 [5]

34		180/200 OK/ ACK (IC-B1 to UE)

When the UE of the calling party receives the 200 (OK) response to the INVITE request the UE can regard the early dialog created for the announcement between the UE and the AS terminated.

### 5.10.3	Insertion of service related media in the originating home network without loopback

Figure 5.10.3-1: Announcement started during the establishment of a communication

Editor's Note: the use of preconditions has to be documented in addition

The steps of the signalling flow are as follows:

1.-5.	INVITE request (UE to S-CSCF-hA)

INVITE is set as described within subclause 5.3.2	Originating roaming to home network with loopback.

6.		Evaluation of initial filter criteria

S-CSCF-hA validates the service profile of this subscriber and evaluates the initial filter criteria. For this example, assume an Application Server/MRFC involvement.

7.		INVITE request (S-CSCF-hA to MFRC) - see example in table 5.10-1

8.		Service logic in AS/MRFC

Service logic in the AS decides to send an announcement to the calling party. In such a case the AS acts a B2BUA. Thus an early dialog for the announcement  and two dialogs will be created in the final call state one from UE-A to the AS and one from the AS to the terminating UE which allows end-to-end communication.

9.		MRFC-MRFP interaction

The MRFC interacts with the MRFP in order to reserve resources for the announcement. As part of the interaction with MRFP the AS receives the necessary media parameters e.g. IP address and port numbers and provide the IP address and port number for the calling party to the MRFP.

10.-15 	183 (AS to UE)

The AS sends a 183 (Session progress) response to S-CSCF. The response includes:

a)	an answer to the SDP received in the INVITE request;

b)	a P-Early-Media header field set to "sendonly";in case where only Announcements will be played. Invoking an interactive voice response system a P-Early-Media header field set to "sendrecv";

c)	the Require header field set to "100rel".

16.		 PRACK/200 OK (AS to UE)

17.		MRFC-MRFP interaction

The MRFC interacts with the MRFP in order to start the announcement.

18.		MRFC-MRFP interaction

The MRFP sends the announcement towards the calling party.

19.		MRFC-MRFP interaction

The complete announcement is sent and the MRFP interacts with the AS/MRFC in order to inform that the announcement is terminated.

20.		MRFC-MRFP interaction

The MRFC interacts with the MRFP in order to release the resources used for the announcement.

21.-26.	199 response (AS-hA to UE-A) (optional)

If the originating UE has indicated support of the 199 (Early Dialog Terminated) response code, the AS sends a 199 (Early Dialog Terminated) provisional response towards the originating UE.

27.		INVITE request (AS-hA to S-CSCF)

The AS sends the INVITE request towards the S-CSCF. The INVITE request contains the same information as the INVITE request received in step 7 with the modification done by AS according to rules and procedures of 3GPP TS 24.229 [4].

28.		INVITE request (S-CSCF to IC-B1)

The S-CSCF decides not to loopback the INVITE and forward the call to the terminating home network.

29		180/200 OK/ ACK (IC-B1 to UE)

When the UE of the calling party receives the 200 (OK) response to the INVITE request the UE can regard the early dialog created for the announcement between the UE and the AS terminated.

## 5.11	PS to CS SRVCC access transfer scenarios

### 5.11.1	General

This subclause describes PS to CS SRVCC access transfer scenarios including the registration and the additional message flow due to PS to CS SRVCC.

For IMS roaming enhanced SRVCC procedure with media anchoring in the ATGWof the visited network and signalling anchored in the SCC-AS of the home network applies. From viewpoint of the remote side these signalling/media anchors do not change due to the SRVCC.

Editor's Note: In case of "originating roaming to home network without loopback" (see subclause 5.4) the media path between the visited network and the home network needs to be re-established since the SIP dialogue between the ATCF in the visited network and the SCC-AS in the home network is re-established. The original dialogue relates to the PS based source leg, the new dialogue relates to the CS based target leg. From the viewpoint of the IC network between the visited network and the home network this results in release of one connection and establishment of an other connection, whereby the IC network is not aware about any link between these two connections.

The 3GPP TS 24.237 [20] defines two types of MSC servers. One MSC server enhanced for ICS and another MSC server enhanced for SIP supporting PS to CS SRVCC. The message flows are the same if PS to CS SRVCC occurs so in the following subclauses both variants are referred to as "MSC server". The only difference between the two types is that when the MSC server is enhanced for ICS, the MSC server will register following the procedures in subclause 5.2 when the PS to CS SRVCC access transfer procedure is completed. After that calls initiated by the UE will look the same as described in subclause 5.3 and subclause 5.4.

### 5.11.2	PS to CS SRVCC information transfer during registration

This subclause describes the PS to CS SRVCC access transfer specifics during registration.

Preconditions to this flow are:

-	the conditions in subclause 5.2.1.1 are fulfilled; and

-	both the visited network support PS to CS access transfer and all MSC servers in the network are enhanced to use SIP signalling towards the IMS.

Figure 5.11.2-1 shows the message flow when a UE registers in IMS over PS before SRVCC occurs as described in subclause 5.11.3.

Figure 5.11.2-1: SRVCC information transfer during registration

The steps of the flow are as follows:

1.	The UE-A registers in IMS and receives a 401 (Unauthorized) response.

The flow is the same as in subclause 5.2.1.1 with the exception that the ATCF is included in the registration path.

2.	REGISTER request (UE-A to P-CSCF-vA1) - see example in table 5.11.2-2.

The UE-A sends a REGISTER request again (now with a authentication challenge response included) as described subclause 5.2.1.1.

Table 5.11.2-2: REGISTER request (UE-A to P-CSCF-vA1)

REGISTER sip:homeA.net SIP/2.0

From:<sip:userA_public1@homeA.net>;tag=4fa3

To:<sip:userA_public1@homeA.net>

P-Access-Network-Info:3GPP-E-UTRAN-TDD;utran-cell-id-3gpp=234151D0FCE11

Supported:path

Contact:<sip:[5555::aaa:bbb:ccc:ddd]>;reg-id=1;+sip.instance="<urn:gsma:imei:90420156-025763-0>";expires=600000;+g.3gpp.ics="principal";+g.3gpp.accesstype="cellular1"

Via:SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd];branch=z9hG4bKnashds7

Other SIP header fields are set according to 3GPP TS 24.229 [4] and 3GPP TS 24.237 [20].

3.	REGISTER request (P-CSCF-vA1 to ATCF-vA1) - see example in table 5.11.2-3.

When the P-CSCF-vA1 receives the REGISTER the P-CSCF-vA1 forwards the REGISTER request to ATCF-vA1 according to local policy. The content of the REGISTER request is not different that the content of the REGISTER request in subclause 5.2.1.1.

Table 5.11.2-3: REGISTER request (P-CSCF-vA1 to ATCF-vA1)

REGISTER sip:homeA.net SIP/2.0

From:

To:

P-Access-Network-Info:3GPP-E-UTRAN-TDD;utran-cell-id-3gpp=234151D0FCE11,3GPP-E-UTRAN-TDD; utran-cell-id-3gpp=234151D0FCE11;network-provided;local-time-zone="UTC+01:00";daylight-saving-time="01"

Supported:

Contact:

Path:<sip:visit-xyz@pcscf-vA1.visited-A.net:5070;lr>

P-Visited-Network-ID:"Visited Network A identifier"

P-Charging-Vector:icid-value="AyretyU0dm+6O2IrT5tAFrbHLso=023551024";orig-ioi="Type 1visited-a"

Via:SIP/2.0/UDP visit-xyz@pcscf-vA1.visited-A.net:5060;branch=z9hG4bKnas56565

Via:SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd];branch=z9hG4bKnashds7

Other SIP header fields are set according to 3GPP TS 24.229[4].

P-Access-Network-Info:	The P-CSCF adds the network provided location including the "local-time-zone" and "daylight saving time" header field parameters. The location information received from the UE-A is kept since it does not indicate that it is network provided.

Via:	The P-CSCF-vA1 adds a Via header field in the REGISTER request.

4.	REGISTER request (ATCF-vA1 to IBCF-vA1) - see example in table 5.11.2-4.

When the ATCF-vA1 receives the REGISTER request, the ATCF-vA1 updates its database for registrations and selects a IBCF according to local policy for routeing the REGISTER request towards the home network.

Table 5.11.2-4: REGISTER request (ATCF-vA1 to IBCF-vA1)

REGISTER sip:homeA.net SIP/2.0

From:

To:

P-Access-Network-Info:

Supported:

Contact:

Path:<sip:atcf-vA1.visited-A.net;lr>,<sip:visit-xyz@pcscf-vA1.visited-A.net:5070;lr>

P-Visited-Network-ID:

P-Charging-Vector:

Feature-Caps:*;+g.3gpp.atcf="tel:+1-237-555-3333",*;+g.3gpp.atcf-mgmt-uri="sip:atcf-vA1.visited-A.net",*;+g.3gpp.atcf-path="sip:termabc@atcf-vA1.visited-A.net",*;+g.3gpp.mid-call,*;+g.3gpp.srvcc-alerting,*;+g.3gpp.ps2cs-srvcc-orig-pre-alerting

Via:SIP/2.0/UDP <atcf-vA1.visited-A.net:5060;branch=z9hG4bKaa3689b>

Via:SIP/2.0/UDP <visit-xyz@pcscf-vA1.visited-A.net:5060;branch=z9hG4bKnas56565>

Via:SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd];branch=z9hG4bKnashds7

Other SIP header fields are set according to 3GPP TS 24.229[4] and 3GPP TS 24.237 [20].

Path:	The ATCF-vA1 adds its address for terminating requests in the Path header field.

Feature-Caps:	The ATCF-vA1 adds a number of feature-capability indicators in the Feature-Caps header field.

-	The g.3gpp.atcf capability-indicator includes the STN-SR that will be used during PS to CS access transfer by the MSC server.

-	The g.3gpp.atcf-mgmt-uri feature-capability indicator includes the URI that will be used by the SCC AS when sending the MESSAGE request with the PS to CS SRVCC information to the ATCF-vA1.

-	The g.3gpp.atcf-path feature-capability indicator is used by ATCF to identify the registration in its database for registrations and will be included by SCC AS in the MESSAGE request with the PS to CS SRVCC information sent to the ATCF-vA1.

-	The g.3gpp.mid-call, g.3gpp.srvcc-alerting and g.3gpp.ps2cs-srvcc-orig-pre-alerting is included to indicate which PS to CS SRVCC features that the visited network supports.

Via:	The ATCF-vA1 adds a Via header field in the REGISTER request.

5.	REGISTER request (IBCF-vA1 to S-CSCF-hA1 via IC-A1, IBCF-hA1 and I-CSCF-hA1) - see example in table 5.11.2-5.

When the IBCF-vA1 receives the REGISTER request, the IBCF-vA1 forwards the REGISTER request in the same way as in subclause 5.2.1.1. There are no more SRVCC specific information added in the REGISTER.

The table 5.11.2-5 shows the content of the REGISTER request when received in S-CSCF-hA1.

Table 5.11.2-5: REGISTER request (IBCF-vA1 to S-CSCF-hA1 via IC-A1, IBCF-hA1 and I-CSCF-hA1)

REGISTER sip:homeA.net SIP/2.0

From:

To:

P-Access-Network-Info:

Supported:

Contact:

Path:<sip:home-abc@ibcf-hA1.visited-A.net:5070;lr>,<sip:proxy-abc@ic-vA1.visited-A.net:5070;lr>,<sip:visit-abc@ibcf-vA1.visited-A.net:5070;lr>,<sip:atcf-vA1.visited-A.net;lr>,<sip:visit-xyz@pcscf-vA1.visited-A.net:5070;lr>

P-Visited-Network-ID:

P-Charging-Vector:icid-value="AyretyU0dm+6O2IrT5tAFrbHLso=023551024";orig-ioi="Type 1visited-a";transit-ioi="ICa.1"

Feature-Caps:

Via:SIP/2.0/UDP reg@i-cscf-hA1.home-A.net;branch=z9hG4bK351g45.1

Via:SIP/2.0/UDP reg@ibcf-hA1.home-A.net;branch=z9hG4bK351g45.1

Via:SIP/2.0/UDP reg@ic-A1.interconnection-A.net;branch=z9hG4bK78a3z

Via:SIP/2.0/UDP reg@ibcf-vA1.visited-A.net;branch=z9hG4bK351g45.1

Via:SIP/2.0/UDP atcf-vA1.visited-A.net:5060;branch=z9hG4bKaa3689b

Via:SIP/2.0/UDP visit-xyz@pcscf-vA1.visited-A.net:5060;branch=z9hG4bKnas56565

Via:SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd];branch=z9hG4bKnashds7

Other SIP header fields are set according to 3GPP TS 24.229[4].

Via:	The IBCF-vA1 added a Via header field.
The IBCF-hA1 added a Via header field.
The I-CSCF-vA1 added a Via header field.

P-Charging-Vector:	The IBCF-hA1 added the "transit-ioi" header field parameter with an IOI value identifying the transit network from where the REGISTER request was received i.e. the interconnect network A.

9-16.	200 (OK) response (S-CSCF-hA1 to UE-A).

The S-CSCF acknowledges the REGISTER request by means of a 200 (OK) response as described in subclause 5.2.1.1. The 200 (OK) response does not contain any SRVCC specific information more than that the ATCF-vA1 is included in the Path header field and in the Via header field.

17.	REGISTER request (S-CSCF-hA1 to SCC AS-hA1) - see example in table 5.11.2-17.

The S-CSCF-hA1 checks the iFC triggers and detects that the SCC AS-hA1 is interested in registrations from this user. The S-CSCF-hA1 sends a 3rd party REGISTER request to the SCC AS-hA1. The S-CSCF-hA1 includes in the 3rd party REGISTER request the content of the initial REGISTER request and the 200 (OK) response to the initial REGISTER request in "sip" message bodies.

Table 5.11.2-17: REGISTER request (S-CSCF-hA1 to SCC AS-hA1)

REGISTER sip: sccas.home1.net /2.0

To: <sip:userA_public1@homeA.net>

From: <sip:scscf-hA1.home1.net>; tag=538ya

Contact: <sip:scscf-hA1.homeA.net>; expires=600000

Via: SIP/2.0/TCP scscf-hA1.home1.net;branch=z9hG499ffhy

Content-Type: multipart/mixed;boundary="boundary1"

Content-Length: (…)

--boundary1

Content-Type: message/sip

REGISTER sip:homeA.net SIP/2.0

From:<sip:userA_public1@homeA.net>;tag=4fa3

To:<sip:userA_public1@homeA.net>

P-Access-Network-Info:3GPP-E-UTRAN-TDD;utran-cell-id-3gpp=234151D0FCE11,3GPP-E-UTRAN-TDD; utran-cell-id-3gpp=234151D0FCE11;network-provided;local-time-zone="UTC+01:00";daylight-saving-time="01"

Supported:path

Contact:<sip:[5555::aaa:bbb:ccc:ddd]>;reg-id=1;+sip.instance="<urn:gsma:imei:90420156-025763-0>";expires=600000;+g.3gpp.ics="principal";+g.3gpp.accesstype="cellular1"

Path:<sip:home-abc@ibcf-hA1.visited-A.net:5070;lr>,

<sip:proxy-abc@ic-vA1.visited-A.net:5070;lr>,

<sip:visit-abc@ibcf-vA1.visited-A.net:5070;lr>,

<sip:atcf-vA1.visited-A.net;lr>,

<sip:visit-xyz@pcscf-vA1.visited-A.net:5070;lr>

P-Visited-Network-ID:"Visited Network A identifier"

P-Charging-Vector:icid-value="AyretyU0dm+6O2IrT5tAFrbHLso=023551024";orig-ioi="Type 1visited-a";transit-ioi="ICa.1"

Feature-Caps:*;+g.3gpp.atcf="tel:+1-237-555-3333",*;+g.3gpp.atcf-mgmt-uri="sip:atcf-vA1.visited-A.net",*;+g.3gpp.atcf-path="sip:termabc@atcf-vA1.visited-A.net",*;+g.3gpp.mid-call,*;+g.3gpp.srvcc-alerting,*;+g.3gpp.ps2cs-srvcc-orig-pre-alerting

Via:SIP/2.0/UDP reg@i-cscf-hA1.home-A.net;branch=z9hG4bK351g45.1

Via:SIP/2.0/UDP reg@ibcf-hA1.home-A.net;branch=z9hG4bK351g45.1

Via:SIP/2.0/UDP reg@ic-A1.interconnection-A.net;branch=z9hG4bK78a3z

Via:SIP/2.0/UDP reg@ibcf-vA1.visited-A.net;branch=z9hG4bK351g45.1

Via:SIP/2.0/UDP atcf-vA1.visited-A.net:5060;branch=z9hG4bKaa3689b

Via:SIP/2.0/UDP visit-xyz@pcscf-vA1.visited-A.net:5060;branch=z9hG4bKnas56565

Via:SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd];branch=z9hG4bKnashds7

--boundary1

Content-Type: message/sip

SIP/2.0 200 OK

From: <sip:userA_public1@homeA.net>;tag=4fa3To: <sip:userA_public1@homeA.net>;tag=5ef4

Service-Route-Header: reg@s-cscf-hA1.home-A.net

P-Charging-Vector: icid-value="AyretyU0dm+6O2IrT5tAFrbHLso=023551024";orig-ioi="visited-a";term-ioi="home-a"

Path:<sip:home-abc@ibcf-hA1.visited-A.net:5070;lr>,

<sip:proxy-abc@ic-vA1.visited-A.net:5070;lr>,

<sip:visit-abc@ibcf-vA1.visited-A.net:5070;lr>,

<sip:atcf-vA1.visited-A.net;lr>,<sip:visit-xyz@pcscf-vA1.visited-A.net:5070;lr>

Via:SIP/2.0/UDP reg@i-cscf-hA1.home-A.net;branch=z9hG4bK351g45.1

Via:SIP/2.0/UDP reg@ibcf-hA1.home-A.net;branch=z9hG4bK351g45.1

Via:SIP/2.0/UDP reg@ic-A1.interconnection-A.net;branch=z9hG4bK78a3z

Via:SIP/2.0/UDP reg@ibcf-vA1.visited-A.net;branch=z9hG4bK351g45.1

Via:SIP/2.0/UDP atcf-vA1.visited-A.net:5060;branch=z9hG4bKaa3689b

Via:SIP/2.0/UDP visit-xyz@pcscf-vA1.visited-A.net:5060;branch=z9hG4bKnas56565

Via:SIP/2.0/UDP [5555::aaa:bbb:ccc:ddd];branch=z9hG4bKnashds7

--boundary1—

Other SIP header fields are set according to 3GPP TS 24.229 [4].

18.	200 (OK) response (SCC AS-hA1 to S-CSCF-hA1).

The SCC AS-hA1 acknowledges the REGISTER request by means of a 200 (OK) response. There is no specific SRVCC access transfer information added by SCC AS-hA1 in the 200 (OK) response.

19.	The SCC AS stores the STN-SR in HSS.

The SCC AS stores the STN-SR (tel:+1-237-555-3333) received in the "+g.3gpp.atcf" Feature-Caps header field parameter in HSS. This information will later be transferred to the MSC server when a PS to CS SRVCC access transfer occurs as described in subclause 5.11.3.

20.	MESSAGE request (SCC AS-hA1 to IBCF-hA1) - see example in table 5.11.2-20.

In order to inform about PS to CS SRVCC access transfer parameters necessary for the ATCF-vA1 during PS to CS SRVCC access transfer, the SCC AS-hA1 sends a MESSAGE request to the IBCF-hA1.

Table 5.11.2-20: MESSAGE request (SCC AS-hA1 to IBCF-hA1)

MESSAGE sip:atcf-vA1.visited-A.net SIP/2.0

From:<sip:sccas-hA1.home-A.net>

To:<sip:atcf-vA1.visited-A.net>

P-Asserted-Identity:sip:sccas-hA1.home-A.net

P-Charging-Vector:icid-value="BrtA76543y+403fg654tBf456fa=023551024";orig-ioi="Type 1home-a"

Content-Length: ...

Content-Type: application/vnd.3gpp.SRVCC-info+xml

Via:SIP/2.0/UDP sccas-hA1.home-A.net:5060;branch=z9hG4bK78956a

<?xml version="1.0" encoding="UTF-8"?>

<SRVCC-infos>

<SRVCC-info ATCF-Path-URI="sip:termabc@atcf-vA1.visited-A.net">

<ATU-STI>sip:sccas-hA1.home-A.net</ATU-STI>

<C-MSISDN>tel:+1-237-555-1111</C-MSISDN>

</SRVCC-info>

</SRVCC-infos>

Other SIP header fields are set according to 3GPP TS 24.229 [4] and 3GPP TS 24.237 [20].

Request-URI/To:	Contains the address received in the g.3gpp.atcf-mgmt-uri feature-capability indicator.

P-Asserted-Identity/From:	Includes the SCC AS-hA1 address.

P-Charging-Vector:	The SCC AS-hA1 assigns a new ICID value and includes a type 1 IOI value in the "orig-ioi". The type IOI value identifies the home network A.

Via:	The SCC AS-hA1 adds a Via header field.

application/vnd.3gpp.SRVCC-info+xml:	Contains the PS to CS SRVCC specific information that will be used by the ATCF-vA1 when SRVCC occurs as described in subclause 5.11.3.

-	The SRVCC-info ATCF-Path-URI identifies the registration in the ATCF-vA1.

-	The ATU-STI is the address in SCC AS where the INVITE request is sent to in the event of a PS to CS SRVCC access transfer.

-	C-MSISDN is the public user identity of the user that the ATCF-vA1 is using to identify the user in the event of a PS to CS SRVCC access transfer.

21. MESSAGE request (IBCF-hA1 to IC-A1) - see example in table 5.11.2-21.

The IBCF-hA1selects to route the MESSAGE request via an IC network and sends the INVITE request according to 3GPP TS 24.229 [4] to the IC-A1.

Table 5.11.2-21: MESSAGE request (IBCF-hA1 to IC-A1)

MESSAGE sip:atcf-vA1.visited-A.net SIP/2.0

From:

P-Asserted-Identity:

To:

P-Charging-Vector:

Content-Length:

Content-Type: application/vnd.3gpp.SRVCC-info+xml

Via:SIP/2.0/UDP ibcf-hA1.home-A.net:5060;branch=z9hG4bK123456

Via:SIP/2.0/UDP sccas-hA1.home-A.net:5060;branch=z9hG4bK78956a

<?xml version="1.0" encoding="UTF-8"?>

<SRVCC-infos>

<SRVCC-info ATCF-Path-URI="sip:termabc@atcf-vA1.visited-A.net">

<ATU-STI>sip:sccas-hA1.home-A.net</ATU-STI>

<C-MSISDN>tel:+1-237-555-1111</C-MSISDN>

</SRVCC-info>

</SRVCC-infos>

Other SIP header fields are set according to 3GPP TS 24.229 [4] and 3GPP TS 24.237 [20].

Via:	The IBCF-hA1 adds a Via header field.

22. MESSAGE request (IC-A1 to IBCF-vA1) - see example in table 5.11.2-22.

The IC-A1 selects an IBCF acting as the entry point in the visited network A and sends the MESSAGE request according to 3GPP TS 24.229 [4] to the IBCF-vA1.

Table 5.11.2-22: MESSAGE request (IC-A1 to IBCF-vA1)

MESSAGE sip:atcf-vA1.visited-A.net SIP/2.0

From:

P-Asserted-Identity:

To:

P-Charging-Vector:

Content-Length: ...

Content-Type: application/vnd.3gpp.SRVCC-info+xml

Via:SIP/2.0/UDP ic-A1.interconnect-A.net:5060;branch=z9hG4bKasdfg

Via:SIP/2.0/UDP ibcf-hA1.home-A.net:5060;branch=z9hG4bK123456

Via:SIP/2.0/UDP sccas-hA1.home-A.net:5060;branch=z9hG4bK78956a

<?xml version="1.0" encoding="UTF-8"?>

<SRVCC-infos>

<SRVCC-info ATCF-Path-URI="sip:termabc@atcf-vA1.visited-A.net">

<ATU-STI>sip:sccas-hA1.home-A.net</ATU-STI>

<C-MSISDN>tel:+1-237-555-1111</C-MSISDN>

</SRVCC-info>

</SRVCC-infos>

Other SIP header fields are set according to 3GPP TS 24.229 [4] and 3GPP TS 24.237 [20].

Via:	The IC-A1 adds a Via header field.

23. MESSAGE request (IBCF-vA1 to ATCF-vA1) - see example in table 5.11.2-23.

When the IBCF-vA1 receives the MESSAGE request, the IBCF-vA1 retrieves the address of the ATCF-vA1 (e.g. by performing a DNS query or using a local database) and forwards the message to the ATCF-vA1.

Table 5.11.2-23: MESSAGE request (IBCF-vA1 to ATCF-vA1)

MESSAGE sip:atcf-vA1.visited-A.net SIP/2.0

From:

P-Asserted-Identity:

To:

P-Charging-Vector:icid-value="BrtA76543y+403fg654tBf456fa=023551024";orig-ioi="Type 1home-a";transit-ioi="ICa.1"

Content-Length: ...

Content-Type: application/vnd.3gpp.SRVCC-info+xml

Via:SIP/2.0/UDP ibcf-vA1.visited-A.net:5060;branch=z9hG4bKlkjhg12

Via:SIP/2.0/UDP ic-A1.interconnect-A.net:5060;branch=z9hG4bKasdfg

Via:SIP/2.0/UDP ibcf-hA1.home-A.net:5060;branch=z9hG4bK123456

Via:SIP/2.0/UDP sccas-hA1.home-A.net:5060;branch=z9hG4bK78956a

<?xml version="1.0" encoding="UTF-8"?>

<SRVCC-infos>

<SRVCC-info ATCF-Path-URI="sip:termabc@atcf-vA1.visited-A.net">

<ATU-STI>sip:sccas-hA1.home-A.net</ATU-STI>

<C-MSISDN>tel:+1-237-555-1111</C-MSISDN>

</SRVCC-info>

</SRVCC-infos>

Other SIP header fields are set according to 3GPP TS 24.229 [4] and 3GPP TS 24.237 [20].

P-Charging-Vector:	The IBCF-vA1 acting as an entry point adds an IOI value in the "transit-ioi" header field parameter. The IOI value identifies the interconnect network from where the MESSAGE request was received i.e. the interconnect network A.

Via:	The IBCF-vA1 adds a Via header field.

24-27. 200 (OK) response (ATCF-vA1 to SCC AS-hA1 via IBCF-vA1, IC-A1 and IBCF-hA1) - see example in table 5.11.2-24.

The ATCF-vA1 acknowledges the receipt of the MESSAGE request by means of a 200 (OK) response according to 3GPP TS 24.229 [4]. No SRVCC specific parameters are included in the 200 (OK) response.

The table 5.11.2-24 shows the content of the 200 (OK) response when received in SCC AS-hA1.

Table 5.11.2-24: 200 (OK) response (ATCF-vA1 to SCC AS-hA1 via IBCF-vA1, IC-A1 and IBCF-hA1)

SIP/2.0 200 OK

P-Charging-Vector:icid-value="BrtA76543y+403fg654tBf456fa=023551024";orig-ioi="Type 1home-a";term-ioi="Type 1visited-A"; transit-ioi="ICa.1"

From:<sip:sccas-hA1.home-A.net>

To:<sip:atcf-vA1.visited-A.net>

Other SIP header fields are set according to 3GPP TS 24.229 [4] and 3GPP TS 24.237 [20].

P-Charging-Vector:	The ATCF-vA1 added a P-Charging-Vector with the same "icid-value" and the same "orig-ioi" value as received in the MESSAGE request and a type 1 IOI value in a "term-ioi" identifying the visited network A.

The IBCF-hA1 added the "transit-ioi" header field parameter with an IOI value identifying the transit network from where the 200 (OK) response was received i.e. the interconnect network A.

Editor's note:	The content of P-Charging-Vector header field is not clear in 3GPP TS 24.237 [20] and needs to be clarified further.

### 5.11.3	PS to CS SRVCC access transfer for a call in the active phase without loopback

This subclause describes the PS to CS SRVCC access transfer of a call in the active phase.

Preconditions to this call flow:

1.	a call is established between a UE-A and a remote UE as described in subclause 5.3. The UE-A is attached to a 4G radio access network (E-UTRAN);

2.	media is anchored in:

a.	the P-CSCF-vA1, the ATCF-vA1 and IBCF-vA1 in the visited network vA;

b.	the IC-A1 in the IC network; and

c.	the IBCF-hA1 and the IBCF-hA2 in the home network hA; and

3.	SIP signalling is anchored in S-CSCF-hA1.

NOTE 1:	The signalling flow is the same for PS to CS SRVCC access transfer for a call in the active phase regardless if the UE-A or the remote UE initiated the call.

Figure 5.11.3-1 shows the SIP message flow when PS to CS SRVCC access transfer of a call in the active phase occurs.

NOTE:	For clarity, the SIP 100 (Trying) messages are not shown in the signalling flow.

Figure 5.11.3-1: SRVCC without loop back message flow

The steps of the flow are as follows:

1.	An call is established as described in subclause 5.3 between UE-A and a remote UE

-	The remote UE can be a UE connected to IMS or an UE connected to CS and the difference compared to the description in subclause 5.3 and this PS to CS SRVCC access transfer scenario is that an ATCF in the visited originated network participated in the call establishment of the call and that the general AS-hA1 in figure 5.3.2-1 is replaced by the access transfer specific AS i.e. the SCC AS-hA1.

-	The media path is UE-A – P-CSCF-vA1 – ATCF-vA1 – IBCF-vA1 – IC-A1 – IBCF-hA1 – IBCF-hA2 – terminating network – remote UE.

2.	During the ongoing call the UE-A is performing measurements on signalling strenght in neighbouring cells and sends continuosly the measurement results to the core network. Due to decreasing signalling strenght in the 4G radio access network the core network takes a decision that the UE shall be handed over to a 3G radio access network and sends a PS to CS SRVCC request to the MSC server-vA1.

3.	INVITE request due to STN-SR (MSC server-vA1 to ATCF-vA1) - see example in table 5.11.3-3.

-	When the MSC server-vA1 receives the PS to CS SRVCC request from the core network in the visited network, the MSC server-vA1 sends an INVITE request to the ATCF-vA1. This INVITE request is referred to as an "INVITE request due to STN-SR" in the 3GPP TS 24.237 [20].

Table 5.11.3-3: INVITE request due to STN-SR (MSC server-vA1 to ATCF-vA1)

INVITE tel:+1-237-555-3333 SIP/2.0

From: <tel:+1-237-555-1111>;tag=171828

To: <tel: +1-237-555-3333>

P-Asserted-Identity: <tel:+1-237-555-1111>

P-Access-Network-Info:3GPP-UTRAN-TDD;utran-cell-id-3gpp=151234D0EAF22;network-provided;local-time-zone="UTC+01:10";daylight-saving-time="01"

P-Charging-Vector:icid-value="023551024"; orig-ioi="Type 1visited-A.net";icid-generated-at="msc1.visit1.net"

Contact: <sip: msc1.visit1.net:1357>;+g.3gpp.icsi-ref="urn%3Aurn-7%3A3gpp-service.ims.icsi.mmtel";+g.3gpp.srvcc-alerting;+g.3gpp.mid-call;+ps2cs-srvss-orig-pre-alerting

Accept:"application/vnd.3gpp.state-and-event-info+xml"

Recv-Info:g.3gpp.state-and-event

Via:SIP/2.0/UDP <atcf-vA1.visited-A.net;branch=z9hG4bk7987fe>

Other SIP headers and SDP according to 3GPP TS 24.229 [4] and 3GPP TS 24.237 [20].

Request-URI /To:	The tel URI (refered to as STN-SR) identifies the ATCF-vA1. The STN-SR is stored by SCC AS-hA1 in HSS during registration of the user as described in subclause 5.11.2 and sent to the MSC server-vA1 in the PS to CS SRVCC request from the core network.

P-Access-Network-Info:	This header field identifies the location of the UE in the 3G access network, the time-zone and daylight saving on that location.

P-Charging-Vector:	The MSC server-vA1 adds the P-Charging-Vector header field containing a new ICID value in the "icid","icid-generated-at" and "orig-ioi" header field parameters. The IOI value is of type 1 and identifies the visited network A.

P-Asserted-Identity/From:	The P-Asserted identity and the From header fields is the C-MSISDN and that is not necessary the same identity used in the original call. For example: In the original call the P-Asserted-Identity header field could be adam@homenetwork.net.

Contact:	The Contact header field can, additional to the contact address, include a number of media feature tags. In this example the g.3gpp.icsi-ref indicates that this is an MMTel call, the g.3gpp.srvcc-alerting indicates that the MSC server-vA1 supports PS to CS access transfer for call in alerting phase, the g.3gpp.mid-call indicates that the MSC server-vA1 supports transfer of held calls and conference calls and the ps2cs-srvss-orig-pre-alerting indicates that the MSC server-vA1 supports PS to CS SRVCC access transfer for calls in the originating pre-alerting phase. The media-feature tags will be used by the SCC AS-vA1 to determine which sessions to be transferred.

4.	200 (OK) response (ATCF-vA1 to MSC server-vA1) – see example in table 5.11.3-4.

When the ATCF-vA1 receives the INVITE due to STN-SR the ATCF-vA1 checks if there is a session that fulfils conditions for being transferred and detects that there is a call in an active phase ongoing between UE-A and a remote UE. To avoid speech gaps, the ATCF-vA1 returns a 200 (OK) response towards the MSC server-vA1 containing media parameters retrieved from the ATGW.

The ATCF-vA1 also adds the "related-icid" and the "related-icid-generated-at" header field parameters containg the "icid-value" and the "icid-genarted-at" header fields parameters used in the original call and can be used for charging correlation of generated CDR records in the visited network A.

Table 5.11.3-4: 200 (OK) response (ATCF-vA1 to MSC server-vA1)

SIP/2.0 200 OK

Record-Route:<sip:atcf-vA1.visited-A.net:5070;lr>

P-Charging-Vector: icid-value="1234bc9876e";icid-generated-at=5555::aaa:bbb:ccc:eeee";orig-ioi="Type 1home-hA.net";term-ioi="Type 1visited-vA"related-icid="023551024"

Feature-Caps:*;+g.3gpp.srvcc-alerting,*;+g.3gpp.mid-call,*;+ps2cs-srvss-orig-pre-alerting

Contact: [5555::bbb:ccc:ddd:aaa]

P-Asserted-Identity:<tel:+4687197378>

Other SIP headers and the SDP according to 3GPP TS 24.229 [4] and 3GPP TS 24.237 [20].

Record-Route:	The ATCF adds its own address in the header field which will be used by the MSC server-vA1 for subsequent SIP requests.

P-Charging-Vector:	The P-Charging-Vector contains the parameters received in the INVITE due to STN-SR and the related-icid header field parameter containing the ICID value recived in the session to be transferred.

Editor's note:	Whether the P-Charging-Vector shall contain term-ioi and/or transit-ioi or not is FFS. At the moment the term-ioi and transit-ioi is not currently specified in 3GPP TS 24.237.

Feature-Caps:	The feature-capability indicators as received on the home leg in the session to be transferred.

Contact:	The contact address of the remote UE is included.

P-Asserted-Identity:	The public user identity of the remote UE is included.

5	ACK request (MSC-vA1 to ATCF-vA1).

The MSC server-vA1 sends the ACK to the ATCF-vA1 according to 3GPP TS 24.229 [4]. The ACK request does not contain any SRVCC specific information.

6.	INVITE request due to ATU-STI (ATCF-vA1 to IBCF-vA1) - see example in table 5.11.3-6.

The ATCF-vA1 creates a new dialog towards SCC AS-hA1 by sending an INVITE due to ATU-STI. Everything except the Request line and the To header field is copied from the received INVITE due to STN-SR.

Table 5.11.3-6: INVITE request due to ATU-STI (ATCF-vA1 to IBCF-vA1)

INVITE sip:ps2cs@sccas1.homeA.net SIP/2.0

From: <tel:+1-237-555-1111>;tag=171828

To: <tel: +1-237-555-3333>

P-Asserted-Identity: <tel:tel:+1-237-555-1111>

P-Access-Network-Info:3GPP-UTRAN-TDD;utran-cell-id-3gpp=151234D0EAF22;network-provided;

local-time-zone="UTC+01:10";daylight-saving-time="01"

P-Charging-Vector: icid-value="023551024"; orig-ioi="Type 1visited-A.net";

icid-generated-at="msc1.visit1.net"

Record-Route:<sip:atcf-vA1.visited-A.net:5070;lr>

Contact: <sip: msc1.visit1.net:1357>;+g.3gpp.icsi-ref="urn%3Aurn-7%3A3gpp-service.ims.icsi.mmtel";+g.3gpp.srvcc-alerting;+g.3gpp.mid-call;+ps2cs-srvss-orig-pre-alerting

Accept:"application/vnd.3gpp.state-and-event-info+xml"

Recv-Info:g.3gpp.state-and-event

Target-Dialog:me03a0s09a2sdfgjkl491777; remote-tag=774321; local-tag=4fa3

Required:tdialog

Via: SIP/2.0/UDP msc-vA1.visit-A.net;branch=z9hG4bk731b87

Other SIP headers and SDP according to 3GPP TS 24.229 [4] and 3GPP TS 24.237 [20].

Request-URI / To:	The SIP URI (refered to as ATU-STI) identifies the SCC AS-hA1. The ATU-STI is sent by SCC AS-hA1 to ATCF-vA1 during registration as described in subclause 5.11.2.

Record-Route:	The address that the ATCF-vA1 wants to receive subsequent in-dialog requests from the SCC AS-hA1.

Target-Dialog:	The identity of the dialog selected by ATCF-vA1.

Required:	The "tdialog" option tag indicate that the support for Target-Dialog header field is required.

7.	INVITE request due to ATU-STI (IBCF-vA1 to IC-A1) - see example in table 5.11.3-7.

When the IBCF-vA1 recieves the INVITE due to ATU-STI, the IBCF-vA1 updates the SDP as described within 3GPP TS 29.079 [5].

The IBCF-vA1 selects, based on local policy, to route the INVITE request due to ATU-STI via interconnect network A and sends the request according to 3GPP TS 24.229 [4] to the IC-A1.

Table 5.11.3-7: INVITE request due to ATU-STI (IBCF-vA1 to IC-A1)

INVITE sip:ps2cs@sccas1.home1.net SIP/2.0

From:

To:

P-Access-Network-Info:

P-Charging-Vector:

Record-Route:<sip:ibcf-vA1.visited-A.net;lr>,<sip:atcf-vA1.visited-A.net:5070;lr>

P-Asserted-Identity:

Contact:

Accept:

Recv-Info:

Target-Dialog:

Require:

Via: SIP/2.0/UDP <ibcf-vA1.visited-A.net;branch=z9hG4bk79c54a>

Via: SIP/2.0/UDP <atcf-vA1.visited-A.net;branch=z9hG4bk7987fe>

Other SIP headers and SDP according to 3GPP TS 24.229 [4] and the 3GPP TS 29.079 [5].

Record-Route: 	Based on topology hiding functionality the IBCF-vA1 will either:

-	forward the Record-Route header field with one visible IBCF-vA1 entry and the ATCF-vA1 entry as one hidden entry; or

-	both the IBCF-vA1 entry and the ATCF-vA1 entry as visible entries.

Via:	Based on topology hiding functionality the IBCF-vA1 will either:

-	forward the Via header fields with one visible IBCF-vA1 entry and the ATCF-vA1 entry as one hidden entry; or

-	both the IBCF-vA1 entry and the ATCF-vA1 entry as visible entries.

NOTE 2:	In the example above both the IBCF-vA1 entry and the ATCF-vA1 entry as visible entries in the Via and Record-Route header fields.

8.	INVITE request due to ATU-STI (IC-A1 to IBCF-hA1) - see example in table 5.11.3-8.

When the IC-A1 receives the initial INVITE request due to ATU-STI and since the IC-A1 according to local policy allows it self to be bypassed by media then the IC-A1 updates the SDP as described within TS 29.079 [5].

The IC-A1 selects an entry point (IBCF-hA1) of the home IMS network hA and sends the INVITE request according to 3GPP TS 24.229 [4] to the IBCF-hA1.

NOTE 3:	The IC-A1 can add a Route header field entry pointing to the entry point of home network hA. However, since this is not necessary for routeing the call it is not shown in table 5.11.3-7.

Table 5.11.3-8: INVITE request due to ATU-STI (IC-A1 to IBCF-hA1)

INVITE sip:ps2cs@sccas1.homeA.net SIP/2.0

From:

To:

P-Access-Network-Info:

P-Charging-Vector:

Target-Dialog:

Require:

Record-Route:<sip:ic-A1.interconnection-A.net;lr>,<sip:ibcf-vA1.visited-A.net;lr>,

<sip:atcf-vA1.visited-A.net:5070;lr>

P-Asserted-Identity:

Contact:

Accept:

Recv-Info:

Via: SIP/2.0/UDP <ic-A1.interconnection-A.net;branch=z9hG4bk73eb11>

Via: SIP/2.0/UDP <ibcf-vA1.visited-A.net;branch=z9hG4bk79c54a>

Via: SIP/2.0/UDP <atcf-vA1.visited-A.net;branch=z9hG4bk7987fe>

Other SIP headers and SDP according to 3GPP TS 24.229 [4] and the 3GPP TS 29.079 [5].

9.	INVITE request due to ATU-STI (IBCF-hA1 to SCC AS-hA1 via I-CSCF) - see example in table 5.11.3-9.

When the IBCF-hA1 receives the INVITE request due to ATU-STI the IBCF-hA1 updates the SDP as described in TS 29.079 [5].

Since there is no Route header field guiding the IBCF-hA1 where to route the call the IBCF-hA1 selects an I-CSCF in the home network and sends the INVITE request due to ATU-STI to the selected I-CSCF according to 3GPP TS 24.229 [4].

The I-CSCF uses PSI routeing to route the call and sends the INVITE request due to ATU-STI directly to SCC AS-hA1.

The table 5.11.3.9 shows the content of the INVITE request due to ATU-STI when received in the SCC AS-hA1.

Table 5.11.3-9: INVITE request due to ATU-STI (IBCF-hA1 to SCC AS-hA1 via I-CSCF)

INVITE sip:ps2cs@sccas1.homeA.net SIP/2.0

From:

To:

P-Access-Network-Info:

P-Charging-Vector:icid-value="023551024";orig-ioi="Type 1visited-A.net";

icid-generated-at="msc1.visit1.net";transit-ioi="ICa.1"

Target-Dialog:

Require:

Record-Route:<ibcf-hA1.home-A.net;lr>,<sip:ic-A1.interconnection-A.net;lr>,

<sip:ibcf-vA1.visited-A.net;lr>,<sip:atcf-vA1.visited-A.net:5070;lr>

P-Asserted-Identity:

Contact:

Accept:

Recv-Info:

Via: SIP/2.0/UDP <icscf-hA1.home-A.net;branch=z9hG4bk7e75ca1>

Via: SIP/2.0/UDP <ibcf-hA1.home-A.net;branch=z9hG4bk769aff3>

Via: SIP/2.0/UDP <ic-A1.interconnection-A.net;branch=z9hG4bk73eb11>

Via: SIP/2.0/UDP <ibcf-vA1.visited-A.net;branch=z9hG4bk79c54a>

Via: SIP/2.0/UDP <atcf-vA1.visited-A.net;branch=z9hG4bk7987fe>

Other SIP headers and SDP according to 3GPP TS 24.229 [4], 3GPP TS 24.237 [20] and the 3GPP TS 29.079 [5].

P-Charging-Vector:	The IBCF-hA1 includes the "transit-ioi" based on from which interconnect network the INVITE request due to ATU-STI is received.

10.	INVITE request (SCC AS-hA1 to S-CSCF-hA1) - see example in table 5.11.3-10.

When the SCC AS-hA1 receives the INVITE request due to ATU-STI the SCC AS-hA1 compares the received SDP with the SDP negotiated in the existing session. The conclusion is that it differs so the SCC AS-hA1 needs to send a re-INVITE request towards the remote UE.

NOTE 4:	When a user is roaming and media is anchored in IBCF the IP addresses will be different in the SDP received in the INVITE request due to ATU-STI compared to those negotiated in the existing session towards the remote UE and that will always result in that a re-INVITE request is needed towards the remote UE.

Table 5.11.3-10: INVITE request (SCC AS-hA1 to S-CSCF-hA1)

INVITE sip:[5555::bbb:ccc:ddd:aaa] SIP/2.0

From:<tel:+1-237-555-1111>;tag=171828

To:<tel:+4687197378>;tag=987651

Route:<sip:scscf-hA1.home-A.net:5070;lr>,<sip:ibcf-hA2.home-A.net:5070;lr>,<The set of routes recorded when the call was established>

Via: SIP/2.0/UDP <scc as-hA1.home-A.net;branch=z9hG4bK3a4f92>

Other SIP headers and updated SDP according to 3GPP TS 24.229 [4], 3GPP TS TS 24.237 [20] and 3GPP TS 29.079 [5]

11.	INVITE request (S-CSCF-hA1 to IBCF-hA2) - see example in table 5.11.3-11.

The S-CSCF-hA1 forwards the re-INVITE request according to procedures in 3GPP TS 24.229 [4].

Table 5.11.3-11: INVITE request (S-CSCF-hA1 to IBCF-hA2)

INVITE sip:[5555::bbb:ccc:ddd:aaa] SIP/2.0

From:<tel:+1-237-555-1111>;tag=171828

To:<tel:+4687197378>;tag=987651

Route:<sip:ibcf-hA2.home-A.net:5070;lr>,<The set of routes recorded when the call was established>

Via: SIP/2.0/UDP <s-cscf-vA1.home-A.net;branch=z9hG4bK354321>

Via: SIP/2.0/UDP <scc as-hA1.home-A.net;branch=z9hG4bK3a4f92>

Other SIP headers and updated SDP according to 3GPP TS 24.229 [4], 3GPP TS TS 24.237 [20] and 3GPP TS 29.079 [5]

12-13.	200 (OK) response (IBCF-hA2 to SCC AS-hA1 via S-CSCF-hA1).

When the IBCF-hA2 receives the 200 (OK) response to the re-INVITE request sent towards the remote UE, the IBCF-hA2 forwards the 200 (OK) response to SCC AS-hA1 according to 3GPP TS 24.229 [4]. The 200 (OK) response does not contain any SRVCC specific information.

14-15.	ACK request (SCC AS-hA1 to IBCF-hA2 via S-CSCF-hA1).

The SCC AS-hA1 sends the ACK to the IBCF-hA2 according to 3GPP TS 24.229 [4]. The ACK request does not contain any SRVCC specific information.

16-20.	200 (OK) response (SCC AS-hA1 to ATCF-vA1 via S-CSCF-hA1, IBCF-hA1, IC-A1 and IBCF-vA1) - see example in table 5.11.3-16.

The SCC AS-hA1 uses the received SDP response and sends a 200 OK response to the INVITE request due to ATU-STI towards the ATCF-vA1.

The table 5.11.3-16 shows the content of the 200 (OK) response when received in ATCF-vA1.

Table 5.11.3-16: 200 (OK) response (SCC AS-hA1 to ATCF-vA1 via S-CSCF-hA1, IBCF-hA1, IC-A1 and IBCF-vA1)

SIP/2.0 200 OK

Record-Route:<ibcf-hA1.home-A.net;lr>,<sip:ic-A1.interconnection-A.net;lr>,

<sip:ibcf-vA1.visited-A.net;lr>,<sip:atcf-vA1b.visited-A.net;lr>

P-Charging-Vectoricid-value=icid-value="023551024"; orig-ioi="Type 1visited-A.net";icid-generated-at="msc1.visit1.net";term-ioi="Type 1home-A.net";related-icid="1234bc9876e";related-icid-generated-at=[5555::aaa:bbb:ccc:eeee]"

Other SIP headers and SDP according to 3GPP TS 24.229 [4].

P-Charging-Vector:	The SCC-AS added the P-Charging-Vector with the "icid-value", the "orig-ioi" value and the "icid-generated-at" as received in the INVITE due to ATU-STI. Additionally the SCC-AS-hA1 added the "related-icid" and the "related-icid-generated-at" header field parameters. The "related-icid" and the "related-icid-generated-at" header field parameters contains the "icid-value" and the "icid-genarted-at" header fields parameters of the original call and can be used for charging correlation of generated CDR records in the interconnect network A.

21-24.	ACK request (ATCF-vA1 to SCC AS-hA1 via IBCF-vA1, IC-A1, IBCF-hA1 and S-CSCF-hA1).

- The ATCF-vA1 sends the ACK to the SCC AS-hA1 according to 3GPP TS 24.229 [4]. The ACK request does not contain any SRVCC specific information.

### 5.11.4	PS to CS SRVCC access transfer for a call in the active phase with loopback

This subclause describes the PS to CS SRVCC access transfer of a call in the active phase with loopback.

Preconditions to this call flow:

1.	a call is established between a UE-A and a remote UE as described in subclause 5.4. The UE-A is attached to a 4G radio access network (E-UTRAN); and

NOTE 1:	The signalling flow is the same for PS to CS SRVCC access transfer for a call in the active phase regardless if the UE-A or the remote UE initiated the call.

2.	The media is anchored in the P-CSCF-vA1, ATCF-vA1 and IBCF-vA2 in the visited network vA.

Figure 5.11.4-1 shows the SIP message flow when PS to CS SRVCC access transfer of a call in the active phase occurs.

## 5.12	Invocation and configuration of services during roaming in a visited network

This case describes how the user within a visited network invokes services via the Ut interface.

GSMA IR.88 [15] describes that the home operator must provide an APN to be used by the UE for XCAP/Ut. For UE details see GSMA IR.92 [8].

The IMS "well-known" APN must not be used as APN for XCAP/Ut as defined in Section 6.3.2 of GSMA IR.88 [15].

In addition GSMA IR.92 [8] describes that for XCAP requests, the UE must be preconfigured or provisioned by the home operator with the APN to be used for XCAP requests.

Editor's Note: It needs to be clarified and described what correlation between the SIP Call Signalling and the http flows are existing.

Editor's Note: It needs to be clarified and described what security mechanisms are used.

NOTE:	For clarity, the SIP 100 (Trying) messages are not shown in the signalling flow.

Figure 5.11.4-1: SRVCC with loopback message flow

The steps of the flow are as follows:

1. A call is established between the UE-A and a remote UE as described in subclause 5.4.

The remote UE can be a UE connected to IMS or an UE connected to CS and the difference compared to the description in subclause 5.4 and this PS to CS SRVCC access transfer scenario is that an ATCF in the visited originated network participated in the call establishment of the call and that the general AS-hA1 in figure 5.4.2-1 is replaced by the access transfer specific SCC AS-hA1.

The media path is UE-A – P-CSCF-vA1 – ATCF-vA1 – IBCF-vA1 – IC-B1 – terminating network – remote UE.

2.	INVITE request due to STN-SR (MSC server-vA1 to ATCF-vA1).

See step 3 in subclause 5.11.3.

3.	200 (OK) response (ATCF-vA1 to MSC server-vA1).

See step 4 in subclause 5.11.3.

4.	ACK request (MSC server-vA1 to ATCF-vA1).

The MSC server acknowledge the receipt of the 200 (OK) response. No SRVCC specific information is added.

5-8.	INVITE request due to ATU-STI (ATCF-vA1 to SCC AS-hA1 via IBCF-vA1, IC-A1, IBCF-hA1/I-CSCF).

See steps 6-9 in subclause 5.11.3.

9-14.	INVITE request (SCC AS-hA1 to IC-B1 via S-CSCF-hA1, IBCF-hA1, IC-A1, IBCF-vA1, TRF and IBCF-vA2) - see example in table 5.11.4-9.

When the SCC AS-hA1 receives the INVITE request due to ATU-STI the SCC AS-hA1 compares the received SDP with the SDP negotiated in the existing session. The conclusion is that it differs so the SCC AS-hA1 needs to send a re-INVITE request towards the remote UE.

The SCC AS-hA1 handles OMR attributes according to 3GPP TS 24.237 [20] subclause 6A.4.4.

NOTE 2:	When a user is roaming and media is anchored in IBCF the IP addresses will be different in the SDP received in the INVITE request due to ATU-STI compared to those negotiated in the existing session towards the remote UE and that will always result in that a re-INVITE request is needed towards the remote UE.

Table 5.11.4-9: INVITE request (SCC AS-hA1 to IC-B1 via S-CSCF-hA1, IBCF-hA1, IC-A1, IBCF-vA1, TRF and IBCF-vA2)

INVITE sip:[5555::bbb:ccc:ddd:aaa] SIP/2.0

From:<tel:+1-237-555-1111>;tag=171828

To:<tel:+4687197378>;tag=987651

Route:<sip:scscf-hA1.home-A.net:5070;lr>,<sip:ibcf-hA2.home-A.net:5070;lr>,

<The set of routes recorded when the call was established>

Via: SIP/2.0/UDP <sccas-hA1.home-A.net;branch=z9hG4bK3a4f92>

Other SIP headers and updated SDP according to 3GPP TS 24.229 [4], 3GPP TS 24.237 [20] and 3GPP TS 29.079 [5]

Via:	Via header fileds are added by SCC AS-hA1, S-CSCF-hA1, IBCF-hA1, IC-A1, IBCF-vA1, TRF and IBCF-vA2.

15-20.	200 (OK) response (IC-B1 to SCC AS-hA1 via TRF, IBCF-vA1, IC-A1, IBCF-hA1 and S-CSCF-hA1).

When receiving a 200 (OK) response to the re-INVITE request from the remote UE the IC-B1 returns a 200 (OK) response. No SRVCC specific information is added.

21-26.	ACK request (SCC AS-hA1 to IC-B1 via S-CSCF-hA1, IBCF-hA1, IC-A1, IBCF-vA1, TRF and IBCF-vA2).

The SCC AS-hA1 acknowledge the receipt of the 200 (OK) response. No SRVCC specific information is added.

27-31.	200 (OK) response (SCC AS-hA1 to ATCF-vA1 via S-CSCF, IBCF-hA1 and IBCF-vA1).

See steps 16-20 in subclause 5.11.3.

31-36.	ACK request (ATCF-vA1 to SCC AS-hA1 via IBCF-vA1, IC-A1, IBCF-hA1/I-CSCF).

The ATCF-vA1 acknowledge the receipt of the 200 (OK) response. No SRVCC specific information is added.

## 5.12	Invocation and configuration of services during roaming in a visited network

This case describes how the user within a visited network invokes services via the Ut interface.

GSMA IR.88 [15] describes that the home operator must provide an APN to be used by the UE for XCAP/Ut. For UE details see GSMA IR.92 [8].

The IMS "well-known" APN must not be used as APN for XCAP/Ut as defined in Section 6.3.2 of GSMA IR.88 [15].

In addition GSMA IR.92 [8] describes that for XCAP requests, the UE must be preconfigured or provisioned by the home operator with the APN to be used for XCAP requests.

Editor's Note: It needs to be clarified and described what correlation between the SIP Call Signalling and the http flows are existing.

Editor's Note: It needs to be clarified and described what security mechanisms are used.

Annex A:
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | TSG # | TSG Doc. | CR | Rev | Subject/Comment | Old | New |
| 2013-09 |  |  |  |  | Initial skeleton provided by rapporteur | - | 0.0.0 |
| 2013-10 |  |  |  |  | Incorporation of PCR1452, PCR1615 and PCR1616 | 0.0.0 | 0.1.0 |
| 2013-12 |  |  |  |  | Incorporation of PCR C3-131817, PCR C3-131819, PCR C3-131836, PCR C3-131866, PCR C3-131916 and PCR C3-131917 | 0.1.0 | 0.2.0 |
| 2014-01 |  |  |  |  | Incorporation of PCR C3-140341 C3-140342 C3-140024 C3-140241 C3-140243 C3-140343 C3-140274 C3-140344 C3-140105 C3-140345 C3-140347 C3-140348 C3-140352 C3-140385 C3-140396 C3-140397 C3-140350 C3-140351 Output in C3-140414 | 0.2.0 | 0.3.0 |
| 2014-02 |  |  |  |  | C3-140346 Editorial Changes (Table Format) | 0.3.0 | 0.3.1 |

Annex X

#### 5.2.1.2	Exampe of an call traversing an IP transport network as transit

In this exampel shown in Figure5.2.1.2-1 the visited network and the home network do not make use of any SIP proxy in between to route the registration messages. Nevertheless there might be various IP backbone networks between the mobile networks which support routing just on an IP level. I.e. there is no need to have direct lines or VPNs between mobile networks.

The SIP layer with IMS registration and call initiaition is executed between the visited network and home network.

For example the registration of the UE addresses an FQDN in the home network domain. With this FQDN the IBCF acting as an exit point in the visited network executes a DNS query and the REGISTER request is routed via IP routers of interconnection backbones to the IBCF acting as the entry point in the home network.

NOTE:	A further possibility is that the IBCF has a mapping table of the FQDN to the related IP addresses of the entry points of the home network.

Figure 5.2.1.2-1: Message flow for registration without IC SIP proxy

Call flow details:

The SIP registration flow itself is the same as for the registration via IC SIP proxys described in subclause 5.2.1.1. The content of the messages differ as follows:

1. The messages Request/Response are passed through the interconnection IP network without any SIP manipulation/handling.

Editor's note: Clarification is needed if the "transit-ioi" header field parameter in the P-Charging-Vector header field is not used since there is no SIP transit network. Due to "transit-ioi" ruling the IBCF-hA1 could include such an "transit-ioi" value for the IP transit. On the other side it is not a real SIP transit. Also the question is if with regard to CDR's written within the home/visited network the "transit-ioi" will have some specific charging intension.

Editor's note: further consideration is needed where this scenario should be placed.