| 3GPP TS 36.101 V16.14.0 (2022-06) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception (Release 16) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2022, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	32

1	Scope	34

2	References	34

3	Definitions, symbols and abbreviations	34

3.1	Definitions	34

3.2	Symbols	36

3.3	Abbreviations	39

4	General	41

4.1	Relationship between minimum requirements and test requirements	41

4.2	Applicability of minimum requirements	41

4.3	Void	41

4.3A	Applicability of minimum requirements (CA, UL-MIMO, ProSe, Dual Connectivity, UE category 0, UE category M1, UE category M2, UE category 1bis, UE category NB1 and NB2, V2X Communication, MBMS UE)	41

4.4	RF requirements in later releases	43

5	Operating bands and channel arrangement	43

5.1	General	43

5.2	Void	43

5.3	Void	43

5.4	Void	43

5.5	Operating bands	43

5.5A	Operating bands for CA	46

5.5B	Operating bands for UL-MIMO	65

5.5C	Operating bands for Dual Connectivity	65

5.5D	Operating bands for ProSe	68

5.5E	Operating bands for UE category 0, UE category M1 and M2 and UE category 1bis	69

5.5F	Operating bands for category NB1 and NB2	69

5.5G	Operating bands for V2X Communication	69

5.6	Channel bandwidth	70

5.6.1	Channel bandwidths per operating band	71

5.6A	Channel bandwidth for CA	74

5.6A.1	Channel bandwidths per operating band for CA	76

5.6B	Channel bandwidth for UL-MIMO	178

5.6B.1	Void	178

5.6C	Channel bandwidth for Dual Connectivity	178

5.6C.1	Void	178

5.6D	Channel bandwidth for ProSe	178

5.6D.1	Channel bandwidths per operating band for ProSe	178

5.6F	Channel bandwidth for category NB1 and NB2	179

5.6G	Channel bandwidth for V2X Communication	180

5.6G.1	Channel bandwidths per operating band for V2X Communication	180

5.7	Channel arrangement	182

5.7.1	Channel spacing	182

5.7.1A	Channel spacing for CA	182

5.7.1F	Channel spacing for category NB1 and NB2	183

5.7.2	Channel raster	183

5.7.2A	Channel raster for CA	183

5.7.2F	Channel raster for category NB1 and NB2	183

5.7.3	Carrier frequency and EARFCN	183

5.7.3F	Carrier frequency and EARFCN for category NB1 and NB2	186

5.7.4	TX–RX frequency separation	186

5.7.4A	TX–RX frequency separation for CA	187

5.7.4E	TX–RX frequency separation for category M1 and M2	187

5.7.4F	TX–RX frequency separation for category NB1 and NB2	188

6	Transmitter characteristics	188

6.1	General	188

6.2	Transmit power	188

6.2.1	Void	188

6.2.2	UE maximum output power	188

6.2.2A	UE maximum output power for CA	191

6.2.2B	UE maximum output power for UL-MIMO	196

6.2.2C	 Void	199

6.2.2D	UE maximum output power for ProSe	199

6.2.2E	UE maximum output power for Category M1 and M2 UE	199

6.2.2F	UE maximum output power for category NB1 and NB2	200

6.2.2G	UE maximum output power for V2X Communication	201

6.2.3	UE maximum output power for modulation / channel bandwidth	203

6.2.3A	UE Maximum Output power for modulation / channel bandwidth for CA	204

6.2.3B	UE maximum output power for modulation / channel bandwidth for UL-MIMO	209

6.2.3D	UE maximum output power for modulation / channel bandwidth for ProSe	209

6.2.3E	UE maximum output power for modulation / channel bandwidth for category M1 and M2	210

6.2.3F	UE maximum output power for modulation / channel bandwidth for category NB1 and NB2	212

6.2.3G	UE maximum output power for modulation / channel bandwidth for V2X Communication	212

6.2.3G.1	MPR for Power class 3 V2X UE	212

6.2.3G.2	MPR for Power class 2 V2X UE	214

6.2.4	UE maximum output power with additional requirements	214

6.2.4A	UE maximum output power with additional requirements for CA	233

6.2.4A.1	A-MPR for CA_NS_01 for CA_1C	235

6.2.4A.2	A-MPR for CA_NS_02 for CA_1C	236

6.2.4A.3	A-MPR for CA_NS_03 for CA_1C	236

6.2.4A.4	A-MPR for CA_NS_04	237

6.2.4A.5	A-MPR for CA_NS_05 for CA_38C	240

6.2.4A.6	A-MPR for CA_NS_06	241

6.2.4A.7	A-MPR for CA_NS_07	242

6.2.4A.8	A-MPR for CA_NS_08	243

6.2.4A.9	Void	244

6.2.4A.10	A-MPR for CA_NS_10	244

6.2.4B	UE maximum output power with additional requirements for UL-MIMO	249

6.2.4D	UE maximum output power with additional requirements for ProSe	249

6.2.4E	UE maximum output power with additional requirements for category M1 and M2 UE	249

6.2.4F	UE maximum output power with additional requirements for category NB1 and NB2 UE	258

6.2.4G	UE maximum output power with additional requirements for V2X Communication	259

6.2.5	Configured transmitted power	261

6.2.5A	Configured transmitted power for CA	292

6.2.5B	Configured transmitted power for UL-MIMO	296

6.2.5C	Configured transmitted power for Dual Connectivity	296

6.2.5D	Configured transmitted power for ProSe	298

6.2.5F	Configured transmitted Power for category NB1 and NB2	299

6.2.5G	Configured transmitted power for V2X Communication	300

6.3	Output power dynamics	304

6.3.1	(Void)	304

6.3.2	Minimum output power	304

6.3.2.1	Minimum requirement	304

6.3.2A	 UE Minimum output power for CA	304

6.3.2A.1	Minimum requirement for CA	305

6.3.2B	 UE Minimum output power for UL-MIMO	305

6.3.2B.1	Minimum requirement	305

6.3.2C	 Void	305

6.3.2D	UE Minimum output power for ProSe	305

6.3.2F	UE Minimum output power for category NB1 and NB2	306

6.3.2G	UE Minimum output power for V2X Communication	306

6.3.3	Transmit OFF power	306

6.3.3.1.	Minimum requirement	306

6.3.3A	 UE Transmit OFF power for CA	307

6.3.3A.1	Minimum requirement for CA	307

6.3.3B	 UE Transmit OFF power for UL-MIMO	307

6.3.3B.1	Minimum requirement	307

6.3.3D	Transmit OFF power for ProSe	308

6.3.3F	Transmit OFF power for category NB1 and NB2	308

6.3.3G	Transmit OFF power for V2X Communication	308

6.3.4	ON/OFF time mask	309

6.3.4.1	General ON/OFF time mask	309

6.3.4.2	PRACH and SRS time mask	310

6.3.4.2.1	PRACH time mask	310

6.3.4.2.2	SRS time mask	311

6.3.4.3	Slot / Sub frame boundary time mask for subframe TTI	313

6.3.4.4	PUCCH / PUSCH / SRS time mask for subframe TTI	315

6.3.4.5	Symbol / Subslot boundary time mask for subslot TTI	317

6.3.4.6	Subslot PUCCH / subslot PUSCH / SRS time mask for subslot TTI	318

6.3.4.7	Symbol / Slot boundary time mask for slot TTI	321

6.3.4.8	Slot PUCCH / slot PUSCH / SRS time mask for slot TTI	322

6.3.4.9	Consecutive subslot and slot TTI or consecutive subslot and subframe TTI  time mask	322

6.3.4.10	Consecutive subframe and subslot TTI or consecutive slot and subslot TTI time mask	322

6.3.4.11	Consecutive TTI and slot TTI or consecutive slot TTI and TTI time mask	323

6.3.4A	ON/OFF time mask for CA	324

6.3.4B	ON/OFF time mask for UL-MIMO	324

6.3.4D	ON/OFF time mask for ProSe	324

6.3.4D.1	 General time mask for ProSe	324

6.3.4D.2	 PSSS/SSSS time mask	325

6.3.4D.3	 PSSS / SSSS / PSBCH time mask	326

6.3.4D.4	 PSSCH / SRS time mask	326

6.3.4F	ON/OFF time mask for category NB1 and NB2	326

6.3.4F.1	General ON/OFF time mask	326

6.3.4F.2	NPRACH time mask	327

6.3.4G	ON/OFF time mask for V2X Communication	327

6.3.4G.1	PSSS / SSSS / PSBCH time mask	328

6.3.5	Power Control	328

6.3.5.1	Absolute power tolerance	328

6.3.5.1.1	 Minimum requirements	328

6.3.5.2	Relative Power tolerance	329

6.3.5.2.1	Minimum requirements	329

6.3.5.3	Aggregate power control tolerance	330

6.3.5.3.1	Minimum requirement	330

6.3.5A	Power control for CA	330

6.3.5A.1	Absolute power tolerance	330

6.3.5A.1.1	 Minimum requirements	330

6.3.5A.2	Relative power tolerance	331

6.3.5A.2.1	Minimum requirements	331

6.3.5A.3	Aggregate power control tolerance	331

6.3.5A.3.1	 Minimum requirements	331

6.3.5B	Power control for UL-MIMO	332

6.3.5D	Power Control for ProSe	332

6.3.5D.1	Absolute power tolerance	332

6.3.5E	Power control for category M1 and M2	332

6.3.5E.1	Absolute power tolerance	332

6.3.5E.2	Relative Power tolerance	332

6.3.5E.3	Aggregate power control tolerance	333

6.3.5E.3.1	Minimum requirement	333

6.3.5F	Power Control for category NB1 and NB2	333

6.3.5F.1	Absolute power tolerance	334

6.3.5F.2	Relative power tolerance	334

6.3.5F.3	Aggregate power control tolerance for category NB1 and NB2	335

6.3.5F.3.1	Minimum requirement	335

6.3.5G	Power Control for V2X Communication	335

6.3.5G.1	Absolute power tolerance	335

6.4	Void	336

6.5	Transmit signal quality	336

6.5.1	Frequency error	336

6.5.1A 	Frequency error for CA	336

6.5.1B	Frequency error for UL-MIMO	336

6.5.1D	Frequency error for ProSe	336

6.5.1E	Frequency error for UE category M1 and M2	336

6.5.1F	Frequency error for UE category NB1 and NB2	337

6.5.1G	Frequency error for V2X Communication	337

6.5.2	Transmit modulation quality	337

6.5.2.1	 Error Vector Magnitude	338

6.5.2.1.1	Minimum requirement	338

6.5.2.2	 Carrier leakage	338

6.5.2.2.1	 Minimum requirements	339

6.5.2.3	In-band emissions	339

6.5.2.3.1	 Minimum requirements	339

6.5.2.4	 EVM equalizer spectrum flatness	340

6.5.2.4.1	Minimum requirements	340

6.5.2A	Transmit modulation quality for CA	341

6.5.2A.1	Error Vector Magnitude	342

6.5.2A.2	Carrier leakage for CA	342

6.5.2A.2.1	Minimum requirements	342

6.5.2A.3	In-band emissions	342

6.5.2A.3.1	Minimum requirement for CA	342

6.5.2B	Transmit modulation quality for UL-MIMO	345

6.5.2B.1	Error Vector Magnitude	345

6.5.2B.2	Carrier leakage	345

6.5.2B.3	In-band emissions	345

6.5.2B.4	EVM equalizer spectrum flatness for UL-MIMO	345

6.5.2D	Transmit modulation quality for ProSe	345

6.5.2D.1	Error Vector Magnitude	345

6.5.2D.2	Carrier leakage	346

6.5.2D.3	In-band emissions	346

6.5.2D.4	EVM equalizer spectrum flatness for ProSe	346

6.5.2E	Transmit modulation quality for category M1 and M2	346

6.5.2E.1	Error Vector Magnitude	346

The Error Vector Magnitude is defined in section 6.5.2.1.	346

6.5.2E.2	Carrier leakage	346

6.5.2E.2.1	Minimum requirements	346

6.5.2E.3	In-band emissions	346

6.5.2E.3.1	Minimum requirements	346

6.5.2F	Transmit modulation quality for Category NB1 and NB2	348

6.5.2F.1	Error Vector Magnitude	348

6.5.2F.2	Carrier leakage	348

6.5.2F.3	In-band emissions	348

6.5.2G	Transmit modulation quality for V2X Communication	349

6.5.2G.1	Error Vector Magnitude	349

6.5.2G.2	Carrier leakage	350

6.5.2G.3	In-band emissions	350

6.5.2G.4	EVM equalizer spectrum flatness	350

6.6	Output RF spectrum emissions	350

6.6.1	Occupied bandwidth	350

6.6.1.1	Additional minimum requirement for E-UTRA (network signalled value “NS_29”)	351

6.6.1A	Occupied bandwidth for CA	351

6.6.1B	 Occupied bandwidth for UL-MIMO	351

6.6.1F	Occupied bandwidth for category NB1 and NB2	352

6.6.1G	Occupied bandwidth for V2X Communication	352

6.6.2	Out of band emission	352

6.6.2.1	Spectrum emission mask	352

6.6.2.1.1	Minimum requirement	352

6.6.2.1A	Spectrum emission mask for CA	353

6.6.2.2	Additional spectrum emission mask	354

6.6.2.2.1	Minimum requirement (network signalled value "NS_03", “NS_11”, "NS_20", and “NS_21”)	354

6.6.2.2.2	Minimum requirement (network signalled value "NS_04")	355

6.6.2.2.3	Minimum requirement (network signalled value "NS_06" or “NS_07”)	355

6.6.2.2.4	Minimum requirement (network signalled value "NS_33" or “NS_34”)	356

6.6.2.2.5	Minimum requirement (network signalled value “NS_27” and “NS_43”)	356

6.6.2.2.6	Minimum requirement (network signalled value "NS_28”)	357

6.6.2.2.7	Minimum requirement (network signalled value "NS_35")	357

6.6.2.2A	Additional Spectrum Emission Mask for CA	358

6.6.2.2A.1	Minimum requirement (network signalled value "CA_NS_04")	358

6.6.2.2A.2	Minimum requirement CA_66B (network signalled value "CA_NS_09")	358

6.6.2.2A.3	Minimum requirement CA_66C (network signalled value "CA_NS_09")	359

6.6.2.2A.4	Minimum requirement CA_48B and CA_48C (network signalled value "CA_NS_10")	359

6.6.2.3	Adjacent Channel Leakage Ratio	360

6.6.2.3.1	Minimum requirement E-UTRA	360

6.6.2.3.1a	Additional minimum requirement for E-UTRA (network signalled value “NS_29”)	361

6.6.2.3.1A	Void	362

6.6.2.3.1Aa	Void	362

6.6.2.3.2	Minimum requirements UTRA	362

6.6.2.3.2A	Minimum requirement UTRA for CA	363

6.6.2.3.3A	Minimum requirements for CA E-UTRA	365

6.6.2.4	Void	366

6.6.2.4.1	Void	366

6.6.2A	Void	366

6.6.2B	 Out of band emission for UL-MIMO	366

6.6.2C	 Void	366

6.6.2D	 Out of band emission for ProSe	366

6.6.2F	Out of band emission for category NB1 and NB2	367

6.6.2F.1	Spectrum emission mask	367

6.6.2F.2	Additional Spectrum Emission Mask for Category NB1 and NB2	367

6.6.2F.2.1	Minimum requirement (network signalled value "NS_02")	367

6.6.2F.2.2	Minimum requirement (network signalled value "NS_03")	367

6.6.2F.3	Adjacent Channel Leakage Ratio for category NB1 and NB2	368

6.6.2G	 Out of band emission for V2X Communication	368

6.6.3	Spurious emissions	369

6.6.3.1	Minimum requirements	369

6.6.3.1A	Minimum requirements for CA	370

6.6.3.2	Spurious emission band UE co-existence	371

6.6.3.2A	Spurious emission band UE co-existence for CA	380

6.6.3.3	Additional spurious emissions	395

6.6.3.3.1 	Minimum requirement (network signalled value "NS_05")	396

6.6.3.3.2 	Minimum requirement (network signalled value “NS_07”)	396

6.6.3.3.3 	Minimum requirement (network signalled value “NS_08”)	396

6.6.3.3.4 	Minimum requirement (network signalled value “NS_09”)	396

6.6.3.3.5 	Minimum requirement (network signalled value "NS_12")	397

6.6.3.3.6 	Minimum requirement (network signalled value “NS_13”)	397

6.6.3.3.7 	Minimum requirement (network signalled value “NS_14”)	397

6.6.3.3.8 	Minimum requirement (network signalled value “NS_15”)	398

6.6.3.3.9 	Minimum requirement (network signalled value “NS_16”)	398

6.6.3.3.10 	Minimum requirement (network signalled value “NS_17”)	398

6.6.3.3.11 	Minimum requirement (network signalled value “NS_18”)	398

6.6.3.3.12 	Minimum requirement (network signalled value “NS_19”)	399

6.6.3.3.13 	Minimum requirement (network signalled value “NS_11”)	399

6.6.3.3.14	Minimum requirement (network signalled value “NS_20”)	399

6.6.3.3.15	Minimum requirement (network signalled value “NS_21”)	399

6.6.3.3.16	Minimum requirement (network signalled value "NS_22")	400

6.6.3.3.17	Minimum requirement (network signalled value “NS_23”)	400

6.6.3.3.18	Void	401

6.6.3.3.19	Minimum requirement (network signalled value "NS_04")	401

6.6.3.3.20	Minimum requirement (network signalled value “NS_24”)	401

6.6.3.3.21	Minimum requirement (network signalled value “NS_25”)	402

6.6.3.3.22	Minimum requirement (network signalled value “NS_26”)	402

6.6.3.3.23	Minimum requirement (network signalled value “NS_27” and “NS_43”)	402

6.6.3.3.24	Minimum requirement (network signalled value “NS_28”)	402

6.6.3.3.25	Minimum requirement (network signalled value “NS_29”)	403

6.6.3.3.26	Minimum requirement (network signalled value “NS_30”)	403

6.6.3.3.27	Minimum requirement (network signalled value “NS_31”)	404

6.6.3.3.28	Minimum requirement (network signalled value “NS_36”)	405

6.6.3.3.29	Minimum requirement (network signalled value “NS_38”)	405

6.6.3.3.30	Minimum requirement (network signalled value “NS_39”)	405

6.6.3.3.31	Minimum requirement (network signalled value “NS_40” and “NS_41”)	406

6.6.3.3.32	Minimum requirement (network signalled value “NS_42”)	406

6.6.3.3.33	Minimum requirement (network signalled value “NS_44”)	406

6.6.3.3.34	Minimum requirement (network signalled value “NS_45”)	406

6.6.3.3.35	Minimum requirement (network signalled value “NS_56”)	407

6.6.3.3A	Additional spurious emissions for CA	407

6.6.3.3A.1	Minimum requirement for CA_1C (network signalled value "CA_NS_01")	408

6.6.3.3A.2 	Minimum requirement for CA_1C (network signalled value "CA_NS_02")	408

6.6.3.3A.3	Minimum requirement for CA_1C (network signalled value "CA_NS_03")	408

6.6.3.3A.4	Minimum requirement for CA_38C (network signalled value "CA_NS_05")	409

6.6.3.3A.5	Minimum requirement for CA_7C (network signalled value "CA_NS_06")	409

6.6.3.3A.6	Minimum requirement for CA_39C and CA_39C-41A (network signalled value "CA_NS_07")	409

6.6.3.3A.7	Minimum requirement for CA_42C (network signalled value "CA_NS_08")	409

6.6.3.3A.8	Minimum requirement for CA_41C and CA_41D (network signalled value "CA_NS_04")	410

6.6.3.3A.9	Void	410

6.6.3.3A.10	Minimum requirement for CA_48B and CA_48C (network signalled value "CA_NS_10")	410

6.6.3A	Void	410

6.6.3B	 Spurious emission for UL-MIMO	411

6.6.3C	 Void	411

6.6.3D	 Spurious emission for ProSe	411

6.6.3F	Spurious emission for category NB1 and NB2	411

6.6.3G	Spurious emission for V2X Communication	411

6.6A	Void	415

6.6B	Void	415

6.7	Transmit intermodulation	415

6.7.1	Minimum requirement	415

6.7.1A	Minimum requirement for CA	416

6.7.1B	Minimum requirement for UL-MIMO	416

6.7.1F	Minimum requirement for category NB1 and NB2	416

6.7.1G	Minimum requirement for V2X Communication	417

6.8	Void	417

6.8A	Void	417

6.8B	 Time alignment error for UL-MIMO	417

6.8B.1	 Minimum Requirements	417

6.8C	Void	418

6.8D	Void	418

6.8E	Void	418

6.8F	Void	418

6.8G	Time alignment error	418

7	Receiver characteristics	418

7.1	General	418

7.2	Diversity characteristics	419

7.3	Reference sensitivity power level	419

7.3.1	Minimum requirements (QPSK)	419

7.3.1A	Minimum requirements (QPSK) for CA	454

7.3.1B	Minimum requirements (QPSK) for UL-MIMO	554

7.3.1D	Minimum requirements (QPSK) for ProSe	554

7.3.1E	Minimum requirements (QPSK) for UE category 0, M1, M2 and 1bis	556

7.3.1F	Minimum requirements for UE category NB1 and NB2	565

7.3.1F.1	Reference sensitivity for UE category NB1 and NB2	565

7.3.1F.2	Void	566

7.3.1G	Minimum requirements (QPSK) for V2X	566

7.3.2	Void	568

7.4	Maximum input level	568

7.4.1	Minimum requirements	569

7.4.1A	Minimum requirements for CA	569

7.4.1B	Minimum requirements for UL-MIMO	570

7.4.1D	Minimum requirements for ProSe	570

7.4.1F	Minimum requirements for category NB1 and NB2	571

7.4.1G	Minimum requirements for V2X	571

7.4A	Void	572

7.4A.1	Void	572

7.5	Adjacent Channel Selectivity (ACS)	572

7.5.1	Minimum requirements	572

7.5.1A	Minimum requirements for CA	573

7.5.1B	Minimum requirements for UL-MIMO	577

7.5.1D	Minimum requirements for ProSe	577

7.5.1F	Minimum requirements for category NB1 and NB2	578

7.5.1G	Minimum requirements for V2X	579

7.6	Blocking characteristics	581

7.6.1	In-band blocking	581

7.6.1.1	Minimum requirements	581

7.6.1.1A	Minimum requirements for CA	583

7.6.1.1D	Minimum requirements for ProSe	586

7.6.1.1F	Minimum requirements for category NB1 and NB2	587

7.6.1.1G	Minimum requirements for V2X	587

7.6.2	Out-of-band blocking	589

7.6.2.1	Minimum requirements	589

7.6.2.1A	Minimum requirements for CA	591

7.6.2.1D	Minimum requirements for ProSe	594

7.6.2.1F	Minimum requirements for category NB1 and NB2	594

7.6.2.1G	Minimum requirements for V2X	595

7.6.3	Narrow band blocking	596

7.6.3.1	Minimum requirements	596

7.6.3.1A	Minimum requirements for CA	597

7.6.3.1D	Minimum requirements for ProSe	598

7.6A	Void	599

7.6B	Blocking characteristics for UL-MIMO	599

7.7	Spurious response	599

7.7.1	Minimum requirements	599

7.7.1A	Minimum requirements for CA	600

7.7.1B	Minimum requirements for UL-MIMO	601

7.7.1D	Minimum requirements for ProSe	601

7.7.1F	Minimum requirements for UE category NB1 and NB2	602

7.7.1G	Minimum requirements for V2X	602

7.8	Intermodulation characteristics	603

7.8.1	Wide band intermodulation	603

7.8.1.1	Minimum requirements	603

7.8.1A	Minimum requirements for CA	604

7.8.1B	Minimum requirements for UL-MIMO	607

7.8.1D	Minimum requirements for ProSe	607

7.8.1F	Minimum requirements for category NB1 and NB2	608

7.8.1G	Minimum requirements	608

7.8.2	Void	609

7.9	Spurious emissions	609

7.9.1	Minimum requirements	610

7.9.1A	Minimum requirements	610

7.10	Receiver image	611

7.10.1	Void	611

7.10.1A	Minimum requirements for CA	611

7.10.1G	Minimum requirements for V2X Communication	611

8	Performance requirement	612

8.1	General	612

8.1.1	Receiver antenna capability	612

8.1.1.1	Simultaneous unicast and MBMS operations	613

8.1.1.2	Dual-antenna receiver capability in idle mode	613

8.1.2	Applicability of requirements	613

8.1.2.1	Applicability of requirements for different channel bandwidths	613

8.1.2.2	Definition of CA capability	613

8.1.2.2A	Definition of dual connectivity capability	618

8.1.2.3	Applicability and test rules for different CA configurations and bandwidth combination sets	619

8.1.2.3A	Applicability and test rules for different dual connectivity configuration and bandwidth combination set	621

8.1.2.3B	Applicability and test rules for different TDD-FDD CA configurations and bandwidth combination sets	622

8.1.2.3C	Applicability and test rules for SDR tests for 4Rx capable UEs	624

8.1.2.3D	Applicability and test rules for different CA with LAA SCell(s) configurations and bandwidth combination sets	624

8.1.2.3E	Applicability and test rules for SDR tests for 8Rx capable UEs	625

8.1.2.4	Test coverage for different number of component carriers	626

8.1.2.5	Applicability of performance requirements for Type B receiver	627

8.1.2.6	Applicability of performance requirements for 4Rx capable UEs	627

8.1.2.6.1	Applicability rule and antenna connection for single carrier tests with 2Rx	627

8.1.2.6.2	Applicability rule and antenna connection for CA and DC tests with 2Rx	629

8.1.2.6.3	Applicability rule and antenna connection for single carrier tests with 4Rx	629

8.1.2.6.4	Applicability rule for 256QAM tests	630

8.1.2.6.5	Applicability rule and antenna connection for CA and DC tests with 4Rx	630

8.1.2.6.6	Applicability rule for Type C with 4Rx	634

8.1.2.6.7	Applicability rule for 1024QAM tests	634

8.1.2.7	Applicability of Enhanced Downlink Control Channel Performance Requirements	634

8.1.2.8	Applicability of performance requirements for CDM-multiplexed DM RS with interfering simultaneous transmission (FRC) with multiple CSI-RS configurations	636

8.1.2.8A	Applicability of performance requirements for UE supporting coverage enhancement	636

8.1.2.9	Applicability of SDR requirements for CA and LAA	637

8.1.2.10	Applicability of performance requirements for Multi-user Superposed Transmission	638

8.1.2.11	Applicability CRS interference mitigation receivers performance requirements	638

8.1.2.12	Applicability of performance requirements for 8Rx capable UEs	639

8.1.2.12.1	Applicability rule and antenna connection for single carrier PDSCH tests	639

8.1.2.12.2	Applicability rule and antenna connection for control channel tests	645

8.1.2.12.3	Applicability rule and antenna connection for CA and DC tests	645

8.1.3	UE category and UE DL category	646

8.2	Demodulation of PDSCH (Cell-Specific Reference Symbols)	646

8.2.1	FDD (Fixed Reference Channel)	646

8.2.1.1	Single-antenna port performance	647

8.2.1.1.1	Minimum Requirement	647

8.2.1.1.2	Void	653

8.2.1.1.3	Void	653

8.2.1.1.4	Minimum Requirement 1 PRB allocation in presence of MBSFN	653

8.2.1.1.4A	Minimum Requirement 1 PRB allocation in presence of FeMBMS Unicast-mixed Cell under CA	653

8.2.1.2	Transmit diversity performance	654

8.2.1.2.1	Minimum Requirement 2 Tx Antenna Port	654

8.2.1.2.2	Minimum Requirement 4 Tx Antenna Port	655

8.2.1.2.3	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)	655

8.2.1.2.3A	Minimum Requirement 2 Tx Antenna Ports (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	657

8.2.1.2.4	Enhanced Performance Requirement Type A - 2 Tx Antenna Ports with TM3 interference model	659

8.2.1.2.5	Enhanced Performance Requirement Type B - 2 Tx Antenna Ports with TM2 interference model	661

8.2.1.2.6	Enhanced Performance Requirement Type B - 2 Tx Antenna Ports with TM9 interference model	662

8.2.1.2.7	Minimum Requirement 2 Tx Antenna Port (Superposed transmission)	663

8.2.1.3	Open-loop spatial multiplexing performance	663

8.2.1.3.1	Minimum Requirement 2 Tx Antenna Port	663

8.2.1.3.1A	Soft buffer management test	667

8.2.1.3.1B	Enhanced Performance Requirement Type C –2Tx Antenna Ports	668

8.2.1.3.1C	Enhanced Performance Requirement Type C - 2 Tx Antenna Ports with TM1 interference	669

8.2.1.3.2	Minimum Requirement 4 Tx Antenna Port	670

8.2.1.3.3	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)	670

8.2.1.3.4	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	674

8.2.1.3.5	Minimum Requirement 2 Tx Antenna Port (Superposed transmission)	676

8.2.1.3.6	Minimum Requirement 2 Tx Antenna Port (network-based CRS interference mitigation)	677

8.2.1.4	Closed-loop spatial multiplexing performance	678

8.2.1.4.1	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Port	678

8.2.1.4.1A	Minimum Requirement Single-Layer Spatial Multiplexing 4 Tx Antenna Port	679

8.2.1.4.1B	Enhanced Performance Requirement Type A - Single-Layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model	679

8.2.1.4.1C	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Ports (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	681

8.2.1.4.1D	Enhanced Performance Requirement Type B - Single-layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model	684

8.2.1.4.1E	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Ports with CRS assistance information	686

8.2.1.4.1F	Minimum Requirement Single-Layer Spatial Multiplexing 4 Tx Antenna Ports with CRS assistance information	687

8.2.1.4.2	Minimum Requirement Multi-Layer Spatial Multiplexing 2 Tx Antenna Port	688

8.2.1.4.2A	Enhanced Performance Requirement Type C – Multi-layer Spatial Multiplexing 2Tx Antenna Ports	689

8.2.1.4.3	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port	689

8.2.1.4.3A	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port for dual connectivity	693

8.2.1.4.4	Minimum Requirement Multi-Layer Spatial Multiplexing 2 Tx Antenna Port (Superposed transmission)	695

8.2.1.5	MU-MIMO	696

8.2.1.6	[Control channel performance: D-BCH and PCH]	696

8.2.1.7	Carrier aggregation with power imbalance	696

8.2.1.7.1	Minimum Requirement	696

8.2.1.8	Intra-band non-contiguous carrier aggregation with timing offset	697

8.2.1.8.1	Minimum Requirement	697

8.2.1.9	HST-SFN performance	698

8.2.1.9.1	Minimum Requirement	698

8.2.1.9.2	Minimum Requirement for Rel-16 further enhanced HST	701

8.2.1.10	Intra-band contiguous carrier aggregation with minimum channel spacing	701

8.2.1.10.1	Minimum Requirement	702

8.2.2	TDD (Fixed Reference Channel)	702

8.2.2.1	Single-antenna port performance	703

8.2.2.1.1	Minimum Requirement	703

8.2.2.1.2	Void	708

8.2.2.1.3	Void	708

8.2.2.1.4	Minimum Requirement 1 PRB allocation in presence of MBSFN	708

8.2.2.2	Transmit diversity performance	708

8.2.2.2.1	Minimum Requirement 2 Tx Antenna Port	708

8.2.2.2.2	Minimum Requirement 4 Tx Antenna Port	709

8.2.2.2.3	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)	710

8.2.2.2.3A	Minimum Requirement 2 Tx Antenna Ports (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	711

8.2.2.2.4	Enhanced Performance Requirement Type A – 2 Tx Antenna Ports with TM3 interference model	713

8.2.2.2.5	Minimum Requirement 2 Tx Antenna Port (when EIMTA-MainConfigServCell-r12 is configured)	715

8.2.2.2.6	Enhanced Performance Requirement Type B - 2 Tx Antenna Ports with TM2 interference model	715

8.2.2.2.7	Enhanced Performance Requirement Type B - 2 Tx Antenna Ports with TM9 interference model	717

8.2.2.2.8	Minimum Requirement 2 Tx Antenna Port (Superposed transmission)	718

8.2.2.3	Open-loop spatial multiplexing performance	719

8.2.2.3.1	Minimum Requirement 2 Tx Antenna Port	719

8.2.2.3.1A	Soft buffer management test	722

8.2.2.3.1B	Enhanced Performance Requirement Type C - 2Tx Antenna Ports	722

8.2.2.3.1C	Enhanced Performance Requirement Type C - 2 Tx Antenna Ports with TM1 interference	723

8.2.2.3.2	Minimum Requirement 4 Tx Antenna Port	724

8.2.2.3.3	Minimum Requirement 2Tx antenna port (demodulation subframe overlaps with aggressor cell ABS)	725

8.2.2.3.4	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	729

8.2.2.3.5	Minimum Requirement 2 Tx Antenna Port (Superposed transmission)	731

8.2.2.3.6	Minimum Requirement 2 Tx Antenna Port (network-based CRS interference mitigation)	732

8.2.2.4	Closed-loop spatial multiplexing performance	733

8.2.2.4.1	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Port	733

8.2.2.4.1A	Minimum Requirement Single-Layer Spatial Multiplexing 4 Tx Antenna Port	734

8.2.2.4.1B	Enhanced Performance Requirement Type A – Single-Layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model	734

8.2.2.4.1C	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Ports (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	736

8.2.2.4.1D	Enhanced Performance Requirement Type B - Single-layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model	738

8.2.2.4.1E	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Ports with CRS assistance information	740

8.2.2.4.1F	Minimum Requirement Single-Layer Spatial Multiplexing 4 Tx Antenna Ports with CRS assistance information	741

8.2.2.4.2	Minimum Requirement Multi-Layer Spatial Multiplexing 2 Tx Antenna Port	743

8.2.2.4.2A	Enhanced Performance Requirement Type C Multi-Layer Spatial Multiplexing 2 Tx Antenna Port	743

8.2.2.4.3	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port	744

8.2.2.4.3A	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port for dual connectivity	748

8.2.2.4.4	Void	749

8.2.2.4.5	Minimum Requirement Multi-Layer Spatial Multiplexing 2 Tx Antenna Port (Superposed transmission)	749

8.2.2.5	MU-MIMO	750

8.2.2.6	[Control channel performance: D-BCH and PCH]	750

8.2.2.7	Carrier aggregation with power imbalance	750

8.2.2.7.1	Minimum Requirement	750

8.2.2.8	Intra-band contiguous carrier aggregation with minimum channel spacing	751

8.2.2.8.1	Minimum Requirement	751

8.2.2.9	HST-SFN performance	752

8.2.2.9.1	Minimum Requirement	752

8.2.2.9.2	Minimum Requirement for Rel-16 further enhanced HST	754

8.2.3	TDD FDD CA (Fixed Reference Channel)	755

8.2.3.1	Single-antenna port performance	756

8.2.3.1.1	Minimum Requirement for FDD PCell	756

8.2.3.1.2	Minimum Requirement for TDD PCell	760

8.2.3.2	Open-loop spatial multiplexing performance 2Tx Antenna port	764

8.2.3.2.1	Minimum Requirement for FDD PCell	764

8.2.3.2.1A	Soft buffer management test for FDD PCell	768

8.2.3.2.2	Minimum Requirement for TDD PCell	769

8.2.3.2.2A	Soft buffer management test for TDD PCell	773

8.2.3.3	Closed-loop spatial multiplexing performance 4Tx Antenna Port	774

8.2.3.3.1	Minimum Requirement for FDD PCell	774

8.2.3.3.2	Minimum Requirement for TDD PCell	778

8.2.3.4	Minimum Requirement for Closed-loop spatial multiplexing performance 4Tx Antenna Port for dual connectivity	782

8.2.3.5	HST-SFN performance	784

8.2.3.5.0	General	784

8.2.3.5.1	Minimum Requirement for FDD PCell	784

8.2.3.5.2	Minimum Requirement for TDD PCell	787

8.2.4	LAA	790

8.2.4.1	Closed-loop spatial multiplexing performance 4Tx Antenna Port	790

8.2.4.1.1	FDD PCell (FDD single carrier)	790

8.2.4.1.2	TDD PCell (TDD single carrier)	794

8.3	Demodulation of PDSCH (User-Specific Reference Symbols)	797

8.3.1	FDD	797

8.3.1.1	Single-layer Spatial Multiplexing	798

8.3.1.1A	Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model	800

8.3.1.1B	Single-layer Spatial Multiplexing (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	803

8.3.1.1C	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with TM9 interference model	806

8.3.1.1D	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with CRS interference model	808

8.3.1.1E	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with TM3 interference model	809

8.3.1.1F	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with TM10 serving cell configuration and TM9 interference model	810

8.3.1.1G	Single-layer Spatial Multiplexing (CRS assistance information is configured)	812

8.3.1.1H	Single-layer Spatial Multiplexing (With Enhanced DMRS table configured)	814

8.3.1.1I	Single-layer Spatial Multiplexing (with assistance information for simultaneous transmition interfering PDSCH)	815

8.3.1.2	Dual-Layer Spatial Multiplexing	817

8.3.1.2A	Enhanced Performance Requirement Type C - Dual-Layer Spatial Multiplexing	818

8.3.1.3	Performance requirements for DCI format 2D and non Quasi Co-located Antenna Ports	819

8.3.1.3.1	Minimum requirement with Same Cell ID (with single NZP CSI-RS resource)	819

8.3.1.3.2	Minimum requirements with Same Cell ID (with multiple NZP CSI-RS resources)	821

8.3.1.3.3	Minimum requirement with Different Cell ID and Colliding CRS (with single NZP CSI-RS resource)	823

8.3.1.3.4	Minimum requirement with Different Cell ID and non-colliding CRS (with single NZP CSI-RS resource and CRS assistance information is configured)	825

8.3.1.3.5	Minimum requirements with different Cell ID and non-colliding CRS (with multiple NZP CSI-RS resources and CRS assistance information is configured)	827

8.3.1.3.6	Minimum requirements for QCL Type C and 2 Layers Spatial Multiplexing	830

8.3.1.4	Performance Requirements for semiOpenLoop transmission	832

8.3.2	TDD	834

8.3.2.1	Single-layer Spatial Multiplexing	834

8.3.2.1A	Single-layer Spatial Multiplexing (with multiple CSI-RS configurations)	836

8.3.2.1B	Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model	838

8.3.2.1C	Single-layer Spatial Multiplexing (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	841

8.3.2.1D	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with TM9 interference	844

8.3.2.1E	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with CRS interference model	846

8.3.2.1F	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with TM3 interference	848

8.3.2.1G	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with TM10 serving cell configuration and TM9 interference model	849

8.3.2.1H	Single-layer Spatial Multiplexing (CRS assistance information is configured)	851

8.3.2.1I	Single-layer Spatial Multiplexing (With Enhanced DMRS table configured)	853

8.3.2.1J	Single-layer Spatial Multiplexing (with assistance information for simultaneous transmition interfering PDSCH)	854

8.3.2.2	Dual-Layer Spatial Multiplexing	856

8.3.2.2A	Enhanced Performance Requirement Type C - Dual-Layer Spatial Multiplexing	856

8.3.2.3	Dual-Layer Spatial Multiplexing (with multiple CSI-RS configurations)	857

8.3.2.4	Performance requirements for DCI format 2D and non Quasi Co-located Antenna Ports	858

8.3.2.4.1	Minimum requirement with Same Cell ID (with single NZP CSI-RS resource)	858

8.3.2.4.2	Minimum requirements with Same Cell ID (with multiple NZP CSI-RS resources)	860

8.3.2.4.3	Minimum requirement with Different Cell ID and Colliding CRS (with single NZP CSI-RS resource)	862

8.3.2.4.4	Minimum requirement with Different Cell ID and non-Colliding CRS (with single NZP CSI-RS resource and CRS assistance information is configured)	864

8.3.2.4.5	Minimum requirements with different Cell ID and non-colliding CRS (with multiple NZP CSI-RS resources and CRS assistance information is configured)	866

8.3.2.4.6	Minimum requirements for QCL Type C and 2 Layers Spatial Multiplexing	869

8.3.2.5	Performance Requirements for semiOpenLoop transmission	871

8.3.3	LAA	873

8.3.3.1	Dual-Layer Spatial Multiplexing with DM-RS	873

8.3.3.1.1	FDD PCell (FDD single carrier)	873

8.3.3.1.2	TDD Pcell (TDD single carrier)	878

8.4	Demodulation of PDCCH/PCFICH	882

8.4.1	FDD	882

8.4.1.1	Single-antenna port performance	883

8.4.1.2	Transmit diversity performance	883

8.4.1.2.1	Minimum Requirement 2 Tx Antenna Port	883

8.4.1.2.2	Minimum Requirement 4 Tx Antenna Port	883

8.4.1.2.3 Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)	884

8.4.1.2.4	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	889

8.4.1.2.5	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Port under Asynchronous Network	895

8.4.1.2.6	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	896

8.4.1.2.7	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Port with Colliding CRS Dominant Interferer	897

8.4.1.2.8	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	898

8.4.1.2.9	Enhanced Downlink Control Channel Performance Requirement Type A - 4 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	899

8.4.2	TDD	900

8.4.2.1	Single-antenna port performance	901

8.4.2.2	Transmit diversity performance	901

8.4.2.2.1	Minimum Requirement 2 Tx Antenna Port	901

8.4.2.2.2	Minimum Requirement 4 Tx Antenna Port	902

8.4.2.2.3	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)	902

8.4.2.2.4	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	906

8.4.2.2.5	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Port with Colliding CRS Dominant Interferer	910

8.4.2.2.6	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	911

8.4.2.2.7	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Port with Colliding CRS Dominant Interferer	912

8.4.2.2.8	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	913

8.4.2.2.9	Enhanced Downlink Control Channel Performance Requirement Type A - 4 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	914

8.4.3	LAA	915

8.4.3.1	Transmit diversity performance	915

8.4.3.1.1	FDD Pcell (FDD single carrier)	915

8.4.3.1.2	TDD Pcell (TDD single carrier)	916

8.5	Demodulation of PHICH	917

8.5.1	FDD	918

8.5.1.1	Single-antenna port performance	918

8.5.1.2	Transmit diversity performance	918

8.5.1.2.1	Minimum Requirement 2 Tx Antenna Port	918

8.5.1.2.2	Minimum Requirement 4 Tx Antenna Port	919

8.5.1.2.3	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)	919

8.5.1.2.4	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	921

8.5.1.2.5	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Ports under Asynchronous Network	924

8.5.1.2.6	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Ports with Non-Colliding CRS Dominant Interferer	925

8.5.1.2.7	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Ports with Colliding CRS Dominant Interferer	926

8.5.1.2.8	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Ports with Non-Colliding CRS Dominant Interferer	927

8.5.2	TDD	928

8.5.2.1	Single-antenna port performance	929

8.5.2.2	Transmit diversity performance	929

8.5.2.2.1	Minimum Requirement 2 Tx Antenna Port	929

8.5.2.2.2	Minimum Requirement 4 Tx Antenna Port	930

8.5.2.2.3	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)	930

8.5.2.2.4	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	932

8.5.2.2.5	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Ports with Colliding CRS Dominant Interferer	934

8.5.2.2.6	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Ports with Non-Colliding CRS Dominant Interferer	935

8.5.2.2.7	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Ports with Colliding CRS Dominant Interferer	936

8.5.2.2.8	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Ports with Non-Colliding CRS Dominant Interferer	937

8.6	Demodulation of PBCH	938

8.6.1	FDD	938

8.6.1.1	Single-antenna port performance	938

8.6.1.2	Transmit diversity performance	939

8.6.1.2.1	Minimum Requirement 2 Tx Antenna Port	939

8.6.1.2.2	Minimum Requirement 4 Tx Antenna Port	939

8.6.1.2.3	Minimum Requirement 2 Tx Antenna Port under Time Domain Measurement Resource Restriction with CRS Assistance Information	939

8.6.2	TDD	941

8.6.2.1	Single-antenna port performance	941

8.6.2.2	Transmit diversity performance	941

8.6.2.2.1	Minimum Requirement 2 Tx Antenna Port	941

8.6.2.2.2	Minimum Requirement 4 Tx Antenna Port	941

8.6.2.2.3	Minimum Requirement 2 Tx Antenna Port under Time Domain Measurement Resource Restriction with CRS Assistance Information	942

8.7	Sustained downlink data rate provided by lower layers	943

8.7.1	FDD (single carrier and CA)	943

8.7.2	TDD (single carrier and CA)	960

8.7.3	FDD (EPDCCH scheduling)	964

8.7.4	TDD (EPDCCH scheduling)	966

8.7.5	TDD FDD CA	968

8.7.5.1	Minimum Requirement FDD PCell	969

8.7.5.2	Minimum Requirement TDD PCell	977

8.7.6	FDD (DC)	986

8.7.7	TDD (DC)	993

8.7.8	TDD FDD (DC)	996

8.7.9	FDD (4 Rx)	999

8.7.10	TDD (4 Rx)	1001

8.7.11	TDD FDD CA (4 Rx)	1003

8.7.11.1	Void	1005

8.7.12	LAA	1005

8.7.12.1	FDD CA in licensed bands	1005

8.7.12.2	TDD CA in licensed bands	1007

8.7.12.3	TDD-FDD CA in licensed bands	1009

8.7.13	FDD DC (4 Rx)	1012

8.7.14	TDD DC (4 Rx)	1013

8.7.15	TDD FDD DC (4 Rx)	1015

8.7.16	FDD (1024QAM and up to 4Rx supported)	1017

8.7.17	TDD (1024QAM and up to 4 Rx supported)	1020

8.7.18	TDD FDD CA (1024QAM and up to 4 Rx supported)	1022

8.7.19	TDD (8 Rx)	1024

8.8	Demodulation of EPDCCH	1026

8.8.1	Distributed Transmission	1026

8.8.1.1	FDD	1026

8.8.1.1.1	Void	1027

8.8.1.2	TDD	1027

8.8.1.2.1	Void	1028

8.8.2	Localized Transmission with TM9	1028

8.8.2.1	FDD	1028

8.8.2.1.1	Void	1029

8.8.2.1.2	Void	1030

8.8.2.2	TDD	1030

8.8.2.2.1	Void	1031

8.8.2.2.2	Void	1031

8.8.3	Localized transmission with TM10 Type B quasi co-location type	1031

8.8.3.1	FDD	1031

8.8.3.2	TDD	1034

8.8.4	Enhanced Downlink Control Channel Performance Requirements Type A - Localized Transmission with CRS Interference Model	1037

8.8.4.1	FDD	1037

8.8.4.2	TDD	1038

8.8.5	Enhanced Downlink Control Channel Performance Requirements Type A - Distributed Transmission with TM9 Interference Model	1040

8.8.5.1	TDD	1040

8.8.6	Enhanced Downlink Control Channel Performance Requirements Type A - Distributed Transmission with TM3 Interference Model	1041

8.8.6.1	FDD	1041

8.9	Demodulation (single receiver antenna)	1042

8.9.1	PDSCH	1042

8.9.1.1	FDD and half-duplex FDD (Fixed Reference Channel)	1042

8.9.1.1.1	Transmit diversity performance (Cell-Specific Reference Symbols)	1042

8.9.1.1.2	Closed-loop spatial multiplexing performance (Cell-Specific Reference Symbols)	1043

8.9.1.1.3	Closed-loop spatial multiplexing performance (User-Specific Reference Symbols)	1045

8.9.1.2	TDD (Fixed Reference Channel)	1048

8.9.1.2.1	Transmit diversity performance (Cell-Specific Reference Symbols)	1048

8.9.1.2.2	 Closed-loop spatial multiplexing performance (Cell-Specific Reference Symbols)	1049

8.9.1.2.3	Closed-loop spatial multiplexing performance (User-Specific Reference Symbols)	1052

8.9.2	PHICH	1054

8.9.2.1	FDD and half-duplex FDD	1054

8.9.2.1.1	Transmit diversity performance	1054

8.9.2.2	TDD	1054

8.9.2.2.1	Transmit diversity performance	1054

8.9.3	PBCH	1055

8.9.3.1	FDD and half-duplex FDD	1055

8.9.3.1.1	Transmit diversity performance	1055

8.9.3.2	TDD	1055

8.9.3.2.1	Transmit diversity performance	1055

8.9.4	PDCCH/PCFICH	1055

8.9.4.1	FDD and half-duplex FDD	1055

8.9.4.1.1	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	1055

8.9.4.1.2	Enhanced Downlink Control Channel Performance Requirement Type A - 4 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	1056

8.9.4.2	TDD	1057

8.9.4.2.1	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	1057

8.9.4.2.2	Enhanced Downlink Control Channel Performance Requirement Type A - 4 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	1058

8.10	Demodulation (4 receiver antenna ports)	1059

8.10.1	PDSCH	1059

8.10.1.1	FDD (Fixed Reference Channel)	1059

8.10.1.1.1	Transmit diversity performance with 2Tx Antenna Ports (Cell-Specific Reference Symbols)	1060

8.10.1.1.1A	Transmit diversity performance wit Enhanced Performance Requirement Type A - 2 Tx Antenna Ports with TM3 interference model	1061

8.10.1.1.2	Open-loop spatial multiplexing performance with 2Tx Antenna Ports (Cell-Specific Reference Symbols)	1062

8.10.1.1.3	Closed-loop spatial multiplexing Enhanced Performance Requirements Type A - Single-Layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model (Cell-Specific Reference Symbols)	1062

8.10.1.1.4	Closed-loop spatial multiplexing performance, Dual-Layer Spatial Multiplexing 4 Tx Antenna Port (Cell-Specific Reference Symbols)	1063

8.10.1.1.4A	Enhanced Performance Requirement Type C - Dual-Layer Spatial Multiplexing with 2Tx Antenna Ports	1064

8.10.1.1.5	Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model (User-Specific Reference Symbols)	1065

8.10.1.1.5A	Single-layer Spatial Multiplexing (User-Specific Reference Symbols)	1068

8.10.1.1.5B	Single-layer Spatial Multiplexing (With Enhanced DMRS table configured)	1069

8.10.1.1.6	Dual-Layer Spatial Multiplexing (User-Specific Reference Symbols)	1070

8.10.1.1.6A	Enhanced Performance Requirement Type C - Dual-Layer Spatial Multiplexing	1072

8.10.1.1.6B	Dual-Layer Spatial Multiplexing with altCQI-Table-1024QAM configured (User-Specific Reference Symbols)	1073

8.10.1.1.7	Open-loop spatial multiplexing, 3 Layer Multiplexing with 4 Tx Antenna Ports (Cell-Specific Reference Symbols)	1074

8.10.1.1.7A	Enhanced Performance Requirement Type C - Open-loop spatial multiplexing, 3 Layer Multiplexing with 4 Tx Antenna Ports (Cell-Specific Reference Symbols)	1074

8.10.1.1.8	Closed-loop spatial multiplexing performance, 4 Layers spatial multiplexing 4 Tx antennas (Cell-Specific Reference Symbols)	1075

8.10.1.1.9	4 Layer Spatial Multiplexing (User-Specific Reference Symbols)	1075

8.10.1.1.9A	Enhanced Performance Requirement Type C - 4 Layer Spatial Multiplexing (User-Specific Reference Symbols)	1077

8.10.1.1.10	Closed loop spatial multiplexing performance - Single-Layer Spatial Multiplexing 2 Tx Antenna Port with CRS assistance information (Cell-Specific Reference Symbols)	1078

8.10.1.1.11	Closed loop spatial multiplexing performance - Single-Layer Spatial Multiplexing 4 Tx Antenna Port with CRS assistance information (Cell-Specific Reference Symbols)	1079

8.10.1.1.12	Closed loop spatial multiplexing performance - Single-Layer Spatial Multiplexing with CRS assistance information (User-Specific Reference Symbols)	1080

8.10.1.1.13	Performance requirements for DCI format 2D and non Quasi Co-located Antenna Ports	1081

8.10.1.1.14	HST-SFN performance	1085

8.10.1.2	TDD (Fixed Reference Channel)	1085

8.10.1.2.1	Transmit diversity performance with 2Tx Antenna Ports (Cell-Specific Reference Symbols)	1086

8.10.1.2.1A	Transmit diversity performance with Enhanced Performance Requirement Type A – 2 Tx Antenna Ports with TM3 interference model	1087

8.10.1.2.2	Open-loop spatial multiplexing performance  with 2Tx Antenna Ports (Cell-Specific Reference Symbols)	1088

8.10.1.2.3	 Closed-loop spatial multiplexing Enhanced Performance Requirements Type A - Single-Layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model (Cell-Specific Reference Symbols)	1088

8.10.1.2.4	Closed-loop spatial multiplexing performance, Dual-Layer Spatial Multiplexing 4 Tx Antenna Ports (Cell-Specific Reference Symbols)	1089

8.10.1.2.4A	Enhanced Performance Requirement Type C - Dual-Layer Spatial Multiplexing with 2Tx Antenna Ports	1090

8.10.1.2.5	Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model (User-Specific Reference Symbols)	1091

8.10.1.2.5A	Single-layer Spatial Multiplexing (with multiple CSI-RS configurations)	1094

8.10.1.2.5B	Single-layer Spatial Multiplexing (With Enhanced DMRS table configured)	1095

8.10.1.2.6	Dual-Layer Spatial Multiplexing (User-Specific Reference Symbols)	1096

8.10.1.2.6A	Enhanced Performance Requirement Type C - Dual-Layer Spatial Multiplexing	1098

8.10.1.2.6B	Dual-Layer Spatial Multiplexing with altCQI-Table-1024QAM configured (User-Specific Reference Symbols)	1099

8.10.1.2.7	Open-loop spatial multiplexing, 3 Layer Multiplexing with 4 Tx Antenna Ports (Cell-Specific Reference Symbols)	1100

8.10.1.2.7A	Enhanced Performance Requirement Type C - Open-loop spatial multiplexing, 3 Layer Multiplexing with 4 Tx Antenna Ports (Cell-Specific Reference Symbols)	1100

8.10.1.2.8	Closed-loop spatial multiplexing performance, 4 Layers spatial multiplexing 4 Tx antennas	1101

8.10.1.2.9	4 Layer Spatial Multiplexing (User-Specific Reference Symbols)	1102

8.10.1.2.9A	Enhanced Performance Requirement Type C - 4 Layer Spatial Multiplexing (User-Specific Reference Symbols)	1103

8.10.1.2.10	Closed loop spatial multiplexing performance - Single-Layer Spatial Multiplexing 2 Tx Antenna Port with CRS assistance information (Cell-Specific Reference Symbols)	1104

8.10.1.2.11	Closed loop spatial multiplexing performance - Single-Layer Spatial Multiplexing 4 Tx Antenna Port with CRS assistance information (Cell-Specific Reference Symbols)	1106

8.10.1.2.12	Closed loop spatial multiplexing performance - Single-Layer Spatial Multiplexing with CRS assistance information (User-Specific Reference Symbols)	1107

8.10.1.2.13	Performance requirements for DCI format 2D and non Quasi Co-located Antenna Ports	1109

8.10.1.2.14	HST-SFN performance	1113

8.10.2	PDCCH/PCFICH	1114

8.10.2.1	FDD	1114

8.10.2.1.1	Single-antenna port performance	1114

8.10.2.1.2	Transmit diversity performance with 2 Tx Antenna Ports	1114

8.10.2.1.3	Transmit diversity performance with 4 Tx Antenna Ports	1114

8.10.2.1.4	Enhanced Downlink Control Channel Performance Requirement Type A - 4 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	1115

8.10.2.2	TDD	1116

8.10.2.2.1	Single-antenna port performance	1116

8.10.2.2.2	Transmit diversity performance with 2 Tx Antenna Ports	1117

8.10.2.2.3	Transmit diversity performance with 4 Tx Antenna Ports	1117

8.10.2.2.4	Enhanced Downlink Control Channel Performance Requirement Type A - 4 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	1117

8.10.3	PHICH	1118

8.10.3.1	 FDD	1118

8.10.3.1.1	Single Tx Antenna Port performance	1119

8.10.3.1.2	Transmit diversity performance with 2 Tx Antenna Ports	1119

8.10.3.1.3	Transmit diversity performance with 4 Tx Antenna Ports	1119

8.10.3.2	TDD	1120

8.10.3.2.1	Single Tx Antenna Port performance	1120

8.10.3.2.2	Transmit diversity performance with 2 Tx Antenna Ports	1121

8.10.3.2.3	Transmit diversity performance with 4 Tx Antenna Ports	1121

8.10.4	ePDCCH	1121

8.10.4.1	Distributed Transmission with 4Rx	1121

8.10.4.1.1	FDD	1121

8.10.4.1.2	TDD	1122

8.10.4.2	Localized Transmission with TM9 and 4Rx	1123

8.10.4.2.1	FDD	1123

8.10.4.2.2	TDD	1124

8.11	Demodulation (UE supporting coverage enhancement)	1125

8.11.1	PDSCH	1126

8.11.1.1	FDD and half-duplex FDD (Fixed Reference Channel)	1126

8.11.1.1.1	Closed-loop spatial multiplexing performance (Cell-Specific Reference Symbols)	1126

8.11.1.1.2	Closed-loop spatial multiplexing performance (User-Specific Reference Symbols)	1130

8.11.1.1.3	Transmit diversity performance (Cell-Specific Reference Symbols)	1132

8.11.1.2	TDD (Fixed Reference Channel)	1137

8.11.1.2.1	Closed-loop spatial multiplexing performance (Cell-Specific Reference Symbols)	1137

8.11.1.2.2	Closed-loop spatial multiplexing performance (User-Specific Reference Symbols)	1142

8.11.1.2.3	Transmit diversity performance (Cell-Specific Reference Symbols)	1144

8.11.2	MPDCCH	1148

8.11.2.1	FDD and half-duplex FDD	1149

8.11.2.1.1	CE Mode A	1150

8.11.2.1.2	CE Mode B	1151

8.11.2.1.3	CE Mode A with TM9 interference model	1151

8.11.2.1.4	CE Mode A with CRS interference model	1153

8.11.2.1.5	CE Mode A and CE Mode B when CRS-ChEstMPDCCH-Config is configured	1154

8.11.2.2.5	CE Mode A and CE Mode B when CRS-ChEstMPDCCH-Config is configured	1155

8.11.2.2	TDD	1157

8.11.2.2.1	CE Mode A	1158

8.11.2.2.2	CE Mode B	1159

8.11.2.2.3	CE Mode A with TM9 interference model	1159

8.11.2.2.4	CE Mode A with CRS interference model	1160

8.11.3	PBCH	1161

8.11.3.1	FDD and half-duplex FDD	1162

8.11.3.1.1	Transmit diversity performance	1162

8.11.3.2	TDD	1163

8.11.3.2.1	Transmit diversity performance	1163

8.12	Demodulation of Narrowband IoT	1163

8.12.1	NPDSCH	1163

8.12.1.1.1	Minimum Requirements for In-band	1164

8.12.1.1.2	Minimum Requirements for Standalone/Guard-band	1165

8.12.1.1.3	Minimum Requirements for Standalone for UE Category NB2	1166

8.12.1.1.4	Minimum Requirements for Standalone for UE with multiple TBs interleaved transmission	1167

8.12.1.2	TDD	1168

8.12.1.2.1	Minimum Requirements for In-band	1168

8.12.1.2.2	Minimum Requirements for Standalone/Guard-band	1169

8.12.1.2.3	Minimum Requirements for Standalone for UE Category NB2	1170

8.12.2	NPDCCH	1170

8.12.2.1	Half-duplex FDD	1170

8.12.2.1.1	Single-antenna performance	1171

8.12.2.1.2	Transmit diversity performance	1172

8.12.2.2	TDD	1172

8.12.2.2.1	Single-antenna performance	1174

8.12.2.2.2	Transmit diversity performance	1174

8.12.3	Demodulation of NPBCH	1174

8.12.3.1	HD-FDD	1175

8.12.3.1.1	Single-antenna port performance with single NPBCH TTI	1175

8.12.3.1.2	Transmit diversity performance	1175

8.12.3.2	TDD	1175

8.12.3.2.1	Single-antenna port performance with single NPBCH TTI	1176

8.12.3.2.2	Transmit diversity performance	1176

8.13	Demodulation of PDSCH CA and DC(4 receiver antenna ports)	1176

8.13.1	FDD (CA and DC)	1176

8.13.1.1	Closed-loop spatial multiplexing performance	1176

8.13.1.1.1	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port	1176

8.13.1.1.2	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port for dual connectivity	1180

8.13.1.1.3	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port with 256QAM	1182

8.13.1.1.4	Minimum Requirement Four-Layer Spatial Multiplexing 4 Tx Antenna Port	1184

8.13.1.2	Dual-Layer Spatial Multiplexing (User-Specific Reference Symbols)	1185

8.13.1.2.1	Minimum Requirement Dual-Layer Spatial Multiplexing 2 Tx Antenna Port	1185

8.13.1.3	Enhanced Performance Requirements Type A Closed-loop spatial multiplexing	1188

8.13.1.3.1	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model (Cell-Specific Reference Symbols)	1188

8.13.1.4	Enhanced Performance Requirement Type A - Single-layer Spatial Multiplexing (User-Specific Reference Symbols)	1190

8.13.1.4.1	Minimum Requirement Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model (User-Specific Reference Symbols)	1190

8.13.2	TDD (CA and DC)	1193

8.13.2.1	Closed-loop spatial multiplexing performance	1194

8.13.2.1.1	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port	1194

8.13.2.1.2	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port for dual connectivity	1196

8.13.2.1.3	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port with 256QAM	1197

8.13.2.1.4	Minimum Requirement Four-Layer Spatial Multiplexing 4 Tx Antenna Port	1198

8.13.2.2	Dual-Layer Spatial Multiplexing (User-Specific Reference Symbols)	1200

8.13.2.2.1	Minimum Requirement Dual-Layer Spatial Multiplexing 2 Tx Antenna Port	1200

8.13.2.4	Enhanced Performance Requirement Type A - Single-layer Spatial Multiplexing (User-Specific Reference Symbols)	1204

8.13.2.4.1	Minimum Requirement Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model (User-Specific Reference Symbols)	1204

8.13.3	TDD-FDD (CA and DC)	1206

8.13.3.1	 Closed-loop spatial multiplexing performance 4Tx Antenna Port	1206

8.13.3.1.1	Minimum Requirement for FDD PCell	1206

8.13.3.1.2	Minimum Requirement for TDD PCell	1210

8.13.3.2	Dual-Layer Spatial Multiplexing (User-Specific Reference Symbols)	1214

8.13.3.2.1	Minimum Requirement Dual-Layer Spatial Multiplexing 2 Tx Antenna Port for FDD PCell	1214

8.13.3.2.2	Minimum Requirement Dual-Layer Spatial Multiplexing 2 Tx Antenna Port for TDD PCell	1217

8.13.3.3	Enhanced Performance Requirements Type A Closed-loop spatial multiplexing	1220

8.13.3.3.1	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model (Cell-Specific Reference Symbols) for FDD PCell	1220

8.13.3.3.2	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model (Cell-Specific Reference Symbols) for TDD PCell	1222

8.13.3.4	Enhanced Performance Requirement Type A - Single-layer Spatial Multiplexing (User-Specific Reference Symbols)	1224

8.13.3.4.1	Minimum Requirement Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model (User-Specific Reference Symbols) for FDD PCell	1224

8.13.3.4.2	Minimum Requirement Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model (User-Specific Reference Symbols) for TDD PCell	1227

8.13.3.5	 Closed-loop spatial multiplexing performance 4Tx Antenna Port for DC	1230

8.13.3.5.1	Minimum Requirement for FDD PCell	1230

8.13.3.5.2	Minimum Requirement for TDD PCell	1232

8.13.3.6	 Closed-loop spatial multiplexing performance 4Tx Antenna Port with 256QAM	1234

8.13.3.6.1	Minimum Requirement for FDD PCell	1234

8.13.3.6.2	Minimum Requirement for TDD PCell	1236

8.13.3.7	 Closed-loop spatial multiplexing performance 4Tx Antenna Port with Four layers	1237

8.13.3.7.1	Minimum Requirement for FDD PCell	1237

8.13.3.7.2	Minimum Requirement for TDD PCell	1239

8.14	Demodulation (UE supporting Short TTI)	1241

8.14.1	Slot-PDSCH and Subslot-PDSCH	1241

8.14.1.1	FDD (Fixed Reference Channel)	1241

8.14.1.1.1	Open-loop spatial multiplexing performance	1241

8.14.1.1.2	Closed-loop spatial multiplexing performance (User-Specific Reference Signals)	1242

8.14.1.2	TDD (Fixed Reference Channel)	1244

8.14.1.2.1	Open-loop spatial multiplexing performance	1244

8.14.1.2.2	Closed-loop spatial multiplexing performance (User-Specific Reference Signals)	1245

8.14.2	SPDCCH	1246

8.14.2.1	FDD	1246

8.14.2.1.1	Mimimum requirement	1247

8.14.2.2	TDD	1247

8.14.2.2.1	Mimimum requirement	1248

8.15	Demodulation (8 receiver antenna ports)	1249

8.15.1	PDSCH	1249

8.15.1.1	Void	1249

8.15.1.2	TDD (Fixed Reference Channel)	1249

8.15.1.2.1	Transmit diversity performance with 2Tx Antenna Ports (Cell-Specific Reference Symbols)	1249

8.15.1.2.2	Open-loop spatial multiplexing performance with 2Tx Antenna Ports (Cell-Specific Reference Symbols)	1250

8.15.1.2.3	8 Layer Spatial Multiplexing (User-Specific Reference Symbols)	1250

8.15.2	CA	1252

8.15.2.1	Void	1252

8.15.2.2	TDD	1252

8.15.2.2.1	Eight Layer Spatial Multiplexing (User-Specific Reference Symbols)	1252

9	Reporting of Channel State Information	1254

9.1	General	1254

9.1.1	Applicability of requirements	1254

9.1.1.1	Applicability of requirements for different channel bandwidths	1254

9.1.1.2	Applicability and test rules for different CA configurations and bandwidth combination sets	1254

9.1.1.2A	Applicability and test rules for different TDD-FDD CA configurations and bandwidth combination sets	1255

9.1.1.3	Test coverage for different number of componenet carriers	1256

9.1.1.4	Applicability of performance requirements for 4Rx capable UEs	1256

9.1.1.4.1	Applicability rule and antenna connection for single carrier tests with 2Rx	1256

9.1.1.4.2	Applicability rule and antenna connection for CA tests with 2Rx	1258

9.1.1.4.3	Applicability rule and antenna connection for single carrier tests with 4Rx	1259

9.1.1.5	Applicability of requirements for UEs supporting coverage enhancement	1259

9.2	 CQI reporting definition under AWGN conditions	1259

9.2.1	Minimum requirement PUCCH 1-0 (Cell-Specific Reference Symbols)	1259

9.2.1.1	FDD	1259

9.2.1.2	TDD	1260

9.2.1.3	FDD (CSI measurements in case two CSI subframe sets are configured)	1261

9.2.1.4	TDD (CSI measurements in case two CSI subframe sets are configured)	1264

9.2.1.5	FDD (CSI measurements in case two CSI subframe sets are configured and with CRS assistance information)	1267

9.2.1.6	TDD (CSI measurements in case two CSI subframe sets are configured and with CRS assistance information)	1270

9.2.1.7	FDD (Modulation and TBS index Table 2 and 4-bit CQI Table 2 are used)	1273

9.2.1.8	TDD (Modulation and TBS index Table 2 and 4-bit CQI Table 2 are used)	1274

9.2.1.9	FDD (Modulation and TBS index Table 3 and 4-bit CQI Table 4 are used)	1274

9.2.1.10	TDD (Modulation and TBS index Table 3 and 4-bit CQI Table 4 are used)	1275

9.2.2	Minimum requirement PUCCH 1-1 (Cell-Specific Reference Symbols)	1276

9.2.2.1	FDD	1276

9.2.2.2	TDD	1277

9.2.3	Minimum requirement PUCCH 1-1 (CSI Reference Symbols)	1278

9.2.3.1	FDD	1278

9.2.3.1A	FDD (With channelMeasRestriction configured)	1279

9.2.3.2	TDD	1280

9.2.3.2A	TDD (With channelMeasRestriction configured)	1281

9.2.4	Minimum requirement PUCCH 1-1 (With Single CSI Process)	1282

9.2.4.1	FDD	1283

9.2.4.1A	FDD (With interferenceMeasRestriction configured)	1285

9.2.4.2	TDD	1288

9.2.4.2A	TDD (With interferenceMeasRestriction configured)	1291

9.2.5	Minimum requirement PUCCH 1-1 (when csi-SubframeSet –r12 and EIMTA-MainConfigServCell-r12 are configured)	1294

9.2.6	Minimum requirement PUSCH 3-0 (Cell-Specific Reference Symbols)	1297

9.2.6.1	Frame structure type 3 with FDD Pcell	1297

9.2.6.2	Frame structure type 3 with TDD Pcell	1299

9.2.7	Minimum requirement PUSCH 3-1 (CSI Reference Symbol)	1301

9.2.7.1	Frame structure type 3 wth FDD Pcell	1301

9.2.7.2	Frame structure type 3 wth TDD Pcell	1303

9.3	CQI reporting under fading conditions	1307

9.3.1	Frequency-selective scheduling mode	1307

9.3.1.1	Minimum requirement PUSCH 3-0 (Cell-Specific Reference Symbols)	1307

9.3.1.1.1	FDD	1307

9.3.1.1.2	TDD	1308

9.3.1.1.3	FDD (CSI measurements in case two CSI subframe sets are configured and with CRS assistance information)	1309

9.3.1.1.4	TDD (CSI measurements in case two CSI subframe sets are configured and with CRS assistance information)	1313

9.3.1.1.5	TDD (when csi-SubframeSet –r12 is configured)	1316

9.3.1.2	Minimum requirement PUSCH 3-1 (CSI Reference Symbol)	1318

9.3.1.2.1	FDD	1318

9.3.1.2.2	TDD	1319

9.3.1.2.3	FDD (Modulation and TBS index Table 2 and 4-bit CQI Table 2 are used)	1321

9.3.1.2.4	TDD (Modulation and TBS index Table 2 and 4-bit CQI Table 2 are used)	1322

9.3.1.2.5	Void	1323

9.3.1.2.6	TDD (when csi-SubframeSet –r12 is configured with one CSI process)	1323

9.3.2	Frequency non-selective scheduling mode	1327

9.3.2.1	Minimum requirement PUCCH 1-0 (Cell-Specific Reference Symbol)	1327

9.3.2.1.1	FDD	1327

9.3.2.1.2	TDD	1329

9.3.2.2	Minimum requirement PUCCH 1-1 (CSI Reference Symbol)	1331

9.3.2.2.1	FDD	1331

9.3.2.2.2	TDD	1332

9.3.3	Frequency-selective interference	1334

9.3.3.1	Minimum requirement PUSCH 3-0 (Cell-Specific Reference Symbol)	1334

9.3.3.1.1	FDD	1334

9.3.3.1.2	TDD	1335

9.3.3.2	Void	1336

9.3.3.2.1	Void	1336

9.3.3.2.2	Void	1336

9.3.4	UE-selected subband CQI	1336

9.3.4.1	Minimum requirement PUSCH 2-0 (Cell-Specific Reference Symbols)	1336

9.3.4.1.1	FDD	1336

9.3.4.1.2	TDD	1337

9.3.4.2	Minimum requirement PUCCH 2-0 (Cell-Specific Reference Symbols)	1338

9.3.4.2.1	FDD	1338

9.3.4.2.2	TDD	1339

9.3.5	Additional requirements for enhanced receiver Type A	1342

9.3.5.1	Minimum requirement PUCCH 1-0 (Cell-Specific Reference Symbol)	1342

9.3.5.1.1	FDD	1342

9.3.5.1.2	TDD	1343

9.3.5.2	Minimum requirement PUCCH 1-1 (CSI Reference Symbol)	1345

9.3.5.2.1	FDD	1345

9.3.5.2.2	TDD	1348

9.3.6	Minimum requirement (With multiple CSI processes)	1351

9.3.6.1	FDD	1351

9.3.6.2	TDD	1356

9.3.7	Minimum requirement PUSCH 3-2	1360

9.3.7.1	FDD	1360

9.3.7.2	TDD	1361

9.3.8	Additional requirements for enhanced receiver Type B	1363

9.3.8.1	Minimum requirement PUCCH 1-1 (Cell-Specific Reference Symbols)	1363

9.3.8.1.1	FDD	1363

9.3.8.1.2	TDD	1364

9.3.8.2	Minimum requirement PUCCH 1-1 (CSI Reference Symbols)	1366

9.3.8.2.1	FDD	1366

9.3.8.2.2	TDD	1368

9.3.8.3	Minimum requirement with CSI process	1371

9.3.8.3.1	FDD	1371

9.3.8.3.2	TDD	1374

9.4	Reporting of Precoding Matrix Indicator (PMI)	1377

9.4.1	Single PMI	1378

9.4.1.1	Minimum requirement PUSCH 3-1 (Cell-Specific Reference Symbols)	1378

9.4.1.1.1	FDD	1378

9.4.1.1.2	TDD	1379

9.4.1.2	 Minimum requirement PUCCH 2-1 (Cell-Specific Reference Symbols)	1380

9.4.1.2.1	FDD	1380

9.4.1.2.2	TDD	1382

9.4.1.3	Minimum requirement PUSCH 3-1 (CSI Reference Symbol)	1383

9.4.1.3.1	FDD	1383

9.4.1.3.2	TDD	1386

9.4.1.3.3	FDD (with Class A 12Tx codebook)	1389

9.4.1.3.4	TDD (with Class A 12Tx codebook)	1391

9.4.1.3.5	FDD (with Class A 24Tx codebook)	1394

9.4.1.3.6	TDD (with Class A 24Tx codebook)	1396

9.4.1.4	Minimum requirement PUCCH 1-1 (CSI Reference Symbol)	1399

9.4.1.4.1	FDD (with 4Tx enhanced codebook)	1399

9.4.1.4.2	TDD (with 4Tx enhanced codebook)	1401

9.4.1.4.3	FDD (with Class B alternative codebook for one CSI-RS resource configured)	1404

9.4.1.4.4	TDD (with Class B alternative codebook for one CSI-RS resource configured)	1406

9.4.1a	Void	1409

9.4.1a.1	Void	1409

9.4.1a.1.1	Void	1409

9.4.1a.1.2	Void	1409

9.4.2	Multiple PMI	1409

9.4.2.1	Minimum requirement PUSCH 1-2 (Cell-Specific Reference Symbols)	1409

9.4.2.1.1	FDD	1409

9.4.2.1.2	TDD	1410

9.4.2.2	 Minimum requirement PUSCH 2-2 (Cell-Specific Reference Symbols)	1411

9.4.2.2.1	FDD	1411

9.4.2.2.2	TDD	1412

9.4.2.3	Minimum requirement PUSCH 1-2 (CSI Reference Symbol)	1413

9.4.2.3.1	FDD	1413

9.4.2.3.2	TDD	1415

9.4.2.3.3	FDD (with 4Tx enhanced codebook)	1418

9.4.2.3.4	TDD (with 4Tx enhanced codebook)	1420

9.4.2.3.5	FDD (with Class A 16Tx codebook)	1422

9.4.2.3.6	TDD (with Class A 16Tx codebook)	1425

9.4.2.3.7	FDD (with Class A 32Tx codebook)	1428

9.4.2.3.8	TDD (with Class A 32Tx codebook)	1431

9.4.2.3.9	FDD (with Class A 16Tx advanced codebook)	1434

9.4.2.3.10	TDD (with Class A 16Tx advanced codebook)	1437

9.4.3	Void	1440

9.5	Reporting of Rank Indicator (RI)	1440

9.5.1	Minimum requirement (Cell-Specific Reference Symbols)	1440

9.5.1.1	FDD	1440

9.5.1.2	TDD	1441

9.5.2	Minimum requirement (CSI Reference Symbols)	1442

9.5.2.1	FDD	1442

9.5.2.2	TDD	1444

9.5.3	Minimum requirement (CSI measurements in case two CSI subframe sets are configured)	1446

9.5.3.1	FDD	1446

9.5.3.2	TDD	1449

9.5.4	Minimum requirement (CSI measurements in case two CSI subframe sets are configured and CRS assistance information are configured)	1452

9.5.4.1	FDD	1452

9.5.4.2	TDD	1455

9.5.5	Minimum requirement (with CSI process)	1458

9.5.5.1	FDD	1459

9.5.5.2	TDD	1462

9.6	 Additional requirements for carrier aggregation	1465

9.6.1	 Periodic reporting on multiple cells (Cell-Specific Reference Symbols)	1465

9.6.1.1	FDD	1465

9.6.1.2	TDD	1471

9.6.1.3	TDD-FDD CA with FDD PCell	1477

9.6.1.4	TDD-FDD CA with TDD PCell	1482

9.7	CSI reporting (Single receiver antenna)	1488

9.7.1	CQI reporting definition under AWGN conditions	1489

9.7.1.1	FDD and half-duplex FDD	1489

9.7.1.2	TDD	1489

9.7.1.3 FDD (Category 1bis UE)	1490

9.7.1.4 TDD (Category 1bis UE)	1491

9.7.2	CQI reporting under fading conditions	1492

9.7.2.1	FDD and half-duplex FDD	1492

9.7.2.2	TDD	1493

9.7.2.3 FDD (Category 1bis UE)	1494

9.7.2.4 TDD (Category 1bis UE)	1495

9.8	CSI reporting (UE supporting coverage enhancement)	1496

9.8.1	CQI reporting definition under AWGN conditions	1496

9.8.1.1	FDD and half-duplex FDD	1496

9.8.1.2	TDD	1497

9.8.2	UE-selected subband CQI	1498

9.8.2.1	FDD and half-duplex FDD	1498

9.8.2.2	TDD	1502

9.8.3	CQI reporting definition for UE supporting 64QAM under AWGN	1505

9.8.3.1	FDD and half-duplex FDD	1505

9.8.3.2	TDD	1506

9.8.4	CQI reporting definition for UE supporting alternative table under AWGN	1507

9.8.4.1	FDD and half-duplex FDD	1507

9.8.4.2	TDD	1508

9.8.5	PMI reporting with PUCCH 1-1 (CSI Reference Symbol)	1509

9.8.5.1	FDD	1510

9.8.5.2	TDD	1511

9.9	CSI reporting for 4Rx UE	1512

9.9.1	CQI reporting definition under AWGN conditions	1512

9.9.1.1	Minimum requirement PUCCH 1-0 with Rank 1 (Cell-Specific Reference Symbols)	1513

9.9.1.1.1	FDD	1513

9.9.1.1.2	TDD	1513

9.9.1.2	Minimum requirement PUCCH 1-1 with Rank 2 (CSI Reference Symbols)	1514

9.9.1.2.1	FDD	1514

9.9.1.2.2	TDD	1515

9.9.1.3	 Minimum requirement PUCCH 1-1 with Rank 4 (Cell-Specific Reference Symbols)	1516

9.9.1.3.1	FDD	1517

9.9.1.3.2	TDD	1517

9.9.1.4	Minimum requirement PUCCH 1-1 with Rank 3 (CSI Reference Symbols)	1518

9.9.1.4.1	FDD	1518

9.9.1.4.2	TDD	1519

9.9.2	CQI reporting definition under fading conditions	1520

9.9.2.1	Minimum requirement PUCCH 1-0 (Cell-Specific Reference Symbol) for enhanced receiver Type A	1520

9.9.2.1.1	FDD	1521

9.9.2.1.2	TDD	1522

9.9.2.2	Minimum requirement PUCCH 1-1 (CSI Reference Symbol) for enhanced receiver Type A	1524

9.9.2.2.1	FDD	1524

9.9.2.2.2	TDD	1527

9.9.3	Reporting of Precoding Matrix Indicator (PMI) for 4Rx UE	1530

9.9.3.1	Minimum requirement PUSCH 3-1 (CSI Reference Symbol)	1531

9.9.3.1.1	TDD	1531

9.9.4	 Reporting of Rank Indicator (RI)	1533

9.9.4.1	Minimum requirement (Cell-Specific Reference Symbols)	1533

9.9.4.1.1	FDD	1533

9.9.4.1.2	TDD	1535

9.9.4.2	Minimum requirement (CSI Reference Symbols)	1536

9.9.4.2.1	FDD	1536

9.9.4.2.2	TDD	1538

9.10	Reporting of CSI-RS Resource Indicator (CRI)	1540

9.10.1	Minimum requirement (PUSCH 3-1)	1541

9.10.1.1	FDD	1541

9.10.1.2	TDD	1543

9.10.2	Minimum requirement (PUSCH 3-1, QCL Type C)	1545

9.10.2.1	FDD	1545

9.10.2.2	TDD	1548

9.11	Reporting of Hybrid Channel state information	1551

9.11.1	Minimum requirement (with eMIMO-Type configured as Class B with more than one CSI-RS resource configured and eMIMO-Type2 as Class B with one CSI-RS resource configured)	1551

9.11.1.1	FDD	1552

9.11.1.2	TDD	1554

9.12	CSI reporting (UE supporting Short TTI)	1556

9.12.1	CQI reporting under fading conditions (Cell-Specific Reference Symbol)	1556

9.12.1.1	FDD	1556

9.12.1.2	TDD	1559

9.12.2	CQI reporting under fading conditions (CSI Reference Symbol)	1560

9.12.2.1	FDD	1560

9.12.2.2	TDD	1563

9.13	CSI reporting for 8Rx UE	1565

9.13.1	CQI reporting definition under AWGN conditions	1565

9.13.1.1	Minimum requirement PUCCH 1-1 with Rank 4 (CSI Reference Symbols)	1565

9.13.1.2.1	Void	1565

9.13.1.2.2	TDD	1565

10	Performance requirement (MBMS)	1566

10.1	FDD (Fixed Reference Channel)	1566

10.1.1	Minimum requirement	1567

10.2	TDD (Fixed Reference Channel)	1568

10.2.1	Minimum requirement	1568

10.3	FDD (Fixed Reference Channel) with FeMBMS	1569

10.3.1	Minimum requirement for FeMBMS Unicast-mixed Cell under CA	1569

10.3.1.1	Minimum requirement with 1.25kHz subcarrier spacing	1569

10.3.1.2	Minimum requirement with 7.5kHz subcarrier spacing	1570

10.3.2	Minimum requirement for FeMBMS Unicast-mixed Cell as Non-Serving Cell	1571

10.3.2.1	Minimum requirement with 1.25kHz subcarrier spacing	1571

10.3.2.2	Minimum requirement with 7.5kHz subcarrier spacing	1572

10.3.3	Minimum requirement for MBMS Dedicated cell	1573

10.3.3.1	Minimum requirement with 1.25kHz subcarrier spacing	1573

10.3.3.2	Minimum requirement with 7.5kHz subcarrier spacing	1574

10.3.3.3	Minimum requirement with 15kHz subcarrier spacing	1575

10.4 FDD with 5G terrestrial broadcast	1576

10.4.1	Minimum requirement for PMCH decoding	1576

10.4.1.1	Minimum requirement with 0.37kHz subcarrier spacing	1576

10.4.1.2	Minimum requirement with 2.5kHz subcarrier spacing	1577

10.4.2	Minimum requirement for CAS detection	1578

10.4.2.1	Minimum requirement for PBCH detection	1578

11	Performance requirement (ProSe Direct Discovery)	1578

11.1	General	1579

11.1.1	Applicability of requirements	1579

11.1.2	Reference DRX configuration	1579

11.2	Demodulation of PSDCH (single link performance)	1579

11.2.1	FDD (in-coverage)	1579

11.2.2	TDD (in-coverage)	1580

11.2.3	FDD (out-of-coverage)	1581

11.3	Power imbalance performance with two links	1582

11.3.1	FDD	1582

11.3.2	TDD	1583

11.4	Multiple timing reference test	1584

11.4.1	FDD	1585

11.5	Maximum Sidelink processes test	1586

11.5.1	FDD	1586

11.5.2	TDD	1587

12	Performance requirement (ProSe Direct Communication)	1589

12.1	General	1589

12.1.1	Applicability of requirements	1589

12.1.1.1	Applicability of requirements for different channel bandwidths	1589

12.1.1.2	Test coverage for different number of component carriers	1589

12.1.1.3	Applicability and test rules for different CA configurations and bandwidth combination sets	1589

12.1.2	Reference DRX configuration	1590

12.2	Demodulation of PSSCH	1590

12.2.1	FDD	1590

12.3	Demodulation of PSCCH	1591

12.3.1	FDD	1592

12.4	Demodulation of PSBCH	1593

12.4.1	FDD	1593

12.5	Power imbalance performance with two links	1593

12.5.1	 FDD	1593

12.6	Multiple timing reference test	1595

12.6.1	FDD	1595

12.7	Maximum Sidelink processes test	1598

12.7.1	FDD	1598

12.8	Sustained downlink data rate with active Sidelink	1599

13	Void	1601

14	Performance requirement (V2X Sidelink Communication)	1601

14.1	General	1601

14.1.1	Applicability of requirements	1601

14.2	Demodulation of PSSCH	1602

14.3	Demodulation of PSCCH	1603

14.4	Power imbalance performance with two links	1603

14.5	Demodulation of PSBCH	1604

14.6	Demodulation of PSSCH with eNB based synchronization	1605

14.7	Soft buffer test	1605

14.8	PSCCH decoding capability test	1606

14.9	Sustained downlink data rate with active sidelink	1607

14.10	Soft buffer test (CA)	1608

14.11	PSCCH/PSSCH decoding capability test (CA)	1609

Annex A (normative):  Measurement channels	1614

A.1	General	1614

A.2	UL reference measurement channels	1614

A.2.1	General	1614

A.2.1.1	Applicability and common parameters	1614

A.2.1.2	Determination of payload size	1614

A.2.1.3	Overview of UL reference measurement channels	1615

A.2.2	Reference measurement channels for FDD	1638

A.2.2.1	Full RB allocation	1638

A.2.2.1.1	QPSK	1638

A.2.2.1.2	16-QAM	1640

A.2.2.1.3	64-QAM	1642

A.2.2.1.4	256 QAM	1643

A.2.2.2	Partial RB allocation	1643

A.2.2.2.1	QPSK	1644

A.2.2.2.2	16-QAM	1647

A.2.2.2.3	64-QAM	1649

A.2.2.2.4	256 QAM	1651

A.2.2.3	Void	1651

A.2.2.4	subPRB allocation	1652

A.2.3	Reference measurement channels for TDD	1652

A.2.3.1	Full RB allocation	1652

A.2.3.1.1	QPSK	1652

A.2.3.1.2	16-QAM	1655

A.2.3.1.3	64-QAM	1658

A.2.3.1.4	256 QAM	1658

A.2.3.2	Partial RB allocation	1658

A.2.3.2.1	QPSK	1659

A.2.3.2.2	16-QAM	1663

A.2.3.2.3	64-QAM	1668

A.2.3.2.4	256 QAM	1670

A.2.3.3	Void	1670

A.2.3.4	subPRB allocation	1671

A.2.4	Reference measurement channels for UE category NB1	1671

A.2.5	Reference measurement channels for LAA	1672

A.2.5.1	Full RB allocation	1672

A.2.5.1.1	QPSK	1672

A.2.5.1.2	16QAM	1673

A.2.5.1.3	64QAM	1673

A.2.5.2	Partial RB allocation	1673

A.2.5.2.1	QPSK	1674

A.2.5.2.2	16QAM	1674

A.2.5.2.3	64QAM	1675

A.3	DL reference measurement channels	1675

A.3.1	General	1675

A.3.1.1	Overview of DL reference measurement channels	1676

A.3.2	Reference measurement channel for receiver characteristics	1701

A.3.3	Reference measurement channels for PDSCH performance requirements (FDD)	1731

A.3.3.1	Single-antenna transmission (Common Reference Symbols)	1731

A.3.3.2	Multi-antenna transmission (Common Reference Symbols)	1736

A.3.3.2.1	Two antenna ports	1736

A.3.3.2.2	Four antenna ports	1744

A.3.3.3	Reference Measurement Channel for UE-Specific Reference Symbols	1749

A.3.3.3.0	Two antenna ports (no CSI-RS)	1749

A.3.3.3.1	Two antenna port (CSI-RS)	1750

A.3.3.3.2	Four antenna ports (CSI-RS)	1753

A.3.3.3.2A	Eight antenna ports (CSI-RS)	1760

A.3.3.3.3	Twelve antenna port (CSI-RS)	1762

A.3.3.3.4	Sixteen antenna port (CSI-RS)	1763

A.3.3.3.5	Twenty-four antenna port (CSI-RS)	1764

A.3.3.3.6	Thirty-two antenna port (CSI-RS)	1765

A.3.4	Reference measurement channels for PDSCH performance requirements (TDD)	1767

A.3.4.1	Single-antenna transmission (Common Reference Symbols)	1767

A.3.4.2	Multi-antenna transmission (Common Reference Signals)	1775

A.3.4.2.1	Two antenna ports	1775

A.3.4.2.2	Four antenna ports	1788

A.3.4.3	Reference Measurement Channels for UE-Specific Reference Symbols	1795

A.3.4.3.1	Single antenna port (Cell Specific)	1795

A.3.4.3.2	Two antenna ports (Cell Specific)	1796

A.3.4.3.3	Two antenna ports (CSI-RS)	1798

A.3.4.3.4	Four antenna ports (CSI-RS)	1806

A.3.4.3.5	Eight antenna ports (CSI-RS)	1813

A.3.4.3.6	Twelve antenna ports (CSI-RS)	1819

A.3.4.3.7	Sixteen antenna ports (CSI-RS)	1820

A.3.4.3.8	Twenty-four antenna ports (CSI-RS)	1821

A.3.4.3.9	Thirty-two antenna ports (CSI-RS)	1822

A.3.5	Reference measurement channels for PDCCH/PCFICH performance requirements	1824

A.3.5.1	FDD	1824

A.3.5.2	TDD	1824

A.3.5.3	LAA	1825

A.3.6	Reference measurement channels for PHICH performance requirements	1825

A.3.7	Reference measurement channels for PBCH performance requirements	1825

A.3.8	Reference measurement channels for MBMS performance requirements	1826

A.3.8.1	FDD	1826

A.3.8.2	TDD	1831

A.3.9	Reference measurement channels for sustained downlink data rate provided by lower layers	1833

A.3.9.1	FDD	1833

A.3.9.2	TDD	1837

A.3.9.3	FDD (EPDCCH scheduling)	1851

A.3.9.4	TDD (EPDCCH scheduling)	1852

A.3.9.5	LAA	1854

A.3.10	Reference Measurement Channels for EPDCCH performance requirements	1856

A.3.10.1	FDD	1856

A.3.10.2	TDD	1856

A.3.11	Reference Measurement Channels for MPDCCH performance requirements	1856

A.3.11.1	FDD and half-duplex FDD	1856

A.3.11.2	TDD	1857

A.3.12	Reference measurement channels for NPDSCH performance requirements	1858

A.3.12.1	In-band	1858

A.3.12.1.2	Two-antenna transmission	1858

A.3.12.2	Standalone/Guard-band	1860

A.3.12.2.1 Single-antenna transmission	1860

A.3.13	Reference measurement channels for NPDCCH performance requirements	1863

A.3.13.1	Half-duplex FDD	1863

A.3.13.2	TDD	1864

A.3.14	Reference measurement channels for NPBCH performance requirements for Cat NB1 UEs	1864

A.3.15	Reference Measurement Channels for LAA SCell with frame structure Type-3	1865

A.3.15.1	Multi-antenna transmission (Common Reference Symbols)	1865

A.3.15.1.1	Four antenna ports	1865

A.3.15.2	Reference Measurement Channel for UE-Specific Reference Symbols	1865

A.3.15.2.1	Two antenna ports (CSI-RS)	1865

A.3.16	Reference measurement channels for Slot-PDSCH and Subslot-PDSCH performance requirements	1867

A.3.16.1	FDD	1867

A.3.16.2	TDD	1871

A.3.17	Reference measurement channels for SPDCCH performance requirements	1872

A.3.17.1	FDD	1872

A.3.17.2	TDD	1873

A.4	CSI reference measurement channels	1873

A.5	OFDMA Channel Noise Generator (OCNG)	1888

A.5.1	OCNG Patterns for FDD	1888

A.5.1.1	OCNG FDD pattern 1: One sided dynamic OCNG FDD pattern	1888

A.5.1.2	OCNG FDD pattern 2: Two sided dynamic OCNG FDD pattern	1889

A.5.1.3	OCNG FDD pattern 3: 49 RB OCNG allocation with MBSFN in 10 MHz	1889

A.5.1.3A	OCNG FDD pattern 3A: 49 RB OCNG allocation with MBSFN enhancement in 10 MHz	1890

A.5.1.4	OCNG FDD pattern 4: One sided dynamic OCNG FDD pattern for MBMS transmission	1890

A.5.1.4A	OCNG FDD pattern 4A: One sided dynamic OCNG FDD pattern for enhanced MBMS transmission	1891

A.5.1.5	OCNG FDD pattern 5: One sided dynamic 16QAM modulated OCNG FDD pattern	1892

A.5.1.6	OCNG FDD pattern 6: dynamic OCNG FDD pattern when user data is in 2 non-contiguous blocks	1893

A.5.1.8	OCNG FDD pattern 8: Dynamic OCNG FDD pattern for TM10 transmission	1894

A.5.2	OCNG Patterns for TDD	1895

A.5.2.1	OCNG TDD pattern 1: One sided dynamic OCNG TDD pattern	1895

A.5.2.2	OCNG TDD pattern 2: Two sided dynamic OCNG TDD pattern	1896

A.5.2.3	OCNG TDD pattern 3: 49 RB OCNG allocation with MBSFN in 10 MHz	1897

A.5.2.4	OCNG TDD pattern 4: One sided dynamic OCNG TDD pattern for MBMS transmission	1897

A.5.2.5	OCNG TDD pattern 5: One sided dynamic 16QAM modulated OCNG TDD pattern	1898

A.5.2.6	OCNG TDD pattern 6: dynamic OCNG TDD pattern when user data is in 2 non-contiguous blocks	1899

A.5.2.8	OCNG TDD pattern 8: Dynamic OCNG TDD pattern for TM10 transmission	1900

A.5.3	OCNG Patterns for Narrowband IoT	1901

A.5.3.1	Narrowband IoT OCNG pattern 1	1901

A.5.4	OCNG Patterns for frame structure type 3	1902

A.5.4.1	OCNG FS3 pattern 1: One sided dynamic OCNG frame structure type 3 pattern	1902

A.5.4.2	OCNG FS3 pattern 2: Two sided dynamic OCNG frame structure 3 pattern	1903

A.6	Sidelink reference measurement channels	1904

A.6.1	General	1904

A.6.1.1	Overview of ProSe reference measurement channels	1904

A.6.2	Reference measurement channel for receiver characteristics	1905

A.6.3	Reference measurement channels for PSDCH performance requirements	1907

A.6.4	Reference measurement channels for PSCCH performance requirements	1908

A.6.5	Reference measurement channels for PSSCH performance requirements	1908

A.6.6	Reference measurement channels for PSBCH performance requirements	1909

A.7	Sidelink reference resource pool configurations	1910

A.7.1	Reference resource pool configurations for ProSe Direct Discovery demodulation tests	1910

A.7.1.1	FDD	1910

A.7.1.2	TDD	1913

A.7.2	Reference resource pool configurations for ProSe Direct Communication demodulation tests	1915

A.7.2.1	FDD	1915

A.8	V2X reference measurement channels	1920

A.8.1	General	1920

A.8.1.1	Overview of V2X reference measurement channels	1921

A.8.2	Reference measurement channel for receiver characteristics	1921

A.8.3	Reference measurement channel for transmitter characteristics	1923

A.8.4	Reference measurement for PSCCH performance requirements	1926

A.8.5	Reference measurement for PSSCH performance requirements	1927

A.8.6	Reference measurement for PSBCH performance requirements	1927

A.9	V2X reference resource pool configurations	1928

Annex B (normative):  Propagation conditions	1931

B.1	Static propagation condition	1931

B.1.1	UE Receiver with 2Rx	1931

B.1.2	UE Receiver with 4Rx	1931

B.1.3	UE Receiver with 8Rx	1932

B.2	Multi-path fading propagation conditions	1933

B.2.1	Delay profiles	1933

B.2.2	Combinations of channel model parameters	1934

B.2.3	MIMO Channel Correlation Matrices	1935

B.2.3.1	Definition of MIMO Correlation Matrices	1935

B.2.3.2	MIMO Correlation Matrices at High, Medium and Low Level	1941

B.2.3A	MIMO Channel Correlation Matrices using cross polarized antennas	1944

B.2.3A.1	Definition of MIMO Correlation Matrices using cross polarized antennas	1945

B.2.3A.2	Spatial Correlation Matrices using cross polarized antennas at eNB and UE sides	1945

B.2.3A.2.1	Spatial Correlation Matrices at eNB side	1945

B.2.3A.2.2	Spatial Correlation Matrices at UE side	1946

B.2.3A.4	Beam steering approach	1949

B.2.3B	MIMO Channel Correlation Matrices using two-dimension cross polarized antennas at eNB and cross polarized antennas at UE	1949

B.2.3B.1	Definition of MIMO Correlation Matrices using two-dimension cross polarized antennas at eNB and cross polarized antennas at UE	1950

B.2.3B.2	Spatial Correlation Matrices using two-dimension cross polarized antennas at eNB and cross polarized antennas at UE	1951

B.2.3B.2.1	Spatial Correlation Matrices at eNB side	1951

B.2.3B.2.2	Spatial Correlation Matrices at UE side	1951

B.2.3B.3	MIMO Correlation Matrices using two-dimension cross polarized antennas at eNB and cross polarized antennas at UE	1951

B.2.3B.4	Beam steering approach	1954

B.2.3B.4A	Beam steering approach with dual cluster beams	1955

B.2.4	Propagation conditions for CQI tests	1956

B.2.4.1	Propagation conditions for CQI tests with multiple CSI processes	1956

B.2.5	Void	1956

B.2.6	MBSFN Propagation Channel Profile	1956

B.2.6.1	Subcarrier spacing 15kHz or 7.5kHz	1956

B.2.6.2	Subcarrier spacing 1.25kHz	1957

B.2.6.3	Subcarrier spacing 0.37kHz	1958

B.2.6.4	Subcarrier spacing 2.5kHz	1958

B.3	High speed train scenario	1959

B.3A	HST-SFN scenario	1960

B.3B	HST-SFN scenario for 500 km/h speed	1964

B.3C	HST scenario for 500 km/h speed	1964

B.4	Beamforming Model	1964

B.4.1	Single-layer random beamforming (Antenna port 5, 7, or 8)	1964

B.4.1A	Single-layer random beamforming (Antenna port 7, 8, 11 or 13 with enhanced DMRS table configured)	1965

B.4.2	Dual-layer random beamforming (antenna ports 7 and 8)	1965

B.4.3	Generic beamforming model (antenna ports 7-14)	1966

B.4.4	Random beamforming for EPDCCH distributed transmission (Antenna port 107 and 109)	1966

B.4.5	Random beamforming for EPDCCH localized transmission (Antenna port 107, 108, 109 or 110)	1967

B.4.6	Beamforming model for CRI test	1967

B.5	Interference models for enhanced performance requirements Type-A	1968

B.5.1	Dominant interferer proportion	1968

B.5.2	Transmission mode 3 interference model	1969

B.5.3	Transmission mode 4 interference model	1969

B.5.4	Transmission mode 9 interference model	1970

B.6	Interference models for enhanced performance requirements Type-B	1970

B.6.1	Transmission mode 2 interference model	1970

B.6.2	Transmission mode 3 interference model	1970

B.6.3	Transmission mode 4 interference model	1971

B.6.4	Transmission mode 9 interference model	1971

B.6.5	CRS interference model	1972

B.6.6	Random interference model	1972

B.7	Interference models for enhanced downlink control channel performance requirements Type A and B	1973

B.7.1	PDCCH, PCFICH and PHICH interference model	1973

B.8	Burst transmission models for Frame structure type 3	1974

B.8.1	Burst transmission model for one LAA SCell	1974

B.8.2	Burst transmission model for multiple LAA SCell(s)	1974

Annex C (normative):  Downlink Physical Channels	1976

C.1	General	1976

C.2	Set-up	1976

C.3	Connection	1976

C.3.1	Measurement of Receiver Characteristics	1976

C.3.2	Measurement of Performance requirements	1977

C.3.3	Aggressor cell power allocation for Measurement of Performance Requirements when ABS is Configured	1978

C.3.4	Power Allocation for Measurement of Performance Requirements when Quasi Co-location Type B: same Cell ID	1979

C.3.5	Simplified CA testing method	1979

C.3.6	Measurement of Receiver Characteristics for Narrowband IoT	1980

Annex D (normative):  Characteristics of the interfering signal	1981

D.1	General	1981

D.2	Interference signals	1981

Annex E (normative):  Environmental conditions	1982

E.1	 General	1982

E.2	 Environmental	1982

E.2.1	Temperature	1982

E.2.2	Voltage	1982

E.2.3	Vibration	1983

Annex F (normative):  Transmit modulation	1984

F.1	Measurement Point	1984

F.2	Basic Error Vector Magnitude measurement	1984

F.3	Basic in-band emissions measurement	1985

F.4	Modified signal under test	1985

F.5	Window length	1987

F.5.1	Timing offset	1987

F.5.2	Window length	1987

F.5.3	Window length for normal CP	1987

F.5.4	Window length for Extended CP	1988

F.5.5	Window length for PRACH	1988

F.5.F	Window length for category NB1	1989

F.6	Averaged EVM	1989

F.6.F	Averaged EVM for category NB1	1990

F.7	Spectrum Flatness	1990

Annex G (informative):  Reference sensitivity level in lower SNR	1991

G.1	General	1991

G.2	Typical receiver sensitivity performance (QPSK)	1991

G.3	Reference measurement channel for REFSENSE in lower SNR	1997

Annex H (normative):  Modified MPR behavior	2000

H.1	Indication of modified MPR behavior	2000

Annex I (normative):  Supported Post Antenna Gain	2001

I.1	Declared Supported Post Antenna Gain for UE	2001

Annex J (informative):  Change history	2002