# Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

In the present document, modal verbs have the following meanings:

shall		indicates a mandatory requirement to do something

shall not	indicates an interdiction (prohibition) to do something

The constructions "shall" and "shall not" are confined to the context of normative provisions, and do not appear in Technical Reports.

The constructions "must" and "must not" are not used as substitutes for "shall" and "shall not". Their use is avoided insofar as possible, and they are not used in a normative context except in a direct citation from an external, referenced, non-3GPP document, or so as to maintain continuity of style when extending or modifying the provisions of such a referenced document.

should		indicates a recommendation to do something

should not	indicates a recommendation not to do something

may		indicates permission to do something

need not	indicates permission not to do something

The construction "may not" is ambiguous and is not used in normative elements. The unambiguous constructions "might not" or "shall not" are used instead, depending upon the meaning intended.

can		indicates that something is possible

cannot		indicates that something is impossible

The constructions "can" and "cannot" are not substitutes for "may" and "need not".

will		indicates that something is certain or expected to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

will not		indicates that something is certain or expected not to happen as a result of action taken by an agency the behaviour of which is outside the scope of the present document

might	indicates a likelihood that something will happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

might not	indicates a likelihood that something will not happen as a result of action taken by some agency the behaviour of which is outside the scope of the present document

In addition:

is	(or any other verb in the indicative mood) indicates a statement of fact

is not	(or any other negative verb in the indicative mood) indicates a statement of fact

The constructions "is" and "is not" do not indicate requirements.

# 1	Scope

The present document establishes the minimum RF characteristics and minimum performance requirements for E-UTRA User Equipment (UE).

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

- References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

- For a specific reference, subsequent revisions do not apply.

- For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	ITU-R Recommendation SM.329-10, "Unwanted emissions in the spurious domain"

[3]	ITU-R Recommendation M.1545: "Measurement uncertainty as it applies to test limits for the terrestrial component of International Mobile Telecommunications-2000".

[4]	3GPP TS 36.211: "Physical Channels and Modulation".

[5]	3GPP TS 36.212: "Multiplexing and channel coding".

[6]	3GPP TS 36.213: "Physical layer procedures".

[7]	3GPP TS 36.331: " Requirements for support of radio resource management ".

[8]	3GPP TS 36.307: " Requirements on User Equipments (UEs) supporting a release-independent frequency band".

[9]	3GPP TS 36.423: "X2 application protocol (X2AP) ".

[10]	3GPP TS 23.303: "Technical Specification Group Services and System Aspects; Proximity-based services (ProSe); Stage 2".

[11]	3GPP TS36.300: "Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall description; Stage 2".

[12]	3GPP TS36.104: "Base Station (BS) radio transmission and reception".

[13]	ETSI TS 102 792: "Intelligent Transport Systems (ITS); Mitigation techniques to avoid interference between European CEN Dedicated Short Range Communication (CEN DSRC) equipment and Intelligent Transport Systems (ITS) operating in the 5 GHz frequency range".

# 3	Definitions, symbols and abbreviations

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply in the case of a single component carrier. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Aggregated Channel Bandwidth: The RF bandwidth in which a UE transmits and receives multiple contiguously aggregated carriers.

Aggregated Transmission Bandwidth Configuration: The number of resource block allocated within the aggregated channel bandwidth.

Carrier aggregation: Aggregation of two or more component carriers in order to support wider transmission bandwidths.

Carrier aggregation band: A set of one or more operating bands across which multiple carriers are aggregated with a specific set of technical requirements.

Carrier aggregation bandwidth class: A class defined by the aggregated transmission bandwidth configuration and maximum number of component carriers supported by a UE.

Carrier aggregation configuration: A combination of CA operating band(s) and CA bandwidth class(es) supported by a UE.

Channel edge: The lowest and highest frequency of the carrier, separated by the channel bandwidth.

Channel bandwidth: The RF bandwidth supporting a single E-UTRA RF carrier with the transmission bandwidth configured in the uplink or downlink of a cell. The channel bandwidth is measured in MHz and is used as a reference for transmitter and receiver RF requirements.

Composite spectrum emission mask: Emission mask requirement for intraband non-contiguous carrier aggregation which is a combination of individual sub-block spectrum emissions masks.

Composite spurious emission requirement: Spurious emission requirement for intraband non-contiguous carrier aggregation which is a combination of individual sub-block spurious emission requirements.

Contiguous carriers: A set of two or more carriers configured in a spectrum block where there are no RF requirements based on co-existence for un-coordinated operation within the spectrum block.

Contiguous resource allocation: A resource allocation of consecutive resource blocks within one carrier or across contiguously aggregated carriers. The gap between contiguously aggregated carriers due to the nominal channel spacing is allowed.

Contiguous spectrum: Spectrum consisting of a contiguous block of spectrum with no sub-block gaps.

Enhanced downlink control channel performance requirements type A: This defines performance requirements for downlink control channel assuming as baseline receiver reference symbol based linear minimum mean square error interference rejection combining plus CRS interference cancellation.

Enhanced downlink control channel performance requirements type B: This defines performance requirements for downlink control channel assuming as baseline receiver reference symbol based enhanced linear minimum mean square error interference rejection combining plus CRS interference cancellation.

Enhanced performance requirements type A: This defines performance requirements assuming as baseline receiver reference symbol based linear minimum mean square error interference rejection combining.

Enhanced performance requirements type B: This defines performance requirements assuming as baseline receiver using network assisted interference cancelation and suppression.

Enhanced performance requirements type C: This defines performance requirements assuming as baseline receiver      inter-stream interference cancellation.

Inter-band carrier aggregation: Carrier aggregation of component carriers in different operating bands.

NOTE:	Carriers aggregated in each band can be contiguous or non-contiguous.

Intra-band contiguous carrier aggregation: Contiguous carriers aggregated in the same operating band.

Intra-band non-contiguous carrier aggregation: Non-contiguous carriers aggregated in the same operating band.

Lower sub-block edge: The frequency at the lower edge of one sub-block. It is used as a frequency reference point for both transmitter and receiver requirements.

Category NB1/NB2 stand-alone operation: category NB1/NB2 is operating standalone when it utilizes its own spectrum, for example the spectrum used by GERAN systems as a replacement of one or more GSM carriers, as well as scattered spectrum for potential IoT deployment.

Category NB1/NB2 guard band operation: category NB1/NB2 is operating in guard band when it utilizes the unused resource block(s) within a E-UTRA carrier’s guard-band.

Category NB1/NB2 in-band operation: category NB1/NB2 is operating in-band when it utilizes the resource block(s) within a normal E-UTRA carrier or within a normal NR carrier plus 15 kHz at each edge (and not within NR minimum guard band).

Non-contiguous spectrum: Spectrum consisting of two or more sub-blocks separated by sub-block gap(s).

ProSe-enabled UE: A UE that supports ProSe requirements and associated procedures.

NOTE: As defined in TS 23.303 [10].

ProSe Direct Communication: A communication between two or more UEs in proximity that are ProSe-enabled.

NOTE: As defined in TS 23.303 [10].

ProSe Direct Discovery: A procedure employed by a ProSe-enabled UE to discover other ProSe-enabled UEs in its vicinity.

NOTE: As defined in TS 23.303 [10].

sTTI : A transmission time interval (TTI) of either one slot or one subslot as defined in TS 36.211 [4] on either uplink or downlink.

Sub-block: This is one contiguous allocated block of spectrum for transmission and reception by the same UE. There may be multiple instances of sub-blocks within an RF bandwidth.

Sub-block bandwidth: The bandwidth of one sub-block.

Sub-block gap: A frequency gap between two consecutive sub-blocks within an RF bandwidth, where the RF requirements in the gap are based on co-existence for un-coordinated operation.

Synchronized operation: Operation of TDD in two different systems, where no simultaneous uplink and downlink occur.

Unsynchronized operation: Operation of TDD in two different systems, where the conditions for synchronized operation are not met.

Upper sub-block edge: The frequency at the upper edge of one sub-block. It is used as a frequency reference point for both transmitter and receiver requirements.

V2X Communication: V2X (Vehicle to Everything) service is operating in ITS spectrum and/or LTE licensed operating bands.

## 3.2	Symbols

For the purposes of the present document, the following symbols apply:

BWChannel	Channel bandwidth

BWChannel,block 	Sub-block bandwidth, expressed in MHz. BWChannel,block= Fedge,block,high- Fedge,block,low.

BWChannel_CA 	Aggregated channel bandwidth, expressed in MHz.

BWGB	Virtual guard band to facilitate transmitter (receiver) filtering above / below edge CCs.

Transmitted energy per RE for reference symbols during the useful part of the symbol, i.e. excluding the cyclic prefix, (average power normalized to the subcarrier spacing) at the eNode B transmit antenna connector

The averaged received energy per RE of the wanted signal during the useful part of the symbol, i.e. excluding the cyclic prefix, at the UE antenna connector; average power is computed within a set of REs used for the transmission of physical channels (including user specific RSs when present), divided by the number of REs within the set, and normalized to the subcarrier spacing

F					Frequency

Fagg_alloc_low	Aggregated Transmission Bandwidth Configuration. The lowest frequency of the simultaneously transmitted resource blocks.

Fagg_alloc_high	Aggregated Transmission Bandwidth Configuration. The highest frequency of the simultaneously transmitted resource blocks.

FInterferer (offset)	Frequency offset of the interferer (between the center frequency of the interferer and the carrier frequency of the carrier measured)

FInterferer			Frequency of the interferer

FIoffset				Frequency offset of the interferer (between the center frequency of the interferer and the closest edge of the carrier measured)

FC	Frequency of the carrier centre frequency

FC_agg	Aggregated Transmission Bandwidth Configuration.  Center frequency of the aggregated carriers.

FC,block, high		Center frequency of the highest transmitted/received carrier in a sub-block.

FC,block, low		Center frequency of the lowest transmitted/received carrier in a sub-block.

FC_low 	The centre frequency of the lowest carrier, expressed in MHz.

FC_high 	The centre frequency of the highest carrier, expressed in MHz.

FDL_low	The lowest frequency of the downlink operating band

FDL_high	The highest frequency of the downlink operating band

FUL_low	The lowest frequency of the uplink operating band

FUL_high	The highest frequency of the uplink operating band

Fedge,block,low 	The lower sub-block edge, where Fedge,block,low = FC,block,low - Foffset.

Fedge,block,high 	The upper sub-block edge, where Fedge,block,high = FC,block,high + Foffset.

Fedge_low 	The lower edge of aggregated channel bandwidth, expressed in MHz.

Fedge_high 	The higher edge of aggregated channel bandwidth, expressed in MHz.

Foffset 	Frequency offset from FC_high to the higher edge or FC_low to the lower edge.

Foffset,block,low	Separation between lower edge of a sub-block and the center of the lowest component carrier within the sub-block

Foffset,block,high	Separation between higher edge of a sub-block and the center of the highest component carrier within the sub-block

Foffset_NS_23	Frequency offset in MHz needed if NS_23 is used

FOOB	The boundary between the E-UTRA out of band emission and spurious emission domains.

The power spectral density of the total input signal (power averaged over the useful part of the symbols within the transmission bandwidth configuration, divided by the total number of RE for this configuration and normalised to the subcarrier spacing) at the UE antenna connector, including the own-cell downlink signal

The total transmitted power spectral density of the own-cell downlink signal (power averaged over the useful part of the symbols within the transmission bandwidth configuration, divided by the total number of RE for this configuration and normalised to the subcarrier spacing) at the eNode B transmit antenna connector

The total received power spectral density of the own-cell downlink signal (power averaged over the useful part of the symbols within the transmission bandwidth configuration, divided by the total number of RE for this configuration and normalised to the subcarrier spacing) at the UE antenna connector

The received power spectral density of the total noise and interference for a certain RE (average power obtained within the RE and normalized to the subcarrier spacing) as measured at the UE antenna connector

LCRB	Transmission bandwidth which represents the length of a contiguous resource block allocation expressed in units of resources blocks

LCtone	Transmission bandwidth which represents the length of a contiguous sub-carrier allocation expressed in units of tones

Ncp	Cyclic prefix length

NDL 	Downlink EARFCN

The power spectral density of a white noise source (average power per RE normalised to the subcarrier spacing), simulating interference from cells that are not defined in a test procedure, as measured at the UE antenna connector

The power spectral density of a white noise source (average power per RE normalized to the subcarrier spacing), simulating interference in non-CRS symbols in ABS subframe from cells that are not defined in a test procedure, as measured at the UE antenna connector.

The power spectral density of a white noise source (average power per RE normalized to the subcarrier spacing), simulating interference in CRS symbols in ABS subframe from all cells that are not defined in a test procedure, as measured at the UE antenna connector.

The power spectral density of a white noise source (average power per RE normalised to the subcarrier spacing), simulating interference in non-ABS subframe from cells that are not defined in a test procedure, as measured at the UE antenna connector

The power spectral density (average power per RE normalised to the subcarrier spacing) of the summation of the received power spectral densities of the strongest interfering cells explicitly defined in a test procedure plus , as measured at the UE antenna connector. The respective power spectral density of each interfering cell relative to  is defined by its associated DIP value, or the respective power spectral density of each interfering cell relative to  is defined by its associated Es/Noc value.

NOffs-DL 	Offset used for calculating downlink EARFCN

NOffs-UL 	Offset used for calculating uplink EARFCN

The power spectral density of a white noise source (average power per RE normalised to the subcarrier spacing) simulating eNode B transmitter impairments as measured at the eNode B transmit antenna connector

NRB	Transmission bandwidth configuration, expressed in units of resource blocks

NRB_agg	The number of the aggregated RBs within the fully allocated Aggregated Channel bandwidth.

NRB_alloc	Total number of simultaneously transmitted resource blocks in Channel bandwidth or Aggregated Channel Bandwidth.

NRB,c	The transmission bandwidth configuration of component carrier c, expressed in units of resource blocks

NRB,largest BW	The largest transmission bandwidth configuration of the component carriers in the bandwidth combination, expressed in units of resource blocks

NRX	Number of receiver antennas

Ntone	Transmission bandwidth configuration for category NB1 and NB2, expressed in units of tones.

Ntone 3.75kHz	Transmission bandwidth configuration for category NB1 and NB2 with 3.75 kHz sub-carrier spacing, expressed in units of tones.

Ntone 15kHz 	Transmission bandwidth configuration for category NB1 and NB2 with 15 kHz sub-carrier spacing, expressed in units of tones.

NUL	Uplink EARFCN.

Rav		Minimum average throughput per RB.

PCMAX	The configured maximum UE output power.

PCMAX, c	The configured maximum UE output power for serving cell c.

PEMAX 	Maximum allowed UE output power signalled by higher layers. Same as IE P-Max, defined in [7].

PEMAX, c	Maximum allowed UE output power signalled by higher layers for serving cell c. Same as IE
P-Max, defined in [7].

PInterferer	Modulated mean power of the interferer

PPowerClass	PPowerClass is the nominal UE power (i.e., no tolerance).

PPowerClass_Default	PPowerClass_Default is the default nominal UE power (i.e., no tolerance) for the band.

PUMAX	The measured configured maximum UE output power.

Puw	Power of an unwanted DL signal

Pw	Power of a wanted DL signal

RBstart 	Indicates the lowest RB index of transmitted resource blocks.

RBend	Indicates the highest RB index of transmitted resource blocks.

Tno_hopping	Transmission period within a TTI duration when consecutive symbols are transmitted without applying any frequency hopping

ΔfOOB	Δ Frequency of Out Of Band emission.

ΔPPowerClass 	Adjustment to maximum output power for a given power class.

ΔRIB,c	Allowed reference sensitivity relaxation due to support for inter-band CA operation, for serving cell c.

ΔRIB,4R	Reference sensitivity adjustment due to support for 4 antenna ports.

ΔRIB,8R	Reference sensitivity adjustment due to support for 8 antenna ports.

ΔTIB,c	Allowed maximum configured output power relaxation due to support for inter-band CA operation, for serving cell c.

TC	Allowed operating band edge transmission power relaxation.

TC,c 	Allowed operating band edge transmission power relaxation for serving cell c.

TProSe	Allowed operating band transmission power relaxation due to support of E-UTRA ProSe on an operating band.

According to Clause 5.2 in TS 36.213 [6]

According to Clause 5.2 in TS 36.213 [6]

Test specific auxiliary variable used for the purpose of downlink power allocation, defined in Annex C.3.2.

Wgap	Sub-block gap size

Wgap_L	Sub-block gap size between lowest two CCs in frequency domain on CA_X-X-X

Wgap_H	Sub-block gap size between highest two CCs in frequency domain on CA_X-X-X

## 3.3	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

ABS	Almost Blank Subframe

ACLR	Adjacent Channel Leakage Ratio

ACS	Adjacent Channel Selectivity

A-MPR	Additional Maximum Power Reduction

AWGN	Additive White Gaussian Noise

BS	Base Station

CA	Carrier Aggregation

CA_X	Intra-band contiguous CA of component carriers in one sub-block within Band X where X is the applicable E-UTRA operating band

CA_X-X	Intra-band non-contiguous CA of component carriers in two sub-blocks within Band X where X is the applicable E-UTRA operating band

CA_X-X-X	Intra-band non-contiguous CA of component carriers in three sub-blocks within Band X where X is the applicable E-UTRA operating band

CA_X-X-X-X	Intra-band non-contiguous CA of component carriers in four sub-blocks within Band X where X is the applicable E-UTRA operating band

CA_X-Y	Inter-band CA of component carrier(s) in one sub-block within Band X and component carrier(s) in one sub-block within Band Y where X and Y are the applicable E-UTRA operating band

CA_X-X-Y	CA of component carriers in two sub-blocks within Band X and component carrier(s) in one sub-block within Band Y where X and Y are the applicable E-UTRA operating bands

CC	Component Carriers

CG	Carrier Group

CPE	Customer Premise Equipment

CPE_X	Customer Premise Equipment for E-UTRA operating band X

CW	Continuous Wave

DC	Dual Connectivity

DC_X-Y	Inter-band DC of component carrier(s) in one sub-block within Band X and component carrier(s) in one sub-block within Band Y where X and Y are the applicable E-UTRA operating band

DL	Downlink

DIP	Dominant Interferer Proportion

EARFCN 	E-UTRA Absolute Radio Frequency Channel Number

EIRP	Effective Isotropic Radiated Power

EPRE	Energy Per Resource Element

E-UTRA 	Evolved UMTS Terrestrial Radio Access

EUTRAN	Evolved UMTS Terrestrial Radio Access Network

EVM	Error Vector Magnitude

FDD	Frequency Division Duplex

FRC	Fixed Reference Channel

GNSS	Global Navigation Satellite Systems

HD	Half-Duplex for Sidelink Operation

HD-FDD	Half- Duplex FDD

ITS	Intelligent Transportation Systems

MCS	Modulation and Coding Scheme

MCG	Master Cell Group

MOP	Maximum Output Power

MPR	Maximum Power Reduction

MSD	Maximum Sensitivity Degradation

OCNG	OFDMA Channel Noise Generator

OFDMA	Orthogonal Frequency Division Multiple Access

OOB	Out-of-band

PA	Power Amplifier

PCC	Primary Component Carrier

P-MPR	Power Management Maximum Power Reduction

ProSe	Proximity-based Services

PSBCH	Physical Sidelink Broadcast CHannel

PSCCH	Physical Sidelink Control CHannel

PSDCH	Physical Sidelink Discovery CHannel

PSS	Primary Synchronization Signal

PSS_RA	PSS-to-RS EPRE ratio for the channel PSS

SSSS	Secondary Sidelink Synchronization Signal

PSSCH	Physical Sidelink Shared CHannel

PSSS	Primary Sidelink Synchronization Signal

RE	Resource Element

REFSENS	Reference Sensitivity power level

r.m.s	Root Mean Square

SCC	Secondary Component Carrier

SCG	Secondary Cell Group

SINR	Signal-to-Interference-and-Noise Ratio

SNR	Signal-to-Noise Ratio

SSS	Secondary Synchronization Signal

SSS_RA	SSS-to-RS EPRE ratio for the channel SSSSSSS	Secondary Sidelink Synchronization Signal

TDD	Time Division Duplex

UE	User Equipment

UL	Uplink

UL-MIMO	Up Link Multiple Antenna transmission

UMTS	Universal Mobile Telecommunications System

UTRA	UMTS Terrestrial Radio Access

UTRAN	UMTS Terrestrial Radio Access Network

V2X	Vehicle to Everything

xCH_RA	xCH-to-RS EPRE ratio for the channel xCH in all transmitted OFDM symbols not containing cell-specific RS

xCH_RB	xCH-to-RS EPRE ratio for the channel xCH in all transmitted OFDM symbols containing cell-specific RS

# 4	General

## 4.1	Relationship between minimum requirements and test requirements

The Minimum Requirements given in this specification make no allowance for measurement uncertainty. The test specification TS 36.521-1 Annex F defines Test Tolerances. These Test Tolerances are individually calculated for each test. The Test Tolerances are used to relax the Minimum Requirements in this specification to create Test Requirements.

The measurement results returned by the Test System are compared - without any modification - against the Test Requirements as defined by the shared risk principle.

The Shared Risk principle is defined in ITU-R M.1545 [3].

## 4.2	Applicability of minimum requirements

a)	In this specification the Minimum Requirements are specified as general requirements and additional requirements. Where the Requirement is specified as a general requirement, the requirement is mandated to be met in all scenarios

b)	For specific scenarios for which an additional requirement is specified, in addition to meeting the general requirement, the UE is mandated to meet the additional requirements.

c)	The reference sensitivity power levels defined in subclause 7.3 are valid for the specified reference measurement channels.

d)	NOTE: Receiver sensitivity degradation may occur when:

1)	The UE simultaneously transmits and receives with bandwidth allocations less than the transmission bandwidth configuration (see Figure 5.6-1), and

2)	Any part of the downlink transmission bandwidth is within an uplink transmission bandwidth from the downlink center subcarrier.

e)	The spurious emissions power requirements are for the long term average of the power. For the purpose of reducing measurement uncertainty it is acceptable to average the measured power over a period of time sufficient to reduce the uncertainty due to the statistical nature of the signal.

f)	The requirements in this specification for TDD operating bands apply for downlink and uplink operations using Frame Structure Type 2 [4] except for Band 46 operating with Frame Structure Type 3.

g)	The requirements related to subslot TTI and/or slot TTI shall apply only if UE supports multiple TTI patterns. And these requirements only apply to subslot and/or slot TTI configurations

## 4.3	Void

## 4.3A	Applicability of minimum requirements (CA, UL-MIMO, ProSe, Dual Connectivity, UE category 0, UE category M1, UE category M2, UE category 1bis, UE category NB1 and NB2, V2X Communication, MBMS UE)

The requirements in clauses 5, 6 and 7 which are specific to CA, UL-MIMO, ProSe, Dual Connectivity,   UE category 0, UE category M1, UE category M2, UE category 1bis, UE category NB1 and NB2 and V2X Communication are specified as suffix A, B, C, D, E, F and G where;

a)	Suffix A additional requirements need to support CA

b)	Suffix B additional requirements need to support UL-MIMO

c)	Suffix C additional requirements need to support Dual Connectivity

d)	Suffix D additional requirements need to support ProSe

e)	Suffix E additional requirements need to support UE category 0, category M1, category M2, and category 1bis

f)	Suffix F additional requirements need to support UE category NB1 and NB2

g)	Suffix G additional requirements need to support V2X Communication

A terminal which supports the above features needs to meet both the general requirements and the additional requirement applicable to the additional subclause (suffix A, B, C, D, E, F and G) in clauses 5, 6 and 7. Where there is a difference in requirement between the general requirements and the additional subclause requirements (suffix A, B, C, D, E, F and G) in clauses 5, 6 and 7, the tighter requirements are applicable unless stated otherwise in the additional subclause.

A terminal which supports more than one feature (CA, UL-MIMO, ProSe, Dual Connectivity, UE category 0, UE category M1, UE category M2, UE category 1bis, UE category NB1 and NB2 and V2X Communication) in clauses 5, 6 and 7 shall meet all of the separate corresponding requirements.

For a terminal supporting CA, compliance with minimum requirements for non-contiguous intra-band carrier aggregation in any given operating band does not imply compliance with minimum requirements for contiguous intra-band carrier aggregation in the same operating band.

For a terminal supporting CA, compliance with minimum requirements for contiguous intra-band carrier aggregation in any given operating band does not imply compliance with minimum requirements for non- contiguous intra-band carrier aggregation in the same operating band.

A terminal which supports a DL CA configuration shall support all the lower order fallback DL CA combinations and it shall support at least one bandwidth combination set for each of the constituent lower order DL combinations containing all the bandwidths specified within each specific combination set of the upper order DL combination.

A terminal which supports CA, for each supported CA configuration, shall support Pcell transmissions in each of the aggregated Component Carriers unless indicated otherwise in clause 5.6A.1.

Terminal supporting Dual Connectivity configuration shall meet the minimum requirements for corresponding CA configuration (suffix A), unless otherwise specified.

For a terminal that supports ProSe Direct Communication and/or ProSe Direct Discovery, the minimum requirements are applicable when

-	the UE is associated with a serving cell on the ProSe carrier, or

-	the UE is not associated with a serving cell on the ProSe carrier and is provisioned with the preconfigured radio parameters for ProSe Direct Communications and/or ProSe Direct Discovery that are associated with known Geographical Area, or

-	the UE is associated with a serving cell on a carrier different than the ProSe carrier, and the radio parameters for ProSe Direct Discovery on the ProSe carrier are provided by the serving cell, or

-	the UE is associated with a serving cell on a carrier different than the ProSe carrier, and has a non-serving cell selected on the ProSe carrier that supports ProSe Direct Discovery and/or ProSe Direct Communication.

When the ProSe UE is not associated with a serving cell on the ProSe carrier, and the UE does not have knowledge of its geographical area, or is provisioned with preconfigured radio parameters that are not associated with any Geographical Area, ProSe transmissions are not allowed, and the requirements in Section 6.3.3D apply.

A terminal that supports simultaneous E-UTRA ProSe sidelink transmissions and E-UTRA uplink transmissions for the inter-band E-UTRA ProSe/E-UTRA bands specified in Table 5.5D-2, shall meet the minimum requirements for the corresponding inter-band UL CA configuration (suffix A), unless otherwise specified. For transmitter characteristics specified in clause 6, the terminal is required to meet the conformance tests for the corresponding inter-band UL CA configuration and is not required to be retested with simultaneous E-UTRA ProSe sidelink and E-UTRA uplink transmissions.

A terminal that supports E-UTRA V2X intra-band multi-carrier operation including carrier aggregation for the band specified in Table 5.5G-3, shall meet the corresponding transmitter characteristics requirements (in subclauses with suffix G in Section 6) only when there are multiple active transmissions on all of the configured carrier components. When there is only one active transmission on one of the configured carrier components, the corresponding requirements for V2X single carrier operation apply for the corresponding active carrier component.

A terminal which supports MBMS (including 15 kHz, 7.5 kHz ,1.25 kHz, 2.5 kHz and 0.37 kHz subcarrier spacing), shall meet the minimum requirements in clauses 5 and 7. A terminal which supports MBMS is not required to support all kinds of subcarrier spacing.

A terminal that supports multiple TTI patterns in different carriers, different TTI patterns can only be used when the carriers are aggregated in inter-band manner. For intra-band carrier aggregation, only same TTI patterns and same TAG are allowed in aggregated carriers.

## 4.4	RF requirements in later releases

The standardisation of new frequency bands and carrier aggregation configurations (downlink and uplink aggregation) may be independent of a release. However, in order to implement a UE that conforms to a particular release but supports a band of operation or a carrier aggregation configuration that is specified in a later release, it is necessary to specify some extra requirements. TS 36.307 [8] specifies requirements on UEs supporting a frequency band or a carrier aggregation configuration that is independent of release.

NOTE:	For UEs conforming to the 3GPP release of the present document, some RF requirements of later releases may be mandatory independent of whether the UE supports the bands specif or carrier aggregation configurations ied in later releases or not. The set of RF requirements of later releases that is also mandatory for UEs conforming to the 3GPP release of the present document is determined by regional regulation.

# 5	Operating bands and channel arrangement

## 5.1	General

The channel arrangements presented in this clause are based on the operating bands and channel bandwidths defined in the present release of specifications.

NOTE:	Other operating bands and channel bandwidths may be considered in future releases.

## 5.2	Void

## 5.3	Void

## 5.4	Void

## 5.5	Operating bands

E-UTRA is designed to operate in the operating bands defined in Table 5.5-1.

Table 5.5-1 E-UTRA operating bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
| 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
| 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
| 4 | 1710 MHz | – | 1755 MHz | 2110 MHz | – | 2155 MHz | FDD |
| 5 | 824 MHz | – | 849 MHz | 869 MHz | – | 894MHz | FDD |
| 61 | 830 MHz | – | 840 MHz | 875 MHz | – | 885 MHz | FDD |
| 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
| 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
| 9 | 1749.9 MHz | – | 1784.9 MHz | 1844.9 MHz | – | 1879.9 MHz | FDD |
| 10 | 1710 MHz | – | 1770 MHz | 2110 MHz | – | 2170 MHz | FDD |
| 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
| 12 | 699 MHz | – | 716 MHz | 729 MHz | – | 746 MHz | FDD |
| 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
| 14 | 788 MHz | – | 798 MHz | 758 MHz | – | 768 MHz | FDD |
| 15 | Reserved |  |  | Reserved |  |  | FDD |
| 16 | Reserved |  |  | Reserved |  |  | FDD |
| 17 | 704 MHz | – | 716 MHz | 734 MHz | – | 746 MHz | FDD |
| 18 | 815 MHz | – | 830 MHz | 860 MHz | – | 875 MHz | FDD |
| 19 | 830 MHz | – | 845 MHz | 875 MHz | – | 890 MHz | FDD |
| 20 | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
| 21 | 1447.9 MHz | – | 1462.9 MHz | 1495.9 MHz | – | 1510.9 MHz | FDD |
| 22 | 3410 MHz | – | 3490 MHz | 3510 MHz | – | 3590 MHz | FDD |
| 231 | 2000 MHz | – | 2020 MHz | 2180 MHz | – | 2200 MHz | FDD |
| 2417 | 1626.5 MHz | – | 1660.5 MHz | 1525 MHz | – | 1559 MHz | FDD |
| 25 | 1850 MHz | – | 1915 MHz | 1930 MHz | – | 1995 MHz | FDD |
| 26 | 814 MHz | – | 849 MHz | 859 MHz | – | 894 MHz | FDD |
| 27 | 807 MHz | – | 824 MHz | 852 MHz | – | 869 MHz | FDD |
| 28 | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
| 29 | N/A |  |  | 717 MHz | – | 728 MHz | FDD2 |
| 3015 | 2305 MHz | – | 2315 MHz | 2350 MHz | – | 2360 MHz | FDD |
| 31 | 452.5 MHz | – | 457.5 MHz | 462.5 MHz | – | 467.5 MHz | FDD |
| 32 |  | N/A |  | 1452 MHz | – | 1496 MHz | FDD2 |
| 33 | 1900 MHz | – | 1920 MHz | 1900 MHz | – | 1920 MHz | TDD |
| 34 | 2010 MHz | – | 2025 MHz | 2010 MHz | – | 2025 MHz | TDD |
| 35 | 1850 MHz | – | 1910 MHz | 1850 MHz | – | 1910 MHz | TDD |
| 36 | 1930 MHz | – | 1990 MHz | 1930 MHz | – | 1990 MHz | TDD |
| 37 | 1910 MHz | – | 1930 MHz | 1910 MHz | – | 1930 MHz | TDD |
| 38 | 2570 MHz | – | 2620 MHz | 2570 MHz | – | 2620 MHz | TDD |
| 39 | 1880 MHz | – | 1920 MHz | 1880 MHz | – | 1920 MHz | TDD |
| 40 | 2300 MHz | – | 2400 MHz | 2300 MHz | – | 2400 MHz | TDD |
| 41 | 2496 MHz |  | 2690 MHz | 2496 MHz |  | 2690 MHz | TDD |
| 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |
| 43 | 3600 MHz | – | 3800 MHz | 3600 MHz | – | 3800 MHz | TDD |
| 44 | 703 MHz | – | 803 MHz | 703 MHz | – | 803 MHz | TDD |
| 45 | 1447 MHz | – | 1467 MHz | 1447 MHz | – | 1467 MHz | TDD |
| 46 | 5150 MHz | – | 5925 MHz | 5150 MHz | – | 5925 MHz | TDD8 |
| 47 | 5855 MHz | – | 5925 MHz | 5855 MHz | – | 5925 MHz | TDD11 |
| 48 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD |
| 49 | 3550 MHz | – | 3700 MHz | 3550 MHz | – | 3700 MHz | TDD16 |
| 50 | 1432 MHz | - | 1517 MHz | 1432 MHz | - | 1517 MHz | TDD13 |
| 51 | 1427 MHz | - | 1432 MHz | 1427 MHz | - | 1432 MHz | TDD13 |
| 52 | 3300 MHz | - | 3400 MHz | 3300 MHz | - | 3400 MHz | TDD |
| 53 | 2483.5 MHz | - | 2495 MHz | 2483.5 MHz | - | 2495 MHz | TDD |
| … |  |  |  |  |  |  |  |
| 64 | Reserved |  |  |  |  |  |  |
| 65 | 1920 MHz | – | 2010 MHz | 2110 MHz | – | 2200 MHz | FDD |
| 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD4 |
| 67 |  | N/A |  | 738 MHz | – | 758 MHz | FDD2 |
| 68 | 698 MHz | – | 728 MHz | 753 MHz | – | 783 MHz | FDD |
| 69 | N/A |  |  | 2570 MHz | – | 2620 MHz | FDD2 |
| 70 | 1695 MHz | – | 1710 MHz | 1995 MHz | – | 2020 MHz | FDD10 |
| 71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
| 72 | 451 MHz | – | 456 MHz | 461 MHz | – | 466 MHz | FDD |
| 73 | 450 MHz | – | 455 MHz | 460 MHz | – | 465 MHz | FDD |
| 74 | 1427 MHz | – | 1470 MHz | 1475 MHz | – | 1518 MHz | FDD |
| 75 |  | N/A |  | 1432 MHz | – | 1517 MHz | FDD2 |
| 76 |  | N/A |  | 1427 MHz | – | 1432 MHz | FDD2 |
| 85 | 698 MHz | – | 716 MHz | 728 MHz | – | 746 MHz | FDD |
| 87 | 410 MHz | – | 415 MHz | 420 MHz | – | 425 MHz | FDD |
| 88 | 412 MHz | – | 417 MHz | 422 MHz | – | 427 MHz | FDD |
| NOTE 1: Band 6, 23 is not applicable NOTE 2: Restricted to E-UTRA operation when carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell. NOTE 3: A UE that complies with the E-UTRA Band 65 minimum requirements in this specification shall also comply with the E-UTRA Band 1 minimum requirements. NOTE 4: The range 2180-2200 MHz of the DL operating band is restricted to E-UTRA operation when carrier aggregation is configured. NOTE 5: A UE that supports E-UTRA Band 66 shall receive in the entire DL operating band NOTE 6: A UE that supports E-UTRA Band 66 and CA operation in any CA band shall also comply with the minimum requirements specified for the DL CA configurations CA_66B, CA_66C and CA_66A-66A. NOTE 7: A UE that complies with the E-UTRA Band 66 minimum requirements in this specification shall also comply with the E-UTRA Band 4 minimum requirements. NOTE 8: This band is an unlicensed band restricted to licensed-assisted operation using Frame Structure Type 3 NOTE 9: In this version of the specification, restricted to E-UTRA DL operation when carrier aggregation is configured. NOTE 10: The range 2010-2020 MHz of the DL operating band is restricted to E-UTRA operation when carrier aggregation is configured and TX-RX separation is 300 MHz The range 2005-2020 MHz of the DL operating band is restricted to E-UTRA operation when carrier aggregation is configured and TX-RX separation is 295 MHz. NOTE 11: This band is unlicensed band used for V2X communication. There is no expected network deployment in this band so Frame Structure Type 1 is used. NOTE 12: A UE that complies with the E-UTRA Band 74 minimum requirements in this specification shall also comply with the E-UTRA Band 11 and Band 21 minimum requirements. NOTE 13: UE that complies with the E-UTRA Band 50 minimum requirements in this specification shall also comply with the E-UTRA Band 51 minimum requirements. NOTE 14: A UE that complies with the E-UTRA Band 75 minimum requirements in this specification shall also comply with the E-UTRA Band 76 minimum requirements. NOTE 15: Uplink transmission is not allowed at this band for UE with external vehicle-mounted antennas. NOTE 16: This band is restricted to licensed-assisted operation using Frame Structure Type 3 NOTE 17: DL operation in this band is restricted to 1526 – 1536 MHz and UL operation is restricted to 1627.5 – 1637.5 MHz and 1646.5 – 1656.5 MHz. |  |  |  |  |  |  |  |

## 5.5A	Operating bands for CA

E-UTRA carrier aggregation is designed to operate in the operating bands defined in Tables 5.5A-1, 5.5A-2, 5.5A-2a, 5.5A-2b, 5.5A-2c and 5.5A-3.

Table 5.5A-1: Intra-band contiguous CA operating bands

| E-UTRA CA Band | E-UTRA Band (Table 5.5.1) |
| --- | --- |
| CA_1 | 1 |
| CA_2 | 2 |
| CA_3 | 3 |
| CA_5 | 5 |
| CA_7 | 7 |
| CA_8 | 8 |
| CA_12 | 12 |
| CA_23 | 23 |
| CA_27 | 27 |
| CA_28 | 28 |
| CA_38 | 38 |
| CA_39 | 39 |
| CA_40 | 40 |
| CA_41 | 41 |
| CA_42 | 42 |
| CA_43 | 43 |
| CA_48 | 48 |
| CA_66 | 66 |
| CA_70 | 70 |

Table 5.5A-2: Inter-band CA operating bands (two bands)

| E-UTRA CA Band | E-UTRA Band (Table 5.5.1) |  |
| --- | --- | --- |
| CA_1-3 | 1, 3 |  |
| CA_1-1-3 | 1,3 |  |
| CA_1-1-5 | 1,5 |  |
| CA_1-1-7 | 1,7 |  |
| CA_1-1-28 | 1,28 |  |
| CA_1-3-3 | 1, 3 |  |
| CA_1-5 | 1, 5 |  |
| CA_1-7 | 1, 7 |  |
| CA_1-7-7 | 1, 7 |  |
| CA_1-8 | 1, 8 |  |
| CA_1-11 | 1, 11 |  |
| CA_1-18 | 1, 18 |  |
| CA_1-19 | 1, 19 |  |
| CA_1-20 | 1, 20 |  |
| CA_1-21 | 1, 21 |  |
| CA_1-26 | 1, 26 |  |
| CA_1-28 | 1, 28 |  |
| CA_1-32 | 1, 32 |  |
| CA_1-38 | 1, 38 |  |
| CA_1-40 | 1, 40 |  |
| CA_1-41 | 1, 41 |  |
| CA_1-42 | 1, 42 |  |
| CA_1-42-42 | 1, 42 |  |
| CA_1-43 | 1, 43 |  |
| CA_1-46 | 1, 46 |  |
| CA_2-4 | 2, 4 |  |
| CA_2-2-4 | 2, 4 |  |
| CA_2-2-4-4 | 2, 4 |  |
| CA_2-4-4 | 2, 4 |  |
| CA_2-5 | 2, 5 |  |
| CA_2-2-5 | 2, 5 |  |
| CA_2-2-7 | 2, 7 |  |
| CA_2-7 | 2, 7 |  |
| CA_2-7-7 | 2, 7 |  |
| CA_2-12 | 2, 12 |  |
| CA_2-2-12 | 2, 12 |  |
| CA_2-2-12-12 | 2, 12 |  |
| CA_2-2-29 | 2, 29 |  |
| CA_2-12-12 | 2, 12 |  |
| CA_2-13 | 2, 13 |  |
| CA_2-2-13 | 2, 13 |  |
| CA_2-14 | 2, 14 |  |
| CA_2-2-14 | 2, 14 |  |
| CA_2-17 | 2, 17 |  |
| CA_2-26 | 2, 26 |  |
| CA_2-28 | 2, 28 |  |
| CA_2-29 | 2, 29 |  |
| CA_2-30 | 2, 30 |  |
| CA_2-2-30 | 2, 30 |  |
| CA_2-46 | 2, 46 |  |
| CA_2-2-46 | 2, 46 |  |
| CA_2-46-46 | 2, 46 |  |
| CA_2-48-48 | 2, 48 |  |
| CA_2-49 | 2, 49 |  |
| CA_2-66 | 2, 66 |  |
| CA_2-48 | 2, 48 |  |
| CA_2-2-66 | 2, 66 |  |
| CA_2-2-66-66 | 2, 66 |  |
| CA_2-66-66 | 2, 66 |  |
| CA_2-66-66-66 | 2, 66 |  |
| CA_2-71 | 2, 71 |  |
| CA_2-2-71 | 2, 71 |  |
| CA_3-5 | 3, 5 |  |
| CA_3-3-5 | 3, 3, 5 |  |
| CA_3-7 | 3, 7 |  |
| CA_3-3-7 | 3, 7 |  |
| CA_3-3-7-7 | 3, 7 |  |
| CA_3-7-7 | 3, 7 |  |
| CA_3-8 | 3, 8 |  |
| CA_3-3-8 | 3, 8 |  |
| CA_3-11 | 3, 11 |  |
| CA_3-18 | 3, 18 |  |
| CA_3-19 | 3, 19 |  |
| CA_3-3-19 | 3, 19 |  |
| CA_3-20 | 3, 20 |  |
| CA_3-3-20 | 3, 20 |  |
| CA_3-3-21 | 3, 21 |  |
| CA_3-3-28 | 3, 28 |  |
| CA_3-3-41 | 3, 41 |  |
| CA_3-3-42 | 3, 42 |  |
| CA_3-21 | 3, 21 |  |
| CA_3-26 | 3, 26 |  |
| CA_3-27 | 3, 27 |  |
| CA_3-28 | 3, 28 |  |
| CA_3-31 | 3, 31 |  |
| CA_3-32 | 3, 32 |  |
| CA_3-38 | 3, 38 |  |
| CA_3-40 | 3, 40 |  |
| CA_3-40-40 | 3, 40 |  |
| CA_3-41 | 3, 41 |  |
| CA_3-42 | 3, 42 |  |
| CA_3-42-42 | 3, 42 |  |
| CA_3-43 | 3, 43 |  |
| CA_3-46 | 3, 46 |  |
| CA_3-3-46 | 3, 46 |  |
| CA_3-69 | 3, 69 |  |
| CA_4-5 | 4, 5 |  |
| CA_4-4-5 | 4, 5 |  |
| CA_4-7 | 4, 7 |  |
| CA_4-4-7 | 4, 7 |  |
| CA_4-7-7 | 4, 7 |  |
| CA_4-12 | 4, 12 |  |
| CA_4-4-12 | 4, 12 |  |
| CA_4-4-12-12 | 4, 12 |  |
| CA_4-12-12 | 4, 12 |  |
| CA_4-13 | 4, 13 |  |
| CA_4-4-13 | 4, 13 |  |
| CA_4-17 | 4, 17 |  |
| CA_4-27 | 4, 27 |  |
| CA_4-28 | 4, 28 |  |
| CA_4-29 | 4, 29 |  |
| CA_4-4-29 | 4, 29 |  |
| CA_4-30 | 4, 30 |  |
| CA_4-4-30 | 4, 30 |  |
| CA_4-46 | 4, 46 |  |
| CA_4-46-46 | 4, 46 |  |
| CA_4-48 | 4, 48 |  |
| CA_4-71 | 4,71 |  |
| CA_4-4-71 | 4, 71 |  |
| CA_5-5-40 | 5, 40 |  |
| CA_5-7 | 5, 7 |  |
| CA_5-7-7 | 5, 7 |  |
| CA_5-12 | 5, 12 |  |
| CA_5-12-12 | 5, 12 |  |
| CA_5-13 | 5 ,13 |  |
| CA_5-17 | 5, 17 |  |
| CA_5-25 | 5, 25 |  |
| CA_5-28 | 5, 28 |  |
| CA_5-29 | 5, 29 |  |
| CA_5-30 | 5, 30 |  |
| CA_5-38 | 5, 38 |  |
| CA_5-40 | 5, 40 |  |
| CA_5-40-40 | 5, 40 |  |
| CA_5-41 | 5, 41 |  |
| CA_5-46 | 5, 46 |  |
| CA_5-48 | 5, 48 |  |
| CA_5-66 | 5, 66 |  |
| CA_5-5-66 | 5, 66 |  |
| CA_5-66-66 | 5, 66 |  |
| CA_5-5-66-66 | 5, 66 |  |
| CA_7-8 | 7, 8 |  |
| CA_7-7-8 | 7, 8 |  |
| CA_7-12 | 7, 12 |  |
| CA_7-20 | 7, 20 |  |
| CA_7-22 | 7, 22 |  |
| CA_7-26 | 7, 26 |  |
| CA_7-7-26 | 7, 26 |  |
| CA_7-28 | 7, 28 |  |
| CA_7-7-28 | 7, 28 |
| CA_7-29 | 7,29 |
| CA_7-7-29 | 7,29 |
| CA_7-30 | 7, 30 |  |
| CA_7-32 | 7, 32 |  |
| CA_7-40 | 7, 40 |  |
| CA_7-42 | 7, 42 |  |
| CA_7-42-42 | 7, 42 |  |
| CA_7-46 | 7, 46 |  |
| CA_7-7-46 | 7, 46 |  |
| CA_7-7-66 | 7, 66 |  |
| CA_7-66 | 7, 66 |  |
| CA_7-66-66 | 7, 66 |  |
| CA_7-7-66-66 | 7, 66 |  |
| CA_8-11 | 8, 11 |  |
| CA_8-20 | 8, 20 |  |
| CA_8-27 | 8, 27 |  |
| CA_8-28 | 8, 28 |  |
| CA_8-32 | 8, 32 |  |
| CA_8-38 | 8, 38 |  |
| CA_8-39 | 8, 39 |  |
| CA_8-40 | 8, 40 |  |
| CA_8-41 | 8, 41 |  |
| CA_8-42 | 8, 42 |  |
| CA_8-46 | 8, 46 |  |
| CA_11-18 | 11, 18 |  |
| CA_11-26 | 11, 26 |  |
| CA_11-28 | 11, 28 |  |
| CA_11-41 | 11, 41 |  |
| CA_11-42 | 11, 42 |  |
| CA_11-46 | 11, 46 |  |
| CA_12-25 | 12, 25 |  |
| CA_12-30 | 12, 30 |  |
| CA_12-46 | 12, 46 |  |
| CA_12-48 | 12, 48 |  |
| CA_12-48 | 12, 48 |  |
| CA_12-66 | 12, 66 |  |
| CA_12-66-66 | 12, 66 |  |
| CA_13-46 | 13, 46 |  |
| CA_13-46-46 | 13, 46 |  |
| CA_13-48 | 13, 48 |  |
| CA_13-48-48 | 13, 48 |  |
| CA_13-66 | 13, 66 |  |
| CA_13-66-66 | 13, 66 |  |
| CA_14-66 | 14, 66 |  |
| CA_14-66-66 | 14, 66 |  |
| CA_14-66-66-66 | 14, 66 |  |
| CA_14-30 | 14, 30 |  |
| CA_18-281 | 18, 28 |  |
| CA_18-42 | 18, 42 |  |
| CA_19-21 | 19, 21 |  |
| CA_19-282 | 19, 28 |  |
| CA_19-42 | 19, 42 |  |
| CA_19-46 | 19, 46 |  |
| CA_20-281 | 20, 28 |  |
| CA_20-31 | 20, 31 |  |
| CA_20-32 | 20, 32 |  |
| CA_20-38 | 20, 38 |  |
| CA_20-40 | 20, 40 |  |
| CA_20-40-40 | 20, 40 |  |
| CA_20-42 | 20, 42 |  |
| CA_20-42-42 | 20, 42 |  |
| CA_20-43 | 20, 43 |  |
| CA_20-67 | 20, 67 |  |
| CA_20-75 | 20, 75 |  |  |
| CA_20-76 | 20, 76 |  |  |
| CA_21-28 | 21, 28 |  |
| CA_21-42 | 21, 42 |  |
| CA_21-46 | 21, 46 |  |
| CA_23-29 | 23, 29 |  |
| CA_25-26 | 25, 26 |  |
| CA_25-25-26 | 25, 26 |  |
| CA_25-41 | 25, 41 |  |
| CA_25-25-41 | 25, 41 |  |
| CA_25-46 | 25, 46 |  |
| CA_26-41 | 26, 41 |  |
| CA_26-46 | 26, 46 |  |
| CA_26-48 | 26,48 |  |
| CA_26-48-48 | 26,48 |  |
| CA_26-66 | 26, 66 |  |
| CA_28-32 | 28, 32 |  |
| CA_28-38 | 28,38 |  |
| CA_28-40 | 28, 40 |  |
| CA_28-41 | 28, 41 |  |
| CA_28-42 | 28, 42 |  |
| CA_28-42-42 | 28, 42 |  |
| CA_28-46 | 28, 46 |  |
| CA_28-66 | 28, 66 |  |
| CA_29-30 | 29, 30 |  |
| CA_29-66 | 29, 66 |  |
| CA_29-66-66 | 29, 66 |  |
| CA_29-70 | 29, 70 |  |
| CA_30-66 | 30, 66 |  |
| CA_30-66-66 | 30, 66 |  |
| CA_32-42 | 32, 42 |  |
| CA_32-43 | 32, 43 |  |
| CA_34-39 | 34, 39 |  |
| CA_34-41 | 34, 41 |  |
| CA_38-40 | 38, 40 |  |
| CA_38-40-40 | 38, 40 |  |
| CA_39-41 | 39, 41 |  |
| CA_39-40 | 39, 40 |  |
| CA_39-42 | 39, 42 |  |
| CA_39-46 | 39, 46 |  |
| CA_40-41 | 40, 41 |  |
| CA_40-42 | 40, 42 |  |
| CA_40-43 | 40, 43 |  |
| CA_40-46 | 40, 46 |  |
| CA_41-42 | 41, 42 |  |
| CA_41-42-42 | 41, 42 |
| CA_41-46 | 41, 46 |  |
| CA_41-48 | 41, 48 |  |
| CA_42-43 | 42,43 |  |
| CA_42-46 | 42, 46 |  |
| CA_46-48 | 46, 48 |  |
| CA_46-48-48 | 46, 48 |  |
| CA_46-66 | 46, 66 |  |
| CA_46-46-66 | 46, 66 |  |
| CA_46-66-66 | 46, 66 |  |
| CA_46-70 | 46, 70 |  |
| CA_46-71 | 46, 71 |  |
| CA_48-66 | 48, 66 |  |
| CA_48-66-66 | 48, 66 |  |
| CA_48-48-66-66 | 48, 66 |  |
| CA_48-48-66 | 48, 66 |  |
| CA_48-71 | 48, 71 |  |
| CA_48-48-71 | 48, 71 |  |
| CA_66-70 | 66,70 |  |
| CA_66-66-70 | 66,70 |  |
| CA_66-71 | 66, 71 |  |
| CA_66-66-71 | 66, 71 |  |
| CA_70-71 | 70, 71 |  |
| NOTE 1: The frequency range in band 28 is restricted for this CA band combination to 703-733 MHz for the UL and 758-788 MHz for the DL. This restriction also apply for any band combinations when CA_20-28 is a subset of a higher order band combination. NOTE 2: The frequency range in band 28 is restricted for this CA band combination to 718-748 MHz for the UL and 773-803 MHz for the DL |  |  |

Table 5.5A-2a: Inter-band CA operating bands (three bands)

| E-UTRA CA Band | E-UTRA Band (Table 5.5.1) |
| --- | --- |
| CA_1-3-5 | 1, 3, 5 |
| CA_1-3-3-5 | 1, 3, 5 |
| CA_1-1-3-5 | 1, 3, 5 |
| CA_1-1-3-7 | 1, 3, 7 |
| CA_1-1-3-3-7 | 1, 3, 7 |
| CA_1-3-7 | 1, 3, 7 |
| CA_1-3-3-7 | 1, 3, 7 |
| CA_1-3-3-7-7 | 1, 3, 7 |
| CA_1-3-7-7 | 1, 3, 7 |
| CA_1-3-8 | 1, 3, 8 |
| CA_1-3-3-8 | 1, 3, 8 |
| CA_1-3-3-43 | 1, 3, 43 |
| CA_1-3-11 | 1, 3, 11 |
| CA_1-3-18 | 1, 3, 18 |
| CA_1-3-19 | 1, 3, 19 |
| CA_1-3-3-19 | 1, 3, 19 |
| CA_1-3-20 | 1, 3, 20 |
| CA_1-3-3-20 | 1, 3, 20 |
| CA_1-3-21 | 1, 3, 21 |
| CA_1-3-3-21 | 1, 3, 21 |
| CA_1-3-26 | 1, 3, 26 |
| CA_1-3-28 | 1, 3, 28 |
| CA_1-3-3-28 | 1, 3, 28 |
| CA_1-1-3-28 | 1, 3, 28 |
| CA_1-1-3-3-28 | 1, 3, 28 |
| CA_1-3-32 | 1, 3, 32 |
| CA_1-3-38 | 1, 3, 38 |
| CA_1-3-40 | 1, 3, 40 |
| CA_1-3-41 | 1, 3, 41 |
| CA_1-3-42 | 1, 3, 42 |
| CA_1-3-3-42 | 1, 3, 42 |
| CA_1-3-42-42 | 1, 3, 42 |
| CA_1-3-43 | 1, 3, 43 |
| CA_1-3-46 | 1, 3, 46 |
| CA_1-5-7 | 1, 5, 7 |
| CA_1-5-7-7 | 1, 5, 7 |
| CA_1-5-282 | 1, 5, 28 |
| CA_1-5-40 | 1, 5, 40 |
| CA_1-5-41 | 1, 5, 41 |
| CA_1-5-46 | 1, 5, 46 |
| CA_1-7-8 | 1, 7, 8 |
| CA_1-7-7-8 | 1, 7, 8 |
| CA_1-7-20 | 1, 7, 20 |
| CA_1-7-7-20 | 1, 7, 20 |
| CA_1-7-26 | 1, 7, 26 |
| CA_1-7-7-26 | 1, 7, 26 |
| CA_1-7-28 | 1, 7, 28 |
| CA_1-7-32 | 1, 7, 32 |
| CA_1-7-38 | 1, 7, 38 |
| CA_1-7-40 | 1, 7, 40 |
| CA_1-7-42 | 1, 7, 42 |
| CA_1-7-46 | 1, 7, 46 |
| CA_1-8-11 | 1, 8, 11 |
| CA_1-8-20 | 1, 8, 20 |
| CA_1-8-28 | 1, 8, 28 |
| CA_1-8-38 | 1, 8, 38 |
| CA_1-8-40 | 1, 8, 40 |
| CA_1-8-42 | 1, 8, 42 |
| CA_1-11-18 | 1, 11, 18 |
| CA_1-11-28 | 1, 11, 28 |
| CA_1-11-42 | 1, 11, 42 |
| CA_1-18-281 | 1, 18, 28 |
| CA_1-18-41 | 1, 18, 41 |
| CA_1-18-42 | 1, 18, 42 |
| CA_1-19-21 | 1, 19, 21 |
| CA_1-19-282 | 1, 19, 28 |
| CA_1-19-42 | 1, 19, 42 |
| CA_1-20-28 | 1, 20, 28 |
| CA_1-20-32 | 1, 20, 32 |
| CA_1-20-38 | 1, 20, 38 |
| CA_1-20-42 | 1, 20, 42 |
| CA_1-20-43 | 1, 20, 43 |
| CA_1-21-28 | 1, 21, 28 |
| CA_1-21-42 | 1, 21, 42 |
| CA_1-28-40 | 1, 28, 40 |
| CA_1-28-42 | 1, 28, 42 |
| CA_1-32-42 | 1, 32, 42 |
| CA_1-32-43 | 1, 32, 43 |
| CA_1-41-42 | 1, 41, 42 |
| CA_1-42-42 | 1, 42, 42 |
| CA_1-42-43 | 1, 42, 43 |
| CA_2-4-5 | 2, 4, 5 |
| CA_2-2-4-5 | 2, 4, 5 |
| CA_2-4-4-5 | 2, 4, 5 |
| CA_2-4-12-12 | 2, 4, 12 |
| CA_2-5-12-12 | 2, 5, 12 |
| CA_2-2-5-30 | 2, 5, 30 |
| CA_2-5-46 | 2, 5, 46 |
| CA_2-2-5-66 | 2, 5, 66 |
| CA_2-2-7-12 | 2, 7, 12 |
| CA_2-2-7-66 | 2, 7, 66 |
| CA_2-7-66-66 | 2, 7, 66 |
| CA_2-2-12-30 | 2, 12, 30 |
| CA_2-2-12-66 | 2, 12, 66 |
| CA_2-2-12-66-66 | 2, 12, 66 |
| CA_2-2-13-66 | 2, 13, 66 |
| CA_2-2-14-66-66 | 2, 14, 66 |
| CA_2-2-30-66 | 2, 30, 66 |
| CA_2-4-7 | 2, 4, 7 |
| CA_2-4-7-7 | 2, 4, 7 |
| CA_2-4-12 | 2, 4, 12 |
| CA_2-2-4-12 | 2, 4, 12 |
| CA_2-2-5-66-66 | 2, 5, 66 |
| CA_2-4-4-12 | 2, 4, 12 |
| CA_2-4-13 | 2, 4, 13 |
| CA_2-4-28 | 2, 4, 28 |
| CA_2-4-29 | 2, 4, 29 |
| CA_2-4-30 | 2, 4, 30 |
| CA_2-4-71 | 2, 4, 71 |
| CA_2-2-4-71 | 2, 4, 71 |
| CA_2-5-7 | 2, 5, 7 |
| CA_2-5-12 | 2, 5, 12 |
| CA_2-2-5-12 | 2, 5, 12 |
| CA_2-5-13 | 2, 5, 13 |
| CA_2-5-28 | 2, 5, 28 |
| CA_2-5-29 | 2, 5, 29 |
| CA_2-5-30 | 2, 5, 30 |
| CA_2-5-46 | 2, 5, 46 |
| CA_2-5-66 | 2, 5, 66 |
| CA_2-5-66-66 | 2, 5, 66 |
| CA_2-7-12 | 2, 7, 12 |
| CA_2-7-13 | 2, 7, 13 |
| CA_2-7-26 | 2, 7, 26 |
| CA_2-7-28 | 2, 7, 28 |
| CA_2-7-29 | 2, 7, 29 |
| CA_2-7-30 | 2, 7, 30 |
| CA_2-7-46 | 2, 7, 46 |
| CA_2-7-66 | 2, 7, 66 |
| CA_2-7-7-66 | 2, 7, 66 |
| CA_2-7-66-66 | 2, 7, 66 |
| CA_2-7-7-66-66 | 2, 7, 66 |
| CA_2-12-30 | 2, 12, 30 |
| CA_2-12-66 | 2, 12, 66 |
| CA_2-12-66-66 | 2, 12, 66 |
| CA_2-13-46 | 2, 13, 46 |
| CA_2-13-48 | 2, 13, 48 |
| CA_2-13-48-48 | 2, 13, 48 |
| CA_2-13-66 | 2, 13, 66 |
| CA_2-13-66-66 | 2, 13, 66 |
| CA_2-14-30 | 2, 14, 30 |
| CA_2-2-14-30 | 2, 14, 30 |
| CA_2-14-66 | 2, 14, 66 |
| CA_2-2-14-66 | 2, 14, 66 |
| CA_2-14-66-66 | 2, 14, 66 |
| CA_2-14-66-66-66 | 2, 14, 66 |
| CA_2-26-66 | 2, 26, 66 |
| CA_2-28-66 | 2, 28, 66 |
| CA_2-2-29-30 | 2, 29, 30 |
| CA_2-29-30 | 2, 29, 30 |
| CA_2-29-66 | 2, 29, 66 |
| CA_2-30-66 | 2, 30, 66 |
| CA_2-30-66-66 | 2, 30, 66 |
| CA_2-46-48 | 2, 46, 48 |
| CA_2-46-66 | 2, 46, 66 |
| CA_2-46-46-66 | 2, 46, 66 |
| CA_2-48-66 | 2, 48,66 |
| CA_2-48-48-66 | 2, 48, 66 |
| CA_2-66-71 | 2, 66, 71 |
| CA_2-2-66-71 | 2, 66, 71 |
| CA_2-66-66-71 | 2, 66, 71 |
| CA_3-5-7 | 3, 5, 7 |
| CA_3-5-7-7 | 3, 5, 7 |
| CA_3-5-28 | 3, 5, 28 |
| CA_3-3-5-282 | 3, 5, 28 |
| CA_3-5-40 | 3, 5, 40 |
| CA_3-5-40-40 | 3, 5, 40 |
| CA_3-5-41 | 3, 5, 41 |
| CA_3-3-7-8 | 3, 7, 8 |
| CA_3-3-7-7-8 | 3, 7, 8 |
| CA_3-7-7-8 | 3, 7, 8 |
| CA_3-7-8 | 3, 7, 8 |
| CA_3-7-20 | 3, 7, 20 |
| CA_3-3-7-20 | 3, 7, 20 |
| CA_3-7-7-20 | 3, 7, 20 |
| CA_3-7-26 | 3, 7, 26 |
| CA_3-7-7-26 | 3, 7, 26 |
| CA_3-7-28 | 3, 7, 28 |
| CA_3-3-7-28 | 3, 7, 28 |
| CA_3-7-32 | 3, 7, 32 |
| CA_3-7-38 | 3, 7, 38 |
| CA_3-7-40 | 3, 7, 40 |
| CA_3-7-42 | 3, 7, 42 |
| CA_3-7-46 | 3, 7, 46 |
| CA_3-8-11 | 3, 8, 11 |
| CA_3-8-20 | 3, 8, 20 |
| CA_3-8-28 | 3, 8, 28 |
| CA_3-8-32 | 3, 8, 32 |
| CA_3-8-38 | 3, 8, 38 |
| CA_3-8-40 | 3, 8, 40 |
| CA_3-8-42 | 3, 8, 42 |
| CA_3-11-18 | 3, 11,18 |
| CA_3-11-26 | 3, 11, 26 |
| CA_3-11-28 | 3, 11, 28 |
| CA_3-18-42 | 3, 18, 42 |
| CA_3-19-21 | 3, 19, 21 |
| CA_3-3-19-21 | 3, 19, 21 |
| CA_3-19-42 | 3, 19, 42 |
| CA_3-20-28 | 3, 20, 28 |
| CA_3-3-20-28 | 3, 20, 28 |
| CA_3-20-32 | 3, 20, 32 |
| CA_3-20-42 | 3, 20, 42 |
| CA_3-20-43 | 3, 20, 43 |
| CA_3-21-28 | 3, 21, 28 |
| CA_3-21-42 | 3, 21, 42 |
| CA_3-28-38 | 3, 28, 38 |
| CA_3-28-40 | 3, 28, 40 |
| CA_3-28-41 | 3, 28, 41 |
| CA_3-28-42 | 3, 28, 42 |
| CA_3-28-42-42 | 3, 28, 42 |
| CA_3-32-42 | 3, 32, 42 |
| CA_3-32-43 | 3, 32, 43 |
| CA_3-32-46 | 3, 32, 46 |
| CA_3-41-42 | 3, 41, 42 |
| CA_3-41-42-42 | 3, 41, 42 |
| CA_3-42-43 | 3, 42, 43 |
| CA_4-5-12 | 4, 5, 12 |
| CA_4-4-5-12 | 4, 5, 12 |
| CA_4-5-12-12 | 4, 5, 12 |
| CA_4-5-13 | 4, 5, 13 |
| CA_4-5-29 | 4, 5, 29 |
| CA_4-5-30 | 4, 5, 30 |
| CA_4-4-5-30 | 4, 5, 30 |
| CA_4-7-12 | 4, 7, 12 |
| CA_4-7-28 | 4, 7, 28 |
| CA_4-12-30 | 4, 12, 30 |
| CA_4-4-12-30 | 4, 12, 30 |
| CA_4-29-30 | 4, 29, 30 |
| CA_4-4-29-30 | 4, 29, 30 |
| CA_5-7-28 | 5, 7, 28 |
| CA_5-7-46 | 5, 7, 46 |
| CA_5-7-66 | 5, 7, 66 |
| CA_5-7-66-66 | 5, 7, 66, 66 |
| CA_5-12-46 | 5, 12, 46 |
| CA_5-12-48 | 5, 12, 48 |
| CA_5-12-66 | 5, 12, 66 |
| CA_5-30-66 | 5, 30, 66 |
| CA_5-30-66-66 | 5, 30, 66 |
| CA_5-40-41 | 5, 40, 41 |
| CA_5-46-66 | 5, 46, 66 |
| CA_5-46-66-66 | 5, 46, 66 |
| CA_5-48-66 | 5, 48, 66 |
| CA_5-48-66-66 | 5, 48, 66 |
| CA_7-8-20 | 7, 8, 20 |
| CA_7-8-38 | 7, 8, 38 |
| CA_7-8-40 | 7, 8, 40 |
| CA_7-12-66 | 7, 12, 66 |
| CA_7-13-66 | 7, 13, 66 |
| CA_7-20-28 | 7, 20, 28 |
| CA_7-20-32 | 7, 20, 32 |
| CA_7-20-38 | 7, 20, 38 |
| CA_7-20-42 | 7, 20, 42 |
| CA_7-26-66 | 7, 26, 66 |
| CA_7-28-38 | 7, 28, 38 |
| CA_7-28-40 | 7, 28, 40 |
| CA_7-29-66 | 7, 29, 66 |
| CA_7-7-29-66 | 7, 29, 66 |
| CA_7-30-66 | 7, 30, 66 |
| CA_7-32-46 | 7, 32, 46 |
| CA_7-46-66 | 7, 46, 66 |
| CA_8-11-28 | 8, 11, 28 |
| CA_8-11-42 | 8, 11, 42 |
| CA_8-20-28 | 8, 20, 28 |
| CA_8-28-41 | 8, 28, 41 |
| CA_8-39-41 | 8, 39 ,41 |
| CA_12-30-66 | 12, 30, 66 |
| CA_12-30-66-66 | 12, 30, 66 |
| CA_13-46-66 | 13, 46, 66 |
| CA_13-48-66 | 13, 48, 66 |
| CA_13-48-48-66 | 13, 48, 66 |
| CA_14-30-66 | 14, 30, 66 |
| CA_14-30-66-66 | 14, 30, 66 |
| CA_19-21-42 | 19, 21, 42 |
| CA_20-32-42 | 20, 32, 42 |
| CA_20-32-43 | 20, 32, 43 |
| CA_20-38-40 | 20, 38, 40 |
| CA_25-26-41 | 25, 26, 41 |
| CA_25-25-26-41 | 25, 26, 41 |
| CA_20-38-40-40 | 20, 38, 40 |
| CA_21-28-42 | 21, 28, 42 |
| CA_29-30-66-66 | 29, 30, 66 |
| CA_28-41-42 | 28, 41, 42 |
| CA_28-41-42-42 | 28, 41, 42 |
| CA_29-30-66 | 29, 30, 66 |
| CA_29-46-66 | 29, 46, 66 |
| CA_29-66-70 | 29, 66, 70 |
| CA_29-66-66-70 | 29, 66, 70 |
| CA_32-42-43 | 32, 42, 43 |
| CA_46-48-66 | 46, 48, 66 |
| CA_46-48-71 | 46, 48, 71 |
| CA_46-48-48-71 | 46, 48, 71 |
| CA_66-70-71 | 66, 70, 71 |
| CA_66-66-70-71 | 66, 70, 71 |
| NOTE 1: The frequency range in band 28 is restricted for this CA band combination to 703-733 MHz for the UL and 758-788 MHz for the DL NOTE 2: The frequency range in band 28 is restricted for this CA band combination to 718-748 MHz for the UL and 773-803 MHz for the DL |  |

Table 5.5A-2b: Inter-band CA operating bands (four bands)

| E-UTRA CA Band | E-UTRA Band (Table 5.5) |
| --- | --- |
| CA_1-3-5-7 | 1, 3, 5, 7 |
| CA_1-3-3-5-7 | 1, 3, 5, 7 |
| CA_1-3-5-7-7 | 1, 3, 5, 7 |
| CA_1-3-5-282 | 1, 3, 5, 28 |
| CA_1-3-5-40 | 1, 3, 5, 40 |
| CA_1-3-5-41 | 1, 3, 5, 41 |
| CA_1-3-7-7-26 | 1, 3, 7, 26 |
| CA_1-3-7-8 | 1, 3, 7, 8 |
| CA_1-3-3-7-8 | 1, 3, 7, 8 |
| CA_1-3-7-7-8 | 1, 3, 7, 8 |
| CA_1-3-3-7-7-8 | 1, 3, 7, 8 |
| CA_1-3-7-20 | 1, 3, 7, 20 |
| CA_1-3-7-7-20 | 1, 3, 7, 20 |
| CA_1-3-3-7-20 | 1, 3, 7, 20 |
| CA_1-3-7-26 | 1, 3, 7, 26 |
| CA_1-3-7-28 | 1, 3, 7, 28 |
| CA_1-1-3-7-28 | 1, 3, 7, 28 |
| CA_1-3-3-7-28 | 1, 3, 7, 28 |
| CA_1-1-3-3-7-28 | 1, 3, 7, 28 |
| CA_1-3-7-7-28 | 1, 3, 7, 28 |
| CA_1-3-7-32 | 1, 3, 7, 32 |
| CA_1-3-7-40 | 1, 3, 7, 40 |
| CA_1-3-7-42 | 1, 3, 7, 42 |
| CA_1-3-7-46 | 1, 3, 7, 46 |
| CA_1-3-8-11 | 1, 3, 8, 11 |
| CA_1-3-8-20 | 1, 3, 8, 20 |
| CA_1-3-8-28 | 1, 3, 8, 28 |
| CA_1-3-8-38 | 1, 3, 8, 38 |
| CA_1-3-11-28 | 1, 3, 11, 28 |
| CA_1-3-8-40 | 1, 3, 8, 40 |
| CA_1-3-8-42 | 1, 3, 8, 42 |
| CA_1-3-18-42 | 1, 3, 18, 42 |
| CA_1-3-19-21 | 1, 3,19, 21 |
| CA_1-3-3-19-21 | 1, 3,19, 21 |
| CA_1-3-19-42 | 1, 3,19, 42 |
| CA_1-3-20-28 | 1, 3, 20, 28 |
| CA_1-3-3-20-28 | 1, 3, 20, 28 |
| CA_1-3-20-32 | 1, 3, 20, 32 |
| CA_1-3-20-42 | 1, 3, 20, 42 |
| CA_1-3-20-43 | 1, 3, 20, 43 |
| CA_1-3-21-28 | 1, 3, 21, 28 |
| CA_1-3-21-42 | 1, 3, 21, 42 |
| CA_1-3-28-40 | 1, 3, 28, 40 |
| CA_1-3-28-42 | 1, 3, 28, 42 |
| CA_1-3-32-42 | 1, 3, 32, 42 |
| CA_1-3-32-43 | 1, 3, 32, 43 |
| CA_1-3-41-42 | 1, 3, 41, 42 |
| CA_1-3-42-43 | 1, 3, 42, 43 |
| CA_1-5-7-282 | 1, 5, 7, 28 |
| CA_1-5-7-46 | 1, 5, 7, 46 |
| CA_1-7-8-20 | 1, 7, 8, 20 |
| CA_1-7-8-40 | 1, 7, 8, 40 |
| CA_1-7-20-28 | 1, 7, 20, 28 |
| CA_1-7-20-32 | 1, 7, 20, 32 |
| CA_1-7-20-42 | 1, 7, 20, 42 |
| CA_1-7-28-40 | 1, 7, 28, 40 |
| CA_1-8-11-28 | 1, 8, 11, 28 |
| CA_1-8-11-42 | 1, 8, 11, 42 |
| CA_1-8-20-28 | 1, 8, 20, 28 |
| CA_1-19-21-42 | 1, 19, 21, 42 |
| CA_1-20-32-42 | 1, 20, 32, 42 |
| CA_1-20-32-43 | 1, 20, 32, 43 |
| CA_1-21-28-42 | 1, 21, 28, 42 |
| CA_1-32-42-43 | 1, 32, 42, 43 |
| CA_2-2-5-12-66 | 2, 5, 12, 66 |
| CA_2-2-5-30-66 | 2, 5, 30, 66 |
| CA_2-2-7-12-66 | 2. 7, 12, 66 |
| CA_2-2-12-30-66 | 2, 12, 30, 66 |
| CA_2-2-14-30-66 | 2, 14, 30, 66 |
| CA_2-4-5-12 | 2, 4, 5, 12 |
| CA_2-4-5-29 | 2, 4, 5, 29 |
| CA_2-4-5-30 | 2, 4, 5, 30 |
| CA_2-4-7-12 | 2, 4, 7, 12 |
| CA_2-4-12-30 | 2, 4, 12, 30 |
| CA_2-4-29-30 | 2, 4, 29, 30 |
| CA_2-5-7-28 | 2, 5, 7, 28 |
| CA_2-5-12-66 | 2, 5, 12, 66 |
| CA_2-5-30-66 | 2, 5, 30, 66 |
| CA_2-5-30-66-66 | 2, 5, 30, 66 |
| CA_2-7-12-66 | 2, 7, 12, 66 |
| CA_2-7-13-66 | 2, 7, 13, 66 |
| CA_2-7-26-66 | 2, 7, 26, 66 |
| CA_2-7-29-66 | 2, 7, 29, 66 |
| CA_2-7-7-29-66 | 2, 7, 29, 66 |
| CA_2-7-46-66 | 2, 7, 46, 66 |
| CA_2-12-30-66 | 2, 12, 30, 66 |
| CA_2-12-30-66-66 | 2, 12, 30, 66 |
| CA_2-13-48-66 | 2, 13, 48, 66 |
| CA_2-13-48-48-66 | 2, 13, 48, 66 |
| CA_2-14-30-66 | 2, 14, 30, 66 |
| CA_2-14-30-66-66 | 2, 14, 30, 66 |
| CA_2-29-30-66 | 2, 29, 30, 66 |
| CA_2-46-48-66 | 2, 46, 48, 66 |
| CA_3-5-7-282 | 3, 5, 7, 28 |
| CA_3-3-5-7-282 | 3, 5, 7, 28 |
| CA_3-7-8-20 | 3, 7, 8, 20 |
| CA_3-7-8-38 | 3, 7, 8, 38 |
| CA_3-7-8-40 | 3, 7, 8, 40 |
| CA_3-7-20-28 | 3, 7, 20, 28 |
| CA_3-7-20-32 | 3, 7, 20, 32 |
| CA_3-7-20-42 | 3, 7, 20, 42 |
| CA_3-7-28-38 | 3, 7, 28, 38 |
| CA_3-7-28-40 | 3, 7, 28, 40 |
| CA_3-7-32-46 | 3, 7, 32, 46 |
| CA_3-8-11-28 | 3, 8, 11, 28 |
| CA_3-8-20-28 | 3, 8, 20, 28 |
| CA_3-19-21-42 | 3, 19, 21, 42 |
| CA_3-20-32-42 | 3, 20, 32, 42 |
| CA_3-20-32-43 | 3, 20, 32, 43 |
| CA_3-21-28-42 | 3, 21, 28, 42 |
| CA_3-28-41-42 | 3, 28, 41, 42 |
| CA_3-32-42-43 | 3, 32, 42, 43 |
| NOTE 1: The frequency range in band 28 is restricted for this CA band combination to 703-733 MHz for the UL and 758-788 MHz for the DL NOTE 2: The frequency range in band 28 is restricted for this CA band combination to 718-748 MHz for the UL and 773-803 MHz for the DL |  |

Table 5.5A-2c: Inter-band CA operating bands (five bands)

| E-UTRA CA Band | E-UTRA Band (Table 5.5) |
| --- | --- |
| CA_1-3-5-7-282 | 1, 3, 5, 7, 28 |
| CA_1-3-7-8-20 | 1, 3, 7, 8, 20 |
| CA_1-3-7-20-28 | 1, 3, 7, 20, 28 |
| CA_1-3-7-20-32 | 1, 3, 7, 20, 32 |
| CA_1-3-7-20-42 | 1, 3, 7, 20, 42 |
| CA_1-3-8-11-28 | 1, 3, 8, 11, 28 |
| CA_1-3-20-32-42 | 1, 3, 20, 32, 42 |
| CA_1-3-20-32-43 | 1, 3, 20, 32, 43 |
| CA_1-3-32-42-43 | 1, 3, 32, 42, 43 |
| NOTE 1: The frequency range in band 28 is restricted for this CA band combination to 703-733 MHz for the UL and 758-788 MHz for the DL NOTE 2: The frequency range in band 28 is restricted for this CA band combination to 718-748 MHz for the UL and 773-803 MHz for the DL |  |

Table 5.5A-3: Intra-band non-contiguous CA operating bands (with two sub-blocks)

| E-UTRA CA Band | E-UTRA Band (Table 5.5) |
| --- | --- |
| CA_1-1 | 1 |
| CA_2-2 | 2 |
| CA_3-3 | 3 |
| CA_4-4 | 4 |
| CA_5-5 | 5 |
| CA_7-7 | 7 |
| CA_12-12 | 12 |
| CA_23-23 | 23 |
| CA_25-25 | 25 |
| CA_40-40 | 40 |
| CA_41-41 | 41 |
| CA_42-42 | 42 |
| CA_43-43 | 43 |
| CA_48-48 | 48 |
| CA_66-66 | 66 |

Table 5.5A-4: Intra-band non-contiguous CA operating bands (with three sub-blocks)

| E-UTRA CA Band | E-UTRA Band (Table 5.5) |
| --- | --- |
| CA_25-25-25 | 25 |
| CA_41-41-41 | 41 |
| CA_48-48-48 | 48 |
| CA_66-66-66 | 66 |

Table 5.5A-5: Intra-band non-contiguous CA operating bands (with four sub-blocks)

| E-UTRA CA Band | E-UTRA Band (Table 5.5) |
| --- | --- |
| CA_48-48-48-48 | 48 |

## 5.5B	Operating bands for UL-MIMO

E-UTRA UL-MIMO is designed to operate in the operating bands defined in Table 5.5B-1.

Table 5.5B-1: Void

## 5.5C	Operating bands for Dual Connectivity

E-UTRA dual connectivity is designed to operate in the operating bands defined in Table 5.5C-1.

Table 5.5C-1: Inter-band dual connectivity operating bands (two bands)

| E-UTRA DC Band | E-UTRA Band (Table 5.5) |
| --- | --- |
| DC_1-3 | 1, 3 |
| DC_1-5 | 1, 5 |
| DC_1-7 | 1, 7 |
| DC_1-8 | 1, 8 |
| DC_1-11 | 1, 11 |
| DC_1-19 | 1, 19 |
| DC_1-20 | 1, 20 |
| DC_1-21 | 1, 21 |
| DC_1-42 | 1, 42 |
| DC_2-4 | 2, 4 |
| DC_2-5 | 2, 5 |
| DC_2-7 | 2, 7 |
| DC_2-12 | 2, 12 |
| DC_2-13 | 2, 13 |
| DC_2-14 | 2, 14 |
| DC_2-30 | 2, 30 |
| DC_2-48 | 2, 48 |
| DC_2-66 | 2, 66 |
| DC_3-5 | 3, 5 |
| DC_3-7 | 3, 7 |
| DC_3-8 | 3, 8 |
| DC_3-11 | 3, 11 |
| DC_3-18 | 3, 18 |
| DC_3-19 | 3, 19 |
| DC_3-20 | 3, 20 |
| DC_3-21 | 3, 21 |
| DC_3-26 | 3, 26 |
| DC_3-28 | 3, 28 |
| DC_3-40 | 3, 40 |
| DC_3-42 | 3, 42 |
| DC_4-5 | 4, 5 |
| DC_4-7 | 4, 7 |
| DC_4-12 | 4, 12 |
| DC_4-13 | 4, 13 |
| DC_4-17 | 4, 17 |
| DC_4-28 | 4, 28 |
| DC_5-7 | 5, 7 |
| DC_5-12 | 5,12 |
| DC_5-17 | 5,17 |
| DC_5-30 | 5, 30 |
| DC_5-40 | 5, 40 |
| DC_5-66 | 5, 66 |
| DC_7-8 | 7, 8 |
| DC_7-20 | 7, 20 |
| DC_7-26 | 7, 26 |
| DC_7-28 | 7, 28 |
| DC_8-39 | 8, 39 |
| DC_8-41 | 8, 41 |
| DC_11-18 | 11, 18 |
| DC_11-26 | 11, 26 |
| DC_12-30 | 12, 30 |
| DC_12-66 | 12, 66 |
| DC_13-66 | 13, 66 |
| DC_14-30 | 14, 30 |
| DC_14-66 | 14, 66 |
| DC_19-21 | 19, 21 |
| DC_19-42 | 19, 42 |
| DC_21-42 | 21, 42 |
| DC_21-28 | 21, 28 |
| DC_25-26 | 25, 26 |
| DC_25-41 | 25, 41 |
| DC_26-46 | 26, 46 |
| DC_26-48 | 26, 48 |
| DC_28-41 | 28, 41 |
| DC_28-42 | 28, 42 |
| DC_30-66 | 30, 66 |
| DC_39-41 | 39, 41 |
| DC_40-42 | 40, 42 |
| DC_41-42 | 41, 42 |

Table 5.5C-2: Inter-band dual connectivity operating bands (three bands)

| E-UTRA DC Band | E-UTRA Band (Table 5.5) |
| --- | --- |
| DC_1-3-19 | 1, 3, 19 |
| DC_1-19-21 | 1, 19, 21 |

## 5.5D	Operating bands for ProSe

E-UTRA ProSe is designed to operate in the operating bands defined in Table 5.5D-1.

Table 5.5D-1 E-UTRA ProSe operating band

| E-UTRA ProSe Band | E-UTRA Operating Band | ProSe UE transmit |  |  | ProSe UE receive |  |  | ProSe Duplex Mode | ProSe Direct |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  | Disc. | Comm. |
| 2 | 2 | 1850 MHz | – | 1910 MHz | 1850 MHz | – | 1910 MHz | HD | Yes |  |
| 3 | 3 | 1710 MHz | – | 1785 MHz | 1710 MHz | – | 1785 MHz | HD | Yes | Yes |
| 4 | 4 | 1710 MHz | – | 1755 MHz | 1710 MHz | – | 1755 MHz | HD | Yes |  |
| 7 | 7 | 2500 MHz | – | 2570 MHz | 2500 MHz | – | 2570 MHz | HD | Yes | Yes |
| 14 | 14 | 788 MHz | – | 798 MHz | 788 MHz | – | 798 MHz | HD | Yes | Yes |
| 20 | 20 | 832 MHz | – | 862 MHz | 832 MHz | – | 862 MHz | HD | Yes | Yes |
| 26 | 26 | 814 MHz | – | 849 MHz | 814 MHz | – | 849 MHz | HD | Yes | Yes |
| 28 | 28 | 703 MHz | – | 748 MHz | 703 MHz | – | 748 MHz | HD | Yes | Yes |
| 31 | 31 | 452.5 MHz | – | 457.5 MHz | 452.5 MHz | – | 457.5 MHz | HD | Yes | Yes |
| 41 | 41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | HD | Yes |  |
| 68 | 68 | 698 MHz | – | 728 MHz | 698 MHz | – | 728 MHz | HD | Yes | Yes |
| 72 | 72 | 451 MHz | – | 456 MHz | 451 MHz | – | 456 MHz | HD | Yes | Yes |

E-UTRA ProSe is designed to operate concurrent with E-UTRA uplink/downlink on the operating bands combinations listed in Table 5.5D-2.

Table 5.5D-2 Inter-band E-UTRA ProSe / E-UTRA operating bands

| E-UTRA ProSe Band Note 1 | E-UTRA band / E-UTRA CA band Note 2 |
| --- | --- |
| 2 | 4 |
|  | CA_2-4Note 3 |
| 28 | 1 |
|  | CA_1-28Note 3 |
| NOTE 1: As specified in Table 5.5D-1 NOTE 2: As specified in Table 5.5-1 and Table 5.5A-2 NOTE 3: Applies when E-UTRA uplink is assigned to one E-UTRA band and ProSe operation is restricted to the uplink frequencies paired with either PCC or SCC. NOTE 4: The concurrency for E-UTRA ProSe Direct Discovery with E-UTRA uplink/downlink applies after allowing for any transmission and/or reception gap requested by the UE. |  |

## 5.5E	Operating bands for UE category 0, UE category M1 and M2 and UE category 1bis

UE category 0 is designed to operate in the E-UTRA operating bands 2, 3, 4, 5, 8, 13, 20, 25, 26 and 28 in both half duplex FDD mode and full-duplex FDD mode and in bands 39, 40 and 41 in TDD mode. The E-UTRA bands are defined in Table 5.5-1.

UE category M1 and M2 is designed to operate in the E-UTRA operating bands 1, 2, 3, 4, 5, 7, 8, 11, 12, 13, 14, 18, 19, 20, 21, 25, 26, 27, 28, 31, 66, 71, 72, 73, 74, 85, 87 and 88 in both half duplex FDD mode and full-duplex FDD mode, and in bands 39, 40, 41, 42 and 43 in TDD mode. The E-UTRA bands are defined in Table 5.5-1.

UE category 1bis is designed to operate in the E-UTRA operating bands 1, 2, 3, 4, 5, 7, 8, 12, 13, 18, 20, 26, 28, 31, 66 and 72 in full duplex FDD mode and in bands 34, 39, 40 and 41 in TDD mode. The E-UTRA bands are defined in Table 5.5-1

## 5.5F	Operating bands for category NB1 and NB2

Category NB1 and NB2 are designed to operate in the E-UTRA operating bands 1, 2, 3, 4, 5, 7, 8, 11, 12, 13, 14, 17, 18, 19, 20, 21, 25, 26, 28, 31, 41, 42, 43, 65, 66, 70, 71, 72, 73, 74, 85, 87 and 88 which are defined in Table 5.5-1. Category NB1 and NB2 are designed to operate in the NR operating bands n1, n2, n3, n5, n7, n8, n12, n14, n18, n20, n25, n26, n28, n41, n65, n66, n70, n71, n74, n90.

Category NB1 and NB2 systems operate in HD-FDD duplex mode or in TDD mode.

In case UE receives network signaling value NS_04 on any of the operating bands listed in Table 5.5F-1 then the lower and upper limit of those bands are shown in Table 5.5F-1 to account for the USA emission requirements.

Table 5.5F-1 E-UTRA operating bands for NB-IoT in the USA

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 2 | 1850.1 MHz | – | 1909.9 MHz | 1930.1 MHz | – | 1989.9 MHz | FDD |
| 4 | 1710.1 MHz | – | 1754.9 MHz | 2110.1 MHz | – | 2154.9 MHz | FDD |
| 5 | 824.1 MHz | – | 848.9 MHz | 869.1 MHz | – | 893.9MHz | FDD |
| 12 | 699 MHz | – | 715.9 MHz | 729 MHz | – | 745.9 MHz | FDD |
| 13 | 777 MHz | – | 786.9 MHz | 746 MHz | – | 755.9 MHz | FDD |
| 17 | 704.1 MHz | – | 715.9 MHz | 734.1 MHz | – | 745.9 MHz | FDD |
| 25 | 1850.1 MHz | – | 1914.9 MHz | 1930.1 MHz | – | 1994.9 MHz | FDD |
| 26 | 814.1 MHz | – | 848.9 MHz | 859.1 MHz | – | 893.9 MHz | FDD |
| 66 | 1710.1 MHz | – | 1779.9 MHz | 2110.1 MHz | – | 2199.9 MHz | FDD |
| 70 | 1695.1 MHz | – | 1709.9 MHz | 1995.1 MHz | – | 2019.9 MHz | FDD |
| 71 | 663.1 MHz | – | 697.9 MHz | 617.1 MHz | – | 651.9 MHz | FDD |
| 85 | 698.1 MHz | – | 715.9 MHz | 728.1 MHz | – | 745.9 MHz | FDD |

## 5.5G	Operating bands for V2X Communication

E-UTRA V2X Communication is designed to operate in the the operating bands defined in Table 5.5G-1.

Table 5.5G-1 V2X operating band

| E-UTRA Operating Band | E-UTRA V2X Operating Band | V2X UE transmit |  |  | V2X UE receive |  |  | Duplex Mode | Interface |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |  |
| 47 | 47 | 5855 MHz |  | 5925 MHz | 5855 MHz |  | 5925 MHz | HD | PC5 |

E-UTRA V2X communication is designed to operate concurrent with E-UTRA uplink/downlink on the operating bands combinations listed in Table 5.5G-2.

Table 5.5G-2 Inter-band con-current V2X operating bands

| V2X con-current band configuration | E-UTRA or V2X Operating Band | Interface | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| V2X_3-47 | 3 | Uu | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 47 | PC5 | 5855 MHz | – | 5925 MHz | 5855 MHz |  | 5925 MHz | HD |
| V2X_5-47 | 5 | Uu | 824 MHz | – | 849 MHz | 869 MHz | – | 894 MHz | FDD |
|  | 47 | PC5 | 5855 MHz | – | 5925 MHz | 5855 MHz | – | 5925 MHz | HD |
| V2X_7-47 | 7 | Uu | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 47 | PC5 | 5855 MHz | – | 5925 MHz | 5855 MHz |  | 5925 MHz | HD |
| V2X_8-47 | 8 | Uu | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 47 | PC5 | 5855 MHz | – | 5925 MHz | 5855 MHz |  | 5925 MHz | HD |
| V2X_20-47 | 20 | Uu | 832 MHz | – | 862 MHz | 791 MHz | – | 821 MHz | FDD |
|  | 47 | PC5 | 5855 MHz | – | 5925 MHz | 5855 MHz |  | 5925 MHz | HD |
| V2X_28-47 | 28 | Uu | 703 MHz | – | 748 MHz | 758 MHz | – | 803 MHz | FDD |
|  | 47 | PC5 | 5855 MHz | – | 5925 MHz | 5855 MHz |  | 5925 MHz | HD |
| V2X_34-47 | 34 | Uu | 2010 MHz | – | 2025 MHz | 2010 MHz | – | 2025 MHz | TDD |
|  | 47 | PC5 | 5855 MHz |  | 5925 MHz | 5855 MHz |  | 5925 MHz | HD |
| V2X_39-47 | 39 | Uu | 1880 MHz | – | 1920 MHz | 1880 MHz | – | 1920 MHz | TDD |
|  | 47 | PC5 | 5855 MHz | – | 5925 MHz | 5855 MHz |  | 5925 MHz | HD |
| V2X_41-47 | 41 | Uu | 2496 MHz | – | 2690 MHz | 2496 MHz |  | 2690 MHz | TDD |
|  | 47 | PC5 | 5855 MHz | – | 5925 MHz | 5855 MHz |  | 5925 MHz | HD |
| V2X_71-47 | 71 | Uu | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |
|  | 47 | PC5 | 5855 MHz | – | 5925 MHz | 5855 MHz |  | 5925 MHz | HD |

E-UTRA V2X communication is also designed to operate for intra-band multi-carrier operation in the operating bands defined in Table 5.5G-3.

Table 5.5G-3: V2X intra-band multi-carrier operation

| V2X multi-carrier Band configuration | V2X operating Band | Interface |
| --- | --- | --- |
| V2X_47 | 47 | PC5 |

## 5.6	Channel bandwidth

Requirements in present document are specified for the channel bandwidths listed in Table 5.6-1.

Table 5.6-1: Transmission bandwidth configuration NRB in E-UTRA channel bandwidths

| Channel bandwidth BWChannel [MHz] | 1.4 | 3 | 5 | 10 | 15 | 20 |
| --- | --- | --- | --- | --- | --- | --- |
| Transmission bandwidth configuration NRB | 6 | 15 | 25 | 50 | 75 | 100 |

Figure 5.6-1 shows the relation between the Channel bandwidth (BWChannel) and the Transmission bandwidth configuration (NRB). The channel edges are defined as the lowest and highest frequencies of the carrier separated by the channel bandwidth, i.e. at FC +/- BWChannel /2.

Transmission

Center subcarrier (corresponds to DC in baseband) is not transmitted in downlink

Active Resource Blocks

Resource block

Transmission bandwidth configuration [NRB]

bandwidth [RB]

Channel bandwidth [MHz]

Figure 5.6-1: Definition of channel bandwidth and transmission bandwidth configuration for one E-UTRA carrier

### 5.6.1	Channel bandwidths per operating band

a)	The requirements in this specification apply to the combination of channel bandwidths and operating bands shown in Table 5.6.1-1. The transmission bandwidth configuration in Table 5.6.1-1 shall be supported for each of the specified channel bandwidths. The same (symmetrical) channel bandwidth is specified for both the TX and RX path.

Table 5.6.1-1: E-UTRA channel bandwidth

| E-UTRA band / Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| 1 |  |  | Yes | Yes | Yes | Yes |
| 2 | Yes | Yes | Yes | Yes | Yes1 | Yes1 |
| 3 | Yes | Yes | Yes | Yes | Yes1 | Yes1 |
| 4 | Yes | Yes | Yes | Yes | Yes | Yes |
| 5 | Yes | Yes | Yes | Yes1 |  |  |
| 6 |  |  | Yes | Yes1 |  |  |
| 7 |  |  | Yes | Yes | Yes3 | Yes1, 3 |
| 8 | Yes | Yes | Yes | Yes1 |  |  |
| 9 |  |  | Yes | Yes | Yes1 | Yes1 |
| 10 |  |  | Yes | Yes | Yes | Yes |
| 11 |  |  | Yes | Yes1 |  |  |
| 12 | Yes | Yes | Yes1 | Yes1 |  |  |
| 13 |  |  | Yes1 | Yes1 |  |  |
| 14 |  |  | Yes1 | Yes1 |  |  |
| ... |  |  |  |  |  |  |
| 17 |  |  | Yes1 | Yes1 |  |  |
| 18 |  |  | Yes | Yes1 | Yes1 |  |
| 19 |  |  | Yes | Yes1 | Yes1 |  |
| 20 |  |  | Yes | Yes1 | Yes1 | Yes1 |
| 21 |  |  | Yes | Yes1 | Yes1 |  |
| 22 |  |  | Yes | Yes | Yes1 | Yes1 |
| 23 | Yes | Yes | Yes | Yes | Yes1 | Yes1 |
| 24 |  |  | Yes | Yes |  |  |
| 25 | Yes | Yes | Yes | Yes | Yes1 | Yes1 |
| 26 | Yes | Yes | Yes | Yes1 | Yes1 |  |
| 27 | Yes | Yes | Yes | Yes1 |  |  |
| 28 |  | Yes | Yes | Yes1 | Yes1 | Yes1, 2 |
| 30 |  |  | Yes | Yes1 |  |  |
| 31 | Yes | Yes1 | Yes1 |  |  |  |
| ... |  |  |  |  |  |  |
| 33 |  |  | Yes | Yes | Yes | Yes |
| 34 |  |  | Yes | Yes | Yes |  |
| 35 | Yes | Yes | Yes | Yes | Yes | Yes |
| 36 | Yes | Yes | Yes | Yes | Yes | Yes |
| 37 |  |  | Yes | Yes | Yes | Yes |
| 38 |  |  | Yes | Yes | Yes3 | Yes3 |
| 39 |  |  | Yes | Yes | Yes3 | Yes3 |
| 40 |  |  | Yes | Yes | Yes | Yes |
| 41 |  |  | Yes | Yes | Yes | Yes |
| 42 |  |  | Yes | Yes | Yes | Yes |
| 43 |  |  | Yes | Yes | Yes | Yes |
| 44 |  | Yes | Yes | Yes | Yes | Yes |
| 45 |  |  | Yes | Yes | Yes | Yes |
| 46 |  |  |  | Yes |  | Yes |
| 47 |  |  |  | Yes |  | Yes |
| 48 |  |  | Yes | Yes | Yes | Yes |
| 49 |  |  |  | Yes |  | Yes |
| 50 |  | Yes | Yes | Yes | Yes | Yes |
| 51 |  | Yes | Yes |  |  |  |
| 52 |  |  | Yes | Yes | Yes | Yes |
| 53 | Yes | Yes | Yes | Yes |  |  |
| … |  |  |  |  |  |  |
| 64 | Reserved |  |  |  |  |  |
| 65 | Yes | Yes | Yes | Yes | Yes | Yes |
| 66 | Yes | Yes | Yes | Yes | Yes | Yes |
| 68 |  |  | Yes | Yes | Yes5 |  |
| 70 |  |  | Yes | Yes | Yes | Yes4 |
| 71 |  |  | Yes | Yes1 | Yes1 | Yes1,6 |
| 72 | Yes | Yes1 | Yes1 |  |  |  |
| 73 | Yes | Yes | Yes |  |  |  |
| 74 | Yes | Yes | Yes | Yes1 | Yes1 | Yes1 |
| 85 |  |  | Yes1 | Yes1 |  |  |
| 87 | Yes | Yes1 | Yes1 |  |  |  |
| 88 | Yes | Yes1 | Yes1 |  |  |  |
| NOTE 1: 1 refers to the bandwidth for which a relaxation of the specified UE receiver sensitivity requirement (subclause 7.3) is allowed. NOTE 2: 2 For the 20 MHz bandwidth, the minimum requirements are specified for E-UTRA UL carrier frequencies confined to either 713-723 MHz or 728-738 MHz NOTE 3: 3 refers to the bandwidth for which the uplink transmission bandwidth can be restricted by the network for some channel assignments in FDD/TDD co-existence scenarios in order to meet unwanted emissions requirements (Clause 6.6.3.2). NOTE 4: 4 For the 20 MHz bandwidth, the minimum requirements are restricted to E-UTRA operation when carrier aggregation is configured. NOTE 5: 5 For the 15 MHz bandwidth, the minimum requirements are specified for E-UTRA UL carrier frequencies confined to either 705.5 MHz or 710.5-720.5 MHz NOTE 6: 6 For the 20MHz bandwidth, the minimum requirements are specified for E-UTRA UL carrier frequencies confined to either 673-678 MHz or 683-688MHz. |  |  |  |  |  |  |

b)	The use of different (asymmetrical) channel bandwidth for the TX and RX is not precluded and is intended to form part of a later release.

## 5.6A	Channel bandwidth for CA

For intra-band contiguous carrier aggregation Aggregated Channel Bandwidth, Aggregated Transmission Bandwidth Configuration and Guard Bands are defined as follows, see Figure 5.6A-1.

FC,low

Lower Edge

Higher Edge

Lowest Carrier Transmission Bandwidth Configuration, NRB,low  [RB]

FC,high

Foffset,low

Highest Carrier Transmission Bandwidth Configuration NRB,high [RB]

Resource block

Aggregated Channel Bandwidth, BWchannel_CA [MHz]

Fedge,low

Fedge,high

For each carrier, the center sub carrier (corresponds to DC in baseband) is not transmitted in downlink

Foffset,high

Guard Band

Guard Band

Aggregated Transmission Bandwidth Configuration, NRB_agg [RB]

Figure 5.6A-1. Definition of Aggregated channel bandwidth and aggregated channel bandwidth edges

The aggregated channel bandwidth, BWChannel_CA, is defined as

BWChannel_CA = Fedge,high - Fedge,low [MHz].

The lower bandwidth edge Fedge,low and the upper bandwidth edge Fedge,high of the aggregated channel bandwidth are used as frequency reference points for transmitter and receiver requirements and are defined by

Fedge,low = FC,low - Foffset,low

Fedge,high = FC,high + Foffset,high

The lower and upper frequency offsets depend on the transmission bandwidth configurations of the lowest and highest assigned edge component carrier and are defined as

Foffset,low = (0.18NRB,low  + f1)/2 + BWGB [MHz]

Foffset,high = (0.18NRB,high + f1)/2 + BWGB [MHz]

where f1 = f for the downlink with f the subcarrier spacing and f1 = 0 for the uplink, while NRB,low and NRB,high are the transmission bandwidth configurations according to Table 5.6-1 for the lowest and highest assigned component carrier, respectively. BWGB denotes the Nominal Guard Band and is defined in Table 5.6A-1, and the factor 0.18 is the PRB bandwidth in MHz.

NOTE:	The values of BWChannel_CA for UE and BS are the same if the lowest and the highest component carriers are identical.

Aggregated Transmission Bandwidth Configuration is the number of the aggregated RBs within the fully allocated Aggregated Channel bandwidth and is defined per CA Bandwidth Class (Table 5.6A-1).

For intra-band non-contiguous carrier aggregation Sub-block Bandwidth and Sub-block edges are defined as follows, see Figure 5.6A-2.

Figure 5.6A-2. Non-contiguous intraband CA terms and definitions

The lower sub-block edge of the Sub-block Bandwidth (BWChannel,block) is defined as

Fedge,block, low = FC,block,low - Foffset,block, low.

The upper sub-block edge of the Sub-block Bandwidth is defined as

Fedge,block,high = FC,block,high + Foffset,block,high .

The Sub-block Bandwidth, BWChannel,block, is defined as follows:

BWChannel,block = Fedge,block,high - Fedge,block,low [MHz]

The lower and upper frequency offsets Foffset,block,low and Foffset,block,high depend on the transmission bandwidth configurations of the lowest and highest assigned edge component carriers within a sub-block and are defined as

Foffset,block,low = (0.18NRB,low + f1) /2 + BWGB [MHz]

Foffset,block,high = (0.18NRB,high + f1)/2 + BWGB [MHz]

where f1 = f for the downlink with f the subcarrier spacing and f1 = 0 for the uplink, while NRB,low and NRB,high are the transmission bandwidth configurations according to Table 5.6-1 for the lowest and highest assigned component carrier within a sub-block, respectively. BWGB denotes the Nominal Guard Band and is defined in Table 5.6A-1, and the factor 0.18 is the PRB bandwidth in MHz.

The sub-block gap size between two consecutive sub-blocks Wgap is defined as

Wgap = Fedge,block n+1,low - Fedge,block n,high [MHz]

Table 5.6A-1: CA bandwidth classes and corresponding nominal guard bands

| CA Bandwidth Class | Aggregated Transmission Bandwidth Configuration | Number of contiguous CC | Nominal Guard Band BWGB |
| --- | --- | --- | --- |
| A | NRB,agg ≤ 100 | 1 | a1 BWChannel(1) - 0.5f1 (NOTE 2) |
| B | 25 < NRB,agg ≤ 100 | 2 | 0.05 max(BWChannel(1),BWChannel(2)) - 0.5f1 |
| C | 100 < NRB,agg ≤ 200 | 2 | 0.05 max(BWChannel(1),BWChannel(2)) - 0.5f1 |
| D | 200 < NRB,agg ≤ 300 | 3 | 0.05 max(BWChannel(1),BWChannel(2), BWChannel(3)) - 0.5f1 |
| E | 300 < NRB,agg ≤ 400 | 4 | 0.05 max(BWChannel(1),BWChannel(2), BWChannel(3) , BWChannel(4)) - 0.5f1 |
| F | 400 < NRB,agg ≤ 500 | 5 | 0.05 max(BWChannel(1),BWChannel(2), BWChannel(3) , BWChannel(4) , BWChannel(5)) - 0.5f1 |
| I | 700 < NRB,agg ≤ 800 | 8 | NOTE 3 |
| NOTE 1: BWChannel(j), j = 1, 2, 3, 4 is the channel bandwidth of an E-UTRA component carrier according to Table 5.6-1 and f1 = f for the downlink with f the subcarrier spacing while f1 = 0 for the uplink. NOTE 2: a1 = 0.16/1.4 for BWChannel(1) = 1.4 MHz whereas a1 = 0.05 for all other channel bandwidths. NOTE 3: Applicable for later releases. |  |  |  |

The channel spacing between centre frequencies of contiguously aggregated component carriers is defined in subclause 5.7.1A.

### 5.6A.1	Channel bandwidths per operating band for CA

The requirements for carrier aggregation in this specification are defined for carrier aggregation configurations with associated bandwidth combination sets. For inter-band carrier aggregation, a carrier aggregation configuration is a combination of operating bands, each supporting a carrier aggregation bandwidth class. For intra-band contiguous carrier aggregation, a carrier aggregation configuration is a single operating band supporting a carrier aggregation bandwidth class.

For each carrier aggregation configuration, requirements are specified for all bandwidth combinations contained in a bandwidth combination set, which is indicated per supported band combination in the UE radio access capability. A UE can indicate support of several bandwidth combination sets per band combination.

Requirements for intra-band contiguous carrier aggregation are defined for the carrier aggregation configurations and bandwidth combination sets specified in Table 5.6A.1-1. Requirements for inter-band carrier aggregation are defined for the carrier aggregation configurations and bandwidth combination sets specified in Table 5.6A.1-2, Table 5.6A.1-2a, Table 5.6A.1-2b and Table 5.6A.1-2c. Requirements for intra-band non-contiguous carrier aggregation are defined for the carrier aggregation configurations and bandwidth combination sets specified in Table 5.6A.1-3.

The DL component carrier combinations for a given CA configuration shall be symmetrical in relation to channel centre unless stated otherwise in Table 5.6A.1-1, Table 5.6A.1-2, Table 5.6A.1-2a, Table 5.6A.1-2b and Table 5.6A.1-2c.

Table 5.6A.1-1: E-UTRA CA configurations and bandwidth combination sets defined for intra-band contiguous CA

|  |  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA configuration | Uplink CA configurations (NOTE 3) | Component carriers in order of increasing carrier frequency |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_1C | CA_1C | 15 | 15 |  |  |  | 40 | 0 |
|  |  | 20 | 20 |  |  |  |  |  |
|  |  | 5, 10, 15 | 20 |  |  |  | 40 | 1 |
|  |  | 20 | 5, 10, 15, 20 |  |  |  |  |  |
| CA_2C |  | 5 | 20 |  |  |  | 40 | 0 |
|  |  | 10 | 15, 20 |  |  |  |  |  |
|  |  | 15 | 10, 15, 20 |  |  |  |  |  |
|  |  | 20 | 5, 10, 15, 20 |  |  |  |  |  |
| CA_3B |  | 5 | 3 |  |  |  | 10 | 0 |
|  |  | 3, 5 | 5 |  |  |  |  |  |
| CA_3C | CA_3C | 5, 10, 15 | 20 |  |  |  | 40 | 0 |
|  |  | 20 | 5, 10, 15, 20 |  |  |  |  |  |
| CA_5B | CA_5B | 5, 10 | 10 |  |  |  | 20 | 0 |
|  |  | 10 | 5 |  |  |  |  |  |
| CA_7B |  | 15 | 5 |  |  |  | 20 | 0 |
| CA_7C | CA_7C | 15 | 15 |  |  |  | 40 | 0 |
|  |  | 20 | 20 |  |  |  |  |  |
|  |  | 10 | 20 |  |  |  | 40 | 1 |
|  |  | 15 | 15, 20 |  |  |  |  |  |
|  |  | 20 | 10, 15, 20 |  |  |  |  |  |
|  |  | 15 | 10, 15 |  |  |  | 40 | 2 |
|  |  | 20 | 15, 20 |  |  |  |  |  |
| CA_8B | CA_8B | 5,10 | 10 |  |  |  | 20 | 0 |
|  |  | 10 | 5 |  |  |  |  |  |
| CA_12B | - | 5 | 5, 10 |  |  |  | 15 | 0 |
| CA_23B | - | 10 | 10 |  |  |  | 20 | 0 |
|  |  | 5 | 15 |  |  |  |  |  |
| CA_27B | - | 1.4, 3, 5 | 5 |  |  |  | 13 | 0 |
|  |  | 1.4, 3 | 10 |  |  |  |  |  |
| CA_28C | - | 5 | 20 |  |  |  | 30 | 0 |
|  |  | 10 | 15, 20 |  |  |  |  |  |
|  |  | 15 | 10, 15 |  |  |  |  |  |
|  |  | 20 | 5, 10 |  |  |  |  |  |
| CA_38C | CA_38C | 15 | 15 |  |  |  | 40 | 0 |
|  |  | 20 | 20 |  |  |  |  |  |
| CA_39C | CA_39C | 5,10,15 | 20 |  |  |  | 35 | 0 |
|  |  | 20 | 5, 10, 15 |  |  |  |  |  |
| CA_40C | CA_40C | 10 | 20 |  |  |  | 40 | 0 |
|  |  | 15 | 15 |  |  |  |  |  |
|  |  | 20 | 10, 20 |  |  |  |  |  |
|  |  | 10, 15 | 20 |  |  |  | 40 | 1 |
|  |  | 15 | 15 |  |  |  |  |  |
|  |  | 20 | 10, 15, 20 |  |  |  |  |  |
| CA_40D | CA_40C, CA_40D | 10, 15, 20 | 20 | 20 |  |  | 60 | 0 |
|  |  | 20 | 10, 15 | 20 |  |  |  |  |
|  |  | 20 | 20 | 10, 15 |  |  |  |  |
|  |  | 15, 20 | 15, 20 | 15, 20 |  |  | 60 | 1 |
| CA_40E | - | 15, 20 | 15, 20 | 15, 20 | 20 |  | 80 | 0 |
| CA_40F | - | 15, 20 | 15, 20 | 15, 20 | 20 | 20 | 100 | 0 |
| CA_41C5 | CA_41C | 10 | 20 |  |  |  | 40 | 0 |
|  |  | 15 | 15, 20 |  |  |  |  |  |
|  |  | 20 | 10, 15, 20 |  |  |  |  |  |
|  |  | 5, 10 | 20 |  |  |  | 40 | 1 |
|  |  | 15 | 15, 20 |  |  |  |  |  |
|  |  | 20 | 5, 10, 15, 20 |  |  |  |  |  |
|  |  | 10 | 15, 20 |  |  |  | 40 | 2 |
|  |  | 15 | 10, 15, 20 |  |  |  |  |  |
|  |  | 20 | 10, 15, 20 |  |  |  |  |  |
|  |  | 10 | 20 |  |  |  | 40 | 3 |
|  |  | 20 | 20 |  |  |  |  |  |
| CA_41D | CA_41C, CA_41D | 10 | 20 | 15 |  |  | 60 | 0 |
|  |  | 10 | 15, 20 | 20 |  |  |  |  |
|  |  | 15 | 20 | 10, 15 |  |  |  |  |
|  |  | 15 | 10, 15, 20 | 20 |  |  |  |  |
|  |  | 20 | 15, 20 | 10 |  |  |  |  |
|  |  | 20 | 10, 15, 20 | 15, 20 |  |  |  |  |
| CA_41E | CA_41C, CA_41D | 15, 20 | 15, 20 | 15, 20 | 20 |  | 80 | 0 |
| CA_41F | CA_41C, CA_41D | 10,15, 20 | 15, 20 | 20 | 20 | 20 | 100 | 0 |
| CA_42C5 | CA_42C | 5, 10, 15, 20 | 20 |  |  |  | 40 | 0 |
|  |  | 20 | 5, 10, 15 |  |  |  |  |  |
|  |  | 10, 15, 20 | 20 |  |  |  | 40 | 1 |
|  |  | 20 | 10, 15 |  |  |  |  |  |
| CA_42D | CA_42C | 5,10,15,20 | 20 | 20 |  |  | 60 | 0 |
|  |  | 20 | 20 | 5,10,15 |  |  |  |  |
|  |  | 10, 15, 20 | 20 | 20 |  |  | 60 | 1 |
|  |  | 20 | 20 | 10, 15 |  |  |  |  |
| CA_42E | CA_42C | 5,10,15,20 | 20 | 20 | 20 |  | 80 | 0 |
|  |  | 20 | 20 | 20 | 5,10,15 |  |  |  |
| CA_42F | CA_42C | 5, 10, 15, 20 | 20 | 20 | 20 | 20 | 100 | 0 |
|  |  | 20 | 20 | 20 | 20 | 5, 10, 15, 20 |  |  |
| CA_43C | - | 5 | 20 |  |  |  | 40 | 0 |
|  |  | 10 | 15, 20 |  |  |  |  |  |
|  |  | 15 | 10, 15, 20 |  |  |  |  |  |
|  |  | 20 | 5, 10, 15, 20 |  |  |  |  |  |
| CA_46C 4 | - | 20 | 20 |  |  |  | 40 | 0 |
|  |  | 20 | 10, 20 |  |  |  | 40 | 1 |
|  |  | 10, 20 | 20 |  |  |  |  |  |
| CA_46D 4 | - | 20 | 20 | 20 |  |  | 60 | 0 |
|  |  | 20 | 20 | 10, 20 |  |  | 60 | 1 |
|  |  | 10, 20 | 20 | 20 |  |  |  |  |
| CA_46E 4 | - | 20 | 20 | 20 | 20 |  | 80 | 0 |
|  |  | 20 | 20 | 20 | 10, 20 |  | 80 | 1 |
|  |  | 10 | 20 | 20 | 20 |  |  |  |
| CA_48B | CA_48B | 10 | 10 |  |  |  | 20 | 0 |
| CA_48C | CA_48C | 5, 10, 15, 20 | 20 |  |  |  | 40 | 0 |
|  |  | 20 | 5, 10, 15 |  |  |  |  |  |
| CA_48D | CA_48C | 5,10,15,20 | 20 | 20 |  |  | 60 | 0 |
|  |  | 20 | 20 | 5,10,15 |  |  |  |  |
| CA_48E | CA_48C | 5,10,15,20 | 20 | 20 | 20 |  | 80 | 0 |
|  |  | 20 | 20 | 20 | 5,10,15 |  |  |  |
| CA_48F | - | 5, 10, 15, 20 | 20 | 20 | 20 | 20 | 100 | 0 |
|  |  | 20 | 20 | 20 | 20 | 5, 10, 15, 20 |  |  |
| CA_66B | CA_66B | 5 | 5, 10, 15 |  |  |  | 20 | 0 |
|  |  | 10 | 5, 10 |  |  |  |  |  |
|  |  | 15 | 5 |  |  |  |  |  |
| CA_66C | CA_66C | 5 | 20 |  |  |  | 40 | 0 |
|  |  | 10 | 15, 20 |  |  |  |  |  |
|  |  | 15 | 10, 15, 20 |  |  |  |  |  |
|  |  | 20 | 5, 10, 15, 20 |  |  |  |  |  |
| CA_66D | - | 5 | 20 | 20 |  |  | 60 | 0 |
|  |  | 20 | 5 | 20 |  |  |  |  |
|  |  | 20 | 20 | 5 |  |  |  |  |
|  |  | 10 | 20 | 15 |  |  |  |  |
|  |  | 15 | 20 | 10 |  |  |  |  |
|  |  | 10, 15, 20 | 15, 20 | 20 |  |  |  |  |
|  |  | 15, 20 | 10 | 20 |  |  |  |  |
|  |  | 15 | 15, 20 | 15 |  |  |  |  |
|  |  | 20 | 15, 20 | 10, 15 |  |  |  |  |
|  |  | 20 | 10 | 15 |  |  |  |  |
| CA_70C | - | 5 | 20 |  |  |  | 25 | 0 |
|  |  | 10 | 15 |  |  |  |  |  |
|  |  | 15 | 10 |  |  |  |  |  |
| NOTE 1: The CA configuration refers to an operating band and a CA bandwidth class specified in Table 5.6A-1 (the indexing letter). Absence of a CA bandwidth class for an operating band implies support of all classes. NOTE 2: For the supported CC bandwidth combinations, the CC downlink and uplink bandwidths are equal. NOTE 3: Uplink CA configurations are the configurations supported by the present release of specifications. NOTE 4: Restricted to E-UTRA operation when inter-band carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell. NOTE 5: 8Rx Requirements are applicable for this band configuration if UE supports 8Rx. |  |  |  |  |  |  |  |  |

Table 5.6A.1-2: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA (two bands)

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations (NOTE 4) | E-UTRA Bands | 1.4 MHz |  | 3 MHz |  |  |  | 5 MHz |  |  |  | 10 MHz |  |  |  |  |  |  | 15 MHz |  |  |  |  |  | 20 MHz |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3A | CA_1A-3A | 1 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 3 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 1 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 1 |
|  |  | 3 |  |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_1A-1A-3A | - | 1 | See CA_1A-1A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 3 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_1A-1A-7A | CA_1A-7A | 1 | See CA_1A-1A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 7 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_1A-1A-7C | CA_7C | 1 | See CA_1A-1A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 7 | See CA_7C in Table 5.6A.1-1 of 36.101 Bandwidth combination set 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-3A | CA_1A-3A | 1 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-1A-3A-3A | - | 1 | See CA_1A-1A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3C | CA_1A-3A, CA_3C | 1 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-1A-3C | CA_3C | 1 | See CA_1A-1A Bandwidth Combination Set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-5A | CA_1A-5A | 1 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 0 |
|  |  | 5 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 1 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 30 | 1 |
|  |  | 5 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-1A-5A | - | 1 | See CA_1A-1A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 5 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1C-5A | - | 1 | See CA_1C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 5 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-7A | CA_1A-7A | 1 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 7 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 1 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 1 |
|  |  | 7 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_1A-7A-7A | CA_1A-7A | 1 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 3 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CA_1A-7A | 1 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  | Yes |  | 60 | 1 |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-7C | CA_1A-7A, CA_7C | 1 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | CA_1A-7A, CA_7C | 1 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 1 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-8A | CA_1A-8A | 1 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 30 | 0 |
|  |  | 8 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 1 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 1 |
|  |  | 8 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 1 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 30 | 2 |
|  |  | 8 |  |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-11A | CA_1A-11A | 1 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 30 | 0 |
|  |  | 11 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-18A | CA_1A-18A | 1 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 35 | 0 |
|  |  | 18 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 1 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 1 |
|  |  | 18 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-19A | CA_1A-19A | 1 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 35 | 0 |
|  |  | 19 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-20A | CA_1A-20A | 1 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 20 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_1A-21A | CA_1A-21A | 1 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 35 | 0 |
|  |  | 21 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-26A | CA_1A-26A | 1 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 35 | 0 |
|  |  | 26 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 1 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 1 |
|  |  | 26 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-28A | CA_1A-28A | 1 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 28 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 1 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 1 |
|  |  | 28 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-1A-28A | - | 1 | See CA_1A-1A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 28 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_1A-32A | - | 1 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 32 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_1A-38A | - | 1 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 38 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_1A-40A | - | 1 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 40 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_1A-40C | - | 1 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 40 | See CA_40C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-41A | CA_1A-41A | 1 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 1 |
|  |  | 41 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_1A-41A8 | - | 1 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 41 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_1A-41C8 | CA_1A-41A CA_1A-41C | 1 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 41 | See CA_41C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-41D8 | - | 1 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 41 | See CA_41D Bandwidth combination set 0 at Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-42A | CA_1A-42A | 1 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 42 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_1A-42A-42A | CA_1A-42A | 1 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 42 | See CA_42A-42A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-42C | CA_1A-42A, CA_1A-42C, CA_42C | 1 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-42A-42C | CA_1A-42A | 1 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 42 | See CA_42A-42C Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-42C-42C | CA_1A-42A | 1 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 100 | 0 |
|  |  | 42 | See CA_42C-42C Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-42D | CA_1A-42A | 1 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 42 | See CA_42D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-42E | CA_1A-42A | 1 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 100 | 0 |
|  |  | 42 | See CA_42E Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-43A | - | 1 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 35 | 0 |
|  |  | 43 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_1A-46A | - | 1 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 1 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 1 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_1A-46C | - | 1 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | - | 1 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 1 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-46D | - | 1 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 1 |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 80 | 1 |
|  |  | 46 | See CA_46D Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-46E | - | 1 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 100 | 0 |
|  |  | 46 | See CA_46E Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 1 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 100 | 1 |
|  |  | 46 | See CA_46E Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1C-3A | - | 1 | See CA_1C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 3 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_2A-4A | CA_2A-4A | 2 | Yes |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 4 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 1 |
|  |  | 4 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 2 |
|  |  | 4 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_2A-2A-4A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 4 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_2A-4A-4A | - | 2 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 4 | See CA_4A-4A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-4A-4A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 4 | See CA_4A-4A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-5A | CA_2A-5A | 2 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 30 | 0 |
|  |  | 5 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 1 |
|  |  | 5 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-5A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 5 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-46D | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 46 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2C-5A | - | 2 | See CA_2C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 5 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-5B | CA_2A-5A | 2 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-5B | - | 2 | See CA_2A-2A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2C-5B | - | 2 | See CA_2C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-7A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 7 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_2A-7A | CA_2A-7A | 2 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 7 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_2A-7A-7A | - | 2 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 7 | See the CA_7A-7A Bandwidth combination set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-7C | - | 2 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 7 | See the CA_7C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-12A | CA_2A-12A | 2 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 30 | 0 |
|  |  | 12 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 30 | 1 |
|  |  | 12 |  |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 2 |
|  |  | 12 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-12A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 12 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-12A-12A | - | 2 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 30 | 0 |
|  |  | 12 | See CA_12A-12A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-12A-12A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 12 | See CA_12A-12A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-12B | CA_2A-12A | 2 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 35 | 0 |
|  |  | 12 | See CA_12B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-12B | - | 2 | See CA_2A-2A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 55 | 0 |
|  |  | 12 | See CA_12B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2C-12A | - | 2 | See CA_2C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 12 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-13A | CA_2A-13A | 2 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 30 | 0 |
|  |  | 13 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 1 |
|  |  | 13 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 30 | 2 |
|  |  | 13 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-13A | CA_2A-13A | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 13 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-14A | CA_2A-14A | 2 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 30 | 0 |
|  |  | 14 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-14A | CA_2A-14A | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 14 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-17A | - | 2 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 0 |
|  |  | 17 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-26A | - | 2 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 35 | 0 |
|  |  | 26 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-28A | - | 2 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 28 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_2A-29A | - | 2 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 0 |
|  |  | 29 |  |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 1 |
|  |  | 29 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 30 | 2 |
|  |  | 29 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-29A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 29 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2C-29A | - | 2 | See CA_2C Bandwidth Combination Set 0 in table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 29 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-30A | CA_2A-30A | 2 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 30 | 0 |
|  |  | 30 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-30A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 30 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2C-30A | - | 2 | See CA_2C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 30 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-46A | CA_2A-46A | 2 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_2A-2A-46A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_2A-46A-46C | - | 2 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 46 | See CA_46A-46C Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-46C | - | 2 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-46C | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-46D | - | 2 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 46 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-46E | - | 2 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 100 | 0 |
|  |  | 46 | See CA_46E Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-46A-46A | - | 2 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 46 | See CA_46A-46A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-46A-46D | - | 2 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 100 | 0 |
|  |  | 46 | See CA_46A-46D Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-48A | CA_2A-48A | 2 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 48 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_2A-48A-48A | CA_2A-48A | 2 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 48 | See CA_48A-48A Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-48C | CA_2A-48A, CA_48C | 2 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-48A-48C | CA_2A-48A | 2 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 48 | See the CA_48A-48C Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-48A-48D | CA_2A-48A | 2 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 100 | 0 |
|  |  | 48 | See CA_48A-48D Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-48C-48C | CA_2A-48A | 2 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  | 100 | 0 |
|  |  | 48 | See CA_48C-48C Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-48D | CA_2A-48A | 2 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 48 | See the CA_48D Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-48E | CA_2A-48A | 2 |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  | Yes |  | 100 | 0 |
|  |  | 48 | See CA_48E Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-49A | CA_2A-49A | 2 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 49 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_2A-66A | CA_2A-66A | 2 | Yes |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 66 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 1 |
|  |  | 66 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 2 |
|  |  | 66 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_2A-66B | CA_66B | 2 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 66 | See CA_66B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-66C | - | 2 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 66 | See CA_66C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-66D | - | 2 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 66 | See CA_66D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-66A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 66 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_2A-2A-66A-66A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-66A-66B | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 66 | See CA_66A-66B Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-66A-66C | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 66 | See CA_66A-66C Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-66A-66A | - | 2 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-66A-66A-66A | - | 2 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 66 | See CA_66A-66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-66A-66B | CA_66B | 2 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 66 | See CA_66A-66B Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-66A-66C |  | 2 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 66 | See CA_66A-66C Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-66B | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 66 | See CA_66B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-66C | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 66 | See CA_66C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-66D |  | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 66 | See CA_66D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2C-66A | - | 2 | See CA_2C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 66 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_2C-66A-66A |  | 2 | See CA_2C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-71A | - | 2 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 71 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 2 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 1 |
|  |  | 71 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-71A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 71 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_3A-5A | CA_3A-5A | 3 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 30 | 0 |
|  |  | 5 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 1 |
|  |  | 5 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 30 | 2 |
|  |  | 5 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 30 | 3 |
|  |  | 5 |  |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 4 |
|  |  | 5 |  |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-3A-5A | - | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 5 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3C-5A | - | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 5 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-7A | CA_3A-7A | 3 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 7 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 1 |
|  |  | 7 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_3A-3A-7A | CA_3A-7A | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 7 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 3 | See CA_3A-3A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 1 |
|  |  | 7 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_3A-3A-7A-7A | CA_3A-7A | 3 | See CA_3A-3A Bandwidth Combination Set 0 in table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 1 in table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 | See CA_3A-3A Bandwidth Combination Set 1 in table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 1 |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 2 in table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-3A-7C | 7C | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 7 | See CA_7C in Table 5.6A.1-1 of 36.101 Bandwidth combination set 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-3A-42D | CA_3A-42A | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 42 | See CA_42D Bandwidth Combination Set 0 in Table 5.6A.1-1: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-7A-7A | CA_3A-7A | 3 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 7 | See CA_7A-7A Bandwidth combination set 1 in table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 50 | 1 |
|  |  | 7 | See CA_7A-7A Bandwidth combination set 2 in table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-7B | - | 3 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 7 | See CA_7B bandwidth combination set 0 in table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-7C | CA_3A-7A CA_7C | 3 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 7 | See CA_7C Bandwidth combination set 1 in table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 1 |
|  |  | 7 | See CA_7C Bandwidth combination set 2 in table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3C-7A | CA_3A-7A CA_3C | 3 | See CA_3C Bandwidth Combination Set 0 in table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 7 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_3C-7C | CA_3A-7A, CA_3C, CA_7C | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 1 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-8A | CA_3A-8A | 3 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 30 | 0 |
|  |  | 8 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 1 |
|  |  | 8 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 30 | 2 |
|  |  | 8 |  |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 30 | 3 |
|  |  | 8 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-3A-8A | CA_3A-8A | 3 | See CA_3A-3A Bandwidth Combination Set 0 in table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 8 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 | See CA_3A-3A Bandwidth Combination Set 1 in table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 40 | 1 |
|  |  | 8 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3C-8A | CA_3A-8A, CA_3C | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 8 |  |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-11A | CA_3A-11A | 3 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 30 | 0 |
|  |  | 11 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-18A | CA_3A-18A | 3 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 35 | 0 |
|  |  | 18 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-19A | CA_3A-19A | 3 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 35 | 0 |
|  |  | 19 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-3A-19A | CA_3A-19A | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 55 | 0 |
|  |  | 19 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-20A | CA_3A-20A | 3 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 30 | 0 |
|  |  | 20 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 1 |
|  |  | 20 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_3A-3A-20A | - | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 20 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_3C-20A | - | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 20 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_3A-21A | CA_3A-21A | 3 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 35 | 0 |
|  |  | 21 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-3A-21A | CA_3A-21A | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 55 | 0 |
|  |  | 21 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-26A | CA_3A-26A | 3 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 35 | 0 |
|  |  | 26 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 1 |
|  |  | 26 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-27A | - | 3 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 30 | 0 |
|  |  | 27 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-28A | CA_3A-28A | 3 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 28 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 3 |  |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 1 |
|  |  | 28 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_3A-3A-28A | - | 3 | See CA_3A-3A Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 28 |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |
| CA_3C-28A | CA_3C | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 28 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_3A-31A | - | 3 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 25 | 0 |
|  |  | 31 |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-32A | - | 3 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 32 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_3C-32A | - | 3 | See the CA_3C Bandwidth combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 32 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_3A-38A | - | 3 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 38 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_3C-38A | - | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 38 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_3A-40A | CA_3A-40A | 3 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 40 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 3 | Yes |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 1 |
|  |  | 40 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_3A-40A-40A | - | 3 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 40 | See CA_40A-40A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-40C | - | 3 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 40 | See CA_40C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-40D | - | 3 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 40 | See CA_40D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-40E | - | 3 |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 40 | See CA_40E Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3C-40A | - | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 40 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_3C-40C | - | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 40 | See CA_40C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-41A | CA_3A-41A | 3 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 41 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 3 |  |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 1 |
|  |  | 41 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_3A-3A-41A | - | 3 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 3 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_3A-41C | CA_3A-41A, CA_3A-41C, CA_41C | 3 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 41 | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-41D | CA_3A-41A, CA_41C | 3 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 41 | See CA_41D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3C-41A | - | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 41 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_3C-41C | - | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 41 | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3C-41D | - | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 41 | See CA_41D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-42A | CA_3A-42A | 3 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 42 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_3A-3A-42A | CA_3A-42A | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 42 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_3A-42C | CA_3A-42A, CA_42C CA_3A-42C | 3 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-42D | CA_3A-42A | 3 |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  | 80 | 0 |
|  |  | 42 | See CA_42D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-3A-42C | CA_3A-42A | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-42A-42A | CA_3A-42A | 3 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  | Yes |  | 60 | 0 |
|  |  | 42 | See CA_42A-42A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-42A-42C | CA_3A-42A, CA_42C | 3 |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  | 80 | 0 |
|  |  | 42 | See CA_42A-42C Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-42C-42C | CA_3A-42A, CA_42C | 3 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 100 | 0 |
|  |  | 42 | See CA_42C-42C Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-42E | CA_3A-42A | 3 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 100 | 0 |
|  |  | 42 | See CA_42E Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-43A | - | 3 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 35 | 0 |
|  |  | 43 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_3A-46A | - | 3 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 3 |  |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 1 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_3A-46C | - | 3 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | - | 3 |  |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 1 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-46D | - | 3 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 80 | 1 |
|  |  | 46 | See CA_46D Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-46E | - | 3 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 100 | 0 |
|  |  | 46 | See CA_46E Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 100 | 1 |
|  |  | 46 | See CA_46E Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-3A-46A |  | 3 | See CA_3A-3A Bandwidth Combination Set 0 in table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_3A-3A-46C | - | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3C-46A | - | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_3C-46C | - | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 46 | See CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3C-46D | - | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-69A | - | 3 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 69 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_4A-5A | CA_4A-5A | 4 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 0 |
|  |  | 5 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 4 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 30 | 1 |
|  |  | 5 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-4A-5A | - | 4 | See CA_4A-4A Bandwidth Combination Set 0 in table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 5 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-5B | CA_5B | 4 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-4A-5B | CA_4A-5A, CA_5B | 4 | See CA_4A-4A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-7A | CA_4A-7A | 4 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 7 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 4 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 1 |
|  |  | 7 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_4A-4A-7A | - | 4 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 40 | 0 |
|  |  | 4 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 4 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 1 |
|  |  | 4 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 7 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_4A-7A-7A | - | 4 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 7 | See the CA_7A-7A Bandwidth combination set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-7C | CA_4A-7A | 4 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-12A | CA_4A-12A | 4 | Yes |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 0 |
|  |  | 12 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 4 | Yes |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 30 | 1 |
|  |  | 12 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 4 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 30 | 2 |
|  |  | 12 |  |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 4 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 3 |
|  |  | 12 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 4 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 30 | 4 |
|  |  | 12 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 4 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 20 | 5 |
|  |  | 12 |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-4A-12A | - | 4 | See CA_4A-4A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 12 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-12A-12A | - | 4 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 30 | 0 |
|  |  | 12 | See CA_12A-12A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-4A-12A-12A | - | 4 | See CA_4A-4A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 12 | See CA_12A-12A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-4A-12B | - | 4 | See CA_4A-4A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 55 | 0 |
|  |  | 12 | See CA_12B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-12B | CA_4A-12A | 4 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 35 | 0 |
|  |  | 12 | See CA_12B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-13A | CA_4A-13A | 4 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 30 | 0 |
|  |  | 13 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 4 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 1 |
|  |  | 13 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-4A-13A | - | 4 | See CA_4A-4A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 13 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-17A | CA_4A-17A | 4 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 0 |
|  |  | 17 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-27A | - | 4 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 30 | 0 |
|  |  | 27 |  |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-28A | CA_4A-28A | 4 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 28 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_4A-29A | - | 4 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 0 |
|  |  | 29 |  |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 4 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 1 |
|  |  | 29 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 4 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 30 | 2 |
|  |  | 29 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-4A-29A | - | 4 | See CA_4A-4A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 29 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-30A | - | 4 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 30 | 0 |
|  |  | 30 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-4A-30A | - | 4 | See CA_4A-4A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 30 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-46A | - | 4 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_4A-46A-46A | - | 4 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 46 | See CA_46A-46A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-46A-46C | - | 4 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 46 | See CA_46A-46C Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-46C | - | 4 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-46D | - | 4 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-46A-46D | - | 4 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 100 | 0 |
|  |  | 46 | See CA_46A-46D Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-48A | - | 4 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 48 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_4A-48C | - | 4 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-48D | - | 4 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 48 | See CA_48D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-48E | - | 4 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  | Yes |  | 100 | 0 |
|  |  | 48 | See CA_48E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-71A | - | 4 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 71 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_4A-4A-71A | - | 4 | See CA_4A-4A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 71 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_5A-7A | CA_5A-7A | 5 | Yes |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 7 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 5 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 1 |
|  |  | 7 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_5A-7A-7A | CA_5A-7A | 5 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 3 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-7C | - | 5 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-12A | CA_5A-12A | 5 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 0 |
|  |  | 12 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-12A-12A | - | 5 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 0 |
|  |  | 12 | See CA_12A-12A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-12B | - | 5 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 25 | 0 |
|  |  | 12 | See CA_12B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-13A | - | 5 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 0 |
|  |  | 13 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-17A | CA_5A-17A | 5 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 0 |
|  |  | 17 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-25A | - | 5 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 25 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_5A-28A | - | 5 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 28 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_5A-29A | - | 5 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 0 |
|  |  | 29 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-30A | CA_5A-30A | 5 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 0 |
|  |  | 30 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5B-30A | - | 5 | See CA_5B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 30 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-38A | - | 5 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 38 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_5A-40A | CA_5A-40A | 5 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 40 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 5 |  |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 1 |
|  |  | 40 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_5A-5A-40A | - | 5 | See CA_5A-5A Bandwidth Combination Set 0 in table 6.140.2-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 40 | 0 |
|  |  | 40 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_5A-40A-40A | - | 5 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 40 | See CA_40A-40A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-40C | - | 5 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 40 | See CA_40C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 5 |  |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 1 |
|  |  | 40 | See CA_40C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-41A | - | 5 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_5A-46A | - | 5 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 5 |  |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 1 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_5A-46C | - | 5 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | - | 5 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 1 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-46D | - | 5 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 5 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 1 |
|  |  | 46 | See CA_46D Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-46E | - | 5 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 46 | See CA_46E of Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 5 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 90 | 1 |
|  |  | 46 | See CA_46E of Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5B-46A | - | 5 | See CA_5B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 40 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_5B-46C | - | 5 | See CA_5B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 46 | See CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5B-46D | - | 5 | See CA_5B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5B-46E | - | 5 | See CA_5B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 46 | See CA_46E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-48A | - | 5 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 48 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_5A-48C | - | 5 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-48D | - | 5 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 48 | See CA_48D Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-66A | CA_5A-66A | 5 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 66 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_5A-5A-66A | CA_5A-66A | 5 | See CA_5A-5A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 40 | 0 |
|  |  | 66 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_5A-5A-66A-66A | CA_5A-66A | 5 | See CA_5A-5A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-5A-66A-66B | CA_5A-66A, CA_66B | 5 | See CA_5A-5A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 66 | See CA_66A-66B Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-5A-66A-66C | CA_5A-66A | 5 | See CA_5A-5A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 66 | See CA_66A-66C Bandwidth Combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-5A-66B | CA_5A-66A, CA_66B | 5 | See CA_5A-5A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 40 | 0 |
|  |  | 66 | See CA_66B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-5A-66C | CA_5A-66A | 5 | See CA_5A-5A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 66 | See CA_66C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-5A-66D | CA_5A-66A | 5 | See CA_5A-5A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 66 | See CA_66D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-66A-66A | CA_5A-66A | 5 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-66A-66C | CA_5A-66A | 5 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 66 | See CA_66A-66C Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-66B | CA_66B | 5 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 66 | See CA_66B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-66C | - | 5 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 66 | See CA_66C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-66D |  | 5 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 66 | See CA_66D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5B-66A | CA_5B | 5 | See CA_5B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 40 | 0 |
|  |  | 66 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_5B-66A-66A |  | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-66A-66B | CA_66B | 5 |  |  |  |  |  |  |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 66 | See CA_66A-66B Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5B-66A-66B | - | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 66 | See CA_66A-66B Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5B-66A-66C | - | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 66 | See CA_66A-66C Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5B-66B | CA_5B, CA_66B | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 40 | 0 |
|  |  | 66 | See CA_66B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5B-66C |  | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 66 | See CA_66C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7A-8A | CA_7A-8A | 7 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 30 | 0 |
|  |  | 8 |  |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 30 | 1 |
|  |  | 8 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 30 | 2 |
|  |  | 8 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7A-7A-8A | CA_7A-8A | 7 | See CA_7A-7A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 8 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 2 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 40 | 1 |
|  |  | 8 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7A-12A | - | 7 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 30 | 0 |
|  |  | 12 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7A-12B | - | 7 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 35 | 0 |
|  |  | 12 | See CA_12B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7A-13A | - | 7 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 30 | 0 |
|  |  | 13 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7C-13A | - | 7 | See CA_7C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 13 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7A-7A-13A | - | 7 | See CA_7A-7A Bandwidth combination set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 13 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7A-20A | CA_7A-20A | 7 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 30 | 0 |
|  |  | 20 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 1 |
|  |  | 20 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 7 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 2 |
|  |  | 20 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_7A-7A-20A | - | 7 | See CA_7A-7A Bandwidth Combination Set 3 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 20 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_7C-20A | - | 7 | See CA_7C Bandwidth Combination Set 1 in table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 20 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_7A-22A | - | 7 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 22 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_7A-26A | CA_7A-26A | 7 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 35 | 0 |
|  |  | 26 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| CA_7A-7A-26A | CA_7A-26A | 7 | See CA_7A-7A bandwidth combination set 3 in table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 55 | 0 |
|  |  | 26 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| CA_7A-28A | CA_7A-28A | 7 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 35 | 0 |
|  |  | 28 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 1 |
|  |  | 28 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_7A-7A-28A | - | 7 | See CA_7A-7A Bandwidth combination set 3 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 28 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_7B-28A | - | 7 | See CA_7B bandwidth combination set 0 in table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 40 | 0 |
|  |  | 28 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_7C-28A | CA_7A-28A CA_7C | 7 | See CA_7C bandwidth combination set 2 in table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 28 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 7 | See CA_7C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 1 |
|  |  | 28 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_7A-29A | - | 7 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 30 | 0 |
|  |  | 29 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7A-7A-29A | - | 7 | See CA_7A-7A Bandwidth combination set 1 in table 5.6A.1-3 of 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 29 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7C-29A | - | 7 | See CA_7C Bandwidth combination set 2 in table 5.6A.1-1 of 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 29 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7A-30A | - | 7 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 30 | 0 |
|  |  | 30 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7A-32A | - | 7 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 32 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_7A-40A | - | 7 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 40 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_7A-40C | - | 7 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 40 | See CA_40C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7A-40D | - | 7 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 40 | See CA_40D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7A-40E | - | 7 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 100 | 0 |
|  |  | 40 | See CA_40E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7A-42A | - | 7 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 42 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_7A-42A-42A | - | 7 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 42 | See CA_42A-42A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7A-46A | - | 7 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 7 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 1 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_7A-7A-46C | - | 7 | See CA_7A-7A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7A-46C | - | 7 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | - | 7 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 1 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7A-46D | - | 7 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 46 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 80 | 1 |
|  |  | 46 | See CA_46D Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7A-46E | - | 7 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 100 | 0 |
|  |  | 46 | See CA_46E Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7A-7A-46E | - | 7 | See CA_7A-7A Bandwidth combination set 1 in table 5.6A.1-3 of 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 120 | 0 |
|  |  | 46 | See CA_46E Bandwidth combination set 0 in table 5.6A.1-3 of 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7C-46C | - | 7 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7C-46D | - | 7 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 46 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7C-46E | - | 7 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 120 | 0 |
|  |  | 46 | See CA_46E Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7A-7A-46A | - | 7 | See CA_7A-7A Bandwidth Combination Set 1 in table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_7A-7A-46D | - | 7 | See CA_7A-7A Bandwidth Combination Set 1 in table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7A-66A | - | 7 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 66 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_7A-7A-66A-66A | - | 7 | See CA_7A-7A Bandwidth combination set 1 in table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7C-66A | - | 7 | See CA_7C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 66 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_7C-46A | - | 7 | See CA_7C Bandwidth Combination set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_7A-7A-66A | - | 7 | See CA_7A-7A Bandwidth combination set 1 in table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 66 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_7A-66A-66A | - | 7 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7C-66A-66A | - | 7 | See CA_7C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_8A-11A | - | 8 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 0 |
|  |  | 11 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_8A-20A | - | 8 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 0 |
|  |  | 20 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 8 |  |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 1 |
|  |  | 20 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 8 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 2 |
|  |  | 20 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_8A-27A | - | 8 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 0 |
|  |  | 27 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_8A-28A | - | 8 |  |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 28 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_8A-32A | - | 8 |  |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 32 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_8A-38A | - | 8 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 38 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_8A-39A | CA_8A-39A | 8 | Yes |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 39 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_8A-39C | - | 8 | Yes |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 45 | 0 |
|  |  | 39 | See CA_39C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_8B-39A | - | 8 | See CA_8B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 40 | 0 |
|  |  | 39 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_8B-39C | - | 8 | See CA_8B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 55 | 0 |
|  |  | 39 | See CA_39C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_8A-40A | - | 8 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 40 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
|  | - | 8 |  |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 1 |
|  |  | 40 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_8A-40C | - | 8 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 40 | See CA_40C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_8A-41A | CA_8A-41A | 8 | Yes |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 8 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 1 |
|  |  | 41 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_8A-41C | - | 8 | Yes |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 41 | See CA_41C bandwidth combination set 3 in table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_8A-41D | - | 8 | Yes |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 41 | See CA_41D bandwidth combination set 0 in table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_8B-41A | - | 8 | See CA_8B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 40 | 0 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_8B-41C | - | 8 | See CA_8B bandwidth combination set 0 in table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 41 | See CA_41C bandwidth combination set 3 in table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_8B-41D | - | 8 | See CA_8B bandwidth combination set 0 in table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 41 | See CA_41D bandwidth combination set 0 in table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_8A-42A | - | 8 | Yes |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 42 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_8A-42C | - | 8 | Yes |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_8A-46A | - | 8 | Yes |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_8A-46C | - | 8 | Yes |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_8A-46D | - | 8 | Yes |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 46 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_8A-46E | - | 8 | Yes |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 46 | See CA_46E Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_8B-46A | - | 8 | See CA_8B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 40 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_8B-46C | - | 8 | See CA_8B bandwidth combination set 0 in table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_8B-46D | - | 8 | See CA_8B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 46 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_11A-18A | CA_11A-18A | 11 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 25 | 0 |
|  |  | 18 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| CA_11A-26A | CA_11A-26A | 11 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 25 | 0 |
|  |  | 26 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| CA_11A-28A | - | 11 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 28 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_11A-41A | - | 11 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 41 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_11A-41C | - | 11 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 41 | See CA_41C bandwidth combination set 0 in table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_11A-42A | - | 11 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 42 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_11A-42C | - | 11 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_11A-46A | - | 11 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_11A-46C | - | 11 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_11A-46D | - | 11 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 46 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_11A-46E | - | 11 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 46 | See CA_46E Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_12A-25A | - | 12 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 25 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_12A-30A | CA_12A-30A | 12 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 0 |
|  |  | 30 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_12A-46A | - | 12 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_12A-48A |  | 12 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 48 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_12A-46C | - | 12 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_12A-46D | - | 12 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 46 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_12A-46E | - | 12 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 46 | See CA_46E Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_12A-48C | - | 12 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_12A-48D | - | 12 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 48 | See CA_48D Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_12A-48E | - | 12 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 48 | See CA_48E Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_12A-66A | CA_12A-66A | 12 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 0 |
|  |  | 66 | Yes |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 12 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 1 |
|  |  | 66 | Yes |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 12 |  |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 2 |
|  |  | 66 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 12 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 3 |
|  |  | 66 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 12 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 4 |
|  |  | 66 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 12 |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 5 |
|  |  | 66 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| CA_12A-66A-66A | - | 12 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_12A-66C | - | 12 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 66 | See CA_66C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_12B-66A | - | 12 | See CA_12B bandwidth combination set 0 in table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 35 | 0 |
|  |  | 66 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_12B-66A-66A | - | 12 | See CA_12B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 55 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_13A-46A | - | 13 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_13A-46A-46A | - | 13 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 46 | See CA_46A-46A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_13A-46A-46C | - | 13 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 46 | See CA_46A-46C Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_13A-46A-46D | - | 13 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 46 | See CA_46A-46D Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_13A-46C | - | 13 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_13A-46D | - | 13 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_13A-46E | - | 13 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 46 | See CA_46E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_13A-48A | - | 13 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 48 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_13A-48A-48A | - | 13 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 48 | See CA_48A-48A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_13A-48A-48C | - | 13 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 48 | See the CA_48A-48C Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_13A-48A-48D | - | 13 |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 48 | See CA_48A-48D Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_13A-48C-48C | - | 13 |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 48 | See CA_48C-48C Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_13A-48C | - | 13 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_13A-48D | - | 13 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 48 | See the CA_48D Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_13A-48E | - | 13 |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 48 | See CA_48E Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_13A-66A | CA_13A-66A | 13 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 66 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_13A-66A-66A | CA_13A-66A | 13 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_13A-66A-66B | - | 13 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 66 | See CA_66A-66B Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_13A-66A-66C | - | 13 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 66 | See CA_66A-66C Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_13A-66B | CA_13A-66A | 13 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 66 | See CA_66B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_13A-66C | CA_13A-66A | 13 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 66 | See CA_66C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_13A-66D | - | 13 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 66 | See CA_66D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_14A-30A | CA_14A-30A | 14 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 0 |
|  |  | 30 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_14A-66A | CA_14A-66A | 14 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 66 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_14A-66A-66A | CA_14A-66A | 14 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_14A-66A-66A-66A | CA_14A-66A | 14 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 66 | See CA_66A-66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_18A-28A | CA_18A-28A | 18 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 25 | 0 |
|  |  | 28 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_18A-41A | - | 18 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 35 | 0 |
|  |  | 41 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_18C-41C | CA_18C-41C | 18 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 55 | 0 |
|  |  | 41 | See CA_41C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_18A-41A | CA_18A-41A | 18 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 35 | 0 |
|  |  | 41 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_18A-41C | CA_18A-41A CA_18A-41C | 18 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 55 | 0 |
|  |  | 41 | See CA_41C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_18A-42C | - | 18 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 55 | 0 |
|  |  | 42 | See the CA_42C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_19A-21A | CA_19A-21A | 19 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 21 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| CA_19A-28A | - | 19 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 25 | 0 |
|  |  | 28 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_19A-42A | CA_19A-42A | 19 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 35 | 0 |
|  |  | 42 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_19A-42C | CA_19A-42A | 19 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 55 | 0 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_19A-42D | - | 19 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 75 | 0 |
|  |  | 42 | See CA_42D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_19A-46A | - | 19 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 35 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_19A-46C | - | 19 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 55 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_19A-46D | - | 19 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 75 | 0 |
|  |  | 46 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_19A-46E | - | 19 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 95 | 0 |
|  |  | 46 | See CA_46E Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_20A-28A7 | - | 20 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 28 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_20A-31A | - | 20 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 25 | 0 |
|  |  | 31 |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_20A-32A | - | 20 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 32 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 20 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 1 |
|  |  | 32 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_20A-38A | - | 20 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 38 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_20A-38C | - | 20 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 38 | See CA_38C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_20A-40A | - | 20 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 40 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 20 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 35 | 1 |
|  |  | 40 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_20A-40A-40A | - | 20 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 55 | 0 |
|  |  | 40 | See CA_40A-40A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_20A-40C | - | 20 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 55 | 0 |
|  |  | 40 | See CA_40C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_20A-40D | - | 20 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 75 | 0 |
|  |  | 40 | See CA_40D Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_20A-41A | - | 20 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 41 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_20A-41C | - | 20 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 41 | See CA_41C in Table 5.6A.1-1 of 36.101 Bandwidth combination set 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_20A-41D | - | 20 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 41 | See CA_41D in Table 5.6A.1-1 of 36.101 Bandwidth combination set 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_20A-42A | - | 20 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 42 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_20A-42A-42A | - | 20 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 42 | See CA_42A-42A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_20A-43A | - | 20 |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 25 | 0 |
|  |  | 43 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_20A-67A | - | 20 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 67 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_20A-75A | - | 20 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 75 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_20A-76A | - | 20 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 25 | 0 |
|  |  | 76 |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_21A-28A | CA_21A-28A | 21 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 25 | 0 |
|  |  | 28 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_21A-42A | CA_21A-42A | 21 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 35 | 0 |
|  |  | 42 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_21A-42C | CA_21A-42A | 21 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 55 | 0 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_21A-42D | - | 21 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 75 | 0 |
|  |  | 42 | See CA_42D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_21A-42E | - | 21 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 95 | 0 |
|  |  | 42 | See CA_42E Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_21A-46A | - | 21 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 35 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_21A-46C | - | 21 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 55 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_21A-46D | - | 21 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 75 | 0 |
|  |  | 46 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_21A-46E | - | 21 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 95 | 0 |
|  |  | 46 | See CA_46E Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_23A-29A | - | 23 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 30 | 0 |
|  |  | 29 |  |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 23 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 1 |
|  |  | 29 |  |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_25A-26A | CA_25A-26A | 25 |  |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 35 | 0 |
|  |  | 26 | Yes |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 25 |  |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 1 |
|  |  | 26 |  |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 25 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 2 |
|  |  | 26 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_25A-25A-26A | CA_25A-26A | 25 | See CA_25A-25A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 45 | 0 |
|  |  | 26 |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_25A-41A | CA_25A-41A | 25 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 41 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_25A-25A-41A | CA_25A-41A | 25 | See CA_25A-25A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 41 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_25A-41C | CA_25A-41A | 25 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 41 | See CA_41C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_25A-25A-41C | CA_25A-41A | 25 | See CA_25A-25A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 41 | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_25A-41D | CA_25A-41A | 25 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 41 | See CA_41D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_25A-25A-41D | CA_25A-41A | 25 | See CA_25A-25A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 41 | See CA_41D bandwidth combination set 0 in table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_25A-41E | CA_25A-41A | 25 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 100 | 0 |
|  |  | 41 | See CA_41E Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_25A-25A-41E | CA_25A-41A | 25 | See CA_25A-25A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 120 | 0 |
|  |  | 41 | See CA_41E bandwidth combination set 0 in table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_25A-41F | CA_25A-41A | 25 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 120 | 0 |
|  |  | 41 | See CA_41F Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_25A-25A-41F | CA_25A-41A | 25 | See CA_25A-25A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 140 | 0 |
|  |  | 41 | See CA_41F bandwidth combination set 0 in table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_25A-46A | - | 25 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_25A-46C | - | 25 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 46 | See CA_46C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_25A-46D | - | 25 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 46 | See CA_46D Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_26A-41A | - | 26 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 35 | 0 |
|  |  | 41 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_26A-41C | - | 26 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 55 | 0 |
|  |  | 41 | See CA_41C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_26A-46A | CA_26A-46A | 26 |  |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_26A-48A | CA_26A-48A | 26 |  |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 48 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_26A-48C | CA_26A-48A | 26 |  |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_26A-48A-48A | CA_26A-48A | 26 |  |  | Yes |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 48 | See CA_48A-48A Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_26A-66A | - | 26 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 35 | 0 |
|  |  | 66 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_28A-32A | - | 28 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 32 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_28A-38A |  | 28 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 38 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_28A-40A | - | 28 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 40 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_28A-40C | - | 28 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 40 | See CA_40C Bandwidth Combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_28A-40D | - | 28 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 40 | See CA_40D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_28A-41A | CA_28A-41A | 28 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 41 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 28 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 1 |
|  |  | 41 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_28A-41C |  | 28 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 41 | See CA_41C Bandwidth Combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_28A-42A | CA_28A-42A | 28 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 42 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_28A-42C | CA_28A-42A, CA_42C | 28 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 42 | See CA_42C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_28A-42A-42A | - | 28 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 42 | See CA_42A-42A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_28A-42D | - | 28 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 42 | See CA_42D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_28A-42A-42C | CA_42C | 28 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 42 | See CA_42A-42C Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_28A-42C-42C | CA_42C | 28 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 100 | 0 |
|  |  | 42 | See CA_42C-42C Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_28A-46A | - | 28 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_28A-46C | - | 28 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_28A-46D | - | 28 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 46 | See CA_46D Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_28A-46E | - | 28 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 100 | 0 |
|  |  | 46 | See CA_46E Bandwidth Combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_28A-66A | - | 28 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 66 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_29A-30A | - | 29 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 20 | 0 |
|  |  | 30 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_29A-66A | - | 29 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 66 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_29A-66C |  | 29 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 66 | See CA_66C Bandwidth Combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_29A-66A-66A |  | 29 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth Combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_29A-70A | - | 29 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 25 | 0 |
|  |  | 70 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| CA_29A-70C | - | 29 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 35 | 0 |
|  |  | 70 | See CA_70C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_30A-66A | CA_30A-66A | 30 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 30 | 0 |
|  |  | 66 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_30A-66A-66A |  | 30 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth Combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_32A-42A | - | 32 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 42 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_32A-43A | - | 32 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 43 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_34A-39A |  | 34 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 35 | 0 |
|  |  | 39 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_34A-41A |  | 34 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 35 | 0 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_38A-40A | - | 38 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 40 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 38 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 1 |
|  |  | 40 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_38A-40A-40A | - | 38 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 40 | See CA_40A-40A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 38 |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  | 60 | 1 |
|  |  | 40 | See CA_40A-40A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_38A-40C | - | 38 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 40 | See CA_40C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 38 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 1 |
|  |  | 40 | See CA_40C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_38A-40D | - | 38 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 40 | See CA_40D Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_39A-40A | - | 39 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 40 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_39A-40C | - | 39 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 40 | See CA_40C Bandwidth Combination Set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_39A-40D | - | 39 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 40 | See CA_40D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_39A-40E | - | 39 |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  | 100 | 0 |
|  |  | 40 | See the CA_40E Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_39C-40A | - | 39 | See CA_39C Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 55 | 0 |
|  |  | 40 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_39C-40C | - | 39 | See CA_39C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 75 | 0 |
|  |  | 40 | See CA_40C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_39C-40D | - | 39 | See the CA_39C Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 95 | 0 |
|  |  | 40 | See the CA_40D Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_39A-41A | CA_39A-41A | 39 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_39A-41C | CA_41C CA_39A-41A CA_39A-41C | 39 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_39A-41D | CA_41C CA_39A-41A | 39 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_39C-41A | CA_39C CA_39A-41A CA_39C-41A | 39 | See CA_39C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 55 | 0 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_39C-41C | CA_39C CA_41C CA_39A-41A | 39 | See CA_39C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 75 | 0 |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_39C-41D | - | 39 | See CA_39C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 95 | 0 |
|  |  | 41 | See CA_41D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_39A-42A | - | 39 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 42 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_39A-42C | - | 39 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_39A-42D | - | 39 |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  | 80 | 0 |
|  |  | 42 | See CA_42D Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_39A-42E | - | 39 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 100 | 0 |
|  |  | 42 | See the CA_42E Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_39C-42A | - | 39 | See CA_39C Bandwidth Combination Set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 55 | 0 |
|  |  | 42 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_39C-42C | - | 39 | See CA_39C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 75 | 0 |
|  |  | 42 | See CA_42C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_39C-42D | - | 39 | See the CA_39C Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 95 | 0 |
|  |  | 42 | See the CA_42D Bandwidth combination set 1 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_39A-46A | - | 39 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_39A-46C | - | 39 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_39A-46D | - | 39 |  |  |  |  |  |  |  |  |  | Yes |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 46 | See the CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_39A-46E | - | 39 |  |  |  |  |  |  |  |  |  | Yes |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 100 | 0 |
|  |  | 46 | See CA_46E Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_39C-46A | - | 39 | See CA_39C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 55 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_39C-46C | - | 39 | See CA_39C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 75 | 0 |
|  |  | 46 | See the CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_39C-46D | - | 39 | See CA_39C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 95 | 0 |
|  |  | 46 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_40A-41A | - | 40 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 41 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_40A-42A | CA_40A-42A | 40 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_40A-42C | - | 40 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_40C-42C | - | 40 | See CA_40C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_40A-43A | - | 40 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 43 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_40A-46A | - | 40 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
|  |  | 40 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 1 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_40A-46C | - | 40 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 40 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 1 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_40A-46D | - | 40 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 40 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 80 | 1 |
|  |  | 46 | See CA_46D Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_40A-46E | - | 40 |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  | 100 | 0 |
|  |  | 46 | See CA_46E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 40 |  |  |  |  |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  | 100 | 1 |
|  |  | 46 | See CA_46E Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_40C-42A | - | 40 | See CA_40C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_40C-46A | - | 40 | See CA_40C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_40C-46C | - | 40 | See CA_40C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_40C-46D | - | 40 | See CA_40C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 46 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_40D-46A | - | 40 | See CA_40D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_40D-46C | - | 40 | See CA_40D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_41A9-42A9 | CA_41A-42A | 41 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_41A-42C | CA_41A-42A, CA_42C, CA_41A-42C | 41 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_41A-42A-42A | - | 41 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 42 | See CA_42A-42A Bandwidth combination set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_41A-42D | - | 41 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 42 | See CA_42D Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_41A-42A-42C | CA_42C | 41 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 42 | See CA_42A-42C Bandwidth combination set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_41A-42C-42C | CA_42C | 41 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 100 | 0 |
|  |  | 42 | See CA_42C-42C Bandwidth combination set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_41C-42A | CA_41A-42A, CA_41C, CA_41C-42A | 41 | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_41C-42C | CA_41A-42A, CA_41C, CA_42C, CA_41C-42C | 41 | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_41C-42A-42A | - | 41 | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 42 | See CA_42A-42A Bandwidth combination set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_41C-42D | - | 41 | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 42 | See CA_42D Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_41C-42A-42C | CA_42C | 41 | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 42 | See CA_42A-42C Bandwidth combination set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_41C-42C-42C | CA_42C | 41 | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 120 | 0 |
|  |  | 42 | See CA_42C-42C Bandwidth combination set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_41D-42A | - | 41 | See CA_41D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 42 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_41D-42C | - | 41 | See CA_41D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 42 | See CA_42C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_41A-46A | - | 41 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_41A-46C | - | 41 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 46 | See CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_41A-46D | - | 41 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_41A-46E | - | 41 |  |  |  |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  | 100 | 0 |
|  |  | 46 | See the CA_46E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_41C-46A | - | 41 | See CA_41C Bandwidth Combination Set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_41C-46C | - | 41 | See CA_41C Bandwidth combination set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 46 | See CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_41C-46D | - | 41 | See the CA_41C Bandwidth combination set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 46 | See the CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_41D-46A | - | 41 | See CA_41D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
| CA_41D-46C | - | 41 | See the CA_41D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 46 | See the CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_41A-48A | - | 41 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 48 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_41A-48C | - | 41 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_41A-48D | - | 41 |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 80 | 0 |
|  |  | 48 | See CA_48D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_41C-48A | CA_41C | 41 | See the CA_41C Bandwidth combination set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 48 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |
| CA_41C-48C | CA_41C | 41 | See the CA_41C Bandwidth combination set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 48 | See the CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_41C-48D | CA_41C | 41 | See the CA_41C Bandwidth combination set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 48 | See the CA_48D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_41D-48A | CA_41C | 41 | See the CA_41D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 48 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |
| CA_41D-48C | CA_41C | 41 | See the CA_41D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 48 | See the CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_42A-43A | - | 42 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 43 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_42A-46A | - | 42 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_46A-48A | - | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 48 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_46A-48A-48A | - | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 48 | See CA_48A-48A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_46A-48C | CA_48C | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  | 60 | 0 |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_46C-48A | - | 46 | See CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 48 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |
| CA_46C-48A-48A | - | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 48 | See CA_48A-48A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_46A-48B | CA_48B | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes | 40 | 0 |
|  |  | 48 | See CA_48B Bandwidth combination set 0 in 36.101 Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_46C-48C | CA_48C | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 48 | See CA_48C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_46C-48B | CA_48B | 46 | See CA_46C Bandwidth combination set 0 in 36.101 Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 48 | See CA_48B Bandwidth combination set 0 in 36.101 Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_46A-48D | CA_48C | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes | 80 | 0 |
|  |  | 48 | See CA_48D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_46D-48A | - | 46 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 48 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |
| CA_46D-48B | CA_48B | 46 | See CA_46D Bandwidth combination set 0 in 36.101 Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 48 | See CA_48B Bandwidth combination set 0 in 36.101 Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_46A-46A-66A | - | 46 | See CA_46A-46A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 66 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_46A-46C-66A | - | 46 | See CA_46A-46C Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 66 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_46A-46D-66A | - | 46 | See CA_46A-46D Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 66 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |
| CA_46A-48E | CA_48C | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  | 100 | 0 |
|  |  | 48 | See CA_48E Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_46C-48D | CA_48C | 46 | See CA_46C Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 48 | See CA_48D Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_46D-48A-48A | - | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 48 | See CA_48A-48A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_46D-48C | - | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_46E-48A | - | 46 | See CA_46E Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 48 |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |
| CA_46E-48B | CA_48B | 46 | See CA_46E Bandwidth combination set 0 in 36.101 Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 48 | See CA_48B Bandwidth combination set 0 in 36.101 Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_46C-66A | - | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 66 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_46A-66A | - | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 66 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_46A-66A-66A | - | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 66 | See the CA_66A-66A Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_46C-66A-66A | - | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_46A-66C | - | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 66 | See the CA_66C Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_46D-66A | - | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 66 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_46D-66A-66A | - | 46 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_46C-48E | CA_48C | 46 | See the CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 120 | 0 |
|  |  | 48 | See the CA_48E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_46E-48C | - | 46 | See the CA_46E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 120 | 0 |
|  |  | 48 | See the CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_46E-66A | - | 46 | See CA_46E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 66 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_46E-66A-66A | - | 46 | See CA_46E Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 120 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_46A-70A | - | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  | 35 | 0 |
|  |  | 70 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| CA_46A-71A | - | 46 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 71 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_46C-71A | - | 46 | See CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 71 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_46D-71A | - | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 71 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_48A-66A | - | 48 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 66 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_48A-48A-66A | - | 48 | See CA_48A-48A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 66 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_48A-48C-66A | - | 48 | See the CA_48A-48C Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 66 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_48A-48C-66B | - | 48 | See CA_48A-48C Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 66 | See CA_66B Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_48A-48C-66C | - | 48 | See CA_48A-48C Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 66 | See CA_66C Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_48A-48D-66A | - | 48 | See CA_48A-48D Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 66 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_48C-48C-66A | - | 48 | See CA_48C-48C Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 66 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_48A-66A-66A | - | 48 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_48A-48A-66A-66A | - | 48 | See CA_48A-48A Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_48A-48A-66B | - | 48 | See CA_48A-48A Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 66 | See CA_66B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_48A-48A-66C | - | 48 | See CA_48A-48A Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 66 | See CA_66C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_48C-66A-66A | - | 48 | See CA_48C Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_48C-66B | - | 48 | See CA_48C Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 66 | See CA_66B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_48C-66C | - | 48 | See CA_48C Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 66 | See CA_66C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_48A-66B | - | 48 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 66 | See CA_66B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_48A-66C | - | 48 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 60 | 0 |
|  |  | 66 | See CA_66C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_48C-66A | - | 48 | See CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 66 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_48D-66A | - | 48 | See the CA_48D Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 66 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_48E-66A | - | 48 | See CA_48E Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 66 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_48A-71A | - | 48 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 71 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_48C-71A | - | 48 | See CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 71 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_48A-48A-71A | - | 48 | See CA_48A-48A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 71 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_66A-70A | - | 66 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 35 | 0 |
|  |  | 70 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| CA_66A-66A-70A | - | 66 | See CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 55 | 0 |
|  |  | 70 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| CA_66A-70C | - | 66 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 45 | 0 |
|  |  | 70 | See CA_70C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_66A-66A-70C | - | 66 | See the CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 65 | 0 |
|  |  | 70 | See the CA_70C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_66C-70A | - | 66 | See CA_66C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 55 | 0 |
|  |  | 70 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |
| CA_66C-70C | - | 66 | See the CA_66C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 65 | 0 |
|  |  | 70 | See the CA_70C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_66A-71A | - | 66 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  | 40 | 0 |
|  |  | 71 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_66C-71A | - | 66 | See CA_66C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 71 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_66A-66A-71A | - | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 71 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_70A-71A | - | 70 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  | 35 | 0 |
|  |  | 71 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| CA_70C-71A | - | 70 | See the CA_70C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 45 | 0 |
|  |  | 71 |  |  |  |  |  |  | Yes |  |  |  | Yes |  |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |  |
| NOTE 1: The CA Configuration refers to a combination of an operating band and a CA bandwidth class specified in Table 5.6A-1 (the indexing letter). Absence of a CA bandwidth class for an operating band implies support of all classes. NOTE 2: For each band combination, all combinations of indicated bandwidths belong to the set. NOTE 3: For the supported CC bandwidth combinations, the CC downlink and uplink bandwidths are equal. NOTE 4: Uplink CA configurations are the configurations supported by the present release of specifications. NOTE 5: For TDD inter-band Carrier Aggregation only non-simultaneous Rx/Tx uplink CA configurations can be supported by UE supporting corresponding DL CA configuration without simultaneous Rx/Tx. NOTE 6: Void NOTE 7: Power imbalance between downlink carriers on Band 20 and Band 28 is assumed to be within [6dB]. NOTE 8: For the corresponding CA configuration, UE may not support Pcell transmissions in this E-UTRA band. NOTE 9: 8Rx Requirements are applicable for this band configuration if UE supports 8Rx. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 5.6A.1-2a: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA (three bands)

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations (NOTE 5) | E-UTRA Bands | 1.4 MHz | 3 MHz |  | 5 MHz |  | 10 MHz |  |  | 15 MHz |  |  | 20 MHz |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3A-5A | CA_1A-3A CA_1A-5A6 CA_3A-5A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 1 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 40 | 1 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_1A-1A-3A-5A | - | 1 | See CA_1A-1A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_1A-1A-3C-5A | CA_1A-3A, CA_1A-5A CA_3A-5A | 1 | See CA_1A-1A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 3 | See CA_3C Bandwidth combination set 0 in table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-3A-5A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | 65 | 0 |
|  |  | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_1C-3A-5A | - | 1 | See CA_1C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-3A-7A-7A | CA_1A-3A, CA_1A-7A, CA_3A-7A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 3 | See the CA_3A-3A Bandwidth combination set 0 in Table below |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | See the CA_7A-7A Bandwidth combination set 1 in Table below |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3C-5A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-7A | CA_1A-3A CA_1A-7A CA_3A-7A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 1 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-1A-3A-7A | - | 1 | See CA_1A-1A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-1A-3C-7A | - | 1 | See the CA_1A-1A Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-1A-3A-3A-7A | CA_1A-3A CA_1A-7A CA_3A-7A | 1 | See the CA_1A-1A Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 3 | See the CA_3A-3A Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-3A-3A-7A | CA_1A-3A, CA_1A-7A, CA_3A-7A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 3 | See the CA_3A-3A Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-1A-3A-3A-7C | CA_7C | 1 | See CA_1A-1A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 120 | 0 |
|  |  | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | See CA_7C Bandwidth combination set 2 in Table 5.6A.1-1 of 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-3A-7C | 7C | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | See CA_7C Bandwidth combination set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-7A-7A | CA_1A-3A CA_1A-7A CA_3A-7A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 3 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 1 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-7C | CA_1A-3A, CA_1A-7A, CA_3A-7A, CA_7C | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 3 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 7 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 1 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 7 | See CA_7C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-1A-3A-7C | CA_7C | 1 | See CA_1A-1A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 7 | See CA_7C Bandwidth combination set 2 in Table 5.6A.1-1 of 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-1A-3C-7A | CA_3C | 1 | See CA_1A-1A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 of 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-1A-3C-7C | CA_3C CA_7C | 1 | See CA_1A-1A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 120 | 0 |
|  |  | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 of 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | See CA_7C Bandwidth combination set 2 in Table 5.6A.1-1 of 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3C-7A | CA_1A-3A, CA_1A-7A, CA_3A-7A, CA_3C | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 1 |
|  |  | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-3C-7C | CA_1A-3A, CA_1A-7A, CA_3A-7A, CA_3C, CA_7C | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | See CA_7C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-8A | CA_1A-3A CA_1A-8A CA_3A-8A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 8 |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 1 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 40 | 1 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 8 |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | 40 | 2 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 8 |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 3 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-3A-8A | CA_1A-3A CA_1A-8A CA_3A-8A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3C-8A | CA_1A-3A CA_1A-8A CA_3A-8A CA_3C | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 8 |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-3A-42C | CA_1A-3A, CA_1A-42A, CA_3A-42A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-11A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 11 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-18A | CA_1A-3A, CA_1A-18A6, CA_3A-18A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 55 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 18 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
| CA_1A-3A-19A | CA_1A-3A CA_1A-19A6 CA_3A-19A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 55 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 19 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
| CA_1A-3A-3A-19A | CA_1A-3A CA_1A-19A6 CA_3A-19A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 75 | 0 |
|  |  | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 19 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
| CA_1A-3A-26A | CA_1A-3A, CA_1A-26A, CA_3A-26A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 26 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 55 | 1 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 26 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
| CA_1A-3A-20A | CA_1A-3A, CA_3A-20A, CA_1A-20A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 20 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-3A-3A-20A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 3 | See CA_3A-3A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 20 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-3C-20A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 20 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-3A-21A | CA_1A-3A, CA_1A-21A, CA_3A-21A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 55 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 21 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
| CA_1A-3A-3A-21A | CA_1A-3A, CA_1A-21A, CA_3A-21A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 75 | 0 |
|  |  | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 21 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
| CA_1A-3A-28A | CA_1A-3A, CA_1A-28A, CA_3A-28A6 | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-1A-3A-28A | - | 1 | See CA_1A-1A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-1A-3A-3A-28A | - | 1 | See CA_1A-1A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-1A-3C-28A | CA_3C | 1 | See CA_1A-1A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 of 36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-3A-3A-28A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 3 | See CA_3A-3A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-3C-28A | CA_3C | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-1A-3C-28A | CA_1A-3A, CA_1A-28A CA_3A-28A | 1 | See CA_1A-1A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-3A-32A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 32 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-3A-38A | CA_1A-3A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 38 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-3C-38A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 38 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-3A-40A | CA_1A-3A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 40 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-3A-40C | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 40 | See CA_40C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3C-40A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 40 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-3C-40C | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 40 | See CA_40C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-41A9 | CA_1A-3A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 41 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-3A-41C9 | CA_1A-3A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 41 | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-41D9 | CA_1A-3A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 41 | See CA_41D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-42A | CA_1A-3A, CA_1A-42A, CA_3A-42A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 42 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-3A-3A-42A | CA_1A-3A, CA_1A-42A, CA_3A-42A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-3A-42A-42A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 42 | See CA_42A-42A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-42A-42C | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 42 | See CA_42A-42C Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-42C | CA_1A-3A, CA_1A-42A, CA_1A-42C, CA_3A-42A, CA_3A-42C CA_42C | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-42C-42C | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 120 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 42 | See CA_42C-42C Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-42D | CA_1A-3A, CA_1A-42A, CA_3A-42A, CA_1A-42C, CA_3A-42C | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 42 | See CA_42D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-43A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | 50 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 43 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-3A-46A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |
|  | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 1 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
| CA_1A-3A-46C | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 | See CA_46C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 1 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 | See CA_46C in Table 5.6A.1-1 of TS 36.101 Bandwidth Combination Set 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-46D | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 | See CA_46D in Table 5.6A.1-1 of TS 36.101 Bandwidth Combination Set 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-46E | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 120 | 0 |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 | See CA_46E in Table 5.6A.1-1 of TS 36.101 Bandwidth Combination Set 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-5A-40A | CA_1A-5A6 | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 40 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-5A-41A11 | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
| CA_1A-5A-46A | CA_1A-5A6 | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
| CA_1A-5A-7A | CA_1A-5A6 CA_1A-7A CA_5A-7A | 1 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 40 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 1 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-5A-7A-7A | CA_1A-5A6 CA_1A-7A CA_5A-7A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 3 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-5A-28A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | 45 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-5A-46C | CA_1A-5A6 | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 46 | See CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-5A-46D | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 90 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-7A-8A | CA_1A-7A, CA_1A-8A CA_7A-8A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 1 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_1A-7A-7A-8A | CA_1A-7A CA_1A-8A CA_7A-8A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 7 | See CA_7A-7A Bandwidth combination set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_1A-7A-20A | CA_1A-7A CA_1A-20A CA_7A-20A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 20 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 1 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 20 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 2 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 20 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-7A-7A-20A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 3 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 20 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-7C-20A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 7 | See CA_7C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 20 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-7A-26A | CA_1A-7A CA_1A-26A, CA_7A-26A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 55 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 26 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
| CA_1A-7A-7A-26A | CA_1A-7A CA_1A-26A, CA_7A-26A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 75 | 0 |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 3 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 26 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
| CA_1A-7A-28A | CA_1A-7A, CA_1A-28A, CA_7A-28A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 55 | 0 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 1 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 28 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 2 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-1A-7A-28A | - | 1 | See CA_1A-1A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-1A-7C-28A | CA_7C | 1 | See CA_1A-1A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 7 | See CA_7C Bandwidth combination set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-7A-7A-28A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 7 | See CA_7A-7A Bandwidth combination set 3 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-7C-28A | CA_1A-7A, CA_1A-28A, CA_7A-28A, CA_7C | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-7A-32A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 32 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-7A-38A16 | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 38 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-7A-40A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 40 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-7A-40C | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 40 | See CA_40C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-7A-42A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 42 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-7A-46A | CA_1A-7A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
| CA_1A-7A-46A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 1 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
| CA_1A-7A-46C | CA_1A-7A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 | See CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-7A-46C | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 1 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 | See CA_46C in Table 5.6A.1-1 of TS 36.101 Bandwidth Combination Set 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-7A-46D | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-7A-46D | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 1 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 | See CA_46D in Table 5.6A.1-1 of TS 36.101 Bandwidth Combination Set 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-7A-46E | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 120 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 | See CA_46E in Table 5.6A.1-1 of TS 36.101 Bandwidth Combination Set 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-8A-11A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 40 | 0 |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 11 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_1A-8A-20A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 20 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-8A-28A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 8 |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-8A-38A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 38 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-8A-40A | CA_1A-8A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 8 |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 40 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-8A-40C | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 40 | See CA_40C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-8A-42A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-8A-42C | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-11A-18A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 45 | 0 |
|  |  | 11 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 18 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 40 | 1 |
|  |  | 11 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 18 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_1A-11A-28A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 11 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-11A-42A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 11 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-11A-42C | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 11 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-18A-28A | CA_1A-18A6 CA_1A-28A CA_18A-28A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 45 | 0 |
|  |  | 18 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 40 | 1 |
|  |  | 18 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_1A-18A-41A | CA_1A-18A CA_1A-41A CA_18A-41A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 55 | 0 |
|  |  | 18 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-18A-41C | CA_1A-18A CA_1A-41A CA_1A-41C CA_18A-41A CA_18A-41C CA_41C | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 75 | 0 |
|  |  | 18 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 41 | See CA_41C Bandwidth combination set 1 in Table 5.6A.1-1 in TS36.101 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-18A-42A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 55 | 0 |
|  |  | 18 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-18A-42C | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 75 | 0 |
|  |  | 18 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-19A-21A | CA_1A-19A6 CA_1A-21A CA_19A-21A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 19 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 21 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
| CA_1A-19A-28A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 45 | 0 |
|  |  | 19 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_1A-19A-42A | CA_1A-19A6, CA_1A-42A, CA_19A-42A6 | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 55 | 0 |
|  |  | 19 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-19A-42C | CA_1A-19A6 CA_1A-42A CA_19A-42A6 | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 75 | 0 |
|  |  | 19 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-20A-28A12 | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 20 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-20A-32A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 20 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 32 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-20A-38A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 20 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 38 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-20A-42A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 20 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 42 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-20A-43A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | 40 | 0 |
|  |  | 20 |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 43 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-21A-28A | CA_1A-21A, CA_1A-28A, CA_21A-28A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 45 | 0 |
|  |  | 21 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_1A-21A-42A | CA_1A-21A, CA_1A-42A, CA_21A-42A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 55 | 0 |
|  |  | 21 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-21A-42C | CA_1A-21A CA_1A-42A CA_21A-42A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 75 | 0 |
|  |  | 21 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-21A-42D | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 95 | 0 |
|  |  | 21 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-28A-40A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 40 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-28A-40C | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 40 | See CA_40C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-28A-42A | CA_1A-28A, CA_1A-42A, CA_28A-42A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-28A-42C | CA_1A-28A, CA_1A-42A, CA_28A-42A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-32A-42A | - | 1 |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  | 55 | 0 |
|  |  | 32 |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |
|  |  | 42 |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |
| CA_1A-32A-43A | - | 1 |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  | 55 | 0 |
|  |  | 32 |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |
|  |  | 43 |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes |  |  |
| CA_1A-41A-42A10 | CA_1A-42A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 41 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 42 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-41A-42C10 | CA_1A-42A, CA_42C, CA_1A-42C | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 41 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-41C-42A10 | CA_1A-42A | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 41 | See CA_41C Bandwidth combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_1A-41C-42C10 | CA_1A-42A, CA_42C, CA_1A-42C | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 41 | See CA_41C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-42A-43A | - | 1 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | 55 | 0 |
|  |  | 42 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 43 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-4A-5A | CA_2A-4A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 4 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-4A-5A | CA_2A-5A CA_4A-5A | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 4 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-12A-66A-66A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-14A-66A-66A | CA_2A-14A CA_14A-66A | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 14 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-4A-5B | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 4 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-4A-7A | CA_2A-4A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 4 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-4A-7A-7A | CA_2A-4A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 4 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 7 | See the CA_7A-7A Bandwidth combination set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-4A-7C | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 4 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 7 | See CA_7C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-4A-4A-5A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 4 | See CA_4A-4A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-4A-12A | CA_2A-4A CA_4A-12A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 4 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-4A-12A-12A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 4 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 12 | See CA_12A-12A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-4A-12B | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 55 | 0 |
|  |  | 4 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 12 | See CA_12B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-4A-12A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 4 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-4A-4A-12A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 4 | See CA_4A-4A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-4A-13A | CA_2A-13A CA_4A-13A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 4 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 13 |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-4A-28A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 4 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-4A-29A | CA_2A-4A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 4 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 29 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-4A-30A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 4 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-4A-71A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 4 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 71 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-2A-4A-71A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 4 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 71 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-5A-7A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-5A-12A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 40 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-5A-12A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-5A-12A-12A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 40 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 12 | See CA_12A-12A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-5A-46C | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-5A-66A | CA_2A-5A CA_5A-66A | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-2A-5A-66A-66A | CA_2A-5A CA_5A-66A | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-5A-66B | CA_2A-5A CA_5A-66A | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-5A-66C | CA_2A-5A CA_5A-66A | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-7A-12A | - | 2 | See CA_2A-2A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-7A-66A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-2A-12B-66A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 75 | 0 |
|  |  | 12 | See CA_12B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-2A-13A-66A | CA_2A-13A CA_13A-66A | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-5A-12B | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 45 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 12 | See CA_12B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-5A-13A | CA_2A-13A6 | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 40 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 13 |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-5A-28A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-5A-29A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 40 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 29 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-5A-30A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 40 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-5A-30A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2C-5A-30A | - | 2 | See CA_2C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-5B-30A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2C-5B-30A | - | 2 | See CA_2C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 5 | See CA_5B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-5A-46A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
| CA_2A-5A-46D | CA_2A-5A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 90 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 46 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-5A-46E | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 110 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 46 | See CA_46E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-5A-48A | CA_2A-48A CA_5A-48A | 2 | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-5A-48C | CA_2A-48A CA_5A-48A CA_2A-5A | 2 | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-5A-48D | CA_2A-5A CA_5A-48A CA_2A-48A | 2 | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 90 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 | See CA_48D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-5A-66A | CA_2A-5A CA_5A-66A CA_2A-66A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-5A-66A-66A | CA_2A-5A CA_5A-66A CA_2A-66A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-5B-66A-66A | CA_2A-5A CA_5A-66A | 2 |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | Yes | 80 | 0 |
|  |  | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-5A-66B | CA_2A-5A CA_5A-66A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-5A-66C | CA_2A-5A CA_5A-66A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-5A-66D | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 90 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-5B-66A | CA_2A-5A CA_5A-66A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-5B-66B | CA_2A-5A CA_5A-66A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-5B-66C | CA_2A-5A CA_5A-66A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-5B-66A-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-5B-66A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-7A-12A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-7A-12B | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 55 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 12 | See CA_12B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-7A-13A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-7C-13A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 7 | See CA_7C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-7A-7A-13A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 7 | See CA_7A-7A Bandwidth combination set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-7A-26A | - | 2 |  | Yes |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 55 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 26 |  | Yes |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
| CA_2A-7A-28A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-7C-28A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-7A-29A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 29 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-7C-29A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 7 | See CA_7C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 29 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-7A-7A-29A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 7 | See CA_7A-7A Bandwidth combination set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 29 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-7A-30A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-7A-46A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |
| CA_2A-7A-7A-46A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 7 | See CA_7A-7A Bandwidth combination set 1 in table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
| CA_2A-7A-46C | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 | See CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-7A-7A-46C | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 7 | See CA_7A-7A Bandwidth combination set 1 in table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 46 | See CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-7A-46D | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 | See CA_46D Bandwidth Combination Set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-7A-7A-46D | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 120 | 0 |
|  |  | 7 | See CA_7A-7A Bandwidth combination set 1 in table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 46 | See CA_46D Bandwidth Combination Set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-7A-46E | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 120 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 | See the CA_46E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-7A-7A-46E | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 140 | 0 |
|  |  | 7 | See CA_7A-7A Bandwidth combination set 1 in table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 46 | See the CA_46E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-7A-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-7A-7A-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 7 | See CA_7A-7A Bandwidth combination set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-7A-7A-66A-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 7 | See CA_7A-7A Bandwidth combination set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-7C-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 7 | See CA_7C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-7C-66A-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 7 | See CA_7C Bandwidth combination set 2 in table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-7A-66A-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-12A-30A | CA_2A-12A6 | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 40 | 0 |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-12A-30A | - | 2 | See CA_2A-2A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2C-12A-30A | - | 2 | See CA_2C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-12A-66A | CA_2A-12A, CA_2A-66A CA_12A-66A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 2 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 40 | 1 |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-2A-12A-66A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-12A-66A-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-12A-66C | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-12B-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 55 | 0 |
|  |  | 12 | See CA_12B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-12B-66A-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 75 | 0 |
|  |  | 12 | See CA_12B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-13A-46A | CA_2A-13A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
| CA_2A-13A-46C | CA_2A-13A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-13A-46D | CA_2A-13A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 90 | 0 |
|  |  | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 46 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-13A-46E | CA_2A-13A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 110 | 0 |
|  |  | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 46 | See CA_46E Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-13A-46A-46D | CA_2A-13A | 2 | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 110 | 0 |
|  |  | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 46 | See CA_46A-46D Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-13A-46A-46C | CA_2A-13A | 2 | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 90 | 0 |
|  |  | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 46 | See CA_46A-46C Bandwidth Combination Set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-13A-46A-46A | CA_2A-13A | 2 | Yes | Yes |  | Yes |  | 70 |  |  | 0 |  |  | Yes |  |  | 70 | 0 |
|  |  | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 46 | See CA_46A-46A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-13A-48A | CA_2A-48A CA_13A-48A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-13A-48A-48A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 | See CA_48A-48A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-13A-48C | CA_2A-48A CA_13A-48A CA_2A-13A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 | See CA_48C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-13A-48D | CA_2A-48A CA_13A-48A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 90 | 0 |
|  |  | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 | See CA_48D Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-13A-48A-48C | CA_2A-13A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 90 | 0 |
|  |  | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 | See CA_48A-48C Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-13A-66A | CA_2A-13A CA_13A-66A CA_2A-66A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-13A-66D | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 90 | 0 |
|  |  | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-13A-66A-66A | CA_2A-13A CA_13A-66A CA_2A-66A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-13A-66A-66B | CA_2A-13A CA_13A-66A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66B Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-13A-66A-66C | CA_2A-13A CA_13A-66A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 90 | 0 |
|  |  | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66C Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-13A-66B | CA_2A-13A CA_13A-66A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-13A-66C | CA_2A-13A CA_13A-66A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-13A-66B | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-13A-66A-66A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-14A-30A | CA_2A-14A CA_14A-30A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 40 | 0 |
|  |  | 14 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-14A-30A | CA_2A-14A CA_14A-30A | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 14 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-14A-66A | CA_2A-14A CA_14A-66A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 14 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-2A-14A-66A | CA_2A-14A CA_14A-66A | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 14 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-14A-66A-66A | CA_2A-14A CA_14A-66A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 14 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-14A-66A-66A-66A | CA_2A-14A CA_14A-66A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 90 | 0 |
|  |  | 14 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-4 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-26A-66A | - | 2 |  | Yes |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 55 | 0 |
|  |  | 26 |  | Yes |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 66 |  | Yes |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-28A-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-29A-30A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 40 | 0 |
|  |  | 29 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-2A-29A-30A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 29 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2C-29A-30A | - | 2 | See CA_2C Bandwidth Combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 29 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_2A-29A-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 29 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-2A-30A-66A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-30A-66A-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-30A-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-46A-48A | CA_2A-48A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
|  |  | 48 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-46A-48C | CA_2A-48A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
|  |  | 48 | See the CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-46A-48D | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
|  |  | 48 | See CA_48D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-46A-48E | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 120 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
|  |  | 48 | See the CA_48E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-46C-48A | CA_2A-48A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 46 | See the CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-46C-48C | CA_2A-48A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 46 | See CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-46D-48A | CA_2A-48A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-46A-66A | CA_2A-66A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-46A-46A-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 46 | See CA_46A-46A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-46C-48D | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 120 | 0 |
|  |  | 46 | See the CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 | See the CA_48D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-46C-48E | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 140 | 0 |
|  |  | 46 | See the CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 | See the CA_48E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-46C-66A | CA_2A-66A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 46 | See CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-46A-66A-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
|  |  | 66 | See the CA_66A-66A Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-46C-66A-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 46 | See the CA_46C Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See the CA_66A-66A Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-46D-66A-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 120 | 0 |
|  |  | 46 | See the CA_46D Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See the CA_66A-66A Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-46E-66A-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 140 | 0 |
|  |  | 46 | See the CA_46E Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See the CA_66A-66A Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-46A-46C-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 46 | See CA_46A-46C Bandwidth Combination Set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-46D-66A | CA_2A-66A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 46 | See CA_46D Bandwidth Combination Set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-46D-48C | CA_2A-48A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 120 | 0 |
|  |  | 46 | See the CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 | See the CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-46E-48A | CA_2A-48A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 120 | 0 |
|  |  | 46 | See the CA_46E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-46E-66A | CA_2A-66A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 120 | 0 |
|  |  | 46 | See CA_46E Bandwidth Combination Set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-46E-48C |  | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 140 | 0 |
|  |  | 46 | See the CA_46E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 | See the CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-48A-66A | CA_2A-48A CA_48A-66A CA_2A-66A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 48 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-48C-66A | CA_2A-48A CA_48A-66A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-48C-66A-66A | CA_48A-66A CA_2A-66A CA_2A-48A | 2 | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-48D-66A | CA_48A-66A CA_2A-48A CA_2A-66A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 48 | See CA_48D Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-48D-66A-66A | CA_48A-66A CA_2A-66A CA_2A-48A | 2 | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 120 | 0 |
|  |  | 48 | See CA_48D Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-48E-66A | CA_48A-66A CA_2A-66A CA_2A-48A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 120 | 0 |
|  |  | 48 | See CA_48E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-48E-66A-66A | CA_48A-66A CA_2A-66A CA_2A-48A | 2 | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 140 | 0 |
|  |  | 48 | See CA_48E Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-48A-48A-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 48 | See CA_48A-48A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-48A-48C-66A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 48 | See CA_48A-48C Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-48A-66A-66A | CA_48A-66A CA_2A-48A CA_2A-66A | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 48 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-66A-71A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 71 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-2A-66A-71A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 71 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-66A-66A-71A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 71 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_2A-66C-71A | - | 2 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 66 | See CA_66C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 71 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-5A-7A | CA_3A-5A, CA_3A-7A, CA_5A-7A | 3 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-5A-7A | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 1 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-3A-5A-7A | - | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-5A-7A-7A | CA_3A-5A, CA_3A-7A, CA_5A-7A | 3 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 3 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-5A-7C | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | See CA_7C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-5A-28A | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-3A-5A-28A | - | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-5A-40A | CA_3A-5A | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 40 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 3 |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 40 | 1 |
|  |  | 5 |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 40 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
| CA_3A-5A-40A-40A | - | 3 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 40 | See CA_40A-40A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-5A-41A | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
| CA_3C-7A-8A | - | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_3A-3A-7A-8A | CA_3A-7A, CA_3A-8A, CA_7A-8A | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 | See CA_3A-3A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 1 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_3A-3A-7A-7A-8A | CA_3A-7A, CA_3A-8A, CA_7A-8A | 3 | See CA_3A-3A Bandwidth Combination Set 0 in table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 1 in table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 | See CA_3A-3A Bandwidth Combination Set 1 in table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 1 |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 2 in table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_3A-7A-7A-8A | CA_3A-7A, CA_3A-8A, CA_7A-8A | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 1 |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 2 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_3A-7A-8A | CA_3A-7A, CA_3A-8A, CA_7A-8A | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | 40 | 0 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 1 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 2 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_3A-7A-20A | CA_3A-7A CA_3A-20A CA_7A-20A | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 20 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 1 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 20 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-7A-7A-20A | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 3 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 20 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-3A-7A-20A | - | 3 | See CA_3A-3A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 20 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3C-7A-20A | - | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 20 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 1 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 20 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3C-7C-20A | - | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 7 | See CA_7C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 20 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-7C-20A | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 7 | See CA_7C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 20 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-7A-26A | CA_3A-7A, CA_3A-26A, CA_7A-26A | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 55 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 26 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
| CA_3A-7A-7A-26A | CA_3A-7A, CA_3A-26A, CA_7A-26A | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 75 | 0 |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 3 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 26 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
| CA_3A-7A-28A | CA_3A-7A, CA_3A-28A6, CA_7A-28A | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-3A-7A-28A | - | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-3A-7C-28A | CA_7C | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-7A-7A-28A | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 7 | See CA_7A-7A Bandwidth combination set 3 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-7C-28A | CA_3A-7A, CA_7C, CA_7A-28A | 3 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 3 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 1 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3C-7A-28A | CA_3C | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 28 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3C-7C-28A | CA_3C CA_7C | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-7A-32A | CA_3A-7A | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 32 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3C-7A-32A | - | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 32 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-7A-38A7 | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 38 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3C-7A-38A7 | - | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 38 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-7A-40A | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 40 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-7A-40C | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 40 | See CA_40C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-7A-42A | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 42 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-7A-46A | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
| CA_3A-7C-46A | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
| CA_3A-7C-46C | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-7C-46D | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 120 | 0 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 46 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-7C-46E | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 140 | 0 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 46 | See CA_46E Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-7A-46C | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-7A-46D | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-7A-46E | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 120 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 | See CA_46E Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-8A-11A | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 40 | 0 |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 11 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_3A-8A-20A | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 40 | 0 |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 20 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_3A-8A-28A | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 8 |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-8A-32A | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 8 |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 32 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-8A-38A | CA_3A-8A | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 38 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3C-8A-38A | - | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 38 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-8A-40A | CA_3A-8A | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 8 |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 40 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-8A-40C | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 40 | See CA_40C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-8A-42A | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-8A-42C | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-11A-18A | CA_3A-11A, CA_3A-18A, CA_11A-18A | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 45 | 0 |
|  |  | 11 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 18 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
| CA_3A-11A-26A | CA_3A-11A, CA_3A-26A, CA_11A-26A | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 45 | 0 |
|  |  | 11 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 26 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
| CA_3A-11A-28A | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 11 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-18A-42A | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 55 | 0 |
|  |  | 18 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-18A-42C | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 75 | 0 |
|  |  | 18 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-19A-21A | CA_3A-19A, CA_3A-21A, CA_19A-21A | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 19 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 21 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
| CA_3A-3A-19A-21A | CA_3A-19A, CA_3A-21A, CA_19A-21A | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 19 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 21 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
| CA_3A-19A-42A | CA_3A-19A, CA_3A-42A, CA_19A-42A6 | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 55 | 0 |
|  |  | 19 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-19A-42C | CA_3A-19A CA_3A-42A CA_19A-42A6 | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 75 | 0 |
|  |  | 19 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-19A-42D | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 95 | 0 |
|  |  | 19 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-20A-28A12 | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 20 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-3A-20A-28A12 | - | 3 | See CA_3A-3A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 20 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3C-20A-28A12 | - | 3 | See CA_3C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 20 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-20A-32A | CA_3A-20A | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 20 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 32 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-20A-42A | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 20 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 42 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-20A-43A | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | 40 | 0 |
|  |  | 20 |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 43 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-21A-28A | CA_3A-21A, CA_3A-28A6, CA_21A-28A | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 45 | 0 |
|  |  | 21 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_3A-21A-42A | CA_3A-21A, CA_3A-42A, CA_21A-42A | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 55 | 0 |
|  |  | 21 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-21A-42C | CA_3A-21A, CA_3A-42A, CA_21A-42A | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 75 | 0 |
|  |  | 21 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-21A-42D | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 95 | 0 |
|  |  | 21 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-28A-38A | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 38 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3C-28A-38A | - | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 38 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-28A-40A | CA_3A-28A6 | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 40 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-28A-40C | CA_3A-28A6 | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 40 | See CA_40C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-28A-40D | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 40 | See CA_40D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-28A-41A | CA_3A-41A | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 41 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-28A-41C | CA_3A-41A | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 41 | See CA_41C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-28A-42A | CA_3A-28A6, CA_3A-42A, CA_28A-42A | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-28A-42A-42A | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42A-42A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-28A-42C | CA_3A-28A6, CA_3A-42A, CA_28A-42A, CA_42C | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-28A-42A-42C | CA_42C | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 90 | 0 |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42A-42C Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-28A-42C-42C | CA_42C | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 110 | 0 |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C-42C Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-28A-42D | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 90 | 0 |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-32A-42A | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | 55 | 0 |
|  |  | 32 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 42 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-32A-43A | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | 55 | 0 |
|  |  | 32 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 43 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-32A-46A | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 32 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
| CA_3A-32A-46C | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 32 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 | See CA_46C in Table 5.6A.1-1 of TS 36.101 Bandwidth Combination Set 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-32A-46D | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 32 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 | See CA_46D in Table 5.6A.1-1 of TS 36.101 Bandwidth Combination Set 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-32A-46E | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 120 | 0 |
|  |  | 32 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 | See CA_46E in Table 5.6A.1-1 of TS 36.101 Bandwidth Combination Set 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-41A-42A | CA_3A-41A, CA_41A-42A, CA_3A-42A | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 41 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 42 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-41A-42A-42A | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 41 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 42 | See CA_42A-42A Bandwidth combination set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-41A-42C | CA_3A-41A, CA_3A-42C, CA_3A-42A, CA_41A-42A, CA_41A-42C, CA_42C | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 41 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-41A-42A-42C | CA_42C | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 41 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 42 | See CA_42A-42C Bandwidth combination set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-41A-42C-42C | CA_42C | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 120 | 0 |
|  |  | 41 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 42 | See CA_42C-42C Bandwidth combination set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-41C-42A | CA_3A-41A, CA_3A-41C, CA_3A-42A, CA_41A-42A, CA_41C CA_41C-42A | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 41 | See CA_41C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_3A-41C-42C | CA_3A-41A, CA_3A-41C, CA_3A-42A, CA_3A-42C, CA_41A-42A, CA_41A-42C CA_41C, CA_41C-42A, CA_42C | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 41 | See CA_41C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-42A-43A | - | 3 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | 55 | 0 |
|  |  | 42 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 43 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_4A-5A-12A | - | 4 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 40 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_4A-5A-12A-12A | - | 4 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 40 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 12 | See CA_12A-12A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-5A-12B | - | 4 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 45 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 12 | See CA_12B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_4A-4A-5A-12A | - | 4 | See CA_4A-4A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_4A-5A-13A | CA_4A-13A6 | 4 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 40 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 13 |  |  |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_4A-5A-29A | - | 4 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 40 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 29 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_4A-5A-30A | - | 4 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 40 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_4A-4A-5A-30A | - | 4 | See CA_4A-4A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_4A-4A-5B-30A | - | 4 | See CA_4A-4A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_4A-5B-30A | - | 4 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_4A-7A-12A | - | 4 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 40 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 4 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 1 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_4A-7A-28A | - | 4 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_4A-12A-30A | CA_4A-12A | 4 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 40 | 0 |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_4A-4A-12A-30A | - | 4 | See CA_4A-4A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_4A-29A-30A | - | 4 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 40 | 0 |
|  |  | 29 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_4A-4A-29A-30A | - | 4 | See CA_4A-4A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 29 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_5A-7A-28A | - | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_5A-7C-28A | - | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_5A-7A-46A | CA_5A-7A | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
| CA_5A-7A-46C | CA_5A-7A | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 | See CA_46C Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-7A-46D | - | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-7A-66A | - | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_5A-7A-66A-66A | - | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-7C-66A | - | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_5A-7C-66A-66A | - | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 7 | See CA_7C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-12A-46A | - | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 40 | 0 |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
| CA_5A-12A-46C | - | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-12A-46D | - | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 46 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-12A-48A | - | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 40 | 0 |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_5A-12A-48C | - | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-12A-48D | - | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 | See the CA_48D Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-30A-66A | - | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 40 | 0 |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_5A-30A-66A-66A | - | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5B-30A-66A | - | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_5B-30A-66A-66A | - | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-46A-66A | - | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_5A-46E-66A | - | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 110 | 0 |
|  |  | 46 | See CA_46E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_5A-46A-66A-66A | - | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-46C-66A-66A | - | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 46 | See CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-46D-66A-66A | - | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 110 | 0 |
|  |  | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-46E-66A-66A | - | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 130 | 0 |
|  |  | 46 | See CA_46E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7A-8A-20A | - | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | 40 | 0 |
|  |  | 8 |  | Yes |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 20 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
| CA_7A-8A-38A13 | - | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 38 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_7A-8A-40A | - | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 40 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_5A-12A-66A | - | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 40 | 0 |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_5A-40A-41A | - | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 40 |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
| CA_5A-46C-66A | - | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_5A-46D-66A | CA_5A-46A CA_5A-66A | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 46 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_5A-48A-66A | CA_48A-66A CA_5A-66A CA_5A-48A | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 48 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_5A-48A-66A-66A | CA_48A-66A CA_5A-66A CA_5A-48A | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 48 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-48C-66A | CA_48A-66A CA_5A-66A CA_5A-48A | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in Table 1.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_5A-48C-66A-66A | CA_48A-66A CA_5A-66A CA_5A-48A | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_5A-48D-66A | CA_48A-66A CA_5A-48A | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 48 | See CA_48D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_5A-48D-66A-66A | CA_48A-66A CA_5A-66A CA_5A-48A | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 110 | 0 |
|  |  | 48 | See CA_48D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7A-8A-40C | - | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 70 | 0 |
|  |  | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 40 | See CA_40C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7A-12A-66A | - | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_7A-12B-66A | - | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 55 | 0 |
|  |  | 12 | See CA_12B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_7A-13A-66A | - | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_7C-13A-66A | - | 7 | See CA_7C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_7A-20A-28A12 | - | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 20 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_7A-20A-32A | CA_7A-20A | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 20 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 32 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_7A-20A-38A8 | - | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 20 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 38 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_7A-26A-66A | - | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 55 | 0 |
|  |  | 26 |  | Yes |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 66 |  | Yes |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_7A-28A-40A | - | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 40 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_7A-28A-40C | - | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 40 | See CA_40C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7A-20A-42A | - | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 20 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 42 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_7A-28A-38A14 | - | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 38 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_7A-29A-66A | - | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 29 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_7A-7A-29A-66A | - | 7 | See CA_7A-7A Bandwidth combination set 1 in table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 29 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_7C-29A-66A | - | 7 | See CA_7C Bandwidth combination set 2 in table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 29 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_7A-30A-66A | - | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 50 | 0 |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_7A-32A-46A | - | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 32 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
| CA_7A-32A-46C | - | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | 80 | 0 |
|  |  | 32 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 | See CA_46C in Table 5.6A.1-1 of TS 36.101 Bandwidth Combination Set 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7A-32A-46D | - | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | 100 | 0 |
|  |  | 32 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 | See CA_46D in Table 5.6A.1-1 of TS 36.101 Bandwidth Combination Set 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7A-32A-46E | - | 7 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  | 120 | 0 |
|  |  | 32 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 46 | See CA_46E in Table 5.6A.1-1 of TS 36.101 Bandwidth Combination Set 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_7A-46A-66A | - | 7 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 60 | 0 |
|  |  | 46 |  |  |  |  |  | Yes |  |  |  |  |  | Yes |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_8A-11A-28A | - | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 40 | 0 |
|  |  | 11 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_8A-11A-42A | - | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 40 | 0 |
|  |  | 11 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_8A-11A-42C | - | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 11 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_8A-20A-28A15 | - | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 20 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_8A-28A-41A | - | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 41 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_8A-39A-41A | - | 8 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 39 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
| CA_12A-30A-66A | - | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 40 | 0 |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_12A-30A-66A-66A | - | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_13A-46A-66A | - | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_13A-46A-66A-66A | - | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_13A-46C-66A | - | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_13A-46C-66A-66A | - | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_13A-46D-66A | CA_13A-66A | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 46 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_13A-46D-66A-66A | - | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 110 | 0 |
|  |  | 46 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_13A-46E-66A | - | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 110 | 0 |
|  |  | 46 | See CA_46E Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_13A-48A-66A | CA_13A-48A CA_13A-66A CA_48A-66A | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 48 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_13A-48A-48A-66A | CA_13A-48A CA_13A-66A CA_48A-66A | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 48 | See CA_48A-48A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_13A-48C-66A | CA_48A-66A CA_13A-66A CA_13A-48A | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 48 | See CA_48C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_13A-48C-66A-66A | CA_48A-66A CA_13A-66A CA_13A-48A | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 48 | See CA_48C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_13A-48D-66A | CA_48A-66A CA_13A-48A | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 48 | See CA_48D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_13A-48D-66A-66A | CA_48A-66A CA_13A-66A CA_13A-48A | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 110 | 0 |
|  |  | 48 | See CA_48D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_13A-48D-66A | - | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 48 | See CA_48D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_13A-48E-66A | - | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 110 | 0 |
|  |  | 48 | See CA_48E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_13A-48A-48C-66A | - | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 48 | See CA_48A-48C Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_13A-48A-66A-66A | CA_48A-66A CA_13A-66A CA_13A-48A | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 48 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_13A-48A-66B | - | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 48 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 66 | See CA_66B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_13A-48A-66C | - | 13 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 48 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 66 | See CA_66C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_14A-30A-66A | CA_14A-30A CA_14A-66A | 14 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 40 | 0 |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_14A-30A-66A-66A | CA_14A-30A CA_14A-66A | 14 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_19A-21A-42A | CA_19A-21A, CA_19A-42A6, CA_21A-42A | 19 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | 50 | 0 |
|  |  | 21 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_19A-21A-42C | CA_19A-21A, CA_19A-42A6, CA_21A-42A | 19 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | 70 | 0 |
|  |  | 21 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_20A-32A-42A | - | 20 |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  | 45 | 0 |
|  |  | 32 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 42 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_20A-32A-43A | - | 20 |  |  |  | Yes |  |  |  |  |  |  |  |  |  |  | 45 | 0 |
|  |  | 32 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 43 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_20A-38A-40A |  | 20 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | 55 | 0 |
|  |  | 38 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 40 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_20A-38A-40A-40A | - | 20 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | 75 | 0 |
|  |  | 38 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 40 | See CA_40A-40A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_20A-38A-40C | - | 20 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | 75 | 0 |
|  |  | 38 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 40 | See CA_40C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_20A-38A-40D | - | 20 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | 95 | 0 |
|  |  | 38 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 40 | See CA_40D Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_21A-28A-42A | CA_21A-28A, CA_21A-42A, CA_28A-42A | 21 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | 45 | 0 |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_21A-28A-42C | CA_21A-28A, CA_21A-42A, CA_28A-42A | 21 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  | 65 | 0 |
|  |  | 28 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_25A-26A-41A | - | 25 |  | Yes |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 55 | 0 |
|  |  | 26 | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_25A-25A-26A-41A | - | 25 | See CA_25A-25A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 65 | 0 |
|  |  | 26 |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_25A-25A-26A-41C | - | 25 | See CA_25A-25A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 85 | 0 |
|  |  | 26 |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 | See CA_41C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_25A-26A-41C | - | 25 |  | Yes |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 75 | 0 |
|  |  | 26 | Yes | Yes |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 41 | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_25A-25A-26A-41D | - | 25 | See CA_25A-25A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 105 | 0 |
|  |  | 26 |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 | See CA_41D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_25A-25A-26A-41E | - | 25 | See CA_25A-25A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 125 | 0 |
|  |  | 26 |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 | See CA_41E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_25A-25A-26A-41F | - | 25 | See CA_25A-25A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 145 | 0 |
|  |  | 26 |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 41 | See CA_41F Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_28A-41A-42A | CA_41A-42A | 28 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 41 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 42 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_28A-41A-42A-42A | - | 28 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 41 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 42 | See CA_42A-42A Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_28A-41A-42C | CA_41A-42A, CA_42C | 28 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 41 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_28A-41A-42A-42C | CA_42C | 28 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 41 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 42 | See CA_42A-42C Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_28A-41A-42C-42C | CA_42C | 28 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 110 | 0 |
|  |  | 41 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 42 | See CA_42C-42C Bandwidth Combination Set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_28A-41C-42A | CA_41A-42A | 28 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 70 | 0 |
|  |  | 41 | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_28A-41C-42C | CA_42C | 28 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 41 | See CA_41C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_29A-30A-66A | - | 29 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 40 | 0 |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_29A-30A-66A-66A | - | 29 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 60 | 0 |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_29A-46A-66A | - | 29 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 50 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_29A-66A-70A | - | 29 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 45 | 0 |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 70 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
| CA_29A-66A-66A-70A | - | 29 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 65 | 0 |
|  |  | 66 | See CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 70 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
| CA_29A-66A-70C | - | 29 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 55 | 0 |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 70 | See CA_70C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_29A-66A-66A-70C | - | 29 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 75 | 0 |
|  |  | 66 | See the CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 70 | See the CA_70C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_29A-66C-70A | - | 29 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 65 | 0 |
|  |  | 66 | See CA_66C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 70 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
| CA_29A-66C-70C | - | 29 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |  |  | 75 | 0 |
|  |  | 66 | See the CA_66C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 70 | See the CA_70C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CA_32A-42A-43A | - | 32 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  | 60 | 0 |
|  |  | 42 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |
|  |  | 43 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |
| CA_46A-48A-66A | CA_48A-66A | 46 |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  | 60 | 0 |
|  |  | 48 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |
|  |  | 66 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |
| CA_46A-48A-71A | - | 46 |  |  |  |  |  |  |  |  |  |  |  |  | Yes |  | 60 | 0 |
|  |  | 48 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |
|  |  | 71 |  |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |
| CA_46C-48A-48A-71A | - | 46 | See CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 48 | See CA_48A-48A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 71 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_46A-48C-66A | CA_48A-66A | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | 80 | 0 |
|  |  | 48 | See the CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_46A-48D-66A | - | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | 100 | 0 |
|  |  | 48 | See the CA_48D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_46A-48E-66A | - | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | 120 | 0 |
|  |  | 48 | See the CA_48E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_46C-48A-66A | CA_48A-66A | 46 | See the CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 48 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_46C-48C-66A | CA_48A-66A | 46 | See the CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 48 | See the CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_46C-48D-66A | - | 46 | See the CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 120 | 0 |
|  |  | 48 | See the CA_48D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_46C-48E-66A | - | 46 | See the CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 140 | 0 |
|  |  | 48 | See the CA_48E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_46D-48A-66A | CA_48A-66A | 46 | See the CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 48 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_46D-48C-66A | CA_48A-66A | 46 | See the CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 120 | 0 |
|  |  | 48 | See the CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_46E-48A-66A | - | 46 | See the CA_46E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 120 | 0 |
|  |  | 48 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_46E-48C-66A | - | 46 | See the CA_46E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 140 | 0 |
|  |  | 48 | See the CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_46A-48A-48A-71A | - | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | 80 | 0 |
|  |  | 48 | See CA_48A-48A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 71 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_46A-48C-71A | - | 46 |  |  |  |  |  |  |  |  |  |  |  | Yes |  |  | 80 | 0 |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 71 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_46C-48A-71A | - | 46 | See CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 48 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
|  |  | 71 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_46C-48C-71A | - | 46 | See CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 71 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_66A-70A-71A | - | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 55 | 0 |
|  |  | 70 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 71 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_66C-70A-71A | - | 66 | See the CA_66C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 75 | 0 |
|  |  | 70 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 71 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_66A-70C-71A | - | 66 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  | 65 | 0 |
|  |  | 70 | See the CA_70C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 71 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_66A-66A-70A-71A | - | 66 | See the CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 75 | 0 |
|  |  | 70 |  |  |  | Yes |  | Yes |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 71 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_66A-66A-70C-71A | - | 66 | See the CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |  |  | 85 | 0 |
|  |  | 70 | See the CA_70C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 71 |  |  |  | Yes |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| CA_66C-70C-71A | - | 66 | See the CA_66C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  | 85 | 0 |
|  |  | 70 | See the CA_70C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 71 |  |  |  |  |  | Yes |  |  | Yes |  |  | Yes |  |  |  |  |
| NOTE 1: The CA Configuration refers to a combination of an operating band and a CA bandwidth class specified in Table 5.6A-1 (the indexing letter). Absence of a CA bandwidth class for an operating band implies support of all classes. NOTE 2: For each band combination, all combinations of indicated bandwidths belong to the set. NOTE 3: For the supported CC bandwidth combinations, the CC downlink and uplink bandwidths are equal. NOTE 4: A terminal which supports a DL CA configuration shall support all the lower order fallback DL CA combinations and it shall support at least one bandwidth combination set for each of the constituent lower order DL combinations containing all the bandwidths specified within each specific combination set of the upper order DL combination. NOTE 5: Uplink CA configurations are the configurations supported by the present release of specifications. NOTE 6: If the UE supports any uplink CA configuration for corresponding downlink CA configuration it shall support this uplink CA configuration. NOTE 7: UL carrier shall be supported in Band 3 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within [6dB]. NOTE 8: UL carrier shall be supported in Band 20 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within [6dB] NOTE 9: UL carrier is only supported on Band 1 or Band 3 not Band 41 because the fall back mode 1UL/2DL CA_1A-41A has the limitation that UL carrier is only supported on Band 1. NOTE 10: UL carrier is only supported on Band 1 or Band 42 not Band 41 because the fall back mode 1UL/2DL CA_1A-41A has the limitation that UL carrier is only supported on Band 1. NOTE 11: UL carrier is only supported on Band 1 or Band 5 not Band 41 because the fall back mode 1UL/2DL CA_1A-41A has the limitation that UL carrier is only supported on Band 1. NOTE 12: Power imbalance between downlink carriers on Band 20 and Band 28 is assumed to be within [6dB]. NOTE 13: UL carrier shall be supported in Band 8 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within [6dB]. NOTE 14: UL carrier shall be supported in Band 28 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within [6dB]. NOTE 15: Power imbalance between downlink carriers on Band 20 and Band 28 is assumed to be within [6dB]. NOTE 16: UL carrier shall be supported in Band 1 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within [6dB]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 5.6A.1-2b: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA (four bands)

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations (NOTE 5) | E-UTRA Bands | 1.4 MHz |  | 3 MHz |  | 5 MHz | 10 MHz | 15 MHz |  | 20 MHz |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3A-5A-7A | CA_1A-3A, CA_1A-5A6, CA_1A-7A, CA_3A-5A, CA_3A-7A, CA_5A-7A | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 70 | 0 |
|  |  | 3 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 5 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 7 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 70 | 1 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 5 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 7 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
| CA_1A-3A-3A-5A-7A | - | 1 |  |  |  |  | Yes | Yes | Yes |  |  |  | 85 | 0 |
|  |  | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 5 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 7 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
| CA_1A-3A-5A-7A-7A | CA_1A-3A, CA_1A-5A6, CA_1A-7A, CA_3A-5A, CA_3A-7A, CA_5A-7A | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 90 | 0 |
|  |  | 3 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 5 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 3 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-5A-28A | - | 1 |  |  |  |  | Yes | Yes | Yes |  |  |  | 65 | 0 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 5 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 28 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-3A-5A-40A | CA_1A-3A, CA_1A-5A6, CA_3A-5A | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 70 | 0 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 5 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 40 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
| CA_1A-3A-5A-41A8 | - | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 70 | 0 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 5 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  |  |  |  | Yes |  |  |  |
| CA_1A-3A-7A-7A-26A | CA_1A-3A, CA_1A-7A, CA_1A-26A, CA_3A-7A, CA_3A-26A, CA_7A-26A | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 95 | 0 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 7 | See the CA_7A-7A Bandwidth combination set 3 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 26 |  |  |  |  | Yes | Yes | Yes |  |  |  |  |  |
| CA_1A-3A-7A-8A | CA_1A-3A, CA_1A-7A, CA_1A-8A, CA_3A-7A, CA_3A-8A, CA_7A-8A | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 70 | 0 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 7 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 8 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 70 | 1 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 7 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 8 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
| CA_1A-3C-7A-8A | - | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 90 | 0 |
|  |  | 3 | See the CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 8 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
| CA_1A-3A-3A-7A-8A | CA_1A-3A, CA_1A-7A, CA_1A-8A, CA_3A-7A, CA_3A-8A, CA_7A-8A | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 90 | 0 |
|  |  | 3 | See the CA_3A-3A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 8 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
| CA_1A-3A-7A-7A-8A | CA_1A-3A, CA_1A-7A, CA_1A-8A, CA_3A-7A, CA_3A-8A, CA_7A-8A | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 90 | 0 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 7 | See the CA_7A-7A Bandwidth combination set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 8 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
| CA_1A-3A-3A-7A-7A-8A | CA_1A-3A, CA_1A-7A, CA_1A-8A, CA_3A-7A, CA_3A-8A, CA_7A-8A | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 110 | 0 |
|  |  | 3 | See the CA_3A-3A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | See the CA_7A-7A Bandwidth combination set 1 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 8 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
| CA_1A-3A-7A-20A | CA_1A-3A, CA_1A-7A, CA_1A-20A, CA_3A-7A, CA_3A-20A, CA_7A-20A | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 80 | 0 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 7 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 20 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 80 | 1 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 7 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 20 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-3A-7C-20A | - | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 100 | 0 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 7 | See CA_7C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 20 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-3C-7A-20A | - | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 100 | 0 |
|  |  | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 20 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-3A-3A-7A-20A | - | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 100 | 0 |
|  |  | 3 | See CA_3A-3A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 20 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-3A-7A-7A-20A | - | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 100 | 0 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 7 | See CA_7A-7A Bandwidth Combination Set 3 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 20 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-3A-7A-26A | CA_1A-3A, CA_1A-7A, CA_1A-26A, CA_3A-7A CA_3A-26A, CA_7A-26A | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 75 | 0 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 7 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 26 |  |  |  |  | Yes | Yes | Yes |  |  |  |  |  |
| CA_1A-3A-7A-28A | CA_1A-3A, CA_1A-7A, CA_1A-28A, CA_3A-7A, CA_3A-28A6, CA_7A-28A | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 80 | 0 |
|  |  | 3 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 7 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 28 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 80 | 1 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 7 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 28 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-3C-7A-28A | CA_3C | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 100 | 0 |
|  |  | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 28 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-3A-7C-28A | CA_1A-3A, CA_1A-7A, CA_1A-28A, CA_3A-7A, CA_3A-28A6, CA_7A-28A, CA_7C | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 100 | 0 |
|  |  | 3 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 7 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
| CA_1A-3C-7C-28A | CA_3C CA_7C | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 120 | 0 |
|  |  | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | See CA_7C Bandwidth Combination Set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-1A-3A-7A-28A | - | 1 | See CA_1A-1A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 7 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 28 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-1A-3A-7C-28A | CA_7C | 1 | See CA_1A-1A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  | 120 | 0 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 7 | See CA_7C Bandwidth combination set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-1A-3C-7A-28A | CA_3C | 1 | See CA_1A-1A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  | 120 | 0 |
|  |  | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 28 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-1A-3C-7C-28A | CA_3C CA_7C | 1 | See CA_1A-1A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  | 140 | 0 |
|  |  | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | See CA_7C Bandwidth combination set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-1A-3A-3A-7A-28A | - | 1 | See CA_1A-1A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  | 120 | 0 |
|  |  | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 28 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-1A-3A-3A-7C-28A | CA_7C | 1 | See CA_1A-1A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  | 140 | 0 |
|  |  | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | See CA_7C Bandwidth combination set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-3A-3A-7A-28A | - | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 100 | 0 |
|  |  | 3 | See CA_3A-3A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 28 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-3A-3A-7C-28A | CA_7C | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 120 | 0 |
|  |  | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 | See CA_7C Bandwidth combination set 2 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-3A-7A-7A-28A | - | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 100 | 0 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 7 | See CA_7A-7A Bandwidth combination set 3 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
| CA_1A-3A-7A-32A | - | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 80 | 0 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 7 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 32 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-3A-7A-38A9 | - | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 80 | 0 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 7 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 38 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-3C-7A-38A9 | - | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 100 | 0 |
|  |  | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 7 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 38 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-3A-7A-40A | - | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 80 | 0 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 7 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 40 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-3A-7A-40C | - | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 100 | 0 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 7 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 40 | See CA_40C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-7A-42A | - | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 80 | 0 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 7 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 42 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-3A-7A-46A | - | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 80 | 0 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 7 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 46 |  |  |  |  |  |  |  |  | Yes |  |  |  |
| CA_1A-3A-7A-46C | - | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 100 | 0 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 7 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-7A-46D | - | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 120 | 0 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 7 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 46 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-7A-46E | - | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 140 | 0 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 7 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 46 | See CA_46E Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-8A-40A | CA_1A-3A, CA_1A-8A, CA_3A-8A | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 70 | 0 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 8 |  |  | Yes |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 40 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-3A-8A-11A | - | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 60 | 0 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 8 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 11 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
| CA_1A-3A-8A-20A | - | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 70 | 0 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 8 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 20 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-3A-8A-28A | - | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 70 | 0 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 8 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 28 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-3A-8A-38A | CA_1A-3A CA_1A-8A CA_3A-8A | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 70 | 0 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 8 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 38 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-3A-8A-42A | - | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 70 | 0 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 8 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-3A-8A-42C | - | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 90 | 0 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 8 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-11A-28A | - | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 70 | 0 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 11 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 28 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-3A-18A-42A | - | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 75 | 0 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 18 |  |  |  |  | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 42 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-3A-18A-42C | - | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 95 | 0 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 18 |  |  |  |  | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-19A-21A | CA_1A-3A, CA_1A-19A6, CA_1A-21A, CA_3A-19A, CA_3A-21A, CA_19A-21A | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 70 | 0 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 19 |  |  |  |  | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 21 |  |  |  |  | Yes | Yes | Yes |  |  |  |  |  |
| CA_1A-3A-19A-42A | CA_1A-3A, CA_1A-19A6, CA_1A-42A, CA_3A-19A, CA_3A-42A, CA_19A-42A6 | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 75 | 0 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 19 |  |  |  |  | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 42 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-3A-3A-19A-21A | CA_1A-3A CA_1A-19A6 CA_1A-21A, CA_3A-19A CA_3A-21A CA_19A-21A | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 90 | 0 |
|  |  | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 19 |  |  |  |  | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 21 |  |  |  |  | Yes | Yes | Yes |  |  |  |  |  |
| CA_1A-3A-19A-42C | CA_1A-3A, CA_1A-19A6, CA_1A-42A, CA_3A-19A, CA_3A-42A, CA_19A-42A6 | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 95 | 0 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 19 |  |  |  |  | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-20A-28A7 | - | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 80 | 0 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 20 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 28 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-3A-3A-20A-28A7 | - | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 100 | 0 |
|  |  | 3 | See CA_3A-3A Bandwidth combination set 0 in in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 20 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 28 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-3A-20A-32A | - | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 70 | 0 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 20 |  |  |  |  |  | Yes |  |  |  |  |  |  |
|  |  | 32 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 1 |  |  |  |  | Yes | Yes | Yes |  |  |  | 55 | 1 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 20 |  |  |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 32 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-3A-20A-42A | - | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 80 | 0 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 20 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 42 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-3A-20A-43A | - | 1 |  |  |  |  | Yes | Yes | Yes |  |  |  | 55 | 0 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 20 |  |  |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 43 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-3A-21A-28A | CA_1A-3A, CA_1A-21A, CA_1A-28A, CA_3A-21A, CA_3A-28A6, CA_21A-28A | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 65 | 0 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 21 |  |  |  |  | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 28 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
| CA_1A-3A-21A-42A | CA_1A-3A, CA_1A-21A, CA_1A-42A, CA_3A-21A, CA_3A-42A, CA_21A-42A | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 75 | 0 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 21 |  |  |  |  | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 42 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-3A-21A-42C | CA_1A-3A, CA_1A-21A, CA_1A-42A, CA_3A-21A, CA_3A-42A, CA_21A-42A | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 95 | 0 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 21 |  |  |  |  | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-28A-40A | - | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 80 | 0 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 28 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 40 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-3A-28A-40C | - | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 100 | 0 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 28 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 40 | See CA_40C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-28A-42A | CA_1A-3A, CA_1A-28A, CA_1A-42A, CA_3A-28A6, CA_3A-42A, CA_28A-42A | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 70 | 0 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 28 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-3A-28A-42C | CA_1A-3A, CA_1A-28A, CA_1A-42A, CA_3A-28A6, CA_3A-42A, CA_28A-42A | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 90 | 0 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 28 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-32A-42A | - | 1 |  |  |  |  | Yes | Yes | Yes |  |  |  | 70 | 0 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 32 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 42 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-3A-32A-43A | - | 1 |  |  |  |  | Yes | Yes | Yes |  |  |  | 70 | 0 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 32 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 43 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-3A-41A-42A | CA_1A-3A CA_1A-42A CA_3A-42A | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 80 | 0 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 41 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 42 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-3A-41C-42A | CA_1A-3A CA_1A-42A CA_3A-42A | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 100 | 0 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 41 | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-3A-41A-42C | CA_1A-3A CA_1A-42A CA_1A-42C CA_3A-42A CA_3A-42C CA_42C | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 100 | 0 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 41 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-41C-42C | CA_1A-3A, CA_1A-42A, CA_1A-42C, CA_3A-42A, CA_3A-42C CA_42C | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 120 | 0 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 41 | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-3A-42A-43A | - | 1 |  |  |  |  | Yes | Yes | Yes |  |  |  | 70 | 0 |
|  |  | 3 |  |  |  |  | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 42 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 43 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-5A-7A-28A | - | 1 |  |  |  |  | Yes | Yes | Yes |  |  |  | 65 | 0 |
|  |  | 5 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 7 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 28 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-20A-32A-42A | - | 1 |  |  |  |  | Yes | Yes | Yes |  |  |  | 60 | 0 |
|  |  | 20 |  |  |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 32 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 42 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-20A-32A-43A | - | 1 |  |  |  |  | Yes | Yes | Yes |  |  |  | 60 | 0 |
|  |  | 20 |  |  |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 32 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 43 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-7A-8A-20A | - | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 70 | 0 |
|  |  | 7 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 8 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 20 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
| CA_1A-7A-8A-40A | - | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 70 | 0 |
|  |  | 7 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 8 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 40 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-5A-7A-46A | CA_1A-5A6, CA_1A-7A, CA_5A-7A | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 70 | 0 |
|  |  | 5 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 7 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 46 |  |  |  |  |  |  |  |  | Yes |  |  |  |
| CA_1A-5A-7A-46C | - | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 90 | 0 |
|  |  | 5 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 7 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 46 | See CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-7A-8A-40C | - | 1 |  |  |  |  | Yes | Yes |  | Yes |  | Yes | 90 | 0 |
|  |  | 7 |  |  |  |  |  | Yes |  | Yes |  | Yes |  |  |
|  |  | 8 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 40 | See CA_40C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-7A-20A-28A7 | - | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 80 | 0 |
|  |  | 7 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 20 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 28 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-7A-20A-32A | - | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 70 | 0 |
|  |  | 7 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 20 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 32 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-7A-20A-42A | - | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 80 | 0 |
|  |  | 7 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 20 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 42 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-7A-28A-40A | - | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 80 | 0 |
|  |  | 7 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 28 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 40 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-7A-28A-40C | - | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 100 | 0 |
|  |  | 7 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 28 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 40 | See CA_40C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-8A-11A-28A | - | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 60 | 0 |
|  |  | 8 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 11 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 28 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-8A-11A-42A | - | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 60 | 0 |
|  |  | 8 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 11 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-8A-11A-42C | - | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 80 | 0 |
|  |  | 8 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 11 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-8A-20A-28A | - | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 70 | 0 |
|  |  | 8 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 20 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 28 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-19A-21A-42A | CA_1A-19A6, CA_1A-21A, CA_1A-42A, CA_19A-21A, CA_19A-42A6, CA_21A-42A | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 70 | 0 |
|  |  | 19 |  |  |  |  | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 21 |  |  |  |  | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 42 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-19A-21A-42C | CA_1A-19A6, CA_1A-21A, CA_1A-42A, CA_19A-21A, CA_19A-42A6, CA_21A-42A | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 90 | 0 |
|  |  | 19 |  |  |  |  | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 21 |  |  |  |  | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-21A-28A-42A | CA_1A-21A, CA_1A-28A, CA_1A-42A, CA_21A-28A, CA_21A-42A, CA_28A-42A | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 65 | 0 |
|  |  | 21 |  |  |  |  | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 28 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_1A-21A-28A-42C | CA_1A-21A, CA_1A-28A, CA_1A-42A, CA_21A-28A, CA_21A-42A, CA_28A-42A | 1 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 85 | 0 |
|  |  | 21 |  |  |  |  | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 28 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
| CA_1A-32A-42A-43A | - | 1 |  |  |  |  | Yes | Yes | Yes |  |  |  | 75 | 0 |
|  |  | 32 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 42 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 43 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_2A-2A-5A-12A-66A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  |  |
| CA_2A-2A-5A-30A-66A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 5 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |
|  |  | 30 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  |  |
| CA_2A-2A-7A-12A-66A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 7 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  |  |
|  |  | 12 |  |  |  | Yes |  | Yes |  |  |  |  |  |  |
|  |  | 66 |  |  |  | Yes |  | Yes |  | Yes |  | Yes |  |  |
| CA_2A-2A-12A-30A-66A | - | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 12 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 30 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 66 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_2A-2A-14A-30A-66A | CA_2A-14A CA_14A-30A CA_14A-66A | 2 | See CA_2A-2A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  | 80 | 0 |
|  |  | 14 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 30 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 66 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_2A-4A-5A-12A | - | 2 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 60 | 0 |
|  |  | 4 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 5 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 12 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
| CA_2A-4A-5A-29A | CA_2A-4A | 2 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 60 | 0 |
|  |  | 4 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 5 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 29 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
| CA_2A-4A-5A-30A | - | 2 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 60 | 0 |
|  |  | 4 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 5 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 30 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
| CA_2A-4A-5B-30A | - | 2 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 70 | 0 |
|  |  | 4 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 5 | See CA_5B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
| CA_2A-4A-7A-12A | - | 2 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 70 | 0 |
|  |  | 4 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 7 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 12 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
| CA_2A-4A-12A-30A | - | 2 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 60 | 0 |
|  |  | 4 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 12 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 30 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
| CA_2A-4A-29A-30A | - | 2 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 60 | 0 |
|  |  | 4 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 29 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 30 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
| CA_2A-5A-7A-28A | - | 2 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 70 | 0 |
|  |  | 5 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 7 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 28 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_2A-5A-12A-66A | - | 2 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 60 | 0 |
|  |  | 5 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 12 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 66 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_2A-5A-30A-66A | - | 2 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 60 | 0 |
|  |  | 5 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 30 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 66 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_2A-5A-30A-66A-66A | - | 2 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 80 | 0 |
|  |  | 5 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 30 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-5B-30A-66A | - | 2 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 70 | 0 |
|  |  | 5 | See CA_5B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 30 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 66 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_2A-5A-46A-66A | - | 2 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 70 | 0 |
|  |  | 5 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 46 |  |  |  |  |  |  |  |  | Yes |  |  |  |
|  |  | 66 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_2A-5A-46C-66A | - | 2 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 90 | 0 |
|  |  | 5 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 46 | See CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_2A-5A-46D-66A | - | 2 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 110 | 0 |
|  |  | 5 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_2A-5A-46E-66A | - | 2 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 130 | 0 |
|  |  | 5 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 46 | See CA_46E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_2A-5A-46A-66A-66A | - | 2 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 90 | 0 |
|  |  | 5 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 46 |  |  |  |  |  |  |  |  | Yes |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-5A-46C-66A-66A | - | 2 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 110 | 0 |
|  |  | 5 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 46 | See CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-5A-46D-66A-66A | - | 2 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 130 | 0 |
|  |  | 5 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-5A-48A-66A | CA_2A-66A CA_2A-48A CA_48A-66A CA_5A-66A CA_5A-48A CA_2A-5A | 2 | Yes |  | Yes |  | Yes | Yes | Yes |  | Yes |  | 70 | 0 |
|  |  | 5 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 48 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 66 | Yes |  | Yes |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_2A-5A-48A-66A-66A | CA_2A-66A CA_2A-48A CA_48A-66A CA_5A-66A CA_5A-48A CA_2A-5A | 2 | Yes |  | Yes |  | Yes | Yes | Yes |  | Yes |  | 90 | 0 |
|  |  | 5 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 48 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-5A-48C-66A | CA_2A-66A CA_2A-48A CA_48A-66A CA_5A-66A CA_5A-48A CA_2A-5A | 2 | Yes |  | Yes |  | Yes | Yes | Yes |  | Yes |  | 90 | 0 |
|  |  | 5 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in Table 5.6A1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | Yes |  | Yes |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_2A-5A-48C-66A-66A | CA_2A-66A CA_2A-48A CA_48A-66A CA_5A-66A CA_5A-48A | 2 | Yes |  | Yes |  | Yes | Yes | Yes |  | Yes |  | 110 | 0 |
|  |  | 5 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in Table 5.6A1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-5A-48D-66A | CA_2A-66A CA_2A-48A CA_48A-66A CA_5A-66A CA_5A-48A CA_2A-5A | 2 | Yes |  | Yes |  | Yes | Yes | Yes |  | Yes |  | 110 | 0 |
|  |  | 5 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 48 | See CA_48D Bandwidth combination set 0 in Table 5.6A1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | Yes |  | Yes |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_2A-5A-48D-66A-66A | CA_2A-66A CA_2A-48A CA_48A-66A CA_5A-66A CA_5A-48A CA_2A-5A | 2 | Yes |  | Yes |  | Yes | Yes | Yes |  | Yes |  | 130 | 0 |
|  |  | 5 | Yes |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 48 | See CA_48D Bandwidth combination set 0 in Table 5.6A1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-7A-12A-66A | - | 2 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 70 | 0 |
|  |  | 7 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 12 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 66 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_2A-7A-12B-66A | - | 2 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 75 | 0 |
|  |  | 7 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 12 | See CA_12B Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_2A-7A-13A-66A | - | 2 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 70 | 0 |
|  |  | 7 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 13 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 66 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_2A-7C-13A-66A | - | 2 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 90 | 0 |
|  |  | 7 | See CA_7C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 13 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 66 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_2A-7A-26A-66A | - | 2 |  |  | Yes |  | Yes | Yes | Yes |  | Yes |  | 75 | 0 |
|  |  | 7 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 26 |  |  | Yes |  | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 66 |  |  | Yes |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_2A-7A-29A-66A | - | 2 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 70 | 0 |
|  |  | 7 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 29 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 66 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_2A-7C-29A-66A | - | 2 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 90 | 0 |
|  |  | 7 | See CA_7C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 29 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 66 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_2A-7A-7A-29A-66A | - | 2 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 90 | 0 |
|  |  | 7 | See CA_7A-7A Bandwidth combination set 3 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 29 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 66 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_2A-7A-46A-66A | - | 2 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 80 | 0 |
|  |  | 7 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 46 |  |  |  |  |  | Yes |  |  | Yes |  |  |  |
|  |  | 66 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_2A-12A-30A-66A | - | 2 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 60 | 0 |
|  |  | 12 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 30 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 66 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_2A-12A-30A-66A-66A | - | 2 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 80 | 0 |
|  |  | 12 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 30 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-13A-46A-66A | CA_2A-13A | 2 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 70 | 0 |
|  |  | 13 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 46 |  |  |  |  |  |  |  |  | Yes |  |  |  |
|  |  | 66 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_2A-13A-46C-66A | CA_2A-13A | 2 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 90 | 0 |
|  |  | 13 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 46 | See CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_2A-13A-46D-66A | CA_2A-13A | 2 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 110 | 0 |
|  |  | 13 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_2A-5A-46E-66A | - | 2 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 130 | 0 |
|  |  | 5 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 46 | See CA_46E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_2A-13A-46A-66A-66A | - | 2 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 90 | 0 |
|  |  | 13 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 46 |  |  |  |  |  |  |  |  | Yes |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-13A-46C-66A-66A | - | 2 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 110 | 0 |
|  |  | 13 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 46 | See CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-13A-46D-66A-66A | - | 2 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 130 | 0 |
|  |  | 13 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 46 | See CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth combination set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-13A-48A-66A | CA_2A-13A CA_2A-66A CA_2A-48A CA_48A-66A CA_13A-66A CA_13A-48A | 2 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 70 | 0 |
|  |  | 13 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 48 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 66 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_2A-13A-48A-66A-66A | CA_2A-66A CA_2A-48A CA_48A-66A CA_13A-66A CA_13A-48A | 2 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 90 | 0 |
|  |  | 13 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 48 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-13A-48C-66A | CA_2A-13A CA_2A-48A CA_2A-66A CA_13A-66A CA_13A-48A CA_48A-66A | 2 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 90 | 0 |
|  |  | 13 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_2A-13A-48C-66A-66A | CA_2A-66A CA_2A-48A CA_48A-66A CA_13A-66A CA_13A-48A | 2 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 110 | 0 |
|  |  | 13 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 48 | See CA_48C Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-13A-48D-66A | CA_2A-66A CA_2A-48A CA_48A-66A CA_13A-66A CA_13A-48A | 2 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 110 | 0 |
|  |  | 13 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 48 | See CA_48D Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_2A-13A-48D-66A-66A | CA_2A-66A CA_2A-48A CA_48A-66A CA_13A-66A CA_13A-48A | 2 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 130 | 0 |
|  |  | 13 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 48 | See CA_48D Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-13A-46E-66A | CA_2A-13A | 2 | Yes |  | Yes |  | Yes | Yes | Yes |  | Yes |  | 130 | 0 |
|  |  | 13 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 46 | See the CA_46E Bandwidth combination set 0 in the Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_2A-13A-48A-48A-66A | CA_2A-13A CA_13A-66A | 2 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 90 | 0 |
|  |  | 13 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 48 | See CA_48A-48A Bandwidth combination set 0 in the Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_2A-14A-30A-66A | CA_2A-14A CA_14A-30A CA_14A-66A | 2 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 60 | 0 |
|  |  | 14 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 30 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 66 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_2A-14A-30A-66A-66A | CA_2A-14A CA_14A-30A CA_14A-66A | 2 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 80 | 0 |
|  |  | 14 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 30 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 66 | See CA_66A-66A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  |  |  |
| CA_2A-29A-30A-66A | - | 2 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 60 | 0 |
|  |  | 29 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 30 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 66 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_2A-46A-48A-66A | CA_2A-48A CA_48A-66A | 2 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 80 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  | Yes |  |  |  |
|  |  | 48 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 66 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_2A-46A-48C-66A | CA_2A-48A CA_48A-66A | 2 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 100 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  | Yes |  |  |  |
|  |  | 48 | See the CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_2A-46A-48D-66A | - | 2 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 120 | 0 |
|  |  | 46 |  |  |  |  |  |  |  |  | Yes |  |  |  |
|  |  | 48 | See the CA_48D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_2A-46C-48A-66A | CA_2A-48A CA_48A-66A | 2 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 100 | 0 |
|  |  | 46 | See the CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 66 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_2A-46C-48C-66A | CA_2A-48A CA_48A-66A | 2 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 120 | 0 |
|  |  | 46 | See the CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 | See the CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_2A-46C-48D-66A | - | 2 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 140 | 0 |
|  |  | 46 | See the CA_46C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 | See the CA_48D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_2A-46D-48A-66A | CA_2A-48A CA_48A-66A | 2 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 120 | 0 |
|  |  | 46 | See the CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 66 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_2A-46D-48C-66A | CA_2A-48A CA_48A-66A | 2 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 140 | 0 |
|  |  | 46 | See the CA_46D Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 | See the CA_48C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 66 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_2A-46E-48A-66A | CA_2A-48A CA_48A-66A | 2 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 140 | 0 |
|  |  | 46 | See CA_46E Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 48 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 66 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_3A-5A-7A-28A | - | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 70 | 0 |
|  |  | 5 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 7 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 28 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_3A-5A-7C-28A | - | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 90 | 0 |
|  |  | 5 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 7 | See CA_7C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 28 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_3A-3A-5A-7A-28A | - | 3 | See CA_3A-3A Bandwidth Combination Set 0 in Table 5.6A.1-3 |  |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 5 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 7 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 28 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_3A-7A-8A-20A | - | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 70 | 0 |
|  |  | 7 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 8 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 20 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_3A-7A-8A-38A9 | - | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 70 | 0 |
|  |  | 7 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 8 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 38 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_3C-7A-8A-38A1 | - | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  | 90 | 0 |
|  |  | 7 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 8 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 38 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_3A-7A-8A-40A | - | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 70 | 0 |
|  |  | 7 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 8 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 40 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_3A-7A-8A-40C | - | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 90 | 0 |
|  |  | 7 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 8 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 40 | See CA_40C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-7A-20A-28A7 | - | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 80 | 0 |
|  |  | 7 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 20 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 28 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_3C-7A-20A-28A7 | - | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 7 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 20 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 28 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
| CA_3A-7A-20A-32A | CA_3A-7A, CA_3A-20A, CA_7A-20A | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 80 | 0 |
|  |  | 7 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 20 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 32 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_3A-7A-20A-42A | - | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 80 | 0 |
|  |  | 7 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 20 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 42 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_3A-7A-28A-38A9 | - | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 80 | 0 |
|  |  | 7 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 28 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 38 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_3C-7A-28A-38A9 | - | 3 | See CA_3C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  | 100 | 0 |
|  |  | 7 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 28 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 38 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_3A-7A-28A-40A | - | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 80 | 0 |
|  |  | 7 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 28 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 40 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_3A-7A-28A-40C | - | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 100 | 0 |
|  |  | 7 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 28 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 40 | See CA_40C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-7A-32A-46A | - | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 80 | 0 |
|  |  | 7 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 32 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 46 |  |  |  |  |  |  |  |  | Yes |  |  |  |
| CA_3A-7A-32A-46C | - | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 100 | 0 |
|  |  | 7 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 32 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 46 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-7A-32A-46D | - | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 120 | 0 |
|  |  | 7 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 32 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 46 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-7A-32A-46E | - | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 140 | 0 |
|  |  | 7 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 32 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 46 | See CA_46E of Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-8A-11A-28A | - | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 60 | 0 |
|  |  | 8 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 11 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 28 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_3A-8A-20A-28A | - | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 70 | 0 |
|  |  | 8 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 20 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 28 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_3A-19A-21A-42A | - | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 70 | 0 |
|  |  | 19 |  |  |  |  | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 21 |  |  |  |  | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 42 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_3A-19A-21A-42C | - | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 90 | 0 |
|  |  | 19 |  |  |  |  | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 21 |  |  |  |  | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-20A-32A-42A | - | 3 |  |  |  |  | Yes | Yes | Yes |  |  |  | 60 | 0 |
|  |  | 20 |  |  |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 32 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 42 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_3A-20A-32A-43A | - | 3 |  |  |  |  | Yes | Yes | Yes |  |  |  | 60 | 0 |
|  |  | 20 |  |  |  |  | Yes |  |  |  |  |  |  |  |
|  |  | 32 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 43 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_3A-21A-28A-42A | - | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 65 | 0 |
|  |  | 21 |  |  |  |  | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 28 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| CA_3A-21A-28A-42C | - | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 85 | 0 |
|  |  | 21 |  |  |  |  | Yes | Yes | Yes |  |  |  |  |  |
|  |  | 28 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-28A-41A-42A | CA_3A-41A, CA_41A-42A | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 70 | 0 |
|  |  | 28 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 42 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
| CA_3A-28A-41A-42C | CA_42C | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 90 | 0 |
|  |  | 28 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 41 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
|  |  | 42 | See CA_42C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-28A-41C-42A | - | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 90 | 0 |
|  |  | 28 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 41 | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 |  |  |  |  |  | Yes | Yes |  | Yes |  |  |  |
| CA_3A-28A-41C-42C | CA_42C | 3 |  |  |  |  | Yes | Yes | Yes |  | Yes |  | 110 | 0 |
|  |  | 28 |  |  |  |  | Yes | Yes |  |  |  |  |  |  |
|  |  | 41 | See the CA_41C Bandwidth combination set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
|  |  | 42 | See the CA_42C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |  |  |  |  |
| CA_3A-32A-42A-43A | - | 3 |  |  |  |  | Yes | Yes | Yes |  |  |  | 75 | 0 |
|  |  | 32 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 42 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
|  |  | 43 |  |  |  |  | Yes | Yes | Yes |  | Yes |  |  |  |
| NOTE 1: The CA Configuration refers to a combination of an operating band and a CA bandwidth class specified in Table 5.6A-1 (the indexing letter). Absence of a CA bandwidth class for an operating band implies support of all classes. NOTE 2: For each band combination, all combinations of indicated bandwidths belong to the set. NOTE 3: For the supported CC bandwidth combinations, the CC downlink and uplink bandwidths are equal. NOTE 4: A terminal which supports a DL CA configuration shall support all the lower order fallback DL CA combinations and it shall support at least one bandwidth combination set for each of the constituent lower order DL combinations containing all the bandwidths specified within each specific combination set of the upper order DL combination. NOTE 5: Uplink CA configurations are the configurations supported by the present release of specifications. NOTE 6: If the UE supports any uplink CA configuration for corresponding downlink CA configuration it shall support this uplink CA configuration. NOTE 7: Power imbalance between downlink carriers on Band 20 and Band 28 is assumed to be within [6dB]. NOTE 8: UL carrier is only supported on Band 1, Band 3 or Band 5 not Band 41 because the fall back mode 2DL/1UL CA_1A-41A has the limitation that UL carrier is only supported on Band 1. NOTE 9: UL carrier shall be supported in Band 1, 3, 8 or 28 only. Power imbalance between downlink carriers on Band 7 and Band 38 is assumed to be within [6dB]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 5.6A.1-2c: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA (five bands)

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations (NOTE 5) | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-3A-5A-7A-28A | - | 1 |  |  | Yes | Yes | Yes |  | 85 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_1A-3A-7A-8A-20A | - | 1 |  |  | Yes | Yes | Yes | Yes | 90 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 20 |  |  |  | Yes | Yes | Yes |  |  |
| CA_1A-3A-7A-20A-28A7 | - | 1 |  |  | Yes | Yes | Yes | Yes | 100 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 20 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_1A-3A-7A-20A-32A | - | 1 |  |  | Yes | Yes | Yes | Yes | 100 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 32 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_1A-3A-7A-20A-42A | - | 1 |  |  | Yes | Yes | Yes | Yes | 100 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_1A-3A-8A-11A-28A | - | 1 |  |  | Yes | Yes | Yes | Yes | 80 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 11 |  |  | Yes | Yes |  |  |  |  |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_1A-3A-20A-32A-42A | - | 1 |  |  | Yes | Yes | Yes |  | 75 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 20 |  |  | Yes |  |  |  |  |  |
|  |  | 32 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_1A-3A-20A-32A-43A | - | 1 |  |  | Yes | Yes | Yes |  | 75 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 20 |  |  | Yes |  |  |  |  |  |
|  |  | 32 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 43 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_1A-3A-32A-42A-43A | - | 1 |  |  | Yes | Yes | Yes |  | 90 | 0 |
|  |  | 3 |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 32 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 43 |  |  | Yes | Yes | Yes | Yes |  |  |
| NOTE 1: The CA Configuration refers to a combination of an operating band and a CA bandwidth class specified in Table 5.6A-1 (the indexing letter). Absence of a CA bandwidth class for an operating band implies support of all classes. NOTE 2: For each band combination, all combinations of indicated bandwidths belong to the set. NOTE 3: For the supported CC bandwidth combinations, the CC downlink and uplink bandwidths are equal. NOTE 4: A terminal which supports a DL CA configuration shall support all the lower order fallback DL CA combinations and it shall support at least one bandwidth combination set for each of the constituent lower order DL combinations containing all the bandwidths specified within each specific combination set of the upper order DL combination. NOTE 5: Uplink CA configurations are the configurations supported by the present release of specifications. NOTE 6: If the UE supports uplink CA for corresponding downlink CA it shall support this uplink CA configuration. NOTE 7: Power imbalance between downlink carriers on Band 20 and Band 28 is assumed to be within [6dB]. |  |  |  |  |  |  |  |  |  |  |

Table 5.6A.1-3: E-UTRA CA configurations and bandwidth combination sets defined for non-contiguous intra-band CA (with two sub-blocks)

|  |  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRACA configuration | Uplink CA configurations (NOTE 1) | Component carriers in order of increasing carrier frequency |  |  |  |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_1A-1A | - | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  |  |  |  |  | 40 | 0 |
| CA_2A-2A | - | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  |  |  |  |  | 40 | 0 |
| CA_3A-3A | - | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  |  |  |  |  | 40 | 0 |
|  |  | 5, 10 | 5, 10, 15, 20 |  |  |  |  |  |  | 30 | 1 |
|  |  | 5 | 3 |  |  |  |  |  |  | 10 | 2 |
|  |  | 3, 5 | 5 |  |  |  |  |  |  |  |  |
| CA_4A-4A | CA_4A-4A | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  |  |  |  |  | 40 | 0 |
|  |  | 5, 10 | 5, 10 |  |  |  |  |  |  | 20 | 1 |
| CA_5A-5A | - | 5,10 | 5,10 |  |  |  |  |  |  | 20 | 0 |
|  |  | 3 | 5 |  |  |  |  |  |  | 8 | 1 |
| CA_7A-7A | - | 5 | 15 |  |  |  |  |  |  | 40 | 0 |
|  |  | 10 | 10, 15 |  |  |  |  |  |  |  |  |
|  |  | 15 | 15, 20 |  |  |  |  |  |  |  |  |
|  |  | 20 | 20 |  |  |  |  |  |  |  |  |
|  |  | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  |  |  |  |  | 40 | 1 |
|  |  | 5, 10, 15, 20 | 5, 10 |  |  |  |  |  |  | 30 | 2 |
|  |  | 10, 15, 20 | 10, 15, 20 |  |  |  |  |  |  | 40 | 3 |
| CA_12A-12A | - | 5 | 5 |  |  |  |  |  |  | 10 | 0 |
| CA_23A-23A | - | 5 | 10 |  |  |  |  |  |  | 15 | 0 |
| CA_25A-25A | - | 5, 10 | 5, 10 |  |  |  |  |  |  | 20 | 0 |
|  |  | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  |  |  |  |  | 40 | 1 |
| CA_40A-40A | - | 10, 20 | 10, 20 |  |  |  |  |  |  | 40 | 0 |
|  |  | 10,15,20 | 10,15,20 |  |  |  |  |  |  | 40 | 1 |
| CA_40A-40C | CA_40C | 20 | See CA_40C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  | 60 | 0 |
|  |  | See CA_40C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  | 20 |  |  |  |  |
| CA_40C-40C | CA_40C | See CA_40C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  | See CA_40C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  | 80 | 0 |
| CA_41A-41A | - | 10, 15, 20 | 10, 15, 20 |  |  |  |  |  |  | 40 | 0 |
|  |  | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  |  |  |  |  | 40 | 1 |
| CA_41A-41C | CA_41C | 5, 10, 15, 20 | See CA_41C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  |  |  | 60 | 0 |
|  |  | See CA_41C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  | 5, 10, 15, 20 |  |  |  |  |
| CA_41A-41D | CA_41C | 5, 10, 15, 20 | See CA_41D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  | 80 | 0 |
|  |  | See CA_41D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 5, 10, 15, 20 |  |  |  |
| CA_41C-41C | CA_41C | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  | 80 | 0 |
| CA_41C-41D | CA_41C | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  | See CA_41D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  | 100 | 0 |
|  |  | See CA_41D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | See CA_41C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |
| CA_42A-42A | - | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  |  |  |  |  | 40 | 0 |
|  |  | 10, 15, 20 | 10, 15, 20 |  |  |  |  |  |  | 40 | 1 |
| CA_42A-42C | CA_42C | 5, 10, 15, 20 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  | 60 | 0 |
|  |  | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  | 5, 10, 15, 20 |  |  |  |  |
|  |  | 10, 15, 20 |  |  | See CA_42C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  | 60 | 1 |
|  |  | See CA_42C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  | 10, 15, 20 |  |  |  |  |
| CA_42A-42D | - | 5, 10, 15, 20 | See CA_42D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  | 80 | 0 |
|  |  | See CA_42D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 5, 10, 15, 20 |  |  |  |
| CA_42C-42C | CA_42C | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  | 80 | 0 |
|  |  | See CA_42C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  |  |  | See CA_42C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  | 80 | 1 |
| CA_43A-43A | - | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  |  |  |  |  | 40 | 0 |
| CA_46A-46A2 | - | 20 | 20 |  |  |  |  |  |  | 40 | 0 |
| CA_46A-46C2 | - | 20 | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  | 60 | 0 |
|  |  | See CA_46C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  | 20 |  |  |  |  |
| CA_46A-46D2 | - | 20 | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  | 80 | 0 |
|  |  | See CA_46D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 20 |  |  |  |
| CA_48A-48A | - | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  |  |  |  |  | 40 | 0 |
| CA_48A-48C | - | 5, 10, 15, 20 | See CA_48C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  | 60 | 0 |
|  |  | See CA_48C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  | 5, 10, 15, 20 |  |  |  |  |
| CA_48A-48D | - | 5, 10, 15, 20 | See CA_48D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  | 80 | 0 |
|  |  | See CA_48D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 5, 10, 15, 20 |  |  |  |
| CA_48C-48C | CA_48C | See CA_48C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  | See CA_48C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  | 80 | 0 |
| CA_48C-48D | - | See CA_48C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  | See CA_48D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  | 100 | 0 |
|  |  | See CA_48D Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | See CA_48C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |
| CA_48A-48E | - | 5, 10, 15, 20 |  | See CA_48E Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  | 100 | 0 |
|  |  | See CA_48E Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  | 5, 10, 15, 20 |  |  |
| CA_66A-66A | - | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  |  |  |  |  | 40 | 0 |
| CA_66A-66B | - | 5, 10, 15, 20 | See CA_66B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  | 40 | 0 |
|  |  | See CA_66B Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  | 5, 10, 15, 20 |  |  |  |  |
| CA_66A-66C | - | 5, 10, 15, 20 | See CA_66C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  | 60 | 0 |
|  |  | See CA_66C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  | 5, 10, 15, 20 |  |  |  |  |
| NOTE 1: Uplink CA configurations are the configurations supported by the present release of specifications. NOTE 2: Restricted to E-UTRA operation when inter-band carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell. |  |  |  |  |  |  |  |  |  |  |  |

Table 5.6A.1-4: E-UTRA CA configurations and bandwidth combination sets defined for non-contiguous intra-band CA (with three sub-blocks)

|  |  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRACA configuration | Uplink CA configurations (NOTE 1) | Component carriers in order of increasing carrier frequency |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_25A-25A-25A | - | 5, 10, 15, 20 | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  | 60 | 0 |
| CA_41A-41A-41A | - | 5, 10, 15, 20 | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  | 60 | 0 |
| CA_41A-41A-41C | CA_41C | 5, 10, 15, 20 | 5, 10, 15, 20 | See CA_41C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  |  | 80 | 0 |
|  |  | 5, 10, 15, 20 | See CA_41C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  | 5, 10, 15, 20 |  |  |  |
|  |  | See CA_41C Bandwidth Combination Set 1 in Table 5.6A.1-1 |  | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  |  |
| CA_48A-48A-48A | - | 5, 10, 15, 20 | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  | 60 | 0 |
| CA_66A-66A-66A | - | 5, 10, 15, 20 | 5, 10, 15, 20 | 5, 10, 15, 20 |  |  | 60 | 0 |

Table 5.6A.1-5: E-UTRA CA configurations and bandwidth combination sets defined for non-contiguous intra-band CA (with four sub-blocks)

|  |  | E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRACA configuration | Uplink CA configurations (NOTE 1) | Component carriers in order of increasing carrier frequency |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| CA_48A-48A-48A-48A | - | 5, 10, 15, 20 | 5, 10, 15, 20 | 5, 10, 15, 20 | 5, 10, 15, 20 |  | 80 | 0 |

## 5.6B	Channel bandwidth for UL-MIMO

The requirements specified in subclause 5.6 are applicable to UE supporting UL-MIMO.

### 5.6B.1	Void

## 5.6C	Channel bandwidth for Dual Connectivity

For E-UTRA DC bands specified in 5.5C, the corresponding E-UTRA CA configurations in 5.6A.1, i.e., dual uplink inter-band carrier aggregation with uplink assigned to two E-UTRA bands, are applicable to Dual Connectivity.

NOTE 1:	Requirements for the dual connectivity configurations are defined in the section corresponding E-UTRA uplink CA configurations, unless otherwise specified.

NOTE 2:	For TDD inter-band dual connectivity configurations, requirements are applicable only for synchronous operation.

### 5.6C.1	Void

Table 5.6C.1-1: Void

Table 5.6C.1-2: Void

## 5.6D	Channel bandwidth for ProSe

### 5.6D.1	Channel bandwidths per operating band for ProSe

The ProSe combination of channel bandwidths and operating bands is shown in Table 5.6D.1-1 and Table 5.6D.1-2. The transmission bandwidth configuration in Table 5.6D.1-1 and Table 5.6D.1-2 shall be supported for each of the specified channel bandwidths. The same (symmetrical) channel bandwidth is specified for both the TX and RX path.

Table 5.6D.1-1 ProSe Direct Discovery channel bandwidth

| E-UTRA ProSe band / ProSe channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA ProSe Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| 2 |  |  | Yes | Yes | Yes | Yes |
| 3 |  |  | Yes | Yes | Yes | Yes |
| 4 |  |  | Yes | Yes | Yes | Yes |
| 7 |  |  | Yes | Yes | Yes | Yes |
| 14 |  |  | Yes | Yes |  |  |
| 20 |  |  | Yes | Yes | Yes | Yes |
| 26 |  |  | Yes | Yes | Yes |  |
| 28 |  |  | Yes | Yes | Yes | Yes |
| 31 |  |  | Yes |  |  |  |
| 41 |  |  | Yes | Yes | Yes | Yes |
| 68 |  |  | Yes | Yes | Yes |  |
| 72 |  |  | Yes |  |  |  |

Table 5.6D.1-2 ProSe Direct Communication channel bandwidth

| E-UTRA ProSe band / ProSe channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA ProSe Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| 3 |  |  |  | Yes |  |  |
| 7 |  |  |  | Yes |  |  |
| 14 |  |  |  | Yes |  |  |
| 20 |  |  |  | Yes |  |  |
| 26 |  |  |  | Yes |  |  |
| 28 |  |  |  | Yes |  |  |
| 31 |  |  | Yes |  |  |  |
| 68 |  |  | Yes | Yes |  |  |
| 72 |  |  | Yes |  |  |  |

## 5.6F	Channel bandwidth for category NB1 and NB2

Channel bandwidth for Category NB1 and NB2 is 200 kHz.

For category NB1 and NB2, requirements in present document are specified for the channel bandwidth listed in Table 5.6F-1.

Table 5.6F-1: Transmission bandwidth configuration NRB, Ntone 15kHz and Ntone 3.75kHz in NB1 and NB2 channel bandwidth

| Channel bandwidth BWChannel [kHz] | 200 |
| --- | --- |
| Transmission bandwidth configuration NRB | 1 |
| Transmission bandwidth configuration Ntone 15kHz | 12 |
| Transmission bandwidth configuration Ntone 3.75kHz | 48 |

Figure 5.6F-1 shows the relation between the Category NB1/NB2 channel bandwidth (BWChannel) and the Category NB1 /NB2 transmission bandwidth configuration (Ntone). The channel edges are defined as the lowest and highest frequencies of the carrier separated by the channel bandwidth, i.e. at FC +/- BWChannel /2.

Figure 5.6F-1 Definition of Channel Bandwidth and Transmission Bandwidth configuration

## 5.6G	Channel bandwidth for V2X Communication

### 5.6G.1	Channel bandwidths per operating band for V2X Communication

E-UTRA V2X Communication channel bandwidths and operating band is shown in Table 5.6G.1-1. The same (symmetrical) channel bandwidth is specified for both the TX and RX path.

Table 5.6G.1-1: V2X Communciation channel bandwidth

| E-UTRA V2X band / V2X channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA V2X Operating Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| 47 |  |  |  | Yes |  | Yes |

For V2X inter-band con-current operation, the V2X Communication channel bandwidths for each operating band is specified in Table 5.6G.1-2.

Table 5.6G.1-2: Inter-band con-current V2X configurations and bandwidth combination sets

| Inter-band con-current V2X configuration | Inter-band con-current V2X configuration for TX | E-UTRA or V2X operating Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum bandwidth [MHz] | Bandwidth combination set |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V2X_3A-47A | V2X_3A-47A | 3 | Yes | Yes | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 47 |  |  |  | Yes |  | Yes |  |  |
| V2X_3A-47B | V2X_3A-47A, V2X_47B | 3 | Yes | Yes | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 47 | See V2X_47B Bandwidth combination set 0 in Table 5.6G.1-4 |  |  |  |  |  |  |  |
| V2X_5A-47A | V2X_5A-47A | 5 |  |  | Yes | Yes |  |  | 30 | 0 |
|  |  | 47 |  |  |  | Yes |  | Yes |  |  |
| V2X_5A-47B | V2X_5A-47A, V2X_47B | 5 | Yes | Yes | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 47 | See V2X_47B Bandwidth combination set 0 in Table 5.6G.1-4 |  |  |  |  |  |  |  |
| V2X_7A_47A | V2X_7A_47A | 7 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 47 |  |  |  | Yes |  | Yes |  |  |
| V2X_7A-47B | V2X_7A-47A, V2X_47B | 7 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 47 | See V2X_47B Bandwidth combination set 0 in Table 5.6G.1-4 |  |  |  |  |  |  |  |
| V2X_8A-47A | V2X_8A-47A | 8 | Yes | Yes | Yes | Yes |  |  | 30 | 0 |
|  |  | 47 |  |  |  | Yes |  | Yes |  |  |
| V2X_8A-47B | V2X_8A-47A, V2X_47B | 8 | Yes | Yes | Yes | Yes |  |  | 40 | 0 |
|  |  | 47 | See V2X_47B Bandwidth combination set 0 in Table 5.6G.1-4 |  |  |  |  |  |  |  |
| V2X_20A-47A | V2X_20A-47A | 20 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 47 |  |  |  | Yes |  | Yes |  |  |
| V2X_20A-47B | V2X_20A-47A, V2X_47B | 20 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 47 | See V2X_47B Bandwidth combination set 0 in Table 5.6G.1-4 |  |  |  |  |  |  |  |
| V2X_28A-47A |  | 28 |  | Yes | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 47 |  |  |  | Yes |  | Yes |  |  |
| V2X_34A-47A | V2X_34A-47A | 34 |  |  | Yes | Yes | Yes |  | 35 | 0 |
|  |  | 47 |  |  |  | Yes |  | Yes |  |  |
| V2X_34A-47B | V2X_34A-47A, V2X_47B | 34 |  |  | Yes | Yes | Yes |  | 40 | 0 |
|  |  | 47 | See V2X_47B Bandwidth combination set 0 in Table 5.6G.1-4 |  |  |  |  |  |  |  |
| V2X_39A-47A | V2X_39A-47A | 39 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 47 |  |  |  | Yes |  | Yes |  |  |
| V2X_39A-47B | V2X_39A-47A, V2X_47B | 39 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 47 | See V2X_47B Bandwidth combination set 0 in Table 5.6G.1-4 |  |  |  |  |  |  |  |
| V2X_39A-47C | V2X_39A-47A, V2X_47C | 39 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 47 | See V2X_47C Bandwidth combination set 0 in Table 5.6G.1-4 |  |  |  |  |  |  |  |
| V2X_41A-47A | V2X_41A-47 | 41 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 47 |  |  |  | Yes |  | Yes |  |  |
| V2X_41A-47B | V2X_41A-47A, V2X_47B | 41 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 47 | See V2X_47B Bandwidth combination set 0 in Table 5.6G.1-4 |  |  |  |  |  |  |  |
| V2X_41A-47C | V2X_41A-47A, V2X_47C | 41 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 47 | See V2X_47C Bandwidth combination set 0 in Table 5.6G.1-4 |  |  |  |  |  |  |  |
| V2X_71A-47A |  | 71 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 47 |  |  |  | Yes |  | Yes |  |  |

V2X Bandwidth Class is specified in Table 5.6G.1-3 for V2X intra-band contiguous multi-carrier operation.

Table 5.6G.1-3: V2X bandwidth classes and corresponding nominal guard bands

| V2X Bandwidth Class | Aggregated Transmission Bandwidth Configuration | Number of contiguous CC | Nominal Guard Band BWGB |
| --- | --- | --- | --- |
| A | NRB,agg ≤ 100 | 1 | 0.05BWChannel(1) - 0.5f1 |
| B | 25 < NRB,agg ≤ 100 | 2 | [0.05 max(BWChannel(1),BWChannel(2)) - 0.5f1] |
| C | 100 < NRB,agg ≤ 200 | 2 | [0.05 max(BWChannel(1),BWChannel(2)) - 0.5f1] |
| C1 | 100 < NRB,agg ≤ 200 | 3 | [0.05 max(BWChannel(1),BWChannel(2)) - 0.5f1] |
| D | 200 < NRB,agg ≤ 300 | 3 | NOTE 3 |
| E | 300 < NRB,agg ≤ 400 | 4 | NOTE 3 |
| F | 400 < NRB,agg ≤ 500 | 5 | NOTE 3 |
| I | 700 < NRB,agg ≤ 800 | 8 | NOTE 3 |
| NOTE 1: BWChannel(j), j = 1, 2, 3, 4 is the channel bandwidth of an E-UTRA component carrier according to Table 5.6-1 and f1 = f for the downlink with f the subcarrier spacing while f1 = 0 for the uplink. NOTE 2: Void NOTE 3: Applicable for later releases. |  |  |  |

For V2X intra-band multi-carrier operation, the V2X communication channel bandwidths for each operating band is specified in Table 5.6G.1-4.

Table 5.6G.1-4: V2X intra-band multi-carrier configurations

|  |  | V2X multi-carrier configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V2X intra-band multi-carrier configuration | V2X intra-band multi-carrier configuration for TX | Component carriers in order of increasing carrier frequency |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] | Channel bandwidths for carrier [MHz] |  |  |
| V2X_47B | V2X_47B | 10 | 10 |  |  |  | 20 | 0 |
| V2X_47C | V2X_47C | 10 | 20 |  |  |  | 30 | 0 |
|  |  | 20 | 10 |  |  |  |  |  |
| V2X_47C1 | V2X_47B | 10 | 10 | 10 |  |  | 30 | 0 |

## 5.7	Channel arrangement

### 5.7.1	Channel spacing

The spacing between carriers will depend on the deployment scenario, the size of the frequency block available and the channel bandwidths. The nominal channel spacing between two adjacent E-UTRA carriers is defined as following:

Nominal Channel spacing = (BWChannel(1) + BWChannel(2))/2

where BWChannel(1) and BWChannel(2) are the channel bandwidths of the two respective E-UTRA carriers. The channel spacing can be adjusted to optimize performance in a particular deployment scenario.

### 5.7.1A	Channel spacing for CA

For intra-band contiguous carrier aggregation with two or more component carriers, the nominal channel spacing between two adjacent E-UTRA component carriers is defined as the following unless stated otherwise:

where BWChannel(1) and BWChannel(2) are the channel bandwidths of the two respective E-UTRA component carriers according to Table 5.6-1 with values in MHz. The channel spacing for intra-band contiguous carrier aggregation can be adjusted to any multiple of 300 kHz less than the nominal channel spacing to optimize performance in a particular deployment scenario.

For intra-band contiguous carrier aggregation with two or more component carriers in Band 46, the requirements apply for both 19.8 MHz and 20.1 MHz nominal carrier spacing between two 20 MHz component carriers, and for 15.0 MHz nominal carrier spacing between 10 MHz and 20 MHz component carriers.

For intra-band non-contiguous carrier aggregation the channel spacing between two or more E-UTRA component carriers in different sub-blocks shall be larger than the nominal channel spacing defined in this subclause.

### 5.7.1F	Channel spacing for category NB1 and NB2

Nominal channel spacing for UE category NB1 and NB2 in stand-alone mode is 200 kHz. For in-band and guard-band cases the nominal channel spacing between two adjacent category NB1 or NB2 carriers is 180 kHz.

### 5.7.2	Channel raster

The channel raster is 100 kHz for all bands, which means that the carrier centre frequency must be an integer multiple of 100 kHz.

### 5.7.2A	Channel raster for CA

For carrier aggregation the channel raster is 100 kHz for all bands, which means that the carrier centre frequency must be an integer multiple of 100 kHz.

### 5.7.2F	Channel raster for category NB1 and NB2

Channel raster for category NB1 and NB2 in-band, guard-band and standalone operation is 100 kHz.

### 5.7.3	Carrier frequency and EARFCN

The carrier frequency in the uplink and downlink is designated by the E-UTRA Absolute Radio Frequency Channel Number (EARFCN) in the range 0 – 262143. The relation between EARFCN and the carrier frequency in MHz for the downlink is given by the following equation, where FDL_low and NOffs-DL are given in Table 5.7.3-1 and NDL is the downlink EARFCN.

FDL = FDL_low + 0.1(NDL – NOffs-DL)

The relation between EARFCN and the carrier frequency in MHz for the uplink is given by the following equation where FUL_low and NOffs-UL are given in Table 5.7.3-1 and NUL is the uplink EARFCN.

FUL = FUL_low + 0.1(NUL – NOffs-UL)

Table 5.7.3-1: E-UTRA channel numbers

| E-UTRA Operating Band | Downlink |  |  | Uplink |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | FDL_low (MHz) | NOffs-DL | Range of NDL | FUL_low (MHz) | NOffs-UL | Range of NUL |
| 1 | 2110 | 0 | 0 – 599 | 1920 | 18000 | 18000 – 18599 |
| 2 | 1930 | 600 | 6001199 | 1850 | 18600 | 18600 – 19199 |
| 3 | 1805 | 1200 | 1200 – 1949 | 1710 | 19200 | 19200 – 19949 |
| 4 | 2110 | 1950 | 1950 – 2399 | 1710 | 19950 | 19950 – 20399 |
| 5 | 869 | 2400 | 2400 – 2649 | 824 | 20400 | 20400 – 20649 |
| 6 | 875 | 2650 | 2650 – 2749 | 830 | 20650 | 20650 – 20749 |
| 7 | 2620 | 2750 | 2750 – 3449 | 2500 | 20750 | 20750 – 21449 |
| 8 | 925 | 3450 | 3450 – 3799 | 880 | 21450 | 21450 – 21799 |
| 9 | 1844.9 | 3800 | 3800 – 4149 | 1749.9 | 21800 | 21800 – 22149 |
| 10 | 2110 | 4150 | 4150 – 4749 | 1710 | 22150 | 22150 – 22749 |
| 11 | 1475.9 | 4750 | 4750 – 4949 | 1427.9 | 22750 | 22750 – 22949 |
| 12 | 729 | 5010 | 5010 – 5179 | 699 | 23010 | 23010 – 23179 |
| 13 | 746 | 5180 | 5180 – 5279 | 777 | 23180 | 23180 – 23279 |
| 14 | 758 | 5280 | 5280 – 5379 | 788 | 23280 | 23280 – 23379 |
| … |  |  |  |  |  |  |
| 17 | 734 | 5730 | 5730 – 5849 | 704 | 23730 | 23730 – 23849 |
| 18 | 860 | 5850 | 5850 – 5999 | 815 | 23850 | 23850 – 23999 |
| 19 | 875 | 6000 | 6000 – 6149 | 830 | 24000 | 24000 – 24149 |
| 20 | 791 | 6150 | 6150 – 6449 | 832 | 24150 | 24150 – 24449 |
| 21 | 1495.9 | 6450 | 6450 – 6599 | 1447.9 | 24450 | 24450 – 24599 |
| 22 | 3510 | 6600 | 6600 – 7399 | 3410 | 24600 | 24600 – 25399 |
| 23 | 2180 | 7500 | 7500 – 7699 | 2000 | 25500 | 25500 – 25699 |
| 24 | 1525 | 7700 | 7700 – 8039 | 1626.5 | 25700 | 25700 – 26039 |
| 25 | 1930 | 8040 | 8040 – 8689 | 1850 | 26040 | 26040 – 26689 |
| 26 | 859 | 8690 | 8690 – 9039 | 814 | 26690 | 26690 – 27039 |
| 27 | 852 | 9040 | 9040 – 9209 | 807 | 27040 | 27040 – 27209 |
| 28 | 758 | 9210 | 9210 – 9659 | 703 | 27210 | 27210 – 27659 |
| 292 | 717 | 9660 | 9660 – 9769 | N/A |  |  |
| 30 | 2350 | 9770 | 9770 – 9869 | 2305 | 27660 | 27660 – 27759 |
| 31 | 462.5 | 9870 | 9870 – 9919 | 452.5 | 27760 | 27760 – 27809 |
| 322 | 1452 | 9920 | 9920 – 10359 | N/A |  |  |
| 33 | 1900 | 36000 | 36000 – 36199 | 1900 | 36000 | 36000 – 36199 |
| 34 | 2010 | 36200 | 36200 – 36349 | 2010 | 36200 | 36200 – 36349 |
| 35 | 1850 | 36350 | 36350 – 36949 | 1850 | 36350 | 36350 – 36949 |
| 36 | 1930 | 36950 | 36950 – 37549 | 1930 | 36950 | 36950 – 37549 |
| 37 | 1910 | 37550 | 37550 – 37749 | 1910 | 37550 | 37550 – 37749 |
| 38 | 2570 | 37750 | 37750 – 38249 | 2570 | 37750 | 37750 – 38249 |
| 39 | 1880 | 38250 | 38250 – 38649 | 1880 | 38250 | 38250 – 38649 |
| 40 | 2300 | 38650 | 38650 – 39649 | 2300 | 38650 | 38650 – 39649 |
| 41 | 2496 | 39650 | 39650 –41589 | 2496 | 39650 | 39650 –41589 |
| 42 | 3400 | 41590 | 41590 – 43589 | 3400 | 41590 | 41590 – 43589 |
| 43 | 3600 | 43590 | 43590 – 45589 | 3600 | 43590 | 43590 – 45589 |
| 44 | 703 | 45590 | 45590 – 46589 | 703 | 45590 | 45590 – 46589 |
| 45 | 1447 | 46590 | 46590 – 46789 | 1447 | 46590 | 46590 – 46789 |
| 46 | 5150 | 46790 | 46790 – 54539 | 5150 | 46790 | 46790 – 54539 |
| 47 | 5855 | 54540 | 54540 - 55239 | 5855 | 54540 | 54540 – 55239 |
| 48 | 3550 | 55240 | 55240 – 56739 | 3550 | 55240 | 55240 – 56739 |
| 49 | 3550 | 56740 | 56740 – 58239 | 3550 | 56740 | 56740 – 58239 |
| 50 | 1432 | 58240 | 58240 - 59089 | 1432 | 58240 | 58240 - 59089 |
| 51 | 1427 | 59090 | 59090 - 59139 | 1427 | 59090 | 59090 - 59139 |
| 52 | 3300 | 59140 | 59140 - 60139 | 3300 | 59140 | 59140 - 60139 |
| 53 | 2483.5 | 60140 | 60140 - 60254 | 2483.5 | 60140 | 60140 - 60254 |
| … |  |  |  |  |  |  |
| 64 | Reserved |  |  |  |  |  |
| 65 | 2110 | 65536 | 65536 – 66435 | 1920 | 131072 | 131072 – 131971 |
| 665 | 2110 | 66436 | 66436 – 67335 | 1710 | 131972 | 131972 – 132671 |
| 672 | 738 | 67336 | 67336 – 67535 | N/A |  |  |
| 68 | 753 | 67536 | 67536 - 67835 | 698 | 132672 | 132672 - 132971 |
| 692 | 2570 | 67836 | 67836 - 68335 | N/A |  |  |
| 706 | 1995 | 68336 | 68336 - 68585 | 1695 | 132972 | 132972 - 133121 |
| 71 | 617 | 68586 | 68586 - 68935 | 663 | 133122 | 133122 - 133471 |
| 72 | 461 | 68936 | 68936 - 68985 | 451 | 133472 | 133472 - 133521 |
| 73 | 460 | 68986 | 68986 - 69035 | 450 | 133522 | 133522 - 133571 |
| 74 | 1475 | 69036 | 69036 - 69465 | 1427 | 133572 | 133572 - 134001 |
| 752 | 1432 | 69466 | 69466 - 70315 | N/A |  |  |
| 762 | 1427 | 70316 | 70316 - 70365 | N/A |  |  |
| 85 | 728 | 70366 | 70366 - 70545 | 698 | 134002 | 134002 - 134181 |
| 87 | 420 | 70546 | 70546 - 70595 | 410 | 134182 | 134182 - 134231 |
| 88 | 422 | 70596 | 70596 - 70645 | 412 | 134232 | 134232 - 134281 |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. This implies that the first 7, 15, 25, 50, 75 and 100 channel numbers at the lower operating band edge and the last 6, 14, 24, 49, 74 and 99 channel numbers at the upper operating band edge shall not be used for channel bandwidths of 1.4, 3, 5, 10, 15 and 20 MHz respectively. NOTE 2: Restricted to E-UTRA operation when carrier aggregation is configured. NOTE 3: For ProSe and V2X the corresponding UL channel number are also specified for the DL for the associated ProSe/V2X operating bands i.e. ProSe_FUL = FUL and ProSe_FDL = FUL; V2X_FUL = FDL and V2X_FDL = FUL. NOTE 4: Requirements for uplink operations are not specified in this version of the specification. NOTE 5: The range 2180-2200 MHz of the DL operating band is restricted to E-UTRA operation when carrier aggregation is configured. NOTE 6: The range 2010-2020 MHz of the DL operating band is restricted to E-UTRA operation when carrier aggregation is configured and TX-RX separation is 300 MHz The range 2005-2020 MHz of the DL operating band is restricted to E-UTRA operation when carrier aggregation is configured and TX-RX separation is 295 MHz. |  |  |  |  |  |  |

### 5.7.3F	Carrier frequency and EARFCN for category NB1 and NB2

The carrier frequency of category NB1/NB2 in the downlink is designated by the E-UTRA Absolute Radio Frequency Channel Number (EARFCN) in the range 0 – 262143 and the Offset of category NB1/NB2 Channel Number to EARFCN in the range {-10,-9,-8,-7,-6,-5,-4,-3,-2,-1,-0.5,0,1,2,3,4,5,6,7,8,9} for FDD and in the range {-10,-9,-8.5,-8,-7,-6,-5,-4.5,-4,-3,-2,-1,-0.5,0,1,2,3,3.5,4,5,6,7,7.5,8,9} for TDD. The relation between EARFCN, Offset of category NB1/NB2 Channel Number to EARFCN and the carrier frequency in MHz for the downlink is given by the following equation, where FDL is the downlink carrier frequency of category NB1/NB2, FDL_low and NOffs-DL are given in table 5.7.3-1, NDL is the downlink EARFCN, MDL is the Offset of category NB1/NB2 Channel Number to downlink EARFCN.

FDL = FDL_low + 0.1(NDL – NOffs-DL) + 0.0025*(2MDL+1)

The carrier frequency of category NB1/NB2 in the uplink is designated by the E-UTRA Absolute Radio Frequency Channel Number (EARFCN) in the range 0 –262143, and the Offset of category NB1/NB2 Channel Number to EARFCN in the range {-10,-9,-8,-7,-6,-5,-4,-3,-2,-1,0,1,2,3,4,5,6,7,8,9} for FDD and in the range {-11,-10,-9.5,-9,-8.5, -8,-7.5,-7,-6.5,-6,-5.5,-5,-4.5,-4,-3.5,-3,-2.5,-2,-1.5,-1,-0.5,0,0.5,1,1.5,2,2.5,3,3.5,4,4.5,5,5.5,6,6.5,7,7.5,8,8.5,9,9.5,10, 11} for TDD. The relation between EARFCN, Offset of category NB1/NB2 Channel Number to EARFCN and the carrier frequency in MHz for the uplink is given by the following equation, where FUL is the uplink carrier frequency of category NB1/NB2, FUL_low and NOffs-UL are given in table 5.7.3-1, NUL is the uplink EARFCN, MUL is the Offset of category NB1/NB2 Channel Number to uplink EARFCN.

FUL = FUL_low + 0.1(NUL – NOffs-UL) + 0.0025*(2MUL)

NOTE 1:	For category NB1/NB2, NDL or NUL is different than the value of EARFCN that corresponds to E-UTRA downlink or uplink carrier frequency for in-band and guard band operation.

NOTE 2:	For FDD MDL = -0.5 is not applicable for in-band and guard band operation. For TDD MDL {-0.5,+3.5,-4.5,+7.5,-8.5} is not applicable for in-band and guard band operation.

NOTE 3:	For the carrier including NPSS/NSSS for in-band and guard band operation, MDL is selected from {-2,-1,0,1}.

NOTE 4:	For the carrier including NPSS/NSSS for stand-alone operation, MDL = -0.5.

### 5.7.4	TX–RX frequency separation

a)	The default E-UTRA TX channel (carrier centre frequency) to RX channel (carrier centre frequency) separation is specified in Table 5.7.4-1 for the TX and RX channel bandwidths defined in Table 5.6.1-1

Table 5.7.4-1: Default UE TX-RX frequency separation

| E-UTRA Operating Band | TX – RX carrier centre frequency separation |
| --- | --- |
| 1 | 190 MHz |
| 2 | 80 MHz. |
| 3 | 95 MHz. |
| 4 | 400 MHz |
| 5 | 45 MHz |
| 6 | 45 MHz |
| 7 | 120 MHz |
| 8 | 45 MHz |
| 9 | 95 MHz |
| 10 | 400 MHz |
| 11 | 48 MHz |
| 12 | 30 MHz |
| 13 | -31 MHz |
| 14 | -30 MHz |
| 17 | 30 MHz |
| 18 | 45 MHz |
| 19 | 45 MHz |
| 20 | -41 MHz |
| 21 | 48 MHz |
| 22 | 100 MHz |
| 23 | 180 MHz |
| 24 | -101.51, -120.5 MHz |
| 25 | 80 MHz |
| 26 | 45 MHz |
| 27 | 45 MHz |
| 28 | 55 MHz |
| 30 | 45 MHz |
| 31 | 10 MHz |
| 65 | 190 MHz |
| 66 | 400 MHz |
| 68 | 55 MHz |
| … |  |
| 70 | 295, 300MHz1 |
| 71 | -46 MHz |
| 72 | 10 MHz |
| 73 | 10 MHz |
| 74 | 48 MHz |
| 85 | 30 MHz |
| 87 | 10 MHz |
| 88 | 10 MHz |
| NOTE 1: Default TX-RX carrier centre frequency separation. |  |

b)	The use of other TX channel to RX channel carrier centre frequency separation is not precluded and is intended to form part of a later release.

### 5.7.4A	TX–RX frequency separation for CA

For intra-band contiguous carrier aggregation, the same TX-RX frequency separation as specified in Table 5.7.4-1 is applied to PCC and SCC, respectively.

### 5.7.4E	TX–RX frequency separation for category M1 and M2

For the category M1 and M2 TX-RX frequency separation is flexible within the assigned channel bandwidth of E-UTRA carrier with the TX-RX frequency separation of the E-UTRA carriers as specified in Table 5.7.4-1.

### 5.7.4F	TX–RX frequency separation for category NB1 and NB2

For in-band and guard-band operation mode, the category NB1 and NB2 TX-RX frequency separation is flexible within the assigned channel bandwidth of E-UTRA carrier with the TX-RX frequency separation of the E-UTRA carriers as specified in Table 5.7.4-1. For stand-alone operation mode the default TX-RX frequency separation is the same as Table 5.7.4-1.

# 6	Transmitter characteristics

## 6.1	General

Unless otherwise stated, the transmitter characteristics are specified at the antenna connector of the UE with a single or multiple transmit antenna(s). For UE with integral antenna only, a reference antenna with a gain of 0 dBi is assumed.

Unless otherwise stated, NB1 and NB2 requirements specified for an E-UTRA band shall also apply for the re-farmed NR band (e.g. if NB1/NB2 requirements are specified for E-UTRA band 1, they shall also apply for NR band n1).

## 6.2	Transmit power

### 6.2.1	Void

### 6.2.2	UE maximum output power

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth for non CA configuration unless otherwise stated. The period of measurement shall be at least as defined in Table 6.2.2-0.

Table 6.2.2-0: Measurement period for UE maximum output power

| TTI pattern | Minimum measurement period |
| --- | --- |
| Subframe | 1ms |
| Slot | 7OS |
| Subslot | 2OS, 3OS |

Table 6.2.2-1: UE Power Class

| EUTRA band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  | 23 | ±2 |  |  |
| 2 |  |  |  |  | 23 | ±22 |  |  |
| 3 | 31 | +2/-3 |  |  | 23 | ±22 |  |  |
| 4 |  |  |  |  | 23 | ±2 |  |  |
| 5 |  |  |  |  | 23 | ±2 |  |  |
| 6 |  |  |  |  | 23 | ±2 |  |  |
| 7 |  |  |  |  | 23 | ±22 |  |  |
| 8 |  |  |  |  | 23 | ±22 |  |  |
| 9 |  |  |  |  | 23 | ±2 |  |  |
| 10 |  |  |  |  | 23 | ±2 |  |  |
| 11 |  |  |  |  | 23 | ±2 |  |  |
| 12 |  |  |  |  | 23 | ±22 |  |  |
| 13 |  |  |  |  | 23 | ±2 |  |  |
| 14 | 31 | +2/-3 |  |  | 23 | ±2 |  |  |
| 17 |  |  |  |  | 23 | ±2 |  |  |
| 18 |  |  |  |  | 23 | ±25 |  |  |
| 19 |  |  |  |  | 23 | ±2 |  |  |
| 20 | 31 | +2/-3 |  |  | 23 | ±22 |  |  |
| 21 |  |  |  |  | 23 | ±2 |  |  |
| 22 |  |  |  |  | 23 | +2/-3.52 |  |  |
| 23 |  |  |  |  | 236 | ±26 |  |  |
| 24 |  |  |  |  | 23 | +2/-32 |  |  |
| 25 |  |  |  |  | 23 | ±22 |  |  |
| 26 |  |  |  |  | 23 | ±22 |  |  |
| 27 |  |  |  |  | 23 | ±2 |  |  |
| 28 | 31 | +2/-3 |  |  | 23 | +2/-2.5 |  |  |
| 30 |  |  |  |  | 23 | ±2 |  |  |
| 31 | 31 | +2/-3 |  |  | 23 | ±2 |  |  |
| … |  |  |  |  |  |  |  |  |
| 33 |  |  |  |  | 23 | ±2 |  |  |
| 34 |  |  |  |  | 23 | ±2 |  |  |
| 35 |  |  |  |  | 23 | ±2 |  |  |
| 36 |  |  |  |  | 23 | ±2 |  |  |
| 37 |  |  |  |  | 23 | ±2 |  |  |
| 38 |  |  | 26 | ±2 | 23 | ±2 |  |  |
| 39 |  |  |  |  | 23 | ±2 |  |  |
| 40 |  |  | 26 | ±2 | 23 | ±2 |  |  |
| 41 |  |  | 26 | ±22 | 23 | ±22 |  |  |
| 42 |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| 43 |  |  |  |  | 23 | +2/-3 |  |  |
| 44 |  |  |  |  | 23 | +2/[-3] |  |  |
| 45 |  |  |  |  | 23 | ±2 |  |  |
| … |  |  |  |  |  |  |  |  |
| 47 |  |  | 26 | ±2 | 23 | ±2 |  |  |
| 48 |  |  |  |  | 23 | +2/-3 |  |  |
| 50 |  |  |  |  | 23 | ±2 |  |  |
| 51 |  |  |  |  | 23 | ±2 |  |  |
| 52 |  |  |  |  | 23 | +2/-3 |  |  |
| 53 |  |  |  |  | 23 | ±2 |  |  |
| 65 |  |  |  |  | 23 | ±2 |  |  |
| 66 |  |  |  |  | 23 | ±2 |  |  |
| 68 |  |  |  |  | 23 | ±2 |  |  |
| … |  |  |  |  |  |  |  |  |
| 70 |  |  |  |  | 23 | ±2 |  |  |
| 71 |  |  |  |  | 23 | +2/-2.5 |  |  |
| 72 | 31 | +2/-3 |  |  | 23 | ±2 |  |  |
| 73 |  |  |  |  | 23 | ±2 |  |  |
| 74 |  |  |  |  | 23 | ±2 |  |  |
| 85 |  |  |  |  | 23 | ±22 |  |  |
| 87 | 31 | +2/-3 |  |  | 23 | ±2 |  |  |
| 88 | 31 | +2/-3 |  |  | 23 | ±2 |  |  |
| NOTE 1: Void NOTE 2: 2 refers to the transmission bandwidths (Figure 5.6-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 3: For the UE which supports both Band 11 and Band 21 operating frequencies, the tolerance is FFS. NOTE 4: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 5: For a UE that supports both Band 18 and Band 26, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB for transmission bandwidths confined within 815 MHz and 818 MHz. NOTE 6: When NS_20 is signalled, the total output power within 2000-2005 MHz shall be limited to 7 dBm. NOTE 7: Void. NOTE 8: Generally, PC1 UE is not targeted for smartphone form factor. NOTE 9: Void. |  |  |  |  |  |  |  |  |

The default power class PPowerClass_Default for an operating band is Power Class 3 unless otherwise stated.

For a power class 2 capable UE operating on Band 41, when an IE P-max as defined in TS 36.331 [7] of 23 dBm or lower is indicated in the cell or if the uplink/downlink configuration is 0 or 6, the requirements for power class 2 are not applicable, and the corresponding requirements for a power class 3 UE shall apply.

For each supported frequency band other than Band 14 and Band 41, the UE shall:

-	if the UE supports a different power class than the default UE power class for the band and the supported power class enables the higher maximum output power than that of the default power class:

-	if the band is a TDD band whose frame configuration is 0 or 6; or

-	if the IE P-Max as defined in TS 36.331 [7] is not provided; or

-	if the IE P-Max as defined in TS 36.331 [7] is provided and set to the maximum output power of the default power class or lower;

-	meet all requirements for the default power class of the operating band in which the UE is operating and set its configured transmitted power as specified in sub-clause 6.2.5;

-	else (i.e the IE P-Max as defined in TS 36.331 [7] is provided and set to the higher value than the maximum output power of the default power class):

-	meet all requirements for the supported power class and set its configured transmitted power class as specified in sub-clause 6.2.5;

### 6.2.2A	UE maximum output power for CA

The following UE Power Classes define the maximum output power for any transmission bandwidth within the aggregated channel bandwidth.

The maximum output power is measured as the sum of the maximum output power at each UE antenna connector. The period of measurement shall be at least as defined in Table 6.2.2A-0a.

Table 6.2.2A-0a: Measurement period for UE maximum output power for CA

| TTI pattern | Minimum measurement period |
| --- | --- |
| Subframe | 1ms |
| Slot | 7OS |
| Subslot | 2OS, 3OS |

For inter-band carrier aggregation with one uplink component carrier assigned to one E-UTRA band the requirements in subclause 6.2.2 apply. For inter-band carrier aggregation with two uplink contiguous component carrier assigned to one E-UTRA band the requirements specified in Table 6.2.2A-1 apply for that band. For inter-band carrier aggregation with one uplink component carrier assigned to one E-UTRA band in Band 38, 40, 41 or 42, the requirements for power class 2 are not applicable and the corresponding requirements for a power class 3 UE shall apply. For inter-band carrier aggregation with one uplink component carrier assigned to one E-UTRA band in Band 3, 20, 28, or 31, the requirements for power class 1 are not applicable and the corresponding requirements for a power class 3 UE shall apply.

For inter-band carrier aggregation with uplink assigned to two E-UTRA bands, UE maximum output power shall be measured over all component carriers from different bands. If each band has separate antenna connectors, maximum output power is measured as the sum of maximum output power at each UE antenna connector. The maximum output power is specified in Table 6.2.2A-0.

For E-UTRA CA bands including an uplink LAA Scell in Band 46, the UE shall meet the following additional requirements for transmission within the frequency ranges 5150-5350 MHz and 5470-5725 MHz:

-	a maximum mean power density of 10 dBm in any 1 MHz band when the network signaling value NS_28 or NS_29 is indicated in the LAA Scell;

-	a maximum mean power density of 11 dBm in any 1 MHz band when the network signaling value NS_30 is indicated in the LAA Scell;

the following additional requirements for transmission within the frequency range 5230-5250 MHz:

-	a maximum mean power density of 4 dBm in any 1 MHz band when the network signaling value NS_31 is indicated in the LAA Scell;

the following additional requirements for transmission within the frequency ranges 5150-5230 MHz, 5250-5350 MHz, 5470-5725 MHz and 5725-5850 MHz:

-	a maximum mean power density of 10 dBm in any 1 MHz band when the network signaling value NS_31 is indicated in the LAA Scell;

where the said network signaling values are specified in clause 6.2.4.

Table 6.2.2A-0: UE Power Class for uplink interband CA (two bands)

| E-UTRA CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1A-3A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_1A-5A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_1A-7A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_1A-8A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_1A-11A |  |  |  |  | 23 | +2/-35 |  |  |
| CA_1A-18A |  |  |  |  | 23 | +2/-35 |  |  |
| CA_1A-19A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_1A-20A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_1A-21A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_1A-26A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_1A-28A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_1A-42A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_1A-42C |  |  |  |  | 23 | +2/-3 |  |  |
| CA_2A-4A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_2A-5A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_2A-7A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_2A-12A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_2A-13A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_2A-14A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_2A-30A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_2A-46A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_2A-48A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_2A-49A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_2A-66A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_3A-5A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_3A-7A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_3A-8A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_3A-11A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_3A-18A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_3A-19A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_3A-20A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_3A-21A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_3A-26A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_3A-28A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_3A-40A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_3A-41C |  |  |  |  | 23 | +2/-32 |  |  |
| CA_3A-42A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_3A-42C |  |  |  |  | 23 | +2/-32 |  |  |
| CA_4A-5A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_4A-7A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_4A-12A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_4A-13A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_4A-17A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_4A-28A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_5A-7A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_5A-12A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_5A-17A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_5A-30A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_5A-40A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_5A-66A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_7A-8A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_7A-20A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_7A-26A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_7A-28A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_8A-39A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_8A-41A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_11A-18A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_11A-26A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_12A-30A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_12A-66A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_13A-66A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_14A-30A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_14A-66A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_18A-28A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_19A-21A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_19A-42A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_21A-28A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_21A-42A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_25A-26A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_25A-41A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_26A-46A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_26A-48A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_28A-41A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_28A-42A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_30A-66A |  |  |  |  | 23 | +2/-3 |  |  |
| CA_39A-41A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_39A-41C |  |  |  |  | 23 | +2/-32 |  |  |
| CA_39C-41A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_40A-42A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_41A-42A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_41A-42C |  |  |  |  | 23 | +2/-32 |  |  |
| CA_41C-42A |  |  |  |  | 23 | +2/-32 |  |  |
| CA_41C-42C |  |  |  |  | 23 | +2/-32 |  |  |
| NOTE 1: Void NOTE 2: 2 refers to the transmission bandwidths (Figure 5.6-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 3: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 4: For inter-band carrier aggregation the maximum power requirement should apply to the total transmitted power over all component carriers (per UE). NOTE 5: For a UE that supports both Band 18 and Band 26, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB for transmission bandwidths confined within 815 MHz and 818 MHz. |  |  |  |  |  |  |  |  |

For uplink intra-band contiguous carrier aggregation the maximum output power is specified in Table 6.2.2A-1. For downlink intra-band contiguous carrier aggregation with a single uplink component carrier configured in the E-UTRA band, the maximum output power is specified in Table 6.2.2-1.

For a power class 2 capable UE operating with intra-band uplink contiguous CA bandwidth class C on Band 41, when an IE P-max as defined in TS 36.331 [7] of 23 dBm or lower is indicated in the cell or if the uplink/downlink configuration is 0 or 6, the requirements for power class 2 are not applicable, and the corresponding requirements for a power class 3 UE shall apply.

Table 6.2.2A-1: CA UE Power Class for intraband contiguous CA

| E-UTRA CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_1C |  |  |  |  | 23 | +2/-2 |  |  |
| CA_3C |  |  |  |  | 23 | +2/-22 |  |  |
| CA_5B |  |  |  |  | 23 | +2/-22 |  |  |
| CA_7C |  |  |  |  | 23 | +2/-22 |  |  |
| CA_8B |  |  |  |  | 23 | +2/-22 |  |  |
| CA_38C |  |  |  |  | 23 | +2/-2 |  |  |
| CA_39C |  |  |  |  | 23 | +2/-2 |  |  |
| CA_40C |  |  |  |  | 23 | +2/-2 |  |  |
| CA_40D |  |  |  |  | 23 | +2/-2 |  |  |
| CA_41C |  |  | 26 | +2/-22 | 23 | +2/-22 |  |  |
| CA_41D |  |  |  |  | 23 | +2/-22 |  |  |
| CA_42C |  |  |  |  | 23 | +2/-3 |  |  |
| CA_48B |  |  |  |  | 23 | +2/-3 |  |  |
| CA_48C |  |  |  |  | 23 | +2/-3 |  |  |
| CA_66B |  |  |  |  | 23 | +2/-2 |  |  |
| CA_66C |  |  |  |  | 23 | +2/-2 |  |  |
| NOTE 1: Void NOTE 2: If all transmitted resource blocks (Figure 5.6A-1) over all component carriers are confined within FUL_low and FUL_low + 4 MHz or/and FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 3: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 4: For intra-band contiguous carrier aggregation the maximum power requirement should apply to the total transmitted power over all component carriers (per UE). |  |  |  |  |  |  |  |  |

For intra-band non-contiguous carrier aggregation with one uplink carrier on the PCC, the requirements in subclause 6.2.2 apply. For intra-band non-contiguous carrier aggregation with two uplink carriers the maximum output power is specified in Table 6.2.2A-2.

Table 6.2.2A-2: UE Power Class for intraband non-contiguous CA

| E-UTRA CA Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA_4A-4A |  |  |  |  | 23 | +2/-2 |  |  |
| NOTE 1: For transmission bandwidths (Figure 5.6-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 2: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 3: For intra-band non-contiguous carrier aggregation the maximum power requirement should apply to the total transmitted power over all component carriers (per UE). |  |  |  |  |  |  |  |  |

### 6.2.2B	UE maximum output power for UL-MIMO

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the maximum output power for any transmission bandwidth within the channel bandwidth is specified in Table 6.2.2B-1. The requirements shall be met with the UL-MIMO configurations specified in Table 6.2.2B-2. For UE supporting UL-MIMO, the maximum output power is measured as the sum of the maximum output power at each UE antenna connector. The period of measurement shall be at least as defined in Table 6.2.2B-0.

Table 6.2.2B-0: Measurement period for UE maximum output power for UL-MIMO

| TTI pattern | Minimum measurement period |
| --- | --- |
| Subframe | 1ms |
| Slot | 7OS |
| Subslot | 2OS, 3OS |

Table 6.2.2B-1: UE Power Class for UL-MIMO in closed loop spatial multiplexing scheme

| EUTRA band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  |  |  | 23 | +2/-3 |  |  |
| 2 |  |  |  |  | 23 | +2/-32 |  |  |
| 3 |  |  |  |  | 23 | +2/-32 |  |  |
| 4 |  |  |  |  | 23 | +2/-3 |  |  |
| 5 |  |  |  |  | 23 | +2/-3 |  |  |
| 6 |  |  |  |  | 23 | +2/-3 |  |  |
| 7 |  |  |  |  | 23 | +2/-32 |  |  |
| 8 |  |  |  |  | 23 | +2/-32 |  |  |
| 9 |  |  |  |  | 23 | +2/-3 |  |  |
| 10 |  |  |  |  | 23 | +2/-3 |  |  |
| 11 |  |  |  |  | 23 | +2/-3 |  |  |
| 12 |  |  |  |  | 23 | +2/-32 |  |  |
| 13 |  |  |  |  | 23 | +2/-3 |  |  |
| 14 |  |  |  |  | 23 | +2/-3 |  |  |
| 17 |  |  |  |  | 23 | +2/-3 |  |  |
| 18 |  |  |  |  | 23 | +2/-3 |  |  |
| 19 |  |  |  |  | 23 | +2/-3 |  |  |
| 20 |  |  |  |  | 23 | +2/-32 |  |  |
| 21 |  |  |  |  | 23 | +2/-3 |  |  |
| 22 |  |  |  |  | 23 | +2/-4.52 |  |  |
| 23 |  |  |  |  | 23 | +2/-3 |  |  |
| 24 |  |  |  |  | 23 | +2/-42 |  |  |
| 25 |  |  |  |  | 23 | +2/-32 |  |  |
| 26 |  |  |  |  | 23 | +2/-32 |  |  |
| 27 |  |  |  |  | 23 | +2/-3 |  |  |
| 28 |  |  |  |  | 23 | +2/[-3] |  |  |
| 30 |  |  |  |  | 23 | +2/-3 |  |  |
| 31 | 31 | +2/-3 |  |  | 23 | +2/-3 |  |  |
| … |  |  |  |  |  |  |  |  |
| 33 |  |  |  |  | 23 | +2/-3 |  |  |
| 34 |  |  |  |  | 23 | +2/-3 |  |  |
| 35 |  |  |  |  | 23 | +2/-3 |  |  |
| 36 |  |  |  |  | 23 | +2/-3 |  |  |
| 37 |  |  |  |  | 23 | +2/-3 |  |  |
| 38 |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| 39 |  |  |  |  | 23 | +2/-3 |  |  |
| 40 |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| 41 |  |  | 26 | +2/-32 | 23 | +2/-32 |  |  |
| 42 |  |  | 26 | +2/-4 | 23 | +2/-4 |  |  |
| 43 |  |  |  |  | 23 | +2/-4 |  |  |
| 44 |  |  |  |  | 23 | +2/[-3] |  |  |
| 45 |  |  |  |  | 23 | +2/-3 |  |  |
| 48 |  |  |  |  | 23 | +2/-3 |  |  |
| 50 |  |  |  |  | 23 | +2/-3 |  |  |
| 51 |  |  |  |  | 23 | +2/-3 |  |  |
| 52 |  |  |  |  | 23 | +2/-3 |  |  |
| 53 |  |  |  |  | 23 | +2/-3 |  |  |
| … |  |  |  |  |  |  |  |  |
| 65 |  |  |  |  | 23 | +2/-3 |  |  |
| 66 |  |  |  |  | 23 | +2/-3 |  |  |
| 68 |  |  |  |  | 23 | +2/-3 |  |  |
| .. |  |  |  |  |  |  |  |  |
| 70 |  |  |  |  | 23 | +2/-3 |  |  |
| 71 |  |  |  |  | 23 | +2/-3 |  |  |
| 72 | 31 | +2/-3 |  |  | 23 | +2/-3 |  |  |
| 73 |  |  |  |  | 23 | +2/-3 |  |  |
| 74 |  |  |  |  | 23 | +2/-3 |  |  |
| NOTE 1: Void NOTE 2: 2 refers to the transmission bandwidths (Figure 5.6-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 3: For the UE which supports both Band 11 and Band 21 operating frequencies, the tolerance is FFS. NOTE 4: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 5: Void. |  |  |  |  |  |  |  |  |

The default power class for an operating band is Power Class 3 unless otherwise stated.

For a power class 2 capable UE operating on Band 41, when an IE P-max as defined in TS 36.331 [7] of 23 dBm or lower is indicated in the cell or if the uplink/downlink configuration is 0 or 6, the requirements for power class 2 are not applicable and the corresponding requirements for a power class 3 UE shall apply.

For each supported frequency band other than Band 41, the UE shall:

-	if the UE supports a different power class than the UE default power class for the band and the supported power class enables the higher maximum output power than that of the default power class:

-	if the band is a TDD band whose frame configuration is 0 or 6; or

-	if the IE P-Max as defined in TS 36.331 [7] is not provided; or

-	if the IE P-Max as defined in TS 36.331 [7] is provided and set to the maximum output power of the default power class or lower;

-	meet all requirements for the default power class of the operating band in which the UE is operating and set its configured transmitted power as specified in sub-clause 6.2.5;

-	else (i.e the IE P-Max as defined in TS 36.331 [7] is provided and set to the higher value than the maximum output power of the default power class):

-	meet all requirements for the supported power class and set its configured transmitted power as specified in sub-clause 6.2.5;

Table 6.2.2B-2: UL-MIMO configuration in closed-loop spatial multiplexing scheme

| Transmission mode | DCI format | Codebook Index |
| --- | --- | --- |
| Mode 2 | DCI format 4 | Codebook index 0 |

If UE is configured for transmission on single-antenna port, the requirements in subclause 6.2.2 apply.

### 6.2.2C		Void

<reserved for future use>

### 6.2.2D	UE maximum output power for ProSe

When UE is configured for simultaneous E-UTRA ProSe sidelink and E-UTRA uplink transmissions for inter-band E-UTRA ProSe / E-UTRA bands specified in Table 5.5D-2, the UE maximum output power shall be as specified in Table 6.2.2A-0 in subclause 6.2.2A for the corresponding inter-band aggregation with uplink assigned to two bands.

If UE is configured to oprerate on single E-UTRA ProSe sidelink band or E-UTRA uplink band specidied in Table 5.5D-1, the requirements in subclause 6.2.2 apply.

### 6.2.2E	UE maximum output power for Category M1 and M2 UE

The following UE Power Classes define the maximum output power for any transmission bandwidth within the channel bandwidth for non CA configuration and UL-MIMO unless otherwise stated. The period of measurement shall be at least one sub frame (1ms).

Table 6.2.2E-1: UE Power Class

| EUTRA band | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) | Class 6 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  |  | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 2 |  |  | 23 | ±22 | 20 | ±22 | 14 | ±2.5 |
| 3 |  |  | 23 | ±22 | 20 | ±22 | 14 | ±2.5 |
| 4 |  |  | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 5 |  |  | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 7 |  |  | 23 | ±22 | 20 | ±22 | 14 | ±2.5 |
| 8 |  |  | 23 | ±22 | 20 | ±22 | 14 | ±2.5 |
| 11 |  |  | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 12 |  |  | 23 | ±22 | 20 | ±22 | 14 | ±2.5 |
| 13 |  |  | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 14 |  |  | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 18 |  |  | 23 | ±25 | 20 | ±25 | 14 | ±2.5 |
| 19 |  |  | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 20 |  |  | 23 | ±22 | 20 | ±22 | 14 | ±2.5 |
| 21 |  |  | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 25 |  |  | 23 | ±22 | 20 | ±2 | 14 | ±2.5 |
| 26 |  |  | 23 | ±22 | 20 | ±22 | 14 | ±2.5 |
| 27 |  |  | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 28 |  |  | 23 | +2/-2.5 | 20 | +2/-2.5 | 14 | ±2.5 |
| 31 | 267 | ±2 | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| … |  |  |  |  |  |  |  |  |
| 39 |  |  | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 40 |  |  | 23 | ±22 | 20 | ±2 | 14 | ±2.5 |
| 41 |  |  | 23 | ±22 | 20 | ±22 | 14 | ±2.5 |
| 42 |  |  | 23 | +2/-3 | 20 | +2/-3 | 14 | ±2.5 |
| 43 |  |  | 23 | +2/-3 | 20 | +2/-3 | 14 | ±2.5 |
| … |  |  |  |  |  |  |  |  |
| 71 |  |  | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 72 | 267 | ±2 | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 73 |  |  | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 74 |  |  | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 85 |  |  | 23 | ±22 | 20 | ±22 | 14 | ±2.5 |
| 87 |  |  | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 88 |  |  | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| NOTE 1: Void NOTE 2: 2 refers to the transmission bandwidths (Figure 5.6-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 3: For the UE which supports both Band 11 and Band 21 operating frequencies, the tolerance is FFS. NOTE 4: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 5: For a UE that supports both Band 18 and Band 26, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB for transmission bandwidths confined within 815 MHz and 818 MHz. NOTE 6: Void NOTE 7: Applicable for category M1 and M2 HD-FDD UE |  |  |  |  |  |  |  |  |

### 6.2.2F	UE maximum output power for category NB1 and NB2

Category NB1 and NB2 UE Power Classes are specified in Table 6.2.2F-1 and define the maximum output power for any transmission bandwidth within the category NB1 and NB2 channel bandwidth. For 3.75 kHz sub-carrier spacing the maximum output power is defined as mean power of measurement which period is atleast one slot (2ms) excluding the 2304Ts gap when UE is not transmitting. For 15kHz sub-carrier spacing the maximum output power is defined as mean power of measurement which period is atleast one sub-frame (1ms).

Table 6.2.2F-1: UE Power Class

| EUTRA band | Class 3 (dBm) | Tolerance (dB) | Class 5 (dBm) | Tolerance (dB) | Class 6 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 2 | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 3 | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 4 | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 5 | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 7 | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 8 | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 11 | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 12 | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 13 | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 14 | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 17 | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 18 | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 19 | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 20 | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 21 | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 25 | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 26 | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 28 | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 31 | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 41 | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 42 | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 43 | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 65 | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 66 | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 70 | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 71 | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 72 | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 73 | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 74 | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 85 | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 87 | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |
| 88 | 23 | ±2 | 20 | ±2 | 14 | ±2.5 |

### 6.2.2G	UE maximum output power for V2X Communication

When UE is configured for E-UTRA V2X sidelink transmissions non-concurrent with E-UTRA uplink transmissions for E-UTRA V2X operating bands specified in Table 5.5G-1, the allowed V2X UE maximum output power for shall be as applied in Table 6.2.2-1 in subclause 6.2.2.

For V2X UE is configured for simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions for inter-band E-UTRA V2X / E-UTRA bands specified in Table 5.5G-2, the UE maximum output power shall be as specified in Table 6.2.2G-1 in subclause 6.2.2G for the corresponding inter-band con-current operation with uplink assigned to two bands.

Table 6.2.2G-1: Inter-band con-current V2X UE Power Class (two bands)

| V2X con-current band Configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V2X_3A-47A |  |  |  |  | 23 | +2/-34 |  |  |
| V2X_5A-47A |  |  |  |  | 23 | +2/-34 |  |  |
| V2X_7A-47A |  |  |  |  | 23 | +2/-34 |  |  |
| V2X_8A-47A |  |  |  |  | 23 | +2/-34 |  |  |
| V2X_20A-47A |  |  |  |  | 23 | +2/-34 |  |  |
| V2X_28A-47A |  |  |  |  | 23 | +2/-34 |  |  |
| V2X_34A-47A |  |  |  |  | 23 | +2/-34 |  |  |
| V2X_39A-47A |  |  |  |  | 23 | +2/-34 |  |  |
| V2X_41A-47A |  |  |  |  | 23 | +2/-34 |  |  |
| V2X_71A-47A |  |  |  |  | 23 | +2/-34 |  |  |
| NOTE 1: The con-current band combinations is used for V2X Service. NOTE 2: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 3: For inter-band con-current aggregation the maximum power requirement apply to the total transmitted power over all component carriers (per UE). NOTE 4: 4 refers to the transmission bandwidths (Figure 5.6-1) confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB |  |  |  |  |  |  |  |  |

For intra-band contiguous multi-carrier operation, the maximum output power is defined in Table 6.2.2G-2.

Table 6.2.2G-2: V2X UE Power Class for intra-band contiguous multi-carrier operation

| V2X intra-band multi-carrier configuration | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| V2X_47B |  |  |  |  | 23 | ±2 |  |  |
| V2X_47C |  |  |  |  | 23 | ±2 |  |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance NOTE 2: For intra-band multi-carrier operation, the maximum power requirement should apply to the total transmitted power over all component carriers (per UE). |  |  |  |  |  |  |  |  |

When a UE is configured for E-UTRA V2X sidelink transmissions in Band 47, the UE shall meet the following additional requirements for transmission within the frequency ranges 5855-5925 MHz:

-	The maximum mean power spectral density shall be restricted to 23 dBm/MHz EIRP when the network signaling value NS_33 or NS_34 is indicated.

where the network signaling values are specified in clause 6.2.4G.

NOTE:	The PSD limit in EIRP shall be converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex I.

For V2X UE supporting Transmit Diversity, if the UE transmits on two connectors at the same time, the maximum output power for any transmission bandwidth within the channel bandwidth is specified in Table 6.2.2G-3. The maximum output power is measured as the sum of the maximum output power at each UE antenna connector. The period of measurement shall be at least one sub frame (1ms).

Table 6.2.2G-3: V2X UE Power Class for Transmit Diversity scheme

| E-UTRA band | Class 1 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 47 |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |

If the UE transmits on one antenna connector at a time, the requirements in Table 6.2.2-1 shall apply to the active antenna connector.

### 6.2.3	UE maximum output power for modulation / channel bandwidth

For UE Power Class 1, 2 and 3, the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2.2-1due to higher order modulation and transmit bandwidth configuration (resource blocks) is specified in Table 6.2.3-1.

Table 6.2.3-1: Maximum Power Reduction (MPR) for Power Class 1, 2 and 3

| Modulation | Channel bandwidth / Transmission bandwidth (NRB) |  |  |  |  |  | MPR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |
| QPSK | > 5 | > 4 | > 8 | > 12 | > 16 | > 18 | ≤ 1 |
| 16 QAM | ≤ 5 | ≤ 4 | ≤ 8 | ≤ 12 | ≤ 16 | ≤ 18 | ≤ 1 |
| 16 QAM | > 5 | > 4 | > 8 | > 12 | > 16 | > 18 | ≤ 2 |
| 64 QAM | ≤ 5 | ≤ 4 | ≤ 8 | ≤ 12 | ≤ 16 | ≤ 18 | ≤ 2 |
| 64 QAM | > 5 | > 4 | > 8 | > 12 | > 16 | > 18 | ≤ 3 |
| 256 QAM | ≥ 1 |  |  |  |  |  | ≤ 5 |

For PRACH, PUCCH and SRS transmissions, the allowed MPR is according to that specified for PUSCH QPSK modulation for the corresponding transmission bandwidth.

For each TTI pattern, the MPR shall be evaluated per Teval period as specified in table 6.2.3-2 and given by the maximum value taken over the transmission(s) within that period; the maximum MPR over TREF is then applied for TREF.

Table 6.2.3-2: MPR evaluation period

| TTI pattern | TREF | Teval |
| --- | --- | --- |
| Subframe | 1 subframe | 1 slot |
| Slot | 7 OS | Min(Tno_hopping, 7OS) |
| Sublot | 2 OS, 3OS | Min(Tno_hopping, 2OS/3OS) |

For UE Power Class 1 and 3 transmissions with non-contiguous resource allocation in single component carrier, the allowed Maximum Power Reduction (MPR) for the maximum output power in table 6.2.2-1, is specified as follows

MPR = CEIL {MA, 0.5}

Where MA is defined as follows for QPSK, 16 QAM and 64 QAM

MA =	8.00-10.12A		; 0.00< A ≤ 0.33

5.67 - 3.07A		; 0.33< A ≤0.77

3.31				; 0.77< A ≤1.00

Where MA is defined as follows for 256 QAM

MA = 8.00-10.12A		; 0.00< A ≤ 0.25

5.50				; 0.25< A < 1.00

Where

A = NRB_alloc / NRB.

CEIL{MA, 0.5} means rounding upwards to closest 0.5dB, i.e. MPR  [3.0, 3.5 4.0 4.5 5.0 5.5 6.0 6.5 7.0 7.5 8.0]

The allowed MPR for transmission on an Scell in Band 46 or Band 49 within a component carrier of a nominal channel bandwidth of 10 MHz or 20 MHz is in accordance with 6.2.3-1 for RIV = ‘11111’ (10 MHz) and L = 10 (20 MHz) with L defined in Clause 8.1.4 of [6]. For all other possible values of the RIV defined in Clause 8.1.4 of [6] the allowed MPR is 2.5 dB for QPSK modulation, 3 dB for 16QAM modulation and 4 dB for 64QAM modulation (256QAM is FFS).

For a power class 2 capable UE operating on Band 41, when an IE P-max as defined in [7] of 23 dBm or lower is indicated in the cell or if the uplink/downlink configuration is 0 or 6, the requirements for power class 2 are not applicable, and the corresponding requirements for a power class 3 UE shall apply.

For each supported frequency band other than Band 14 and Band 41, the UE shall:

-	if the UE supports a different power class than the default UE power class for the band and the supported power class enables the higher maximum output power than that of the default power class:

-	if the band is a TDD band whose frame configuration is 0 or 6; or

-	if the IE P-Max as defined in TS 36.331 [7] is not provided; or

-	if the IE P-Max as defined in TS 36.331 [7] is provided and set to the maximum output power of the default power class or lower;

-	meet all requirements for the default power class of the operating band in which the UE is operating and set its configured transmitted power as specified in sub-clause 6.2.5;

-	else (i.e the IE P-Max as defined in TS 36.331 [7] is provided and set to the higher value than the maximum output power of the default power class):

-	meet all requirements for the supported power class and set its configured transmitted power class as specified in sub-clause 6.2.5.

For UE Power Class 2 transmissions with non-contiguous resource allocation in single component carrier, the allowed Maximum Power Reduction (MPR) for the maximum output power is not specified in this version of the specification.

For the UE maximum output power modified by MPR, the power limits specified in subclause 6.2.5 apply.

### 6.2.3A	UE Maximum Output power for modulation / channel bandwidth for CA

For inter-band carrier aggregation with one uplink component carrier assigned to one E-UTRA band, the requirements in subclause 6.2.3 apply. For inter-band carrier aggregation with two uplink contiguous component carrier assigned to one E-UTRA band specified in this clause for intra-band contiguous carrier aggregation apply for that band.

For inter-band carrier aggregation with one component carrier per operating band and the uplink active in two E-UTRA bands, the requirements in subclause 6.2.3 apply for each uplink component carrier.

For intra-band contiguous carrier aggregation the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2.2A-1due to higher order modulation and contiguously aggregated transmit bandwidth configuration (resource blocks) is specified in Table 6.2.3A-1 for UE power class 3 CA bandwidth classes B and C, in Table 6.2.3A-1a for UE power class 2 CA bandwidth class C, and Table 6.2.3A-2 for UE power class 3 CA bandwidth class D. In case the modulation format is different on different component carriers then the MPR is determined by the rules applied to higher order of those modulations.

Table 6.2.3A-1: Maximum Power Reduction (MPR) for Power Class 3

| Modulation | CA bandwidth Class B and C / Smallest Component Carrier Transmission Bandwidth Configuration |  |  |  | MPR (dB) |
| --- | --- | --- | --- | --- | --- |
|  | 25 RB | 50 RB | 75 RB | 100 RB |  |
| QPSK | > 8 and ≤ 25 | > 12 and ≤ 50 | > 16 and ≤ 75 | > 18 and ≤ 100 | ≤ 1 |
| QPSK | > 25 | > 50 | > 75 | > 100 | ≤ 2 |
| 16 QAM | ≤ 8 | ≤ 12 | ≤ 16 | ≤ 18 | ≤ 1 |
| 16 QAM | > 8 and ≤ 25 | > 12 and ≤ 50 | > 16 and ≤ 75 | > 18 and ≤ 100 | ≤ 2 |
| 16 QAM | > 25 | > 50 | > 75 | > 100 | ≤ 3 |
| 64 QAM | ≤ 8 and allocation wholly contained within a single CC | ≤ 12 and allocation wholly contained within a single CC | ≤ 16 and allocation wholly contained within a single CC | ≤ 18 and allocation wholly contained within a single CC | ≤ 2 |
| 64 QAM | > 8 or allocation extends across two CC's | > 12 or allocation extends across two CC's | > 16 or allocation extends across two CC's | > 18 or allocation extends across two CC's | ≤ 3 |
| 256 QAM | ≥ 1 |  |  |  | ≤ 5 |

Table 6.2.3A-1a: Maximum Power Reduction (MPR) for Power Class 2

| Modulation | CA bandwidth Class C / Smallest Component Carrier Transmission Bandwidth Configuration |  |  |  | MPR (dB) |
| --- | --- | --- | --- | --- | --- |
|  | 25 RB | 50 RB | 75 RB | 100 RB |  |
| QPSK | > 6 and ≤ 25 | > 6 and ≤ 50 | > 6 and ≤ 75 | > 6 and ≤ 100 | ≤ 1 |
| QPSK | > 25 | > 50 | > 75 | > 100 | ≤ 2 |
| 16 QAM | ≤ 6 | ≤ 8 | ≤ 16 | ≤ 18 | ≤ 1.5 |
| 16 QAM | > 6 and ≤ 25 | > 8 and ≤ 50 | > 16 and ≤ 75 | > 18 and ≤ 100 | ≤ 2 |
| 16 QAM | > 25 | > 50 | > 75 | > 100 | ≤ 3 |
| 64 QAM | ≤ 8 and allocation wholly contained within a single CC | ≤ 12 and allocation wholly contained within a single CC | ≤ 16 and allocation wholly contained within a single CC | ≤ 18 and allocation wholly contained within a single CC | ≤ 2 |
| 64 QAM | > 8 or allocation extends across two CC's | > 12 or allocation extends across two CC's | > 16 or allocation extends across two CC's | > 18 or allocation extends across two CC's | ≤ 3 |
| 256 QAM | ≥ 1 |  |  |  | ≤ 6 |

Table 6.2.3A-2: Maximum Power Reduction (MPR) for Class 3

| Modulation | CA bandwidth Class D |  |  |  |  | MPR (dB) |
| --- | --- | --- | --- | --- | --- | --- |
|  | 50 RB + 75 RB + 100RB | 50 RB + 100 RB + 100 RB | 75 RB + 75 RB + 100 RB | 75 RB + 100 RB + 100 RB | 100 RB + 100 RB + 100 RB |  |
| QPSK | > 12 and ≤ 50 | > 12 and ≤ 50 | > 16 and ≤ 75 | > 16 and ≤ 75 | > 18 and ≤ 100 | ≤ 1 |
| QPSK | > 50 and ≤ 125 | > 50 and ≤ 150 | > 75 and ≤ 150 | > 75 and ≤ 175 | > 100 and ≤ 200 | ≤ 2 |
| QPSK | > 125 | > 150 | > 150 | > 175 | > 200 | ≤ 3 |
| 16 QAM | ≤ 12 | ≤ 12 | ≤ 16 | ≤ 16 | ≤ 18 | ≤ 1 |
| 16 QAM | > 12 and ≤ 50 | > 12 and ≤ 50 | > 16 and ≤ 75 | > 16 and ≤ 75 | > 18 and ≤ 100 | ≤ 2 |
| 16 QAM | > 50 and ≤ 125 | > 50 and ≤ 150 | > 75 and ≤ 150 | > 75 and ≤ 175 | > 100 and ≤ 200 | ≤ 3 |
| 16 QAM | > 125 | > 150 | > 150 | >175 | > 200 | ≤ 3.5 |
| 64 QAM | ≤ 12 allocation wholly contained within a single CC | ≤ 12 and allocation wholly contained within a single CC | ≤ 16 and allocation wholly contained within a single CC | ≤ 16 and allocation wholly contained within a single CC | ≤ 18 and allocation wholly contained within a single CC | ≤ 2 |
| 64 QAM | > 12 allocation wholly contained within a single CC or allocation extends across two CC’s | > 12 allocation wholly contained within a single CC or allocation extends across two CC’s | > 16 allocation wholly contained within a single CC or allocation extends across two CC’s | > 16 allocation wholly contained within a single CC or allocation extends across two CC’s | > 18 allocation wholly contained within a single CC or allocation extends across two CC’s | ≤ 3 |
| 64 QAM | allocation extends across three CC’s | allocation extends across three CC’s | allocation extends across three CC’s | allocation extends across three CC’s | allocation extends across three CC’s | ≤ 4.0 |
| 256 QAM | ≥ 1 |  |  |  |  | ≤ 5.5 |

For PUCCH and SRS transmissions, the allowed MPR is according to that specified for PUSCH QPSK modulation for the corresponding transmission bandwidth.

For UE power class 3 intra-band contiguous carrier aggregation bandwidth class C with non-contiguous resource allocation, the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2.2A-1 is specified as follows

MPR = CEIL { min(MA, MIM5), 0.5}

Where MA is defined as follows for QPSK, 16 QAM and 64 QAM

MA = 	8.2						; 0 ≤ A < 0.025

9.2 - 40A 			; 0.025	≤ A < 0.05

8 – 16A				; 0.05	≤ A < 0.25

4.83 – 3.33A			; 0.25 ≤ A ≤ 0.4,

3.83 – 0.83A			; 0.4 ≤ A ≤ 1,

Where MA is defined as follows for 256 QAM

MA = 	8.2						; 0 ≤ A < 0.025

9.2 - 40A				; 0.025 ≤ A < 0.05

8 – 16A				; 0.05 ≤ A < 0.16

5.5					; 0.16 ≤ A < 1

and MIM5 is defined as follows

MIM5 =	4.5			; IM5 < 1.5 * BWChannel_CA

6.0		; 1.5 * BWChannel_CA ≤ IM5 <  BWChannel_CA/2 + FOOB

MA		; IM5 ≥ BWChannel_CA/2 + FOOB

For UE power class 2 intra-band contiguous carrier aggregation bandwidth class C with non-contiguous resource allocation, the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2.2A-1 is specified as follows

MPR = CEIL { min(MA, MIM5), 0.5}

Where MA is defined as follows for QPSK, 16 QAM and 64 QAM

MA = 	8.2						; 0 ≤ A < 0.04

9.2 - 40A 				; 0.04 ≤ A < 0.075

8 – 16A				; 0.075 ≤ A < 0.25

4.83 – 3.33A			; 0.25 ≤ A ≤ 0.4,

3.83 – 0.83A			; 0.4 ≤ A ≤ 1,

Where MA is defined FFS for 256 QAM

and MIM5 is defined as follows

MIM5 =	5.0			; IM5 < 1.5 * BWChannel_CA

6.0		; 1.5 * BWChannel_CA ≤ IM5 <  BWChannel_CA/2 + FOOB

MA		; IM5 ≥ BWChannel_CA/2 + FOOB

For UE power class 3 intra-band contiguous carrier aggregation bandwidth class B with non-contiguous resource allocation, the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2.2A-1 is specified as follows

MPR = CEIL { MA, 0.5}

Where MA is defined as follows for QPSK, 16 QAM and 64 QAM

MA =	10.5 – 17.5A			; 0 ≤ A < 0.2

8.5 – 7.5A 			; 0.2 ≤ A < 0.6

5.5 – 2.5A 			; 0.6 ≤ A ≤ 1

Where MA is defined as follows for 256 QAM

MA =	10.5 – 17.5A			; 0 ≤ A < 0.2

8.5 – 7.5A 			; 0.2 ≤ A < 0.4

5.5  					; 0.4 ≤ A ≤ 1

Where

A = NRB_alloc / NRB_agg.

IM5 = max( | FC_agg  – (3*Fagg_alloc_low – 2*Fagg_alloc_high) |,  | FC_agg  – (3*Fagg_alloc_high – 2*Fagg_alloc_low) | )

FC_agg = (Fedge_high + Fedge_low)/2

For UE power class 3 intra-band contiguous carrier aggregation bandwidth class D with non-contiguous resource allocation, the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2.3A-2 is specified as follows

MPR = CEIL { min(MA, MIM5), 0.5}

Where MA is defined as follows for QPSK, 16 QAM and 64 QAM

MA = 	8.2						; 0 ≤ A < 0.025

9.2 - 40A 			; 0.025	 ≤ A < 0.05

8 – 16A				; 0.05	≤ A < 0.25

4.0					; 0.25 ≤ A < 1

Where MA is defined as follows for 256 QAM

MA =		8.2 				; 0 ≤ A < 0.025

9.2 - 40A			; 0.025 ≤ A < 0.05

8 – 16A			; 0.05 ≤ A < 0.16

5.5				; 0.16 ≤ A < 1

and MIM5 is defined as follows

MIM5 =	4.5			; IM5 < 1.5 * BWChannel_CA

6.0		; 1.5 * BWChannel_CA ≤ IM5 <  BWChannel_CA/2 + FOOB

MA		; IM5 ≥ BWChannel_CA/2 + FOOB

CEIL{MA, 0.5} means rounding upwards to closest 0.5dB, i.e. MPR[3.0, 3.5, 4.0, 4.5, 5.0, 5.5, 6.0, 6.5, 7.0, 7.5, 8.0, 8.5].

For intra-band non-contiguous carrier aggregation with one uplink carrier, the requirements in subclause 6.2.3 apply.

For intra-band non-contiguous carrier aggregation with two uplink carriers MPR is specified for E-UTRA CA configurations with a maximum possible WGAP ≤ 35 MHz; the allowed MPR is

MPR = CEIL {MN, 0.5}

where MN is defined as follows

MN=		-0.125 N + 18.25		; 2 ≤ N ≤ 50

-0.0333 N + 13.67		; 50 < N ≤ 200

where N= NRB_alloc is the number of allocated resource blocks. Clause 6.2.3 does not apply in addition. E-UTRA CA configurations with a maximum possible Wgap > 35 MHz and their corresponding MPR are intended to form part of a later release.

For intra-band carrier aggregation, the MPR is evaluated per Teval period specified in table 6.2.3A-3 and given by the maximum value taken over the transmission(s) on all component carriers within that period; the maximum MPR over TREF is then applied for TREF.

Table 6.2.3A-3: MPR evaluation period for CA

| TTI pattern | TREF | Teval |
| --- | --- | --- |
| Subframe | 1 subframe | 1 slot |
| Slot | 7 OS | Min(Tno_hopping, 7OS) |
| Sublot | 2 OS, 3OS | Min(Tno_hopping, 2OS/3OS) |

For combinations of intra-band and inter-band carrier aggregation with three uplink component carriers (up to two contiguously aggregated carriers per band), the requirements specified in subclause 6.2.3 apply for the E-UTRA band supporting one component carrier, and for the E-UTRA band supporting two contiguous component carriers the requirements specified in subclause 6.2.3A apply.

For the UE maximum output power modified by MPR, the power limits specified in subclause 6.2.5A apply.

### 6.2.3B	UE maximum output power for modulation / channel bandwidth for UL-MIMO

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2.2B-1 is specified in Table 6.2.3-1. The requirements shall be met with UL-MIMO configurations defined in Table 6.2.2B-2. For UE supporting UL-MIMO, the maximum output power is measured as the sum of the maximum output power at each UE antenna connector.

For the UE maximum output power modified by MPR, the power limits specified in subclause 6.2.5B apply.

If UE is configured for transmission on single-antenna port, the requirements in subclause 6.2.3 apply.

### 6.2.3D	UE maximum output power for modulation / channel bandwidth for ProSe

When UE is configured for E-UTRA ProSe sidelink transmissions non-concurrent with E-UTRA uplink transmissions for E-UTRA ProSe operating bands specified in Table 5.5D-1, this subclause specifies the allowed Maximum Power Reduction (MPR) power for ProSe physical channels and signals due to higher order modulation and transmit bandwidth configuration (resource blocks).

The allowed MPR for the maximum output power for ProSe physical channels PSDCH, PSCCH, PSSCH, and PSBCH shall be as specified in subclause 6.2.3 for  PUSCH for the corresponding modulation and transmission bandwidth.

The allowed MPR for the maximum output power for ProSe physical signal PSSS shall be as be as specified in subclause 6.2.3 for PUSCH QPSK modulation for the corresponding transmission bandwidth.

The allowed MPR for the maximum output power for ProSe physical signal SSSS is specified in Table 6.2.3D-1.

For a power class 2 capable UE operating on Band 41, the corresponding requirements for a power class 3 UE apply when an IE P-max as defined in [7] of 23 dBm or lower is indicated in the cell or if the uplink/downlink configuration is 0 or 6.

For each supported frequency band other than Band 14 and Band 41, the UE shall:

-	if the UE supports a different power class than the default UE power class for the band and the supported power class enables the higher maximum output power than that of the default power class:

-	if the band is a TDD band whose frame configuration is 0 or 6; or

-	if the IE P-Max as defined in TS 36.331 [7] is not provided; or

-	if the IE P-Max as defined in TS 36.331 [7] is provided and set to the maximum output power of the default power class or lower;

-	meet all requirements for the default power class of the operating band in which the UE is operating and set its configured transmitted power as specified in sub-clause 6.2.5;

-	else (i.e the IE P-Max as defined in TS 36.331 [7] is provided and set to the higher value than the maximum output power of the default power class):

-	meet all requirements for the supported power class and set its configured transmitted power class as specified in sub-clause 6.2.5.

Table 6.2.3D-1: Maximum Power Reduction (MPR) for SSSS for Power Class 1, 2 and 3

| Channel bandwidth | MPR for SSSS (dB) |
| --- | --- |
| 1.4 MHz |  |
| 3.0 MHz |  |
| 5.0 MHz | ≤ 4 |
| 10 MHz | ≤ 4 |
| 15 MHz | ≤ 4 |
| 20 MHz | ≤ 4 |

When UE is configured for simultaneous E-UTRA ProSe sidelink and E-UTRA uplink transmissions for inter-band E-UTRA ProSe / E-UTRA bands specified in Table 5.5D-2, the requirements in subclause 6.2.3D apply for ProSe transmission and the requirements in subclause 6.2.3 apply for uplink transmission.

### 6.2.3E	UE maximum output power for modulation / channel bandwidth for category M1 and M2

For category M1 UE Power Class 3, 5 and 6, the allowed Maximum Power Reduction (MPR) for the maximum output power specified in Table 6.2.2E-1 due to higher order modulation and transmit bandwidth configuration (resource blocks) is specified in Tables 6.2.3E-1, 6.2.3E-2 and 6.2.3E-5 respectively.

For category M2 UE Power Class 3, 5 and 6, the allowed Maximum Power Reduction (MPR) for the maximum output power specified in Table 6.2.2E-1 due to higher order modulation and transmit bandwidth configuration (resource blocks) is specified in Table 6.2.3E-3, Table 6.2.3E-4 and Table 6.2.3E-6 respectively.

For subPRB allocation of category M1 UE of Power Class 3, there is no MPR applies. For subPRB allocation of category M2 UE of Power Class 3, the allowed MPR due to higher order modulation and transmit bandwidth configuration (subcarrier) is specified in in Table 6.2.3E-7.

Table 6.2.3E-1: Maximum Power Reduction (MPR) for category M1 UE for Power Class 2 and 3

| Modulation | Channel bandwidth / Transmission bandwidth (NRB) |  |  |  |  |  | MPR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |
| QPSK | >2 | >2 | >1 | >4 | - | - | ≤ 1 |
| QPSK | >5 | >5 | - | - | - | - | ≤ 2 |
| 16 QAM | ≤ 2 | ≤ 2 | >1 | >3 | - | - | ≤ 1 |
| 16QAM | >2 | >2 | >3 | >5 | - | - | ≤ 2 |
| NOTE: MPR only applicable for NRB ≥ 1 |  |  |  |  |  |  |  |

Table 6.2.3E-2: Maximum Power Reduction (MPR) for category M1 for Power Class 5

| Modulation | Channel bandwidth / Transmission bandwidth (NRB) |  |  |  |  |  | MPR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |
| QPSK | >2 | >2 | >3 | >5 | - | - | ≤ 1 |
| QPSK | >5 | >5 | - | - | - | - | ≤ 2 |
| 16 QAM | ≤ 2 | ≤ 2 | >3 | >5 | - | - | ≤ 1 |
| 16QAM | >2 | >2 | >5 | - | - | - | ≤ 2 |
| NOTE: MPR only applicable for NRB ≥ 1 |  |  |  |  |  |  |  |

Table 6.2.3E-3: Maximum Power Reduction (MPR) for category M2 UE for Power Class 2 and 3

| Modulation | Channel bandwidth / Transmission bandwidth (NRB) |  |  |  |  |  | MPR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |
| QPSK | >2 | >2 | >1 | >4 | > 16 | > 18 | ≤ 1 |
| QPSK | >5 | >5 | - | - | - | - | ≤ 2 |
| 16 QAM | ≤ 2 | ≤ 2 | >1 | >3 | ≤ 16 | ≤ 18 | ≤ 1 |
| 16QAM | >2 | >2 | >3 | >5 | > 16 | > 18 | ≤ 2 |
| NOTE: MPR only applicable for NRB ≥ 1 |  |  |  |  |  |  |  |

Table 6.2.3E-4: Maximum Power Reduction (MPR) for category M2 UE for Power Class 5

| Modulation | Channel bandwidth / Transmission bandwidth (NRB) |  |  |  |  |  | MPR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |
| QPSK | >2 | >2 | >3 | >5 | > 16 | > 18 | ≤ 1 |
| QPSK | >5 | >5 | - | - | - | - | ≤ 2 |
| 16 QAM | ≤ 2 | ≤ 2 | >3 | >5 | ≤ 16 | ≤ 18 | ≤ 1 |
| 16QAM | >2 | >2 | >5 | - | > 16 | > 18 | ≤ 2 |
| NOTE: MPR only applicable for NRB ≥ 1 |  |  |  |  |  |  |  |

Table 6.2.3E-5: Maximum Power Reduction (MPR) for category M1 for Power Class 6

| Modulation | Channel bandwidth / Transmission bandwidth (NRB) |  |  |  |  |  | MPR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |
| QPSK | >2 | >2 | >3 | >5 | - | - | ≤ 1 |
| QPSK | >5 | >5 | - | - | - | - | ≤ 2 |
| 16 QAM | ≤ 2 | ≤ 2 | >3 | >5 | - | - | ≤ 1 |
| 16QAM | >2 | >2 | >5 | - | - | - | ≤ 2 |

Table 6.2.3E-6: Maximum Power Reduction (MPR) for category M2 UE for Power Class 6

| Modulation | Channel bandwidth / Transmission bandwidth (NRB) |  |  |  |  |  | MPR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |
| QPSK | >2 | >2 | >3 | >5 | > 16 | > 18 | ≤ 1 |
| QPSK | >5 | >5 | - | - | - | - | ≤ 2 |
| 16 QAM | ≤ 2 | ≤ 2 | >3 | >5 | ≤ 16 | ≤ 18 | ≤ 1 |
| 16QAM | >2 | >2 | >5 | - | > 16 | > 18 | ≤ 2 |

Table 6.2.3E-7: Maximum Power Reduction (MPR) for category M2 UE for Power Class 3 for subPRB allocation

| Channel bandwidth | 5 MHz |  | 10 MHz |  |  |  | 15MHz |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Modulation | BPSK | QPSK | BPSK |  | QPSK |  | BPSK |  | QPSK |  |
| WB index for 2 subcarrier transmission | 0 | - | 0 | 1 | - |  | 0 | 2 | - |  |
| MPR | ≤0.5 | - | ≤0.5 | ≤0.5 | - |  | ≤0.5 | ≤0.5 | - |  |
| WB index for 3 subcarrier transmission | - | 0 | - |  | 0 | 1 | - |  | 0 | 2 |
| MPR | - | ≤2 | - |  | ≤2 | ≤2 | - |  | ≤2 | ≤2 |
| WB index for 6 subcarrier transmission | - | 0 | - |  | 0 | 1 | - |  | 0 | 2 |
| MPR | - | ≤1 | - |  | ≤1 | ≤1 | - |  | ≤1 | ≤1 |
| NOTE: WB is specified in TS 36.211 [4] |  |  |  |  |  |  |  |  |  |  |

For PRACH, PUCCH and SRS transmissions, the allowed MPR is according to that specified for PUSCH QPSK modulation for the corresponding transmission bandwidth.

For each subframe, the MPR is evaluated per slot and given by the maximum value taken over the transmission(s) within the slot; the maximum MPR over the two slots is then applied for the entire subframe.

For the UE maximum output power modified by MPR, the power limits specified in subclause 6.2.5 apply.

No other MPR requirement than those specified in tables 6.2.3E-1 and Table 6.2.3E-2 and Table 6.2.3E-5 applies to category M1 and those specified in tables 6.2.3E-3 and Table 6.2.3E-4 and Table 6.2.3E-6 applies to category M2 UE.

### 6.2.3F	UE maximum output power for modulation / channel bandwidth for category NB1 and NB2

For UE category NB1 and NB2 power class 3 and 5 the allowed Maximum Power Reduction (MPR) for the maximum output power given in Table 6.2.2F-1 is specified in Table 6.2.3F-1.

Table 6.2.3F-1: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 3 and 5

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| MPR | ≤ 0.5 dB | 0 dB |  | ≤ 0.5 dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| MPR | ≤ 1 dB |  | ≤ 1 dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| MPR | ≤ 2 dB |  |  |  |

For UE category NB1 and NB2 power class 6 the allowed Maximum Power Reduction (MPR) for the maximum output power given in Table 6.2.2F-1 is specified in Table 6.2.3F-2.

Table 6.2.3F-2: Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 6

| Modulation | QPSK |
| --- | --- |
| MPR for 3 Tones allocation | ≤ 0.5 dB |
| MPR for 6 Tones allocation | ≤ 1 dB |
| MPR for 12 Tones allocation | ≤ 1.5 dB |

For the UE maximum output power modified by MPR, the power limits specified in sub-clause 6.2.5F apply.

### 6.2.3G	UE maximum output power for modulation / channel bandwidth for V2X Communication

When UE is configured for E-UTRA V2X sidelink transmissions non-concurrent with E-UTRA uplink transmissions for E-UTRA V2X operating bands specified in Table 5.5G-1, this subclause specifies the allowed Maximum Power Reduction (MPR) power for V2X physical channels and signals due to PSCCH and PSSCH simultaneous transmission.

#### 6.2.3G.1	MPR for Power class 3 V2X UE

For contiguous allocation of PSCCH and PSSCH simultaneous transmission, the allowed MPR for the maximum output power for V2X physical channels PSCCH and PSSCH shall be as specified in Table 6.2.3G.1-1 for power class 3.

Table 6.2.3G.1-1: Maximum Power Reduction (MPR) for power class 3 V2X Communication (Contiguous PSCCH and PSSCH transmission)

| Modulation | Channel bandwidth / Transmission bandwidth (NRB) |  |  |  |  |  | MPR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |
| QPSK |  |  |  |  |  |  | ≤ 1.5 |
| 16 QAM |  |  |  |  |  |  | ≤ 2 |
| 64QAM |  |  |  |  |  |  | ≤ 3 |

For non-contiguous allocation of PSCCH and PSSCH simultaneous transmission, the allowed MPR for the maximum output power for V2X physical channels PSCCH and PSSCH shall be as specified as follows

MPR = CEIL {MA, 0.5}

Where MA is defined as follows

MA =	4.5		; 0.00< A ≤ 0.2

5.5 –5.833A		; 0.2< A ≤0.6

2.0					; 0.6< A ≤1.00

Where

A = NRB_alloc / NRB.

CEIL{MA, 0.5} means rounding upwards to closest 0.5dB.

The allowed MPR for the maximum output power for V2X physical channels PSBCH and PSSS shall be as specified in subclause 6.2.3 for the corresponding modulation and transmission bandwidth.

The allowed MPR for the maximum output power for V2X physical signal SSSS is specified in Table 6.2.3D-1.

When UE is configured for simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions for inter-band E-UTRA V2X / E-UTRA bands specified in Table 5.5G-2, the allowed MPR requirements in subclause 6.2.3G apply for V2X  PSSCH and PSCCH transmission. The allowed MPR requirements in subclause 6.2.3D apply for other V2X sidelink transmission (PSBCH/PSSS/SSSS). The MPR requirements in subclause 6.2.3 apply for uplink transmission.

For intra-band contiguous multi-carrier operation bandwidth class B the allowed Maximum Power Reduction (MPR) for the maximum output power in Table 6.2.2G.1-2 due to higher order modulation is specified as follows.

Table 6.2.3G.1-2: Void

MPR = CEIL { MA, 0.5}

Where MA is defined as follows for QPSK, 16 QAM and 64 QAM

MA = 	6.5					; 0 ≤ A < 0.1

8 - 15A 			; 0.1 ≤ A < 0.2

5.75 – 3.75A		; 0.2 ≤ A < 0.6

3.5			      ; 0.6≤ A ≤ 1For intra-band contiguous multi-carrier operation bandwidth class C the allowed Maximum Power Reduction (MPR) for the maximum output power can be specified as follows. In case the modulation format is different on different component carriers then the MPR is determined by the rules applied to higher order of those modulations.

MPR = CEIL { MA, 0.5}

Where MA is defined as follows for QPSK, 16 QAM and 64 QAM

[MA = 	6.5					; 0 ≤ A < 0.1

8.5 - 20A 			; 0.1 ≤ A < 0.2

5.25 – 2.5A		; 0.2 ≤ A < 0.6

3.5			      ; 0.6≤ A ≤ 1]

#### 6.2.3G.2	MPR for Power class 2 V2X UE

For contiguous allocation of PSCCH and PSSCH simultaneous transmission, the allowed MPR for the maximum output power for V2X physical channels PSCCH and PSSCH shall be as specified in Table 6.2.3G.2-1 for power class 2.

Table 6.2.3G.2-1: Maximum Power Reduction (MPR) for power class 2 V2X Communication (Contiguous PSCCH and PSSCH transmission)

| Modulation | Channel bandwidth / Transmission bandwidth (NRB) |  |  |  |  |  | MPR (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |
| QPSK |  |  |  |  |  |  | ≤ 2 |
| 16 QAM |  |  |  |  |  |  | ≤ 2.5 |
| 64QAM |  |  |  |  |  |  | ≤ 3 |

For non-contiguous allocation of PSCCH and PSSCH simultaneous transmission, the allowed MPR for the maximum output power for V2X physical channels PSCCH and PSSCH shall be as specified as follows:

MPR = CEIL {MA, 0.5}

Where MA is defined as follows

For 10MHz channel bandwidth         MA =	4.5		         ; 0.0< A ≤ 0.2

8.5 – 20.0A		; 0.2< A ≤0.3

2.5				   ; 0.3< A ≤1.00

For 20MHz channel bandwidth         MA =	9.0		         ; 0.0< A ≤ 0.1

12.0 – 30.0A		; 0.1< A ≤0.3

3.0				   ; 0.3< A ≤1.00

Where

A = NRB_alloc / NRB.

CEIL{MA, 0.5} means rounding upwards to closest 0.5dB.

### 6.2.4	UE maximum output power with additional requirements

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction (A-MPR) is allowed for the output power as specified in Table 6.2.2-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

For UE Power Class 1, 2 and 3 the specific requirements and identified subclauses are specified in Table 6.2.4-1 along with the allowed A-MPR values that may be used to meet these requirements. The allowed A-MPR values specified below in Table 6.2.4.-1 to 6.2.4-15 are in addition to the allowed MPR requirements specified in subclause 6.2.3.

Table 6.2.4-1: Additional Maximum Power Reduction (A-MPR)

| Network Signalling value | Requirements (subclause) | E-UTRA Band | Channel bandwidth (MHz) | Resources Blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 | 6.6.2.1.1 | Table 5.5-1 | 1.4, 3, 5, 10, 15, 20 | Table 5.6-1 | N/A |
| NS_03 | 6.6.2.2.1 | 2, 4,10, 23, 25, 35, 36, 66, 70 | 3 | >5 | ≤ 1 |
|  |  |  | 5 | >6 | ≤ 1 |
|  |  |  | 10 | >6 | ≤ 1 |
|  |  |  | 15 | >8 | ≤ 1 |
|  |  |  | 20 | >10 | ≤ 1 |
| NS_04 | 6.6.2.2.2, 6.6.3.3.19 | 41 | 5, 10, 15, 20 | Table 6.2.4-4, Table 6.2.4-4a |  |
| NS_05 | 6.6.3.3.1 | 1 | 10,15,20 | ≥ 50 (NOTE1) | ≤ 1 (NOTE1) |
|  |  |  | 15, 20 | Table 6.2.4-18 (NOTE2) |  |
|  |  | 65 (NOTE 3) | 10,15,20 | ≥ 50 | ≤ 1 (NOTE 1) |
|  |  |  | 15,20 | Table 6.2.4-18 (NOTE 2) |  |
| NS_06 | 6.6.2.2.3 | 12, 13, 14, 17 | 1.4, 3, 5, 10 | Table 5.6-1 | N/A |
|  |  | 85 | 5, 10 | Table 5.6-1 | N/A |
| NS_07 | 6.6.2.2.3 6.6.3.3.2 | 13 | 10 | Table 6.2.4-2 |  |
| NS_08 | 6.6.3.3.3 | 19 | 10, 15 | > 44 | ≤ 3 |
| NS_09 | 6.6.3.3.4 | 21 | 10, 15 | > 40 | ≤ 1 |
|  |  |  |  | > 55 | ≤ 2 |
|  |  | 74 (NOTE 6) | 10, 15 | > 40 | ≤ 1 |
|  |  |  |  | > 55 | ≤ 2 |
| NS_10 |  | 20 | 15, 20 | Table 6.2.4-3 |  |
| NS_11 | 6.6.2.2.1 6.6.3.3.13 | 23 | 1.4, 3, 5, 10, 15, 20 | Table 6.2.4-5 |  |
| NS_12 | 6.6.3.3.5 | 26 | 1.4, 3, 5, 10, 15 | Table 6.2.4-6 |  |
| NS_13 | 6.6.3.3.6 | 26 | 5 | Table 6.2.4-7 |  |
| NS_14 | 6.6.3.3.7 | 26 | 10, 15 | Table 6.2.4-8 |  |
| NS_15 | 6.6.3.3.8 | 26 | 1.4, 3, 5, 10, 15 | Table 6.2.4-9 Table 6.2.4-10 |  |
| NS_16 | 6.6.3.3.9 | 27 | 3, 5, 10 | Table 6.2.4-11, Table 6.2.4-12, Table 6.2.4-13 |  |
| NS_17 | 6.6.3.3.10 | 28 | 5, 10 | Table 5.6-1 | N/A |
| NS_18 | 6.6.3.3.11 | 28 | 5 | ≥ 2 | ≤ 1 |
|  |  |  | 10, 15, 20 | ≥ 1 | ≤ 4 |
| NS_19 | 6.6.3.3.12 | 44 | 10, 15, 20 | Table 6.2.4-14 |  |
| NS_20 | 6.2.2 6.6.2.2.1 6.6.3.3.14 | 23 | 5, 10, 15, 20 | Table 6.2.4-15 |  |
| NS_21 | 6.6.2.2.1 6.6.3.3.15 | 30 | 5, 10 | Table 6.2.4-16 |  |
| NS_22 | 6.6.3.3.16 | 42(NOTE 8), 43 | 5, 10, 15, 20 | Table 6.2.4-17 |  |
| NS_23 | 6.6.3.3.17 | 42(NOTE 8), 43 | 5, 10, 15, 20 | N/A |  |
| NS_24 | 6.6.3.3.20 | 65 (NOTE 4) | 5, 10, 15, 20 | Table 6.2.4-19 |  |
| NS_25 | 6.6.3.3.21 | 65 (NOTE 4) | 5, 10, 15, 20 | Table 6.2.4-20 |  |
| NS_26 | 6.6.3.3.22 | 68 | 10, 15 | Table 6.2.4-21 |  |
| NS_27 | 6.6.2.2.5, 6.6.3.3.23 | 48 | 5, 10, 15, 20 | Table 6.2.4-22 |  |
| NS_28 | 6.2.2A, 6.6.2.2.6 6.6.3.3.24 | 46 (NOTE 5) | 20 | Table 6.2.4-23 |  |
| NS_29 | 6.2.2A, 6.6.2.3.1a, 6.6.3.3.25 | 46 (NOTE 5) | 20 | Table 6.2.4-24 |  |
| NS_30 | 6.2.2A, 6.6.3.3.26 | 46 (NOTE 5) | 20 | Table 6.2.4-25 |  |
| NS_31 | 6.2.2A, 6.6.3.3.27 | 46 (NOTE 5) | 20 | Table 6.2.4-26 |  |
| NS_32 | - | - | - | - | - |
| … |  |  |  |  |  |
| NS_35 | 6.6.2.2.7 | 71 | 5, 10, 15, 20 | N/A |  |
| NS_36 | 6.6.3.3.28 | 68 | 5, 10, 15 | Table 6.2.4-27 |  |
| NS_38 | 6.6.3.3.29 | 74 | 1.4, 3, 5, 10, 15, 20 | Table 6.2.4-28 |  |
| NS_39 | 6.6.3.3.30 | 74 | 10, 15, 20 | Table 6.2.4-29 |  |
| NS_40 | 6.6.3.3.31 | 51 | 3, 5 | Table 6.2.4-30a, Table 6.2.4-30b |  |
| NS_41 | 6.6.3.3.31 | 50 | 3, 5, 10, 15, 20 | Table 6.2.4-31 |  |
| NS_42 | 6.6.3.3.32 | 50 | 3, 5, 10, 15, 20 | Table 6.2.4-32 |  |
| NS_43 | 6.6.2.2.5 6.6.3.3.23 | 49 | 20 | Table 6.2.4-33 |  |
| NS_44 | 6.6.3.3.33 | 38 (Note 7) | 5, 10, 15, 20 | Table 6.2.4-34 |  |
| NS_45 | 6.6.3.3.34 | 53 | 1.4, 3, 5, 10 | N/A |  |
| .. |  |  |  |  |  |
| NS_56 | 6.6.3.3.35 | 24 | 5, 10 | Table 6.2.4-34a |  |
| NOTE 1: Applicable when the lower edge of the assigned E-UTRA UL channel bandwidth frequency is larger than or equal to the upper edge of PHS band (1915.7 MHz) + 4 MHz + the channel BW assigned, where channel BW is as defined in subclause 5.6. A-MPR for operations below this frequency is not covered in this version of specifications except for the channel assignments in NOTE2 as the emissions requirement in 6.6.3.3.1 may not be met. For 10MHz channel bandwidth whose carrier frequency is larger than or equal to 1945 MHz or 15 MHz channel bandwidth whose carrier frequency is larger than or equal to 1947.5 MHz, no A-MPR applies. NOTE 2: Applicable when carrier frequency is 1932.5 MHz for 15MHz channel bandwidth or 1930 MHz for 20MHz channel bandwidth case. NOTE 3: Applicable when the E-UTRA carrier is within 1920-1980 MHz. NOTE 4: Applicable when the upper edge of the channel bandwidth frequency is greater than 1980MHz. NOTE 5: Applicable only for an LAA Scell configured in Band 46. NOTE 6: Applicable when the E-UTRA carrier is within 1447.9 – 1462.9 MHz NOTE 7: Applicable for power class 2 UE in E-UTRA carrier with channel bandwidth is confined within 2570 MHz and 2615 MHz NOTE 8: Not applicable for power class 2 UE in Band 42 |  |  |  |  |  |

Table 6.2.4-2: A-MPR for "NS_07"

| Parameters | Region A |  | Region B |  | Region C |
| --- | --- | --- | --- | --- | --- |
| RBstart | 0 - 12 |  | 13 – 18 | 19 – 42 | 43 – 49 |
| LCRB [RBs] | 6-8 | 1 to 5 and 9-50 | ≥8 | ≥18 | ≤2 |
| A-MPR [dB] | ≤ 8 | ≤ 12 | ≤ 12 | ≤ 6 | ≤ 3 |
| NOTE 1; RBstart indicates the lowest RB index of transmitted resource blocks NOTE 2; LCRB is the length of a contiguous resource block allocation NOTE 3: For intra-subframe frequency hopping between two regions, notes 1 and 2 apply on a per slot basis. For intra-slot or intra-subslot frequency hopping between two regions, notes 1 and 2 apply on a per Tno_hopping basis. NOTE 4; For intra-subframe frequency hopping between two regions, the larger A-MPR value of the two regions may be applied for both slots in the subframe. For intra-slot frequency hopping between two regions, the larger A-MPR value of the two regions may be applied for the slot. For intra-subslot frequency hopping between two regions, the larger A-MPR value of the two regions may be applied for the subslot. |  |  |  |  |  |

Table 6.2.4-3: A-MPR for "NS_10"

| Channel bandwidth [MHz] | Parameters | Region A |
| --- | --- | --- |
| 15 | RBstart | 0 – 10 |
|  | LCRB [RBs] | 1 -20 |
|  | A-MPR [dB] | ≤ 2 |
| 20 | RBstart | 0 – 15 |
|  | LCRB [RBs] | 1 -20 |
|  | A-MPR [dB] | ≤ 5 |
| NOTE 1: RBstart indicates the lowest RB index of transmitted resource blocks NOTE 2: LCRB is the length of a contiguous resource block allocation NOTE 3: For intra-subframe frequency hopping which intersects Region A, notes 1 and 2 apply on a per slot basis. For intra-slot or intra-subslot frequency hopping which intersects Region A, notes 1 and 2 apply on a Tno_hopping basis. NOTE 4: For intra-subframe frequency hopping which intersect Region A, the larger A-MPR value may be applied for both slots in the subframe. For intra-slot frequency hopping which intersects Region A, the larger A-MPR value may be applied for the slot. For intra-subslot frequency hopping which intersects Region A, the larger A-MPR value may be applied for the subslot. |  |  |

Table 6.2.4-4: A-MPR requirements for "NS_04" for Power Class 3 UE

| Channel bandwidth [MHz] | Parameters |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | Fc [MHz] | ≤ 2500.5 |  |  |  |  |  | > 2500.5 |
|  | RBstart | 0 - 8 |  |  |  | 9 – 24 |  | 0 - 24 |
|  | LCRB [RBs] | > 0 |  |  |  | > 0 |  | > 0 |
|  | A-MPR [dB] | ≤ 2 |  |  |  | 0 |  | 0 |
| 10 | Fc [MHz] | ≤ 2504 |  |  |  |  |  | > 2504 |
|  | RBstart | 0 - 8 |  |  |  | 9 - 35 | 36 - 49 | 0 - 49 |
|  | LCRB [RBs] | ≤ 15 | > 15 and < 25 |  | ≥ 25 | N/A | > 0 | > 0 |
|  | RBstart + LCRB [RBs] | N/A | N/A |  | N/A | ≥ 45 | N/A | N/A |
|  | A-MPR [dB] | ≤ 3 | ≤ 1 |  | ≤ 2 | ≤ 1 | 0 | 0 |
| 15 | Fc [MHz] | ≤ 2510.8 |  |  |  |  |  | > 2510.8 |
|  | RBstart | 0 - 13 |  |  |  | 14 – 59 | 60 – 74 | 0 - 74 |
|  | LCRB [RBs] | ≤ 18 or ≥ 36 |  | > 18 and < 36 |  | N/A | > 0 | > 0 |
|  | RBstart + LCRB [RBs] | N/A |  | N/A |  | ≥ 62 | N/A | N/A |
|  | A-MPR [dB] | ≤ 3 |  | ≤ 1 |  | ≤ 1 | 0 | 0 |
| 20 | Fc [MHz] | ≤ 2517.5 |  |  |  |  |  | > 2517.5 |
|  | RBstart | 0 – 22 |  |  |  | 23 – 76 | 77 – 99 | 0 - 99 |
|  | LCRB [RBs] | ≤ 18 or ≥ 40 |  | > 18 and < 40 |  | N/A | > 0 | > 0 |
|  | RBstart + LCRB [RBs] | N/A |  | N/A |  | ≥ 86 | N/A | N/A |
|  | A-MPR [dB] | ≤ 3 |  | ≤ 1 |  | ≤ 1 | 0 | 0 |
| NOTE 1: RBstart indicates the lowest RB index of transmitted resource blocks NOTE 2: LCRB is the length of a contiguous resource block allocation NOTE 3: For intra-subframe frequency hopping which intersects regions, notes 1 and 2 apply on a per slot basis. For intra-slot or intra-subslot frequency hopping which intersects regions, notes 1 and 2 apply on a per Tno_hopping basis. NOTE 4: For intra-subframe frequency hopping which intersects regions, the larger A-MPR value may be applied for both slots in the subframe. For intra-slot frequency hopping which intersects regions, the larger A-MPR value may be applied for the slot. For intra-subslot frequency hopping which intersects regions, the larger A-MPR value may be applied for the subslot. |  |  |  |  |  |  |  |  |

Table 6.2.4-4a: A-MPR requirements for "NS_04" for Power Class 2 UE

| Channel bandwidth [MHz] | Parameters |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | Fc [MHz] | ≤ 2500.5 |  |  |  |  | > 2500.5 |
|  | RBstart | 0 - 8 |  |  | 9 - 24 |  | 0 - 24 |
|  | LCRB [RBs] | > 0 |  |  | > 0 |  | > 0 |
|  | A-MPR [dB] | ≤ 3 |  |  | 0 |  | 0 |
| 10 | Fc [MHz] | ≤ 2504 |  |  |  |  | > 2504 |
|  | RBstart | 0 – 8 |  |  | 9 - 35 | 36 - 49 | 0 - 49 |
|  | LCRB [RBs] | ≤ 15 | > 15 and < 25 | ≥ 25 | N/A | > 0 | > 0 |
|  | RBstart + LCRB [RBs] | N/A | N/A | N/A | ≥ 45 | N/A | N/A |
|  | A-MPR [dB] | ≤ 5 | ≤ 2 | ≤ 3 | ≤ 1 | 0 | 0 |
| 15 | Fc [MHz] | ≤ 2510.8 |  |  |  |  | > 2510.8 |
|  | RBstart | 0 – 13 |  |  | 14 – 59 | 60 – 74 | 0 - 74 |
|  | LCRB [RBs] | ≤ 18 | > 18 and < 36 | ≥ 36 | N/A | > 0 | > 0 |
|  | RBstart + LCRB [RBs] | N/A | N/A | N/A | ≥ 62 | N/A | N/A |
|  | A-MPR [dB] | ≤ 5 | ≤ 2 | ≤ 4 | ≤ 3 | 0 | 0 |
| 20 | Fc [MHz] | ≤ 2517.5 |  |  |  |  | > 2517.5 |
|  | RBstart | 0 – 22 |  |  | 23 – 76 | 77 – 99 | 0 - 99 |
|  | LCRB [RBs] | ≤ 18 | > 18 and < 40 | ≥ 40 | N/A | > 0 | > 0 |
|  | RBstart + LCRB [RBs] | N/A | N/A | N/A | ≥ 86 | N/A | N/A |
|  | A-MPR [dB] | ≤ 5 | ≤ 2 | ≤ 4 | ≤ 3 | 0 | 0 |
| NOTE 1: RBstart indicates the lowest RB index of transmitted resource blocks NOTE 2: LCRB is the length of a contiguous resource block allocation NOTE 3: For intra-subframe frequency hopping which intersects regions, notes 1 and 2 apply on a per slot basis. For intra-slot or intra-subslot frequency hopping which intersects regions, notes 1 and 2 apply on a per Tno_hopping basis. NOTE 4: For intra-subframe frequency hopping which intersects regions, the larger A-MPR value may be applied for both slots in the subframe. For intra-slot frequency hopping which intersects regions, the larger A-MPR value may be applied for the slot. For intra-subslot frequency hopping which intersects regions, the larger A-MPR value may be applied for the subslot. |  |  |  |  |  |  |  |

For a power class 2 capable UE operating in Band 41, A-MPR according to Table 6.2.4-4 for power class 3 is allowed when an IE P-max as defined in [7] of 23 dBm or lower is indicated in the cell or if the uplink/downlink configuration is 0 or 6.

Table 6.2.4-5: A-MPR for "NS_11"

| Channel Bandwidth [MHz] | Parameters |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3 | Fc [MHz] | <2004 |  |  | ≥2004 |  |  |  |  |  |  |  |  |  |  |
|  | LCRB [RBs] | 1-15 |  |  | >5 |  |  |  |  |  |  |  |  |  |  |
|  | A-MPR [dB] | ≤5 |  |  | ≤ 1 |  |  |  |  |  |  |  |  |  |  |
| 5 | Fc [MHz] | <2004 |  |  | 2004 ≤ Fc <2007 |  |  |  |  |  |  | ≥2007 |  |  |  |
|  | LCRB [RBs] | 1-25 |  |  | 1-6 & 15-25 |  |  |  | 8-12 |  |  | >6 |  |  |  |
|  | A-MPR [dB] | ≤7 |  |  | ≤ 4 |  |  |  | 0 |  |  | ≤ 1 |  |  |  |
| 10 | Fc [MHz] | 2005 ≤ Fc <2015 |  |  |  |  |  |  | 2015 |  |  |  |  |  |  |
|  | RBstart | 0-49 |  |  |  |  |  |  | 0-49 |  |  |  |  |  |  |
|  | LCRB [RBs] | 1-50 |  |  |  |  |  |  | 1-50 |  |  |  |  |  |  |
|  | A-MPR [dB] | ≤ 12 |  |  |  |  |  |  | 0 |  |  |  |  |  |  |
| 15 | Fc [MHz] | <2012.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RBstart | 0-4 | 5-21 |  |  |  |  |  |  |  | 22-56 |  |  |  | 57-74 |
|  | LCRB [RBs] | ≥1 | 7-50 |  |  | 0-6 & ≥50 |  |  |  |  | ≤25 |  | >25 |  | >0 |
|  | A-MPR [dB] | ≤15 | ≤7 |  |  | ≤10 |  |  |  |  | 0 |  | ≤6 |  | ≤15 |
|  | Fc [MHz] | 2012.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RBstart | 0-12 |  | 13-39 |  |  |  |  |  | 40-65 |  |  |  | 66-74 |  |
|  | LCRB [RBs] | ≥1 |  | ≥30 |  |  |  | <30 |  | ≥ (69 – RBstart) |  |  |  | ≥1 |  |
|  | A-MPR [dB] | ≤10 |  | ≤6 |  |  |  | 0 |  | ≤2 |  |  |  | ≤6.5 |  |
| 20 | Fc [MHz] | 2010 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RBstart | 0-12 | 13-29 |  |  |  |  |  |  | 30-68 |  |  |  |  | 69-99 |
|  | LCRB [RBs] | ≥1 | 10-60 |  |  |  | 1-9 & >60 |  |  | 1-24 |  |  | ≥25 |  | ≥1 |
|  | A-MPR [dB] | ≤15 | ≤7 |  |  |  | ≤10 |  |  | 0 |  |  | ≤7 |  | ≤15 |

Table 6.2.4-6: A-MPR for "NS_12"

| Channel bandwidth [MHz] | Parameters | Region A |  | Region B |
| --- | --- | --- | --- | --- |
| 1.4 | RBstart | 0 |  | 1-2 |
|  | LCRB [RBs] | ≤3 | ≥4 | ≥4 |
|  | A-MPR [dB] | ≤3 | ≤6 | ≤3 |
| 3 | RBstart | 0-3 |  | 4-5 |
|  | LCRB [RBs] | 1-15 |  | ≥9 |
|  | A-MPR [dB] | ≤4 |  | ≤3 |
| 5 | RBstart | 0-6 |  | 0-9 |
|  | LCRB [RBs] | ≤8 |  | ≥9 |
|  | A-MPR [dB] | ≤5 |  | ≤3 |
| 10 | RBstart | 0-15 |  | 0-22 |
|  | LCRB [RBs] | ≤18 |  | ≥20 |
|  | A-MPR [dB] | ≤4 |  | ≤2 |
| 15 | RBstart | 0-30 |  | 0-30 |
|  | LCRB [RBs] | ≤30 |  | ≥32 |
|  | A-MPR [dB] | ≤4 |  | ≤3 |

Table 6.2.4-7: A-MPR for "NS_13"

| Channel bandwidth [MHz] | Parameters | Region A |  |
| --- | --- | --- | --- |
| 5 | RBstart | 0-2 |  |
|  | LCRB [RBs] | ≤5 | ≥18 |
|  | A-MPR [dB] | ≤3 | ≤2 |

Table 6.2.4-8: A-MPR for "NS_14"

| Channel bandwidth [MHz] | Parameters | Region A |  |
| --- | --- | --- | --- |
| 10 | RBstart | 0 |  |
|  | LCRB [RBs] | ≤5 | =50 |
|  | A-MPR [dB] | ≤3 | ≤1 |
| 15 | RBstart | ≤8 |  |
|  | LCRB [RBs] | ≤16 | ≥50 |
|  | A-MPR [dB] | ≤3 | ≤1 |

Table 6.2.4-9: A-MPR for "NS_15" for E-UTRA highest channel edge > 845 MHz and ≤ 849 MHz

| Channel bandwidth [MHz] | Parameters | Region A | Region B | Region C |
| --- | --- | --- | --- | --- |
| 1.4 | RBend [RB] |  |  | 4-5 |
|  | A-MPR [dB] |  |  | ≤3 |
| 3 | RBend [RB] | 0-1 | 8-12 | 13-14 |
|  | LCRB [RB] | ≤2 | ≥8 | >0 |
|  | A-MPR [dB] | ≤4 | ≤4 | ≤9 |
| 5 | RBend [RB] | 0-4 | 12-19 | 20-24 |
|  | LCRB [RB] | ≤2 | ≥8 | >0 |
|  | A-MPR [dB] | ≤4 | ≤5 | ≤9 |
| 10 | RBend [RB] | 0-12 | 23-36 | 37-49 |
|  | LCRB [RB] | ≤2 | ≥15 | >0 |
|  | A-MPR [dB] | ≤4 | ≤6 | ≤9 |
| 15 | RBend [RB] | 0-20 | 26-53 | 54-74 |
|  | LCRB [RB] | ≤2 | ≥20 | >0 |
|  | A-MPR [dB] | ≤4 | ≤5 | ≤9 |

Table 6.2.4-10: A-MPR for "NS_15" for E-UTRA highest channel edge ≤ 845 MHz

| Channel bandwidth [MHz] | Parameters | Region A | Region B | Region C |
| --- | --- | --- | --- | --- |
| 5 | RBend [RB] |  |  | 19-24 |
|  | LCRB [RB] |  |  | ≥18 |
|  | A-MPR [dB] |  |  | ≤2 |
| 10 | RBend [RB] | 0-4 | 29-44 | 45-49 |
|  | LCRB [RB] | ≤2 | ≥24 | >0 |
|  | A-MPR [dB] | ≤4 | ≤4 | ≤9 |
| 15 | RBend [RB] | 0-12 | 44-61 | 62-74 |
|  | LCRB [RB] | ≤2 | ≥20 | >0 |
|  | A-MPR [dB] | ≤4 | ≤5 | ≤9 |

Table 6.2.4-11: A-MPR for "NS_16" with channel lower edge at ≥807 MHz and <808.5 MHz

| Channel bandwidth [MHz] | Parameter | Region A | Region B | Region C | Region D | Region E |
| --- | --- | --- | --- | --- | --- | --- |
| 3 MHz | RBstart | 0 | 1-2 |  |  |  |
|  | LCRB [RBs] | ≥12 | 12 |  |  |  |
|  | A-MPR [dB] | ≤2 | ≤1 |  |  |  |
| 5 MHz | RBstart | 0-1 | 2 | 2-9 | 2-5 |  |
|  | LCRB [RBs] | 1 - 25 | 12 | 15-18 | 20 |  |
|  | A-MPR [dB] | ≤5 | ≤1 | ≤2 | ≤3 |  |
| 10 MHz | RBstart | 0 - 8 | 0-14 |  | 15-20 | 15-24 |
|  | LCRB [RBs] | 1 - 12 | 15-20 | ≥24 | ≥30 | 24-27 |
|  | A-MPR [dB] | ≤5 | ≤3 | ≤7 | ≤3 | ≤1 |

Table 6.2.4-12: A-MPR for "NS_16" with channel lower edge at ≥808.5 MHz and <812 MHz

| Channel bandwidth [MHz] | Parameter | Region A | Region B | Region C | Region D | Region E |
| --- | --- | --- | --- | --- | --- | --- |
| 5 MHz | RBstart | 0 | 0-1 | 1-5 |  |  |
|  | LCRB [RBs] | 16-20 | ≥24 | 16-20 |  |  |
|  | A-MPR [dB] | ≤2 | ≤3 | ≤1 |  |  |
| 10 MHz | RBstart | 0-6 |  | 0-10 | 0-14 | 11-20 |
|  | LCRB [RBs] | 1-12 | 15-20 | 24-32 | ≥36 | 24-32 |
|  | A-MPR [dB] | ≤5 | ≤2 | ≤4 | ≤5 | ≤1 |

Table 6.2.4-13: A-MPR for "NS_16" with channel lower edge at ≥812 MHz

| Channel bandwidth [MHz] | Parameter | Region A | Region B | Region C | Region D |
| --- | --- | --- | --- | --- | --- |
| 10 MHz | RBstart | 0 - 9 | 0 | 1-14 | 0-5 |
|  | LCRB [RBs] | 27-32 | 36-40 | 36-40 | ≥45 |
|  | A-MPR [dB] | ≤1 | ≤2 | ≤1 | ≤3 |

Table 6.2.4-14: A-MPR for "NS_19"

| Channel bandwidth [MHz] | Parameters | Region A |  | Region B |
| --- | --- | --- | --- | --- |
| 10 | RBstart |  |  | 0-6 |
|  | LCRB [RBs] |  |  | ≥40 |
|  | A-MPR [dB] |  |  | ≤1 |
| 15 | RBstart | 0-6 |  | 7-20 |
|  | LCRB [RBs] | ≤18 | ≥36 | ≥42 |
|  | A-MPR [dB] | ≤2 | ≤3 | ≤2 |
| 20 | RBstart | 0-14 |  | 15-30 |
|  | LCRB [RBs] | ≤40 | ≥45 | ≥50 |
|  | A-MPR [dB] | ≤2 | ≤3 | ≤2 |

Table 6.2.4-15: A-MPR for "NS_20"

| Channel Bandwidth [MHz] | Parameters |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | Fc [MHz] | < 2007.5 |  |  | 2007.5 ≤ Fc < 2012.5 |  |  |  |  |  |  |  |  |  |  |  | 2012.5 ≤ Fc ≤ 2017.5 |  |  |  |
|  | RBstart | ≤24 |  |  | 0-3 |  |  |  |  |  |  |  | 4-6 |  |  |  | ≤24 |  |  |  |
|  | LCRB [RBs] | >0 |  |  | 15-19 |  |  |  | ≥20 |  |  |  | ≥18 |  |  |  | 1-25 |  |  |  |
|  | A-MPR [dB] | ≤17 |  |  | ≤1 |  |  |  | ≤4 |  |  |  | ≤2 |  |  |  | ≤ 0 |  |  |  |
| 10 | Fc [MHz] | 2005 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RBstart | 0-25 |  |  |  |  | 26-34 |  |  |  |  |  |  |  |  | 35-49 |  |  |  |  |
|  | LCRB [RBs] | >0 |  |  |  |  | 8-15 |  |  |  |  | >15 |  |  |  | >0 |  |  |  |  |
|  | A-MPR [dB] | ≤16 |  |  |  |  | ≤2 |  |  |  |  | ≤5 |  |  |  | ≤ 6 |  |  |  |  |
|  | Fc [MHz] | 2015 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RBstart | 0-5 |  |  |  |  |  |  |  |  | 6-10 |  |  |  |  |  |  |  |  |  |
|  | LCRB [RBs] | ≥32 |  |  |  |  |  |  |  |  | ≥40 |  |  |  |  |  |  |  |  |  |
|  | A-MPR [dB] | ≤4 |  |  |  |  |  |  |  |  | ≤2 |  |  |  |  |  |  |  |  |  |
| 15 | Fc [MHz] | 2012.5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RBstart | 0-14 |  |  |  |  |  | 15-24 |  |  |  |  |  |  |  |  | 25-39 |  | 61-74 |  |
|  | LCRB [RBs] | 1-9 & 40-75 |  | 10-39 |  |  |  | 24-29 |  |  |  |  |  | ≥30 |  |  | ≥36 |  | ≤6 |  |
|  | A-MPR [dB] | ≤11 |  | ≤6 |  |  |  | ≤1 |  |  |  |  |  | ≤7 |  |  | ≤5 |  | ≤6 |  |
| 20 | Fc [MHz] | 2010 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RBstart | 0-21 | 22-31 |  |  |  |  |  |  | 32-38 |  |  |  |  | 39-49 |  |  | 50-68 |  | 69-99 |
|  | LCRB [RBs] | >0 | 1-9 & 31-75 |  |  | 10-30 |  |  |  | ≥15 |  |  |  |  | ≥24 |  |  | ≥25 |  | >0 |
|  | A-MPR [dB] | ≤17 | ≤12 |  |  | ≤6 |  |  |  | ≤9 |  |  |  |  | ≤7 |  |  | ≤5 |  | ≤16 |
| NOTE 1: When NS_20 is signaled the minimum requirements for the 10 MHz bandwidth are specified for E-UTRA UL carrier center frequencies of 2005 MHz or 2015 MHz. NOTE 2: When NS_20 is signaled the minimum requirements for the 15 MHz channel bandwidth are specified for E-UTRA UL carrier center frequency of 2012.5 MHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4-16: A-MPR for "NS_21"

| Channel Bandwidth [MHz] | Parameters | Region A |  | Region B |  |
| --- | --- | --- | --- | --- | --- |
| 10 | RBstart | 0 – 6 | 0 – 6 | N/A | N/A |
|  | RBend | N/A | N/A | 43 – 49 | 43 – 49 |
|  | LCRB [RBs] | 1 – 2 | 3 – 12, 32 - 50 | 1 – 2 | 3 – 12, 32 - 50 |
|  | A-MPR [dB] | ≤ 4 | ≤ 3 | ≤ 4 | ≤ 3 |

Table 6.2.4-17: A-MPR for "NS_22"

| Channel bandwidth [MHz] | Parameters | Region A | Region B | Region C | Region D |
| --- | --- | --- | --- | --- | --- |
| 5 | No A-MPR is needed for 5 MHz channel bandwidth |  |  |  |  |
| 10 | RBstart | 0-13 | 0-17 | ≤ 6 | ≥12 |
|  | LCRB [RBs] | > 36 | 33-36 | ≤ 32 | ≤ 32 |
|  | RBstart + LCRB [RBs] | N/A | N/A | N/A | ≥44 |
|  | A-MPR [dB] | ≤ 4 | ≤ 3 | ≤ 3 | ≤ 3 |
| 15 | RBstart | 0-24 | 0-38 | ≤ 14 | ≥ 23 |
|  | LCRB [RBs] | > 50 | 37-50 | ≤ 36 | ≤ 36 |
|  | RBstart + LCRB [RBs] | N/A | N/A | N/A | ≥59 |
|  | A-MPR [dB] | ≤ 5 | ≤ 4 | ≤ 3 | ≤ 3 |
| 20 | RBstart | 0-35 | 0-51 | ≤ 21 | ≥ 31 |
|  | LCRB [RBs] | > 64 | 49-64 | ≤ 48 | ≤ 48 |
|  | RBstart + LCRB [RBs] | N/A | N/A | N/A | ≥79 |
|  | A-MPR [dB] | ≤ 5 | ≤ 4 | ≤ 3 | ≤ 3 |
| NOTE 1; RBstart indicates the lowest RB index of transmitted resource blocks NOTE 2; LCRB is the length of a contiguous resource block allocation NOTE 3: For intra-subframe frequency hopping between two regions, notes 1 and 2 apply on a per slot basis. For intra-slot or intra-subslot frequency hopping between two regions, notes 1 and 2 apply on a per Tno_hopping basis. NOTE 4; For intra-subframe frequency hopping between two regions, the larger A-MPR value of the two regions may be applied for both slots in the subframe. For intra-slot frequency hopping between two regions, the larger A-MPR value may be applied for the slot. For intra-subslot frequency hopping between two regions, the larger A-MPR value may be applied for the subslot. |  |  |  |  |  |

Table 6.2.4-18: A-MPR for "NS_05"

| Channel Bandwidth [MHz] | Parameters |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 15 | Fc [MHz] | 1932.5 |  |  |  |  |  |  |  |  |  |  |
|  | RBstart | 0-7 | 8 – 66 |  |  |  |  |  | 67-74 |  |  |  |
|  | LCRB [RBs] | ≥1 | ≤30 |  | 31 – 54 |  | >54 |  | ≤6 |  | >6 |  |
|  | A-MPR [dB] | ≤11 | 0 |  | ≤3 |  | ≤5 |  | ≤5 |  | ≤1 |  |
| 20 | Fc [MHz] | 1930 |  |  |  |  |  |  |  |  |  |  |
|  | RBstart | 0-23 | 24-75 |  |  |  |  |  |  | 76-99 |  |  |
|  | LCRB [RBs] | ≥1 | ≤24 | 25 – 40 |  | 41 – 50 |  | > 50 |  | ≤6 |  | >6 |
|  | A-MPR [dB] | ≤11 | 0 | ≤3 |  | ≤5 |  | ≤10 |  | ≤5 |  | ≤1 |

Table 6.2.4-18E: A-MPR requirements for "NS_05" for Cat-M2 power class 3 UE

| BW [MHz] | 20 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Fc [MHz] | 1930 |  |  |  |  |
| (NBindex,RBstart) | (0,<6) | (0,<6) | (1,<6) | (2,<6) | (2,<6) |
| LCRB | 9, 12 | ≥15 | ≥15 | 18 | 24 |
| A-MPR [dB] | ≤1 | ≤4 | ≤3 | ≤1 | ≤2 |
| NOTE 1: NBindex is the narrowband index that is defined in 6.2.7 in [4]. The resource block assignment is defined within the narrowband as defined in 5.3.3.1.12 and 5.3.3.1.13 in [5]. |  |  |  |  |  |

Table 6.2.4-19: A-MPR for "NS_24"

| Channel Bandwidth [MHz] | Parameters |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | Fc [MHz] | Fc > [1987.5] |  |  |  |  |  |  |  |  |  |
|  | RBstart | 0 - 24 |  |  |  |  |  |  |  |  |  |
|  | LCRB [RBs] | 0 - 24 |  |  |  |  |  |  |  |  |  |
|  | A-MPR [dB] | ≤ 10 |  |  |  |  |  |  |  |  |  |
| 10 | Fc [MHz] | 1975 < Fc ≤ 1985 |  |  |  |  |  |  | 1985<Fc≤1995 |  | Fc>1995 |
|  | RBstart | 0 - 1 | 2 - 14 |  | 15 - 26 | 36 - 49 |  |  | 0 - 49 |  | 0 - 49 |
|  | LCRB [RBs] | > 10 | ≥ 35 |  | N/A | ≤ 2 |  | > 11 | 0 - 49 |  | 0 - 49 |
|  | RBend | N/A | N/A |  | > 48 | N/A |  | N/A | N/A |  | N/A |
|  | A-MPR [dB] | ≤ 2 | ≤ 2 |  | 1 | ≤ 3 |  | ≤ 1 | ≤ 9 |  | ≤ 17 |
| 15 | Fc [MHz] | 1972.5 < Fc ≤ 1987.5 |  |  |  |  |  |  |  | Fc > 1987.5 |  |
|  | RBstart | 0 - 11 |  |  |  |  | 12 - 74 |  |  | 0 - 74 |  |
|  | LCRB [RBs] | ≤ 45 |  | > 45 |  |  | > 3 |  |  | 0 - 74 |  |
|  | RBend | N/A |  | N/A |  |  | ≥ 45 |  |  | N/A |  |
|  | A-MPR [dB] | ≤ 2 |  | ≤ 8 |  |  | ≤ 7 |  |  | ≤ 17 |  |
| 20 | Fc [MHz] | Fc > 1970 |  |  |  |  |  |  |  |  |  |
|  | RBstart | 0 - 99 |  |  |  |  |  |  |  |  |  |
|  | LCRB [RBs] | 0 - 99 |  |  |  |  |  |  |  |  |  |
|  | A-MPR [dB] | ≤ 17 |  |  |  |  |  |  |  |  |  |

Table 6.2.4-20: A-MPR for "NS_25"

| Channel Bandwidth [MHz] |  | Parameters |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | Fc [MHz] |  | Fc > [1997.5] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | RBstart |  | 0 - 9 |  |  |  |  |  |  |  | 10 - 24 |  |  |  |  |  |  |  |
|  | LCRB [RBs] |  | > 12 |  |  |  |  |  |  |  | N/A |  |  |  |  |  |  |  |
|  | RBend |  | N/A |  |  |  |  |  |  |  | ≥ 22 |  |  |  |  |  |  |  |
|  | A-MPR [dB] |  | ≤ 5 |  |  |  |  |  |  |  | ≤ 2 |  |  |  |  |  |  |  |
| 10 | Fc [MHz] |  | 1975 < Fc ≤ 1985 |  |  | 1985 < Fc ≤ 1995 |  |  |  |  |  |  |  | Fc > 1995 |  |  |  |  |
|  | RBstart |  | 0-1 | 2-49 |  | 0 |  |  |  | 1 - 18 |  | 19-49 |  | 0-6 |  | 7-15 |  | 16-49 |
|  | LCRB [RBs] |  | > 10 | N/A |  | ≤ 25 |  | > 25 |  | > 25 |  | N/A |  | N/A |  | > 20 |  | N/A |
|  | RBend |  | N/A | > 48 |  | N/A |  | N/A |  | N/A |  | > 42 |  | N/A |  | N/A |  | > 35 |
|  | A-MPR [dB] |  | ≤ 1 | ≤ 1 |  | ≤ 1 |  | ≤ 5 |  | ≤ 5 |  | ≤ 1 |  | ≤ 10 |  | ≤ 7 |  | ≤ 11 |
| 15 | Fc [MHz] |  | 1972.5 < Fc ≤ 1987.5 |  |  |  |  |  |  |  |  |  |  |  |  |  | Fc > 1987.5 |  |
|  | RBstart |  | 0 - 4 |  | 5 - 30 |  |  |  | 31 - 62 |  |  |  | 63 - 74 |  |  |  | 0 - 74 |  |
|  | LCRB [RBs] |  | ≥ 15 |  | ≥ 45 |  |  |  | N/A |  |  |  | N/A |  |  |  | 0 - 74 |  |
|  | RBend |  | N/A |  | N/A |  |  |  | > 71 |  |  |  | N/A |  |  |  | N/A |  |
|  | A-MPR [dB] |  | ≤ 4 |  | ≤ 3 |  |  |  | ≤ 1 |  |  |  | ≤ 1 |  |  |  | ≤ 13 |  |
| 20 | Fc [MHz] |  | 1970 < Fc ≤ 1990 |  |  |  |  |  |  |  |  |  |  |  | Fc > 1990 |  |  |  |
|  | RBstart |  | 0 - 13 |  |  |  | 14 - 40 |  |  |  | 41 - 99 |  |  |  | 0 - 99 |  |  |  |
|  | LCRB [RBs] |  | N/A |  |  |  | ≥ 32 |  |  |  | N/A |  |  |  | 0 - 99 |  |  |  |
|  | RBend |  | N/A |  |  |  | N/A |  |  |  | > 72 |  |  |  | N/A |  |  |  |
|  | A-MPR [dB] |  | ≤ 11 |  |  |  | ≤ 11 |  |  |  | ≤13 |  |  |  | ≤ 13 |  |  |  |

Table 6.2.4-21: A-MPR for "NS_26"

| Bandwidth (MHz) | RBstart | L_crb | A-MPR |
| --- | --- | --- | --- |
| 10 | 0 - 10 | ≥ 1 | ≤ 1 |
| 15 | 0 - 17 | ≥ 1 | ≤ 1 |

Table 6.2.4-22: A-MPR for "NS_27"

| Parameters |  |  |  |  |
| --- | --- | --- | --- | --- |
| Channel bandwidth [MHz] | RBstart | RBend | LCRB | A-MPR |
| 15 | 0 – 6 |  | ≤ 15 | ≤ 4 dB |
|  |  | 68 – 74 |  |  |
|  | ≥ 0 |  | ≥ 60 | ≤ 2 dB |
| 20 | 0 – 12 |  | ≤ 20 | ≤ 4 dB |
|  |  | 87 – 99 |  |  |
|  | 13 – 15 |  | ≤ 3 | ≤ 1 dB |
|  |  | 84 – 86 |  |  |
|  | ≥ 0 |  | ≥ 60 | ≤ 3 dB |

Table 6.2.4-23: A-MPR for "NS_28"

| Parameters |  |  |  |
| --- | --- | --- | --- |
| Channel Bandwidth [MHz] | Carrier centre frequency (FC) [MHz] | Uplink resource allocation | A-MPR [dB] |
| 20 | 5160 ≤ FC < 5179.8 5320.2 < FC ≤ 5340 5480 ≤ FC < 5499.8 | L = 10 (RIV = 19) | 9 |
|  |  | Any other RIV | 8 |
|  | 5680.2 < FC ≤ 5715 | L = 10 (RIV = 19) | 9 |
|  |  | Any other RIV | 8 |
|  | 5179.8 ≤ FC ≤ 5320.2 5499.8 ≤ FC ≤ 5680.2 | L = 10 (RIV = 19) | 2 |
|  |  | Any other RIV | 2 |
| NOTE 1: The carrier centre frequencies and corresponding EARFCN allowed for operation in Band 46 are specified in [12]. The uplink resource allocation is defined in Clause 8.1.4 of [6]. |  |  |  |

Table 6.2.4-24: A-MPR for "NS_29"

| Parameters |  |  |  |
| --- | --- | --- | --- |
| Channel Bandwidth [MHz] | Carrier centre frequency (FC) [MHz] | Uplink resource allocation | A-MPR [dB] |
| 20 | 5179.8 ≤ FC ≤ 5320.2 5499.8 ≤ FC ≤ 5700.2 | L = 10 (RIV = 19) | 2 |
|  |  | Any other RIV | 1 |
| NOTE 1: The carrier centre frequencies and corresponding EARFCN allowed for operation in Band 46 are specified in [12]. The uplink resource allocation is defined in Clause 8.1.4 of [6]. |  |  |  |

Table 6.2.4-25: A-MPR for "NS_30"

| Parameters |  |  |  |
| --- | --- | --- | --- |
| Channel Bandwidth [MHz] | Carrier centre frequency (FC) [MHz] | Uplink resource allocation | A-MPR [dB] |
| 20 | 5160 ≤ FC < 5179.8 5320.2 < FC ≤ 5340 | L = 10 (RIV = 19) | 15 |
|  |  | Any other RIV | 16 |
|  | 5179.8 ≤ FC < 5199.8 5300.2 < FC ≤ 5320.2 | L = 10 (RIV = 19) | 3 |
|  |  | Any other RIV | 3 |
|  | 5480 ≤ FC < 5499.8 | L = 10 (RIV = 19) | 11 |
|  |  | Any other RIV | 11 |
|  | 5680.2 < FC ≤ 5715 | L = 10 (RIV = 19) | 6 |
|  |  | Any other RIV | 6 |
|  | 5199.8 ≤ FC ≤ 5300.2 5499.8 ≤ FC ≤ 5680.2 | L = 10 (RIV = 19) | 1 |
|  |  | Any other RIV | N/A |
| NOTE 1: The carrier centre frequencies and corresponding EARFCN allowed for operation in Band 46 are specified in [12]. The uplink resource allocation is defined in Clause 8.1.4 of [6]. The uplink resource allocation is defined in Clause 8.1.4 of [6]. |  |  |  |

Table 6.2.4-26: A-MPR for "NS_31"

| Channel Bandwidth [MHz] | Carrier centre frequency (FC) [MHz] | Uplink resource allocation | A-MPR [dB] |
| --- | --- | --- | --- |
| 20 | 5239.8 ≤ FC ≤ 5240.2 | L = 10 (RIV = 19) | 7 |
|  |  | Any other RIV | 7 |
|  | 5160 ≤ FC < 5179.8 5260 ≤ FC < 5279.8 5320.2 < FC ≤ 5340 5480 ≤ FC < 5499.8 | L = 10 (RIV = 19) | 6 |
|  |  | Any other RIV | 6 |
|  | 5680.2 < FC ≤ 5715 5805.2 < FC ≤ 5840 | L = 10 (RIV = 19) | 6 |
|  |  | Any other RIV | 6 |
|  | 5735 ≤ FC < 5764.8 | L = 10 (RIV = 19) | 6 |
|  |  | Any other RIV | 6 |
|  | 5179.8 ≤ FC ≤ 5220.2 5279.8 ≤ FC ≤ 5320.2 5499.8 ≤ FC ≤ 5680.2 5764.8 ≤ FC ≤ 5805.2 | L = 10 (RIV = 19) | 2 |
|  |  | Any other RIV | 1 |
| NOTE 1: The carrier centre frequencies and corresponding EARFCN allowed for operation in Band 46 are specified in [12]. The uplink resource allocation is defined in Clause 8.1.4 of [6]. |  |  |  |

Table 6.2.4-27: A-MPR for “NS_36”

| Channel Bandwidth [MHz] | Parameters |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 5 MHz | Fc [MHz] | 700.5 ≤ Fc < 705.5 |  |  |  |
|  | RBstart | 0 |  | 1-4 |  |
|  | LCRB [RBs] | 1 | ≥ 15 | 15 - 23 | 24 |
|  | A-MPR [dB] | ≤ 2 | ≤ 4 | ≤ 1 | ≤ 3 |
| 10 MHz | Fc [MHz] | 703 ≤ Fc < 708 |  |  |  |
|  | RBstart | 0 - 8 | 9 - 12 | 13 - 39 | 40 - 43 |
|  | LCRB [RBs] | > 0 | > 12 | > 16 | > 0 |
|  | A-MPR [dB] | ≤ 10 | ≤ 8 | ≤ 6 | ≤ 6 |
| 15 MHz | Fc [MHz] | Fc = 705.5 |  |  |  |
|  | RBstart | 0 - 16 | 17 - 23 | 24 - 56 | 57 - 60 |
|  | LCRB [RBs] | > 0 | > 12 | > 20 | < 6 |
|  | A-MPR [dB] | ≤ 10 | ≤ 9 | ≤ 7 | ≤ 6 |
| NOTE 1: RBstart indicates the lowest RB index of transmitted resource blocks NOTE 2: LCRB is the length of a contiguous resource block allocation NOTE 3: For intra-subframe frequency hopping between two regions, notes 1 and 2 apply on a per slot basis. For intra-slot or intra-subslot frequency hopping between two regions, notes 1 and 2 apply on a per Tno_hopping basis. NOTE 4: For intra-subframe frequency hopping between two regions, the larger A-MPR value of the two regions may be applied for both slots in the subframe. For intra-slot frequency hopping between two regions, the larger A-MPR value may be applied for the slot. For intra-subslot frequency hopping between two regions, the larger A-MPR value may be applied for the subslot. |  |  |  |  |  |

Table 6.2.4-28: A-MPR for "NS_38"

| Channel Bandwidth [MHz] | Carrier centre frequency (FC) [MHz] | Parameters | Region A |  | Region B |  | Region C |  | Region D |  | Region E |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.4 | 1427.7 ≤ FC < 1429.1 | A-MPR [dB] | ≤ [16] |  |  |  |  |  |  |  |  |  |
| 3 | 1428.5 ≤ FC < 1431.5 | RBstart | 0 |  | 1– 2 |  | 3 – 6 |  |  |  |  |  |
|  |  | LCRB [RBs] | 1 | ≥2 | 2 | ≥ 3 | 5-7 | ≥8 |  |  |  |  |
|  |  | A-MPR [dB] | ≤ 11 | ≤ 16 | ≤ 9 | ≤ 15 | ≤ 12 | ≤ 14 |  |  |  |  |
| 5 | 1429.5 ≤ FC < 1434.5 | RBstart | 0 - 1 |  | 2 – 4 |  | 5 – 7 |  | 8 – 10 |  |  |  |
|  |  | LCRB [RBs] | 2 | ≥3 | 4-6 | ≥ 7 | 8-10 | ≥11 | 12-14 | ≥15 |  |  |
|  |  | A-MPR [dB] | ≤ 12 | ≤ 16 | ≤ 13 | ≤ 16 | ≤ 12 | ≤ 15 | ≤ 10 | ≤ 12 |  |  |
| 10 | 1432 ≤ FC < 1442 | RBstart | 0 |  | 1 – 4 |  | 5 – 9 |  | 10 – 14 |  | 15 – 21 |  |
|  |  | LCRB [RBs] | 3-4 | ≥5 | 5-8 | ≥ 9 | 9-16 | ≥17 | 15-23 | ≥24 | 24-29 | ≥30 |
|  |  | A-MPR [dB] | ≤ 13 | ≤ 16 | ≤ 13 | ≤ 16 | ≤ 13 | ≤ 16 | ≤ 11 | ≤ 14 | ≤ 11 | ≤ 13 |
| 15 | 1434.5 ≤ FC < 1447.5 | RBstart | 0 -1 |  | 2 – 7 |  | 8 – 17 |  | 18 – 23 |  | 24 – 30 |  |
|  |  | LCRB [RBs] | 5-10 | ≥11 | 8-12 | ≥ 13 | 15-25 | ≥26 | 26-40 | ≥41 | ≥ 36 |  |
|  |  | A-MPR [dB] | ≤ 14 | ≤ 16 | ≤ 13 | ≤ 16 | ≤ 13 | ≤ 16 | ≤ 12 | ≤ 14 | ≤ 12 |  |
| 20 | 1437 ≤ FC < 1450 | RBstart | 0 -4 |  | 5 – 9 |  | 10 – 19 |  | 20 – 29 |  | 30 – 40 |  |
|  |  | LCRB [RBs] | 6-10 | ≥11 | 11-20 | ≥ 21 | 17-32 | ≥33 | 28-45 | ≥46 | 41-54 | ≥55 |
|  |  | A-MPR [dB] | ≤ 13 | ≤ 16 | ≤ 13 | ≤ 16 | ≤ 13 | ≤ 16 | ≤ 12 | ≤ 14 | ≤ 11 | ≤ 13 |

Table 6.2.4-29: A-MPR for "NS_39"

| Channel Bandwidth [MHz] | Carrier centre frequency (FC) [MHz] | Parameters | Region A |  |  | Region B |  | Region C |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 1462 < FC ≤1465 | RBstart | 43-50 |  |  |  |  |  |  |
|  |  | LCRB [RBs] | ≤ 3 | > 3 and ≤ 25 | ≥ 32 |  |  |  |  |
|  |  | A-MPR [dB] | ≤ 4 | ≤ 3 | ≤ 3 |  |  |  |  |
| 15 | 1456.3 < FC ≤ 1462.5 | RBstart | 56-75 |  |  | 64-75 |  | 60-75 |  |
|  |  | LCRB [RBs] | ≥ 36 |  |  | ≥ 13 and ≤ 35 |  | ≥ 5 and ≤ 12 | < 5 |
|  |  | A-MPR [dB] | ≤ 3 |  |  | ≤ 2 |  | ≤ 3 | ≤ 4 |
| 20 | 1450.8 < FC ≤ 1460 | RBstart | 71-100 |  |  | >76 |  |  |  |
|  |  | LCRB [RBs] | ≥ 36 |  |  | ≥ 7 and ≤ 35 | < 7 |  |  |
|  |  | A-MPR [dB] | ≤ 3 |  |  | ≤ 3 | ≤ 4 |  |  |

Table 6.2.4-30a: A-MPR for "NS_40"

| Channel bandwidth confined to 1427-1432MHz (B51) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth [MHz] | Carrier centre frequency (FC) [MHz] | Parameters | Region A |  | Region B |  | Region C |  | Region D |  |
| 3MHz | 1428.5 MHz ≤ FC ≤ 1430.5 MHZ | RBstart | 0 |  | 1– 2 |  | 3 – 6 |  |  |  |
|  |  | LCRB [RBs] | 1 | ≥2 | 2 | ≥ 3 | 5-6 | ≥8 |  |  |
|  |  | A-MPR [dB] | ≤ 11 | ≤ 16 | ≤ 9 | ≤ 15 | ≤ 12 | ≤ 14 |  |  |
| 5MHz | 1429.5 MHz ≤ FC ≤ 1432 MHz | RBstart | 0 - 1 |  | 2 – 4 |  | 5 – 7 |  | 8 – 10 |  |
|  |  | LCRB [RBs] | 2 | ≥3 | 4-6 | ≥ 7 | 8-10 | ≥11 | 12 | ≥15 |
|  |  | A-MPR [dB] | ≤ 12 | ≤ 16 | ≤ 13 | ≤ 16 | ≤ 12 | ≤ 15 | ≤ 10 | ≤ 12 |

Table 6.2.4-30b: A-MPR for "NS_40"

| Channel bandwidth confined to 1429-1432MHz (B51) |  |  |  |
| --- | --- | --- | --- |
| Channel bandwidth [MHz] | Carrier centre frequency (FC) [MHz] | Parameters | Region A |
| 3MHz | 1430.5 MHz < FC ≤ 1432 MHZ | RBstart | 0 |
|  |  | LCRB [RBs] | 12 |
|  |  | A-MPR [dB] | ≤ 9 |

Table 6.2.4-31: A-MPR for "NS_41"

| Channel bandwidth confined to 1432-1452MHz (B50) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth [MHz] | Parameters | Region A | Region B |  | Region C | Region D | Region E | Region F | Region G | Region H |
| 3MHz | RBstart |  |  |  |  |  |  |  |  |  |
|  | LCRB [RBs] |  |  |  |  |  |  |  |  |  |
|  | A-MPR [dB] |  |  |  |  |  |  |  |  |  |
| 5MHz | RBstart |  |  |  |  |  |  |  |  |  |
|  | LCRB [RBs] |  |  |  |  |  |  |  |  |  |
|  | A-MPR [dB] |  |  |  |  |  |  |  |  |  |
| 10MHz | RBstart | 0 – 4 | 5 – 6 |  |  |  |  |  |  |  |
|  | LCRB [RBs] | ≥ 36 | ≥ 40 |  |  |  |  |  |  |  |
|  | A-MPR [dB] | ≤ 11 | ≤ 10 |  |  |  |  |  |  |  |
| 15MHz | RBstart | 0-1 | 02-06 |  | 07-11 | 12-13 | 14-15 |  |  |  |
|  | LCRB [RBs] | ≥ 36 | ≥ 40 |  | ≥ 48 | 26-40 | ≥ 54 |  |  |  |
|  | A-MPR [dB] | ≤ 12 | ≤ 12 |  | ≤ 11 | ≤ 10 | ≤ 10 |  |  |  |
| 20 MHz | RBstart | 0 | 1 – 4 |  | 5-6 | 7-9 | 10 – 11 | 12 – 16 | 17 – 20 | 21 – 26 |
|  | LCRB [RBs] | ≥ 36 | 45-53 | ≥ 54 | ≥ 33 | ≥ 48 | ≥ 50 | ≥ 60 | ≥ 64 | ≥ 64 |
|  | A-MPR [dB] | ≤ 12 | ≤ 11 | ≤ 12 | ≤ 12 | ≤ 12 | ≤ 12 | ≤ 12 | ≤ 11 | ≤ 10 |

Table 6.2.4-32: A-MPR for “NS_42”

| Channel bandwidth [MHz] | Carrier centre frequency (FC) [MHz] | Parameters | Region A |  | Region B |  | Region C |  | Region D |  | Region E |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 3MHz | 1512.5 MHz ≤ FC ≤ 1515.5 MHZ | RBstart | 0 – 3 |  | 4 – 7 |  | 8-10 |  | 11-14 |  |  |  |
|  |  | LCRB [RBs] | 8-11 | ≥ 12 | ≥ 8 |  | ≥ 3 |  | ≥ 1 |  |  |  |
|  |  | A-MPR [dB] | ≤ 3 | ≤ 5 | ≤ 6 |  | ≤ 3 |  | ≤ 3 |  |  |  |
| 3MHz | 1511 MHz ≤ FC < 1512.5 MHz | RBstart | 0 |  |  |  |  |  |  |  |  |  |
|  |  | LCRB [RBs] | 12 |  |  |  |  |  |  |  |  |  |
|  |  | A-MPR [dB] | ≤ 1 |  |  |  |  |  |  |  |  |  |
| 5MHz | 1507 MHz ≤ FC ≤ 1514.5 MHz | RBstart | 0 – 5 |  | 6 – 11 |  | 12-16 |  | 17-24 |  |  |  |
|  |  | LCRB [RBs] | 12-16 | ≥ 17 | 9-12 | ≥ 13 | 5-8 | ≥ 9 | 1-6 | ≥ 7 |  |  |
|  |  | A-MPR [dB] | ≤ 5 | ≤ 9 | ≤ 6 | ≤ 10 | ≤ 4 | ≤ 9 | ≤ 4 | ≤ 6 |  |  |
| 10MHz | 1502 MHz ≤ FC ≤ 1512 MHz | RBstart | 0 – 22 |  | 23 – 27 |  | 28-35 |  | 36-49 |  |  |  |
|  |  | LCRB [RBs] | ≥ 20 |  | 15-19 | ≥ 20 | 3-14 | ≥ 15 | 1-9 | ≥ 10 |  |  |
|  |  | A-MPR [dB] | ≤ 4 |  | ≤ 4 | ≤ 9 | ≤ 4 | ≤ 10 | ≤ 3 | ≤ 8 |  |  |
| 10MHz | 1497 MHz ≤ FC < 1502 MHz | RBstart | 0 – 5 |  |  |  |  |  |  |  |  |  |
|  |  | LCRB [RBs] | 40 |  |  |  |  |  |  |  |  |  |
|  |  | A-MPR [dB] | ≤ 1 |  |  |  |  |  |  |  |  |  |
| 15MHz | 1497 MHz ≤ FC ≤ 1509.5 MHz | RBstart | 0 – 22 |  | 23 – 48 |  | 49-60 |  | 61-74 |  |  |  |
|  |  | LCRB [RBs] | ≥ 36 |  | 9-23 | ≥ 24 | 1-14 | ≥ 15 | ≥ 1 |  |  |  |
|  |  | A-MPR [dB] | ≤ 4 |  | ≤ 3 | ≤ 5 | ≤ 3 | ≤ 9 | ≤ 4 |  |  |  |
| 20 MHz | 1492 MHz ≤ FC ≤ 1507 MHz | RBstart | 0 – 20 |  | 21 – 40 |  | 41 – 60 |  | 61 – 80 |  | 81 – 99 |  |
|  |  | LCRB [RBs] | 24-47 | ≥ 48 | ≥ 40 |  | 15-29 | ≥ 30 | 1-17 | ≥18 | 1-14 | ≥ 15 |
|  |  | A-MPR [dB] | ≤ 2 | ≤ 4 | ≤ 3 |  | ≤2 | ≤ 4 | ≤ 3 | ≤ 4 | ≤ 3 | ≤ 6 |

Table 6.2.4-32a: Void

Table 6.2.4-32b: Void

Table 6.2.4-33: A-MPR for “NS_43”

| Parameters |  |  |  |
| --- | --- | --- | --- |
| Channel bandwidth (MHz) | Carrier centre frequency (FC) (MHz) | Uplink resource allocation | A-MPR (dB) |
| 20 | 3580 ≤ FC ≤ 3670 | L = 10 (RIV = 19) | [1] |
|  |  | Any other RIV | [2] |
|  | 3560 ≤ FC < 3580 3670 < FC ≤ 3690 | L = 10 (RIV = 19) | [4] |
|  |  | Any other RIV | [4] |
| NOTE 1: The uplink resource allocation is defined in Clause 8.1.4 of TS 36.213 [6]. |  |  |  |

Table 6.2.4-34: A-MPR requirements for “NS_44” for Power Class 2 UE

| Channel bandwidth [MHz] | Parameters |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 10 | Fc [MHz] | ≥2605 |  |  |  | <2605 |
|  | RBstart | 0 – 5 |  | 38-49 |  | 0 - 49 |
|  | LCRB [RBs] | ≥ 45 |  | < 13 |  | > 0 |
|  | RBstart + LCRB [RBs] | N/A |  | NA |  | N/A |
|  | A-MPR [dB] | ≤ 1 |  | ≤ 1 |  | 0 |
| 15 | Fc [MHz] | ≥2597.5 |  |  |  | <2597.5 |
|  | RBstart | 0 – 18 | 19-30 |  | 54-74 | 0 - 74 |
|  | LCRB [RBs] | N/A | ≥ 45& <56 |  | < 17 | > 0 |
|  | RBstart + LCRB [RBs] | ≥ 63 | N/A |  | NA | N/A |
|  | A-MPR [dB] | ≤ 1 | ≤ 1 |  | ≤ 1 | 0 |
| 20 | Fc [MHz] | ≥2590 |  |  |  | <2590 |
|  | RBstart | 0 – 31 | 32-55 |  | 70-99 | 0 - 99 |
|  | LCRB [RBs] | N/A | ≥ 45& <68 |  | < 19 | > 0 |
|  | RBstart + LCRB [RBs] | ≥ 76 | N/A |  | NA | N/A |
|  | A-MPR [dB] | ≤ 2 | ≤ 1 |  | ≤ 1 | 0 |

Table 6.2.4-34a: A-MPR for "NS_56"

| Channel bandwidth confined to 1627.5- 1637.5MHz |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel bandwidth | Carrier centre frequency (FC) (MHz) | Parameters | Region A | Region B |  | Region C | Region D | Region E | Region F | Region G |
| 5 MHz | 1630.0, 1630.3 | RBstart | ≤ 8 | ≤ 8 |  | N/A | N/A | N/A | N/A | N/A |
|  |  | LCRB [RBs] | ≤ 8 | > 8 |  | N/A | N/A | N/A | N/A | N/A |
|  |  | A-MPR [dB] | 8 | 2 |  | N/A | N/A | N/A | N/A | N/A |
|  | 1635.0 | No A-MPR needed |  |  |  |  |  |  |  |  |
|  | 1649.0 |  |  |  |  |  |  |  |  |  |
|  | 1654.0 |  |  |  |  |  |  |  |  |  |
| 10 MHz | 1632.5 | RBstart | ≤ 5 | ≤ 5 | 6 to18 | ≤ 18 | 35 to 39 | 35 to 39 | ≥ 40 | ≥ 40 |
|  |  | LCRB [RBs] | ≤ 5 | 6 to 12 | ≤ 12 | > 12 | ≤ 7 | > 7 | ≤ 7 | > 7 |
|  |  | A-MPR [dB] | 7 | 5 |  | 7 | 4 | 2 | 5 | 3 |
|  | 1651.5 | No A-MPR needed |  |  |  |  |  |  |  |  |

For PRACH, PUCCH and SRS transmissions, the allowed A-MPR is according to that specified for PUSCH QPSK modulation for the corresponding transmission bandwidth.

For each TTI pattern, the A-MPR shall be evaluated per Teval period as specified in table 6.2.4-35 and given by the maximum value taken over the transmission(s) within that period; the maximum A-MPR over the TREF is then applied for TREF.

Table 6.2.4-35: A-MPR evaluation period

| TTI pattern | TREF | Teval |
| --- | --- | --- |
| Subframe | 1 subframe | 1 slot |
| Slot | 7 OS | Min(Tno_hopping, 7OS) |
| Sublot | 2 OS, 3OS | Min(Tno_hopping, 2OS/3OS) |

For the UE maximum output power modified by A-MPR, the power limits specified in subclause 6.2.5 apply.

### 6.2.4A	UE maximum output power with additional requirements for CA

Additional ACLR, spectrum emission and spurious emission requirements for carrier aggregation can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction (A-MPR) is allowed for the CA Power Class as specified in Table 6.2.2A-1.

If for intra-band carrier aggregation the UE is configured for transmissions on a single serving cell, then subclauses 6.2.3 and 6.2 4 apply with the Network Signaling value indicated by the field additionalSpectrumEmission.

For intra-band contiguous aggregation with the UE configured for transmissions on two serving cells, the maximum output power reduction specified in Table 6.2.4A-1 is allowed for all serving cells of the applicable uplink CA configurations according to the CA network signalling value indicated by the field additionalSpectrumEmissionSCell-r10. Then clause 6.2.3A does not apply, i.e. the carrier aggregation MPR = 0dB, unless the value indicated is CA_NS_09 or CA_NS_31. For uplink 64 QAM and 256 QAM, the applied maximum output power reduction is obtained by taking the maximum value of MPR requirements specified in Table 6.2.3A-1 and A-MPR requirements specified in Table 6.2.4A-1.

Table 6.2.4A-1: Additional Maximum Power Reduction (A-MPR) for intra-band contiguous CA

| CA Network Signalling value | Requirements (subclause) | Uplink CA Configuration | A-MPR [dB] (subclause) |
| --- | --- | --- | --- |
| CA_NS_01 | 6.6.3.3A.1 | CA_1C | 6.2.4A.1 |
| CA_NS_02 | 6.6.3.3A.2 | CA_1C | 6.2.4A.2 |
| CA_NS_03 | 6.6.3.3A.3 | CA_1C | 6.2.4A.3 |
| CA_NS_04 | 6.6.2.2A.1, 6.6.3.3A.8 | CA_41C, CA_41D | 6.2.4A.4 |
| CA_NS_05 | 6.6.3.3A.4 | CA_38C | 6.2.4A.5 |
| CA_NS_06 | 6.6.3.3A.5 | CA_7C | 6.2.4A.6 |
| CA_NS_07 | 6.6.3.3A.6 | CA_39C | 6.2.4A.7 |
| CA_NS_08 | 6.6.3.3A.7 | CA_42C | 6.2.4A.8 |
| CA_NS_09 | 6.6.2.2A.2 | CA_66B | N/A |
|  | 6.6.2.2A.3 | CA_66C | N/A |
| CA_NS_10 | 6.6.2.2A.4 6.6.3.3A.10 | CA_48B, CA_48C | 6.2.4A.10 |
| … |  |  |  |
| CA_NS_31 | NOTE 1 | Table 5.6A.1-1 (NOTE 1) | N/A |
| CA_NS_32 | Reserved |  |  |
| NOTE 1: Applicable for uplink CA configurations listed in Table 5.6A.1-1 for which none of the additional requirements in subclauses 6.6.2.2A or 6.6.3.3A apply. NOTE 2: The index of the sequence CA_NS corresponds to the value of additionalSpectrumEmissionSCell-r10. |  |  |  |

If for intra-band non-contigous carrier aggregation the UE is configured for transmissions on a single serving cell, then subclauses 6.2.3 and 6.2 4 apply with the Network Signaling value indicated by the field additionalSpectrumEmission.

For intra-band non-contiguous carrier aggregation with the UE configured for transmissions on two serving cells, the maximum output power reduction specified in Table 6.2.4A-2 is allowed for all serving cells of the applicable uplink CA configurations according to the CA network signalling value indicated by the field additionalSpectrumEmissionSCell-r10. MPR as specified in subclause 6.2.3A is not allowed in addition, unless A-MPR is N/A.

Table 6.2.4A-2: Additional Maximum Power Reduction (A-MPR) for intra-band non-contiguous CA

| CA Network Signalling value | Additional requirements for sub-blocks in order of increasing uplink carrier frequency |  | Uplink CA Configuration | A-MPR for sub-blocks in order of increasing uplink carrier frequency |
| --- | --- | --- | --- | --- |
|  | Requirements (subclause) | Requirements (subclause) |  | A-MPR [dB] (subclause) |
| CA_NC_NS_01 | 6.6.2.2.1 (NS_03) | 6.6.2.2.1 (NS_03) | CA_4A-4A | N/A |
| … |  |  |  |  |
| CA_NC_NS_31 | NOTE 1 | NOTE 1 | Table 5.6A.1-3 (NOTE 1) | N/A |
| CA_NC_NS_32 | Reserved |  |  |  |
| NOTE 1: Applicable for uplink CA configurations listed in Table 5.6A.1-3 for which the additional requirements in subclause 6.6.2.1.1 (indicated by NS_01) applies in each sub-block. NOTE 2: The index of the sequence CA_NC_NS corresponds to the value of additionalSpectrumEmissionSCell-r10. |  |  |  |  |

If for inter-band carrier aggregation the UE is configured for transmissions on a single serving cell, then subclauses 6.2.3 and 6.2 4 apply with the Network Signaling value indicated by the field additionalSpectrumEmission.

For inter-band carrier aggregation with the UE configured for transmissions on two serving cells the maximum output power reduction specified in Table 6.2.4-1 is allowed for each serving cell of the applicable uplink CA configuration according to the Network Signaling value indicated by the field additionalSprectrumEmission for the PCC and the CA network signalling value indicated by the field additionalSpectrumEmissionSCell-r10 for the SCC. The value of additionalSpectrumEmissionSCell-r10 is equal to that of additionalSprectrumEmission configured on the SCC. MPR as specified in subclause 6.2.3A is allowed in addition.

For PUCCH and SRS transmissions, the allowed A-MPR is according to that specified for PUSCH QPSK modulation for the corresponding transmission bandwidth.

For intra-band carrier aggregation, the A-MPR shall be evaluated per Teval period as specified in table 6.2.4A-3 and given by the maximum value taken over the transmission(s) on all component carriers within that period; the maximum A-MPR over TREF is then applied for the entire TREF.

Table 6.2.4A-3: A-MPR evaluation Teval period

| TTI pattern | TREF | Teval |
| --- | --- | --- |
| Subframe | 1 subframe | 1 slot |
| Slot | 7 OS | Min(Tno_hopping, 7OS) |
| Sublot | 2 OS, 3OS | Min(Tno_hopping, 2OS/3OS) |

For combinations of intra-band and inter-band carrier aggregation with the UE configured for transmission on three serving cells (up to two contiguously aggregated carriers per band), the maximum output power reduction is specified as follows. For the band supporting one serving cell the maximum output power reduction specified in Table 6.2.4-1 is allowed according to the Network Signaling value indicated by the field additionalSprectrumEmission for the PCC and the CA network signalling value indicated by the field additionalSpectrumEmissionSCell-r10 for the SCC. The value of additionalSpectrumEmissionSCell-r10 is equal to that of additionalSprectrumEmission configured on the SCC. MPR as specified in subclause 6.2.3A is allowed in addition. For the band supporting intra-band contiguous aggregation with the UE configured for transmissions on two serving cells, the maximum output power reduction specified in Table 6.2.4A-1 is allowed for all serving cells of the applicable uplink CA configurations according to the CA network signalling value indicated by the field additionalSpectrumEmissionSCell-r10. Then clause 6.2.3A does not apply, i.e. the carrier aggregation MPR = 0dB, unless the value indicated is CA_NS_31. For uplink 64 QAM and 256 QAM, the applied maximum output power reduction is obtained by taking the maximum value of MPR requirements specified in Table 6.2.3A-1 and A-MPR requirements specified in Table 6.2.4A-1.

For the UE maximum output power modified by A-MPR specified in table 6.2.4A-1, the power limits specified in subclause 6.2.5A apply.

#### 6.2.4A.1	A-MPR for CA_NS_01 for CA_1C

If the UE is configured to CA_1C and it receives IE CA_NS_01 the allowed maximum output power reduction applied to transmissions on the PCC and the SCC for contiguously aggregated signals is specified in table 6.2.4A.1-1.

Table 6.2.4A.1-1: Contiguous allocation A-MPR for CA_NS_01

| CA_1C: CA_NS_01 | RBstart | LCRB [RBs] | RBstart + LCRB [RBs] | A-MPR for QPSK, 16 QAM, 64 QAM and 256 QAM [dB] |
| --- | --- | --- | --- | --- |
| 100 RB / 100 RB | 0 – 23 and 176 – 199 | > 0 | N/A | ≤ 12.0 |
|  | 24 – 105 | > 64 | N/A | ≤ 6.0 |
|  | 106 – 175 | N/A | > 175 | ≤ 5.0 |
| 75 RB / 75 RB | 0 – 6 and 143 – 149 | 0 < LCRB ≤ 10 | N/A | ≤ 11.0 |
|  |  | > 10 | N/A | ≤ 6.0 |
|  | 7 – 90 | > 44 | N/A | ≤ 5.0 |
|  | 91 – 142 | N/A | > 142 | ≤ 2.0 |
| NOTE 1: RB_start indicates the lowest RB index of transmitted resource blocks NOTE 2: L_CRB is the length of a contiguous resource block allocation NOTE 3: For intra-subframe frequency hopping which intersects regions, notes 1 and 2 apply on a per slot basis. For intra-slot or intra-subslot frequency hopping which intersects regions, notes 1 and 2 apply on a per Tno_hopping basis. NOTE 4: For intra-subframe frequency hopping which intersects regions, the larger A-MPR value may be applied for both slots in the subframe. For intra-slot frequency hopping which intersects regions, the larger A-MPR value may be applied for the slot. For intra-subslot frequency hopping which intersects regions, the larger A-MPR value may be applied for the subslot. |  |  |  |  |

If the UE is configured to CA_1C and it receives IE CA_NS_01 the allowed maximum output power reduction applied to transmissions on the PCell and the SCell with non-contiguous resource allocation is defined as follows

A-MPR = CEIL {MA, 0.5}

Where MA is defined as follows

MA =	-22.5 A + 17			; 0 ≤ A < 0.20

-11.0 A + 14.7		; 0.20 ≤ A < 0.70

-1.7 A + 8.2			 ; 0.70 ≤ A ≤ 1

Where A = NRB_alloc / NRB_agg.

#### 6.2.4A.2	A-MPR for CA_NS_02 for CA_1C

If the UE is configured to CA_1C and it receives IE CA_NS_02 the allowed maximum output power reduction applied to transmission on the PCC and the SCC for contiguously aggregated signals is specified in Table 6.2.4A.2-1.

Table 6.2.4A.2-1: Contiguous allocation A-MPR for CA_NS_02

| CA_1C: CA_NS_02 | RBend | LCRB [RBs] | A-MPR for QPSK, 16 QAM, 64 QAM and 256 QAM [dB] |
| --- | --- | --- | --- |
| 100 RB / 100 RB | 0 –20 | > 0 | ≤ 4 dB |
|  | 21 – 46 | > 0 | ≤ 3 dB |
|  | 47 – 99 | > RBend - 20 | ≤ 3 dB |
|  | 100 – 184 | > 75 | ≤ 6 dB |
|  | 185 – 199 | > 0 | ≤ 10 dB |
| 75 RB / 75 RB | 0 – 48 | > 0 | ≤ 2 dB |
|  | 49 – 80 | > RBend - 20 | ≤ 3 dB |
|  | 81 – 129 | > 60 | ≤ 5 dB |
|  | 130 – 149 | > 84 | ≤ 6 dB |
|  | 130 – 149 | 1 – 84 | ≤ 2 dB |

If the UE is configured to CA_1C and it receives IE CA_NS_02 the allowed maximum output power reduction applied to transmissions on the PCell and the SCell with non-contiguous resource allocation is defined as follows:

A-MPR = CEIL {MA, 0.5}

Where MA is defined as follows

MA =	-22.5 A + 17			; 0 ≤ A < 0.20

-11.0 A + 14.7		; 0.20 ≤ A < 0.70

-1.7 A + 8.2			 ; 0.70 ≤ A ≤ 1

Where A = NRB_alloc / NRB_agg.

#### 6.2.4A.3	A-MPR for CA_NS_03 for CA_1C

If the UE is configured to CA_1C and it receives IE CA_NS_03 the allowed maximum output power reduction applied to transmission on the PCC and the SCC for contiguously aggregated signals is specified in Table 6.2.4A.3-1.

Table 6.2.4A.3-1: Contiguous allocation A-MPR for CA_NS_03

| CA_1C: CA_NS_03 | RBend | LCRB [RBs] | A-MPR for QPSK, 16 QAM, 64 QAM and 256 QAM [dB] |
| --- | --- | --- | --- |
| 100 RB / 100 RB | 0 – 26 | > 0 | ≤ 10 dB |
|  | 27 – 63 | ≥ RBend - 27 | ≤ 6 dB |
|  | 27 – 63 | < RBend - 27 | ≤ 1 dB |
|  | 64 – 100 | > RBend - 20 | ≤ 4 dB |
|  | 101 – 171 | > 68 | ≤ 7 dB |
|  | 172 – 199 | > 0 | ≤ 10 dB |
| 75 RB / 75 RB | 0 – 20 | > 0 | ≤ 10 dB |
|  | 21 – 45 | > 0 | ≤ 4 dB |
|  | 46 – 75 | > RBend – 13 | ≤ 2 dB |
|  | 76 – 95 | > 45 | ≤ 5 dB |
|  | 96 – 149 | > 43 | ≤ 8 dB |
|  | 120 – 149 | 1 - 43 | ≤ 6 dB |

If the UE is configured to CA_1C and it receives IE CA_NS_03 the allowed maximum output power reduction applied to transmissions on the PCell and the SCell with non-contiguous resource allocation is defined as follows:

A-MPR = CEIL {MA, 0.5}

Where MA is defined as follows

MA =	-23.33A + 17.5		; 0 ≤ A < 0.15

-7.65A + 15.15		; 0.15 ≤ A ≤ 1

Where A = NRB_alloc / NRB_agg.

#### 6.2.4A.4	A-MPR for CA_NS_04

If the UE is configured to CA_41C or any uplink inter-band CA configuration containing CA_41C and it receives IE CA_NS_04 the allowed maximum output power reduction applied to transmission on two component carriers for contiguously aggregated signals is specified in Table 6.2.4A.4-1 and Table 6.2.4A.4-1A for UE power class 3 and in Table 6.2.4A.4-2 for UE power class 2.

Table 6.2.4A.4-1: Contiguous Allocation A-MPR for CA_NS_04 (power class 3), Bandwidth Class C

| CA Bandwidth Class C | RBStart | LCRB [RBs] | RBstart + LCRB [RBs] | A-MPR for QPSK [dB] | A-MPR for 16 QAM, 64 QAM and 256 QAM [dB] |
| --- | --- | --- | --- | --- | --- |
| 25 RB / 100 RB | 0 – 34 and 90 – 124 | >0 | N/A | ≤3dB | ≤3.5dB |
|  | 35 – 89 | N/A | >90 | ≤1dB | ≤2.5dB |
| 50RB / 100 RB | 0 – 44 and 105 – 149 | >0 | N/A | ≤4dB | ≤4dB |
|  | 45 – 104 | N/A | >105 | ≤3dB | ≤4dB |
| 75 RB / 75 RB | 0 – 44 and 105 – 149 | >0 | N/A | ≤4dB | ≤4dB |
|  | 45 – 104 | N/A | >105 | ≤4dB | ≤4dB |
| 100 RB / 75 RB | 0 – 49 and 125 – 174 | >0 | N/A | ≤4dB | ≤4dB |
|  | 50 - 124 | N/A | >125 | ≤3dB | ≤4dB |
| 100 RB / 100 RB | 0 – 59 and 140 – 199 | >0 | N/A | ≤3dB | ≤4dB |
|  | 60– 139 | N/A | >140 | ≤4dB | ≤4dB |
| NOTE 1: RBstart indicates the lowest RB index of transmitted resource blocks NOTE 2: LCRB is the length of a contiguous resource block allocation NOTE 3: For intra-subframe frequency hopping which intersects regions, notes 1 and 2 apply on a per slot basis. For intra-slot or intra-subslot frequency hopping which intersects regions, notes 1 and 2 apply on a per Tno_hopping basis.. NOTE 4: For intra-subframe frequency hopping which intersects regions, the larger A-MPR value may be applied for both slots in the subframe. For intra-slot frequency hopping which intersects regions, the larger A-MPR value may be applied for the slot. For intra-subslot frequency hopping which intersects regions, the larger A-MPR value may be applied for the subslot. |  |  |  |  |  |

Table 6.2.4A.4-1A: Contiguous Allocation A-MPR for CA_NS_04 (power class 3), Bandwidth Class D

| CA Bandwidth Class D | RBStart | LCRB [RBs] | RBstart + LCRB [RBs] | A-MPR for QPSK [dB] | A-MPR for 16 QAM, 64 QAM and 256 QAM [dB] |
| --- | --- | --- | --- | --- | --- |
| 50 RB / 75 RB/ 100 RB | 0 – 64 and 161 – 224 | >0 | N/A | ≤4dB | ≤4.5dB |
|  | 65 – 160 | N/A | >161 | ≤3dB | ≤3.5dB |
| 50 RB / 100 RB / 100 RB | 0 – 72 and 178 – 249 | >0 | N/A | ≤4dB | ≤4.5dB |
|  | 73 – 177 | N/A | >178 | ≤3dB | ≤3.5dB |
| 75 RB / 75 RB / 100 RB | 0 – 72 and 178 – 249 | >0 | N/A | ≤4dB | ≤4.5dB |
|  | 73 – 177 | N/A | >178 | ≤3dB | ≤3.5dB |
| 75 RB / 100 RB / 100 RB | 0 – 78 and 197 – 274 | >0 | N/A | ≤3.5dB | ≤4dB |
|  | 79 - 196 | N/A | >197 | ≤2.5dB | ≤3dB |
| 100 RB / 100 RB / 100 RB | 0 – 96 and 204 – 300 | >0 | N/A | ≤4dB | ≤4dB |
|  | 97– 203 | N/A | >204 | ≤2.5dB | ≤3dB |
| NOTE 1: RBstart indicates the lowest RB index of transmitted resource blocks NOTE 2: LCRB is the length of a contiguous resource block allocation NOTE 3: For intra-subframe frequency hopping which intersects regions, notes 1 and 2 apply on a per slot basis. For intra-slot or intra-subslot frequency hopping which intersects regions, notes 1 and 2 apply on a per Tno_hopping basis.. NOTE 4: For intra-subframe frequency hopping which intersects regions, the larger A-MPR value may be applied for both slots in the subframe. For intra-slot frequency hopping which intersects regions, the larger A-MPR value may be applied for the slot. For intra-subslot frequency hopping which intersects regions, the larger A-MPR value may be applied for the subslot. |  |  |  |  |  |

Table 6.2.4A.4-2: Contiguous Allocation A-MPR for CA_NS_04 (power class 2)

| CA Bandwidth Class C | Lower edge cutoff frequency [MHz]5 | RBStart | LCRB [RBs] | RBstart + LCRB [RBs] | A-MPR per modulation [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | QPSK | 16QAM | 64QAM | 256QAM |
| 25 RB / 100 RB | 2513.5 | 0 – 42 | >0 | N/A | ≤5 | ≤5 | ≤5 | 6.5 |
|  |  | 43 – 81 | N/A | >82 | ≤1 | ≤1.5 | ≤1.5 | 3 |
|  |  | 82 – 124 | >0 | N/A | ≤1 | ≤1.5 | ≤1.5 | 2.5 |
| 50 RB / 100 RB | 2518.4 | 0 – 52 | >0 | N/A | ≤5 | ≤5 | ≤5 | 6.5 |
|  |  | 53 – 94 | N/A | >95 | ≤1 | ≤1.5 | ≤1.5 | 3 |
|  |  | 95 – 149 | >0 | N/A | ≤1 | ≤1.5 | ≤1.5 | 2.5 |
| 75 RB / 75 RB | 2519.0 | 0 – 54 | >0 | N/A | ≤5 | ≤5 | ≤5 | 6.5 |
|  |  | 55 – 94 | N/A | >95 | ≤2 | ≤2.5 | ≤2.5 | 3.5 |
|  |  | 95 – 149 | >0 | N/A | ≤1.5 | ≤2 | ≤2 | 3 |
| 75 RB / 100 RB | 2523.4 | 0 – 64 | >0 | N/A | ≤5 | ≤5 | ≤5 | 6.5 |
|  |  | 65 – 114 | N/A | >115 | ≤2 | ≤2.5 | ≤2.5 | 3.5 |
|  |  | 115 – 174 | >0 | N/A | ≤1 | ≤1.5 | ≤2 | 3 |
| 100 RB / 100 RB | 2528.3 | 0 – 69 | >0 | N/A | ≤5 | ≤5 | ≤5 | 6.5 |
|  |  | 70 – 129 | N/A | >130 | ≤2 | ≤2.5 | ≤2.5 | 3.5 |
|  |  | 130 – 199 | >0 | N/A | ≤1.5 | ≤1.5 | ≤2 | 3 |
| NOTE 1: RBstart indicates the lowest RB index of transmitted resource blocks NOTE 2: LCRB is the length of a contiguous resource block allocation NOTE 3: For intra-subframe frequency hopping which intersects regions, notes 1 and 2 apply on a per slot basis NOTE 4: For intra-subframe frequency hopping which intersects regions, the larger A-MPR value may be applied for both slots in the subframe NOTE 5: The A-MPR values in this table shall apply when the lower edge of the aggregated channel bandwidth (Figure 5.6A-1) is less than or equal to the lower edge cutoff frequency specified in this table for the corresponding CA bandwidth combination. When the lower edge of the aggregated channel bandwidth exceeds the lower edge cutoff frequency, then the A-MPR shall be equal to the MPR specified in Table 6.2.3A-1a. |  |  |  |  |  |  |  |  |

If the UE is configured to CA_41C or any uplink inter-band CA configuration containing CA_41C and it receives IE CA_NS_04 the allowed maximum output power reduction applied to transmissions on two serving cells assigned to Band 41 with non-contiguous resource allocation is defined as follows for UE power class 3

A-MPR = CEIL {MA, 0.5}

Where MA is defined as follows

MA 	= 	11,	 				0≤ A < 0.05

= -55.0A + 13.75,		0.05≤ A < 0.15

= -4.0A + 6.10, 			0.15≤ A < 0.40

= -0.83A + 4.83,		0.40 ≤ A ≤ 1

Where A = NRB_alloc / NRB_agg.

If the UE is configured to CA_41C or any uplink inter-band CA configuration containing CA_41C and it receives IE CA_NS_04 the allowed maximum output power reduction applied to transmissions on two serving cells assigned to Band 41 with non-contiguous resource allocation is defined as follows for UE power class 2

A-MPR = CEIL {MA, 0.5}

If the UE is configured to CA_41D or any uplink inter-band CA configuration containing CA_41D and it receives IE CA_NS_04 the allowed maximum output power reduction applied to transmissions on two serving cells assigned to Band 41 with non-contiguous resource allocation is defined as follows for UE power class 3

A-MPR = CEIL {MA, 0.5}

Where MA is defined as follows

MA 	= 	11.5,	 				0≤ A < 0.05

= -55.0A + 14.25,		0.05≤ A < 0.15

= -4.0A + 6.60, 			0.15≤ A < 0.40

= -0.833A + 5.333,		0.40 ≤ A ≤ 1

Where A = NRB_alloc / NRB_agg.

Where MA is defined as follows when the lower edge of the aggregated channel bandwidth (Table 5.6A-1) is less than or equal to the lower edge cutoff frequency specified in Table 6.2.4A.4-2 for the corresponding CA bandwidth combination

MA 	= 13.0,		 				0 ≤ A < 0.05

= 15.33 – 46.67A,		0.05 ≤ A < 0.20

= -7.0 – 5.0A, 			0.20 ≤ A < 0.50

= 4.5,						0.50 ≤ A ≤ 1

And MA is defined as follows when the lower edge of the aggregated channel bandwidth exceeds the lower edge cutoff frequency specified in Table 6.2.4A.4-2 for the corresponding CA bandwidth combination

MA 	= 8.2,		 				0 ≤ A < 0.04

= 9.8 – 40.0A,			0.04 ≤ A < 0.075

= 8.0 – 16.0A, 			0.075 ≤ A < 0.25

= 4.83 – 3.33A,			0.25 ≤ A < 0.40

= 3.83 – 0.83A,			0.40 ≤ A ≤ 1

Where A = NRB_alloc / NRB_agg.

#### 6.2.4A.5	A-MPR for CA_NS_05 for CA_38C

If the UE is configured to CA_38C and it receives IE CA_NS_05 the allowed maximum output power reduction applied to transmission on the PCC and the SCC for contiguously aggregated signals is specified in Table 6.2.4A.5-1.

Table 6.2.4A.5-1: Contigous Allocation A-MPR for CA_NS_05

| CA_38C | RBend | LCRB [RBs] | A-MPR for QPSK, 16 QAM, 64 QAM and 256 QAM [dB] |
| --- | --- | --- | --- |
| 100RB/100RB | 0 – 12 | >0 | ≤ 5 dB |
|  | 13 – 79 | > RBend – 13 | ≤ 2 dB |
|  | 80 – 180 | >60 | ≤ 6 dB |
|  | 181 – 199 | > 0 | ≤ 11 dB |
| 75RB/75RB | 0 – 70 | > max (0, RBend -10) | ≤ 2 dB |
|  | 71- 108 | > 60 | ≤ 5 dB |
|  | 109 – 139 | >0 | ≤ 5 dB |
|  | 140 – 149 | ≤ 70 | ≤ 2 dB |
|  | 140 – 149 | >70 | ≤ 6 dB |
| NOTE 1: RBend indicates the highest RB index of transmitted resource blocks NOTE 2: LCRB is the length of a contiguous resource block allocation NOTE 3: For intra-subframe frequency hopping which intersects regions, notes 1 and 2 apply on a per slot basis. For intra-slot or intra-subslot frequency hopping which intersects regions, notes 1 and 2 apply on a per Tno_hopping basis. NOTE 4: For intra-subframe frequency hopping which intersects regions, the larger A-MPR value may be applied for both slots in the subframe. For intra-slot frequency hopping which intersects regions, the larger A-MPR value may be applied for the slot. For intra-subslot frequency hopping which intersects regions, the larger A-MPR value may be applied for the subslot. |  |  |  |

If the UE is configured to CA_38C and it receives IE CA_NS_05 the allowed maximum output power reduction applied to transmissions on the PCell and the SCell with non-contiguous resource allocation is defined as follows

A-MPR = CEIL {MA, 0.5}

Where MA is defined as follows

MA = -14.17 A + 16.50		; 0 ≤ A < 0.60

-2.50 A + 9.50			; 0.60 ≤ A ≤ 1

Where A = NRB_alloc / NRB_agg.

#### 6.2.4A.6	A-MPR for CA_NS_06

If the UE is configured to CA_7C and it receives IE CA_NS_06 the allowed maximum output power reduction applied to transmission on the PCC and the SCC for contiguously aggregated signals is specified in Table 6.2.4A.6-1.

Table 6.2.4A.6-1: Contiguous Allocation A-MPR for CA_NS_06

| CA Bandwidth Class C | RBend | LCRB [RBs] | A-MPR for QPSK, 16 QAM, 64 QAM and 256 QAM [dB] |
| --- | --- | --- | --- |
| 100RB/100RB | 0 –22 | >0 | ≤ 4 dB |
|  | 23 – 99 | > max(0,RBend – 25) | ≤ 2 dB |
|  | 100 – 142 | > 75 | ≤ 3 dB |
|  | 143 – 177 | >70 | ≤ 5 dB |
|  | 178 – 199 | > 0 | ≤ 10 dB |
| 75RB/75RB | 0 – 7 | >0 | ≤ 5 dB |
|  | 8- 74 | > max(0,RBend – 10) | ≤ 2 dB |
|  | 75 – 109 | >64 | ≤ 2 dB |
|  | 110 – 144 | >35 | ≤ 6 dB |
|  | 145 – 149 | >0 | ≤ 10 dB |
| 50RB/100RB and 100RB/50RB | 0 – 10 | > 0 | ≤ 5 dB |
|  | 11 – 75 | > max(0, RB_End – 25) | ≤ 2 dB |
|  | 76 – 103 | > 50 | ≤ 3 dB |
|  | 104 – 144 | > 25 | ≤ 6 dB |
|  | 145 – 149 | > 0 | ≤ 10 dB |
| 75RB/100RB and 100RB/75RB | 0 – 15 | > 0 | ≤ 5 dB |
|  | 16 – 75 | > max(0, RB_End – 15) | ≤ 2 dB |
|  | 76 – 120 | > 50 | ≤ 3 dB |
|  | 121 – 160 | > 50 | ≤ 6 dB |
|  | 161 – 174 | > 0 | ≤ 10 dB |

If the UE is configured to CA_7C and it receives IE CA_NS_06 the allowed maximum output power reduction applied to transmissions on the PCell and the SCell with non-contiguous resource allocation is defined as follows:

A-MPR = CEIL {MA, 0.5}

Where MA is defined as follows

MA =	-13.33A + 17.5				; 0 ≤ A < 0.15

-6.47A + 16.47				; 0.15 ≤ A ≤ 1

Where A = NRB_alloc / NRB_agg.

#### 6.2.4A.7	A-MPR for CA_NS_07

If the UE is configured to CA_39C or any uplink inter-band CA configuration containing CA_39C and it receives IE CA_NS_07 the allowed maximum output power reduction applied to transmission on two component carriers for contiguously aggregated signals is specified in Table 6.2.4A.7-1.

Table 6.2.4A.7-1: Contiguous Allocation A-MPR for CA_NS_07

| CA_39C: CA_NS_07 | RBStart | LCRB [RBs] | A-MPR for QPSK, 16 QAM, 64 QAM and 256 QAM [dB] |
| --- | --- | --- | --- |
| 75 RB / 100 RB and 100 RB / 75 RB | 0 – 13 | > 0 | ≤ 11 |
|  | 14 – 50 | ≤ 60 | ≤ 3 |
|  | 14 – 100 | > 60 | ≤ 7 |
|  | 101 – 155 | > max(155 - RBstart , 0) | ≤ 2 |
|  | 156 – 174 | > 0 | ≤ 5 |
| 50 RB / 100 RB and 100 RB / 50 RB | 0 – 5 | > 0 | ≤ 11 |
|  | 6 – 42 | ≤ 25 | ≤ 3 |
|  |  | > 25 | ≤ 6 |
|  | 43 – 80 | > 50 | ≤ 5 |
|  | 81 – 138 | > 20 | ≤ 2 |
|  | 139 – 149 | > 0 | ≤ 5 |
| 25 RB / 100 RB and 100 RB / 25 RB | 0 – 32 | ≥ 84 | ≤ 6 |
|  |  | < 84 | ≤ 4 |
|  | 33 – 60 | > 50 | ≤ 3 |
|  | 61 – 124 | > 20 | ≤ 3 |

If the UE is configured to CA_39C or any uplink inter-band CA configuration containing CA_39C and it receives IE CA_NS_07 the allowed maximum output power reduction applied to transmissions on two serving cells assigned to Band 39 with non-contiguous resource allocation is defined as follows

A-MPR = CEIL {MA, 0.5}

Where MA is defined as follows

MA = -16. 25A + 21			; 0 ≤ A < 0. 80

-2.50 A + 10.00			; 0.80 ≤ A ≤ 1

Where A = NRB_alloc / NRB_agg

#### 6.2.4A.8	A-MPR for CA_NS_08

If the UE is configured to CA_42C and it receives IE CA_NS_08 the allowed maximum output power reduction applied to transmission on the PCC and the SCC for contiguously aggregated signals is specified in Table 6.2.4A.8-1.

Table 6.2.4A.8-1: Contiguous Allocation A-MPR for CA_NS_08

| CA_42C: CA_NS_08 | RBstart | Condition | RBend | LCRB [RBs] | A-MPR for QPSK, 16 QAM, 64 QAM and 256 QAM [dB] |
| --- | --- | --- | --- | --- | --- |
| 100RB / 100RB | ≤ 21 | Or | ≥ 178 | ≤ 25 | ≤ 12 |
|  |  |  |  | > 25 and ≤ 80 | ≤ 6 |
|  | ≥ 0 | N/A | N/A | > 80 and ≤ 172 | ≤ 8 |
|  |  |  |  | > 172 | ≤ 9 |
|  | > 21 and ≤ 58 | Or | ≥ 141 and < 178 | < 48 | ≤ 3 |
|  | > 21 | And | < 178 | ≥ 48 and ≤ 80 | ≤ 4 |
| 100RB / 75RB And 75RB / 100RB | ≤ 12 | Or | ≥ 162 | ≤ 25 | ≤ 12 |
|  |  |  |  | > 25 and ≤ 75 | ≤ 6 |
|  | ≥ 0 | N/A | N/A | > 75 and <172 | ≤ 8 |
|  |  |  |  | ≥172 | 9 |
|  | > 12 and ≤ 49 | Or | ≥ 125 and < 162 | < 54 | ≤ 3 |
|  | > 12 | And | < 162 | ≥ 54 and ≤75 | ≤ 5 |
|  | > 49 | And | < 125 | ≥ 36 and < 54 | ≤ 2 |
| 100RB / 50RB And 50RB / 100RB | ≤ 5 | Or | ≥ 144 | ≤ 16 | ≤ 12 |
|  |  |  |  | > 16 and ≤ 61 | ≤ 6 |
|  | ≥ 0 | N/A | N/A | > 61 | ≤ 8 |
|  | > 5 | And | < 144 | ≥ 36 and ≤ 61 | ≤ 5 |
|  | > 5 and ≤ 41 | Or | ≥ 108 and < 144 | < 36 | ≤ 3 |
| 100RB / 25RB And 25RB / 100RB | ≤ 31 | Or | ≥ 92 | ≤ 34 | ≤ 4 |
|  |  |  |  | > 34 and ≤ 44 | ≤ 5 |
|  | ≥ 0 | N/A | N/A | > 44 | ≤ 8 |
| NOTE 1: RBstart indicates the lowest RB index of transmitted resource blocks NOTE 2: LCRB is the length of a contiguous resource block allocation NOTE 3: RBend indicates the highest RB index of transmitted resource blocks NOTE 4: If condition is “and” both RBstart and RBend constraints need to be met. If condition is “or” either RBstart or RBend constraints need to be met NOTE 5: For intra-subframe frequency hopping which intersects regions, notes 1, 2, 3 and 4 apply on a per slot basis. For intra-slot or intra-subslot frequency hopping which intersects regions, notes 1,2,3 and 4 apply on a per Tno_hopping basis. NOTE 6: For intra-subframe frequency hopping which intersects regions, the larger A-MPR value may be applied for both slots in the subframe. For intra-slot frequency hopping which intersects regions, the larger A-MPR value may be applied for the slot. For intra-subslot frequency hopping which intersects regions, the larger A-MPR value may be applied for the subslot. |  |  |  |  |  |

If the UE is configured to CA_42C and it receives IE CA_NS_08 the allowed maximum output power reduction applied to transmissions on the PCell and the SCell with non-contiguous resource allocation is defined as follows

A-MPR = CEIL {MA, 0.5}

Where MA is defined as follows

MA =	20						; 0 ≤ A < 0.025

23 – 120A			; 0.025 ≤ A < 0.05

17.53 – 10.59A		; 0.05 ≤ A ≤ 0.9

8						; 0.9 ≤ A ≤ 1

Where A = NRB_alloc / NRB_agg.

#### 6.2.4A.9	Void

#### 6.2.4A.10	A-MPR for CA_NS_10

If the UE is configured to CA_48C and it receives IE CA_NS_10 the allowed maximum output power reduction applied to transmission on the PCC and the SCC for contiguously aggregated signals is specified in Table 6.2.4A.10-2 or Table 6.2.4A.10-3. Which table is determined by the position of the carrier centre frequency in Table 6.2.4A.10-1.

Table 6.2.4A.10-1: A-MPR regions for CA_48C

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | A-MPR |
| --- | --- | --- |
| 20+5 / 5 + 20 | FL + BWCA/2 ≤ FC < FL + 3*BWCA/2 - 10 MHz FH - 3*BWCA/2 + 10 MHz ≤ FC ≤ FH - BWCA/2 | Table 6.2.4A.10-2 |
|  | FL + 3*BWCA/2 - 10 MHz ≤ FC < FH - 3*BWCA/2 + 10 MHz | Table 6.2.4A.10-3 |
| 20+10 / 10 + 20 | FL + BWCA/2 ≤ FC < FL + 3*BWCA/2 - 10 MHz FH - 3*BWCA/2 + 10 MHz ≤ FC ≤ FH - BWCA/2 | Table 6.2.4A.10-2 |
|  | FL + 3*BWCA/2 - 10 MHz ≤ FC < FH - 3*BWCA/2 + 10 MHz | Table 6.2.4A.10-3 |
| 20+15 / 15 + 20 | FL + BWCA/2 ≤ FC< FL + 3*BWCA/2 - 10 MHz FH - 3*BWCA/2 + 10 MHz ≤ FC ≤ FH - BWCA/2 | Table 6.2.4A.10-2 |
|  | FL + 3*BWCA/2 - 10 MHz ≤ FC < FH - 3*BWCA/2 + 10 MHz | Table 6.2.4A.10-3 |
| 20+20 | FL + BWCA/2 ≤ FC < FL + 3*BWCA/2 - 10 MHz FH - 3*BWCA/2 + 10 MHz ≤ FC ≤ FH - BWCA/2 | Table 6.2.4A.10-2 |
|  | FL + 3*BWCA/2 - 10 MHz ≤ FC < FH - 3*BWCA/2 + 10 MHz | Table 6.2.4A.10-3 |
| NOTE: FL = 3550 MHz, FH = 3700 MHz and BWCA is the combined bandwidth of the contiguous CCs in the CA combination indicated. |  |  |

Table 6.2.4A.10-2: A-MPR regions for CA_48C at the band edge

| BWs [MHz] | RB_start | L_CRB | A-MPR [dB] |
| --- | --- | --- | --- |
| 20+5 / 5 + 20 | 0 – 7 and 117 - 124 |  | ≤ 12 |
|  | 8 - 25 | <85 | ≤ 5.5 |
|  |  | ≥ 85 | ≤ 6.5 |
|  | 26 - 116 |  | ≤ 5 |
| 20+10 / 10 + 20 | 0 - 13 and 135 - 149 |  | ≤ 11 |
|  | 14 - 33 | <85 | ≤ 5 |
|  |  | ≥85 | ≤ 7 |
|  | 34 - 134 |  | ≤ 6 |
| 20+15 / 15 + 20 | 0 - 22 and 152 -174 |  | ≤ 11 |
|  | 23 - 42 | <95 | ≤ 5.5 |
|  |  | ≥95 | ≤ 7 |
|  | 43 - 151 |  | ≤ 6 |
| 20+20 | 0 - 31 and 165 - 199 |  | ≤ 11 |
|  | 32 - 51 | <100 | ≤ 5.5 |
|  |  | ≥100 | ≤ 7 |
|  | 52 - 164 | <100 | ≤ 4.5 |
|  |  | ≥100 | ≤ 6 |

Table 6.2.4A.10-3: A-MPR regions for CA_48C at the band center (“range for lower A-MPR”)

| BWs [MHz] | RB_start | L_CRB | A-MPR [dB] |
| --- | --- | --- | --- |
| 20+5 / 5 + 20 | 0 – 7 and 117 - 124 |  | ≤ 4 |
|  | 8 - 25 | <85 | ≤ 2 |
|  |  | ≥85 | ≤ 2.5 |
|  | 26 - 116 |  | ≤ 2 |
| 20+10 / 10 + 20 | 0 - 13 and 135 - 149 |  | ≤ 4.5 |
|  | 14 - 33 | <85 | ≤4.5 |
|  |  | ≥85 | ≤ 2.5 |
|  | 34 - 134 |  | ≤ 1.5 |
| 20+15 / 15 + 20 | 0 - 22 and 152 -174 |  | ≤ 4.5 |
|  | 23 - 42 | <95 | ≤ 1 |
|  |  | ≥95 | ≤ 2.5 |
|  | 43 - 151 |  | ≤ 1 |
| 20+20 | 0 - 31 and 165 - 199 |  | ≤ 4.5 |
|  | 32 - 51 | <100 | ≤ 1 |
|  |  | ≥100 | ≤ 1.5 |
|  | 52 - 164 | <100 | ≤ 1 |
|  |  | ≥100 | ≤ 1 |

If the UE is configured to CA_48C and it receives IE CA_NS_10 the allowed maximum output power reduction applied to transmissions on the PCell and the SCell with non-contiguous resource allocation is defined with both an edge and a center scenario and is determined in Table 6.2.4A.10-4.

Table 6.2.4A.10-4: A-MPR regions for CA_48C

| Channel Bandwidth, MHz | Carrier Centre Frequency, Fc, MHz | A-MPR |
| --- | --- | --- |
| 20+5 / 5 + 20 | FL + BWCA/2 ≤ FC < FL + 3*BWCA/2 - 10 MHz FH - 3*BWCA/2 + 10 MHz ≤ FC ≤ FH - BWCA/2 | Edge |
|  | FL + 3*BWCA/2 - 10 MHz ≤ FC < FH - 3*BWCA/2 + 10 MHz | Center |
| 20+10 / 10 + 20 | FL + BWCA/2 ≤ FC < FL + 3*BWCA/2 - 10 MHz FH - 3*BWCA/2 + 10 MHz ≤ FC ≤ FH - BWCA/2 | Edge |
|  | FL + 3*BWCA/2 - 10 MHz ≤ FC < FH - 3*BWCA/2 + 10 MHz | Center |
| 20+15 / 15 + 20 | FL + BWCA/2 ≤ FC< FL + 3*BWCA/2 - 10 MHz FH - 3*BWCA/2 + 10 MHz ≤ FC ≤ FH - BWCA/2 | Edge |
|  | FL + 3*BWCA/2 - 10 MHz ≤ FC < FH - 3*BWCA/2 + 10 MHz | Center |
| 20+20 | FL + BWCA/2 ≤ FC < FL + 3*BWCA/2 - 10 MHz FH - 3*BWCA/2 + 10 MHz ≤ FC ≤ FH - BWCA/2 | Edge |
|  | FL + 3*BWCA/2 - 10 MHz ≤ FC < FH - 3*BWCA/2 + 10 MHz | Center |
| NOTE: FL = 3550 MHz, FH = 3700 MHz and BWCA is the combined bandwidth of the contiguous CCs in the CA combination indicated. |  |  |

The Edge scenario is defined as follows

A-MPR = CEIL {MA, 0.5}

where MA is defined as follows

MA =      18.00 - 10.00 A; 			0 ≤ A < 0.05

18.50 - 20.00 A;			0.05 ≤ A < 0.2

15.50 - 5.00 A;			0.2 ≤ A < 1

where A = NRB_alloc / NRB_agg.

The Center scenario is defined as follows

A-MPR = CEIL {MA, 0.5}

where MA is defined as follows

MA =      11.50 - 10.00 A; 			0 ≤ A < 0.15

10.88 - 5.88 A;			0.15 ≤ A < 1

where A = NRB_alloc / NRB_agg

For CA_48B contiguous resource allocation when 3560 MHz ≤ Fagg_alloc_low and Fagg_alloc_high <= 3690 MHz

if allocation is inner 1 then A-MPR = 0 dB where inner 1 is defined as

RBStart,Low = max(1, floor(LCRB/2))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = NRB_agg – RBStart,Low – LCRB

with following conditions

RBStart,Low  ≤  RBStart  ≤  RBStart,High, and

LCRB  ≤  ceil(NRB_agg /2)

Inner 1 region exceptions thresholds are

RBstart < 12 or RBend ≥ 92 for BWChannel_CA = 20MHz

For which AMPR = 4 dB.

else A-MPR= 4 dB

For CA_48B contiguous resource allocation when Fagg_alloc_low < 3560 MHz

if allocation is inner 3 then A-MPR = 0 dB

Inner 3 region exceptions thresholds are

RBstart < 30 for BWChannel_CA = 20MHz

For which AMPR = 7dB.

where inner 3 is defined as

NRB_agg /4 < RBStart < NRB_agg 3/4  LCRB  AND LCRB < NRB_agg/4

else A-MPR = 7 dB.

For CA_48B contiguous resource allocation when Fagg_alloc_high > 3690 MHz

if allocation is inner 3 then A-MPR = 0 dB

Inner 3 region exceptions thresholds are

RBstart > 70 for BWChannel_CA = 20MHz

For which AMPR = 7dB.

where inner 3 is defined as

NRB_agg /4 < RBStart < NRB_agg 3/4  LCRB  AND LCRB < NRB_agg/4

else A-MPR = 7 dB.

For CA_48B non-contiguous resource allocation when 3560 MHz ≤ Fagg_alloc_low and  Fagg_alloc_high <= 3690 MHz

A = NRB_alloc / NRB_agg

A-MPR= 			13.00;         			0.00 <= A <= 0.08

13.78 - 9.78 A; 		0.08 < A <= 1.00

For CA_48B non-contiguous resource allocation when Fagg_alloc_low < 3560 MHz or Fagg_alloc_high > 3690 MHz

A-MPR= 		13.00;     			0.00 <= A <= 0.08

14.13  -14.06 A; 		0.08 < A <= 0.40

9.17 – 1.67 A;		0.40 < A <= 1.00

### 6.2.4B	UE maximum output power with additional requirements for UL-MIMO

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the A-MPR values specified in subclause 6.2.4 shall apply to the maximum output power specified in Table 6.2.2B-1. The requirements shall be met with the UL-MIMO configurations specified in Table 6.2.2B-2. For UE supporting UL-MIMO, the maximum output power is measured as the sum of the maximum output power at each UE antenna connector. Unless stated otherwise, an A-MPR of 0 dB shall be used.

For the UE maximum output power modified by A-MPR, the power limits specified in subclause 6.2.5B apply.

If UE is configured for transmission on single-antenna port, the requirements in subclause 6.2.4 apply.

### 6.2.4D	UE maximum output power with additional requirements for ProSe

When UE is configured for E-UTRA ProSe sidelink transmissions non-concurrent with E-UTRA uplink transmissions for E-UTRA ProSe operating bands specified in Table 5.5D-1, the allowed A-MPR for the maximum output power for ProSe physical channels PSDCH, PSCCH, PSSCH, and PSBCH shall be as specified in subclause 6.2.4 for  PUSCH for the corresponding modulation and transmission bandwidth.

The allowed A-MPR for the maximum output power for ProSe physical signal PSSS and SSSS shall be as be as specified in subclause 6.2.4 for PUSCH QPSK modulation for the corresponding transmission bandwidth.

When UE is configured for simultaneous E-UTRA ProSe sidelink and E-UTRA uplink transmissions for inter-band E-UTRA ProSe / E-UTRA bands specified in Table 5.5D-2, the requirements in subclause 6.2.4D apply for ProSe transmission and the requirements in subclause 6.2.4 apply for uplink transmission.

### 6.2.4E	UE maximum output power with additional requirements for category M1 and M2 UE

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction (A-MPR) is allowed for the output power as specified in Table 6.2.2E-1 and Table 6.2.4E-2. Unless stated otherwise, an A-MPR of 0 dB shall be used.

For UE Power Class 3 and 5 the specific requirements and identified subclauses are specified in Table 6.2.4E-1 and Table 6.2.4E-2 along with the allowed A-MPR values that may be used to meet these requirements. The allowed A-MPR values specified below in Table 6.2.4E-1 and Table 6.2.4E-2 and from 6.2.4-2 to 6.2.4-15 are in addition to the allowed MPR requirements specified in subclause 6.2.3E.

Table 6.2.4E-1: Additional Maximum Power Reduction (A-MPR) for category M1 UE

| Network Signalling value | Requirements (subclause) | E-UTRA Band | Resources Blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- |
| NS_01 | 6.6.2.1.1 | Table 5.5-1 | Table 5.6-1 | N/A |
| NS_03 | 6.6.2.2.1 | 2, 4 | Table 5.6-1 | N/A |
| NS_04 | 6.6.2.2.2 | 41 | Table 6.2.4E-3 |  |
| NS_05 | 6.6.3.3.1 | 1 | Table 5.6-1 | N/A |
| NS_06 | 6.6.2.2.3 | 12, 13, 14, 85 | Table 5.6-1 | N/A |
| NS_07 | 6.6.2.2.3 6.6.3.3.2 | 13 | Table 6.2.4E-4 |  |
| NS_08 | 6.6.3.3.3 | 19 | Table 5.6-1 | N/A |
| NS_09 | 6.6.3.3.4 | 21, 74 | Table 5.6-1 | N/A |
| NS_10 |  | 20 | Table 5.6-1 | N/A |
| NS_12 | 6.6.3.3.5 | 26 | Table 6.2.4E-5 |  |
| NS_13 | 6.6.3.3.6 | 26 | Table 5.6-1 | N/A |
| NS_14 | 6.6.3.3.7 | 26 | Table 5.6-1 | N/A |
| NS_15 | 6.6.3.3.8 | 26 | Table 6.2.4-9 |  |
| NS_16 | 6.6.3.3.9 | 27 | Table 5.6-1 | N/A |
| NS_17 | 6.6.3.3.10 | 28 | Table 5.6-1 | N/A |
| NS_18 | 6.6.3.3.11 | 28 | Table 5.6-1 | N/A |
| NS_22 | 6.6.3.3.16 | 42, 43 | Table 5.6-1 | N/A |
| NS_23 | 6.6.3.3.17 | 42, 43 | Table 5.6-1 | N/A |
| NS_32 | - | - | - | - |
| NS_35 | 6.6.2.2.7 | 71 | Table 5.6-1 | N/A |
| NS_38 | 6.6.3.3.29 | 74 | Table 5.6-1 | N/A |
| NS_39 | 6.6.3.3.30 | 74 | Table 5.6-1 | N/A |

Table 6.2.4E-2: Additional Maximum Power Reduction (A-MPR) for category M2 UE

| Network Signalling value | Requirements (subclause) | E-UTRA Band | Narrowband bandwidth | Resources Blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 | 6.6.2.1.1 | Table 5.5-1 | 1.4, 3, 5 | Table 5.6-1 | N/A |
| NS_03 | 6.6.2.2.1 | 2, 4 | 3,5 | Table 6.2.4-1 |  |
| NS_04 | 6.6.2.2.2 | 41 | 5 | Table 6.2.4-1 |  |
| NS_05 | 6.6.3.3.1 | 1 | 1.4, 3, 5 | Table 6.2.4-18E |  |
| NS_06 | 6.6.2.2.3 | 12, 13, 14, 85 | 1.4, 3, 5 | Table 5.6-1 | N/A |
| NS_07 | 6.6.2.2.3 6.6.3.3.2 | 13 |  | [TBD] |  |
| NS_08 | 6.6.3.3.3 | 19 | 1.4, 3, 5 | Table 5.6-1 | N/A |
| NS_09 | 6.6.3.3.4 | 21 | 1.4, 3, 5 | Table 5.6-1 | N/A |
| NS_10 |  | 20 | 1.4, 3, 5 | Table 5.6-1 | N/A |
| NS_12 | 6.6.3.3.5 | 26 | 1.4, 3, 5 | Table 6.2.4-1 |  |
| NS_13 | 6.6.3.3.6 | 26 | 5 | Table 6.2.4-1 |  |
| NS_14 | 6.6.3.3.7 | 26 | N/A | Table 5.6-1 | N/A |
| NS_15 | 6.6.3.3.8 | 26 | 1.4, 3, 5 | Table 5.6-1 |  |
| NS_16 | 6.6.3.3.9 | 27 | 3,5 | Table 6.2.4-1 |  |
| NS_17 | 6.6.3.3.10 | 28 | 5 | Table 6.2.4-1 |  |
| NS_18 | 6.6.3.3.11 | 28 | 5 | Table 6.2.4-1 |  |
| NS_22 | 6.6.3.3.16 | 42, 43 | 1.4, 3, 5 | Table 5.6-1 | N/A |
| NS_23 | 6.6.3.3.17 | 42, 43 | 1.4, 3, 5 | Table 5.6-1 | N/A |
| NS_32 | - | - |  | - | - |
| NS_35 | 6.6.2.2.7 | 71 | 1.4, 3, 5 | Table 5.6-1 | N/A |

Table 6.2.4E-3: A-MPR for "NS_04" for Cat-M1

| Channel bandwidth [Hz] | Parameters | Region |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 5 | Fc [MHz] | ≤ 2500.5 |  |  |  |  |
|  | (NBindex, RBstart ) | (0, 0-5) |  |  | (1, 0-1) |  |
|  | LCRB [RBs] | > 0 |  |  |  |  |
|  | A-MPR [dB] | ≤ 2 |  |  |  |  |
| 10 | Fc [MHz] | ≤ 2504 |  |  |  |  |
|  | (NBindex, RBstart ) | (0, 0-5) |  |  | (1, 0-1) |  |
|  | LCRB [RBs] | >0 |  |  |  |  |
|  | A-MPR [dB] | ≤ 3 |  |  |  |  |
| 15 | Fc [MHz] | ≤ 2510.8 |  |  |  |  |
|  | (NBindex, RBstart ) | (0, 0-5) |  | (1, 0-5) |  |  |
|  | LCRB [RBs] | >0 |  |  |  |  |
|  | A-MPR [dB] | ≤ 3 |  |  |  |  |
| 20 | Fc [MHz] | ≤ 2517.5 |  |  |  |  |
|  | (NBindex, RBstart ) | (0, 0-5) | (1, 0-5) |  | (2, 0-5) | (3, 0-2) |
|  | LCRB [RBs] | >0 |  |  |  |  |
|  | A-MPR [dB] | ≤ 3 |  |  |  |  |
| NOTE 1: RBstart indicates the lowest RB index of transmitted resource blocks NOTE 2: LCRB is the length of a contiguous resource block allocation NOTE 3: For intra-subframe frequency hopping which intersects regions, notes 1 and 2 apply on a per slot basis NOTE 4: For intra-subframe frequency hopping which intersects regions, the larger A-MPR value may be applied for both slots in the subframe NOTE 5: For CAT-M1 device, the NB index is the starting index allocated from DCI[6], the RBstart is indexed within the NB allocated to cat-M1 device. |  |  |  |  |  |  |

Table 6.2.4E-4: A-MPR for "NS_07" for Cat-M1

| BW [MHz] | 5 |  |  |  | 10 |  |
| --- | --- | --- | --- | --- | --- | --- |
| (NBindex,RBstart) | (0,<6) | (0,<6) | (3,<6) | (3,<6) | (0,<6) | (7,<6) |
| LCRB | >4 and <7 | >1 and ≤4 | >4 and <7 | >1 and ≤4 | >2and <7 | >2and <7 |
| AMPR [dB] | 2 | 1 | 2 | 1 | 1 | 1 |
| NOTE 1: NBindex is the narrowband index that is defined in 6.2.7 in [4]. The resource block assignment is defined within the narrowband as defined in 5.3.3.1.12 and 5.3.3.1.13 in [5]. |  |  |  |  |  |  |

Table 6.2.4E-5: A-MPR for "NS_12" for Cat-M1

| Channel bandwidth [MHz] | Parameters | Region |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 1.4 | (NBindex, RBstart ) | (0,0) |  | (0,1-2) |  |
|  | LCRB [RBs] | ≤3 | ≥4 | ≥4 |  |
|  | A-MPR [dB] | ≤3 | ≤6 | ≤3 |  |
| 3 | (NBindex, RBstart ) | (0,0-2) |  |  |  |
|  | LCRB [RBs] | >0 |  |  |  |
|  | A-MPR [dB] | ≤4 |  |  |  |
| 5 | (NBindex, RBstart ) | (0, 0-5) |  |  |  |
|  | LCRB [RBs] | >0 |  |  |  |
|  | A-MPR [dB] | ≤5 |  |  |  |
| 10 | (NBindex, RBstart ) | (0, 0-5) | (1, 0-5) |  | (2,0-2) |
|  | LCRB [RBs] | >0 |  |  |  |
|  | A-MPR [dB] | ≤4 |  |  |  |
| 15 | (NBindex, RBstart ) | (0-5,0-5) |  |  |  |
|  | LCRB [RBs] | >0 |  |  |  |
|  | A-MPR [dB] | ≤4 |  |  |  |

For subPRB allocation, the allowed A-MPR values specified below in Table 6.2.4E-6 and Table 6.2.4E-7 for category M1 UE and category M2 UE respectively in addition to the allowed MPR requirements specified in subclause 6.2.3E.

Table 6.2.4E-6: Additional Maximum Power Reduction (A-MPR) for category M1 UE for subPRB allocation

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |
| --- | --- | --- | --- |
| NS_01 | 6.6.2.1.1 | Table 5.5-1 | N/A |
| NS_03 | 6.6.2.2.1 | 2, 4 | Table 6.2.4E-8 |
| NS_04 | 6.6.2.2.2 | 41 | Table 6.2.4E-9 |
| NS_05 | 6.6.3.3.1 | 1 | [N/A] |
| NS_06 | 6.6.2.2.3 | 12, 13, 14 | Table 6.2.4E-13 |
| NS_07 | 6.6.2.2.3 6.6.3.3.2 | 13 | Table 6.2.4E-23 |
| NS_08 | 6.6.3.3.3 | 19 | [N/A] |
| NS_09 | 6.6.3.3.4 | 21, 74 | [N/A] |
| NS_10 |  | 20 | [N/A] |
| NS_12 | 6.6.3.3.5 | 26 | Table 6.2.4E-14 |
| NS_13 | 6.6.3.3.6 | 26 | [N/A] |
| NS_14 | 6.6.3.3.7 | 26 | [N/A] |
| NS_15 | 6.6.3.3.8 | 26 | [N/A] |
| NS_16 | 6.6.3.3.9 | 27 | [N/A] |
| NS_17 | 6.6.3.3.10 | 28 | [N/A] |
| NS_18 | 6.6.3.3.11 | 28 | [N/A] |
| NS_22 | 6.6.3.3.16 | 42, 43 | [N/A] |
| NS_23 | 6.6.3.3.17 | 42, 43 | [N/A] |
| NS_32 | - | - | - |
| NS_35 | 6.6.2.2.7 | 71 | Table 6.2.4E-15 |
| NS_38 | 6.6.3.3.29 | 74 | Table 6.2.4E-16 |
| NS_39 | 6.6.3.3.30 | 74 | [N/A] |

Table 6.2.4E-7: Additional Maximum Power Reduction (A-MPR) for category M2 UE for subPRB allocation

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |
| --- | --- | --- | --- |
| NS_01 | 6.6.2.1.1 | Table 5.5-1 | N/A |
| NS_03 | 6.6.2.2.1 | 2, 4 | Table 6.2.4E-10 |
| NS_04 | 6.6.2.2.2 | 41 | Table 6.2.4E-11 |
| NS_05 | 6.6.3.3.1 | 1 | Table 6.2.4E-17 |
| NS_06 | 6.6.2.2.3 | 12, 13,14 | Table 6.2.4E-18 |
| NS_07 | 6.6.2.2.3 6.6.3.3.2 | 13 | Table 6.2.4E-12 |
| NS_08 | 6.6.3.3.3 | 19 | N/A |
| NS_09 | 6.6.3.3.4 | 21 | N/A |
| NS_10 |  | 20 | N/A |
| NS_12 | 6.6.3.3.5 | 26 | Table 6.2.4E-19 |
| NS_13 | 6.6.3.3.6 | 26 | Table 6.2.4E-20 |
| NS_14 | 6.6.3.3.7 | 26 | N/A |
| NS_15 | 6.6.3.3.8 | 26 | Table 6.2.4E-21 |
| NS_16 | 6.6.3.3.9 | 27 | Table 6.2.4E-22 |
| NS_17 | 6.6.3.3.10 | 28 | N/A |
| NS_18 | 6.6.3.3.11 | 28 | N/A |
| NS_22 | 6.6.3.3.16 | 42, 43 | [N/A] |
| NS_23 | 6.6.3.3.17 | 42, 43 | [N/A] |
| NS_32 | - | - | - |
| NS_35 | 6.6.2.2.7 | 71 | N/A |
| NS_38 | 6.6.3.3.29 | 74 | Table 6.2.4E-24 |
| NS_39 | 6.6.3.3.30 | 74 | N/A |

Table 6.2.4E-8: A-MPR for "NS_03" for Cat-M1 with sub-PRB allocation

| BW [MHz] | 5 MHz |  |  |  |
| --- | --- | --- | --- | --- |
| (NBindex,SCstart) | (0, ≤9) | (0, ≥63) | (0, ≤9) | (0, ≥63) |
| Lcsc | 2 | 2 | 3,6 | 3,6 |
| AMPR [dB] | ≤[0.5] | ≤[0.5] | ≤[1] | ≤[1] |
| NOTE 1: NBindex is the narrowband index that is defined in 6.2.7 in [4]. The resource block assignment is defined within the narrowband as defined in 5.3.3.1.12 and 5.3.3.1.13 in [5]. NOTE 2: Lcsc is the length of the continuous subcarrier, SCstart is the subcarrier offset relative to the first subcarrier of the first PRB of NB indicated with NBindex. |  |  |  |  |

Table 6.2.4E-9: A-MPR for "NS_04" for Cat-M1 with sub-PRB allocation

| BW [MHz] | 5 MHz |  |
| --- | --- | --- |
| (NBindex,SCstart) | (0, ≤9) | (0, ≤9) |
| Lcsc | 2 | 3,6 |
| AMPR [dB] | ≤[1] | ≤[1.5] |
| NOTE 1: NBindex is the narrowband index that is defined in 6.2.7 in [4]. The resource block assignment is defined within the narrowband as defined in 5.3.3.1.12 and 5.3.3.1.13 in [5]. NOTE 2: Lcsc is the length of the continuous subcarrier, SCstart is the subcarrier offset relative to the first subcarrier of the first PRB of NB indicated with NBindex. |  |  |

Table 6.2.4E-10: A-MPR for "NS_03" for Cat-M2 with sub-PRB allocation

| BW [MHz] | 5 MHz |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| (NBindex,SCstart) | (0,≤72) | (0, ≥216) |  |  |  |  |
| Lcsc | 3,6 | 3,6 |  |  |  |  |
| AMPR [dB] | ≤[1] | ≤[1] |  |  |  |  |
|  | 10 MHz |  |  |  |  |  |
| (NBindex,SCstart) | (0,≤60) | (1,≤24) | (3, ≥264) | (4, ≥228) |  |  |
| Lcsc | 3,6 | 3,6 | 3,6 | 3,6 |  |  |
| AMPR [dB] | ≤[1] | ≤[1] | ≤[1] | ≤[1] |  |  |
| NOTE 1: NBindex is the narrowband index that is defined in 6.2.7 in [4]. The resource block assignment is defined within the narrowband as defined in 5.3.3.1.12 and 5.3.3.1.13 in [5]. NOTE 2: Lcsc is the length of the continuous subcarrier, SCstart is the subcarrier offset relative to the first subcarrier of the first PRB of NB indicated with NBindex. |  |  |  |  |  |  |

Table 6.2.4E-11: A-MPR for "NS_04" for Cat-M2 with sub-PRB allocation

| BW [MHz] | 5MHz |  |
| --- | --- | --- |
| (NBindex,SCstart) | (0,≤72) | (0, ≥216) |
| Lcsc | ≥2 |  |
| AMPR [dB] | ≤[2] |  |

Table 6.2.4E-12: A-MPR for "NS_07" for Cat-M2 with sub-PRB allocation

| BW [MHz] | 10 MHz |  |  |
| --- | --- | --- | --- |
| (NBindex,SCstart) | (0, <48) | (0,48-84) | (0, ≥240) |
| Lcsc | ≥2 | ≥2 | ≥2 |
| AMPR [dB] | ≤[12] | ≤[6] | ≤[14.5] |
| (NBindex,SCstart) | (1, <24) | (1,24-72) | (1, ≥264) |
| Lcsc | ≥2 | ≥2 | ≥2 |
| AMPR [dB] | ≤[12] | ≤[6] | ≤[14.5] |
| (NBindex,SCstart) | (2, <60) |  | (2, ≥252) |
| Lcsc | ≥2 |  | ≥2 |
| AMPR [dB] | ≤[6] |  | ≤[1.5] |
| (NBindex,SCstart) | (3, <48) |  | (3, ≥216) |
| Lcsc | ≥2 |  | ≥2 |
| AMPR [dB] | ≤[6] |  | ≤[1.5] |
| (NBindex,SCstart) | (4, <36) |  | (4, ≥192) |
| Lcsc | ≥2 |  | ≥2 |
| AMPR [dB] | ≤[3] |  | ≤[1.5] |

Table 6.2.4E-13: A-MPR for "NS_06" for Cat-M1 with sub-PRB allocation

| BW [MHz] | 1.4 | 3 | 5 |
| --- | --- | --- | --- |
| (NBindex,SCstart) | (0, < 15) and (0, > 54) | ([0], < 15) | ([0], < 15) |
| Lcsc | [2] | [2] | [2] |
| AMPR [dB] | ≤[0.5] | ≤[0.5] | ≤[0.5] |
| NOTE 1: NBindex is the narrowband index that is defined in 6.2.7 in [4]. The resource block assignment is defined within the narrowband as defined in 5.3.3.1.12 and 5.3.3.1.13 in [5]. NOTE2: Lcsc is the length of the continuous subcarrier, SCstart is the subcarrier offset relative to the first subcarrier of the first PRB of NB indicated with NBindex. |  |  |  |

Table 6.2.4E-14: A-MPR for "NS_12" for Cat-M1 with sub-PRB allocation

| Channel bandwidth [MHz] | Parameters | Region |  |  |
| --- | --- | --- | --- | --- |
| 1.4 | (NBindex,SCstart ) | ([0], ≤ [3]) | ([0], ≤ [3]) | ([0], ≤ [3]) |
|  | Lcsc | [2] | [3] | [6] |
|  | A-MPR [dB] | ≤ [5] | ≤ [5] | ≤ [5] |
| 3 | (NBindex,SCstart) | ([0], ≤ [3]) | ([0], ≤ [3]) | ([0], ≤ [3]) |
|  | Lcsc | [2] | [3] | [6] |
|  | A-MPR [dB] | ≤ [5] | ≤ [5] | ≤ [5] |
| 5 | (NBindex,SCstart) | ([0], ≤ [3]) | ([0], ≤ [3]) | ([0], ≤ [3]) |
|  | Lcsc | [2] | [3] | [6] |
|  | A-MPR [dB] | ≤ [5] | ≤ [5] | ≤ [5] |

Table 6.2.4E-15: A-MPR for "NS_35" for Cat-M1 with sub-PRB allocation

| BW [MHz] | 5 |
| --- | --- |
| (NBindex,SCstart) | ([0], [0]) |
| Lcsc | [2] |
| AMPR [dB] | ≤ [0.5] |
| NOTE 1: NBindex is the narrowband index that is defined in 6.2.7 in [4]. The resource block assignment is defined within the narrowband as defined in 5.3.3.1.12 and 5.3.3.1.13 in [5]. NOTE2: Lcsc is the length of the continuous subcarrier, SCstart is the subcarrier offset relative to the first subcarrier of the first PRB of NB indicated with NBindex. |  |

Table 6.2.4E-16: A-MPR for "NS_38" for Cat-M1 with sub-PRB allocation for E-UTRA lowest channel edge > 1427 MHz and ≤ 1447 MHz

| Channel bandwidth [MHz] | Parameters | Region |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1.4 | (NBindex,SCstart) | ([0], < 27) | ([0], > 51) |  |  |  |  |
|  | Lcsc | 2,3,6 | [3] |  |  |  |  |
|  | A-MPR [dB] | ≤ [7] | ≤ [1] |  |  |  |  |
| 3 | (NBindex,SCstart) | ([0], < 21) | ([0], > 60) |  |  |  |  |
|  | Lcsc | 2,3,6 | 2,3,6 |  |  |  |  |
|  | A-MPR [dB] | ≤ [7] | ≤ [1] |  |  |  |  |
| 5 | (NBindex,SCstart) | ([0], < 21) | ([0], > 60) |  |  |  |  |
|  | Lcsc | [2] | [2] |  |  |  |  |
|  | A-MPR [dB] | ≤ [7] | ≤ [1] |  |  |  |  |
| 10 | (NBindex,SCstart) | (0, < 15) |  |  |  |  |  |
|  | Lcsc | 2,3,6 |  |  |  |  |  |
|  | A-MPR [dB] | ≤ [7] |  |  |  |  |  |
| 15 | (NBindex,SCstart) | (0, < 12) |  |  |  |  |  |
|  | Lcsc | 2,3,6 |  |  |  |  |  |
|  | A-MPR [dB] | ≤ [7] |  |  |  |  |  |

Table 6.2.4E-17: A-MPR for "NS_05" for Cat-M2 with sub-PRB allocation

| BW [MHz] | 5 |  | 10 |  | 15 |  | 20 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (WBindex,SCstart) | (0,≤36) | (0, ≤6) | (0,≤30) | (0,≤12) | (0,≤24) | (0,≤12) | (0,0) | (0,0) |
| Lcsc | 2 | 6 | 2 | 6 | 2 | 6 | 2 | 3 |
| AMPR [dB] | ≤[1.5] | ≤ [0.5] | ≤[1.5] | ≤[0.5] | ≤[1.5] | ≤[0.5] | ≤[1.5] | ≤[0.5] |
| NOTE 1: WBindex is the wideband index that is defined in [4]. NOTE2: Lcsc is the length of the continuous subcarrier, SCstart is the subcarrier offset relative to the first subcarrier of the first PRB indicated with WBindex |  |  |  |  |  |  |  |  |

Table 6.2.4E-18: A-MPR for "NS_06" for Cat-M2 with sub-PRB allocation

| BW [MHz] | 5 MHz |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| (NBindex,SCstart) | (0,≤72) | (0, ≥216) |  |  |  |  |
| Lcsc | 3,6 | 3,6 |  |  |  |  |
| AMPR [dB] | ≤[1] | ≤[1] |  |  |  |  |
|  | 10 MHz |  |  |  |  |  |
| (NBindex,SCstart) | (0,≤60) | (1,≤24) | (3, ≥264) | (4, ≥228) |  |  |
| Lcsc | 3,6 | 3,6 | 3,6 | 3,6 |  |  |
| AMPR [dB] | ≤[1] | ≤[1] | ≤[1] | ≤[1] |  |  |
| NOTE 1: NBindex is the narrowband index that is defined in 6.2.7 in [4]. The resource block assignment is defined within the narrowband as defined in 5.3.3.1.12 and 5.3.3.1.13 in [5]. NOTE 2: Lcsc is the length of the continuous subcarrier, SCstart is the subcarrier offset relative to the first subcarrier of the first PRB of NB indicated with NBindex. |  |  |  |  |  |  |

Table 6.2.4E-19: A-MPR for "NS_12" for Cat-M2 with sub-PRB allocation for E-UTRA lower channel edge >= 814.2 MHz and ≤ 829.2 MHz

| BW [MHz] | 5 MHz |
| --- | --- |
| (WBindex,SCstart) | (0, <84) |
| Lcsc | ≥2 |
| AMPR [dB] | ≤[5] |
| BW [MHz] | 10 MHz |
| (WBindex,SCstart) | (0, <72) |
| Lcsc | ≥2 |
| AMPR [dB] | ≤[5] |
| BW [MHz] | 15 MHz |
| (WBindex,SCstart) | (0, <66) |
| Lcsc | ≥2 |
| AMPR [dB] | ≤[5] |
| NOTE 1: NBindex is the narrowband index that is defined in 6.2.7 in [4]. The resource block assignment is defined within the narrowband as defined in 5.3.3.1.12 and 5.3.3.1.13 in [5]. NOTE 2: WBindex is the wideband index that is defined in [4]. NOTE 3: Lcsc is the length of the continuous subcarrier, SCstart is the subcarrier offset relative to the first subcarrier of the first PRB indicated with NBindex or WBindex |  |

Table 6.2.4E-20: A-MPR for "NS_13" for Cat-M2 with sub-PRB allocation for E-UTRA lower channel edge >= 819 MHz and ≤ 824 MHz

| BW [MHz] | 5 |  |
| --- | --- | --- |
| (WBindex,SCstart) | (0, <30) | (0, <30) |
| Lcsc | 2 | 3,6 |
| AMPR [dB] | ≤[3] | ≤[5] |
| NOTE 1: WBindex is the wideband index that is defined in [4]. NOTE 2: Lcsc is the length of the continuous subcarrier, SCstart is the subcarrier offset relative to the first subcarrier of the first PRB indicated with WBindex |  |  |

Table 6.2.4E-21: A-MPR for "NS_15" for Cat-M2 with sub-PRB allocation for E-UTRA highest channel edge > 834 MHz and ≤ 849 MHz

| BW [MHz] | 5 MHz |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| (WBindex,SCstart) | (0, <54) | (0, <54) | (0, <54) | (0, 200-234) | (0, >234) | (0, >234) |
| Lcsc | 2 | 3 | 6 | ≥2 | 2,3 | 6 |
| AMPR [dB] | ≤[10.5] | ≤[8.5] | ≤[5.5] | ≤[3.5] | ≤[10] | ≤[9] |
| BW [MHz] | 10 MHz |  |  |  |  |  |
| (WBindex,SCstart) | (1, <48) | (1, <48) | (1, <48) | (1, 200-234) | (1, >234) | (1, >234) |
| Lcsc | 2 | 3 | 6 | ≥2 | 2,3 | 6 |
| AMPR [dB] | ≤[10.5] | ≤[8.5] | ≤[5.5] | ≤[3.5] | ≤[10] | ≤[9] |
| BW [MHz] | 15 MHz |  |  |  |  |  |
| (WBindex,SCstart) | (2, <48) | (2, <48) | (2, <48) | (2, 200-234) | (2, >234) | (2, >234) |
| Lcsc | 2 | 3 | 6 | ≥2 | 2,3 | 6 |
| AMPR [dB] | ≤[10.5] | ≤[8.5] | ≤[5.5] | ≤[3.5] | ≤[10] | ≤[9] |
| NOTE 1: WBindex is the wideband index that is defined in [4]. NOTE 2: Lcsc is the length of the continuous subcarrier, SCstart is the subcarrier offset relative to the first subcarrier of the first PRB indicated with WBindex |  |  |  |  |  |  |

Table 6.2.4E-22: A-MPR for "NS_16" for Cat-M2 with sub-PRB allocation for E-UTRA lowest channel edge > 807 MHz and ≤ 812 MHz

| BW [MHz] | 5 MHz |
| --- | --- |
| (WBindex,SCstart) | (0, <6) |
| Lcsc | 2 |
| AMPR [dB] | ≤[7] |
| NOTE 1: WBindex is the wideband index that is defined in [4]. NOTE 2: Lcsc is the length of the continuous subcarrier, SCstart is the subcarrier offset relative to the first subcarrier of the first PRB indicated with WBindex |  |

Table 6.2.4E-23: A-MPR for “NS_07” for Cat-M1 with sub-PRB allocation

| BW [MHz] | 10 |
| --- | --- |
| (NBindex,SCstart) | (0, < 9) |
| Lcsc | 2 |
| AMPR [dB] | ≤ 0.5 |
| NOTE 1: NBindex is the narrowband index that is defined in 6.2.7 in [4]. The resource block assignment is defined within the narrowband as defined in 5.3.3.1.12 and 5.3.3.1.13 in [5]. NOTE 2: Lcsc is the length of the continuous subcarrier, SCstart is the subcarrier offset relative to the first subcarrier of the first PRB of NB indicated with NBindex. |  |

Table 6.2.4E-24: A-MPR for “NS_38” for Cat-M2 with sub-PRB allocation for E-UTRA lowest channel edge > 1427 MHz and ≤ 1447 MHz

| Channel bandwidth [MHz] | Parameters | Region |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 3 | (NBindex,SCstart ) | (0, 0-27) | (0, 30-42) | (0, 45-51) | (0, 96-120) | (0, 123-168) |
|  | Lcsc | 2,3 ,6 | 2,3 ,6 | 2,3 ,6 | 2,3 ,6 | 2,3 ,6 |
|  | A-MPR [dB] | ≤ [16] | ≤ [13] | ≤ [10] | ≤ [7] | ≤ [3] |
| 5 | (WBindex,SCstart ) | (0, < 96) | (0, > 219) |  |  |  |
|  | Lcsc | 2,3,6 | 2,3,6 |  |  |  |
|  | A-MPR [dB] | ≤ [7] | ≤ [1] |  |  |  |
| 10 | (WBindex,SCstart ) | (0, < 93) | (0, > 219) |  |  |  |
|  | Lcsc | 2,3,6 | 2,3,6 |  |  |  |
|  | A-MPR [dB] | ≤ [7] | ≤ [1] |  |  |  |
| 15 | (WBindex,SCstart ) | (0, < 87) | (0, > 234) |  |  |  |
|  | Lcsc | 2,3,6 | 2,3,6 |  |  |  |
|  | A-MPR [dB] | ≤ [7] | ≤ [1] |  |  |  |
| 20 | (WBindex,SCstart ) | (0, < 81) | (0, > 234) |  |  |  |
|  | Lcsc | 2,3,6 | 2,3,6 |  |  |  |
|  | A-MPR [dB] | ≤ [7] | ≤ [1] |  |  |  |

No other A-MPR requirement than those specified in table 6.2.4E-1, table 6.2.4E-2, table 6.2.4E-6 and table 6.2.4E-7 applies to category M1 and M2 UE.

### 6.2.4F	UE maximum output power with additional requirements for category NB1 and NB2 UE

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction (A-MPR) is allowed for the output power are specified. For the agreed E-UTRA bands for category NB1 and NB2 UE an A-MPR of 0 dB shall be allowed unless specified otherwise.

For UE Power Class 3 and 5 the specific requirements and identified subclauses are specified in Table 6.2.4F-1 along with the allowed A-MPR values that may be used to meet these requirements. The allowed A-MPR values specified below in Table 6.2.4F-1 are in addition to the allowed MPR requirements specified in subclause 6.2.3F-1.

Table 6.2.4F-1: Additional Maximum Power Reduction (A-MPR) for category NB1 and NB2 UE

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |
| --- | --- | --- | --- |
| NS_01 | 6.6.2F.1 | Operating bands defined in 5.5F | N/A |
| NS_02 | 6.6.2F.2.1 | [1, 2, 3, 5, 8, 11, 12, 13, 17, 18, 19, 20, 21, 25, 26, 28, 66, 70 and 85] | [N/A] |
| NS_03 | 6.6.2F.2.2 | [1, 2, 3, 5, 8, 11, 12, 13, 17, 18, 19, 20, 21, 25, 26, 28, 66, 70 and 85] | [N/A] |
| NS_04 | 5.5F | 2, 4, 5, 12, 13, 17, 25, 26, 66, 71, 85 | N/A |

### 6.2.4G	UE maximum output power with additional requirements for V2X Communication

For QPSK the MPR requirements specified in subclause 6.2.3G does not apply, i.e. MPR = 0dB. For 16QAM and 64 QAM, the applied maximum output power reduction is obtained by taking the maximum value of MPR requirements specified in subclause 6.2.3G and A-MPR requirements specified in subclause 6.2.4G.

When UE is configured for E-UTRA V2X sidelink transmissions non-concurrent with E-UTRA uplink transmissions for E-UTRA V2X operating bands specified in Table 5.5G-1, the maximum output power reduction specified as

A-MPR = CEIL {MA, 0.5}

Where MA is defined as follows

MA = A-MPRBase + Gpost connector * A-MPRStep

CEIL{MA, 0.5} means rounding upwards to closest 0.5dB.

A-MPRBase  and A-MPRStep  are specified in Tables 6.2.4G-1, 6.2.4G-2, 6.2.4G-3 is allowed when network signalling value is provided. The supported post antenna connector gain Gpost connector is declared by the UE following the principle described in annex I.

NOTE: the A-MPRstep is the increase in A-MPR allowance to allow UE to meet tighter conducted A-SE and A-SEM requirements with higher value of declared Gpost connector. A-MPRBase is the default A-MPR value when no Gpost connector is declared.  A-MPRBase  and A-MPRstep vary depending on channel frequency and RB allocation. For channel frequencies and RB allocations that are close to the frequency range 5815-5855MHz, those value are much higher due to stringent emission requirement in this range.

Table 6.2.4G-1: Additional Maximum Power Reduction (A-MPR) for power class 3 V2X UE

| Network Signalling value | Requirements (subclause) | E-UTRA Band | Channel bandwidth (MHz) | A-MPR (dB) |
| --- | --- | --- | --- | --- |
| NS_33 | 6.6.2.2.4 (A-SEM) 6.6.3.2 (A-SE) 6.6.2G | 47 | 10 | Table 6.2.4G-2 |

Table 6.2.4G-2: A-MPR for NS_33

| Resource pool | Carrier frequency(MHz) | Resources Blocks (NRB) | Start Resource Block | A-MPRBase | A-MPRStep |
| --- | --- | --- | --- | --- | --- |
| Adjacent | 5860 | ≤ 6 | 0 | 20 | 0.86 |
|  |  |  | 5,6 | 8 | 0.64 |
|  |  |  | 10,12 | 6 | 0.50 |
|  |  |  | ≥ 15 | 5 | 0.93 |
|  |  | >6 and ≤ 10 | 0 | 15.5 | 0.86 |
|  |  |  | 5, 6 | 8 | 0.64 |
|  |  |  | 10, 12 | 6 | 0.50 |
|  |  |  | ≥ 15 | 5 | 0.93 |
|  |  | >10 and ≤22 | 0 | 15 | 0.71 |
|  |  |  | 5, 6 | 11.5 | 0.64 |
|  |  |  | 10, 12 | 10 | 0.57 |
|  |  |  | 15, 18 | 6 | 0.57 |
|  |  |  | 20, 24, 25 | 5 | 0.57 |
|  |  |  | ≥ 30 | 4.5 | 0.64 |
|  |  | >22 | 0, 5, 6 | 12.5 | 0.71 |
|  |  |  | 10, 12 | 10.5 | 0.57 |
|  |  |  | 15, 18 | 9.5 | 0.64 |
|  |  |  | 20, 24, 25 | 6.5 | 0.71 |
|  | 5870, 5910, 5920 | <20 | ≥ 0 | 3 | 0.64 |
|  |  | ≥ 20 and ≤45 |  | 1.5 | 0.43 |
|  |  | >45 |  | 2.5 | 0.36 |
|  | 5880, 5890, 5900 | <10 | ≥ 0 | 3 | 0.43 |
|  |  | ≥ 10 and ≤38 |  | 1.5 | 0.50 |
|  |  | >38 |  | 2 | 0.43 |
| Non-Adjacent | 5860 | ≤ 5 | ≥ 0 | 13.5 | 1 |
|  |  | >5 |  | 11.5 | 1 |
|  | 5870, 5910, 5920 | ≤ 5 | ≥ 0 | 5 | 1 |
|  |  | >5 and ≤42 |  | 3 | 1 |
|  |  | >42 |  | 4.5 | 1 |
|  | 5880, 5890, 5900 | ≤18 | ≥ 0 | 3.5 | 1 |
|  |  | >18 and ≤42 |  | 2.5 | 1 |
|  |  | >42 |  | 3 | 1 |

The allowed A-MPR for the maximum output power for V2X physical signal PSBCH and PSSS/SSSS shall be as be as specified in subclause 6.2.4 for the corresponding modulation and transmission bandwidth.

When UE is configured for simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions for inter-band E-UTRA V2X / E-UTRA bands specified in Table 5.5G-2, the requirements in subclause 6.2.4G apply for V2X  PSSCH and PSCCH transmission. The allowed A-MPR requirements in subclause 6.2.4D apply for other V2X sidelink transmission (PSBCH/PSSS/SSSS). The A-MPR requirements in subclause 6.2.4 apply for uplink transmission.

When UE is configured for E-UTRA V2X sidelink transmissions non-concurrent with E-UTRA uplink transmissions for E-UTRA V2X operating bands specified in Table 5.5G-1, the allowed A-MPR for the maximum output power for V2X physical channels PSCCH and PSSCH shall be as specified in Table 6.2.4G-3 and 6.2.4G-4 for V2X UE  power class 2.

Table 6.2.4G-3: Additional Maximum Power Reduction (A-MPR) for power class 2 V2X UE

| Network Signalling value | Requirements (subclause) | E-UTRA Band | Channel bandwidth (MHz) | A-MPR (dB) |
| --- | --- | --- | --- | --- |
| NS_34 | 6.6.2.2.4 (A-SEM) 6.6.3.2 (A-SE) 6.6.2G | 47 | 10 | Table 6.2.4G-4 |

| Table 6.2.4G-4: A-MPR for NS_34 Resource pool | Carrier frequency(MHz) | Resources Blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- |
| Adjacent | 5860 | <20 | 15 |
|  |  | >20 and <30 | 13 |
|  |  | >30 | 12 |
|  | 5870, 5900, 5880, 5890, 5900 | <15 or >40 | 1 |
| Non-Adjacent | 5860 | - | 12.5 |
|  | 5910, 5920 | <10 | 5.5 |
|  |  | >10 and <20 | 3 |
|  | 5870, 5880, 5890, 5900 | <10 | 5.5 |

### 6.2.5	Configured transmitted power

The UE is allowed to set its configured maximum output power PCMAX,c for serving cell c. The configured maximum output power PCMAX,c is set within the following bounds:

PCMAX_L,c ≤  PCMAX,c  ≤  PCMAX_H,c with

PCMAX_L,c = MIN {PEMAX,c – TC,c,  (PPowerClass – ΔPPowerClass) – MAX(MPRc + A-MPRc + ΔTIB,c + TC,c + TProSe, P-MPRc)}

PCMAX_H,c = MIN {PEMAX,c,  PPowerClass – ΔPPowerClass}

where

-	PEMAX,c is the value given by IE P-Max for serving cell c, defined in [7];

-	PPowerClass is the maximum UE power specified in Table 6.2.2-1 without taking into account the tolerance specified in the Table 6.2.2-1;

-	ΔPPowerClass = 3 dB for a power class 2 capable UE operating in Band 41, when P-max of 23 dBm or lower is indicated or if the uplink/downlink configuration is 0 or 6 in the cell; otherwise, ΔPPowerClass = 0 dB

-	ΔPPowerClass = PPowerClass – PPowerClass_Default dB for UE operating in Band 14, when P-max of 23 dBm or lower is indicated in the cell; otherwise, ΔPPowerClass = 0 dB.

-	MPRc and A-MPRc for serving cell c are specified in subclause 6.2.3 and subclause 6.2.4, respectively;

-	TIB,c is the additional tolerance for serving cell c as specified in Table 6.2.5-2; TIB,c = 0 dB otherwise;

-	TC,c = 1.5 dB when NOTE 2 in Table 6.2.2-1 applies;

-	TC,c = 0 dB when NOTE 2 in Table 6.2.2-1 does not apply;

-	TProSe = 0.1 dB when the UE supports ProSe Direct Discovery and/or ProSe Direct Communication on the corresponding E-UTRA ProSe band; TProSe = 0 dB otherwise.

-	For a power class higher than default UE power class capable UE except for operating in Band 14 and Band 41, ΔPPowerClass = PPowerClass – PPowerClass_Default dB, when the band is a TDD band whose frame configuration is 0 or 6; or P-max is not indicated in the cell; or P-Max is provided and set to the maximum output power of the default power class or lower, otherwise, ΔPPowerClass = 0 dB.

P-MPRc is the allowed maximum output power reduction for

a)	ensuring compliance with applicable electromagnetic energy absorption requirements and addressing unwanted emissions / self desense requirements in case of simultaneous transmissions on multiple RAT(s) for scenarios not in scope of 3GPP RAN specifications;

b)	ensuring compliance with applicable electromagnetic energy absorption requirements in case of proximity detection is used to address such requirements that require a lower maximum output power.

The UE shall apply P-MPR c for serving cell c only for the above cases. For UE conducted conformance testing P-MPR shall be 0 dB

NOTE 1:	P-MPRc was introduced in the PCMAX,c equation such that the UE can report to the eNB the available maximum output transmit power. This information can be used by the eNB for scheduling decisions.

NOTE 2: P-MPRc may impact the maximum uplink performance for the selected UL transmission path.

TREF and Teval are specified in Table 6.2.5-0 for different TTI patterns. For each TREF, the PCMAX_L,c for serving cell c is evaluated perTeval and given by the minimum value taken over the transmission(s) within the Teval; the minimum PCMAX_ L,c over the one or more Teval is then applied for the entire TREF. PPowerClass shall not be exceeded by the UE during any period of time.

Table 6.2.5-0: PCMAX evaluation window for different TTI patterns

| TTI pattern | TREF | Teval | Teval with frequency hopping |
| --- | --- | --- | --- |
| Subframe | 1 subframe | 1 slot | 1 slot |
| Subslot | 2 OS | 2 OS | Min(Tno_hopping, 2OS) |
| Slot | 7 OS | 7 OS | Min(Tno_hopping, 7OS) |

The measured configured maximum output power PUMAX,c shall be within the following bounds:

PCMAX_L,c  –  MAX{TL,c, T(PCMAX_L,c)}  ≤  PUMAX,c  ≤  PCMAX_H,c  +  T(PCMAX_H,c).

where the tolerance T(PCMAX,c) for applicable values of PCMAX,c is specified in Table 6.2.5-1, and Table 6.2.5-1A. The tolerance TL,c is the absolute value of the lower tolerance for the applicable operating band as specified in Table 6.2.2-1.

Table 6.2.5-1: PCMAX tolerance

| PCMAX,c (dBm) | Tolerance T(PCMAX,c) (dB) |
| --- | --- |
| 23 < PCMAX,c ≤ 33 | 2.0 |
| 21 ≤ PCMAX,c ≤ 23 | 2.0 |
| 20 ≤ PCMAX,c < 21 | 2.5 |
| 19 ≤ PCMAX,c < 20 | 3.5 |
| 18 ≤ PCMAX,c < 19 | 4.0 |
| 13 ≤ PCMAX,c < 18 | 5.0 |
| 8 ≤ PCMAX,c < 13 | 6.0 |
| -40 ≤ PCMAX,c < 8 | 7.0 |

Table 6.2.5-1A: PCMAX tolerance for power class 5

| PCMAX,c (dBm) | Tolerance T(PCMAX,c) (dB) |
| --- | --- |
| PCMAX,c = 20 | 2.0 |
| 19 ≤ PCMAX,c < 20 | 3.5 |
| 18 ≤ PCMAX,c < 19 | 4.0 |
| 13 ≤ PCMAX,c < 18 | 5.0 |
| 8 ≤ PCMAX,c < 13 | 6.0 |
| -40 ≤ PCMAX,c < 8 | 7.0 |

Table 6.2.5-1B: PCMAX tolerance for power class 6 for category M1 and M2 UE

| PCMAX,c (dBm) | Tolerance T(PCMAX,c) (dB) |
| --- | --- |
| 13 ≤ PCMAX,c ≤ 14 | 2.5 |
| 12 ≤ PCMAX,c < 13 | 3.5 |
| 11 ≤ PCMAX,c < 12 | 4 |
| 8 ≤ PCMAX,c < 11 | 5 |
| 5 ≤ PCMAX,c < 8 | 6.0 |
| -40 ≤ PCMAX,c < 5 | 7.0 |

For the UE which supports inter-band carrier aggregation configurations with the uplink assigned to one or two E-UTRA bands the ΔTIB,c is defined for applicable bands in Table 6.2.5-2, Table 6.2.5-3 and Table 6.2.5-4 where unless otherwise stated, the same ΔTIB,c is applicable to E-UTRA band(s) part for CA configurations which have the same E-UTRA operating band combination.

Table 6.2.5-2: ΔTIB,c (two bands)

| E-UTRA operating band combination |  | E-UTRA Band |  | ΔTIB,c [dB] |  |
| --- | --- | --- | --- | --- | --- |
| CA_1-3, CA_1-1-3, CA_1-1-3-3, CA_1-3-3 |  | 1 |  | 0.3 |  |
|  |  | 3 |  | 0.3 |  |
| CA_1-5 |  | 1 |  | 0.3 |  |
|  |  | 5 |  | 0.3 |  |
| CA_1-7, CA_1-1-7, CA_1-7-7 |  | 1 |  | 0.5 |  |
|  |  | 7 |  | 0.6 |  |
| CA_1-8 |  | 1 |  | 0.3 |  |
|  |  | 8 |  | 0.3 |  |
| CA_1-11 |  | 1 |  | 0.3 |  |
|  |  | 11 |  | 0.3 |  |
| CA_1-18 |  | 1 |  | 0.3 |  |
|  |  | 18 |  | 0.3 |  |
| CA_1-19 |  | 1 |  | 0.3 |  |
|  |  | 19 |  | 0.3 |  |
| CA_1-20 |  | 1 |  | 0.3 |  |
|  |  | 20 |  | 0.3 |  |
| CA_1-21 |  | 1 |  | 0.3 |  |
|  |  | 21 |  | 0.3 |  |
| CA_1-26 |  | 1 |  | 0.3 |  |
|  |  | 26 |  | 0.3 |  |
| CA_1-28, CA_1-1-28 |  | 1 |  | 0.3 |  |
|  |  | 28 |  | 0.6 |  |
| CA_1-32 |  | 1 |  | 0.5 |  |
|  |  | 32 |  | N/A |  |
| CA_1-38 |  | 1 |  | 0.5 |  |
|  |  | 38 |  | 0.5 |  |
| CA_1-40 |  | 1 |  | 0.5 |  |
|  |  | 40 |  | 0.5 |  |
| CA_1-418 |  | 1 |  | 0.5 |  |
|  |  | 41 |  | 0.5 |  |
| CA_1-42, CA_1-42-42 |  | 1 |  | 0.3 |  |
|  |  | 42 |  | 0.8 |  |
| CA_1-43 |  | 1 |  | 0.3 |  |
|  |  | 43 |  | 0.8 |  |
| CA_1-46 |  | 1 |  | 0 |  |
| CA_2-4, CA_2-2-4, CA_2-4-4, CA_2-2-4-4 |  | 2 |  | 0.5 |  |
|  |  | 4 |  | 0.5 |  |
| CA_2-5, CA_2-2-5 |  | 2 |  | 0.3 |  |
|  |  | 5 |  | 0.3 |  |
| CA_2-7, CA_2-2-7, CA_2-7-7 |  | 2 |  | 0.5 |  |
|  |  | 7 |  | 0.5 |  |
| CA_2-12, CA_2-2-12, CA_2-12-12, CA_2-2-12-12 |  | 2 |  | 0.3 |  |
|  |  | 12 |  | 0.3 |  |
| CA_2-13, CA_2-2-13 |  | 2 |  | 0.3 |  |
|  |  | 13 |  | 0.3 |  |
| CA_2-14, CA_2-2-14 |  | 2 |  | 0.3 |  |
|  |  | 14 |  | 0.3 |  |
| CA_2-17 |  | 2 |  | 0.3 |  |
|  |  | 17 |  | 0.8 |  |
| CA_2-26 |  | 2 |  | 0.3 |  |
|  |  | 26 |  | 0.3 |  |
| CA_2-28 |  | 2 |  | 0.3 |  |
|  |  | 28 |  | 0.3 |  |
| CA_2-29, CA_2-2-29 |  | 2 |  | 0.3 |  |
| CA_2-30, CA_2-2-30 |  | 2 |  | 0.5 |  |
|  |  | 30 |  | 0.3 |  |
| CA_2-46, CA_2-2-46, CA_2-46-46 |  | 2 |  | 0 |  |
| CA_2-48, CA_2-48-48 |  | 2 |  | 0.6 |  |
|  |  | 48 |  | 0.8 |  |
| CA_2-49 |  | 2 |  | 0.6 |  |
| CA_2-66, CA_2-2-66, CA_2-66-66, CA_2-2-66-66, CA_2-66-66-66 |  | 2 |  | 0.5 |  |
|  |  | 66 |  | 0.5 |  |
| CA_2-71, CA_2-2-71 |  | 2 |  | 0.3 |  |
|  |  | 71 |  | 0.3 |  |
| CA_3-5, CA_3-3-5 |  | 3 |  | 0.3 |  |
|  |  | 5 |  | 0.3 |  |
| CA_3-7, CA_3-3-7, CA_3-7-7, CA_3-3-7-7 |  | 3 |  | 0.5 |  |
|  |  | 7 |  | 0.5 |  |
| CA_3-8, CA_3-3-8 |  | 3 |  | 0.3 |  |
|  |  | 8 |  | 0.3 |  |
| CA_3-11 |  | 3 |  | 0.8 |  |
|  |  | 11 |  | 0.9 |  |
| CA_3-18 |  | 3 |  | 0.3 |  |
|  |  | 18 |  | 0.3 |  |
| CA_3-19, CA_3-3-19 |  | 3 |  | 0.3 |  |
|  |  | 19 |  | 0.3 |  |
| CA_3-20, CA_3-3-20 |  | 3 |  | 0.3 |  |
|  |  | 20 |  | 0.3 |  |
| CA_3-21, CA_3-3-21 |  | 3 |  | 0.8 |  |
|  |  | 21 |  | 0.9 |  |
| CA_3-26 |  | 3 |  | 0.3 |  |
|  |  | 26 |  | 0.3 |  |
| CA_3-27 |  | 3 |  | 0.3 |  |
|  |  | 27 |  | 0.3 |  |
| CA_3-28 |  | 3 |  | 0.3 |  |
|  |  | 28 |  | 0.3 |  |
| CA_3-31 |  | 3 |  | 0.3 |  |
|  |  | 31 |  | 0.6 |  |
| CA_3-32 |  | 3 |  | 0.5 |  |
| CA_3-38 |  | 3 |  | 0,5 |  |
|  |  | 38 |  | 0,5 |  |
| CA_3-40, CA_3-40-40 |  | 3 |  | 0.5 |  |
|  |  | 40 |  | 0.5 |  |
| CA_3-41, CA_3-3-41 |  | 3 |  | 0.5 |  |
|  |  | 41 |  | 0.310 |  |
|  |  |  |  | 0.811 |  |
| CA_3-42, CA_3-3-42, CA_3-42-42 |  | 3 |  | 0.6 |  |
|  |  | 42 |  | 0.8 |  |
| CA_3-43 |  | 3 |  | 0.3 |  |
|  |  | 43 |  | 0.8 |  |
| CA_3-46, CA_3-3-46 |  | 3 |  | 0 |  |
| CA_3-69 |  | 3 |  | 0.5 |  |
| CA_4-5, CA_4-4-5 |  | 4 |  | 0.3 |  |
|  |  | 5 |  | 0.3 |  |
| CA_4-7, CA_4-4-7, CA_4-7-7 |  | 4 |  | 0.5 |  |
|  |  | 7 |  | 0.5 |  |
| CA_4-12, CA_4-4-12, CA_4-12-12, CA_4-4-12-12 |  | 4 |  | 0.3 |  |
|  |  | 12 |  | 0.8 |  |
| CA_4-13, CA_4-4-13 |  | 4 |  | 0.3 |  |
|  |  | 13 |  | 0.3 |  |
| CA_4-17 |  | 4 |  | 0.3 |  |
|  |  | 17 |  | 0.8 |  |
| CA_4-27 |  | 4 |  | 0.3 |  |
|  |  | 27 |  | 0.3 |  |
| CA_4-28 |  | 4 |  | 0.3 |  |
|  |  | 28 |  | 0.6 |  |
| CA_4-29, CA_4-4-29 |  | 4 |  | 0.3 |  |
| CA_4-30, CA_4-4-30 |  | 4 |  | 0.5 |  |
|  |  | 30 |  | 0.3 |  |
| CA_4-46, CA_4-46-46 |  | 4 |  | 0 |  |
| CA_4-48 |  | 4 |  | 0.3 |  |
|  |  | 48 |  | 0.8 |  |
| CA_4-71, CA_4-4-71 |  | 4 |  | 0.3 |  |
|  |  | 71 |  | 0.3 |  |
| CA_5-7, CA_5-7-7 |  | 5 |  | 0.3 |  |
|  |  | 7 |  | 0.3 |  |
| CA_5-12, CA_5-12-12 |  | 5 |  | 0.8 |  |
|  |  | 12 |  | 0.4 |  |
| CA_5-13 |  | 5 |  | 0.5 |  |
|  |  | 13 |  | 0.5 |  |
| CA_5-17 |  | 5 |  | 0.8 |  |
|  |  | 17 |  | 0.4 |  |
| CA_5-25 |  | 5 |  | 0.3 |  |
|  |  | 25 |  | 0.3 |  |
| CA_5-28 |  | 5 |  | 0.5 |  |
|  |  | 28 |  | 0.5 |  |
| CA_5-29 |  | 5 |  | 0.5 |  |
| CA_5-30 |  | 5 |  | 0.3 |  |
|  |  | 30 |  | 0.3 |  |
| CA_5-38 |  | 5 |  | 0.3 |  |
|  |  | 38 |  | 0.3 |  |
| CA_5-40, CA_5-5-40, CA_5-40-40 |  | 5 |  | 0.3 |  |
|  |  | 40 |  | 0.3 |  |
| CA_5-41 |  | 5 |  | 0.3 |  |
|  |  | 41 |  | 0.3 |  |
| CA_5-46 |  | 5 |  | 0 |  |
| CA_5-48 |  | 5 |  | 0.3 |  |
|  |  | 48 |  | 0.3 |  |
| CA_5-66, CA_5-5-66, CA_5-66-66, CA_5-5-66-66 |  | 5 |  | 0.3 |  |
|  |  | 66 |  | 0.3 |  |
| CA_7-8, CA_7-7-8 |  | 7 |  | 0.3 |  |
|  |  | 8 |  | 0.6 |  |
| CA_7-12 |  | 7 |  | 0.3 |  |
|  |  | 12 |  | 0.3 |  |
| CA_7-13 |  | 7 |  | 0.3 |  |
|  |  | 13 |  | 0.3 |  |
| CA_7-20, CA_7-7-20 |  | 7 |  | 0.3 |  |
|  |  | 20 |  | 0.3 |  |
| CA_7-22 |  | 7 |  | 0.5 |  |
|  |  | 22 |  | 0.8 |  |
| CA_7-26, CA_7-7-26 |  | 7 |  | 0.3 |  |
|  |  | 26 |  | 0.3 |  |
| CA_7-28, CA_7-7-28 |  | 7 |  | 0.3 |  |
|  |  | 28 |  | 0.3 |  |
| CA_7-29 CA_7-7-29 |  | 7 |  | 0.3 |  |
| CA_7-30 |  | 7 |  | 0.5 |  |
|  |  | 30 |  | 0.5 |  |
| CA_7-32 |  | 7 |  | 0.7 |  |
| CA_7-40 |  | 7 |  | 0.5 |  |
|  |  | 40 |  | [0.6] |  |
| CA_7-42, CA_7-42-42 |  | 7 |  | 0.5 |  |
|  |  | 42 |  | 0.8 |  |
| CA_7-46, CA_7-7-46 |  | 7 |  | 0 |  |
| CA_7-66, CA_7-7-66, CA_7-66-66, CA_7-7-66-66 |  | 7 |  | 0.5 |  |
|  |  | 66 |  | 0.5 |  |
| CA_8-11 |  | 8 |  | 0.3 |  |
|  |  | 11 |  | 0.4 |  |
| CA_8-20 |  | 8 |  | 0.4 |  |
|  |  | 20 |  | 0.4 |  |
| CA_8-27 |  | 8 |  | 0.8 |  |
|  |  | 27 |  | 0.8 |  |
| CA_8-2814 |  | 8 |  | 0.6 |  |
|  |  | 28 |  | 0.5 |  |
| CA_8-32 |  | 8 |  | 0.3 |  |
| CA_8-38 |  | 8 |  | 0.3 |  |
|  |  | 38 |  | 0.3 |  |
| CA_8-39 |  | 8 |  | 0,3 |  |
|  |  | 39 |  | 0,3 |  |
| CA_8-40 |  | 8 |  | 0.3 |  |
|  |  | 40 |  | 0.3 |  |
| CA_8-41 |  | 8 |  | 0.3 |  |
|  |  | 41 |  | 0.3 |  |
| CA_8-42 |  | 8 |  | 0.6 |  |
|  |  | 42 |  | 0.8 |  |
| CA_8-46 |  | 8 |  | 0 |  |
| CA_11-18 |  | 11 |  | 0.3 |  |
|  |  | 18 |  | 0.3 |  |
| CA_11-26 |  | 11 |  | 0.3 |  |
|  |  | 26 |  | 0.3 |  |
| CA_11-28 |  | 11 |  | 0.4 |  |
|  |  | 28 |  | 0.6 |  |
| CA_11-41 |  | 11 |  | 0.3 |  |
|  |  | 41 |  | 0.3 |  |
| CA_11-42 |  | 11 |  | 0.4 |  |
|  |  | 42 |  | 0.8 |  |
| CA_11-46 |  | 11 |  | 0 |  |
| CA_12-25 |  | 12 |  | 0.3 |  |
|  |  | 25 |  | 0.3 |  |
| CA_12-30 |  | 12 |  | 0.3 |  |
|  |  | 30 |  | 0.3 |  |
| CA_12-46 |  | 12 |  | 0 |  |
|  |  | 46 |  | 0 |  |
| CA_12-48 |  | 12 |  | 0.3 |  |
|  |  | 48 |  | 0.3 |  |
| CA_12-66, CA_12-66-66 |  | 12 |  | 0.8 |  |
|  |  | 66 |  | 0.3 |  |
| CA_13-46, CA_13-46-46 |  | 13 |  | 0 |  |
| CA_13-48, CA_13-48-48 |  | 13 |  | 0.3 |  |
|  |  | 48 |  | 0.3 |  |
| CA_13-66, CA_13-66-66 |  | 13 |  | 0.3 |  |
|  |  | 66 |  | 0.3 |  |
| CA_14-30 |  | 14 |  | 0.3 |  |
|  |  | 30 |  | 0.3 |  |
| CA_14-66, CA_14-66-66, CA_14-66-66-66 |  | 14 |  | 0.3 |  |
|  |  | 66 |  | 0.3 |  |
| CA_18-289 |  | 18 |  | 0.5 |  |
|  |  | 28 |  | 0.5 |  |
| CA_18-41 |  | 18 |  | 0.3 |  |
|  |  | 41 |  | 0.3 |  |
| CA_19-21 |  | 19 |  | 0.3 |  |
|  |  | 21 |  | 0.4 |  |
| CA_19-289 |  | 19 |  | 0.5 |  |
|  |  | 28 |  | 0.5 |  |
| CA_18-42 |  | 18 |  | 0.3 |  |
|  |  | 42 |  | 0.8 |  |
| CA_19-42 |  | 19 |  | 0.3 |  |
|  |  | 42 |  | 0.8 |  |
| CA_19-46 |  | 19 |  | 0 |  |
| CA_20-28 |  | 20 |  | 0.5 |  |
|  |  | 28 |  | 0.5 |  |
| CA_20-31 |  | 20 |  | 0.5 |  |
|  |  | 31 |  | 0.5 |  |
| CA_20-32 |  | 20 |  | 0.3 |  |
| CA_20-38 |  | 20 |  | 0.3 |  |
|  |  | 38 |  | 0.3 |  |
| CA_20-40, CA_20-40-40 |  | 20 |  | 0.3 |  |
|  |  | 40 |  | 0.3 |  |
| CA_20-41 |  | 20 |  | 0.3 |  |
|  |  | 41 |  | 0.3 |  |
| CA_20-42, CA_20-42-42 |  | 20 |  | 0.6 |  |
|  |  | 42 |  | 0.8 |  |
| CA_20-43 |  | 20 |  | 0.3 |  |
|  |  | 43 |  | 0.8 |  |
| CA_20-67 |  | 20 |  | 0.5 |  |
| CA_20-75 |  | 20 |  | 0.3 |  |
| CA_20-76 |  | 20 |  | 0.3 |  |
| CA_21-28 |  | 21 |  | 0.4 |  |
|  |  | 28 |  | 0.3 |  |
| CA_21-42 |  | 21 |  | 0.4 |  |
|  |  | 42 |  | 0.8 |  |
| CA_21-46 |  | 21 |  | 0 |  |
| CA_23-29 |  | 23 |  | 0.3 |  |
| CA_25-26, CA_25-25-26 |  | 25 |  | 0.3 |  |
|  |  | 26 |  | 0.3 |  |
| CA_25-41, CA_25-25-41 |  | 25 |  | 0.5 |  |
|  |  | 41 |  | 0.410 |  |
|  |  |  |  | 0.911 |  |
| CA_25-46 |  | 25 |  | 0 |  |
|  |  | 46 |  | 0 |  |
| CA_26-41 |  | 26 |  | 0.3 |  |
|  |  | 41 |  | 0.3 |  |
| CA_26-46 |  | 26 |  | 0 |  |
| CA_26-48, CA_26-48-48 |  | 26 |  | 0.3 |  |
|  |  | 48 |  | 0.3 |  |
| A_26-66 |  | 26 |  | 0.3 |  |
|  |  | 66 |  | 0.3 |  |
| CA_28-32 |  | 28 |  | 0.3 |  |
| CA_28-38 |  | 28 |  | 0.3 |  |
|  |  | 38 |  | 0.3 |  |
| CA_28-40 |  | 28 |  | 0.3 |  |
|  |  | 40 |  | 0.3 |  |
| CA_28-41 |  | 28 |  | 0.3 |  |
|  |  | 41 |  | 0.3 |  |
| CA_28-42, CA_28-42-42 |  | 28 |  | 0.5 |  |
|  |  | 42 |  | 0.8 |  |
| CA_28-46 |  | 28 |  | 0 |  |
| CA_28-66 |  | 28 |  | 0.6 |  |
|  |  | 66 |  | 0.3 |  |
| CA_29-30 |  | 30 |  | 0.3 |  |
| CA_29-66, CA_29-66-66 |  | 66 |  | 0.3 |  |
| CA_29-70 |  | 70 |  | 0.3 |  |
| CA_30-66, CA_30-66-66 |  | 30 |  | 0.3 |  |
|  |  | 66 |  | 0.5 |  |
| CA_32-42 |  | 42 |  | 0.8 |  |
| CA_32-43 |  | 43 |  | 0.8 |  |
| CA_34-39 |  | 34 |  | 01 |  |
|  |  | 39 |  | 01 |  |
| CA_34-41 |  | 34 |  | 01 |  |
|  |  | 41 |  | 01 |  |
| CA_38-40, CA_38-40-40 |  | 38 |  | 04 |  |
|  |  | 40 |  | 04 |  |
| CA_39-40 |  | 39 |  | 04 |  |
|  |  | 40 |  | 04 |  |
| CA_39-41 |  | 39 |  | 04 |  |
|  |  | 41 |  | 04 |  |
| CA_39-41 |  | 39 |  | 0.57 |  |
|  |  | 41 |  | 0.57 |  |
| CA_39-42 |  | 39 |  | 04 |  |
|  |  | 42 |  | 0.54 |  |
| CA_39-46 |  | 39 |  | 0 |  |
| CA_40-41 |  | 40 |  | 0.54 |  |
|  |  | 41 |  | 0.54 |  |
| CA_40-42 |  | 40 |  | 04 |  |
|  |  | 42 |  | 0.54 |  |
| CA_40-43 |  | 40 |  | 04 |  |
|  |  | 43 |  | 0.54 |  |
| CA_40-46 |  | 40 |  | 0 |  |
| CA_41-42, CA_41-42-42 |  | 41 |  | 04 |  |
|  |  | 42 |  | 0.54 |  |
| CA_41-42, CA_41-42-42 |  | 41 |  | 0.37 |  |
|  |  | 42 |  | 0.87 |  |
| CA_41-46 |  | 41 |  | 0 |  |
| CA_41-48 |  | 41 |  | 04 |  |
|  |  | 48 |  | 0.54 |  |
| CA_42-43 |  | 42 |  | 04 |  |
|  |  | 43 |  | 04 |  |
| CA_42-46 |  | 42 |  | [0.5] |  |
| CA_46-48, CA_46-48-48 |  | 48 |  | 0.8 |  |
| CA_46-66, CA_46-46-66, CA_46-66-66 |  | 66 |  | 0 |  |
| CA_46-70 |  | 70 |  | 0 |  |
| CA_46-71 |  | 71 |  | 0 |  |
| CA_48-66, CA_48-48-66, CA_48-66-66, CA_48-48-66-66 |  | 48 |  | 0.8 |  |
|  |  | 66 |  | 0.6 |  |
| CA_48-71, CA_48-48-71 |  | 48 |  | 0.3 |  |
|  |  | 71 |  | 0.3 |  |
| CA_66-70, CA_66-66-70 |  | 66 |  | 0.5 |  |
|  |  | 70 |  | 0.5 |  |
| CA_66-71, CA_66-66-71 |  | 66 |  | 0.3 |  |
|  |  | 71 |  | 0.3 |  |
| CA_70-71 |  | 70 |  | 0.3 |  |
|  |  | 71 |  | 0.6 |  |
| NOTE 1: The above additional tolerances are only applicable for the E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations NOTE 2: The above additional tolerances also apply in non-aggregated operation for the supported E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations NOTE 3: In case the UE supports more than one of the above 2DL inter-band carrier aggregation configurations and a E-UTRA operating band belongs to more than one 2DL inter-band carrier aggregation configurations then: - When the E-UTRA operating band frequency range is ≤ 1GHz, the applicable additional tolerance shall be the average of the 2DL tolerances above, truncated to one decimal place for that operating band among the supported 2DL CA configurations. In case there is a harmonic relation between low band UL and high band DL, then the maximum tolerance among the different supported 2DL carrier aggregation configurations involving such band shall be applied - When the E-UTRA operating band frequency range is >1GHz, the applicable additional 2DL tolerance shall be the maximum tolerance above that applies for that operating band among the supported 2DL CA configurations NOTE 4: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. NOTE 5: Unless otherwise specified, in case the UE supports more than one of the above 3DL inter-band carrier aggregation configurations and a E-UTRA operating band belongs to more than one 3DL inter-band carrier aggregation configurations then: - When the E-UTRA operating band frequency range is ≤ 1GHz and the tolerances are the same, the value applies to the band. If the tolerances are different, the applicable additional 3DL tolerance is FFS. In case there is a harmonic relation between low band UL and high band DL, then the maximum tolerance among the different supported 3DL carrier aggregation configurations involving such band shall be applied - When the E-UTRA operating band frequency range is >1GHz, the applicable additional 3DL tolerance shall be the maximum tolerance above that applies for that operating band among the supported 3DL CA configurations. NOTE 6: The above additional tolerances applicable for the E-UTRA operating bands that belong to the supported highest order inter-band carrier aggregation configuration, also applies to the same E-UTRA operating bands that belong to a supported lower order CA configuration. NOTE 7: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. NOTE 8: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in the FDD band. NOTE 9: For Band 28, the requirements only apply for the restricted frequency range specified for this CA configuration (Table 5.5A-2). NOTE 10: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 11: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. NOTE 12: For UE supporting E-UTRA band 65 and CA configurations including Band 1, the Band 65 ΔTIB,c is the max(Band 65 ΔTIB,c , Band 1 ΔTIB,c) NOTE 13: For UE supporting E-UTRA band 42, 43 or 48 and CA configurations including Band 42, 43 or 48, the applicable ΔTIB,c in Band 42, 43, or 48 is the max(Band 42 ΔTIB,c , Band 43 ΔTIB,c, Band 48 ΔTIB,c). NOTE 14: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 8. |  |  |  |  |  |

NOTE:	The above additional tolerances do not apply to supported UTRA operating bands with frequency range below 1 GHz that correspond to the E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations when such bands are belonging only to band combination(s) where one band is <1GHz and another band is >1.7GHz and there is no harmonic relationship between the low band UL and high band DL. Otherwise the above additional tolerances also apply to supported UTRA operating bands that correspond to the E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations.

NOTE:	To meet the TIB,c requirements for CA_3A-7A with state-of-the-art technology, an increase in power consumption of the UE may be required. It is also expected that as the state-of-the-art technology evolves in the future, this possible power consumption increase can be reduced or eliminated.

Table 6.2.5-3: ΔTIB,c (three bands)

| E-UTRA operating band combination | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1-3-5, CA_1-1-3-5, CA_1-3-3-5 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 5 | 0.3 |
| CA_1-3-7, CA_1-1-3-7, CA_1-1-3-3-7, CA_1-3-3-7, CA_1-3-3-7-7, CA_1-3-7-7 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
| CA_1-3-8, CA_1-3-3-8 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 8 | 0.3 |
| CA_1-3-11 | 1 | 0.3 |
|  | 3 | 0.8 |
|  | 11 | 0.9 |
| CA_1-3-18 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 18 | 0.3 |
| CA_1-3-19, CA_1-3-3-19 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 19 | 0.3 |
| CA_1-3-20, CA_1-3-3-20 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 20 | 0.3 |
| CA_1-3-21, CA_1-3-3-21 | 1 | 0.3 |
|  | 3 | 0.8 |
|  | 21 | 0.9 |
| CA_1-3-26 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 26 | 0.3 |
| CA_1-3-28, CA_1-1-3-28, CA_1-3-3-28, CA_1-1-3-28 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 28 | 0.6 |
| CA_1-3-32 | 1 | 0.5 |
|  | 3 | 0.5 |
| CA_1-3-38 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 38 | 0.5 |
| CA_1-3-40 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 40 | 0.5 |
| CA_1-3-41 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 41 | 0.35/0.86 |
| CA_1-3-42, CA_1-3-3-42 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 42 | 0.8 |
| CA_1-3-43 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 43 | 0.8 |
| CA_1-3-46 | 1 | 0.3 |
|  | 3 | 0.3 |
| CA_1-5-7, CA_1-5-7-7 | 1 | 0.5 |
|  | 5 | 0.3 |
|  | 7 | 0.6 |
| CA_1-5-28 | 1 | 0.3 |
|  | 5 | 0.5 |
|  | 28 | 0.6 |
| CA_1-5-40 | 1 | 0.5 |
|  | 5 | 0.3 |
|  | 40 | 0.5 |
| CA_1-5-41 | 1 | 0.5 |
|  | 5 | 0.3 |
|  | 41 | 0.5 |
| CA_1-5-46 | 1 | 0.3 |
|  | 5 | 0.3 |
| CA_1-7-8, CA_1-7-7-8 | 1 | 0.5 |
|  | 7 | 0.6 |
|  | 8 | 0.6 |
| CA_1-7-20, CA_1-7-7-20 | 1 | 0.5 |
|  | 7 | 0.6 |
|  | 20 | 0.3 |
| CA_1-7-26, CA_1-7-7-26 | 1 | 0.5 |
|  | 7 | 0.6 |
|  | 26 | 0.3 |
| CA_1-7-28 | 1 | 0.5 |
|  | 7 | 0.6 |
|  | 28 | 0.6 |
| CA_1-7-32 | 1 | 0.7 |
|  | 7 | 0.7 |
| CA_1-7-38 | 1 | 0.5 |
| CA_1-7-40 | 1 | 0.6 |
|  | 7 | 0.8 |
|  | 40 | 0.9 |
| CA_1-7-42 | 1 | 0.6 |
|  | 7 | 0.6 |
|  | 42 | 0.8 |
| CA_1-7-46 | 1 | 0.5 |
|  | 7 | 0.6 |
| CA_1-8-11 | 1 | 0.3 |
|  | 8 | 0.3 |
|  | 11 | 0.4 |
| CA_1-8-20 | 1 | 0.3 |
|  | 8 | 0.4 |
|  | 20 | 0.4 |
| CA_1-8-2811 | 1 | 0.3 |
|  | 8 | 0.6 |
|  | 28 | 0.6 |
| CA_1-8-38 | 1 | 0.5 |
|  | 8 | 0.3 |
|  | 38 | 0.5 |
| CA_1-8-40 | 1 | 0.5 |
|  | 8 | 0.3 |
|  | 40 | 0.5 |
| CA_1-8-42 | 1 | 0.3 |
|  | 8 | 0.6 |
|  | 42 | 0.8 |
| CA_1-11-18 | 1 | 0.3 |
|  | 11 | 0.4 |
|  | 18 | 0.3 |
| CA_1-11-28 | 1 | 0.3 |
|  | 11 | 0.4 |
|  | 28 | 0.6 |
| CA_1-11-42 | 1 | 0.3 |
|  | 11 | 0.4 |
|  | 42 | 0.8 |
| CA_1-18-28 | 1 | 0.3 |
|  | 18 | 0.5 |
|  | 28 | 0.5 |
| CA_1-18-41 | 1 | 0.5 |
|  | 18 | 0.3 |
|  | 41 | 0.5 |
| CA_1-18-42 | 1 | 0.3 |
|  | 18 | 0.3 |
|  | 42 | 0.8 |
| CA_1-19-21 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | 21 | 0.4 |
| CA_1-19-28 | 1 | 0.3 |
|  | 19 | 0.5 |
|  | 28 | 0.5 |
| CA_1-19-42 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
| CA_1-20-28 | 1 | 0.3 |
|  | 20 | 0.6 |
|  | 28 | 0.6 |
| CA_1-20-32 | 1 | 0.5 |
|  | 20 | 0.3 |
| CA_1-20-38 | 1 | 0.5 |
|  | 20 | 0.3 |
|  | 38 | 0.5 |
| CA_1-20-42 | 1 | 0.3 |
|  | 20 | 0.3 |
|  | 42 | 0.8 |
| CA_1-20-43 | 1 | 0.3 |
|  | 20 | 0.3 |
|  | 43 | 0.8 |
| CA_1-21-28 | 1 | 0.3 |
|  | 21 | 0.4 |
|  | 28 | 0.6 |
| CA_1-21-42 | 1 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
| CA_1-28-40 | 1 | 0.6 |
|  | 28 | 0.3 |
|  | 40 | 0.5 |
| CA_1-28-42 | 1 | 0.3 |
|  | 28 | 0.6 |
|  | 42 | 0.8 |
| CA_1-32-42 | 1 | 0.5 |
|  | 42 | 0.8 |
| CA_1-32-43 | 1 | 0.5 |
|  | 43 | 0.8 |
| CA_1-41-428, 13 | 1 | 0.5 |
|  | 41 | 0.5 |
|  | 42 | 0.8 |
| CA_1-42-4313 | 1 | 0.3 |
|  | 42 | 0.8 |
|  | 43 | 0.8 |
| CA_2-4-5, CA_2-2-4-5, CA_2-4-4-5 | 2 | 0.5 |
|  | 4 | 0.5 |
|  | 5 | 0.3 |
| CA_2-4-7, CA_2-4-7-7 | 2 | 0.5 |
|  | 4 | 0.5 |
|  | 7 | 0.5 |
| CA_2-4-12, CA_2-2-4-12, CA_2-4-4-12, CA_2-4-12-12 | 2 | 0.5 |
|  | 4 | 0.5 |
|  | 12 | 0.8 |
| CA_2-4-13 | 2 | 0.5 |
|  | 4 | 0.5 |
|  | 13 | 0.3 |
| CA_2-4-28 | 2 | 0.5 |
|  | 4 | 0.5 |
|  | 28 | 0.8 |
| CA_2-4-29 | 2 | [0.5] |
|  | 4 | 0.5 |
| CA_2-4-30 | 2 | 0.5 |
|  | 4 | 0.5 |
|  | 30 | 0.3 |
| CA_2-4-71, CA_2-2-4-71 | 2 | 0,5 |
|  | 4 | 0.5 |
|  | 71 | 0.3 |
| CA_2-5-12, CA_2-2-5-12, CA_2-5-12-12 | 2 | 0.3 |
|  | 5 | 0.8 |
|  | 12 | 0.4 |
| CA_2-5-7 | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 7 | 0.5 |
| CA_2-5-13 | 2 | 0.3 |
|  | 5 | 0.5 |
|  | 13 | 0.5 |
| CA_2-5-28 | 2 | 0.3 |
|  | 5 | 0.8 |
|  | 28 | 0.4 |
| CA_2-5-29 | 2 | 0.3 |
|  | 5 | 0.5 |
| CA_2-5-30, CA_2-2-5-30 | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 30 | 0.3 |
| CA_2-5-46 | 2 | 0.3 |
|  | 5 | 0.3 |
| CA_2-5-66, CA_2-2-5-66, CA_2-5-66-66 | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 66 | 0.5 |
| CA_2-7-12, CA_2-2-7-12 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 12 | 0.3 |
| CA_2-7-13, CA_2-7-7-13 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 13 | 0.3 |
| 2 | 2 | 0.5 |
| 7 | 7 | 0.5 |
| 28 | 26 | 0.3 |
| CA_2-7-28 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 28 | 0.3 |
| CA_2-7-29, CA_2-7-7-29 | 2 | 0.5 |
|  | 7 | 0.5 |
| CA_2-7-30 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 30 | 0.5 |
| CA_2-7-46 | 2 | 0.5 |
|  | 7 | 0.5 |
| CA_2-7-66, CA_2-2-7-66, CA_2-7-7-66, CA_2-7-7-66-66, CA_2-7-66-66 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
| CA_2-12-30, CA_2-2-12-30 | 2 | 0.5 |
|  | 12 | 0.3 |
|  | 30 | 0.3 |
| CA_2-12-66, CA_2-2-12-66, CA_2-12-66-66, CA_2-2-12-66-66 | 2 | 0.5 |
|  | 12 | 0.8 |
|  | 66 | 0.5 |
| CA_2-13-46 | 2 | 0.3 |
|  | 13 | 0.3 |
| CA_2-13-48, CA_2-13-48-48 | 2 | 0.6 |
|  | 13 | 0.3 |
|  | 48 | 0.8 |
| CA_2-13-66, CA_2-2-13-66, CA_2-13-66-66 | 2 | 0.5 |
|  | 13 | 0.3 |
|  | 66 | 0.5 |
| CA_2-14-30, CA_2-2-14-30 | 2 | 0.5 |
|  | 14 | 0.3 |
|  | 30 | 0.5 |
| CA_2-14-66, CA_2-2-14-66, CA_2-14-66-66, CA_2-2-14-66-66, CA_2-14-66-66-66 | 2 | 0.5 |
|  | 14 | 0.3 |
|  | 66 | 0.5 |
| CA_2-26-66 | 2 | 0.5 |
|  | 26 | 0.3 |
|  | 66 | 0.5 |
| CA_2-28-66 | 2 | 0.5 |
|  | 28 | 0.6 |
|  | 66 | 0.5 |
| CA_2-29-30, CA_2-2-29-30 | 2 | 0.5 |
|  | 30 | 0.3 |
| CA_2-29-66 | 2 | 0.5 |
|  | 66 | 0.5 |
| CA_2-30-66, CA_2-2-30-66, CA_2-30-66-66 | 2 | 0.5 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
| CA_2-46-48 | 2 | 0.6 |
|  | 48 | 0.8 |
| CA_2-46-66, CA_2-46-46-66, CA_2-46-66-66 | 2 | 0.5 |
|  | 66 | 0.5 |
| CA_2-48-66, CA_2-48-48-66 | 2 | 0.6 |
|  | 48 | 0.8 |
|  | 66 | 0.6 |
| CA_2-66-71 CA_2-2-66-71 CA_2-66-66-71 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | 71 | 0.3 |
| CA_3-5-7, CA_3-5-7-7, CA_3-3-5-7 | 3 | 0.5 |
|  | 5 | 0.3 |
|  | 7 | 0.5 |
| CA_3-5-28 CA_3-3-5-28 | 3 | 0.3 |
|  | 5 | 0.5 |
|  | 28 | 0.5 |
| CA_3-5-40, CA_3-5-40-40 | 3 | 0.5 |
|  | 5 | 0.3 |
|  | 40 | 0.5 |
| CA_3-5-41 | 3 | 0.5 |
|  | 5 | 0.318 |
|  | 41 | 0.35 |
|  |  | 0.86 |
| CA_3-7-8, CA_3-3-7-8, CA_3-3-7-7-8 | 3 | 0.5 |
|  | 7 | 0.5 |
|  | 8 | 0.6 |
| CA_3-7-20, CA_3-3-7-20, CA_3-7-7-20 | 3 | 0.5 |
|  | 7 | 0.5 |
|  | 20 | 0.3 |
| CA_3-7-26, CA_3-7-7-26 | 3 | 0.5 |
|  | 7 | 0.5 |
|  | 26 | 0.3 |
| CA_3-7-28, CA_3-3-7-28 | 3 | 0.5 |
|  | 7 | 0.5 |
|  | 28 | 0.3 |
| CA_3-7-32 | 3 | 0.7 |
|  | 7 | 0.7 |
| CA_3-7-38 | 3 | 0.5 |
|  | 7 | 0.5 |
|  | 38 | 0.5 |
| CA_3-7-40 | 3 | 0.6 |
|  | 7 | 0.8 |
|  | 40 | 0.9 |
| CA_3-7-42 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 42 | 0.8 |
| CA_3-7-46 | 3 | 0.5 |
|  | 7 | 0.5 |
| CA_3-8-11 | 3 | 0.8 |
|  | 8 | 0.3 |
|  | 11 | 0.9 |
| CA_3-8-20 | 3 | 0.3 |
|  | 8 | 0.4 |
|  | 20 | 0.4 |
| CA_3-8-2812 | 3 | 0.3 |
|  | 8 | 0.6 |
|  | 28 | 0.5 |
| CA_3-8-32 | 3 | 0.8 |
|  | 8 | 0.3 |
| CA_3-8-38 | 3 | 0.5 |
|  | 8 | 0.3 |
|  | 38 | 0.5 |
| CA_3-8-40 | 3 | 0.5 |
|  | 8 | 0.3 |
|  | 40 | 0.5 |
| CA_3-8-42 | 3 | 0.6 |
|  | 8 | 0.6 |
|  | 42 | 0.8 |
| CA_3-11-18 | 3 | 0.8 |
|  | 11 | 0.9 |
|  | 18 | 0.3 |
| CA_3-11-26 | 3 | 0.8 |
|  | 11 | 0.9 |
|  | 26 | 0.3 |
| CA_3-11-28 | 3 | 0.8 |
|  | 11 | 0.9 |
|  | 28 | 0.6 |
| CA_3-18-42 | 3 | 0.6 |
|  | 18 | 0.3 |
|  | 42 | 0.8 |
| CA_3-19-21, CA_3-3-19-21 | 3 | 0.8 |
|  | 19 | 0.3 |
|  | 21 | 0.9 |
| CA_3-19-42 | 3 | 0.6 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
| CA_3-20-28, CA_3-3-20-28 | 3 | 0.3 |
|  | 20 | 0.5 |
|  | 28 | 0.5 |
| CA_3-20-32 | 3 | 0.5 |
|  | 20 | 0.3 |
| CA_3-20-42 | 3 | 0.6 |
|  | 20 | 0.3 |
|  | 42 | 0.8 |
| CA_3-20-43 | 3 | 0.3 |
|  | 20 | 0.3 |
|  | 43 | 0.8 |
| CA_3-21-28 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | 28 | 0.3 |
| CA_3-21-42 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | 42 | 0.8 |
| CA_3-28-38 | 3 | 0.5 |
|  | 28 | 0.5 |
|  | 38 | 0.5 |
| CA_3-28-40 | 3 | 0.5 |
|  | 28 | 0.3 |
|  | 40 | 0.5 |
| CA_3-28-41 | 3 | 0.5 |
|  | 28 | 0.3 |
|  | 41 | 0.35/0.86 |
| CA_3-28-42, CA_3-28-42-42 | 3 | 0.6 |
|  | 28 | 0.5 |
|  | 42 | 0.8 |
| CA_3-32-42 | 3 | 0.6 |
|  | 42 | 0.8 |
| CA_3-32-43 | 3 | 0.3 |
|  | 43 | 0.8 |
| CA_3-32-46 | 3 | 0.5 |
| CA_3-41-4214 CA_3-41-42-42 | 3 | 1 |
|  | 41 | 0.35/0.86 |
|  | 42 | 0.8 |
| CA_3-42-4313 | 3 | 0.6 |
|  | 42 | 0.8 |
|  | 43 | 0.8 |
| CA_4-5-12, CA_4-5-12-12, CA_4-4-5-12 | 4 | 0.3 |
|  | 5 | 0.8 |
|  | 12 | 0.8 |
| CA_4-5-13 | 4 | 0.3 |
|  | 5 | 0.5 |
|  | 13 | 0.5 |
| CA_4-5-29 | 4 | 0.3 |
|  | 5 | 0.5 |
| CA_4-5-30, CA_4-4-5-30 | 4 | 0.5 |
|  | 5 | 0.3 |
|  | 30 | 0.3 |
| CA_4-7-12 | 4 | 0.5 |
|  | 7 | 0.5 |
|  | 12 | 0.8 |
| CA_4-7-28 | 4 | 0.5 |
|  | 7 | 0.5 |
|  | 28 | 0.6 |
| CA_4-12-30, CA_4-4-12-30 | 4 | 0.5 |
|  | 12 | 0.8 |
|  | 30 | 0.3 |
| CA_4-29-30, CA_4-4-29-30 | 4 | 0.5 |
|  | 30 | 0.3 |
| CA_5-7-28 | 5 | 0.5 |
|  | 7 | 0.3 |
|  | 28 | 0.5 |
| CA_5-7-46 | 5 | 0.3 |
|  | 7 | 0.3 |
| CA_5-7-66 CA_5-7-66-66 | 5 | 0.3 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
| CA_5-12-46 | 5 | 0.8 |
|  | 12 | 0.4 |
| CA_5-12-48 | 5 | 0.8 |
|  | 12 | 0.4 |
|  | 48 | 0.3 |
| CA_5-12-66 | 5 | 0.3 |
|  | 12 | 0.8 |
|  | 66 | 0.8 |
| CA_5-30-66, CA_5-30-66-66 | 5 | 0.3 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
| CA_5-40-41 | 5 | 0.3 |
|  | 40 | 0.5 |
|  | 41 | 0.5 |
| CA_5-46-66, CA_5-46-66-66 | 5 | 0.3 |
|  | 66 | 0.3 |
| CA_5-48-66 | 5 | 0.3 |
|  | 48 | 0.8 |
|  | 66 | 0.6 |
| CA_7-8-20 | 7 | 0.3 |
|  | 8 | 0.6 |
|  | 20 | [0.6] |
| CA_7-8-38 | 7 | 0.5 |
|  | 8 | 0.5 |
|  | 38 | 0.5 |
| CA_7-8-40 | 7 | 0.5 |
|  | 8 | 0.6 |
|  | 40 | 0.6 |
| CA_7-12-66 | 7 | 0.5 |
|  | 12 | 0.8 |
|  | 66 | 0.5 |
| CA_7-13-66 | 7 | 0.5 |
|  | 13 | 0.3 |
|  | 66 | 0.5 |
| CA_7-20-28 | 7 | 0.3 |
|  | 20 | 0.6 |
|  | 28 | 0.6 |
| CA_7-20-32 | 7 | 0.7 |
|  | 20 | 0.3 |
|  | 32 | N/A |
| CA_7-20-38 | 7 | 0.3 |
|  | 20 | 0.3 |
|  | 38 | 0.3 |
| CA_7-20-42 | 7 | 0.3 |
|  | 20 | 0.3 |
|  | 42 | 0.8 |
| CA_7-26-66 | 7 | 0.5 |
|  | 26 | 0.3 |
|  | 66 | 0.5 |
| CA_7-28-38 | 7 | 0.3 |
|  | 28 | 0.3 |
|  | 38 | 0.3 |
| CA_7-28-40 | 7 | 0.5 |
|  | 28 | 0.3 |
|  | 40 | 0.6 |
| CA_7-29-66 | 7 | 0.5 |
|  | 66 | 0.5 |
| CA_7-30-66 | 7 | 0.5 |
|  | 30 | 0.5 |
|  | 66 | 0.5 |
| CA_7-32-46 | 7 | 0.7 |
| CA_7-46-66 | 7 | 0.5 |
|  | 66 | 0.5 |
| CA_8-11-2817 | 8 | 0.6 |
|  | 11 | 0.4 |
|  | 28 | 0.6 |
| CA_8-11-42 | 8 | 0.6 |
|  | 11 | 0.4 |
|  | 42 | 0.8 |
| CA_8-20-28 | 8 | 0.6 |
|  | 20 | 0.5 |
|  | 28 | 0.5 |
| CA_8-28-4115 | 8 | 0.6 |
|  | 28 | 0.5 |
|  | 41 | 0.3 |
| CA_8-39-41 | 8 | 0.3 |
|  | 39 | 0.319 |
|  | 41 | 0.319 |
| CA_12-30-66, CA_12-30-66-66 | 12 | 0.8 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
| CA_13-46-66 | 13 | 0.3 |
|  | 66 | 0.3 |
| CA_13-48-66 | 13 | 0.3 |
|  | 48 | 0.8 |
|  | 66 | 0.6 |
| CA_14-30-66, CA_14-30-66-66 | 14 | 0.3 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
| CA_19-21-42 | 19 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
| CA_20-32-42 | 20 | 0.5 |
|  | 42 | 0.8 |
| CA_20-32-43 | 20 | 0.3 |
|  | 43 | 0.8 |
| CA_20-38-4020, CA_20-38-40-4020 | 20 | 0.3 |
|  | 38 | 0.3 |
|  | 40 | 0.3 |
| CA_21-28-42 | 21 | 0.4 |
|  | 28 | 0.5 |
|  | 42 | 0.8 |
| CA_25-26-41, CA_25-25-26-41 | 25 | 0.3 |
|  | 26 | 0.3 |
|  | 41 | 0.3 |
| CA_28-41-4210, CA_28-41-42-4210 | 28 | 0.5 |
|  | 41 | 0.31 |
|  | 42 | 0.81 |
| CA_29-30-66, CA_29-30-66-66 | 30 | 0.3 |
|  | 66 | 0.5 |
| CA_29-46-66 | 66 | 0.3 |
| CA_29-66-70, CA_29-66-66-70 | 66 | 0.5 |
|  | 70 | 0.5 |
| CA_32-42-4313 | 42 | 0.8 |
|  | 43 | 0.8 |
| CA_46-48-66 | 48 | 0.8 |
|  | 66 | 0.6 |
| CA_46-48-71, CA_46-48-48-71 | 46 | 0 |
|  | 48 | 0.8 |
|  | 71 | 0.3 |
| CA_66-70-71, CA_66-66-70-71 | 66 | 0.5 |
|  | 70 | 0.5 |
|  | 71 | 0.6 |
| NOTE 1: The above additional tolerances are only applicable for the E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations NOTE 2: The above additional tolerances also apply in non-aggregated operation for the supported E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations NOTE 3: Unless otherwise specified, in case the UE supports more than one of the above 3DL inter-band carrier aggregation configurations and a E-UTRA operating band belongs to more than one 3DL inter-band carrier aggregation configurations then: - When the E-UTRA operating band frequency range is ≤ 1GHz and the tolerances are the same, the value applies to the band. If the tolerances are different, the applicable additional 3DL tolerance is FFS. In case there is a harmonic relation between low band UL and high band DL, then the maximum tolerance among the different supported 3DL carrier aggregation configurations involving such band shall be applied - When the E-UTRA operating band frequency range is >1GHz, the applicable additional 3DL tolerance shall be the maximum tolerance above that applies for that operating band among the supported 3DL CA configurations NOTE 4: The above additional tolerances applicable for the E-UTRA operating bands that belong to the supported highest order inter-band carrier aggregation configuration, also applies to the same E-UTRA operating bands that belong to a supported lower order CA configuration. NOTE 5: The requirement is specified for the frequency range of 2545-2690MHz. NOTE 6: The requirement is specified for the frequency range of 2496-2545MHz. NOTE 7: For UE supporting E-UTRA band 65 and CA configurations including Band 1, the Band 65 ΔTIB,c is the max(Band 65 ΔTIB,c , Band 1 ΔTIB,c) NOTE 8: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 1 or Band 42. NOTE 9: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx on Band 41 and Band 42. NOTE 10: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx among TDD bands. NOTE 11: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 1 or Band 8 NOTE 12: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 3 or Band 8. NOTE 13: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx among TDD bands. NOTE 14: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx among TDD bands. NOTE 15: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 8 or Band 41. NOTE 16: For UE supporting E-UTRA band 42, 43 or 48 and CA configurations including Band 42, 43 or 48, the applicable ΔTIB,c in Band 42, 43, or 48 is the max(Band 42 ΔTIB,c , Band 43 ΔTIB,c, Band 48 ΔTIB,c). NOTE 17: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 8 or Band 11. NOTE 18: The values in the table reflect what can be achieved with the present state of the art technology. They shall be reconsidered when the state of the art technology progresses. NOTE 19: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRAN band and without simultaneous Rx/Tx on band 39 and band 41 NOTE 20: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx among TDD bands |  |  |

Table 6.2.5-4: ΔTIB,c (four bands)

| E-UTRA operating band combination | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1-3-5-7, CA_1-3-3-5-7, CA_1-3-5-7-7 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 5 | 0.3 |
|  | 7 | 0.6 |
| CA_1-3-5-28 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 5 | 0.5 |
|  | 28 | 0.6 |
| CA_1-3-5-40 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 5 | 0.3 |
|  | 40 | 0.5 |
| CA_1-3-5-41 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 5 | 0.39 |
|  | 41 | 0.57 |
|  |  | 0.88 |
| CA_1-3-7-8, CA_1-3-3-7-8, CA_1-3-7-7-8, CA_1-3-3-7-7-8 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 8 | 0.6 |
| CA_1-3-7-20, CA_1-3-3-7-20, CA_1-3-7-7-20 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 20 | 0.3 |
| CA_1-3-7-26, CA_1-3-7-7-26 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 26 | 0.3 |
| CA_1-3-7-28, CA_1-1-3-7-28, CA_1-1-3-3-7-28, CA_1-3-3-7-28, CA_1-3-7-7-28 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 28 | 0.6 |
| CA_1-3-7-32 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
| CA_1-3-7-38 | 1 | 0.6 |
|  | 3 | 0.6 |
| CA_1-3-7-40 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.8 |
|  | 40 | 0.9 |
| CA_1-3-7-42 | 1 | 0.7 |
|  | 3 | 0.7 |
|  | 7 | 0.7 |
|  | 42 | 0.8 |
| CA_1-3-7-46 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
| CA_1-3-8-11 | 1 | 0.3 |
|  | 3 | 0.8 |
|  | 8 | 0.3 |
|  | 11 | 0.9 |
| CA_1-3-8-20 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 8 | 0.4 |
|  | 20 | 0.4 |
| CA_1-3-8-2810 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 8 | 0.6 |
|  | 28 | 0.6 |
| CA_1-3-8-38 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 8 | 0.3 |
|  | 38 | 0.5 |
| CA_1-3-8-40 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 8 | 0.3 |
|  | 40 | 0.5 |
| CA_1-3-8-42 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 8 | 0.6 |
|  | 42 | 0.8 |
| CA_1-3-11-28 | 1 | 0.3 |
|  | 3 | 0.8 |
|  | 11 | 0.9 |
|  | 28 | 0.6 |
| CA_1-3-18-42 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 18 | 0.3 |
|  | 42 | 0.8 |
| CA_1-3-19-21 | 1 | 0.3 |
|  | 3 | 0.8 |
|  | 19 | 0.3 |
|  | 21 | 0.9 |
| CA_1-3-19-42, CA_1-3-3-19-21 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 19 | 0.3 |
|  | 42 | 0.8 |
| CA_1-3-20-28, CA_1-3-3-20-28 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 20 | 0.6 |
|  | 28 | 0.6 |
| CA_1-3-20-32 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 20 | 0.3 |
| CA_1-3-20-42 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 20 | 0.3 |
|  | 42 | 0.8 |
| CA_1-3-20-43 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 20 | 0.3 |
|  | 43 | 0.8 |
| CA_1-3-21-28 | 1 | 0.3 |
|  | 3 | 0.8 |
|  | 21 | 0.9 |
|  | 28 | 0.6 |
| CA_1-3-21-42 | 1 | 0.6 |
|  | 3 | 0.8 |
|  | 21 | 0.9 |
|  | 42 | 0.8 |
| CA_1-3-28-40 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 28 | 0.6 |
|  | 40 | 0.5 |
| CA_1-3-28-42 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 28 | 0.6 |
|  | 42 | 0.8 |
| CA_1-3-32-42 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 42 | 0.8 |
| CA_1-3-32-43 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 43 | 0.8 |
| CA_1-3-41-42 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 41 | 0.5 |
|  | 42 | 0.8 |
| CA_1-3-42-4313 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 42 | 0.8 |
|  | 43 | 0.8 |
| CA_1-5-7-28 | 1 | 0.6 |
|  | 5 | 0.5 |
|  | 7 | 0.6 |
|  | 28 | 0.6 |
| CA_1-5-7-46 | 1 | 0.5 |
|  | 5 | 0.3 |
|  | 7 | 0.6 |
| CA_1-7-8-20 | 1 | 0.5 |
|  | 7 | 0.6 |
|  | 8 | 0.6 |
|  | 20 | 0.6 |
| CA_1-7-8-40 | 1 | 0.6 |
|  | 7 | 0.8 |
|  | 8 | 0.6 |
|  | 40 | 0.9 |
| CA_1-7-20-28 | 1 | 0.5 |
|  | 7 | 0.6 |
|  | 20 | 0.6 |
|  | 28 | 0.6 |
| CA_1-7-20-32 | 1 | 0.7 |
|  | 7 | 0.7 |
|  | 20 | 0.3 |
| CA_1-7-20-42 | 1 | 0.6 |
|  | 7 | 0.7 |
|  | 20 | 0.4 |
|  | 42 | 0.8 |
| CA_1-7-28-40 | 1 | 0.6 |
|  | 7 | 0.8 |
|  | 28 | 0.6 |
|  | 40 | 0.9 |
| CA_1-8-11-2811 | 1 | 0.3 |
|  | 8 | 0.6 |
|  | 11 | 0.4 |
|  | 28 | 0.6 |
| CA_1-8-11-42 | 1 | 0.3 |
|  | 8 | 0.6 |
|  | 11 | 0.4 |
|  | 42 | 0.8 |
| CA_1-8-20-28 | 1 | 0.3 |
|  | 8 | 0.6 |
|  | 20 | 0.6 |
|  | 28 | 0.6 |
| CA_1-19-21-42 | 1 | 0.3 |
|  | 19 | 0.3 |
|  | 21 | 0.4 |
|  | 42 | 0.8 |
| CA_1-20-32-42 | 1 | 0.5 |
|  | 20 | 0.4 |
|  | 42 | 0.8 |
| CA_1-20-32-43 | 1 | 0.5 |
|  | 20 | 0.3 |
|  | 43 | 0.8 |
| CA_1-21-28-42 | 1 | 0.3 |
|  | 21 | 0.4 |
|  | 28 | 0.6 |
|  | 42 | 0.8 |
| CA_1-32-42-4313 | 1 | 0.5 |
|  | 42 | 0.8 |
|  | 43 | 0.8 |
| CA_2-4-5-12 | 2 | 0.5 |
|  | 4 | 0.5 |
|  | 5 | 0.8 |
|  | 12 | 0.8 |
| CA_2-4-5-29 | 2 | 0.5 |
|  | 4 | 0.5 |
|  | 5 | 0.5 |
| CA_2-4-5-30 | 2 | 0.5 |
|  | 4 | 0.5 |
|  | 5 | 0.3 |
|  | 30 | 0.3 |
| CA_2-4-7-12 | 2 | 0.5 |
|  | 4 | 0.5 |
|  | 7 | 0.5 |
|  | 12 | 0.8 |
| CA_2-4-12-30 | 2 | 0.5 |
|  | 4 | 0.5 |
|  | 12 | 0.8 |
|  | 30 | 0.3 |
| CA_2-4-29-30 | 2 | 0.5 |
|  | 4 | 0.5 |
|  | 30 | 0.3 |
| CA_2-5-7-28 | 2 | 0.5 |
|  | 5 | 0.6 |
|  | 7 | 0.6 |
|  | 28 | 0.6 |
| CA_2-5-12-66, CA_2-2-5-12-66 | 2 | 0.5 |
|  | 5 | 0.8 |
|  | 12 | 0.5 |
|  | 66 | 0.8 |
| CA_2-5-30-66, CA_2-2-5-30-66, CA_2-5-30-66-66 | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
| CA_2-5-46-66, CA_2-5-46-66-66 | 2 | 0.5 |
|  | 5 | 0.3 |
|  | 66 | 0.5 |
| CA_2-7-46-66 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 46 | 0 |
|  | 66 | 0.5 |
| CA_2-7-13-66 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 13 | 0.3 |
|  | 66 | 0.5 |
| CA_2-7-26-66 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 26 | 0.3 |
|  | 66 | 0.5 |
| CA_2-12-30-66, CA_2-2-12-30-66, CA_2-12-30-66-66 | 2 | 0.5 |
|  | 12 | 0.8 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
| CA_2-13-46-66, CA_2-13-46-66-66 | 2 | 0.5 |
|  | 13 | 0.3 |
|  | 66 | 0.5 |
| CA_2-13-48-66, CA_2-13-48-48-66 | 2 | 0.6 |
|  | 13 | 0.3 |
|  | 48 | 0.8 |
|  | 66 | 0.6 |
| CA_2-14-30-66, CA_2-2-14-30-66, CA_2-14-30-66-66 | 2 | 0.5 |
|  | 14 | 0.3 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
| CA_2-7-12-66, CA_2-2-7-12-66 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 12 | 0.8 |
|  | 66 | 0.5 |
| CA_2-7-29-66, CA_2-7-7-29-66 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
| CA_2-29-30-66 | 2 | 0.5 |
|  | 30 | 0.3 |
|  | 66 | 0.5 |
| CA_2-46-48-66 | 2 | 0.6 |
|  | 48 | 0.8 |
|  | 66 | 0.6 |
| CA_3-5-7-28, CA_3-3-5-7-28 | 3 | 0.5 |
|  | 5 | 0.5 |
|  | 7 | 0.5 |
|  | 28 | 0.5 |
| CA_3-7-8-20 | 3 | 0.5 |
|  | 7 | 0.5 |
|  | 8 | 0.4 |
|  | 20 | 0.4 |
| CA_3-7-8-38 | 3 | 0.5 |
|  | 7 | 0.5 |
|  | 8 | 0.5 |
|  | 38 | 0.5 |
| CA_3-7-8-40 | 3 | 0.6 |
|  | 7 | 0.8 |
|  | 8 | 0.6 |
|  | 40 | 0.9 |
| CA_3-7-20-28 | 3 | 0.5 |
|  | 7 | 0.5 |
|  | 20 | 0.6 |
|  | 28 | 0.5 |
| CA_3-7-20-32 | 3 | 0.7 |
|  | 7 | 0.7 |
|  | 20 | 0.3 |
| CA_3-7-20-42 | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 20 | 0.3 |
|  | 42 | 0.8 |
| CA_3-7-28-38 | 3 | 0.5 |
|  | 7 | 0.5 |
|  | 28 | 0.5 |
|  | 38 | 0.5 |
| CA_3-7-28-40 | 3 | 0.6 |
|  | 7 | 0.8 |
|  | 28 | 0.3 |
|  | 40 | 0.9 |
| CA_3-7-32-46 | 3 | 0.7 |
|  | 7 | 0.7 |
| CA_3-8-11-2812 | 3 | 0.8 |
|  | 8 | 0.6 |
|  | 11 | 0.9 |
|  | 28 | 0.6 |
| CA_3-8-20-28 | 3 | 0.3 |
|  | 8 | 0.6 |
|  | 20 | 0.5 |
|  | 28 | 0.5 |
| CA_3-19-21-42 | 3 | 0.8 |
|  | 19 | 0.3 |
|  | 21 | 0.9 |
|  | 42 | 0.8 |
| CA_3-20-32-42 | 3 | 0.6 |
|  | 20 | 0.4 |
|  | 42 | 0.8 |
| CA_3-20-32-43 | 3 | 0.5 |
|  | 20 | 0.3 |
|  | 43 | 0.8 |
| CA_3-21-28-42 | 3 | 0.8 |
|  | 21 | 0.9 |
|  | 28 | 0.5 |
|  | 42 | 0.8 |
| CA_3-28-41-42 | 3 | 1 |
|  | 28 | 0.5 |
|  | 41 | 0.37/0.88 |
|  | 42 | 0.8 |
| CA_3-32-42-4313 | 3 | 0.6 |
|  | 42 | 0.8 |
|  | 43 | 0.8 |
| NOTE 1: The above additional tolerances are only applicable for the E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations. NOTE 2: The above additional tolerances also apply in non-aggregated operation for the supported E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations. NOTE 3: Tolerances for a UE supporting multiple 4DL inter-band CA configurations are FFS. NOTE 4: The above additional tolerances applicable for the E-UTRA operating bands that belong to the supported highest order inter-band carrier aggregation configuration, also applies to the same E-UTRA operating bands that belong to a supported lower order CA configuration. NOTE 5: For UE supporting E-UTRA band 65 and CA configurations including Band 1, the Band 65 ΔTIB,c is the max(Band 65 ΔTIB,c , Band 1 ΔTIB,c). NOTE 6: For UE supporting E-UTRA band 42, 43 or 48 and CA configurations including Band 42, 43 or 48, the applicable ΔTIB,c in Band 42, 43, or 48 is the max(Band 42 ΔTIB,c , Band 43 ΔTIB,c, Band 48 ΔTIB,c). NOTE 7: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 8: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. NOTE 9: The values in the table reflect what can be achieved with the present state of the art technology. They shall be reconsidered when the state of the art technology progresses. NOTE 10: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 1, Band 3 or Band 8. NOTE 11: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 1, Band 8 or Band 11. NOTE 12: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 3, Band 8 or Band 11. NOTE 13: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx among TDD bands. |  |  |

Table 6.2.5-5: ΔTIB,c (five bands)

| E-UTRA operating band combination | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1-3-5-7-28 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 5 | 0.5 |
|  | 7 | 0.6 |
|  | 28 | 0.6 |
| CA_1-3-7-8-20 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 8 | 0.6 |
|  | 20 | 0.6 |
| CA_1-3-7-20-28 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 7 | 0.6 |
|  | 20 | 0.6 |
|  | 28 | 0.6 |
| CA_1-3-7-20-32 | 1 | 0.7 |
|  | 3 | 0.7 |
|  | 7 | 0.7 |
|  | 20 | 0.3 |
| CA_1-3-7-20-42 | 1 | 0.7 |
|  | 3 | 0.7 |
|  | 7 | 0.7 |
|  | 20 | 0.3 |
|  | 42 | 0.8 |
| CA_1-3-8-11-28 | 1 | 0.3 |
|  | 3 | 0.8 |
|  | 8 | 0.6 |
|  | 11 | 0.9 |
|  | 28 | 0.6 |
| CA_1-3-20-32-42 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 20 | 0.4 |
|  | 42 | 0.8 |
| CA_1-3-20-32-43 | 1 | 0.5 |
|  | 3 | 0.5 |
|  | 20 | 0.3 |
|  | 43 | 0.8 |
| CA_1-3-32-42-43 | 1 | 0.6 |
|  | 3 | 0.6 |
|  | 42 | 0.8 |
|  | 43 | 0.8 |
| NOTE 1: The above additional tolerances are only applicable for the E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations. NOTE 2: The above additional tolerances also apply in non-aggregated operation for the supported E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations. NOTE 3: Tolerances for a UE supporting multiple 5DL inter-band CA configurations are FFS. NOTE 4: The above additional tolerances applicable for the E-UTRA operating bands that belong to the supported highest order inter-band carrier aggregation configuration, also applies to the same E-UTRA operating bands that belong to a supported lower order CA configuration. |  |  |

NOTE:	The above additional tolerances do not apply to supported UTRA operating bands with frequency range below 1 GHz that correspond to the E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations when such bands are belonging only to band combination(s) where one band is <1GHz and other bands are >1.7GHz and there is no harmonic relationship between the low band UL and high band DL. Otherwise the above additional tolerances also apply to supported UTRA operating bands that correspond to the E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations.

### 6.2.5A	Configured transmitted power for CA

For uplink carrier aggregation the UE is allowed to set its configured maximum output power PCMAX,c for serving cell c and its total configured maximum output power PCMAX.

The configured maximum output power PCMAX,c  on serving cell c shall be set as specified in subclause 6.2.5.

For uplink inter-band carrier aggregation, MPRc and A-MPRc apply per serving cell c and are specified in subclause 6.2.3 and subclause 6.2.4, respectively. P-MPR c accounts for power management for serving cell c. PCMAX,c  is calculated under the assumption that the transmit power is increased independently on all component carriers.

For uplink intra-band contiguous and non-contiguous carrier aggregation, MPRc = MPR and A-MPRc = A-MPR with MPR and A-MPR specified in subclause 6.2.3A and subclause 6.2.4A respectively. There is one power management term for the UE, denoted P-MPR, and P-MPR c = P-MPR. PCMAX,c  is calculated under the assumption that the transmit power is increased by the same amount in dB on all component carriers.

The total configured maximum output power PCMAX shall be set within the following bounds:

PCMAX_L ≤ PCMAX ≤ PCMAX_H

For uplink inter-band carrier aggregation with one serving cell c per operating band when same TTI pattern is used in all aggregated serving cells,

PCMAX_L = MIN {10log10∑ MIN [ pEMAX,c/ (tC,c),  pPowerClass/(mprc·a-mprc·tC,c ·tIB,c·tProSe) , pPowerClass/pmprc], PPowerClass}

PCMAX_H = MIN{10 log10 ∑ pEMAX,c , PPowerClass}

where

-	pEMAX,c is the linear value of PEMAX, c which is given by IE P-Max for serving cell c in [7];

-	PPowerClass is the maximum UE power specified in Table 6.2.2A-1 without taking into account the tolerance specified in the Table 6.2.2A-1; pPowerClass is the linear value of PPowerClass;

-	mpr c and a-mpr c are the linear values of MPR c and A-MPR c as specified in subclause 6.2.3 and subclause 6.2.4, respectively;

-	pmprc is the linear value of P-MPR c;

-	tC,c is the linear value of TC,ctC,c = 1.41 when NOTE 2 in Table 6.2.2-1 applies for a serving cell c, otherwise tC,c = 1;

-	tIB,c  is the linear value of the inter-band relaxation term TIB,c of the serving cell c as specified in Table 6.2.5-2; otherwise tIB,c

- 	tProSe is the linear value of TProSe and applies as specified in subclause 6.2.5.

For uplink intra-band contiguous and non-contiguous carrier aggregation when same TTI pattern is used in all aggregated serving cells,

PCMAX_L  = MIN{10 log10 ∑ pEMAX,c  - TC , (PPowerClass – ΔPPowerClass) – MAX(MPR + A-MPR + ΔTIB,c + TC + TProSe, P-MPR ) }

PCMAX_H  = MIN{10 log10 ∑ pEMAX,c , PPowerClass}

where

-	pEMAX,c is the linear value of PEMAX,c which is given by IE P-Max for serving cell c in [7];

-	PPowerClass is the maximum UE power specified in Table 6.2.2A-1 without taking into account the tolerance specified in the Table 6.2.2A-1;

ΔPPowerClass = 3 dB for a power class 2 capable UE operating in Band 41, when P-max of 23 dBm or lower is indicated or if the uplink/downlink configuration is 0 or 6 in the cell; otherwise, ΔPPowerClass = 0 dB

-	MPR and A-MPR are specified in subclause 6.2.3A and subclause 6.2.4A respectively;

-	TIB,c is the additional tolerance for serving cell c as specified in Table 6.2.5-2;

-	P-MPR is the power management term for the UE;

-	TC is the highest value TC,c among all serving cells c in the TREF over all Teval durations. TC,c = 1.5 dB when NOTE 2 in Table 6.2.2A-1 applies to the serving cell c, otherwise TC,c = 0 dB;

- 	TProSe applies as specified in subclause 6.2.5.

For uplink inter-band carrier aggregation with one serving cell c per operating band when at least one different TTI patterns is used in aggregated cells, the UE is allowed to set its configured maximum output power PCMAX,c(i),i for serving cell c(i) of TTI length i, i = 1,2,3 and its total configured maximum output power PCMAX.

The configured maximum output power PCMAX,c(i),i (p) in TTI p of serving cell c(i) on TTI length i shall be set within the following bounds:

PCMAX_L,c(i),i (p) ≤  PCMAX,c(i), i (p) ≤  PCMAX_H,c(i),i (p)

where PCMAX_L,c(i),i (p) and PCMAX_H,c(i),i (p) are the limits for a serving cell c(i) of TTI length i as specified in subclause 6.2.5.

The total UE configured maximum output power PCMAX (p,q,k) in a TTI p of TTI length 1 ,  a TTI q of TTI length 2 and a TTI k of TTI length 3 that overlap in time shall be set within the following bounds unless stated otherwise:

PCMAX_L(p,q,k) ≤  PCMAX (p,q,k)  ≤  PCMAX_H (p,q,k)

When p, q, k are of different lengths and belong to different cells:

PCMAX_L (p,q,k) = MIN {10 log10 [pCMAX_L,c(1),1 (p) + pCMAX_L,c(2),2 (q)+ + pCMAX_L,c(3),3 (k)], PPowerClass}

PCMAX_H (p,q,k) = MIN {10 log10 [pCMAX_H,c(1),1 (p) + pCMAX_H,c(2),2 (q) + pCMAX_H,c(3),3 (k)], PPowerClass}

where pCMAX_L,c(i),i and pCMAX_H,c(i),i are the respective limits PCMAX_L,c(i),iand PCMAX_H,c(i),i expressed in linear scale.

For combinations of intra-band and inter-band carrier aggregation with UE configured for transmission on three serving cells (up to two contiguously aggregated carriers per operating band),

For the case when p and q belong to the same band and k belongs to a different band but p, q and k are of the same TTI pattern.

PCMAX_L = MIN {10log10∑(pCMAX_L, Bi), PPowerClass}

PCMAX_H = MIN{10 log10 ∑ pEMAX,c , PPowerClass}

For the case when p and q belong to the same band and are of the same TTI pattern while k belong to a different band and is of different TTI pattern then:

PCMAX_L (p,q,k) = MIN {10 log10 [pCMAX_L,Bi(p) + pCMAX_L,c(3),3 (k)], PPowerClass}

PCMAX_H (p,q,k) = MIN {10 log10 [pCMAX_H,Bi (p) + pCMAX_H,c(3),3 (k)], PPowerClass}

where

-	pEMAX,c is the linear value of PEMAX, c which is given by IE P-Max for serving cell c in [7];

-	PPowerClass is the maximum UE power specified in Table 6.2.2A-0 without taking into account the tolerance specified in the Table 6.2.2A-0; pPowerClass is the linear value of PPowerClass;

- 	pCMAX_L, Bi is the linear values of PCMAX_L as specified in corresponding operating band. PCMAX_L,c specified for single carrier in subclause 6.2.5 applies for operating band supporting one serving cell. PCMAX_L specified for uplink intra-band contiguous carrier aggregation in subclause 6.2.5A applies for operating band supporting two contiguous serving cells.

- 	intra-band carriers use the same TTI patterns.

TREF and Teval are specified in Table 6.2.5A-0 when same and different TTI patterns are used in aggregated carriers. For each TREF, the PCMAX_L is evaluated per Teval and given by the minimum value taken over the transmission(s) within the Teval; the minimum PCMAX_L over the one or more Teval is then applied for the entire TREF. PPowerClass shall not be exceeded by the UE during any period of time.

Table 6.2.5A-0: PCMAX evaluation window for different TTI patterns

| TTI duration | TREF | Teval |
| --- | --- | --- |
| Different TTI duration in different aggregated carrier | TREF of longer TTI | Teval of shortest TTI |

If the UE is configured with multiple TAGs and transmissions of the UE on TTI i for any serving cell in one TAG overlap some portion of the first symbol of the transmission on TTI i +1 for a different serving cell in another TAG, the UE minimum of PCMAX_L for TTIs i and i + 1 applies for any overlapping portion of TTIs i and i + 1. PPowerClass shall not be exceeded by the UE during any period of time.

In case PC2 and uplink intra-band contiguous CA capable UE receives pEMAX,c in Scell then that applies both to Scell and Pcell once the Scell is activated.

The measured maximum output power PUMAX over all serving cells with same TTI pattern shall be within the following range:

PCMAX_L  – MAX{TL, TLOW(PCMAX_L) }  ≤  PUMAX  ≤  PCMAX_H  +  THIGH(PCMAX_H)

PUMAX = 10 log10 ∑ pUMAX,c

where pUMAX,c  denotes the measured maximum output power for serving cell c expressed in linear scale. The tolerances TLOW(PCMAX) and THIGH(PCMAX) for applicable values of PCMAX are specified in Table 6.2.5A-1 and Table 6.2.5A-2 for inter-band carrier aggregation and intra-band carrier aggregation, respectively. The tolerance TL is the absolute value of the lower tolerance for applicable E-UTRA CA configuration as specified in Table 6.2.2A-0, Table 6.2.2A-1 and Table 6.2.2A-2 for inter-band carrier aggregation, intra-band contiguous carrier aggregation and intra-band non-contiguous carrier aggregation, respectively.

The measured maximum output power PUMAX over all serving cells, when atleast one TTI has a different TTI pattern, shall be within the following range:

P’CMAX_L–  MAX{TL, TLOW (P’CMAX_L)} ≤  P’UMAX  ≤  P’CMAX_H + THIGH (P’CMAX_H)

P’UMAX = 10 log10 ∑ p’UMAX,c

where p’UMAX,c  denotes the average measured maximum output power for serving cell c expressed in linear scale over TREF. The tolerances TLOW(P’CMAX) and THIGH(P’CMAX) for applicable values of P’CMAX are specified in Table 6.2.5A-1 and Table 6.2.5A-2 for inter-band carrier aggregation and intra-band carrier aggregation, respectively. The tolerance TL is the absolute value of the lower tolerance for applicable E-UTRA CA configuration as specified in Table 6.2.2A-0, Table 6.2.2A-1 and Table 6.2.2A-2 for inter-band carrier aggregation, intra-band contiguous carrier aggregation and intra-band non-contiguous carrier aggregation, respectively.

where:

P’CMAX_L  = MIN{ MIN {10log10∑(pCMAX_L, Bi), PPowerClass} over all overlapping TTIs in TREF}

P’CMAX_H = MAX{ MIN{10 log10 ∑ pEMAX,c , PPowerClass} over all overlapping TTIs in TREF}

Table 6.2.5A-1: PCMAX tolerance for uplink inter-band CA (two bands)

| PCMAX (dBm) | Tolerance TLOW(PCMAX) (dB) | Tolerance THIGH(PCMAX) (dB) |
| --- | --- | --- |
| PCMAX = 23 | 3.0 | 2.0 |
| 22 ≤ PCMAX < 23 | 5.0 | 2.0 |
| 21 ≤ PCMAX < 22 | 5.0 | 3.0 |
| 20 ≤ PCMAX < 21 | 6.0 | 4.0 |
| 16 ≤ PCMAX < 20 | 5.0 |  |
| 11 ≤ PCMAc < 16 | 6.0 |  |
| -40 ≤ PCMAX < 11 | 7.0 |  |

Table 6.2.5A-2: PCMAX tolerance for uplink intra-band CA

| PCMAX (dBm) | Tolerance TLOW(PCMAX) (dB) | Tolerance THIGH(PCMAX) (dB) |
| --- | --- | --- |
| 21 ≤ PCMAX ≤ 26 | 2.0 |  |
| 20 ≤ PCMAX < 21 | 2.5 |  |
| 19 ≤ PCMAX < 20 | 3.5 |  |
| 18 ≤ PCMAX < 19 | 4.0 |  |
| 13 ≤ PCMAX < 18 | 5.0 |  |
| 8 ≤ PCMAX < 13 | 6.0 |  |
| -40 ≤ PCMAX < 8 | 7.0 |  |

### 6.2.5B	Configured transmitted power for UL-MIMO

For UE supporting UL-MIMO, the transmitted power is configured per each UE.

The definitions of configured maximum output power PCMAX,c, the lower bound PCMAX_L,c, and the higher bound PCMAX_H,c specified in subclause 6.2.5 shall apply to UE supporting UL-MIMO, where

-	PPowerClass, ΔPPowerClass and TC,c are specified in subclause 6.2.2B;

-	MPR,c is specified in subclause 6.2.3B;

-	A-MPR,c is specified in subclause 6.2.4B.

The measured configured maximum output power PUMAX,c for serving cell c shall be within the following bounds:

PCMAX_L,c  –  MAX{TL, T LOW(PCMAX_L,c)}  ≤  PUMAX,c  ≤  PCMAX_H,c  +  T HIGH(PCMAX_H,c)

where TLOW(PCMAX_L,c) and THIGH(PCMAX_H,c) are defined as the tolerance and applies to PCMAX_L,c and PCMAX_H,c separately, while TL is the absolute value of the lower tolerance in Table 6.2.2B-1 for the applicable operating band.

For UE with two transmit antenna connectors in closed-loop spatial amultiplexing scheme, the tolerance is specified in Table 6.2.5B-1. The requirements shall be met with UL-MIMO configurations specified in Table 6.2.2B-2.

Table 6.2.5B-1: PCMAX,c tolerance in closed-loop spatial multiplexing scheme

| PCMAX,c (dBm) | Tolerance TLOW(PCMAX_L,c) (dB) | Tolerance THIGH(PCMAX_H,c) (dB) |
| --- | --- | --- |
| PCMAX,c =23 | 3.0 | 2.0 |
| 22 ≤ PCMAX,c < 23 | 5.0 | 2.0 |
| 21 ≤ PCMAX,c < 22 | 5.0 | 3.0 |
| 20 ≤ PCMAX,c < 21 | 6.0 | 4.0 |
| 16 ≤ PCMAX,c < 20 | 5.0 |  |
| 11 ≤ PCMAX,c < 16 | 6.0 |  |
| -40 ≤ PCMAX,c < 11 | 7.0 |  |

If UE is configured for transmission on single-antenna port, the requirements in subclause 6.2.5 apply.

### 6.2.5C	Configured transmitted power for Dual Connectivity

For inter-band dual connectivity with one uplink serving cell per CG, the UE is allowed to set its configured maximum output power PCMAX,c(i),i for serving cell c(i) of CG i, i = 1,2, and its total configured maximum output power PCMAX.

The configured maximum output power PCMAX,c(i),i (p) in subframe p of  serving cell c(i) on CG i shall be set within the following bounds:

PCMAX_L,c(i),i (p) ≤  PCMAX,c(i), i (p) ≤  PCMAX_H,c(i),i (p)

where PCMAX_L,c(i),i (p) and PCMAX_H,c(i),i (p) are the limits for a serving cell c(i) of CG i as specified in subclause 6.2.5.

The total UE configured maximum output power PCMAX (p,q) in a subframe p of CG 1 and a subframe q of CG 2 that overlap in time shall be set within the following bounds for synchronous and asynchronous operation unless stated otherwise:

PCMAX_L(p,q) ≤  PCMAX (p,q)  ≤  PCMAX_H (p,q)

with

PCMAX_L (p,q) = MIN {10 log10 [pCMAX_L,c(1),1 (p) + pCMAX_L,c(2),2 (q)], PPowerClass}

PCMAX_H (p,q) = MIN {10 log10 [pCMAX_H,c(1),1 (p) + pCMAX_H,c(2),2 (q)], PPowerClass}

where pCMAX_L,c(i),i is pCMAX_H,c(i),i are the respective limits PCMAX_L,c(i),i (p) and PCMAX_H,c(i),i (p) expressed in linear scale.

If the UE is configured in Dual Connectivity and synchronous transmissions of the UE on subframe p for a serving cell in one CG overlaps some portion of the first symbol of the transmission on subframe q +1 for a different serving cell in the other CG, the UE minimum of PCMAX_L between subframes pairs (p, q) and (p+1, q +1) respectively applies for any overlapping portion of subframes (p, q) and (p +1, q+1). PPowerClass shall not be exceeded by the UE during any period of time.

The measured total maximum output power PUMAX over both CGs is

PUMAX = 10 log10 [pUMAX,c(1),1 + pUMAX,c(2),2],

where pUMAX,c(i),i  denotes the measured output power of serving cell c(i) of CG i expressed in linear scale.

If the UE is configured in Dual Connectivity and synchronous transmissions

PCMAX_L(p, q)   –  TLOW (PCMAX_L(p, q))  ≤  PUMAX  ≤  PCMAX_H(p, q)  + THIGH (PCMAX_H(p, q))

where PCMAX_L (p,q) and PCMAX_H (p,q) are the limits for the pair (p,q) and with the tolerances TLOW(PCMAX) and THIGH(PCMAX) for applicable values of PCMAX specified in Table 6.2.5C-1. PCMAX_L may be modified for any overlapping portion of subframes (p, q) and (p +1, q+1).

If the UE is configured in Dual Connectivity and asynchronous transmissions, the subframes of the leading CG are taken as reference subframes for the measurement of the total configured output power PUMAX.  If subframe p of CG 1 and subframe q of CG 2 overlap in time in their respective slot 0 and

1.	if p leads in time over q, then p is the reference subframe and the (p,q) and (p,q-1) pairs are considered for determining the PCMAX tolerance

2.	if q leads in time over p, then q is the reference subframe and the (p-1,q) and (p,q) pairs are considered for determining the PCMAX tolerance;

for the reference subframe p duration (when subframe p in CG 1 leads):

P’CMAX_L   = MIN {PCMAX_L   (p,q) , PCMAX_L  (p,q-1)}

P’CMAX_H  = MAX {PCMAX_H   (p,q) , PCMAX_H  (p,q-1)}

while for the reference subframe q duration (when subframe q in CG 2 leads):

P’CMAX_L   = MIN {PCMAX_L   (p-1,q) , PCMAX_L  (p,q)}

P’CMAX_H  = MAX {PCMAX_H   (p-1,q) , PCMAX_H  (p,q)}

where PCMAX_L   and PCMAX_H are the applicable limits for each overlapping subframe pairs  (p,q) , (p, q-1) and (p-1,q). The measured total configured maximum output power PUMAX shall be within the following bounds:

P’CMAX_L   –  TLOW (P’CMAX_L)  ≤  PUMAX  ≤  P’CMAX_H + THIGH (P’CMAX_H)

with the tolerances TLOW(PCMAX) and THIGH(PCMAX) for applicable values of PCMAX specified in Table 6.2.5C-1.

Table 6.2.5C-1: PCMAX tolerance for inter-band Dual Connectivity

| PCMAX(dBm) | Tolerance TLOW(PCMAX_L )(dB) | Tolerance THIGH ( PCMAX_H )(dB) |
| --- | --- | --- |
| PCMAX = 23 | 3.0 | 2.0 |
| 22 ≤PCMAX,< 23 | 5.0 | 2.0 |
| 21 ≤ PCMAX< 22 | 5.0 | 3.0 |
| 20 ≤ PCMAX, < 21 | 6.0 | 4.0 |
| 16 ≤ PCMAX < 20 | 5.0 |  |
| 11 ≤ PCMAX, < 16 | 6.0 |  |
| -40 ≤ PCMAX < 11 | 7.0 |  |

### 6.2.5D	Configured transmitted power for ProSe

When UE is configured for E-UTRA ProSe sidelink transmissions non-concurrent with E-UTRA uplink transmissions for E-UTRA ProSe operating bands specified in Table 5.5D-1, the configured maximum output power PCMAX,c and power boundary requirement specified in subclause 6.2.5 shall apply to UE supporting ProSe, where

-	MPRc is specified in subclause 6.2.3D;

-	A-MPRc is specified in subclause 6.2.4D;

-	TProSe = 0.1 dB.

For  and, PEMAX,c is the value given by IE P-Max for serving cell c, defined by [7], when present. PEMAX,c is the value given by IE maxTxPower, defined by [7], when the UE is not associated with a serving cell on the ProSe carrier .

For, PEMAX,c is the value given by the IE discMaxTxPower in [7].

For, PEMAX,c is the value given by the IE maxTxPower in [7] when the ProSe UE is not associated with a serving cell on the ProSe carrier. When the UE is associated with a serving cell, then PEMAX,c is the value given by the IE P-Max when PSBCH/SLSS transmissions is triggered for ProSe Direct communication as specified in [7], and is the value given by the IE discMaxTxPower in [7] otherwise.

For, the value is as calculated for  and applying the MPR for SSSS as specified in Section 6.2.3D.

When a UE is configured for simultaneous E-UTRA ProSe sidelink and E-UTRA uplink transmissions for inter-band E-UTRA ProSe / E-UTRA bands specified in Table 5.5D-2, the UE is allowed to set its configured maximum output power PCMAX,c,E-UTRA and PCMAX,c,ProSe for the configured E-UTRA uplink carrier and the configured E-UTRA ProSe carrier, respectively, and its total configured maximum output power PCMAX,c.

The configured maximum output power PCMAX c,E-UTRA(p) in subframe p for the configured E-UTRA uplink carrier shall be set within the bounds:

PCMAX_L,c,E-UTRA (p) ≤  PCMAX,c,E-UTRA (p) ≤  PCMAX_H,c,E-UTRA (p)

where PCMAX_L,c,E-UTRA and PCMAX_H,c,E-UTRA are the limits for a serving cell c as specified in subclause 6.2.5.

The configured maximum output power PCMAX c,ProSe (q) in subframe q for the configured E-UTRA ProSe carrier shall be set within the bounds:

PCMAX,c,ProSe (q) ≤  PCMAX_H,c,ProSe (q)

where PCMAX_H,c,ProSe is the limit as specified in subclause 6.2.5D.

The total UE configured maximum output power PCMAX (p,q) in a subframe p of an E-UTRA uplink carrier and a subframe q of an E-UTRA ProSe sidelink that overlap in time shall be set within the following bounds for synchronous and asynchronous operation unless stated otherwise:

PCMAX_L (p,q) ≤  PCMAX (p,q)  ≤  PCMAX_H (p,q)

with

PCMAX_L (p,q) =  PCMAX_L,c,E-UTRA (p)

PCMAX_H (p,q) = MIN {10 log10 [pCMAX_H,c,E-UTRA (p) + pCMAX_H,c,ProSe (q)], PPowerClass}

where pCMAX_H,c,ProSe and pCMAX_H,c,E-UTRA are the limits PCMAX_H,c,ProSe (q) and PCMAX_H,c,E-UTRA (p) expressed in linear scale.

The measured total maximum output power PUMAX over both the E-UTRA uplink and E-UTRA ProSe carriers is

PUMAX = 10 log10 [pUMAX,c,E-UTRA + pUMAX,c,ProSe],

where pUMAX,c,E-UTRA  denotes the measured output power of serving cell c for the configured E-UTRA uplink carrier, and pUMAX,c,ProSe denotes the measured output power for the configured E-UTRA ProSe carrier expressed in linear scale.

When a UE is configured for synchronous ProSe and uplink transmissions,

PCMAX_L(p, q)   –  TLOW (PCMAX_L(p, q))  ≤  PUMAX  ≤  PCMAX_H(p, q)  + THIGH (PCMAX_H(p, q))

where PCMAX_L (p,q) and PCMAX_H (p,q) are the limits for the pair (p,q) and with the tolerances TLOW(PCMAX) and THIGH(PCMAX) for applicable values of PCMAX specified in Table 6.2.5C-1. PCMAX_L may be modified for any overlapping portion of subframes (p, q) and (p +1, q+1).

When a UE is configured for asynchronous ProSe and uplink transmissions, the carrier configured for uplink transmission is taken as the reference. If subframe p for the E-UTRA uplink carrier and subframe q for the E-UTRA ProSe carrier overlap in time and

1.	if uplink carrier leads in time over q, then p is the reference subframe and, the (p,q) and (p,q-1) pairs are considered for determining the PCMAX tolerance

2.	if ProSe carrier leads in time over p, then p is the reference subframe and, the (p,q) and (p,q+1) pairs are considered for determining the PCMAX tolerance

For the reference subframe p duration when uplink carrier leads:

P'CMAX_L   = PCMAX_L,,cE-UTRA (p)

P'CMAX_H  = MAX {PCMAX_H   (p,q-1) , PCMAX_H  (p,q)}

For the reference subframe p duration when ProSe carrier leads:

P'CMAX_L   = PCMAX_L,cE-UTRA (p)

P'CMAX_H  = MAX {PCMAX_H   (p,q) , PCMAX_H  (p,q+1)}

where PCMAX_L,,cE-UTRA (p) and PCMAX_H are the applicable limits for each overlapping subframe pairs (p,q) , (p, q+1) , (p, q-1). The measured total configured maximum output power PUMAX shall be within the following bounds:

P’CMAX_L   –  TLOW (P’CMAX_L)  ≤  PUMAX  ≤  P’CMAX_H + THIGH (P’CMAX_H)

with the tolerances TLOW(PCMAX) and THIGH(PCMAX) for applicable values of PCMAX specified in Table 6.2.5C-1.

### 6.2.5F	Configured transmitted Power for category NB1 and NB2

For each slot i the category NB1and NB2 UE is allowed to set its configured maximum output power PCMAX,c. The configured maximum output power PCMAX,c is set within the following bounds:

PCMAX_L,c ≤  PCMAX,c  ≤  PCMAX_H,c

Where

	PCMAX_L,c = MIN { PEMAX,c ,  PPowerClass – MPRc – A-MPRc}

	PCMAX_H,c = MIN { PEMAX,c,  PPowerClass}

	PEMAX,c is the value given to IE P-Max, defined in [7]

	PPowerClass is the maximum category NB1 and NB2 UE power specified in Table 6.2.2F-1 without taking into account the associated tolerance

	MPRc is specified in subclause 6.2.3F

	A-MPRc = 0dB unless otherwise stated.

The measurement period for PUMAX,c is at least one sub-frame (1ms) for 15 KHz channel spacing, and at least a 2ms slot (excluding the 2304Ts gap when UE is not transmitting) respectively for the 3.75 KHz channel spacing. The measured maximum output power PUMAX,c shall be within the following bounds:

PCMAX_L,c –  T(PCMAX_L,c)  ≤  PUMAX,c  ≤  PCMAX_H,c  +  T(PCMAX_H,c)

Where T(PCMAX) is defined by the tolerance table below and applies to PCMAX_L,c and PCMAX_H,c separately.

Table 6.2.5F-1: PCMAX tolerance for power class 3

| PCMAX (dBm) | Tolerance T(PCMAX) (dB) |
| --- | --- |
| 21 ≤ PCMAX ≤ 23 | 2.0 |
| 20 ≤ PCMAX < 21 | 2.5 |
| 19 ≤ PCMAX < 20 | 3.5 |
| 18 ≤ PCMAX < 19 | 4.0 |
| 13 ≤ PCMAX < 18 | 5.0 |
| 8 ≤ PCMAX < 13 | 6.0 |
| -40 ≤ PCMAX < 8 | 7.0 |

Table 6.2.5F-2: PCMAX tolerance for power class 5

| PCMAX,c (dBm) | Tolerance T(PCMAX,c) (dB) |
| --- | --- |
| 18 ≤ PCMAX ≤ 20 | 2.0 |
| 17 ≤ PCMAX,c < 18 | 2.5 |
| 16 ≤ PCMAX,c < 17 | 3.5 |
| 15 ≤ PCMAX,c < 16 | 4.0 |
| 10 ≤ PCMAX,c < 15 | 5.0 |
| 5 ≤ PCMAX,c < 10 | 6.0 |
| -40 ≤ PCMAX,c < 5 | 7.0 |

Table 6.2.5F-3: PCMAX tolerance for power class 6

| PCMAX,c (dBm) | Tolerance T(PCMAX,c) (dB) |
| --- | --- |
| 13 ≤ PCMAX,c ≤ 14 | 2.5 |
| 12 ≤ PCMAX,c < 13 | 3.5 |
| 11 ≤ PCMAX,c < 12 | 4 |
| 8 ≤ PCMAX,c < 11 | 5 |
| 5 ≤ PCMAX,c < 8 | 6.0 |
| -40 ≤ PCMAX,c < 5 | 7.0 |

### 6.2.5G	Configured transmitted power for V2X Communication

When UE is configured for E-UTRA V2X sidelink transmissions non-concurrent with E-UTRA uplink transmissions for E-UTRA V2X  operating bands specified in Table 5.5G-1, the V2X UE is allowed to set its configured maximum output power PCMAX,c for component carrier c. The configured maximum output power PCMAX,c is set within the following bounds:

PCMAX_L,c ≤  PCMAX,c  ≤  PCMAX_H,c with

PCMAX_L,c = MIN {PEMAX,c – TC,c,  PPowerClass –– MAX(MPRc + A-MPRc + ΔTIB,c + TC,c + TProSe, P-MPRc), PRegulatory,c }

PCMAX_H,c = MIN {PEMAX,c,  PPowerClass,  PRegulatory,c }

where

-	For the total transmitted power PCMAX,c of PSSCH and PSCCH, PEMAX,c is the value given by IE maxTxPower, defined by [7], when the UE is not associated with a serving cell on the V2X carrier.

-	For, PEMAX,c is the value given by the IE maxTxPower in [7] when the UE is not associated with a serving cell on the V2X carrier.

-	For, the value is as calculated for  and applying the MPR for SSSS as specified in Section 6.2.3D.

-	PPowerClass is the maximum UE power specified in Table 6.2.2-1 without taking into account the tolerance specified in the Table 6.2.2-1;

-	MPRc and A-MPRc for serving cell c are specified in subclause 6.2.3G and subclause 6.2.4G, respectively;

-	TIB,c, TC,c, TProSe and P-MPRc are specified in subclause 6.2.5

-	PRegulatory,c= 10 - Gpost connector dBm when the V2X UE is within the protected zone [13] of CEN DSRC tolling system and operating in Band 47; PRegulatory,c= 33 - Gpost connector dBm otherwise.

The maximum output power PCMAX,PSSCH and PCMAX,PSCCH are derived from PCMAX,c based on the PSD offset following subclause 14.1.1.5 in [6]. For all cases, the PSD difference between PSCCH and PSSCH shall be the same as the PSD offset value.

For the measured configured maximum output power PUMAX,c for E-UTRA V2X sidelink transmissions non-concurrent with E-UTRA uplink transmissions, the same requirement as in subclause 6.2.5 shall be applied.

When a UE is configured for simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions for inter-band E-UTRA V2X / E-UTRA bands specified in Table 5.5G-2, the UE is allowed to set its configured maximum output power PCMAX,c,E-UTRA and PCMAX,c,V2X for the configured E-UTRA uplink carrier and the configured E-UTRA V2X carrier, respectively, and its total configured maximum output power PCMAX,c. The TIB,c of PCMAX,c,E-UTRA is specified in Table 6.2.5G-1.

The configured maximum output power PCMAX c,E-UTRA(p) in subframe p for the configured E-UTRA uplink carrier shall be set within the bounds:

PCMAX_L,c,E-UTRA (p) ≤  PCMAX,c,E-UTRA (p) ≤  PCMAX_H,c,E-UTRA (p)

where PCMAX_L,c,E-UTRA and PCMAX_H,c,E-UTRA are the limits for a serving cell c as specified in subclause 6.2.5.

The configured maximum output power PCMAX c,V2X (q) in subframe q for the configured E-UTRA V2X carrier shall be set within the bounds:

PCMAX,c,V2X (q) ≤  PCMAX_H,c,V2X (q)

where PCMAX_H,c,V2X is the limit as specified in subclause 6.2.5G.

The total UE configured maximum output power PCMAX (p,q) in a subframe p of an E-UTRA uplink carrier and a subframe q of an E-UTRA V2X sidelink that overlap in time shall be set within the following bounds for synchronous and asynchronous operation unless stated otherwise:

PCMAX_L (p,q) ≤  PCMAX (p,q)  ≤  PCMAX_H (p,q)

with

PCMAX_L (p,q) =  PCMAX_L,c,E-UTRA (p)

PCMAX_H (p,q) = 10 log10 [pCMAX_H,c,E-UTRA (p) + pCMAX_H,c,V2X (q)]

where pCMAX_H,c,V2X and pCMAX_H,c,E-UTRA are the limits PCMAX_H,c,V2X (q) and PCMAX_H,c,E-UTRA (p) expressed in linear scale.

The measured total maximum output power PUMAX over both the E-UTRA uplink and E-UTRA V2X carriers is

PUMAX = 10 log10 [pUMAX,c,E-UTRA + pUMAX,c,V2X],

where pUMAX,c,E-UTRA  denotes the measured output power of serving cell c for the configured E-UTRA uplink carrier, and pUMAX,c,V2X  denotes the measured output power for the configured E-UTRA V2X carrier expressed in linear scale.

When a UE is configured for synchronous V2X sidelink and uplink transmissions,

PCMAX_L(p, q)   –  TLOW (PCMAX_L(p, q))  ≤  PUMAX  ≤  PCMAX_H(p, q)  + THIGH (PCMAX_H(p, q))

where PCMAX_L (p,q) and PCMAX_H (p,q) are the limits for the pair (p,q) and with the tolerances TLOW(PCMAX) and THIGH(PCMAX) for applicable values of PCMAX specified in Table 6.2.5G-2. PCMAX_L may be modified for any overlapping portion of subframes (p, q) and (p +1, q+1).

When a UE is configured for asynchronous V2X and uplink transmissions, the subframe p for the E-UTRA uplink carrier and subframe q for the E-UTRA V2X carrier overlap in time and

1.	if uplink carrier leads in time over q and V2X UE sidelink transmission has SCI whose “Priority” field is set to a value less than the high layer parameter thresSL-TxPrioritization,  then p is the reference subframe and the (p,q) and (p,q-1) pairs are considered for determining the PCMAX tolerance

2.	if uplink carrier leads in time over q and V2X UE sidelink transmission has SCI whose “Priority” field is set to a value larger than the high layer parameter thresSL-TxPrioritization,  then q is the reference subframe and the (p,q) and (p+1,q) pairs are considered for determining the PCMAX tolerance

3.	if V2X carrier leads in time over p and V2X UE sidelink transmission has SCI whose “Priority” field is set to a value less than the high layer parameter thresSL-TxPrioritization, then p is the reference subframe and the (p,q) and (p,q+1) pairs are considered for determining the PCMAX tolerance

4.	if V2X carrier leads in time over p and V2X UE sidelink transmission has SCI whose “Priority” field is set to a value larger than the high layer parameter thresSL-TxPrioritization,, then q is the reference subframe and the (p-1,q) and (p,q) pairs are considered for determining the PCMAX tolerance

For the reference subframe p duration when uplink carrier leads:

P'CMAX_L   = PCMAX_L,,cE-UTRA (p)

P'CMAX_H  = MAX {PCMAX_H   (p,q-1) , PCMAX_H  (p,q)}

For the reference subframe p duration when V2X carrier leads:

P'CMAX_L   = PCMAX_L,c,E-UTRA (p)

P'CMAX_H  = MAX {PCMAX_H   (p,q) , PCMAX_H  (p,q+1)}

For the reference subframe q duration when uplink carrier leads:

P'CMAX_L   = PCMAX_L,c, E-UTRA (q)

P'CMAX_H  = MAX {PCMAX_H   (p,q) , PCMAX_H  (p+1,q)}

For the reference subframe q duration when V2X carrier leads:

P'CMAX_L   = PCMAX_L,c, E-UTRA (p)

P'CMAX_H  = MAX {PCMAX_H   (p-1,q) , PCMAX_H  (p,q)}

where PCMAX_L,,cE-UTRA (p) and PCMAX_H are the applicable limits for each overlapping subframe pairs above 4case with (p,q), (p, q-1) or (p,q), (p, q+1) or (p,q), (p+1,q) or (p,q), (p-1, q). The measured total configured maximum output power PUMAX shall be within the following bounds:

P’CMAX_L   –  TLOW (P’CMAX_L)  ≤  PUMAX  ≤  P’CMAX_H + THIGH (P’CMAX_H)

with the tolerances TLOW(PCMAX) and THIGH(PCMAX) for applicable values of PCMAX specified in Table 6.2.5G-2.

For intra-band contiguous multi-carrier operation, MPRc = MPR and A-MPRc = A-MPR with MPR and A-MPR specified in subclause 6.2.3G and subclause 6.2.4G respectively. There is one power management term for the UE, denoted P-MPR, and P-MPR c = P-MPR. PCMAX,c  is calculated under the assumption that the transmit power is increased by the same amount in dB on all component carriers.

The total configured maximum output power PCMAX shall be set within the following bounds:

PCMAX_L ≤ PCMAX ≤ PCMAX_H

PCMAX_L  = MIN{10 log10 ∑ pEMAX,c  - TC , PPowerClass – MAX(MPR + A-MPR + ΔTIB,c + TC + TProSe, P-MPR ), PRegulatory }

PCMAX_H  = MIN{10 log10 ∑ pEMAX,c , PPowerClass, PRegulatory }

where

-	pEMAX,c is the linear value of PEMAX,c which is given by IE maxTxPower in [7];

-	PPowerClass is the maximum UE power specified in Table 6.2.2G-1 without taking into account the tolerance specified in the Table 6.2.2G-1;

-	MPR and A-MPR are specified in subclause 6.2.3G and subclause 6.2.4G respectively;

-	TIB,c is the additional tolerance for serving cell c as specified in Table 6.2.5-2;

-	P-MPR is the power management term for the UE;

-	TC is the highest value TC,c among all serving cells c in the subframe over both timeslots. TC,c = 1.5 dB when NOTE 2 in Table 6.2.2-1 applies, otherwise TC,c = 0 dB;

- 	TProSe applies as specified in subclause 6.2.5.

-	PRegulatory= 10 - Gpost connector dBm when V2X UE is within the protected zone [13] of CEN DSRC tolling system and operating in Band 47; PRegulatory= 33 - Gpost connector dBm otherwise.

NOTE:	The supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex I.

Table 6.2.5G-1: ΔTIB,c for inter-band con-current V2X operation (two bands)

| V2X con-current band Combination | E-UTRA or V2X Operating Band | ΔTIB,c [dB] |
| --- | --- | --- |
| V2X_3-47 | 3 | 0.0 |
| V2X_5-47 | 5 | 0.2 |
| V2X_7-47 | 7 | 0.0 |
| V2X_8-47 | 8 | 0.0 |
| V2X_20-47 | 20 | 0.2 |
| V2X_28A-47A | 28 | 0.2 |
| V2X_34-47 | 34 | 0.0 |
| V2X_39-47 | 39 | 0.0 |
| V2X_41-47 | 41 | 0.0 |
| V2X_71A-47A | 71 | 0.0 |

For V2X UE supporting Transmit Diversity, the transmitted power is configured per each UE.

If the UE transmits on two antenna connectors at the same time, the tolerance is specified in Table 6.2.5G-2 and 6.2.5G-3 for PC2 and PC3 V2X UE respectively.

Table 6.2.5G-2: PCMAX,c tolerance in Transmit Diversity scheme for PC2 V2X UE

| PCMAX,c (dBm) | Tolerance TLOW(PCMAX_L,c) (dB) | Tolerance THIGH(PCMAX_H,c) (dB) |
| --- | --- | --- |
| PCMAX,c =26 | 3.0 | 2.0 |
| 23 ≤ PCMAX,c < 26 | 4.0 | 2.0 |
| 22 ≤ PCMAX,c < 23 | 5.0 | 2.0 |
| 21 ≤ PCMAX,c < 22 | 5.0 | 3.0 |
| 20 ≤ PCMAX,c < 21 | 6.0 | 4.0 |
| 16 ≤ PCMAX,c < 20 | 5.0 |  |
| 11 ≤ PCMAX,c < 16 | 6.0 |  |
| -30 ≤ PCMAX,c < 11 | 7.0 |  |

Table 6.2.5G-3: PCMAX,c tolerance in Transmit Diversity scheme for PC3 V2X UE

| PCMAX,c (dBm) | Tolerance TLOW(PCMAX_L,c) (dB) | Tolerance THIGH(PCMAX_H,c) (dB) |
| --- | --- | --- |
| PCMAX,c =23 | 3.0 | 2.0 |
| 22 ≤ PCMAX,c < 23 | 5.0 | 2.0 |
| 21 ≤ PCMAX,c < 22 | 5.0 | 3.0 |
| 20 ≤ PCMAX,c < 21 | 6.0 | 4.0 |
| 16 ≤ PCMAX,c < 20 | 5.0 |  |
| 11 ≤ PCMAX,c < 16 | 6.0 |  |
| -40 ≤ PCMAX,c < 11 | 7.0 |  |

If the UE transmits on one antenna connector at a time, the requirements in Table 6.2.5-1 apply to the active antenna connector.

## 6.3	Output power dynamics

### 6.3.1	(Void)

### 6.3.2	Minimum output power

The minimum controlled output power of the UE is defined as the broadband transmit power of the UE, i.e. the power in the channel bandwidth for all transmit bandwidth configurations (resource blocks), when the power is set to a minimum value.

#### 6.3.2.1	Minimum requirement

The minimum output power is defined as the mean power in one sub-frame (1ms). The minimum output power shall not exceed the values specified in Table 6.3.2.1-1.

Table 6.3.2.1-1: Minimum output power

|  | Channel bandwidth / Minimum output power / Measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Minimum output power | -40 dBm |  |  |  |  |  |
| Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |

### 6.3.2A		UE Minimum output power for CA

For inter-band carrier aggregation with uplink assigned to two E-UTRA bands and intra-band contiguous and non-contiguous carrier aggregation, the minimum controlled output power of the UE is defined as the transmit power of the UE per component carrier, i.e., the power in the channel bandwidth of each component carrier for all transmit bandwidth configurations (resource blocks), when the power on both component carriers are set to a minimum value.

#### 6.3.2A.1	Minimum requirement for CA

For inter-band carrier aggregation with uplink assigned to two E-UTRA bands, the minimum output power is defined per carrier and the requirement is specified in subclause 6.3.2.1. If two contiguous component carriers are assigned to one E-UTRA band, the requirements in subclause 6.3.2A.1 apply for those component carriers.

For intra-band contiguous and non-contiguous carrier aggregation the minimum output power is defined as the mean power in one sub-frame (1ms). The minimum output power shall not exceed the values specified in Table 6.3.2A.1-1.

Table 6.3.2A.1-1: Minimum output power for intra-band contiguous and non-contiguous CA UE

|  | CC Channel bandwidth / Minimum output power / Measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Minimum output power | -40 dBm |  |  |  |  |  |
| Measurement bandwidth |  |  | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |

### 6.3.2B		UE Minimum output power for UL-MIMO

For UE supporting UL-MIMO, the minimum controlled output power is defined as the broadband transmit power of the UE, i.e. the sum of the power in the channel bandwidth for all transmit bandwidth configurations (resource blocks) at each transmit antenna connector, when the UE power is set to a minimum value.

#### 6.3.2B.1	Minimum requirement

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the minimum output power is defined as the sum of the mean power at each transmit connector in one sub-frame (1ms). The minimum output power shall not exceed the values specified in Table 6.3.2B.1-1.

Table 6.3.2B.1-1: Minimum output power

|  | Channel bandwidth / Minimum output power / Measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Minimum output power | -40 dBm |  |  |  |  |  |
| Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |

If UE is configured for transmission on single-antenna port, the requirements in subclause 6.3.2 apply.

### 6.3.2C		Void

<reserved for future use>

### 6.3.2D	UE Minimum output power for ProSe

When UE is configured for E-UTRA ProSe sidelink transmissions non-concurrent with E-UTRA uplink transmissions for E-UTRA ProSe operating bands specified in Table 5.5D-1, the requirements in subclause 6.3.2 apply for ProSe transmission.

When UE is configured for simultaneous E-UTRA ProSe sidelink and E-UTRA uplink transmissions for inter-band E-UTRA ProSe / E-UTRA bands specified in Table 5.5D-2, the requirements in subclause 6.3.2A apply as specified for the corresponding inter-band aggregation with uplink assigned to two bands.

### 6.3.2F	UE Minimum output power for category NB1 and NB2

For category NB1 and NB2 UE the single-tone and multi-tone transmission minimum output power requirement for the channel bandwidth is -40 dBm. For 3.75kHz sub-carrier spacing the minimum output power is defined as mean power in one slot (2ms) excluding the 2304Ts gap when UE is not transmitting. For 15kHz sub-carrier spacing the minimum output power is defined as mean power in one sub-frame (1ms).

### 6.3.2G	UE Minimum output power for V2X Communication

When UE is configured for E-UTRA V2X sidelink transmissions non-concurrent with E-UTRA uplink transmissions for E-UTRA V2X operating bands specified in Table 5.5G-1, the minimum output power shall not exceed the values specified in Table 6.3.2G-1.

Table 6.3.2G-1: Minimum output power

|  | Channel bandwidth / Minimum output power / Measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Minimum output power | -30 dBm |  |  |  |  |  |
| Measurement bandwidth |  |  |  | 9.0 MHz |  | 18 MHz |

When UE is configured for simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions for inter-band E-UTRA V2X / E-UTRA bands specified in Table 5.5G-2, the requirements specified in subclause 6.3.2 shall apply for the uplink and the requirements specified in subclause 6.3.2G shall apply for the sidelink.

For intra-band contiguous E-UTRA V2X multiple carrier transmissions, the requirements specified in subclause 6.3.2G shall apply for each sidelink carrier.

For V2X UE supporting Transmit Diversity, if the UE transmits on two antenna connectors at the same time, the minimum output power is defined as the sum of the mean power at each transmit connector in one sub-frame (1ms). The minimum output power shall not exceed the values specified for single carrier.

If the UE transmits on aone antenna connector at a time, the requirements specified for single carrier shall apply to the active antenna connector.

### 6.3.3	Transmit OFF power

Transmit OFF power is defined as the mean power when the transmitter is OFF. The transmitter is considered to be OFF when the UE is not allowed to transmit or during periods when the UE is not transmitting a sub-frame. During DTX and measurements gaps, the UE is not considered to be OFF.

#### 6.3.3.1.	Minimum requirement

The transmit OFF power is defined as the mean power in a duration of at least one sub-frame (1ms) excluding any transient periods. The transmit OFF power shall not exceed the values specified in Table 6.3.3.1-1.

Table 6.3.3.1-1: Transmit OFF power

|  | Channel bandwidth / Transmit OFF power / Measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Transmit OFF power | -50 dBm |  |  |  |  |  |
| Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |

### 6.3.3A		UE Transmit OFF power for CA

For inter-band carrier aggregation with uplink assigned to two E-UTRA bands and intra-band contiguous and non-contiguous carrier aggregation, transmit OFF power is defined as the mean power per component carrier when the transmitter is OFF on all component carriers. The transmitter is considered to be OFF when the UE is not allowed to transmit or during periods when the UE is not transmitting a sub-frame. During measurements gaps, the UE is not considered to be OFF.

#### 6.3.3A.1	Minimum requirement for CA

For inter-band carrier aggregation with uplink assigned to two E-UTRA bands, transmit OFF power requirement is defined per carrier and the requirement is specified in subclause 6.3.3.1. If two contiguous component carriers are assigned to one E-UTRA band, the requirements in subclause 6.3.3A.1 apply for those component carriers.

For intra-band contiguous and non-contiguous carrier aggregation the transmit OFF power is defined as the mean power in a duration of at least one sub-frame (1ms) excluding any transient periods. The transmit OFF power shall not exceed the values specified in Table 6.3.3A.1-1.

Table 6.3.3A.1-1: Transmit OFF power for intra-band contiguous and non-contiguos CA UE

|  | CC Channel bandwidth / Transmit OFF power / Measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Transmit OFF power | -50 dBm |  |  |  |  |  |
| Measurement bandwidth |  |  | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |

### 6.3.3B		UE Transmit OFF power for UL-MIMO

For UE supporting UL-MIMO, the transmit OFF power is defined as the mean power at each transmit antenna connector when the transmitter is OFF at all transmit antenna connectors. The transmitter is considered to be OFF when the UE is not allowed to transmit or during periods when the UE is not transmitting a sub-frame. During DTX and measurements gaps, the UE is not considered to be OFF.

#### 6.3.3B.1	Minimum requirement

The transmit OFF power is defined as the mean power at each transmit antenna connector in a duration of at least one sub-frame (1ms) excluding any transient periods. The transmit OFF power at each transmit antenna connector shall not exceed the values specified in Table 6.3.3B.1-1.

Table 6.3.3B.1-1: Transmit OFF power per antenna port

|  | Channel bandwidth / Transmit OFF power/ Measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Transmit OFF power | -50 dBm |  |  |  |  |  |
| Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |

### 6.3.3D	Transmit OFF power for ProSe

When UE is configured for E-UTRA ProSe sidelink transmissions non-concurrent with E-UTRA uplink transmissions for E-UTRA ProSe operating bands specified in Table 5.5D-1, the Prose UE shall meet the Transmit OFF power at all times when the UE is not associated with a serving cell on the ProSe carrier and does not have knowledge of its geographical area or is provisioned with pre-configured radio parameters that are not associated with any known Geographical Area.

The requirements specified in subclause 6.3.3 shall apply to UE supporting ProSe when

-	the UE is associated with a serving cell on the ProSe carrier, or

-	the UE is not associated with a serving cell on the ProSe carrier and is provisioned with the preconfigured radio parameters for ProSe Direct Communications and/or ProSe Direct Discovery that are associated with known Geographical Area, or

-	the UE is associated with a serving cell on a carrier different than the ProSe carrier, and the radio parameters for ProSe Direct Discovery on the ProSe carrier are provided by the serving cell, or

-	the UE is associated with a serving cell on a carrier different than the ProSe carrier, and has a non-serving cell selected on the ProSe carrier that supports ProSe Direct Discovery and/or ProSe Direct Communication.

When UE is configured for simultaneous E-UTRA ProSe sidelink and E-UTRA uplink transmissions for inter-band E-UTRA ProSe / E-UTRA bands specified in Table 5.5D-2, transmit OFF power is defined as the mean power per component carrier when the transmitter is OFF on all component carriers. During measurement gaps and transmission/reception gaps for ProSe, the UE is not considered to be OFF. Transmit OFF power requirement as specified in subclause 6.3.3 apply per carrier.

### 6.3.3F	Transmit OFF power for category NB1 and NB2

For category NB1 and NB2 UE the transmit OFF power requirement for the channel bandwidth is -50 dBm. For 3.75kHz sub-carrier spacing the transmit OFF power is defined as mean power in one slot (2ms) excluding the 2304Ts gap when UE is not transmitting. For 15kHz sub-carrier spacing the transmit OFF power is defined as mean power in one sub-frame (1ms).

### 6.3.3G	Transmit OFF power for V2X Communication

When UE is configured for E-UTRA V2X sidelink transmissions non-concurrent with E-UTRA uplink transmissions for E-UTRA V2X operating bands specified in Table 5.5G-1, the V2X UE shall meet the Transmit OFF power in subclause 6.3.3D.

When UE is configured for simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions for inter-band E-UTRA V2X / E-UTRA bands specified in Table 5.5G-2, the requirements in subclause 6.3.3A apply for as specified for the corresponding inter-band con-current operation with uplink assigned to two bands.

For intra-band contiguous E-UTRA V2X multiple carrier transmissions, the requirements in subclause 6.3.3A apply as specified for the corresponding intra band contiguous carrier aggregation.

The transmit OFF power is defined as the mean power at each transmit antenna connector.

The transmit OFF power at each transmit antenna connector shall not exceed the values specified for single carrier.

### 6.3.4	ON/OFF time mask

#### 6.3.4.1	General ON/OFF time mask

The General ON/OFF time mask defines the observation period between Transmit OFF and ON power and between Transmit ON and OFF power. ON/OFF scenarios include; the beginning or end of DTX, measurement gap, contiguous, and non contiguous transmission

The OFF power measurement period is defined in a duration of at least one subframe, or one slot or one subslot for sTTI, excluding any transient periods. The ON power is defined as the mean power over one subframe, or one slot or one subslot for sTTI, excluding any transient period.

There are no additional requirements on UE transmit power beyond that which is required in subclause 6.2.2 and subclause 6.6.2.3

The transient period length shall be no longer than the specified value in Table 6.3.4.1-1.

Table 6.3.4.1-1: Transient period length depending on transmission length

| TTI pattern | Channel BW (MHz) | TTI Duration | Transient period length (µs) |
| --- | --- | --- | --- |
| Subframe TTI |  | 1ms | 20 |
| Slot TTI | 1.4 | 1 slot | 20 |
|  | > 1.4 | 1 slot | 10 |
| Subslot TTI | 1.4 | 2os or 3 os | 20 |
|  | > 1.4 | 2os or 3 os | 10 |

Figure 6.3.4.1-1: General ON/OFF time mask for subframe TTI and for Frame Structure Type 1 and Frame Structure Type 2

For Frame Structure Type 3 the general ON/OFF mask is specified in 6.3.4.1-1A with the PUSCH starting position modified by relative to the start of the sub-frame as indicated in the associated DCI, where and the basic time unit are specified in [4]. At the end of the sub-frame  and  with  denoting the duration of the last SC-FDMA symbol when the bit indicating the PUSCH ending symbol in the associated DCI has value ‘0’ and ‘1’ as specified in [5], respectively; the OFF power requirement applies 5 s after the end of the last symbol transmitted.

Figure 6.3.4.1-1A: General ON/OFF time mask for subframe TTI and for Frame Structure Type 3

Figure 6.3.4.1-1B: General ON/OFF time mask for sTTI and for Frame Structure Type 1 and Frame Structure Type 2

#### 6.3.4.2	PRACH and SRS time mask

##### 6.3.4.2.1	PRACH time mask

The PRACH ON power is specified as the mean power over the PRACH measurement period excluding any transient periods as shown in Figure 6.3.4.2-1. The measurement period for different PRACH preamble format is specified in Table 6.3.4.2-1.

There are no additional requirements on UE transmit power beyond that which is required in subclause 6.2.2 and subclause 6.6.2.3

Table 6.3.4.2-1: PRACH ON power measurement period

| PRACH preamble format | Measurement period (ms) |
| --- | --- |
| 0 | 0.9031 |
| 1 | 1.4844 |
| 2 | 1.8031 |
| 3 | 2.2844 |
| 4 | 0.1479 |

Figure 6.3.4.2-1: PRACH ON/OFF time mask

##### 6.3.4.2.2	SRS time mask

In the case a single SRS transmission, the ON power is defined as the mean power over the symbol duration excluding any transient period; Figure 6.3.4.2.2-1 and Figure 6.3.4.2.2-1A.

In the case a dual SRS transmission, the ON power is defined as the mean power for each symbol duration excluding any transient period. Figure 6.3.4.2.2-2

There are no additional requirements on UE transmit power beyond that which is required in subclause 6.2.2 and subclause 6.6.2.3

Figure 6.3.4.2.2-1: Single SRS time mask for Frame Structure Type 1 and Frame Structure Type 2

For Frame Structure Type 3 and single SRS transmission, the SRS time mask is specified in 6.3.4.2-2A; the OFF power requirement applies [5] s after the end of the SRS symbol.

Figure 6.3.4.2.2-1A: Single SRS time mask for Frame Structure Type 3

Figure 6.3.4.2.2-2: Dual SRS time mask for the case of UpPTS transmissions

For SRS transmission mapped to two or more OFDM symbols the ON power is defined as the mean power for each symbol duration excluding any transient period. For consecutive SRS transmissions without power change, Figure 6.3.4.2.2-3 applies.

SRS

End of OFF power

requirement

Start of OFF power requirement

SRS ON power requirement on consecutive symbols

20µs

Transient period

SRS

SRS

SRS

20µs

Transient period

Figure 6.3.4.2.2-3: Consecutive SRS time mask for the case when no power change is required

When power change between consecutive SRS transmissions is required, then Figure 6.3.4.2.2-4 and Figure 6.3.4.2.2-5 apply.

SRS

SRS ON power requirement

SRS

SRS

20µs

20µs

20µs

SRS

End of OFF power

requirement

Start of OFF power requirement

SRS ON power requirement

SRS ON power requirement

SRS ON power requirement

20µs

Transient period

20µs

Transient period

20µs

Transient period

20µs

Transient period

20µs

Transient period

Figure 6.3.4.2.2-4: Consecutive SRS time mask for the case when power change is required

SRS

Ant. ‘x’

SRS

Ant. ‘x’

SRS

Ant. ‘x’

SRS

Ant. ‘x’

SRS

Ant. ‘x’

SRS

Ant. ‘x’

SRS ON power requirement

20µs

20µs

20µs

End of OFF power

requirement

Start of OFF power requirement

SRS ON power requirement

SRS ON power requirement

SRS ON power requirement

20µs

Transient period

20µs

Transient period

20µs

Transient period

20µs

Transient period

20µs

Transient period

SRS

Ant. ‘x’

SRS

Ant. ‘y’

SRS

Ant. ‘x’

SRS

Ant. ‘x’

Figure 6.3.4.2.2-5: Time mask for SRS antenna switching

The above transient period applies to all the transmit CCs in CA with the CC sounding SRS. UE RF requirements do not apply during this transient period.

#### 6.3.4.3	Slot / Sub frame boundary time mask for subframe TTI

The sub frame boundary time mask defines the observation period between the previous/subsequent sub–frame and the (reference) sub-frame. A transient period at a slot boundary within a sub-frame is only allowed in the case of Intra-sub frame frequency hopping. For the cases when the subframe contains SRS the time masks in subclause 6.3.4.4 apply.

There are no additional requirements on UE transmit power beyond that which is required in subclause 6.2.2 and subclause 6.6.2.3

Figure 6.3.4.3-1: Transmission power template for Frame Structure Type 1 and Frame Structure Type 2

For Frame Structure Type 3 the sub-frame boundary time mask is specified in Figure 6.3.4.3-1A when the bit indicating the PUSCH ending symbol in the associated DCI has value ‘1’ and the PUSCH starting position is modified by  in the following subframe (clause 6.3.4.1);  denotes the duration of the ending SC-FDMA symbol. the OFF power requirement applies 5 s after the end of the last symbol transmitted.

Figure 6.3.4.3-1A: Transmission power template when the bit in the associated DCI indcating the PUSCH ending symbol has value ‘1' for Frame Structure Type 3

For Frame Structure Type 3 the first slot boundary time mask is specified in Figure 6.3.4.3-1B when the PUSCH mode is 3 indicated in DCI [4]. The PUSCH starting position modified by relative to the start of the sub-frame as indicated in the associated DCI, where and the basic time unit are specified in TS 36.211 [4]. At the end of the first slot tend=3∙Tsymb or  tend=0 with  denoting the duration of one SC-FDMA symbol when the bit indicating the PUSCH ending symbol in the associated DCI is either fourth or seventh symbol  as specified in TS 36.212 [5], respectively; the OFF power requirement applies 5 s after the end of the last symbol transmitted.

Figure 6.3.4.3-1B: Transmission power template for the first slot in one subframe for Frame Structure Type 3

For Frame Structure Type 3 the second slot boundary time mask is specified in Figure 6.3.4.3-1C when the PUSCH mode is 2 indicated in DCI [4]. The PUSCH starting position modified by relative to the start of the second slot as indicated in the associated DCI, where and the basic time unit are specified in TS 36.211 [4]. At the end of the second slot  tend=Tsymb or  tend=0 with  denoting the duration of one SC-FDMA symbol when the bit indicating the PUSCH ending symbol in the associated DCI is either thirteenth or fourteenth symbol  as specified in TS 36.212 [5], respectively; the OFF power requirement applies 5 s after the end of the last symbol transmitted.

For Frame Structure Type 3 the second slot boundary time mask specified in Figure 6.3.4.3-1C can also be applied  when the PUSCH mode is 1 indicated in DCI [4] and transmition starts at the eighth symbol. The PUSCH starting position tD=0 relative to the start of the second slot. At the end of the second slot  tend=Tsymb or  tend=0  with  denoting the duration of one SC-FDMA symbol when the bit indicating the PUSCH ending symbol in the associated DCI is either thirteenth or fourteenth symbol as specified in TS 36.212 [5], respectively; the OFF power requirement applies 5 s after the end of the last symbol transmitted.

Figure 6.3.4.3-1C: Transmission power template for the second slot in one subframe for Frame Structure Type 3

#### 6.3.4.4	PUCCH / PUSCH / SRS time mask for subframe TTI

The PUCCH/PUSCH/SRS time mask defines the observation period between sounding reference symbol (SRS) and an adjacent PUSCH/PUCCH symbol and subsequent sub-frame. The time masks apply for all types of frame structures and their allowed PUCCH/PUSCH/SRS transmissions unless otherwise stated.

There are no additional requirements on UE transmit power beyond that which is required in subclause 6.2.2 and subclause 6.6.2.3

Figure 6.3.4.4-1: PUCCH/PUSCH/SRS time mask when there is a transmission before SRS but not after for Frame Structure Type 1 and Frame Structure Type 2

For Frame Structure Type 3 the PUSCH/SRS time mask when there is a transmission before SRS but not after is specified in Figure 6.3.4.4-1A; the OFF power requirement applies 5 s after the end of the last symbol transmitted.

Figure 6.3.4.4-1A: PUSCH/SRS time mask when there is a transmission before SRS but not after for Frame Structure Type 3

Figure 6.3.4.4-2: PUCCH/PUSCH/SRS time mask when there is transmission before and after SRS

Figure 6.3.4.4-3: PUCCH/PUSCH/SRS time mask when there is a transmission after SRS but not before

Figure 6.3.4.4-4: SRS time mask when there is FDD SRS blanking for Frame Structure Type 1 and Frame Structure Type 2

For Frame Structure Type 3 the PUSCH/SRS time mask with transmission after the SRS symbol and the PUSCH starting position modified by  in the following subframe (clause 6.3.4.1) is specified in Figure 6.3.4.4-4A when there is SRS blanking.

Figure 6.3.4.4-4A: SRS time mask when there is SRS blanking for Frame Structure Type 3

#### 6.3.4.5	Symbol / Subslot boundary time mask for subslot TTI

The subslot boundary time mask defines the observation period between the previous/subsequent subslot and the (reference) subslot. A transient period at a symbol boundary within a subslot is only allowed in the case of Intra-subslot frequency hopping. For the cases when the subslot contains SRS the time masks in subclause 6.3.4.6 apply.

There are no additional requirements on UE transmit power beyond that which is required in subclause 6.2.2 and subclause 6.6.2.3

Following time masks requirements shall be applied:

-	the transient period shall be equally shared between  two consecutive Reference symbols or Data symbols (figure 6.3.4.5-1 and figure 6.3.4.5-4).

-	Otherwise, the transient period shall be placed in the Reference symbol (figure 6.3.4.5-2 and figure 6.3.4.5-3).

Figure 6.3.4.5-1: Transmission power template for subslot TTI – transient period shared

Figure 6.3.4.5-2: Transmission power template for subslot TTI – transient period not shared

Figure 6.3.4.5-3: Transmission power template for subslot TTI – transient period not shared

Figure 6.3.4.5-4: Transmission power template for subslot TTI – transient period shared

#### 6.3.4.6	Subslot PUCCH / subslot PUSCH / SRS time mask for subslot TTI

The subslot PUCCH/subslot PUSCH/SRS time mask defines the observation period between sounding reference symbol (SRS) in the last symbol in subslot N and an adjacent subslot PUSCH/subslot PUCCH symbol in subslot N+1.

There are no additional requirements on UE transmit power beyond that which is required in subclause 6.2.2 and subclause 6.6.2.3

Following time masks requirement shall be applied when SRS is either transmitted or blanked:

-	the transient period shall be placed in Reference symbol when the transient is in between Reference symbol and SRS (figure 6.3.4.6-1, figure 6.3.4.6-2, figure 6.3.4.6-5 and figure 6.3.4.6-7).

-	the transient period shall be equally shared when the transien is in between  Data symbol and SRS (figure 6.3.4.6-3 and figure 6.3.4.6-4).

Figure 6.3.4.6-1: subslot PUSCH/SRS time mask when there is a Reference symbol before SRS (or SRS blanking) and data symbol after

Figure 6.3.4.6-2:subslot PUSCH/SRS time mask when there is a Reference symbol before SRS (or SRS blanking) and Reference symbol after

Figure 6.3.4.6-3: subslot PUSCH/SRS time mask when there is a data symbol before SRS (or SRS blanking) and data symbol after

Figure 6.3.4.6-4: subslot PUSCH/SRS time mask when there is a data symbol before SRS (or SRS blanking) and Reference symbol after

Figure 6.3.4.6-5: subslot PUSCH/SRS time mask when there is a no symbol before SRS

Figure 6.3.4.6-6: subslot PUSCH/SRS time mask when there is a no symbol after SRS

Figure 6.3.4.6-7: subslot PUSCH/SRS time mask when there is a no symbol before and after SRS

#### 6.3.4.7	Symbol / Slot boundary time mask for slot TTI

The slot boundary time mask defines the observation period between the previous/subsequent slot and the (reference) slot. A transient period at a symbol boundary within a slot is only allowed in the case of Intra slot frequency hopping. For the cases when the slot contains SRS the time masks in subclause 6.3.4.8 shall apply.

There are no additional requirements on UE transmit power beyond that which is required in subclause 6.2.2 and subclause 6.6.2.3

For slot boundary, the time maks specified in subclause 6.3.4.4 shall apply with a transient time of 10µs intead of 20µs.

For frequency hopping within the slot, the time masks specified in subclause 6.3.4.5 shall apply.

#### 6.3.4.8	Slot PUCCH / slot PUSCH / SRS time mask for slot TTI

The slot PUCCH/slot PUSCH/SRS time mask defines the observation period between sounding reference symbol (SRS) and an adjacent slot PUSCH/slot PUCCH symbol and subsequent slot.

There are no additional requirements on UE transmit power beyond that which is required in subclause 6.2.2 and subclause 6.6.2.3

The time masks specified in subclause 6.3.4.4 shall apply.

#### 6.3.4.9	Consecutive subslot and slot TTI or consecutive subslot and subframe TTI  time mask

The consecutive subslot and slot boundary time mask or consecutive subslot and subframe boundary time mask defines the observation period between the subslot and the slot or subframe.

There are no additional requirements on UE transmit power beyond that which is required in subclause 6.2.2 and subclause 6.6.2.3.

In this case, the transient period shall be placed in the subframe TTI or the slot TTI (figure 6.3.4.9-1)

Figure 6.3.4.9-1: subslot TTI and subframe TTI boundary

#### 6.3.4.10	Consecutive subframe and subslot TTI or consecutive slot and subslot TTI time mask

The consecutive subframe and subslot boundary time mask or consecutive slot and subslot boundary time mask defines the observation period between the slot or subframe and the subslot.

There are no additional requirements on UE transmit power beyond that which is required in subclause 6.2.2 and subclause 6.6.2.3

Figure 6.3.4.10-1: Subframe TTI and subslot TTI boundary with SRS in last subframe TTI symbol and Reference Symbol in first subslot TTI symbol

Figure 6.3.4.10-2: Subframe TTI and subslot TTI boundary with SRS in last subframe TTI symbol and data Symbol in first subslot TTI symbol

When the last symbol of the Subframe or slot is not SRS then the transient period is placed in the Subframe or Slot.

Figure 6.3.4.10-3: subframe TTI and subslot TTI boundary

#### 6.3.4.11	Consecutive TTI and slot TTI or consecutive slot TTI and TTI time mask

The consecutive subframe and slot boundary time mask or consecutive slot and subframe boundary time mask defines the observation period between the subframe and the slot or the slot and the subframe.

There are no additional requirements on UE transmit power beyond that which is required in subclause 6.2.2 and subclause 6.6.2.3

The time masks at subframe boundary specified in subclause 6.3.4.3 or at slot boundary specified in subclause 6.3.4.7 shall apply.

### 6.3.4A	ON/OFF time mask for CA

For inter-band carrier aggregation with uplink assigned to two E-UTRA bands and intra-band contiguous and non-contiguous carrier aggregation, the general output power ON/OFF time mask specified in subclause 6.3.4.1 is applicable for each component carrier during the ON power period and the transient periods. The OFF period as specified in subclause 6.3.4.1 shall only be applicable for each component carrier when all the component carriers are OFF.

### 6.3.4B	ON/OFF time mask for UL-MIMO

For UE supporting UL-MIMO, the ON/OFF time mask requirements in subclause 6.3.4 apply at each transmit antenna connector.

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the general ON/OFF time mask requirements specified in subclause 6.3.4.1 apply to each transmit antenna connector. The requirements shall be met with the UL-MIMO configurations specified in Table 6.2.2B-2.

If UE is configured for transmission on single-antenna port, the requirements in subclause 6.3.4 apply.

### 6.3.4D	ON/OFF time mask for ProSe

For ProSe Direct Discovery and ProSe Direct Communications, additional requirements on ON/OFF time masks for ProSe physical channels and signals are specified in this clause.

When UE is configured for simultaneous E-UTRA ProSe sidelink and E-UTRA uplink transmissions for inter-band E-UTRA ProSe / E-UTRA bands specified in Table 5.5D-2, the requirements in subclause 6.3.4D apply for ProSe transmission and the requirements in subclause 6.3.4 apply for uplink transmission.

#### 6.3.4D.1		General time mask for ProSe

The General ON/OFF time mask defines the observation period between the Transmit OFF and ON power and between Transmit ON and OFF power for PSDCH, PSCCH, and PSSCH transmissions in a subframe wherein the last symbol is punctured to create a guard period.

There are no additional requirements on UE transmit power beyond that which is required in subclause 6.2.2 and subclause 6.6.2.3.

Figure 6.3.4D.1-1: PSDCH/PSCCH/PSSCH time mask

#### 6.3.4D.2		PSSS/SSSS time mask

The PSSS time mask / SSSS time mask defines the observation period between the Transmit OFF and ON power and between Transmit ON and OFF power for PSSS/SSSS transmissions in a subframe when not multiplexed with PSBCH in that subframe.

There are no additional requirements on UE transmit power beyond that which is required in subclause 6.2.2 and subclause 6.6.2.3.

Figure 6.3.4D.2-1: PSSS time mask for normal CP transmission (when not time-multiplexed with PSBCH)

Figure 6.3.4D.2-2: PSSS time mask for extended CP transmission (when not time-multiplexed with PSBCH)

Figure 6.3.4D.2-3: SSSS time mask (when not time-multiplexed with PSBCH)

#### 6.3.4D.3		PSSS / SSSS / PSBCH time mask

The PSSS/SSSS/PSBCH time mask defines the observation period between SSSS and adjacent PSSS/PSBCH symbols in a subframe, with last symbol punctured to create a guard period.

There are no additional requirements on UE transmit power beyond that which is required in subclause 6.2.2 and subclause 6.6.2.3.

Figure 6.3.4D.3-1: PSSS/SSSS/PBCH time mask for normal CP transmission

Figure 6.3.4D.3-2: PSSS/SSSS/PBCH time mask for extended CP transmission

#### 6.3.4D.4		PSSCH / SRS time mask

The PSSCH/SRS time mask defines the observation period between sounding reference symbol (SRS) and an adjacent PSSCH symbol and subsequent sub-frame.

There are no additional requirements on UE transmit power beyond that which is required in subclause 6.2.2 and subclause 6.6.2.3.

The PSSCH/SRS time mask shall follow the PUSCH/PUCCH/SRS time mask as specified in subclause 6.3.4.4.

### 6.3.4F	ON/OFF time mask for category NB1 and NB2

#### 6.3.4F.1	General ON/OFF time mask

E-UTRA general ON/OFF time mask in subclause 6.3.4.1 applies for category NB1 and NB2 UE with an exception that for 3.75kHz sub-carrier spacing the transmit OFF power is defined as mean power in one slot (2ms) and for 15kHz sub-carrier spacing the transmit OFF power is defined as mean power in one sub-frame (1ms), excluding any transient periods. The ON power is defined as the mean power over one RU excluding any transient periods.

#### 6.3.4F.2	NPRACH time mask

The NPRACH ON power is specified as the mean power over the NPRACH measurement period excluding any transient periods as shown in Figure 6.3.4F.2-1. The measurement period for different NPRACH preamble format is specified in Table 6.3.4F.2-1.

There are no additional requirements on UE transmit power beyond that which is required in subclause 6.2.2F and subclause 6.6.2.3F.

Table 6.3.4F.2-1: NPRACH ON power measurement period

| NPRACH preamble format | Measurement period (ms) |
| --- | --- |
| 0 | 5.6 |
| 1 | 6.4 |

Figure 6.3.4F.2-1: NPRACH ON/OFF time mask

### 6.3.4G	ON/OFF time mask for V2X Communication

For V2X Communications, additional requirements on ON/OFF time masks for V2X physical channels and signals are specified in this clause.

The General ON/OFF time mask in subclause 6.3.4D.1 and PSSS/SSSS time mask in subcluse 6.3.4D.2 are applied for E-UTRA V2X sidelink UE.

When UE is configured for simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions for inter-band E-UTRA V2X / E-UTRA bands specified in Table 5.5G-2, the requirements in subclause 6.3.4G apply for the V2X sidelink transmission and the requirements in subclause 6.3.4 apply for the E-UTRA uplink transmission.

For intra-band contiguous multi-carrier operation the general ON/OFF time mask is applicable for each component carrier during the ON power period and the transient periods. The OFF period shall only be applicable for each component carrier when all the component carriers are OFF.

For V2X UE supporting Transmit Diversity, the ON/OFF time mask requirements apply at each transmit antenna connector.

If the UE transmits on two antenna connectorsat the same time, the general ON/OFF time mask requirements apply to each transmit antenna connector.

If the UE transmits on one antenna connector at a time, the general ON/OFF time mask requirements apply to the active antenna connector.

#### 6.3.4G.1	PSSS / SSSS / PSBCH time mask

The PSSS/SSSS/PSBCH time mask for V2X UE defines the observation period between SSSS and adjacent PSSS/PSBCH symbols in a subframe, with last symbol punctured to create a guard period.

Figure 6.3.4G.1-1: PSSS/SSSS/PSBCH time mask for normal CP transmission for V2X Service

Figure 6.3.4G.1-2: PSSS/SSSS/PSBCH time mask for extended CP transmission for V2X Service

### 6.3.5	Power Control

#### 6.3.5.1	Absolute power tolerance

Absolute power tolerance is the ability of the UE transmitter to set its initial output power to a specific value for the first sub-frame at the start of a contiguous transmission or non-contiguous transmission with a transmission gap larger than 20ms. This tolerance includes the channel estimation error (the absolute RSRP accuracy requirement specified in subclause 9.1 of TS 36.133). In the case of a PRACH transmission, the absolute tolerance is specified for the first preamble. The absolute power tolerance includes the channel estimation error (the absolute RSRP accuracy requirement specified in subclause 9.1 of TS 36.133).

##### 6.3.5.1.1		Minimum requirements

The minimum requirement for absolute power tolerance is given in Table 6.3.5.1.1-1 over the power range bounded by the Maximum output power as defined in subclause 6.2.2 and the Minimum output power as defined in subclause 6.3.2.

For operating bands under NOTE 2 in Table 6.2.2-1, the absolute power tolerance as specified in Table 6.3.5.1.1-1 is relaxed by reducing the lower limit by 1.5 dB when the transmission bandwidth is confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high.

Table 6.3.5.1.1-1: Absolute power tolerance

| Conditions | Tolerance |
| --- | --- |
| Normal | ± 9.0 dB |
| Extreme | ± 12.0 dB |

#### 6.3.5.2	Relative Power tolerance

The relative power tolerance is the ability of the UE transmitter to set its output power in a target sub-frame relatively to the power of the most recently transmitted reference sub-frame if the transmission gap between these sub-frames is ≤ 20 ms.

For PRACH transmission, the relative tolerance is the ability of the UE transmitter to set its output power relatively to the power of the most recently transmitted preamble. The measurement period for the PRACH preamble is specified in Table 6.3.4.2-1.

##### 6.3.5.2.1	Minimum requirements

The requirements specified in Table 6.3.5.2.1-1 apply when the power of the target and reference sub-frames are within the power range bounded by the Minimum output power as defined in subclause 6.3.2 and the measured PUMAX as defined in subclause 6.2.5 (i.e, the actual power as would be measured assuming no measurement error). This power shall be within the power limits specified in subclause 6.2.5.

To account for RF Power amplifier mode changes 2 exceptions are allowed for each of two test patterns. The test patterns are a monotonically increasing power sweep and a monotonically decreasing power sweep over a range bounded by the requirements of minimum power and maximum power specified in subclauses 6.3.2 and 6.2.2. For these exceptions the power tolerance limit is a maximum of ±6.0 dB in Table 6.3.5.2.1-1

Table 6.3.5.2.1-1 Relative power tolerance for transmission (normal conditions)

| Power step P (Up or down) [dB] | All combinations of PUSCH and PUCCH transitions [dB] | All combinations of PUSCH/PUCCH and SRS transitions between sub-frames [dB] | PRACH [dB] |
| --- | --- | --- | --- |
| ΔP < 2 | ±2.5 (NOTE 3) | ±3.0 | ±2.5 |
| 2 ≤ ΔP < 3 | ±3.0 | ±4.0 | ±3.0 |
| 3 ≤ ΔP < 4 | ±3.5 | ±5.0 | ±3.5 |
| 4 ≤ ΔP ≤ 10 | ±4.0 | ±6.0 | ±4.0 |
| 10 ≤ ΔP < 15 | ±5.0 | ±8.0 | ±5.0 |
| 15 ≤ ΔP | ±6.0 | ±9.0 | ±6.0 |
| NOTE 1: For extreme conditions an additional ± 2.0 dB relaxation is allowed NOTE 2: For operating bands under NOTE 2 in Table 6.2.2-1, the relative power tolerance is relaxed by increasing the upper limit by 1.5 dB if the transmission bandwidth of the reference sub-frames is confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high and the target sub-frame is not confined within any one of these frequency ranges; if the transmission bandwidth of the target sub-frame is confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high and the reference sub-frame is not confined within any one of these frequency ranges, then the tolerance is relaxed by reducing the lower limit by 1.5 dB. NOTE 3: For PUSCH to PUSCH transitions with the allocated resource blocks fixed in frequency and no transmission gaps other than those generated by downlink subframes, DwPTS fields or Guard Periods for TDD: for a power step ΔP ≤ 1 dB, the relative power tolerance for transmission is ±1.0 dB. |  |  |  |

The power step (ΔP) is defined as the difference in the calculated setting of the UE Transmit power between the target and reference sub-frames with the power setting according to subclause 5.1 of [TS 36.213]. The error is the difference between ΔP and the power change measured at the UE antenna port with the power of the cell-specific reference signals kept constant. The error shall be less than the relative power tolerance specified in Table 6.3.5.2.1-1.

For sub-frames not containing an SRS symbol, the power change is defined as the relative power difference between the mean power of the original reference sub-frame and the mean power of the target subframe not including transient durations. The mean power of successive sub-frames shall be calculated according to Figure 6.3.4.3-1 and Figure 6.3.4.1-1 if there is a transmission gap between the reference and target sub-frames.

If at least one of the sub-frames contains an SRS symbol, the power change is defined as the relative power difference between the mean power of the last transmission within the reference sub-frame and the mean power of the first transmission within the target sub-frame not including transient durations. A transmission is defined as PUSCH, PUCCH or an SRS symbol. The mean power of the reference and target sub-frames shall be calculated according to Figures 6.3.4.1-1, 6.3.4.2-1, 6.3.4.4-1, 6.3.4.4-2 and 6.3.4.4-3 for these cases.

#### 6.3.5.3	Aggregate power control tolerance

Aggregate power control tolerance is the ability of a UE to maintain its power in non-contiguous transmission within 21 ms in response to 0 dB TPC commands with respect to the first UE transmission, when the power control parameters specified in TS 36.213 are constant. For HD-FDD UEs that support coverage enhancement (CE), the requirements on aggregate power control tolerance in 6.3.5E.3 apply.

##### 6.3.5.3.1	Minimum requirement

The UE shall meet the requirements specified in Table 6.3.5.3.1-1 for aggregate power control over the power range bounded by the minimum output power as defined in subclause 6.3.2 and the maximum output power as defined in subclause 6.2.2.

Table 6.3.5.3.1-1: Aggregate power control tolerance

| TPC command | UL channel | Aggregate power tolerance within 21 ms |
| --- | --- | --- |
| 0 dB | PUCCH | ±2.5 dB |
| 0 dB | PUSCH | ±3.5 dB |
| NOTE: The UE transmission gap is 4 ms. TPC command is transmitted via PDCCH 4 subframes preceding each PUCCH/PUSCH transmission. |  |  |

### 6.3.5A	Power control for CA

The requirements apply for one single PUCCH, PUSCH or SRS transmission of contiguous PRB allocation per component carrier with power setting in accordance with Clause 5.1 of [6].

#### 6.3.5A.1	Absolute power tolerance

The absolute power tolerance is the ability of the UE transmitter to set its initial output power to a specific value for the first sub-frame at the start of a contiguous transmission or non-contiguous transmission with a transmission gap on each active component carriers larger than 20ms. For component carriers with Frame Structure Type 3 the absolute power toerlance requirements apply when the said transmission gaps are larger than 40 ms. The requirement can be tested by time aligning any transmission gaps on the component carriers.

When SRS carrier based switching is used, then the above mentioned absolute power tolerance is the ability of the UE transmitter to set its initial output power to a specific value for the first sub-frame at the start of a contiguous transmission or non-contiguous transmission with a transmission gap on component carriers (to which SRS switching occurs) larger than 40ms.

##### 6.3.5A.1.1		Minimum requirements

For inter-band carrier aggregation with uplink assigned to two E-UTRA bands, the absolute power control tolerance is specified on each component carrier exceed the minimum output power as defined in subclause 6.3.2A and the total power is limited by maximum output power as defined in subclause 6.2.2A. The requirements defined in Table 6.3.5.1.1-1 shall apply on each component carrier with all component carriers active. The requirements can be tested by time aligning any transmission gaps on all the component carriers.

For intra-band contiguous carrier aggregation bandwidth class B, C and D and intra-band non-contiguous carrier aggregation the absolute power control tolerance per component carrier is given in Table 6.3.5.1.1-1.

#### 6.3.5A.2	Relative power tolerance

##### 6.3.5A.2.1	Minimum requirements

For inter-band carrier aggregation with uplink assigned to two E-UTRA bands, the relative power tolerance is specified when the power of the target and reference sub-frames on each component carrier exceed the minimum output power as defined in subclause 6.3.2A and the total power is limited by PUMAX as defined in subclause 6.2.5A. The requirements shall apply on each component carrier with all component carriers active. The UE transmitter shall have the capability of changing the output power independently on all component carriers in the uplink and:

a)	the requirements for all combinations of PUSCH and PUCCH transitions per component carrier is given in Table 6.3.5.2.1-1.

b)	for SRS the requirements for combinations of PUSCH/PUCCH and SRS transitions between subframes given in Table 6.3.5.2.1-1 apply per component carrier when the target and reference subframes are configured for either simultaneous SRS or simultaneous PUSCH.

c)	for RACH the requirements apply for the primary cell and are given in Table 6.3.5.2.1-1.

For component carriers with Frame Structure Type 3 the requirements for the target sub-frame relative to the power of the most recently transmitted reference sub-frame shall be met with a transmission gap ≤ 40 ms.

For intra-band contiguous carrier aggregation bandwidth class B, C and D and intra-band non-contiguous carrier aggregation, the requirements apply when the power of the target and reference sub-frames on each component carrier exceed -20 dBm and the total power is limited by PUMAX as defined in subclause 6.2.5A. For the purpose of these requirements, the power in each component carrier is specified over only the transmitted resource blocks.

The UE shall meet the following requirements for transmission on both assigned component carriers when the average transmit power per PRB is aligned across both assigned carriers in the reference sub-frame:

a)	for all possible combinations of PUSCH and PUCCH transitions per component carrier, the corresponding requirements given in Table 6.3.5.2.1-1;

b)	for SRS transitions on each component carrier, the requirements for combinations of PUSCH/PUCCH and SRS transitions given in Table 6.3.5.2.1-1 with simultaneous SRS of constant SRS bandwidth allocated in the target and reference subrames;

c)	for RACH on the primary component carrier, the requirements given in Table 6.3.5.2.1-1 for PRACH.

For a) and b) above, the power step P between the reference and target subframes shall be set by a TPC command and/or an uplink scheduling grant transmitted by means of an appropriate DCI Format.

For a), b) and c) above, two exceptions are allowed for each component carrier for a power per carrier ranging from -20 dBm to PUMAX,c as defined in subclause 6.2.5. For these exceptions the power tolerance limit is ±6.0 dB in Table 6.3.5.2.1-1.

#### 6.3.5A.3	Aggregate power control tolerance

Aggregate power control tolerance is the ability of a UE to maintain its power in non-contiguous transmission within 21 ms in response to 0 dB TPC commands with respect to the first UE transmission, when the power control parameters specified in [6] are constant on all active component carriers.

##### 6.3.5A.3.1		Minimum requirements

For inter-band carrier aggregation with uplink assigned to two E-UTRA bands, the aggregate power tolerance is specified on each component carrier exceed the minimum output power as defined in subclause 6.3.2A and the total power is limited by maximum output power as defined in subclause 6.2.2A. The requirements defined in Table 6.3.5.3.1-1 shall apply on each component carrier with all component carriers active. The requirements can be tested by time aligning any transmission gaps on both the component carriers.

For intra-band contiguous carrier aggregation bandwidth class B, C and D and intra-band non-contiguous carrier aggregation, the aggregate power tolerance per component carrier is given in Table 6.3.5.3.1-1 with either simultaneous PUSCH or simultaneous PUCCH-PUSCH (if supported by the UE) configured. The average power per PRB shall be aligned across both assigned carriers before the start of the test. The requirement can be tested with the transmission gaps time aligned between component carriers.

### 6.3.5B	Power control for UL-MIMO

For UE supporting UL-MIMO, the power control tolerance applies to the sum of output power at each transmit antenna connector.

The power control requirements specified in subclause 6.3.5 apply to UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme. The requirements shall be met with UL-MIMO configurations specified in Table 6.2.2B-2, wherein

-	The Maximum output power requirements for UL-MIMO are specified in subclause 6.2.2B

-	The Minimum output power requirements for UL-MIMO are specified in subclause 6.3.2B

-	The requirements for configured transmitted power for UL-MIMO are specified in subclause 6.2.5B.

If UE is configured for transmission on single-antenna port, the requirements in subclause 6.3.5 apply.

### 6.3.5D	Power Control for ProSe

When UE is configured for simultaneous E-UTRA ProSe sidelink and E-UTRA uplink transmissions for inter-band E-UTRA ProSe / E-UTRA bands specified in Table 5.5D-2, the requirements in subclause 6.3.5D apply for ProSe transmission and the requirements in subclause 6.3.5 apply for uplink transmission.

#### 6.3.5D.1	Absolute power tolerance

For ProSe transmissions, the absolute power tolerance requirements specified in subclause 6.3.5.1 shall apply for each ProSe transmission.

### 6.3.5E	Power control for category M1 and M2

#### 6.3.5E.1	Absolute power tolerance

The absolute power tolerance requirements specified in subclause 6.3.5.1 apply, wherein

-	The Maximum output power requirements are specified in subclause 6.2.2E

-	The Minimum output power requirements are specified in subclause 6.3.2

-	The requirements for configured transmitted power are specified in subclause 6.2.5.

#### 6.3.5E.2	Relative Power tolerance

The relative power tolerance requirements specified in subclause 6.3.5.2 apply, wherein

-	The Maximum output power requirements are specified in subclause 6.2.2E

-	The Minimum output power requirements are specified in subclause 6.3.2

-	The requirements for configured transmitted power are specified in subclause 6.2.5.

#### 6.3.5E.3	Aggregate power control tolerance

Aggregate power control tolerance is the ability of a UE to maintain its power in non-contiguous transmission in response to 0 dB TPC commands with respect to the first UE transmission, when the power control parameters specified in TS 36.213 are constant.

For category M1 and M2 TDD and FD-FDD UEs, the aggregate power control tolerance requirements specified in Table 6.3.5E.3.1-0 apply. For category M1 and M2 HD-FDD UEs and for continuous uplink transmissions of duration ≤ 64 ms, the aggregate power control tolerance requirements specified in Table 6.3.5E.3.1-0 apply.

For category M1 and M2 HD-FDD UEs and for continuous uplink transmissions of duration > 64 ms, the aggregate power control tolerance requirements specified in Table 6.3.5E.3.1-1 apply.

##### 6.3.5E.3.1	Minimum requirement

The category M1 and M2 TDD and FD-FDD UEs shall meet the requirements specified in Table 6.3.5E.3.1-0 for aggregate power control over the power range bounded by the minimum output power as defined in subclause 6.3.2,  the maximum output power as defined in subclause 6.2.2E, and the requirements for configured transmitted power are specified in subclause 6.2.5.

The category M1 and M2 HD-FDD UEs and for continuous uplink transmissions of duration ≤ 64 ms, shall meet the requirements specified in Table 6.3.5E.3.1-0 for aggregate power control over the power range bounded by the minimum output power as defined in subclause 6.3.2,  the maximum output power as defined in subclause 6.2.2E, and the requirements for configured transmitted power are specified in subclause 6.2.5.

Table 6.3.5E.3.1-0: Aggregate power control tolerance

| TPC command | UL channel | Aggregate power tolerance within 21 ms 2 |
| --- | --- | --- |
| 0 dB | PUCCH | ±2.5 dB |
| 0 dB | PUSCH | ±3.5 dB |
| NOTE 1: The UE transmission gap is 4 ms for full-duplex FDD and TDD. For UE of half-duplex FDD with the channel bandwidth 5 MHz / 10 MHz / 15 MHz / 20 MHz, the transmission gap is 1 ms after subframe #4 and 7 ms after subframe #6. For UE of half-duplex FDD with the CBW 1.4 / 3 MHz, the transmission gap is 9 ms. TPC command is transmitted via MPDCCH 4 subframes preceding each PUCCH/PUSCH transmission. NOTE 2: For UE of half-duplex FDD with the CBW 1.4 / 3 MHz, the test interval is 41 ms. |  |  |

The category M1 and M2 HD-FDD UE and for continuous uplink transmissions of duration > 64 ms shall meet the requirements specified in Table 6.3.5E.3.1-1 for aggregate power control over the power range bounded by the minimum output power as defined in subclause 6.3.2 and the maximum output power as defined in subclause 6.2.2E.

Table 6.3.5E.3.1-1: Aggregate power control tolerance

| TPC command | UL channel | Aggregate power tolerance within 129 ms |
| --- | --- | --- |
| 0 dB | PUCCH | ±2.5 dB |
| 0 dB | PUSCH | ±3.5 dB |
| NOTE: The UE transmission gap is 5 ms. TPC command is transmitted via MPDCCH 4 subframes preceding each PUCCH/ PUSCH transmission. |  |  |

### 6.3.5F	Power Control for category NB1 and NB2

Power control requirements in this clause apply for category NB1 and NB2 UE.

#### 6.3.5F.1	Absolute power tolerance

The minimum requirement for absolute power tolerance is given in Table 6.3.5F.1-1 over the power range bounded by the Maximum output power as defined in subclause 6.2.2F and the Minimum output power as defined in subclause 6.3.2F.

Table 6.3.5F.1-1: Absolute power tolerance - I

| Conditions | Tolerance |
| --- | --- |
| Normal | ± 9.0 dB |
| Extreme | ±12.0 dB |

In case of -15 dB ≤ Ês/Iot < -6 dB, the absolute power tolerance given in Table 6.3.5F.1-2 applies if the UE transmit power is not mandated to be PCMAX,c according to the UE uplink power control procedure or random access procedure in Section 16 of [6] (e.g. the lowest configured repetition level is used for NPRACH transmission or the number of repetitions of the allocated NPUSCH RUs is no more than 2).

Table 6.3.5F.1-2: Absolute power tolerance - II

| Conditions | Tolerance |
| --- | --- |
| Normal | ± 13.3 dB |
| Extreme | ± 16.3 dB |

#### 6.3.5F.2	Relative power tolerance

Category NB1 and NB2 UE relative power control requirement is defined for NPRACH power step values of 0, 2, 4 and 6 dB. For NPRACH transmission, the relative tolerance is the ability of the UE transmitter to set its output power relatively to the power of the most recently transmitted preamble. The measurement period for the NPRACH preamble is specified in Table 6.3.4F.2-1.

The requirements specified in Table 6.3.5F.2-1 apply when the power of the target and reference sub-frames are within the power range bounded by the Minimum output power as defined in subclause 6.3.2F and the maximum output power as defined in subclause 6.2.2F.

Table 6.3.5F.2-1: Relative power tolerance for category NB1 and NB2 NPRACH transmission (normal conditions)

| Power step P [dB] | NPRACH [dB] |
| --- | --- |
| ΔP = 0 | ±1.5 |
| ΔP = 2 | ±2.0 |
| ΔP = 4 | ±3.5 |
| ΔP = 6 | ±4.0 |
| NOTE: For extreme conditions an additional ± 2.0 dB relaxation is allowed. |  |

The power step (ΔP) is defined as the difference in the calculated setting of the UE transmit power between the target and reference sub-frames. The error is the difference between ΔP and the power change measured at the UE antenna port with the power of the cell-specific reference signals kept constant. The error shall be less than the relative power tolerance specified in Table 6.3.5F.2-1.

#### 6.3.5F.3	Aggregate power control tolerance for category NB1 and NB2

Category NB1 and NB2 aggregate power control tolerance is the ability of a UE to maintain its output power in non-contiguous transmission with respect to the first UE transmission, when the uplink power control parameters as defined in TS 36.213 are constant and α is set to 0.

##### 6.3.5F.3.1	Minimum requirement

The UE shall meet the requirements specified in Table 6.3.5F.3.1-1 for aggregate power control over the power range bounded by the minimum output power as defined in subclause 6.3.2F and the maximum output power as defined in subclause 6.2.2F.

Table 6.3.5F.3.1-1: Aggregate power control tolerance for HD-FDD

| UL channel | Aggregate power tolerance |  |
| --- | --- | --- |
|  | 15 kHz / 12 tones within 53 ms | 15 kHz / 1 tone within 104 ms |
| NPUSCH | ±3.5 dB |  |
| NOTE: For five consecutive UE transmissions the transmission gaps are 12 ms for 12 tone and 16 ms for single tone transmissions. Uplink scheduling grant is transmitted via NPDCCH eight subframes before NPUSCH transmission. |  |  |

Table 6.3.5F.3.1-2: Aggregate power control tolerance for TDD

| UL channel | Aggregate power tolerance |  |
| --- | --- | --- |
|  | 15 kHz / 12 tones within 81 ms | 15 kHz / 1 tone within 177 ms |
| NPUSCH | ±3.5 dB |  |
| NOTE: For five consecutive UE transmissions the transmission gaps are 19 ms for 12 tone and 23 ms for single tone transmissions. Uplink scheduling grant is transmitted via NPDCCH eight subframes before NPUSCH transmission. |  |  |

### 6.3.5G	Power Control for V2X Communication

When UE is configured for E-UTRA V2X sidelink transmissions non-concurrent with E-UTRA uplink transmissions for E-UTRA V2X operating bands specified in Table Table 5.5G-1, the requirements in subclause 6.3.5G.1 apply for E-UTRA V2X sidelink transmission.

When UE is configured for simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions for inter-band E-UTRA V2X / E-UTRA bands specified in Table 5.5G-2, the requirements in subclause 6.3.5G.1 apply for V2X sidelink transmission and the requirements in subclause 6.3.5 apply for the E-UTRA uplink transmission.

For V2X UE supporting Transmit Diversity, if the UE transmits on two antenna connectors at the same time, the power control tolerance for single carrier shall apply to the sum of output power at each transmit antenna connector.

If the UE transmitson one -antenna connector at a time, the requirements for single carrier shall apply to the active antenna connector.

#### 6.3.5G.1	Absolute power tolerance

Absolute power tolerance is the ability of the UE to set its output power to a specific value for each subframe.

For V2X sidelink communication transmissions in the operating bands specified in Table 5.5G-1, the minimum requirement for absolute power tolerance is given in Table 6.3.5G.1-1 over the power range bounded by the Maximum output power as defined in subclause 6.2.2G and the Minimum output power as defined in subclause 6.3.2G.

For operating bands under NOTE 2 in Table 6.2.2-1, the absolute power tolerance as specified in Table 6.3.5G.1-1 is relaxed by reducing the lower limit by 1.5 dB when the transmission bandwidth is confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high.

Table 6.3.5G.1-1: Absolute power tolerance

| Conditions | Tolerance |
| --- | --- |
| Normal | ± 9.0 dB |
| Extreme | ± 12.0 dB |

For intra-band contiguous multi-carrier operation the absolute power control tolerance specified in Table 6.3.5G.1-1 shall apply for each component carrier.

## 6.4	Void

## 6.5	Transmit signal quality

### 6.5.1	Frequency error

The UE modulated carrier frequency shall be accurate to within ±0.1 PPM observed over a period of one time slot (0.5 ms) compared to the carrier frequency received from the E-UTRA Node B

### 6.5.1A 	Frequency error for CA

For inter-band carrier aggregation with uplink assigned to two E-UTRA bands, the frequency error requirements defined in subclause 6.5.1 shall apply on each component carrier with all component carriers active.

For intra-band contiguous carrier aggregation the UE modulated carrier frequencies per band shall be accurate to within ±0.1 PPM observed over a period of one timeslot compared to the carrier frequency of primary component carrier received from the E-UTRA in the corresponding band.

For intra-band non-contiguous carrier aggregation the requirements in Section 6.5.1 applies per component carrier.

### 6.5.1B	Frequency error for UL-MIMO

For UE(s) supporting UL-MIMO, the UE modulated carrier frequency at each transmit antenna connector shall be accurate to within ±0.1 PPM observed over a period of one time slot (0.5 ms) compared to the carrier frequency received from the E-UTRA Node B.

### 6.5.1D	Frequency error for ProSe

The UE modulated carrier frequency for ProSe sidelink transmissions shall be accurate to within ±0.1 PPM observed over a period of one time slot (0.5 ms) compared to the carrier frequency received from the synchronization source. The synchronization source can be E-UTRA Node B or a ProSe UE transmitting sidelink synchronization signals.

When UE is configured for simultaneous E-UTRA ProSe sidelink and E-UTRA uplink transmissions for inter-band E-UTRA ProSe / E-UTRA bands specified in Table 5.5D-2, the requirements in subclause 6.5.1D apply for ProSe transmission and the requirements in subclause 6.5.1 apply for uplink transmission.

### 6.5.1E	Frequency error for UE category M1 and M2

For category M1 and M2 TDD UEs and FD-FDD UEs, the frequency error requirements in Clause 6.5.1 apply.

For category M1 and M2 HD-FDD UEs and for continuous uplink transmissions of duration ≤ 64 ms, the frequency error requirements in Clause 6.5.1 apply.

For category M1 and M2 HD-FDD UEs and for continuous uplink transmissions of duration > 64 ms, the UE modulated carrier frequency shall be accurate to within the limits in Table 6.5.1E-1 observed over a period of one time slot (0.5 ms) compared to the carrier frequency received from the E-UTRA Node B.

Table 6.5.1E-1: Frequency error requirement for HD-FDD UE category M1 and M2

| Carrier frequency [GHz] | Frequency error [ppm] |
| --- | --- |
| ≤1 | ±0.2 |
| >1 | ±0.1 |

### 6.5.1F	Frequency error for UE category NB1 and NB2

For UE category NB1 and NB2, the UE modulated carrier frequency shall be accurate to within the following limits

Table 6.5.1F-1: Frequency error requirement for UE category NB1 and NB2

| Carrier frequency [GHz] | Frequency error [ppm] |
| --- | --- |
| ≤1 | ±0.2 |
| >1 | ±0.1 |

Observed over a period of one time slot (0.5 ms for 15 kHz sub-carrier spacing and 2 ms excluding the 2304Ts gap for 3.75 kHz sub-carrier spacing) and averaged over 72/LCtone slots (where LCtone = {1, 3, 6, 12} is the number of sub-carriers used for the transmission), compared to the carrier frequency received from the E-UTRA Node B.

### 6.5.1G	Frequency error for V2X Communication

The UE modulated carrier frequency for V2X sidelink transmissions shall be accurate to within ±0.1 PPM observed over a period of one time slot (0.5 ms) compared to the absolute frequency in case of using GNSS synchronization source. The same requirements applied over a period of one time slot (0.5 ms) compared to the relative frequency in case of using the E-UTRA Node B or V2X UE sidelink synchronization signals.

When UE is configured for simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions for inter-band E-UTRA V2X / E-UTRA bands specified in Table 5.5G-2, the requirements in subclause 6.5.1G apply for V2X sidelink transmission and the requirements in subclause 6.5.1 apply for the E-UTRA uplink transmission.

For V2X UE supporting Transmit Diversity, if the UE transmits on two antenna connectors at the same time, the UE modulated carrier frequency at each transmit antenna connector shall be accurate to within ±0.1 PPM observed over a period of one time slot (0.5 ms) in case of using GNSS synchronization source. The same requirements applied over a period of one time slot (0.5 ms) compared to the relative frequency in case of using the E-UTRA Node B or V2X UE sidelink synchronization signals.

If the UE transmits on one antenna connector at a time, the requirements for single carrier shall apply to the active antenna connector.

### 6.5.2	Transmit modulation quality

Transmit modulation quality defines the modulation quality for expected in-channel RF transmissions from the UE. The transmit modulation quality is specified in terms of:

-	Error Vector Magnitude (EVM) for the allocated resource blocks (RBs)

-	EVM equalizer spectrum flatness derived from the equalizer coefficients generated by the EVM measurement process

-	Carrier leakage

-	In-band emissions for the non-allocated RB

All the parameters defined in subclause 6.5.2 are defined using the measurement methodology specified in Annex F.

#### 6.5.2.1		Error Vector Magnitude

The Error Vector Magnitude is a measure of the difference between the reference waveform and the measured waveform. This difference is called the error vector. Before calculating the EVM the measured waveform is corrected by the sample timing offset and RF frequency offset. Then the carrier leakage shall be removed from the measured waveform before calculating the EVM.

The measured waveform is further modified by selecting the absolute phase and absolute amplitude of the Tx chain. The EVM result is defined after the front-end IDFT as the square root of the ratio of the mean error vector power to the mean reference power expressed as a %.

The basic EVM measurement interval in the time domain is one preamble sequence for the PRACH, and as specified in Table 6.5.2.1-1 for the PUCCH and PUSCH in the time domain. When the PUSCH or PUCCH transmission slot or subslot is shortened due to multiplexing with SRS, the EVM measurement interval is reduced by one symbol, accordingly. Likewise, when the PUSCH starting position is modified or when second last symbol is the ending symbol of the PUSCH subframe for Frame Structure Type 3, the EVM measurement interval is reduced accordingly. The PUSCH or PUCCH EVM measurement interval is also reduced when the mean power, modulation or allocation between slots or subslots is expected to change. In the case of PUSCH transmission, the measurement interval is reduced by a time interval equal to the sum of 5 μs and the applicable exclusion period defined in subclause 6.3.4, adjacent to the boundary where the power change is expected to occur. The PUSCH exclusion period is applied to the signal obtained after the front-end IDFT. In the case of PUCCH transmission with power change, the PUCCH EVM measurement interval is reduced by one symbol adjacent to the boundary where the power change is expected to occur.

Table 6.5.2.1-1: Measurement interval for EVM

| TTI pattern | Measurement interval |
| --- | --- |
| Subframe | 7OS |
| Slot | 7OS |
| Subslot | 2OS, 3OS |

##### 6.5.2.1.1	Minimum requirement

The RMS average of the basic EVM measurements for 10 subframes excluding any transient period for the average EVM case, and 60 subframes excluding any transient period for the reference signal EVM case, for the different modulations schemes shall not exceed the values specified in Table 6.5.2.1.1-1 for the parameters defined in Table 6.5.2.1.1-2. For EVM evaluation purposes, [all PRACH preamble formats 0-4 and] all PUCCH formats 1, 1a, 1b, 2, 2a and 2b are considered to have the same EVM requirement as QPSK modulated.

Table 6.5.2.1.1-1: Minimum requirements for Error Vector Magnitude

| Parameter | Unit | Average EVM Level | Reference Signal EVM Level |
| --- | --- | --- | --- |
| QPSK or BPSK | % | 17.5 | 17.5 |
| 16 QAM | % | 12.5 | 12.5 |
| 64 QAM | % | 8 | 8 |
| 256 QAM | % | 3.5 | 3.5 |

Table 6.5.2.1.1-2: Parameters for Error Vector Magnitude

| Parameter | Unit | Level |
| --- | --- | --- |
| UE Output Power | dBm | -40 |
| UE Output Power for 256 QAM | dBm | -30 |
| Operating conditions |  | Normal conditions |

#### 6.5.2.2		Carrier leakage

Carrier leakage is an additive sinusoid waveform that has the same frequency as a modulated waveform carrier frequency. The measurement interval is one slot in the time domain.

##### 6.5.2.2.1		Minimum requirements

The relative carrier leakage power is a power ratio of the additive sinusoid waveform and the modulated waveform. The relative carrier leakage power shall not exceed the values specified in Table 6.5.2.2.1-1.

Table 6.5.2.2.1-1: Minimum requirements for relative carrier leakage power

| Parameters | Relative limit (dBc) | Applicable frequencies |
| --- | --- | --- |
| Output power >10 dBm | -28 | Carrier center frequency < 1 GHz |
|  | -25 | Carrier center frequency ≥ 1 GHz |
| 0 dBm ≤ Output power ≤10 dBm | -25 |  |
| -30 dBm ≤ Output power ≤0 dBm | -20 |  |
| -40 dBm Output power < -30 dBm (NOTE 1) | -10 |  |
| NOTE 1: The requirement in this power range is not applicable for V2X sidelink transmission for E-UTRA V2X operating bands specified in Table 5.5G-1. |  |  |

#### 6.5.2.3	In-band emissions

The in-band emission is defined as the average across 12 sub-carrier and as a function of the RB offset from the edge of the allocated UL transmission bandwidth. The in-band emission is measured as the ratio of the UE output power in a non–allocated RB to the UE output power in an allocated RB.

The basic in-band emissions measurement interval is defined over one slot in the time domain. When the PUSCH or PUCCH transmission slot is shortened due to multiplexing with SRS, the in-band emissions measurement interval is reduced by one SC-FDMA symbol, accordingly. Likewise, when the PUSCH starting position is modified or when the second last symbol is the ending symbol of the PUSCH sub-frame for Frame Structure Type 3, the in-band emissions measurement interval is reduced accordingly.

##### 6.5.2.3.1		Minimum requirements

The relative in-band emission shall not exceed the values specified in Table 6.5.2.3.1-1.

Table 6.5.2.3.1-1: Minimum requirements for in-band emissions

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | For Frame Structure Type 1 and Frame Structure Type 2: For Frame Structure Type 3, 10 MHz and 20 MHz channel bandwidths: max-10-6∆RB-1,-57 dBm/180 kHz-PRB |  | Any non-allocated (NOTE 2) |
| IQ Image | dB | -28 | Image frequencies when carrier center frequency < 1 GHz and Output power > 10 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -25 | Image frequencies when carrier center frequency < 1 GHz and Output power ≤ 10 dBm |  |
|  |  | -25 | Image frequencies when carrier center frequency ≥ 1 GHz |  |
| Carrier leakage | dBc | -28 | Output power > 10 dBm and carrier center frequency < 1 GHz | Carrier frequency (NOTES 4, 5) |
|  |  | -25 | Output power > 10 dBm and carrier center frequency ≥ 1 GHz |  |
|  |  | -25 | 0 dBm ≤ Output power ≤10 dBm |  |
|  |  | -20 | -30 dBm ≤ Output power ≤ 0 dBm |  |
|  |  | -10 | -40 dBm Output power < -30 dBm (NOTE 12) |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of PRB - 30 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in NOTE 10. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. For Frame Structure Type 3 and 20 MHz channel bandwidth, the requirement applies for QPSK, 16QAM and 64QAM modulation with for any non-allocated RB andin the uplink scheduling grant with specified in [6]. For Frame Structure Type 3 and 10 MHz channel bandwidth in Band 49, the requirement applies for QPSK, 16QAM and 64QAM modulation with for any non-allocated RB andin the uplink scheduling grant. NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the centre carrier frequency, but excluding any allocated RBs. For Frame Structure Type 3 with 20 MHz channel bandwidth, the applicable frequency is for QPSK, 16QAM and 64QAM modulation. For Frame Structure Type 3 and 10 MHz channel bandwidth in Band 49, the applicable frequency is for QPSK, 16QAM and 64QAM modulation. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. For Frame Structure Type 3 and 20 MHz channel bandwidth, the requirement applies for QPSK, 16QAM and 64QAM modulation within the uplink scheduling grant. For Frame Structure Type 3 and 10 MHz channel bandwidth in Band 49, the requirement applies for QPSK, 16QAM and 64QAM modulation within the uplink scheduling grant. NOTE 5: The applicable frequencies for this limit are those that are enclosed in the RBs containing the DC frequency if is odd, or in the two RBs immediately adjacent to the DC frequency if is even, but excluding any allocated RB. NOTE 6: is the Transmission Bandwidth (see Figure 5.6-1). NOTE 7: is the Transmission Bandwidth Configuration (see Figure 5.6-1). NOTE 8: is the limit specified in Table 6.5.2.1.1-1 for the modulation format used in the allocated RBs. NOTE 9: is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. or for the first adjacent RB outside of the allocated bandwidth. NOTE 10: is the transmitted power per 180 kHz in allocated RBs, measured in dBm. [NOTE 11: For V2V-V2X waveforms, the requirements are applied when PSSCH and PSCCH are adjacent in frequency. The limit values (General, IQ Image or Carrier leakage) of each channel is calculated separately as Pgeneral, PSSCH, PIQ, PSSCH, PLO, PSSCH, Pgeneral, PSCCH, PIQ, PSCCH, PLO, PSCCH ,respectively. In-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of PRB, PSSCH - 30 dB and the PsumIBE.SSCH, where PsumIBE.SSCH is the linear sum of Pgeneral, PSSCH, PIQ, PSSCH, PLO, PSSCH, Pgeneral, PSCCH, PIQ, PSCCH, PLO, PSCCH.] NOTE 12: The requirement in this power range is not applicable for V2X sidelink transmission for E-UTRA V2X operating bands specified in Table 5.5G-1. |  |  |  |  |

NOTE:	For Frame Structure 3 and operations in Band 46, in-band emissions requirements are not specified for the 10 MHz channel bandwidth.

#### 6.5.2.4		EVM equalizer spectrum flatness

The zero-forcing equalizer correction applied in the EVM measurement process (as described in Annex F) must meet a spectral flatness requirement for the EVM measurement to be valid. The EVM equalizer spectrum flatness is defined in terms of the maximum peak-to-peak ripple of the equalizer coefficients (dB) across the allocated uplink block. The basic measurement interval is the same as for EVM.

##### 6.5.2.4.1	Minimum requirements

The peak-to-peak variation of the EVM equalizer coefficients contained within the frequency range of the uplink allocation shall not exceed the maximum ripple specified in Table 6.5.2.4.1-1 for normal conditions. For uplink allocations contained within both Range 1 and Range 2, the coefficients evaluated within each of these frequency ranges shall meet the corresponding ripple requirement and the following additional requirement: the relative difference between the maximum coefficient in Range 1 and the minimum coefficient in Range 2 must not be larger than 5 dB, and the relative difference between the maximum coefficient in Range 2 and the minimum coefficient in Range 1 must not be larger than 7 dB (see Figure 6.5.2.4.1-1).

The EVM equalizer spectral flatness shall not exceed the values specified in Table 6.5.2.4.1-2 for extreme conditions. For uplink allocations contained within both Range 1 and Range 2, the coefficients evaluated within each of these frequency ranges shall meet the corresponding ripple requirement and the following additional requirement: the relative difference between the maximum coefficient in Range 1 and the minimum coefficient in Range 2 must not be larger than 6 dB, and the relative difference between the maximum coefficient in Range 2 and the minimum coefficient in Range 1 must not be larger than 10 dB (see Figure 6.5.2.4.1-1).

Table 6.5.2.4.1-1: Minimum requirements for EVM equalizer spectrum flatness (normal conditions)

| Frequency range | Maximum ripple [dB] |
| --- | --- |
| FUL_Meas – FUL_Low ≥ 3 MHz and FUL_High – FUL_Meas ≥ 3 MHz (Range 1) | 4 (p-p) |
| FUL_Meas – FUL_Low < 3 MHz or FUL_High – FUL_Meas < 3 MHz (Range 2) | 8 (p-p) |
| NOTE 1: FUL_Meas refers to the sub-carrier frequency for which the equalizer coefficient is evaluated NOTE 2: FUL_Low and FUL_High refer to each E-UTRA frequency band specified in Table 5.5-1 |  |

Table 6.5.2.4.1-2: Minimum requirements for EVM equalizer spectrum flatness (extreme conditions)

| Frequency range | Maximum Ripple [dB] |
| --- | --- |
| FUL_Meas – FUL_Low ≥ 5 MHz and FUL_High – FUL_Meas ≥ 5 MHz (Range 1) | 4 (p-p) |
| FUL_Meas – FUL_Low < 5 MHz or FUL_High – FUL_Meas < 5 MHz (Range 2) | 12 (p-p) |
| NOTE 1: FUL_Meas refers to the sub-carrier frequency for which the equalizer coefficient is evaluated NOTE 2: FUL_Low and FUL_High refer to each E-UTRA frequency band specified in Table 5.5-1 |  |

f

FUL_High

FUL_High – 3(5) MHz

< 4(4) dBp-p

Range 1

Range 2

max(Range 1)-min(Range 2) < 5(6) dB

max(Range 2)-min(Range 1) < 7(10) dB

< 8(12) dBp-p

Figure 6.5.2.4.1-1: The limits for EVM equalizer spectral flatness with the maximum allowed variation of the coefficients indicated (the ETC minimum requirement within brackets).

### 6.5.2A	Transmit modulation quality for CA

For inter-band carrier aggregation with uplink assigned to two E-UTRA bands, the requirements shall apply on each component carrier as defined in clause 6.5.2 with all component carriers active. If two contiguous component carriers are assigned to one E-UTRA band, the requirements in subclauses 6.5.2A.1, 6.5.2A.2, and 6.5.2A.3 apply for those component carriers.

The requirements in this clause apply with PCC and SCC in the UL configured and activated: PCC with PRB allocation and SCC without PRB allocation and without CSI reporting and SRS configured.

#### 6.5.2A.1	Error Vector Magnitude

For the intra-band contiguous and non-contiguous carrier aggregation, the Error Vector Magnitude requirement should be defined for each component carrier. Requirements only apply with PRB allocation in one of the component carriers. Similar transmitter impairment removal procedures are applied for CA waveform before EVM calculation as is specified for non-CA waveform in sub-section 6.5.2.1.

When a single component carrier is configured Table 6.5.2.1.1-1 apply.

The EVM requirements are according to Table 6.5.2A.1-1 if CA is configured in uplink with the parameters defined in Table 6.5.2.1.1-2.

Table 6.5.2A.1-1: Minimum requirements for Error Vector Magnitude

| Parameter | Unit | Average EVM Level per CC | Reference Signal EVM Level |
| --- | --- | --- | --- |
| QPSK or BPSK | % | 17.5 | 17.5 |
| 16 QAM | % | 12.5 | 12.5 |
| 64 QAM | % | 8 | 8 |
| 256 QAM | % | 3.5 | 3.5 |

#### 6.5.2A.2	Carrier leakage for CA

Carrier leakage is an additive sinusoid waveform that is confined within the aggrecated transmission bandwidth configuration. The carrier leakage requirement is defined for each component carrier and is measured on the component carrier with PRBs allocated. The measurement interval is one slot in the time domain.

#### 6.5.2A.2.1	Minimum requirements

The relative carrier leakage power is a power ratio of the additive sinusoid waveform and the modulated waveform. The relative carrier leakage power shall not exceed the values specified in Table 6.5.2A.2.1-1.

Table 6.5.2A.2.1-1: Minimum requirements for Relative Carrier Leakage Power

| Parameters | Relative Limit (dBc) |
| --- | --- |
| Output power >0 dBm | -25 |
| -30 dBm ≤ Output power ≤0 dBm | -20 |
| -40 dBm Output power < -30 dBm | -10 |

#### 6.5.2A.3	In-band emissions

##### 6.5.2A.3.1	Minimum requirement for CA

For intra-band contiguous carrier aggregation bandwidth class B, C and D, the requirements in Table 6.5.2A.3.1-1 and 6.5.2A.3.1-2 apply within the aggregated transmission bandwidth configuration with both component carrier (s) active and one single contiguous PRB allocation of bandwidth  at the edge of the aggregated transmission bandwidth configuration.

The inband emission is defined as the interference falling into the non allocated resource blocks for all component carriers. The measurement method for the inband emissions in the component carrier with PRB allocation is specified in annex F. For a non allocated component carrier a spectral measurement is specified.

For intra-band non-contiguous carrier aggregation the requirements for in-band emissions should be defined for each component carrier. Requirements only apply with PRB allocation in one of the component carriers according to Table 6.5.2.3.1.

Table 6.5.2A.3.1-1: Minimum requirements for in-band emissions (allocated component carrier)

| Parameter | Unit | Limit |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB |  |  | Any non-allocated (NOTE 2) |
| IQ Image | dB | -25 |  | Exception for IQ image (NOTE 3) |
| Carrier leakage | dBc | -25 | Output power > 0 dBm | Exception for Carrier frequency (NOTE 4) |
|  |  | -20 | -30 dBm ≤ Output power ≤ 0 dBm |  |
|  |  | -10 | -40 dBm Output power < -30 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of PRB - 30 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in NOTE 9. The limit is evaluated in each non-allocated RB. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs NOTE 3: Exceptions to the general limit are allowed for up to +1 RBs within a contiguous width of +1 non-allocated RBs. The measurement bandwidth is 1 RB. NOTE 4: Exceptions to the general limit are allowed for up to two contiguous non-allocated RBs. The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in the non-allocated RB to the measured total power in all allocated RBs. NOTE 5: is the Transmission Bandwidth (see Figure 5.6-1) not exceeding NOTE 6: is the Transmission Bandwidth Configuration (see Figure 5.6-1) of the component carrier with RBs allocated. NOTE 7: is the limit specified in Table 6.5.2.1.1-1 for the modulation format used in the allocated RBs. NOTE 8: is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. or for the first adjacent RB outside of the allocated bandwidth). NOTE 9: is the transmitted power per 180 kHz in allocated RBs, measured in dBm. NOTE 10: For V2X intra-band contiguous multi-carrier operation the in-band emission requirement (General, IQ Image or Carrier leakage) of each channel is calculated separately as Pgeneral, PSSCH,c(i), PIQ, PSSCH,c(i), PLO, PSSCH,c(i), Pgeneral, PSCCH,c(i), PIQ, PSCCH,c(i), PLO, PSCCH,c(i) for each carrier respectively. In-band emissions combined limit for the aggregated bandwidth is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the PsumIBE.CA, where PsumIBE.CA is the linear sum of Pgeneral, PSSCH,c(i), PIQ, PSSCH,c(i), PLO, PSSCH,c(i), Pgeneral, PSCCH,c(i), PIQ, PSCCH,c(i), PLO, PSCCH,c(i) for all carriers. |  |  |  |  |

Table 6.5.2A.3.1-2: Minimum requirements for in-band emissions (not allocated component carrier)

| Para-meter | Unit | Meas BW NOTE 1 | Limit |  | remark | Applicable Frequencies |
| --- | --- | --- | --- | --- | --- | --- |
| General | dB | BW of 1 RB (180KHz rectangular) |  |  | The reference value is the average power per allocated RB in the allocated component carrier | Any RB in the non allocated component carrier. The frequency raster of the RBs is derived when this component carrier is allocated with RBs |
| IQ Image | dB | BW of 1 RB (180KHz rectangular) | -25 NOTE 2 |  | The reference value is the average power per allocated RB in the allocated component carrier | The frequencies of the contiguous non-allocated RBs are unknown. The frequency raster of the RBs is derived when this component carrier is allocated with RBs |
| Carrier leakage | dBc | BW of 1 RB (180KHz rectangular) | NOTE 3 |  | The reference value is the total power of the allocated RBs in the allocated component carrier | The frequencies of the up to 2 non-allocated RBs are unknown. The frequency raster of the RBs is derived when this component carrier is allocated with RBs |
|  |  |  | -25 | Output power > 0 dBm |  |  |
|  |  |  | -20 | -30 dBm ≤ Output power ≤ 0 dBm |  |  |
|  |  |  | -10 | -40 dBm Output power < -30 dBm |  |  |
| NOTE1: Resolution BWs smaller than the measurement BW may be integrated to achieve the measurement bandwidth. NOTE 2: Exceptions to the general limit is are allowed for up to +1 RBs within a contiguous width of +1 non-allocated RBs. NOTE 3: Two Exceptions to the general limit are allowed for up to two contiguous non-allocated RBs NOTE 4: NOTES 1, 5, 6, 7, 8, 9 from Table 6.5.2A.3.1-1 apply for Table 6.5.2A.3.1-2 as well. NOTE 5: for measured non-allocated RB in the non allocated component carrier may take non-integer values when the carrier spacing between the CCs is not a multiple of RB. NOTE 6: For V2X intra-band contiguous multi-carrier operation the in-band emission requirement (General, IQ Image or Carrier leakage) of each channel is calculated separately as Pgeneral, PSSCH,c(i), PIQ, PSSCH,c(i), PLO, PSSCH,c(i), Pgeneral, PSCCH,c(i), PIQ, PSCCH,c(i), PLO, PSCCH,c(i) for each carrier respectively. In-band emissions combined limit for the aggregated bandwidth is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the PsumIBE.CA, where PsumIBE.CA is the linear sum of Pgeneral, PSSCH,c(i), PIQ, PSSCH,c(i), PLO, PSSCH,c(i), Pgeneral, PSCCH,c(i), PIQ, PSCCH,c(i), PLO, PSCCH,c(i) for all carriers. |  |  |  |  |  |  |

### 6.5.2B	Transmit modulation quality for UL-MIMO

For UE supporting UL-MIMO, the transmit modulation quality requirements are specified at each transmit antenna connector.

If UE is configured for transmission on single-antenna port, the requirements in subclause 6.5.2 apply.

The transmit modulation quality is specified in terms of:

-	Error Vector Magnitude (EVM) for the allocated resource blocks (RBs)

-	EVM equalizer spectrum flatness derived from the equalizer coefficients generated by the EVM measurement process

-	Carrier leakage (caused by IQ offset)

-	In-band emissions for the non-allocated RB

#### 6.5.2B.1	Error Vector Magnitude

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the Error Vector Magnitude requirements specified in Table 6.5.2.1.1-1 which is defined in subclause 6.5.2.1 apply at each transmit antenna connector. The requirements shall be met with the UL-MIMO configurations specified in Table 6.2.2B-2.

#### 6.5.2B.2	Carrier leakage

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the Relative Carrier Leakage Power requirements specified in Table 6.5.2.2.1-1 which is defined in subclause 6.5.2.2 apply at each transmit antenna connector. The requirements shall be met with the UL-MIMO configurations specified in Table 6.2.2B-2.

#### 6.5.2B.3	In-band emissions

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the In-band Emission requirements specified in Table 6.5.2.3.1-1 which is defined in subclause 6.5.2.3 apply at each transmit antenna connector. The requirements shall be met with the uplink MIMO configurations specified in Table 6.2.2B-2.

#### 6.5.2B.4	EVM equalizer spectrum flatness for UL-MIMO

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the EVM Equalizer Spectrum Flatness requirements specified in Table 6.5.2.4.1-1 and Table 6.5.2.4.1-2 which are defined in subclause 6.5.2.4 apply at each transmit antenna connector. The requirements shall be met with the UL-MIMO configurations specified in Table 6.2.2B-2.

### 6.5.2D	Transmit modulation quality for ProSe

The requirements in this clause apply to ProSe sidelink transmissions.

When UE is configured for simultaneous E-UTRA ProSe sidelink and E-UTRA uplink transmissions for inter-band E-UTRA ProSe / E-UTRA bands specified in Table 5.5D-2, the requirements in subclause 6.5.2D apply for ProSe transmission and the requirements in subclause 6.5.2 apply for uplink transmission.

#### 6.5.2D.1	Error Vector Magnitude

For ProSe sidelink physical channels PSDCH, PSCCH, PSSCH, and PSBCH, the Error Vector Magnitude requirements shall be as specified for PUSCH in subclause 6.5.2.1 for the corresponding modulation and transmission bandwidth. When ProSe transmissions are shortened due to transmission gap of 1 symbol at the end of the subframe, the EVM measurement interval is reduced by one symbol, accordingly.

For PSBCH the duration over which EVM is averaged shall be 24 subframes.

This requirement is not applicable for ProSe physical signals PSSS and SSSS.

#### 6.5.2D.2	Carrier leakage

The requirements of subcaluse 6.5.2.2 shall apply for ProSe transmissions.

#### 6.5.2D.3	In-band emissions

For ProSe sidelink physical channels PSDCH, PSCCH, PSSCH, and PSBCH, the In-band emissions requirements shall be as specified for PUSCH in subclause 6.5.2.3 for the corresponding modulation and transmission bandwidth. When ProSe transmissions are shortened due to transmission gap of 1 symbol at the end of the subframe, the In-band emissions measurement interval is reduced by one symbol, accordingly.

#### 6.5.2D.4	EVM equalizer spectrum flatness for ProSe

The requirements of subcaluse 6.5.2.4 shall apply for ProSe transmissions.

### 6.5.2E	Transmit modulation quality for category M1 and M2

#### 6.5.2E.1	Error Vector Magnitude

#### The Error Vector Magnitude is defined in section 6.5.2.1.

#### 6.5.2E.2	Carrier leakage

Carrier leakage is an additive sinusoid waveform that has the same frequency as a modulated waveform carrier frequency. For UE of UL Categories M1 and M2, the sinusoid waveform may lie at the center of the narrowband assigned for transmission. The measurement interval is one slot in the time domain.

##### 6.5.2E.2.1	Minimum requirements

The relative carrier leakage power is a power ratio of the additive sinusoid waveform and the modulated waveform. The relative carrier leakage power at the center of the channel bandwidth or the center of the narrowband assigned for transmission shall not exceed the values specified in Table 6.5.2.2.1-1.

#### 6.5.2E.3	In-band emissions

The in-band emission is defined in clause 6.5.2.3 and measurement condition specified in Annex F.

##### 6.5.2E.3.1	Minimum requirements

The relative in-band emission when the center carrier frequency is either at the center of channel bandwidth or at the center of the narrowband assigned for transmission shall not exceed the values specified in Table 6.5.2E.3.1-1

Table 6.5.2E.3.1-1: Minimum requirements for in-band emissions

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB | -18-5∙¦∆SubG-1)/LSCG |  | Any non-allocated Subcarrier Group within the subPRB allocation (NOTE 11,12,13) |
|  |  |  |  | Any non-allocated (NOTE 2) |
| IQ Image | dB | -28 | Image frequencies when carrier center frequency < 1 GHz and Output power > 10 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -25 | Image frequencies when carrier center frequency < 1 GHz and Output power ≤ 10 dBm |  |
|  |  | -25 | Image frequencies when carrier center frequency ≥ 1 GHz |  |
| Carrier leakage | dBc | -28 | Output power > 10 dBm and carrier center frequency < 1 GHz | Carrier frequency (NOTES 4, 5) |
|  |  | -25 | Output power > 10 dBm and carrier center frequency ≥ 1 GHz |  |
|  |  | -25 | 0 dBm ≤ Output power ≤10 dBm |  |
|  |  | -20 | -30 dBm ≤ Output power ≤ 0 dBm |  |
|  |  | -10 | -40 dBm Output power < -30 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of PRB - 30 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in NOTE 10. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the centre carrier frequency, but excluding any allocated RBs. For UE of UL Categories M1 and M2, the applicable frequencies shall alternatively include those found by reflection on the center of the assigned narrowband, but excluding any allocated RBs. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit are those that are enclosed in the RBs containing the DC frequency if is odd, or in the two RBs immediately adjacent to the DC frequency if is even, but excluding any allocated RB. For UE of UL Categories M1 and M2, the applicable frequencies shall alternatively be the centre frequency of the supported 6RBs. NOTE 6: is the Transmission Bandwidth (see Figure 5.6-1). NOTE 7: is the Transmission Bandwidth Configuration (see Figure 5.6-1). NOTE 8: is the limit specified in Table 6.5.2.1.1-1 for the modulation format used in the allocated RBs. NOTE 9: is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. or for the first adjacent RB outside of the allocated bandwidth. NOTE 10: is the transmitted power per 180 kHz in allocated RBs, measured in dBm. NOTE 11: The measurement bandwidth is 1 subcarrier group ( [3] subcarrier per subcarrier group) and the limit is expressed as a ratio of measured power in one non-allocated subcarrier group to the measured total power in all allocated subcarrier. NOTE 12: ∆SubG is the starting frequency offset between the allocated subcarrier group and the measured non-allocated subcarrier group (e.g. ∆SubG=1 or ∆SubG =-1 for the first adjacent subcarrier group outside the allocated subcarrier group.) NOTE 13: LSCG is the Transmission bandwidth (number of subcarrier group). |  |  |  |  |

### 6.5.2F	Transmit modulation quality for Category NB1 and NB2

#### 6.5.2F.1	Error Vector Magnitude

The RMS average of the basic EVM measurements for 240/LCtone slots excluding any transient period for the average EVM case, where LCtone = {1, 3, 6, 12} is the number of subcarriers for the category NB1 and NB2 transmission, for the different modulations schemes shall not exceed the values specified in Table 6.5.2.1.1-1 for the parameters defined in Table 6.5.2.1.1-2. For EVM evaluation purposes, both NPRACH formats are considered to have the same EVM requirement as QPSK modulated.

#### 6.5.2F.2	Carrier leakage

Carrier leakage is an additive sinusoid waveform that has the same frequency as a modulated waveform carrier frequency. The measurement interval is one slot in the time domain. The relative carrier leakage power is a power ratio of the additive sinusoid waveform and the modulated waveform. The relative carrier leakage power of category NB1 or NB2 UE shall not exceed the values specified in Table 6.5.2F.2-1.

Table 6.5.2F.2-1: Minimum requirements for relative carrier leakage power

| Parameters | Relative limit (dBc) |
| --- | --- |
| 0 dBm ≤ Output power | -25 |
| -30 dBm ≤ Output power ≤ 0 dBm | -20 |
| -40 dBm Output power < -30 dBm | -10 |

#### 6.5.2F.3	In-band emissions

The in-band emission is defined as a function of the tone offset from the edge of the allocated UL transmission tone(s) within the transmission bandwidth configuration. The in-band emission is measured as the ratio of the UE output power in a non–allocated tone to the UE output power in an allocated tone. The basic in-band emissions measurement interval is defined over one slot in the time domain.

The category NB1 and NB2 UE relative in-band emission shall not exceed the values specified in Table 6.5.2F.3-1.

Table 6.5.2F.3-1: Minimum requirements for in-band emissions

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB |  |  | Any non-allocated (NOTE 2) |
| IQ Image | dB | -25 |  | Image frequencies (NOTES 2, 3) |
| Carrier leakage | dBc | -25 | 0 dBm ≤ Output power | Carrier frequency (NOTES 4, 5) |
|  |  | -20 | -30 dBm ≤ Output power ≤ 0 dBm |  |
|  |  | -10 | -40 dBm Output power < -30 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated tone. For each such tone, the minimum requirement is calculated as the higher of Ptone - 30 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. Ptone is defined in NOTE 9. NOTE 2: The measurement bandwidth is 1 tone and the limit is expressed as a ratio of measured power in one non-allocated tone to the measured average power per allocated tone, where the averaging is done across all allocated tones. NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the centre carrier frequency, but excluding any allocated tones. NOTE 4: The measurement bandwidth is 1 tone and the limit is expressed as a ratio of measured power in one non-allocated tone to the measured total power in all allocated tones. NOTE 5: The applicable frequencies for this limit are those that are enclosed in the tones containing the DC frequency if is odd, or in the two tones immediately adjacent to the DC frequency if is even, but excluding any allocated tone. NOTE 6: is the Transmission Bandwidth (tones). NOTE 7: is the Transmission Bandwidth Configuration (tones). NOTE 8: is the starting frequency offset between the allocated tone and the measured non-allocated tone. (e.g. or for the first adjacent tone outside of the allocated bandwidth. NOTE 9: is the transmitted power per 3.75 kHz or 15 kHz in allocated tones, measured in dBm. |  |  |  |  |

### 6.5.2G	Transmit modulation quality for V2X Communication

The requirements in this clause apply to V2X sidelink transmissions.

When UE is configured for simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions for inter-band E-UTRA V2X / E-UTRA bands specified in Table 5.5G-2, the requirements in subclause 6.5.2G apply for V2X sidelink transmission and the requirements in subclause 6.5.2 apply for the E-UTRA uplink transmission.

For V2X UE supporting Transmit Diversity, if the UE transmits on two antenna-connectors at the same time, the transmit modulation quality requirements for single carrier shall apply to each transmit antenna connector.

If V2X UE transmits on one-antenna connector at a time, the requirements specified for single carrier apply to the active antenna connector.

#### 6.5.2G.1	Error Vector Magnitude

For V2X physical channels PSCCH, PSSCH and PSBCH, the Error Vector Magnitude requirements shall be as specified for PUSCH in subclause 6.5.2.1 for the corresponding modulation and transmission bandwidth.

For V2X sidelink physical channels PSCCH, PSSCH and PSBCH, the Error Vector Magnitude requirements shall be as specified separately for PSSCH and PSCCH for the corresponding modulation and transmission bandwidth. The measurement period for EVM of PSSCH and PSCCH is 15 subframes. The measurement period for reference signal EVM is 30 subframes. When V2X transmissions are shortened due to transmission gap of 1 symbol at the end of the subframe, the EVM measurement interval is reduced by one symbol, accordingly.

For PSBCH the duration over which EVM is averaged shall be 24 subframes.

For intra-band contiguous multi-carrier operation the EVM requirement shall apply for each component carrier.

#### 6.5.2G.2	Carrier leakage

The requirements of subcaluse 6.5.2.2 shall apply for V2X transmissions.

For intra-band contiguous multi-carrier operation the carrier leakage requirement of subcaluse 6.5.2A.2 shall apply.

#### 6.5.2G.3	In-band emissions

For V2X sidelink physical channels PSCCH, PSSCH and PSBCH, the In-band emissions requirements shall be as specified for PUSCH in subclause 6.5.2.3 for the corresponding modulation and transmission bandwidth. When V2X transmissions are shortened due to transmission gap of 1 symbol at the end of the subframe, the In-band emissions measurement interval is reduced by one symbol, accordingly.

For intra-band contiguous multi-carrier operation the in-band emission requirement of subcaluse 6.5.2A.3 shall apply.

#### 6.5.2G.4	EVM equalizer spectrum flatness

The requirements of subcaluse 6.5.2.4 shall apply for V2X transmissions.

For intra-band contiguous multi-carrier operation the EVM equalizer spectrum flatness requirement of subcaluse 6.5.2.4 shall apply for each component carrier.

## 6.6	Output RF spectrum emissions

The output UE transmitter spectrum consists of the three components; the emission within the occupied bandwidth (channel bandwidth), the Out Of Band (OOB) emissions and the far out spurious emission domain.

Figure 6.6-1: Transmitter RF spectrum

### 6.6.1	Occupied bandwidth

Occupied bandwidth is defined as the bandwidth containing 99 % of the total integrated mean power of the transmitted spectrum on the assigned channel. The occupied bandwidth for all transmission bandwidth configurations (Resources Blocks) shall be less than the channel bandwidth specified in Table 6.6.1-1

Table 6.6.1-1: Occupied channel bandwidth

|  | Occupied channel bandwidth / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Channel bandwidth (MHz) | 1.4 | 3 | 5 | 10 | 15 | 20 |

#### 6.6.1.1	Additional minimum requirement for E-UTRA (network signalled value “NS_29”)

For E-UTRA CA bands including one uplink LAA Scell in Band 46 with "NS_29" indicated, the occupied bandwidth for all transmission bandwidth configurations (Resources Blocks) shall be less than or equal to 19 MHz and 19.7MHz for E-UTRA carriers of 20 MHz bandwidth assigned within 5150-5350 MHz and 5470-5725 MHz, respectively.

### 6.6.1A	Occupied bandwidth for CA

For inter-band carrier aggregation with one component carrier per operating band and the uplink active in two E-UTRA bands the occupied bandwidth is defined per component carrier. Occupied bandwidth is the bandwidth containing 99 % of the total integrated mean power of the transmitted spectrum on assigned channel bandwidth on the component carrier. The occupied bandwidth shall be less than the channel bandwidth specified in Table 6.6.1-1.

For intra-band contiguous carrier aggregation the occupied bandwidth is a measure of the bandwidth containing 99 % of the total integrated power of the transmitted spectrum. The OBW shall be less than the aggregated channel bandwidth defined in subclause 5.6A.

For intra-band non-contiguous carrier aggregation sub-block occupied bandwidth is defined as the bandwidth containing 99 % of the total integrated mean power of the transmitted spectrum on the sub-block. In case the sub-block consist of one component carrier the occupied bandwidth of the sub-block shall be less than the channel bandwidth specified in Table 6.6.1-1.

For combinations of intra-band and inter-band carrier aggregation with three uplink component carriers (up to two contiguously aggregated carriers per band), the occupied bandwidth is the bandwidth containing 99 % of the total integrated mean power of the transmitted spectrum on each E-UTRA band. The OBW shall be less than the channel bandwidth as specified in Table 6.6.1-1 for the E-UTRA band supporting one component carrier. The OBW shall be less than the aggregated channel bandwidth as specified in subclause 5.6A for the E-UTRA band supporting two contiguous component carriers.

### 6.6.1B		Occupied bandwidth for UL-MIMO

For UE supporting UL-MIMO, the requirements for occupied bandwidth is specified at each transmit antenna connector. The occupied bandwidth is defined as the bandwidth containing 99 % of the total integrated mean power of the transmitted spectrum on the assigned channel at each transmit antenna connector.

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the occupied bandwidth at each transmitter antenna shall be less than the channel bandwidth specified in Table 6.6.1B-1. The requirements shall be met with the UL-MIMO configurations specified in Table 6.2.2B-2.

Table 6.6.1B-1: Occupied channel bandwidth

|  | Occupied channel bandwidth / Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Channel bandwidth (MHz) | 1.4 | 3 | 5 | 10 | 15 | 20 |

If UE is configured for transmission on single-antenna port, the requirements in subclause 6.6.1 apply.

### 6.6.1F	Occupied bandwidth for category NB1 and NB2

The occupied bandwidth is defined as the bandwidth containing 99 % of the total integrated mean power of the transmitted spectrum on the assigned channel at the transmit antenna connector. Occupied bandwidth shall be less than the channel bandwidth of category NB1 and NB2 specified in Section 5.6F.

### 6.6.1G	Occupied bandwidth for V2X Communication

When UE is configured for E-UTRA V2X sidelink transmissions non-concurrent with E-UTRA uplink transmissions for E-UTRA V2X operating bands specified in Table Table 5.5G-1, the requirements in subclause 6.6.1 apply for E-UTRA V2X sidelink transmission.

When UE is configured for simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions for inter-band E-UTRA V2X / E-UTRA bands specified in Table 5.5G-2, the requirements in subclause 6.6.1 apply for V2X sidelink transmission and the E-UTRA uplink transmission.

For intra-band contiguous multi-carrier operation, the occupied bandwidth is a measure of the bandwidth containing 99 % of the total integrated power of the transmitted spectrum. The OBW shall be less than the aggregated channel bandwidth defined in subclause 5.6A.

For V2X UE supporting Transmit Diversity, if the UE transmits on two antenna connectors at the same time, the requirements for occupied bandwidth is specified at each transmit antenna connector and the occupied bandwidth at each transmitter antenna shall be less than the channel bandwidth specified for single carrier.

If V2X UE transmits on one antenna connector at a time, the requirements specified for single carrier shall apply to the active antenna connector.

### 6.6.2	Out of band emission

The Out of band emissions are unwanted emissions immediately outside the assigned channel bandwidth resulting from the modulation process and non-linearity in the transmitter but excluding spurious emissions. This out of band emission limit is specified in terms of a spectrum emission mask and an Adjacent Channel Leakage power Ratio.

#### 6.6.2.1	Spectrum emission mask

The spectrum emission mask of the UE applies to frequencies (ΔfOOB) starting from the  edge of the assigned E-UTRA channel bandwidth. For frequencies offset greater than ΔfOOB as specified in Table 6.6.2.1.1-1 the spurious requirements in subclause 6.6.3 are applicable.

##### 6.6.2.1.1	Minimum requirement

The power of any UE emission shall not exceed the levels specified in Table 6.6.2.1.1-1 for the specified channel bandwidth.

Table 6.6.2.1.1-1: General E-UTRA spectrum emission mask

| Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -10 | -13 | -15 | -18 | -20 | -21 | 30 kHz |
| 1-2.5 | -10 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| 2.5-2.8 | -25 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| 2.8-5 |  | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| 5-6 |  | -25 | -13 | -13 | -13 | -13 | 1 MHz |
| 6-10 |  |  | -25 | -13 | -13 | -13 | 1 MHz |
| 10-15 |  |  |  | -25 | -13 | -13 | 1 MHz |
| 15-20 |  |  |  |  | -25 | -13 | 1 MHz |
| 20-25 |  |  |  |  |  | -25 | 1 MHz |

NOTE:	 As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

#### 6.6.2.1A	Spectrum emission mask for CA

For inter-band carrier aggregation with one component carrier per operating band and the uplink active in two E-UTRA bands, the spectrum emission mask of the UE is defined per component carrier while both component carriers are active and the requirements are specified in subclauses 6.6.2.1 and 6.6.2.2. If for some frequency spectrum emission masks of component carriers overlap then spectrum emission mask allowing higher power spectral density applies for that frequency. If for some frequency a component carrier spectrum emission mask overlaps with the channel bandwidth of another component carrier, then the emission mask does not apply for that frequency.

For intra-band contiguous carrier aggregation the spectrum emission mask of the UE applies to frequencies (ΔfOOB) starting from the  edge of the aggregated channel bandwidth (Table 5.6A-1) For intra-band contiguous carrier aggregation the bandwidth class B, C and D, the power of any UE emission shall not exceed the levels specified in Table 6.6.2.1A-0, Table 6.6.2.1A-1 and Table 6.6.2.1A-2 for the specified channel bandwidth.

Table 6.6.2.1A-0: General E-UTRA CA spectrum emission mask for Bandwidth Class B

| ΔfOOB (MHz) | 25RB+25RB (9.8MHz) | 25RB+50RB (14.95 MHz) | 25RB+75RB (19.8MHz) | 50RB+50RB (19.9 MHz) | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- |
| 0-1 | -18 | -20 | -21 | -21 | 30 kHz |
| 1-5 | -10 | -10 | -10 | -10 | 1 MHz |
| 5-9.8 | -13 | -13 | -13 | -13 | 1 MHz |
| 9.8-14.8 | -25 | -13 | -13 | -13 | 1 MHz |
| 14.8-14.95 |  | -13 | -13 | -13 | 1 MHz |
| 14.95-19.80 |  | -25 | -13 | -13 | 1 MHz |
| 19.80-19.90 |  | -25 | -25 | -13 | 1 MHz |
| 19.90-19.95 |  | -25 | -25 | -25 | 1 MHz |
| 19.95-24.80 |  |  | -25 | -25 | 1 MHz |
| 24.80-24.90 |  |  |  | -25 | 1 MHz |

Table 6.6.2.1A-1: General E-UTRA CA spectrum emission mask for Bandwidth Class C

| Spectrum emission limit [dBm]/BWChannel_CA |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 25RB+100RB (24.95MHz) | 50RB+75RB (24.75 MHz) | 50RB+100RB (29.9 MHz) | 75RB+75RB (30 MHz) | 75RB+100RB (34.85 MHz) | 100RB+100RB (39.8 MHz) | Measurement bandwidth |
| 0-1 | -22 | -22 | -22.5 | -22.5 | -23.5 | -24 | 30 kHz |
| 1-5 | -10 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| 5-24.75 | -13 | -13 | -13 | -13 | -13 | -13 | 1 MHz |
| 24.75-24.95 | -13 | -25 | -13 | -13 | -13 | -13 | 1 MHz |
| 24.95-29.75 | -25 | -25 | -13 | -13 | -13 | -13 | 1 MHz |
| 29.75-29.9 | -25 |  | -13 | -13 | -13 | -13 | 1 MHz |
| 29.9-29.95 | -25 |  | -25 | -13 | -13 | -13 | 1 MHz |
| 29.95-30 |  |  | -25 | -13 | -13 | -13 | 1 MHz |
| 30-34.85 |  |  | -25 | -25 | -13 | -13 | 1 MHz |
| 34.85-34.9 |  |  | -25 | -25 | -25 | -13 | 1 MHz |
| 34.9-35 |  |  |  | -25 | -25 | -13 | 1 MHz |
| 35-39.8 |  |  |  |  | -25 | -13 | 1 MHz |
| 39.8-39.85 |  |  |  |  | -25 | -25 | 1 MHz |
| 39.85-44.8 |  |  |  |  |  | -25 | 1 MHz |

Table 6.6.2.1A-2: General E-UTRA CA spectrum emission mask for Bandwidth Class D

| Spectrum emission limit [dBm]/BWChannel_CA |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB | 50RB+75RB+100RB | 75RB+75RB+75RB | 50RB+100RB+100 RB | 75RB+75RB+100RB | 75RB+100RB+100RB | 100RB+100RB+100 RB | Measurement bandwidth |
| (MHz) | (44.6MHz) | (45MHz) | (49.7MHz) | (49.85MHz) | (54.65MHz) | (59.6MHz) |  |
|  0-1 | -22 | -22 | -22.5 | -22.5 | -23.5 | -24 | 30 kHz |
|  1-5 | -10 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
|  5-44.6 | -13 | -13 | -13 | -13 | -13 | -13 | 1 MHz |
|  44.6-45 | -25 | -13 | -13 | -13 | -13 | -13 | 1 MHz |
|  45-49.6 | -25 | -25 | -13 | -13 | -13 | -13 | 1 MHz |
|  49.6-49.7 |  | -25 | -13 | -13 | -13 | -13 | 1 MHz |
|  49.7-49.85 |  | -25 | -25 | -13 | -13 | -13 | 1 MHz |
|  49.85-50 |  | -25 | -25 | -25 | -13 | -13 | 1 MHz |
|  50-54.65 |  |  | -25 | -25 | -13 | -13 | 1 MHz |
|  54.65-54.7 |  |  | -25 | -25 | -25 | -13 | 1 MHz |
|  54.7-54.85 |  |  |  | -25 | -25 | -13 | 1 MHz |
|  54.85-59.6 |  |  |  |  | -25 | -13 | 1 MHz |
|  59.6-59.65 |  |  |  |  | -25 | -25 | 1 MHz |
|  59.65-64.6 |  |  |  |  |  | -25 | 1 MHz |

For intra-band non-contiguous carrier aggregation transmission the spectrum emission mask requirement is defined as a composite spectrum emissions mask. Composite spectrum emission mask applies to frequencies up to  ΔfOOB starting from the edges of the sub-blocks. Composite spectrum emission mask is defined as follows

a)	Composite spectrum emission mask is a combination of individual sub-block spectrum emissions masks

b)	In case the sub-block consist of one component carrier the sub-lock general spectrum emission mask is defined in subclause 6.6.2.1.1

c)	If for some frequency sub-block spectrum emission masks overlap then spectrum emission mask allowing higher power spectral density applies for that frequency

d)	If for some frequency a sub-block spectrum emission mask overlaps with the sub-block bandwidth of another sub-block, then the emission mask does not apply for that frequency.

For combinations of intra-band and inter-band carrier aggregation with three uplink component carriers (up to two contiguously aggregated carriers per band), the spectrum emission mask of the UE is defined per E-UTRA band while all component carriers are active. For the E-UTRA band supporting one component carrier the requirements in subclauses 6.6.2.1 and 6.6.2.2 apply. For the E-UTRA band supporting two contiguous component carriers the requirements specified in subclause 6.6.2.1A apply. If for some frequency spectrum emission masks of single component carrier and two contiguous component carriers overlap then spectrum emission mask allowing higher power spectral density applies for that frequency. If for some frequency spectrum emission masks of single component carrier or two contiguous component carriers overlap then the emission mask does not apply for that frequency.

#### 6.6.2.2	Additional spectrum emission mask

This requirement is specified in terms of an "additional spectrum emission" requirement.

##### 6.6.2.2.1	Minimum requirement (network signalled value "NS_03", “NS_11”, "NS_20", and “NS_21”)

Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

When "NS_03", "NS_11", "NS_20" or "NS_21" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.2.2.1-1.

Table 6.6.2.2.1-1: Additional requirements

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -10 | -13 | -15 | -18 | -20 | -21 | 30 kHz |
| 1-2.5 | -13 | -13 | -13 | -13 | -13 | -13 | 1 MHz |
| 2.5-2.8 | -25 | -13 | -13 | -13 | -13 | -13 | 1 MHz |
| 2.8-5 |  | -13 | -13 | -13 | -13 | -13 | 1 MHz |
| 5-6 |  | -25 | -13 | -13 | -13 | -13 | 1 MHz |
| 6-10 |  |  | -25 | -13 | -13 | -13 | 1 MHz |
| 10-15 |  |  |  | -25 | -13 | -13 | 1 MHz |
| 15-20 |  |  |  |  | -25 | -13 | 1 MHz |
| 20-25 |  |  |  |  |  | -25 | 1 MHz |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

##### 6.6.2.2.2	Minimum requirement (network signalled value "NS_04")

Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

When "NS_04" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.2.2.2-1.

Table 6.6.2.2.2-1: Additional requirements

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -15 | -18 | -20 | -21 | 30 kHz |
| 1-2.5 | -10 | -10 | -10 | -10 | 1 MHz |
| 2.5-2.8 | -10 | -10 | -10 | -10 | 1 MHz |
| 2.8-5 | -10 | -10 | -10 | -10 | 1 MHz |
| 5-6 | -13 | -13 | -13 | -13 | 1 MHz |
| 6-9 | -25 | -13 | -13 | -13 | 1 MHz |
| 9-10 | -25 | -25 | -13 | -13 | 1 MHz |
| 10-13.5 |  | -25 | -13 | -13 | 1 MHz |
| 13.5-15 |  | -25 | -25 | -13 | 1 MHz |
| 15-18 |  |  | -25 | -13 | 1 MHz |
| 18-20 |  |  | -25 | -25 | 1 MHz |
| 20-25 |  |  |  | -25 | 1 MHz |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

##### 6.6.2.2.3	Minimum requirement (network signalled value "NS_06" or “NS_07”)

Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

When "NS_06" or “NS_07” is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.2.2.3-1.

Table 6.6.2.2.3-1: Additional requirements

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | Measurement bandwidth |
| 0-0.1 | -13 | -13 | -15 | -18 | 30 kHz |
| 0.1-1 | -13 | -13 | -13 | -13 | 100 kHz |
| 1-2.5 | -13 | -13 | -13 | -13 | 1 MHz |
| 2.5-2.8 | -25 | -13 | -13 | -13 | 1 MHz |
| 2.8-5 |  | -13 | -13 | -13 | 1 MHz |
| 5-6 |  | -25 | -13 | -13 | 1 MHz |
| 6-10 |  |  | -25 | -13 | 1 MHz |
| 10-15 |  |  |  | -25 | 1 MHz |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

##### 6.6.2.2.4	Minimum requirement (network signalled value "NS_33" or “NS_34”)

The additional spectrum mask in Table 6.6.2.2.4-1 applies for E-UTRA V2X UE within 5 855 MHz to 5 950 MHz according to ETSI EN 302 571. Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

When "NS_33" or “NS_34” is indicated in the cell, the power of any V2X UE emission shall not exceed the levels specified in Table 6.6.2.2.4-1.

Table 6.6.2.2.4-1: Additional requirements for 10MHz channel bandwidth

| Spectrum emission limit (dBm EIRP)/ Channel bandwidth |  |  |
| --- | --- | --- |
| ΔfOOB (MHz) | 10 MHz | Measurement bandwidth |
| 0-0.5 | [-13-12∆fOOBMHz] | 100 kHz |
| 0.5-5 | [-19-169∆fOOBMHz-0.5] | 100 kHz |
| 5-10 | [-27-2∆fOOBMHz-5.0] | 100 kHz |

NOTE 1:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

NOTE 2:	Additional SEM for V2X overrides any other requirements in frequency range 5855-5950MHz.

NOTE 3:	The EIRP requirement is converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex I.

##### 6.6.2.2.5	Minimum requirement (network signalled value “NS_27” and “NS_43”)

Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

When “NS_27” or “NS_43” is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.2.2.5-1.

Table 6.6.2.2.5-1: Additional requirements

|  | Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -13 | -13 | -13 | -13 | Note 1 |
| 1-10 | -13 | -13 | -13 | -13 | 1 MHz |
| 10-15 |  | -25 | -25 | -25 | 1 MHz |
| 15-20 |  |  | -25 | -25 | 1 MHz |
| 20-25 |  |  |  | -25 | 1 MHz |
| Note 1: The measurement bandwidth is 1% of the applicable E-UTRA channel bandwidth. |  |  |  |  |  |

##### 6.6.2.2.6	Minimum requirement (network signalled value "NS_28”)

When "NS_28” is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.2.2.6-1 for E-UTRA channels assigned within the frequency ranges 5150-5350 and 5470-5725 MHz.

Table 6.6.2.2.6-1: Additional requirements

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement [dBm] | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 0.5 MHz | 0.5 MHz f_offset < 1 MHz | 10 – 20(f_offset/MHz) dB | 1 MHz |
| 0.5 MHz f < 9.5MHz | 1 MHz f_offset < 10 MHz | -10 – 8/9(f_offset/MHz – 1) dB | 1 MHz |
| 9.5 MHz f < 19.5 MHz | 10 MHz f_offset < 20 MHz | -28 – 1.2(f_offset/MHz – 10) dB | 1 MHz |
| 19.5 MHz f | 20 MHz f_offset | -30 | 1 MHz |
| NOTE 1: The measurement filter -3dB point is that closest to the channel edge. NOTE 2: The requirement applies when the offset of the measurement filter centre frequency is such that both -3 dB points of the measurement filter are confined within any of the two frequency ranges 5150-5250 MHz and 5470-5725 MHz. |  |  |  |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

##### 6.6.2.2.7	Minimum requirement (network signalled value "NS_35")

Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

When "NS_35" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.2.2.7-1.

Table 6.6.2.2.7-1: Additional requirements

| ΔfOOB (MHz) | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth (unless otherwise stated) |
| --- | --- | --- | --- | --- | --- |
| 0-0.1 | -15 | -18 | -20 | -21 | 30 kHz |
| 0.1-6 | -13 | -13 | -13 | -13 | 100 kHz |
| 6-10 | -251 | -13 | -13 | -13 | 100 kHz |
| 10-15 |  | -251 | -13 | -13 | 100 kHz |
| 15-20 |  |  | -251 | -13 | 100 kHz |
| 20-25 |  |  |  | -25 | 1 MHz |
| Note 1: The measurement bandwidth shall be 1 MHz |  |  |  |  |  |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

#### 6.6.2.2A	Additional Spectrum Emission Mask for CA

This requirement is specified in terms of an "additional spectrum emission" requirement.

##### 6.6.2.2A.1	Minimum requirement (network signalled value "CA_NS_04")

Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

When "CA_NS_04" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.2.2A.1-1.

Table 6.6.2.2A.1-1: Additional requirements

| Spectrum emission limit [dBm]/BWChannel_CA |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 50+75RB (24.75 MHz) | 25+100RB (24.95 MHz) | 50+100RB (29.9 MHz) | 75+75RB (30 MHz) | 75+100RB (34.85 MHz) | 100+100RB (39.8 MHz) | Measurement bandwidth |
| 0-1 | -22 | -22 | -22.5 | -23 | -23.5 | -24 | 30 kHz |
| 1-5 | -10 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| 5-22.95 | -13 | -13 | -13 | -13 | -13 | -13 | 1 MHz |
| 22.95-23.25 | -13 | -25 | -13 | -13 | -13 | -13 | 1 MHz |
| 23.25-27.9 | -25 | -25 | -13 | -13 | -13 | -13 | 1 MHz |
| 27.9-28.5 | -25 | -25 | -25 | -13 | -13 | -13 | 1 MHz |
| 28.5-29.75 | -25 | -25 | -25 | -25 | -13 | -13 | 1 MHz |
| 29.75-29.95 |  | -25 | -25 | -25 | -13 | -13 | 1 MHz |
| 29.95-32.85 |  |  | -25 | -25 | -13 | -13 | 1 MHz |
| 32.85-34.9 |  |  | -25 | -25 | -25 | -13 | 1 MHz |
| 34.9-35 |  |  |  | -25 | -25 | -13 | 1 MHz |
| 35-37.8 |  |  |  |  | -25 | -13 | 1 MHz |
| 37.8-39.85 |  |  |  |  | -25 | -25 | 1 MHz |
| 39.85-44.8 |  |  |  |  |  | -25 | 1 MHz |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

##### 6.6.2.2A.2	Minimum requirement CA_66B (network signalled value "CA_NS_09")

Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

When "CA_NS_09" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.2.2A.2-1.

Table 6.6.2.2A.2-1: Additional requirements

| ΔfOOB (MHz) | 25RB+25RB (9.8MHz) | 25RB+50RB (14.95 MHz) | 25RB+75RB (19.8MHz) | 50RB+50RB (19.9 MHz) | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- |
| 0-1 | -18 | -20 | -21 | -21 | 30 kHz |
| 1-5 | -13 | -13 | -13 | -13 | 1 MHz |
| 5-9.8 | -13 | -13 | -13 | -13 | 1 MHz |
| 9.8-14.8 | -25 | -13 | -13 | -13 | 1 MHz |
| 14.8-14.95 |  | -13 | -13 | -13 | 1 MHz |
| 14.95-19.80 |  | -25 | -13 | -13 | 1 MHz |
| 19.80-19.90 |  | -25 | -25 | -13 | 1 MHz |
| 19.90-19.95 |  | -25 | -25 | -25 | 1 MHz |
| 19.95-24.80 |  |  | -25 | -25 | 1 MHz |
| 24.80-24.90 |  |  |  | -25 | 1 MHz |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

##### 6.6.2.2A.3	Minimum requirement CA_66C (network signalled value "CA_NS_09")

Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

When "CA_NS_09" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.2.2A.3-1.

Table 6.6.2.2A.3-1: Additional requirements

| Spectrum emission limit [dBm]/BWChannel_CA |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 25RB+100RB (24.95MHz) | 50RB+75RB (24.75 MHz) | 50RB+100RB (29.9 MHz) | 75RB+75RB (30 MHz) | 75RB+100RB (34.85 MHz) | 100RB+100RB (39.8 MHz) | Measurement bandwidth |
| 0-1 | -22 | -22 | -22.5 | -22.5 | -23.5 | -24 | 30 kHz |
| 1-5 | -13 | -13 | -13 | -13 | -13 | -13 | 1 MHz |
| 5-24.75 | -13 | -13 | -13 | -13 | -13 | -13 | 1 MHz |
| 24.75-24.95 | -13 | -25 | -13 | -13 | -13 | -13 | 1 MHz |
| 24.95-29.75 | -25 | -25 | -13 | -13 | -13 | -13 | 1 MHz |
| 29.75-29.9 | -25 |  | -13 | -13 | -13 | -13 | 1 MHz |
| 29.9-29.95 | -25 |  | -25 | -13 | -13 | -13 | 1 MHz |
| 29.95-30 |  |  | -25 | -13 | -13 | -13 | 1 MHz |
| 30-34.85 |  |  | -25 | -25 | -13 | -13 | 1 MHz |
| 34.85-34.9 |  |  | -25 | -25 | -25 | -13 | 1 MHz |
| 34.9-35 |  |  |  | -25 | -25 | -13 | 1 MHz |
| 35-39.8 |  |  |  |  | -25 | -13 | 1 MHz |
| 39.8-39.85 |  |  |  |  | -25 | -25 | 1 MHz |
| 39.85-44.8 |  |  |  |  |  | -25 | 1 MHz |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

##### 6.6.2.2A.4	Minimum requirement CA_48B and CA_48C (network signalled value "CA_NS_10")

Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

When "CA_NS_10" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.2.2A.4-1.

Table 6.6.2.2A.4-1: Additional requirements for “CA_NS_10”

|  | Spectrum emission limit (dBm) / measurement bandwidth for each channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB MHz | 25+100RB (24.95MHz) | 50+50RB (19.9 MHz) | 50+100RB (29.9 MHz) | 75+100RB (34.85 MHz) | 100+100RB (39.8 MHz) | Measurement bandwidth |
| ± 0 - 1 | -13 |  |  |  |  | 1 % channel bandwidth |
| ± 1 - X | -13 |  |  |  |  | 1 MHz |
| < – X or > X when 3540 MHz < ΔfOOB < 3710 MHz | -25 |  |  |  |  |  |
| NOTE: X is aggregated channel bandwidth as defined in clause 5.6A |  |  |  |  |  |  |

NOTE:	As a general rule, the resolution bandwidth of the measuring equipment should be equal to the measurement bandwidth. However, to improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

#### 6.6.2.3	Adjacent Channel Leakage Ratio

Adjacent Channel Leakage power Ratio (ACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency. ACLR requirements for one E-UTRA carrier are specified for two scenarios for an adjacent E-UTRA and /or UTRA channel as shown in Figure 6.6.2.3-1.

Figure 6.6.2.3-1: Adjacent Channel Leakage requirements for one E-UTRA carrier

##### 6.6.2.3.1	Minimum requirement E-UTRA

E-UTRA Adjacent Channel Leakage power Ratio (E-UTRAACLR) is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency at nominal channel spacing. The assigned E-UTRA channel power and adjacent E-UTRA channel power are measured with rectangular filters with measurement bandwidths specified in Table 6.6.2.3.1-1, Table 6.6.2.3.1-2, and Table 6.6.2.3.1-3. If the measured adjacent channel power is greater than –50dBm then the E-UTRAACLR shall be higher than the value specified in Table 6.6.2.3.1-1, Table 6.6.2.3.1-2, and Table 6.6.2.3.1-3.

For a power class 2 capable UE operating on Band 41, when an IE P-max as defined in [7] of 23 dBm or lower is indicated in the cell or if the uplink/downlink configuration is 0 or 6, the requirements for power class 2 are not applicable,  and the corresponding requirements for a power class 3 UE shall apply.

For each supported frequency band other than Band 14 and Band 41, the UE shall:

-	if the UE supports a different power class than the default UE power class for the band and the supported power class enables the higher maximum output power than that of the default power class:

-	if the band is a TDD band whose frame configuration is 0 or 6; or

-	if the IE P-Max as defined in TS 36.331 [7] is not provided; or

-	if the IE P-Max as defined in TS 36.331 [7] is provided and set to the maximum output power of the default power class or lower;

-	meet all requirements for the default power class of the operating band in which the UE is operating and set its configured transmitted power as specified in sub-clause 6.2.5;

-	else (i.e the IE P-Max as defined in TS 36.331 [7] is provided and set to the higher value than the maximum output power of the default power class):

-	meet all requirements for the supported power class and set its configured transmitted power class as specified in sub-clause 6.2.5.

Table 6.6.2.3.1-1: General requirements for E-UTRAACLR

|  | Channel bandwidth / E-UTRAACLR1 / Measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| E-UTRAACLR1 | 30 dB | 30 dB | 30 dB | 30 dB | 30 dB | 30 dB |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| Adjacent channel centre frequency offset [MHz] | +1.4 / -1.4 | +3.0 / -3.0 | +5 / -5 | +10 / -10 | +15 / -15 | +20 / -20 |

Table 6.6.2.3.1-2: Additional E-UTRAACLR requirements for Power Class 1

|  | Channel bandwidth / E-UTRAACLR1 / Measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| E-UTRAACLR1 | 37 dB | 37 dB | 37 dB | 37 dB | 37 dB | 37 dB |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| Adjacent channel centre frequency offset [MHz] | +1.4 / -1.4 | +3.0 / -3.0 | +5 / -5 | +10 / -10 | +15 / -15 | +20 / -20 |
| NOTE 1: E-UTRAACLR1 shall be applicable for >23dBm |  |  |  |  |  |  |

Table 6.6.2.3.1-3: Additional E-UTRAACLR requirements for Power Class 2

|  | Channel bandwidth / E-UTRAACLR1 / Measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| E-UTRAACLR1 | N/A | N/A | 31 dB | 31 dB | 31 dB | 31 dB |
| E-UTRA channel Measurement bandwidth | N/A | N/A | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| Adjacent channel centre frequency offset [MHz] | N/A | N/A | +5 / -5 | +10 / -10 | +15 / -15 | +20 / -20 |

##### 6.6.2.3.1a	Additional minimum requirement for E-UTRA (network signalled value “NS_29”)

When "NS_29" is indicated in the cell, the UE emission shall meet the additional requirements specified in Table 6.6.2.3.1a-1 for E-UTRA channels assigned within the frequency ranges 5150-5350 MHz and 5470-5725 MHz. The assigned E-UTRA channel power and alternative adjacent E-UTRA channel power are measured with rectangular filters with measurement bandwidths specified in Table 6.6.2.3.1a-1. If the measured alternative adjacent channel power is greater than –50dBm then the E-UTRAACLR2 shall be higher than the value specified in Table 6.6.2.3.1a-1.

Table 6.6.2.3.1a-1: Additional E-UTRAACLR requirement

|  | Channel bandwidth / E-UTRAACLR2 / Measurement bandwidth |
| --- | --- |
|  | 20 MHz |
| E-UTRAACLR2 | 40 dBc |
| E-UTRA channel Measurement bandwidth | NOTE 1 |
| Adjacent channel centre frequency offset [MHz] | +40 / -40 |
| NOTE 1: 18 MHz for E-UTRA channels assigned within 5150-5350 MHz; 19 MHz for E-UTRA channels assigned within 5470-5725 MHz. |  |

##### 6.6.2.3.1A	Void

##### 6.6.2.3.1Aa	Void

##### 6.6.2.3.2	Minimum requirements UTRA

UTRA Adjacent Channel Leakage power Ratio (UTRAACLR) is the ratio of the filtered mean power centred on the assigned E-UTRA channel frequency to the filtered mean power centred on an adjacent(s) UTRA channel frequency.

UTRA Adjacent Channel Leakage power Ratio is specified for both the first UTRA adjacent channel (UTRAACLR1) and the 2nd UTRA adjacent channel (UTRAACLR2). The UTRA channel power is measured with a RRC bandwidth filter with roll-off factor =0.22. The assigned E-UTRA channel power is measured with a rectangular filter with measurement bandwidth specified in Table 6.6.2.3.2-1. If the measured UTRA channel power is greater than –50dBm then the UTRAACLR shall be higher than the value specified in Table 6.6.2.3.2-1.

UTRAACLR is not applicable to the power class 3 UE operating in Band 7, 12, 13, 17, 20, 24, 27, 30, 33, 35, 36, 37, 38, 40, 43, 44, 45, 47, 48, 50, 51, 52, 68, 70, 71, 85 and Scell operation in Band 46, 49.

UTRAACLR is not applicable to the power class 2 UE operating in Band 38, 40, 41, 42 or 47 and Scell operation in Band 46.

UTRAACLR is not applicable to the power class 1 UE operating in Band 3, 20, 28, 31 or 72.

Table 6.6.2.3.2-1: Requirements for UTRAACLR1/2

|  | Channel bandwidth / UTRAACLR1/2 / Measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| UTRAACLR1 | 33 dB | 33 dB | 33 dB | 33 dB | 33 dB | 33 dB |
| Adjacent channel centre frequency offset [MHz] | 0.7+BWUTRA/2 / -0.7-BWUTRA/2 | 1.5+BWUTRA/2 / -1.5-BWUTRA/2 | +2.5+BWUTRA/2 / -2.5-BWUTRA/2 | +5+BWUTRA/2 / -5-BWUTRA/2 | +7.5+BWUTRA/2 / -7.5-BWUTRA/2 | +10+BWUTRA/2 / -10-BWUTRA/2 |
| UTRAACLR2 | - | - | 36 dB | 36 dB | 36 dB | 36 dB |
| Adjacent channel centre frequency offset [MHz] | - | - | +2.5+3*BWUTRA/2 / -2.5-3*BWUTRA/2 | +5+3*BWUTRA/2 / -5-3*BWUTRA/2 | +7.5+3*BWUTRA/2 / -7.5-3*BWUTRA/2 | +10+3*BWUTRA/2 / -10-3*BWUTRA/2 |
| E-UTRA channel Measurement bandwidth | 1.08 MHz | 2.7 MHz | 4.5 MHz | 9.0 MHz | 13.5 MHz | 18 MHz |
| UTRA 5MHz channel Measurement bandwidth (NOTE 1) | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz | 3.84 MHz |
| UTRA 1.6MHz channel measurement bandwidth (NOTE 2) | 1.28 MHz | 1.28 MHz | 1.28 MHz | 1.28MHz | 1.28MHz | 1.28MHz |
| NOTE 1: Applicable for E-UTRA FDD co-existence with UTRA FDD in paired spectrum. NOTE 2: Applicable for E-UTRA TDD co-existence with UTRA TDD in unpaired spectrum. |  |  |  |  |  |  |

##### 6.6.2.3.2A	Minimum requirement UTRA for CA

For inter-band carrier aggregation with one component carrier per operating band and the uplink active in two E-UTRA bands, the UTRA Adjacent Channel Leakage power Ratio (UTRAACLR) is the ratio of the filtered mean power centred on the assigned channel bandwidth on the component carrier to the filtered mean power centred on an adjacent channel frequency. The UTRA Adjacent Channel Leakage power Ratio is defined per carrier and the requirement is specified in subclause 6.6.2.3.2.

For intra-band contiguous carrier aggregation the UTRA Adjacent Channel Leakage power Ratio (UTRAACLR) is the ratio of the filtered mean power centred on the aggregated channel bandwidth to the filtered mean power centred on an adjacent(s) UTRA channel frequency.

For intra-band non-contiguous carrier aggregation when all sub-blocks consist of one component carrier the UTRA Adjacent Channel Leakage power Ratio (UTRAACLR) is the ratio of the sum of the filtered mean powers centered on the assigned sub-block frequencies to the filtered mean power centred on an adjacent(s) UTRA channel frequency. UTRAACLR1/2 requirements are applicaple for all sub-blocks and are specified in Table 6.6.2.3.2A-2. UTRAACLR1 is required to be met in the sub-block gap when the gap bandwidth Wgap is 5MHz≤Wgap <15MHz. Both UTRAACLR1 and UTRAACLR2 are required to be met in the sub-block gap when the gap bandwidth Wgap is 15MHz≤Wgap.

For combinations of intra-band and inter-band carrier aggregation with three uplink component carriers (up to two contiguously aggregated carriers per band), the UTRA Adjacent Channel Leakage power Ratio (UTRAACLR) is defined as follows. For the E-UTRA band supporting one component carrier, the UTRA Adjacent Channel Leakage power Ratio (UTRAACLR) is the ratio of the filtered mean power centred on the assigned channel bandwidth of the component carrier to the filtered mean power centred on an adjacent(s) UTRA channel frequency and the requirements specified in subclause 6.6.2.3.2 apply. For the E-UTRA band supporting two contiguous component carriers the UTRA Adjacent Channel Leakage power Ratio (UTRAACLR) is the ratio of the filtered mean power centred on the aggregated channel bandwidth to the filtered mean power centred on an adjacent(s) UTRA channel frequency and the requirements specified in subclause 6.6.2.3.2A apply.

UTRA Adjacent Channel Leakage power Ratio is specified for both the first UTRA adjacent channel (UTRAACLR1) and the 2nd UTRA adjacent channel (UTRAACLR2). The UTRA channel power is measured with a RRC bandwidth filter with roll-off factor =0.22. The assigned aggregated channel bandwidth power is measured with a rectangular filter with measurement bandwidth specified in Table 6.6.2.3.2A-1 for intraband contiguous carrier aggregation or 6.6.2.3.2A-2 for intraband non-contiguous carrier aggregation. If the measured UTRA channel power is greater than –50dBm then the UTRAACLR shall be higher than the value specified in Table 6.6.2.3.2A-1 for intraband contiguous carrier aggregation or 6.6.2.3.2A-2 for intraband non-contiguous carrier aggregation.

For carrier aggregation with one or two uplink component carriers, the UTRAACLR requirements for the PC3 UE are not applicable to the uplink component carrier(s) assigned to one of the E-UTRA band in Band 7, 12, 13, 17, 20, 24, 27, 30, 33, 35, 36, 37, 38, 40, 43, 44, 45, 46, 47, 48, 49, 50, 51, 52, 68, 70, 71 or 85.

Table 6.6.2.3.2A-1: Requirements for UTRAACLR1/2

|  | CA bandwidth class / UTRAACLR1/2 / measurement bandwidth |
| --- | --- |
|  | CA bandwidth class B, C and D |
| UTRAACLR1 | 33 dB |
| Adjacent channel centre frequency offset (in MHz) | + BWChannel_CA /2 + BWUTRA/2 / - BWChannel_CA / 2 - BWUTRA/2 |
| UTRAACLR2 | 36 dB |
| Adjacent channel centre frequency offset (in MHz) | + BWChannel_CA /2 + 3*BWUTRA/2 / - BWChannel_CA /2 – 3*BWUTRA/2 |
| CA E-UTRA channel Measurement bandwidth | BWChannel_CA - 2* BWGB |
| UTRA 5MHz channel Measurement bandwidth (NOTE 1) | 3.84 MHz |
| UTRA 1.6MHz channel measurement bandwidth (NOTE 2) | 1.28 MHz |
| NOTE 1: Applicable for E-UTRA FDD co-existence with UTRA FDD in paired spectrum. NOTE 2: Applicable for E-UTRA TDD co-existence with UTRA TDD in unpaired spectrum. |  |

Table 6.6.2.3.2A-2: Requirements for intraband non-contiguous CA UTRAACLR1/2

|  | UTRAACLR1/2 / measurement bandwidth |
| --- | --- |
| UTRAACLR1 | 33 dB |
| Adjacent channel centre frequency offset (in MHz) | + Fedge,block,high + BWUTRA/2 / - Fedge,block,low - BWUTRA/2 |
| UTRAACLR2 | 36 dB |
| Adjacent channel centre frequency offset (in MHz) | + Fedge,block,high + 3*BWUTRA/2 / - Fedge,block,low – 3*BWUTRA/2 |
| Sub-block measurement bandwidth | BWChannel,block - 2* BWGB |
| UTRA 5 MHz channel Measurement bandwidth (NOTE 1) | 3.84 MHz |
| UTRA 1.6 MHz channel measurement bandwidth (NOTE 2) | 1.28 MHz |
| NOTE 1: Applicable for E-UTRA FDD co-existence with UTRA FDD in paired spectrum. NOTE 2: Applicable for E-UTRA TDD co-existence with UTRA TDD in unpaired spectrum. |  |

##### 6.6.2.3.3A	Minimum requirements for CA E-UTRA

For intra-band contiguous carrier aggregation the carrier aggregation E-UTRA Adjacent Channel Leakage power Ratio (CA E-UTRAACLR) is the ratio of the filtered mean power centred on the aggregated channel bandwidth to the filtered mean power centred on an adjacent aggregated channel bandwidth at nominal channel spacing. The assigned aggregated channel bandwidth power and adjacent aggregated channel bandwidth power are measured with rectangular filters with measurement bandwidths specified in Table 6.6.2.3.3A-1. If the measured adjacent channel power is greater than –50dBm then the E-UTRAACLR shall be higher than the value specified in Table 6.6.2.3.3A-1 and Table 6.6.2.3.3A-1a.

Table 6.6.2.3.3A-1: General requirements for CA E-UTRAACLR

|  | CA bandwidth class / CA E-UTRAACLR / Measurement bandwidth |
| --- | --- |
|  | CA bandwidth class B, C and D |
| CA E-UTRAACLR | 30 dB |
| CA E-UTRA channel Measurement bandwidth | BWChannel_CA - 2* BWGB |
| Adjacent channel centre frequency offset (in MHz) | + BWChannel_CA / - BWChannel_CA |

Table 6.6.2.3.3A-1a: Additional requirements for CA E-UTRAACLR for UL CA_41C Power Class 2

|  | CA bandwidth class / CA E-UTRAACLR / Measurement bandwidth |
| --- | --- |
|  | CA bandwidth class C |
| CA E-UTRAACLR | 31 dB |
| CA E-UTRA channel Measurement bandwidth | BWChannel_CA - 2* BWGB |
| Adjacent channel centre frequency offset (in MHz) | + BWChannel_CA / - BWChannel_CA |

For inter-band carrier aggregation with one component carrier per operating band and the uplink active in two E-UTRA bands, E-UTRA Adjacent Channel Leakage power Ratio (E-UTRAACLR) is the ratio of the filtered mean power centred on the assigned channel bandwidth on a component carrier to the filtered mean power centred on an adjacent channel frequency. The E-UTRA Adjacent Channel Leakage power Ratio is defined per carrier and the requirement is specified in subclause 6.6.2.3.1.

For intra-band non-contiguous carrier aggregation when all sub-blocks consist of one component carrier the E-UTRA Adjacent Channel Leakage power Ratio (E-UTRAACLR) is the ratio of the sum of the filtered mean powers centred on the assigned sub-block frequencies to the filtered mean power centred on an adjacent channel frequency at nominal channel spacing. In case the sub-block gap bandwidth Wgap is smaller than of the sub-block bandwidth then for that sub-block no E-UTRAACLR requirement is set for the gap. In case the sub-block gab bandwidth Wgap is smaller than either of the sub-block bandwidths then no E- UTRAACLR requirement is set for the gap.The assigned E-UTRA sub-block power and adjacent E-UTRA channel power are measured with rectangular filters with measurement bandwidths specified in Table 6.6.2.3.3A-2. If the measured adjacent channel power is greater than –50dBm then the E-UTRAACLR shall be higher than the value specified in Table 6.6.2.3.3A-2.

Table 6.6.2.3.3A-2: General requirements for non-contiguous intraband CA E-UTRAACLR

|  | CC and adjacent channel bandwidth / E-UTRAACLR / Measurement bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| E-UTRAACLR1 | 30 dB | 30 dB | 30 dB | 30 dB | 30 dB | 30 dB |
| CC and adjacent channel measurement bandwidth [MHz] | 1.08 | 2.7 | 4.5 | 9 | 13.5 | 18 |
| Adjacent channel centre frequency offset [MHz] | + 1.4 / - 1.4 | + 3 / - 3 | + 5 / - 5 | + 10 / - 10 | + 15 / - 15 | + 20 / - 20 |

For combinations of intra-band and inter-band carrier aggregation with three uplink component carriers (up to two contiguously aggregated carriers per band), the E-UTRA Adjacent Channel Leakage power Ratio (E-UTRAACLR) is defined as follows. For the E-UTRA band supporting one component carrier, the E-UTRA Adjacent Channel Leakage power Ratio (UTRAACLR) is the ratio of the filtered mean power centred on the assigned channel bandwidth of the component carrier to the filtered mean power centred on an adjacent channel frequency and the requirements in subclause 6.6.2.3.1 apply. For the E-UTRA band supporting two contiguous component carriers the E-UTRA Adjacent Channel Leakage power Ratio (E-UTRAACLR) is the ratio of the filtered mean power centred on the aggregated channel bandwidth to the filtered mean power centred on an adjacent(s) aggregated channel bandwidth at nominal channel spacing and the requirements of CA E-UTRAACLR specified in subclause 6.6.2.3.3A apply.

#### 6.6.2.4	Void

##### 6.6.2.4.1	Void

### 6.6.2A	Void

<reserved for future use>

### 6.6.2B		Out of band emission for UL-MIMO

For UE supporting UL-MIMO, the requirements for Out of band emissions resulting from the modulation process and non-linearity in the transmitters are specified at each transmit antenna connector.

For UEs with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the requirements in subclause 6.6.2 apply to each transmit antenna connector. The requirements shall be met with the UL-MIMO configurations specified in Table 6.2.2B-2.

If UE is configured for transmission on single-antenna port, the requirements in subclause 6.6.3 apply.

### 6.6.2C		Void

<reserved for future use>

### 6.6.2D		Out of band emission for ProSe

When UE is configured for E-UTRA ProSe sidelink transmissions non-concurrent with E-UTRA uplink transmissions for E-UTRA ProSe operating bands specified in Table 5.5D-1, the requirements in subclause 6.6.2 apply.

When UE is configured for simultaneous E-UTRA ProSe sidelink and E-UTRA uplink transmissions for inter-band E-UTRA ProSe / E-UTRA bands specified in Table 5.5D-2, the requirements in subclause 6.6.2 apply per E-UTRA ProSe sidelink and E-UTRA uplink transmission as specified for the corresponding inter-band aggregation with uplink assigned to two bands.

### 6.6.2F	Out of band emission for category NB1 and NB2

#### 6.6.2F.1	Spectrum emission mask

The spectrum emission mask of the category NB1 and NB2 UE applies to frequencies (ΔfOOB) starting from the  edge of the assigned category NB1 or NB2 channel bandwidth. For frequencies greater than (ΔfOOB) as specified in Table 6.6.2F.1-1 the spurious requirements in subclause 6.6.3 are applicable.

The power of any category NB1 or NB2 UE emission shall not exceed the levels specified in Table 6.6.2F.1-1. The spectrum emission limit between each ΔfOOB is linearly interpolated.

Table 6.6.2F.1-1: category NB1 and NB2 UE spectrum emission mask

| ΔfOOB (kHz) | Emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 0 | 26 | 30 kHz |
| 100 | -5 | 30 kHz |
| 150 | -8 | 30 kHz |
| 300 | -29 | 30 kHz |
| 500-1700 | -35 | 30 kHz |

In addition to the spectrum emission mask requirement in Table 6.6.2F.1-1 a category NB1 or NB2 UE shall also meet the applicable E-UTRA spectrum emission mask requirement in sub-clause 6.6.2. E-UTRA spectrum emission requirement applies for frequencies that are Foffset away from edge of NB1 or NB2 channel edge as defined in Table 6.6.2F.1-2.

Table 6.6.2F.1-2: Foffset for category NB1 and NB2 UE spectrum emission mask

| Channel BW (MHz) | Foffset [kHz] |
| --- | --- |
| 1.4 | 165 |
| 3 | 190 |
| 5 | 200 |
| 10 | 225 |
| 15 | 240 |
| 20 | 245 |

Note:	Foffset in Table 6.6.2F.1-2 is used to guarantee co-existence for guard-band operation.

#### 6.6.2F.2	Additional Spectrum Emission Mask for Category NB1 and NB2

This requirement is specified in terms of an "additional spectrum emission" requirement.

##### 6.6.2F.2.1	Minimum requirement (network signalled value "NS_02")

Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell broadcast message.

When "NS_02" is indicated in the cell, the NB-IoT channel is deployed in the lower guard-band of a 10MHz E-UTRA channel and the separation between the two channel centres is equal to 4.695 MHz. The power of any UE emission shall not exceed the levels specified in Table 6.6.2.1.1-1 for the specified E-UTRA channel bandwidth and the levels specified in Table 6.6.2F.1-1 for the NB-IoT channel.

Note:	UEs that meet the above emission requirement would automatically meet the E-UTRA additional spectrum emission masks as defined in 6.6.2.2 for the applicable operating bands.

##### 6.6.2F.2.2	Minimum requirement (network signalled value "NS_03")

Additional spectrum emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell broadcast message.

When "NS_03" is indicated in the cell, the NB-IoT channel is deployed in the upper guard-band of a 10MHz E-UTRA channel and the separation between the two channel centres is equal to 4.695 MHz. The power of any UE emission shall not exceed the levels specified in Table 6.6.2.1.1-1 for the specified E-UTRA channel bandwidth and the levels specified in Table 6.6.2F.1-1 for the NB-IoT channel.

Note:	UEs that meet the above emission requirement would automatically meet the E-UTRA additional spectrum emission masks as defined in 6.6.2.2 for the applicable operating bands.

#### 6.6.2F.3	Adjacent Channel Leakage Ratio for category NB1 and NB2

Adjacent Channel Leakage power Ratio is the ratio of the filtered mean power centred on the assigned channel frequency to the filtered mean power centred on an adjacent channel frequency. The assigned category NB1or NB2 channel power and adjacent channel power are measured with filters and measurement bandwidths specified in Table 6.6.2F.3-1. If the measured adjacent channel power is greater than –50dBm then the category NB1 or NB2 UE ACLR shall be higher than the value specified in Table 6.6.2F.3-1. GSMACLR requirement is intended for protection of GSM system. UTRAACLR requirement is intended for protection of UTRA and E-UTRA systems.

Table 6.6.2F.3-1: category NB1 and NB2 UE ACLR requirements

|  | GSMACLR | UTRAACLR |
| --- | --- | --- |
| ACLR | 20 dB | 37 dB |
| Adjacent channel center frequency offset from category NB1 or NB2 Channel edge | ±200 kHz | ±2.5 MHz |
| Adjacent channel measurement bandwidth | 180 kHz | 3.84 MHz |
| Measurement filter | Rectangular | RRC-filter α=0.22 |
| Category NB1 and NB2 channel measurement bandwidth | 180 kHz | 180 kHz |
| Category NB1 and NB2 channel Measurement filter | Rectangular | Rectangular |

### 6.6.2G		Out of band emission for V2X Communication

When UE is configured for E-UTRA V2X sidelink transmissions non-concurrent with E-UTRA uplink transmissions for E-UTRA V2X operating bands specified in Table 5.5G-1, the requirements in subclause 6.6.2 apply except for the ACLR requirements for power class 2 V2X UE.

When UE is configured for simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions for inter-band E-UTRA V2X / E-UTRA bands specified in Table 5.5G-2, the requirements in subclause 6.6.2 apply per V2X sidelink transmission and E-UTRA uplink transmission as specified for the corresponding inter-band con-current operation with uplink assigned to two bands.

For intra-band contiguous multi-carrier operation, the general CA spectrum emission mask for CA Bandwidth Class B specified in subclause 6.6.2.1A shall apply for V2X Bandwdith Class B, the general CA spectrum emission mask for CA Bandwidth Class C specified in subclause 6.6.2.1A shall apply for V2X Bandwdith Class C and C1.

For intra-band contiguous multi-carrier operation, the E-UTRA ACLR requirment for CA Bandwidth Class B specified in subclause 6.6.2.3.3A shall apply for V2X Bandwdith Class B, the general CA spectrum emission mask for CA Bandwidth Class C specified in subclause 6.6.2.3.3A shall apply for V2X Bandwdith Class C and C1.

For power class 2 V2X UE, the assigned channel power and adjacent channel power are measured with rectangular filters with measurement bandwidths specified in Table 6.6.2G-1. If the measured adjacent channel power is greater than –50dBm then ACLR shall be higher than the value specified in Table 6.6.2G-1.

Table 6.6.2G-1: ACLR requirements for power class 2 V2X Communication

|  | Channel bandwidth / ACLR / Measurement bandwidth |  |
| --- | --- | --- |
|  | 10 MHz | 20 MHz |
| ACLR | 31 dB | 31 dB |
| E-UTRA channel Measurement bandwidth | 9.0 MHz | 18 MHz |
| Adjacent channel centre frequency offset [MHz] | +10 / -10 | +20 / -20 |

For V2X UE supporting Transmit Diversity, if the UE transmits on two antenna connectors at the same time, the requirements specified for single carrier apply to each transmit antenna connector.

If V2X UE transmits on one antenna connector at a time, the requirements specified for single carrier shall  apply to the active antenna connector.

### 6.6.3	Spurious emissions

Spurious emissions are emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emissions, intermodulation products and frequency conversion products, but exclude out of band emissions unless otherwise stated. The spurious emission limits are specified in terms of general requirements inline with SM.329 [2] and E-UTRA operating band requirement to address UE co-existence.

To improve measurement accuracy, sensitivity and efficiency, the resolution bandwidth may be smaller than the measurement bandwidth. When the resolution bandwidth is smaller than the measurement bandwidth, the result should be integrated over the measurement bandwidth in order to obtain the equivalent noise bandwidth of the measurement bandwidth.

#### 6.6.3.1	Minimum requirements

Unless otherwise stated, the spurious emission limits apply for the frequency ranges that are more than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth. The spurious emission limits in Table 6.6.3.1-2 apply for all transmitter band configurations (NRB) and channel bandwidths.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

Table 6.6.3.1-1: Boundary between E-UTRA out of band and spurious emission domain

| Channel bandwidth | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| --- | --- | --- | --- | --- | --- | --- |
| OOB boundary FOOB (MHz) | 2.8 | 6 | 10 | 15 | 20 | 25 |

Table 6.6.3.1-2: Spurious emissions limits

| Frequency Range | Maximum Level | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 9 kHz f < 150 kHz | -36 dBm | 1 kHz |  |
| 150 kHz f < 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1 GHz f < 12.75 GHz | -30 dBm | 1 MHz |  |
| 12.75 GHz ≤ f < 5th harmonic of the upper frequency edge of the UL operating band in GHz | -30 dBm | 1 MHz | 1 |
| 12.75 GHz < f < 26GHz | -30dBm | 1MHz | 2 |
| NOTE 1: Applies for Band 22, Band 42, Band 43 and Band 48. NOTE 2: Applies for Band 46 and Band 47. |  |  |  |

#### 6.6.3.1A	Minimum requirements for CA

This clause specifies the spurious emission requirements for carrier aggregation.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

For inter-band carrier aggregation with one component carrier per operating band and the uplink active in two E-UTRA bands, the spurious emission requirement Table 6.6.3.1-2 apply for the frequency ranges that are more than FOOB as defined in Table 6.6.3.1-1 away from edges of the assigned channel bandwidth on a component carrier. If for some frequency a spurious emission requirement of individual component carrier overlaps with the spectrum emission mask or channel bandwidth of another component carrier then it does not apply.

NOTE:	For inter-band carrier aggregation with uplink assigned to two E-UTRA bands the requirements in Table 6.6.3.1-2 could be verified by measuring spurious emissions at the specific frequencies where second and third order intermodulation products generated by the two transmitted carriers can occur; in that case, the requirements for remaining applicable frequencies in Table 6.6.3.1-2 would be considered to be verified by the measurements verifying the one uplink inter-band CA spurious emission requirement.

For intra-band contiguous carrier aggregation the spurious emission limits apply for the frequency ranges that are more than FOOB (MHz) in Table 6.6.3.1A-1 from the edge of the aggregated channel bandwidth (Table 5.6A-1). For frequencies ΔfOOB greater than FOOB as specified in Table 6.6.3.1A-1the spurious emission requirements in Table 6.6.3.1-2 are applicable.

Table 6.6.3.1A-1: Boundary between E-UTRA out of band and spurious emission domain for intra-band contiguous carrier aggregation

| CA Bandwidth Class | OOB boundary FOOB (MHz) |
| --- | --- |
| A | Table 6.6.3.1-1 |
| B | BWChannel_CA + 5 |
| C | BWChannel_CA + 5 |

For intra-band non-contiguous carrier aggregation transmission the spurious emission requirement is defined as a composite spurious emission requirement. Composite spurious emission requirement applies to frequency ranges that are more than FOOB away from the edges of the sub-blocks. Composite spurious emission requirement is defined as follows

a)	Composite spurious emission requirement is a combination of individual sub-block spurious emission requirements

b)	In case the sub-block consist of one component carrier the sub-lock spurious emission requirement and FOOB are defined in subclause 6.6.3.1

c)	If for some frequency an individual sub-block spurious emission requirement overlaps with the general spectrum emission mask or the sub-block bandwidth of another sub-block then it does not apply

For combinations of intra-band and inter-band carrier aggregation with three uplink component carriers (up to two contiguously aggregated carriers per band), the spurious emission requirememnt is defined as follows. For the E-UTRA band supporting one component carrier the requirements in Table 6.6.3.1-2 apply for frequency ranges that are more than FOOB (MHz) from the edges of assigned channel bandwidth as defined in Table 6.6.3.1-1. For the E-UTRA band supporting two contiguous component carriers the requirements in Table 6.6.3.1-2 apply for frequency ranges that are more than FOOB (MHz) from the edges of assigned aggregated channel bandwidth as defined in Table 6.6.3.1A-1. If for some frequency a spurious emission requirement of a single component carrier or two contiguous component carriers overlap with the spurious emission requirement or channel bandwidth of another component carrier or two contiguously aggregated carriers then it does not apply.

#### 6.6.3.2	Spurious emission band UE co-existence

This clause specifies the requirements for the specified E-UTRA band, for coexistence with protected bands.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

Table 6.6.3.2-1: Requirements

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| 1 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 38, 40, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76, 87, 88 NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 34 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1880 |  | 1895 | -40 | 1 | 15, 27 |
|  | Frequency range | 1895 |  | 1915 | -15.5 | 5 | 15, 26, 27 |
|  | Frequency range | 1915 |  | 1920 | +1.6 | 5 | 15, 26, 27, 44 |
| 2 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 42, 50, 51, 53, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 43, 48 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 3 | E-UTRA Band 1, 5, 7, 8, 11, 18, 19, 20, 21, 26, 27, 28, 31, 32, 33, 34, 38, 39, 40, 41, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73,74, 75, 76, 87, 88 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 22, 42 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 |  |
| 4 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 41, 43, 48, 50, 51, 53, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 5 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 50, 51, 65, 66, 70, 71, 73, 74, 85 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
|  | E-UTRA Band 41, 52, 53 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 18, 19 | FDL_low | - | FDL_high | -40 | 1 | 39 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 39 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 39 |
| 6 | E-UTRA Band 1, 9, 11, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 860 | - | 875 | -37 | 1 |  |
|  | Frequency range | 875 | - | 895 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1919.6 | -41 | 0.3 | 7 |
|  |  | 1884.5 | - | 1915.7 |  |  | 8 |
| 7 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 20, 22, 26, 27, 28, 29, 30, 31, 32, 33, 34, 40, 42, 43, 50, 51, 52, 65, 66, 67, 68, 72, 74, 75, 76, 85, 87, 88 NR Band n77,n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 15, 21, 26 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 15, 21, 26 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 15, 21 |
| 8 | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3, 7, 22, 41, 42, 43 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 23 |
|  | Frequency range | 860 | - | 890 | -40 | 1 | 15, 23 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 23 |
| 9 | E-UTRA Band 1, 3, 11, 18, 19, 21, 26, 28, 34 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| 10 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 41, 43, 66, 70, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 22, 42, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 11 | E-UTRA Band 1, 3, 11, 18, 19, 21, 28, 34, 40, 42, 65 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| 12 | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 53, 70, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 48, 50, 51, 66 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| 13 | E-UTRA Band 2, 4, 5, 12, 13, 17, 25, 26, 27, 29, 41, 48, 50, 51, 53, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 14 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 24, 30, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 15 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 15 |
| 14 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 23, 24, 25, 26, 27, 29, 30, 41, 48, 53, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 769 | - | 775 | -35 | 0.00625 | 12, 15 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 | 12, 15 |
| 17 | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 70, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 48, 50, 51, 53, 66 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| 18 | E-UTRA Band 1, 3, 11, 21, 34, 40, 42, 65 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 15 |
|  | Frequency range | 860 | - | 890 | -40 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| 19 | E-UTRA Band 1, 3, 11, 21, 28, 34, 40, 42, 65 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| 20 | E-UTRA Band 1, 3, 7, 8, 22, 31, 32, 33, 34, 40, 43, 50, 51, 65, 67, 68, 72, 74, 75, 76, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 38, 42, 52, 69 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 788 | -50 | 1 |  |
| 21 | E-UTRA Band 1, 3, 18, 19, 28, 34, 40, 42, 65 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 2545 | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 | - | 2645 | -50 | 1 |  |
| 22 | E-UTRA Band 1, 3, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 38, 39, 40, 43, 65, 67, 68, 69, 72, 75, 76, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 3510 | - | 3525 | -40 | 1 | 15 |
|  | Frequency range | 3525 | - | 3590 | -50 | 1 |  |
| 23 | E-UTRA Band 4, 5, 12, 13, 14, 17, 23, 24, 26, 27, 29, 30, 41, 66 | FDL_low | - | FDL_high | -50 | 1 |  |
| 24 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 48, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 25 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 27, 28, 29, 30, 41, 42, 53, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 25 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | E-UTRA Band 43, 48 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 26 | E-UTRA Band 1, 2, 3, 4, 5, 11, 12, 13, 14, 17, 18,19, 21, 24, 25, 26, 29, 30, 31, 34, 39, 40, 42, 43, 48, 50, 51, 65, 66, 70, 71, 73,74, 85 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41, 53 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 703 | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 | - | 803 | -40 | 1 | 15 |
|  | Frequency range | 945 | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| 27 | E-UTRA Band 1, 2, 3, 4, 5, 7, 12, 13, 14, 17, 25, 26, 27, 29, 30, 31, 38, 40, 41, 42, 43, 65, 66, 73, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 28 | FDL_low | - | 790 | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 799 | - | 805 | -35 | 0.00625 |  |
| 28 | E-UTRA Band 1, 4, 22, 32, 42, 43, 50, 51, 65, 66, 73, 74, 75, 76 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 19, 25 |
|  | E-UTRA Band 2, 3, 5, 7, 8, 18, 19, 20, 25, 26, 27, 31, 34, 38, 40, 41, 52, 72, 87, 88 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 11, 21 | FDL_low | - | FDL_high | -50 | 1 | 19, 24 |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 15, 35 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 34 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 15 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 15 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8, 19 |
| 30 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 29, 30, 38, 41, 48, 53, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 |  |
| 31 | E-UTRA Band 1, 5, 7, 8, 20, 22, 26, 27, 28, 31, 32, 33, 34, 38, 40, 42, 43, 50, 51, 52, 65, 67, 68, 69, 74, 75, 76, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 470 | - | 694 | -42 | 8 |  |
| … |  |  |  |  |  |  |  |
| 33 | E-UTRA Band 1, 7, 8, 20, 22, 28, 31, 32, 34, 38, 40, 42, 43, 52, 65, 67, 69, 72, 73, 75, 76, 87, 88 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | E-UTRA Band 43 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| 34 | E-UTRA Band 1, 3, 7, 8, 11, 18, 19, 20, 21, 22, 26, 28, 31, 32, 33, 38,39, 40, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 69, 72, 73, 74, 75, 76, 87, 88 NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 5 |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2, 5 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| 35 |  |  |  |  |  |  |  |
| 36 |  |  |  |  |  |  |  |
| 37 |  |  | - |  |  |  |  |
| 38 | E-UTRA Band 1, 2, 3, 4, 5, 8, 12, 13, 14, 17, 20, 22, 27, 28, 29, 30, 31, 32, 33, 34, 40, 42, 43, 50, 51, 52, 65, 66, 67, 68, 72, 74, 75, 76, 85, 87, 88 NR Band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2620 | - | 2645 | -15.5 | 5 | 15, 22, 26 |
|  | Frequency range | 2645 | - | 2690 | -40 | 1 | 15, 22 |
| 39 | E-UTRA Band 1, 8, 22, 26, 28, 34, 40, 41, 42, 44, 45, 50, 51, 52, 73, 74 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1805 |  | 1855 | -40 | 1 | 33 |
|  | Frequency range | 1855 |  | 1880 | -15.5 | 5 | 15,26,33 |
| 40 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 33, 34, 38, 39, 41, 42, 43, 44, 45, 50, 51, 52, 65, 67, 68, 69, 72, 73, 74, 75, 76, 87, 88 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| 41 | E-UTRA Band 1, 2, 3, 4, 5, 8, 12, 13 , 14, 17, 24, 25, 26, 27, 28, 29, 30, 34, 39, 40, 42, 44, 45, 48, 50, 51, 52, 65, 66, 70, 71, 73, 74, 85 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 9, 11, 18, 19, 21 | FDL_low | - | FDL_high | -50 | 1 | 30 |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 |  | 1915.7 | -41 | 0.3 | 8, 30 |
| 42 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 11, 18, 19, 20, 21, 25, 26, 27, 28, 31, 32, 33, 34, 38, 40, 41, 44, 45, 50, 51, 65, 66, 67, 68, 69, 72, 73, 74, 75, 76, 87, 88 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
| 43 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 20, 25, 26, 27, 28, 31,32, 33, 34, 38, 40, 50, 51, 65, 66, 67, 68, 69, 72, 73, 74, 75, 76, 85, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
| 44 | E-UTRA Band 1, 40, 42, 45 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 3, 5, 8, 34, 39, 41, 73 | FDL_low | - | FDL_high | -50 | 1 |  |
| 45 | E-UTRA Band 1, 3, 5, 8, 34, 39, 40, 41, 42, 44, 52, 73 | FDL_low | - | FDL_high | -50 | 1 |  |
| … |  |  |  |  |  |  |  |
| 47 | E-UTRA Band 1, 3, 5, 7, 8, 22, 26, 28, 34, 39, 40, 41, 42, 44, 45, 65, 68, 72, 73 NR band n77, n78 , n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 5925 | - | 5950 | -30 EIRP | 1 | 38, 40, 43 |
|  | Frequency range | 5815 | - | 5855 | -30 EIRP | 1 | 38, 43, 45 |
| 48 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
| 50 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 17, 20, 26, 28, 29, 31, 34, 38, 39, 40, 41, 42, 43, 48, 52, 65, 66, 67, 68, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
| 51 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 17, 20, 26, 28, 29, 31, 34, 38, 39, 40, 41, 42, 43, 48, 52, 65, 66, 67, 68, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
| 52 | E-UTRA Band 1, 3, 5, 7, 8, 20, 28, 31, 33, 34, 38, 39, 40, 41, 45, 47, 50, 51, 68, 72, 73, 74, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
| 53 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 48, 66, 70, 71, 85, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 |  |
| 65 | E-UTRA Band 1, 3, 7, 8, 20, 22, 28, 31, 32, 38, 40, 42, 43, 50, 51, 65, 68, 69, 72, 74, 75, 76, 87, 88 NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 5, 11, 18, 19, 21, 26, 27, 41 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 34 | FDL_low | - | FDL_high | -50 | 1 | 36 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 37 |
|  | Frequency range | 1900 | - | 1915 | -15.5 | 5 | 15, 26, 27 |
|  | Frequency range | 1915 | - | 1920 | +1.6 | 5 | 15, 26, 27,44 |
| 66 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 38, 41, 43, 50, 51, 53, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42, 48, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 68 | E-UTRA Band 3, 7, 8, 20, 28, 31, 38, 40, 47, 72, 74, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 1, 22, 42, 43, 50, 51, 52, 65 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| … |  |  |  |  |  |  |  |
| 70 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 48, 53, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 71 | E-UTRA Band 4, 5, 12, 13, 14, 17, 24, 26, 30, 48, 53, 66, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 2, 25, 41, 70, NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 29 | FDL_low | - | FDL_high | -38 | 1 | 15 |
|  | E-UTRA Band 71 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| 72 | E-UTRA Band 1, 7, 20, 22, 28, 31, 32, 33, 34, 38, 42, 43, 47, 52, 65, 68, 72, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 8, 40 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 470 | - | 694 | -42 | 8 |  |
| 73 | E-UTRA Band 1, 26, 28, 33, 34, 39, 41, 42, 43, 44, 45, 47, 52 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3, 5, 8, 27, 40 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| 74 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 17, 18, 19, 20, 26, 28, 29, 31, 34, 38, 39, 40, 41, 42, 43, 48, 52, 65, 66, 67, 68, 85 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 8 |
|  | Frequency range | 1400 | - | 1427 | -32 | 27 | 15, 41 |
|  | Frequency range | 1475 | - | 1488 | -50 | 1 | 42 |
|  | Frequency range | 1488 | - | 1518 | -50 | 1 | 15 |
| 85 | E-UTRA Band 2, 5, 13, 14, 17, 24, 25, 26, 27, 30, 41, 53, 70, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 4, 48, 51, 66 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 12, 85 | FDL_low | - | FDL_high | -50 | 1 | 15 |
| 87 | E-UTRA Band 1, 3, 7, 8, 22, 28, 31, 32, 33, 34, 38, 40, 42, 43, 47, 52, 65, 68, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band, 20 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 87, 88 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | Frequency range | 470 | - | 694 | -42 | 8 |  |
| 88 | E-UTRA Band 1, 3, 7, 8, 20, 22, 28, 31, 32, 33, 34, 38, 40, 42, 43, 47, 52, 65, 68, 72 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 87 | FDL_low | - | FDL_high | -20 | 1 | 15 |
|  | E-UTRA Band 88 | FDL_low | - | FDL_high | -50 | 1 | 15 |
|  | Frequency range | 470 | - | 694 | -42 | 8 |  |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: N/A NOTE 4: N/A NOTE 5: For non synchronised TDD operation to meet these requirements some restriction will be needed for either the operating band or protected band NOTE 6: N/A NOTE 7: Applicable when co-existence with PHS system operating in 1884.5 -1919.6MHz. NOTE 8: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. NOTE 9: N/A NOTE 10: N/A NOTE 11: Whether the applicable frequency range should be 793-805MHz instead of 799-805MHz is TBD NOTE 12: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB NOTE 13: N/A NOTE 14: N/A NOTE 15: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 16: N/A NOTE 17: N/A NOTE 18: N/A NOTE 19: Applicable when the assigned E-UTRA carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 20: N/A NOTE 21: This requirement is applicable for any channel bandwidths within the range 2500 - 2570 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2560.5 - 2562.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2552 - 2560 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 22: This requirement is applicable for power class 3 UE for any channel bandwidths within the range 2570 - 2615 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 2605.5 - 2607.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 2597 - 2605 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. For power class 2 UE for any channel bandwidths within the range 2570 - 2615 MHz, NS_44 shall apply. For power class 2 or 3 UE for carriers with channel bandwidth overlapping the frequency range 2615 - 2620 MHz the requirement applies with the maximum output power configured to +19 dBm in the IE P-Max. NOTE 23: This requirement is applicable only for the following cases: - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth. - for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. NOTE 24: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 25: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 26: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 27: This requirement is applicable for any channel bandwidths within the range 1920 - 1980 MHz with the following restriction: for carriers of 15 MHz bandwidth when carrier centre frequency is within the range 1927.5 - 1929.5 MHz and for carriers of 20 MHz bandwidth when carrier centre frequency is within the range 1930 - 1938 MHz the requirement is applicable only for an uplink transmission bandwidth less than or equal to 54 RB. NOTE 28: N/A NOTE 29: N/A NOTE 30: This requirement applies when the E-UTRA carrier is confined within 2545-2575MHz or 2595-2645MHz and the channel bandwidth is 10 or 20 MHz NOTE 31: N/A NOTE 32: Void NOTE 33: This requirement is only applicable for carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for carriers of 15 MHz bandwidth when carrier center frequency is within the range 1892.5 - 1894.5 MHz and for carriers of 20 MHz bandwidth when carrier center frequency is within the range 1895 - 1903 MHz. NOTE 34: This requirement is applicable for 5 and 10 MHz E-UTRA channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart<48. NOTE 35: This requirement is applicable in the case of a 10 MHz E-UTRA carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 36: This requirement is applicable for E-UTRA channel bandwidth allocated within 1920-1980 MHz. NOTE 37: Applicable when the upper edge of the channel bandwidth frequency is greater than 1980MHz. NOTE 38: Applicable when NS_33 or NS_34 is configured by the pre-configured radio parameters. NOTE 39: Applicable only when the assigned E-UTRA carrier is confined within 824 MHz and 849 MHz for UE category M1, M2 and UE category NB1 and NB2. NOTE 40: In the frequency range x-5950MHz, SE requirement of -30dBm/MHz should be applied; where x = max (5925, fc + 15), where fc is the channel centre frequency. NOTE 41: Applicable for all bandwidths, and when the lower edge of the assigned E-UTRA UL channel bandwidth frequency is greater than or equal to 1427 MHz + the channel BW assigned for 1.4, 3, 5 and 10 MHz bandwidth, and when the lower edge of the assigned E-UTRA UL channel bandwidth frequency is greater than or equal to 1440 MHz for 15 and 20 MHz bandwidth. This requirement shall be verified with UE transmission power configured as high as possible but no higher than 15 dBm. NOTE 42: Applicable for 1.4 , 3 and 5 MHz bandwidth, and when the upper edge of the assigned E-UTRA UL channel bandwidth frequency is less than or equal to 1467 MHz assigned for10 MHz bandwidth, and when the upper edge of the assigned E-UTRA UL channel bandwidth frequency is less than or equal to 1463.8 MHz for 15 MHz bandwidth, and when the upper edge of the assigned E-UTRA UL channel bandwidth frequency is less than or equal to 1460.8 MHz for 20 MHz bandwidth. NOTE 43: The EIRP requirement is converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex I. NOTE 44: For category NB1 and NB2 UE when carrier centre frequency is 1920.1 MHz, in case of single-tone uplink transmission the requirement is applicable only for sub-carrier index > 2. NOTE 45: Resolution BW is 10% of the measurement BW and the result should be integrated to achieve the measurement bandwidth. The sweep time shall be set at least as (sweep points)*(symbol length) to improve the measurement accuracy. |  |  |  |  |  |  |  |

NOTE:	The restriction on the maximum uplink transmission to 54 RB in Notes 21, 22, and 27 of Table 6.6.3.2-1 and the restriction on the single-tone uplink transmission to sub-carrier index > 2 in Note 44 of Table 6.6.3.2-1 are intended for conformance testing and may be applied to network operation to facilitate coexistence when the aggressor and victim bands are deployed in the same geographical area. The applicable spurious emission requirement of -15.5 dBm/5MHz is a least restrictive technical condition for FDD/TDD coexistence and may have to be revised in the future.

When "NS_33" or “NS 34” is configured from pre-configured radio parameters or the cell and the indication from upper layers has indicated that the UE is within the protection zone of CEN DSRC devices or HDR DSRC devices, the power of any V2X UE emission shall fulfil either one of the two set of conditions.

|  | Maximum Transmission Power (dBm EIRP) | Emission Limit in Frequency Range 5795-5815 (dBm/MHz EIRP) |
| --- | --- | --- |
| Condition 1 | 10 | -65 |
| Condition 2 | 10 | -45 |

#### 6.6.3.2A	Spurious emission band UE co-existence for CA

This clause specifies the requirements for inter-band carrier aggregation configurations with the uplink assigned to two E-UTRA bands for coexistence with protected bands. When both constituent bands have common coexistence band protection requirements as specified in clause 6.6.3.2, the requirements are also applied to the carrier aggregation configuration

As exceptions, the additional requirements in Table 6.6.3.2A-0 apply on each component carrier with all component carriers are active.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

Table 6.6.3.2A-0: Requirements for uplink inter-band carrier aggregation (two bands)

| E-UTRA CA Configuration | Spurious emission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_1-5 | Frequency range | 859 |  | - | 869 | -27 | 1 |  |
| CA_1-8 | Frequency range | 860 |  | - | 890 | -40 | 1 | 3, 11 |
| CA_1-11 | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_1-18 | Frequency range | 758 |  | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 |  | - | 803 | -40 | 1 | 3 |
|  | Frequency range | 860 |  | - | 890 | -40 | 1 |  |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_1-19 | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_1-20 | Frequency range | 758 |  | - | 788 | -50 | 1 |  |
| CA_1-21 | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_1-26 | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 703 |  | - | 799 | -50 | 1 |  |
|  |  | 799 |  | - | 803 | -40 | 1 | 3 |
| CA_1-28 | Frequency range | 470 |  | - | 694 | -42 | 8 | 3, 22 |
|  | Frequency range | 470 |  | - | 710 | -26.2 | 6 | 23 |
|  | Frequency range | 758 |  | - | 773 | -32 | 1 | 3 |
|  | Frequency range | 773 |  | - | 803 | -50 | 1 |  |
|  | Frequency range | 662 |  | - | 694 | -26.2 | 6 | 3 |
| CA_2-5 | Frequency range | 859 |  | - | 869 | -27 | 1 |  |
| CA_2-13 | Frequency range | 769 |  | - | 775 | -35 | 0.00625 | 3 |
|  | Frequency range | 799 |  | - | 805 | -35 | 0.00625 | 3 |
| CA_2-14 | Frequency range | 769 |  | - | 775 | -35 | 0.00625 | 3 |
|  | Frequency range | 799 |  | - | 805 | -35 | 0.00625 | 3 |
| CA_3-5 | Frequency range | 859 |  | - | 869 | -27 | 1 |  |
| CA_3-8 | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4, 11 |
|  | Frequency range | 860 |  | - | 890 | -40 | 1 | 3,11,17 |
| CA_3-11 | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_3-18 | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_3-19 | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 3, 4 |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_3-20 | Frequency range | 758 |  | - | 788 | -50 | 1 |  |
| CA_3-21 | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_3-26 | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
|  | Frequency range | 703 |  | - | 799 | -50 | 1 |  |
|  |  | 799 |  | - | 803 | -40 | 1 | 3 |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
| CA_3-28 | Frequency range | 470 |  | - | 710 | -26.2 | 6 | 23 |
|  | Frequency range | 758 |  | - | 773 | -32 | 1 | 3 |
|  | Frequency range | 773 |  | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4, 5 |
| CA_3-40 | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 |  |
| CA_3-41 | Frequency range | 1884.5 |  |  | 1915.7 | -41 | 0.3 | 4, 18 |
| CA_3-42 | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
| CA_4-5 | Frequency range | 859 |  | - | 869 | -27 | 1 |  |
| CA_4-13 | Frequency range | 769 |  | - | 775 | -35 | 0.00625 | 3 |
|  | Frequency range | 799 |  | - | 805 | -35 | 0.00625 | 3 |
| CA_4-28 | Frequency range | 470 |  | - | 710 | -26.2 | 6 | 23 |
|  | Frequency range | 758 |  |  | 773 | -32 | 1 | 3 |
|  | Frequency range | 773 |  |  | 803 | -50 | 1 |  |
| CA_5-7 | Frequency range | 859 |  | - | 869 | -27 | 1 |  |
| CA_5-12 | Frequency range | 859 |  | - | 869 | -27 | 1 |  |
| CA_5-17 | Frequency range | 859 |  | - | 869 | -27 | 1 |  |
| CA_5-40 | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
| CA_7-26 | Frequency range | 703 |  | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 |  | - | 803 | -40 | 1 | 3 |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 |  |
| CA_7-28 | Frequency range | 758 |  | - | 773 | -32 | 1 | 3 |
|  | Frequency range | 773 |  | - | 803 | -50 | 1 |  |
| CA_8-41 | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4, 11 |
| CA_11-18 | Frequency range | 860 |  | - | 890 | -40 | 1 | 3 |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
| CA_11-26 | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 | 2 |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
| CA_13-66 | Frequency range | 769 |  | - | 775 | -35 | 0.00625 | 3 |
|  | Frequency range | 799 |  | - | 805 | -35 | 0.00625 | 3 |
| CA_14-30 | Frequency range | 769 |  | - | 775 | -35 | 0.00625 | 3 |
|  | Frequency range | 799 |  | - | 805 | -35 | 0.00625 | 3 |
| CA_14-66 | Frequency range | 769 |  | - | 775 | -35 | 0.00625 | 3 |
|  | Frequency range | 799 |  | - | 805 | -35 | 0.00625 | 3 |
| CA_18-28 | Frequency range | 470 |  | - | 710 | -26.2 | 6 | 23 |
|  | Frequency range | 758 |  | - | 773 | -32 | 1 | 3 |
|  | Frequency range | 773 |  | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 |  | - | 803 | -40 | 1 | 3 |
|  | Frequency range | 860 |  | - | 890 | -40 | 1 |  |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 | 3 |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_19-21 | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_19-42 | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_21-28 | Frequency range | 470 |  | - | 710 | -26.2 | 6 | 23 |
|  | Frequency range | 773 |  | - | 803 | -50 | 1 |  |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4, 5 |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_21-42 | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4 |
|  | Frequency range | 2545 |  | - | 2575 | -50 | 1 |  |
|  | Frequency range | 2595 |  | - | 2645 | -50 | 1 |  |
| CA_26-46 | Frequency range | 703 |  | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 |  | - | 803 | -40 | 1 | 2 |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 3 |
| CA_26-48 | Frequency range | 703 |  | - | 799 | -50 | 1 |  |
|  | Frequency range | 799 |  | - | 803 | -40 | 1 | 2 |
|  | Frequency range | 945 |  | - | 960 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 3 |
| CA_28-41 | Frequency range | 470 | - |  | 694 | -42 | 8 | 3, 22 |
|  | Frequency range | 470 | - |  | 710 | -26.2 | 6 | 23 |
|  | Frequency range | 662 | - |  | 694 | -26.2 | 6 | 3 |
|  | Frequency range | 758 | - |  | 773 | -32 | 1 | 3 |
|  | Frequency range | 773 | - |  | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 | - |  | 1915.7 | -41 | 0.3 | 4, 5, 18 |
| CA_28-42 | Frequency range | 470 |  | - | 710 | -26.2 | 6 | 23 |
|  | Frequency range | 758 |  | - | 773 | -32 | 1 | 3 |
|  | Frequency range | 773 |  | - | 803 | -50 | 1 |  |
|  | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 4, 5 |
| CA_40-42 | Frequency range | 1884.5 |  | - | 1915.7 | -41 | 0.3 | 8 |
| CA_41-42 | Frequency range | 1884.5 |  |  | 1915.7 | -41 | 0.3 | 4, 18 |
| NOTE 1: Void NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. In case the exceptions are allowed due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3 or 4 for the 2nd, 3rd or 4th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the aggregated channel bandwidth. NOTE 4: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. NOTE 5: Applicable when the assigned E-UTRA carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 6: Void NOTE 7: Void NOTE 8: Void NOTE 9: Void NOTE10: Void NOTE 11: This requirement is applicable only for the following cases: - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 902.5 MHz ≤ Fc < 907.5 MHz with an uplink transmission bandwidth less than or equal to 20 RB - for carriers of 5 MHz channel bandwidth when carrier centre frequency (Fc) is within the range 907.5 MHz ≤ Fc ≤ 912.5 MHz without any restriction on uplink transmission bandwidth. - for carriers of 10 MHz channel bandwidth when carrier centre frequency (Fc) is Fc = 910 MHz with an uplink transmission bandwidth less than or equal to 32 RB with RBstart > 3. NOTE 12: Void NOTE13: Void NOTE 14: Void NOTE 15: Void NOTE 16: Void NOTE 17: This requirement is applicable only when Band 3 transmission frequency is less than or equal to 1765 MHz. NOTE 18: This requirement applies when the E-UTRA carrier is confined within 2545-2575MHz or 2595-2645MHz and the channel bandwidth is 10 or 20 MHz NOTE 19: Void NOTE 20: Void NOTE 21: Void NOTE 22: This requirement is applicable in the case of a 10 MHz E-UTRA carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 23: This requirement is applicable for 5 and 10 MHz E-UTRA channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart<48.NOTE 24: Void NOTE 25: Void |  |  |  |  |  |  |  |  |

Table 6.6.3.2A-1: Requirements for intraband carrier aggregation

| E-UTRA CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_1 | E-UTRA Band 1, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 38, 40, 41, 42, 43, 44, 50, 51, 52, 65, 67, 72, 73, 74, 75, 76 NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 10 |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_3 | E-UTRA Band 1, 7, 8, 20, 26, 27, 28, 31, 32, 33, 34, 38, 40, 41, 43, 44, 50, 51, 65, 67, 72, 73, 74, 75, 76 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 10 |
|  | E-UTRA Band 22, 42, 52 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_5 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 12, 13, 14, 17, 24, 25, 28, 29, 30, 31, 34, 38, 40, 42, 43, 45, 48, 65, 66, 70, 71, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 52, 53 NR Band n77, n78,n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_7 | E-UTRA Band 1, 3, 7, 8, 20, 22, 27, 28, 29, 30. 31, 32, 33, 34, 40, 42, 43, 50, 51, 52, 65, 67, 72, 74, 75, 76 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_8 | E-UTRA Band 1, 20, 28, 31, 32, 33, 34, 38, 39, 40, 50, 51, 72, 73, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA band 3 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA band 7 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 8 | FDL_low | - | FDL_high | -50 | 1 | 10 |
|  | E-UTRA Band 22, 41, 42, 43, 52 NR Band n77, n78,n79 | FDL_low |  | FDL_high | -50 | 1 | 2 |
| CA_38 | E-UTRA Band 1,3, 8, 20, 22, 27, 28, 29, 30, 31, 32, 33, 34, 40, 42, 43, 50, 51, 52, 65, 67, 72, 74, 75, 76 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_39 | E-UTRA Band 22, 34, 40, 41, 42, 44, 50, 51, 52, 73, 74 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_40 | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 27, 28, 31, 32, 33, 34, 38, 39, 41, 42, 43, 44, 50, 51, 52, 65, 67, 72, 73, 74, 75, 76 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 15 |
| CA_41 | E-UTRA Band 1, 2, 3, 4, 5, 8, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 34, 39, 40, 42, 44, 50, 51, 52, 65, 66, 70, 71, 73, 74, 85 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| CA_42 | E-UTRA Band 1, 2, 3, 4, 5, 7, 8, 11, 18, 19, 20, 21, 25, 26, 27, 28, 31, 32, 33, 34, 38, 40, 41, 44, 50, 51, 65, 66, 67, 72, 73, 74, 75, 76 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 |  |
| CA_48 | E-UTRA Band 2, 4, 5, 12, 13, 14, 17, 24, 25, 26, 29, 30, 41, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
| CA_66 | E-UTRA Band 2, 4, 5, 7, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 38, 41, 43, 50, 51, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 42, 48 NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3, 4, [5] for the 2nd, 3rd, 4th [or 5th] harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval NOTE 3: To meet these requirements some restriction will be needed for either the operating band or protected band NOTE 4: N/A NOTE 5: N/A NOTE 6: N/A NOTE 7: N/A NOTE 8: N/A NOTE 9: N/A NOTE 10: The requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the aggregated channel bandwidth. NOTE 11: N/A NOTE 12: N/A NOTE 13: N/A NOTE 14: N/A NOTE 15: Applicable when co-existence with PHS system operating in 1884.5 -1915.7MHz. |  |  |  |  |  |  |  |

Table 6.6.3.2A-2: Requirements for intraband non-contiguous CA

| E-UTRA CA Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| CA_4-4 | E-UTRA Band 2, 4, 5, 10, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 41, 43, 50, 51, 53, 66, 70, 71, 74, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 22, 42 NR Band n7 | FDL_low | - | FDL_high | -50 | 1 | 2 |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd or 3rd harmonic spurious emissions. Due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2 or 3 for the 2nd or 3rd harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. |  |  |  |  |  |  |  |

#### 6.6.3.3	Additional spurious emissions

These requirements are specified in terms of an additional spectrum emission requirement. Additional spurious emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell handover/broadcast message.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

##### 6.6.3.3.1 	Minimum requirement (network signalled value "NS_05")

When "NS_05" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3.3.1-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth.

Table 6.6.3.3.1-1: Additional requirements (PHS)

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) |  |  |  | Measurement bandwidth | NOTE |
| --- | --- | --- | --- | --- | --- | --- |
|  | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| 1884.5 f 1915.7 | -41 | -41 | -41 | -41 | 300 KHz | 1 |

Table 6.6.3.3.1-2: Void

##### 6.6.3.3.2 	Minimum requirement (network signalled value “NS_07”)

When "NS_07" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3.3.2-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth.

Table 6.6.3.3.2-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 10 MHz |  |
| 769 ≤ f ≤ 775 | -57 | 6.25 kHz |
| NOTE: The emissions measurement shall be sufficiently power averaged to ensure standard standard deviation < 0.5 dB. |  |  |

##### 6.6.3.3.3 	Minimum requirement (network signalled value “NS_08”)

When “NS 08” is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3.3.3-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth.

Table 6.6.3.3.3-1: Additional requirement

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- |
|  | 5MHz | 10MHz | 15MHz |  |
| 860 ≤ f ≤ 890 | -40 | -40 | -40 | 1 MHz |

##### 6.6.3.3.4 	Minimum requirement (network signalled value “NS_09”)

When “NS 09” is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3.3.4-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth.

Table 6.6.3.3.4-1: Additional requirement

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- |
|  | 5MHz | 10MHz | 15MHz |  |
| 1475.9 ≤ f ≤ 1510.9 | -35 | -35 | -35 | 1 MHz |

NOTE 1:	Void.

NOTE 2:	To improve measurement accuracy, A-MPR values for NS_09 specified in Table 6.2.4-1 in subclause 6.2.4 are derived based on 100 kHz RBW.

##### 6.6.3.3.5 	Minimum requirement (network signalled value "NS_12")

When “NS 12” is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3.3.5-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth.

Table 6.6.3.3.5-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 1.4 MHz, 3 MHz, 5 MHz, 10 MHz, 15 MHz |  |
| 806 ≤ f ≤ 813.5 | -42 | 6.25 kHz |
| NOTE 1: The requirement applies for E-UTRA carriers with lower channel edge at or above 814.2 MHz. NOTE 2: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB. |  |  |

##### 6.6.3.3.6 	Minimum requirement (network signalled value “NS_13”)

When “NS 13” is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3.3.6-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth.

Table 6.6.3.3.6-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 1.4, 3, 5 MHz |  |
| 806 ≤ f ≤ 816 | -42 | 6.25 kHz |
| NOTE 1: The requirement applies for E-UTRA carriers with lower channel edge at or above 819 MHz. NOTE 2: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB. |  |  |

##### 6.6.3.3.7 	Minimum requirement (network signalled value “NS_14”)

When “NS 14” is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3.3.7-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth.

Table 6.6.3.3.7-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 10 MHz, 15 MHz |  |
| 806 ≤ f ≤ 816 | -42 | 6.25 kHz |
| NOTE 1: The requirement applies for E-UTRA carriers with lower channel edge at or above 824 MHz. NOTE 2: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB. |  |  |

##### 6.6.3.3.8 	Minimum requirement (network signalled value “NS_15”)

When “NS 15” is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3.3.8-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth.

Table 6.6.3.3.8-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 1.4 MHz, 3 MHz, 5 MHz, 10 MHz, 15 MHz |  |
| 851 ≤ f ≤ 859 | -53 | 6.25 kHz |
| NOTE 1: The emissions measurement shall be sufficiently power averaged to ensure a standard deviation < 0.5 dB. |  |  |

##### 6.6.3.3.9 	Minimum requirement (network signalled value “NS_16”)

When “NS_16” is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3.3.9-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth.

Table 6.6.3.3.9-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 1.4, 3, 5, 10 MHz |  |  |
| 790 ≤ f ≤ 803 | -32 | 1 MHz |  |

##### 6.6.3.3.10 	Minimum requirement (network signalled value “NS_17”)

When “NS_17” is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3.3.10-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.3.1-1 from the edge of the channel bandwidth.

Table 6.6.3.3.10-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5, 10 MHz |  |  |
| 470 ≤ f ≤ 710 | -26.2 | 6 MHz | 1 |
| NOTE 1: Applicable when the assigned E-UTRA carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. |  |  |  |

##### 6.6.3.3.11 	Minimum requirement (network signalled value “NS_18”)

When “NS_18” is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3.3.11-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth.

Table 6.6.3.3.11-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5, 10, 15, 20 MHz |  |  |
| 692-698 | -26.2 | 6 MHz |  |

##### 6.6.3.3.12 	Minimum requirement (network signalled value “NS_19”)

When “NS_19” is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3.3.12-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth.

Table 6.6.3.3.12-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 3, 5, 10, 15, 20 MHz |  |  |
| 662 ≤ f ≤ 694 | -25 | 8 MHz |  |

##### 6.6.3.3.13 	Minimum requirement (network signalled value “NS_11”)

When “NS_11” is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3.3.13-1. These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth.

Table 6.6.3.3.13-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 1.4, 3, 5, 10, 15, 20 MHz |  |
| E-UTRA Band 2 | -50 | 1 MHz |
| 1998 ≤ f ≤ 1999 | -21 | 1 MHz |
| 1997 ≤ f < 1998 | -27 | 1 MHz |
| 1996 ≤ f < 1997 | -32 | 1 MHz |
| 1995 ≤ f < 1996 | -37 | 1 MHz |
| 1990 ≤ f < 1995 | -40 | 1 MHz |

##### 6.6.3.3.14	Minimum requirement (network signalled value “NS_20”)

When “NS_20” is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3.3.14-1. These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth.

Table 6.6.3.3.14-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5, 10, 15, 20 MHz |  |
| 1990 ≤ f < 1999 | -40 | 1 MHz |
| 1999 ≤ f ≤ 2000 | -40 | NOTE 1 |
| NOTE 1: The measurement bandwidth is 1% of the applicable E-UTRA channel bandwidth. |  |  |

##### 6.6.3.3.15	Minimum requirement (network signalled value “NS_21”)

When “NS_21” is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3.3.15-1. These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth.

Table 6.6.3.3.15-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5, 10 MHz |  |
| 2200 ≤ f < 2288 | -40 | 1 MHz |
| 2288 ≤ f < 2292 | -37 | 1 MHz |
| 2292 ≤ f < 2296 | -31 | 1 MHz |
| 2296 ≤ f < 2300 | -25 | 1 MHz |
| 2320 ≤ f < 2324 | -25 | 1 MHz |
| 2324 ≤ f < 2328 | -31 | 1 MHz |
| 2328 ≤ f < 2332 | -37 | 1 MHz |
| 2332 ≤ f ≤ 2395 | -40 | 1 MHz |

##### 6.6.3.3.16	Minimum requirement (network signalled value "NS_22")

When "NS 22" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3.3.16-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth.

Table 6.6.3.3.16-1: Additional requirement

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | MBW |
| --- | --- | --- |
|  | 5, 10, 15, 20 MHz |  |
| 3400 ≤ f ≤ 3800 | -23 (NOTE 1, NOTE 3) | 5 MHz |
|  | -40 (NOTE 2) | 1 MHz |
| NOTE 1: This requirement applies within an offset between 5 MHz and 25 MHz from the lower and from the upper edge of the channel bandwidth, whenever these frequencies overlap with the specified frequency band. NOTE 2: This requirement applies from 3400 MHz to 25 MHz below the lower E-UTRA channel edge and from 25 MHz above the upper E-UTRA channel edge to 3800 MHz. NOTE 3: This emission limit might imply risk of harmful interference to UE(s) operating in the protected operating band |  |  |

##### 6.6.3.3.17	Minimum requirement (network signalled value “NS_23”)

When "NS 23" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3.3.17-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth.

Table 6.6.3.3.17-1: Additional requirement

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | MBW |
| --- | --- | --- |
|  | 5, 10, 15, 20 MHz |  |
| 3400 ≤ f ≤ 3800 | -23 (NOTE 1, NOTE 4) | 5 MHz |
|  | -40 (NOTE 2) | 1 MHz |
| NOTE 1: This requirement applies within an offset between 5 MHz + Foffset_NS_23 and 25 MHz + Foffset_NS_23 from the lower and from the upper edges of the channel bandwidth, whenever these frequencies overlap with the specified frequency band. NOTE 2: This requirement applies from 3400 MHz to 25 MHz + Foffset_NS_23 below the lower E-UTRA channel edge and from 25 MHz + Foffset_NS_23 above the upper E-UTRA channel edge to 3800 MHz. NOTE 3: Foffset_NS_23 is: 0 MHz for 5 MHz channel BW, 5 MHz for 10 MHz channel BW, 9 MHz for 15 MHz channel BW and 12 MHz for 20 MHz channel BW. NOTE 4: This emission limit might imply risk of harmful interference to UE(s) operating in the protected operating band |  |  |

##### 6.6.3.3.18	Void

Table 6.6.3.3.18-1: Void

##### 6.6.3.3.19	Minimum requirement (network signalled value "NS_04")

When "NS 04" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3.3.19-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth.

Table 6.6.3.3.19-1: Additional requirements

| Frequency band | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5, 10, 15, 20 MHz |  |
| 2490.5 MHz ≤ f < 2495 MHz | -13 | 1 MHz |
| 9 kHz < f < 2490.5 MHz | -25 | 1 MHz |

##### 6.6.3.3.20	Minimum requirement (network signalled value “NS_24”)

When "NS_24" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3.3.20-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth.

Table 6.6.3.3.20-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz, 20 MHz |  |
| Band 34 | -50 | MHz |
| NOTE 1: This requirement applies at a frequency offset equal or larger than 5 MHz from the upper edge of the channel bandwidth, whenever these frequencies overlap with the specified frequency band. |  |  |

##### 6.6.3.3.21	Minimum requirement (network signalled value “NS_25”)

When "NS_25" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3.3.21-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth.

Table 6.6.3.3.21-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz, 20 MHz |  |
| Band 34 | -40 | MHz |
| NOTE 1: This requirement applies at a frequency offset equal or larger than 5 MHz from the upper edge of the channel bandwidth, whenever these frequencies overlap with the specified frequency band. |  |  |

##### 6.6.3.3.22	Minimum requirement (network signalled value “NS_26”)

When "NS_26" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3.3.22-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth.

Table 6.6.3.3.22-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5 MHz, 10 MHz, 15 MHz |  |
| 686 ≤ f ≤ 694 | -25 | 8MHz |

##### 6.6.3.3.23	Minimum requirement (network signalled value “NS_27” and “NS_43”)

When "NS_27" or “NS_43” is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3.3.23-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth.

Table 6.6.3.3.23-1: Additional requirements

| Frequency band | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5, 10, 15, 20 MHz |  |
| 9 kHz f 3530 MHz | -40 | 1 MHz |
| 3720 MHz f 5th harmonic of the upper frequency edge of the UL operating band |  |  |

##### 6.6.3.3.24	Minimum requirement (network signalled value “NS_28”)

When "NS_28" is indicated in the cell, the power of any UE emission for E-UTRA channels assigned within 5150-5350 MHz and 5470-5725 MHz shall not exceed the levels specified in Table 6.6.3.3.24-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth.

Table 6.6.3.3.24-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 20 MHz |  |
| 47 ≤ f ≤ 74 | -54 | 100 kHz |
| 87.5 ≤ f ≤ 118 | -54 | 100 kHz |
| 174 ≤ f ≤ 230 | -54 | 100 kHz |
| 470 ≤ f ≤ 862 | -54 | 100 kHz |
| 1000 ≤ f ≤ 5150 | -30 | 1 MHz |
| 5350 ≤ f ≤ 5470 | -30 | 1 MHz |
| 5725 ≤ f ≤ 26000 | -30 | 1 MHz |

##### 6.6.3.3.25	Minimum requirement (network signalled value “NS_29”)

When "NS_29" is indicated in the cell, the power of any UE emission for E-UTRA channels assigned within 5150-5350 and 5470-5725 MHz shall not exceed the levels specified in Table 6.6.3.3.25-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth.

Table 6.6.3.3.25-1: Additional requirements

| Centre Frequency Fc [MHz] | Protected range [MHz] | Frequency difference f between centre frequency – 5240 (for Fc=5180, 5200, 5220, 5240) 5260 (for Fc=5260, 5280, 5300, 5320) (MHz) | Minimum requirement [dBm] | Measurement bandwidth |
| --- | --- | --- | --- | --- |
| 5180, 5200, 5220, 5240 | 5135 ≤ f ≤ 5142 | - | -26 | 1 MHz |
|  | 5142 < f ≤ 5150 | - | -18 |  |
|  | 5250 ≤ f < 5251 | ≥ 10 and < 11 | 10(10 - f) |  |
|  | 5251 ≤ f < 5260 | ≥ 11 and < 20 | -10 – 8/9(f – 11) |  |
|  | 5260 ≤ f < 5266.7 | ≥ 20 and < 26.7 | -18 – 1.2(f – 20) |  |
|  | 5266.7 ≤ f ≤ 5365 | - | -26 |  |
| 5260, 5280, 5300, 5320 | 5135 ≤ f ≤ 5233.3 | - | -26 |  |
|  | 5233.3 < f ≤ 5240 | ≥ 20 and < 26.7 | -18 – 1.2(f – 20) |  |
|  | 5240 < f ≤ 5249 | ≥ 11 and < 20 | -10 – 8/9(f – 11) |  |
|  | 5249 < f ≤ 5250 | ≥ 10 and < 11 | 10(10 - f) |  |
|  | 5350 ≤ f ≤ 5365 | - | -26 |  |
| 5500, 5520, 5540, 5560, 5580, 5600, 5620, 5640, 5660, 5680, 5700 | 5455 ≤ f ≤ 5460 | - | -26 |  |
|  | 5460 < f ≤ 5470 | - | -19 |  |
|  | 5725 ≤ f < 5740 | - | -19 |  |
|  | 5740 ≤ f ≤ 5745 | - | -26 |  |

##### 6.6.3.3.26	Minimum requirement (network signalled value “NS_30”)

When "NS_30" is indicated in the cell, the power of any UE emission for E-UTRA channels assigned within 5150-5350 MHz, 5470-5725 MHz and 5725-5850 MHz shall not exceed the levels specified in Table 6.6.3.3.26-1, Table 6.6.3.3.26-2 and Table 6.6.3.3.26-3, respectively. These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth.

Table 6.6.3.3.26-1: Additional requirements for E-UTRA channels assigned within 5150-5350 MHz

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 20 MHz |  |
| 4500 ≤ f ≤ 5150 | -41 | 1 MHz |
| 5350 ≤ f ≤ 5460 | -41 |  |

Table 6.6.3.3.26-2: Additional requirements for E-UTRA channels assigned within 5470-5725 MHz

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 20 MHz |  |
| 4500 ≤ f ≤ 5150 | -41 | 1 MHz |
| 5350 ≤ f ≤ 5460 | -41 |  |
| 5460 < f ≤ 5470 | -27 |  |
| 5725 ≤ f | -27 |  |

Table 6.6.3.3.26-3: Additional requirements for E-UTRA channels assigned within 5725-5850 MHz

| Frequency offset of measurement filter -3dB point, f | Frequency offset of measurement filter centre frequency, f_offset | Minimum requirement [dBm] | Measurement bandwidth |
| --- | --- | --- | --- |
| 0 MHz f < 5 MHz | 0.5 MHz f_offset < 5.5 MHz | 27 – 2.28(f_offset/MHz – 0.5) | 1 MHz |
| 5 MHz f < 25 MHz | 5.5 MHz f_offset < 25.5 MHz | 15.6 – 0.28(f_offset/MHz – 0.5) | 1 MHz |
| 25 MHz f < 75 MHz | 25.5 MHz f_offset < 75.5 MHz | 10 – 0.74(f_offset/MHz – 0.5) | 1 MHz |
| 75 MHz f | 75.5 MHz f_offset | -27 | 1 MHz |
| NOTE 1: The frequency offset f_offset is below and above the range 5725-5850 MHz; the measurement filter -3dB point is that closest to the range 5725-5850 MHz NOTE 2: The requirement applies when the offset of the measurement filter centre frequency is such that both -3 dB points of the measurement filter are confined within the frequency range 5725-5850 MHz. |  |  |  |

##### 6.6.3.3.27	Minimum requirement (network signalled value “NS_31”)

When "NS_31" is indicated in the cell, the power of any UE emission for E-UTRA channels assigned within 5150-5250 MHz, 5250-5350 MHz, 5470-5725 MHz and 5725-5850 MHz shall not exceed the levels specified in Table 6.6.3.3.27-1, Table 6.6.3.3.27-2, Table 6.6.3.3.27-3 and Table 6.6.3.3.27-4, respectively. These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth.

Table 6.6.3.3.27-1: Additional requirements for E-UTRA channels assigned within 5150-5250 MHz

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 20 MHz |  |
| f ≤ 5150 | -27 | 1 MHz |
| f ≥ 5250 | -27 |  |

Table 6.6.3.3.27-2: Additional requirements for E-UTRA channels assigned within 5250-5350 MHz

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 20 MHz |  |
| f ≤ 5250 | -27 | 1 MHz |
| f ≥ 5350 | -27 |  |

Table 6.6.3.3.27-3: Additional requirements for E-UTRA channels assigned within 5470-5725 MHz

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 20 MHz |  |
| f ≤ 5470 | -27 | 1 MHz |
| f ≥ 5725 | -27 |  |

Table 6.6.3.3.27-4: Additional requirements for E-UTRA channels assigned within 5725-5850 MHz

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 20 MHz |  |
| f ≤ 5725 | -27 | 1 MHz |
| f ≥ 5850 | -27 |  |

##### 6.6.3.3.28	Minimum requirement (network signalled value “NS_36”)

When "NS_36" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3.3.28-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth.

Table 6.6.3.3.28-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5 MHz, 10 MHz and 15 MHz |  |
| 470 ≤ f ≤ 694 | -42 | 8MHz |
| NOTE: For a 5 MHz E-UTRA carrier confined within 698MHz and 703MHz, this requirement shall be met in normal conditions only. The requirement is relaxed to -30dBm in extreme conditions. |  |  |

##### 6.6.3.3.29	Minimum requirement (network signalled value “NS_38”)

When "NS_38" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3.3.29-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth.

Table 6.6.3.3.29-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 1.4 MHz, 3 MHz, 5 MHz, 10 MHz, 15 MHz, 20 MHz |  |
| 1400 ≤ f ≤ 1427 | -32 | 27MHz |
| NOTE 1: This requirement shall be verified with UE transmission power configured as high as possible but no higher than 15 dBm. |  |  |

##### 6.6.3.3.30	Minimum requirement (network signalled value “NS_39”)

When "NS_39" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3.3.30-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth.

Table 6.6.3.3.30-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 1.4 MHz, 3 MHz, 5 MHz, 10 MHz, 15 MHz, 20 MHz |  |
| 1475 ≤ f ≤ 1488 | -28 | 1MHz |

##### 6.6.3.3.31	Minimum requirement (network signalled value “NS_40” and “NS_41”)

When "NS_40" or "NS_41" is indicated in the cell, the power of any UE emission for E-UTRA channels assigned within 1427-1432MHz (B51) and 1432-1452MHz (B50) shall not exceed the levels specified in Table 6.6.3.3.31-1. These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth.

Table 6.6.3.3.31-1: Additional requirements for E-UTRA channels assigned within 1427-1452MHz

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 3, 5, 10, 15, 20 MHz |  |
| 1400 ≤ f ≤ 1427 | -32 | 27 MHz |
| NOTE 1: This requirement shall be verified with UE transmission power configured as high as possible but no higher than 15 dBm. |  |  |

##### 6.6.3.3.32	Minimum requirement (network signalled value “NS_42”)

When "NS_42" is indicated in the cell, the power of any UE emission for E-UTRA channels assigned within 1492-1517 MHz (B50) shall not exceed the levels specified in Table 6.6.3.3.32-1. These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth.

Table 6.6.3.3.32-1: Additional requirements for E-UTRA channels assigned within 1492-1517 MHz

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 3, 5, 10, 15, 20 MHz |  |
| 1518 ≤ f ≤ 1559 | -30 | 1 MHz |

##### 6.6.3.3.33	Minimum requirement (network signalled value “NS_44”)

When "NS_44" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3.3.33-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth.

Table 6.6.3.3.33-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5, 10, 15, 20 |  |  |
| 2620-2645 | -15.5 | 5MHz | 1 |
| 2645-2690 | -40 | 1MHz | 1 |
| Note 1: The E-UTRA carrier with channel bandwidth is confined within 2570 MHz and 2615 MHz |  |  |  |

##### 6.6.3.3.34	Minimum requirement (network signalled value “NS_45”)

When "NS_45" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Tables 6.6.3.3.35-1 and 6.6.3.3.35-2. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth.

Table 6.6.3.3.34-1: Additional requirements for 1.4, 3 and 5 MHz channel bandwidths

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 1.4, 3, 5 |  |  |
| 0.009 < f ≤ 2477.5 | -25 | 1 MHz |  |
| 2477.5 < f ≤ 2478.5 | -13 | 1 MHz |  |
| 2478.5< f ≤ 2483.5 | -10 | 1 MHz |  |
| 2495 ≤ f < 2496 | -13 | 1% of Channel Bandwidth |  |
| 2496< f ≤ 2501 | -13 | 1 MHz |  |
| 2501 ≤ f ≤ 5th harmonic of the upper frequency edge of the UL operating band | -25 | 1 MHz |  |

Table 6.6.3.3.34-2: Additional requirements for 10 MHz channel bandwidth

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 10 |  |  |
| 0.009 < f ≤ 2473.5 | -25 | 1 MHz |  |
| 2473.5 < f ≤ 2478.5 | -13 | 1 MHz |  |
| 2478.5< f ≤ 2483.5 | -10 | 1 MHz |  |
| 2495 ≤ f < 2496 | -13 | 1% of Channel Bandwidth |  |
| 2496< f ≤ 2505 | -13 | 1 MHz |  |
| 2505 ≤ f ≤ 5th harmonic of the upper frequency edge of the UL operating band | -25 | 1 MHz |  |

##### 6.6.3.3.35	Minimum requirement (network signalled value “NS_56”)

When "NS_56" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3.3.35-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 from the edge of the channel bandwidth.

Table 6.6.3.3.35-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit1 (dBm) | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
|  | 5 MHz, 10MHz |  |  |
| 1541 ≤ f ≤ 1559 | -102 | 2kHz | Averaged over any 2 millisecond active transmission interval |
| 1559≤ f ≤ 1608 | -85 | 700Hz |  |
| 1608≤ f ≤ 1610 | -85 +5/2 (f-1608) | 700Hz |  |
| 1610≤ f ≤ 1625 | -80+ 66/15 (f-1610) | 700Hz |  |
| 1541 ≤ f ≤ 1608 | -75 | 1MHz | Averaged over any 2 millisecond active transmission interval |
| 1608≤ f ≤ 1610 | -75 + 5/2 (f-1608) | 1MHz |  |
| 1610≤ f ≤ 1627.5 | -70+ 57/17.5 (f-1610) | 1MHz |  |
| 1627.5 | -37 | 4kHz |  |
| 1638.5 ≤f ≤ 1645.5 | -28 | 4kHz |  |
| 1657.5 ≤f ≤ 1660.5 | -28 | 4kHz |  |
| NOTE 1: The EIRP requirement in regulation is converted to conducted requirement using a 0 dBi antenna. |  |  |  |

#### 6.6.3.3A	Additional spurious emissions for CA

These requirements are specified in terms of an additional spectrum emission requirement. Additional spurious emission requirements are signalled by the network to indicate that the UE shall meet an additional requirement for a specific deployment scenario as part of the cell reconfiguration message.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

##### 6.6.3.3A.1	Minimum requirement for CA_1C (network signalled value "CA_NS_01")

When "CA_NS_01" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3.3A.1-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1A-1 from the edge of the aggregated channel bandwidth.

Table 6.6.3.3A.1-1: Additional requirements (PHS)

| Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA band 34 | FDL_low | - | FDL_high | -50 | 1 |  |
| Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 1 |
| NOTE 1: Applicable when the aggregated channel bandwidth is confined within frequency range 1940 – 1980 MHz |  |  |  |  |  |  |

##### 6.6.3.3A.2 	Minimum requirement for CA_1C (network signalled value "CA_NS_02")

When "CA_NS_02" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3.3A.2-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1A-1 from the edge of the aggregated channel bandwidth.

Table 6.6.3.3A.2-1: Additional requirements

| Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA band 34 | FDL_low | - | FDL_high | -50 | 1 |  |
| Frequency range | 1900 | - | 1915 | -15.5 | 5 | 1, 2 |
| Frequency range | 1915 | - | 1920 | +1.6 | 5 | 1, 2 |
| NOTE 1: The requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 2: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |  |  |  |  |

##### 6.6.3.3A.3	Minimum requirement for CA_1C (network signalled value "CA_NS_03")

When "CA_NS_03" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3.3A.3-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1A-1 from the edge of the aggregated channel bandwidth.

Table 6.6.3.3A.3-1: Additional requirements

| Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA band 34 | FDL_low | - | FDL_high | -50 | 1 |  |
| Frequency range | 1880 | - | 1895 | -40 | 1 |  |
| Frequency range | 1895 | - | 1915 | -15.5 | 5 | 1, 2 |
| Frequency range | 1915 | - | 1920 | +1.6 | 5 | 1, 2 |
| NOTE 1: The requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 2: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |  |  |  |  |

##### 6.6.3.3A.4	Minimum requirement for CA_38C (network signalled value "CA_NS_05")

When "CA_NS_05" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3.3A.4-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1A-1 from the edge of the aggregated channel bandwidth. This requirement is applicable for carriers with aggregated channel bandwidths confined in 2570 - 2615 MHz.

Table 6.6.3.3A.4-1: Additional requirements

| Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency range | 2620 | - | 2645 | -15.5 | 5 | 1, 2, 3 |
| Frequency range | 2645 | - | 2690 | -40 | 1 | 1, 3 |
| NOTE 1: The requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 2: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 3: This requirement is applicable for carriers with aggregated channel bandwidths confined in 2570-2615 MHz. |  |  |  |  |  |  |

##### 6.6.3.3A.5	Minimum requirement for CA_7C (network signalled value "CA_NS_06")

When "CA_NS_06" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3.3A.5-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1A-1 from the edge of the aggregated channel bandwidth.

Table 6.6.3.3A.5-1: Additional requirements

| Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency range | 2570 | - | 2575 | +1.6 | 5 | 1, 2 |
| Frequency range | 2575 | - | 2595 | -15.5 | 5 | 1, 2 |
| Frequency range | 2595 | - | 2620 | -40 | 1 |  |
| NOTE 1: The requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 2: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |  |  |  |  |

##### 6.6.3.3A.6	Minimum requirement for CA_39C and CA_39C-41A (network signalled value "CA_NS_07")

When "CA_NS_07" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3.3A.6-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1A-1 from the edge of the aggregated channel bandwidth.

Table 6.6.3.3A.6-1: Additional requirements

| Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| --- | --- | --- | --- | --- | --- | --- |
| Frequency range | 1805 | - | 1855 | -40 | 1 | 1 |
| Frequency range | 1855 | - | 1880 | -15.5 | 5 | 1, 2, 3 |
| NOTE 1: This requirement is applicable for carriers with aggregated channel bandwidths confined in 1885-1920 MHz. NOTE 2: The requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the channel bandwidth. NOTE 3: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |  |  |  |  |

##### 6.6.3.3A.7	Minimum requirement for CA_42C (network signalled value "CA_NS_08")

When "CA_NS_08" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3.3A.7-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1A-1 from the edge of the aggregated channel bandwidth.

Table 6.6.3.3A.7-1: Additional requirements

| Frequency band (MHz) | Aggregated bandwidth / Spectrum emission limit (dBm) | MBW |
| --- | --- | --- |
|  | 25, 30, 35, 40 MHz (Note 1) |  |
| 3400 ≤ f ≤ 3800 | -23 (Note 2, Note 4) | 5 MHz |
|  | -40 (Note 3) | 1 MHz |
| NOTE 1: Possible aggregated bandwidth for CA_42C as specified in Table 5.6A.1-1. NOTE 2: This requirement applies within an offset between 5 MHz and 25 MHz from the lower and from the upper edge of the channel bandwidth, whenever these frequencies overlap with the specified frequency band. NOTE 3: This requirement applies from 3400 MHz to 25 MHz below the lower E-UTRA channel edge and from 25 MHz above the upper E-UTRA channel edge to 3800 MHz. NOTE 4: This emission limit might imply risk of harmful interference to UE(s) operating in the protected operating band. |  |  |

##### 6.6.3.3A.8	Minimum requirement for CA_41C and CA_41D (network signalled value "CA_NS_04")

When "CA_NS_04" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3.3A.8-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1A-1 from the edge of the aggregated channel bandwidth.

Table 6.6.3.3A.8-1: Additional requirements

| Frequency band | Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
| 2490.5 MHz ≤ f < 2495 MHz | -13 | 1 MHz |
| 9 kHz < f < 2490.5 MHz | -25 | 1 MHz |

##### 6.6.3.3A.9	Void

##### 6.6.3.3A.10	Minimum requirement for CA_48B and CA_48C (network signalled value "CA_NS_10")

When "CA_NS_10" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3.3A.10-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1A-1 from the edge of the aggregated channel bandwidth.

Table 6.6.3.3A.10-1: Additional requirements

| Frequency range (MHz) | Aggregated bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 5, 10, 15, 20, 40 MHz |  |
| 9 kHz – 3530 MHz | -40 | 1 MHz |
| 3530 MHz – 3540 MHz | -25 |  |
| 3710 MHz – 3720 MHz | -25 |  |
| 3720 MHz – 12.75 GHz | -40 |  |

### 6.6.3A	Void

<reserved for future use>

### 6.6.3B		Spurious emission for UL-MIMO

For UE supporting UL-MIMO, the requirements for Spurious emissions which are caused by unwanted transmitter effects such as harmonics emission, parasitic emissions, intermodulation products and frequency conversion products are specified at each transmit antenna connector.

For UEs with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the requirements in subclause 6.6.3 apply to each transmit antenna connector. The requirements shall be met with the UL-MIMO configurations specified in Table 6.2.2B-1.

If UE is configured for transmission on single-antenna port, the general requirements in subclause 6.6.3 apply.

### 6.6.3C		Void

<reserved for future use>

### 6.6.3D		Spurious emission for ProSe

When UE is configured for E-UTRA ProSe sidelink transmissions non-concurrent with E-UTRA uplink transmissions for E-UTRA ProSe operating bands specified in Table 5.5D-1, the requirements in subclause 6.6.3 apply.

When UE is configured for simultaneous E-UTRA ProSe sidelink and E-UTRA uplink transmissions for inter-band E-UTRA ProSe / E-UTRA bands specified in Table 5.5D-2, the UE co-existence requirements in Table 6.6.3.2A-0 in subclause 6.6.3.2A apply as specified for the corresponding inter-band aggregation with uplink assigned to two bands.

### 6.6.3F	Spurious emission for category NB1 and NB2

When UE is configured for category NB1 or NB2 uplink transmissions the requirements in subclause 6.6.3 apply with an exception that boundary between category NB1 or NB2 out of band and spurious emission domain shall be FOOB = 1.7 MHz.

### 6.6.3G	Spurious emission for V2X Communication

When UE is configured for E-UTRA V2X sidelink transmissions non-concurrent with E-UTRA uplink transmissions for E-UTRA V2X operating bands specified in Table 5.5G-1, the requirements in subclause 6.6.3 apply.

When UE is configured for simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions for inter-band E-UTRA V2X / E-UTRA bands specified in Table 5.5G-2, the UE-coexistence requirements in Table 6.6.3G-0 in subclause 6.6.3G apply as as specified for the corresponding inter-band con-current operation with uplink assigned to two bands.

Table 6.6.3G-0: Requirements for inter-band con-current V2X operation

| V2X con-current band Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| V2X_3A-47A | E-UTRA Band 1, 5, 7, 8, 26, 28, 34, 39, 40, 44, 45, 65, 87, 88 NR band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 3 | FDL_low | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA Band 22, 41, 42, 52 NR band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 5925 | - | 5950 | -30 | 1 | 7,8 |
|  | Frequency range | 5815 | - | 5855 | -30 | 1 | 7 |
| V2X_5A-47A | E-UTRA Band 1, 3, 5, 7, 8, 10, 12, 13, 14, 17, 40, 53, 65, 85 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 26 | 859 | - | 869 | -27 | 1 |  |
|  | E-UTRA Band 41, 52 NR band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 5925 | - | 5950 | -30 | 1 | 7, 8 |
|  | Frequency range | 5815 | - | 5855 | -30 | 1 | 7 |
| V2X_7A-47A | E-UTRA Band 1, 3, 5, 7, 8, 22, 26, 28, 34, 39, 40, 41, 42, 44, 45, 52, 65, 87, 88 NR band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 2570 | - | 2575 | +1.6 | 5 | 3, 6, 4 |
|  | Frequency range | 2575 | - | 2595 | -15.5 | 5 | 3, 6, 4 |
|  | Frequency range | 2595 | - | 2620 | -40 | 1 | 3, 6 |
|  | Frequency range | 5925 | - | 5950 | -30 | 1 | 7, 8 |
|  | Frequency range | 5815 | - | 5855 | -30 | 1 | 7 |
| V2X_8A-47A | E-UTRA Band 1, 5, 26, 28, 34, 39, 40, 44, 45, 65, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 7, 22, 41, 42, 52 NR band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 3, 8 | FDL_low | - | FDL_high | -50 | 1 | 2, 3 |
|  | Frequency range | 5925 | - | 5950 | -30 | 1 | 7, 8 |
|  | Frequency range | 5815 | - | 5855 | -30 | 1 | 7 |
| V2X_20A-47A | E-UTRA Band 1, 3, 7, 8, 22, 31, 32, 33, 34, 40, 43, 65, 67, 87, 88 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 20 | FDL_low | - | FDL_high | -50 | 1 | 3 |
|  | E-UTRA Band 38, 42, 52, 69 NR band n77, n78 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 758 | - | 788 | -50 | 1 |  |
|  | Frequency range | 5925 | - | 5950 | -30 | 1 | 7, 8 |
|  | Frequency range | 5815 | - | 5855 | -30 | 1 | 7 |
| V2X_28A-47A | E-UTRA Band 1, 22, 42, 43, 65 NR band n77, n78, 87, 88 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | E-UTRA Band 1 | FDL_low | - | FDL_high | -50 | 1 | 10, 11 |
|  | E-UTRA Band 3, 7, 8, 20, 31, 38, 40 NR band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 470 | - | 694 | -42 | 8 | 3, 12 |
|  | Frequency range | 470 | - | 710 | -26.2 | 6 | 13 |
|  | Frequency range | 662 | - | 694 | -26.2 | 6 | 3 |
|  | Frequency range | 758 | - | 773 | -32 | 1 | 3 |
|  | Frequency range | 773 | - | 803 | -50 | 1 |  |
|  | Frequency range | 5925 | - | 5950 | -30 | 1 | 7, 8 |
|  | Frequency range | 5815 | - | 5855 | -30 | 1 | 7 |
| V2X_34A-47A | E-UTRA Band 1, 3, 5, 7, 8, 11, 18, 19, 20, 21, 22, 26, 28, 31, 32, 33, 34, 38,39, 40, 41, 42, 43, 44, 45, 52, 65, 67, 69, 87, 88 NR Band n78, n79 | FDL_low | - | FDL_high | -50 | 1 | 9 |
|  | NR Band n77 | FDL_low | - | FDL_high | -50 | 1 | 2, 9 |
|  | Frequency range | 5925 | - | 5950 | -30 | 1 | 7, 8 |
|  | Frequency range | 5815 | - | 5855 | -30 | 1 | 7 |
| V2X_39A-47A | E-UTRA Band 1, 3,5,7,8, 22, 26, 28, 34, 39, 40, 41, 42, 44, 45, 52, 65 NR Band n79 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n77,n78 | FDL_low | - | FDL_high | -50 | 1 | 2, 9 |
|  | Frequency range | 1805 | - | 1855 | [-40] | 1 | 5 |
|  | Frequency range | 1855 | - | 1880 | [-15.5] | 5 | 3, 4, 5 |
|  | Frequency range | 5925 | - | 5950 | -30 | 1 | 7, 8 |
|  | Frequency range | 5815 | - | 5855 | -30 | 1 | 7 |
| V2X_41A-47A | E-UTRA Band 1, 3, 5, 7, 8, 22, 26, 28, 34, 39, 40, 41, 42, 44, 45, 52, 65 NR Band n77, n78 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | NR Band n79 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 5925 | - | 5950 | -30 | 1 | 7, 8 |
|  | Frequency range | 5815 | - | 5855 | -30 | 1 | 7 |
| V2X_71A-47A | E-UTRA Band 5, 26, 53 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | E-UTRA Band 41 | FDL_low | - | FDL_high | -50 | 1 | 2 |
|  | Frequency range | 5925 | - | 5950 | -30 | 1 | 7, 8 |
|  | Frequency range | 5815 | - | 5855 | -30 | 1 | 7 |
| NOTE 1: FDL_low and FDL_high refer to each E-UTRA frequency band specified in Table 5.5-1 NOTE 2: As exceptions, measurements with a level up to the applicable requirements defined in Table 6.6.3.1-2 are permitted for each assigned E-UTRA carrier used in the measurement due to 2nd, 3rd, 4th [or 5th] harmonic spurious emissions. In case the exceptions are allowed due to spreading of the harmonic emission the exception is also allowed for the first 1 MHz frequency range immediately outside the harmonic emission on both sides of the harmonic emission. This results in an overall exception interval centred at the harmonic emission of (2MHz + N x LCRB x 180kHz), where N is 2, 3 or 4 for the 2nd, 3rd or 4th harmonic respectively. The exception is allowed if the measurement bandwidth (MBW) totally or partially overlaps the overall exception interval. NOTE 3: These requirements also apply for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1-1 and Table 6.6.3.1A-1 from the edge of the aggregated channel bandwidth. NOTE 4: For these adjacent bands, the emission limit could imply risk of harmful interference to UE(s) operating in the protected operating band. NOTE 5: This requirement is only applicable for carriers with bandwidth confined within 1885-1920 MHz (requirement for carriers with at least 1RB confined within 1880 - 1885 MHz is not specified). This requirement applies for an uplink transmission bandwidth less than or equal to 54 RB for carriers of 15 MHz bandwidth when carrier center frequency is within the range 1892.5 - 1894.5 MHz and for carriers of 20 MHz bandwidth when carrier center frequency is within the range 1895 - 1903 MHz. NOTE 6: As exceptions, measurements with a level up to the applicable requirement of -38 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 2nd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 2nd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 7: Applicable when NS_33 or NS_34 is configured by the pre-configured radio parameters. NOTE 8: In the frequency range x-5950MHz, SE requirement of -30dBm/MHz should be applied; where x = max (5925, fc + 15), where fc is the channel centre frequency. NOTE 9: For non synchronised TDD operation to meet these requirements some restriction will be needed for either the operating band or protected band NOTE 10: Applicable when the assigned E-UTRA carrier is confined within 718 MHz and 748 MHz and when the channel bandwidth used is 5 or 10 MHz. NOTE 11: As exceptions, measurements with a level up to the applicable requirement of -36 dBm/MHz is permitted for each assigned E-UTRA carrier used in the measurement due to 3rd harmonic spurious emissions. An exception is allowed if there is at least one individual RB within the transmission bandwidth (see Figure 5.6-1) for which the 3rd harmonic totally or partially overlaps the measurement bandwidth (MBW). NOTE 12: This requirement is applicable in the case of a 10 MHz E-UTRA carrier confined within 703 MHz and 733 MHz, otherwise the requirement of -25 dBm with a measurement bandwidth of 8 MHz applies. NOTE 13: This requirement is applicable for 5 and 10 MHz E-UTRA channel bandwidth allocated within 718-728MHz. For carriers of 10 MHz bandwidth, this requirement applies for an uplink transmission bandwidth less than or equal to 30 RB with RBstart > 1 and RBstart<48. |  |  |  |  |  |  |  |

For intra-band contiguous multi-carrier operation, the boundary between E-UTRA out of band and spurious emission domain for intra-band contiguous carrier aggregation specified in Table 6.6.3.1A-1 shall apply.

For intra-band contiguous multi-carrier operation, the spurious emission requirements in Table 6.6.3G-1 shall apply for coexistence with protected bands.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

Table 6.6.3G-1: Requirements for intraband multi-carrier V2X operation

| V2X multi-carrier Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| V2X_47B | E-UTRA Band 1, 3, 5, 7, 8, 22, 26, 28, 34, 39, 40, 41, 42, 44, 45, 50, 51, 52, 65 NR band n77, n78, n79 | FDL_low | - | FDL_high | -50 | 1 |  |

For V2X UEs supportingTransmit Diversity, the requirements specified for single carrier shall apply to each transmit antenna connector.

If V2X UE is configured for transmission on single-antenna connector, the general requirements specified for single carrier shall apply to the active antenna connector.

## 6.6A	Void

## 6.6B	Void

## 6.7	Transmit intermodulation

The transmit intermodulation performance is a measure of the capability of the transmitter to inhibit the generation of signals in its non linear elements caused by presence of the wanted signal and an interfering signal reaching the transmitter via the antenna.

### 6.7.1	Minimum requirement

User Equipment(s) transmitting in close vicinity of each other can produce intermodulation products, which can fall into the UE, or eNode B receive band as an unwanted interfering signal. The UE intermodulation attenuation is defined by the ratio of the mean power of the wanted signal to the mean power of the intermodulation product when an interfering CW signal is added at a level below the wanted signal at each of the transmitter antenna port with the other antenna port(s) if any is terminated. Both the wanted signal power and the intermodulation product power are measured through E-UTRA rectangular filter with measurement bandwidth shown in Table 6.7.1-1.

The requirement of transmitting intermodulation is prescribed in Table 6.7.1-1.

Table 6.7.1-1: Transmit Intermodulation

| BW Channel (UL) | 5MHz |  | 10MHz |  | 15MHz |  | 20MHz |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Interference Signal Frequency Offset | 5MHz | 10MHz | 10MHz | 20MHz | 15MHz | 30MHz | 20MHz | 40MHz |
| Interference CW Signal Level | -40dBc |  |  |  |  |  |  |  |
| Intermodulation Product | -29dBc | -35dBc | -29dBc | -35dBc | -29dBc | -35dBc | -29dBc | -35dBc |
| Measurement bandwidth | 4.5MHz | 4.5MHz | 9.0MHz | 9.0MHz | 13.5MHz | 13.5MHz | 18MHz | 18MHz |

### 6.7.1A	Minimum requirement for CA

User Equipment(s) transmitting in close vicinity of each other can produce intermodulation products, which can fall into the UE, or eNode B receive band as an unwanted interfering signal. The UE intermodulation attenuation is defined by the ratio of the mean power of the wanted signal to the mean power of the intermodulation product on both component carriers when an interfering CW signal is added at a level below the wanted signal at each of the transmitter antenna port with the other antenna port(s) if any is terminated. Both the wanted signal power and the intermodulation product power are measured through rectangular filter with measurement bandwidth shown in Table 6.7.1A-1.

For inter-band carrier aggregation with one component carrier per operating band and the uplink active in two E-UTRA bands, the requirement is specified in Table 6.7.1-1 which shall apply on each component carrier with both component carriers active.

For intra-band contiguous carrier aggregation the requirement of transmitting intermodulation is specified in Table 6.7.1A-1.

Table 6.7.1A-1: Transmit Intermodulation

| CA bandwidth class(UL) | B and C |  |
| --- | --- | --- |
| Interference Signal Frequency Offset | BWChannel_CA | 2*BWChannel_CA |
| Interference CW Signal Level | -40dBc |  |
| Intermodulation Product | -29dBc | -35dBc |
| Measurement bandwidth | BWChannel_CA- 2* BWGB |  |

For combinations of intra-band and inter-band carrier aggregation with three uplink component carriers (up to two contiguously aggregated carriers per band) transmit intermodulations is defined as follows. For the E-UTRA band supporting one component carrier the requirement specified in Table 6.7.1-1 apply. For the E-UTRA band supporting two contiguous component carriers the requirements specified in Table 6.7.1A-1 apply.

### 6.7.1B	Minimum requirement for UL-MIMO

For UE supporting UL-MIMO, the transmit intermodulation requirements are specified at each transmit antenna connector and the wanted signal is defined as the sum of output power at each transmit antenna connector.

For UEs with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the requirements in subclause 6.7.1 apply to each transmit antenna connector. The requirements shall be met with the UL-MIMO configurations specified in Table 6.2.2B-2.

If UE is configured for transmission on single-antenna port, the requirements in subclause 6.7.1 apply.

### 6.7.1F	Minimum requirement for category NB1 and NB2

The UE category NB1 and NB2 transmitter intermodulation attenuation is defined by the ratio of the mean power of the wanted signal to the mean power of the intermodulation product as defined in Table 6.7.1F-1 when an interfering CW signal is added at a level below the wanted signal at the transmitter antenna port. Both the wanted signal power and the intermodulation product power are measured through rectangular filter with measurement bandwidth shown in Table 6.7.1F-1.

Table 6.7.1F-1: UE category NB1 and NB2 transmitter IM requirement

| Parameters for transmitter intermodulation |  |  |
| --- | --- | --- |
| BW Channel (UL) | 15 kHz (1 tone at sub-carrier 5 or 6) |  |
| Interference Signal Frequency Offset | 180 kHz | 360 kHz |
| Interference CW Signal Level | -40dBc |  |
| Intermodulation Product | -20 dBc | -39 dBc |
| Measurement bandwidth | 30 kHz | 30 kHz |

### 6.7.1G	Minimum requirement for V2X Communication

When UE is configured for E-UTRA V2X sidelink transmissions non-concurrent with E-UTRA uplink transmissions for E-UTRA V2X operating bands specified in Table Table 5.5G-1, the requirements in subclause 6.7.1 apply for E-UTRA V2X sidelink transmission.

When UE is configured for simultaneous E-UTRA V2X sidelink and E-UTRA uplink transmissions for inter-band E-UTRA V2X / E-UTRA bands specified in Table 5.5G-2, the requirements in subclause 6.7.1 apply for V2X sidelink transmission and the E-UTRA uplink transmission.

For intra-band contiguous multi-carrier operation, the transmit intermodulation requirement for CA Bandwidth Class B specified in subclause 6.7.1A shall apply for V2X Bandwdith Class B, the general CA spectrum emission mask for CA Bandwidth Class C specified in subclause 6.7.1A shall apply for V2X Bandwdith Class C and C1.

For V2X UE supporting Transmit Diversity, if the UE transmits on two antenna connectors at the same time, the requirements specified for single carrier shall apply to each transmit antenna connector. If the UE transmits on one antenna connector, the requirements specified for single carrier shall apply to the active antenna connector.

## 6.8	Void

## 6.8A	Void

## 6.8B		Time alignment error for UL-MIMO

For UE(s) with multiple transmit antenna connectors supporting UL-MIMO, this requirement applies to frame timing differences between transmissions on multiple transmit antenna connectors in the closed-loop spatial multiplexing scheme.

The time alignment error (TAE) is defined as the average frame timing difference between any two transmissions on different transmit antenna connectors.

### 6.8B.1		Minimum Requirements

For UE(s) with multiple transmit antenna connectors, the Time Alignment Error (TAE) shall not exceed 130 ns.

## 6.8C	Void

## 6.8D	Void

## 6.8E	Void

## 6.8F	Void

## 6.8G	Time alignment error

For V2X UE(s) with two  transmit antenna connectors in Transmit Diversity scheme, this requirement applies to frame timing differences between transmissions on two transmit antenna connectors.The Time Alignment Error (TAE) shall not exceed [260] ns.

# 7	Receiver characteristics

## 7.1	General

Unless otherwise stated the receiver characteristics are specified at the antenna connector(s) of the UE. For UE(s) with an integral antenna only, a reference antenna(s) with a gain of 0 dBi is assumed for each antenna port(s). UE with an integral antenna(s) may be taken into account by converting these power levels into field strength requirements, assuming a 0 dBi gain antenna. . For UEs with more than one receiver antenna connector, identical interfering signals shall be applied to each receiver antenna port if more than one of these is used (diversity).

The levels of the test signal applied to each of the antenna connectors shall be as defined in the respective sections below.

With the exception of subclause 7.3, the requirements shall be verified with the network signalling value NS_01 configured (Table 6.2.4-1).

All the parameters in clause 7 are defined using the UL reference measurement channels specified in Annexes A.2.2 and A.2.3, the DL reference measurement channels specified in Annex A.3.2 and using the set-up specified in Annex C.3.1.

For the additional requirements for intra-band non-contiguous carrier aggregation of two or more sub-blocks, an in-gap test refers to the case when the interfering signal is located at a negative offset with respect to the assigned lowest channel frequency of the highest sub-block and located at a positive offset with respect to the assigned highest channel frequency of the lowest sub-block.

For the additional requirements for intra-band non-contiguous carrier aggregation of two or more sub-blocks, an out-of-gap test refers to the case when the interfering signal(s) is (are) located at a positive offset with respect to the assigned channel frequency of the highest carrier frequency, or located at a negative offset with respect to the assigned channel frequency of the lowest carrier frequency.

For the additional requirements for intra-band non-contiguous carrier aggregation of two or more sub-blocks with channel bandwidth larger than or equal to 5 MHz, the existing adjacent channel selectivity requirements, in-band blocking requirements (for each case), and narrow band blocking requirements apply for in-gap tests only if the corresponding interferer frequency offsets with respect to the two measured carriers satisfy the following condition in relation to the sub-block gap size Wgap for at least one of these carriers j = 1,2, so that the interferer frequency position does not change the nature of the core requirement tested:

Wgap ≥ 2∙|FInterferer (offset),j|  – BWChannel(j)

where FInterferer (offset),j for a sub-block with a single component carrier is the interferer frequency offset with respect to carrier j as specified in subclause 7.5.1, subclause 7.6.1 and subclause 7.6.3 for the respective requirement and BWChannel(j) the channel bandwidth of carrier j. FInterferer (offset),j for a sub-block with two or more contiguous component carriers is the interference frequency offset with respect to the carrier adjacent to the gap is specified in subclause 7.5.1A, 7.6.1A and 7.6.3A. The interferer frequency offsets for adjacent channel selectivity, each in-band blocking case and narrow- band blocking shall be tested separately with a single in-gap interferer at a time.

For a ProSe UE that supports both ProSe Direct Discovery and ProSe Direct Communication, the receiver characteristics specified in clause 7 for ProSe Direct Communication shall apply.

For ProSe Direct Discovery and ProSe Direct Communication on E-UTRA ProSe operating bands that correspond to TDD E-UTRA operating bands as specified in subclause 5.5D, the only additional requirement for ProSe specified in subcaluse 7.4.1D is applicable.

## 7.2	Diversity characteristics

The requirements in Section 7 assume that the receiver is equipped with two Rx port as a baseline. These requirements apply to all UE categories unless stated otherwise. Additional requirements apply for UE(s) equipped with four Rx ports. These additional requirements also apply for supported band combinations for which the UE can operate using up to four Rx ports while configured with carrier aggregation. With the exception of subclause 7.9 all requirements shall be verified by using both (all) antenna ports simultaneously.

NOTE:	for an operating band in which the UE can operate using up to four Rx ports, it suffices to verify for conformance the additional requirements applicable for four Rx ports [except for REFSENS].

NOTE:	Implementation of 4 antenna ports for all operating bands supported by the UE is not mandated.

For a category 0, a category [M 1] , category 1bis, category NB1 and NB2 UE the requirements in Section 7 assume that the receiver is equipped with single Rx port.

## 7.3	Reference sensitivity power level

The reference sensitivity power level REFSENS is the minimum mean power applied to each one of the UE antenna ports for all UE categories except category 0, category M1, category M2, and category 1bis, or to the single antenna port for UE category 0, UE category M1, category M2, and UE category 1bis, at which the throughput shall meet or exceed the requirements for the specified reference measurement channel.

The throughput for the REFSENS test is measured based on the Transmission Mode 1 unless specified otherwise.

### 7.3.1	Minimum requirements (QPSK)

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.1-1 and Table 7.3.1-2

Table 7.3.1-1: Reference sensitivity QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
| 2 | -102.7 | -99.7 | -98 | -95 | -93.2 | -92 | FDD |
| 3 | -101.7 | -98.7 | -97 | -94 | -92.2 | -91 | FDD |
| 4 | -104.7 | -101.7 | -100 | -97 | -95.2 | -94 | FDD |
| 5 | -103.2 | -100.2 | -98 | -95 |  |  | FDD |
| 6 |  |  | -100 | -97 |  |  | FDD |
| 7 |  |  | -98 | -95 | -93.2 | -92 | FDD |
| 8 | -102.2 | -99.2 | -97 | -94 |  |  | FDD |
| 9 |  |  | -99 | -96 | -94.2 | -93 | FDD |
| 10 |  |  | -100 | -97 | -95.2 | -94 | FDD |
| 11 |  |  | -100 | -97 |  |  | FDD |
| 12 | -101.7 | -98.7 | -97 | -94 |  |  | FDD |
| 13 |  |  | -97 | -94 |  |  | FDD |
| 14 |  |  | -97 | -94 |  |  | FDD |
| … |  |  |  |  |  |  |  |
| 17 |  |  | -97 | -94 |  |  | FDD |
| 18 |  |  | -1007 | -977 | -95.27 |  | FDD |
| 19 |  |  | -100 | -97 | -95.2 |  | FDD |
| 20 |  |  | -97 | -94 | -91.2 | -90 | FDD |
| 21 |  |  | -100 | -97 | -95.2 |  | FDD |
| 22 |  |  | -97 | -94 | -92.2 | -91 | FDD |
| 23 | -104.7 | -101.7 | -100 | -97 | -95.2 | -94 | FDD |
| 24 |  |  | -100 | -97 |  |  | FDD |
| 25 | -101.2 | -98.2 | -96.5 | -93.5 | -91.7 | -90.5 | FDD |
| 26 | -102.7 | -99.7 | -97.56 | -94.56 | -92.76 |  | FDD |
| 27 | -103.2 | -100.2 | -98 | -95 |  |  | FDD |
| 28 |  | -100.2 | -98.5 | -95.5 | -93.7 | -91 | FDD |
| 30 |  |  | -99 | -96 |  |  | FDD |
| 31 | -99.0 | -95.7 | -93.5 |  |  |  | FDD |
| … |  |  |  |  |  |  |  |
| 33 |  |  | -100 | -97 | -95.2 | -94 | TDD |
| 34 |  |  | -100 | -97 | -95.2 |  | TDD |
| 35 | -106.2 | -102.2 | -100 | -97 | -95.2 | -94 | TDD |
| 36 | -106.2 | -102.2 | -100 | -97 | -95.2 | -94 | TDD |
| 37 |  |  | -100 | -97 | -95.2 | -94 | TDD |
| 38 |  |  | -100 | -97 | -95.2 | -94 | TDD |
| 39 |  |  | -100 | -97 | -95.2 | -94 | TDD |
| 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |
| 41 |  |  | -98 | -95 | -93.2 | -92 | TDD |
| 42 |  |  | -99 | -96 | -94.2 | -93 | TDD |
| 43 |  |  | -99 | -96 | -94.2 | -93 | TDD |
| 44 |  | [-100.2] | [-98] | [-95] | [-93.2] | [-92] | TDD |
| 45 |  |  | -100 | -97 | -95.2 | -94 | TDD |
| 48 |  |  | -99 | -96 | -94.2 | -93 | TDD |
| 50 |  | -102.2 | -100 | -97 | -95.2 | -94 | TDD |
| 51 |  | -102.2 | -100 |  |  |  | TDD |
| 52 |  |  | -99 | -96 | -94.2 | -93 | TDD |
| 53 | -106.2 | -102.2 | -100 | -97 |  |  | TDD |
| ... |  |  |  |  |  |  |  |
| 65 | -104.2 | -101.2 | -99.5 | -96.5 | -94.7 | -93.5 | FDD |
| 66 | -104.2 | -101.2 | -99.5 | -96.5 | -94.7 | -93.5 | FDD |
| 68 |  |  | -98.5 | -95.5 | -93.7 |  | FDD |
| … |  |  |  |  |  |  |  |
| 70 |  |  | -100 | -97 | -95.2 | -94 | FDD |
| 71 |  |  | -97.2 | -94.2 | -92.0 | -87.5 | FDD |
| 72 | -99.0 | -95.7 | -93.5 |  |  |  | FDD |
| 73 | -99.0 | -95.7 | -93.5 |  |  |  | FDD |
| 74 | -104.78 | -101.78 | -99.58 | -96.58 | -94.78 | -89.88 | FDD |
| 85 |  |  | -97 | -94 |  |  | FDD |
| 87 | -99.0 | -95.7 | -93.5 |  |  |  | FDD |
| 88 | -99.0 | -95.7 | -93.5 |  |  |  | FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5 NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: The signal power is specified per port NOTE 4: For the UE which supports both Band 3 and Band 9 the reference sensitivity level is FFS. NOTE 5: For the UE which supports both Band 11 and Band 21 the reference sensitivity level is FFS. NOTE 6: 6 indicates that the requirement is modified by -0.5 dB when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 7: For a UE that support both Band 18 and Band 26, the reference sensitivity level for Band 26 applies for the applicable channel bandwidths. NOTE 8: 8 indicates that the requirement is modified by -0.5 dB when the assigned E-UTRA channel bandwidth is confined within 1475.9-1510.9 MHz. |  |  |  |  |  |  |  |

For UE(s) equipped with 4 antenna ports, the minimum requirement for reference sensitivity in Table 7.3.1-1 shall be modified by the amount given in ΔRIB,4R in Table 7.3.1-1a for the applicable E-UTRA bands.

Table 7.3.1-1a: ΔRIB,4R

| E-UTRA Band | ΔRIB,4R [dB] |
| --- | --- |
| 1, 2, 3, 4, 7, 20, 21, 25, 30, 34, 39, 40, 41, 66 | - 2.7 |
| 42, 43, 52 | - 2.2 |

For UE(s) equipped with 8 antenna ports, the minimum requirement for reference sensitivity in Table 7.3.1-1 shall be modified by the amount given in ΔRIB,8R in Table 7.3.1-1aa for the applicable E-UTRA bands.

Table 7.3.1-1aa: ΔRIB,8R

| E-UTRA Band | ΔRIB,8R [dB] |
| --- | --- |
| 41, 42, 43 | - 4 |

For UE(s) supporting power class 1 in any of the E-UTRA bands given in table 7.3.1-1b, the following exceptions due to the high power leakage or blocking issue shall apply. The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.1-1b and Table 7.3.1-2.

Table 7.3.1-1b: Reference sensitivity for power class 1 QPSK PREFSENS (Exception due to high power issue)

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| 20 |  |  | -92.8 | -90.9 | -89.5 | -88.5 | FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5 NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: The signal power is specified per port. |  |  |  |  |  |  |  |

The reference receive sensitivity (REFSENS) requirement specified in Table 7.3.1-1 (two antenna ports) and Table 7.3.1-1a (four antenna ports) shall be met for an uplink transmission bandwidth less than or equal to that specified in Table 7.3.1-2.

NOTE:	Table 7.3.1-2 is intended for conformance tests and does not necessarily reflect the operational conditions of the network, where the number of uplink and downlink allocated resource blocks will be practically constrained by other factors. Typical receiver sensitivity performance with HARQ retransmission enabled and using a residual BLER metric relevant for e.g. Speech Services is given in the Annex G (informative).For the UE which supports inter-band carrier aggregation configuration with the uplink in one or two E-UTRA bands, the minimum requirement for reference sensitivity in Table 7.3.1-1 and Table 7.3.1-1a shall be increased by the amount given in ΔRIB,c in Table 7.3.1-1A, Table 7.3.1-1B and Table 7.3.1-1C for the applicable E-UTRA bands where unless otherwise stated, the same ΔRIB,c is applicable to E-UTRA band(s) part for CA configurations which have the same E-UTRA operating band combination.

Table 7.3.1-1A: ΔRIB,c (two bands)

| E-UTRA operating band combination |  | E-UTRA Band |  | ΔRIB,c [dB] |  |
| --- | --- | --- | --- | --- | --- |
| CA_1-3, CA_1-1-3, CA_1-1-3-3, CA_1-3-3 |  | 1 |  | 0 |  |
|  |  | 3 |  | 0 |  |
| CA_1-5, CA_1-1-5 |  | 1 |  | 0 |  |
|  |  | 5 |  | 0 |  |
| CA_1-7, CA_1-1-7, CA_1-7-7 |  | 1 |  | 0 |  |
|  |  | 7 |  | 0 |  |
| CA_1-8 |  | 1 |  | 0 |  |
|  |  | 8 |  | 0 |  |
| CA_1-11 |  | 1 |  | 0 |  |
|  |  | 11 |  | 0 |  |
| CA_1-18 |  | 1 |  | 0 |  |
|  |  | 18 |  | 0 |  |
| CA_1-19 |  | 1 |  | 0 |  |
|  |  | 19 |  | 0 |  |
| CA_1-20 |  | 1 |  | 0 |  |
|  |  | 20 |  | 0 |  |
| CA_1-21 |  | 1 |  | 0 |  |
|  |  | 21 |  | 0 |  |
| CA_1-26 |  | 1 |  | 0 |  |
|  |  | 26 |  | 0 |  |
| CA_1-28, CA_1-1-28 |  | 1 |  | 0 |  |
|  |  | 28 |  | 0.2 |  |
| CA_1-32 |  | 1 |  | 0 |  |
|  |  | 32 |  | 0 |  |
| CA_1-38 |  | 1 |  | 0 |  |
|  |  | 38 |  | 0 |  |
| CA_1-40 |  | 1 |  | 0 |  |
|  |  | 40 |  | 0 |  |
| CA_1-418 |  | 1 |  | 0 |  |
|  |  | 41 |  | 0 |  |
| CA_1-42, CA_1-42-42 |  | 1 |  | 0 |  |
|  |  | 42 |  | 0.5 |  |
| CA_1-43 |  | 1 |  | 0 |  |
|  |  | 43 |  | 0.5 |  |
| CA_1-46 |  | 1 |  | 0 |  |
| CA_2-4, CA_2-2-4, CA_2-4-4, CA_2-2-4-4 |  | 2 |  | 0.3 |  |
|  |  | 4 |  | 0.3 |  |
| CA_2-5, CA_2-2-5 |  | 2 |  | 0 |  |
|  |  | 5 |  | 0 |  |
| CA_2-7, CA_2-2-7, CA_2-7-7 |  | 2 |  | 0 |  |
|  |  | 7 |  | 0 |  |
| CA_2-12, CA_2-2-12, CA_2-12-12, CA_2-2-12-12 |  | 2 |  | 0 |  |
|  |  | 12 |  | 0 |  |
| CA_2-13, CA_2-2-13 |  | 2 |  | 0 |  |
|  |  | 13 |  | 0 |  |
| CA_2-14, CA_2-2-14 |  | 2 |  | 0 |  |
|  |  | 14 |  | 0 |  |
| CA_2-17 |  | 2 |  | 0 |  |
|  |  | 17 |  | 0.5 |  |
| CA_2-26 |  | 2 |  | 0 |  |
|  |  | 26 |  | 0 |  |
| CA_2-28 |  | 2 |  | 0 |  |
|  |  | 28 |  | 0 |  |
| CA_2-29, CA_2-2-29 |  | 2 |  | 0 |  |
| CA_2-30, CA_2-2-30 |  | 2 |  | 0.4 |  |
|  |  | 30 |  | 0.5 |  |
| CA_2-46, CA_2-2-46 |  | 2 |  | 0 |  |
| CA_2-48, CA_2-48-48 |  | 2 |  | 0.2 |  |
|  |  | 48 |  | 0.5 |  |
| CA_2-49 |  | 2 |  | 0.2 |  |
| CA_2-66, CA_2-2-66, CA_2-66-66, CA_2-2-66-66, CA_2-66-66-66 |  | 2 |  | 0.3 |  |
|  |  | 66 |  | 0.3 |  |
| CA_2-71, CA_2-2-71 |  | 2 |  | 0 |  |
|  |  | 71 |  | 0 |  |
| CA_3-5, CA_3-3-5 |  | 3 |  | 0 |  |
|  |  | 5 |  | 0 |  |
| CA_3-7, CA_3-3-7, CA_3-7-7, CA_3-3-7-7 |  | 3 |  | 0 |  |
|  |  | 7 |  | 0 |  |
| CA_3-8, CA_3-3-8 |  | 3 |  | 0 |  |
|  |  | 8 |  | 0 |  |
| CA_3-11 |  | 3 |  | 0.3 |  |
|  |  | 11 |  | 0.5 |  |
| CA_3-18 |  | 3 |  | 0 |  |
|  |  | 18 |  | 0 |  |
| CA_3-19, CA_3-3-19 |  | 3 |  | 0 |  |
|  |  | 19 |  | 0 |  |
| CA_3-20, CA_3-3-20 |  | 3 |  | 0 |  |
|  |  | 20 |  | 0 |  |
| CA_3-21, CA_3-3-21 |  | 3 |  | 0.3 |  |
|  |  | 21 |  | 0.5 |  |
| CA_3-26 |  | 3 |  | 0 |  |
|  |  | 26 |  | 0 |  |
| CA_3-27 |  | 3 |  | 0 |  |
|  |  | 27 |  | 0 |  |
| CA_3-28 |  | 3 |  | 0 |  |
|  |  | 28 |  | 0 |  |
| CA_3-31 |  | 3 |  | 0 |  |
|  |  | 31 |  | 0.2 |  |
| CA_3-32 |  | 3 |  | 0 |  |
|  |  | 32 |  | 0 |  |
| CA_3-38 |  | 3 |  | 0 |  |
|  |  | 38 |  | 0 |  |
| CA_3-40, CA_3-40-40 |  | 3 |  | 0 |  |
|  |  | 40 |  | 0 |  |
| CA_3-41, CA_3-3-41 |  | 3 |  | 0 |  |
|  |  | 41 |  | 010 |  |
|  |  |  |  | 0.511 |  |
| CA_3-42, CA_3-3-42, CA_3-42-42 |  | 3 |  | 0.2 |  |
|  |  | 42 |  | 0.5 |  |
| CA_3-43 |  | 3 |  | 0 |  |
|  |  | 43 |  | 0.5 |  |
| CA_3-46, CA_3-3-46 |  | 3 |  | 0 |  |
| CA_4-5, CA_4-4-5 |  | 4 |  | 0 |  |
|  |  | 5 |  | 0 |  |
| CA_4-7, CA_4-4-7, CA_4-7-7 |  | 4 |  | 0.5 |  |
|  |  | 7 |  | 0.5 |  |
| CA_4-12, CA_4-4-12, CA_4-12-12, CA_4-4-12-12 |  | 4 |  | 0 |  |
|  |  | 12 |  | 0.5 |  |
| CA_4-13, CA_4-4-13 |  | 4 |  | 0 |  |
|  |  | 13 |  | 0 |  |
| CA_4-17 |  | 4 |  | 0 |  |
|  |  | 17 |  | 0.5 |  |
| CA_4-27 |  | 4 |  | 0 |  |
|  |  | 27 |  | 0 |  |
| CA_4-28 |  | 4 |  | 0 |  |
|  |  | 28 |  | 0.2 |  |
| CA_4-29, CA_4-4-29 |  | 4 |  | 0 |  |
| CA_4-30, CA_4-4-30 |  | 4 |  | 0.4 |  |
|  |  | 30 |  | 0.5 |  |
| CA_4-46 |  | 4 |  | 0 |  |
| CA_4-48 |  | 4 |  | 0 |  |
|  |  | 48 |  | 0.5 |  |
| CA_4-71, CA_4-4-71 |  | 4 |  | 0 |  |
|  |  | 71 |  | 0 |  |
| CA_5-7, CA_5-7-7 |  | 5 |  | 0 |  |
|  |  | 7 |  | 0 |  |
| CA_5-12, CA_5-12-12 |  | 5 |  | 0.5 |  |
|  |  | 12 |  | 0.3 |  |
| CA_5-13 |  | 5 |  | 0 |  |
|  |  | 13 |  | 0 |  |
| CA_5-17 |  | 5 |  | 0.5 |  |
|  |  | 17 |  | 0.3 |  |
| CA_5-25 |  | 5 |  | 0 |  |
|  |  | 25 |  | 0 |  |
| CA_5-28 |  | 5 |  | 0 |  |
|  |  | 28 |  | 0 |  |
| CA_5-29 |  | 5 |  | 0 |  |
| CA_5-30 |  | 5 |  | 0 |  |
|  |  | 30 |  | 0 |  |
| CA_5-38 |  | 5 |  | 0 |  |
|  |  | 38 |  | 0 |  |
| CA_5-40, CA_5-5-40, CA_5-40-40 |  | 5 |  | 0 |  |
|  |  | 40 |  | 0 |  |
| CA_5-41 |  | 5 |  | 0 |  |
|  |  | 41 |  | 0 |  |
| CA_5-48 |  | 5 |  | 0 |  |
|  |  | 48 |  | 0 |  |
| CA_5-66, CA_5-5-66, CA_5-66-66, CA_5-5-66-66 |  | 5 |  | 0 |  |
|  |  | 66 |  | 0 |  |
| CA_7-8, CA_7-7-8 |  | 7 |  | 0 |  |
|  |  | 8 |  | 0.2 |  |
| CA_7-12 |  | 7 |  | 0 |  |
|  |  | 12 |  | 0 |  |
| CA_7-13 |  | 7 |  | 0 |  |
|  |  | 13 |  | 0 |  |
| CA_7-20, CA_7-7-20 |  | 7 |  | 0 |  |
|  |  | 20 |  | 0 |  |
| CA_7-22 |  | 7 |  | 0 |  |
|  |  | 22 |  | 0.5 |  |
| CA_7-26, CA_7-7-26 |  | 7 |  | 0 |  |
|  |  | 26 |  | 0 |  |
| CA_7-28, CA_7-7-28 |  | 7 |  | 0 |  |
|  |  | 28 |  | 0 |  |
| CA_7-29, CA_7-7-29 |  | 7 |  | 0 |  |
| CA_7-30 |  | 7 |  | 0.5 |  |
|  |  | 30 |  | 0.5 |  |
| CA_7-32 |  | 7 |  | 0 |  |
|  |  | 32 |  | 0 |  |
| CA_7-40 |  | 7 |  | 0 |  |
|  |  | 40 |  | 0.5 |  |
| CA_7-42, CA_7-42-42 |  | 7 |  | 0 |  |
|  |  | 42 |  | 0.5 |  |
| CA_7-46, CA_7-7-46 |  | 7 |  | 0 |  |
| CA_7-66, CA_7-7-66, CA_7-66-66, CA_7-7-66-66 |  | 7 |  | 0.5 |  |
|  |  | 66 |  | 0.5 |  |
| CA_8-11 |  | 8 |  | 0 |  |
|  |  | 11 |  | 0 |  |
| CA_8-20 |  | 8 |  | 0 |  |
|  |  | 20 |  | 0 |  |
| CA_8-27 |  | 8 |  | 0.3 |  |
|  |  | 27 |  | 0.3 |  |
| CA_8-2813 |  | 8 |  | 0.2 |  |
|  |  | 28 |  | 0.1 |  |
| CA_8-32 |  | 8 |  | 0 |  |
|  |  | 32 |  | 0 |  |
| CA_8-38 |  | 8 |  | 0 |  |
|  |  | 38 |  | 0 |  |
| CA_8-39 |  | 8 |  | 0 |  |
|  |  | 39 |  | 0 |  |
| CA_8-40 |  | 8 |  | 0 |  |
|  |  | 40 |  | 0 |  |
| CA_8-41 |  | 8 |  | 0 |  |
|  |  | 41 |  | 0 |  |
| CA_8-42 |  | 8 |  | 0.2 |  |
|  |  | 42 |  | 0.5 |  |
| CA_8-46 |  | 8 |  | 0 |  |
| CA_11-18 |  | 11 |  | 0 |  |
|  |  | 18 |  | 0 |  |
| CA_11-26 |  | 11 |  | 0 |  |
|  |  | 26 |  | 0 |  |
| CA_11-28 |  | 11 |  | 0 |  |
|  |  | 28 |  | 0.2 |  |
| CA_11-41 |  | 11 |  | 0 |  |
|  |  | 41 |  | 0 |  |
| CA_11-42 |  | 11 |  | 0 |  |
|  |  | 42 |  | 0.5 |  |
| CA_11-46 |  | 11 |  | 0 |  |
| CA_12-25 |  | 12 |  | 0 |  |
|  |  | 25 |  | 0 |  |
| CA_12-30 |  | 12 |  | 0 |  |
|  |  | 30 |  | 0 |  |
| CA_12-46 |  | 12 |  | 0 |  |
|  |  | 46 |  | 0 |  |
| CA_12-48 |  | 12 |  | 0 |  |
|  |  | 48 |  | 0 |  |
| CA_12-66, CA_12-66-66 |  | 12 |  | 0.5 |  |
|  |  | 66 |  | 0 |  |
| CA_13-46, CA_13-46-46 |  | 13 |  | 0 |  |
| CA_13-48, CA_13-48-48 |  | 13 |  | 0 |  |
|  |  | 48 |  | 0 |  |
| CA_13-66, CA_13-66-66 |  | 13 |  | 0 |  |
|  |  | 66 |  | 0 |  |
| CA_14-30 |  | 14 |  | 0 |  |
|  |  | 30 |  | 0 |  |
| CA_14-66, CA_14-66-66, CA_14-66-66-66 |  | 14 |  | 0 |  |
|  |  | 66 |  | 0 |  |
| CA_18-289 |  | 18 |  | 0 |  |
|  |  | 28 |  | 0 |  |
| CA_18-41 |  | 18 |  | 0 |  |
|  |  | 41 |  | 0 |  |
| CA_18-42 |  | 18 |  | 0 |  |
|  |  | 42 |  | 0.5 |  |
| CA_19-21 |  | 19 |  | 0 |  |
|  |  | 21 |  | 0 |  |
| CA_19-289 |  | 19 |  | 0 |  |
|  |  | 28 |  | 0 |  |
| CA_19-42 |  | 19 |  | 0 |  |
|  |  | 42 |  | 0.5 |  |
| CA_19-46 |  | 19 |  | 0 |  |
| CA_20-28 |  | 20 |  | 0 |  |
|  |  | 28 |  | 0 |  |
| CA_20-31 |  | 20 |  | 0 |  |
|  |  | 31 |  | 0 |  |
| CA_20-32 |  | 20 |  | 0 |  |
| CA_20-38 |  | 20 |  | 0 |  |
|  |  | 38 |  | 0 |  |
| CA_20-40, CA_20-40-40 |  | 20 |  | 0 |  |
|  |  | 40 |  | 0 |  |
| CA_20-41 |  | 20 |  | 0 |  |
|  |  | 41 |  | 0 |  |
| CA_20-42, CA_20-42-42 |  | 20 |  | 0 |  |
|  |  | 42 |  | 0.5 |  |
| CA_20-43 |  | 20 |  | 0 |  |
|  |  | 43 |  | 0.5 |  |
| CA_20-67 |  | 20 |  | 0 |  |
| CA_20-75 |  | 20 |  | 0 |  |
| CA_20-76 |  | 20 |  | 0 |  |
| CA_21-28 |  | 21 |  | 0 |  |
|  |  | 28 |  | 0 |  |
| CA_21-42 |  | 21 |  | 0 |  |
|  |  | 42 |  | 0.5 |  |
| CA_21-46 |  | 21 |  | 0 |  |
| CA_23-29 |  | 23 |  | 0 |  |
| CA_25-26, CA_25-25-26 |  | 25 |  | 0 |  |
|  |  | 26 |  | 0 |  |
| CA_25-41, CA_25-25-41 |  | 25 |  | 0 |  |
|  |  | 41 |  | 010 |  |
|  |  |  |  | 0.511 |  |
| CA_25-46 |  | 25 |  | 0 |  |
|  |  | 46 |  | 0 |  |
| CA_26-41 |  | 26 |  | 0 |  |
|  |  | 41 |  | 0 |  |
| CA_26-46 |  | 26 |  | 0 |  |
| CA_26-48, CA_26-48-48 |  | 26 |  | 0 |  |
|  |  | 48 |  | 0 |  |
| CA_26-66 |  | 26 |  | 0 |  |
|  |  | 66 |  | 0 |  |
| CA_28-32 |  | 28 |  | 0 |  |
| CA_28-38 |  | 28 |  | 0 |  |
|  |  | 38 |  | 0 |  |
| CA_28-40 |  | 28 |  | 0 |  |
|  |  | 40 |  | 0 |  |
| CA_28-41 |  | 28 |  | 0 |  |
|  |  | 41 |  | 0 |  |
| CA_28-42, CA_28-42-42 |  | 28 |  | 0.2 |  |
|  |  | 42 |  | 0.5 |  |
| CA_28-46 |  | 28 |  | 0 |  |
| CA_28-66 |  | 28 |  | 0.2 |  |
|  |  | 66 |  | 0 |  |
| CA_29-30 |  | 30 |  | 0 |  |
| CA_29-66, CA_29-66-66 |  | 66 |  | 0 |  |
| CA_29-70 |  | 70 |  | 0 |  |
| CA_30-66, CA_30-66-66 |  | 30 |  | 0.5 |  |
|  |  | 66 |  | 0.4 |  |
| CA_32-42 |  | 42 |  | 0.5 |  |
| CA_32-43 |  | 43 |  | 0.5 |  |
| CA_34-39 |  | 34 |  | 0.21 |  |
|  |  | 39 |  | 0.21 |  |
| CA_34-41 |  | 34 |  | 0.21 |  |
|  |  | 41 |  | 0.21 |  |
| CA_38-40, CA_38-40-40 |  | 38 |  | 0.54 |  |
|  |  | 40 |  | 0.54 |  |
| CA_39-40 |  | 39 |  | 0.34 |  |
|  |  | 40 |  | 0.34 |  |
| CA_39-41 |  | 39 |  | 0.24 |  |
|  |  | 41 |  | 0.24 |  |
| CA_39-41 |  | 39 |  | 0.27 |  |
|  |  | 41 |  | 0.27 |  |
| CA_39-42 |  | 39 |  | 04 |  |
|  |  | 42 |  | 0.54 |  |
| CA_39-46 |  | 39 |  | 0 |  |
| CA_40-41 |  | 40 |  | 04 |  |
|  |  | 41 |  | 04 |  |
| CA_40-42 |  | 40 |  | 0.44 |  |
|  |  | 42 |  | 0.54 |  |
| CA_40-43 |  | 40 |  | 0.44 |  |
|  |  | 43 |  | 0.54 |  |
| CA_40-46 |  | 40 |  | 0 |  |
| CA_41-42, CA_41-42-42 |  | 41 |  | 0.44 |  |
|  |  | 42 |  | 0.54 |  |
| CA_41-42, CA_41-42-42 |  | 41 |  | 07 |  |
|  |  | 42 |  | 0.57 |  |
| CA_41-46 |  | 41 |  | 0 |  |
| CA_41-48 |  | 41 |  | 04 |  |
|  |  | 48 |  | 0.54 |  |
| CA_42-43 |  | 42 |  | 04 |  |
|  |  | 43 |  | 04 |  |
| CA_42-46 |  | 42 |  | [0] |  |
| CA_46-48, CA_46-48-48 |  | 48 |  | 0.5 |  |
| CA_46-66, CA_46-66-66 |  | 66 |  | 0 |  |
| CA_46-70 |  | 70 |  | 0 |  |
| CA_46-71 |  | 71 |  | 0 |  |
| CA_48-66, CA_48-48-66, CA_48-66-66, CA_48-48-66-66 |  | 48 |  | 0.5 |  |
|  |  | 66 |  | 0.2 |  |
| CA_48-71, CA_48-48-71 |  | 48 |  | 0 |  |
|  |  | 71 |  | 0 |  |
| CA_66-70, CA_66-66-70 |  | 66 |  | 0 |  |
|  |  | 70 |  | 0 |  |
| CA_66-71, CA_66-66-71 |  | 66 |  | 0 |  |
|  |  | 71 |  | 0 |  |
| CA_70-71 |  | 70 |  | 0 |  |
|  |  | 71 |  | 0 |  |
| NOTE 1: The above additional tolerances are only applicable for the E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations NOTE 2: The above additional tolerances also apply in intra-band and non-aggregated operation for the supported E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations NOTE 3: In case the UE supports more than one of the above 2DL inter-band carrier aggregation configurations and a E-UTRA operating band belongs to more than one 2DL inter-band carrier aggregation configurations then: - When the E-UTRA operating band frequency range is ≤ 1GHz, the applicable additional tolerance shall be the average of the 2DL tolerances in Table 7.3.1-1A, truncated to one decimal place that would apply for that operating band among the supported 2DL CA configurations. In case there is a harmonic relation between low band UL and high band DL, then the maximum tolerance among the different supported 2DL carrier aggregation configurations involving such band shall be applied - When the E-UTRA operating band frequency range is >1GHz, the applicable additional tolerance shall be the maximum 2DL tolerance in Table 7.3.1-1A that would apply for that operating band among the supported 2DL CA configurations NOTE 4: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. NOTE 5: Unless otherwise specified, in case the UE supports more than one of the above 3DL inter-band carrier aggregation configurations and a E-UTRA operating band belongs to more than one 3DL inter-band carrier aggregation configurations then: - When the E-UTRA operating band frequency range is ≤ 1GHz and the tolerances are the same, the value applies to the band. If the tolerances are different, the applicable additional 3DL tolerance is FFS. In case there is a harmonic relation between low band UL and high band DL, then the maximum tolerance among the different supported 3DL carrier aggregation configurations involving such band shall be applied - When the E-UTRA operating band frequency range is >1GHz, the applicable additional 3DL tolerance shall be the maximum tolerance above that applies for that operating band among the supported 3DL CA configurations. NOTE 6: The above additional tolerances applicable for the E-UTRA operating bands that belong to the supported highest order inter-band carrier aggregation configuration, also applies to the same E-UTRA operating bands that belong to a supported lower order CA configuration. NOTE 7: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx. NOTE 8: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in the FDD band. NOTE 9: For Band 28, the requirements only apply for the restricted frequency range specified for this CA configuration (Table 5.5A-2). NOTE 10: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 11: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. NOTE 12: For UE supporting E-UTRA band 42, 43 or 48 and CA configurations including Band 42, 43 or 48, the applicable ΔRIB,c in Band 42, 43, or 48 is the max(Band 42 ΔRIB,c , Band 43 ΔRIB,c, Band 48 ΔRIB,c). NOTE 13: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 8. |  |  |  |  |  |

NOTE:	To meet the RIB,c requirements for CA_20A-28A state-of-the-art filter combiner technology is needed.

Table 7.3.1-1B: ΔRIB,c (three bands)

| E-UTRA operating band combination | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1-3-5, CA_1-1-3-5, CA_1-3-3-5 | 1 | 0 |
|  | 3 | 0 |
|  | 5 | 0 |
| CA_1-3-7, CA_1-1-3-7, CA_1-3-3-7, CA_1-3-7-7, CA_1-3-3-7-7 | 1 | 0 |
|  | 3 | 0 |
|  | 7 | 0 |
| CA_1-3-8, CA_1-3-3-8 | 1 | 0 |
|  | 3 | 0 |
|  | 8 | 0 |
| CA_1-3-11 | 1 | 0 |
|  | 3 | 0.3 |
|  | 11 | 0.5 |
| CA_1-3-18 | 1 | 0 |
|  | 3 | 0 |
|  | 18 | 0 |
| CA_1-3-19, CA_1-3-3-19 | 1 | 0 |
|  | 3 | 0 |
|  | 19 | 0 |
| CA_1-3-20, CA_1-3-3-20 | 1 | 0 |
|  | 3 | 0 |
|  | 20 | 0 |
| CA_1-3-21, CA_1-3-3-21 | 1 | 0 |
|  | 3 | 0.3 |
|  | 21 | 0.5 |
| CA_1-3-26 | 1 | 0 |
|  | 3 | 0 |
|  | 26 | 0 |
| CA_1-3-28, CA_1-1-3-28, CA_1-3-3-28, CA_1-1-3-28 | 1 | 0 |
|  | 3 | 0 |
|  | 28 | 0.2 |
| CA_1-3-32 | 1 | 0 |
|  | 3 | 0 |
|  | 32 | 0 |
| CA_1-3-38 | 1 | 0 |
|  | 3 | 0 |
|  | 38 | 0 |
| CA_1-3-40 | 1 | 0 |
|  | 3 | 0 |
|  | 40 | 0 |
| CA_1-3-41 | 1 | 0 |
|  | 3 | 0 |
|  | 41 | 05/0.56 |
| CA_1-3-42, CA_1-3-3-42 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 42 | 0.5 |
| CA_1-3-43 | 1 | 0 |
|  | 3 | 0 |
|  | 43 | 0.5 |
| CA_1-3-46 | 1 | 0 |
|  | 3 | 0 |
| CA_1-5-7, CA_1-5-7-7 | 1 | 0 |
|  | 5 | 0 |
|  | 7 | 0 |
| CA_1-5-28 | 1 | 0 |
|  | 5 | 0 |
|  | 28 | 0.2 |
| CA_1-5-40 | 1 | 0 |
|  | 5 | 0 |
|  | 40 | 0 |
| CA_1-5-41 | 1 | 0 |
|  | 5 | 0 |
|  | 41 | 0 |
| CA_1-5-46 | 1 | 0 |
|  | 5 | 0 |
| CA_1-7-8, CA_1-7-7-8 | 1 | 0 |
|  | 7 | 0 |
|  | 8 | 0.2 |
| CA_1-7-20, CA_1-7-7-20 | 1 | 0 |
|  | 7 | 0 |
|  | 20 | 0 |
| CA_1-7-26, CA_1-7-7-26 | 1 | 0 |
|  | 7 | 0 |
|  | 26 | 0 |
| CA_1-7-28 | 1 | 0 |
|  | 7 | 0 |
|  | 28 | 0.2 |
| CA_1-7-32 | 1 | 0 |
|  | 7 | 0 |
|  | 32 | 0 |
| CA_1-7-38 | 1 | 0 |
|  | 7 | 0 |
|  | 38 | 0.2 |
| CA_1-7-40 | 1 | 0 |
|  | 7 | 0.3 |
|  | 40 | 0.8 |
| CA_1-7-42 | 1 | 0.2 |
|  | 7 | 0.2 |
|  | 42 | 0.5 |
| CA_1-7-46 | 1 | 0 |
|  | 7 | 0 |
| CA_1-8-11 | 1 | 0 |
|  | 8 | 0 |
|  | 11 | 0 |
| CA_1-8-20 | 1 | 0 |
|  | 8 | 0 |
|  | 20 | 0 |
| CA_1-8-2810 | 1 | 0 |
|  | 8 | 0.2 |
|  | 28 | 0.2 |
| CA_1-8-38 | 1 | 0 |
|  | 8 | 0 |
|  | 38 | 0 |
| CA_1-8-40 | 1 | 0 |
|  | 8 | 0 |
|  | 40 | 0 |
| CA_1-8-42 | 1 | 0 |
|  | 8 | 0.2 |
|  | 42 | 0.5 |
| CA_1-11-18 | 1 | 0 |
|  | 11 | 0 |
|  | 18 | 0 |
| CA_1-11-28 | 1 | 0 |
|  | 11 | 0 |
|  | 28 | 0.2 |
| CA_1-11-42 | 1 | 0 |
|  | 11 | 0 |
|  | 42 | 0.5 |
| CA_1-18-28 | 1 | 0 |
|  | 18 | 0 |
|  | 28 | 0 |
| CA_1-18-41 | 1 | 0 |
|  | 18 | 0 |
|  | 41 | 0 |
| CA_1-18-42 | 1 | 0 |
|  | 18 | 0 |
|  | 42 | 0.5 |
| CA_1-19-21 | 1 | 0 |
|  | 19 | 0 |
|  | 21 | 0 |
| CA_1-19-28 | 1 | 0 |
|  | 19 | 0 |
|  | 28 | 0 |
| CA_1-19-42 | 1 | 0 |
|  | 19 | 0 |
|  | 42 | 0.5 |
| CA_1-20-28 | 1 | 0 |
|  | 20 | 0.2 |
|  | 28 | 0.2 |
| CA_1-20-32 | 1 | 0 |
|  | 20 | 0 |
|  | 32 | 0 |
| CA_1-20-38 | 1 | 0 |
|  | 20 | 0 |
|  | 38 | 0 |
| CA_1-20-42 | 1 | 0 |
|  | 20 | 0 |
|  | 42 | 0.5 |
| CA_1-20-43 | 1 | 0 |
|  | 20 | 0 |
|  | 43 | 0.5 |
| CA_1-21-28 | 1 | 0 |
|  | 21 | 0 |
|  | 28 | 0.2 |
| CA_1-21-42 | 1 | 0 |
|  | 21 | 0 |
|  | 42 | 0.5 |
| CA_1-28-40 | 1 | 0 |
|  | 28 | 0.2 |
|  | 40 | 0 |
| CA_1-28-42 | 1 | 0 |
|  | 28 | 0.2 |
|  | 42 | 0.5 |
| CA_1-32-42 | 1 | 0 |
|  | 42 | 0.5 |
| CA_1-32-43 | 1 | 0 |
|  | 43 | 0.5 |
| CA_1-41-427, 12 | 1 | 0 |
|  | 41 | 0 |
|  | 42 | 0.5 |
| CA_1-42-4313 | 1 | 0 |
|  | 42 | 0.5 |
|  | 43 | 0.5 |
| CA_2-4-5, CA_2-2-4-5, CA_2-4-4-5 | 2 | 0.3 |
|  | 4 | 0.3 |
|  | 5 | 0 |
| CA_2-4-7, CA_2-4-7-7 | 2 | 0.3 |
|  | 4 | 0.5 |
|  | 7 | 0.5 |
| CA_2-4-12, CA_2-2-4-12, CA_2-4-4-12, CA_2-4-12-12 | 2 | 0.3 |
|  | 4 | 0.3 |
|  | 12 | 0.5 |
| CA_2-4-13 | 2 | 0.3 |
|  | 4 | 0.3 |
|  | 13 | 0 |
| CA_2-4-28 | 2 | 0.3 |
|  | 4 | 0.3 |
|  | 28 | 0.5 |
| CA_2-4-29 | 2 | 0.3 |
|  | 4 | 0.3 |
| CA_2-4-30 | 2 | 0.4 |
|  | 4 | 0.4 |
|  | 30 | 0.5 |
| CA_2-4-71, CA_2-2-4-71 | 2 | 0.3 |
|  | 4 | 0.3 |
|  | 71 | 0 |
| CA_2-5-12, CA_2-2-5-12, CA_2-5-12-12 | 2 | 0 |
|  | 5 | 0.5 |
|  | 12 | 0.3 |
| CA_2-5-7 | 2 | 0 |
|  | 5 | 0 |
|  | 7 | 0 |
| CA_2-5-13 | 2 | 0 |
|  | 5 | 0 |
|  | 13 | 0 |
| CA_2-5-28 | 2 | 0 |
|  | 5 | 0.5 |
|  | 28 | 0.3 |
| CA_2-5-29 | 2 | 0 |
|  | 5 | 0 |
| CA_2-5-30, CA_2-2-5-30 | 2 | 0.4 |
|  | 5 | 0 |
|  | 30 | 0.5 |
| CA_2-5-46 | 2 | 0 |
|  | 5 | 0 |
| CA_2-5-66, CA_2-2-5-66, CA_2-5-66-66, CA_2-2-5-66-66 | 2 | 0.3 |
|  | 5 | 0 |
|  | 66 | 0.3 |
| CA_2-7-12, CA_2-2-7-12 | 2 | 0 |
|  | 7 | 0 |
|  | 12 | 0 |
| CA_2-7-13, CA_2-7-7-13 | 2 | 0 |
|  | 7 | 0 |
|  | 13 | 0 |
| CA_2-7-26 | 2 | 0 |
|  | 7 | 0 |
|  | 26 | 0 |
| CA_2-7-28 | 2 | 0 |
|  | 7 | 0 |
|  | 28 | 0 |
| CA_2-7-29, CA_2-7-7-29 | 2 | 0 |
|  | 7 | 0 |
| CA_2-7-30 | 2 | 0.4 |
|  | 7 | 0 |
|  | 30 | 0.5 |
| CA_2-7-46 | 2 | 0 |
|  | 7 | 0 |
| CA_2-7-66, CA_2-2-7-66, CA_2-7-7-66, CA_2-7-66-66 | 2 | 0.3 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
| CA_2-12-30, CA_2-2-12-30 | 2 | 0.4 |
|  | 12 | 0 |
|  | 30 | 0.5 |
| CA_2-12-66, CA_2-2-12-66, CA_2-2-12-66-66, CA_2-12-66-66 | 2 | 0.3 |
|  | 12 | 0.5 |
|  | 66 | 0.3 |
| CA_2-13-46 | 2 | 0 |
|  | 13 | 0 |
| CA_2-13-48, CA_2-13-48-48 | 2 | 0.2 |
|  | 13 | 0 |
|  | 48 | 0.5 |
| CA_2-13-66, CA_2-2-13-66, CA_2-13-66-66 | 2 | 0.3 |
|  | 13 | 0 |
|  | 66 | 0.3 |
| CA_2-14-30, CA_2-2-14-30 | 2 | 0.3 |
|  | 14 | 0 |
|  | 30 | 0.3 |
| CA_2-14-66, CA_2-2-14-66, CA_2-2-14-66-66, CA_2-14-66-66-66 | 2 | 0.3 |
|  | 14 | 0 |
|  | 66 | 0.3 |
| CA_2-26-66 | 2 | 0 |
|  | 26 | 0 |
|  | 66 | 0 |
| CA_2-28-66 | 2 | 0.3 |
|  | 28 | 0.2 |
|  | 66 | 0.3 |
| CA_2-29-30, CA_2-2-29-30 | 2 | 0.4 |
|  | 30 | 0.5 |
| CA_2-29-66 | 2 | 0.3 |
|  | 66 | 0.3 |
| CA_2-30-66, CA_2-2-30-66, CA_2-30-66-66 | 2 | 0.4 |
|  | 30 | 0.5 |
|  | 66 | 0.4 |
| CA_2-46-48 | 2 | 0.3 |
|  | 48 | 0.5 |
| CA_2-46-66, CA_2-46-46-66, CA_2-46-66-66 | 2 | 0 |
|  | 66 | 0 |
| CA_2-48-66, CA_2-48-48-66 | 2 | 0.3 |
|  | 48 | 0.5 |
|  | 66 | 0.3 |
| CA_2-66-71, CA_2-2-66-71, CA_2-66-66-71 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | 71 | 0 |
| CA_3-5-7, CA_3-5-7-7, CA_3-3-5-7 | 3 | 0 |
|  | 5 | 0 |
|  | 7 | 0 |
| CA_3-5-28 CA_3-3-5-28 | 3 | 0 |
|  | 5 | 0.1 |
|  | 28 | 0.1 |
| CA_3-5-40, CA_3-5-40-40 | 3 | 0 |
|  | 5 | 0 |
|  | 40 | 0 |
| CA_3-5-41 | 3 | 0 |
|  | 5 | 0 |
|  | 41 | 05 |
|  |  | 0.56 |
| CA_3-7-8,CA_3-3-7-8, CA_3-7-7-8, CA_3-3-7-7-8, | 3 | 0 |
|  | 7 | 0 |
|  | 8 | 0.2 |
| CA_3-7-20, CA_3-3-7-20, CA_3-7-7-20 | 3 | 0 |
|  | 7 | 0 |
|  | 20 | 0 |
| CA_3-7-26 | 3 | 0 |
|  | 7 | 0 |
|  | 26 | 0 |
| CA_3-7-28, CA_3-3-7-28 | 3 | 0 |
|  | 7 | 0 |
|  | 28 | 0 |
| CA_3-7-32 | 3 | 0 |
|  | 7 | 0 |
| CA_3-7-38 | 3 | 0 |
|  | 7 | 0 |
|  | 38 | 0.2 |
| CA_3-7-40 | 3 | 0 |
|  | 7 | 0.3 |
|  | 40 | 0.8 |
| CA_3-7-42 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 42 | 0.5 |
| CA_3-7-46 | 3 | 0 |
|  | 7 | 0 |
| CA_3-8-11 | 3 | 0.3 |
|  | 8 | 0 |
|  | 11 | 0.5 |
| CA_3-8-20 | 3 | 0 |
|  | 8 | 0 |
|  | 20 | 0 |
| CA_3-8-2811 | 3 | 0 |
|  | 8 | 0.2 |
|  | 28 | 0.1 |
| CA_3-8-32 | 3 | 0.3 |
|  | 8 | 0 |
|  | 32 | 0.5 |
| CA_3-8-38 | 3 | 0 |
|  | 8 | 0 |
|  | 38 | 0 |
| CA_3-8-40 | 3 | 0 |
|  | 8 | 0 |
|  | 40 | 0 |
| CA_3-8-42 | 3 | 0.2 |
|  | 8 | 0.2 |
|  | 42 | 0.5 |
| CA_3-11-18 | 3 | 0.3 |
|  | 11 | 0.5 |
|  | 18 | 0 |
| CA_3-11-26 | 3 | 0.3 |
|  | 11 | 0.5 |
|  | 26 | 0 |
| CA_3-11-28 | 3 | 0.3 |
|  | 11 | 0.5 |
|  | 28 | 0.2 |
| CA_3-18-42 | 3 | 0.2 |
|  | 18 | 0 |
|  | 42 | 0.5 |
| CA_3-19-21 | 3 | 0.3 |
|  | 19 | 0 |
|  | 21 | 0.5 |
| CA_3-19-42 | 3 | 0.2 |
|  | 19 | 0 |
|  | 42 | 0.5 |
| CA_3-20-28, CA_3-3-20-28 | 3 | 0 |
|  | 20 | 0.1 |
|  | 28 | 0.1 |
| CA_3-20-32 | 3 | 0 |
|  | 20 | 0 |
|  | 32 | 0 |
| CA_3-20-42 | 3 | 0.2 |
|  | 20 | 0 |
|  | 42 | 0.5 |
| CA_3-20-43 | 3 | 0 |
|  | 20 | 0 |
|  | 43 | 0.5 |
| CA_3-21-28 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | 28 | 0 |
| CA_3-21-42 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | 42 | 0.5 |
| CA_3-28-38 | 3 | 0 |
|  | 28 | 0 |
|  | 38 | 0.2 |
| CA_3-28-40 | 3 | 0 |
|  | 28 | 0 |
|  | 40 | 0 |
| CA_3-28-41 | 3 | 0 |
|  | 28 | 0 |
|  | 41 | 05/0.56 |
| CA_3-28-42, CA_3-28-42-42 | 3 | 0.2 |
|  | 28 | 0.2 |
|  | 42 | 0.5 |
| CA_3-32-42 | 3 | 0.2 |
|  | 32 | 0 |
|  | 42 | 0.5 |
| CA_3-32-43 | 3 | 0 |
|  | 32 | 0 |
|  | 43 | 0.5 |
| CA_3-32-46 | 3 | 0 |
|  | 32 | 0 |
| CA_3-41-4213 CA_3-41-42-42 | 3 | 0.5 |
|  | 41 | 05/0.56 |
|  | 42 | 0.5 |
| CA_3-42-4313 | 3 | 0.2 |
|  | 42 | 0.5 |
|  | 43 | 0.5 |
| CA_4-5-12, CA_4-4-5-12, CA_4-5-12-12 | 4 | 0 |
|  | 5 | 0.5 |
|  | 12 | 0.5 |
| CA_4-5-13 | 4 | 0 |
|  | 5 | 0 |
|  | 13 | 0 |
| CA_4-5-29 | 4 | 0 |
|  | 5 | 0 |
| CA_4-5-30, CA_4-4-5-30 | 4 | 0.4 |
|  | 5 | 0 |
|  | 30 | 0.5 |
| CA_4-7-12 | 4 | 0.5 |
|  | 7 | 0.5 |
|  | 12 | 0.5 |
| CA_4-7-28 | 4 | 0.5 |
|  | 7 | 0.5 |
|  | 28 | 0.2 |
| CA_4-12-30, CA_4-4-12-30 | 4 | 0.4 |
|  | 12 | 0.5 |
|  | 30 | 0.5 |
| CA_4-29-30, CA_4-4-29-30 | 4 | 0.4 |
|  | 30 | 0.5 |
| CA_5-7-28 | 5 | 0 |
|  | 7 | 0 |
|  | 28 | 0 |
| CA_5-7-46 | 5 | 0 |
|  | 7 | 0 |
| CA_5-7-66 | 5 | 0 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
| CA_5-12-46 | 5 | 0.5 |
|  | 12 | 0.3 |
| CA_5-12-48 | 5 | 0.5 |
|  | 12 | 0.3 |
|  | 48 | 0 |
| CA_5-12-66 | 5 | 0 |
|  | 12 | 0.5 |
|  | 66 | 0.5 |
| CA_5-30-66, CA_5-30-66-66 | 5 | 0 |
|  | 30 | 0.5 |
|  | 66 | 0.4 |
| CA_5-40-41 | 5 | 0 |
|  | 40 | 0 |
|  | 41 | 0 |
| CA_5-46-66, CA_5-46-66-66 | 5 | 0 |
|  | 66 | 0 |
| CA_5-48-66 | 5 | 0 |
|  | 48 | 0.5 |
|  | 66 | 0.2 |
| CA_7-8-20 | 7 | 0 |
|  | 8 | 0.2 |
|  | 20 | [0.2] |
| CA_7-8-38 | 7 | 0 |
|  | 8 | 0 |
|  | 38 | 0.2 |
| CA_7-8-40 | 7 | 0 |
|  | 8 | 0.2 |
|  | 40 | 0.5 |
| CA_7-12-66 | 7 | 0.5 |
|  | 12 | 0.5 |
|  | 66 | 0.5 |
| CA_7-13-66 | 7 | 0.5 |
|  | 13 | 0 |
|  | 66 | 0.5 |
| CA_7-20-28 | 7 | 0 |
|  | 20 | 0.2 |
|  | 28 | 0.2 |
| CA_7-20-32 | 7 | 0 |
|  | 20 | 0 |
|  | 32 | 0 |
| CA_7-20-38 | 7 | 0 |
|  | 20 | 0 |
|  | 38 | 0.2 |
| CA_7-20-42 | 7 | 0 |
|  | 20 | 0 |
|  | 42 | 0.5 |
| CA_7-26-66 | 7 | 0 |
|  | 26 | 0 |
|  | 66 | 0 |
| CA_7-28-38 | 7 | 0 |
|  | 28 | 0 |
|  | 38 | 0.2 |
| CA_7-28-40 | 7 | 0 |
|  | 28 | 0 |
|  | 40 | 0.5 |
| CA_7-29-66 | 7 | 0.5 |
|  | 66 | 0.5 |
| CA_7-30-66 | 7 | 0.5 |
|  | 30 | 0.5 |
|  | 66 | 0.5 |
| CA_7-32-46 | 7 | 0 |
|  | 32 | 0 |
| CA_7-46-66 | 7 | 0.5 |
|  | 66 | 0.5 |
| CA_8-11-2816 | 8 | 0.2 |
|  | 11 | 0 |
|  | 28 | 0.2 |
| CA_8-11-42 | 8 | 0.2 |
|  | 11 | 0 |
|  | 42 | 0.5 |
| CA_8-20-28 | 8 | 0 |
|  | 20 | 0 |
|  | 28 | 0 |
| CA_8-28-4114 | 8 | 0.2 |
|  | 28 | 0.1 |
|  | 41 | 0 |
| CA_8-39-41 | 8 | 0 |
|  | 39 | 0.217 |
|  | 41 | 0.217 |
| CA_12-30-66, CA_12-30-66-66 | 12 | 0.5 |
|  | 30 | 0.5 |
|  | 66 | 0.4 |
| CA_13-46-66 | 13 | 0 |
|  | 66 | 0 |
| CA_13-48-66, CA_13-48-48-66 | 13 | 0 |
|  | 48 | 0.5 |
|  | 66 | 0.2 |
| CA_14-30-66, CA_14-30-66-66 | 14 | 0 |
|  | 30 | 0.5 |
|  | 66 | 0.4 |
| CA_19-21-42 | 19 | 0 |
|  | 21 | 0 |
|  | 42 | 0.5 |
| CA_20-32-42 | 20 | 0 |
|  | 32 | 0 |
|  | 42 | 0.5 |
| CA_20-32-43 | 20 | 0 |
|  | 32 | 0 |
|  | 43 | 0.5 |
| CA_20-38-40, CA_20-38-40-4018 | 20 | 0 |
|  | 38 | 0.5 |
|  | 40 | 0.5 |
| CA_21-28-42 | 21 | 0 |
|  | 28 | 0.2 |
|  | 42 | 0.5 |
| CA_25-26-41, CA_25-25-26-41 | 25 | 0 |
|  | 26 | 0 |
|  | 41 | 0.5 |
| CA_28-41-429, CA_28-41-42-429 | 28 | 0.2 |
|  | 41 | 0.41 |
|  | 42 | 0.51 |
| CA_29-30-66, CA_29-30-66-66 | 30 | 0.5 |
|  | 66 | 0.4 |
| CA_29-46-66 | 66 | 0 |
| CA_29-66-70, CA_29-66-66-70 | 66 | 0 |
|  | 70 | 0 |
| CA_32-42-4313 | 32 | 0 |
|  | 42 | 0.5 |
|  | 43 | 0.5 |
| CA_46-48-66 | 48 | 0.5 |
|  | 66 | 0.3 |
| CA_46-48-71, CA_46-48-48-71 | 46 | 0 |
|  | 48 | 0.5 |
|  | 71 | 0 |
| CA_66-70-71, CA_66-66-70-71 | 66 | 0 |
|  | 70 | 0 |
|  | 71 | 0 |
| NOTE 1: The above additional tolerances are only applicable for the E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations. NOTE 2: The above additional tolerances also apply in intra-band and non-aggregated operation for the supported E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations. NOTE 3: Unless otherwise specified, in case the UE supports more than one of the above 3DL inter-band carrier aggregation configurations and a E-UTRA operating band belongs to more than one 3DL inter-band carrier aggregation configurations then: - When the E-UTRA operating band frequency range is ≤ 1GHz and the tolerances are the same, the value applies to the band. If the tolerances are different, the applicable additional 3DL tolerance is FFS. In case there is a harmonic relation between low band UL and high band DL, then the maximum tolerance among the different supported 3DL carrier aggregation configurations involving such band shall be applied - When the E-UTRA operating band frequency range is >1GHz, the applicable additional 3DL tolerance shall be the maximum tolerance above that applies for that operating band among the supported 3DL CA configurations. NOTE 4: The above additional tolerances applicable for the E-UTRA operating bands that belong to the supported highest order inter-band carrier aggregation configuration, also applies to the same E-UTRA operating bands that belong to a supported lower order CA configuration. NOTE 5: The requirement is specified for the frequency range of 2545-2690MHz. NOTE 6: The requirement is specified for the frequency range of 2496-2545MHz. NOTE 7: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 1 or Band 42. NOTE 8: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx on Band 41 and Band 42. NOTE 9: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx among TDD bands. NOTE 10: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 1 or Band 8. NOTE 11: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 3 or Band 8. NOTE 12: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx among TDD bands. NOTE 13: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx among TDD bands. NOTE 14: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 8 or Band 41. NOTE 15: For UE supporting E-UTRA band 42, 43 or 48 and CA configurations including Band 42, 43 or 48, the applicable ΔRIB,c in Band 42, 43, or 48 is the max(Band 42 ΔRIB,c , Band 43 ΔRIB,c, Band 48 ΔRIB,c). NOTE 16: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 8 or Band 11. NOTE17: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRAN band and without simultaneous Rx/Tx on Band 39 and Band 41. NOTE 18: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx among TDD bands. |  |  |

Table 7.3.1-1C: ΔRIB,c (four bands)

| E-UTRA operating band combination | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1-3-5-7, CA_1-3-3-5-7, CA_1-3-5-7-7 | 1 | 0 |
|  | 3 | 0 |
|  | 5 | 0 |
|  | 7 | 0 |
| CA_1-3-5-28 | 1 | 0 |
|  | 3 | 0 |
|  | 5 | 0.1 |
|  | 28 | 0.2 |
| CA_1-3-5-40 | 1 | 0 |
|  | 3 | 0 |
|  | 5 | 0 |
|  | 40 | 0 |
| CA_1-3-5-41 | 1 | 0 |
|  | 3 | 0 |
|  | 5 | 0 |
|  | 41 | 06 |
|  |  | 0.57 |
| CA_1-3-7-8, CA_1-3-3-7-8, CA_1-3-7-7-8, CA_1-3-3-7-7-8 | 1 | 0 |
|  | 3 | 0 |
|  | 7 | 0 |
|  | 8 | 0.2 |
| CA_1-3-7-20, CA_1-3-3-7-20, CA_1-3-7-7-20 | 1 | 0 |
|  | 3 | 0 |
|  | 7 | 0 |
|  | 20 | 0 |
| CA_1-3-7-26, CA_1-3-7-7-26 | 1 | 0 |
|  | 3 | 0 |
|  | 7 | 0 |
|  | 26 | 0 |
| CA_1-3-7-28, CA_1-1-3-7-28, CA_1-1-3-3-7-28, CA_1-3-3-7-28, CA_1-3-7-7-28 | 1 | 0 |
|  | 3 | 0 |
|  | 7 | 0 |
|  | 28 | 0.2 |
| CA_1-3-7-32 | 1 | 0 |
|  | 3 | 0 |
|  | 7 | 0 |
|  | 32 | 0 |
| CA_1-3-7-38 | 1 | 0 |
|  | 3 | 0 |
|  | 7 | 0 |
|  | 38 | 0 |
| CA_1-3-7-40 | 1 | 0 |
|  | 3 | 0 |
|  | 7 | 0.3 |
|  | 40 | 0.8 |
| CA_1-3-7-42 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | 7 | 0.3 |
|  | 42 | 0.5 |
| CA_1-3-7-46 | 1 | 0 |
|  | 3 | 0 |
|  | 7 | 0 |
|  | 46 | 0 |
| CA_1-3-8-11 | 1 | 0 |
|  | 3 | 0.3 |
|  | 8 | 0 |
|  | 11 | 0.5 |
| CA_1-3-8-20 | 1 | 0 |
|  | 3 | 0 |
|  | 8 | 0 |
|  | 20 | 0 |
| CA_1-3-8-288 | 1 | 0 |
|  | 3 | 0 |
|  | 8 | 0.2 |
|  | 28 | 0.2 |
| CA_1-3-8-38 | 1 | 0 |
|  | 3 | 0 |
|  | 8 | 0 |
|  | 38 | 0 |
| CA_1-3-11-28 | 1 | 0 |
|  | 3 | 0.3 |
|  | 11 | 0.5 |
|  | 28 | 0.2 |
| CA_1-3-8-40 | 1 | 0 |
|  | 3 | 0 |
|  | 8 | 0 |
|  | 40 | 0 |
| CA_1-3-8-42 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 8 | 0.2 |
|  | 42 | 0.5 |
| CA_1-3-18-42 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 18 | 0 |
|  | 42 | 0.5 |
| CA_1-3-19-21 | 1 | 0 |
|  | 3 | 0.3 |
|  | 19 | 0 |
|  | 21 | 0.5 |
| CA_1-3-19-42, CA_1-3-3-19-21 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 19 | 0 |
|  | 42 | 0.5 |
| CA_1-3-20-28, CA_1-3-3-20-28 | 1 | 0 |
|  | 3 | 0 |
|  | 20 | 0.2 |
|  | 28 | 0.2 |
| CA_1-3-20-32 | 1 | 0 |
|  | 3 | 0 |
|  | 20 | 0 |
|  | 32 | 0 |
| CA_1-3-20-42 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 20 | 0 |
|  | 42 | 0.5 |
| CA_1-3-20-43 | 1 | 0 |
|  | 3 | 0 |
|  | 20 | 0 |
|  | 43 | 0.5 |
| CA_1-3-21-28 | 1 | 0 |
|  | 3 | 0.3 |
|  | 21 | 0.5 |
|  | 28 | 0.2 |
| CA_1-3-21-42 | 1 | 0.2 |
|  | 3 | 0.3 |
|  | 21 | 0.5 |
|  | 42 | 0.5 |
| CA_1-3-28-40 | 1 | 0 |
|  | 3 | 0 |
|  | 28 | 0.2 |
|  | 40 | 0 |
| CA_1-3-28-42 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 28 | 0.2 |
|  | 42 | 0.5 |
| CA_1-3-32-42 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 32 | 0 |
|  | 42 | 0.5 |
| CA_1-3-32-43 | 1 | 0 |
|  | 3 | 0 |
|  | 32 | 0 |
|  | 43 | 0.5 |
| CA_1-3-41-42 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 41 | 0 |
|  | 42 | 0.5 |
| CA_1-3-42-4311 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 42 | 0.5 |
|  | 43 | 0.5 |
| CA_1-5-7-28 | 1 | 0 |
|  | 5 | 0.1 |
|  | 7 | 0 |
|  | 28 | 0.2 |
| CA_1-5-7-46 | 1 | 0 |
|  | 5 | 0 |
|  | 7 | 0 |
| CA_1-7-8-20 | 1 | 0 |
|  | 7 | 0 |
|  | 8 | 0.2 |
|  | 20 | 0.2 |
| CA_1-7-8-40 | 1 | 0 |
|  | 7 | 0.3 |
|  | 8 | 0.2 |
|  | 40 | 0.8 |
| CA_1-7-20-28 | 1 | 0 |
|  | 7 | 0 |
|  | 20 | 0.2 |
|  | 28 | 0.2 |
| CA_1-7-20-32 | 1 | 0 |
|  | 7 | 0 |
|  | 20 | 0 |
|  | 32 | 0 |
| CA_1-7-20-42 | 1 | 0.2 |
|  | 7 | 0.2 |
|  | 20 | 0.2 |
|  | 42 | 0.5 |
| CA_1-7-28-40 | 1 | 0 |
|  | 7 | 0.3 |
|  | 28 | 0.2 |
|  | 40 | 0.8 |
| CA_1-8-11-289 | 1 | 0 |
|  | 8 | 0.2 |
|  | 11 | 0 |
|  | 28 | 0.2 |
| CA_1-8-11-42 | 1 | 0 |
|  | 8 | 0.2 |
|  | 11 | 0 |
|  | 42 | 0.5 |
| CA_1-8-20-28 | 1 | 0 |
|  | 8 | 0.2 |
|  | 20 | 0.2 |
|  | 28 | 0.2 |
| CA_1-19-21-42 | 1 | 0 |
|  | 19 | 0 |
|  | 21 | 0 |
|  | 42 | 0.5 |
| CA_1-20-32-42 | 1 | 0 |
|  | 20 | 0 |
|  | 32 | 0 |
|  | 42 | 0.5 |
| CA_1-20-32-43 | 1 | 0 |
|  | 20 | 0 |
|  | 32 | 0 |
|  | 43 | 0.5 |
| CA_1-21-28-42 | 1 | 0 |
|  | 21 | 0 |
|  | 28 | 0.2 |
|  | 42 | 0.5 |
| CA_1-32-42-4311 | 1 | 0 |
|  | 32 | 0 |
|  | 42 | 0.5 |
|  | 43 | 0.5 |
| CA_2-4-5-12 | 2 | 0.3 |
|  | 4 | 0.3 |
|  | 5 | 0.5 |
|  | 12 | 0.5 |
| CA_2-4-5-29 | 2 | 0.3 |
|  | 4 | 0.3 |
|  | 5 | 0 |
| CA_2-4-5-30 | 2 | 0.4 |
|  | 4 | 0.4 |
|  | 5 | 0 |
|  | 30 | 0.5 |
| CA_2-4-7-12 | 2 | 0.3 |
|  | 4 | 0.3 |
|  | 7 | 0.5 |
|  | 12 | 0.5 |
| CA_2-4-12-30 | 2 | 0.4 |
|  | 4 | 0.4 |
|  | 12 | 0.5 |
|  | 30 | 0.5 |
| CA_2-4-29-30 | 2 | 0.4 |
|  | 4 | 0.4 |
|  | 30 | 0.5 |
| CA_2-5-7-28 | 2 | 0 |
|  | 5 | 0.2 |
|  | 7 | 0 |
|  | 28 | 0.2 |
| CA_2-5-12-66, CA_2-2-5-12-66 | 2 | 0.3 |
|  | 5 | 0.5 |
|  | 12 | 0.5 |
|  | 66 | 0.3 |
| CA_2-5-30-66, CA_2-2-5-30-66, CA_2-5-30-66-66 | 2 | 0.4 |
|  | 5 | 0 |
|  | 30 | 0.5 |
|  | 66 | 0.4 |
| CA_2-5-46-66, CA_2-5-46-66-66 | 2 | 0.3 |
|  | 5 | 0 |
|  | 66 | 0.3 |
| CA_2-7-12-66, CA_2-2-7-12-66 | 2 | 0.3 |
|  | 7 | 0.5 |
|  | 12 | 0.5 |
|  | 66 | 0.3 |
| CA_2-7-13-66 | 2 | 0.3 |
|  | 7 | 0.5 |
|  | 13 | 0 |
|  | 66 | 0.5 |
| CA_2-7-26-66 | 2 | 0.3 |
|  | 7 | 0.5 |
|  | 26 | 0 |
|  | 66 | 0.5 |
| CA_2-7-29-66, CA_2-7-7-29-66 | 2 | 0.3 |
|  | 7 | 0.5 |
|  | 66 | 0.5 |
| CA_2-7-46-66 | 2 | 0.3 |
|  | 7 | 0.5 |
|  | 46 | 0 |
|  | 66 | 0.5 |
| CA_2-12-30-66, CA_2-2-12-30-66, CA_2-12-30-66-66 | 2 | 0.4 |
|  | 12 | 0.5 |
|  | 30 | 0.5 |
|  | 66 | 0.4 |
| CA_2-13-46-66, CA_2-13-46-66-66 | 2 | 0.3 |
|  | 13 | 0 |
|  | 66 | 0.3 |
| CA_2-13-48-66, CA_2-13-48-48-66 | 2 | 0.3 |
|  | 13 | 0 |
|  | 48 | 0.5 |
|  | 66 | 0.3 |
| CA_2-14-30-66, CA_2-2-14-30-66, CA_2-14-30-66-66 | 2 | 0.4 |
|  | 14 | 0 |
|  | 30 | 0.5 |
|  | 66 | 0.4 |
| CA_2-29-30-66 | 2 | 0.4 |
|  | 30 | 0.5 |
|  | 66 | 0.4 |
| CA_2-46-48-66 | 2 | 0.3 |
|  | 48 | 0.5 |
|  | 66 | 0.3 |
| CA_3-5-7-28, CA_3-3-5-7-28 | 3 | 0 |
|  | 5 | 0.1 |
|  | 7 | 0 |
|  | 28 | 0.1 |
| CA_3-7-8-20 | 3 | 0 |
|  | 7 | 0 |
|  | 8 | 0 |
|  | 20 | 0 |
| CA_3-7-8-38 | 3 | 0 |
|  | 7 | 0 |
|  | 8 | 0 |
|  | 38 | 0.2 |
| CA_3-7-8-40 | 3 | 0 |
|  | 7 | 0.3 |
|  | 8 | 0.2 |
|  | 40 | 0.8 |
| CA_3-7-20-28 | 3 | 0 |
|  | 7 | 0 |
|  | 20 | 0.2 |
|  | 28 | 0.1 |
| CA_3-7-20-32 | 3 | 0 |
|  | 7 | 0 |
|  | 20 | 0 |
|  | 32 | 0 |
| CA_3-7-20-42 | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 20 | 0 |
|  | 42 | 0.5 |
| CA_3-7-28-38 | 3 | 0 |
|  | 7 | 0 |
|  | 28 | 0 |
|  | 38 | 0.2 |
| CA_3-7-28-40 | 3 | 0 |
|  | 7 | 0.3 |
|  | 28 | 0 |
|  | 40 | 0.8 |
| CA_3-7-32-46 | 3 | 0 |
|  | 7 | 0 |
|  | 32 | 0 |
| CA_3-8-11-2810 | 3 | 0.3 |
|  | 8 | 0.2 |
|  | 11 | 0.5 |
|  | 28 | 0.2 |
| CA_3-8-20-28 | 3 | 0 |
|  | 8 | 0.2 |
|  | 20 | 0.1 |
|  | 28 | 0.1 |
| CA_3-19-21-42 | 3 | 0.3 |
|  | 19 | 0 |
|  | 21 | 0.5 |
|  | 42 | 0.5 |
| CA_3-20-32-42 | 3 | 0.2 |
|  | 20 | 0 |
|  | 32 | 0 |
|  | 42 | 0.5 |
| CA_3-20-32-43 | 3 | 0 |
|  | 20 | 0 |
|  | 32 | 0 |
|  | 43 | 0.5 |
| CA_3-21-28-42 | 3 | 0.3 |
|  | 21 | 0.5 |
|  | 28 | 0.2 |
|  | 42 | 0.5 |
| CA_3-28-41-42 | 3 | 0.5 |
|  | 28 | 0.2 |
|  | 41 | 0.46/0.57 |
|  | 42 | 0.5 |
| CA_3-32-42-4311 | 3 | 0.2 |
|  | 32 | 0 |
|  | 42 | 0.5 |
|  | 43 | 0.5 |
| NOTE 1: The above additional tolerances are only applicable for the E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations. NOTE 2: The above additional tolerances also apply in intra-band and non-aggregated operation for the supported E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations. NOTE 3: Tolerances for a UE supporting multiple 4DL inter-band CA configurations are FFS. NOTE 4: The above additional tolerances applicable for the E-UTRA operating bands that belong to the supported highest order inter-band carrier aggregation configuration, also applies to the same E-UTRA operating bands that belong to a supported lower order CA configuration. NOTE 5: For UE supporting E-UTRA band 42, 43 or 48 and CA configurations including Band 42, 43 or 48, the applicable ΔRIB,c in Band 42, 43, or 48 is the max(Band 42 ΔRIB,c , Band 43 ΔRIB,c, Band 48 ΔRIB,c). NOTE 6: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 7: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. NOTE 8: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 1, Band 3 or Band 8. NOTE 9: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 1, Band 8 or Band 11. NOTE 10: Only applicable for UE supporting inter-band carrier aggregation with the uplink active in Band 3, Band 8 or Band 11. NOTE 11: Applicable for UE supporting inter-band carrier aggregation without simultaneous Rx/Tx among TDD bands. |  |  |

Table 7.3.1-1D: ΔRIB,c (five bands)

| E-UTRA operating band combination | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_1-3-5-7-28 | 1 | 0 |
|  | 3 | 0 |
|  | 5 | 0.1 |
|  | 7 | 0 |
|  | 28 | 0.2 |
| CA_1-3-7-8-20 | 1 | 0 |
|  | 3 | 0 |
|  | 7 | 0 |
|  | 8 | 0.2 |
|  | 20 | 0.2 |
| CA_1-3-7-20-28 | 1 | 0 |
|  | 3 | 0 |
|  | 7 | 0 |
|  | 20 | 0.2 |
|  | 28 | 0.2 |
| CA_1-3-7-20-32 | 1 | 0 |
|  | 3 | 0 |
|  | 7 | 0 |
|  | 20 | 0 |
|  | 32 | 0 |
| CA_1-3-7-20-42 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 7 | 0.2 |
|  | 20 | 0 |
|  | 42 | 0.5 |
| CA_1-3-8-11-28 | 1 | 0 |
|  | 3 | 0.3 |
|  | 8 | 0.2 |
|  | 11 | 0.5 |
|  | 28 | 0.2 |
| CA_1-3-20-32-42 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 20 | 0 |
|  | 32 | 0 |
|  | 42 | 0.5 |
| CA_1-3-20-32-43 | 1 | 0 |
|  | 3 | 0 |
|  | 20 | 0 |
|  | 32 | 0 |
|  | 43 | 0.5 |
| CA_1-3-32-42-43 | 1 | 0.2 |
|  | 3 | 0.2 |
|  | 32 | 0 |
|  | 42 | 0.5 |
|  | 43 | 0.5 |
| NOTE 1: The above additional tolerances are only applicable for the E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations. NOTE 2: The above additional tolerances also apply in intra-band and non-aggregated operation for the supported E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations. NOTE 3: Tolerances for a UE supporting multiple 5DL inter-band CA configurations are FFS. NOTE 4: The above additional tolerances applicable for the E-UTRA operating bands that belong to the supported highest order inter-band carrier aggregation configuration, also applies to the same E-UTRA operating bands that belong to a supported lower order CA configuration. |  |  |

NOTE :	The above additional tolerances do not apply to supported UTRA operating bands with frequency range below 1 GHz that correspond to the E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations when such bands are belonging only to band combination(s) where one band is <1GHz and other bands are >1.7GHz and there is no harmonic relationship between the low band UL and high band DL. Otherwise the above additional tolerances also apply to supported UTRA operating bands that correspond to the E-UTRA operating bands that belong to the supported inter-band carrier aggregation configurations.

Table 7.3.1-2: Uplink configuration for reference sensitivity

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| 1 |  |  | 25 | 50 | 75 | 100 | FDD |
| 2 | 6 | 15 | 25 | 50 | 501 | 501 | FDD |
| 3 | 6 | 15 | 25 | 50 | 501 | 501 | FDD |
| 4 | 6 | 15 | 25 | 50 | 75 | 100 | FDD |
| 5 | 6 | 15 | 25 | 251 |  |  | FDD |
| 6 |  |  | 25 | 251 |  |  | FDD |
| 7 |  |  | 25 | 50 | 75 | 751 | FDD |
| 8 | 6 | 15 | 25 | 251 |  |  | FDD |
| 9 |  |  | 25 | 50 | 501 | 501 | FDD |
| 10 |  |  | 25 | 50 | 75 | 100 | FDD |
| 11 |  |  | 25 | 251 |  |  | FDD |
| 12 | 6 | 15 | 201 | 201 |  |  | FDD |
| 13 |  |  | 201 | 201 |  |  | FDD |
| 14 |  |  | 151 | 151 |  |  | FDD |
| ... |  |  |  |  |  |  |  |
| 17 |  |  | 201 | 201 |  |  | FDD |
| 18 |  |  | 25 | 251 | 251 |  | FDD |
| 19 |  |  | 25 | 251 | 251 |  | FDD |
| 20 |  |  | 25 | 201 | 203 | 203 | FDD |
| 21 |  |  | 25 | 251 | 251 |  | FDD |
| 22 |  |  | 25 | 50 | 501 | 501 | FDD |
| 23 | 6 | 15 | 25 | 50 | 75 | 100 | FDD |
| 24 |  |  | 25 | 50 |  |  | FDD |
| 25 | 6 | 15 | 25 | 50 | 501 | 501 | FDD |
| 26 | 6 | 15 | 25 | 251 | 251 |  | FDD |
| 27 | 6 | 15 | 25 | 251 |  |  | FDD |
| 28 |  | 15 | 25 | 251 | 251 | 251 | FDD |
| 30 |  |  | 25 | 251 |  |  | FDD |
| 31 | 6 | 54 | 54 |  |  |  | FDD |
| … |  |  |  |  |  |  |  |
| 33 |  |  | 25 | 50 | 75 | 100 | TDD |
| 34 |  |  | 25 | 50 | 75 |  | TDD |
| 35 | 6 | 15 | 25 | 50 | 75 | 100 | TDD |
| 36 | 6 | 15 | 25 | 50 | 75 | 100 | TDD |
| 37 |  |  | 25 | 50 | 75 | 100 | TDD |
| 38 |  |  | 25 | 50 | 75 | 100 | TDD |
| 39 |  |  | 25 | 50 | 75 | 100 | TDD |
| 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| 41 |  |  | 25 | 50 | 75 | 100 | TDD |
| 42 |  |  | 25 | 50 | 75 | 100 | TDD |
| 43 |  |  | 25 | 50 | 75 | 100 | TDD |
| 44 |  | 15 | 25 | 50 | 75 | 100 | TDD |
| 45 |  |  | 25 | 50 | 75 | 100 | TDD |
| 48 |  |  | 25 | 50 | 75 | 100 | TDD |
| 50 |  | 15 | 25 | 50 | 75 | 100 | TDD |
| 51 |  | 15 | 25 |  |  |  | TDD |
| 52 |  |  | 25 | 50 | 75 | 100 | TDD |
| 53 | 6 | 15 | 25 | 50 |  |  |  |
| … |  |  |  |  |  |  |  |
| 65 | 6 | 15 | 25 | 50 | 75 | 100 | FDD |
| 66 | 6 | 15 | 25 | 50 | 75 | 100 | FDD |
| 68 |  |  | 25 | 251 | 251 |  | FDD |
| … |  |  |  |  |  |  |  |
| 70 |  |  | 25 | 50 | 75 |  | FDD |
| 71 |  |  | 25 | 251 | 201 | 201 | FDD |
| 72 | 6 | 54 | 54 |  |  |  | FDD |
| 73 | 6 | 54 | 54 |  |  |  | FDD |
| 74 | 6 | 15 | 25 | 251 | 251 | 251 | FDD |
| 85 |  |  | 201 | 201 |  |  | FDD |
| 87 | 6 | 54 | 54 |  |  |  | FDD |
| 88 | 6 | 54 | 54 |  |  |  | FDD |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1). NOTE 2: For the UE which supports both Band 11 and Band 21 the uplink configuration for reference sensitivity is FFS. NOTE 3: 3 refers to Band 20; in the case of 15MHz channel bandwidth, the UL resource blocks shall be located at RBstart 11 and in the case of 20MHz channel bandwidth, the UL resource blocks shall be located at RBstart 16 NOTE 4: 4 refers to Bands 31, 72, 73, 87 and 88; in the case of 3 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 9 and in the case of 5 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 10. |  |  |  |  |  |  |  |

Unless given by Table 7.3.1-3, the minimum requirements specified in Tables 7.3.1-1, 7.3.1-1a and 7.3.1-2 shall be verified with the network signalling value NS_01 (Table 6.2.4-1) configured.

Table 7.3.1-3: Network signalling value for reference sensitivity

| E-UTRA Band | Network Signalling value |
| --- | --- |
| 2 | NS_03 |
| 4 | NS_03 |
| 10 | NS_03 |
| 12 | NS_06 |
| 13 | NS_06 |
| 14 | NS_06 |
| 17 | NS_06 |
| 19 | NS_08 |
| 21 | NS_09 |
| 23 | NS_03 |
| 25 | NS_03 |
| 30 | NS_21 |
| 48 | NS_27 |
| 53 | NS_45 |
| 66 | NS_03 |
| 70 | NS_03 |
| 71 | NS_35 |
| 85 | NS_06 |

### 7.3.1A	Minimum requirements (QPSK) for CA

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one E-UTRA band the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.1-1, Table 7.3.1-1a and Table 7.3.1-2. The reference sensitivity is defined to be met with all downlink component carriers active and one of the uplink carriers active. The uplink resource blocks shall be located as close as possible to the primary downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1). The primary downlink operating band is the downlink band of the active uplink operating band. The UE shall meet the requirements specified in subclause 7.3.1 with the following exceptions.

For the bands supporting 4 antenna ports which are in Table 7.3.1-1a, the minimum requirements for reference sensitivity in the reference sensitivity exception tables shall be modified by the amount given in ΔRIB,4R in Table 7.3.1-1a for the applicable E-UTRA bands unless otherwise specified.

For the bands supporting 8 antenna ports which are in Table 7.3.1-1aa, the minimum requirements for reference sensitivity in the reference sensitivity exception tables shall be modified by the amount given in ΔRIB,8R in Table 7.3.1-1aa for the applicable E-UTRA bands unless otherwise specified.

For the UE that supports any of the E-UTRA CA configurations given in Table 7.3.1A-0a, exceptions to the requirements for a band(s) specified in subclause 7.3.1 are allowed when the band(s) is impacted by harmonic interference from the uplink transmission in a lower-frequency band of the same CA configuration. For these exceptions, the UE shall meet the requirements specified in Table 7.3.1A-0a and Table 7.3.1A-0b.

Table 7.3.1A-0a: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issue)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3A-5A-7A-28A5,6 | 133 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
| CA_1A-3A-5A-28A 5,6 | 133 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
| CA_1A-3A-5A-41A8,19,28 | 5 |  |  | N/A | N/A |  |  | FDD |
|  | 41 |  |  | N/A | N/A | N/A | N/A | TDD |
| CA_1A-3A-7A-8A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_1A-3A-7A-8A4,5,6 | 733 |  |  | -88 | -87.4 | -87 | -86.7 | FDD |
| CA_1A-3C-7A-8A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_1A-3C-7A-8A5,6 | 733 |  |  | -88 | -87.4 | -87 | -86.7 | FDD |
| CA_1A-3A-7A-8A-20A5,6 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_1A-3A-7A-8A-20A5,6 | 733 |  |  | -88 | -87.4 | -87 | -86.7 | FDD |
| CA_1A-3A-7A-20A-28A5,6 | 133 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
| CA_1A-3A-7A-20A-42A9,10 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_1A-3A-7A-20A-42A11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_1A-3A-7A-28A5,6 | 133 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
| CA_1A-3A-7A-42A9,10 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_1A-3A-7A-42A11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_1A-3A-8A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_1A-3A-8A-11A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_1A-3A-8A-20A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_1A-3A-8A-11A-28A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_1A-3A-8A-28A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_1A-3A-8A-38A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_1A-3A-11A-28A5,6 | 133 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
| CA_1A-3A-11A-28A9,10 | 11 |  |  | -75.2 | -75.2 |  |  | FDD |
| CA_1A-3A-8A-40A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_1A-3A-8A-42A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_1A-3A-8A-42A9,10 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_1A-3A-8A-42A11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_1A-3A-8A-42A12,13 | 4233 |  |  | -84.8 | -84.7 | -84.6 | -84.5 | TDD |
| CA_1A-3A-18A-42A9,10 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_1A-3A-18A-42A11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_1A-3A-19A-42A9,10 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_1A-3A-19A-42A11 | 42 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_1A-3A-20A-28A5,6 | 133 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
| CA_1A-3A-20A-32A-42A9,10 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_1A-3A-20A-32A-42A11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_1A-3A-20A-32A-42A12,13 | 4233 |  |  | -84.8 | -84.7 | -84.6 | -84.5 | TDD |
| CA_1A-3A-21A-28A4,21 | 21 |  |  | N/A | N/A | N/A |  | FDD |
| CA_1A-3A-21A-42A22,23 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_1A-3A-21A-42A24 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_1A-3A-28A-40A15,16 | 28 |  |  | -60.7 | -60.7 | -60.7 | -60.7 | FDD |
| CA_1A-3A-28A-42A22,23 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_1A-3A-28A-42A24 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_1A-3A-28A5,6 CA_1A-1A-3A-28A CA_1A-1A-3C-28A CA_1A-3A-3A-28A | 133 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
| CA_1A-3A-32A-42A9,10 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_1A-3A-32A-42A11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_1A-3A-42A-43A9,10 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_1A-3A-42A-43A11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_1A-3A-32A-42A-43A9,10 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_1A-3A-32A-42A-43A11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_1A-3A-41A-42A9,10 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_1A-3A-41A-42A11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_1A-3A-42A9,10 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_1A-3A-42A11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_1A-3A-3A-42A9,10 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_1A-3A-3A-42A11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_1A-3A-42A-42A9,10 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_1A-3A-42A-42A11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_1A-3A-42A-42C9,10 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_1A-3A-42A-42C11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_1A-3A-42C-42C9,10 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_1A-3A-42C-42C11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_1A-5A-7A-28A5,6 | 133 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
| CA_1A-5A-28A5,6 | 133 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
| CA_1A-5A-41A8,19 | 5 |  |  | N/A | N/A |  |  | FDD |
|  | 41 |  |  | N/A | N/A | N/A | N/A | TDD |
| CA_1A-7A-8A5,6, CA_1A-7A-7A-8A5,6 | 733 |  |  | -88 | -87.4 | -87 | -86.7 | FDD |
| CA_1A-7A-8A-20A | 133 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
| CA_1A-7A-8A-40A5,6 | 733 |  |  |  | -87.1 | -86.7 | -86.4 | FDD |
| CA_1A-7A-20A-28A5,6 | 133 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
| CA_1A-7A-28A5,6 | 133 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
| CA_1A-7A-28A-40A15,16 | 28 |  |  | -60.7 | -60.7 | -60.7 | -60.7 | FDD |
| CA_1A-8A-20A-28A5,6 | 133 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
| CA_1A-8A-42A12,13 CA_1A-8A-42C12,13 | 4233 |  |  | -84.8 | -84.7 | -84.6 | -84.5 | TDD |
| CA_1A-8A-11A-42A12,13 | 4233 |  |  | -84.8 | -84.7 | -84.6 | -84.5 | TDD |
| CA_1A-11A-28A5,6 | 133 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
| CA_1A-11A-28A9,10 | 11 |  |  | -75.2 | -75.2 |  |  | FDD |
| CA_1A-18A-28A14 | 1 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_1A-19A-28A14 | 133 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_1A-20A-28A | 133 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
| CA_1A-20A-32A-42A12,13 | 4233 |  |  | -84.8 | -84.7 | -84.6 | -84.5 | TDD |
| CA_1A-21A-28A4,21 | 21 |  |  | N/A | N/A | N/A |  | FDD |
| CA_1A-21A-28A-42A4,21 | 21 |  |  | N/A | N/A | N/A |  | FDD |
| CA_1A-28A5,6,14 | 133 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
| CA_1A-1A-28A5,6,14 | 1 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
| CA_1A-28A-40A15,16 CA_1A-28A-40C15,16 | 28 |  |  | -60.7 | -60.7 | -60.7 | -60.7 | FDD |
| CA_1A-28A-40A5,6 CA_1A-28A-40C5,6 | 133 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
| CA_1A-28A-42A5,6,17,18 | 133 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
|  | 4233 |  |  | -85.7 | -85.4 | -85.1 | -84.9 | TDD |
| CA_2A-46A15,16 | 2 |  |  | -70 | -67 | -65.2 | -64 | FDD |
| CA_2A-2A-46A15,16 | 2 |  |  | -70 | -67 | -65.2 | -64 | FDD |
| CA_2A-48A9,10 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_2A-48A11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_2A-71A36 | 2 |  |  | -93.4 | -94 | -92.5 | -91.4 | FDD |
| CA_2A-71A37 | 2 |  |  | -96.8 | -94 | -92.5 | -91.4 | FDD |
| CA_2A-71A15,16 | 71 |  |  | -70.4 | -70.4 | -70.4 | -70.4 | FDD |
| CA_2A-2A-71A36 | 2 |  |  | -93.4 | -94 | -92.5 | -91.4 | FDD |
| CA_2A-2A-71A37 | 2 |  |  | -96.8 | -94 | -92.5 | -91.4 | FDD |
| CA_2A-2A-71A15,16 | 71 |  |  | -70.4 | -70.4 | -70.4 | -70.4 | FDD |
| CA_2A-4A-12A5,6 | 433 |  |  | -90 | -89.5 | -89 | -88.5 | FDD |
| CA_2A-4A-28A5,6 | 433 |  |  | -90 | -89.5 | -89 | -88.5 | FDD |
| CA_2A-4A-71A36, CA_2A-2A-4A-71A36 | 2 |  |  | -93.1 | -93.7 | -92.2 | -91.1 | FDD |
| CA_2A-4A-71A37, CA_2A-2A-4A-71A37 | 2 |  |  | -96.5 | -93.7 | -92.2 | -91.1 | FDD |
| CA_2A-4A-71A15,16, CA_2A-2A-4A-71A15,16 | 71 |  |  | -70.4 | -70.4 | -70.4 | -70.4 | FDD |
| CA_2A-4A-5A-12A5,6 | 433 |  |  | -90 | -89.5 | -89 | -88.5 | FDD |
| CA_2A-4A-7A-12A5,6 | 433 |  |  | -90 | -89.5 | -89 | -88.5 | FDD |
| CA_2A-4A-12A-30A5,6 | 433 |  |  | -90 | -89.5 | -89 | -88.5 | FDD |
| CA_2A-5A-12A-66A 5,6 | 66 |  |  | -89.5 | -89 | -88.5 | -88 | FDD |
| CA_2A-7A-12A-66A5,6 | 6633 |  |  | -89.5 | -89 | -88.5 | -88 | FDD |
| CA_2A-12A-66A5,6 | 6633 |  |  | -89.5 | -89 | -88.5 | -88 | FDD |
| CA_2A-13A-48A9,10 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_2A-13A-48A11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_2A-13A-48A-66A9,10 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_2A-13A-48A-66A11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_2A-28A-66A5,6 | 66 |  |  | -89,5 | -88,9 | -88,5 | -88,2 | FDD |
| CA_2A-48A-48A10,31 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_2A-48A-48A11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_2A-48A-48C10,31 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_2A-48A-48C11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_2A-48C9,10 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_2A-48C11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_2A-48D9,10 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_2A-48D11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_2A-48A-66A10,32, CA_2A-48C-66A10,32 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_2A-48A-66A11, CA_2A-48C-66A11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_2A-66A-71A36 CA_2A-2A-66A-71A36 CA_2A-66A-66A-71A36 CA_2A-66C-71A36 | 2 |  |  | -93.1 | -93.7 | -92.2 | -91.1 | FDD |
| CA_2A-66A-71A37 CA_2A-2A-66A-71A37 CA_2A-66A-66A-71A37 CA_2A-66C-71A37 | 2 |  |  | -96.5 | -93.7 | -92.2 | -91.1 | FDD |
| CA_2A-66A-71A15,16 CA_2A-2A-66A-71A15,16 CA_2A-66A-66A-71A15,16 CA_2A-66C-71A15,16 | 71 |  |  | -70.4 | -70.4 | -70.4 | -70.4 | FDD |
| CA_3A-5A-41A8,19,28 | 5 |  |  | N/A | N/A |  |  | FDD |
|  | 41 |  |  | N/A | N/A | N/A | N/A | TDD |
| CA_3A-7A-8A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_3A-7A-8A5,6 | 733 |  |  | -88 | -87.4 | -87 | -86.7 | FDD |
| CA_3C-7A-8A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_3C-7A-8A5,6 | 733 |  |  | -88 | -87.4 | -87 | -86.7 | FDD |
| CA_3A-7A-8A-20A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_3A-7A-8A-20A5,6 | 733 |  |  | -88 | -87.4 | -87 | -86.7 | FDD |
| CA_1A-3A-8A-20A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_3A-7A-8A-38A 4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_3A-7A-8A-40A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_3A-7A-8A-40A5,6 | 733 |  |  |  | -87.1 | -86.7 | -86.4 | FDD |
| CA_3A-7A-20A-42A9,10 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-7A-20A-42A11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-7A-28A-40A15,16 | 28 |  |  | -60.7 | -60.7 | -60.7 | -60.7 | FDD |
| CA_3A-7A-42A9,10 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-7A-42A11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-8A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_3A-8A-11A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_3A-8A-11A-28A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_3A-8A-32A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_3A-8A-38A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_3C-8A-38A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_3A-8A-40A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_3A-8A-40C4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_3A-8A-42A4 CA_3A-8A-42C4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_3A-8A-42A9,10 CA_3A-8A-42C9,10 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-8A-42A11 CA_3A-8A-42C11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-8A-42A12,13 CA_3A-8A-42C12,13 | 4233 |  |  | -84.8 | -84.7 | -84.6 | -84.5 | TDD |
| CA_3A-11A-28A9,10 | 11 |  |  | -75.2 | -75.2 |  |  | FDD |
| CA_3A-18A-42A9,10 CA_3A-18A-42C9,10 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-18A-42A11 CA_3A-18A-42C11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-19A-21A-42A25,26 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-19A-21A-42A27 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-19A-21A-42C25,26 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-19A-21A-42C27 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-19A-42A9,10 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-19A-42A11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-20A-32A-42A9,10 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-20A-32A-42A11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-20A-32A-42A12,13 | 4233 |  |  | -84.8 | -84.7 | -84.6 | -84.5 | TDD |
| CA_3A-20A-42A9,10 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-20A-42A11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-21A-28A4,21 | 21 |  |  | N/A | N/A | N/A |  | FDD |
| CA_3A-21A-28A-42A9,10 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-21A-28A-42A11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-21A-28A-42C9,10 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-21A-28A-42C11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-21A-42A9,10 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-21A-42A11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-28A-40A15,16 | 28 |  |  | -60.7 | -60.7 | -60.7 | -60.7 | FDD |
| CA_3A-28A-41A-42A9,10,29 | 41 |  |  |  | -94.5 | -92.7 | -91.5 | TDD |
|  | 4233 |  |  |  | -71.7 | -71.7 | -71.7 |  |
| CA_3A-28A-41A-42A11,29 | 41 |  |  |  | -94.5 | -92.7 | -91.5 | TDD |
|  | 4233 |  |  |  | -94.7 | -93.2 | -92.5 |  |
| CA_3A-28A-41A-42A17,18, 29 | 41 |  |  |  | -94.5 | -92.7 | -91.5 | TDD |
|  | 4233 |  |  |  | -85.4 | -85.1 | -84.9 |  |
| CA_3A-28A-42A9,10 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-28A-42A11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-28A-42A17,18 | 4233 |  |  | -85.7 | -85.4 | -85.1 | -84.9 | TDD |
| CA_3A-31A12,13 | 333 |  |  | -86.9 | -86.4 | -86 | -85.6 | FDD |
| CA_3A-32A-42A9,10 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-32A-42A11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-42A9,10 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-42A11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-42A-42A9,10 | 4233 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-42A-42A11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-3A-42A11 | 4233 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-42A-43A9. 10 | 42 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-42A-43A11 | 42 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-32A-42A-43A9. 10 | 42 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-32A-42A-43A11 | 42 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_4A-5A-12A5,6 | 433 |  |  | -90 | -89.5 | -89 | -88.5 | FDD |
| CA_4A-7A-12A5,6 | 433 |  |  | -90 | -89.5 | -89 | -88.5 | FDD |
| CA_4A-7A -28A5,6 | 433 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
| CA_4A-12A5,6 | 433 | -89.2 | -89.2 | -90 | -89.5 | -89 | -88.5 | FDD |
| CA_4A-12A-30A5,6 | 433 |  |  | -90 | -89.5 | -89 | -88.5 | FDD |
| CA_4A-17A5,6 | 433 |  |  | -90 | -89.5 |  |  | FDD |
| CA_4A-28A5,6 | 433 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
| CA_5A-12A-66A5,6 | 6633 |  |  | -90 | -89.5 | -89 | -88.5 | FDD |
| CA_5A-38A8,19 CA_5A-41A8,19 CA_5A-40A-41A8,19 | 5 |  |  | N/A | N/A |  |  | FDD |
|  | 38,41 |  |  | N/A | N/A | N/A | N/A | TDD |
| CA_5A-48A-66A9,10 CA_5A-48A-66A-66A9,10 | 4833 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_5A-48A-66A11 CA_5A-48A-66A-66A11 | 4833 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_7A-8A5,6 | 733 |  |  | -88 | -87.4 | -87 | -86.7 | FDD |
| CA_7A-8A-20A5,6 | 733 |  |  |  | -87.4 | -87 | -86.7 | FDD |
| CA_7A-12A-66A5,6 | 6633 |  |  | -89.5 | -89 | -88.5 | -88 | FDD |
| CA_7A-12B-66A5,6 | 6633 |  |  | -89.5 | -89 | -88.5 | -88 | FDD |
| CA_7A-20A-38A8 | 38 |  |  | N/A | N/A | N/A | N/A | TDD |
| CA_7A-28A-40A15,16 CA_7A-28A-40C15,16 | 28 |  |  | -60.7 | -60.7 | -60.7 | -60.7 | FDD |
| CA_8A-28A-41A28 | 41 |  |  | N/A | N/A | N/A | N/A | TDD |
| CA_8A-39A-41A8 | 41 |  |  | N/A | N/A | N/A | N/A | TDD |
| CA_8A-41A8 | 41 |  |  | N/A | N/A | N/A | N/A | TDD |
| CA_8A-42A12,13 | 4233 |  |  | -84.8 | -84.7 | -84.6 | -84.5 | TDD |
| CA_8A_11A_42A12, 13 CA_8A_11A_42C12, 13 | 4233 |  |  | -84.8 | -84.7 | -84.6 | -84.5 | TDD |
| CA_11A-28A9,10 | 1133 |  |  | -75.2 | -75.2 |  |  | FDD |
| CA_12A-30A-66A5,6 | 6633 |  |  | -89.5 | -89 | -88.5 | -88 | FDD |
| CA_12A-66A5,6 | 6633 | -88.7 | -88.7 | -89.5 | -89 | -88.5 | -88 | FDD |
| CA_13A-48A-66A9,10 CA_13A-48A-66A-66A9,10 CA_13A-48A-66B9,10 CA_13A-48A-66C9,10 CA_13A-48A-48A-66A9,10 CA_13A-48C-66A9,10 CA_13A-48D-66A9,10 CA_13A-48A-48C-66A9,10 | 4833 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_13A-48A-66A11 CA_13A-48A-66A-66A11 CA_13A-48A-66B11 CA_13A-48A-66C11 CA_13A-48A-48A-66A11 CA_13A-48C-66A11 CA_13A-48D-66A11 CA_13A-48A-48C-66A11 CA_13A-48E-66A | 4833 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_18A-41A19 CA_18A-41C19 | 41 |  |  | N/A | N/A | N/A | N/A | TDD |
| CA_20A-32A-42A12,13 | 20 |  |  | -97 |  |  |  | FDD |
|  | 32 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 4233 |  |  | -84.8 | -84.7 | -84.6 | -84.5 | TDD |
| CA_20A-38A8 CA_20A-38C8 | 20 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_20A-38A-40D15,16 | 20 |  |  | -60.7 | -60.7 | -60.7 |  | FDD |
| CA_20A-40A15,16 | 2033 |  |  | -60.7 | -60.7 | -60.7 | -60.7 | FDD |
| CA_20A-40A-40A15,16 | 2033 |  |  | -60.7 | -60.7 | -60.7 |  | FDD |
| CA_20A-38A-40A15,16 | 2033 |  |  | -60.7 | -60.7 | -60.7 |  | FDD |
| CA_20A-40C15,16 | 2033 |  |  | -60.7 | -60.7 | -60.7 |  | FDD |
| CA_20A-40D15,16 | 20 |  |  | -60.7 | -60.7 | -60.7 |  | FDD |
| CA_20A-38A-40A-40A15,16 | 20 |  |  | -60.7 | -60.7 | -60.7 |  | FDD |
| CA_20A-38A-40C15,16 | 20 |  |  | -60.7 | -60.7 | -60.7 |  | FDD |
| CA_20A-42A12,13, CA_20A-42A-42A12,13 | 4233 |  |  | -84.8 | -84.7 | -84.6 | -84.5 | TDD |
| CA_20A-41A5,6 CA_20A-41C5,6 CA_20A-41D5,6 | 41 |  |  | -85.1 | -84.7 | -84.8 | -84.6 | TDD |
| CA_21A-28A4,21 | 21 |  |  | N/A | N/A | N/A |  | FDD |
| CA_21A-28A-42A4,21 | 21 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_25A-26A-41A38 | 41 |  |  | N/A | N/A | N/A | N/A | TDD |
| CA_26A-41A8,19 CA_25A-25A-26A-41A8,19 CA_25A-26A-41C8,19 | 26 |  |  | N/A | N/A | N/A |  | FDD |
|  | 41 |  |  | N/A | N/A | N/A | N/A | TDD |
| CA_28A-32A9,10 | 32 |  |  | -72.2 | -72.2 | -72.2 | -72.2 | FDD |
| CA_28A-32A11 | 32 |  |  | -97.6 | -95.2 | -93.7 | -93.0 | FDD |
| CA_28A-40A15,16 | 28 |  |  | -60.7 | -60.7 | -60.7 | -60.7 | FDD |
| CA_28A-41A-42A17,18 | 4233 |  |  |  | -85.4 | -85.1 | -84.9 | TDD |
| CA_28A-42A17,18 | 4233 |  |  | -85.7 | -85.4 | -85.1 | -84.9 | TDD |
| CA_28A-66A5,6 | 66 |  |  | -89.5 | -88.9 | -88.5 | -88.2 | FDD |
| CA_48A-66A9,10 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_48A-66A11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_48A-66B9,10 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_48A-66B11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_48A-66C9,10 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_48A-66C11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_48A-48A-66A9,10 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_48A-48A-66A11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_48A-66A-66A9,10 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_48A-66A-66A11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_48A-48A-66A-66A9,10 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_48A-48A-66A-66A11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_48A-48A-66B9,10 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_48A-48A-66B11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_48A-48A-66C9,10 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_48A-48A-66C11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_48C-66A-66A9,10 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
|  | 66 |  |  | -99.3 | -96.3 | -94.5 | -93.2 | FDD |
| CA_48C-66A-66A11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
|  | 66 |  |  | -99.3 | -96.3 | -94.5 | -93.2 | FDD |
| CA_48C-66B9,10 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_48C-66B11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_48C-66C9,10 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_48C-66C11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_48C-66A9,10 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_48C-66A11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_48E-66A 9,10 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_48E-66A 11 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_66A-70A-71A5,6 | 70 |  |  | -90 | -89.5 | -89.2 |  | FDD |
| CA_66A-66A-70A-71A5,6 | 70 |  |  | -90 | -89.5 | -89.2 |  | FDD |
| CA_66A-70C-71A5,6, 35 | 70 |  |  | -90 | -89.5 | -89.2 | -89 | FDD |
| CA_66A-66A-70C-71A5,6,35 | 70 |  |  | -90 | -89.5 | -89.2 | -89 | FDD |
| CA_66C-70A-71A5,6 | 70 |  |  | -90 | -89.5 | -89.2 |  | FDD |
| CA_66C-70C-71A5,6,35 | 70 |  |  | -90 | -89.5 | -89.2 | -89 | FDD |
| CA_70A-71A5,6 | 70 |  |  | -90 | -89.5 | -89.2 |  | FDD |
| CA_70C-71A5,6,35 | 70 |  |  | -90 | -89.5 | -89.2 | -89 | FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD/FS3 as described in Annex A.5.1.1/A.5.2.1/A.5.4.1. NOTE 3: The signal power is specified per port NOTE 4: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity for all active downlink component carriers is only verified when this is not the case (the requirements specified in clause 7.3.1 apply unless otherwise specified). NOTE 5: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 6: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 7: Void. NOTE 8: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity is only verified when this is not the case (the requirements specified in clause 7.3.1 apply). NOTE 9: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_3A-42A, CA_3A-3A-42A, CA_3A-42A-42A, CA_1A-3A-20A-32A-42A, CA_3A-42A-43A, CA_3A-32A-42A-43A, CA_1A-3A-42A, CA_2A-13A-48A-66A, CA_2A-48A, CA_2A-48C, CA_2A-48D, CA_48A-66A, CA_3A-7A-42A, CA_3A-19A-42A, CA_3A-20A-42A, CA_3A-28A-42A, CA_1A-3A-7A-42A, CA_5A-48A-66A, CA_5A-48A-66A-66A, CA_13A-48A-66A, CA_13A-48A-66A-66A, CA_13A-48A-66B, CA_13A-48A-66C, CA_13A-48A-48A-66A, CA_13A-48C-66A, CA_13A-48D-66A, CA_13A-48A-48C-66A, CA_28A-32A, CA_48A-66A-66A, CA_48A-66B , CA_48A-66C, CA_48A-48A-66A, CA_48C-66A, CA_48A-48A-66A-66A, CA_48A-48A-66B, CA_48A-48A-66C, CA_48C-66B, CA_48C-66C, CA_48E-66A, CA_1A-3A-18A-42A, CA_1A-3A-19A-42A, CA_1A-3A-32A-42A, CA_1A-3A-41A-42A, CA_3A-7A-20A-42A, CA_3A-20A-32A-42A, CA_3A-28A-41A-42A, CA_3A-18A-42A, CA_3A-18A-42C, CA_3A-8A-42A and CA_3A-8A-42C. FHD = 0MHz for CA_11A-28A, CA_1A-11A-28A and CA_3A-11A-28A. NOTE 10: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 11: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 12: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 4th transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 13: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 14: For the UE that supports CA_1A-18A-28A or CA_1A-19A-28A, no requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity should only be verified when this is not the case (the requirements specified in clause 7.3.1 apply). NOTE 15: These requirements apply when there is at least one individual RE within the downlink transmission bandwidth of the victim (lower) band for which the 3rd harmonic is within the uplink transmission bandwidth or the uplink adjacent channel’s transmission bandwidth of an aggressor (higher) band. NOTE 16: The requirements should be verified for UL EARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 17: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 5th transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 18: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 19: No requirements apply for the case that there is at least one individual RE within the uplink transmission bandwidth of the relative higher band and when the frequency range of relative higher band’s uplink channel bandwidth or uplink 1st adjacent channel bandwidth is fully or partially overlapped with the 3 times of the frequency range of the relative lower band’s downlink channel bandwidth. The reference sensitivity is only verified when this is not the case (the requirements specified in clause 7.3.1 apply). NOTE 20: Void NOTE 21: No requirements apply when there is at least one individual RE on band 28 uplink outside frequencies 728 – 738 MHz. The reference sensitivity is only verified when all configured RE’s are confined within frequencies 728 – 738 MHz (the requirements specified in clause 7.3.1 of [6] apply). NOTE 22: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_1A-3A-21A-42A. NOTE 23: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 24: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 25: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_3A-19A-21A-42A. NOTE 26: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 27: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 28: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of either Band 5 or Band 8 for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of Band 41. The reference sensitivity for all active downlink component carriers is only verified when this is not the case (the requirements specified in clause 7.3.1 apply). NOTE 29: The B41 requirements are modified by -0.1dB when carrier frequency of the assigned E-UTRA channel bandwidth is within 2545-2690 MHz. NOTE 30: Void NOTE 31: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_2A-48A-48A and CA_2A-48A-48C NOTE 32: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_2A-48A-66A, CA_2A-48A-66A-66A and CA_2A-48C-66A NOTE 33: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. NOTE 34: Void NOTE 35: These exceptions for the intra-band class C carriers should be verified one carrier at a time, according to note 6 frequency arrangements. No exceptions apply for the carrier which is not under REFSENS exception test. NOTE 36: These requirements apply when the lower edge frequency of the 5 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. NOTE 37: These requirements apply when the lower edge frequency of the 10 MHz, 15 MHz, or 20 MHz uplink channel in Band 71 is located at or below 668 MHz and the downlink channel in Band 2 is located with its upper edge at 1990 MHz. Note 38: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 3nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 15 MHz for CA_26A-41A, CA_25A-26A-41A. |  |  |  |  |  |  |  |  |

Table 7.3.1A-0b: Uplink configuration for the low band (exceptions due to harmonic issue)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-3A-5A-7A-28A | 28 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_1A-3A-5A-28A | 28 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_1A-3A-7A-8A | 8 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_1A-3C-7A-8A | 8 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_1A-3A-7A-8A-20A | 8 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_1A-3A-7A-20A-28A | 28 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_1A-3A-7A-20A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_1A-3A-7A-28A | 28 |  |  |  | 16 | 25 | 25 | FDD |
| CA_1A-3A-7A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_1A-3A-8A-42A | 8 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_1A-3A-8A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_1A-3A-11A-28A4 | 28 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_1A-3A-11A-28A5 | 28 |  |  | 12 | 25 |  |  | FDD |
| CA_1A-3A-18A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_1A-3A-19A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_1A-3A-28A CA_1A-1A-3A-28A CA_1A-3A-3A-28A | 28 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_1A-3A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_1A-3A-3A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_1A-3A-42A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_1A-3A-42A-42C | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_1A-3A-42C-42C | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_1A-3A-20A-28A | 28 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_1A-3A-21A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_1A-3A-28A-40A | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_1A-3A-28A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_1A-3A-41A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_1A-5A-7A-28A | 28 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_1A-7A-8A | 8 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_1A-3A-32A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_1A-3A-42A-43A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_1A-7A-8A-20A CA_1A-7A-7A-8A | 8 |  |  | 8 | 16 |  |  | FDD |
| CA_1A-7A-8A-40A | 8 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_1A-7A-20A-28A | 28 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_1A-7A-28A | 28 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_1A-7A-28A-40A | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_1A-8A-20A-28A | 28 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_1A-8A-42A CA_1A-8A-42C | 8 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_1A-8A-11A-42A | 8 |  |  | e | 16 | 25 | 25 | FDD |
| CA_1A-11A-28A4 | 28 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_1A-11A-28A5 | 28 |  |  | 12 | 25 |  |  | FDD |
| CA_1A-20A-28A | 28 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_1A-20A-32A-42A | 20 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_1A-3A-20A-32A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
|  | 20 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_1A-3A-32A-42A-43A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_1A-28A | 28 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_1A-1A-28A | 28 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_1A-28A-40A | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_1A-28A-42A | 28 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_2A-46A | 46 |  |  |  |  |  | 100 | TDD |
| CA_2A-2A-46A | 46 |  |  |  |  |  | 100 | TDD |
| CA_2A-71A | 71 |  |  | 25 | 251 | 201 | 201 | FDD |
| CA_2A-71A | 2 |  |  | 25 | 50 | 50 | 50 | FDD |
| CA_2A-2A-71A | 71 |  |  | 25 | 50 | 50 | 50 | FDD |
| CA_2A-4A-12A | 12 |  |  | 8 | 16 | 20 | 20 | FDD |
| CA_2A-4A-28A | 28 |  |  | 8 | 16 | 25 |  | FDD |
| CA_2A-4A-71A CA_2A-2A-4A-71A | 71 |  |  | 25 | 50 | 50 | 50 | FDD |
| CA_2A-4A-5A-12A | 12 |  |  | 8 | 16 |  |  | FDD |
| CA_2A-4A-7A-12A | 12 |  |  | 8 | 16 |  |  | FDD |
| CA_2A-4A-12A-30A | 12 |  |  | 8 | 16 |  |  | FDD |
| CA_2A-5A-12A-66A | 12 |  |  | 8 | 16 | 20 | 20 | FDD |
| CA_2A-7A-12A-66A | 12 |  |  | 8 | 16 |  |  | FDD |
| CA_2A-12A-66A | 12 |  |  | 8 | 16 |  |  | FDD |
| CA_2A-13A-48A | 2 |  |  | 25 | 50 | 501 | 501 | FDD |
| CA_2A-13A-48A-66A | 2 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 66 |  |  | 121 | 251 | 361 | 501 |  |
| CA_2A-48A CA_2A-48A-48A CA_2A-48A-48C CA_2A-48D | 2 |  |  | 25 | 50 | 501 | 501 | FDD |
| CA_2A-48A-66A CA_2A-48C-66A CA_2A-48A-66A-66A | 2 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 66 |  |  | 121 | 251 | 361 | 501 | FDD |
| CA_2A-66A-71A CA_2A-2A-66A-71A CA_2A-66A-66A-71A CA_2A-66C-71A | 71 |  |  | 25 | 50 | 50 | 50 | FDD |
| CA_3A-7A-8A | 8 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_3C-7A-8A | 8 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_3A-7A-20A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_3A-7A-8A-40A | 8 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_3A-7A-28A-40A | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3A-7A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_3A-8A-32A | 8 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_3A-8A-42A CA_3A-8A-42C | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_3A-8A-42A CA_3A-8A-42C | 8 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_3A-11A-28A | 28 |  |  | 12 | 25 |  |  | FDD |
| CA_3A-18A-42A CA_3A-18A-42C | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_3A-19A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_3A-19A-21A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_3A-20A-32A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
|  | 20 |  |  | 8 | 16 | 25 | 25 |  |
| CA_3A-19A-21A-42C | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_3A-20A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_3A-21A-28A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_3A-21A-28A-42C | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_3A-21A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_3A-28A-40A | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3A-28A-41A-42A | 3 |  |  |  | 25 | 36 | 50 | FDD |
| CA_3A-28A-41A-42A | 28 |  |  |  | 10 | 15 | 20 | FDD |
| CA_3A-31A | 31 |  |  | 5 | 5 | 5 | 5 | FDD |
| CA_3A-32A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_3A-32A-42A-43A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_3A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_3A-3A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_3A-42A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_3A-42A-43A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_4A-5A-12A | 12 |  |  | 8 | 16 | 20 | 20 | FDD |
| CA_4A-7A-12A | 12 |  |  | 8 | 16 | 20 | 20 | FDD |
| CA_4A-7A-28A | 28 |  |  | [8] | [16] | [25] | [25] | FDD |
| CA_4A-12A | 12 | 2 | 5 | 8 | 16 | 20 | 20 | FDD |
| CA_4A-12A-30A | 12 |  |  | 8 | 16 |  |  | FDD |
| CA_4A-17A | 17 |  |  | 8 | 16 |  |  | FDD |
| CA_4A-28A | 28 |  |  | [8] | [16] | [25] | [25] | FDD |
| CA_5A-12A-66A | 12 |  |  | 8 | 16 | 20 | 20 | FDD |
| CA_5A-48A-66A CA_5A-48A-66A-66A | 66 |  |  | 121 | 251 | 361 | 501 | FDD |
| CA_7A-8A | 8 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_7A-8A-20A | 8 |  |  |  | 16 | 25 | 25 | FDD |
| CA_7A-12A-66A | 12 |  |  | 8 | 16 |  |  | FDD |
| CA_7A-12B-66A | 12 |  |  | 8 | 16 |  |  | FDD |
| CA_7A-28A-40A | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_8A-42A | 8 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_8A_11A_42A CA_8A_11A_42C | 8 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_11A-28A | 28 |  |  | 12 | 25 |  |  | FDD |
| CA_12A-30A-66A | 12 |  |  | 8 | 16 |  |  | FDD |
| CA_12A-66A | 12 | 2 | 5 | 8 | 16 | 20 | 20 | FDD |
| CA_13A-48A-66A CA_13A-48A-66A-66A CA_13A-48A-66B CA_13A-48A-66C CA_13A-48A-48A-66A CA_13A-48C-66A CA_13A-48D-66A CA_13A-48A-48C-66A CA_13A-48E-66A | 66 |  |  | 121 | 251 | 361 | 501 | FDD |
| CA_20A-32A-42A | 20 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_20A-40A3 CA_20A-40D3 CA_20A-38A-40A-40A3 CA_20A-38A-40C3 | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_20A-40C3 | 40 |  |  | 25 | 50 | 75 |  | TDD |
| CA_20A-40A-40A3 | 40 |  |  | 25 | 50 | 75 |  | TDD |
| CA_20A-38A-40A3 | 40 |  |  | 25 | 50 | 75 |  | TDD |
| CA_20A-38A-40D3 | 40 |  |  | 25 | 50 | 75 |  | TDD |
| CA_20A-41A CA_20A-41C CA_20A-41D | 20 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_20A-42A, CA_20A-42A-42A | 20 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_28A-32A | 28 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_28A-40A | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_28A-41A-42A | 28 |  |  |  | 10 | 15 | 20 | FDD |
| CA_28A-42A | 28 |  |  | 5 | 10 | 15 | 20 | FDD |
| CA_28A-66A | 28 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_48A-66A | 66 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_48A-48A-66A | 66 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_48A-48A-66A-66A | 66 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_48A-48A-66B | 66 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_48A-48A-66C | 66 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_48C-66A-66A | 66 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_48C-66B | 66 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_48C-66C | 66 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_48A-66A-66A | 66 |  |  | 121 | 251 | 361 | 501 | FDD |
| CA_48A-66B | 66 |  |  | 121 | 251 | 361 | 501 | FDD |
| CA_48A-66C | 66 |  |  | 121 | 251 | 361 | 501 | FDD |
| CA_48C-66A | 66 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_48E-66A | 66 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_66A-70A-71A | 71 |  |  | 8 | 16 | 20 |  | FDD |
| CA_66A-66A-70A-71A | 71 |  |  | 8 | 16 | 20 |  | FDD |
| CA_66A-70C-71A | 71 |  |  | 8 | 16 | 20 | 20 | FDD |
| CA_66A-66A-70C-71A | 71 |  |  | 8 | 16 | 20 | 20 | FDD |
| CA_66C-70A-71A | 71 |  |  | 8 | 16 | 20 |  | FDD |
| CA_66C-70C-71A | 71 |  |  | 8 | 16 | 20 | 20 | FDD |
| CA_70A-71A | 71 |  |  | 8 | 16 | 20 |  | FDD |
| CA_70C-71A | 71 |  |  | 8 | 16 | 20 | 20 | FDD |
| NOTE 1: refers to the UL resource blocks, which shall be centred within the transmission bandwidth configuration for the channel bandwidth. NOTE 2: the UL configuration applies regardless of the channel bandwidth of the low band unless the UL resource blocks exceed that specified in Table 7.3.1-2 for the uplink bandwidth in which case the allocation according to Table 7.3.1-2 applies. NOTE 3: 3 refers to the UL resource blocks shall be located between 2373-2400MHz. NOTE 4: These configurations apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 5: These configurations apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a high band. |  |  |  |  |  |  |  |  |

For the UE that supports any of the E-UTRA CA configurations given in Table 7.3.1A-0bA, exceptions to the requirements for a band(s) specified in subclause 7.3.1 are allowed when the band(s) is impacted by the uplink being active within a specified frequency range as noted in Table 7.3.1A-0bA. For these exceptions, the UE shall meet the requirements specified in Table 7.3.1A-0bA and Table 7.3.1A-0bB.

Table 7.3.1A-0bA: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions for two bands due to close proximity of UL to DL channel)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3A4 | 39 |  | [-95.7] | -94 | -91.5 | -90 | -89 | FDD |
| CA_1A-3A5 | 3 |  | -98.7 | -97 | -94 | -92.2 | -91 | FDD |
| CA_1A-1A-3A4 | 39 |  |  | -94 | -91.5 | -90 | -89 | FDD |
| CA_1A-1A-3A5 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
| CA_1A-1A-3A-3A4 | 39 |  | [-95.7] | -94 | -91.5 | -90 | -89 | FDD |
| CA_1A-1A-3A-3A5 | 3 |  | -98.7 | -97 | -94 | -92.2 | -91 | FDD |
| CA_1A-3A-3A4 | 39 |  |  | -94 | -91.5 | -90 | -89 | FDD |
| CA_1A-3A-3A5 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
| CA_1A-3C4 | 39 |  |  | -94 | -91.5 | -90 | -89 | FDD |
| CA_1A-3C5 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
| CA_1A-1A-3C4 | 39 |  |  | -94 | -91.5 | -90 | -89 | FDD |
| CA_1A-1A-3C5 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
| CA_1C-3A4 | 39 |  |  | -94 | -91.5 | -90 | -89 | FDD |
| CA_1C-3A5 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
| CA_18A-28A6 | 28 |  |  | -94 | -92.5 |  |  | FDD |
| CA_19A-28A7 | 28 |  |  | -94 | -92 |  |  | FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: The signal power is specified per port NOTE 4: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in Band 3, the requirement applies regardless of channel bandwidth in Band 1. NOTE 5: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth in Band 3, the requirement applies regardless of channel bandwidth in Band 1. NOTE 6: These requirements apply when the uplink is active in Band 18 and the downlink channels in Band 28 are confined within the restricted frequency range specified for this CA configuration (Table 5.5A-2). For each channel bandwidth in Band 28, the requirement applies regardless of channel bandwidth in Band 18. NOTE 7: These requirements apply when the uplink is active in Band 19 and the downlink channels in Band 28 are allocated at the middle of the restricted frequency range specified for this CA configuration (Table 5.5A-2). For each channel bandwidth in Band 28, the requirement applies regardless of channel bandwidth in Band 19. NOTE 8: Void NOTE 9: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |

Table 7.3.1A-0bB: Uplink configuration for the uplink band (exceptions for two bands due to close proximity of UL to DL channel)

| E-UTRA Band / Channel bandwidth of the affected DL band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-3A1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-3A1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1A-1A-3A 1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |  |
| CA_1A-1A-3A 1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |  |
| CA_1A-1A-3A-3A1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |  |
| CA_1A-1A-3A-3A1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |  |
| CA_1A-3C1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-3C1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1A-1A-3C1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-1A-3C1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1C-3A1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1C-3A1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_18A-28A4 | 18 |  |  | 18 | 18 |  |  | FDD |
| CA_19A-28A4 | 19 |  |  | 18 | 18 |  |  | FDD |
| NOTE 1: refers to the UL resource blocks shall be located as close as possible to the downlink channel in Band 3 but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1) in the uplink channel in Band 1. NOTE 2: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz NOTE 3: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. NOTE 4: refers to the UL resource blocks shall be located as close as possible to the downlink channel in Band 28 but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1). |  |  |  |  |  |  |  |  |

For the UE that supports any of the E-UTRA CA configurations given in Table 7.3.1A-0bC, exceptions to the requirements for a band(s) specified in subclause 7.3.1 are allowed when the band(s) is impacted by the uplink being active within a specified frequency range as noted in Table 7.3.1A-0bC. For these exceptions, the UE shall meet the requirements specified in Table 7.3.1A-0bC and Table 7.3.1A-0bD.

Table 7.3.1A-0bC: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions for three bands due to close proximity of UL to DL channel)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3A-5A4 | 312 |  |  | -94 | -91.5 | -90 | -89 | FDD |
| CA_1A-3A-5A5 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
| CA_1A-1A-3A-5A4 | 312 |  |  | -94 | -91.5 | -90 | -89 | FDD |
| CA_1A-1A-3A-5A5 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
| CA_1A-1A-3C-5A4 | 312 |  |  | -94 | -91.5 | -90 | -89 | FDD |
| CA_1A-1A-3C-5A5 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
| CA_1A-3A-3A-5A4 | 39 |  |  | -94 | -91.5 | -90 | -89 | FDD |
| CA_1A-3A-3A-5A5 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
| CA_1C-3A-5A4 | 312 |  |  | -94 | -91.5 | -90 | -89 | FDD |
| CA_1C-3A-5A5 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
| CA_1A-3A-3A-7A-7A | 34,12 |  |  | -94 | -91.5 | -90 | -89 | FDD |
|  | 35 |  |  | -97 | -94 | -92.2 | -91 |  |
| CA_1A-3C-5A4 | 312 |  |  | -94 | -91.5 | -90 | -89 | FDD |
| CA_1A-3C-5A5 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
| CA_1A-3A-7A9 | 312 |  |  | -94 | -91.5 | -90 | -89 | FDD |
| CA_1A-3A-7A10 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
| CA_1A-1A-3A-7A9 | 312 |  |  | -94 | -91.5 | -90 | -89 | FDD |
| CA_1A-1A-3A-7A10 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
| CA_1A-3A-3A-7A | 39,12 |  |  | -94 | -91.5 | -90 | -89 | FDD |
|  | 310 |  |  | -97 | -94 | -92.2 | -91 |  |
| CA_1A-3A-7A-7A | 39,12 |  |  | -94 | -91.5 | -90 | -89 | FDD |
|  | 310 |  |  | -97 | -94 | -92.2 | -91 |  |
| CA_1A-3A-7C9 | 312 |  |  |  | -91.5 | -90 | -89 | FDD |
| CA_1A-3A-3A-7C9 | 312 |  |  | -94 | -91.5 | -90 | -89 | FDD |
| CA_1A-3A-7C10 | 3 |  |  |  | -94 | -92.2 | -91 | FDD |
| CA_1A-3A-3A-7C10 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
| CA_1A-3C-7A9 | 312 |  |  | -94 | -91.5 | -90 | -89 | FDD |
| CA_1A-3C-7A10 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
| CA_1A-1A-3C-7A9 | 312 |  |  | -94 | -91.5 | -90 | -89 | FDD |
| CA_1A-1A-3C-7A10 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
| CA_1A-3C-7C | 39,12 |  |  | -94 | -91.5 | -90 | -89 | FDD |
|  | 310 |  |  | -97 | -94 | -92.2 | -91 |  |
| CA_1A-3A-8A4 | 312 |  |  | -94 | -91.5 | -90 | -89 | FDD |
| CA_1A-3A-8A5 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
| CA_1A-3A-3A-8A4 | 312 |  |  | -94 | -91.5 | -90 | -89 | FDD |
| CA_1A-3A-3A-8A5 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
| CA_1A-3C-8A4 | 312 |  |  | -94 | -91.5 | -90 | -89 | FDD |
| CA_1A-3C-8A5 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
| CA_1A-3A-11A4 | 312 |  |  | -94 | -91.5 | -90 | -89 | FDD |
| CA_1A-3A-11A5 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
| CA_1A-3A-19A4 | 312 |  |  | -94 | -91.5 | -90 | -89 | FDD |
| CA_1A-3A-19A5 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
| CA_1A-3A-3A-19A4 | 312 |  |  | -94 | -91.5 | -90 | -89 | FDD |
| CA_1A-3A-3A-19A5 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
| CA_1A-3A-20A4 CA_1A-3A-3A-20A4 | 312 |  |  | -94 | -91.5 | -90 | -89 | FDD |
| CA_1A-3A-20A5 CA_1A-3A-3A-20A5 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
| CA_1A-3C-20A4 | 312 |  |  | -94 | -91.5 | -90 | -89 | FDD |
| CA_1A-3C-20A5 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
| CA_1A-3A-21A4 | 312 |  |  | -94 | -91.5 | -90 | -89 | FDD |
| CA_1A-3A-21A5 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
| CA_1A-3A-3A-21A4 | 312 |  |  | -94 | -91.5 | -90 | -89 | FDD |
| CA_1A-3A-3A-21A5 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
| CA_1A-3A-26A4 | 312 |  |  | -94 | -91.5 | -90 | -89 | FDD |
| CA_1A-3A-26A5 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
| CA_1A-3A-28A4 CA_1A-1A-3A-28A4 CA_1A-3A-3A-28A4 | 312 |  |  | -94 | -91.5 | -90 | -89 | FDD |
| CA_1A-3A-28A5 CA_1A-1A-3A-28A5 CA_1A-3A-3A-28A5 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
| CA_1A-3C-28A4 | 312 |  |  | -94 | -91.5 | -90 | -89 | FDD |
| CA_1A-3C-28A5 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
| CA_1A-3A-40A | 34,12 |  |  | -94 | -91.5 | -90 | -89 | FDD |
|  | 35 |  |  | -97 | -94 | -92.2 | -91 |  |
|  | 40 |  |  | [-93.4] | -91.3 | -90 | -88.9 | TDD |
| CA_1A-3A-40C | 34,12 |  |  | -94 | -91.5 | -90 | -89 | FDD |
|  | 35 |  |  | -97 | -94 | -92.2 | -91 |  |
|  | 40 |  |  | [-93.4] | -91.3 | -90 | -88.9 | TDD |
| CA_1A-3C-40A | 34,12 |  |  | -94 | -91.5 | -90 | -89 | FDD |
|  | 35 |  |  | -97 | -94 | -92.2 | -91 |  |
|  | 40 |  |  | [-93.4] | -91.3 | -90 | -88.9 | TDD |
| CA_1A-3C-40C | 34,12 |  |  | -94 | -91.5 | -90 | -89 | FDD |
|  | 35 |  |  | -97 | -94 | -92.2 | -91 |  |
|  | 40 |  |  | [-93.4] | -91.3 | -90 | -88.9 | TDD |
| CA_1A-3A-42A4 | 312 |  |  | -93.8 | -91.3 | -89.8 | -88.8 | FDD |
| CA_1A-3A-42A5 | 3 |  |  | -96.8 | -93.8 | -92 | -90.8 | FDD |
| CA_1A-3A-3A-42A4 | 312 |  |  | -93.8 | -91.3 | -89.8 | -88.8 | FDD |
| CA_1A-3A-3A-42A5 | 3 |  |  | -96.8 | -93.8 | -92 | -90.8 | FDD |
| CA_1A-3A-42A-42A4 | 312 |  |  | -93.8 | -91.3 | -89.8 | -88.8 | FDD |
| CA_1A-3A-42A-42A5 | 3 |  |  | -96.8 | -93.8 | -92 | -90.8 | FDD |
| CA_1A-3A-42A-42C4 | 312 |  |  | -93.8 | -91.3 | -89.8 | -88.8 | FDD |
| CA_1A-3A-42A-42C5 | 3 |  |  | -96.8 | -93.8 | -92 | -90.8 | FDD |
| CA_1A-3A-42C4 | 312 |  |  | -93.8 | -91.3 | -89.8 | -88.8 | FDD |
| CA_1A-3A-42C5 | 3 |  |  | -96.8 | -93.8 | -92 | -90.8 | FDD |
| CA_1A-3A-3A-42C4 | 312 |  |  | -94 | -91.5 | -90 | -89 | FDD |
| CA_1A-3A-3A-42C5 | 3 |  |  | -96.8 | -93.8 | -92 | -90.8 | FDD |
| CA_1A-3A-42C-42C4 | 312 |  |  | -93.8 | -91.3 | -89.8 | -88.8 | FDD |
| CA_1A-3A-42C-42C5 | 3 |  |  | -96.8 | -93.8 | -92 | -90.8 | FDD |
| CA_1A-3A-42D4 | 312 |  |  | -93.8 | -91.3 | -89.8 | -88.8 | FDD |
| CA_1A-3A-42D5 | 3 |  |  | -96.8 | -93.8 | -92 | -90.8 | FDD |
| CA_1A-18A-28A6 | 28 |  |  | -94 | -92.5 |  |  | FDD |
| CA_1A-19A-28A8 | 28 |  |  | -94 | -92 |  |  | FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: The signal power is specified per port NOTE 4: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in the Bands other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 5: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth in the Bands other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 6: These requirements apply when the uplink is active in Band 18 and the downlink channels in Band 28 are confined within the restricted frequency range specified for this CA configuration (Table 5.5A-2). For each channel bandwidth in Band 28, the requirement applies regardless of channel bandwidth in Band 18. NOTE 7: Void NOTE 8: These requirements apply when the uplink is active in Band 19 and the downlink channels in Band 28 are allocated at the middle of the restricted frequency range specified for this CA configuration (Table 5.5A-2). For each channel bandwidth in Band 28, the requirement applies regardless of channel bandwidth in Band 19. NOTE 9: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in Band 3 and Band 7, the requirement applies regardless of channel bandwidth in Band 1. NOTE 10: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth in Band 3 and Band 7, the requirement applies regardless of channel bandwidth in Band 1. NOTE 11: Void NOTE 12: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |

Table 7.3.1A-0bD: Uplink configuration for the uplink band (exceptions for three bands due to close proximity of UL to DL channel)

| E-UTRA Band / Channel bandwidth of the affected DL band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-3A-5A1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-3A-5A1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1A-1A-3A-5A1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-1A-3A-5A1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1C-3A-5A1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1C-3A-5A1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1A-3A-3A-7A-7A | 11,2 |  |  | 25 | 25 | 25 | 25 | FDD |
|  | 11,3 |  |  | 25 | 45 | 45 | 45 |  |
| CA_1A-3C-5A1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-3C-5A1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1A-3A-7A1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-3A-7A1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1A-1A-3A-7A1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-1A-3A-7A1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1A-1A-3C-7A1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-1A-3C-7A1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1A-3A-3A-7A1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-3A-3A-7C1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-3A-3A-7A1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1A-3A-3A-7C1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1A-3A-7A-7A1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-3A-7A-7A1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1A-3C-7A1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-3C-7A1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1A-3A-7C1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-3A-7C1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1A-3C-7C1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-3C-7C1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1A-3A-8A1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-3A-8A1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1A-3A-3A-8A1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-3A-3A-8A1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1A-3C-8A1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-3C-8A1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1A-3A-11A1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-3A-11A1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1A-3A-19A1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-3A-19A1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1A-3A-3A-19A1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-3A-3A-19A1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1A-3A-20A1, 2 CA_1A-3A-3A-20A1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-3A-20A1, 3 CA_1A-3A-3A-20A1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1A-3C-20A1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-3C-20A1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1A-3A-21A1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-3A-21A1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1A-3A-3A-21A1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-3A-3A-21A1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1A-3A-26A1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-3A-26A1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1A-3A-28A1, 2 CA_1A-1A-3A-28A1, 2 CA_1A-3A-3A-28A1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-3A-28A1, 3 CA_1A-1A-3A-28A1, 3 CA_1A-3A-3A-28A1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1A-3C-28A1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-3C-28A1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1A-3A-40A CA_1A-3A-40C CA_1A-3C-40A CA_1A-3C-40C | 11,2 |  |  | 25 | 25 | 25 | 25 | FDD |
|  | 11,3 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1A-3A-42A1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-3A-42A1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1A-3A-3A-42A1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-3A-3A-42A1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1A-3A-42A-42A1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-3A-42A-42A1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1A-3A-42A-42C1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-3A-42A-42C1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1A-3A-42C1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-3A-42C1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1A-3A-42C-42C1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-3A-42C-42C1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1A-3A-42D1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-3A-42D1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1A-3A-3A-42C1, 2 | 1 |  |  | 25 | 25 | 25 | 25 | FDD |
| CA_1A-3A-3A-42C1, 3 | 1 |  |  | 25 | 45 | 45 | 45 | FDD |
| CA_1A-18A-28A4 | 18 |  |  | 18 | 18 |  |  | FDD |
| CA_1A-19A-28A4 | 19 |  |  | 18 | 18 |  |  | FDD |
| NOTE 1: refers to the UL resource blocks shall be located as close as possible to the downlink channel in Band 3 but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1) in the uplink channel in Band 1. NOTE 2: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz NOTE 3: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. NOTE 4: refers to the UL resource blocks shall be located as close as possible to the downlink channel in Band 28 but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1). |  |  |  |  |  |  |  |  |

For the UE that supports any of the E-UTRA CA configurations given in Table 7.3.1A-0bD1, exceptions to the requirements for a band(s) specified in subclause 7.3.1 are allowed when the band(s) is impacted by the uplink  being active within a specified frequency range as noted in Table 7.3.1A-0bD1. For these exceptions, the UE shall meet the requirements specified in Table 7.3.1A-0bD1 and Table 7.3.1A-0bD2.

Table 7.3.1A-0bD1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions for four bands due to close proximity of UL to DL channel)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3A-5A-7A CA_1A-3A-3A-5A-7A CA_1A-3A-5A-7A-7A | 34,9 |  |  | -94 | -91.5 | -90 | -89 | FDD |
|  | 35 |  |  | -97 | -94 | -92.2 | -91 |  |
| CA_1A-3A-5A-28A | 34,9 |  |  | -94 | -91.5 | -90 | -89 | FDD |
|  | 35 |  |  | -97 | -94 | -92.2 | -91 |  |
| CA_1A-3A-5A-40A | 34,9 |  |  | -94 | -91.5 | -90 | -89 | FDD |
|  | 35 |  |  | -97 | -94 | -92.2 | -91 |  |
|  | 40 |  |  |  | -91.9 | -90.4 | -89.4 | TDD |
| CA_1A-3A-5A-41A7,8 | 34,9 |  |  | -94 | -91.5 | -90 | -89 | FDD |
|  | 35 |  |  | -97 | -94 | -92.2 | -91 |  |
|  | 41 |  |  |  |  |  | [88.1] | TDD |
| CA_1A-3C-7A-8A CA_1A-3A-3A-7A-8A CA_1A-3A-7A-7A-8A CA_1A-3A-3A-7A-7A-8A | 34,9 |  |  | -94 | -91.5 | -90 | -89 | FDD |
|  | 35 |  |  | -97 | -94 | -92.2 | -91 |  |
| CA_1A-3C-7A-20A | 34,9 |  |  | -94 | -91.5 | -90 | -89 | FDD |
|  | 35 |  |  | -97 | -94 | -92.2 | -91 |  |
| CA_1A-3A-7A-7A-26A | 34,9 |  |  |  | -91.5 | -90 | -89 | FDD |
|  | 35 |  |  | -97 | -94 | -92.2 | -91 |  |
| CA_1A-3A-7A-8A | 34,9 |  |  | -94 | -91.5 | -90 | -89 | FDD |
|  | 35 |  |  | -97 | -94 | -92.2 | -91 |  |
| CA_1A-3A-7A-20A | 34,9 |  |  | -94 | -91.5 | -90 | -89 | FDD |
|  | 35 |  |  | -97 | -94 | -92.2 | -91 |  |
| CA_1A-3A-3A-7A-20A CA_1A-3A-7A-7A-20A | 34,9 |  |  | -94 | -91.5 | -90 | -89 | FDD |
|  | 35 |  |  | -97 | -94 | -92.2 | -91 | FDD |
| CA_1A-3A-7A-26A | 34,9 |  |  |  | -91.5 | -90 | -89 | FDD |
|  | 35 |  |  | -97 | -94 | -92.2 | -91 |  |
| CA_1A-3A-7A-28A | 34,9 |  |  |  | [-91.5] | [-90] | [-89] | FDD |
|  | 35 |  |  |  | -94 | -92.2 | -91 |  |
| CA_1A-3A-7C-28A CA_1A-1A-3A-7A-28A CA_1A-1A-3A-7C-28A CA_1A-1A-3C-7A-28A CA_1A-1A-3C-7C-28A CA_1A-3A-3A-7C-28A CA_1A-1A-3A-3A-7A-28A CA_1A-1A-3A-3A-7C-28A | 34,9 |  |  |  | [-91.5] | [-90] | [-89] | FDD |
|  | 35 |  |  |  | -94 | -92.2 | -91 |  |
| CA_1A-3A-7A-38A CA_1A-3C-7A-38A | 3e,9 |  |  | -93.8 | -91.3 | -89.8 | -88.8 | FDD |
|  | 35 |  |  | -96.8 | -93.8 | -92 | -90.8 |  |
| CA_1A-3A-7A-42A | 34,9 |  |  | -93.8 | -91.3 | -89.8 | -88.8 | FDD |
|  | 35 |  |  | -96.8 | -93.8 | -92 | -90.8 |  |
| CA_1A-3A-8A-11A | 34,9 |  |  | -94 | -91.5 | -90 | -89 | FDD |
|  | 35 |  |  | -97 | -94 | -92.2 | -91 |  |
| CA_1A-3A-8A-28A | 34,9 |  |  | -94 | -91.5 | -90 | -89 | FDD |
|  | 35 |  |  | -97 | -94 | -92.2 | -91 |  |
| CA_1A-3A-8A-38A | 34,9 |  |  | -94 | -91.5 | -90 | -89 | FDD |
|  | 35 |  |  | -97 | -94 | -92.2 | -91 |  |
| CA_1A-3A-11A-28A | 34,9 |  |  | -94 | -91.5 | -90 | -89 | FDD |
|  | 35 |  |  | -97 | -94 | -92.2 | -91 |  |
| CA_1A-3A-8A-40A | 34,9 |  |  | -94 | -91.5 | -90 | -89 | FDD |
|  | 35 |  |  | -97 | -94 | -92.2 | -91 |  |
|  | 40 |  |  | [-93.4] | -91.9 | -90.4 | -89.4 | TDD |
| CA_1A-3A-8A-42A CA_1A-3A-8A-42C | 34,9 |  |  | -93.8 | -91.3 | -89.8 | -88.8 | FDD |
|  | 35 |  |  | -96.8 | -93.8 | -92 | -90.8 |  |
| CA_1A-3A-18A-42A CA_1A-3A-18A-42C | 34,9 |  |  | -93.8 | -91.3 | -89.8 | -88.8 | FDD |
|  | 35 |  |  | -96.8 | -93.8 | -92 | -90.8 |  |
| CA_1A-3A-19A-21A4 | 39 |  |  | -93.7 | -91.2 | -89.7 | -88.7 | FDD |
| CA_1A-3A-19A-21A5 | 3 |  |  | -96.7 | -93.7 | -91.9 | -90.7 | FDD |
| CA_1A-3A-3A-19A-21A4 | 39 |  |  | -93.7 | -91.2 | -89.7 | -88.7 | FDD |
| CA_1A-3A-3A-19A-21A5 | 3 |  |  | -96.7 | -93.7 | -91.9 | -90.7 | FDD |
| CA_1A-3A-7A-40C4 | 39 |  |  | -94 | -91.5 | -90 | -89 | FDD |
|  | 40 |  |  | -92.6 | -90.5 | -89.2 | -88.1 | TDD |
| CA_1A-3A-7A-40C5 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
|  | 40 |  |  | -92.6 | -90.5 | -89.2 | -88.1 | TDD |
| CA_1A-3A-19A-42A | 34,9 |  |  | -93.8 | -91.3 | -89.8 | -88.8 | FDD |
|  | 35 |  |  | -96.8 | -93.8 | -92 | -90.8 |  |
| CA_1A-3A-19A-42C | 34,9 |  |  | -93.8 | -91.3 | -89.8 | -88.8 | FDD |
|  | 35 |  |  | -96.8 | -93.8 | -92 | -90.8 |  |
| CA_1A-3A-20A-28A | 34,9 |  |  | -94 | -91.5 | -90 | -89 | FDD |
|  | 35 |  |  | -97 | -94 | -92.2 | -91 |  |
| CA_1A-3A-3A-20A-28A | 34,9 |  |  | -94 | -91.5 | -90 | -89 | FDD |
|  | 35 |  |  | -97 | -94 | -92.2 | -91 | FDD |
| CA_1A-3A-20A-42A | 34,9 |  |  | -93.8 | -91.3 | -89.8 | -88.8 | FDD |
|  | 35 |  |  | -96.8 | -93.8 | -92 | -90.8 |  |
| CA_1A-3A-20A-43A | 34,9 |  |  | -94 | -91.5 | -90 |  | FDD |
|  | 35 |  |  | -97 | -94 | -92.2 |  |  |
| CA_1A-3A-21A-28A4 | 39 |  |  | -93.7 | -91.2 | -89.7 | -88.7 | FDD |
| CA_1A-3A-21A-28A5 | 3 |  |  | -96.7 | -93.7 | -91.9 | -90.7 | FDD |
| CA_1A-3A-21A-42A4 | 39 |  |  | -93.7 | -91.2 | -89.7 | -88.7 | FDD |
| CA_1A-3A-21A-42A5 | 3 |  |  | -96.7 | -93.7 | -91.9 | -90.7 | FDD |
| CA_1A-3A-21A-42C | 34,9 |  |  | -93.7 | -91.2 | -89.7 | -88.7 | FDD |
|  | 35 |  |  | -96.7 | -93.7 | -91.9 | -90.7 |  |
| CA_1A-3A-28A-40A | 34,9 |  |  | -94 | -91.5 | -90 | -89 | FDD |
|  | 35 |  |  | -97 | -94 | -92.2 | -91 |  |
|  | 40 |  |  | [-93.4] | -91.9 | -90.4 | -89.4 | TDD |
| CA_1A-3A-28A-40C | 34,9 |  |  | -94 | -91.5 | -90 | -89 | FDD |
|  | 35 |  |  | -97 | -94 | -92.2 | -91 |  |
|  | 40 |  |  | -92.6 | -90.5 | -89.2 | -88.1 | TDD |
| CA_1A-3A-28A-42A4 | 39 |  |  | -93.8 | -91.3 | -89.8 | -88.8 | FDD |
| CA_1A-3A-28A-42A5 | 3 |  |  | -96.8 | -93.8 | -92 | -90.8 | FDD |
| CA_1A-3A-28A-42C | 34,9 |  |  | -93.8 | -91.3 | -89.8 | -88.8 | FDD |
|  | 35 |  |  | -96.8 | -93.8 | -92 | -90.8 |  |
| CA_1A-3A-32A-42A | 34,9 |  |  | -93.8 | -91.3 | -89.8 |  | FDD |
|  | 35 |  |  | -96.8 | -93.8 | -92 |  |  |
| CA_1A-3A-32A-43A | 34,9 |  |  | -94 | -91.5 | -90 |  | FDD |
|  | 35 |  |  | -97 | -94 | -92.2 |  |  |
| CA_1A-3A-41A-42A CA_1A-3A-41C-42A CA_1A-3A-41A-42C CA_1A-3A-41C-42C | 34,9 |  |  | -93.8 | -91.3 | -89.8 | -88.8 | FDD |
|  | 35 |  |  | -96.8 | -93.8 | -92 | -90.8 |  |
| CA_1A-3A-42A-43A | 34,9 |  |  | -93.8 | -91.3 | -89.8 |  | FDD |
|  | 35 |  |  | -96.8 | -93.8 | -92 |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: The signal power is specified per port NOTE 4: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in the bands other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 5: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth in the bands other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 6: Void NOTE 7: The Band 41 requirements are modified by -0.5dB when carrier frequency of the assigned E-UTRA channel bandwidth is within 2545-2690MHz. NOTE 8: The Band 41 requirements also apply to the supported CA_1A-41A. NOTE 9: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |

Table 7.3.1A-0bD2: Uplink configuration for the low band (exceptions for four bands due to close proximity of UL to DL channel)

| E-UTRA Band / Channel bandwidth of the affected DL band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-3A-5A-7A CA_1A-3A-3A-5A-7A CA_1A-3A-5A-7A-7A CA_1A-3A-5A-28A CA_1A-3A-5A-40A CA_1A-3A-5A-41A CA_1A-3A-7A-7A-26A CA_1A-3A-7A-8A CA_1A-3C-7A-8A CA_1A-3A-3A-7A-8A CA_1A-3A-7A-7A-8A CA_1A-3A-3A-7A-7A-8A CA_1A-3A-7A-20A CA_1A-3C-7A-20A CA_1A-3A-3A-7A-20A CA_1A-3A-7A-7A-20A CA_1A-3A-7A-26A CA_1A-3A-7A-28A CA_1A-3A-7C-28A CA_1A-1A-3A-7A-28A CA_1A-1A-3A-7C-28A CA_1A-1A-3C-7A-28A CA_1A-1A-3C-7C-28A CA_1A-3A-3A-7C-28A CA_1A-1A-3A-3A-7A-28A CA_1A-1A-3A-3A-7C-28A CA_1A-3A-7A-38A CA_1A-3C-7A-38A CA_1A-3A-7A-40C CA_1A-3A-7A-42A CA_1A-3A-8A-11A CA_1A-3A-8A-28A CA_1A-3A-8A-38A CA_1A-3A-11A-28A CA_1A-3A-8A-40A CA_1A-3A-8A-42A CA_1A-3A-8A-42C CA_1A-3A-18A-42A CA_1A-3A-18A-42C CA_1A-3A-19A-21A CA_1A-3A-3A-19A-21A CA_1A-3A-19A-42A CA_1A-3A-19A-42C CA_1A-3A-20A-28A CA_1A-3A-3A-20A-28A CA_1A-3A-20A-42A CA_1A-3A-20A-43A CA_1A-3A-21A-28A CA_1A-3A-21A-42A CA_1A-3A-21A-42C CA_1A-3A-28A-40A CA_1A-3A-28A-40C CA_1A-3A-28A-42A CA_1A-3A-32A-42A CA_1A-3A-32A-43A CA_1A-3A-41A-42A CA_1A-3A-41C-42A CA_1A-3A-41A-42C CA_1A-3A-41C-42C CA_1A-3A-42A-43A | 11,2 |  |  | 25 | 25 | 25 | 25 | FDD |
|  | 11,3 |  |  | 25 | 45 | 45 | 45 |  |
| NOTE 1: refers to the UL resource blocks shall be located as close as possible to the downlink channel in Band 3 but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1) in the uplink channel in Band 1. NOTE 2: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz NOTE 3: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. |  |  |  |  |  |  |  |  |

For the UE that supports any of the E-UTRA CA configurations given in Table 7.3.1A-0bD3, exceptions are allowed when the uplink is active within a specified frequency range as noted in Table 7.3.1A-0bD3. For these exceptions, the UE shall meet the requirements specified in Table 7.3.1A-0bD3 and Table 7.3.1A-0bD4.

Table 7.3.1A-0bD3: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions for five bands due to close proximity of UL to DL channel)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3A-5A-7A-28A | 34,9 |  |  | -94 | -91.5 | -90 | -89 | FDD |
|  | 35 |  |  | -97 | -94 | -92.2 | -91 |  |
| CA_1A-3A-7A-8A-20A | 34,7 |  |  | -94 | -91.5 | -90 | -89 | FDD |
|  | 35 |  |  | -97 | -94 | -92.2 | -91 |  |
| CA_1A-3A-7A-20A-28A | 34,7 |  |  | -94 | -91.5 | -90 | -89 | FDD |
|  | 35 |  |  | -97 | -94 | -92.2 | -91 |  |
| CA_1A-3A-7A-20A-42A | 34,7 |  |  | -93.8 | -91.3 | -89.8 | -88.8 | FDD |
|  | 35 |  |  | -96.8 | -93.8 | -92 | -90.8 |  |
| CA_1A-3A-20A-32A-42A | 34,7 |  |  | -93.8 | -91.3 | -89.8 |  | FDD |
|  | 35 |  |  | -96.8 | -93.8 | -92 |  |  |
| CA_1A-3A-20A-32A-43A | 34,7 |  |  | -94 | -91.5 | -90 |  | FDD |
|  | 35 |  |  | -97 | -94 | -92.2 |  |  |
| CA_1A-3A-32A-42A-43A | 34,7 |  |  | -93.8 | -91.3 | -89.8 |  | FDD |
|  | 35 |  |  | -96.8 | -93.8 | -92 |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: The signal power is specified per port NOTE 4: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in the bands other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 5: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth in the bands other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 6: Void NOTE 7: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |

Table 7.3.1A-0bD4: Uplink configuration for the low band (exceptions for five bands due to close proximity of UL to DL channel)

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-3A-5A-7A-28A CA_1A-3A-7A-8A-20A CA_1A-3A-7A-20A-28A CA_1A-3A-7A-20A-42A CA_1A-3A-20A-32A-42A CA_1A-3A-20A-32A-43A CA_1A-3A-32A-42A-43A | 11,2 |  |  | 25 | 25 | 25 | 25 | FDD |
|  | 11,3 |  |  | 25 | 45 | 45 | 45 |  |
| NOTE 1: refers to the UL resource blocks shall be located as close as possible to the downlink channel in Band 3 but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1) in the uplink channel in Band 1. NOTE 2: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. NOTE 3: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. |  |  |  |  |  |  |  |  |

For the UE that supports any of the E-UTRA CA configurations given in Table 7.3.1A-0bE, exceptions to the requirements for a band(s) specified in subclause 7.3.1 are allowed when the band(s) is impacted by uplink being active in the applicable active UL bands of the same CA configuration in Table 7.3.1A-0bE. For these exceptions, the UE shall meet the reference sensitivities specified in Table 7.3.1A-0bE and Table 7.3.1A-0bF.

Table 7.3.1A-0bE: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to cross band isolation issues of TDD and FDD bands)

| EUTRA CA Configuration | EUTRA band | Channel bandwidth |  |  |  |  |  | Duplex mode | Applicable active UL band |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) |  |  |  |
| CA_1A-3A-5A-40A | 4019 |  |  |  | -92.9 | -91.3 | -90.2 | TDD | 3 |  |
| CA_1A-3A-5A-40A | 119 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |  |
|  | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 |  |  |  |
| CA_1A-3A-5A-41A5 | 41 |  |  |  |  |  | [-88.1] | TDD | 3 |  |
| CA_1A-3A-7A-20A-42A | 4219 |  |  | -95.6 | -93 | -91.5 | -90.4 | TDD | 7 |  |
| CA_1A-3A-7A-20A-42A | 719 |  |  |  | -93 | -91.3 | -90.1 | FDD | 42 |  |
| CA_1A-3A-7A-38A CA_1A-3C-7A-38A | 719 |  |  | -93.3 | -90.7 | -89.2 | -88.1 | FDD | 1 |  |
|  | 3819 |  |  | -93.3 | -90.7 | -89.2 | -88.1 | TDD |  |  |
| CA_1A-3A-7A-38A CA_1A-3C-7A-38A | 719 |  |  | -93.3 | -90.7 | -89.2 | -88.1 | FDD | 3 |  |
|  | 3819 |  |  | -93.3 | -90.7 | -89.2 | -88.1 | TDD |  |  |
| CA_1A-3A-7A-40A15 | 319 |  |  | -94 | -91.5 | -90 | -89 | FDD | 1 |  |
|  | 4019 |  |  | -92.6 | -90.5 | -89.2 | -88.1 | TDD |  |  |
| CA_1A-3A-7A-40A16 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 1 |  |
|  | 4019 |  |  | -92.6 | -90.5 | -89.2 | -88.1 | TDD |  |  |
| CA_1A-3A-7A-40A | 4019 |  |  | -94.6 | -92.1 | -90.5 | -89.4 | TDD | 3 |  |
| CA_1A-3A-7A-40A | 4019 |  |  | -96 | -93.3 | -91.7 | -90.6 | TDD | 7 |  |
| CA_1A-3A-7A-40A | 119 |  |  | -91.7 | -89.5 | -87.9 | -86.9 | FDD | 40 |  |
|  | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 |  |  |  |
|  | 719 |  |  |  | -94 | -92.4 | -91.2 |  |  |  |
| CA_1A-3A-7A-40C | 40 |  |  | -94.6 | -92.1 | -90.5 | -89.4 | TDD | 3 |  |
| CA_1A-3A-7A-40C | 40 |  |  | -96 | -93.3 | -91.7 | -90.6 | TDD | 7 |  |
| CA_1A-3A-7A-40C | 1 |  |  | -91.7 | -89.5 | -87.9 | -86.9 | FDD | 40 |  |
|  | 3 |  |  | -94.2 | -91.2 | -89.5 | -88.3 |  |  |  |
|  | 7 |  |  |  | -94 | -92.4 | -91.2 |  |  |  |
| CA_1A-3A-7A-42A | 4219 |  |  | -95.6 | -93 | -91.5 | -90.4 | TDD | 7 |  |
|  | 719 |  |  |  | -93 | -91.3 | -90.1 | FDD | 42 |  |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |  |  |
| CA_1A-3A-8A-40A | 4019 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD | 3 |  |
| CA_1A-3A-8A-40A | 119 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |  |
|  | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 |  |  |  |
| CA_1A-3A-28A-40A CA_1A-3A-28A-40C | 119 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |  |
|  | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD |  |  |
|  | 28 |  |  | -96.8 | -94.1 | -92.5 | -89.8 | FDD |  |  |
| CA_1A-3A-28A-40A CA_1A-3A-28A-40C | 4019 |  |  | [-93.4] | -91.9 | -90.4 | -89.4 | TDD | 1 |  |
| CA_1A-3A-28A-40A CA_1A-3A-28A-40C | 4019 |  |  | -94.6 | -92.1 | -90.5 | -89.4 | TDD | 3 |  |
| CA_1A-3A-28A-40A CA_1A-3A-28A-40C | 4019 |  |  | -95.1 | -92.9 | -91.4 | -90.5 | TDD | 28 |  |
| CA_1A-3A-38A12 CA_1A-3C-38A12 | 319 |  |  | -94 | -91.5 | -90 | -89 | FDD | 1 |  |
|  | 38 |  |  | -97.1 | -94.4 | -92.8 | -91.7 | TDD |  |  |
|  | 38 |  |  | -97.1 | -94.4 | -92.8 | -91.7 | TDD | 3 |  |
|  | 119 |  |  | -98.1 | -95.1 | -93.3 | -92.1 | FDD | 38 |  |
|  | 319 |  |  | -95.1 | -92.1 | -90.3 | -89.1 |  |  |  |
| CA_1A-3A-38A13 CA_1A-3C-38A13 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 1 |  |
|  | 38 |  |  | -97.1 | -94.4 | -92.8 | -91.7 | TDD |  |  |
|  | 38 |  |  | -97.1 | -94.4 | -92.8 | -91.7 | TDD | 3 |  |
|  | 119 |  |  | -98.1 | -95.1 | -93.3 | -92.1 | FDD | 38 |  |
|  | 319 |  |  | -95.1 | -92.1 | -90.3 | -89.1 |  |  |  |
| CA_1A-3A-40A | 4019 |  |  | -100 | -92.9 | -91.3 | -90.2 | TDD | 3 |  |
| CA_1A-3A-40A | 119 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |  |
|  | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 |  |  |  |
| CA_1A-3A-40C | 4019 |  |  | -100 | -92.9 | -91.3 | -90.2 | TDD | 3 |  |
| CA_1A-3A-40C | 119 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |  |
|  | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 |  |  |  |
| CA_1A-3C-40A | 4019 |  |  | -100 | -92.9 | -91.3 | -90.2 | TDD | 3 |  |
| CA_1A-3C-40A | 119 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |  |
|  | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 |  |  |  |
| CA_1A-3A-41A12,14 CA_1A-3A-41C12,14 CA_1A-3A-41D12,14 | 319 |  |  | -94 | -91.5 | -90 | -89 | FDD | 1 |  |
|  | 4119 |  |  | -93.3 | -90.7 | -89.2 | -88.1 | TDD |  |  |
|  | 4119 |  |  | -93.3 | -90.7 | -89.2 | -88.1 | TDD | 3 |  |
| CA_1A-3A-41A13,14 CA_1A-3A-41C13,14 CA_1A-3A-41D13,14 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 1 |  |
|  | 4119 |  |  | -93.3 | -90.7 | -89.2 | -88.1 | TDD |  |  |
|  | 4119 |  |  | -93.3 | -90.7 | -89.2 | -88.1 | TDD | 3 |  |
| CA_1A-3A-41A-42A12,14 CA_1A-3A-41A-42C12,14 CA_1A-3A-41C-42A12,14 CA_1A-3A-41C-42C12,14 | 319 |  |  | -94 | -91.5 | -90 | -89 | FDD | 1 |  |
|  | 4119 |  |  | -93.3 | -90.7 | -89.2 | -88.1 | TDD |  |  |
|  | 4119 |  |  | -93.3 | -90.7 | -89.2 | -88.1 | TDD | 3 |  |
| CA_1A-3A-41A-42A13,14 CA_1A-3A-41A-42C13,14 CA_1A-3A-41C-42A13,14 CA_1A-3A-41C-42C13,14 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD | 1 |  |
|  | 4119 |  |  | -93.3 | -90.7 | -89.2 | -88.1 | TDD |  |  |
|  | 4119 |  |  | -93.3 | -90.7 | -89.2 | -88.1 | TDD | 3 |  |
| CA_1A-3C-40C | 4019 |  |  | -100 | -92.9 | -91.3 | -90.2 | TDD | 3 |  |
| CA_1A-3C-40C | 119 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |  |
|  | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 |  |  |  |
| CA_1A-5A-40A | 4019 |  |  |  | -91.9 | -90.4 | -89.4 | TDD | 1 |  |
| CA_1A-5A-40A | 119 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |  |
| CA_1A-7A-8A-40A | 4019 |  |  | -92.6 | -90.5 | -89.2 | -88.1 | TDD | 1 |  |
| CA_1A-7A-8A-40A | 4019 |  |  | -96 | -93.3 | -91.7 | -90.6 | TDD | 7 |  |
| CA_1A-7A-8A-40A | 119 |  |  | -91.7 | -89.5 | -87.9 | -86.9 | FDD | 40 |  |
|  | 719 |  |  |  | -94 | -92.4 | -91.2 |  |  |  |
| CA_1A-7A-8A-40C | 4019 |  |  | -92.6 | -90.5 | -89.2 | -88.1 | TDD | 1 |  |
| CA_1A-7A-8A-40C | 4019 |  |  | -96 | -93.3 | -91.7 | -90.6 | TDD | 7 |  |
| CA_1A-7A-8A-40C | 119 |  |  | -91.7 | -89.5 | -87.9 | -86.9 | FDD | 40 |  |
|  | 719 |  |  |  | -94 | -92.4 | -91.2 |  |  |  |
| CA_1A-7A-20A-42A | 4219 |  |  | -95.6 | -93 | -91.5 | -90.4 | TDD | 7 |  |
| CA_1A-7A-20A-42A | 719 |  |  | -96.2 | -93.2 | -91.5 | -90.3 | FDD | 42 |  |
| CA_1A-7A-28A-40A CA_1A-7A-28A-40C | 119 |  |  | -91.7 | -89.5 | -87.9 | -86.9 | FDD | 40 |  |
|  | 719 |  |  | -96.8 | -94 | -92.4 | -91.2 | FDD |  |  |
|  | 28 |  |  | -96.8 | -94.1 | -92.5 | -89.8 | FDD |  |  |
| CA_1A-7A-28A-40A CA_1A-7A-28A-40C | 4019 |  |  | -92.6 | -90.5 | -89.2 | -88.1 | TDD | 1 |  |
| CA_1A-7A-28A-40A CA_1A-7A-28A-40C | 4019 |  |  | -96 | -93.3 | -91.7 | -90.6 | TDD | 7 |  |
| CA_1A-7A-28A-40A CA_1A-7A-28A-40C | 4019 |  |  | -95.1 | -92.9 | -91.4 | -90.5 | TDD | 28 |  |
| CA_1A-7A-38A | 719 |  |  | -93.3 | -90.7 | -89.2 | -88.1 | FDD | 1 |  |
|  | 38 |  |  | -93.3 | -90.7 | -89.2 | -88.1 | TDD |  |  |
| CA_1A-7A-40A | 4019 |  |  | -92.6 | -90.5 | -89.2 | -88.1 | TDD | 1 |  |
| CA_1A-7A-40A | 4019 |  |  | -96 | -93.3 | -91.7 | -90.6 | TDD | 7 |  |
| CA_1A-7A-40A | 119 |  |  | -91.7 | -89.5 | -87.9 | -86.9 | FDD | 40 |  |
|  | 719 |  |  |  | -94 | -92.4 | -91.2 |  |  |  |
| CA_1A-7A-40C | 4019 |  |  | -92.6 | -90.5 | -89.2 | -88.1 | TDD | 1 |  |
| CA_1A-7A-40C | 4019 |  |  | -96 | -93.3 | -91.7 | -90.6 | TDD | 7 |  |
| CA_1A-7A-40C | 119 |  |  | -91.7 | -89.5 | -87.9 | -86.9 | FDD | 40 |  |
|  | 719 |  |  |  | -94 | -92.4 | -91.2 |  |  |  |
| CA_1A-7A-42A | 4219 |  |  | -95.6 | -93 | -91.5 | -90.4 | TDD | 7 |  |
|  | 719 |  |  | -96.2 | -93.2 | -91.5 | -90.3 | FDD | 42 |  |
| CA_1A-8A-40A | 4019 |  |  | [-93.4] | -91.9 | -90.4 | -89.4 | TDD | 1 |  |
| CA_1A-8A-40A | 119 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |  |
| CA_1A-8A-40C | 4019 |  |  | [-93.4] | -91.9 | -90.4 | -89.4 | TDD | 1 |  |
| CA_1A-8A-40C | 119 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |  |
| CA_1A-28A-40A CA_1A-28A-40C | 119 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |  |
|  | 28 |  |  | -96.8 | -94.1 | -92.5 | -89.8 | FDD |  |  |
| CA_1A-28A-40A CA_1A-28A-40C | 4019 |  |  | [-93.4] | -91.9 | -90.4 | -89.4 | TDD | 1 |  |
| CA_1A-28A-40A CA_1A-28A-40C | 4019 |  |  | -95.1 | -92.9 | -91.4 | -90.5 | TDD | 28 |  |
| CA_1A-40A | 4019 |  |  | [-93.4] | -91.9 | -90.4 | -89.4 | TDD | 1 |  |
| CA_1A-40A | 119 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |  |
| CA_1A-40C | 4019 |  |  | [-93.4] | -91.9 | -90.4 | -89.4 | TDD | 1 |  |
| CA_1A-40C | 119 |  |  | -91.7 | [-89.5] | [-87.9] | [-86.9] | FDD | 40 |  |
| CA_3A-5A-40A | 4019 |  |  |  | -92.9 | -91.3 | -90.2 | TDD | 3 |  |
| CA_3A-5A-40A | 319 |  | [-95.3] | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
| CA_3A-5A-41A5 | 41 |  |  |  |  |  | [-88.1] | TDD | 3 |  |
| CA_3A-5A-41A5,18 | 3 |  |  | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |  |
|  | 41 |  |  |  |  |  | -91.5 | TDD |  |  |
| CA_3A-7A-8A-38A | 719 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] | FDD | 3 |  |
|  | 38 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] | TDD |  |  |
| CA_3A-7A-8A-38A | 719 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] | FDD | 8 |  |
|  | 38 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] | TDD |  |  |
| CA_3C-7A-8A-38A | 719 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] | FDD | 3 |  |
|  | 38 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] | TDD |  |  |
| CA_3C-7A-8A-38A | 719 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] |  | 3 |  |
|  | 38 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] |  |  |  |
| CA_3A-7A-8A-40A | 4019 |  |  | -94.6 | -92.1 | -90.5 | -89.4 | TDD | 3 |  |
| CA_3A-7A-8A-40A | 4019 |  |  | -96 | -93.3 | -91.7 | -90.6 | TDD | 7 |  |
| CA_3A-7A-8A-40C | 4019 |  |  | -94.6 | -92.1 | -90.5 | -89.4 | TDD | 3 |  |
| CA_3A-7A-8A-40C | 4019 |  |  | -96 | -93.3 | -91.7 | -90.6 | TDD | 7 |  |
| CA_3A-7A-8A-40A | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
|  | 719 |  |  |  | -94 | -92.4 | -91.2 |  |  |  |
| CA_3A-7A-8A-40C | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
|  | 719 |  |  |  | -94 | -92.4 | -91.2 |  |  |  |
| CA_3A-7A-28A-38A | 719 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] | FDD | 3 |  |
|  | 38 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] | TDD |  |  |
| CA_3A-7A-28A-38A | 719 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] | FDD | 28 |  |
|  | 38 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] | TDD |  |  |
| CA_3C-7A-28A-38A | 719 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] | FDD | 3 |  |
|  | 38 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] | TDD |  |  |
| CA_3C-7A-28A-38A | 719 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] | FDD | 3 |  |
|  | 38 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] | TDD |  |  |
| CA_3A-7A-28A-40A CA_3A-7A-28A-40C | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
|  | 719 |  |  | -96.8 | -94 | -92.4 | -91.2 | FDD |  |  |
|  | 28 |  |  | -96.8 | -94.1 | -92.5 | -89.8 | FDD |  |  |
| CA_3A-7A-28A-40A CA_3A-7A-28A-40C | 4019 |  |  | -94.6 | -92.1 | -90.5 | -89.4 | TDD | 3 |  |
| _3A-7A-28A-40A CA_3A-7A-28A-40C | 4019 |  |  | -96 | -93.3 | -91.7 | -90.6 | TDD | 7 |  |
| CA_3A-7A-28A-40A CA_3A-7A-28A-40C | 4019 |  |  | -95.1 | -92.9 | -91.4 | -90.5 | TDD | 28 |  |
| CA_3A-7A-38A | 719 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] | FDD | 3 |  |
|  | 38 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] | TDD |  |  |
| CA_3C-7A-38A | 719 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] | FDD | 3 |  |
|  | 38 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] | TDD |  |  |
| CA_3A-7A-40A | 4019 |  |  | -94.6 | -92.1 | -90.5 | -89.4 | TDD | 3 |  |
| CA_3A-7A-40A | 4019 |  |  | -96 | -93.3 | -91.7 | -90.6 | TDD | 7 |  |
| CA_3A-7A-40A | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
|  | 719 |  |  | -96.8 | -94 | -92.4 | -91.2 |  |  |  |
| CA_3A-7A-40C | 4019 |  |  | -94.6 | -92.1 | -90.5 | -89.4 | TDD | 3 |  |
| CA_3A-7A-40C | 4019 |  |  | -96 | -93.3 | -91.7 | -90.6 | TDD | 7 |  |
| CA_3A-7A-40C | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
|  | 719 |  |  | -96.8 | -94 | -92.4 | -91.2 |  |  |  |
| CA_3A-7A-42A | 4219 |  |  | -95.6 | -93 | -91.5 | -90.4 | TDD | 7 |  |
|  | 3 |  |  | [-96.6] | [-93.6] | [-91.8] | [-90.6] | FDD | 42 |  |
|  | 719 |  |  | -96.2 | -93.2 | -91.5 | -90.3 |  |  |  |
| CA_3A-8A-40A | 4019 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD | 3 |  |
| CA_3A-8A-40C | 4019 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD | 3 |  |
| CA_3A-8A-40C | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
| CA_3A-8A-40A | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
| CA_3A-20A-32A | 32 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | FDD | 3 |  |
| CA_3A-28A-40A | 4019 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD | 3 |  |
| CA_3A-28A-40A | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
| CA_3A-28A-40A | 4019 |  |  | -95.1 | -92.9 | -91.4 | -90.5 | TDD | 28 |  |
| CA_3A-28A-40C | 4019 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD | 3 |  |
| CA_3A-28A-40C | 4019 |  |  | -95.1 | -92.9 | -91.4 | -90.5 | TDD | 28 |  |
| CA_3A-28A-40C | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
|  | 28 |  |  | -96.8 | -94.1 | -92.5 | -89.8 |  |  |  |
| CA_3A-28A-40D | 4019 |  |  |  | -92.9 | -91.3 | -90.2 | TDD | 3 |  |
| CA_3A-28A-40D | 4019 |  |  |  | -92.9 | -91.4 | -90.5 | TDD | 28 |  |
| CA_3A-28A-40D | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
|  | 28 |  |  | -96.8 | -94.1 | -92.5 | -89.8 |  |  |  |
| CA_3A-28A-41A5 | 4119 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD | 3 |  |
|  | 319 |  |  | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |  |
| CA_3A-28A-41C5 | 4119 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD | 3 |
|  | 319 |  |  | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |
| CA_3A-28A-41A-42A | 319 |  |  | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |
|  | 41 |  |  |  | -94.517 | -92.717 | -91.517 | TDD |  |
|  | 4119 |  |  |  | [-90.7] | [-89.2] | [-88.1] | TDD | 3 |
| CA_3A-28A-41A-42C | 4119 |  |  |  | [-90.7] | [-89.2] | [-88.1] | TDD | 3 |  |
|  | 319 |  |  | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |  |
|  | 41 |  |  |  | -94.517 | -92.717 | -91.517 | TDD |  |  |
| CA_3A-28A-41C-42A | 4119 |  |  |  | [-90.7] | [-89.2] | [-88.1] | TDD | 3 |  |
|  | 319 |  |  | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |  |
|  | 41 |  |  |  | -94.517 | -92.717 | -91.517 | TDD |  |  |
| CA_3A-40A | 4019 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD | 3 |  |
| CA_3A-40A | 319 | [-97.4] | [-95.3] | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
| CA_3A-40A-40A | 4019 |  |  |  | -92.9 |  | -90.2 | TDD | 3 |  |
| CA_3A-40A-40A | 319 |  |  | -94.2 | -91.2 |  |  | FDD | 40 |  |
| CA_3A-40C | 4019 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD | 3 |  |
| CA_3A-40D | 4019 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD | 3 |
| CA_3A-40D | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |
| CA_3A-40E | 4019 |  |  |  |  | -91.3 | -90.2 | TDD | 3 |  |
| CA_3A-40E | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
| CA_3A-40C | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
| CA_3C-40A | 4019 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD | 3 |  |
| CA_3C-40A | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
| CA_3C-40C | 4019 |  |  | -95.4 | -92.9 | -91.3 | -90.2 | TDD | 3 |  |
| CA_3C-40C | 319 |  |  | -94.2 | -91.2 | -89.5 | -88.3 | FDD | 40 |  |
| CA_3A-41A5 | 319 |  | [-95.3] | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |  |
|  | 4119 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD | 3 |  |
| CA_3A-41C5 | 319 |  |  | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |  |
|  | 4119 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD | 3 |  |
| CA_3A-41D5 | 319 |  |  | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |  |
|  | 4119 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD | 3 |  |
| CA_3A-3A-41A | 3 |  | [-95.3] | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |
|  | 41 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD | 3 |
| CA_3C-41A5 | 319 |  |  | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |  |
|  | 4119 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD | 3 |  |
| CA_3C-41C5 | 319 |  |  | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |  |
|  | 4119 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD | 3 |  |
| CA_3C-41D5 | 319 |  |  | [-94] | [-91] | [-89.2] | [-87.9] | FDD | 41 |
|  | 4119 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD | 3 |
| CA_3A-41A-42A5,6,7,8 | 4119 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD | 3 |  |
|  | 4219 |  |  | -71.7 | -71.7 | -71.7 | -71.7 |  |  |  |
| CA_3A-41A-42A5,6,9 | 4119 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD | 3 |  |
|  | 4219 |  |  | -97.1 | -94.7 | -93.2 | -92.5 |  |  |  |
| CA_3A-41A-42A5,6,10 | 319 |  |  | [-93.5] | [-90.5] | [-88.7] | [-87.4] | FDD | 41 |  |
| CA_3A-41A-42C5,6,7,8 | 4119 |  |  |  | [-90.7] | [-89.2] | [-88.1] | TDD | 3 |  |
|  | 4219 |  |  |  | -71.7 | -71.7 | -71.7 |  |  |  |
| CA_3A-41A-42C5,6,9 | 4119 |  |  |  | [-90.7] | [-89.2] | [-88.1] | TDD | 3 |  |
|  | 4219 |  |  |  | -94.7 | -93.2 | -92.5 |  |  |  |
| CA_3A-41A-42C5,6,10 | 319 |  |  | [-93.5] | [-90.5] | [-88.7] | [-87.4] | FDD | 41 |  |
|  | 42 |  |  |  | -95.5 | -93.7 | -92.5 | TDD |  |  |
| CA_3A-41C-42A5,6,7,8 | 4119 |  |  |  | [-90.7] | [-89.2] | [-88.1] | TDD | 3 |  |
|  | 4219 |  |  |  | -71.7 | -71.7 | -71.7 |  |  |  |
| CA_3A-41C-42A5,6,9 | 4119 |  |  |  | [-90.7] | [-89.2] | [-88.1] | TDD | 3 |  |
|  | 42 |  |  |  | -94.7 | -93.2 | -92.5 |  |  |  |
| CA_3A-41C-42A5,6,10 | 319 |  |  | [-93.5] | [-90.5] | [-88.7] | [-87.4] | FDD | 41 |  |
| CA_3A-41C-42C5,6,7,8 | 4119 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD | 3 |  |
|  | 4219 |  |  | -71.7 | -71.7 | -71.7 | -71.7 |  |  |  |
| CA_3A-41C-42C5,6,9 | 4119 |  |  | [-93.3] | [-90.7] | [-89.2] | [-88.1] | TDD | 3 |  |
|  | 4219 |  |  | -97.1 | -94.7 | -93.2 | -92.5 |  |  |  |
| CA_3A-41C-42C5,6,10 | 319 |  |  | [-93.5] | [-90.5] | [-88.7] | [-87.4] | FDD | 41 |  |
| CA_7A-8A-38A | 719 |  |  |  | [-91.2] | [-89.7] | [-88.6] | FDD | 8 |  |
|  | 38 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] | TDD |  |  |
| CA_7A-8A-40A | 4019 |  |  | -96.3 | -93.6 | -92 | -90.9 | TDD | 7 |  |
|  | 719 |  |  | -88 | -87.4 | -87 | -86.7 | FDD | 8 |  |
|  | 719 |  |  | -97.1 | -94.3 | -92.7 | -91.5 | FDD | 40 |  |
| CA_7A-8A-40C | 4019 |  |  | -96.3 | -93.6 | -92 | -90.9 | TDD | 7 |  |
|  | 719 |  |  | -88 | -87.4 | -87 | -86.7 | FDD | 8 |  |
|  | 719 |  |  | -97.1 | -94.3 | -92.7 | -91.5 | FDD | 40 |  |
| CA_7A-20A-42A | 4219 |  |  | -95.6 | -93 | -91.5 | -90.4 | TDD | 7 |  |
|  | 719 |  |  | -96.2 | -93.2 | -91.5 | -90.3 | FDD | 42 |  |
| CA_7A-28A-38A | 719 |  |  |  | [-91.2] | [-89.7] | [-88.6] | FDD | 8 |  |
|  | 38 |  |  | [-93.8] | [-91.2] | [-89.7] | [-88.6] | TDD |  |  |
| CA_7A-28A-40A CA_7A-28A-40C | 719 |  |  | -96.8 | -94 | -92.4 | -91.2 | FDD | 40 |  |
|  | 28 |  |  | -96.8 | -94.1 | -92.5 | -89.8 | FDD |  |  |
| CA_7A-28A-40A CA_7A-28A-40C | 4019 |  |  | -96 | -93.3 | -91.7 | -90.6 | TDD | 7 |  |
| CA_7A-28A-40A CA_7A-28A-40C | 4019 |  |  | -95.1 | -92.9 | -91.4 | -90.5 | TDD | 28 |  |
| CA_7A-40A | 4019 |  |  | -96.3 | -93.6 | -92 | -90.9 | TDD | 7 |  |
|  | 719 |  |  | -97.1 | -94.3 | -92.7 | -91.5 | FDD | 40 |  |
| CA_7A-40C | 4019 |  |  | -96.3 | -93.6 | -92 | -90.9 | TDD | 7 |  |
|  | 719 |  |  | -97.1 | -94.3 | -92.7 | -91.5 | FDD | 40 |  |
| CA_7A-40D | 4019 |  |  | -96.3 | -93.6 | -92 | -90.9 | TDD | 7 |  |
|  | 719 |  |  | -97.1 | -94.3 | -92.7 | -91.5 | FDD | 40 |  |
| CA_7A-40E | 4019 |  |  | -96.3 | -93.6 | -92 | -90.9 | TDD | 7 |  |
|  | 719 |  |  | -97.1 | -94.3 | -92.7 | -91.5 | FDD | 40 |  |
| CA_7A-42A | 4219 |  |  | -95.6 | -93 | -91.5 | -90.4 | TDD | 7 |  |
|  | 719 |  |  | -96.2 | -93.2 | -91.5 | -90.3 | FDD | 42 |  |
| CA_7A-42A-42A | 4219 |  |  | -95.6 | -93 | -91.5 | -90.4 | TDD | 7 |  |
|  | 719 |  |  | -96.2 | -93.2 | -91.5 | -90.3 | FDD | 42 |  |
| CA_25A-41A, CA_25A-25A-41A, CA_25A-41C, CA_25A-25A-41C, CA_25A-41D, CA_25A-25A-41D, CA_25A-41E, CA_25A-25A-41E, CA_25A-41F, CA_25A-25A-41F | 25 |  |  | [-95.9] | [-92.9] | [-91.1] | [-89.9] | FDD | 41 |  |
| CA_28A-40A | 40 |  |  | -95.1 | -92.9 | -91.4 | -90.5 | TDD | 28 |  |
| CA_28A-40A | 28 |  |  | -96.8 | -94.1 | -92.5 | -89.8 | FDD | 40 |  |
| CA_28A-40C | 40 |  |  | -95.1 | -92.9 | -91.4 | -90.5 | TDD | 28 |  |
| CA_28A-40C | 28 |  |  | -96.8 | -94.1 | -92.5 | -89.8 | FDD | 40 |  |
| CA_28A-40D | 40 |  |  | -95.1 | -92.9 | -91.4 | -90.5 | TDD | 28 |  |
| CA_28A-40D | 28 |  |  | -96.8 | -94.1 | -92.5 | -89.8 | FDD | 40 |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: The signal power is specified per port NOTE 4: These requirements apply regardless of the channel bandwidth and the location of UL band. NOTE 5: The B41 requirements are modified by -0.5dB when carrier frequency of the assigned E-UTRA channel bandwidth is within 2545-2690 MHz. NOTE 6: The antenna isolation for MSD calculation is assumed as 10 dB. For conducted mode REFSENS test such antenna isolation is not observed as the antennas are disconnected. Additionally antenna isolation assumption is under discussion depending on the frequency range NOTE 7: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_3A-42A, CA_3A-42C, CA_3A-42D, CA_3A-3A-42C, CA_3A-42A-42C, CA_1A-3A-42A, CA_1A-3A-42C, CA_1A-3A-42D, CA_3A-19A-42A, CA_3A-19A-42C, CA_1A-3A-19A-42A, CA 3A-41A-42A, CA_3A-41A-42C, CA_3A-41C-42A, and CA_3A-41C-42C. NOTE 8: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 9: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 10: Only applicable for UE supporting inter-band carrier aggregation with uplink in one E-UTRA band and without simultaneous Rx/Tx. NOTE 11: Void NOTE 12: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in Band 3 and Band 41, the requirement applies regardless of channel bandwidth in Band 1. NOTE 13: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth in Band 3 and Band 41, the requirement applies regardless of channel bandwidth in Band 1. NOTE 14: The B41 requirements also apply to the supported CA_1A-41A. NOTE 15: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth other than Band 1, the requirement applies regardless of channel bandwidth in Band 1 NOTE 16: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth other than Band 1, the requirement applies regardless of channel bandwidth in Band 1. NOTE 17: The B41 requirements are modified by -0.1dB when carrier frequency of the assigned E-UTRA channel bandwidth is within 2545-2690 MHz. NOTE 18: No requirements apply when there is at least one individual RE within the downlink transmission bandwidth of band5 for which the 3rd receiver harmonic is within the uplink transmission bandwidth of band41. The reference sensitivity is only verified when this is not the case (the requirements specified in clause 7.3.1 apply). NOTE 19: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. |  |  |  |  |  |  |  |  |  |  |

Table 7.3.1A-0bF: Uplink configuration for reference sensitivity (exceptions due to cross band isolation issues of TDD and FDD bands)

| E-UTRA Band / Channel bandwidth of the affected DL band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| CA_1A-3A-5A-41A | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
| CA_1A-3A-7A-20A-42A | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 42 |  |  | 25 | 50 | 75 | 100 | FDD |
| CA_1A-3A-7A-38A CA_1A-3C-7A-38A | 1 |  |  | 25 | 50 | 75 | 100 | FDD |
|  | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
| CA_1A-3A-7A-40A CA_1A-3A-7A-40C | 11,3 |  |  | 25 | 25 | 25 | 25 | FDD |
|  | 11,4 |  |  | 25 | 45 | 45 | 45 | FDD |
|  | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_1A-3A-7A-42A | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 42 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_1A-3A-28A-40A CA_1A-3A-28A-40C | 1 |  |  | 25 | 50 | 75 | 100 | FDD |
|  | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 28 |  |  | 25 | 251 | 251 | 251 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_1A-3A-38A CA_1A-3C-38A | 11,3 |  |  | 25 | 25 | 25 | 25 | FDD |
|  | 11,4 |  |  | 25 | 45 | 45 | 45 | FDD |
|  | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 38 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_1A-3A-41A CA_1A-3A-41C CA_1A-3A-41D | 1 |  |  | 253 | 251,3 | 251,3 | 251,3 | FDD |
|  |  |  |  | 254 | 451,4 | 451,4 | 451,4 | FDD |
|  | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
| CA_1A-3A-41A-42A CA_1A-3A-41A-42C CA_1A-3A-41C-42A CA_1A-3A-41C-42C | 1 |  |  | 253 | 251,3 | 251,3 | 251,3 | FDD |
|  |  |  |  | 254 | 451,4 | 451,4 | 451,4 | FDD |
|  | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
| CA_1A-5A-40A | 1 |  |  | 25 | 50 | 75 | 100 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_1A-7A-8A-40A | 1 |  |  | 25 | 50 | 75 | 100 | FDD |
|  | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_1A-7A-8A-40C | 1 |  |  | 25 | 50 | 75 | 100 | FDD |
|  | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_1A-7A-28A-40A CA_1A-7A-28A-40C | 1 |  |  | 25 | 50 | 75 | 100 | FDD |
|  | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 28 |  |  | 25 | 251 | 251 | 251 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_1A-8A-40A | 1 |  |  | 25 | 50 | 75 | 100 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_1A-8A-40C | 1 |  |  | 25 | 50 | 75 | 100 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_1A-7A-20A-42A | 1 |  |  | 25 | 50 | 75 | 100 | FDD |
|  | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 20 |  |  | 25 | 201 | 205 | 205 | FDD |
|  | 42 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_1A-7A-38A | 1 |  |  | 25 | 45 | 451 | 451 | FDD |
| CA_1A-7A-40A | 1 |  |  | 25 | 50 | 75 | 100 | FDD |
|  | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_1A-7A-40C | 1 |  |  | 25 | 50 | 75 | 100 | FDD |
|  | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_1A-7A-42A | 1 |  |  | 25 | 50 | 75 | 100 | FDD |
|  | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 42 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_1A-40A | 1 |  |  | 25 | 50 | 75 | 100 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_1A-28A-40A CA_1A-28A-40C | 1 |  |  | 25 | 50 | 75 | 100 | FDD |
|  | 28 |  |  | 25 | 251 | 251 | 251 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3A-5A-41A | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 41 |  |  |  |  |  | 100 | TDD |
| CA_3A-7A-8A-38A | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
| CA_3C-7A-8A-38A | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
| CA_3A-7A-8A-40A | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3A-7A-8A-40C | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3A-7A-28A-38A | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
| CA_3C-7A-28A-38A | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
| CA_3A-7A-28A-40A CA_3A-7A-28A-40C | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 28 |  |  | 25 | 251 | 251 | 251 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3A-7A-38A | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
| CA_3C-7A-38A | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
| CA_3A-7A-40A | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3A-7A-40C | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3A-7A-42A | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 42 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3A-8A-40A | 3 | 6 | 15 | 25 | 50 | 501 | 501 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3A-8A-40C | 3 | 6 | 15 | 25 | 50 | 501 | 501 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3A-28A-41A | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 28 |  |  | 25 | 251 | 251 | 251 | FDD |
|  | 41 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3A-28A-41C | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 28 |  |  | 25 | 251 | 251 | 251 | FDD |
|  | 41 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3A-28A-41A-42A | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 28 |  |  | 25 | 251 |  |  | FDD |
|  | 41 |  |  |  | 50 | 75 | 100 | TDD |
|  | 42 |  |  |  | 50 | 75 | 100 | TDD |
| CA_3A-28A-41A-42C | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 41 |  |  |  | 50 | 75 | 100 | TDD |
| CA_3A-28A-41C-42A | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 41 |  |  |  | 50 | 75 | 100 | TDD |
| CA_3A-40A CA_3A-40C CA_3A-40D CA_3A-40E CA_1A-3A-40A CA_1A-3C-40A CA_1A-3A-40C CA_1A-3C-40C CA_3A-5A-40A CA_3A-28A-40A CA_3A-28A-40C CA_1A-3A-5A-40A CA_1A-3A-8A-40A | 3 | 6 | 15 | 25 | 50 | 501 | 501 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3A-40A-40A | 3 |  |  | 25 | 50 |  |  | FDD |
|  | 40 |  |  |  | 50 |  | 100 | TDD |
| CA_3A-41A CA_3A-3A-41A | 3 |  | 15 | 25 | 50 | 501 | 501 | FDD |
|  | 41 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3A-41C CA_3C-41A CA_3C-41D | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 41 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3A-41D | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 41 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3C-41C | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 41 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3A-41A-42A | 3 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 41 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_7A-8A-38A | 8 |  |  | 25 | 50 |  |  | FDD |
| CA_7A-8A-40A | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 8 |  |  | 8 | 16 |  |  |  |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_7A-8A-40C | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 8 |  |  | 8 | 16 |  |  |  |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_7A-20A-42A | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 20 |  |  | 25 | 50 | 75 | 100 |  |
|  | 42 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_7A-28A-38A | 28 |  |  | 25 | 50 | 501 | 501 | FDD |
| CA_7A-28A-40A CA_7A-28A-40C | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 28 |  |  | 25 | 251 | 251 | 251 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_7A-40A, CA_7A-40C, CA_7A-40D, CA_7A-40E | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_7A-42A, CA_7A-42A-42A | 7 |  |  | 25 | 50 | 75 | 751 | FDD |
|  | 42 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_25A-41A, CA_25A-41C, CA_25A-41D, CA_25A-41E, CA_25A-41F, CA_25A-25A-41A, CA_25A-25A-41C, CA_25A-25A-41D, CA_25A-25A-41E, CA_25A-25A-41F | 25 |  |  | 25 | 50 | 75 | 100 | FDD |
|  | 41 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_28A-40A, CA_28A-40C | 28 |  |  | 25 | 251 | 251 | 251 | FDD |
|  | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1). NOTE 2: the UL configuration applies regardless of the channel bandwidth of the low band unless the UL resource blocks exceed that specified in Table 7.3.1-2 for the uplink bandwidth in which case the allocation according to Table 7.3.1-2 applies. NOTE 3: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. NOTE 4: UL allocation when the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. NOTE 5: 5 refers to Band 20; in the case of 15MHz channel bandwidth, the UL resource blocks shall be located at RBstart 11 and in the case of 20MHz channel bandwidth, the UL resource blocks shall be located at RBstart 16 |  |  |  |  |  |  |  |  |

For band combinations including operating bands without uplink band (as noted in Table 5.5-1), the requirements are specified in Table 7.3.1A-0d and for any uplink band with uplink configuration specified in Table 7.3.1-2.

Table 7.3.1A-0d: Reference sensitivity QPSK PREFSENS (CA with a SDL band)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3A-7A-20A-32A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 35,6 |  |  | -94 | -91 | -90 | -89 |  |
|  | 37 |  |  | -97 | -94 | -92.2 | -91 |  |
|  | 7 |  |  |  | -95 | -93.2 | -92 |  |
|  | 20 |  |  | -97 | -94 | -91.2 | -90 |  |
|  | 32 |  |  | -100 | -97 | -95.2 | -94 |  |
| CA_1A-3A-20A-32A6 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 3 |  |  | -94 | -91.5 | -90 | -89 |  |
|  | 20 |  |  | -97 | -94 | -91.2 | -90 |  |
|  | 32 |  |  | -100 | -97 | -95.2 | -94 |  |
| CA_1A-3A-20A-32A-42A | 1 |  |  | -100 | -97 | -95.2 |  | FDD |
|  | 3 |  |  | -97 | -94 | -92.2 |  | FDD |
|  | 20 |  |  | -97 |  |  |  | FDD |
|  | 32 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | FDD |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |
| CA_1A-3A-20A-32A-43A | 1 |  |  | -100 | -97 | -95.2 |  | FDD |
|  | 3 |  |  | -97 | -94 | -92.2 |  | FDD |
|  | 20 |  |  | -97 |  |  |  | FDD |
|  | 32 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | FDD |
|  | 43 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |
| CA_1A-3A-20A-32A7 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |
|  | 20 |  |  | -97 | -94 | -91.2 | -90 |  |
|  | 32 |  |  | -100 | -97 | -95.2 | -94 |  |
| CA_1A-3A-32A6 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 3 |  |  | -94 | -91.5 | -90 | -89 |  |
|  | 32 |  |  | -100 | -97 | -95.2 | -94 |  |
| CA_1A-3A-32A7 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |
|  | 32 |  |  | -100 | -97 | -95.2 | -94 |  |
| CA_1A-3A-7A-32A6 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 3 |  |  | -94 | -91.5 | -90 | -89 |  |
|  | 7 |  |  | -98 | -95 | -93.2 | -92 |  |
|  | 32 |  |  | -100 | -97 | -95.2 | -94 |  |
| CA_1A-3A-7A-32A7 | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |
|  | 7 |  |  | -98 | -95 | -93.2 | -92 |  |
|  | 32 |  |  | -100 | -97 | -95.2 | -94 |  |
| CA_1A-3A-32A-42A7 | 1 |  |  | -99.8 | -96.8 | -95 |  | FDD |
|  | 3 |  |  | -96.8 | -93.8 | -92 |  |  |
|  | 32 |  |  | -99.5 | -96.5 | -94.7 | -93.5 |  |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |
| CA_1A-3A-32A-43A7 | 1 |  |  | -100 | -97 | -95.2 |  | FDD |
|  | 3 |  |  | -97 | -94 | -92.2 |  |  |
|  | 32 |  |  | -99.5 | -96.5 | -94.7 | -93.5 |  |
|  | 43 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |
| CA_1A-3A-32A-42A-43A | 1 |  |  | -99.8 | -96.8 | -95 |  | FDD |
|  | 3 |  |  | -96.8 | -93.8 | -92 |  | FDD |
|  | 32 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | FDD |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |
|  | 43 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |
| CA_1A-20A-32A-42A | 1 |  |  | -100 | -97 | -95.2 |  | FDD |
|  | 20 |  |  | -97 |  |  |  |  |
|  | 32 |  |  | -100 | -97 | -95.2 | -94 |  |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |
| CA_1A-20A-32A-43A | 1 |  |  | -100 | -97 | -95.2 |  | FDD |
|  | 20 |  |  | -97 |  |  |  |  |
|  | 32 |  |  | -100 | -97 | -95.2 | -94 |  |
|  | 43 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |
| CA_1A-32A | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 32 |  |  | -100 | -97 | -95.2 | -94 |  |
| CA_1A-32A-42A | 1 |  |  | -100 | -97 | -95.2 |  | FDD |
|  | 32 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |
| CA_1A-32A-43A | 1 |  |  | -100 | -97 | -95.2 |  | FDD |
|  | 32 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 43 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |
| CA_1A-32A-42A-43A | 1 |  |  | -100 | -97 | -95.2 |  | FDD |
|  | 32 |  |  | -100 | -97 | -95.2 | -94 |  |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |
|  | 43 |  |  | -98.5 | -95.5 | -93.7 | -92.5 |  |
| CA_2A-4A-5A-29A | 2 |  |  | -97.7 | -94.7 | -92.9 | -91.7 | FDD |
|  | 4 |  |  | -99.7 | -96.7 | -94.9 | -93.7 |  |
|  | 5 |  |  | -98 | -95 |  |  |  |
|  | 29 |  |  | -97 | -94 |  |  |  |
| CA_2A-4A-29A | 2 |  |  | -97.7 | -94.7 | -92.9 | -91.7 | FDD |
|  | 4 |  |  | -99.7 | -96.7 | -94.9 | -93.7 |  |
|  | 29 |  |  | -97 | -94 |  |  |  |
| CA_2A-4A-29A-30A | 2 |  |  | -97.6 | -94.6 | -92.8 | -91.6 | FDD |
|  | 4 |  |  | -99.6 | -96.6 | -94.8 | -93.6 |  |
|  | 29 |  |  | -97 | -94 |  |  |  |
|  | 30 |  |  | -98.5 | -95.5 |  |  |  |
| CA_2A-5A-29A | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 5 |  |  | -98 | -95 |  |  |  |
|  | 29 |  |  | -97 | -94 |  |  |  |
| CA_2A-7A-29A CA_2A-7C-29A | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 7 |  |  |  | -95 | -93.2 | -92 |  |
|  | 29 |  |  | -97 | -94 |  |  |  |
| CA_2A-7A-7A-29A | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 7 |  |  | -98 | -95 | -93.2 | -92 |  |
|  | 29 |  |  | -97 | -94 |  |  |  |
| CA_2A-7A-29A-66A CA_2A-7C-29A-66A CA_2A-7A-7A-29A-66A | 2 |  |  | -97.7 | -94.7 | -92.9 | -91.7 | FDD |
|  | 7 |  |  |  | -94.5 | -92.7 | -91.5 |  |
|  | 29 |  |  | -97 | -94 |  |  |  |
|  | 66 |  |  | -99 | -96 | -94.2 | -93 |  |
| CA_2A-29A | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 29 |  | -98.7 | -97 | -94 |  |  |  |
| CA_2A-2A-29A | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 29 |  |  | -97 | -94 |  |  |  |
| CA_2C-29A | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 29 |  |  | -97 | -94 |  |  |  |
| CA_2A-29A-30A | 2 |  |  | -97.6 | -94.6 | -92.8 | -91.6 | FDD |
|  | 29 |  |  | -97 | -94 |  |  |  |
|  | 30 |  |  | -98.5 | -95.5 |  |  |  |
| CA_2A-2A-29A-30A | 2 |  |  | -97.6 | -94.6 | -92.8 | -91.6 | FDD |
|  | 29 |  |  | -97 | -94 |  |  |  |
|  | 30 |  |  | -98.5 | -95.5 |  |  |  |
| CA_2C-29A-30A | 2 |  |  | -97.6 | -94.6 | -92.8 | -91.6 | FDD |
|  | 29 |  |  | -97 | -94 |  |  |  |
|  | 30 |  |  | -98.5 | -95.5 |  |  |  |
| CA_2A-29A-30A-66A | 2 |  |  | -97.6 | -94.6 | -92.8 | -91.6 | FDD |
|  | 29 |  |  | -97 | -94 |  |  |  |
|  | 30 |  |  | -98.5 | -95.5 |  |  |  |
|  | 66 |  |  | -99.1 | -96.1 | -94.3 | -93.1 |  |
| CA_2A-29A-66A | 2 |  |  | -97.7 | -94.7 | -92.9 | -91.7 | FDD |
|  | 29 |  |  | -97 | -94 |  |  |  |
|  | 66 |  |  | -99.2 | -96.2 | -94.4 | -93.2 |  |
| CA_4A-4A-29A | 4 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 29 |  |  | -97 | -94 |  |  |  |
| CA_3A-7A-20A-32A | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
|  | 7 |  |  | -98 | -95 | -93.2 | -92 |  |
|  | 20 |  |  | -97 | -94 | -91.2 | -90 |  |
|  | 32 |  |  | -99.5 | -96.5 | -94.7 | -93.5 |  |
| CA_3A-7A-32A | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
|  | 7 |  |  | -98 | -95 | -93.2 | -92 |  |
|  | 32 |  |  | -100 | -97 | -95.2 | -94 |  |
| CA_3A-20A-32A-42A | 3 |  |  | -96.8 | -93.8 | -92 |  | FDD |
|  | 20 |  |  | -97 |  |  |  |  |
|  | 32 |  |  | -99.5 | -96.5 | -94.7 | -93.5 |  |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |
| CA_3A-20A-32A-43A | 3 |  |  | -97 | -94 | -92.2 |  | FDD |
|  | 20 |  |  | -97 |  |  |  |  |
|  | 32 |  |  | -99.5 | -96.5 | -94.7 | -93.5 |  |
|  | 43 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |
| CA_3A-32A | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
|  | 32 |  |  | -99.5 | -96.5 | -94.7 | -93.5 |  |
| CA_3C-32A | 35 |  |  | -97 | -94 | -92.2 | -91 | FDD |
|  | 32 |  |  | -99.5 | -96.5 | -94.7 | -93.5 |  |
| CA_3A-32A-42A | 3 |  |  | -96.8 | -93.8 | -92 |  | FDD |
|  | 32 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | FDD |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |
| CA_3A-32A-43A | 3 |  |  | -97 | -94 | -92.2 |  | FDD |
|  | 32 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | FDD |
|  | 43 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |
| CA_3A-32A-42A-43A | 3 |  |  | -96.8 | -93.8 | -92 |  | FDD |
|  | 32 |  |  | -99.5 | -96.5 | -94.7 | -93.5 |  |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |
|  | 43 |  |  | -98.5 | -95.5 | -93.7 | -92.5 |  |
| CA_3A-69A | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
|  | 69 |  |  | -100 | -97 | -95.2 | -94 |  |
| CA_4A-4A-29A-30A | 4 |  |  | -99.6 | -96.6 | -94.8 | -93.6 | FDD |
|  | 29 |  |  | -97 | -94 |  |  |  |
|  | 30 |  |  | -98.5 | -95.5 |  |  |  |
| CA_4A-5A-29A | 4 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 5 |  |  | -98 | -95 |  |  |  |
|  | 29 |  |  | -97 | -94 |  |  |  |
| CA_4A-29A | 4 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 29 |  | -98.7 | -97 | -94 |  |  |  |
| CA_4A-29A-30A | 4 |  |  | -99.6 | -96.6 | -94.8 | -93.6 | FDD |
|  | 29 |  |  | -97 | -94 |  |  |  |
|  | 30 |  |  | -98.5 | -95.5 |  |  |  |
| CA_5A-29A | 5 |  |  | -98 | -95 |  |  | FDD |
|  | 29 |  |  | -97 | -94 |  |  |  |
| CA_7A-20A-32A | 7 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 20 |  |  | -97 | -94 | -91.2 | -90 |  |
|  | 32 |  |  | -100 | -97 | -95.2 | -94 |  |
| CA_7A-29A CA_7A-7A-29A CA_7C-29A | 7 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 29 |  |  | -97 | -94 |  |  |  |
| CA_7A-29A-66A | 7 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 29 |  |  | -97 | -94 |  |  |  |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 |  |
| CA_7A-7A-29A-66A | 7 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 29 |  |  | -97 | -94 |  |  |  |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 |  |
| CA_7C-29A-66A | 7 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 29 |  |  | -97 | -94 |  |  |  |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 |  |
| CA_7A-32A | 7 |  |  |  | -95 | -93.2 | -92 | FDD |
|  | 32 |  |  | -100 | -97 | -95.2 | -94 |  |
| CA_20A-32A | 20 |  |  | -97 | -94 | -91.2 | -90 | FDD |
|  | 32 |  |  | -100 | -97 | -95.2 | -94 |  |
| CA_20A-32A-42A | 20 |  |  | -97 |  |  |  | FDD |
|  | 32 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |
| CA_20A-32A-43A | 20 |  |  | -97 |  |  |  | FDD |
|  | 32 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 43 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |
| CA_20A-75A | 20 |  |  | -97 | -94 | -91.2 | -90 | FDD |
|  | 75 |  |  | -100 | -97 | -95.2 | -94 |  |
| CA_20A-76A | 20 |  |  | -97 | -94 | -91.2 | -90 | FDD |
|  | 76 |  |  | -100 |  |  |  |  |
| CA_20A-67A | 20 |  |  | -97 | -94 | -91.2 | -90 | FDD |
|  | 67 |  |  | -100 | -97 | -95.2 | -94 |  |
| CA_23A-29A | 23 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 29 |  | -98.7 | -97 | -94 |  |  |  |
| CA_29A-30A | 29 |  |  | -97 | -94 |  |  | FDD |
|  | 30 |  |  | -99 | -96 |  |  |  |
| CA_29A-30A-66A CA_29A-30A-66A-66A | 29 |  |  | -97 | -94 |  |  | FDD |
|  | 30 |  |  | -98.5 | -95.5 |  |  |  |
|  | 66 |  |  | -99.1 | -96.1 | -94.3 | -93.1 |  |
| CA_29A-66A | 29 |  |  | -97 | -94 |  |  | FDD |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 |  |
| CA_29A-66C | 29 |  |  | -97 | -94 |  |  | FDD |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 |  |
| CA_29A-66A-66A | 29 |  |  | -97 | -94 |  |  | FDD |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 |  |
| CA_29A-66A-70A | 29 |  |  | -97 | -94 |  |  | FDD |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 |  |
|  | 70 |  |  | -100 | -97 | -95.2 |  |  |
| CA_29A-66A-70C | 29 |  |  | -97 | -94 |  |  | FDD |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 |  |
|  | 70 |  |  | -100 | -97 | -95.2 | -94 |  |
| CA_29A-66C-70A | 29 |  |  | -97 | -94 |  |  | FDD |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 |  |
|  | 70 |  |  | -100 | -97 | -95.2 |  |  |
| CA_29A-66A-66A-70A | 29 |  |  | -97 | -94 |  |  | FDD |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 |  |
|  | 70 |  |  | -100 | -97 | -95.2 |  |  |
| CA_29A-66C-70C | 29 |  |  | -97 | -94 |  |  | FDD |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 |  |
|  | 70 |  |  | -100 | -97 | -95.2 | -94 |  |
| CA_29A-66A-66A-70C | 29 |  |  | -97 | -94 |  |  | FDD |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 |  |
|  | 70 |  |  | -100 | -97 | -95.2 | -94 |  |
| CA_29A-70A | 29 |  |  | -97 | -94 |  |  | FDD |
|  | 70 |  |  | -100 | -97 | -95.2 | -94 |  |
| CA_29A-70C | 29 |  |  | -97 | -94 |  |  | FDD |
|  | 70 |  |  | -100 | -97 | -95.2 | -94 |  |
| CA_32A-42A | 32 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |
| CA_32A-43A | 32 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 43 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |
| CA_32A-42A-43A | 32 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 42 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |
|  | 43 |  |  | -98.5 | -95.5 | -93.7 | -92.5 | TDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: The signal power is specified per port. NOTE 4: Void NOTE 5: Applicable only if operation with 4 antenna ports is supported in the band with carrier aggregation configured. NOTE 6: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is < 60 MHz. For each channel bandwidth in Band 3, the requirement applies regardless of channel bandwidth in Band 1. NOTE 7: These requirements apply when the uplink is active in Band 1 and the separation between the lower edge of the uplink channel in Band 1 and the upper edge of the downlink channel in Band 3 is ≥ 60 MHz. For each channel bandwidth in Band 3, the requirement applies regardless of channel bandwidth in Band 1. |  |  |  |  |  |  |  |  |

Table 7.3.1A-0e: Void

For band combinations including operating band 46 (Table 5.5-1), the requirements are specified in Table 7.3.1A-0eA for the uplink in any band other than band 46 with the uplink configuration specified in Table 7.3.1-2 and Table 7.3.1A-0eC.

For band combinations including operating band 49 (Table 5.5-1), the requirements are specified in Table 7.3.1A-0eA for the uplink in any band other than Band 49 with uplink configurations specified in Table 7.3.1-2 and measurement exclusion region in Table 7.3.1A-0eD.

Table 7.3.1A-0eA: Reference sensitivity QPSK PREFSENS (CA with band 46 or Band 49)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3A-7A-46A CA_1A-3A-7A-46C CA_1A-3A-7A-46D CA_1A-3A-7A-46E | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |
|  | 7 |  |  | -98 | -95 | -93.2 | -92 |  |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_1A-3A-46A CA_1A-3A-46C CA_1A-3A-46D CA_1A-3A-46E | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 3 |  |  | -97 | -94 | -92.2 | -91 |  |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
| CA_1A-5A-7A-46A CA_1A-5A-7A-46C | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 5 |  |  | -98 | -95 |  |  |  |
|  | 7 |  |  |  | -95 | -93.2 | -92 |  |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_1A-5A-46A CA_1A-5A-46C CA_1A-5A-46D | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 5 |  |  | -98 | -95 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_2A-5A-46A | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 5 |  |  | -98 | -95 |  |  |  |
|  | 46 |  |  |  |  |  | -90 |  |
| CA_2A-5A-46A-66A CA_2A-5A-46C-66A CA_2A-5A-46D-66A CA_2A-5A-46E-66A CA_2A-5A-46A-66A-66A CA_2A-5A-46C-66A-66A CA_2A-5A-46D-66A-66A | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 5 |  |  | -98 | -95 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | FDD |
| CA_2A-7A-46A-66A | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 7 |  |  | -98 | -95 | -93.2 | -92 |  |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | FDD |
| CA_2A-13A-46A | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 13 |  |  | -97 | -94 |  |  |  |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_1A-7A-46A CA_1A-7A-46C CA_1A-7A-46D CA_1A-7A-46E | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 7 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_1A-46A CA_1A-46C CA_1A-46D CA_1A-46E | 1 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
| CA_2A-5A-46C | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 5 |  |  | -98 | -95 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_2A-5A-46D CA_2A-5A-46E | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 5 |  |  | -98 | -95 |  |  |  |
|  | 46 |  |  |  |  |  | -90 |  |
| CA_2A-7A-46A CA_2A-7A-7A-46A CA_2A-7A-46C CA_2A-7A-7A-46C CA_2A-7A-46D CA_2A-7A-7A-46D CA_2A-7A-46E CA_2A-7A-7A-46E | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 7 |  |  | -98 | -95 | -93.2 | -92 |  |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
| CA_2A-13A-46C | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 13 |  |  | -97 | -94 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_2A-13A-46A-66A CA_2A-13A-46C-66A CA_2A-13A-46D-66A CA_2A-13A-46A-66A-66A CA_2A-13A-46C-66A-66A CA_2A-13A-46D-66A-66A | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 13 |  |  | -97 | -94 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | FDD |
| CA_2A-46A CA_2A-46E CA_2A-2A-46A CA_2A-2A-46D CA_2A-46C CA_2A-46A-46A CA_2A-2A-46C CA_2A-46A-46C CA_2A-46D CA_2A-46A-46D | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_2A-46A-48A10,11 CA_2A-46A-48C10,11 CA_2A-46A-48D10,11 CA_2A-46A-48E10,11 CA_2A-46C-48A10,11 CA_2A-46C-48C10,11 CA_2A-46C-48D10,11 CA_2A-46C-48E10,11 CA_2A-46D-48A10,11 CA_2A-46D-48C10,11 CA_2A-46E-48A10,11 CA_2A-46E-48C10,11 | 2 |  |  | -97.8 | -94.8 | -93.0 | -91.7 | FDD |
|  | 46 |  |  |  |  |  | -83 | TDD |
|  | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_2A-46A-48A12 CA_2A-46A-48C12 CA_2A-46A-48D12 CA_2A-46A-48E12 CA_2A-46C-48A12 CA_2A-46C-48C12 CA_2A-46C-48D12 CA_2A-46C-48E12 CA_2A-46D-48A12 CA_2A-46E-48A12 CA_2A-46D-48C12 CA_2A-46E-48C12 | 2 |  |  | -97.8 | -94.8 | -93.0 | -91.7 | FDD |
|  | 46 |  |  |  |  |  | -83 | TDD |
|  | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_2A-46A-48A-66A10,11 CA_2A-46A-48C-66A10,11 CA_2A-46A-48D-66A10,11 CA_2A-46C-48A-66A10,11 CA_2A-46C-48C-66A10,11 CA_2A-46C-48D-66A10,11 CA_2A-46D-48A-66A10,11 CA_2A-46D-48C-66A10,11 CA_2A-46E-48A-66A10,11 | 2 |  |  | -97.7 | -94.7 | -92.9 | -91.7 | FDD |
|  | 46 |  |  |  |  |  | -83 | TDD |
|  | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
|  | 66 |  |  | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
| CA_2A-46A-48A-66A12 CA_2A-46A-48D-66A12 CA_2A-46C-48C-66A12 CA_2A-46C-48D-66A12 CA_2A-46D-48A-66A 12 CA_2A-46D-48C-66A12 CA_2A-46E-48A-66A12 | 2 |  |  | -97.7 | -94.7 | -92.9 | -91.7 | FDD |
|  | 46 |  |  |  |  |  | -83 | TDD |
|  | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
|  | 66 |  |  | -99.3 | -96.3 | -94.5 | -93.3 | FDD |
| CA_2A-46A-48C-66A12 CA_2A-46C-48A-66A12 | 2 |  |  | -97.7 | -94.7 | -92.9 | -91.7 | FDD |  |
|  | 46 |  |  |  |  |  | -83 | TDD |  |
|  | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |  |
|  | 66 |  |  | -99.3 | -96.3 | -94.5 | -93.3 | FDD |  |
| CA_2A-46A-66A | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | FDD |
| CA_2A-46A-46A-66A CA_2A-46C-66A | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |  |
|  | 46 |  |  |  |  |  | -90 | TDD |  |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | FDD |  |
| CA_2A-46A-46C-66A CA_2A-46D-66A CA_2A-46E-66A CA_2A-46A-66A-66A, CA_2A-46C-66A-66A, CA_2A-46D-66A-66A, CA_2A-46E-66A-66A | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | FDD |
| CA_2A-49A9 | 2 |  |  | -97.8 | -94.8 | -93.0 | -91.7 | FDD |
|  | 49 |  |  |  | -95.5 |  | -92.5 | TDD |
| CA_3A-7A-46C | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
|  | 7 |  |  | -98 | -95 | -93.2 | -92 |  |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_3A-7A-46D | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
|  | 7 |  |  | -98 | -95 | -93.2 | -92 |  |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_3A-7A-46E | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
|  | 7 |  |  | -98 | -95 | -93.2 | -92 |  |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_3A-46A CA_3A-46C CA_3A-46D CA_3A-46E | 3 |  | -98.7 | -97 | -94 | -92.2 | -91 | FDD |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
| CA_3C-46A | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_3A-3A-46A CA_3C-46C CA_3C-46D CA_3A-3A-46C | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_3A-7A-46A CA_3A-7C-46A CA_3A-7C-46C CA_3A-7C-46D CA_3A-7C-46E | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
|  | 7 |  |  | -98 | -95 | -93.2 | -92 |  |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_3A-32A-46A CA_3A-32A-46C CA_3A-32A-46D CA_3A-32A-46E | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
|  | 32 |  |  | -100 | -97 | -95.2 | -94 | SDL |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_3A-7A-32A-46A CA_3A-7A-32A-46C CA_3A-7A-32A-46D CA_3A-7A-32A-46E | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
|  | 7 |  |  |  | -95 | -93.2 | -92 | FDD |
|  | 32 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_4A-46A CA_4A-46C CA_4A-46A-46A CA_4A-46A-46C CA_4A-46D CA_4A-46A-46D | 4 |  |  | -100 | -97 | -95.2 | -94 | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_5A-7A-46A CA_5A-7A-46C CA_5A-7A-46D | 5 |  |  | -98 | -95 |  |  | FDD |
|  | 7 |  |  |  | -95 | -93.2 | -92 | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_5A-12A-46A, CA_5A-12A-46C, CA_5A-12A-46D | 5 |  |  | -98 | -95 |  |  | FDD |
|  | 12 |  |  | -97 | -94 |  |  |  |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_5A-46A CA_5A-46C CA_5A-46D CA_5A-46E CA_5B-46A CA_5B-46C CA_5B-46D CA_5B-46E | 5 |  |  | -98 | -95 |  |  | FDD |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
| CA_5A-46A-66A | 5 |  |  | -98 | -95 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 |  |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 |  |
| CA_5A-46C-66A | 5 |  |  | -98 | -95 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | FDD |
| CA_5A-46D-66A, CA_5A-46E-66A, CA_5A-46A-66A-66A, CA_5A-46C-66A-66A, CA_5A-46D-66A-66A, CA_5A-46E-66A-66A | 5 |  |  | -98 | -95 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 |  |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 |  |
| CA_7A-46A CA_7A-46C CA_7A-7A-46C CA_7A-46D CA_7A-46E CA_7C-46A CA_7C-46C CA_7C-46D CA_7C-46E | 7 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
| CA_7A-7A-46A CA_7A-7A-46D CA_7A-7A-46E | 7 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_7A-32A-46A CA_7A-32A-46C CA_7A-32A-46D CA_7A-32A-46E | 7 |  |  |  | -95 | -93.2 | -92 | FDD |
|  | 32 |  |  | -100 | -97 | -95.2 | -94 | SDL |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_7A-46A-66A | 7 |  |  | -97.5 | -94.5 | -92.7 | -91.5 | FDD |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
|  | 66 |  |  | -99 | -96 | -94.2 | -93 | FDD |
| CA_8A-46A CA_8A-46D CA_8A-46E CA_8B-46C CA_8B-46D | 8 | -102.2 | -99.2 | -97 | -94 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_8A-46C | 8 | -102.2 | -99.2 | -97 | -94 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_8B-46A | 8 | -102.2 | -99.2 | -97 | -94 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_11A-46A | 11 |  |  | -100 | -97 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_11A-46C | 11 |  |  | -100 | -97 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_11A-46D | 11 |  |  | -100 | -97 | -95.2 |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_11A-46E | 11 |  |  | -100 | -97 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_12A-46A CA_12A-46C CA_12A-46D | 12 |  |  | -97 | -94 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_12A-46E | 12 |  |  | -97 | -94 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_13A-46A CA_13A-46A-46A CA_13A-46A-46C CA_13A-46A-46D CA_13A-46C CA_13A-46D CA_13A-46E | 13 |  |  | -97 | -94 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_13A-46A-66A CA_13A-46C-66A CA_13A-46D-66A CA_13A-46A-66A-66A CA_13A-46C-66A-66A CA_13A-46D-66A-66A CA_13A-46E-66A | 13 |  |  | -97 | -94 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | FDD |
| CA_19A-46A CA_19A-46C CA_19A-46D CA_19A-46E | 19 |  |  | -100 | -97 | -95.2 |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_21A-46A CA_21A-46C CA_21A-46D CA_21A-46E | 21 |  |  | -100 | -97 | -95.2 |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_25A-46A CA_25A-46C CA_25A-46D | 25 | -101.2 | -98.2 | -96.5 | -93.5 | -91.7 | -90.5 | FDD |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
| CA_26A-46A | 26 |  | -99.7 | -97.57 | -94.57 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_28A-46A CA_28A-46C CA_28A-46D CA_28A-46E | 28 |  |  | -98.5 | -95.5 | -93.7 | -91 | FDD |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
| CA_29A-46A-66A | 29 |  |  | -97 | -94 |  |  | FDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | FDD |
| CA_39A-46A | 39 |  |  | -100 | -97 | -95.2 | -94 | TDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_39A-46C | 39 |  |  | -100 | -97 | -95.2 | -94 | TDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_39C-46A | 39 |  |  | -100 | -97 | -95.2 | -94 | TDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_39A-46E CA_39C-46C CA_39C-46D | 39 |  |  | -100 | -97 | -95.2 | -94 | TDD |
|  | 46 |  |  |  |  |  | -90 |  |
| CA_40A-46A CA_40A-46D CA_40A-46E CA_40C-46C CA_40C-46D CA_40D-46A CA_40D-46C | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |
|  | 46 |  |  |  | -93 |  | -90 | TDD |
| CA_40A-46C | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_40C-46A | 40 |  |  | -100 | -97 | -95.2 | -94 | TDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_41A-46A CA_41A-46C CA_41A-46D CA_41A-46E CA_41C-46A CA_41C-46C CA_41C-46D CA_41D-46A CA_41D-46C | 41 |  |  | -98 | -95 | -93.2 | -92 | TDD |
|  | 46 |  |  |  |  |  | -90 | TDD |
| CA_42A-46A | 42 |  |  | -99 | -96 | -94.2 | -93 | TDD |
|  | 46 |  |  |  |  |  | -83 | TDD |
| CA_46A-48A CA_46A-48A-48A CA_46A-48C CA_46A-48D CA_46A-48E CA_46C-48A CA_46C-48A-48A CA_46C-48C CA_46C-48D CA_46C-48E CA_46D-48A CA_46D-48C CA_46D-48A-48A CA_46E-48A CA_46E-48C | 46 |  |  |  |  |  | -83 | TDD |
|  | 48 |  |  | -99 | -96 | -94.2 | -93 | TDD |
| CA_46A-48A-66A10,11 CA_46A-48C-66A10,11 CA_46A-48D-66A10,11 CA_46A-48E-66A10,11 CA_46C-48A-66A10,11 CA_46C-48C-66A10,11 CA_46C-48D-66A10, 11 CA_46C-48E-66A10,11 CA_46D-48A-66A10,11 CA_46D-48C-66A 10,11 CA_46E-48A-66A10,11 CA_46E-48C-66A10,11 | 46 |  |  |  |  |  | -83 | FDD |
|  | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
|  | 66 |  |  | -99.3 | -96.3 | -94.5 | -93.2 | FDD |
| CA_46A-48A-66A12 CA_46A-48C-66A12 CA_46A-48D-66A12 CA_46A-48E-66A12 CA_46C-48A-66A12 CA_46C-48C-66A12 CA_46C-48D-66A12 CA_46C-48E-66A12 CA_46D-48A-66A12 CA_46D-48C-66A 12 CA_46E-48A-66A12 CA_46E-48C-66A12 | 46 |  |  |  |  |  | -83 | FDD |
|  | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
|  | 66 |  |  | -99.3 | -96.3 | -94.5 | -93.2 | FDD |
| CA_46A-48A-71A CA_46A-48A-48A-71A CA_46A-48C-71A CA_46C-48A-71A CA_46C-48C-71A CA_46C-48A-48A-71A | 46 |  |  |  |  |  | -83 | TDD |  |
|  | 48 |  |  | -99 | -96 | -94.2 | -93 | TDD |  |
|  | 71 |  |  | -97.2 | -94.2 | -92.0 | -87.5 | FDD |  |
| CA_46A-66A CA_46A-46A-66A CA_46A-66A-66A CA_46A-66C CA_46A-46C-66A CA_46A-46D-66A CA_46C-66A CA_46C-66A-66A CA_46D-66A CA_46D-66A-66A CA_46E-66A CA_46E-66A-66A | 46 |  |  |  |  |  | -90 | TDD |
|  | 66 |  |  | -99.5 | -96.5 | -94.7 | -93.5 | FDD |
| CA_46A-70A | 46 |  |  |  |  |  | -90 | TDD |
|  | 70 |  |  | -100 | -97 | -95.2 |  | FDD |
| CA_46A-71A CA_46C-71A CA_46D-71A | 46 |  |  |  |  |  | -90 | FDD |
|  | 71 |  |  | -97.2 | -94.2 | -92.0 | -87.5 | TDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD/FS3 as described in Annex A.5.1.1/A.5.2.1/A.5.4.1. NOTE 3: The signal power is specified per port. NOTE 4: Void NOTE 5: The requirement for B46 does not apply when there is at least one individual RE within the B46 downlink transmission bandwidth which falls into the reference sensitivity exclusion region as specified in Table 7.3.1A-0eC. NOTE 6: Void NOTE 7: 7 indicates that the requirement is modified by -0.5 dB when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 8: When Band 46 have self interference problems by dual uplink CA, then the requirements not apply in exclusion zone which is frequency range within (harmonics frequency region + FHD) and IMD frequency region as follow. NOTE 9: The requirement for B49 does not apply when there is at least one individual RE within the B49 downlink transmission bandwidth which falls into the reference sensitivity exclusion region as specified in Table 7.3.1A-0eD. NOTE 10: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band which excludes band 46 and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_2-46-48, CA_46-48-66, and CA_2-46-48-66. For harmonic issue not related with band 46, the uplink configuration of CA_2-48, CA_48-66 and CA_2-48-66 in Table 7.3.1A-0b can be used. NOTE 11: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 12: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. IMD frequency range DL_CA configuration; UL_CA configuration; Exclusion zone center frequency; Exclusion zone BW. CA_1A-3A-46A; CA_1A-3A; 2*fc_1A + fc_3A; 2*BW_1A + BW_3A. CA_1A-3A-46A; CA_1A-3A; fc_1A – 2*fc_3A; BW_1A + 2*BW_3A. IMD frequency range DL_CA configuration; UL_CA configuration; Exclusion zone center frequency; Exclusion zone BW. CA_1A-5A-46A; CA_1A-5A; 2*fc_1A + 2*fc_5A; 2*BW_1A + 2*BW_5A. CA_1A-7A-46A; CA_1A-7A; 3*fc_7A - fc_1A; 3*BW_7A + BW_1A. CA_5A-7A-46A; CA_5A-7A; 2*fc_7A + fc_5A; 2*BW_7A + BW_5A. IMD frequency range DL_CA configuration; UL_CA configuration; Exclusion zone center frequency; Exclusion zone BW. CA_2A-5A-46D; CA_2A-5A; 3*fc_5A + 1*fc_2A; 3*BW_5A + 1*BW_2A. CA_2A-5A-46D; CA_2A-5A; 2*fc_5A – 3*fc_2A; 2*BW_5A + 3*BW_2A. IMD frequency range DL_CA configuration; UL_CA configuration; Exclusion zone center frequency; Exclusion zone BW. CA_5A-46D-66A; CA_5A-66A; 3*fc_66A + 1*fc_5A; 3*BW_66A + 1*BW_5A. CA_5A-46D-66A; CA_5A-66A; 2*fc_66A – 3*fc_5A; 2*BW_66A + 3*BW_5A. CA_5A-46D-66A; CA_5A-66A; 2*fc_66A + 3*fc_5A; 2*BW_66A + 3*BW_5A. IMD frequency range DL_CA configuration; UL_CA configuration; Exclusion zone center frequency; Exclusion zone BW. CA_13A-46D-66A; CA_13A-66A; 2*fc_66A + 2*fc_13A; 2*BW_66A + 2*BW_13A. CA_13A-46D-66A; CA_13A-66A; 2*fc_66A + 3*fc_13A; 2*BW_66A + 3*BW_13A. IMD frequency range DL_CA configuration; UL_CA configuration; Exclusion zone center frequency; Exclusion zone BW. CA_2A-13A-46D CA_2A-13A-46A-46D CA_2A-13A-46A-46C CA_2A-13A-46C CA_2A-13A-46A-46A CA_2A-13A-46A CA_2A-13A-46E; CA_2A-13A; 3*fc_13A + 1*fc_2A; 3*BW_13A + 1*BW_2A. IMD frequency range DL_CA configuration; UL_CA configuration; Exclusion zone center frequency; Exclusion zone BW. CA_2A-46E-66A CA_2A-46D-66A CA_2A-46C-66A CA_2A-46A-66A; CA_2A-66A; 2*fc_2A + 1*fc_66A 2*fc_66A + 1*fc_2A 1*fc_2A – 4*fc_66A 1*fc_66A – 4*fc_2A; 2*BW_2A + 1*BW_66A 2*BW_66A + 1*BW_2A 1*BW_2A + 4*BW_66A 1*BW_66A + 4*BW_2A. IMD frequency range DL_CA configuration; UL_CA configuration; Exclusion zone center frequency; Exclusion zone BW. CA_2A-46E-48A, CA_2A-46D-48A CA_2A-46D-48C CA_2A-46C-48C CA_2A-46A-48C CA_2A-46C-48A CA_2A-46A-48A; CA_2A-48A; 1*fc_2A + 1*fc_48A 2*fc_48A – 1*fc_2A; 1*BW_2A + 1*BW_48A 2*BW_48A + 1*BW_2A. IMD frequency range DL_CA configuration; UL_CA configuration; Exclusion zone center frequency; Exclusion zone BW. CA_46D-48C-66A CA_46C-48C-66A CA_46A-48C-66A CA_46D-48A-66A CA_46C-48A-66A CA_46A-48A-66A; CA_48A-66A; 1*fc_48A + 1*fc_66A 2*fc_48A – 1*fc_66A; 1*BW_48A + 1*BW_66A 2*BW_48A + 1*BW_66A. |

Table 7.3.1A-0eB: Void

Table 7.3.1A-0eC specifies the Band 46 reference measurement exclusion region for different licensed component carriers and channel bandwidth. The exclusion region is defined according to the licensed component carrier channel bandwidth. The UL configurations to be adopted for the test are specified in Table 7.3.1-2. The exclusion region in Table 7.3.1A-0eC is specified for the case of 10MHz and 20MHz channel bandwidth in Band 46.

Table 7.3.1A-0eC: Band 46 Reference sensitivity measurement exclusion region in MHz.

| Licensed Component Carriers / E-UTRA Band / Harmonic order / Channel BW in UL |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Licensed Component Carriers | Harmonic order | 5MHz | 10MHz | 15MHz | 20MHz |
| 1 | 3 | +/- 15 | +/- 23 | +/- 35 | +/- 45 |
| 2 | 3 | +/- 15 | +/- 23 | +/- 35 | +/- 45 |
| 3 | 3 | +/- 15 | +/- 23 | +/- 35 | +/- 45 |
| 4 | 3 | +/- 15 | +/- 23 | +/- 35 | +/- 45 |
| 5 | 7 | +/- 10 | +/- 20 |  |  |
| 71 | 2 | +/- 15 | +/- 25 | +/- 38 | +/- 50 |
| 8 | 6 | +/- 10 | +/- 20 |  |  |
| 11 | 4 | +/- 15 | +/- 25 |  |  |
| 12 | 8 | +/- 10 | +/- 20 |  |  |
| 13 | 7 | +/- 10 | +/- 20 |  |  |
| 19 | 7 | +/- 10 | +/- 20 | +/- 30 |  |
| 21 | 4 | +/- 15 | +/- 25 | +/- 38 |  |
| 25 | 3 | +/- 15 | +/- 23 | +/- 35 | +/- 45 |
| 26 | 7 | +/- 10 | +/- 20 | +/- 30 |  |
| 28 | 7 | +/- 10 | +/- 20 | +/- 30 | +/- 40 |
| 28 | 8 | +/- 10 | +/- 20 | +/- 30 | +/- 40 |
| 39 | 3 | +/- 15 | +/- 23 | +/- 35 | +/- 45 |
| 41 | 2 | +/- 15 | +/- 25 | +/- 38 | +/- 50 |
| 66 | 3 | +/- 15 | +/- 23 | +/- 35 | +/- 45 |
| NOTE 1: Even though UL harmonic does not fall directly into Band 46 the exclusion region still applies. NOTE 2: The center of the exclusion region is obtained by multiplying the UL channel center frequency by the harmonic order. |  |  |  |  |  |

Table 7.3.1A-0eD specifies the Band 49 reference measurement exclusion region for different licensed component carriers and channel bandwidth. The exclusion region is defined according to the licensed component carrier channel bandwidth. The UL configurations to be adopted for the test are specified in Table 7.3.1-2.

Table 7.3.1A-0eD: Band 49 reference sensitivity measurement exclusion region in MHz.

| Licensed Component Carriers / E-UTRA Band / Harmonic order / Channel BW in UL |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Licensed Component Carriers | Harmonic order | 5MHz | 10MHz | 15MHz | 20MHz |
| 21 | 2 | +/- 12.5 | +/- 25 | +/- 37.5 | +/- 50 |
| NOTE 1: Even though UL harmonic does not fall directly into Band 49 the exclusion region still applies. NOTE 2: The center of the exclusion region is obtained by multiplying the UL channel center frequency by the harmonic order. |  |  |  |  |  |

In all cases for single uplink inter-band CA, unless given by Table 7.3.1-3 for the band with the active uplink carrier, the applicable reference sensitivity requirements shall be verified with the network signalling value NS_01 (Table 6.2.4-1) configured.

For inter-band carrier aggregation with one component carrier per operating band (up to four downlinks) and the uplink assigned to two E-UTRA bands the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.1-1, Table 7.3.1-1a and Table 7.3.1-2. The reference sensitivity is defined to be met with all downlink component carriers active and both of the uplink carriers active.

For E-UTRA CA configurations with uplink and downlink assigned to two E-UTRA bands given in Table 7.3.1A-0f, the reference sensitivity is defined only for the specific uplink and downlink test points which are specified in Table 7.3.1A-0f. For E-UTRA CA configurations with uplink assigned to two E-UTRA bands and downlink assigned to three E-UTRA bands given in Table 7.3.1A-0g, the reference sensitivity is defined only for the specific uplink and downlink test points which are specified in Table 7.3.1A-0g. For these test points the reference sensitivity requirement specified in Table 7.3.1-1 and Table 7.3.1-1a are relaxed by the amount of the corresponding parameter MSD given in Table 7.3.1A-0f and Table 7.3.1A-0g.

The allowed exceptions defined in Table 7.3.1A-0a and Table 7.3.1A-0b for inter-band carrier aggregation with a single active uplink are also applicable for dual uplink operation.

Table 7.3.1A-0f: 2DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | UL Fc (MHz) | UL/DL BW (MHz) | UL CLRB | DL Fc (MHz) | MSD (dB) | Duplex mode |  |
| CA_1A-3A | 1 | 1950 | 5 | 25 | 2140 | 23 | FDD | IMD3 |
|  | 3 | 1760 | 5 | 25 | 1855 | N/A |  | N/A |
| CA_1A-8A | 1 | 1965 | 5 | 25 | 2155 | 6 | FDD | IMD4 |
|  | 8 | 887.5 | 5 | 25 | 932.5 | N/A |  | N/A |
| CA_2A-4A | 2 | 1860 | 20 | 502 | 1940 | 5 | FDD | IMD3 |
|  | 4 | 1752.5 | 5 | 25 | 2152.5 | N/A |  | N/A |
| CA_2A-4A | 2 | 1868.3 | 5 | 25 | 1948.3 | N/A | FDD | N/A |
|  | 4 | 1735 | 5 | 25 | 2135 | 5 |  | IMD5 |
| CA_2A-46A | 2 | 1880 | 5 | 25 | 1960 | 12.0 | FDD | IMD34 |
|  | 46 | 5720 | 20 | 100 | 5720 | N/A |  | N/A |
| CA_2A-48A | 2 | 1852.5 | 5 | 25 | 1932.5 | [12] | FDD | IMD4 |
|  | 48 | 3625 | 20 | 100 | 3625 | N/A | TDD | N/A |
| CA_2A-49A | 2 | 1852.5 | 5 | 25 | 1932.5 | [12] | FDD | IMD4 |
|  | 49 | 3625 | 20 | 100 | 3625 | N/A | TDD | N/A |
| CA_2A-66A | 2 | 1855 | 5 | 25 | 1935 | 20 | FDD | IMD3 |
|  | 66 | 1775 | 5 | 25 | 2175 | N/A |  | N/A |
| CA_2A-66A | 2 | 1883.3 | 5 | 25 | 1963.3 | N/A | FDD | N/A |
|  | 66 | 1750 | 5 | 25 | 2150 | 4 |  | IMD5 |
| CA_3A-5A | 3 | 1771 | 10 | 50 | 1866 | 4 | FDD | IMD4 |
|  | 5 | 838 | 5 | 25 | 883 | N/A |  | N/A |
| CA_3A-5A | 3 | 1721 | 10 | 50 | 1816 | N/A | FDD | N/A |
|  | 5 | 838 | 5 | 25 | 883 | 24 |  | IMD24 |
| CA_3A-7A | 3 | 1730 | 5 | 25 | 1825 | N/A | FDD | N/A |
|  | 7 | 2535 | 10 | 50 | 2655 | 13 |  | IMD4 |
| CA_3A-8A | 3 | 1755 | 10 | 50 | 1850 | N/A | FDD | N/A |
|  | 8 | 900 | 5 | 25 | 945 | 8 |  | IMD44 |
| CA_3A-8A | 3 | 1747.5 | 10 | 50 | 1842.5 | 6.4 | FDD | IMD5 |
|  | 8 | 897.5 | 5 | 25 | 942.5 | N/A |  | N/A |
| CA_3A-18A | 3 | 1721 | 5 | 25 | 1816 | 4 | FDD | IMD4 |
|  | 18 | 823 | 5 | 25 | 868 | N/A |  | N/A |
| CA_3A-19A | 3 | 1771 | 5 | 25 | 1866 | 4 | FDD | IMD4 |
|  | 19 | 838 | 5 | 25 | 883 | N/A |  | N/A |
| CA_3A-19A | 3 | 1721 | 5 | 25 | 1816 | N/A | FDD | N/A |
|  | 19 | 838 | 5 | 25 | 883 | 27 |  | IMD24 |
| CA_3A-20A | 3 | 1775 | 5 | 25 | 1870 | 4 | FDD | IMD4 |
|  | 20 | 840 | 5 | 25 | 799 | N/A |  | N/A |
| CA_3A-20A | 3 | 1735 | 5 | 25 | 1830 | N/A | FDD | N/A |
|  | 20 | 847 | 5 | 25 | 806 | 9 |  | IMD4 |
| CA_3A-26A | 3 | 1771 | 5 | 25 | 1866 | 4 | FDD | IMD4 |
|  | 26 | 838 | 5 | 25 | 883 | N/A |  | N/A |
| CA_3A-26A | 3 | 1721 | 5 | 25 | 1816 | N/A | FDD | N/A |
|  | 26 | 838 | 5 | 25 | 883 | 26 |  | IMD24 |
| CA_3A-41A | 3 | 1740 | 5 | 25 | 1835 | 8.2 | FDD | IMD4 |
|  | 41 | 2657.5 | 5 | 25 | 2657.5 | N/A | TDD | N/A |
| CA_3A-42A | 3 | 1740 | 5 | 25 | 1835 | 29.8 | FDD | IMD24 |
|  | 42 | 3575 | 5 | 25 | 3575 | N/A | TDD | N/A |
| CA_3A-42A | 3 | 1765 | 5 | 25 | 1860 | 8.0 | FDD | IMD44 |
|  | 42 | 3435 | 5 | 25 | 3435 | N/A | TDD | N/A |
| CA_4A-5A | 4 | 1721 | 5 | 25 | 2121 | N/A | FDD | N/A |
|  | 5 | 838 | 5 | 25 | 883 | 26 |  | IMD24 |
| CA_4A-7A | 4 | 1730 | 5 | 25 | 2130 | N/A | FDD | N/A |
|  | 7 | 2535 | 5 | 25 | 2655 | 15 |  | IMD4 |
| CA_5A-7A | 5 | 834 | 5 | 25 | 879 | 12 | FDD | IMD34 |
|  | 7 | 2547 | 10 | 50 | 2667 | N/A |  | N/A |
| CA_5A-66A | 5 | 838 | 5 | 25 | 883 | 30 | FDD | IMD24 |
|  | 66 | 1721 | 5 | 25 | 2121 | N/A |  | N/A |
| CA_7A-20A | 7 | 2512 | 10 | 50 | 2632 | N/A | FDD | N/A |
|  | 20 | 851 | 5 | 25 | 810 | 12 |  | IMD34 |
| CA_7A-26A | 7 | 2556 | 5 | 25 | 2676 | N/A | FDD-FDD | N/A |
|  | 26 | 837 | 5 | 25 | 882 | 16.0 |  | IMD3 |
|  | 7 | 2567.5 | 5 | 25 | 2687.5 | 2.5 | FDD-FDD | IMD5 |
|  | 26 | 816.5 | 5 | 25 | 861.5 | N/A |  | N/A |
| CA_8A-41A | 8 | 882.5 | 5 | 25 | 927.5 | 12.1 | FDD | IMD34 |
|  | 41 | 2685 | 10 | 50 | 2685 | N/A | TDD | N/A |
| CA_19A-42A3 | 19 | N/A | N/A | N/A | N/A | N/A | FDD | N/A |
|  | 42 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
| CA_21A-28A | 21 | 1450.4 | 5 | 25 | 1498.4 | [2.5] | FDD | IMD5 |
|  | 28 | 735.5 | 5 | 25 | 790.5 | N/A | TDD | N/A |
| CA_21A-42A3 | 21 | N/A | N/A | N/A | N/A | N/A | FDD | N/A |
|  | 42 | N/A | N/A | N/A | N/A | N/A | TDD | N/A |
| CA_28A-42A | 28 | 705.5 | 5 | 25 | 760.5 | [5.5] | FDD | IMD5 |
|  | 42 | 3582.5 | 5 | 25 | 3582.5 | N/A | TDD | N/A |
| CA_40A-42A | 40 | 2350 | 5 | 25 | 2350 | N/A | TDD | N/A |
|  | 42 | 3500 | 5 | 25 | 3550 | 5 |  | IMD4 |
|  | 40 | 2350 | 5 | 25 | 2350 | 5 | TDD | IMD4 |
|  | 42 | 3525 | 5 | 25 | 3525 | N/A |  | N/A |
| NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,c) as defined in subclause 6.2.5A NOTE 2: RBSTART = 0 NOTE 3: No requirements apply when there is at least one individual RE within the intermodulation generated by the dual uplink is within the downlink transmission bandwidth of the FDD band. The reference sensitivity should only be verified when this is not the case (the requirements specified in clause 7.3.1 apply). NOTE 4: This band is subject to IMD5 also which MSD is not specified. NOTE 5: Void NOTE 6: For operations with 4 antenna ports, the MSD in the applicable bands shall be modified by the absolute value of ΔRIB,4R in Table 7.3.1-1a when MSD > 0. |  |  |  |  |  |  |  |  |

Table 7.3.1A-0g: 3DL/2UL interband Reference sensitivity QPSK PREFSENS and uplink/downlink configurations

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |  |  | Source of IMD |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA | EUTRA CA | EUTRA band | UL Fc | UL BW | UL | DL Fc | DL BW | MSD | Duplex mode |  |
| DL Configuration | UL Configuration |  | (MHz) | (MHz) | CLRB | (MHz) | (MHz) | (dB) |  |  |
| CA_1A-3A-28A | CA_1A-28A | 1 | 1975 | 5 | 25 | 2165 | 5 | N/A | FDD | N/A |
|  |  | 28 | 710.5 | 5 | 25 | 765.5 | 5 | N/A |  | N/A |
|  |  | 3 | 1723.5 | 5 | 25 | 1818.5 | 5 | 4.0 |  | IMD5 |
|  | CA_3A-28A | 3 | 1780 | 5 | 25 | 1875 | 5 | N/A | FDD | N/A |
|  |  | 28 | 710.5 | 5 | 25 | 765.5 | 5 | N/A |  | N/A |
|  |  | 1 | 1949 | 5 | 25 | 2139 | 5 | 11.0 |  | IMD4 |
| CA_1A-3A-40A | CA_1A-3A | 1 | 1950 | 5 | 25 | 2140 | 5 | N/A | FDD | N/A |
|  |  | 3 | 1735 | 5 | 25 | 1830 | 5 | N/A | FDD | N/A |
|  |  | 40 | 2380 | 5 | 25 | 2380 | 5 | 8.0 | TDD | IMD5 |
| CA_1A-3A-41A | CA_1A-3A | 1 | 1977.5 | 5 | 25 | 2167.5 | 5 | N/A | FDD | N/A |
|  |  | 3 | 1712.5 | 5 | 25 | 1807.5 | 5 | N/A | FDD | N/A |
|  |  | 41 | 2507.5 | 5 | 25 | 2507.5 | 5 | 5.0 | TDD | IMD5 |
| CA_1A-3A-42A | CA_1A-3A | 1 | 1922.5 | 5 | 25 | 2112.5 | 5 | N/A | FDD | N/A |
|  |  | 3 | 1782.5 | 5 | 25 | 1877.5 | 5 | N/A | FDD | N/A |
|  |  | 42 |  |  |  | 3425 | 5 | 13.0 | TDD | IMD4 |
| CA_1A-5A-7A | CA_1A-7A | 1 | 1968 | 5 | 25 | 2158 | 5 | N/A | FDD | N/A |
|  |  | 7 | 2512 | 10 | 50 | 2632 | 10 | N/A |  | N/A |
|  |  | 5 | 835 | 5 | 25 | 880 | 5 | 1.0 |  | IMD5 |
| CA_1A-5A-40A | CA_1A-5A | 1 | 1977.5 | 5 | 25 | 2167.5 | 5 | N/A | FDD | N/A |
|  |  | 5 | 826.5 | 5 | 25 | 871.5 | 5 | N/A | FDD | N/A |
|  |  | 40 | 2305 | 10 | 50 | 2305 | 10 | 9.0 | TDD | IMD4 |
| CA_1A-7A-26A | CA_1A-7A | 1 | 1965 | 5 | 25 | 2155 | 5 | N/A | FDD | N/A |
|  |  | 7 | 2510 | 10 | 50 | 2630 | 10 | N/A |  | N/A |
|  |  | 26 | 830 | 5 | 50 | 875 | 5 | 3.5 |  | IMD5 |
| CA_1A-7A-28A | CA_1A-7A | 1 | 1935 | 5 | 25 | 2125 | 5 | N/A | FDD | N/A |
|  |  | 7 | 2510 | 10 | 50 | 2630 | 10 | N/A |  | N/A |
|  |  | 28 | 730 | 10 | 50 | 785 | 10 | 4.5 |  | IMD5 |
|  | CA_1A-28A | 1 | 1935 | 5 | 25 | 2125 | 5 | N/A | FDD | N/A |
|  |  | 28 | 730 | 10 | 50 | 785 | 10 | N/A |  | N/A |
|  |  | 7 | 2545 | 10 | 50 | 2665 | 10 | 28.0 |  | IMD2 |
| CA_1A-28A-42A | CA_1A-28A | 1 | 1955 | 5 | 25 | 2145 | 5 | N/A | FDD | N/A |
|  |  | 28 | 735 | 5 | 25 | 790 | 5 | N/A | FDD | N/A |
|  |  | 42 | 3425 | 5 | 25 | 3425 | 5 | 15.0 | TDD | IMD3 |
|  | CA_28A-42A | 28 | 710.5 | 5 | 25 | 765.5 | 5 | N/A | FDD | N/A |
|  |  | 42 | 3560 | 5 | 25 | 3560 | 5 | N/A | TDD | N/A |
|  |  | 1 | 1949 | 5 | 25 | 2139 | 5 | 11.0 | FDD | IMD3 |
| CA_2A-12A-30A | CA_2A-12A | 2 | 1885 | 5 | 25 | 1965 | 5 | N/A | FDD | N/A |
|  |  | 12 | 708.5 | 5 | 25 | 738.5 | 5 | N/A |  | N/A |
|  |  | 30 | 2308 | 5 | 25 | 2353 | 5 | 12.0 |  | IMD4 |
| CA_2A-2A-4A-5A | CA_2A-5A | 2 | 1900 | 5 | 25 | 1980 | 5 | N/A | FDD | N/A |
|  |  | 5 | 834 | 5 | 25 | 879 | 5 |  |  | N/A |
|  |  | 4 | 1732 | 5 | 25 | 2132 | 5 | 7.6 |  | IMD4 |
| CA_2A-4A-13A | CA_2A-13A | 2 | 1855 | 5 | 25 | 1935 | 5 | N/A | FDD | N/A |
|  |  | 13 | 782 | 5 | 25 | 751 | 5 |  |  | N/A |
|  |  | 4 | 1746 | 5 | 25 | 2146 | 5 | 7.6 |  | IMD4 |
|  | CA_4A-13A | 4 | 1750 | 5 | 25 | 2150 | 5 | N/A | FDD | N/A |
|  |  | 13 | 780 | 5 | 25 | 749 | 5 |  |  | N/A |
|  |  | 2 | 1860 | 5 | 25 | 1940 | 5 | 6.2 |  | IMD4 |
| CA_2A-2A-5A-66A-66A, CA_2A-5A-66A, CA_2A-5A-66B, CA_2A-5A-66C, CA_2A-5B-66A, CA_2A-5B-66B, CA_2A-5B-66C, CA_2A-2A-5A-66A, CA_2A-2A-5A-66B, CA_2A-2A-5A-66C, CA_2A-5A-66A-66A | CA_2A-5A | 2 | 1900 | 5 | 25 | 1980 | 5 | N/A | FDD | N/A |
|  |  | 5 | 834 | 5 | 25 | 879 | 5 |  |  | N/A |
|  |  | 66 | 1712 | 5 | 25 | 2132 | 5 | 7.2 |  | IMD4 |
| CA_2A-5B-66A-66A | CA_2A-5A | 2 | 1900 | 5 | 25 | 1980 | 5 | N/A | FDD | N/A |
|  |  | 5 | 834 | 5 | 25 | 879 | 5 |  |  | N/A |
|  |  | 66 | 1712 | 5 | 25 | 2132 | 5 | 7.2 |  | IMD4 |
| CA_2A-13A-66A-66B | CA_2A-13A | 2 | 1860 | 5 | 25 | 1940 | 5 | N/A | FDD | N/A |
|  |  | 13 | 782 | 5 | 25 | 751 | 5 |  |  | N/A |
|  |  | 66 | 1736 | 5 | 25 | 2156 | 5 | 7.2 |  | IMD4 |
| CA_2A-13A-66A-66B | CA_13A-66A | 2 | 1880 | 5 | 25 | 1960 | 5 | 6.2 | FDD | IMD4 |
|  |  | 13 | 782 | 5 | 25 | 751 | 5 | N/A |  | N/A |
|  |  | 66 | 1762 | 5 | 25 | 2162 | 5 |  |  | N/A |
| CA_2A-48A-66A CA_2A-48C-66A | CA_48A-66A | 2 | 1880 | 5 | 25 | 1960 | 5 | 28.3 | FDD-TDD | IMD2 |
|  |  | 48 | 3695 | 5 | 25 | 3695 | 5 | N/A |  | N/A |
|  |  | 66 | 1735 | 5 | 25 | 2135 | 5 | N/A |  | N/A |
| CA_2A-48A-66A CA_2A-48C-66A | CA_2A-48A | 2 | 1905 | 5 | 25 | 1985 | 5 | N/A | FDD-TDD | N/A |
|  |  | 48 | 3560 | 5 | 25 | 3560 | 5 | N/A |  | N/A |
|  |  | 66 | 1755 | 5 | 25 | 2155 | 5 | 12.1 |  | IMD4 |
| CA_3A-5A-7A | CA_3A-5A | 3 | 1780 | 10 | 50 | 1875 | 10 | N/A | FDD | N/A |
|  |  | 5 | 845 | 5 | 25 | 890 | 5 | N/A |  | N/A |
|  |  | 7 | 2505 | 10 | 50 | 2625 | 10 | 30.0 |  | IMD21 |
|  | CA_3A-7A | 3 | 1725 | 10 | 50 | 1820 | 10 | N/A | FDD | N/A |
|  |  | 7 | 2565 | 10 | 50 | 2685 | 10 | N/A |  | N/A |
|  |  | 5 | 840 | 5 | 25 | 885 | 5 | 19.0 |  | IMD3 |
| CA_3A-7A-8A | CA_3A-7A | 3 | 1735 | 5 | 25 | 1830 | 5 | N/A | FDD | N/A |
|  |  | 7 | 2530 | 10 | 50 | 2650 | 10 |  |  | N/A |
|  |  | 8 | 895 | 5 | 25 | 940 | 5 | 18.0 |  | IMD3 |
|  | CA_3A-8A | 3 | 1780 | 5 | 25 | 1875 | 5 | N/A | FDD | N/A |
|  |  | 8 | 890 | 5 | 25 | 935 | 5 |  |  | N/A |
|  |  | 7 | 2550 | 10 | 50 | 2670 | 10 | 29.0 |  | IMD2+IMD34 |
| CA_3A-7A-20A | CA_3A-7A | 3 | 1737 | 5 | 25 | 1832 | 5 | N/A | FDD | N/A |
|  |  | 7 | 2543 | 10 | 50 | 2663 | 10 | N/A |  | N/A |
|  |  | 20 | 847 | 10 | 20 | 806 | 10 | 10.5 |  | IMD2 |
|  | CA_3A-20A | 3 | 1775 | 10 | 50 | 1870 | 10 | N/A | FDD | N/A |
|  |  | 20 | 855 | 5 | 25 | 896 | 5 | N/A |  | N/A |
|  |  | 7 | 2510 | 10 | 50 | 2630 | 10 | 26.0 |  | IMD21 |
| CA_3A-7A-26A | CA_3A-7A | 3 | 1720 | 5 | 25 | 1815 | 5 | N/A | FDD | N/A |
|  |  | 7 | 2560 | 10 | 50 | 2680 | 10 | N/A |  | N/A |
|  |  | 26 | 835 | 5 | 25 | 880 | 5 | 17.5 |  | IMD3 |
| CA_3A-7A-26A | CA_3A-26A | 3 | 1780 | 5 | 25 | 1875 | 5 | N/A | FDD | N/A |
|  |  | 26 | 845 | 5 | 25 | 890 | 5 | N/A |  | N/A |
|  |  | 7 | 2505 | 10 | 50 | 2625 | 10 | 29.0 |  | IMD21 |
| CA_3A-7A-28A | CA_3A-7A | 3 | 1747 | 5 | 25 | 1842 | 5 | N/A | FDD | N/A |
|  |  | 7 | 2543 | 5 | 25 | 2663 | 5 | N/A |  | N/A |
|  |  | 28 | 741 | 5 | 25 | 796.0 | 5 | 20.0 |  | IMD2 |
|  | CA_3A-28A | 3 | 1712.5 | 5 | 25 | 1807.5 | 5 | N/A | FDD | N/A |
|  |  | 28 | 743 | 5 | 25 | 798 | 5 | N/A |  | N/A |
|  |  | 7 | 2562 | 5 | 25 | 2682 | 5 | 17.0 |  | IMD3 |
|  | CA_7A-28A | 7 | 2543 | 5 | 25 | 2663 | 5 | N/A | FDD | N/A |
|  |  | 28 | 710.5 | 5 | 25 | 765.5 | 5 | N/A |  | N/A |
|  |  | 3 | 1737.5 | 5 | 25 | 1832.5 | 5 | 26.0 |  | IMD2 |
| CA_3A-7A-32A | CA_3A-7A | 3 | 1775 | 5 | 25 | 1870 | 5 | N/A | FDD | N/A |
|  |  | 7 | 2510 | 10 | 50 | 2630 | 10 | N/A |  | N/A |
|  |  | 32 | - | - | - | 1470 | 5 | 10.5 |  | IMD4 |
| CA_3A-8A-38A | CA_3A-8A | 3 | 1720 | 5 | 25 | 1815 | 5 | N/A | FDD-TDD | N/A |
|  |  | 8 | 890 | 5 | 25 | 935 | 5 | N/A |  | N/A |
|  |  | 38 | 2610 | 5 | 25 | 2610 | 5 | 26.4 |  | IMD2 |
|  | CA_3A-8A | 3 | 1750 | 5 | 25 | 1845 | 5 | N/A | FDD-TDD | N/A |
|  |  | 8 | 900 | 5 | 25 | 945 | 5 | N/A |  | N/A |
|  |  | 38 | 2600 | 5 | 25 | 2600 | 5 | 15.7 |  | IMD3 |
| CA_3A-11A-18A | CA_3A-11A | 3 | 1725 | 5 | 25 | 1820 | 5 | N/A | FDD | N/A |
|  |  | 11 | 1440 | 5 | 25 | 1448 | 5 | N/A |  | N/A |
|  |  | 18 | 825 | 5 | 25 | 870 | 5 | 4.9 |  | IMD5 |
|  | CA_11A-18A | 11 | 1432 | 5 | 25 | 1481 | 5 | N/A | FDD | N/A |
|  |  | 18 | 820 | 5 | 25 | 865 | 5 | N/A |  | N/A |
|  |  | 3 | 1753 | 5 | 25 | 1848 | 5 | 4.0 |  | IMD5 |
| CA_3A-11A-26A | CA_3A-11A | 3 | 1725 | 5 | 25 | 1820 | 5 | N/A | FDD | N/A |
|  |  | 11 | 1440 | 5 | 25 | 1448 | 5 | N/A |  | N/A |
|  |  | 26 | 825 | 5 | 25 | 870 | 5 | 4.9 |  | IMD5 |
|  | CA_3A-26A | 3 | 1782.5 | 5 | 25 | 1877.5 | 5 | N/A | FDD | N/A |
|  |  | 26 | 816.5 | 5 | 25 | 861.5 | 5 | N/A |  | N/A |
|  |  | 11 | 1435.5 | 5 | 25 | 1483.5 | 5 | 5.0 |  | IMD5 |
|  | CA_11A-26A | 11 | 1440 | 5 | 25 | 1488 | 5 | N/A | FDD | N/A |
|  |  | 26 | 824 | 5 | 25 | 869 | 5 | N/A |  | N/A |
|  |  | 3 | 1761 | 5 | 25 | 1856 | 5 | 4.5 |  | IMD5 |
| CA_3A-19A-21A | CA_19A-21A | 19 | 832.5 | 5 | 25 | 877.5 | 5 | N/A | FDD | N/A |
|  |  | 21 | 1460.4 | 5 | 25 | 1508.4 | 5 | N/A |  | N/A |
|  |  | 3 | 1774.6 | 5 | 25 | 1869.6 | 5 | 4.0 |  | IMD5 |
| CA_3A-21A-28A | CA_3A-21A | 3 | 1782 | 5 | 25 | 1877 | 5 | N/A | FDD | N/A |
|  |  | 21 | 1451 | 5 | 25 | 1499 | 5 | N/A |  | N/A |
|  |  | 28 | 734 | 5 | 25 | 789 | 5 | 3.0 |  | IMD5 |
| CA_3A-28A-41A | CA_3A-41 | 3 | 1720 | 5 | 25 | 1815 | 5 | N/A | FDD | N/A |
|  |  | 41 | 2510 | 5 | 25 | 2510 | 5 | N/A | TDD | N/A |
|  |  | 28 | 735 | 5 | 25 | 790 | 5 | 26.0 | FDD | IMD21 |
| CA_3A-41A-42A | CA_41A-42A | 41 | 2640 | 10 | 50 | 2640 | 10 | N/A | TDD | N/A |
|  |  | 42 | 3425 | 10 | 50 | 3425 | 10 |  | TDD | N/A |
|  |  | 3 | 1760 | 5 | 25 | 1855 | 5 | 16.0 | FDD | IMD3 |
| CA_5A-46D-66A | CA_5A_46D | 5 | 834 | 5 | 25 | 879 | 5 | N/A | FDD-TDD | N/A |
|  |  | 46 | 5491 | 20 | 100 | 5491 | 20 |  |  | N/A |
|  |  | 66 | 1755 | 5 | 25 | 2155 | 5 | 0.3 |  | IMD5 |
| CA_13A-48A-66A | CA_13A-48A | 13 | 782 | 5 | 25 | 751 | 5 | N/A | FDD-TDD | N/A |
|  |  | 48 | 3695 | 5 | 25 | 3695 | 5 | N/A |  | N/A |
|  |  | 66 | 1731 | 5 | 25 | 2131 | 5 | 17.1 |  | IMD3 |
| CA_19A-21A-42A | CA_19A-21A | 19 | 842.5 | 5 | 25 | 887.5 | 5 | N/A | FDD | N/A |
|  |  | 21 | 1450.4 | 5 | 25 | 1498.4 | 5 | N/A | FDD | N/A |
|  |  | 42 | 3508.7 | 5 | 25 | 3508.7 | 5 | 13.0 | TDD | IMD4 |
|  | CA_21A-42A | 21 | 1460.4 | 5 | 25 | 1508.4 | 5 | N/A | FDD | N/A |
|  |  | 42 | 3500 | 5 | 25 | 3500 | 5 | N/A | FDD | N/A |
|  |  | 19 | 836.2 | 5 | 25 | 881.2 | 5 | 13.0 | TDD | IMD4 |
| CA_28A-41A-42A | CA_41A-42A | 41 | 2672 | 10 | 50 | 2672 | 10 | N/A | TDD | N/A |
|  |  | 42 | 3460 | 10 | 50 | 3460 | 10 |  | TDD | N/A |
|  |  | 28 | 733 | 5 | 25 | 788 | 5 | 26.0 | FDD | IMD2 |
| CA_1A-21A-42A6 | CA_1A-42A | 1 |  |  |  |  |  |  | FDD | N/A |
|  |  | 42 |  |  |  |  |  |  | TDD | N/A |
|  |  | 21 |  |  |  |  |  |  | FDD | N/A |
| CA_2A-5A-48A CA_2A-5A-48C CA_2A-5A-48D | CA_5A-48A | 2 | 1882 | 5 | 25 | 1962 | 5 | 15.6 | FDD-TDD | IMD3 |
|  |  | 5 | 839 | 5 | 25 | 884 | 5 | N/A |  | N/A |
|  |  | 48 | 3640 | 5 | 25 | 3640 | 5 | N/A |  | N/A |
| CA_2A-5A-48C CA_2A-5A-48D | CA_2A-5A | 2 | 1905 | 5 | 25 | 1985 | 5 | N/A | FDD-TDD | N/A |
|  |  | 5 | 844 | 5 | 25 | 889 | 5 | N/A |  | N/A |
|  |  | 48 | 3593 | 5 | 25 | 3593 | 5 | 16.6 |  | IMD3 |
| CA_2A-13A-48A CA_2A-13A-48C CA_2A-13A-48D | CA_13A-48A | 2 | 1903.5 | 5 | 25 | 1983.5 | 5 | 15.6 | FDD-TDD | IMD3 |
|  |  | 13 | 784.5 | 5 | 25 | 753.5 | 5 | N/A |  | N/A |
|  |  | 48 | 3552.5 | 5 | 25 | 3552.5 | 5 | N/A |  | N/A |
| CA_2A-48A-66A, CA_2A-48D-66A, CA_2A-48E-66A, CA_2A-48A-66A-66A, CA_2A-48C-66A-66A, CA_2A-48D-66A-66A, CA_2A-48E-66A-66A | CA_2A-66A | 2 | 1855 | 5 | 25 | 1935 | 5 | N/A | FDD-TDD | N/A |
|  |  | 48 | 3625 | 5 | 25 | 3625 | 5 | 32.0 |  | IMD2 |
|  |  | 66 | 1770 | 5 | 25 | 2190 | 5 | N/A |  | N/A |
| CA_2A-14A-66A, CA_2A-2A-14A-66A, CA_2A-14A-66A-66A, CA_2A-2A-14A-66A-66A, CA_2A-14A-66A-66A-66A | CA_2A-14A | 2 | 1870 | 5 | 25 | 1950 | 5 | N/A | FDD | N/A |
|  |  | 14 | 793 | 5 | 25 | 763 | 5 | N/A |  | N/A |
|  |  | 66 | 1734 | 5 | 25 | 2154 | 5 | 7.2 |  | IMD4 |
|  | CA_14A-66A | 2 | 1874 | 5 | 25 | 1954 | 5 | 6.2 | FDD | IMD4 |
|  |  | 14 | 793 | 5 | 25 | 763 | 5 | N/A |  | N/A |
|  |  | 66 | 1770 | 5 | 25 | 2190 | 5 | N/A |  | N/A |
| NOTE 1: This band is subject to IMD3 also which MSD is not specified. NOTE 1: Both of the transmitters shall be set min(+20 dBm, PCMAX_L,c) as defined in subclause 6.2.5A NOTE 2: RBSTART = 0 NOTE 3: Void NOTE 4: This MSD requirement apply with both IMD2 and IMD3 products should be generated. NOTE 5: For operations with 4 antenna ports, the MSD in the applicable bands shall be modified by the absolute value of ΔRIB,4R in Table 7.3.1-1a when MSD > 0. NOTE 6: Due to the spectrum holdings of the operator, the deployed frequency ranges do not result MSD to interested downlink channel. Therefore, no requirements apply for this CA configuration. |  |  |  |  |  |  |  |  |  |  |

For intra-band contiguous carrier aggregation the throughput of each component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.1-1, Table 7.3.1-1a, Table 7.3.1-1A, Table 7.3.1-1B, Table 7.3.1-1C, Table 7.3.1A-0h and Table 7.3.1A-1. For operating bands with an unpaired DL part (as noted in Table 5.5-1), the power levels in Table 7.3.1-1 and Table 7.3.1-1a also apply for an SCC assigned in the unpaired part. The requirement is verified using an uplink CA configuration with the largest number of carriers supported by the UE. Table 7.3.1A-0h, Table 7.3.1A-1 and Table 7.3.1A-2 specifies the maximum number of allocated uplink resource blocks for which the intra-band contiguous carrier aggregation reference sensitivity requirement shall be met. The PCC and SCC allocations as defined in Table 7.3.1A-0h, Table 7.3.1A-1 and Table 7.3.1A-2 form a contiguous allocation where TX–RX frequency separations of the component carriers are as defined in Table 5.7.4-1. In case downlink CA configuration has additional SCC(s) compared to uplink CA configuration those are configured furthers away from uplink band. For UE(s) supporting one uplink carrier, the uplink configuration of the PCC shall be in accordance with Table 7.3.1-2 and the downlink PCC carrier center frequency shall be configured closer to uplink operating band than any of the downlink SCC center frequency. Unless given by Table 7.3.1-3, the reference sensitivity requirements shall be verified with the network signalling value NS_01 (Table 6.2.4-1) configured.

Table 7.3.1A-0h: Intra-band contiguous CA uplink configuration for reference sensitivity for Bandwidth Class B

| CA configuration / CC combination / NRB_agg / Duplex mode |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Uplink CA configuration | 25RB+25RB |  | 50RB+25RB |  | 50RB+50RB |  | 75RB+25RB |  | Duplex Mode |
|  | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |
| CA_5B | N/A | N/A | 25 | 0 | 25 | 0 | N/A | N/A | FDD |
| CA_8B | N/A | N/A | 25 | 0 | 25 | 0 | N/A | N/A | FDD |
| CA_66B | 25 | 25 | 50 | 25 | 50 | 50 | 75 | 25 | FDD |
| NOTE 1: The carrier centre frequency of SCC in the UL operating band is configured closer to the DL operating band. NOTE 2: The transmitted power over both PCC and SCC shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 3: The UL resource blocks in both PCC and SCC shall be confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1). NOTE 4: The UL resource blocks in PCC shall be located as close as possible to the downlink operating band, while the UL resource blocks in SCC shall be located as far as possible from the downlink operating band. NOTE 5: In case a CA configuration consists of CC channel bandwidths which are unequal in bandwidth the PCC channel bandwidth shall be the larger one for reference sensitivity test. |  |  |  |  |  |  |  |  |  |

Table 7.3.1A-1: Intra-band contiguous CA uplink configuration for reference sensitivity for Bandwidth Class C

| CA configuration / CC combination / NRB_agg / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Uplink CA configuration | 100RB+25RB |  | 100RB+50RB |  | 75RB+75RB |  | 75RB+50RB |  | 100RB+75RB |  | 100RB+100RB |  | Duplex Mode |
|  | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC | PCC | SCC |  |
| CA_1C | N/A | N/A | N/A | N/A | 75 | 54 | N/A | N/A | N/A | N/A | 100 | 30 | FDD |
| CA_3C | 50 | 0 | 50 | 0 | N/A | N/A | N/A | N/A | 50 | 0 | 50 | 0 | FDD |
| CA_7C | N/A | N/A | 75 | 0 | 75 | 0 | 75 | 0 | 75 | 0 | 75 | 0 | FDD |
| CA_38C | N/A | N/A | N/A | N/A | 75 | 75 | N/A | N/A | N/A | N/A | 100 | 100 | TDD |
| CA_39C | 100 | 25 | 100 | 50 | N/A | N/A | N/A | N/A | 100 | 75 | N/A | N/A | TDD |
| CA_40C | N/A | N/A | 100 | 50 | 75 | 75 | N/A | N/A | 100 | 75 | 100 | 100 | TDD |
| CA_41C | 100 | 25 | 100 | 50 | 75 | 75 | 75 | 50 | 100 | 75 | 100 | 100 | TDD |
| CA_42C | 100 | 25 | 100 | 50 | N/A | N/A | N/A | N/A | 100 | 75 | 100 | 100 | TDD |
| CA_48C | 100 | 25 | 100 | 50 | N/A | N/A | N/A | N/A | 100 | 75 | 100 | 100 | TDD |
| CA_66C | 100 | 25 | 100 | 50 | 75 | 75 | 75 | 50 | 100 | 75 | 100 | 100 | FDD |
| NOTE 1: The carrier centre frequency of SCC in the UL operating band is configured closer to the DL operating band. NOTE 2: The transmitted power over both PCC and SCC shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 3: The UL resource blocks in both PCC and SCC shall be confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1). NOTE 4: The UL resource blocks in PCC shall be located as close as possible to the downlink operating band, while the UL resource blocks in SCC shall be located as far as possible from the downlink operating band. NOTE 5: In case a CA configuration consists of CC channel bandwidths which are unequal in bandwidth the PCC channel bandwidth shall be the larger one for reference sensitivity test. NOTE 6: Void. NOTE 7: Void |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 7.3.1A-2: Intra-band contiguous CA uplink configuration for reference sensitivity for Bandwidth Class D

| CA configuration / CC combination / NRB_agg / Duplex mode |  |  |  |  |
| --- | --- | --- | --- | --- |
| Uplink CA configuration | CC combination | Number of uplink resource blocks per CC |  |  |
|  |  | PCC | SCC | SCC |
| CA_40D | 75RB+75RB+75RB 100RB+75RB+75RB 100RB+100RB+50RB 100RB+100RB+75RB 100RB+100RB+100RB | NOTE 6 |  |  |
| CA_41D | 75RB+75RB+75RB 100RB+75RB+75RB 100RB+100RB+50RB 100RB+100RB+75RB 100RB+100RB+100RB | NOTE 6 |  |  |
| NOTE 1: The carrier centre frequency of SCC in the UL operating band is configured closer to the DL operating band. NOTE 2: The transmitted power over both PCC and SCC shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 3: The UL resource blocks in both PCC and SCC shall be confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1). NOTE 4: The UL resource blocks in PCC shall be located as close as possible to the downlink operating band, while the UL resource blocks in SCC shall be located as far as possible from the downlink operating band. NOTE 5: In case a CA configuration consists of CC channel bandwidths which are unequal in bandwidth the PCC channel bandwidth shall be the larger one for reference sensitivity test. NOTE 6: All uplink CCs are fully allocated |  |  |  |  |

For intra-band non-contiguous carrier aggregation with one uplink carrier and two or more downlink sub-blocks, the throughput of each downlink component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) and parameters specified in Table 7.3.1-1, Table 7.3.1-1a, Table 7.3.1-1A, Table 7.3.1-1B, Table 7.3.1-1C and Table 7.3.1A-3 with the reference sensitivity power level increased by  RIBNC given in Table 7.3.1A-3 for the SCC(s). For aggregation of more than two downlink FDD carriers with one uplink carrier the reference sensitivity is defined only for the specific uplink and downlink test points which are specified in Table 7.3.1A-3. The requirements apply with all downlink carriers active. Unless given by Table 7.3.1-3, the reference sensitivity requirements shall be verified with the network signalling value NS_01 (Table 6.2.4-1) configured.

Table 7.3.1A-3: Intra-band non-contiguous CA with one uplink configuration for reference sensitivity

| CA configuration | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation | ΔRIBNC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- |
| CA_1A-1A | 25RB + 25RB | 0.0 < Wgap ≤ 50.0 | 251 | 0.5 | FDD |
|  | 25RB + 50RB | 0.0 < Wgap ≤ 45.0 | 251 | 0.5 |  |
|  | 25RB + 75RB | 0.0 < Wgap ≤ 40.0 | 251 | 0.5 |  |
|  | 25RB + 100RB | 0.0 < Wgap ≤ 35.0 | 251 | 0.5 |  |
|  | 50RB + 25RB | 0.0 < Wgap ≤ 45.0 | 501 | 0.5 |  |
|  | 50RB + 50RB | 0.0 < Wgap ≤ 40.0 | 501 | 0.5 |  |
|  | 50RB + 75RB | 0.0 < Wgap ≤ 35.0 | 501 | 0.5 |  |
|  | 50RB + 100RB | 0.0 < Wgap ≤ 30.0 | 501 | 0.5 |  |
|  | 75RB + 25RB | 0.0 < Wgap ≤ 40.0 | 751 | 0.5 |  |
|  | 75RB + 50RB | 0.0 < Wgap ≤ 35.0 | 751 | 0.5 |  |
|  | 75RB + 75RB | 0.0 < Wgap ≤ 30.0 | 751 | 0.5 |  |
|  | 75RB + 100RB | 0.0 < Wgap ≤ 25.0 | 751 | 0.5 |  |
|  | 100RB + 25RB | 20.0 < Wgap ≤ 35.0 | 901 | 0.5 |  |
|  |  | 0.0 < Wgap ≤ 20.0 | 1001 | 0.5 |  |
|  | 100RB + 50RB | 15.0 < Wgap ≤ 30.0 | 901 | 0.5 |  |
|  |  | 0.0 < Wgap ≤ 15.0 | 1001 | 0.5 |  |
|  | 100RB + 75RB | 10.0 < Wgap ≤ 25.0 | 901 | 0.5 |  |
|  |  | 0.0 < Wgap ≤ 10.0 | 1001 | 0.5 |  |
|  | 100RB + 100RB | 5.0 < Wgap ≤ 20.0 | 901 | 0.5 |  |
|  |  | 0.0 < Wgap ≤ 5.0 | 1001 | 0.5 |  |
| CA_2A-2A | 25RB+25RB | 30.0 < Wgap ≤ 50.0 | 121 | 5.3 | FDD |
|  |  | 0.0 < Wgap ≤ 30.0 | 251 | 0 |  |
|  | 25RB+50RB | 25.0 < Wgap ≤ 45.0 | 121 | 4.4 |  |
|  |  | 0.0 < Wgap ≤ 25.0 | 251 | 0 |  |
|  | 25RB+75RB | 20.0 < Wgap ≤ 40.0 | 121 | 4.2 |  |
|  |  | 0.0 < Wgap ≤ 20.0 | 251 | 0 |  |
|  | 25RB+100RB | 15.0 < Wgap ≤ 35.0 | 121 | 3.8 |  |
|  |  | 0.0 < Wgap ≤ 15.0 | 251 | 0 |  |
|  | 50RB+25RB | 15.0 < Wgap ≤ 45.0 | 121 | 5.9 |  |
|  |  | 0.0 < Wgap ≤ 15.0 | 321 | 0 |  |
|  | 50RB+50RB | 10.0 < Wgap ≤ 40.0 | 121 | 4.6 |  |
|  |  | 0.0 < Wgap ≤ 10.0 | 321 | 0 |  |
|  | 50RB+75RB | 5.0 < Wgap ≤ 35.0 | 121 | 4.1 |  |
|  |  | 0.0 < Wgap ≤ 5.0 | 321 | 0 |  |
|  | 50RB+100RB | 0.0 < Wgap ≤ 30.0 | 121 | 4.0 |  |
|  | 75RB+25RB | 10.0 < Wgap ≤ 40.0 | 1212 | 6.7 |  |
|  |  | 0.0 < Wgap ≤ 10.0 | 361 | 0 |  |
|  | 75RB+50RB | 5.0 < Wgap ≤ 35.0 | 1212 | 5.4 |  |
|  |  | 0.0 < Wgap ≤ 5.0 | 361 | 0 |  |
|  | 75RB+75RB | 0.0 < Wgap ≤ 30.0 | 1212 | 4.6 |  |
|  | 75RB+100RB | 0.0 < Wgap ≤ 25.0 | 1212 | 4.2 |  |
|  | 100RB+25RB | 0.0 < Wgap ≤ 35.0 | 1613 | 7.2 |  |
|  | 100RB+50RB | 0.0 < Wgap ≤ 30.0 | 1613 | 5.8 |  |
|  | 100RB+75RB | 0.0 < Wgap ≤ 25.0 | 1613 | 5.0 |  |
|  | 100RB+100RB | 0.0 < Wgap ≤ 20.0 | 1613 | 4.6 |  |
| CA_3A-3A | 15RB+25RB | 47.0 < Wgap ≤ 67.0 | 121 | 4.5 | FDD |
|  |  | 0.0 < Wgap ≤ 47.0 | 151 | 0 |  |
|  | 25RB+15RB | 47.0 < Wgap ≤ 67.0 | 121 | 4.9 |  |
|  |  | 0.0 < Wgap ≤ 47.0 | 251 | 0 |  |
|  | 25RB+25RB | 45.0 < Wgap ≤ 65.0 | 121 | 4.7 |  |
|  |  | 0.0 < Wgap ≤ 45.0 | 251 | 0 |  |
|  | 25RB+50RB | 40.0 < Wgap ≤ 60.0 | 121 | 3.8 |  |
|  |  | 0.0 < Wgap ≤ 40.0 | 251 | 0 |  |
|  | 25RB+75RB | 35.0 < Wgap ≤ 55.0 | 121 | 3.6 |  |
|  |  | 0.0 < Wgap ≤ 35.0 | 251 | 0 |  |
|  | 25RB+100RB | 30.0 < Wgap ≤ 50.0 | 121 | 3.4 |  |
|  |  | 0.0 < Wgap ≤ 30.0 | 251 | 0 |  |
|  | 50RB+25RB | 30.0 < Wgap ≤ 60.0 | 129 | 5.1 |  |
|  |  | 0.0 < Wgap ≤ 30.0 | 321 | 0 |  |
|  | 50RB+50RB | 25.0 < Wgap ≤ 55.0 | 129 | 4.3 |  |
|  |  | 0.0 < Wgap ≤ 25.0 | 321 | 0 |  |
|  | 50RB+75RB | 20.0 < Wgap ≤ 50.0 | 129 | 3.8 |  |
|  |  | 0.0 < Wgap ≤ 20.0 | 321 | 0 |  |
|  | 50RB+100RB | 15.0 < Wgap ≤ 45.0 | 129 | 3.4 |  |
|  |  | 0.0 < Wgap ≤ 15.0 | 321 | 0 |  |
|  | 75RB+25RB | 25.0 < Wgap ≤ 55.0 | 1210 | 6.0 |  |
|  |  | 0.0 < Wgap ≤ 25.0 | 321 | 0 |  |
|  | 75RB+50RB | 20.0 < Wgap ≤ 50.0 | 1210 | 4.7 |  |
|  |  | 0.0 < Wgap ≤ 20.0 | 321 | 0 |  |
|  | 75RB+75RB | 15.0 < Wgap ≤ 45.0 | 1210 | 4.2 |  |
|  |  | 0.0 < Wgap ≤ 15.0 | 321 | 0 |  |
|  | 75RB+100RB | 10.0 < Wgap ≤ 40.0 | 1210 | 3.8 |  |
|  |  | 0.0 < Wgap ≤ 10.0 | 321 | 0 |  |
|  | 100RB+25RB | 15.0 < Wgap ≤ 50.0 | 1611 | 6.5 |  |
|  |  | 0.0 < Wgap ≤ 15.0 | 321 | 0 |  |
|  | 100RB+50RB | 10.0 < Wgap ≤ 45.0 | 1611 | 5.1 |  |
|  |  | 0.0 < Wgap ≤ 10.0 | 321 | 0 |  |
|  | 100RB+75RB | 5.0 < Wgap ≤ 40.0 | 1611 | 4.5 |  |
|  |  | 0.0 < Wgap ≤ 5.0 | 321 | 0 |  |
|  | 100RB+100RB | 0.0 < Wgap ≤ 35.0 | 1611 | 4.1 |  |
| CA_4A-4A | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | FDD |
| CA_5A-5A | 25RB+25RB | NOTE 7 | 121 | 5.3 | FDD |
|  | 25RB+50RB | NOTE 7 | 121 | 4.4 |  |
|  | 50RB+25RB | NOTE 7 | 121 | 5.9 |  |
|  | 50RB+50RB | NOTE 7 | 121 | 4.6 |  |
| CA_7A-7A | 25RB+25RB | 0< Wgap ≤ 60 | 25 | 0.0 | FDD |
|  | 25RB+50RB | 0 < Wgap ≤ 55 | 25 | 0.0 |  |
|  | 25RB+75RB | 0 < Wgap ≤ 50 | 25 | 0.0 |  |
|  | 25RB+100RB | 0 < Wgap ≤ 45 | 25 | 0.0 |  |
|  | 50RB+25RB | 30 < Wgap ≤ 55 | 321 | 0.0 |  |
|  |  | 0 < Wgap ≤ 30 | 50 | 0.0 |  |
|  | 50RB+50RB | 25.0 < Wgap ≤ 50.0 | 321 | 0.0 |  |
|  |  | 0.0 < Wgap ≤ 25.0 | 50 | 0.0 |  |
|  | 50RB+75RB | 20 < Wgap ≤ 45 | 321 | 0.0 |  |
|  |  | 0 < Wgap ≤ 20 | 50 | 0.0 |  |
|  | 50RB+100RB | 15 < Wgap ≤ 40 | 321 | 0.0 |  |
|  |  | 0 < Wgap ≤ 15 | 50 | 0.0 |  |
|  | 75RB+25RB | 20.0 < Wgap ≤ 50.0 | 321 | 0.0 |  |
|  |  | 0.0 < Wgap ≤ 20.0 | 501 | 0.0 |  |
|  | 75RB+50RB | 20.0 < Wgap ≤ 45.0 | 321 | 0.0 |  |
|  |  | 0.0 < Wgap ≤ 20.0 | 501 | 0.0 |  |
|  | 75RB+75RB | 15.0 < Wgap ≤ 40.0 | 321 | 0.0 |  |
|  |  | 0.0 < Wgap ≤ 15.0 | 501 | 0.0 |  |
|  | 75RB+100RB | 10 < Wgap ≤ 35 | 321 | 0.0 |  |
|  |  | 0 < Wgap ≤ 10 | 501 | 0.0 |  |
|  | 100RB+25RB | 25 < Wgap ≤ 45 | 321 | 0.0 |  |
|  |  | 0 < Wgap ≤ 25 | 451 | 0.0 |  |
|  | 100RB+50RB | 20 < Wgap ≤ 40 | 321 | 0.0 |  |
|  |  | 0 < Wgap ≤ 20 | 451 | 0.0 |  |
|  | 100RB+75RB | 15.0 < Wgap ≤ 35.0 | 361 | 0.0 |  |
|  |  | 0.0 < Wgap ≤ 15.0 | 501 | 0.0 |  |
|  | 100RB+100RB | 15.0 < Wgap ≤ 30.0 | 321 | 0.0 |  |
|  |  | 0.0 < Wgap ≤ 15.0 | 451 | 0.0 |  |
| CA_12A-12A | 25RB+25RB | 0.0 < Wgap ≤ 7.0 | 518 | 3 | FDD |
| CA_23A-23A | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | FDD |
| CA_25A-25A | 25RB+25RB | 30.0 < Wgap ≤ 55.0 | 101 | 5.0 | FDD |
|  |  | 0.0 < Wgap ≤ 30.0 | 251 | 0.0 |  |
|  | 25RB+50RB | 25.0 < Wgap ≤ 50.0 | 101 | 4.5 |  |
|  |  | 0.0 < Wgap ≤ 25.0 | 251 | 0.0 |  |
|  | 25RB+75RB | 20 < Wgap ≤ 45 | 101 | 4.3 |  |
|  |  | 0 < Wgap ≤ 20 | 251 | 0 |  |
|  | 25RB+100RB | 15 < Wgap ≤ 40 | 101 | 4.1 |  |
|  |  | 0 < Wgap ≤ 15 | 251 | 0 |  |
|  | 50RB+25RB | 15.0 < Wgap ≤ 50.0 | 104 | 5.5 |  |
|  |  | 0.0 < Wgap ≤ 15.0 | 321 | 0.0 |  |
|  | 50RB+50RB | 10.0 < Wgap ≤ 45.0 | 104 | 5.0 |  |
|  |  | 0.0 < Wgap ≤ 10.0 | 321 | 0.0 |  |
|  | 50RB+75RB | 5 < Wgap ≤ 40 | 104 | 4.5 |  |
|  |  | 0 < Wgap ≤ 5 | 321 | 0 |  |
|  | 50RB+100RB | 0 < Wgap ≤ 35 | 104 | 4.2 |  |
|  | 75RB+25RB | 10 < Wgap ≤ 45 | 1014 | 7.6 |  |
|  |  | 0 < Wgap ≤ 10 | 321 | 0 |  |
|  | 75RB+50RB | 5 < Wgap ≤ 40 | 1014 | 6.7 |  |
|  |  | 0 < Wgap ≤ 5 | 321 | 0 |  |
|  | 75RB+75RB | 0 < Wgap ≤ 35 | 1014 | 5.6 |  |
|  | 75RB+100RB | 0 < Wgap ≤ 30 | 1014 | 4.8 |  |
|  | 100RB+25RB | 0 < Wgap ≤ 40 | 1215 | 8 |  |
|  | 100RB+50RB | 0 < Wgap ≤ 35 | 1215 | 6.7 |  |
|  | 100RB+75RB | 0 < Wgap ≤ 30 | 1215 | 6.1 |  |
|  | 100RB+100RB | 0 < Wgap ≤ 25 | 1215 | 5.7 |  |
| CA_25A-25A-25A | 25RB+25RB+25RB | Wgap_L = Wgap_H = 25 MHz | 101 | 5.0 | FDD |
|  |  | Wgap_L = Wgap_H = 5 MHz | 251 | 0.0 |  |
|  | 50RB+50RB+50RB | Wgap_L = Wgap_H = 17.5 MHz | 104 | 5.0 |  |
|  | 75RB+75RB+75RB | Wgap_L = Wgap_H = 10 MHz | 1014 | 5.6 |  |
|  | 100RB+100RB+100RB | Wgap_L = Wgap_H = 2.5 MHz | 1215 | 5.7 |  |
| CA_40A-40A | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_40A-40C | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_40C-40C | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_41A-41A | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_41A-41C | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_41A-41D | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_41C-41C | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_41A-41A-41A | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_41A-41A-41C | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_42A-42A | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_42A-42C | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_42A-42D | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_42C-42C | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_48A-48A | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_48A-48A-48A | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_48A-48A-48A-48A | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_48A-48C | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_48A-48D | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_48A-48E | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_48C-48C | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_48C-48D | NOTE 6 | NOTE 7 | NOTE 8 | 0.0 | TDD |
| CA_66A-66A | NOTE 6 | NOTE 7 | NOTE 8, NOTE 16 | 0.0 | FDD |
| CA_66A-66A-66A | NOTE 6 | NOTE 7 | NOTE 8, NOTE 16 | 0.0 | FDD |
| CA_66A-66B | NOTE 6 | NOTE 7 | NOTE 8, NOTE 16 | 0.0 | FDD |
| CA_66A-66C | NOTE 6 | NOTE 7 | NOTE 8, NOTE 16 | 0.0 | FDD |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission. NOTE 2: Wgap is the sub-block gap between the two sub-blocks. NOTE 3: The carrier center frequency of PCC in the UL operating band is configured closer to the DL operating band. NOTE 4: 4 refers to the UL resource blocks shall be located at RBstart=33. NOTE 5: For the TDD intra-band non-contiguous CA configurations, the minimum requirements apply only in synchronized operation between all component carriers. NOTE 6: All combinations of channel bandwidths defined in Table 5.6A.1-3. NOTE 7: All applicable sub-block gap sizes. NOTE 8: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.6-1. In case of uplink sub-block is TDD intra-band contiguous CA then the uplink PCC and SCC allocations are the same as NRB_agg defined in Table 7.3.1A-1. NOTE 9: 9 refers to the UL resource blocks shall be located at RBstart=25. NOTE 10: 10 refers to the UL resource blocks shall be located at RBstart=35. NOTE 11: 11 refers to the UL resource blocks shall be located at RBstart=50. NOTE 12: 12 refers to the UL resource blocks shall be located at RBstart=39. NOTE 13: 13 refers to the UL resource blocks shall be located at RBstart=57. NOTE 14: 14 refers to the UL resource blocks shall be located at RBstart=44. NOTE 15: 15 refers to the UL resource blocks shall be located at RBstart=62. NOTE 16: The carrier center frequency of PCC in the DL operating band is configured closer to the UL operating band. NOTE 17: Void NOTE 18: 18 refers to the UL resource blocks shall be located at RBstart=12. NOTE 19: For operations with 4 antenna ports, the MSD in the applicable bands shall be modified by the absolute value of ΔRIB,4R in Table 7.3.1-1a when MSD > 0. |  |  |  |  |  |

For intra-band non-contiguous carrier aggregation with two uplink and downlink carriers the reference sensitivity is defined to be met with both downlink and uplink carriers activated. The downlink PCC and SCC minimum requirements for reference sensitivity power level as specified in Table 7.3.1-1, Table 7.3.1-1a, Table 7.3.1-1A, Table 7.3.1-1B and Table 7.3.1-1C are increased by amount of ΔR2UL_PCC and ΔR2UL_SCC  which are defined in Table 7.3.1A-4 when uplink PCC and SCC allocations are according to the Table 7.3.1A-4.

Table 7.3.1A-4: Intra-band non-contiguous CA with two uplinks configuration for reference sensitivity

| CA configuration | Aggregated channel bandwidth (PCC+SCC) | Wgap / [MHz] | UL PCC allocation | UL SCC allocation | ΔR2UL_PCC (dB) | ΔR2UL_SCC (dB) | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CA_4A-4A | NOTE 2 | NOTE 3 | NOTE 4 | NOTE 5 | 0.0 | 0.0 | FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 2: All combinations of channel bandwidths defined in Table 5.6A.1-3. NOTE 3: All applicable sub-block gap sizes. NOTE 4: The PCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.6-1. NOTE 5: The SCC allocation is same as Transmission bandwidth configuration NRB as defined in Table 5.6-1. |  |  |  |  |  |  |  |

For combinations of intra-band and inter-band carrier aggregation, the requirement is defined with an uplink configuration in accordance with Table 7.3.1A-3 when the uplink is active in the band supporting two or more non-contigous component carriers, Table 7.3.1A-1 when the uplink (up to two contiguously aggregated uplink carriers) is active in a band supporting two contiguous component carriers and in accordance with Table 7.3.1-2 when an uplink is active in a band supporting one carrier per band. The downlink PCC shall be configured closer to the uplink operating band than the downlink SCC(s) when the uplink is active in band(s) supporting contiguous aggregation. The carrier center frequency of PCC in the UL operating band is configured closer to the DL operating band when the uplink is active in band(s) supporting non-contiguous aggregation. For these uplink configurations, the UE shall meet the reference sensitivity requirements for intra-band non-contiguous carrier aggregation of two or more downlink sub-blocks, the requirements for intra-band contiguous carrier aggregation for the contiguously aggregated downlink carriers and for any remaining component carrier(s) the requirements specified in subclause 7.3.1. For the two or more component carriers within the same band, RIBNC = 0 dB for all sub-block gaps (Table 7.3.1A-3) when the uplink is active in another band. All downlink carriers shall be active throughout the tests and the requirements for the downlinks shall be met with all uplink carriers active in each band capable of UL operation. For component carriers configured in Band 46, the said requirements for intra-band carrier aggregation of downlink carriers are replaced by the requirements in Table 7.3.1A-0eA for the uplink in any band other than band 46 with the uplink configuration specified in Table 7.3.1-2. Unless given by Table 7.3.1-3, the reference sensitivity requirements shall be verified with the network signalling value NS_01 (Table 6.2.4-1) configured.

For the UE that supports any of combinations of intra-band and inter-band carrier aggregation given in Table 7.3.1A-5, exceptions to the requirements for a band(s) specified in subclause 7.3.1 are allowed when the band(s) is impacted by harmonic interference from the uplink being active in a lower-frequency band of the same CA configuration. For these exceptions, the UE shall meet the requirements specified in Table 7.3.1A-5 and Table 7.3.1A-6.

Table 7.3.1A-5: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions due to harmonic issues in the combinations of intra-band and inter-band CA)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-3A-3A-7A-8A4 CA_1A-3A-7A-7A-8A4 CA_1A-3A-3A-7A-7A-8A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_1A-3A-3A-7A-8A5,6 CA_1A-3A-7A-7A-8A5,6 CA_1A-3A-3A-7A-7A-8A5,6 | 733 |  |  | -88 | -87.4 | -87 | -86.7 | FDD |
| CA_1A-1A-3A-7A-28A5,6 CA_1A-1A-3A-7C-28A5,6 CA_1A-1A-3C-7A-28A5,6 CA_1A-1A-3C-7C-28A5,6 CA_1A-1A-3A-3A-7A-28A5,6 CA_1A-1A-3A-3A-7C-28A5,6 CA_1A-3A-7C-28A5,6 CA_1A-3C-7A-28A5,6 CA_1A-3C-7C-28A5,6 CA_1A-3A-3A-7A-28A5,6 CA_1A-3A-3A-7C-28A5,6 CA_1A-3A-7A-7A-28A5,6 | 121 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
| CA_1A-3C-8A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_1A-3A-8A-42C4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_1A-3A-8A-42C8,9 | 4221 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_1A-3A-8A-42C10 | 4221 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_1A-3A-8A-42C15,16 | 4221 |  |  | -84.8 | -84.7 | -84.6 | -84.5 | TDD |
| CA_1A-3A-18A-42C8,9 | 4221 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_1A-3A-18A-42C10 | 4221 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_1A-3A-19A-42C8,9 | 4221 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_1A-3A-19A-42C10 | 4221 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_1A-3A-3A-20A-28A | 121 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
| CA_1A-3A-21A-42C8,9 | 4221 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_1A-3A-21A-42C10 | 4221 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_1A-3C-28A | 121 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
| CA_1A-3A-28A-40C16,17 | 28 |  |  | -60.7 | -60.7 | -60.7 | -60.7 | FDD |
| CA_1A-3A-28A-42C8,9 | 4221 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_1A-3A-28A-42C10 | 4221 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_1A-3A-41A-42C8,9 CA_1A-3A-41C-42A8,9 CA_1A-3A-41C-42C8,9 | 4221 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_1A-3A-41A-42C10 CA_1A-3A-41C-42A10 CA_1A-3A-41C-42C10 | 4221 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_1A-3A-42C8,9 | 4221 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_1A-3A-42D8,9 | 4221 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_1A-7A-8A-40C5,6 | 733 |  |  |  | -87.1 | -86.7 | -86.4 | FDD |
| CA_1A-7C-28A5,6 | 121 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
| CA_1A-7A-28A-40C16,17 | 28 |  |  | -60.7 | -60.7 | -60.7 | -60.7 | FDD |
| CA_1A-8A-11A-42C15,16 | 4221 |  |  | -84.8 | -84.7 | -84.6 | -84.5 | TDD |
| CA_1A-3A-42C10 | 4221 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_1A-3A-42D10 | 4221 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_1A-3A-3A-42C9,10 | 4221 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_1A-3A-3A-42C11 | 4221 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_1A-21A-28A-42C4, 14 | 21 |  |  | N/A | N/A | N/A |  | FDD |
|  | 42 |  |  | N/A | N/A | N/A | N/A | TDD |
| CA_1A-28A-42C5,6,12,13 | 121 |  |  | -89.8 | -89.4 | -89 | -88.7 | FDD |
|  | 42 |  |  | -85.7 | -85.4 | -85.1 | -84.9 | TDD |
| CA_2A-2A-4A-12A5,6 | 421 |  |  | -90 | -89.5 | -89 | -88.5 | FDD |
| CA_2A-2A-5A-12A-66A5,6 | 6621 |  |  | -89.5 | -89 | -88.5 | -88 | FDD |
| CA_2A-2A-7A-12A-66A5,6 | 6621 |  |  | -89.5 | -89 | -88.5 | -88 | FDD |
| CA_2A-2A-12A-66A-66A5,6 | 6621 |  |  | -89.5 | -89 | -88.5 | -88 | FDD |
| CA_2A-13A-48A-48A8,9 CA_2A-13A-48C8,9 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_2A-13A-48A-48A10 CA_2A-13A-48C10 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_2A-7A-12B-66A5,6 | 6621 |  |  | -89.5 | -89 | -88.5 | -88 | FDD |
| CA_2A-2A-12A-30A-66A5,6 | 6621 |  |  | -89.5 | -89 | -88.5 | -88 | FDD |
| CA_2A-2A-12A-66A5,6 | 6621 |  |  | -89.5 | -89 | -88.5 | -88 | FDD |
| CA_2A-2A-12B-66A5,6 | 6621 |  |  | -89.5 | -89 | -88.5 | -88 | FDD |
| CA_2A-12A-30A-66A-66A5,6 | 6621 |  |  | -89.5 | -89 | -88.5 | -88 | FDD |
| CA_2A-12B-66A5,6 | 1621 |  |  | -89.5 | -89 | -88.5 | -88 | FDD |
| CA_2A-12B-66A-66A5,6 | 6621 |  |  | -89.5 | -89 | -88.5 | -88 | FDD |
| CA_2A-4A-4A-12A5,6 | 421 |  |  | -90 | -89.5 | -89 | -88.5 | FDD |
| CA_2A-4A-12A-12A5,6 | 421 |  |  | -90 | -89.5 | -89 | -88.5 | FDD |
| CA_2A-4A-12B5,6 | 421 |  |  | -90 | -89.5 | -89 | -88.5 | FDD |
| CA_2A-48A-48C9, 22 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_2A-48A-48C1 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_2A-48A-48D8,9 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_2A-48A-48D10 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_2A-48C-48C8,9 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_2A-48C-48C10 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_2A-48D9, 22 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_2A-48D10 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_2A-48E8,9 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_2A-48E10 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_2A-12A-66A-66A5,6 | 6621 |  |  | -89.5 | -89 | -88.5 | -88 | FDD |
| CA_2A-12A-66C5,6 | 6621 |  |  | -89.5 | -89 | -88.5 | -88 | FDD |
| CA_2A-13A-48D8,9 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_2A-13A-48D10 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_2A-13A-48A-48C8,9 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_2A-13A-48A-48C10 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_2A-13A-48C-66A8,9 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_2A-13A-48C-66A10 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_2A-13A-48A-48A-66A8,9 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_2A-13A-48A-48A-66A10 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_2A-48D-66A8,9 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_2A-48D-66A10 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_2A-48E-66A8,9 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_2A-48E-66A10 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_2A-48A-48A-66A8,9 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_2A-48A-48A-66A10 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_2A-48A-48C-66A8,9 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_2A-48A-48C-66A10 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-3A-7A-7A-8A4 | 3 |  |  | N/A | N/A | N/A | N/A |  |
| CA_3A-3A-7A-7A-8A5,6 | 721 |  |  | -88 | -87,4 | -87 | -86,7 | FDD |
| CA_3A-3A-7A-8A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_3A-3A-7A-8A5,6 | 721 |  |  | -88 | -87.4 | -87 | -86.7 | FDD |
| CA_3A-3A-8A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_3A-7A-7A-8A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
|  | 7 |  |  | N/A | N/A | N/A | N/A |  |
| CA_3A-7A-7A-8A4,5,6 | 3 |  |  | -97 | -94 | -92.2 | -91 | FDD |
|  | 721 |  |  | -88 | -87.4 | -87 | -86.7 |  |
| CA_3C-7A-8A-38A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
|  | 7 |  |  | N/A | N/A | N/A | N/A |  |
| CA_3A-7A-8A-40C4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
|  | 7 |  |  |  | N/A | N/A | N/A |  |
| CA_3A-7A-8A-40C5,6 | 733 |  |  |  | -87.1 | -86.7 | -86.4 | FDD |
| CA_3A-7A-28A-40C16,17 | 28 |  |  | -60.7 | -60.7 | -60.7 | -60.7 | FDD |
| CA_3A-19A-42C8,9 | 4221 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-19A-42C10 | 4221 |  |  | -97.1 | -94.7 | -93.2 | -92.5 |  |
| CA_3A-19A-42D8,9 | 4221 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-19A-42D10 | 4221 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-21A-42C1,2 | 4221 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-21A-42C3 | 4221 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-21A-42D8,9 | 4221 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-21A-42D10 | 4221 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-28A-40C8,9 | 28 |  |  | -60.7 | -60.7 | -60.7 | -60.7 | FDD |
| CA_3A-28A-40D8,9 | 28 |  |  | -60.7 | -60.7 | -60.7 | -60.7 | FDD |
| CA_3A-28A-41A-42C8,9,20 | 4221 |  |  |  | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-28A-41A-42C10,20 | 4221 |  |  |  | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-28A-41A-42C12,13,20 | 4221 |  |  |  | -85.4 | -85.1 | -84.9 | TDD |
| CA_3A-28A-41C-42A8,9,20 | 4221 |  |  |  | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-28A-41C-42A10,20 | 4221 |  |  |  | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-28A-41C-42A12,13,20 | 4221 |  |  |  | -85.4 | -85.1 | -84.9 | TDD |
| CA_3A-28A-41C-42C8,9,20 | 4221 |  |  |  | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-28A-41C-42C10,20 | 4221 |  |  |  | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-28A-41C-42C12,13,20 | 4221 |  |  |  | -85.4 | -85.1 | -84.9 | TDD |
| CA_3A-28A-42C8,9 | 4221 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-28A-42C10 | 4221 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-28A-42C12,13 | 4221 |  |  | -85.7 | -85.4 | -85.1 | -84.9 | TDD |
| CA_3A-28A-42D8,9 | 4221 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-28A-42D10 | 4221 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-28A-42D12,13 | 4221 |  |  | -85.7 | -85.4 | -85.1 | -84.9 | TDD |
| CA_3A-28A-42A-42A8,9 | 4221 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-28A-42A-42A10 | 4221 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-28A-42A-42A12,13 | 4221 |  |  | -85.7 | -85.4 | -85.1 | -84.9 | TDD |
| CA_3A-28A-42A-42C8,9 | 4221 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-28A-42A-42C10 | 4221 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-28A-42A-42C12,13 | 4221 |  |  | -85.7 | -85.4 | -85.1 | -84.9 | TDD |
| CA_3A-28A-42C-42C8,9 | 4221 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-28A-42C-42C10 | 4221 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-28A-42C-42C12,13 | 4221 |  |  | -85.7 | -85.4 | -85.1 | -84.9 | TDD |
| CA_3A-41A-42A-42A8,9,20 | 4221 |  |  |  | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-41A-42A-42A10,20 | 4221 |  |  |  | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-41A-42A-42C8,9,20 | 4221 |  |  |  | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-41A-42A-42C10,20 | 4221 |  |  |  | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-41A-42C-42C8,9,20 | 4221 |  |  |  | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-41A-42C-42C10,20 | 4221 |  |  |  | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-42C8,9 | 4221 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-42C10 | 4221 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-42D8,9 | 4221 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-42D10 | 4221 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-3A-42C8,9 | 4221 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-3A-42C10 | 4221 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-3A-42D9,10 | 4221 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-3A-42D11 | 4221 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-42A-42C8,9 | 4221 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-42A-42C10 | 4221 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-42C-42C8, 9,10 | 4221 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-42C-42C11 | 4221 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3A-42E9,10 | 4221 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_3A-42E11 | 4221 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_3C-8A4 | 3 |  |  | N/A | N/A | N/A | N/A | FDD |
| CA_4A-4A-5A-12A5,6 | 421 |  |  | -90 | -89.5 | -89 | -88.5 | FDD |
| CA_4A-4A-12A5,6 | 4 |  |  | -90 | -89.5 | -89 | -88.5 | FDD |
| CA_4A-4A-12A-12A5,6 | 421 |  |  | -90 | -89.5 | -89 | -88.5 | FDD |
| CA_4A-4A-12A-30A5,6 | 421 |  |  | -90 | -89.5 | -89 | -88.5 | FDD |
| CA_4A-4A-12B5,6 | 421 |  |  | -90 | -89.5 | -89 | -88.5 | FDD |
| CA_4A-5A-12B5,6 | 421 |  |  | -90 | -89.5 | -89 | -88.5 | FDD |
| CA_4A-12A-12A5,6 | 421 |  |  | -90 | -89.5 | -89 | -88.5 | FDD |
| CA_4A-12B5,6 | 421 |  |  | -90 | -89.5 | -89 | -88.5 | FDD |
| CA_4A-5A-12A-12A5,6 | 421 |  |  | -90 | -89.5 | -89 | -88.5 | FDD |
| CA_7A-7A-8A5,6 | 721 |  |  | -88 | -87.4 | -87 | -86.7 | FDD |
| CA_8A-41C7 | 41 |  |  |  | N/A | N/A | N/A | TDD |
| CA_8A-42C15,16 | 4221 |  |  | -84.8 | -84.7 | -84.6 | -84.5 | TDD |
| CA_12A-30A-66A-66A5,6 | 6621 |  |  | -89.5 | -89 | -88.5 | -88 | FDD |
| CA_12A-66A-66A5,6 | 6621 |  |  | -89.5 | -89 | -88.5 | -88 | FDD |
| CA_12A-66C5,6 | 6621 |  |  | -89.5 | -89 | -88.5 | -88 | FDD |
| CA_12B-66A5,6 | 6621 |  |  | -89.5 | -89 | -88.5 | -88 | FDD |
| CA_12B-66A-66A5,6 | 6621 |  |  | -89.5 | -89 | -88.5 | -88 | FDD |
| CA_20A-38A-40A-40A15,16 | 20 |  |  | -60.7 | -60.7 | -60.7 |  | FDD |
| CA_20A-38A-40C15,16 | 20 |  |  | -60.7 | -60.7 | -60.7 |  | FDD |
| CA_20A-40D7 | 20 |  |  | -60.7 | -60.7 | -60.7 |  | FDD |
| CA_21A-28A-42C4,14 | 21 |  |  | N/A | N/A | N/A | N/A | FDD |
|  | 42 |  |  | N/A | N/A |  |  | TDD |
| CA_25A-25A-26A-41C7 | 41 |  |  | N/A | N/A | N/A | N/A | TDD |
| CA_26A-41C7 | 41 |  |  | N/A | N/A | N/A | N/A | TDD |
| CA_28A-40C16,17 | 2821 |  |  | -60.7 | -60.7 | -60.7 | -60.7 | FDD |
| CA_28A-40D16,17 | 2821 |  |  | -60.7 | -60.7 | -60.7 | -60.7 | FDD |
| CA_28A-41A-42C12, 13 | 4221 |  |  |  | -85.4 | -85.1 | -84.9 | TDD |
| CA_28A-41C-42A12, 13 | 4221 |  |  |  | -85.4 | -85.1 | -84.9 | TDD |
| CA_28A-41C-42C12,13 | 4221 |  |  |  | -85.4 | -85.1 | -84.9 | TDD |
| CA_28A-42C12,13 CA_28A-42D12,13 | 4221 |  |  | -85.7 | -85.4 | -85.1 | -84.9 | TDD |
| CA_28A-41A-42A-42A12, 13 | 4221 |  |  |  | -85.4 | -85.1 | -84.9 | TDD |
| CA_28A-41A-42A-42C12, 13 | 4221 |  |  |  | -85.4 | -85.1 | -84.9 | TDD |
| CA_28A-41A-42A-42C12, 13 | 4221 |  |  |  | -85.4 | -85.1 | -84.9 | TDD |
| CA_28A-42A-42A12,13 CA_28A-42A-42C12,13 CA_28A-42C-42C12,13 | 4221 |  |  | -85.7 | -85.4 | -85.1 | -84.9 | TDD |
| CA_48A-48C-66A8,9 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_48A-48C-66A10 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_48A-48C-66B 8,9 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_48A-48C-66B 10 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_48A-48C-66C 8,9 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_48A-48C-66C 10 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_48A-48D-66A 8,9 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_48A-48D-66A 10 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_48C-48C-66A 8,9 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_48C-48C-66A 10 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| CA_48D-66A8,9 | 48 |  |  | -71.7 | -71.7 | -71.7 | -71.7 | TDD |
| CA_48D-66A10 | 48 |  |  | -97.1 | -94.7 | -93.2 | -92.5 | TDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: The signal power is specified per port NOTE 4: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity for all active downlink component carriers is only verified when this is not the case (the requirements specified in clause 7.3.1 apply unless otherwise specified). NOTE 5: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 6: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 7: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity is only verified when this is not the case (the requirements specified in clause 7.3.1 apply). NOTE 8: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_3A-42C, CA_3A-42D, CA_3A-3A-42C, CA_3A-42A-42C, CA_3A-42C-42C, CA_1A-3A-18A-42C, CA_1A-3A-19A-42C, CA_1A-3A-21A-42C, CA_1A-3A-41A-42C, CA_1A-3A-41C-42A, CA_1A-3A-41C-42C, CA_1A-3A-42C, CA_3A-28A-42C, CA_3A-28A-42D, CA_3A-19A-42C, CA_3A-19A-42D, CA_3A-21A-42D, CA_3A-28A-40C, CA_3A-28A-40D, CA_3A-28A-41A-42C, CA_3A-28A-41C-42A, CA_3A-28A-41C-42C, CA_3A-28A-42A-42A, CA_3A-28A-42A-42C, CA_3A-28A-42C-42C, CA_3A-41A-42A-42A, CA_3A-41A-42A-42C, CA_3A-41A-42C-42C, CA_2A-48C-48C, CA_2A-48A-48D, CA_48A-48C-66C, CA_48A-48C-66B, CA_48A-48D-66A, CA_48C-48C-66A, CA_48D-66A, CA_48A-48C-66A, CA_2A-13A-48D, CA_2A-13A-48A-48C, CA_2A-13A-48C-66A, CA_2A-13A-48A-48A-66A, CA_2A-48D-66A, CA_2A-48E-66A, CA_2A-48A-48C-66A and CA_2A-48E. NOTE 9: The requirements should be verified for UL EARFCN of the aggressor (lower) band (superscript LB) such that in MHz and with carrier frequency in the victim (higher) band in MHz and the channel bandwidth configured in the lower band. NOTE 10: The requirements are only applicable to channel bandwidths with a carrier frequency at MHz offset from in the victim (higher band) with , whereandare the channel bandwidths configured in the aggressor (lower) and victim (higher) bands in MHz, respectively. NOTE 11: Void NOTE 12: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 5th transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 13: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 14: No requirements apply when there is at least one individual RE on band 28 uplink outside frequencies 728 – 738 MHz. The reference sensitivity is only verified when all configured RE’s are confined within frequencies 728 – 738 MHz (the requirements specified in clause 7.3.1 of [6] apply). NOTE 15: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 4th transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 16: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 17: These requirements apply when there is at least one individual RE within the downlink transmission bandwidth of the victim (lower) band for which the 3rd harmonic is within the uplink transmission bandwidth or the uplink adjacent channel’s transmission bandwidth of an aggressor (higher) band. NOTE 18: The requirements should be verified for UL EARFCN of the aggressor (higher) band (superscript HB) such that in MHz and with the carrier frequency in the victim (lower) band and the channel bandwidth configured in the higher band. NOTE 19: The requirement for B46 does not apply when there is at least one individual RE within the B46 downlink transmission bandwidth which falls into the reference sensitivity exclusion region as specified in Table 6.2.4-3. NOTE 20: The B41 requirements are modified by -0.1dB when carrier frequency of the assigned E-UTRA channel bandwidth is within 2545-2690 MHz. NOTE 21: Applicable for the operations with 2 or 4 antenna ports supported in the band with carrier aggregation configured. NOTE 22: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the aggressor (lower) band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of a victim (higher) band and a range FHD above and below the edge of this downlink transmission bandwidth. The value FHD depends on the E-UTRA configuration: FHD = 10 MHz for CA_2A-48A-48C |  |  |  |  |  |  |  |  |

Table 7.3.1A-6: Uplink configuration for the low band (exceptions due to harmonic issues in the combinations of intra-band and inter-band CA)

| E-UTRA Band / Channel bandwidth of the high band / NRB / Duplex mode |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | UL band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex mode |
| CA_1A-3A-3A-7A-8A CA_1A-3A-7A-7A-8A CA_1A-3A-3A-7A-7A-8A | 8 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_1A-1A-3A-7A-28A CA_1A-1A-3A-7C-28A CA_1A-1A-3C-7A-28A CA_1A-1A-3C-7C-28 CA_1A-1A-3A-3A-7A-28A CA_1A-1A-3A-3A-7C-28A CA_1A-3A-7C-28A CA_1A-3C-7A-28A CA_1A-3C-7C-28A CA_1A-3A-3A-7A-28A CA_1A-3A-3A-7C-28A CA_1A-3A-7A-7A-28A | 28 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_1A-3A-8A-42C | 8 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_1A-3A-8A-42C | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_1A-3A-18A-42C | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_1A-3A-19A-42C | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_1A-3A-3A-20A-28A | 28 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_1A-3A-21A-42C | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_1A-3C-28A | 28 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_1A-3A-28A-40C | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_1A-3A-28A-42C | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_1A-3A-42C | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_1A-3A-42D | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_1A-3A-3A-42C | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_1A-7A-8A-40C | 8 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_1A-7C-28A | 28 |  |  |  | 16 | 25 | 25 | FDD |
| CA_1A-7A-28A-40C | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_1A-8A-11A-42C | 8 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_1A-28A-42C | 28 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_2A-2A-4A-12A | 12 |  |  | 8 | 16 | 20 | 20 | FDD |
| CA_2A-2A-5A-12A-66A | 12 |  |  | 8 | 16 | 20 | 20 | FDD |
| CA_2A-2A-7A-12A-66A | 12 |  |  | 8 | 16 |  |  | FDD |
| CA_2A-2A-12A-30A-66A | 12 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_2A-2A-12A-66A | 12 |  |  | 8 | 16 |  |  | FDD |
| CA_2A-2A-12A-66A-66A | 12 |  |  | 8 | 16 | 20 | 20 | FDD |
| CA_2A-2A-12B-66A | 12 |  |  | 8 | 16 | 20 | 20 | FDD |
| CA_2A-4A-4A-12A | 12 |  |  | 8 | 16 | 20 | 20 | FDD |
| CA_2A-4A-12A-12A | 12 |  |  | 8 | 16 | 20 | 20 | FDD |
| CA_2A-4A-12B | 12 |  |  | 8 | 16 |  |  | FDD |
| CA_2A-7A-12B-66A | 12 |  |  | 8 | 16 |  |  | FDD |
| CA_2A-12A-30A-66A-66A | 12 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_2A-12A-66A-66A | 12 |  |  | 8 | 16 |  |  | FDD |
| CA_2A-12A-66C | 12 |  |  | 8 | 16 |  |  | FDD |
| CA_2A-12B-66A | 12 |  |  | 8 | 16 | 20 | 20 | FDD |  |
| CA_2A-12B-66A-66A | 12 |  |  | 8 | 16 | 20 | 20 | FDD |
| CA_2A-13A-48A-48A CA_2A-13A-48C | 2 |  |  | 25 | 50 | 501 | 501 | FDD |
| CA_2A-13A-48D | 2 |  |  | 25 | 50 | 501 | 501 | FDD |
| CA_2A-13A-48A-48C | 2 |  |  | 25 | 50 | 501 | 501 | FDD |
| CA_2A-13A-48C-66A | 2 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 66 |  |  | 121 | 251 | 361 | 501 | FDD |
| CA_2A-13A-48A-48A-66A | 2 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 66 |  |  | 121 | 251 | 361 | 501 | FDD |
| CA_2A-48A-48D | 2 |  |  | 25 | 50 | 501 | 501 | FDD |
| CA_2A-48C-48C | 2 |  |  | 25 | 50 | 501 | 501 | FDD |
| CA_2A-48D | 2 |  |  | 25 | 50 | 501 | 501 | FDD |
| CA_2A-48E | 2 |  |  | 25 | 50 | 501 | 501 | FDD |
| CA_2A-48A-48A-66A | 2 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 66 |  |  | 121 | 251 | 361 | 501 | FDD |
| CA_2A-48D-66A | 2 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 66 |  |  | 121 | 251 | 361 | 501 | FDD |
| CA_2A-48E-66A | 2 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 66 |  |  | 121 | 251 | 361 | 501 | FDD |
| CA_2A-48A-48C-66A | 2 |  |  | 25 | 50 | 501 | 501 | FDD |
|  | 66 |  |  | 121 | 251 | 361 | 501 | FDD |
| CA_3A-3A-7A-7A-8A | 8 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_3A-3A-7A-8A | 8 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_3A-7A-7A-8A | 8 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_3A-7A-8A-40C | 8 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_3A-19A-42C | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_3A-19A-42D | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_3A-7A-28A-40C | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3A-42C | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_3A-3A-42D | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_3A-42C-42C | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_3A-42E | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_3A-21A-42C | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_3A-21A-42D | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_3A-28A-40C | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_3A-28A-40D | 40 |  |  |  | 50 | 75 | 100 | TDD |
| CA_3A-28A-41A-42C | 3 |  |  |  | 25 | 36 | 50 | FDD |
|  | 28 |  |  |  | 10 | 15 | 20 | FDD |
| CA_3A-28A-41C-42A | 3 |  |  |  | 25 | 36 | 50 | FDD |
|  | 28 |  |  |  | 10 | 15 | 20 | FDD |
| CA_3A-28A-41C-42C | 3 |  |  |  | 25 | 36 | 50 | FDD |
|  | 28 |  |  |  | 10 | 15 | 20 | FDD |
| CA_3A-28A-42C | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
|  | 28 |  |  | 5 | 10 | 15 | 20 | FDD |
| CA_3A-28A-42A-42A | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
|  | 28 |  |  | 5 | 10 | 15 | 20 | FDD |
| CA_3A-28A-42A-42C | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
|  | 28 |  |  | 5 | 10 | 15 | 20 | FDD |
| CA_3A-28A-42C-42C | 3 |  |  | 12 | 25 | 36 | 50 | FDD |
|  | 28 |  |  | 5 | 10 | 15 | 20 | FDD |
| CA_3A-41A-42A-42A | 3 |  |  |  | 25 | 36 | 50 | FDD |
| CA_3A-41A-42A-42C | 3 |  |  |  | 25 | 36 | 50 | FDD |
| CA_3A-41A-42C-42C | 3 |  |  |  | 25 | 36 | 50 | FDD |
| CA_4A-4A-5A-12A | 12 |  |  | 8 | 16 | 20 | 20 | FDD |
| CA_4A-4A-12A | 12 |  |  | 8 | 16 | 20 | 20 | FDD |
| CA_4A-4A-12A-12A | 12 |  |  | 8 | 16 | 20 | 20 | FDD |
| CA_4A-4A-12A-30A | 12 |  |  | 8 | 16 | 20 | 20 | FDD |
| CA_4A-4A-12B | 12 |  |  | 8 | 16 |  |  | FDD |
| CA_4A-12A-12A | 12 |  |  | 8 | 16 | 20 | 20 | FDD |
| CA_4A-12B | 12 |  |  | 8 | 16 | 20 | 20 | FDD |
| CA_4A-5A-12A-12A | 12 |  |  | 8 | 16 | 20 | 20 | FDD |
| CA_4A-5A-12B | 12 |  |  | 8 | 16 |  |  | FDD |
| CA_7A-7A-8A | 8 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_8A-42C | 8 |  |  | 8 | 16 | 25 | 25 | FDD |
| CA_12A-30A-66A-66A | 12 |  |  | 8 | 16 |  |  | FDD |
| CA_12A-66A-66A | 12 |  |  | 8 | 16 | 20 | 20 | FDD |
| CA_12A-66C | 12 |  |  | 8 | 16 | 20 | 20 | FDD |
| CA_12B-66A | 12 |  |  | 8 | 16 | 20 | 20 | FDD |
| CA_12B-66A-66A | 12 |  |  | 8 | 16 | 20 | 20 | FDD |
| CA_20A-38A-40A-40A3 | 40 |  |  | 25 | 50 | 75 |  | TDD |
| CA_20A-38A-40C3 | 40 |  |  | 25 | 50 | 75 |  | TDD |
| CA_20A-40D3 | 40 |  |  | 25 | 50 | 75 |  | TDD |
| CA_28A-40C | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_28A-40D | 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| CA_28A-41A-42C | 28 |  |  |  | 10 | 15 | 20 | FDD |
| CA_28A-41C-42A | 28 |  |  |  | 10 | 15 | 20 | FDD |
| CA_28A-41C-42C | 28 |  |  |  | 10 | 15 | 20 | FDD |
| CA_28A-41A-42A-42A | 28 |  |  |  | 10 | 15 | 20 | FDD |
| CA_28A-41A-42A-42C | 28 |  |  |  | 10 | 15 | 20 | FDD |
| CA_28A-41A-42C-42C | 28 |  |  |  | 10 | 15 | 20 | FDD |
| CA_28A-42C CA_28A-42D | 28 |  |  | 5 | 10 | 15 | 20 | FDD |
| CA_28A-42A-42A CA_28A-42A-42C CA_28A-42C-42C | 28 |  |  | 5 | 10 | 15 | 20 | FDD |
| CA_48A-48C-66A | 66 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_48A-48C-66B | 66 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_48A-48C-66C | 66 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_48A-48D-66A | 66 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_48C-48C-66A | 66 |  |  | 12 | 25 | 36 | 50 | FDD |
| CA_48D-66A | 66 |  |  | 12 | 25 | 36 | 50 | FDD |
| NOTE 1: refers to the UL resource blocks, which shall be centred within the transmission bandwidth configuration for the channel bandwidth. NOTE 2: the UL configuration applies regardless of the channel bandwidth of the low band unless the UL resource blocks exceed that specified in Table 7.3.1-2 for the uplink bandwidth in which case the allocation according to Table 7.3.1-2 applies. NOTE 3: refers to the UL resource blocks shall be located between 2373-2400MHz. |  |  |  |  |  |  |  |  |

### 7.3.1B	Minimum requirements (QPSK) for UL-MIMO

For UE with two transmitter antenna connectors in closed-loop spatial multiplexing scheme, the minimum requirements in Clause 7.3.1 shall be met with the UL-MIMO configurations specified in Table 6.2.2B-2. For UL-MIMO, the parameter PUMAX is the total transmitter power over the two transmits power over the two transmit antenna connectors.

### 7.3.1D	Minimum requirements (QPSK) for ProSe

When UE is configured for E-UTRA ProSe reception non-concurrent with E-UTRA uplink transmissions for E-UTRA ProSe operating bands specified in Table 5.5D-1, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.6.2 with parameters specified in Table 7.3.1D-1 and Table 7.3.1D-2.

Table 7.3.1D-1: Reference sensitivity for ProSe Direct Discovery QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA ProSe Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| 2 |  |  | -104.1 | -104.1 | -104.1 | -104.1 | HD |
| 3 |  |  | -103.1 | -103.1 | -103.1 | -103.1 | HD |
| 4 |  |  | -106.1 | -106.1 | -106.1 | -106.1 | HD |
| 7 |  |  | -103.8 | -103.8 | -103.8 | -103.8 | HD |
| 14 |  |  | -103.1 | -103.1 |  |  | HD |
| 20 |  |  | -103.2 | -103.2 | -102.2 | -102.2 | HD |
| 26 |  |  | -103.55 | -103.5 5 | -103.5 5 |  | HD |
| 28 |  |  | -104.4 | -104.4 | -104.4 | -102.9 | HD |
| 31 |  |  | -99.5 |  |  |  | HD |
| 68 |  |  | -104.4 | -104.4 | -104.4 |  | HD |
| 72 |  |  | -99.5 |  |  |  | HD |
| NOTE 1: Reference measurement channel is A.6.2 NOTE 2: The signal power is specified per port NOTE 3: For the UE which supports both Band 3 and Band 9 the reference sensitivity level is FFS. NOTE 4: For the UE which supports both Band 11 and Band 21 the reference sensitivity level is FFS. NOTE 5: 5 indicates that the requirement is modified by -0.5 dB when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 6: For a UE that support both Band 18 and Band 26, the reference sensitivity level for Band 26 applies for the applicable channel bandwidths. |  |  |  |  |  |  |  |

Table 7.3.1D-2: Reference sensitivity for ProSe Direct Communication QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA ProSe Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| 3 |  |  |  | -97.6 |  |  | HD |
| 7 |  |  |  | -98.3 |  |  | HD |
| 14 |  |  |  | -97.6 |  |  | HD |
| 20 |  |  |  | -97.7 |  |  | HD |
| 26 |  |  |  | -98.0 5 |  |  | HD |
| 28 |  |  |  | -98.9 |  |  | HD |
| 31 |  |  | -96.7 |  |  |  | HD |
| 68 |  |  | -101.7 | -98.9 |  |  | HD |
| 72 |  |  | -96.7 |  |  |  | HD |
| NOTE 1: Reference measurement channel is A.6.2 NOTE 2: The signal power is specified per port NOTE 3: For the UE which supports both Band 3 and Band 9 the reference sensitivity level is FFS. NOTE 4: For the UE which supports both Band 11 and Band 21 the reference sensitivity level is FFS. NOTE 5: 5 indicates that the requirement is modified by -0.5 dB when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. NOTE 6: For a UE that support both Band 18 and Band 26, the reference sensitivity level for Band 26 applies for the applicable channel bandwidths. |  |  |  |  |  |  |  |

NOTE:	Table 7.3.1D-1/ Table 7.3.1D-2 is intended for conformance tests and does not necessarily reflect the operational conditions of the network, where the number of allocated resource blocks will be practically constrained by other factors.

For the UE which supports ProSe in an operating band as specified in Section 5.5D, and the UE also supports a E-UTRA downlink inter-band carrier aggregation configuration in Table 7.3.1-1A or Table 7.3.1-1B, the minimum requirement for reference sensitivity in Table 7.3.1D-1 and Table 7.3.1D-2 shall be increased by the amount given in ΔRIB,c in Table 7.3.1-1A and Table 7.3.1-1B for the corresponding E-UTRA ProSe band.

When UE is configured for E-UTRA ProSe reception on PCC for the inter-band E-UTRA ProSe / E-UTRA bands specified in Table 5.5D-2, there are no further requirements for reference sensitivity beyond those specified above when only PCC is configured in Table 7.3.1D-1 and Table 7.3.1D-2.

When UE is configured for E-UTRA ProSe reception on SCC or a non-serving carrier concurrent with E-UTRA uplink for inter-band E-UTRA ProSe / E-UTRA bands specified in Table 5.5D-2, E-UTRA ProSe throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.6.2 with parameters specified in Table 7.3.1D-1 and Table 7.3.1D-2. The reference sensitivity is defined to be met with E-UTRA uplink assigned to one band (that differs from the ProSe operating band) and all E-UTRA downlink carriers active. The E-UTRA uplink resource blocks shall be located as close as possible to E-UTRA ProSe operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1). The uplink configuration for the E-UTRA operating band is specified in Table 7.3.1D-3.

NOTE:	The E-UTRA uplink channel bandwidth and transmission bandwidth specified in this Table 7.3.1D-3 are intended for conformance tests and does not restrict the operating conditions of the network.

Table 7.3.1D-3: Uplink configuration for E-UTRA band / E-UTRA CA band

| Inter-band E-UTRA ProSe/E-UTRA configuration |  | E-UTRA UL band / Channel BW / NRB / Duplex mode |  |  |  |
| --- | --- | --- | --- | --- | --- |
| E-UTRA ProSe band | E-UTRA band / E-UTRA CA band | E-UTRA UL band | Channel Bandwidth (MHz) | NRB | Duplex Mode |
| 2 | 4 | 4 | 5 | 25 | FDD |
| 2 | CA_2-4 | 4 | 5 | 25 | FDD |
| 28 | 1 | 1 | 5 | 25 | FDD |
| 28 | CA_1-28 | 1 | 5 | 25 | FDD |
| NOTE 1: For E-UTRA ProSe reception on SCC, the channel bandwith of the E-UTRA downlink SCC is set same as the ProSe channel bandwidth for which reference sensitivity is being measured. |  |  |  |  |  |

### 7.3.1E	Minimum requirements (QPSK) for UE category 0, M1, M2 and 1bis

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.3.1E-1A/Table 7.3.1E-1B and Table 7.3.1E-2 for category 0,  Table 7.3.1E-3/Table 7.3.1E-4 for category M1, and Table 7.3.1E-6/Table 7.3.1E-7 for category 1bis, and Table 7.3.1E-8/Table 7.3.1E-9 for category M2.

Table 7.3.1E-1A: Reference sensitivity for FDD and TDD UE category 0 QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| 2 | -100.2 | -97.2 | -95.5 | -92.5 | -90.7 | -89.5 | FDD |
| 3 | -99.2 | -96.2 | -94.5 | -91.5 | -89.7 | -88.5 | FDD |
| 4 | -102.2 | -99.2 | -97.5 | -94.5 | -92.7 | -91.5 | FDD |
| 5 | -100.7 | -97.7 | -95.5 | -92.5 |  |  | FDD |
| 8 | -99.7 | -96.7 | -94.5 | -91.5 |  |  | FDD |
| 13 |  |  | -94 | -91 |  |  | FDD |
| 20 |  |  | -94.5 | -91.5 | -88.2 | -87 | FDD |
| 25 | -98.7 | -95.7 | -94 | -91 | -89.2 | -88 | FDD |
| 26 | -100.2 | -97.2 | -953 | -923 | -90.23 |  | FDD |
| 28 |  | -97.7 | -96 | -93 | -91.2 | -88.5 | FDD |
| 39 |  |  | -97.5 | -94.5 | -92.7 | -91.5 | TDD |
| 40 |  |  | -97.5 | -94.5 | -92.7 | -91.5 | TDD |
| 41 |  |  | -95.5 | -92.5 | -90.7 | -89.5 | TDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5 NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: The requirement is modified by -0.5 dB when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz |  |  |  |  |  |  |  |

Table 7.3.1E-1B: Reference sensitivity for HD-FDD UE category 0 QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| 2 | -101 | -98 | -96.3 | -93.3 | -91.5 | -90.3 | HD-FDD |
| 3 | -100 | -97 | -95.3 | -92.3 | -90.5 | -89.3 | HD-FDD |
| 4 | -103 | -100 | -98.3 | -95.3 | -93.5 | -92.3 | HD-FDD |
| 5 | -101.5 | -98.5 | -96.3 | -93.3 |  |  | HD-FDD |
| 8 | -100.5 | -97.5 | -95.3 | -92.3 |  |  | HD-FDD |
| 13 |  |  | -95.3 | -92.3 |  |  | HD-FDD |
| 20 |  |  | -95.3 | -92.3 | -89.5 | -88.3 | HD-FDD |
| 25 | -99.5 | -96.5 | -94.8 | -91.8 | -90 | -88.8 | HD-FDD |
| 26 | -101 | -98 | -95.8 | -92.8 |  |  | HD-FDD |
| 28 |  | -98.5 | -96.8 | -93.8 | -92 | -89.3 | HD-FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5 NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 |  |  |  |  |  |  |  |

The reference receive sensitivity (REFSENS) requirement specified in Table 7.3.1E-1A/Table 7.3.1E-1B shall be met for an uplink transmission bandwidth less than or equal to that specified in Table 7.3.1E-2.

Unless given by Table 7.3.1-3, the minimum requirements specified in Table 7.3.1E-1A/Table 7.3.1E-1B shall be verified with the network signalling value NS_01 (Table 6.2.4E-1) configured.

NOTE:	Table 7.3.1E-2 is intended for conformance tests and does not necessarily reflect the operational conditions of the network, where the number of uplink and downlink allocated resource blocks will be practically constrained by other factors. Typical receiver sensitivity performance with HARQ retransmission enabled and using a residual BLER metric relevant for e.g. Speech Services is given in the Annex G (informative).

Table 7.3.1E-2: FDD and TDD UE category 0 Uplink configuration for reference sensitivity

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| 2 | 6 | 15 | 25 | 361 | 361 | 361 | FDD and HD-FDD |
| 3 | 6 | 15 | 25 | 361 | 361 | 361 | FDD and HD-FDD |
| 4 | 6 | 15 | 25 | 361 | 361 | 361 | FDD and HD-FDD |
| 5 | 6 | 15 | 25 | 251 |  |  | FDD and HD-FDD |
| 8 | 6 | 15 | 25 | 251 |  |  | FDD and HD-FDD |
| 13 |  |  | 201 | 201 |  |  | FDD and HD-FDD |
| 20 |  |  | 25 | 201 | 202 | 202 | FDD and HD-FDD |
| 25 | 6 | 15 | 25 | 361 | 361 | 361 | FDD and HD-FDD |
| 26 | 6 | 15 | 25 | 251 | 251 |  | FDD and HD-FDD |
| 28 |  | 15 | 25 | 251 | 251 | 251 | FDD and HD-FDD |
| 39 |  |  | 25 | 361 | 361 | 361 | TDD |
| 40 |  |  | 25 | 361 | 361 | 361 | TDD |
| 41 |  |  | 25 | 361 | 361 | 361 | TDD |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1). NOTE 2: 2 refers to Band 20; in the case of 15MHz channel bandwidth, the UL resource blocks shall be located at RBstart 11 and in the case of 20MHz channel bandwidth, the UL resource blocks shall be located at RBstart 16. |  |  |  |  |  |  |  |

Table 7.3.1E-3: Reference sensitivity for FDD and TDD UE category M1 QPSK PREFSENS

| E-UTRA Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 1 | -102.2 | FDD |
| 2 | -100.2 | FDD |
| 3 | -99.2 | FDD |
| 4 | -102.2 | FDD |
| 5 | -100.7 | FDD |
| 7 | -100.2 | FDD |
| 8 | -99.7 | FDD |
| 11 | -102.23 | FDD |
| 12 | -99.2 | FDD |
| 13 | -98.7 | FDD |
| 14 | -98.7 | FDD |
| 18 | -102.24 | FDD |
| 19 | -102.2 | FDD |
| 20 | -99.7 | FDD |
| 21 | -102.23 | FDD |
| 25 | -98.7 | FDD |
| 26 | -100.2 | FDD |
| 27 | -100.7 | FDD |
| 28 | -100.7 | FDD |
| 31 | -96.5 | FDD |
| … |  |  |
| 39 | -103.7 | TDD |
| 40 | -103.7 | TDD |
| 41 | -101.7 | TDD |
| 42 | -102.7 | TDD |
| 43 | -102.7 | TDD |
| … |  |  |
| 71 | 99.4 | FDD |
| 72 | -96.5 | FDD |
| 73 | -96.5 | FDD |
| 74 | -101.78 | FDD |
| 85 | -99.2 | FDD |
| 87 | -96.5 | FDD |
| 88 | -96.5 | FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5 NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: For the UE which supports both Band 11 and Band 21 the reference sensitivity level is FFS. NOTE 4: For a UE that support both Band 18 and Band 26, the reference sensitivity level for Band 26 applies for the applicable channel bandwidths. NOTE 5: For cat M1 the same reference sensitivity requirement applies for all applicable channel bandwidths (Table 5.6.1-1) NOTE 6: The reference receive sensitivity shall be met for an uplink transmission bandwidth less than or equal to 6 RB except for band 31 and 72. For band 31 and 72; in the case of 3 MHz channel bandwidth 5 RB applies and the UL resource blocks shall be located at RBstart 9. In case of 5 MHz channel bandwidth 5 RB applies and the UL resource blocks shall be located at RBstart 10. NOTE 7: The UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth. NOTE 8: 8 indicates that the requirement is modified by -0.5 dB when the assigned E-UTRA channel bandwidth is confined within 1475.9-1510.9 MHz. |  |  |

Table 7.3.1E-4: Reference sensitivity for HD-FDD UE category M1 QPSK PREFSENS

| E-UTRA Band | REFSENS (dBm) | Duplex Mode |
| --- | --- | --- |
| 1 | -103 | HD-FDD |
| 2 | -101 | HD-FDD |
| 3 | -100 | HD-FDD |
| 4 | -103 | HD-FDD |
| 5 | -101.5 | HD-FDD |
| 7 | -101 | HD-FDD |
| 8 | -100.5 | HD-FDD |
| 11 | -1033 | HD-FDD |
| 12 | -100 | HD-FDD |
| 13 | -100 | HD-FDD |
| 14 | -100 | HD-FDD |
| 18 | -1034 | HD-FDD |
| 19 | -103 | HD-FDD |
| 20 | -100.5 | HD-FDD |
| 21 | -1033 | HD-FDD |
| 25 | -99.5 | HD-FDD |
| 26 | -101 | HD-FDD |
| 27 | -101.5 | HD-FDD |
| 28 | -101.5 | HD-FDD |
| 31 | -97.3 | HD-FDD |
| … |  |  |
| 71 | -100.2 | HD-FDD |
| 72 | -97.3 | HD-FDD |
| 73 | -97.3 | HD-FDD |
| 74 | -103 | HD-FDD |
| 85 | -100 | HD-FDD |
| 87 | -97.3 | HD-FDD |
| 88 | -97.3 | HD-FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5 NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: For the UE which supports both Band 11 and Band 21 the reference sensitivity level is FFS. NOTE 4: For a UE that support both Band 18 and Band 26, the reference sensitivity level for Band 26 applies for the applicable channel bandwidths. NOTE 5: For cat M1 the same reference sensitivity requirement applies for all applicable channel bandwidths (Table 5.6.1-1) |  |  |

The reference receive sensitivity (REFSENS) requirement specified in Table 7.3.1E-3/Table 7.3.1E-4 shall be met for an uplink transmission bandwidth less than or equal to that specified in Table 7.3.1E-5.

NOTE:	Table 7.3.1E-5 is intended for conformance tests and does not necessarily reflect the operational conditions of the network, where the number of uplink and downlink allocated resource blocks will be practically constrained by other factors. Typical receiver sensitivity performance with HARQ retransmission enabled and using a residual BLER metric relevant for e.g. Speech Services is given in the Annex G (informative).

Table 7.3.1E-5: FDD and TDD UE category M1 Uplink configuration for reference sensitivity

| E-UTRA Band |  | NRB |  | Duplex Mode |  |
| --- | --- | --- | --- | --- | --- |
| 1 |  | 61 |  | FDD and HD-FDD |  |
| 2 |  | 61 |  | FDD and HD-FDD |  |
| 3 |  | 61 |  | FDD and HD-FDD |  |
| 4 |  | 61 |  | FDD and HD-FDD |  |
| 5 |  | 61 |  | FDD and HD-FDD |  |
| 7 |  | 61 |  | FDD and HD-FDD |  |
| 8 |  | 61 |  | FDD and HD-FDD |  |
| 11 |  | 61 |  | FDD and HD-FDD |  |
| 12 |  | 61 |  | FDD and HD-FDD |  |
| 13 |  | 61 |  | FDD and HD-FDD |  |
| 14 |  | 61 |  | FDD and HD-FDD |  |
| 18 |  | 61 |  | FDD and HD-FDD |  |
| 19 |  | 61 |  | FDD and HD-FDD |  |
| 20 |  | 61 |  | FDD and HD-FDD |  |
| 21 |  | 61 |  | FDD and HD-FDD |  |
| 25 |  | 61 |  | FDD and HD-FDD |  |
| 26 |  | 61 |  | FDD and HD-FDD |  |
| 27 |  | 61 |  | FDD and HD-FDD |  |
| 28 |  | 61 |  | FDD and HD-FDD |  |
| 31 |  | 61 |  | FDD and HD-FDD |  |
| … |  |  |  |  |  |
| 39 |  | 61 |  | TDD |  |
| 40 |  | 61 |  | TDD |  |
| 41 |  | 61 |  | TDD |  |
| 42 |  | 61 |  | TDD |  |
| 43 |  | 61 |  | TDD |  |
| … |  |  |  |  |  |
| 71 |  | 61 |  | FDD and HD-FDD |  |
| 72 |  | 61 |  | FDD and HD-FDD |  |
| 73 |  | 61 |  | FDD and HD-FDD |  |
| 74 |  | 61 |  | FDD and HD-FDD |  |
| 85 |  | 61 |  | FDD and HD-FDD |  |
| 87 |  | 61 |  | FDD and HD-FDD |  |
| 88 |  | 61 |  | FDD and HD-FDD |  |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1). |  |  |  |  |  |

Table 7.3.1E-6: Reference sensitivity for FDD and TDD UE category 1bis QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| 1 |  |  | -97.5 | -94 | -92.2 | -91 | FDD |
| 2 | -100.2 | -97.2 | -95.5 | -92 | -90.2 | -89 | FDD |
| 3 | -99.2 | -96.2 | -94.5 | -91 | -89.2 | -88 | FDD |
| 4 | -102.2 | -99.2 | -97.5 | -94 | -92.2 | -91 | FDD |
| 5 | -100.7 | -97.7 | -95.5 | -92.5 |  |  | FDD |
| 7 |  |  | -95.5 | -92 | -90.2 | -89 | FDD |
| 8 | -99.7 | -96.7 | -94.5 | -91.5 |  |  | FDD |
| 12 | -98.7 | -95.7 | -94 | -91 |  |  | FDD |
| 13 |  |  | -94 | -91 |  |  | FDD |
| 18 |  |  | -97.5 | -94.5 | -92.7 |  | FDD |
| 20 |  |  | -94.5 | -91.5 | -88.2 | -87 | FDD |
| 26 | -100.2 | -97.2 | -95.03 | -92.03 | -90.23 |  | FDD |
| 28 |  | -97.7 | -96.0 | -93.0 | -91.2 | -88.5 | FDD |
| 31 | -96.5 | -92.5 | -90.5 |  |  |  | FDD |
| 34 |  |  | -97.5 | -94.5 | -92.7 |  |  |
| 39 |  |  | -97.5 | -94.5 | -92.7 | -91.5 | TDD |
| 40 |  |  | -97.5 | -94.5 | -92.7 | -91.5 | TDD |
| 41 |  |  | -95.5 | -92.5 | -90.7 | -89.5 | TDD |
| 66 | -101.7 | -98.7 | -97 | -93.5 | -91.7 | -90.5 | FDD |
| 72 | -96.5 | -92.5 | -90.5 |  |  |  | FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5 NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: 3 indicates that the requirement is modified by -0.5 dB when the carrier frequency of the assigned E-UTRA channel bandwidth is within 865-894 MHz. |  |  |  |  |  |  |  |

Table 7.3.1E-7: FDD and TDD UE category 1bis Uplink configuration for reference sensitivity

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| 1 |  |  | 25 | 50 | 75 | 100 | FDD |
| 2 | 6 | 15 | 25 | 50 | 501 | 501 | FDD |
| 3 | 6 | 15 | 25 | 50 | 501 | 501 | FDD |
| 4 | 6 | 15 | 25 | 50 | 75 | 100 | FDD |
| 5 | 6 | 15 | 25 | 251 |  |  | FDD |
| 7 |  |  | 25 | 50 | 75 | 751 | FDD |
| 8 | 6 | 15 | 25 | 251 |  |  | FDD |
| 12 | 6 | 15 | 201 | 201 |  |  | FDD |
| 13 |  |  | 201 | 201 |  |  | FDD |
| 18 |  |  | 25 | 25 | 25 |  | FDD |
| 20 |  |  | 25 | 201 | 202 | 202 | FDD |
| 26 | 6 | 15 | 25 | 251 | 251 |  | FDD |
| 28 |  | 15 | 25 | 251 | 251 | 251 | FDD |
| 31 | 6 | 53 | 53 |  |  |  | FDD |
| 34 |  |  | 25 | 50 | 75 |  |  |
| 39 |  |  | 25 | 50 | 75 | 100 | TDD |
| 40 |  |  | 25 | 50 | 75 | 100 | TDD |
| 41 |  |  | 25 | 50 | 75 | 100 | TDD |
| 66 | 6 | 15 | 25 | 50 | 75 | 100 | FDD |
| 72 | 6 | 53 | 53 |  |  |  | FDD |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1). NOTE 2: 2 refers to Band 20; in the case of 15MHz channel bandwidth, the UL resource blocks shall be located at RBstart 11 and in the case of 20MHz channel bandwidth, the UL resource blocks shall be located at RBstart 16. NOTE 3: 3 refers to Bands 31 and 72; in the case of 3 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 9 and in the case of 5 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 10. |  |  |  |  |  |  |  |

Table 7.3.1E-8: Reference sensitivity for FDD /TDD UE category M2 QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| 1 |  |  | -97.7 | -97.7 | -97.7 | -97.7 | FDD |
| 2 | -100.2 | -97.2 | -95.7 | -95.7 | -95.7 | -95.7 | FDD |
| 3 | -99.2 | -96.2 | -94.7 | -94.7 | -94.7 | -94.7 | FDD |
| 4 | -102.2 | -99.2 | -97.7 | -97.7 | -97.7 | -97.7 | FDD |
| 5 | -100.7 | -97.7 | -95.7 | -95.7 |  |  | FDD |
| 7 |  |  | -95.7 | -95.7 | -95.7 | -95.7 | FDD |
| 8 | -99.7 | -96.7 | -94.7 | -94.7 |  |  | FDD |
| 11 |  |  | -97.7 | -97.7 |  |  |  |
| 12 | -99.2 | -96.2 | -94.7 | -94.7 |  |  | FDD |
| 13 |  |  | -94.2 | -94.2 |  |  | FDD |
| … |  |  |  |  |  |  |  |
| 18 |  |  | -97.7 | -97.7 | -97.7 |  | FDD |
| 19 |  |  | -97.7 | -97.7 | -97.7 |  | FDD |
| 20 |  |  | -94.7 | -94.7 | -94.7 | -94.7 | FDD |
| 21 |  |  | -97.7 | -97.7 | -97.7 |  | FDD |
| 25 | -98.7 | -95.7 | -94.2 | -94.2 | -94.2 | -94.2 | FDD |
| 26 | -100.2 | -97.2 | -95.2 | -95.2 | -95.2 |  | FDD |
| 27 | -100.7 | -97.7 | -95.7 | -95.7 |  |  | FDD |
| 28 |  | -97.7 | -96.2 | -96.2 | -96.2 | -96.2 | FDD |
| 31 | -96.5 | -93.2 | -91.2 |  |  |  | FDD |
| … |  |  |  |  |  |  |  |
| 39 |  |  | -97.7 | -97.7 | -97.7 | -97.7 | TDD |
| 40 |  |  | -97.7 | -97.7 | -97.7 | -97.7 | TDD |
| 41 |  |  | -95.7 | -95.7 | -95.7 | -95.7 | TDD |
| 42 |  |  | -96.7 | -96.7 | -96.7 | -96.7 | TDD |
| 43 |  |  | -96.7 | -96.7 | -96.7 | -96.7 | TDD |
| 66 | -101.7 | -98.7 | -97.2 | -97.2 | -97.2 | -97.2 | FDD |
| … |  |  |  |  |  |  |  |
| 71 | [-99.4] | [-95.4] | [-93.4] | [-93.4] | [-93.4] | [-93.4] | FDD |
| 72 | [-96.5] | [-92.5] | [-90.5] |  |  |  | FDD |
| 73 | [-96.5] | [-92.5] | [-90.5] |  |  |  | FDD |
| 85 |  |  | -94.7 | -94.7 |  |  | FDD |
| 87 | -96.5 | -92.5 | -90.5 |  |  |  | FDD |
| 88 | -96.5 | -92.5 | -90.5 |  |  |  | FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5 NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: For the UE which supports both Band 11 and Band 21 the reference sensitivity level is FFS. NOTE 4: For a UE that support both Band 18 and Band 26, the reference sensitivity level for Band 26 applies for the applicable channel bandwidths. NOTE 5: The UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth. |  |  |  |  |  |  |  |

Table 7.3.1E-9: Reference sensitivity for HD-FDD category M2 QPSK PREFSENS

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| 1 |  |  | -97 | -97 | -97 | -97 | HD-FDD |
| 2 | -101 | -97 | -95 | -95 | -95 | -95 | HD-FDD |
| 3 | -100 | -96 | -94 | -94 | -94 | -94 | HD-FDD |
| 4 | -103 | -99 | -97 | -97 | -97 | -97 | HD-FDD |
| 5 | -101.5 | -97.5 | -95.5 | -95.5 |  |  | HD-FDD |
| 7 |  |  | -95 | -95 | -95 | -95 | HD-FDD |
| 8 | -100.5 | -96.5 | -94.5 | -94.5 |  |  | HD-FDD |
| 11 |  |  | -97 | -97 |  |  | HD-FDD |
| 12 | -100 | -96 | -94 | -94 |  |  | HD-FDD |
| 13 |  |  | -94 | -94 |  |  | HD-FDD |
| 14 |  |  | -94 | -94 |  |  | HD-FDD |
| 18 |  |  | -97 | -97 | -97 |  | HD-FDD |
| 19 |  |  | -97 | -97 | -97 |  | HD-FDD |
| 20 |  |  | -94.5 | -94.5 | -94.5 | -94.5 | HD-FDD |
| 21 |  |  | -97 | -97 | -97 |  | HD-FDD |
| 25 | -99.5 | -95.5 | -93.5 | -93.5 | -93.5 | -93.5 | HD-FDD |
| 26 | -101 | -97 | -95 | -95 | -95 |  | HD-FDD |
| 27 | -101.5 | -97.5 | -95.5 | -95.5 |  |  | HD-FDD |
| 28 |  | -97.5 | -95.5 | -95.5 | -95.5 | -95.5 | HD-FDD |
| 31 | -97.3 | -93.3 | -91.3 |  |  |  | HD-FDD |
| 71 | -100.2 | -96.2 | -94.2 | -94.2 | -94.2 | -94.2 | HD-FDD |
| 72 | -97.3 | -93.3 | -91.3 |  |  |  | HD-FDD |
| 73 | -97.3 | -93.3 | -91.3 |  |  |  | HD-FDD |
| 85 |  |  | -94 | -94 |  |  | HD-FDD |
| 87 | -97.3 | -93.3 | -91.3 |  |  |  | HD-FDD |
| 88 | -97.3 | -93.3 | -91.3 |  |  |  | HD-FDD |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5 NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: For the UE which supports both Band 11 and Band 21 the reference sensitivity level is FFS. NOTE 4: For a UE that support both Band 18 and Band 26, the reference sensitivity level for Band 26 applies for the applicable channel bandwidths. |  |  |  |  |  |  |  |

Table 7.3.1E-10: FDD/HD-FDD and TDD UE category M2 Uplink configuration for reference sensitivity

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| 1 |  |  | 24 | 24 | 24 | 24 | FDD/HD-FDD |
| 2 | 6 | 15 | 24 | 24 | 24 | 24 | FDD/HD-FDD |
| 3 | 6 | 15 | 24 | 24 | 24 | 24 | FDD/HD-FDD |
| 4 | 6 | 15 | 24 | 24 | 24 | 24 | FDD/HD-FDD |
| 5 | 6 | 15 | 24 | 24 |  |  | FDD/HD-FDD |
| 7 |  |  | 24 | 24 | 24 | 24 | FDD/HD-FDD |
| 8 | 6 | 15 | 24 | 24 |  |  | FDD/HD-FDD |
| 11 |  |  | 24 | 24 |  |  | FDD/HD-FDD |
| 12 | 6 | 15 | 201 | 201 |  |  | FDD/HD-FDD |
| 13 |  |  | 201 | 201 |  |  | FDD/HD-FDD |
| 14 |  |  | 151 | 151 |  |  | FDD/HD-FDD |
| 18 |  |  | 24 | 24 | 24 |  | FDD/HD-FDD |
| 19 |  |  | 24 | 24 | 24 |  | FDD/HD-FDD |
| 20 |  |  | 24 | 201 | 203 | 203 | FDD/HD-FDD |
| 21 |  |  | 24 | 241 | 241 |  | FDD/HD-FDD |
| 25 | 6 | 15 | 24 | 24 | 24 | 24 | FDD/HD-FDD |
| 26 | 6 | 15 | 24 | 24 | 24 |  | FDD/HD-FDD |
| 27 | 6 | 15 | 24 | 24 |  |  | FDD/HD-FDD |
| 28 |  | 15 | 24 | 24 | 24 | 24 | FDD/HD-FDD |
| 31 | 6 | 54 | 54 |  |  |  | FDD/HD-FDD |
| … |  |  |  |  |  |  |  |
| 39 |  |  |  | 24 | 24 | 24 | TDD |
| 40 |  |  |  | 24 | 24 | 24 | TDD |
| 41 |  |  |  | 24 | 24 | 24 | TDD |
| 42 |  |  |  | 24 | 24 | 24 | TDD |
| 43 |  |  |  | 24 | 24 | 24 | TDD |
| … |  |  |  |  |  |  |  |
| 71 | 6 | 15 | 24 | 24 | 24 | 24 | FDD/HD-FDD |
| 72 | 6 | 54 | 54 |  |  |  | FDD/HD-FDD |
| 73 | 6 | 54 | 54 |  |  |  | FDD/HD-FDD |
| 85 |  |  | 201 | 201 |  |  | FDD/HD-FDD |
| 87 | 6 | 54 | 54 |  |  |  | FDD/HD-FDD |
| 88 | 6 | 54 | 54 |  |  |  | FDD/HD-FDD |
| NOTE 1: 1 refers to the UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1). NOTE 2: For the UE which supports both Band 11 and Band 21 the uplink configuration for reference sensitivity is FFS. NOTE 3: 3 refers to Band 20; in the case of 15MHz channel bandwidth, the UL resource blocks shall be located at RBstart 11 and in the case of 20MHz channel bandwidth, the UL resource blocks shall be located at RBstart 16 NOTE 4: 4 refers to Bands 31, 72 and 73; in the case of 3 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 9 and in the case of 5 MHz channel bandwidth, the UL resource blocks shall be located at RBstart 10. |  |  |  |  |  |  |  |

### 7.3.1F	Minimum requirements for UE category NB1 and NB2

#### 7.3.1F.1	Reference sensitivity for UE category NB1 and NB2

The category NB1 and NB2 UE throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in Annex A.3.2  with received signal level as specified in Table 7.3.1F.1-1. Requirement in Table 7.3.1F.1-1 applies for any uplink configuration.

Table 7.3.1F.1-1: Reference sensitivity for UE category NB1 and NB2

| Operating band | REFSENS [dBm] |
| --- | --- |
| According to subclause 5.5F | - 108.2 |

#### 7.3.1F.2	Void

### 7.3.1G	Minimum requirements (QPSK) for V2X

When UE is configured for E-UTRA V2X reception non-concurrent with E-UTRA uplink transmissions for E-UTRA V2X operating bands specified in Table 5.5G-1, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.8.2 with parameters specified in Table 7.3.1G-1.

Table 7.3.1G-1: Reference sensitivity of E-UTRA V2X Bands (PC5)

| Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA V2X Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| 47 |  |  |  | -90.4 |  | -87.5 | HD |
| NOTE 1: Reference measurement channel is defined in A.8.2. NOTE 2: The signal power is specified per port. |  |  |  |  |  |  |  |

Table 7.3.1.G-1a: Sidelink TX configuration for reference sensitivity of E-UTRA V2X Bands (PC5)

| E-UTRA Band / Channel bandwidth / NRB / Duplex mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA V2X Band | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Duplex Mode |
| 47 |  |  |  | 50 |  | 98 | HD |

When UE is configured for E-UTRA V2X reception on V2X carrier con-current with E-UTRA uplink and downlink for inter-band E-UTRA V2X / E-UTRA bands specified in Table 5.5G-2 with one or multiple contiguous carriers in V2X sidelink, E-UTRA V2X sidelink throughput for each component carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.8.2 with parameters specified in Table 7.3.1G-2. Also the E-UTRAdownlink throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.3.3.2.

For the UE which supports V2X in an operating band as specified in Table 5.5G-2, and the UE also supports a E-UTRA downlink inter-band con-current configuration in Table 7.3.1G-2A, the minimum requirement for reference sensitivity in Table 7.3.1G-1 and Table 7.3.1G-2 shall be increased by the amount given in ΔRIB,c in Table 7.3.1G-2A for the corresponding E-UTRA V2X band.

Table 7.3.1G-2: Reference sensitivity for V2X Communication QPSK PREFSENS

| Inter-band V2X reception |  | Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA V2X Band | E-UTRA or V2X band | E-UTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode |
| Band 47 | Band 3 | 3 | -101.7 | -98.7 | -97 | -94 | -92.2 | -91 | FDD |
|  |  | 47 |  |  |  | -90.4 |  | -87.5 | HD |
| Band 47 | Band 5 | 5 |  |  | -98 | -95 |  |  | FDD |
|  |  | 47 |  |  |  | -90.4 |  | -87.5 | HD |
| Band 47 | Band 7 | 7 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  |  | 47 |  |  |  | -90.4 |  | -87.5 | HD |
| Band 47 | Band 8 | 8 | -102.2 | -99.2 | -97 | -94 |  |  | FDD |
|  |  | 47 |  |  |  | -90.4 |  | -87.5 | HD |
| Band 47 | Band 20 | 20 |  |  | -97 | -94 | -91.2 | -90 | FDD |
|  |  | 47 |  |  |  | -90.4 |  | -87.5 | HD |
| Band 47 | Band 28 | 28 |  | -100.2 | -98.5 | -95.5 | -93.7 | -91 | FDD |
|  |  | 47 |  |  |  | -90.4 |  | -87.5 | HD |
| Band 47 | Band 34 | 34 |  |  | -100 | -97 | -95.2 |  | TDD |
|  |  | 47 |  |  |  | -90.4 |  | -87.5 | HD |
| Band 47 | Band 39 | 39 |  |  | -100 | -97 | -95.2 | -94 | TDD |
|  |  | 47 |  |  |  | -90.4 |  | -87.5 | HD |
| Band 47 | Band 41 | 41 |  |  | -98 | -95 | -93.2 | -92 | TDD |
|  |  | 47 |  |  |  | -90.4 |  | -87.5 | HD |
| Band 47 | Band 71 | 71 |  |  | -97.2 | -94.2 | -92 | -87.5 | FDD |
|  |  | 47 |  |  |  | -90.4 |  | -87.5 | HD |

Table 7.3.1G-2A: ΔRIB,c (two bands)

| V2X inter-band con-current band Combination | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| V2X_3-47 | 3 | 0 |
| V2X_5-47 | 5 | 0.2 |
| V2X_7-47 | 7 | 0 |
| V2X_8-47 | 8 | 0 |
| V2X_20-47 | 20 | 0.2 |
| V2X_28A-47A | 28 | 0.2 |
| V2X_34-47 | 34 | 0 |
| V2X_39-47 | 39 | 0 |
| V2X_41-47 | 41 | 0 |
| V2X_71A-47A | 71 | 0 |

The reference sensitivity is defined to be met with E-UTRA uplink assigned to one band (that differs from the V2X operating band) and all E-UTRA downlink carriers active. The E-UTRA uplink resource blocks shall be located as close as possible to E-UTRA V2X operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1). The uplink configuration for the E-UTRA operating band is specified in Table 7.3.1G-3 and 7.3.1G-4. The REFSENS of Uu downlink and PC5 sidelink will be tested at the same time.

Table 7.3.1G-3: Uplink configuration for REFSENS of E-UTRA V2X Bands

| Inter-band E-UTRA V2X /E-UTRA configuration |  | E-UTRA UL band / Channel BW / NRB / Duplex mode |  |  |  |
| --- | --- | --- | --- | --- | --- |
| E-UTRA V2X band (PC5) | E-UTRA or V2X operating band (Uu) | E-UTRA or V2X UL band (Uu) | Channel Bandwidth (MHz) | NRB | Duplex Mode |
| 47 | 3 | 3 | 10 | 50 | FDD |
| 47 | 5 | 5 | 10 | 50 | FDD |
| 47 | 7 | 7 | 10 | 50 | FDD |
| 47 | 8 | 8 | 10 | 50 | FDD |
| 47 | 20 | 20 | 10 | 50 | FDD |
| 47 | 28 | 28 | 10 | 50 | FDD |
| 47 | 34 | 34 | 10 | 50 | TDD |
| 47 | 39 | 39 | 10 | 50 | TDD |
| 47 | 41 | 41 | 10 | 50 | TDD |
| 47 | 71 | 71 | 10 | 50 | FDD |

Table 7.3.1G-4: Sidelink TX configuration for REFSENS of E-UTRA V2X Bands

| Inter-band E-UTRA V2X /E-UTRA configuration |  | E-UTRA UL band / Channel BW / NRB / Duplex mode |  |  |  |
| --- | --- | --- | --- | --- | --- |
| E-UTRA V2X band (PC5) | E-UTRA or V2X operating band (Uu) | E-UTRA or V2X band (PC5) | Channel Bandwidth (MHz) | NRB | Duplex Mode |
| 47 | 3 | 47 | 10 | 50 | HD |
| 47 | 5 | 47 | 10 | 50 | HD |
| 47 | 7 | 47 | 10 | 50 | HD |
| 47 | 8 | 47 | 10 | 50 | HD |
| 47 | 20 | 47 | 10 | 50 | HD |
| 47 | 28 | 47 | 10 | 50 | HD |
| 47 | 34 | 47 | 10 | 50 | HD |
| 47 | 39 | 47 | 10 | 50 | HD |
| 47 | 41 | 47 | 10 | 50 | HD |
| 47 | 71 | 47 | 10 | 50 | HD |

For intra-band contiguous multi-carrier operation, the reference sensitivity requirement specified in Table 7.3.1G-1 shall apply for each component carrier with all carriers active. The requirement is applied for multi-carrier intra-band con-current receptions when 2 carrier transmissions are activated at the same time.

Table 7.3.1G-5: Sidelink TX configuration for REFSENS of E-UTRA V2X Bands for intra-band multi-carrier operation

| V2X configuration | 50RB+50RB |  | 100RB+50RB |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- |
|  | PCC | SCC | PCC | SCC |  |
| V2X_47B | 50 | 50 | N/A | N/A | HD |
| V2X_47C | N/A | N/A | 100 | 50 | HD |

### 7.3.2	Void

## 7.4	Maximum input level

This is defined as the maximum mean power received at the UE antenna port, at which the specified relative throughput shall meet or exceed the minimum requirements for the specified reference measurement channel.

### 7.4.1	Minimum requirements

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.4.1-1. For operating bands with an unpaired DL part (as noted in Table 5.5-1), the requirements only apply for carriers assigned in the paired part.

Table 7.4.1-1: Maximum input level

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Power in Transmission Bandwidth Configuration | dBm | -252 |  |  |  |  |  |
|  |  | -273 |  |  |  |  |  |
|  |  | -274 |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3.1-2 with PCMAX_L as defined in subclause 6.2.5. NOTE 2: Reference measurement channel is Annex A.3.2: 64QAM, R=3/4 variant with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. NOTE 3: Reference measurement channel is Annex A.3.2: 256QAM, R=4/5 variant with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. NOTE 4: Reference measurement channel is Annex A.3.2: 1024QAM, R=4/5 variant with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. |  |  |  |  |  |  |  |

### 7.4.1A	Minimum requirements for CA

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one E-UTRA band the maximum input level is defined with the uplink active on the band(s) other than the band whose downlink is being tested. For E-UTRA CA configurations including an operating band without uplink band or an operating band with an unpaired DL part, the requirements for all downlinks shall be met with the single uplink carrier active in each band capable of UL operation. The UE shall meet the requirements specified in subclause 7.4.1 for each component carrier while all downlink carriers are active.

For intra-band contiguous carrier aggregation maximum input level is defined as the powers received at the UE antenna port over the Transmission bandwidth configuration of each CC, at which the specified relative throughput shall meet or exceed the minimum requirements for the specified reference measurement channel over each component carrier.

The downlink SCC(s) shall be configured at nominal channel spacing to the PCC. For FDD the PCC shall be configured closest to the uplink band. All downlink carriers shall be active throughout the test. The uplink output power shall be set as specified in Table 7.4.1A-1 with the uplink configuration set according to Table 7.3.1A-1 for the applicable carrier aggregation configuration. For UE(s) supporting one uplink carrier, the uplink configuration of the PCC shall be in accordance with Table 7.3.1-2.

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels over each component carrier as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.4.1A-1. For operating bands with an unpaired DL part (as noted in Table 5.5-1), the requirements also apply for an SCC assigned in the unpaired part with parameters specified in Table 7.4.1A-1.

For intra-band non-contiguous carrier aggregation with one uplink carrier and two or more downlink sub-blocks, each larger than or equal to 5 MHz, the maximum input level requirements are defined with the uplink configuration in accordance with Table 7.3.1A-3. For this uplink configuration, the UE shall meet the requirements for each sub-block as specified in Table 7.4.1-1 and Table 7.4.1A-1 for one component carrier and two component carriers per sub-block, respectively. The throughput of each downlink component carrier shall be ≥ 95% of the maximum throughput of the specified reference measurement channel as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1). The requirements apply with all downlink carriers active.

Table 7.4.1A-1: Maximum input level for intra-band contiguous CA

| Rx Parameter | Units | CA Bandwidth Class |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | A | B | C | D | E | F |
| Power in largest Transmission Bandwidth Configuration CC | dBm |  | -282 | -252 | -252 | -262 | -272 |
|  |  |  | -303 | -273 | -273 | [-28]3 | [-29]3 |
|  |  |  | -304 | -274 | -274 | [-28]4 | [-29]4 |
| Power in each other CC | dBm |  | -28+ 10log(NRB,c /NRB,largest BW) 2 | -25 + 10log(NRB,c /NRB,largest BW) 2 | -25 + 10log(NRB,c /NRB,largest BW) 2 | -26 + 10log(NRB,c /NRB,largest BW) 2 | -27 + 10log(NRB,c /NRB,largest BW) 2 |
|  |  |  | -30+ 10log(NRB,c /NRB,largest BW) 3 | -27 + 10log(NRB,c /NRB,largest BW) 3 | -27 + 10log(NRB,c /NRB,largest BW) 3 | [-28] + 10log(NRB,c /NRB,largest BW) 3 | [-29] + 10log(NRB,c /NRB,largest BW) 3 |
|  |  |  | -30+ 10log(NRB,c /NRB,largest BW) 4 | -27 + 10log(NRB,c /NRB,largest BW) 4 | -27 + 10log(NRB,c /NRB,largest BW) 4 | [-28] + 10log(NRB,c /NRB,largest BW) 4 | [-29] + 10log(NRB,c /NRB,largest BW) 4 |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,c or PCMAX_L as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is Annex A.3.2: 64QAM, R=3/4 variant with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. NOTE 3: Reference measurement channel is Annex A.3.2: 256QAM, R=4/5 variant with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. NOTE 4: Reference measurement channel is Annex A.3.2: 1024QAM, R=4/5 variant with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. |  |  |  |  |  |  |  |

For combinations of intra-band and inter-band carrier aggregation and one uplink assigned to one E-UTRA band, the requirement is defined with the uplink active in a band other than that supporting the downlink(s) under test. The uplink configuration shall be in accordance with Table 7.3.1A-3 when the uplink is active in the band supporting two or more non-contiguous component carriers, Table 7.3.1A-1 when the uplink is active in a band supporting two contiguous component carriers and in accordance with Table 7.3.1-2 when the uplink is active in a band supporting one carrier per band. The downlink PCC shall be configured closer to the uplink operating band than the downlink SCC(s) when the uplink is active in band(s) supporting contiguous aggregation. For these uplink configurations, the UE shall meet the maximum input-level requirements for intra-band non-contiguous carrier aggregation of two or more downlink sub-blocks, the requirements for intra-band contiguous carrier aggregation for the contiguously aggregated downlink carriers and for any remaining component carrier(s) the the requirements specified in subclause 7.4.1. All downlink carriers shall be active throughout the tests and the requirements for the downlinks shall be met with the single uplink carrier active in each band capable of UL operation.

### 7.4.1B	Minimum requirements for UL-MIMO

For UE with two transmitter antenna connectors in closed-loop spatial multiplexing, the minimum requirements in Clause 7.4.1 shall be met with the UL-MIMO configurations specified in Table 6.2.2B-2. For UL-MIMO, the parameter PCMAX_L is defined as the total transmitter power over the two transmit antenna connectors.

### 7.4.1D	Minimum requirements for ProSe

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.6.2.

Table 7.4.1D-1: Maximum input level for ProSe

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Power in Transmission Bandwidth Configuration | dBm | -22 |  |  |  |  |  |
| NOTE 1: Reference measurement channel is Annex A.6.2 |  |  |  |  |  |  |  |

### 7.4.1F	Minimum requirements for category NB1 and NB2

Category NB1 and NB2 UE maximum input level requirement is – 25 dBm. For this input level the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in Annex A.3.2.

### 7.4.1G	Minimum requirements for V2X

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.8.2 with parameters specified in Table 7.4.1G-1.

Table 7.4.1G-1: Maximum input level

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Power in Transmission Bandwidth Configuration | dBm |  |  |  | -222 |  | -222 |
|  |  |  |  |  | -233 |  | -233 |
| NOTE 1: Reference measurement channel is defined in Annex A.8.2. NOTE 2: This requirement is applicable for 16QAM. NOTE 3: This requirement is applicable for 64QAM. |  |  |  |  |  |  |  |

When UE is configured for simultaneous E-UTRA V2X sidelink and E-UTRA downlink reception for inter-band E-UTRA V2X / E-UTRA bands specified in Table 5.5G-2, the requirements in subclause 7.4.1G apply for the E-UTRA V2X sidelink reception and the requirements in subclause 7.4.1 apply for the E-UTRA downlink reception while all downlink carriers are active.

For intra-band contiguous multi-carrier operation, maximum input level is defined as the powers received at the UE antenna port over the Transmission bandwidth configuration of each CC, at which the specified relative throughput shall meet or exceed the minimum requirements for the specified reference measurement channel over each component carrier.

Table 7.4.1G-2: Maximum input level for intra-band contiguous multi-carrier for V2X UE

| Rx Parameter | Units | V2X Bandwidth Class |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | A | B | C/C1 | D | E | F |
| Power in largest Transmission Bandwidth Configuration CC | dBm |  | -222 | -222 |  |  |  |
|  |  |  | -233 | -233 |  |  |  |
| Power in each other CC | dBm |  | -22+ 10log(NRB,c /NRB,largest BW) 2 | -222+ 10log(NRB,c /NRB,largest BW) 2 |  |  |  |
|  |  |  | -23+ 10log(NRB,c /NRB,largest BW) 3 | -23+ 10log(NRB,c /NRB,largest BW) 3 |  |  |  |
| NOTE 1: Void NOTE 2: The requirement is applied for multi-carrier intra-band con-current receptions when 2 carrier transmissions are activated at the same time. NOTE 3: This requirement is applicable for 64QAM. |  |  |  |  |  |  |  |

## 7.4A	Void

### 7.4A.1	Void

## 7.5	Adjacent Channel Selectivity (ACS)

Adjacent Channel Selectivity (ACS) is a measure of a receiver's ability to receive a E-UTRA signal at its assigned channel frequency in the presence of an adjacent channel signal at a given frequency offset from the centre frequency of the assigned channel. ACS is the ratio of the receive filter attenuation on the assigned channel frequency to the receive filter attenuation on the adjacent channel(s).

### 7.5.1	Minimum requirements

The UE shall fulfil the minimum requirement specified in Table 7.5.1-1 for all values of an adjacent channel interferer up to –25 dBm. However it is not possible to directly measure the ACS, instead the lower and upper range of test parameters are chosen in Table 7.5.1-2 and Table 7.5.1-3 where the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1). For operating bands with an unpaired DL part (as noted in Table 5.5-1), the requirements only apply for carriers assigned in the paired part.

Table 7.5.1-1: Adjacent channel selectivity

|  |  | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rx Parameter | Units | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| ACS | dB | 33.0 | 33.0 | 33.0 | 33.0 | 30 | 27 |

Table 7.5.1-2: Test parameters for Adjacent channel selectivity, Case 1

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + 14 dB |  |  |  |  |  |
| PInterferer | dBm | REFSENS +45.5dB | REFSENS +45.5dB | REFSENS +45.5dB | REFSENS +45.5dB | REFSENS +42.5dB | REFSENS +39.5dB |
| BWInterferer | MHz | 1.4 | 3 | 5 | 5 | 5 | 5 |
| FInterferer (offset) | MHz | 1.4+0.0025 / -1.4-0.0025 | 3+0.0075 / -3-0.0075 | 5+0.0025 / -5-0.0025 | 7.5+0.0075 / -7.5-0.0075 | 10+0.0125 / -10-0.0125 | 12.5+0.0025 / -12.5-0.0025 |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3.1-2 with PCMAX_L as defined in subclause 6.2.5. NOTE 2: The interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1. NOTE 3: The REFSENS power level is specified in Table 7.3.1-1 and Table 7.3.1-1a for two and four antenna ports, respectively. NOTE 4: For DL category M1 and M2 UE, the reference sensitivity for category M1 in table 7.3.1E-3 and category M2 in Table 7.3.1E-8 should be used as REFSENS for the power in Transmission Bandwidth Configuration and PInterferer. NOTE5: For DL category M1 and M2 UE, the parameters for the applicable channel bandwidth apply. |  |  |  |  |  |  |  |

Table 7.5.1-3: Test parameters for Adjacent channel selectivity, Case 2

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Power in Transmission Bandwidth Configuration | dBm | -56.5 | -56.5 | -56.5 | -56.5 | -53.5 | -50.5 |
| PInterferer | dBm | -25 |  |  |  |  |  |
| BWInterferer | MHz | 1.4 | 3 | 5 | 5 | 5 | 5 |
| FInterferer (offset) | MHz | 1.4+0.0025 / -1.4-0.0025 | 3+0.0075 / -3-0.0075 | 5+0.0025 / -5-0.0025 | 7.5+0.0075 / -7.5-0.0075 | 10+0.0125 / -10-0.0125 | 12.5+0.0025 / -12.5-0.0025 |
| NOTE 1: The transmitter shall be set to 24dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3.1-2 with PCMAX_L as defined in subclause 6.2.5. NOTE 2: The interferer consists of the Reference measurement channel specified in Annex 3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1. |  |  |  |  |  |  |  |

### 7.5.1A	Minimum requirements for CA

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one E-UTRA band, the adjacent channel requirements are defined with the uplink active on the band(s) other than the band whose downlink is being tested. The UE shall meet the requirements specified in subclause 7.5.1 for each component carrier while all downlink carriers are active. For E-UTRA CA configurations including an operating band without uplink operation or an operating band with an unpaired DL part (as noted in Table 5.5-1), the requirements for all downlinks shall be met with the single uplink carrier active in each band capable of UL operation. For a component carrier configured in Band 46 or Band 49, the requirements specified in subclause 7.5.1 are replaced by the requirements in Table 7.5.1A-0a with test parameters in Table 7.5.1A-0b and Table 7.5.1A-0c.

Table 7.5.1A-0a: Adjacent channel selectivity

| E-UTRA band | Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| 46 | ACS | dB |  |  |  | 33 |  | 27 |
| 49 | ACS | dB |  |  |  | 33 |  | 33 |

Table 7.5.1A-0b: Test parameters for Adjacent channel selectivity, Case 1

| E-UTRA Band | Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| 46 | Power in Transmission Bandwidth Configuration | dBm | REFSENS + 14 dB |  |  |  |  |  |
|  | PInterferer | dBm |  |  |  | REFSENS+45.5dB |  | REFSENS +39.5dB |
|  | BWInterferer | MHz |  |  |  | 20 |  | 20 |
|  | FInterferer (offset) | MHz |  |  |  | 15+0.0075/-15-0.0075 |  | 20+0.0025 / -20-0.0025 |
| 49 | Power in Transmission Bandwidth Configuration | dBm | REFSENS + 14 dB |  |  |  |  |  |
|  | PInterferer | dBm |  |  |  | REFSENS+45.5dB |  | REFSENS+45.5dB |
|  | BWInterferer | MHz |  |  |  | 10 |  | 20 |
|  | FInterferer (offset) | MHz |  |  |  | 10+0.0125 / -10-0.0125 |  | 20+0.0025 / -20-0.0025 |
| NOTE 1: In a band capable of uplink operation, the transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3.1-2 with PCMAX_L as defined in subclause 6.2.5. NOTE 2: The interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD/FS3 as described in Annex A.5.1.1/A.5.2.1/A.5.4.1 and set-up according to Annex C.3.1. |  |  |  |  |  |  |  |  |

Table 7.5.1A-0c: Test parameters for Adjacent channel selectivity, Case 2

| E-UTRA band | Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| 46 | Power in Transmission Bandwidth Configuration | dBm |  |  |  | -56.5 |  | -50.5 |
|  | PInterferer | dBm | -25 |  |  |  |  |  |
|  | BWInterferer | MHz |  |  |  | 20 |  | 20 |
|  | FInterferer (offset) | MHz |  |  |  | 15+0.0075/-15-0.0075 |  | 20+0.0025 / -20-0.0025 |
| 49 | Power in Transmission Bandwidth Configuration |  |  |  |  | -56.5 |  | -56.5 |
|  | PInterferer |  | -25 |  |  |  |  |  |
|  | BWInterferer |  |  |  |  | 10 |  | 20 |
|  | FInterferer (offset) |  |  |  |  | 10+0.0125 / -10-0.0125 |  | 20+0.0025 / -20-0.0025 |
| NOTE 1: In a band capable of unplink operation, the transmitter shall be set to 24dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3.1-2 with PCMAX_L as defined in subclause 6.2.5. NOTE 2: The interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD/FS3 as described in Annex A.5.1.1/A.5.2.1/A.5.4.1 and set-up according to Annex C.3.1. |  |  |  |  |  |  |  |  |

For E-UTRA CA configurations listed in Table 7.3.1A-0a under conditions for which reference sensitivity for the operating band being tested is N/A, the adjacent channel requirements of subclause 7.5.1A do not apply.

For intra-band contiguous carrier aggregation the downlink SCC(s) shall be configured at nominal channel spacing to the PCC. For FDD, the PCC shall be configured closest to the uplink band. All downlink carriers shall be active throughout the test. The uplink output power shall be set as specified in Table 7.5.1A-2 and Table 7.5.1A-3 with the uplink configuration set according to Table 7.3.1A-1 for the applicable carrier aggregation configuration. For UE(s) supporting one uplink carrier, the uplink configuration of the PCC shall be in accordance with Table 7.3.1-2. The UE shall fulfil the minimum requirement specified in Table 7.5.1A-1 for an adjacent channel interferer on either side of the aggregated downlink signal at a specified frequency offset and for an interferer power up to -25 dBm. The throughput of each carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Tables 7.5.1A-2 and 7.5.1A-3. For operating bands with an unpaired DL part (as noted in Table 5.5-1), the requirements also apply for an SCC assigned in the unpaired part with parameters specified in Tables 7.5.1A-2 and 7.5.1A-3.

For intra-band non-contiguous carrier aggregation with one uplink carrier and two or more downlink sub-blocks, each larger than or equal to 5 MHz, the adjacent channel selectivity requirements are defined with the uplink configuration in accordance with Table 7.3.1A-3. For this uplink configuration, the UE shall meet the requirements for each sub-block as specified in subclauses 7.5.1 and 7.5.1A for one component carrier and two component carriers per sub-block, respectively. The UE shall fulfil the minimum requirements all values of a single adjacent channel interferer in-gap and out-of-gap up to a –25 dBm interferer power while all downlink carriers are active. For the lower range of test parameters (Case 1), the interferer power Pinterferer shall be set to the maximum of the levels given by the carriers of the respective sub-blocks as specified in Table 7.5.1-2 and Table 7.5.1A-2 for one component carrier and two component carriers per sub-block, respectively. The wanted signal power levels for the carriers of each sub-block shall then be adjusted relative to Pinterferer in accordance with the ACS requirement for each sub-block (Table 7.5.1-1 and Table 7.5.1A-1). For the upper range of test parameters (Case 2) for which the interferer power Pinterferer is -25 dBm (Table 7.5.1-3 and Table 7.5.1A-3) the wanted signal power levels for the carriers of each sub-block shall be adjusted relative to Pinterferer like for Case 1.

Table 7.5.1A-1: Adjacent channel selectivity

|  |  | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Rx Parameter | Units | B | C | D | E | F |
| ACS | dB | 27 | 24 | 22.2 | 21 | 20 |

Table 7.5.1A-2: Test parameters for Adjacent channel selectivity, Case 1

| Rx Parameter | Units | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | B | C | D | E | F |
| Pw in Transmission Bandwidth Configuration, per CC |  | REFSENS + 14 dB | REFSENS + 14 dB | REFSENS + 14 dB | REFSENS + 14 dB | REFSENS + 14 dB |
| PInterferer | dBm | Aggregated power + 25.5 dB | Aggregated power + 22.5 dB | Aggregated power + 20.7 dB | Aggregated power + 19.5 dB | Aggregated power + 18.5 dB |
| BWInterferer | MHz | 5 | 5 | 5 | 5 | 5 |
| FInterferer (offset) | MHz | 2.5 + Foffset / -2.5 - Foffset | 2.5 + Foffset / -2.5 - Foffset | 2.5 + Foffset / -2.5 - Foffset | 2.5 + Foffset / -2.5 - Foffset | 2.5 + Foffset / -2.5 - Foffset |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,c or PCMAX_L as defined in subclause 6.2.5A. NOTE 2: The interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1 NOTE 3: The Finterferer (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the adjacent channel interferer and shall be further adjusted to MHz to be offset from the sub-carrier raster. |  |  |  |  |  |  |

Table 7.5.1A-3: Test parameters for Adjacent channel selectivity, Case 2

| Rx Parameter | Units | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | B | C | D | E | F |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | -50.5 +10log10(NRB,c/ NRB agg) | -47.5 +10log10(NRB,c/NRB agg) | -45.7+10log10(NRB,c/NRB agg) | -44.5 +10log10(NRB,c/NRB agg) | -43.5 +10log10(NRB,c/NRB agg) |
| PInterferer | dBm | -25 |  |  |  |  |
| BWInterferer | MHz | 5 | 5 | 5 | 5 | 5 |
| FInterferer (offset) | MHz | 2.5+ Foffset / -2.5- Foffset | 2.5+ Foffset / -2.5- Foffset | 2.5+ Foffset / -2.5- Foffset | 2.5+ Foffset / -2.5- Foffset | 2.5+ Foffset / -2.5- Foffset |
| NOTE 1: The transmitter shall be set to 24dB below PCMAX_L,c or PCMAX_L as defined in subclause 6.2.5A. NOTE 2: The interferer consists of the Reference measurement channel specified in Annex 3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1 NOTE 3: The Finterferer (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the adjacent channel interferer and shall be further adjusted to MHz to be offset from the sub-carrier raster. |  |  |  |  |  |  |

For combinations of intra-band and inter-band carrier aggregation and one uplink carrier assigned to one E-UTRA band, the requirement is defined with the uplink active in each band other than that supporting the downlink(s) under test. The uplink configuration shall be in accordance with Table 7.3.1A-3 when the uplink is active in the band supporting two or more non-contiguous component carriers, Table 7.3.1A-1 when the uplink is active in a band supporting two contiguous component carriers and in accordance with Table 7.3.1-2 when the uplink is active in a band supporting one carrier per band. The downlink PCC shall be configured closer to the uplink operating band than the downlink SCC(s) when the uplink is active in band(s) supporting contiguous aggregation. For these uplink configurations, the UE shall meet the adjacent channel selectivity requirements for intra-band non-contiguous carrier aggregation with RIBNC = 0 dB for all sub-block gaps (Table 7.3.1A-3) for the two or more non-contiguous downlink sub-blocks, the requirements for intra-band contiguous carrier aggregation for the contiguously aggregated downlink carriers and for any remaining component carrier(s) the requirements specified in subclause 7.5.1. For contiguously aggregated component carriers configured in Band 46, the said requirements for intra-band contiguous carrier aggregation of downlink carriers are replaced by requirements in Table 7.5.1A-4 with test parameters in Table 7.5.1A-5 and Table 7.5.1A-6. For non-contiguously aggregated component carriers configured in Band 46, the said requirements are applied to each sub-block for in-gap and out-of-gap interferers. For the sub-block with a single component carrier, the requirement is replaced by Table 7.5.1A-0a with test parameters in Table 7.5.1A-0b and Table 7.5.1A-0c. For the sub-block with two or more contiguous component carriers, the requirement is replaced by Table 7.5.1A-4 with test parameters in Table 7.5.1A-5 and Table 7.5.1A-6. All downlink carriers shall be active throughout the tests and the requirements for downlinks shall be met with the single uplink carrier active in each band capable of UL operation.

Table 7.5.1A-4: Adjacent channel selectivity

| E-UTRA band | Rx Parameter | Units | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | B | C | D | E | F |
| 46 | ACS | dB |  | 24 | 22.2 | 21 |  |

Table 7.5.1A-5: Test parameters for Adjacent channel selectivity, Case 1

| E-UTRA Band | Rx Parameter | Units | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | B | C | D | E | F |
| 46 | Pw in Transmission Bandwidth Configuration, per CC |  |  | REFSENS + 14 dB | REFSENS + 14 dB | REFSENS + 14 dB |  |
|  | PInterferer | dBm |  | Aggregated power + 22.5 dB | Aggregated power + 20.7 dB | Aggregated power + 19.5 dB |  |
|  | BWInterferer | MHz |  | 20 | 20 | 20 |  |
|  | FInterferer (offset) | MHz |  | 10 + Foffset / -10 - Foffset | 10 + Foffset / -10 - Foffset | 10 + Foffset / -10 - Foffset |  |
| NOTE 1*: In a band capable of uplink operation, the transmitter shall be set to 4dB below PCMAX_L,c or PCMAX_L as defined in subclause 6.2.5A. NOTE 2: The interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD/FS3 as described in Annex A.5.1.1/A.5.2.1/A.5.4.1 and set-up according to Annex C.3.1. NOTE 3: The Finterferer (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the adjacent channel interferer and shall be further adjusted to MHz to be offset from the sub-carrier raster. |  |  |  |  |  |  |  |

Table 7.5.1A-6: Test parameters for Adjacent channel selectivity, Case 2

| E-UTRA band | Rx Parameter | Units | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | B | C | D | E | F |
| 46 | Pw in Transmission Bandwidth Configuration, per CC | dBm |  | -47.5 +10log10(NRB,c/NRB agg) | -45.7 +10log10(NRB,c/NRB agg) | -44.5 +10log10(NRB,c/NRB agg) |  |
|  | PInterferer | dBm | -25 |  |  |  |  |
|  | BWInterferer | MHz |  | 20 | 20 | 20 |  |
|  | FInterferer (offset) | MHz |  | 10 + Foffset / -10 - Foffset | 10 + Foffset / -10 - Foffset | 10 + Foffset / -10 - Foffset |  |
| NOTE 1: In a band capable of uplink operation, the transmitter shall be set to 24dB below PCMAX_L,c or PCMAX_L as defined in subclause 6.2.5A. NOTE 2: The interferer consists of the Reference measurement channel specified in Annex 3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD/FS3 as described in Annex A.5.1.1/A.5.2.1/A.5.4.1 and set-up according to Annex C.3.1. NOTE 3: The Finterferer (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the adjacent channel interferer and shall be further adjusted to MHz to be offset from the sub-carrier raster. |  |  |  |  |  |  |  |

### 7.5.1B	Minimum requirements for UL-MIMO

For UE(s) with two transmitter antenna connectors in closed-loop spatial multiplexing scheme, the minimum requirements in Clause 7.5.1 shall be met with the UL-MIMO configurations specified in Table 6.2.2B-2. For UL-MIMO, the parameter PCMAX_L is defined as the total transmitter power over the two transmit antenna connectors.

### 7.5.1D	Minimum requirements for ProSe

The UE shall fulfil the minimum requirement specified in Table 7.5.1D-1 for all values of an adjacent channel interferer up to –25 dBm. However it is not possible to directly measure the ACS, instead the lower and upper range of test parameters are chosen in Table 7.5.1D-2 and Table 7.5.1D-3 where the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.6.2.

Table 7.5.1D-1: Adjacent channel selectivity for ProSe

|  |  | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rx Parameter | Units | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| ACS | dB |  |  | 33.0 | 33.0 | 30 | 27 |

Table 7.5.1D-2: Test parameters for Adjacent channel selectivity for ProSe, Case 1

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Power in Transmission Bandwidth Configuration | dBm | PREFSENS_ProSe + 14 dB |  |  |  |  |  |
| PInterferer | dBm |  |  | REFSENS +45.5dB | REFSENS +45.5dB | REFSENS +42.5dB | REFSENS +39.5dB |
| BWInterferer | MHz |  |  | 5 | 5 | 5 | 5 |
| FInterferer (offset) | MHz |  |  | 5+0.0025 / -5-0.0025 | 7.5+0.0075 / -7.5-0.0075 | 10+0.0125 / -10-0.0125 | 12.5+0.0025 / -12.5-0.0025 |
| NOTE 1: The interferer is QPSK modulated PUSCH containing data and reference symbols. Normal cyclic prefix is used. The data content shall be uncorrelated to the wanted signal and modulated according to clause 5 of TS36.211. |  |  |  |  |  |  |  |

Table 7.5.1D-3: Test parameters for Adjacent channel selectivity for ProSe, Case 2

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Power in Transmission Bandwidth Configuration | dBm |  |  | -56.5 | -56.5 | -53.5 | -50.5 |
| PInterferer | dBm | -25 |  |  |  |  |  |
| BWInterferer | MHz |  |  | 5 | 5 | 5 | 5 |
| FInterferer (offset) | MHz |  |  | 5+0.0025 / -5-0.0025 | 7.5+0.0075 / -7.5-0.0075 | 10+0.0125 / -10-0.0125 | 12.5+0.0025 / -12.5-0.0025 |
| NOTE 1: The interferer is QPSK modulated PUSCH containing data and reference symbols. Normal cyclic prefix is used. The data content shall be uncorrelated to the wanted signal and modulated according to clause 5 of TS36.211. |  |  |  |  |  |  |  |

### 7.5.1F	Minimum requirements for category NB1 and NB2

Category NB1 and NB2 UE shall fulfil the minimum requirement specified in Table 7.5.1F-1 for all values of an adjacent channel interferer up to –25 dBm. However it is not possible to directly measure the ACS, instead the lower and upper range of test parameters are chosen in Table 7.5.1F-1 where the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in Annex A.3.2.

Table 7.5.1F: Adjacent channel selectivity parameters for category NB1 and NB2

| ACS1 test Parameters |  |  |
| --- | --- | --- |
| Interferer | GSM (GMSK) | E-UTRA |
| Category NB1 or NB2 signal power (Pwanted ) / dBm | REFSENS + 14 dB |  |
| Interferer signal power (PInterferer ) / dBm | REFSENS + 42 dB | REFSENS + 47 dB |
| Interferer bandwidth | 200 kHz | 5 MHz |
| Interferer offset from category NB1 or NB2 channel edge | ±200 kHz | ±2.5 MHz |
| ACS2 test Parameters |  |  |
| Interferer | GSM (GMSK) | E-UTRA |
| Category NB1 or NB2 signal power (Pwanted ) / dBm | -53 dBm | -58 dBm |
| Interferer signal power (PInterferer ) / dBm | -25 dBm |  |
| Interferer bandwidth | 200 kHz | 5 MHz |
| Interferer offset from category NB1 or NB2 channel edge | ±200 kHz | ±2.5 MHz |

### 7.5.1G	Minimum requirements for V2X

The V2X UE shall fulfil the minimum requirement specified in Table 7.5.1G-1 for all values of an adjacent channel interferer up to -22 dBm. However it is not possible to directly measure the ACS, instead the lower and upper range of test parameters are chosen in Table 7.5.1G-2 and Table 7.5.1G-3 where the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.8.2.

Table 7.5.1G-1: Adjacent channel selectivity for V2X

|  |  | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rx Parameter | Units | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| ACS | dB |  |  |  | 33.0 |  | 27 |

Table 7.5.1G-2: Test parameters for Adjacent channel selectivity for V2X, Case 1

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Power in Transmission Bandwidth Configuration | dBm | PREFSENS_V2X + 14 dB |  |  |  |  |  |
| PInterferer | dBm |  |  |  | PREFSENS_V2X +45.5dB |  | PREFSENS_V2X +39.5dB |
| BWInterferer | MHz |  |  |  | 10 |  | 10 |
| FInterferer (offset) | MHz |  |  |  | 10+0.0125 / -10-0.0125 |  | 15+0.0075 / -15-0.0075 |
| NOTE 1: The interferer is QPSK modulated PUSCH containing data and reference symbols. Normal cyclic prefix is used. The data content shall be uncorrelated to the wanted signal and modulated according to clause 5 of TS36.211. |  |  |  |  |  |  |  |

Table 7.5.1G-3: Test parameters for Adjacent channel selectivity for V2X, Case 2

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Power in Transmission Bandwidth Configuration | dBm |  |  |  | -53.5 |  | -47.5 |
| PInterferer | dBm | -22 |  |  |  |  |  |
| BWInterferer | MHz |  |  |  | 10 |  | 10 |
| FInterferer (offset) | MHz |  |  |  | 10+0.0125 / -10-0.0125 |  | 15+0.0075 / -15-0.0075 |
| NOTE 1: The interferer is QPSK modulated PUSCH containing data and reference symbols. Normal cyclic prefix is used. The data content shall be uncorrelated to the wanted signal and modulated according to clause 5 of TS36.211. |  |  |  |  |  |  |  |

When UE is configured for simultaneous E-UTRA V2X sidelink and E-UTRA downlink reception for inter-band E-UTRA V2X / E-UTRA bands specified in Table 5.5G-2, the requirements in subclause 7.5.1G apply for the E-UTRA V2X sidelink reception and the requirements in subclause 7.5.1 apply for the E-UTRA downlink reception while all downlink carriers are active.

For intra-band contiguous multi-carrier operation, the V2X UE shall fulfil the minimum requirement specified in Table 7.5.1G-4 to Table 7.5.1G-6 where the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.8.2.

Table 7.5.1G-4: Adjacent channel selectivity for intra-band contiguous multi-carrier for V2X UE

|  |  | V2X Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Rx Parameter | Units | B | C/ C1 | D | E | F |
| ACS | dB | 30 | 24 |  |  |  |

Table 7.5.1G-5: Test parameters for Adjacent channel selectivity, Case 1

| Rx Parameter | Units | V2X Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | B | C/ C1 | D | E | F |
| Pw in Transmission Bandwidth Configuration, per CC |  | REFSENS + 14 dB | REFSENS + 14 dB |  |  |  |
| PInterferer | dBm | Aggregated power + 28.5 dB | Aggregated power + 22.5 dB |  |  |  |
| BWInterferer | MHz | 10 | 10 |  |  |  |
| FInterferer (offset) | MHz | 5 + Foffset / -5 - Foffset | 5 + Foffset / -5 - Foffset |  |  |  |
| NOTE 1: The Finterferer (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the adjacent channel interferer and shall be further adjusted to MHz to be offset from the sub-carrier raster. NOTE 2: The requirement is applied for multi-carrier intra-band con-current receptions when 2 carrier transmissions are activated at the same time. |  |  |  |  |  |  |

Table 7.5.1G-6: Test parameters for Adjacent channel selectivity, Case 2

| Rx Parameter | Units | V2X Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | B | C/ C1 | D | E | F |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | -50.5 +10log10(NRB,c/ NRB agg) | -47.5 +10log10(NRB,c/ NRB agg) |  |  |  |
| PInterferer | dBm | -22 |  |  |  |  |
| BWInterferer | MHz | 10 | 10 |  |  |  |
| FInterferer (offset) | MHz | 5+ Foffset / -5- Foffset | 5+ Foffset / -5- Foffset |  |  |  |
| NOTE 1: The Finterferer (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the adjacent channel interferer and shall be further adjusted to MHz to be offset from the sub-carrier raster. NOTE 2: The requirement is applied for multi-carrier intra-band con-current receptions when 2 carrier transmissions are activated at the same time. |  |  |  |  |  |  |

## 7.6	Blocking characteristics

The blocking characteristic is a measure of the receiver's ability to receive a wanted signal at its assigned channel frequency in the presence of an unwanted interferer on frequencies other than those of the spurious response or the adjacent channels, without this unwanted input signal causing a degradation of the performance of the receiver beyond a specified limit. The blocking performance shall apply at all frequencies except those at which a spurious response occur.

### 7.6.1	In-band blocking

In-band blocking is defined for an unwanted interfering signal falling into the UE receive band or into the first 15 MHz below or above the UE receive band at which the relative throughput shall meet or exceed the minimum requirement for the specified measurement channels.

For CA configurations including Band 46, in-band blocking in Band 46 is defined for a 20 MHz unwanted interfering signal falling into the UE receive band or into the first 60 MHz below or above the UE receive band (Table 7.6.1.1A-0a and Table 7.6.1.1A-0b).

For CA configurations including Band 49, in-band blocking in Band 49 is defined for an unwanted interfering signal falling into the UE receive band or into the first 60 MHz below or above the UE receive band (Table 7.6.1.1A-0a and Table 7.6.1.1A-0b).

#### 7.6.1.1	Minimum requirements

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Tables 7.6.1.1-1 and 7.6.1.1-2. For operating bands with an unpaired DL part (as noted in Table 5.5-1), the requirements only apply for carriers assigned in the paired part.

Table 7.6.1.1-1: In band blocking parameters

| Rx parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |  |
|  |  | 6 | 6 | 6 | 6 | 7 | 9 |
| BWInterferer | MHz | 1.4 | 3 | 5 | 5 | 5 | 5 |
| FIoffset, case 1 | MHz | 2.1+0.0125 | 4.5+0.0075 | 7.5+0.0125 | 7.5+0.0025 | 7.5+0.0075 | 7.5+0.0125 |
| FIoffset, case 2 | MHz | 3.5+0.0075 | 7.5+0.0075 | 12.5+0.0075 | 12.5+0.0125 | 12.5+0.0025 | 12.5+0.0075 |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3.1-2 with PCMAX_L as defined in subclause 6.2.5. NOTE 2: The interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1. NOTE 3: The REFSENS power level is specified in Table 7.3.1-1 and Table 7.3.1-1a for two and four antenna ports, respectively. NOTE 4: For DL category M1 and M2 UE, the reference sensitivity for category M1 in table 7.3.1E-3 and category M2 in Table 7.3.1E-8 should be used as REFSENS for the power in Transmission Bandwidth Configuration. NOTE5: For DL category M1 and M2 UE, the parameters for the applicable channel bandwidth apply. |  |  |  |  |  |  |  |

Table 7.6.1.1-2: In-band blocking

| E-UTRA band | Parameter | Unit | Case 1 | Case 2 | Case 3 | Case 4 | Case 5 | Case 6 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -56 | -44 | Void | Void | -38 | -15 |
|  | FInterferer (offset) | MHz | =-BW/2 – FIoffset,case 1 & =+BW/2 + FIoffset,case 1 | ≤-BW/2 – FIoffset,case 2 & ≥+BW/2 + FIoffset,case 2 |  |  | -BW/2 - 11 |  |
| 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 17, 18, 19, 20, 21, 22, 23, 25, 26, 27, 28, 31, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42, 43, 44, 45, 48, 50, 51, 52, 53, 65, 66, 68, 70, 72, 73, 74, 85, 87, 88 | FInterferer | MHz | (NOTE 2) | FDL_low – 15 to FDL_high + 15 |  |  |  |  |
| 30 | FInterferer | MHz | (NOTE 2) | FDL_low – 15 to FDL_high + 15 |  |  | FDL_low – 11 |  |
| 71 | FInterferer | MHz | (NOTE 2) | FDL_low – 12 to FDL_high + 15 |  |  |  | FDL-low - 12 |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band NOTE 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -BW/2 - FIoffset, case 1 and b. the carrier frequency +BW/2 + FIoffset, case 1 NOTE 3: FInterferer range values for unwanted modulated interfering signal are interferer center frequencies |  |  |  |  |  |  |  |  |

For the UE which supports inter band CA configuration in Table 7.3.1-1A, PInterferer power defined in Table 7.6.1.1-2 is increased by the amount given by ΔRIB,c in Table 7.3.1-1A.

#### 7.6.1.1A	Minimum requirements for CA

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one E-UTRA band the in-band blocking requirements are defined with the uplink active on the band(s) other than the band whose downlink is being tested. For adjacent downlink bands separated by less than 30 MHz the frequency separation between the center frequencies of adjacent component carriers belonging to different bands shall be ≥ BW1/2 + BW2/2 + 2FIoffset,case j for Case j interferers, j = 1,2, where BWk/2 are the channel bandwidths of carrier k, k = 1,2. The UE shall meet the requirements specified in subclause 7.6.1.1 for each component carrier while all downlink carriers are active. For the UE which supports inter band CA configuration in Table 7.3.1-1A, PInterferer power defined in Table 7.6.1.1-2 is increased by the amount given by ΔRIB,c in Table 7.3.1-1A. For E-UTRA CA configurations including an operating band without uplink operation or an operating band with an unpaired DL part (as noted in Table 5.5-1), the requirements for all downlinks shall be met with the single uplink carrier active in each band capable of UL operation. The requirements for the component carrier configured in the operating band without uplink operation are specified in Table 7.6.1.1A-0, Table 7.6.1.1A-0a and Table 7.6.1.1A-0b. The requirements for a component carrier configured in Band 49 are specified in Table 7.6.1.1A-0a and Table 7.6.1.1A-0b.

Table 7.6.1.1A-0: In-band blocking for additional operating bands for carrier aggregation

| E-UTRA band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -56 | -44 |
|  | FInterferer (offset) | MHz | =-BW/2 – FIoffset,case 1 & =+BW/2 + FIoffset,case 1 | ≤-BW/2 – FIoffset,case 2 & ≥+BW/2 + FIoffset,case 2 |
| 29, 32, 67, 69, 75, 76 | FInterferer | MHz | (NOTE 2) | FDL_low – 15 to FDL_high + 15 |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band NOTE 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -BW/2 - FIoffset, case 1 and b. the carrier frequency +BW/2 + FIoffset, case 1 NOTE 3: FInterferer range values for unwanted modulated interfering signal are interferer center frequencies |  |  |  |  |

Table 7.6.1.1A-0a: In band blocking parameters for additional operating bands for carrier aggregation

| E-UTRA band | Rx parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| 46 (NOTE 3) | Power in Transmission Bandwidth Configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |  |
|  |  |  |  |  |  | 6 |  | 9 |
|  | BWInterferer | MHz |  |  |  | 20 |  | 20 |
|  | FIoffset, case 1 | MHz |  |  |  | 30+0.0025 |  | 30+0.0125 |
|  | FIoffset, case 2 | MHz |  |  |  | 50+0.0125 |  | 50+0.0075 |
| 49 (NOTE 3) | Power in Transmission Bandwidth Configuration | dBm |  |  |  | 6 |  | 6 |
|  | BWInterferer | MHz |  |  |  | 10 |  | 20 |
|  | FIoffset, case 1 | MHz |  |  |  | 15+0.025 |  | 30+0.125 |
|  | FIoffset, case 2 | MHz |  |  |  | 25+0.075 |  | 50+0.075 |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3.1-2 with PCMAX_L as defined in subclause 6.2.5. NOTE 2: The interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD/FS3 as described in Annex A.5.1.1/A.5.2.1/A.5.4.1 and set-up according to Annex C.3.1 NOTE 3: The interferer consists of the Reference measurement channel specified in Annex A.3.2 (TBD) |  |  |  |  |  |  |  |  |

Table 7.6.1.1A-0b: In-band blocking for additional operating bands for carrier aggregation

| E-UTRA band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -50 | -44 |
|  | FInterferer (offset) | MHz | =-BW/2 – FIoffset,case 1 & =+BW/2 + FIoffset,case 1 | ≤-BW/2 – FIoffset,case 2 & ≥+BW/2 + FIoffset,case 2 |
| 46, 49 | FInterferer | MHz | (Note 2) | FDL_low – 60 to FDL_high + 60 |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz or 60 MHz below or above the UE receive band NOTE 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -BW/2 - FIoffset, case 1 and b. the carrier frequency +BW/2 + FIoffset, case 1 NOTE 3: FInterferer range values for unwanted modulated interfering signal are interferer center frequencies |  |  |  |  |

For E-UTRA CA configurations listed in Table 7.3.1A-0a under conditions for which reference sensitivity for the operating band being tested is N/A, the in-band blocking requirements of subclause 7.6.1.1A do not apply.

For intra-band contiguous carrier aggregation the downlink SCC(s) shall be configured at nominal channel spacing to the PCC. For FDD, the PCC shall be configured closest to the uplink band. All downlink carriers shall be active throughout the test. The uplink output power shall be set as specified in Table 7.6.1.1A-1 with the uplink configuration set according to Table 7.3.1A-1 for the applicable carrier aggregation configuration. For UE(s) supporting one uplink carrier, the uplink configuration of the PCC shall be in accordance with Table 7.3.1-2. The UE shall fulfil the minimum requirement in presence of an interfering signal specified in Tables 7.6.1.1A-1 and Tables 7.6.1.1A-2 being on either side of the aggregated signal. The throughput of each carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Tables 7.6.1.1A-1 and 7.6.1.1A-2. For operating bands with an unpaired DL part (as noted in Table 5.5-1), the requirements also apply for an SCC assigned in the unpaired part with parameters specified in Tables 7.6.2.1A-1 and 7.6.2.1A-2.

For intra-band non-contiguous carrier aggregation with one uplink carrier and two or more downlink sub-blocks, each larger than or equal to 5 MHz, the in-band blocking requirements are defined with the uplink configuration in accordance with Table 7.3.1A-3. For this uplink configuration, the UE shall meet the requirements for each sub-block as specified in subclause 7.6.1.1 and in this subclause for one component carrier and two component carriers per sub-block, respectively. The requirements apply for in-gap and out-of-gap interferers while all downlink carriers are active.

Table 7.6.1.1A-1: In band blocking parameters

| Rx Parameter | Units | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | B | C | D | E | F |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + CA Bandwidth Class specific value below |  |  |  |  |
|  |  | 9 | 12 | 13.8 | 15 | 16 |
| BWInterferer | MHz | 5 | 5 | 5 | 5 | 5 |
| FIoffset, case 1 | MHz | 7.5 | 7.5 | 7.5 | 7.5 | 7.5 |
| FIoffset, case 2 | MHz | 12.5 | 12.5 | 12.5 | 12.5 | 12.5 |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,c or PCMAX_L as defined in subclause 6.2.5A NOTE 2: The interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 and set-up according to Annex C.3.1 |  |  |  |  |  |  |

Table 7.6.1.1A-2: In-band blocking

| CA configuration | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -56 | -44 |
|  | FInterferer (offset) | MHz | =-Foffset– FIoffset,case 1 & =+Foffset + FIoffset,case 1 | ≤-Foffset– FIoffset,case 2 & ≥+Foffset + FIoffset,case 2 |
| CA_1C, CA_2C, CA_3B, CA_3C, CA_5B, CA_7B, CA_7C, CA_8B, CA_12B, CA_23B, CA_27B, CA_28C, CA_38C, CA_39C, CA_40C, CA_40D, CA_40E, CA_40F, CA_41C, CA_41D, CA_41E, CA_41F, CA_42C, CA_42D, CA_42E, CA_42F, CA_43C, CA_48B, CA_48C, CA_48D, CA_48E, CA_48F, CA_66B, CA_66C, CA_66D, CA_70C | FInterferer (Range) | MHz | (NOTE 2) | FDL_low – 15 to FDL_high + 15 |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band NOTE 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -Foffset - FIoffset, case 1 and b. the carrier frequency +Foffset + FIoffset, case 1 NOTE 3: Foffset is the frequency offset from the center frequency of the CC being tested to the edge of aggregated channel bandwidth. NOTE 4: The Finterferer (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the interferer and shall be further adjusted to MHz to be offset from the sub-carrier raster. |  |  |  |  |

For combinations of intra-band and inter-band carrier aggregation and one uplink carrier assigned to one E-UTRA band, the requirement is defined with the uplink active in the band other than that supporting the downlink(s) under test. The uplink configuration shall be in accordance with Table 7.3.1A-3 when the uplink is active in the band supporting two or more non-contiguous component carriers, Table 7.3.1A-1 when the uplink is active in a band supporting two contiguous component carriers and in accordance with Table 7.3.1-2 when the uplink is active in a band supporting one carrier per band. The downlink PCC shall be configured closer to the uplink operating band than the downlink SCC(s) when the uplink is active in band(s) supporting contiguous aggregation. For these uplink configurations, the UE shall meet the in-band blocking requirements for intra-band non-contiguous carrier aggregation with RIBNC = 0 dB for all sub-block gaps (Table 7.3.1A-3) for the two or more non-contiguous downlink sub-blocks, the requirements for intra-band contiguous carrier aggregation for the contiguously aggregated downlink carriers and for any remaining component carrier(s) the requirements specified in subclause 7.6.1. For contiguously aggregated component carriers configured in Band 46, the said requirements for intra-band contiguous carrier aggregation of downlink carriers are replaced by requirements in Table 7.6.1.1A-3 and 7.6.1.1A-4. For non-contiguously aggregated component carriers configured in Band 46, the said requirements are applied to each sub-block for in-gap and out-of-gap interferers. For the sub-block with a single component carrier, the requirement is replaced by Table 7.6.1.1A-0a and 7.6.1.1A-0b. For the sub-block with two or more contiguous component carriers, the requirement is replaced by Table 7.6.1.1A-3 and 7.6.1.1A-4. All downlink carriers shall be active throughout the tests and the requirements for the downlinks shall be met with the single uplink carrier active in each band capable of uplink operation.

Table 7.6.1.1A-3: In band blocking parameters

| E-UTRA Band | Rx Parameter | Units | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | B | C | D | E | F |
| 46 | Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + CA Bandwidth Class specific value below |  |  |  |  |
|  |  |  |  | 12 | 13.8 | 15 |  |
|  | BWInterferer | MHz |  | 20 | 20 | 20 |  |
|  | FIoffset, case 1 | MHz |  | 30 | 30 | 30 |  |
|  | FIoffset, case 2 | MHz |  | 50 | 50 | 50 |  |
| NOTE 1: In a band capable of uplink operation, the transmitter shall be set to 4dB below PCMAX_L,c or PCMAX_L as defined in subclause 6.2.5A NOTE 2: The interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD/FS3 as described in Annex A.5.1.1/A.5.2.1/A.5.4.1 and set-up according to Annex C.3.1 |  |  |  |  |  |  |  |

Table 7.6.1.1A-4: In-band blocking

| E-UTRA Band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -50 | -44 |
|  | FInterferer (offset) | MHz | =-Foffset– FIoffset,case 1 & =+Foffset + FIoffset,case 1 | ≤-Foffset– FIoffset,case 2 & ≥+Foffset + FIoffset,case 2 |
| 46 | FInterferer (Range) | MHz | (Note 2) | FDL_low – 60 to FDL_high + 60 |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band NOTE 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -Foffset - FIoffset, case 1 and b. the carrier frequency +Foffset + FIoffset, case 1 NOTE 3: Foffset is the frequency offset from the center frequency of the CC being tested to the edge of aggregated channel bandwidth. NOTE 4: The Finterferer (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the interferer and shall be further adjusted to MHz to be offset from the sub-carrier raster. |  |  |  |  |

#### 7.6.1.1D	Minimum requirements for ProSe

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.6.2.

Table 7.6.1.1D-1: In band blocking parameters for ProSe Direct Discovery

| Rx parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Power in Transmission Bandwidth Configuration | dBm | PREFSENS_ProSe + channel bandwidth specific value below + Poffset |  |  |  |  |  |
|  |  |  |  | 6 | 6 | 7 | 9 |
| BWInterferer | MHz |  |  | 5 | 5 | 5 | 5 |
| FIoffset, case 1 | MHz |  |  | 7.5+0.0125 | 7.5+0.0025 | 7.5+0.0075 | 7.5+0.0125 |
| FIoffset, case 2 | MHz |  |  | 12.5+0.0075 | 12.5+0.0125 | 12.5+0.0025 | 12.5+0.0075 |
| Poffset | dB |  |  | 10.9 | 13.9 | 15.7 | 16.9 |
| NOTE 1: The interferer is QPSK modulated PUSCH containing data and reference symbols. Normal cyclic prefix is used. The data content shall be uncorrelated to the wanted signal and modulated according to clause 5 of TS36.211 |  |  |  |  |  |  |  |

Table 7.6.1.1D-2: In band blocking parameters for ProSe Direct Communication

| Rx parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Power in Transmission Bandwidth Configuration | dBm | PREFSENS_ProSe + channel bandwidth specific value below |  |  |  |  |  |
|  |  |  |  | 6 | 6 | 7 | 9 |
| BWInterferer | MHz |  |  | 5 | 5 | 5 | 5 |
| FIoffset, case 1 | MHz |  |  | 7.5+0.0125 | 7.5+0.0025 | 7.5+0.0075 | 7.5+0.0125 |
| FIoffset, case 2 | MHz |  |  | 12.5+0.0075 | 12.5+0.0125 | 12.5+0.0025 | 12.5+0.0075 |
| NOTE 1: The interferer is QPSK modulated PUSCH containing data and reference symbols. Normal cyclic prefix is used. The data content shall be uncorrelated to the wanted signal and modulated according to clause 5 of TS36.211 |  |  |  |  |  |  |  |

Table 7.6.1.1D-3: In-band blocking for ProSe

| E-UTRA ProSe band | Parameter | Unit |  | Case 1 | Case 2 |
| --- | --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -56 |  | -44 |
|  | FInterferer (offset) | MHz | =-BW/2 – FIoffset,case 1 & =+BW/2 + FIoffset,case 1 |  | ≤-BW/2 – FIoffset,case 2 & ≥+BW/2 + FIoffset,case 2 |
| 2,3,4,7,14, 20,26,28,31 | FInterferer | MHz |  | (NOTE 2) | FDL_low – 15 to FDL_high + 15 |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band NOTE 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -BW/2 - FIoffset, case 1 and b. the carrier frequency +BW/2 + FIoffset, case 1 NOTE 3: FInterferer range values for unwanted modulated interfering signal are interferer center frequencies |  |  |  |  |  |

For the UE which supports inter band CA configuration in Table 7.3.1-1A, PInterferer power defined in Table 7.6.1.1D-3 is increased by the amount given by ΔRIB,c in Table 7.3.1-1A.

#### 7.6.1.1F	Minimum requirements for category NB1 and NB2

Category NB1 and NB2 UE throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in Annex A.3.2 with parameters specified in Table 7.6.1.1F-1.

Table 7.6.1.1F-1: In-band blocking parameters for category NB1 and NB2

| IBB1 test Parameters |  |
| --- | --- |
| Category NB1 or NB2 signal power (Pwanted ) / dBm | REFSENS + 6 dB |
| Interferer | E-UTRA |
| Interferer signal power (PInterferer ) / dBm | - 56 dBm |
| Interferer bandwidth | 5 MHz |
| Interferer offset from category NB1 or NB2 channel edge | +7.5 MHz + 0.005 MHz and -7.5 MHz - 0.005 MHz |
| IBB2 test Parameters |  |
| Category NB1 or NB2 signal power (Pwanted ) / dBm | REFSENS + 6 dB |
| Interferer | E-UTRA |
| Interferer signal power (PInterferer ) / dBm | - 44 dBm |
| Interferer bandwidth | 5 MHz |
| Interferer offset range from category NB1 or NB2 channel edge | From +12.5 MHz to FDL_high + 15 MHz and From -12.5 MHz to FDL_low - 15 MHz |

#### 7.6.1.1G	Minimum requirements for V2X

The V2X UE throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.8.2 with paramteters defined in Table 7.6.1.1G-1 and Table 7.6.1.1G-2.

Table 7.6.1.1G-1: In band blocking parameters

| Rx parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Power in Transmission Bandwidth Configuration | dBm | PREFSENS_V2X + channel bandwidth specific value below |  |  |  |  |  |
|  |  |  |  |  | 6 |  | 9 |
| BWInterferer | MHz |  |  |  | 10 |  | 10 |
| FIoffset, case 1 | MHz |  |  |  | 15+0.0025 |  | 15+0.005 |
| FIoffset, case 2 | MHz |  |  |  | 25+0.0075 |  | 25+0.0025 |
| NOTE 1: The interferer is QPSK modulated PUSCH containing data and reference symbols. Normal cyclic prefix is used. The data content shall be uncorrelated to the wanted signal and modulated according to clause 5 of TS36.211 |  |  |  |  |  |  |  |

Table 7.6.1.1G-2: In-band blocking

| E-UTRA V2X band | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -44 | -44 |
|  | FInterferer (offset) | MHz | =-BW/2 – FIoffset,case 1 & =+BW/2 + FIoffset,case 1 | ≤-BW/2 – FIoffset,case 2 & ≥+BW/2 + FIoffset,case 2 |
| 47 | FInterferer | MHz | (NOTE 2) | FDL_low –30 to FDL_high + 30 |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 15 MHz below or above the UE receive band NOTE 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -BW/2 - FIoffset, case 1 and b. the carrier frequency +BW/2 + FIoffset, case 1 NOTE 3: FInterferer range values for unwanted modulated interfering signal are interferer center frequencies |  |  |  |  |

When UE is configured for simultaneous E-UTRA V2X sidelink and E-UTRA downlink reception for inter-band E-UTRA V2X / E-UTRA bands specified in Table 5.5G-2, the requirements in subclause 7.6.1.1G apply for the E-UTRA V2X sidelink reception and the requirements in subclause 7.6.1.1 apply for the E-UTRA downlink reception while all downlink carriers are active.

For intra-band contiguous multi-carrier operation, the V2X UE throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.8.2 with paramteters defined in Table 7.6.1.1G-3 and Table 7.6.1.1G-4.

Table 7.6.1.1G-3: In band blocking parameters for intra-band contiguous multi-carrier for V2X UE

| Rx Parameter | Units | V2X Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | B | C/ C1 | D | E | F |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + V2X Bandwidth Class specific value below |  |  |  |  |
|  |  | 9 | 12 |  |  |  |
| BWInterferer | MHz | 10 | 10 |  |  |  |
| FIoffset, case 1 | MHz | 15 | 15 |  |  |  |
| FIoffset, case 2 | MHz | 25 | 25 |  |  |  |

Table 7.6.1.1G-4: In-band blocking for intra-band contiguous multi-carrier for V2X UE

| V2X multi-carrier configuration | Parameter | Unit | Case 1 | Case 2 |
| --- | --- | --- | --- | --- |
|  | PInterferer | dBm | -44 | -44 |
|  | FInterferer (offset) | MHz | =-Foffset– FIoffset,case 1 & =+Foffset + FIoffset,case 1 | ≤-Foffset– FIoffset,case 2 & ≥+Foffset + FIoffset,case 2 |
| V2X_47B, V2X_47C, V2X_47C1 | FInterferer (Range) | MHz | (NOTE 2) | FDL_low – 30 to FDL_high + 30 |
| NOTE 1: For certain bands, the unwanted modulated interfering signal may not fall inside the UE receive band, but within the first 30 MHz below or above the UE receive band NOTE 2: For each carrier frequency the requirement is valid for two frequencies: a. the carrier frequency -Foffset - FIoffset, case 1 and b. the carrier frequency +Foffset + FIoffset, case 1 NOTE 3: Foffset is the frequency offset from the center frequency of the CC being tested to the edge of aggregated channel bandwidth. NOTE 4: The Finterferer (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the interferer and shall be further adjusted to MHz to be offset from the sub-carrier raster. NOTE 5: The requirement is applied for multi-carrier intra-band con-current receptions when 2 carrier transmissions are activated at the same time. |  |  |  |  |

### 7.6.2	Out-of-band blocking

Out-of-band band blocking is defined for an unwanted CW interfering signal falling more than 15 MHz below or above the UE receive band. For the first 15 MHz below or above the UE receive band the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.1 and subclause 7.6.1 shall be applied.

For CA configurations including Band 46 or Band 49, out-of-band band blocking is defined for an unwanted CW interfering signal falling more than 60 MHz below or above the UE receive band (see Table 7.6.2.1A-0a). For the first 60 MHz below or above the UE receive band the appropriate in-band blocking or adjacent channel selectivity in subclause 7.5.1A and subclause 7.6.1A shall be applied.

#### 7.6.2.1	Minimum requirements

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Tables 7.6.2.1-1 and 7.6.2.1-2. For operating bands with an unpaired DL part (as noted in Table 5.5-1), the requirements only apply for carriers assigned in the paired part.

For Table 7.6.2.1-2 in frequency range 1, 2 and 3, up to exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a 1MHz step size, where  is the number of resource blocks in the downlink transmission bandwidth configuration (see Figure 5.6-1). For these exceptions the requirements of subclause 7.7 Spurious response are applicable.

For Table 7.6.2.1-2 in frequency range 4, up to exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a 1MHz step size, where  is the number of resource blocks in the downlink transmission bandwidth configurations (see Figure 5.6-1) and  is the number of resource blocks allocated in the uplink. For these exceptions the requirements of clause 7.7 spurious response are applicable.

Table 7.6.2.1-1: Out-of-band blocking parameters

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |  |
|  |  | 6 | 6 | 6 | 6 | 7 | 9 |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3.1-2 with PCMAX_L as defined in subclause 6.2.5. NOTE 2: Reference measurement channel is specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2. NOTE 3: The REFSENS power level is specified in Table 7.3.1-1 and Table 7.3.1-1a for two and four antenna ports, respectively. NOTE 4: For DL category M1 and M2 UE, the reference sensitivity for category M1 in table 7.3.1E-3 and category M2 in Table 7.3.1E-8 should be used as REFSENS for the power in Transmission Bandwidth Configuration. NOTE5: For DL category M1 and M2 UE, the parameters for the applicable channel bandwidth apply. |  |  |  |  |  |  |  |

Table 7.6.2.1-2: Out of band blocking

| E-UTRA band | Parameter | Units | Frequency |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Range 1 | Range 2 | Range 3 | Range 4 |
|  | PInterferer | dBm | -44 | -30 | -15 | -15 |
| 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 30, 31, 33, 34, 35, 36, 37, 38, 39, 40, 41, 42 (NOTE 2), 43 (NOTE 2), 44, 45, 48 (NOTE 2), 50, 51, 52 (NOTE 6), 539, 65, 66, 68, 70, 71, 72, 73, 74, 85, 87, 88 | FInterferer (CW) | MHz | FDL_low -15 to FDL_low -60 | FDL_low -60 to FDL_low -85 | FDL_low -85 to 1 MHz | - |
|  |  |  | FDL_high +15 to FDL_high + 60 | FDL_high +60 to FDL_high +85 | FDL_high +85 to +12750 MHz | - |
| 2, 5, 12, 17, 85 | FInterferer | MHz | - | - | - | FUL_low - FUL_high (NOTE 5) |
| NOTE 1: For the UE which supports both Band 11 and Band 21 the out of blocking is FFS. NOTE 2: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 2800 MHz and FInterferer < 4400 MHz. The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 2800 MHz and FInterferer < 4800 MHz when UE supports both E-UTRA band B42 and NR bands n77, n78. NOTE 3: For the UE that supports both Band 4 and Band 66, the out-of-blocking frequency range for Band 4 is defined relative to FDL_low and FDL_high of Band 66. NOTE 4: For a UE supporting CA_20A-28A, CA_1A-3A-7A-20A-28A, CA_1A-3A-20A-28A, CA_1A-3A-3A-20A-28A, CA_1A-7A-20A-28A, CA_1A-20A-28A, CA_3A-7A-20A-28A, CA_3A-20A-28A or CA_7A-20A-28A the requirements for Band 20 and Band 28 apply with FDL_low given by the lower limit of the restricted operating frequency range in Band 28 and FDL_high by Band 20 (Table 5.5A-2). NOTE 5: Range 4 requirement does not apply to category M1 and M2. NOTE 6: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 2700 MHz and FInterferer < 4000 MHz. NOTE 7: For band 51 the FDL_high of band 50 is applied as FDL_high for band 51. NOTE 8: For UEs supporting both bands 38 and 41, the FDL_high and FDL_low of band 41 is applied as FDL_high and FDL_low for band 38. NOTE 9: The power level of the interferer (PInterferer) for Range 3 shall be modified to [-20 dBm] for FInterferer > [2580 MHz] and FInterferer < [2775 MHz]. |  |  |  |  |  |  |

#### 7.6.2.1A	Minimum requirements for CA

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one E-UTRA band, the out-of-band blocking requirements are defined with the uplink active on the band(s) other than the band whose downlink is being tested. The throughput in the downlink measured shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Tables 7.6.2.1-1 and 7.6.2.1A-0. For E-UTRA CA configurations including an operating band without uplink operation (as noted in Table 5.5-1), the requirements for all downlinks shall be met with the uplink active in the band(s) capable of UL operation. For the E-UTRA CA configurations  with band 46 or Band 49, the parameters specified in Table 7.6.2.1A-0 are replaced by those specified in Table 7.6.2.1A-0a. The UE shall meet these requirements for each component carrier while all downlink carriers are active.

For inter-band carrier aggregation with one component carrier per operating band and the uplink active in two E-UTRA bands, the out-of-band blocking requirements specified above shall be met with the transmitter power for the uplink set to 7 dB below PCMAX_L,c  for each serving cell c.

For E-UTRA CA configurations including an operating band without uplink band or an operating band with an unpaired DL part (as noted in Table 5.5-1), the requirements for all downlinks shall be met with the single uplink carrier active in each band capable of UL operation. For E-UTRA CA configurations listed in Table 7.3.1A-0a under conditions for which reference sensitivity for the operating band being tested is N/A, the out-of-band blocking requirements of subclause 7.6.2.1A do not apply.

Table 7.6.2.1A-0: out-of-band blocking for inter-band carrier aggregation

| Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- |
| Pw | dBm | Table 7.6.2.1-1 for all component carriers |  |  |
| Pinterferer | dBm | -44 + RIB,c | -30 + RIB,c | -15 + RIB,c |
| Finterferer (CW) | MHz | -60 < f – FDL_Low(j) < -15 or 15 < f – FDL_High(j) < 60 | -85 < f – FDL_Low(j) ≤ -60 or 60 ≤ f – FDL_High(j) < 85 | 1 ≤ f ≤ FDL_Low(1) – 85 or FDL_High(j) + 85 ≤ f ≤ FDL_Low(j+1) – 85 with j < X or FDL_High(X) + 85 ≤ f ≤ 12750 |
| NOTE 1: FDL_Low(j) and FDL_High(j) denote the respective lower and upper frequency limits of the operating band containing carrier j, j = 1,…,X, with carriers numbered in increasing order of carrier frequency and X the number of component carriers in the band combination. NOTE 2: For FDL_Low(j+1) – FDL_High(j) < 145 MHz and FInterferer in FDL_High(j) < f < FDL_Low(j+1) with j < X, FInterferer can be in both Range 1 and Range 2. Then the lower of the PInterferer applies. NOTE 3: For FDL_Low(j) – 15 MHz ≤ f ≤ FDL_High(j) + 15 MHz the appropriate adjacent channel selectivity and in-band blocking requirments in the respective subclauses 7.5.1A and 7.6.1.1A shall be applied for carrier j. NOTE 4: RIB,c according to Table 7.3.1-1A applies when serving cell c is measured. NOTE 5: For inter-band CA combinations containing Bands 42, 43,or 48, the interferer with respect to Band 42, Band 43, or Band 48 shall have power level (PInterferer) for Range 3 modified to -20 + RIB,c dBm for FInterferer > 2800 MHz and FInterferer < 4400 MHz. NOTE 6: For inter-band CA combinations containing Bands 7 and 38 simultaneously, for FInterferer Bands 7 and 38 are considered as one single band as follows: FDL_Low = 2570 MHz and FDL_High = 2690 MHz. For Range 2, the following applies for FDL_Low: -95 < f – FDL_Low ≤ -60 or 60 ≤ f – FDL_High < 85 . For Range 3 the following applies 1 ≤ f ≤ FDL_Low -95 or FDL_High + 85 ≤ f ≤ 12750. For UEs supporting both bands 38 and 41, FDL_Low = 2496 MHz. NOTE 7: For CA_20A-28A, CA_1A-3A-7A-20A-28A, CA_1A-3A-20A-28A, CA_1A-3A-3A-20A-28A, CA_1A-7A-20A-28A, CA_1A-20A-28A, CA_3A-7A-20A-28A, CA_3A-20A-28A and CA_7A-20A-28A the FDL_low(1) is given by the lower limit of the restricted operating frequency range in Band 28 and FDL_high(2) by Band 20 (Table 5.5A-2). NOTE 8: For inter-band CA combinations including DL in band 76, the FDL_high of band 75 is applied as FDL_high for band 76. NOTE 9: For CA band combinations including DL in band 32, FDL_high of band 75 is applied as FDL_high for band 32, and FDL_low of band 76 is applied as FDL_low for band 32. |  |  |  |  |

Table 7.6.2.1A-0a: out-of-band blocking for inter-band carrier aggregation with band 46 or Band 49 and with one active uplink

| Parameter | Unit | Range 1 | Range 2 | Range 3 |
| --- | --- | --- | --- | --- |
| Pwanted | dBm | Table 7.6.2.1-1 for component carriers in bands with j ≤ K and Table 7.6.1.1A-0a for component carriers in bands with j > K |  |  |
| Pinterferer | dBm | -44 + RIB,c | -30 + RIB,c | -15 + RIB,c (NOTE 5) |
| Finterferer (CW) | MHz | -60 < f – FDL_Low(j) < -15 with j ≤ K or 15 < f – FDL_High(j) < 60 with j ≤ K | -856 < f – FDL_Low(j) ≤ -60 or 60 ≤ f – FDL_High(j) < 856 | 1 ≤ f ≤ FDL_Low(j) – 856 or FDL_High(j) + 856 ≤ f ≤ 12750 |
| NOTE 1: FDL_Low(j) and FDL_High(j), j = 1,…,K,…N, denote the respective lower and upper frequency limits of the (non-overlapping) operating bands of the CA configuration numbered in increasing order of frequency, with N the number of bands in the band combination and K the number of bands with FDL_High ≤ 3800 MHz for Band 46 and FDL_High ≤ 2700 MHz for Band 49. NOTE 2: For FDL_Low(j) – 15 MHz ≤ f ≤ FDL_High(j) + 15 MHz the appropriate adjacent channel selectivity and in-band blocking requirements in the respective subclauses 7.5.1A and 7.6.1.1A shall be applied for carrier j ≤ K . NOTE 3: For FDL_Low(j) – 60 MHz ≤ f ≤ FDL_High(j) + 60 MHz the appropriate adjacent channel selectivity and in-band blocking requirements in the respective subclauses 7.5.1A and 7.6.1.1A shall be applied for carrier K < j ≤ N. NOTE 4: RIB,c according to Table 7.3.1-1A applies when serving cell c is measured. NOTE 5: The power level (PInterferer) for Range 3 is modified to -20 dBm for FInterferer > 4400 MHz except for band combinations with Band 42 or Band 43 for which PInterferer for Range 3 is modified to -20 dBm for FInterferer > 2800 MHz. NOTE 6: The -85 MHz and 85 MHz offset from FDL_Low(j) and FDL_High(j) is modified to -200 MHz and 200 MHz for carrier j in Band 46. |  |  |  |  |

For Table 7.6.2.1A-0 and Table 7.6.2.1A-0b in frequency ranges 1, 2 and 3, up to  exceptions per downlink are allowed for spurious response frequencies for one active uplink when measured using a step size of 1 MHz.

For Table 7.6.2.1A-0 in frequency ranges 1, 2 and 3, up to 2∙ exceptions per downlink are allowed for spurious response frequencies for two active uplinks when measured using a step size of 1 MHz. For these exceptions the requirements in clause 7.7.1A apply.

For intra-band contiguous carrier aggreagations the downlink SCC(s) shall be configured at nominal channel spacing to the PCC. For FDD, the PCC shall be configured closest to the uplink band. All downlink carriers shall be active throughout the test. The uplink output power shall be set as specified in Table 7.6.2.1A-1 with the uplink configuration set according to Table 7.3.1A-1 for the applicable carrier aggregation configuration. For UE(s) supporting one uplink carrier, the uplink configuration of the PCC shall be in accordance with Table 7.3.1-2.

The UE shall fulfil the minimum requirement in presence of an interfering signal specified in Tables 7.6.2.1A-1 and Tables 7.6.2.1A-2 being on either side of the aggregated signal. The throughput of each carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Tables 7.6.2.1A-1 and 7.6.2.1A-2. For operating bands with an unpaired DL part (as noted in Table 5.5-1), the requirements also apply for an SCC assigned in the unpaired part with parameters specified in Tables 7.6.2.1A-1 and 7.6.2.1A-2.

For Table 7.6.2.1A-2 in frequency range 1, 2 and 3, up to exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a 1MHz step size. For these exceptions the requirements of subclause 7.7 Spurious response are applicable.

Table 7.6.2.1A-1: Out-of-band blocking parameters

| Rx Parameter | Units | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | B | C | D | E | F |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + CA Bandwidth Class specific value below |  |  |  |  |
|  |  | 9 | 9 | 9 | 9 | 9 |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,c or PCMAX_L as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2. |  |  |  |  |  |  |

Table 7.6.2.1A-2: Out of band blocking

| CA configuration | Parameter | Units | Frequency |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Range 1 | Range 2 | Range 3 |
|  | PInterferer | dBm | -44 | -30 | -15 |
| CA_1C, CA_2C, CA_3B, CA_3C, CA_5B, CA_7B, CA_7C, CA_8B, CA_12B, CA_23B, CA_27B, CA_28C, CA_38C, CA_39C, CA_40C, CA_40D, CA_40E, CA_40F, CA_41C, CA_41D, CA_41E, CA_41F, CA_42C1, CA_42D1, CA_42E1, CA_42F1, CA_43C1, CA_48B1, CA_48C1, CA_48D1, CA_48E1, CA_48F1, CA_66B, CA_66C, CA_66D, CA_70C | FInterferer (CW) | MHz | FDL_low -15 to FDL_low -60 | FDL_low -60 to FDL_low -85 | FDL_low -85 to 1 MHz |
|  |  |  | FDL_high +15 to FDL_high + 60 | FDL_high +60 to FDL_high +85 | FDL_high +85 to +12750 MHz |
| NOTE 1: The power level of the interferer (PInterferer) for this CA configuration for Range 3 shall be modified to -20 dBm for FInterferer > 2800 MHz and FInterferer < 4400 MHz. The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 2800 MHz and FInterferer < 4800 MHz when UE supports both E-UTRA band B42 and NR bands n77, n78. |  |  |  |  |  |

For intra-band non-contiguous carrier aggregation with one uplink carrier and two or more downlink sub-blocks, the out-of-band blocking requirements are defined with the uplink configuration in accordance with table 7.3.1A-3. For this uplink configuration, the UE shall meet the requirements for each sub-block as specified in subclauses 7.6.2.1 and 7.6.2.1A for one component carrier and two or more component carriers per sub-block, respectively. The requirements apply with all downlink carriers active.

For Table 7.6.2.1-2 in frequency range 1, 2 and 3, up to exceptions per assigned E-UTRA channel per sub-block of the E-UTRA CA configuration are allowed for spurious response frequencies for one active uplink when measured using a 1MHz step size. For these exceptions the requirements of subclause 7.7 spurious response are applicable.

For Table 7.6.2.1-2 in frequency range 4, up to exceptions per assigned E-UTRA channel per sub-block of the E-UTRA CA configuration are allowed for spurious response frequencies for one active uplink when measured using a 1MHz step size. For these exceptions the requirements of clause 7.7 spurious response are applicable.

For intra-band non-contiguous carrier aggregation with two uplink carriers and two or more downlink carriers, the out-of-band blocking requirements are defined with the uplink configuration of the PCC and SCC being in accordance with Table 7.3.1A-4 and powers of both carriers set to PCMAX_L,c – 7 dBm. The UE shall meet the requirements specified in subclause 7.6.2.1 for each component carrier while both downlink carriers are active.

For Table 7.6.2.1-2 in frequency range 1, 2 and 3, up to 2∙ exceptions per assigned E-UTRA channel per sub-block of the E-UTRA CA configuration are allowed for spurious response frequencies for two active uplinks in the same operating band when measured using a 1MHz step size. For these exceptions the requirements of subclause 7.7 spurious response are applicable.

For Table 7.6.2.1-2 in frequency range 4, up to 2∙ exceptions per assigned E-UTRA channel per sub-block of the E-UTRA CA configuration are allowed for spurious response frequencies for two active uplinks in the same operating band when measured using a 1MHz step size. For these exceptions the requirements of clause 7.7 spurious response are applicable.

For combinations of intra-band and inter-band carrier aggregation and the uplink assigned to one E-UTRA band, the requirement is defined with the uplink active in a band other than that supporting the downlink(s) under test. The uplink configuration shall be in accordance with Table 7.3.1A-3 when the uplink is active in the band supporting two or more non-contiguous component carriers, Table 7.3.1A-1 when the uplink is active in a band supporting two contiguous component carriers and in accordance with Table 7.3.1-2 when the uplink is active in a band supporting one carrier per band. The downlink PCC shall be configured closer to the uplink operating band than the downlink SCC(s) when the uplink is active in band(s) supporting contiguous aggregation. For the two or more non-contiguous component carriers within the same band, Pwanted in Table 7.6.2.1A-0 is set using RIBNC = 0 dB for all sub-block gaps (Table 7.3.1A-3) while a band supporting contiguously aggregated carriers the out-of-band blocking parameters in Table 7.6.2.1-1 are replaced by those specified in Table 7.6.2.1A-1. For each downlink the UE shall meet the out-of-band blocking requirements applicable for inter-band carrier aggregation with one component carrier per operating band with the following exception. For each component carrier of the E-UTRA CA Configurations with band 46 or band 49, the requirements specified in Table 7.6.2.1A-0 are replaced by those in Table 7.6.2.1A-0a. All downlink carriers shall be active throughout the tests and the requirements for the downlinks shall be met with the single uplink carrier active in each band capable of UL operation.

#### 7.6.2.1D	Minimum requirements for ProSe

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.6.2 with parameters specified in Tables 7.6.2.1D-1, 7.6.2.1D-2 and 7.6.2.1D-3.

For Table 7.6.2.1D-3 in frequency range 1, 2 and 3, up to exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a 1MHz step size, where  is the number of resource blocks in the downlink transmission bandwidth configuration (see Figure 5.6-1). For these exceptions the requirements of subclause 7.7 Spurious response are applicable.

Table 7.6.2.1D-1: Out-of-band blocking parameters for ProSe Direct Discovery

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Power in Transmission Bandwidth Configuration | dBm | PREFSENS_ProSe + channel bandwidth specific value below + Poffset |  |  |  |  |  |
|  |  |  |  | 6 | 6 | 7 | 9 |
| Poffset | dB |  |  | 10.9 | 13.9 | 15.7 | 16.9 |
| NOTE 2: Reference measurement channel is specified in Annex A.6.2. |  |  |  |  |  |  |  |

Table 7.6.2.1D-2: Out-of-band blocking parameters for ProSe Direct Communication

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Power in Transmission Bandwidth Configuration | dBm | PREFSENS_ProSe + channel bandwidth specific value below |  |  |  |  |  |
|  |  |  |  | 6 | 6 | 7 | 9 |
| NOTE 1: Reference measurement channel is specified in Annex A.6.2. |  |  |  |  |  |  |  |

Table 7.6.2.1D-3: Out of band blocking for ProSe

| E-UTRA ProSe band | Parameter | Units | Frequency |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Range 1 | Range 2 | Range 3 |
|  | PInterferer | dBm | -44 | -30 | -15 |
| 2,3,4,7,14, 20,26,28,31,72 | FInterferer (CW) | MHz | FDL_low -15 to FDL_low -60 | FDL_low -60 to FDL_low -85 | FDL_low -85 to 1 MHz |
|  |  |  | FDL_high +15 to FDL_high + 60 | FDL_high +60 to FDL_high +85 | FDL_high +85 to +12750 MHz |
| NOTE 1: For the UE which supports both Band 11 and Band 21 the out of blocking is FFS. |  |  |  |  |  |

#### 7.6.2.1F	Minimum requirements for category NB1 and NB2

The category NB1 and NB2 UE throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.3.2 with parameters specified in Table 7.6.2.1F-1.

For Table 7.6.2.1F-1 in frequency range 1, 2 and 3, up to 24 exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a 1MHz step size. For these exceptions the requirements of subclause 7.7.1F spurious response are applicable.

Table 7.6.2.1F-1: Out-of-band blocking parameters for category NB1 and NB2 UE

| Parameter | Units | Frequency |  |  |
| --- | --- | --- | --- | --- |
|  |  | Range 1 | Range 2 | Range 3 |
| Pwanted | dBm | REFSENS + 6 dB |  |  |
| Pinterferer (CW) | dBm | -44 | -30 | -15 |
| Finterferer range | MHz | FDL_low - 15 to FDL_low - 60 | FDL_low - 60 to FDL_low - 85 | FDL_low - 85 to 1 MHz |
|  | MHz | FDL_high + 15 to FDL_high + 60 | FDL_high + 60 to FDL_high + 85 | FDL_high + 85 to 12750 MHz |
| NOTE 1: For operating bands which downlink band frequency range is between 617 MHz < f < 1 GHz the power level of the interferer (PInterferer) for Range 3 shall be modified to: -18 dBm for the frequency range which is bounded by FDL_low - 150 MHz of the lowest band that UE supports in frequency range 617 MHz < f < 1 GHz and FDL_high + 150 MHz of the highest band that UE supports in frequency range 617 MHz < f < 1 GHz. NOTE 2: For operating bands which downlink band frequency range is between 1475.9 MHz < f < 2690 MHz the power level of the interferer (PInterferer) for Range 3 shall be modified to: -20 dBm for the frequency range which is bounded by FDL_low- 200 MHz of the lowest band that UE supports in frequency range 1475.9 MHz < f < 2690 MHz and FDL_high + 200 MHz of the highest band that UE supports supports in frequency range 1475.9 MHz < f < 2690 MHz. NOTE 3: For operating bands which downlink band frequency range is between 460 MHz < f < 467.5 MHz the power level of the interferer (PInterferer) for Range 3 shall be modified to: – 18 dBm for the frequency range which is bounded by FDL_low - 150 MHz of the lowest band that UE supports in frequency range 460 MHz < f < 467.5 0 MHz and FDL_high + 150 MHz of the highest band that UE supports supports in frequency range 460 MHz < f < 467.5. NOTE 4: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 2800 MHz and FInterferer < 4400 MHz. |  |  |  |  |

#### 7.6.2.1G	Minimum requirements for V2X

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.8.2 with parameters specified in Tables 7.6.2.1G-1, 7.6.2.1G-2.

For Table 7.6.2.1G-2 in frequency range 1, 2 and 3, up to exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a 1MHz step size, where  is the number of resource blocks in the downlink transmission bandwidth configuration (see Figure 5.6-1). For these exceptions the requirements of subclause 7.7 spurious response are applicable.

Table 7.6.2.1G-1: Out-of-band blocking parameters

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Power in Transmission Bandwidth Configuration | dBm | PREFSENS_V2X + channel bandwidth specific value below |  |  |  |  |  |
|  |  |  |  |  | 6 |  | 9 |
| NOTE 1: Reference measurement channel is specified in Annex A.8.2. |  |  |  |  |  |  |  |

Table 7.6.2.1G-2: Out of band blocking

| E-UTRA V2X band | Parameter | Units | Frequency |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Range 1 | Range 2 | Range 3 |
|  | PInterferer | dBm | -44 | -30 | -15 |
| 47 | FInterferer (CW) | MHz | FDL_low -30 to FDL_low -60 | FDL_low -60 to FDL_low -85 | FDL_low -85 to 1 MHz |
|  |  |  | FDL_high +30 to FDL_high + 60 | FDL_high +60 to FDL_high +85 | FDL_high +85 to +12750 MHz |
| NOTE: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 4400 MHz. |  |  |  |  |  |

When UE is configured for simultaneous E-UTRA V2X sidelink and E-UTRA downlink reception for inter-band E-UTRA V2X / E-UTRA bands specified in Table 5.5G-2, the requirements in subclause 7.6.2.1G apply for the E-UTRA V2X sidelink reception and the requirements in subclause 7.6.2.1 apply for the E-UTRA downlink reception while all downlink carriers are active.

For intra-band contiguous multi-carrier operation, the V2X UE throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.8.2 with parameters specified in Tables 7.6.2.1G-3 and 7.6.2.1G-4.

For Table 7.6.2.1G-4 in frequency range 1, 2 and 3, up to exceptions are allowed for spurious response frequencies in each assigned frequency channel when measured using a 1MHz step size. For these exceptions the requirements of subclause 7.7 spurious response are applicable.

Table 7.6.2.1G-3: Out-of-band blocking parameters for intra-band contiguous multi-carrier for V2X UE

| Rx Parameter | Units | V2X Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | B | C/ C1 | D | E | F |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + V2X Bandwidth Class specific value below |  |  |  |  |
|  |  | 9 | 9 |  |  |  |

Table 7.6.2.1G-4: Out of band blocking for intra-band contiguous multi-carrier for V2X UE

| V2X multi-carrier configuration | Parameter | Units | Frequency |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Range 1 | Range 2 | Range 3 |
|  | PInterferer | dBm | -44 | -30 | -15 |
| V2X_47B, V2X_47C, V2X_47C1 | FInterferer (CW) | MHz | FDL_low -30 to FDL_low -60 | FDL_low -60 to FDL_low -85 | FDL_low -85 to 1 MHz |
|  |  |  | FDL_high +30 to FDL_high + 60 | FDL_high +60 to FDL_high +85 | FDL_high +85 to +12750 MHz |
| NOTE 1: The power level of the interferer (PInterferer) for Range 3 shall be modified to -20 dBm for FInterferer > 4400 MHz. NOTE 2: The requirement is applied for multi-carrier intra-band con-current receptions when 2 carrier transmissions are activated at the same time. |  |  |  |  |  |

### 7.6.3	Narrow band blocking

This requirement is measure of a receiver's ability to receive a E-UTRA signal at its assigned channel frequency in the presence of an unwanted narrow band CW interferer at a frequency, which is less than the nominal channel spacing.

#### 7.6.3.1	Minimum requirements

The relative throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.6.3.1-1. For operating bands with an unpaired DL part (as noted in Table 5.5-1), the requirements only apply for carriers assigned in the paired part.

Table 7.6.3.1-1: Narrow-band blocking

| Parameter | Unit | Channel Bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Pw | dBm | PREFSENS + channel-bandwidth specific value below |  |  |  |  |  |
|  |  | 22 | 18 | 16 | 13 | 14 | 16 |
| Puw (CW) | dBm | -55 | -55 | -55 | -55 | -55 | -55 |
| Fuw (offset for f = 15 kHz) | MHz | 0.9075 | 1.7025 | 2.7075 | 5.2125 | 7.7025 | 10.2075 |
| Fuw (offset for f = 7.5 kHz) | MHz |  |  |  |  |  |  |
| NOTE 1: The transmitter shall be set a 4 dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3.1-2 with PCMAX_L as defined in subclause 6.2.5. NOTE 2: Reference measurement channel is specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. NOTE 3: The PREFSENS power level is specified in Table 7.3.1-1 and Table 7.3.1-1a for two and four antenna ports, respectively. NOTE 4: For DL category M1 and M2 UE, the reference sensitivity for category M1 in table 7.3.1E-3 and category M2 in Table 7.3.1E-8 should be used as PREFSENS for Pw. NOTE5: For DL category M1 and M2 UE, the parameters for the applicable channel bandwidth apply. NOTE6: For DL category M1 UE, the parameter, Pw, for all the channel bandwidth will be PREFSENS +22 dBm. NOTE7: For DL category M2 UE, the parameter, Pw, for all channel bandwidths greater than 3 MHz will be PREFSENS +16 dBm. |  |  |  |  |  |  |  |

For the UE which supports inter-band CA configuration in Table 7.3.1-1A, PUW power defined in Table 7.6.3.1-1 is increased by the amount given by ΔRIB,c in Table 7.3.1-1A.

#### 7.6.3.1A	Minimum requirements for CA

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one E-UTRA band the narrow-band blocking requirements are defined with the uplink active on the band(s) other than the band whose downlink is being tested. The UE shall meet the requirements specified in subclause 7.6.3.1 for each component carrier while all downlink carriers are active. For E-UTRA CA configurations including an operating band without uplink band or an operating band with an unpaired DL part (as noted in Table 5.5-1), the requirements for all downlinks shall be met with the single uplink carrier active in each band capable of UL operation. For E-UTRA CA configurations listed in Table 7.3.1A-0a under conditions for which reference sensitivity for the operating band being tested is N/A, the narrow-band blocking requirements of subclause 7.6.3.1A do not apply. For E-UTRA CA configurations with a component carrier assigned in Band 46, narrow-band blocking requirements do not apply in the presence of a narrow-band interferer in Band 46.

For intra-band contiguous carrier aggregation the downlink SCC(s) shall be configured at nominal channel spacing to the PCC. For FDD, the PCC shall be configured closest to the uplink band. All downlink carriers shall be active throughout the test. The uplink output power shall be set as specified in Table 7.6.3.1A-1 with the uplink configuration set according to Table 7.3.1A-1 for the applicable carrier aggregation configuration. For UE(s) supporting one uplink, the uplink configuration of the PCC shall be in accordance with Table 7.3.1-2. The UE shall fulfil the minimum requirement in presence of an interfering signal specified in Table 7.6.3.1A-1 being on either side of the aggregated signal. The throughput of each carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.6.3.1A-1. For operating bands with an unpaired DL part (as noted in Table 5.5-1), the requirements also apply for an SCC assigned in the unpaired part with parameters specified in Table 7.6.3.1A-1.

For intra-band non-contiguous carrier aggregation with one uplink carrier and two or more downlink sub-blocks, the narrow band blocking requirements are defined with the uplink configuration in accordance with Table 7.3.1A-3. For this uplink configuration, the UE shall meet the requirements for each sub-block as specified in subclauses 7.6.3.1 and 7.6.3.1A for one component carrier and two component carriers per sub-block, respectively. The requirements apply for in-gap and out-of-gap interferers while all downlink carriers are active.

Table 7.6.3.1A-1: Narrow-band blocking

| Parameter | Unit | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | B | C | D | E | F |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + CA Bandwidth Class specific value below |  |  |  |  |
|  |  | 16 | 164 | 16 | 16 | 16 |
| Puw (CW) | dBm | -55 | -55 | -55 | -55 | -55 |
| Fuw (offset for f = 15 kHz) | MHz | - Foffset – 0.2 / + Foffset + 0.2 | - Foffset – 0.2 / + Foffset + 0.2 | - Foffset – 0.2 / + Foffset + 0.2 | - Foffset – 0.2 / + Foffset + 0.2 | - Foffset – 0.2 / + Foffset + 0.2 |
| Fuw (offset for f = 7.5 kHz) | MHz |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,c or PCMAX_L as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. NOTE 3: The Fuw (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the interferer and shall be further adjusted to MHz to be offset from the sub-carrier raster. NOTE 4: The requirement is applied for the band combinations whose component carriers’ BW≥5 MHz. |  |  |  |  |  |  |

For combinations of intra-band and inter-band carrier aggregation and one uplink carrier assigned to one E-UTRA band, the requirement is defined with the uplink active in a band other than that supporting the downlink(s) under test. The uplink configuration shall be in accordance with Table 7.3.1A-3 when the uplink is active in the band supporting two or more non-contiguous component carriers, Table 7.3.1A-1 when the uplink is active in a band supporting two contiguous component carriers and in accordance with Table 7.3.1-2 when the uplink is active in a band supporting one carrier per band. The downlink PCC shall be configured closer to the uplink operating band than the downlink SCC(s) when the uplink is active in band(s) supporting contiguous aggregation. For these uplink configurations, the UE shall meet the narrow-band blocking requirements for intra-band non-contiguous carrier aggregation with RIBNC = 0 dB for all sub-block gaps (Table 7.3.1A-3) for the two or more non-contiguous downlink sub-blocks, the requirements for intra-band contiguous carrier aggregation for the contiguously aggregated downlink carriers and for any remaining component carrier(s) the requirements specified in subclause 7.6.3. For E-UTRA CA configurations with component carriers assigned in Band 46, narrow-band blocking requirements do not apply in the presence of a narrow-band interferer in Band 46. All downlink carriers shall be active throughout the tests and the requirements for the downlinks shall be met with the single uplink carrier active in each band capable of UL operation.

#### 7.6.3.1D	Minimum requirements for ProSe

The relative throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.6.2 with parameters specified in Table 7.6.3.1D-1 and Table 7.6.3.1D-2.

Table 7.6.3.1D-1: Narrow-band blocking for ProSe Direct Discovery

| Parameter | Unit | Channel Bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Pw | dBm | PREFSENS_ProSe + channel-bandwidth specific value below + Poffset |  |  |  |  |  |
|  |  |  |  | 16 | 13 | 14 | 16 |
| Puw (CW) | dBm |  |  | -55 | -55 | -55 | -55 |
| Poffset | dB |  |  | 10.9 | 13.9 | 15.7 | 16.9 |
| Fuw (offset for f = 15 kHz) | MHz |  |  | 2.7075 | 5.2125 | 7.7025 | 10.2075 |
| Fuw (offset for f = 7.5 kHz) | MHz |  |  |  |  |  |  |
| NOTE 1: Reference measurement channel is specified in Annex A.6.2. |  |  |  |  |  |  |  |

Table 7.6.3.1D-2: Narrow-band blocking for ProSe Direct Communication

| Parameter | Unit | Channel Bandwidth |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Pw | dBm | PREFSENS_ProSe + channel-bandwidth specific value below |  |  |  |  |  |
|  |  |  |  | 16 | 13 | 14 | 16 |
| Puw (CW) | dBm |  |  | -55 | -55 | -55 | -55 |
| Fuw (offset for f = 15 kHz) | MHz |  |  | 2.7075 | 5.2125 | 7.7025 | 10.2075 |
| Fuw (offset for f = 7.5 kHz) | MHz |  |  |  |  |  |  |
| NOTE 1: Reference measurement channel is specified in Annex A.6.2. |  |  |  |  |  |  |  |

For the UE which supports inter-band CA configuration in Table 7.3.1-1A, PUW power defined in Table 7.6.3.1D-1 and  Table 7.6.3.1D-2 is increased by the amount given by ΔRIB,c in Table 7.3.1-1A.

## 7.6A	Void

<Reserved for future use>

## 7.6B	Blocking characteristics for UL-MIMO

For UE with two transmitter antenna connectors in closed-loop spatial multiplexing scheme, the minimum requirements in subclause 7.6 shall be met with the UL-MIMO configurations specified in Table 6.2.2B-2. For UL-MIMO, the parameter PCMAX_L is defined as the total transmitter power over the two transmit antenna connectors.

## 7.7	Spurious response

Spurious response is a measure of the receiver's ability to receive a wanted signal on its assigned channel frequency without exceeding a given degradation due to the presence of an unwanted CW interfering signal at any other frequency at which a response is obtained i.e. for which the out of band blocking limit as specified in subclause 7.6.2 is not met.

### 7.7.1	Minimum requirements

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Tables 7.7.1-1 and 7.7.1-2. For operating bands with an unpaired DL part (as noted in Table 5.5-1), the requirements only apply for carriers assigned in the paired part.

Table 7.7.1-1: Spurious response parameters

| Rx parameter | Units | Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz |  | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |  |  |
|  |  | 6 | 6 |  | 6 | 6 | 7 | 9 |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3.1-2. N OTE 2: Reference measurement channel is specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. NOTE 3: The REFSENS power level is specified in Table 7.3.1-1 and Table 7.3.1-1a for two and four antenna ports, respectively. |  |  |  |  |  |  |  |  |

Table 7.7.1-2: Spurious response

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -44 |
| FInterferer | MHz | Spurious response frequencies |

For the UE which supports inter-band CA configuration in Table 7.3.1-1A, Pinterferer power defined in Table 7.7.1-2 is increased by the amount given by ΔRIB,c in Table 7.3.1-1A.

### 7.7.1A	Minimum requirements for CA

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one E-UTRA band the spurious response requirements are defined with the uplink active on the band(s) other than the band whose downlink is being tested. The throughput measured in each downlink with Finterferer in Table 7.6.2.1A-0 and Table 7.6.2.1A-0a at spurious response frequencies shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Tables 7.7.1-1 and 7.7.1-2. The UE shall meet these requirements for each component carrier while all downlink carriers are active.

For inter-band carrier aggregation with one component carrier per operating band and the uplink active in two E-UTRA bands, the spurious response requirements applicable specified above shall be met with the transmitter power for the uplink set to 7 dB below PCMAX_L,c  for each serving cell c.

For E-UTRA CA configurations including an operating band without uplink band or an operating band with an unpaired DL part (as noted in Table 5.5-1), the requirements for all downlinks shall be met with the single uplink carrier active in each band capable of UL operation. For E-UTRA CA configurations listed in Table 7.3.1A-0a under conditions for which reference sensitivity for the operating band being tested is N/A, the spurious response requirements of subclause 7.7.1A do not apply.

For intra-band contiguous carrier aggregation the downlink SCC(s) shall be configured at nominal channel spacing to the PCC. For FDD, the PCC shall be configured closest to the uplink band. All downlink carriers shall be active throughout the test. The uplink output power shall be set as specified in Table 7.7.1A-1 with the uplink configuration set according to Table 7.3.1A-1 for the applicable carrier aggregation configuration. For UE(s) supporting one uplink carrier, the uplink configuration of the PCC shall be in accordance with Table 7.3.1-2. The throughput of each carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Tables 7.7.1A-1 and 7.7.1A-2. For operating bands with an unpaired DL part (as noted in Table 5.5-1), the requirements also apply for an SCC assigned in the unpaired part with parameters specified in Tables 7.7.1A-1 and 7.7.1A-2

For intra-band non-contiguous carrier aggregation with one uplink carrier and two or more downlink sub-blocks, the spurious response requirements are defined with the uplink configuration in accordance with Table 7.3.1A-3. For this uplink configuration, the UE shall meet the requirements for each sub-block as specified in subclauses 7.7.1 and 7.7.1A for one component carrier and two component carriers per sub-block, respectively. The requirements apply with all downlink carriers active.

For intra-band non-contiguous carrier aggregation with two uplink carriers and two or more downlink carriers, the spurious response requirements applicable specified above shall be met with the transmitter powers for the uplinks set to PCMAX_L,c – 7 dBm.

Table 7.7.1A-1: Spurious response parameters

| Rx Parameter | Units | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | B | C | D | E | F |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + CA Bandwidth Class specific value below |  |  |  |  |
|  |  | 9 | 9 | 9 | 9 | 9 |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,c or PCMAX_L as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. |  |  |  |  |  |  |

Table 7.7.1A-2: Spurious response

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -44 |
| FInterferer | MHz | Spurious response frequencies |

For combinations of intra-band and inter-band carrier aggregation and one uplink carrier assigned to one E-UTRA band, the requirement is defined with the uplink active in a band other than that supporting the downlink(s) under test. The uplink configuration shall be in accordance with Table 7.3.1A-3 when the uplink is active in the band supporting two or more non-contiguous component carriers, Table 7.3.1A-1 when the uplink is active in a band supporting two contiguous component carriers and in accordance with Table 7.3.1-2 when the uplink is active in a band supporting one carrier per band. The downlink PCC shall be configured closer to the uplink operating band than the downlink SCC(s) when the uplink is active in band(s) supporting contiguous aggregation. For the two or more non-contiguous component carriers within the same band, Pwanted in Table 7.6.2.1A-0 is set using RIBNC = 0 dB for all sub-block gaps (Table 7.3.1A-3) while a band supporting contiguously aggregated carriers the out-of-band blocking parameters in Table 7.7.1-1 are replaced by those specified in Table 7.7.1A-1. For each downlink the UE shall meet the spurious-response requirements applicable for inter-band carrier aggregation with one component carrier per operating band. All downlink carriers shall be active throughout the tests and the requirements for the downlinks shall be met with the single uplink carrier active in each band capable of UL operation.

### 7.7.1B	Minimum requirements for UL-MIMO

For UE with two transmitter antenna connectors in closed-loop spatial multiplexing scheme, the minimum requirements in Clause 7.7.1 shall be met with the UL-MIMO configurations specified in Table 6.2.2B-2. For UL-MIMO, the parameter PCMAX_L is defined as the total transmitter power over the two transmit antenna connectors.

### 7.7.1D	Minimum requirements for ProSe

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.6.2 with parameters specified in Tables 7.7.1D-1, 7.7.1D-2, and 7.7.1D-3.

Table 7.7.1D-1: Spurious response parameters for ProSe Direct Discovery

| Rx parameter | Units | Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz |  | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Power in Transmission Bandwidth Configuration | dBm | PREFSENS_ProSe + channel bandwidth specific value below+ Poffset |  |  |  |  |  |  |
|  |  |  |  |  | 6 | 6 | 7 | 9 |
| Poffset | dB |  |  |  | 10.9 | 13.9 | 15.7 | 16.9 |
| NOTE 1: Reference measurement channel is specified in Annex A.6.2. |  |  |  |  |  |  |  |  |

Table 7.7.1D-2: Spurious response parameters for ProSe Direct Communication

| Rx parameter | Units | Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz |  | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Power in Transmission Bandwidth Configuration | dBm | PREFSENS_ProSe + channel bandwidth specific value below |  |  |  |  |  |  |
|  |  |  |  |  | 6 | 6 | 7 | 9 |
| NOTE 1: Reference measurement channel is specified in Annex A.6.2. |  |  |  |  |  |  |  |  |

Table 7.7.1D-3: Spurious response for ProSe

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -44 |
| FInterferer | MHz | Spurious response frequencies |

For the UE which supports inter-band CA configuration in Table 7.3.1-1A, Pinterferer power defined in Table 7.7.1D-3 is increased by the amount given by ΔRIB,c in Table 7.3.1-1A.

### 7.7.1F	Minimum requirements for UE category NB1 and NB2

The category NB1 and NB2 UE throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in Annexe A.3.2 with parameters specified in Tables 7.7.1F-1.

Table 7.7.1F-1: Spurious response parameters for UE category NB1 and NB2

| Parameter | Unit | Level |
| --- | --- | --- |
| Psignal | dBm | REFSENS+6 |
| PInterferer (CW) | dBm | -44 |
| FInterferer | MHz | Spurious response frequencies |
| Number of spurious response frequencies |  | 24 (in OOB range 1, 2, 3) |
| NOTE 1: Reference measurement channel is specified in Annex A.3.2. NOTE 2: The REFSENS power level is specified in 7.3.1F.1-1. NOTE 3: OOB range 1, 2, 3 refers to Table 7.6.2.1F-1. |  |  |

### 7.7.1G	Minimum requirements for V2X

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.8.2 with parameters specified in Tables 7.7.1G-1.

Table 7.7.1G-1: Spurious response parameters

| Rx parameter | Units | Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz |  | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Power in Transmission Bandwidth Configuration | dBm | PREFSENS_V2X + channel bandwidth specific value below |  |  |  |  |  |  |
|  |  |  |  |  |  | 6 |  | 9 |
| NOTE 1: Reference measurement channel is specified in Annex A.8.2. |  |  |  |  |  |  |  |  |

Table 7.7.1G-2: Spurious response

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -44 |
| FInterferer | MHz | Spurious response frequencies |

When UE is configured for simultaneous E-UTRA V2X sidelink and E-UTRA downlink reception for inter-band E-UTRA V2X / E-UTRA bands specified in Table 5.5G-2, the requirements in subclause 7.7.1G apply for the E-UTRA V2X sidelink reception and the requirements in subclause 7.7.1 apply for the E-UTRA downlink reception while all downlink carriers are active.

For intra-band contiguous multi-carrier operation, the V2X UE throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.8.2 with parameters specified in Table 7.7.1G-3 and Table 7.7.1G-4.

Table 7.7.1G-3: Spurious response parameters for intra-band contiguous multi-carrier for V2X UE

| Rx Parameter | Units | V2X Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | B | C/ C1 | D | E | F |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + V2X Bandwidth Class specific value below |  |  |  |  |
|  |  | 9 | 9 |  |  |  |
| NOTE 1: The requirement is applied for multi-carrier intra-band con-current receptions when 2 carrier transmissions are activated at the same time. |  |  |  |  |  |  |

Tables 7.7.1G-4: Spurious response for intra-band contiguous multi-carrier for V2X UE

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -44 |
| FInterferer | MHz | Spurious response frequencies |
| NOTE 1: The requirement is applied for multi-carrier intra-band con-current receptions when 2 carrier transmissions are activated at the same time. |  |  |

## 7.8	Intermodulation characteristics

Intermodulation response rejection is a measure of the capability of the receiver to receiver a wanted signal on its assigned channel frequency in the presence of two or more interfering signals which have a specific frequency relationship to the wanted signal.

### 7.8.1	Wide band intermodulation

The wide band intermodulation requirement is defined following the same principles using modulated E-UTRA carrier and CW signal as interferer.

#### 7.8.1.1	Minimum requirements

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.8.1.1 for the specified wanted signal mean power in the presence of two interfering signals. For operating bands with an unpaired DL part (as noted in Table 5.5-1), the requirements only apply for carriers assigned in the paired part.

Table 7.8.1.1-1: Wide band intermodulation

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz |  | 3 MHz |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Power in Transmission Bandwidth Configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |  |  |  |
|  |  | 12 |  |  | 8 | 6 | 6 | 7 | 9 |
| PInterferer 1 (CW) | dBm | -46 |  |  |  |  |  |  |  |
| PInterferer 2 (Modulated) | dBm | -46 |  |  |  |  |  |  |  |
| BWInterferer 2 |  | 1.4 | 3 |  |  | 5 |  |  |  |
| FInterferer 1 (Offset) | MHz | -BW/2 –2.1 / +BW/2+ 2.1 | -BW/2 –4.5 / +BW/2 + 4.5 |  |  | -BW/2 – 7.5 / +BW/2 + 7.5 |  |  |  |
| FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |  |  |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3.1-2 with PCMAX_L as defined in subclause 6.2.5. NOTE 2: Reference measurement channel is specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. NOTE 3: The modulated interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 with set-up according to Annex C.3.1The interfering modulated signal is 5MHz E-UTRA signal as described in Annex D for channel bandwidth 5MHz. NOTE 4: The REFSENS power level is specified in Table 7.3.1-1 and Table 7.3.1-1a for two and four antenna ports, respectively. NOTE 5: For DL category M1 and M2 UE, the reference sensitivity for category M1 in table 7.3.1E-3 and category M2 in Table 7.3.1E-8 should be used as REFSENS for the power in Transmission Bandwidth Configuration. NOTE6: For DL category M1 and M2 UE, the parameters for the applicable channel bandwidth apply, and BW refers to the corresponding channel bandwidth. |  |  |  |  |  |  |  |  |  |

For the UE which supports inter band CA configuration in Table 7.3.1-1A, Pinterferer1 and Pinterferer2 powers defined in Table 7.8.1.1-1 are increased by the amount given by ΔRIB,c in Table 7.3.1-1A.

### 7.8.1A	Minimum requirements for CA

For inter-band carrier aggregation with one component carrier per operating band and the uplink assigned to one E-UTRA band the wide band intermodulation requirements are defined with the uplink active on the band(s) other than the band whose downlink is being tested. The UE shall meet the requirements specified in subclause 7.8.1.1 for each component carrier while all downlink carriers are active. For E-UTRA CA configurations including an operating band without uplink band or an operating band with an unpaired DL part (as noted in Table 5.5-1), the requirements for all downlinks shall be met with the single uplink carrier active in each band capable of UL operation. For a component carrier configured in Band 46 or Band 49, the requirements specified in subclause 7.8.1.1 are replaced by the requirements in Table 7.8.1-1A-0.

Table 7.8.1.1A-0: Wide band intermodulation

| E-UTRA band | Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1.4 MHz | 3 MHz |  | 5 MHz | 10 MHz |  | 15 MHz |  | 20 MHz |
| 46 | Power in Transmission Bandwidth Configuration | dBm | REFSENS + channel bandwidth specific value below |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  | 6 |  |  |  | 9 |
|  | PInterferer 1 (CW) | dBm | -46 |  |  |  |  |  |  |  |  |
|  | PInterferer 2 (Modulated) | dBm | -46 |  |  |  |  |  |  |  |  |
|  | BWInterferer 2 |  |  |  |  |  |  | 20 |  |  | 20 |
|  | FInterferer 1 (Offset) | MHz |  |  |  |  |  | -BW/2-30/+BW/2+30 |  |  | -BW/2 – 30 / +BW/2 + 30 |
|  | FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |  |  |  |  |  |  |  |  |
| 49 | Power in Transmission Bandwidth Configuration | dBm |  |  |  |  | 6 |  |  |  | 6 |
|  | PInterferer 1 (CW) | dBm | -46 |  |  |  |  |  |  |  |  |
|  | PInterferer 2 (Modulated) | dBm | -46 |  |  |  |  |  |  |  |  |
|  | BWInterferer 2 | MHz |  |  |  |  | 10 |  |  |  | 20 |
|  | FInterferer 1 (Offset) | MHz |  |  |  |  | -BW/2-15 / +BW/2+15 |  |  |  | -BW/2 – 30 / +BW/2 + 30 |
|  | FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |  |  |  |  |  |  |  |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L at the minimum uplink configuration specified in Table 7.3.1-2 with PCMAX_L as defined in subclause 6.2.5. NOTE 2: Reference measurement channel is specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD/FS3 as described in Annex A.5.1.1/A.5.2.1/A.5.4.1. NOTE 3: The modulated interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. |  |  |  |  |  |  |  |  |  |  |  |

For E-UTRA CA configurations listed in Table 7.3.1A-0a under conditions for which reference sensitivity for the operating band being tested is N/A, the wideband intermodulation requirements of subclause 7.8.1A do not apply.

For intra-band contiguous carrier aggegation the downlink SCC(s) shall be configured at nominal channel spacing to the PCC, For FDD, the PCC shall be configured closest to the uplink band. All downlink carriers shall be active throughout the test. The uplink output power shall be set as specified in Table 7.8.1A-1 with the uplink configuration set according to Table 7.3.1A-1 for the applicable carrier aggreagation configuration. For UE(s) supporting one uplink carrier, the uplink configuration of the PCC shall be in accordance with Table 7.3.1-2. The UE shall fulfil the minimum requirement in presence of an interfering signal specified in Table 7.8.1A-1 being on either side of the aggregated signal. The throughput of each carrier shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.2.2, A.2.3 and A.3.2 (with one sided dynamic OCNG Pattern OP.1 FDD/TDD for the DL-signal as described in Annex A.5.1.1/A.5.2.1) with parameters specified in Table 7.8.1A-1. For operating bands with an unpaired DL part (as noted in Table 5.5-1), the requirements also apply for an SCC assigned in the unpaired part with parameters specified in Tables 7.8.1A-1.

Table 7.8.1A-1: Wide band intermodulation

| Rx parameter | Units | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | B | C | D | E | F |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + CA Bandwidth Class specific value below |  |  |  |  |
|  |  | 9 | 12 | 13.8 | 15 | 16 |
| PInterferer 1 (CW) | dBm | -46 |  |  |  |  |
| PInterferer 2 (Modulated) | dBm | -46 |  |  |  |  |
| BWInterferer 2 | MHz | 5 | 5 | 5 | 5 | 5 |
| FInterferer 1 (Offset) | MHz | –Foffset-7.5 / + Foffset+7.5 | –Foffset-7.5 / + Foffset+7.5 | –Foffset-7.5 / + Foffset+7.5 | –Foffset-7.5 / + Foffset+7.5 | –Foffset-7.5 / + Foffset+7.5 |
| FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |  |  |  |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,c or PCMAX_L as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1. NOTE 3: The modulated interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 with set-up according to Annex C.3.1. NOTE 4: The interfering modulated signal is 5MHz E-UTRA signal as described in Annex D for channel bandwidth 5MHz; NOTE 5: The Finterferer 1 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the CW interferer and Finterferer 2 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the modulated interferer. |  |  |  |  |  |  |

For intra-band non-contiguous carrier aggregation with one uplink carrier and two or more downlink sub-blocks, the wide band intermodulation requirements are defined with the uplink configuration in accordance with Table 7.3.1A-3. For this uplink configuration, the UE shall meet the requirements for each sub-block as specified in subclauses 7.8.1.1 and in this subclause for one component carrier and two or more component carriers per sub-block, respectively. The requirements apply for out-of-gap interferers while all downlink carriers are active.

For combinations of intra-band and inter-band carrier aggregation and one uplink carrier assigned to one E-UTRA band, the requirement is defined with the uplink active in a band other than that supporting the downlink(s) under test. The uplink configuration shall be in accordance with Table 7.3.1A-3 when the uplink is active in the band supporting two or more non-contiguous component carriers, Table 7.3.1A-1 when the uplink is active in a band supporting two contiguous component carriers and in accordance with Table 7.3.1-2 when the uplink is active in a band supporting one carrier per band. For these uplink configurations, the UE shall meet the wide-band intermodulation requirements for intra-band non-contiguous carrier aggregation with RIBNC = 0 dB for all sub-block gaps (Table 7.3.1A-3) for the two or more non-contiguous downlink sub-blocks, the requirements for intra-band contiguous carrier aggregation for the contiguously aggregated downlink carriers and for any remaining component carrier(s) the requirements specified in subclause 7.8.1. For contiguously aggregated component carriers configured in Band 46, the said requirements for intra-band contiguous carrier aggregation of two or more downlink carriers are replaced by requirements in Table 7.8.1A-2. For non-contiguously aggregated component carriers configured in Band 46, the said requirements are applied to each sub-block for out-of-gap interferers. For the sub-block with a single component carrier, the requirement is replaced by Table 7.8.1.1A-0. For the sub-block with two or more contiguous component carriers, the requirement is replaced by Table 7.8.1.1A-2. All downlink carriers shall be active throughout the tests and the requirements for the downlinks shall be met with the single uplink carrier active in each band capable of UL operation.

Table 7.8.1A-2: Wide band intermodulation

| E-UTRA Band | Rx parameter | Units | CA Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | B | C | D | E | F |
| 46 | Power per CC in Aggregated Transmission Bandwidth Configuration | dBm | REFSENS + CA Bandwidth Class specific value below |  |  |  |  |
|  |  |  |  | 12 | 13.8 | 15 |  |
|  | PInterferer 1 (CW) | dBm | -46 |  |  |  |  |
|  | PInterferer 2 (Modulated) | dBm | -46 |  |  |  |  |
|  | BWInterferer 2 | MHz |  | 20 | 20 | 20 |  |
|  | FInterferer 1 (Offset) | MHz |  | –Foffset-30 / + Foffset+30 | –Foffset-30 / + Foffset+30 | –Foffset-30 / + Foffset+30 |  |
|  | FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |  |  |  |  |
| NOTE 1: The transmitter shall be set to 4dB below PCMAX_L,c or PCMAX_L as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD/FS3 as described in Annex A.5.1.1/A.5.2.1/A.5.4.1. NOTE 3: The modulated interferer consists of the Reference measurement channel specified in Annex A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD/FS3 as described in Annex A.5.1.1/A.5.2.1/A.5.4.1 with set-up according to Annex C.3.1. NOTE 4: The interfering modulated signal is 20 MHz E-UTRA signal as described in Annex D interference setting 2; NOTE 5: The Finterferer 1 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the CW interferer and Finterferer 2 (offset) is the frequency separation of the center frequency of the carrier closest to the interferer and the center frequency of the modulated interferer. |  |  |  |  |  |  |  |

### 7.8.1B	Minimum requirements for UL-MIMO

For UE(s) with two transmitter antenna connectors in closed-loop spatial multiplexing scheme, the minimum requirements in subclause 7.8.1 shall be met with the UL-MIMO configurations specified in Table 6.2.2B-2. For UL-MIMO, the parameter PCMAX_L is defined as the total transmitter power over the two transmit antenna connectors.

### 7.8.1D	Minimum requirements for ProSe

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.6.2 with parameters specified in Table 7.8.1D-1, Table 7.8.1D-2, and Table 7.8.1D-3 for the specified wanted signal mean power in the presence of two interfering signals

Table 7.8.1D-1: Wide band intermodulation parameters for ProSe Direct Discovery

| Rx parameter | Units | Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz |  | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Poffset | dB |  |  |  | 10.9 | 13.9 | 15.7 | 16.9 |

Table 7.8.1D-2: Wide band intermodulation for ProSe Direct Communication

| Rx parameter | Units | Channel bandwidth |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz |  | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Poffset | dB |  |  |  | 0 | 0 | 0 | 0 |

Table 7.8.1D-3: Wide band intermodulation for ProSe

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz |  | 3 MHz |  | 5 MHz | 10 MHz | 15 MHz | 20 MHz |
| Power in Transmission Bandwidth Configuration | dBm | PREFSENS_ProSe + channel bandwidth specific value below+ Poffset |  |  |  |  |  |  |  |
|  |  | 12 |  |  | 8 | 6 | 6 | 7 | 9 |
| PInterferer 1 (CW) | dBm | -46 |  |  |  |  |  |  |  |
| PInterferer 2 (Modulated) | dBm | -46 |  |  |  |  |  |  |  |
| BWInterferer 2 |  | 1.4 | 3 |  |  | 5 |  |  |  |
| FInterferer 1 (Offset) | MHz | -BW/2 –2.1 / +BW/2+ 2.1 | -BW/2 –4.5 / +BW/2 + 4.5 |  |  | -BW/2 – 7.5 / +BW/2 + 7.5 |  |  |  |
| FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |  |  |  |  |  |  |  |
| NOTE 1: Reference measurement channel is specified in Annex A.6.2 NOTE 2: The interferer is QPSK modulated PUSCH containing data and reference symbols. Normal cyclic prefix is used. The data content shall be uncorrelated to the wanted signal and modulated according to clause 5 of TS36.211 |  |  |  |  |  |  |  |  |  |

For the UE which supports inter band CA configuration in Table 7.3.1-1A, Pinterferer1 and Pinterferer2 powers defined in Table 7.8.1D-3 are increased by the amount given by ΔRIB,c in Table 7.3.1-1A.

### 7.8.1F	Minimum requirements for category NB1 and NB2

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in Annex A.3.2 with parameters specified in Table 7.8.1F-1 for the specified wanted signal mean power in the presence of two interfering signals.

Table 7.8.1F-1: Wide band intermodulation for category NB1 and NB2

| Parameters for wideband intermodulation |  |
| --- | --- |
| Category NB1 or NB2 signal power | REFSENS + 12 dB |
| CW interferer signal power | - 46 dBm |
| 1.4 MHz E-UTRA interferer signal power | - 46 dBm |
| CW interferer offset | ± 2.2 MHz |
| 1.4 MHz E-UTRA interferer offset | ± 4.4 MHz |

### 7.8.1G	Minimum requirements

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.8.2 with parameters specified in Table 7.8.1G-1 for the specified wanted signal mean power in the presence of two interfering signals

Table 7.8.1G-1: Wide band intermodulation

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz |  | 15 MHz |  |  | 20 MHz |  |
| Power in Transmission Bandwidth Configuration | dBm | PREFSENS_V2X + channel bandwidth specific value below |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  | 6 |  |  |  |  | 9 |
| PInterferer 1 (CW) | dBm | -46 |  |  |  |  |  |  |  |  |  |
| PInterferer 2 (Modulated) | dBm | -46 |  |  |  |  |  |  |  |  |  |
| BWInterferer 2 |  |  |  |  |  | 10 |  |  |  |  | 10 |
| FInterferer 1 (Offset) | MHz |  |  |  |  | -BW/2 – 15 / +BW/2 + 15 |  |  |  |  | -BW/2 – 15 / +BW/2 + 15 |
| FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |  |  |  |  |  |  |  |  |  |
| NOTE 1: Reference measurement channel is specified in Annex A.8.2 NOTE 2: The interferer is QPSK modulated PUSCH containing data and reference symbols. Normal cyclic prefix is used. The data content shall be uncorrelated to the wanted signal and modulated according to clause 5 of TS36.211 |  |  |  |  |  |  |  |  |  |  |  |

When UE is configured for simultaneous E-UTRA V2X sidelink and E-UTRA downlink reception for inter-band E-UTRA V2X / E-UTRA bands specified in Table 5.5G-2, the requirements in subclause 7.8.1G apply for the E-UTRA V2X sidelink reception and the requirements in subclause 7.8.1 apply for the E-UTRA downlink reception while all downlink carriers are active.

For intra-band contiguous multi-carrier operation, the V2X UE throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex A.8.2 with parameters specified in Table 7.8.1G-2 for the specified wanted signal mean power in the presence of two interfering signals.

Table 7.8.1G-2: Wide band intermodulation for intra-band contiguous multi-carrier for V2X UE

| Rx parameter | Units | V2X Bandwidth Class |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | B | C/ C1 | D | E | F |
| Pw in Transmission Bandwidth Configuration, per CC | dBm | REFSENS + V2X Bandwidth Class specific value below |  |  |  |  |
|  |  | 9 | 12 |  |  |  |
| PInterferer 1 (CW) | dBm | -46 |  |  |  |  |
| PInterferer 2 (Modulated) | dBm | -46 |  |  |  |  |
| BWInterferer 2 | MHz | 10 | 10 |  |  |  |
| FInterferer 1 (Offset) | MHz | –Foffset-15 / + Foffset+15 | –Foffset-15 / + Foffset+15 |  |  |  |
| FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |  |  |  |  |
| NOTE 1: The requirement is applied for multi-carrier intra-band con-current receptions when 2 carrier transmissions are activated at the same time. |  |  |  |  |  |  |

### 7.8.2	Void

## 7.9	Spurious emissions

The spurious emissions power is the power of emissions generated or amplified in a receiver that appear at the UE antenna connector.

### 7.9.1	Minimum requirements

The power of any narrow band CW spurious emission shall not exceed the maximum level specified in Table 7.9.1-1

Table 7.9.1-1: General receiver spurious emission requirements

| Frequency band | Measurement bandwidth | Maximum level | NOTE |
| --- | --- | --- | --- |
| 30MHz f < 1GHz | 100 kHz | -57 dBm |  |
| 1GHz f 12.75 GHz | 1 MHz | -47 dBm |  |
| 12.75 GHz f 5th harmonic of the upper frequency edge of the DL operating band in GHz | 1 MHz | -47 dBm | 1 |
| 12.75 GHz – 26GHz | 1 MHz | -47dBm | 3 |
| NOTE 1: Applies only for Band 22, Band 42, Band 43, Band 48 and Band 49 NOTE 2: Unused PDCCH resources are padded with resource element groups with power level given by PDCCH_RA/RB as defined in Annex C.3.1. NOTE 3: Applies for Band 46 and Band 47 |  |  |  |

In addition, for a V2X UE operating in Region 1, the power of any spurious emission shall not exceed the levels specified in Table 7.9.1-2.

Table 7.9.1-2: Additional RX spurious emissions limits in Region 1

| Frequency Range | Maximum Level | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 5795 MHz f < 5815MHz | -65 dBm (EIRP) | 1 MHz |  |
| NOTE: The EIRP requirement is converted to conducted requirement depend on the supported post antenna connector gain Gpost connector declared by the UE following the principle described in annex I. The additional Rx spurious emission limits, only apply when the network signaling value NS_33 or NS_34 is indicated. |  |  |  |

### 7.9.1A	Minimum requirements

For E-UTRA CA configurations including an operating band without uplink band (as noted in Table 5.5-1), the power of any narrow band CW spurious emission shall not exceed the maximum level specified in Table 7.9.1A-1.

Table 7.9.1A-1: General receiver spurious emission requirements

| Frequency band | Measurement bandwidth | Maximum level | NOTE |
| --- | --- | --- | --- |
| 30MHz f < 1GHz | 100 kHz | -57 dBm |  |
| 1GHz f 12.75 GHz | 1 MHz | -47 dBm |  |
| 12.75 GHz f 26 GHz | 1 MHz | -47 dBm | 3 |
| NOTE 1: Unused PDCCH resources are padded with resource element groups with power level given by PDCCH_RA/RB as defined in Annex C.3.1. NOTE 2: The requirements apply when the UE is configured for carrier aggregation but is not transmitting. NOTE 3: Applies only for Band 46 |  |  |  |

## 7.10	Receiver image

### 7.10.1	Void

### 7.10.1A	Minimum requirements for CA

Receiver image rejection is a measure of a receiver's ability to receive the E-UTRA signal on one component carrier while it is also configured to receive an adjacent aggregated carrier. Receiver image rejection ratio is the ratio of the wanted received power on a sub-carrier being measured to the unwanted image power received on the same sub-carrier when both sub-carriers are received with equal power at the UE antenna connector.

For intra-band contiguous carrier aggregation the UE shall fulfil the minimum requirement specified in Table 7.10.1A-1 for all values of aggregated input signal up to –22 dBm.

Table 7.10.1A-1: Receiver image rejection

|  | CA bandwidth class |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rx parameter | Units | A | B | C | D | E | F |
| Receiver image rejection | dB |  | 25 | 25 | 25 | 25 | 25 |

### 7.10.1G	Minimum requirements for V2X Communication

Receiver image rejection is a measure of a receiver's ability to receive the E-UTRA V2X signal on one component carrier while it is also configured to receive another aggregated carrier. Receiver image rejection ratio is the ratio of the wanted received power on a sub-carrier being measured to the unwanted image power received on the same sub-carrier when both sub-carriers are received with equal power at the UE antenna connector.

For intra-band contiguous multi-carrier operation, the UE shall fulfil the minimum requirement specified in Table 7.10.1G-1 for all values of aggregated input signal.

Table 7.10.1G-1: Receiver image rejection

|  | V2X Bandwidth Class |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Rx parameter | Units | A | B | C/ C1 | D | E | F |
| Receiver image rejection | dB |  | 30 | 30 |  |  |  |
| NOTE 1: The requirement is applied for multi-carrier intra-band con-current receptions when 2 carrier transmissions are activated at the same time. |  |  |  |  |  |  |  |