| 3GPP TS 36.101 V17.12.0 (2023-12) |  |
| --- | --- |
| Technical Specification |  |
| 3rd Generation Partnership Project; Technical Specification Group Radio Access Network; Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE) radio transmission and reception (Release 17) |  |
| The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP. The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented. This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification. Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices. |  |

3GPP Postal address 3GPP support office address 650 Route des Lucioles - Sophia Antipolis Valbonne - FRANCE Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16 Internet http://www.3gpp.org

Copyright Notification No part may be reproduced except as authorized by written permission. The copyright and the foregoing restriction extend to reproduction in all media. © 2023, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC). All rights reserved. UMTS™ is a Trade Mark of ETSI registered for the benefit of its members 3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	32

1	Scope	34

2	References	34

3	Definitions, symbols and abbreviations	34

3.1	Definitions	34

3.2	Symbols	36

3.3	Abbreviations	39

4	General	41

4.1	Relationship between minimum requirements and test requirements	41

4.2	Applicability of minimum requirements	41

4.3	Void	41

4.3A	Applicability of minimum requirements (CA, UL-MIMO, ProSe, Dual Connectivity, UE category 0, UE category M1, UE category M2, UE category 1bis, UE category NB1 and NB2, V2X Communication, MBMS UE)	41

4.4	RF requirements in later releases	43

5	Operating bands and channel arrangement	43

5.1	General	43

5.2	Void	43

5.3	Void	43

5.4	Void	43

5.5	Operating bands	43

5.5A	Operating bands for CA	46

5.5B	Operating bands for UL-MIMO	67

5.5C	Operating bands for Dual Connectivity	67

5.5D	Operating bands for ProSe	70

5.5E	Operating bands for UE category 0, UE category M1 and M2 and UE category 1bis	71

5.5F	Operating bands for category NB1 and NB2	71

5.5G	Operating bands for V2X Communication	72

5.6	Channel bandwidth	73

5.6.1	Channel bandwidths per operating band	74

5.6A	Channel bandwidth for CA	77

5.6A.1	Channel bandwidths per operating band for CA	79

5.6B	Channel bandwidth for UL-MIMO	202

5.6B.1	Void	202

5.6C	Channel bandwidth for Dual Connectivity	202

5.6C.1	Void	202

5.6D	Channel bandwidth for ProSe	202

5.6D.1	Channel bandwidths per operating band for ProSe	202

5.6F	Channel bandwidth for category NB1 and NB2	203

5.6G	Channel bandwidth for V2X Communication	204

5.6G.1	Channel bandwidths per operating band for V2X Communication	204

5.7	Channel arrangement	206

5.7.1	Channel spacing	206

5.7.1A	Channel spacing for CA	206

5.7.1F	Channel spacing for category NB1 and NB2	207

5.7.2	Channel raster	207

5.7.2A	Channel raster for CA	207

5.7.2F	Channel raster for category NB1 and NB2	207

5.7.3	Carrier frequency and EARFCN	207

5.7.3F	Carrier frequency and EARFCN for category NB1 and NB2	210

5.7.4	TX–RX frequency separation	211

5.7.4A	TX–RX frequency separation for CA	211

5.7.4E	TX–RX frequency separation for category M1 and M2	212

5.7.4F	TX–RX frequency separation for category NB1 and NB2	212

6	Transmitter characteristics	212

6.1	General	212

6.2	Transmit power	212

6.2.1	Void	212

6.2.2	UE maximum output power	212

6.2.2A	UE maximum output power for CA	215

6.2.2B	UE maximum output power for UL-MIMO	220

6.2.2C	 Void	223

6.2.2D	UE maximum output power for ProSe	223

6.2.2E	UE maximum output power for Category M1 and M2 UE	223

6.2.2F	UE maximum output power for category NB1 and NB2	224

6.2.2G	UE maximum output power for V2X Communication	225

6.2.3	UE maximum output power for modulation / channel bandwidth	227

6.2.3A	UE Maximum Output power for modulation / channel bandwidth for CA	228

6.2.3B	UE maximum output power for modulation / channel bandwidth for UL-MIMO	233

6.2.3D	UE maximum output power for modulation / channel bandwidth for ProSe	233

6.2.3E	UE maximum output power for modulation / channel bandwidth for category M1 and M2	234

6.2.3F	UE maximum output power for modulation / channel bandwidth for category NB1 and NB2	236

6.2.3G	UE maximum output power for modulation / channel bandwidth for V2X Communication	236

6.2.3G.1	MPR for Power class 3 V2X UE	236

6.2.3G.2	MPR for Power class 2 V2X UE	238

6.2.4	UE maximum output power with additional requirements	238

6.2.4A	UE maximum output power with additional requirements for CA	257

6.2.4A.1	A-MPR for CA_NS_01 for CA_1C	259

6.2.4A.2	A-MPR for CA_NS_02 for CA_1C	260

6.2.4A.3	A-MPR for CA_NS_03 for CA_1C	260

6.2.4A.4	A-MPR for CA_NS_04	261

6.2.4A.5	A-MPR for CA_NS_05 for CA_38C	264

6.2.4A.6	A-MPR for CA_NS_06	265

6.2.4A.7	A-MPR for CA_NS_07	266

6.2.4A.8	A-MPR for CA_NS_08	267

6.2.4A.9	Void	268

6.2.4A.10	A-MPR for CA_NS_10	268

6.2.4B	UE maximum output power with additional requirements for UL-MIMO	273

6.2.4D	UE maximum output power with additional requirements for ProSe	273

6.2.4E	UE maximum output power with additional requirements for category M1 and M2 UE	273

6.2.4F	UE maximum output power with additional requirements for category NB1 and NB2 UE	284

6.2.4G	UE maximum output power with additional requirements for V2X Communication	284

6.2.5	Configured transmitted power	287

6.2.5A	Configured transmitted power for CA	322

6.2.5B	Configured transmitted power for UL-MIMO	326

6.2.5C	Configured transmitted power for Dual Connectivity	326

6.2.5D	Configured transmitted power for ProSe	328

6.2.5F	Configured transmitted Power for category NB1 and NB2	329

6.2.5G	Configured transmitted power for V2X Communication	330

6.3	Output power dynamics	334

6.3.1	(Void)	334

6.3.2	Minimum output power	334

6.3.2.1	Minimum requirement	334

6.3.2A	 UE Minimum output power for CA	334

6.3.2A.1	Minimum requirement for CA	335

6.3.2B	 UE Minimum output power for UL-MIMO	335

6.3.2B.1	Minimum requirement	335

6.3.2C	 Void	335

6.3.2D	UE Minimum output power for ProSe	335

6.3.2F	UE Minimum output power for category NB1 and NB2	336

6.3.2G	UE Minimum output power for V2X Communication	336

6.3.3	Transmit OFF power	336

6.3.3.1.	Minimum requirement	336

6.3.3A	 UE Transmit OFF power for CA	337

6.3.3A.1	Minimum requirement for CA	337

6.3.3B	 UE Transmit OFF power for UL-MIMO	337

6.3.3B.1	Minimum requirement	337

6.3.3D	Transmit OFF power for ProSe	338

6.3.3F	Transmit OFF power for category NB1 and NB2	338

6.3.3G	Transmit OFF power for V2X Communication	338

6.3.4	ON/OFF time mask	339

6.3.4.1	General ON/OFF time mask	339

6.3.4.2	PRACH and SRS time mask	340

6.3.4.2.1	PRACH time mask	340

6.3.4.2.2	SRS time mask	341

6.3.4.3	Slot / Sub frame boundary time mask for subframe TTI	343

6.3.4.4	PUCCH / PUSCH / SRS time mask for subframe TTI	345

6.3.4.5	Symbol / Subslot boundary time mask for subslot TTI	347

6.3.4.6	Subslot PUCCH / subslot PUSCH / SRS time mask for subslot TTI	348

6.3.4.7	Symbol / Slot boundary time mask for slot TTI	351

6.3.4.8	Slot PUCCH / slot PUSCH / SRS time mask for slot TTI	352

6.3.4.9	Consecutive subslot and slot TTI or consecutive subslot and subframe TTI  time mask	352

6.3.4.10	Consecutive subframe and subslot TTI or consecutive slot and subslot TTI time mask	352

6.3.4.11	Consecutive TTI and slot TTI or consecutive slot TTI and TTI time mask	353

6.3.4A	ON/OFF time mask for CA	354

6.3.4B	ON/OFF time mask for UL-MIMO	354

6.3.4D	ON/OFF time mask for ProSe	354

6.3.4D.1	 General time mask for ProSe	354

6.3.4D.2	 PSSS/SSSS time mask	355

6.3.4D.3	 PSSS / SSSS / PSBCH time mask	356

6.3.4D.4	 PSSCH / SRS time mask	356

6.3.4F	ON/OFF time mask for category NB1 and NB2	356

6.3.4F.1	General ON/OFF time mask	356

6.3.4F.2	NPRACH time mask	357

6.3.4G	ON/OFF time mask for V2X Communication	357

6.3.4G.1	PSSS / SSSS / PSBCH time mask	358

6.3.5	Power Control	358

6.3.5.1	Absolute power tolerance	358

6.3.5.1.1	 Minimum requirements	358

6.3.5.2	Relative Power tolerance	359

6.3.5.2.1	Minimum requirements	359

6.3.5.3	Aggregate power control tolerance	360

6.3.5.3.1	Minimum requirement	360

6.3.5A	Power control for CA	360

6.3.5A.1	Absolute power tolerance	360

6.3.5A.1.1	 Minimum requirements	360

6.3.5A.2	Relative power tolerance	361

6.3.5A.2.1	Minimum requirements	361

6.3.5A.3	Aggregate power control tolerance	361

6.3.5A.3.1	 Minimum requirements	361

6.3.5B	Power control for UL-MIMO	362

6.3.5D	Power Control for ProSe	362

6.3.5D.1	Absolute power tolerance	362

6.3.5E	Power control for category M1 and M2	362

6.3.5E.1	Absolute power tolerance	362

6.3.5E.2	Relative Power tolerance	362

6.3.5E.3	Aggregate power control tolerance	363

6.3.5E.3.1	Minimum requirement	363

6.3.5F	Power Control for category NB1 and NB2	363

6.3.5F.1	Absolute power tolerance	364

6.3.5F.2	Relative power tolerance	364

6.3.5F.3	Aggregate power control tolerance for category NB1 and NB2	365

6.3.5F.3.1	Minimum requirement	365

6.3.5G	Power Control for V2X Communication	365

6.3.5G.1	Absolute power tolerance	365

6.4	Void	366

6.5	Transmit signal quality	366

6.5.1	Frequency error	366

6.5.1A 	Frequency error for CA	366

6.5.1B	Frequency error for UL-MIMO	366

6.5.1D	Frequency error for ProSe	366

6.5.1E	Frequency error for UE category M1 and M2	366

6.5.1F	Frequency error for UE category NB1 and NB2	367

6.5.1G	Frequency error for V2X Communication	367

6.5.2	Transmit modulation quality	367

6.5.2.1	 Error Vector Magnitude	368

6.5.2.1.1	Minimum requirement	368

6.5.2.2	 Carrier leakage	368

6.5.2.2.1	 Minimum requirements	369

6.5.2.3	In-band emissions	369

6.5.2.3.1	 Minimum requirements	369

6.5.2.4	 EVM equalizer spectrum flatness	370

6.5.2.4.1	Minimum requirements	370

6.5.2A	Transmit modulation quality for CA	371

6.5.2A.1	Error Vector Magnitude	372

6.5.2A.2	Carrier leakage for CA	372

6.5.2A.2.1	Minimum requirements	372

6.5.2A.3	In-band emissions	372

6.5.2A.3.1	Minimum requirement for CA	372

6.5.2B	Transmit modulation quality for UL-MIMO	375

6.5.2B.1	Error Vector Magnitude	375

6.5.2B.2	Carrier leakage	375

6.5.2B.3	In-band emissions	375

6.5.2B.4	EVM equalizer spectrum flatness for UL-MIMO	375

6.5.2D	Transmit modulation quality for ProSe	375

6.5.2D.1	Error Vector Magnitude	375

6.5.2D.2	Carrier leakage	376

6.5.2D.3	In-band emissions	376

6.5.2D.4	EVM equalizer spectrum flatness for ProSe	376

6.5.2E	Transmit modulation quality for category M1 and M2	376

6.5.2E.1	Error Vector Magnitude	376

6.5.2E.2	Carrier leakage	376

6.5.2E.2.1	Minimum requirements	376

6.5.2E.3	In-band emissions	376

6.5.2E.3.1	Minimum requirements	376

6.5.2F	Transmit modulation quality for Category NB1 and NB2	378

6.5.2F.1	Error Vector Magnitude	378

6.5.2F.2	Carrier leakage	378

6.5.2F.3	In-band emissions	378

6.5.2G	Transmit modulation quality for V2X Communication	379

6.5.2G.1	Error Vector Magnitude	379

6.5.2G.2	Carrier leakage	380

6.5.2G.3	In-band emissions	380

6.5.2G.4	EVM equalizer spectrum flatness	380

6.6	Output RF spectrum emissions	380

6.6.1	Occupied bandwidth	380

6.6.1.1	Additional minimum requirement for E-UTRA (network signalled value “NS_29”)	381

6.6.1A	Occupied bandwidth for CA	381

6.6.1B	 Occupied bandwidth for UL-MIMO	381

6.6.1F	Occupied bandwidth for category NB1 and NB2	382

6.6.1G	Occupied bandwidth for V2X Communication	382

6.6.2	Out of band emission	382

6.6.2.1	Spectrum emission mask	382

6.6.2.1.1	Minimum requirement	382

6.6.2.1A	Spectrum emission mask for CA	383

6.6.2.2	Additional spectrum emission mask	384

6.6.2.2.1	Minimum requirement (network signalled value "NS_03", “NS_11”, "NS_20", and “NS_21”)	384

6.6.2.2.2	Minimum requirement (network signalled value "NS_04")	385

6.6.2.2.3	Minimum requirement (network signalled value "NS_06" or “NS_07”)	385

6.6.2.2.4	Minimum requirement (network signalled value "NS_33" or “NS_34”)	386

6.6.2.2.5	Minimum requirement (network signalled value “NS_27” and “NS_43”)	386

6.6.2.2.6	Minimum requirement (network signalled value "NS_28”)	387

6.6.2.2.7	Minimum requirement (network signalled value "NS_35")	387

6.6.2.2A	Additional Spectrum Emission Mask for CA	388

6.6.2.2A.1	Minimum requirement (network signalled value "CA_NS_04")	388

6.6.2.2A.2	Minimum requirement CA_66B (network signalled value "CA_NS_09")	388

6.6.2.2A.3	Minimum requirement CA_66C (network signalled value "CA_NS_09")	389

6.6.2.2A.4	Minimum requirement CA_48B and CA_48C (network signalled value "CA_NS_10")	389

6.6.2.3	Adjacent Channel Leakage Ratio	390

6.6.2.3.1	Minimum requirement E-UTRA	390

6.6.2.3.1a	Additional minimum requirement for E-UTRA (network signalled value “NS_29”)	391

6.6.2.3.1A	Void	392

6.6.2.3.1Aa	Void	392

6.6.2.3.2	Minimum requirements UTRA	392

6.6.2.3.2A	Minimum requirement UTRA for CA	393

6.6.2.3.3A	Minimum requirements for CA E-UTRA	395

6.6.2.4	Void	396

6.6.2.4.1	Void	396

6.6.2A	Void	396

6.6.2B	 Out of band emission for UL-MIMO	396

6.6.2C	 Void	396

6.6.2D	 Out of band emission for ProSe	396

6.6.2F	Out of band emission for category NB1 and NB2	397

6.6.2F.1	Spectrum emission mask	397

6.6.2F.2	Additional Spectrum Emission Mask for Category NB1 and NB2	397

6.6.2F.2.1	Minimum requirement (network signalled value "NS_02")	397

6.6.2F.2.2	Minimum requirement (network signalled value "NS_03")	397

6.6.2F.3	Adjacent Channel Leakage Ratio for category NB1 and NB2	398

6.6.2G	 Out of band emission for V2X Communication	398

6.6.3	Spurious emissions	399

6.6.3.1	Minimum requirements	399

6.6.3.1A	Minimum requirements for CA	400

6.6.3.2	Spurious emission band UE co-existence	401

6.6.3.2A	Spurious emission band UE co-existence for CA	411

6.6.3.3	Additional spurious emissions	419

6.6.3.3.1 	Minimum requirement (network signalled value "NS_05")	420

6.6.3.3.2 	Minimum requirement (network signalled value “NS_07”)	420

6.6.3.3.3 	Minimum requirement (network signalled value “NS_08”)	420

6.6.3.3.4 	Minimum requirement (network signalled value “NS_09”)	420

6.6.3.3.5 	Minimum requirement (network signalled value "NS_12")	421

6.6.3.3.6 	Minimum requirement (network signalled value “NS_13”)	421

6.6.3.3.7 	Minimum requirement (network signalled value “NS_14”)	421

6.6.3.3.8 	Minimum requirement (network signalled value “NS_15”)	422

6.6.3.3.9 	Minimum requirement (network signalled value “NS_16”)	422

6.6.3.3.10 	Minimum requirement (network signalled value “NS_17”)	422

6.6.3.3.11 	Minimum requirement (network signalled value “NS_18”)	422

6.6.3.3.12 	Minimum requirement (network signalled value “NS_19”)	423

6.6.3.3.13 	Minimum requirement (network signalled value “NS_11”)	423

6.6.3.3.14	Minimum requirement (network signalled value “NS_20”)	423

6.6.3.3.15	Minimum requirement (network signalled value “NS_21”)	424

6.6.3.3.16	Minimum requirement (network signalled value "NS_22")	424

6.6.3.3.17	Minimum requirement (network signalled value “NS_23”)	424

6.6.3.3.18	Void	425

6.6.3.3.19	Minimum requirement (network signalled value "NS_04")	425

6.6.3.3.20	Minimum requirement (network signalled value “NS_24”)	425

6.6.3.3.21	Minimum requirement (network signalled value “NS_25”)	426

6.6.3.3.22	Minimum requirement (network signalled value “NS_26”)	426

6.6.3.3.23	Minimum requirement (network signalled value “NS_27” and “NS_43”)	426

6.6.3.3.24	Minimum requirement (network signalled value “NS_28”)	426

6.6.3.3.25	Minimum requirement (network signalled value “NS_29”)	427

6.6.3.3.26	Minimum requirement (network signalled value “NS_30”)	427

6.6.3.3.27	Minimum requirement (network signalled value “NS_31”)	428

6.6.3.3.28	Minimum requirement (network signalled value “NS_36”)	429

6.6.3.3.29	Minimum requirement (network signalled value “NS_38”)	429

6.6.3.3.30	Minimum requirement (network signalled value “NS_39”)	429

6.6.3.3.31	Minimum requirement (network signalled value “NS_40” and “NS_41”)	430

6.6.3.3.32	Minimum requirement (network signalled value “NS_42”)	430

6.6.3.3.33	Minimum requirement (network signalled value “NS_44”)	430

6.6.3.3.34	Minimum requirement (network signalled value “NS_45”)	430

6.6.3.3.35	Minimum requirement (network signalled value “NS_56”)	431

6.6.3.3A	Additional spurious emissions for CA	431

6.6.3.3A.1	Minimum requirement for CA_1C (network signalled value "CA_NS_01")	432

6.6.3.3A.2 	Minimum requirement for CA_1C (network signalled value "CA_NS_02")	432

6.6.3.3A.3	Minimum requirement for CA_1C (network signalled value "CA_NS_03")	432

6.6.3.3A.4	Minimum requirement for CA_38C (network signalled value "CA_NS_05")	433

6.6.3.3A.5	Minimum requirement for CA_7C (network signalled value "CA_NS_06")	433

6.6.3.3A.6	Minimum requirement for CA_39C and CA_39C-41A (network signalled value "CA_NS_07")	433

6.6.3.3A.7	Minimum requirement for CA_42C (network signalled value "CA_NS_08")	433

6.6.3.3A.8	Minimum requirement for CA_41C and CA_41D (network signalled value "CA_NS_04")	434

6.6.3.3A.9	Void	434

6.6.3.3A.10	Minimum requirement for CA_48B and CA_48C (network signalled value "CA_NS_10")	434

6.6.3A	Void	434

6.6.3B	 Spurious emission for UL-MIMO	435

6.6.3C	 Void	435

6.6.3D	 Spurious emission for ProSe	435

6.6.3F	Spurious emission for category NB1 and NB2	435

6.6.3G	Spurious emission for V2X Communication	435

6.6A	Void	437

6.6B	Void	437

6.7	Transmit intermodulation	437

6.7.1	Minimum requirement	437

6.7.1A	Minimum requirement for CA	438

6.7.1B	Minimum requirement for UL-MIMO	438

6.7.1F	Minimum requirement for category NB1 and NB2	439

6.7.1G	Minimum requirement for V2X Communication	439

6.8	Void	439

6.8A	Void	439

6.8B	 Time alignment error for UL-MIMO	439

6.8B.1	 Minimum Requirements	439

6.8C	Void	440

6.8D	Void	440

6.8E	Void	440

6.8F	Void	440

6.8G	Time alignment error	440

7	Receiver characteristics	440

7.1	General	440

7.2	Diversity characteristics	441

7.3	Reference sensitivity power level	441

7.3.1	Minimum requirements (QPSK)	441

7.3.1A	Minimum requirements (QPSK) for CA	481

7.3.1B	Minimum requirements (QPSK) for UL-MIMO	528

7.3.1D	Minimum requirements (QPSK) for ProSe	528

7.3.1E	Minimum requirements (QPSK) for UE category 0, M1, M2 and 1bis	530

7.3.1F	Minimum requirements for UE category NB1 and NB2	540

7.3.1F.1	Reference sensitivity for UE category NB1 and NB2	540

7.3.1F.2	Void	541

7.3.1G	Minimum requirements (QPSK) for V2X	541

7.3.2	Void	543

7.4	Maximum input level	543

7.4.1	Minimum requirements	544

7.4.1A	Minimum requirements for CA	544

7.4.1B	Minimum requirements for UL-MIMO	545

7.4.1D	Minimum requirements for ProSe	545

7.4.1F	Minimum requirements for category NB1 and NB2	546

7.4.1G	Minimum requirements for V2X	546

7.4A	Void	547

7.4A.1	Void	547

7.5	Adjacent Channel Selectivity (ACS)	547

7.5.1	Minimum requirements	547

7.5.1A	Minimum requirements for CA	548

7.5.1B	Minimum requirements for UL-MIMO	552

7.5.1D	Minimum requirements for ProSe	552

7.5.1F	Minimum requirements for category NB1 and NB2	553

7.5.1G	Minimum requirements for V2X	554

7.6	Blocking characteristics	556

7.6.1	In-band blocking	556

7.6.1.1	Minimum requirements	556

7.6.1.1A	Minimum requirements for CA	558

7.6.1.1D	Minimum requirements for ProSe	561

7.6.1.1F	Minimum requirements for category NB1 and NB2	562

7.6.1.1G	Minimum requirements for V2X	562

7.6.2	Out-of-band blocking	564

7.6.2.1	Minimum requirements	564

7.6.2.1A	Minimum requirements for CA	566

7.6.2.1D	Minimum requirements for ProSe	569

7.6.2.1F	Minimum requirements for category NB1 and NB2	569

7.6.2.1G	Minimum requirements for V2X	570

7.6.3	Narrow band blocking	571

7.6.3.1	Minimum requirements	571

7.6.3.1A	Minimum requirements for CA	572

7.6.3.1D	Minimum requirements for ProSe	573

7.6A	Void	574

7.6B	Blocking characteristics for UL-MIMO	574

7.7	Spurious response	574

7.7.1	Minimum requirements	574

7.7.1A	Minimum requirements for CA	575

7.7.1B	Minimum requirements for UL-MIMO	576

7.7.1D	Minimum requirements for ProSe	576

7.7.1F	Minimum requirements for UE category NB1 and NB2	577

7.7.1G	Minimum requirements for V2X	577

7.8	Intermodulation characteristics	578

7.8.1	Wide band intermodulation	578

7.8.1.1	Minimum requirements	578

7.8.1A	Minimum requirements for CA	579

7.8.1B	Minimum requirements for UL-MIMO	582

7.8.1D	Minimum requirements for ProSe	582

7.8.1F	Minimum requirements for category NB1 and NB2	583

7.8.1G	Minimum requirements	583

7.8.2	Void	584

7.9	Spurious emissions	584

7.9.1	Minimum requirements	585

7.9.1A	Minimum requirements	585

7.10	Receiver image	586

7.10.1	Void	586

7.10.1A	Minimum requirements for CA	586

7.10.1G	Minimum requirements for V2X Communication	586

8	Performance requirement	643

8.1	General	643

8.1.1	Receiver antenna capability	643

8.1.1.1	Simultaneous unicast and MBMS operations	644

8.1.1.2	Dual-antenna receiver capability in idle mode	644

8.1.2	Applicability of requirements	644

8.1.2.1	Applicability of requirements for different channel bandwidths	644

8.1.2.2	Definition of CA capability	644

8.1.2.2A	Definition of dual connectivity capability	649

8.1.2.3	Applicability and test rules for different CA configurations and bandwidth combination sets	650

8.1.2.3A	Applicability and test rules for different dual connectivity configuration and bandwidth combination set	653

8.1.2.3B	Applicability and test rules for different TDD-FDD CA configurations and bandwidth combination sets	654

8.1.2.3C	Applicability and test rules for SDR tests for 4Rx capable UEs	656

8.1.2.3D	Applicability and test rules for different CA with LAA SCell(s) configurations and bandwidth combination sets	656

8.1.2.3E	Applicability and test rules for SDR tests for 8Rx capable UEs	657

8.1.2.4	Test coverage for different number of component carriers	658

8.1.2.5	Applicability of performance requirements for Type B receiver	659

8.1.2.6	Applicability of performance requirements for 4Rx capable UEs	659

8.1.2.6.1	Applicability rule and antenna connection for single carrier tests with 2Rx	659

8.1.2.6.2	Applicability rule and antenna connection for CA and DC tests with 2Rx	661

8.1.2.6.3	Applicability rule and antenna connection for single carrier tests with 4Rx	661

8.1.2.6.4	Applicability rule for 256QAM tests	662

8.1.2.6.5	Applicability rule and antenna connection for CA and DC tests with 4Rx	662

8.1.2.6.6	Applicability rule for Type C with 4Rx	666

8.1.2.6.7	Applicability rule for 1024QAM tests	666

8.1.2.7	Applicability of Enhanced Downlink Control Channel Performance Requirements	666

8.1.2.8	Applicability of performance requirements for CDM-multiplexed DM RS with interfering simultaneous transmission (FRC) with multiple CSI-RS configurations	668

8.1.2.8A	Applicability of performance requirements for UE supporting coverage enhancement	668

8.1.2.9	Applicability of SDR requirements for CA and LAA	669

8.1.2.10	Applicability of performance requirements for Multi-user Superposed Transmission	670

8.1.2.11	Applicability CRS interference mitigation receivers performance requirements	670

8.1.2.12	Applicability of performance requirements for 8Rx capable UEs	671

8.1.2.12.1	Applicability rule and antenna connection for single carrier PDSCH tests	671

8.1.2.12.2	Applicability rule and antenna connection for control channel tests	677

8.1.2.12.3	Applicability rule and antenna connection for CA and DC tests	677

8.1.3	UE category and UE DL category	678

8.2	Demodulation of PDSCH (Cell-Specific Reference Symbols)	678

8.2.1	FDD (Fixed Reference Channel)	678

8.2.1.1	Single-antenna port performance	679

8.2.1.1.1	Minimum Requirement	679

8.2.1.1.2	Void	685

8.2.1.1.3	Void	685

8.2.1.1.4	Minimum Requirement 1 PRB allocation in presence of MBSFN	685

8.2.1.1.4A	Minimum Requirement 1 PRB allocation in presence of FeMBMS Unicast-mixed Cell under CA	685

8.2.1.2	Transmit diversity performance	686

8.2.1.2.1	Minimum Requirement 2 Tx Antenna Port	686

8.2.1.2.2	Minimum Requirement 4 Tx Antenna Port	687

8.2.1.2.3	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)	687

8.2.1.2.3A	Minimum Requirement 2 Tx Antenna Ports (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	689

8.2.1.2.4	Enhanced Performance Requirement Type A - 2 Tx Antenna Ports with TM3 interference model	691

8.2.1.2.5	Enhanced Performance Requirement Type B - 2 Tx Antenna Ports with TM2 interference model	693

8.2.1.2.6	Enhanced Performance Requirement Type B - 2 Tx Antenna Ports with TM9 interference model	694

8.2.1.2.7	Minimum Requirement 2 Tx Antenna Port (Superposed transmission)	695

8.2.1.3	Open-loop spatial multiplexing performance	695

8.2.1.3.1	Minimum Requirement 2 Tx Antenna Port	695

8.2.1.3.1A	Soft buffer management test	699

8.2.1.3.1B	Enhanced Performance Requirement Type C –2Tx Antenna Ports	700

8.2.1.3.1C	Enhanced Performance Requirement Type C - 2 Tx Antenna Ports with TM1 interference	701

8.2.1.3.2	Minimum Requirement 4 Tx Antenna Port	702

8.2.1.3.3	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)	702

8.2.1.3.4	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	706

8.2.1.3.5	Minimum Requirement 2 Tx Antenna Port (Superposed transmission)	708

8.2.1.3.6	Minimum Requirement 2 Tx Antenna Port (network-based CRS interference mitigation)	709

8.2.1.4	Closed-loop spatial multiplexing performance	710

8.2.1.4.1	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Port	710

8.2.1.4.1A	Minimum Requirement Single-Layer Spatial Multiplexing 4 Tx Antenna Port	711

8.2.1.4.1B	Enhanced Performance Requirement Type A - Single-Layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model	711

8.2.1.4.1C	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Ports (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	713

8.2.1.4.1D	Enhanced Performance Requirement Type B - Single-layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model	716

8.2.1.4.1E	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Ports with CRS assistance information	718

8.2.1.4.1F	Minimum Requirement Single-Layer Spatial Multiplexing 4 Tx Antenna Ports with CRS assistance information	719

8.2.1.4.2	Minimum Requirement Multi-Layer Spatial Multiplexing 2 Tx Antenna Port	720

8.2.1.4.2A	Enhanced Performance Requirement Type C – Multi-layer Spatial Multiplexing 2Tx Antenna Ports	721

8.2.1.4.3	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port	721

8.2.1.4.3A	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port for dual connectivity	725

8.2.1.4.4	Minimum Requirement Multi-Layer Spatial Multiplexing 2 Tx Antenna Port (Superposed transmission)	727

8.2.1.5	MU-MIMO	728

8.2.1.6	[Control channel performance: D-BCH and PCH]	728

8.2.1.7	Carrier aggregation with power imbalance	728

8.2.1.7.1	Minimum Requirement	728

8.2.1.8	Intra-band non-contiguous carrier aggregation with timing offset	729

8.2.1.8.1	Minimum Requirement	729

8.2.1.9	HST-SFN performance	730

8.2.1.9.1	Minimum Requirement	730

8.2.1.9.2	Minimum Requirement for Rel-16 further enhanced HST	733

8.2.1.10	Intra-band contiguous carrier aggregation with minimum channel spacing	733

8.2.1.10.1	Minimum Requirement	734

8.2.2	TDD (Fixed Reference Channel)	734

8.2.2.1	Single-antenna port performance	735

8.2.2.1.1	Minimum Requirement	735

8.2.2.1.2	Void	740

8.2.2.1.3	Void	740

8.2.2.1.4	Minimum Requirement 1 PRB allocation in presence of MBSFN	740

8.2.2.2	Transmit diversity performance	740

8.2.2.2.1	Minimum Requirement 2 Tx Antenna Port	740

8.2.2.2.2	Minimum Requirement 4 Tx Antenna Port	741

8.2.2.2.3	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)	742

8.2.2.2.3A	Minimum Requirement 2 Tx Antenna Ports (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	743

8.2.2.2.4	Enhanced Performance Requirement Type A – 2 Tx Antenna Ports with TM3 interference model	745

8.2.2.2.5	Minimum Requirement 2 Tx Antenna Port (when EIMTA-MainConfigServCell-r12 is configured)	747

8.2.2.2.6	Enhanced Performance Requirement Type B - 2 Tx Antenna Ports with TM2 interference model	747

8.2.2.2.7	Enhanced Performance Requirement Type B - 2 Tx Antenna Ports with TM9 interference model	749

8.2.2.2.8	Minimum Requirement 2 Tx Antenna Port (Superposed transmission)	750

8.2.2.3	Open-loop spatial multiplexing performance	751

8.2.2.3.1	Minimum Requirement 2 Tx Antenna Port	751

8.2.2.3.1A	Soft buffer management test	754

8.2.2.3.1B	Enhanced Performance Requirement Type C - 2Tx Antenna Ports	754

8.2.2.3.1C	Enhanced Performance Requirement Type C - 2 Tx Antenna Ports with TM1 interference	755

8.2.2.3.2	Minimum Requirement 4 Tx Antenna Port	756

8.2.2.3.3	Minimum Requirement 2Tx antenna port (demodulation subframe overlaps with aggressor cell ABS)	757

8.2.2.3.4	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	761

8.2.2.3.5	Minimum Requirement 2 Tx Antenna Port (Superposed transmission)	763

8.2.2.3.6	Minimum Requirement 2 Tx Antenna Port (network-based CRS interference mitigation)	764

8.2.2.4	Closed-loop spatial multiplexing performance	765

8.2.2.4.1	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Port	765

8.2.2.4.1A	Minimum Requirement Single-Layer Spatial Multiplexing 4 Tx Antenna Port	766

8.2.2.4.1B	Enhanced Performance Requirement Type A – Single-Layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model	766

8.2.2.4.1C	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Ports (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	768

8.2.2.4.1D	Enhanced Performance Requirement Type B - Single-layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model	770

8.2.2.4.1E	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Ports with CRS assistance information	772

8.2.2.4.1F	Minimum Requirement Single-Layer Spatial Multiplexing 4 Tx Antenna Ports with CRS assistance information	773

8.2.2.4.2	Minimum Requirement Multi-Layer Spatial Multiplexing 2 Tx Antenna Port	775

8.2.2.4.2A	Enhanced Performance Requirement Type C Multi-Layer Spatial Multiplexing 2 Tx Antenna Port	775

8.2.2.4.3	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port	776

8.2.2.4.3A	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port for dual connectivity	780

8.2.2.4.4	Void	781

8.2.2.4.5	Minimum Requirement Multi-Layer Spatial Multiplexing 2 Tx Antenna Port (Superposed transmission)	781

8.2.2.5	MU-MIMO	782

8.2.2.6	[Control channel performance: D-BCH and PCH]	782

8.2.2.7	Carrier aggregation with power imbalance	782

8.2.2.7.1	Minimum Requirement	782

8.2.2.8	Intra-band contiguous carrier aggregation with minimum channel spacing	783

8.2.2.8.1	Minimum Requirement	783

8.2.2.9	HST-SFN performance	784

8.2.2.9.1	Minimum Requirement	784

8.2.2.9.2	Minimum Requirement for Rel-16 further enhanced HST	786

8.2.3	TDD FDD CA (Fixed Reference Channel)	787

8.2.3.1	Single-antenna port performance	788

8.2.3.1.1	Minimum Requirement for FDD PCell	788

8.2.3.1.2	Minimum Requirement for TDD PCell	792

8.2.3.2	Open-loop spatial multiplexing performance 2Tx Antenna port	796

8.2.3.2.1	Minimum Requirement for FDD PCell	796

8.2.3.2.1A	Soft buffer management test for FDD PCell	800

8.2.3.2.2	Minimum Requirement for TDD PCell	801

8.2.3.2.2A	Soft buffer management test for TDD PCell	805

8.2.3.3	Closed-loop spatial multiplexing performance 4Tx Antenna Port	806

8.2.3.3.1	Minimum Requirement for FDD PCell	806

8.2.3.3.2	Minimum Requirement for TDD PCell	810

8.2.3.4	Minimum Requirement for Closed-loop spatial multiplexing performance 4Tx Antenna Port for dual connectivity	814

8.2.3.5	HST-SFN performance	816

8.2.3.5.0	General	816

8.2.3.5.1	Minimum Requirement for FDD PCell	816

8.2.3.5.2	Minimum Requirement for TDD PCell	819

8.2.4	LAA	822

8.2.4.1	Closed-loop spatial multiplexing performance 4Tx Antenna Port	822

8.2.4.1.1	FDD PCell (FDD single carrier)	822

8.2.4.1.2	TDD PCell (TDD single carrier)	826

8.3	Demodulation of PDSCH (User-Specific Reference Symbols)	829

8.3.1	FDD	829

8.3.1.1	Single-layer Spatial Multiplexing	830

8.3.1.1A	Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model	832

8.3.1.1B	Single-layer Spatial Multiplexing (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	835

8.3.1.1C	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with TM9 interference model	838

8.3.1.1D	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with CRS interference model	840

8.3.1.1E	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with TM3 interference model	841

8.3.1.1F	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with TM10 serving cell configuration and TM9 interference model	842

8.3.1.1G	Single-layer Spatial Multiplexing (CRS assistance information is configured)	844

8.3.1.1H	Single-layer Spatial Multiplexing (With Enhanced DMRS table configured)	846

8.3.1.1I	Single-layer Spatial Multiplexing (with assistance information for simultaneous transmition interfering PDSCH)	847

8.3.1.2	Dual-Layer Spatial Multiplexing	849

8.3.1.2A	Enhanced Performance Requirement Type C - Dual-Layer Spatial Multiplexing	850

8.3.1.3	Performance requirements for DCI format 2D and non Quasi Co-located Antenna Ports	851

8.3.1.3.1	Minimum requirement with Same Cell ID (with single NZP CSI-RS resource)	851

8.3.1.3.2	Minimum requirements with Same Cell ID (with multiple NZP CSI-RS resources)	853

8.3.1.3.3	Minimum requirement with Different Cell ID and Colliding CRS (with single NZP CSI-RS resource)	855

8.3.1.3.4	Minimum requirement with Different Cell ID and non-colliding CRS (with single NZP CSI-RS resource and CRS assistance information is configured)	857

8.3.1.3.5	Minimum requirements with different Cell ID and non-colliding CRS (with multiple NZP CSI-RS resources and CRS assistance information is configured)	859

8.3.1.3.6	Minimum requirements for QCL Type C and 2 Layers Spatial Multiplexing	862

8.3.1.4	Performance Requirements for semiOpenLoop transmission	864

8.3.2	TDD	866

8.3.2.1	Single-layer Spatial Multiplexing	866

8.3.2.1A	Single-layer Spatial Multiplexing (with multiple CSI-RS configurations)	868

8.3.2.1B	Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model	870

8.3.2.1C	Single-layer Spatial Multiplexing (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	873

8.3.2.1D	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with TM9 interference	876

8.3.2.1E	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with CRS interference model	878

8.3.2.1F	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with TM3 interference	880

8.3.2.1G	Enhanced Performance Requirement Type B – Single-layer Spatial Multiplexing with TM10 serving cell configuration and TM9 interference model	881

8.3.2.1H	Single-layer Spatial Multiplexing (CRS assistance information is configured)	883

8.3.2.1I	Single-layer Spatial Multiplexing (With Enhanced DMRS table configured)	885

8.3.2.1J	Single-layer Spatial Multiplexing (with assistance information for simultaneous transmition interfering PDSCH)	886

8.3.2.2	Dual-Layer Spatial Multiplexing	888

8.3.2.2A	Enhanced Performance Requirement Type C - Dual-Layer Spatial Multiplexing	888

8.3.2.3	Dual-Layer Spatial Multiplexing (with multiple CSI-RS configurations)	889

8.3.2.4	Performance requirements for DCI format 2D and non Quasi Co-located Antenna Ports	890

8.3.2.4.1	Minimum requirement with Same Cell ID (with single NZP CSI-RS resource)	890

8.3.2.4.2	Minimum requirements with Same Cell ID (with multiple NZP CSI-RS resources)	892

8.3.2.4.3	Minimum requirement with Different Cell ID and Colliding CRS (with single NZP CSI-RS resource)	894

8.3.2.4.4	Minimum requirement with Different Cell ID and non-Colliding CRS (with single NZP CSI-RS resource and CRS assistance information is configured)	896

8.3.2.4.5	Minimum requirements with different Cell ID and non-colliding CRS (with multiple NZP CSI-RS resources and CRS assistance information is configured)	898

8.3.2.4.6	Minimum requirements for QCL Type C and 2 Layers Spatial Multiplexing	901

8.3.2.5	Performance Requirements for semiOpenLoop transmission	903

8.3.3	LAA	905

8.3.3.1	Dual-Layer Spatial Multiplexing with DM-RS	905

8.3.3.1.1	FDD PCell (FDD single carrier)	905

8.3.3.1.2	TDD Pcell (TDD single carrier)	910

8.4	Demodulation of PDCCH/PCFICH	914

8.4.1	FDD	914

8.4.1.1	Single-antenna port performance	915

8.4.1.2	Transmit diversity performance	915

8.4.1.2.1	Minimum Requirement 2 Tx Antenna Port	915

8.4.1.2.2	Minimum Requirement 4 Tx Antenna Port	915

8.4.1.2.3 Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)	916

8.4.1.2.4	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	921

8.4.1.2.5	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Port under Asynchronous Network	927

8.4.1.2.6	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	928

8.4.1.2.7	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Port with Colliding CRS Dominant Interferer	929

8.4.1.2.8	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	930

8.4.1.2.9	Enhanced Downlink Control Channel Performance Requirement Type A - 4 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	931

8.4.2	TDD	932

8.4.2.1	Single-antenna port performance	933

8.4.2.2	Transmit diversity performance	933

8.4.2.2.1	Minimum Requirement 2 Tx Antenna Port	933

8.4.2.2.2	Minimum Requirement 4 Tx Antenna Port	934

8.4.2.2.3	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)	934

8.4.2.2.4	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	938

8.4.2.2.5	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Port with Colliding CRS Dominant Interferer	942

8.4.2.2.6	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	943

8.4.2.2.7	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Port with Colliding CRS Dominant Interferer	944

8.4.2.2.8	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	945

8.4.2.2.9	Enhanced Downlink Control Channel Performance Requirement Type A - 4 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	946

8.4.3	LAA	947

8.4.3.1	Transmit diversity performance	947

8.4.3.1.1	FDD Pcell (FDD single carrier)	947

8.4.3.1.2	TDD Pcell (TDD single carrier)	948

8.5	Demodulation of PHICH	949

8.5.1	FDD	950

8.5.1.1	Single-antenna port performance	950

8.5.1.2	Transmit diversity performance	950

8.5.1.2.1	Minimum Requirement 2 Tx Antenna Port	950

8.5.1.2.2	Minimum Requirement 4 Tx Antenna Port	951

8.5.1.2.3	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)	951

8.5.1.2.4	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	953

8.5.1.2.5	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Ports under Asynchronous Network	956

8.5.1.2.6	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Ports with Non-Colliding CRS Dominant Interferer	957

8.5.1.2.7	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Ports with Colliding CRS Dominant Interferer	958

8.5.1.2.8	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Ports with Non-Colliding CRS Dominant Interferer	959

8.5.2	TDD	960

8.5.2.1	Single-antenna port performance	961

8.5.2.2	Transmit diversity performance	961

8.5.2.2.1	Minimum Requirement 2 Tx Antenna Port	961

8.5.2.2.2	Minimum Requirement 4 Tx Antenna Port	962

8.5.2.2.3	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS)	962

8.5.2.2.4	Minimum Requirement 2 Tx Antenna Port (demodulation subframe overlaps with aggressor cell ABS and CRS assistance information are configured)	964

8.5.2.2.5	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Ports with Colliding CRS Dominant Interferer	966

8.5.2.2.6	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Ports with Non-Colliding CRS Dominant Interferer	967

8.5.2.2.7	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Ports with Colliding CRS Dominant Interferer	968

8.5.2.2.8	Enhanced Downlink Control Channel Performance Requirement Type B - 2 Tx Antenna Ports with Non-Colliding CRS Dominant Interferer	969

8.6	Demodulation of PBCH	970

8.6.1	FDD	970

8.6.1.1	Single-antenna port performance	970

8.6.1.2	Transmit diversity performance	971

8.6.1.2.1	Minimum Requirement 2 Tx Antenna Port	971

8.6.1.2.2	Minimum Requirement 4 Tx Antenna Port	971

8.6.1.2.3	Minimum Requirement 2 Tx Antenna Port under Time Domain Measurement Resource Restriction with CRS Assistance Information	971

8.6.2	TDD	973

8.6.2.1	Single-antenna port performance	973

8.6.2.2	Transmit diversity performance	973

8.6.2.2.1	Minimum Requirement 2 Tx Antenna Port	973

8.6.2.2.2	Minimum Requirement 4 Tx Antenna Port	973

8.6.2.2.3	Minimum Requirement 2 Tx Antenna Port under Time Domain Measurement Resource Restriction with CRS Assistance Information	974

8.7	Sustained downlink data rate provided by lower layers	975

8.7.1	FDD (single carrier and CA)	975

8.7.2	TDD (single carrier and CA)	992

8.7.3	FDD (EPDCCH scheduling)	996

8.7.4	TDD (EPDCCH scheduling)	998

8.7.5	TDD FDD CA	1000

8.7.5.1	Minimum Requirement FDD PCell	1001

8.7.5.2	Minimum Requirement TDD PCell	1009

8.7.6	FDD (DC)	1018

8.7.7	TDD (DC)	1025

8.7.8	TDD FDD (DC)	1028

8.7.9	FDD (4 Rx)	1031

8.7.10	TDD (4 Rx)	1033

8.7.11	TDD FDD CA (4 Rx)	1035

8.7.11.1	Void	1037

8.7.12	LAA	1037

8.7.12.1	FDD CA in licensed bands	1037

8.7.12.2	TDD CA in licensed bands	1039

8.7.12.3	TDD-FDD CA in licensed bands	1041

8.7.13	FDD DC (4 Rx)	1044

8.7.14	TDD DC (4 Rx)	1045

8.7.15	TDD FDD DC (4 Rx)	1047

8.7.16	FDD (1024QAM and up to 4Rx supported)	1049

8.7.17	TDD (1024QAM and up to 4 Rx supported)	1052

8.7.18	TDD FDD CA (1024QAM and up to 4 Rx supported)	1054

8.7.19	TDD (8 Rx)	1056

8.8	Demodulation of EPDCCH	1058

8.8.1	Distributed Transmission	1058

8.8.1.1	FDD	1058

8.8.1.1.1	Void	1059

8.8.1.2	TDD	1059

8.8.1.2.1	Void	1060

8.8.2	Localized Transmission with TM9	1060

8.8.2.1	FDD	1060

8.8.2.1.1	Void	1061

8.8.2.1.2	Void	1062

8.8.2.2	TDD	1062

8.8.2.2.1	Void	1063

8.8.2.2.2	Void	1063

8.8.3	Localized transmission with TM10 Type B quasi co-location type	1063

8.8.3.1	FDD	1063

8.8.3.2	TDD	1066

8.8.4	Enhanced Downlink Control Channel Performance Requirements Type A - Localized Transmission with CRS Interference Model	1069

8.8.4.1	FDD	1069

8.8.4.2	TDD	1070

8.8.5	Enhanced Downlink Control Channel Performance Requirements Type A - Distributed Transmission with TM9 Interference Model	1072

8.8.5.1	TDD	1072

8.8.6	Enhanced Downlink Control Channel Performance Requirements Type A - Distributed Transmission with TM3 Interference Model	1073

8.8.6.1	FDD	1073

8.9	Demodulation (single receiver antenna)	1074

8.9.1	PDSCH	1074

8.9.1.1	FDD and half-duplex FDD (Fixed Reference Channel)	1074

8.9.1.1.1	Transmit diversity performance (Cell-Specific Reference Symbols)	1074

8.9.1.1.2	Closed-loop spatial multiplexing performance (Cell-Specific Reference Symbols)	1075

8.9.1.1.3	Closed-loop spatial multiplexing performance (User-Specific Reference Symbols)	1077

8.9.1.2	TDD (Fixed Reference Channel)	1080

8.9.1.2.1	Transmit diversity performance (Cell-Specific Reference Symbols)	1080

8.9.1.2.2	 Closed-loop spatial multiplexing performance (Cell-Specific Reference Symbols)	1081

8.9.1.2.3	Closed-loop spatial multiplexing performance (User-Specific Reference Symbols)	1084

8.9.2	PHICH	1086

8.9.2.1	FDD and half-duplex FDD	1086

8.9.2.1.1	Transmit diversity performance	1086

8.9.2.2	TDD	1086

8.9.2.2.1	Transmit diversity performance	1086

8.9.3	PBCH	1087

8.9.3.1	FDD and half-duplex FDD	1087

8.9.3.1.1	Transmit diversity performance	1087

8.9.3.2	TDD	1087

8.9.3.2.1	Transmit diversity performance	1087

8.9.4	PDCCH/PCFICH	1087

8.9.4.1	FDD and half-duplex FDD	1087

8.9.4.1.1	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	1087

8.9.4.1.2	Enhanced Downlink Control Channel Performance Requirement Type A - 4 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	1088

8.9.4.2	TDD	1089

8.9.4.2.1	Enhanced Downlink Control Channel Performance Requirement Type A - 2 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	1089

8.9.4.2.2	Enhanced Downlink Control Channel Performance Requirement Type A - 4 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	1090

8.10	Demodulation (4 receiver antenna ports)	1091

8.10.1	PDSCH	1091

8.10.1.1	FDD (Fixed Reference Channel)	1091

8.10.1.1.1	Transmit diversity performance with 2Tx Antenna Ports (Cell-Specific Reference Symbols)	1092

8.10.1.1.1A	Transmit diversity performance wit Enhanced Performance Requirement Type A - 2 Tx Antenna Ports with TM3 interference model	1093

8.10.1.1.2	Open-loop spatial multiplexing performance with 2Tx Antenna Ports (Cell-Specific Reference Symbols)	1094

8.10.1.1.3	Closed-loop spatial multiplexing Enhanced Performance Requirements Type A - Single-Layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model (Cell-Specific Reference Symbols)	1094

8.10.1.1.4	Closed-loop spatial multiplexing performance, Dual-Layer Spatial Multiplexing 4 Tx Antenna Port (Cell-Specific Reference Symbols)	1095

8.10.1.1.4A	Enhanced Performance Requirement Type C - Dual-Layer Spatial Multiplexing with 2Tx Antenna Ports	1096

8.10.1.1.5	Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model (User-Specific Reference Symbols)	1097

8.10.1.1.5A	Single-layer Spatial Multiplexing (User-Specific Reference Symbols)	1100

8.10.1.1.5B	Single-layer Spatial Multiplexing (With Enhanced DMRS table configured)	1101

8.10.1.1.6	Dual-Layer Spatial Multiplexing (User-Specific Reference Symbols)	1102

8.10.1.1.6A	Enhanced Performance Requirement Type C - Dual-Layer Spatial Multiplexing	1104

8.10.1.1.6B	Dual-Layer Spatial Multiplexing with altCQI-Table-1024QAM configured (User-Specific Reference Symbols)	1105

8.10.1.1.7	Open-loop spatial multiplexing, 3 Layer Multiplexing with 4 Tx Antenna Ports (Cell-Specific Reference Symbols)	1106

8.10.1.1.7A	Enhanced Performance Requirement Type C - Open-loop spatial multiplexing, 3 Layer Multiplexing with 4 Tx Antenna Ports (Cell-Specific Reference Symbols)	1106

8.10.1.1.8	Closed-loop spatial multiplexing performance, 4 Layers spatial multiplexing 4 Tx antennas (Cell-Specific Reference Symbols)	1107

8.10.1.1.9	4 Layer Spatial Multiplexing (User-Specific Reference Symbols)	1107

8.10.1.1.9A	Enhanced Performance Requirement Type C - 4 Layer Spatial Multiplexing (User-Specific Reference Symbols)	1109

8.10.1.1.10	Closed loop spatial multiplexing performance - Single-Layer Spatial Multiplexing 2 Tx Antenna Port with CRS assistance information (Cell-Specific Reference Symbols)	1110

8.10.1.1.11	Closed loop spatial multiplexing performance - Single-Layer Spatial Multiplexing 4 Tx Antenna Port with CRS assistance information (Cell-Specific Reference Symbols)	1111

8.10.1.1.12	Closed loop spatial multiplexing performance - Single-Layer Spatial Multiplexing with CRS assistance information (User-Specific Reference Symbols)	1112

8.10.1.1.13	Performance requirements for DCI format 2D and non Quasi Co-located Antenna Ports	1113

8.10.1.1.14	HST-SFN performance	1117

8.10.1.2	TDD (Fixed Reference Channel)	1117

8.10.1.2.1	Transmit diversity performance with 2Tx Antenna Ports (Cell-Specific Reference Symbols)	1118

8.10.1.2.1A	Transmit diversity performance with Enhanced Performance Requirement Type A – 2 Tx Antenna Ports with TM3 interference model	1119

8.10.1.2.2	Open-loop spatial multiplexing performance  with 2Tx Antenna Ports (Cell-Specific Reference Symbols)	1120

8.10.1.2.3	 Closed-loop spatial multiplexing Enhanced Performance Requirements Type A - Single-Layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model (Cell-Specific Reference Symbols)	1120

8.10.1.2.4	Closed-loop spatial multiplexing performance, Dual-Layer Spatial Multiplexing 4 Tx Antenna Ports (Cell-Specific Reference Symbols)	1121

8.10.1.2.4A	Enhanced Performance Requirement Type C - Dual-Layer Spatial Multiplexing with 2Tx Antenna Ports	1122

8.10.1.2.5	Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model (User-Specific Reference Symbols)	1123

8.10.1.2.5A	Single-layer Spatial Multiplexing (with multiple CSI-RS configurations)	1126

8.10.1.2.5B	Single-layer Spatial Multiplexing (With Enhanced DMRS table configured)	1127

8.10.1.2.6	Dual-Layer Spatial Multiplexing (User-Specific Reference Symbols)	1128

8.10.1.2.6A	Enhanced Performance Requirement Type C - Dual-Layer Spatial Multiplexing	1130

8.10.1.2.6B	Dual-Layer Spatial Multiplexing with altCQI-Table-1024QAM configured (User-Specific Reference Symbols)	1131

8.10.1.2.7	Open-loop spatial multiplexing, 3 Layer Multiplexing with 4 Tx Antenna Ports (Cell-Specific Reference Symbols)	1132

8.10.1.2.7A	Enhanced Performance Requirement Type C - Open-loop spatial multiplexing, 3 Layer Multiplexing with 4 Tx Antenna Ports (Cell-Specific Reference Symbols)	1132

8.10.1.2.8	Closed-loop spatial multiplexing performance, 4 Layers spatial multiplexing 4 Tx antennas	1133

8.10.1.2.9	4 Layer Spatial Multiplexing (User-Specific Reference Symbols)	1134

8.10.1.2.9A	Enhanced Performance Requirement Type C - 4 Layer Spatial Multiplexing (User-Specific Reference Symbols)	1135

8.10.1.2.10	Closed loop spatial multiplexing performance - Single-Layer Spatial Multiplexing 2 Tx Antenna Port with CRS assistance information (Cell-Specific Reference Symbols)	1136

8.10.1.2.11	Closed loop spatial multiplexing performance - Single-Layer Spatial Multiplexing 4 Tx Antenna Port with CRS assistance information (Cell-Specific Reference Symbols)	1138

8.10.1.2.12	Closed loop spatial multiplexing performance - Single-Layer Spatial Multiplexing with CRS assistance information (User-Specific Reference Symbols)	1139

8.10.1.2.13	Performance requirements for DCI format 2D and non Quasi Co-located Antenna Ports	1140

8.10.1.2.14	HST-SFN performance	1144

8.10.2	PDCCH/PCFICH	1145

8.10.2.1	FDD	1145

8.10.2.1.1	Single-antenna port performance	1145

8.10.2.1.2	Transmit diversity performance with 2 Tx Antenna Ports	1145

8.10.2.1.3	Transmit diversity performance with 4 Tx Antenna Ports	1146

8.10.2.1.4	Enhanced Downlink Control Channel Performance Requirement Type A - 4 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	1146

8.10.2.2	TDD	1148

8.10.2.2.1	Single-antenna port performance	1148

8.10.2.2.2	Transmit diversity performance with 2 Tx Antenna Ports	1148

8.10.2.2.3	Transmit diversity performance with 4 Tx Antenna Ports	1149

8.10.2.2.4	Enhanced Downlink Control Channel Performance Requirement Type A - 4 Tx Antenna Port with Non-Colliding CRS Dominant Interferer	1149

8.10.3	PHICH	1150

8.10.3.1	 FDD	1150

8.10.3.1.1	Single Tx Antenna Port performance	1150

8.10.3.1.2	Transmit diversity performance with 2 Tx Antenna Ports	1151

8.10.3.1.3	Transmit diversity performance with 4 Tx Antenna Ports	1151

8.10.3.2	TDD	1151

8.10.3.2.1	Single Tx Antenna Port performance	1152

8.10.3.2.2	Transmit diversity performance with 2 Tx Antenna Ports	1152

8.10.3.2.3	Transmit diversity performance with 4 Tx Antenna Ports	1153

8.10.4	ePDCCH	1153

8.10.4.1	Distributed Transmission with 4Rx	1153

8.10.4.1.1	FDD	1153

8.10.4.1.2	TDD	1154

8.10.4.2	Localized Transmission with TM9 and 4Rx	1155

8.10.4.2.1	FDD	1155

8.10.4.2.2	TDD	1156

8.11	Demodulation (UE supporting coverage enhancement)	1157

8.11.1	PDSCH	1158

8.11.1.1	FDD and half-duplex FDD (Fixed Reference Channel)	1158

8.11.1.1.1	Closed-loop spatial multiplexing performance (Cell-Specific Reference Symbols)	1158

8.11.1.1.2	Closed-loop spatial multiplexing performance (User-Specific Reference Symbols)	1162

8.11.1.1.3	Transmit diversity performance (Cell-Specific Reference Symbols)	1164

8.11.1.2	TDD (Fixed Reference Channel)	1170

8.11.1.2.1	Closed-loop spatial multiplexing performance (Cell-Specific Reference Symbols)	1170

8.11.1.2.2	Closed-loop spatial multiplexing performance (User-Specific Reference Symbols)	1175

8.11.1.2.3	Transmit diversity performance (Cell-Specific Reference Symbols)	1177

8.11.2	MPDCCH	1181

8.11.2.1	FDD and half-duplex FDD	1182

8.11.2.1.1	CE Mode A	1183

8.11.2.1.2	CE Mode B	1184

8.11.2.1.3	CE Mode A with TM9 interference model	1184

8.11.2.1.4	CE Mode A with CRS interference model	1186

8.11.2.1.5	CE Mode A and CE Mode B when CRS-ChEstMPDCCH-Config is configured	1187

8.11.2.2.5	CE Mode A and CE Mode B when CRS-ChEstMPDCCH-Config is configured	1188

8.11.2.2	TDD	1190

8.11.2.2.1	CE Mode A	1191

8.11.2.2.2	CE Mode B	1192

8.11.2.2.3	CE Mode A with TM9 interference model	1192

8.11.2.2.4	CE Mode A with CRS interference model	1193

8.11.3	PBCH	1194

8.11.3.1	FDD and half-duplex FDD	1195

8.11.3.1.1	Transmit diversity performance	1195

8.11.3.2	TDD	1196

8.11.3.2.1	Transmit diversity performance	1196

8.12	Demodulation of Narrowband IoT	1196

8.12.1	NPDSCH	1196

8.12.1.1.1	Minimum Requirements for In-band	1197

8.12.1.1.2	Minimum Requirements for Standalone/Guard-band	1198

8.12.1.1.3	Minimum Requirements for Standalone for UE Category NB2	1199

8.12.1.1.4	Minimum Requirements for Standalone for UE with multiple TBs interleaved transmission	1200

8.12.1.1.5	Minimum Requirements for Standalone for UE with 16-QAM	1200

8.12.1.2	TDD	1202

8.12.1.2.1	Minimum Requirements for In-band	1202

8.12.1.2.2	Minimum Requirements for Standalone/Guard-band	1203

8.12.1.2.3	Minimum Requirements for Standalone for UE Category NB2	1204

8.12.1.2.4	Minimum Requirements for Standalone for UE with 16-QAM	1204

8.12.2	NPDCCH	1205

8.12.2.1	Half-duplex FDD	1205

8.12.2.1.1	Single-antenna performance	1206

8.12.2.1.2	Transmit diversity performance	1207

8.12.2.2	TDD	1207

8.12.2.2.1	Single-antenna performance	1209

8.12.2.2.2	Transmit diversity performance	1209

8.12.3	Demodulation of NPBCH	1209

8.12.3.1	HD-FDD	1210

8.12.3.1.1	Single-antenna port performance with single NPBCH TTI	1210

8.12.3.1.2	Transmit diversity performance	1210

8.12.3.2	TDD	1210

8.12.3.2.1	Single-antenna port performance with single NPBCH TTI	1211

8.12.3.2.2	Transmit diversity performance	1211

8.13	Demodulation of PDSCH CA and DC(4 receiver antenna ports)	1211

8.13.1	FDD (CA and DC)	1211

8.13.1.1	Closed-loop spatial multiplexing performance	1211

8.13.1.1.1	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port	1211

8.13.1.1.2	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port for dual connectivity	1215

8.13.1.1.3	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port with 256QAM	1217

8.13.1.1.4	Minimum Requirement Four-Layer Spatial Multiplexing 4 Tx Antenna Port	1219

8.13.1.2	Dual-Layer Spatial Multiplexing (User-Specific Reference Symbols)	1220

8.13.1.2.1	Minimum Requirement Dual-Layer Spatial Multiplexing 2 Tx Antenna Port	1220

8.13.1.3	Enhanced Performance Requirements Type A Closed-loop spatial multiplexing	1223

8.13.1.3.1	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model (Cell-Specific Reference Symbols)	1223

8.13.1.4	Enhanced Performance Requirement Type A - Single-layer Spatial Multiplexing (User-Specific Reference Symbols)	1225

8.13.1.4.1	Minimum Requirement Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model (User-Specific Reference Symbols)	1225

8.13.2	TDD (CA and DC)	1228

8.13.2.1	Closed-loop spatial multiplexing performance	1229

8.13.2.1.1	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port	1229

8.13.2.1.2	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port for dual connectivity	1231

8.13.2.1.3	Minimum Requirement Multi-Layer Spatial Multiplexing 4 Tx Antenna Port with 256QAM	1232

8.13.2.1.4	Minimum Requirement Four-Layer Spatial Multiplexing 4 Tx Antenna Port	1234

8.13.2.2	Dual-Layer Spatial Multiplexing (User-Specific Reference Symbols)	1235

8.13.2.2.1	Minimum Requirement Dual-Layer Spatial Multiplexing 2 Tx Antenna Port	1235

8.13.2.4	Enhanced Performance Requirement Type A - Single-layer Spatial Multiplexing (User-Specific Reference Symbols)	1239

8.13.2.4.1	Minimum Requirement Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model (User-Specific Reference Symbols)	1239

8.13.3	TDD-FDD (CA and DC)	1241

8.13.3.1	 Closed-loop spatial multiplexing performance 4Tx Antenna Port	1242

8.13.3.1.1	Minimum Requirement for FDD PCell	1242

8.13.3.1.2	Minimum Requirement for TDD PCell	1245

8.13.3.2	Dual-Layer Spatial Multiplexing (User-Specific Reference Symbols)	1249

8.13.3.2.1	Minimum Requirement Dual-Layer Spatial Multiplexing 2 Tx Antenna Port for FDD PCell	1249

8.13.3.2.2	Minimum Requirement Dual-Layer Spatial Multiplexing 2 Tx Antenna Port for TDD PCell	1252

8.13.3.3	Enhanced Performance Requirements Type A Closed-loop spatial multiplexing	1255

8.13.3.3.1	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model (Cell-Specific Reference Symbols) for FDD PCell	1255

8.13.3.3.2	Minimum Requirement Single-Layer Spatial Multiplexing 2 Tx Antenna Port with TM4 interference model (Cell-Specific Reference Symbols) for TDD PCell	1257

8.13.3.4	Enhanced Performance Requirement Type A - Single-layer Spatial Multiplexing (User-Specific Reference Symbols)	1259

8.13.3.4.1	Minimum Requirement Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model (User-Specific Reference Symbols) for FDD PCell	1259

8.13.3.4.2	Minimum Requirement Enhanced Performance Requirement Type A – Single-layer Spatial Multiplexing with TM9 interference model (User-Specific Reference Symbols) for TDD PCell	1262

8.13.3.5	 Closed-loop spatial multiplexing performance 4Tx Antenna Port for DC	1265

8.13.3.5.1	Minimum Requirement for FDD PCell	1265

8.13.3.5.2	Minimum Requirement for TDD PCell	1267

8.13.3.6	 Closed-loop spatial multiplexing performance 4Tx Antenna Port with 256QAM	1269

8.13.3.6.1	Minimum Requirement for FDD PCell	1269

8.13.3.6.2	Minimum Requirement for TDD PCell	1271

8.13.3.7	 Closed-loop spatial multiplexing performance 4Tx Antenna Port with Four layers	1272

8.13.3.7.1	Minimum Requirement for FDD PCell	1272

8.13.3.7.2	Minimum Requirement for TDD PCell	1274

8.14	Demodulation (UE supporting Short TTI)	1276

8.14.1	Slot-PDSCH and Subslot-PDSCH	1276

8.14.1.1	FDD (Fixed Reference Channel)	1276

8.14.1.1.1	Open-loop spatial multiplexing performance	1276

8.14.1.1.2	Closed-loop spatial multiplexing performance (User-Specific Reference Signals)	1277

8.14.1.2	TDD (Fixed Reference Channel)	1279

8.14.1.2.1	Open-loop spatial multiplexing performance	1279

8.14.1.2.2	Closed-loop spatial multiplexing performance (User-Specific Reference Signals)	1280

8.14.2	SPDCCH	1281

8.14.2.1	FDD	1281

8.14.2.1.1	Mimimum requirement	1282

8.14.2.2	TDD	1282

8.14.2.2.1	Mimimum requirement	1283

8.15	Demodulation (8 receiver antenna ports)	1284

8.15.1	PDSCH	1284

8.15.1.1	Void	1284

8.15.1.2	TDD (Fixed Reference Channel)	1284

8.15.1.2.1	Transmit diversity performance with 2Tx Antenna Ports (Cell-Specific Reference Symbols)	1284

8.15.1.2.2	Open-loop spatial multiplexing performance with 2Tx Antenna Ports (Cell-Specific Reference Symbols)	1285

8.15.1.2.3	8 Layer Spatial Multiplexing (User-Specific Reference Symbols)	1285

8.15.2	CA	1287

8.15.2.1	Void	1287

8.15.2.2	TDD	1287

8.15.2.2.1	Eight Layer Spatial Multiplexing (User-Specific Reference Symbols)	1287

9	Reporting of Channel State Information	1289

9.1	General	1289

9.1.1	Applicability of requirements	1289

9.1.1.1	Applicability of requirements for different channel bandwidths	1289

9.1.1.2	Applicability and test rules for different CA configurations and bandwidth combination sets	1289

9.1.1.2A	Applicability and test rules for different TDD-FDD CA configurations and bandwidth combination sets	1290

9.1.1.3	Test coverage for different number of componenet carriers	1291

9.1.1.4	Applicability of performance requirements for 4Rx capable UEs	1291

9.1.1.4.1	Applicability rule and antenna connection for single carrier tests with 2Rx	1291

9.1.1.4.2	Applicability rule and antenna connection for CA tests with 2Rx	1293

9.1.1.4.3	Applicability rule and antenna connection for single carrier tests with 4Rx	1294

9.1.1.5	Applicability of requirements for UEs supporting coverage enhancement	1294

9.2	 CQI reporting definition under AWGN conditions	1294

9.2.1	Minimum requirement PUCCH 1-0 (Cell-Specific Reference Symbols)	1294

9.2.1.1	FDD	1294

9.2.1.2	TDD	1295

9.2.1.3	FDD (CSI measurements in case two CSI subframe sets are configured)	1296

9.2.1.4	TDD (CSI measurements in case two CSI subframe sets are configured)	1299

9.2.1.5	FDD (CSI measurements in case two CSI subframe sets are configured and with CRS assistance information)	1302

9.2.1.6	TDD (CSI measurements in case two CSI subframe sets are configured and with CRS assistance information)	1305

9.2.1.7	FDD (Modulation and TBS index Table 2 and 4-bit CQI Table 2 are used)	1308

9.2.1.8	TDD (Modulation and TBS index Table 2 and 4-bit CQI Table 2 are used)	1309

9.2.1.9	FDD (Modulation and TBS index Table 3 and 4-bit CQI Table 4 are used)	1309

9.2.1.10	TDD (Modulation and TBS index Table 3 and 4-bit CQI Table 4 are used)	1310

9.2.2	Minimum requirement PUCCH 1-1 (Cell-Specific Reference Symbols)	1311

9.2.2.1	FDD	1311

9.2.2.2	TDD	1312

9.2.3	Minimum requirement PUCCH 1-1 (CSI Reference Symbols)	1313

9.2.3.1	FDD	1313

9.2.3.1A	FDD (With channelMeasRestriction configured)	1314

9.2.3.2	TDD	1315

9.2.3.2A	TDD (With channelMeasRestriction configured)	1316

9.2.4	Minimum requirement PUCCH 1-1 (With Single CSI Process)	1317

9.2.4.1	FDD	1318

9.2.4.1A	FDD (With interferenceMeasRestriction configured)	1320

9.2.4.2	TDD	1323

9.2.4.2A	TDD (With interferenceMeasRestriction configured)	1326

9.2.5	Minimum requirement PUCCH 1-1 (when csi-SubframeSet –r12 and EIMTA-MainConfigServCell-r12 are configured)	1329

9.2.6	Minimum requirement PUSCH 3-0 (Cell-Specific Reference Symbols)	1332

9.2.6.1	Frame structure type 3 with FDD Pcell	1332

9.2.6.2	Frame structure type 3 with TDD Pcell	1334

9.2.7	Minimum requirement PUSCH 3-1 (CSI Reference Symbol)	1336

9.2.7.1	Frame structure type 3 wth FDD Pcell	1336

9.2.7.2	Frame structure type 3 wth TDD Pcell	1338

9.3	CQI reporting under fading conditions	1342

9.3.1	Frequency-selective scheduling mode	1342

9.3.1.1	Minimum requirement PUSCH 3-0 (Cell-Specific Reference Symbols)	1342

9.3.1.1.1	FDD	1342

9.3.1.1.2	TDD	1343

9.3.1.1.3	FDD (CSI measurements in case two CSI subframe sets are configured and with CRS assistance information)	1344

9.3.1.1.4	TDD (CSI measurements in case two CSI subframe sets are configured and with CRS assistance information)	1348

9.3.1.1.5	TDD (when csi-SubframeSet –r12 is configured)	1351

9.3.1.2	Minimum requirement PUSCH 3-1 (CSI Reference Symbol)	1353

9.3.1.2.1	FDD	1353

9.3.1.2.2	TDD	1354

9.3.1.2.3	FDD (Modulation and TBS index Table 2 and 4-bit CQI Table 2 are used)	1356

9.3.1.2.4	TDD (Modulation and TBS index Table 2 and 4-bit CQI Table 2 are used)	1357

9.3.1.2.5	Void	1358

9.3.1.2.6	TDD (when csi-SubframeSet –r12 is configured with one CSI process)	1358

9.3.2	Frequency non-selective scheduling mode	1362

9.3.2.1	Minimum requirement PUCCH 1-0 (Cell-Specific Reference Symbol)	1362

9.3.2.1.1	FDD	1362

9.3.2.1.2	TDD	1364

9.3.2.2	Minimum requirement PUCCH 1-1 (CSI Reference Symbol)	1366

9.3.2.2.1	FDD	1366

9.3.2.2.2	TDD	1367

9.3.3	Frequency-selective interference	1369

9.3.3.1	Minimum requirement PUSCH 3-0 (Cell-Specific Reference Symbol)	1369

9.3.3.1.1	FDD	1369

9.3.3.1.2	TDD	1370

9.3.3.2	Void	1371

9.3.3.2.1	Void	1371

9.3.3.2.2	Void	1371

9.3.4	UE-selected subband CQI	1371

9.3.4.1	Minimum requirement PUSCH 2-0 (Cell-Specific Reference Symbols)	1371

9.3.4.1.1	FDD	1371

9.3.4.1.2	TDD	1372

9.3.4.2	Minimum requirement PUCCH 2-0 (Cell-Specific Reference Symbols)	1373

9.3.4.2.1	FDD	1373

9.3.4.2.2	TDD	1375

9.3.5	Additional requirements for enhanced receiver Type A	1377

9.3.5.1	Minimum requirement PUCCH 1-0 (Cell-Specific Reference Symbol)	1377

9.3.5.1.1	FDD	1377

9.3.5.1.2	TDD	1378

9.3.5.2	Minimum requirement PUCCH 1-1 (CSI Reference Symbol)	1380

9.3.5.2.1	FDD	1380

9.3.5.2.2	TDD	1383

9.3.6	Minimum requirement (With multiple CSI processes)	1386

9.3.6.1	FDD	1386

9.3.6.2	TDD	1391

9.3.7	Minimum requirement PUSCH 3-2	1395

9.3.7.1	FDD	1395

9.3.7.2	TDD	1396

9.3.8	Additional requirements for enhanced receiver Type B	1398

9.3.8.1	Minimum requirement PUCCH 1-1 (Cell-Specific Reference Symbols)	1398

9.3.8.1.1	FDD	1398

9.3.8.1.2	TDD	1399

9.3.8.2	Minimum requirement PUCCH 1-1 (CSI Reference Symbols)	1401

9.3.8.2.1	FDD	1401

9.3.8.2.2	TDD	1403

9.3.8.3	Minimum requirement with CSI process	1406

9.3.8.3.1	FDD	1406

9.3.8.3.2	TDD	1409

9.4	Reporting of Precoding Matrix Indicator (PMI)	1412

9.4.1	Single PMI	1413

9.4.1.1	Minimum requirement PUSCH 3-1 (Cell-Specific Reference Symbols)	1413

9.4.1.1.1	FDD	1413

9.4.1.1.2	TDD	1414

9.4.1.2	 Minimum requirement PUCCH 2-1 (Cell-Specific Reference Symbols)	1415

9.4.1.2.1	FDD	1415

9.4.1.2.2	TDD	1417

9.4.1.3	Minimum requirement PUSCH 3-1 (CSI Reference Symbol)	1418

9.4.1.3.1	FDD	1418

9.4.1.3.2	TDD	1421

9.4.1.3.3	FDD (with Class A 12Tx codebook)	1424

9.4.1.3.4	TDD (with Class A 12Tx codebook)	1426

9.4.1.3.5	FDD (with Class A 24Tx codebook)	1429

9.4.1.3.6	TDD (with Class A 24Tx codebook)	1431

9.4.1.4	Minimum requirement PUCCH 1-1 (CSI Reference Symbol)	1434

9.4.1.4.1	FDD (with 4Tx enhanced codebook)	1434

9.4.1.4.2	TDD (with 4Tx enhanced codebook)	1436

9.4.1.4.3	FDD (with Class B alternative codebook for one CSI-RS resource configured)	1439

9.4.1.4.4	TDD (with Class B alternative codebook for one CSI-RS resource configured)	1441

9.4.1a	Void	1444

9.4.1a.1	Void	1444

9.4.1a.1.1	Void	1444

9.4.1a.1.2	Void	1444

9.4.2	Multiple PMI	1444

9.4.2.1	Minimum requirement PUSCH 1-2 (Cell-Specific Reference Symbols)	1444

9.4.2.1.1	FDD	1444

9.4.2.1.2	TDD	1445

9.4.2.2	 Minimum requirement PUSCH 2-2 (Cell-Specific Reference Symbols)	1446

9.4.2.2.1	FDD	1446

9.4.2.2.2	TDD	1447

9.4.2.3	Minimum requirement PUSCH 1-2 (CSI Reference Symbol)	1448

9.4.2.3.1	FDD	1448

9.4.2.3.2	TDD	1450

9.4.2.3.3	FDD (with 4Tx enhanced codebook)	1453

9.4.2.3.4	TDD (with 4Tx enhanced codebook)	1455

9.4.2.3.5	FDD (with Class A 16Tx codebook)	1457

9.4.2.3.6	TDD (with Class A 16Tx codebook)	1460

9.4.2.3.7	FDD (with Class A 32Tx codebook)	1463

9.4.2.3.8	TDD (with Class A 32Tx codebook)	1466

9.4.2.3.9	FDD (with Class A 16Tx advanced codebook)	1469

9.4.2.3.10	TDD (with Class A 16Tx advanced codebook)	1472

9.4.3	Void	1475

9.5	Reporting of Rank Indicator (RI)	1475

9.5.1	Minimum requirement (Cell-Specific Reference Symbols)	1475

9.5.1.1	FDD	1475

9.5.1.2	TDD	1476

9.5.2	Minimum requirement (CSI Reference Symbols)	1477

9.5.2.1	FDD	1477

9.5.2.2	TDD	1479

9.5.3	Minimum requirement (CSI measurements in case two CSI subframe sets are configured)	1481

9.5.3.1	FDD	1481

9.5.3.2	TDD	1484

9.5.4	Minimum requirement (CSI measurements in case two CSI subframe sets are configured and CRS assistance information are configured)	1487

9.5.4.1	FDD	1487

9.5.4.2	TDD	1490

9.5.5	Minimum requirement (with CSI process)	1493

9.5.5.1	FDD	1494

9.5.5.2	TDD	1497

9.6	 Additional requirements for carrier aggregation	1500

9.6.1	 Periodic reporting on multiple cells (Cell-Specific Reference Symbols)	1500

9.6.1.1	FDD	1500

9.6.1.2	TDD	1506

9.6.1.3	TDD-FDD CA with FDD PCell	1512

9.6.1.4	TDD-FDD CA with TDD PCell	1517

9.7	CSI reporting (Single receiver antenna)	1523

9.7.1	CQI reporting definition under AWGN conditions	1524

9.7.1.1	FDD and half-duplex FDD	1524

9.7.1.2	TDD	1524

9.7.1.3 FDD (Category 1bis UE)	1525

9.7.1.4 TDD (Category 1bis UE)	1526

9.7.2	CQI reporting under fading conditions	1527

9.7.2.1	FDD and half-duplex FDD	1527

9.7.2.2	TDD	1528

9.7.2.3 FDD (Category 1bis UE)	1529

9.7.2.4 TDD (Category 1bis UE)	1530

9.8	CSI reporting (UE supporting coverage enhancement)	1531

9.8.1	CQI reporting definition under AWGN conditions	1531

9.8.1.1	FDD and half-duplex FDD	1531

9.8.1.2	TDD	1532

9.8.2	UE-selected subband CQI	1533

9.8.2.1	FDD and half-duplex FDD	1533

9.8.2.2	TDD	1537

9.8.3	CQI reporting definition for UE supporting 64QAM under AWGN	1540

9.8.3.1	FDD and half-duplex FDD	1540

9.8.3.2	TDD	1541

9.8.4	CQI reporting definition for UE supporting alternative table under AWGN	1542

9.8.4.1	FDD and half-duplex FDD	1542

9.8.4.2	TDD	1543

9.8.5	PMI reporting with PUCCH 1-1 (CSI Reference Symbol)	1544

9.8.5.1	FDD	1545

9.8.5.2	TDD	1546

9.9	CSI reporting for 4Rx UE	1547

9.9.1	CQI reporting definition under AWGN conditions	1547

9.9.1.1	Minimum requirement PUCCH 1-0 with Rank 1 (Cell-Specific Reference Symbols)	1548

9.9.1.1.1	FDD	1548

9.9.1.1.2	TDD	1548

9.9.1.2	Minimum requirement PUCCH 1-1 with Rank 2 (CSI Reference Symbols)	1549

9.9.1.2.1	FDD	1549

9.9.1.2.2	TDD	1550

9.9.1.3	 Minimum requirement PUCCH 1-1 with Rank 4 (Cell-Specific Reference Symbols)	1551

9.9.1.3.1	FDD	1552

9.9.1.3.2	TDD	1552

9.9.1.4	Minimum requirement PUCCH 1-1 with Rank 3 (CSI Reference Symbols)	1553

9.9.1.4.1	FDD	1553

9.9.1.4.2	TDD	1554

9.9.2	CQI reporting definition under fading conditions	1555

9.9.2.1	Minimum requirement PUCCH 1-0 (Cell-Specific Reference Symbol) for enhanced receiver Type A	1555

9.9.2.1.1	FDD	1556

9.9.2.1.2	TDD	1557

9.9.2.2	Minimum requirement PUCCH 1-1 (CSI Reference Symbol) for enhanced receiver Type A	1560

9.9.2.2.1	FDD	1560

9.9.2.2.2	TDD	1563

9.9.3	Reporting of Precoding Matrix Indicator (PMI) for 4Rx UE	1566

9.9.3.1	Minimum requirement PUSCH 3-1 (CSI Reference Symbol)	1567

9.9.3.1.1	TDD	1567

9.9.4	 Reporting of Rank Indicator (RI)	1569

9.9.4.1	Minimum requirement (Cell-Specific Reference Symbols)	1569

9.9.4.1.1	FDD	1569

9.9.4.1.2	TDD	1571

9.9.4.2	Minimum requirement (CSI Reference Symbols)	1572

9.9.4.2.1	FDD	1572

9.9.4.2.2	TDD	1574

9.10	Reporting of CSI-RS Resource Indicator (CRI)	1576

9.10.1	Minimum requirement (PUSCH 3-1)	1577

9.10.1.1	FDD	1577

9.10.1.2	TDD	1579

9.10.2	Minimum requirement (PUSCH 3-1, QCL Type C)	1581

9.10.2.1	FDD	1581

9.10.2.2	TDD	1584

9.11	Reporting of Hybrid Channel state information	1587

9.11.1	Minimum requirement (with eMIMO-Type configured as Class B with more than one CSI-RS resource configured and eMIMO-Type2 as Class B with one CSI-RS resource configured)	1587

9.11.1.1	FDD	1588

9.11.1.2	TDD	1590

9.12	CSI reporting (UE supporting Short TTI)	1592

9.12.1	CQI reporting under fading conditions (Cell-Specific Reference Symbol)	1592

9.12.1.1	FDD	1592

9.12.1.2	TDD	1595

9.12.2	CQI reporting under fading conditions (CSI Reference Symbol)	1596

9.12.2.1	FDD	1596

9.12.2.2	TDD	1599

9.13	CSI reporting for 8Rx UE	1601

9.13.1	CQI reporting definition under AWGN conditions	1601

9.13.1.1	Minimum requirement PUCCH 1-1 with Rank 4 (CSI Reference Symbols)	1601

9.13.1.2.1	Void	1601

9.13.1.2.2	TDD	1601

9.14	CSI reporting of Narrowband IoT	1602

9.14.1	CQI reporting definition under AWGN conditions	1602

9.14.1.1	Half-duplex FDD	1602

9.14.1.2	TDD	1603

10	Performance requirement (MBMS)	1604

10.1	FDD (Fixed Reference Channel)	1604

10.1.1	Minimum requirement	1605

10.2	TDD (Fixed Reference Channel)	1606

10.2.1	Minimum requirement	1606

10.3	FDD (Fixed Reference Channel) with FeMBMS	1607

10.3.1	Minimum requirement for FeMBMS Unicast-mixed Cell under CA	1607

10.3.1.1	Minimum requirement with 1.25kHz subcarrier spacing	1607

10.3.1.2	Minimum requirement with 7.5kHz subcarrier spacing	1608

10.3.2	Minimum requirement for FeMBMS Unicast-mixed Cell as Non-Serving Cell	1609

10.3.2.1	Minimum requirement with 1.25kHz subcarrier spacing	1609

10.3.2.2	Minimum requirement with 7.5kHz subcarrier spacing	1610

10.3.3	Minimum requirement for MBMS Dedicated cell	1611

10.3.3.1	Minimum requirement with 1.25kHz subcarrier spacing	1611

10.3.3.2	Minimum requirement with 7.5kHz subcarrier spacing	1612

10.3.3.3	Minimum requirement with 15kHz subcarrier spacing	1613

10.4 FDD with 5G terrestrial broadcast	1614

10.4.1	Minimum requirement for PMCH decoding	1614

10.4.1.1	Minimum requirement with 0.37kHz subcarrier spacing	1614

10.4.1.2	Minimum requirement with 2.5kHz subcarrier spacing	1615

10.4.2	Minimum requirement for CAS detection	1616

10.4.2.1	Minimum requirement for PBCH detection	1616

11	Performance requirement (ProSe Direct Discovery)	1616

11.1	General	1617

11.1.1	Applicability of requirements	1617

11.1.2	Reference DRX configuration	1617

11.2	Demodulation of PSDCH (single link performance)	1617

11.2.1	FDD (in-coverage)	1617

11.2.2	TDD (in-coverage)	1618

11.2.3	FDD (out-of-coverage)	1619

11.3	Power imbalance performance with two links	1620

11.3.1	FDD	1620

11.3.2	TDD	1621

11.4	Multiple timing reference test	1622

11.4.1	FDD	1623

11.5	Maximum Sidelink processes test	1624

11.5.1	FDD	1624

11.5.2	TDD	1625

12	Performance requirement (ProSe Direct Communication)	1627

12.1	General	1627

12.1.1	Applicability of requirements	1627

12.1.1.1	Applicability of requirements for different channel bandwidths	1627

12.1.1.2	Test coverage for different number of component carriers	1627

12.1.1.3	Applicability and test rules for different CA configurations and bandwidth combination sets	1627

12.1.2	Reference DRX configuration	1628

12.2	Demodulation of PSSCH	1628

12.2.1	FDD	1628

12.3	Demodulation of PSCCH	1629

12.3.1	FDD	1630

12.4	Demodulation of PSBCH	1631

12.4.1	FDD	1631

12.5	Power imbalance performance with two links	1631

12.5.1	 FDD	1631

12.6	Multiple timing reference test	1633

12.6.1	FDD	1633

12.7	Maximum Sidelink processes test	1636

12.7.1	FDD	1636

12.8	Sustained downlink data rate with active Sidelink	1637

13	Void	1639

14	Performance requirement (V2X Sidelink Communication)	1639

14.1	General	1639

14.1.1	Applicability of requirements	1639

14.2	Demodulation of PSSCH	1640

14.3	Demodulation of PSCCH	1641

14.4	Power imbalance performance with two links	1641

14.5	Demodulation of PSBCH	1642

14.6	Demodulation of PSSCH with eNB based synchronization	1643

14.7	Soft buffer test	1643

14.8	PSCCH/PSSCH decoding capability test	1644

14.9	Sustained downlink data rate with active sidelink	1645

14.10	Soft buffer test (CA)	1646

14.11	PSCCH/PSSCH decoding capability test (CA)	1647

Annex A (normative):  Measurement channels	1646

A.1	General	1646

A.2	UL reference measurement channels	1646

A.2.1	General	1646

A.2.1.1	Applicability and common parameters	1646

A.2.1.2	Determination of payload size	1646

A.2.1.3	Overview of UL reference measurement channels	1647

A.2.2	Reference measurement channels for FDD	1670

A.2.2.1	Full RB allocation	1670

A.2.2.1.1	QPSK	1670

A.2.2.1.2	16-QAM	1672

A.2.2.1.3	64-QAM	1674

A.2.2.1.4	256 QAM	1675

A.2.2.2	Partial RB allocation	1675

A.2.2.2.1	QPSK	1676

A.2.2.2.2	16-QAM	1679

A.2.2.2.3	64-QAM	1681

A.2.2.2.4	256 QAM	1683

A.2.2.3	Void	1683

A.2.2.4	subPRB allocation	1684

A.2.3	Reference measurement channels for TDD	1684

A.2.3.1	Full RB allocation	1684

A.2.3.1.1	QPSK	1684

A.2.3.1.2	16-QAM	1687

A.2.3.1.3	64-QAM	1689

A.2.3.1.4	256 QAM	1690

A.2.3.2	Partial RB allocation	1690

A.2.3.2.1	QPSK	1691

A.2.3.2.2	16-QAM	1695

A.2.3.2.3	64-QAM	1700

A.2.3.2.4	256 QAM	1702

A.2.3.3	Void	1702

A.2.3.4	subPRB allocation	1703

A.2.4	Reference measurement channels for UE category NB1	1703

A.2.5	Reference measurement channels for LAA	1704

A.2.5.1	Full RB allocation	1704

A.2.5.1.1	QPSK	1704

A.2.5.1.2	16QAM	1705

A.2.5.1.3	64QAM	1705

A.2.5.2	Partial RB allocation	1705

A.2.5.2.1	QPSK	1706

A.2.5.2.2	16QAM	1706

A.2.5.2.3	64QAM	1707

A.3	DL reference measurement channels	1707

A.3.1	General	1707

A.3.1.1	Overview of DL reference measurement channels	1708

A.3.2	Reference measurement channel for receiver characteristics	1733

A.3.3	Reference measurement channels for PDSCH performance requirements (FDD)	1763

A.3.3.1	Single-antenna transmission (Common Reference Symbols)	1763

A.3.3.2	Multi-antenna transmission (Common Reference Symbols)	1768

A.3.3.2.1	Two antenna ports	1768

A.3.3.2.2	Four antenna ports	1777

A.3.3.3	Reference Measurement Channel for UE-Specific Reference Symbols	1782

A.3.3.3.0	Two antenna ports (no CSI-RS)	1782

A.3.3.3.1	Two antenna port (CSI-RS)	1783

A.3.3.3.2	Four antenna ports (CSI-RS)	1786

A.3.3.3.2A	Eight antenna ports (CSI-RS)	1793

A.3.3.3.3	Twelve antenna port (CSI-RS)	1795

A.3.3.3.4	Sixteen antenna port (CSI-RS)	1796

A.3.3.3.5	Twenty-four antenna port (CSI-RS)	1797

A.3.3.3.6	Thirty-two antenna port (CSI-RS)	1798

A.3.4	Reference measurement channels for PDSCH performance requirements (TDD)	1800

A.3.4.1	Single-antenna transmission (Common Reference Symbols)	1800

A.3.4.2	Multi-antenna transmission (Common Reference Signals)	1808

A.3.4.2.1	Two antenna ports	1808

A.3.4.2.2	Four antenna ports	1821

A.3.4.3	Reference Measurement Channels for UE-Specific Reference Symbols	1828

A.3.4.3.1	Single antenna port (Cell Specific)	1828

A.3.4.3.2	Two antenna ports (Cell Specific)	1829

A.3.4.3.3	Two antenna ports (CSI-RS)	1831

A.3.4.3.4	Four antenna ports (CSI-RS)	1839

A.3.4.3.5	Eight antenna ports (CSI-RS)	1846

A.3.4.3.6	Twelve antenna ports (CSI-RS)	1852

A.3.4.3.7	Sixteen antenna ports (CSI-RS)	1853

A.3.4.3.8	Twenty-four antenna ports (CSI-RS)	1854

A.3.4.3.9	Thirty-two antenna ports (CSI-RS)	1855

A.3.5	Reference measurement channels for PDCCH/PCFICH performance requirements	1857

A.3.5.1	FDD	1857

A.3.5.2	TDD	1857

A.3.5.3	LAA	1858

A.3.6	Reference measurement channels for PHICH performance requirements	1858

A.3.7	Reference measurement channels for PBCH performance requirements	1858

A.3.8	Reference measurement channels for MBMS performance requirements	1859

A.3.8.1	FDD	1859

A.3.8.2	TDD	1864

A.3.9	Reference measurement channels for sustained downlink data rate provided by lower layers	1866

A.3.9.1	FDD	1866

A.3.9.2	TDD	1870

A.3.9.3	FDD (EPDCCH scheduling)	1884

A.3.9.4	TDD (EPDCCH scheduling)	1885

A.3.9.5	LAA	1887

A.3.10	Reference Measurement Channels for EPDCCH performance requirements	1889

A.3.10.1	FDD	1889

A.3.10.2	TDD	1889

A.3.11	Reference Measurement Channels for MPDCCH performance requirements	1889

A.3.11.1	FDD and half-duplex FDD	1889

A.3.11.2	TDD	1890

A.3.12	Reference measurement channels for NPDSCH performance requirements	1891

A.3.12.1	In-band	1891

A.3.12.1.2	Two-antenna transmission	1891

A.3.12.2	Standalone/Guard-band	1893

A.3.12.2.1 Single-antenna transmission	1893

A.3.13	Reference measurement channels for NPDCCH performance requirements	1896

A.3.13.1	Half-duplex FDD	1896

A.3.13.2	TDD	1897

A.3.14	Reference measurement channels for NPBCH performance requirements for Cat NB1 UEs	1897

A.3.15	Reference Measurement Channels for LAA SCell with frame structure Type-3	1898

A.3.15.1	Multi-antenna transmission (Common Reference Symbols)	1898

A.3.15.1.1	Four antenna ports	1898

A.3.15.2	Reference Measurement Channel for UE-Specific Reference Symbols	1898

A.3.15.2.1	Two antenna ports (CSI-RS)	1898

A.3.16	Reference measurement channels for Slot-PDSCH and Subslot-PDSCH performance requirements	1900

A.3.16.1	FDD	1900

A.3.16.2	TDD	1904

A.3.17	Reference measurement channels for SPDCCH performance requirements	1905

A.3.17.1	FDD	1905

A.3.17.2	TDD	1906

A.4	CSI reference measurement channels	1906

A.5	OFDMA Channel Noise Generator (OCNG)	1921

A.5.1	OCNG Patterns for FDD	1921

A.5.1.1	OCNG FDD pattern 1: One sided dynamic OCNG FDD pattern	1921

A.5.1.2	OCNG FDD pattern 2: Two sided dynamic OCNG FDD pattern	1922

A.5.1.3	OCNG FDD pattern 3: 49 RB OCNG allocation with MBSFN in 10 MHz	1922

A.5.1.3A	OCNG FDD pattern 3A: 49 RB OCNG allocation with MBSFN enhancement in 10 MHz	1923

A.5.1.4	OCNG FDD pattern 4: One sided dynamic OCNG FDD pattern for MBMS transmission	1923

A.5.1.4A	OCNG FDD pattern 4A: One sided dynamic OCNG FDD pattern for enhanced MBMS transmission	1924

A.5.1.5	OCNG FDD pattern 5: One sided dynamic 16QAM modulated OCNG FDD pattern	1925

A.5.1.6	OCNG FDD pattern 6: dynamic OCNG FDD pattern when user data is in 2 non-contiguous blocks	1926

A.5.1.8	OCNG FDD pattern 8: Dynamic OCNG FDD pattern for TM10 transmission	1927

A.5.2	OCNG Patterns for TDD	1928

A.5.2.1	OCNG TDD pattern 1: One sided dynamic OCNG TDD pattern	1928

A.5.2.2	OCNG TDD pattern 2: Two sided dynamic OCNG TDD pattern	1929

A.5.2.3	OCNG TDD pattern 3: 49 RB OCNG allocation with MBSFN in 10 MHz	1930

A.5.2.4	OCNG TDD pattern 4: One sided dynamic OCNG TDD pattern for MBMS transmission	1930

A.5.2.5	OCNG TDD pattern 5: One sided dynamic 16QAM modulated OCNG TDD pattern	1931

A.5.2.6	OCNG TDD pattern 6: dynamic OCNG TDD pattern when user data is in 2 non-contiguous blocks	1932

A.5.2.8	OCNG TDD pattern 8: Dynamic OCNG TDD pattern for TM10 transmission	1933

A.5.3	OCNG Patterns for Narrowband IoT	1934

A.5.3.1	Narrowband IoT OCNG pattern 1	1934

A.5.4	OCNG Patterns for frame structure type 3	1935

A.5.4.1	OCNG FS3 pattern 1: One sided dynamic OCNG frame structure type 3 pattern	1935

A.5.4.2	OCNG FS3 pattern 2: Two sided dynamic OCNG frame structure 3 pattern	1936

A.6	Sidelink reference measurement channels	1937

A.6.1	General	1937

A.6.1.1	Overview of ProSe reference measurement channels	1937

A.6.2	Reference measurement channel for receiver characteristics	1938

A.6.3	Reference measurement channels for PSDCH performance requirements	1940

A.6.4	Reference measurement channels for PSCCH performance requirements	1941

A.6.5	Reference measurement channels for PSSCH performance requirements	1941

A.6.6	Reference measurement channels for PSBCH performance requirements	1942

A.7	Sidelink reference resource pool configurations	1943

A.7.1	Reference resource pool configurations for ProSe Direct Discovery demodulation tests	1943

A.7.1.1	FDD	1943

A.7.1.2	TDD	1946

A.7.2	Reference resource pool configurations for ProSe Direct Communication demodulation tests	1948

A.7.2.1	FDD	1948

A.8	V2X reference measurement channels	1953

A.8.1	General	1953

A.8.1.1	Overview of V2X reference measurement channels	1954

A.8.2	Reference measurement channel for receiver characteristics	1954

A.8.3	Reference measurement channel for transmitter characteristics	1956

A.8.4	Reference measurement for PSCCH performance requirements	1959

A.8.5	Reference measurement for PSSCH performance requirements	1960

A.8.6	Reference measurement for PSBCH performance requirements	1960

A.9	V2X reference resource pool configurations	1961

Annex B (normative):  Propagation conditions	1964

B.1	Static propagation condition	1964

B.1.1	UE Receiver with 2Rx	1964

B.1.2	UE Receiver with 4Rx	1964

B.1.3	UE Receiver with 8Rx	1965

B.2	Multi-path fading propagation conditions	1966

B.2.1	Delay profiles	1966

B.2.2	Combinations of channel model parameters	1967

B.2.3	MIMO Channel Correlation Matrices	1968

B.2.3.1	Definition of MIMO Correlation Matrices	1968

B.2.3.2	MIMO Correlation Matrices at High, Medium and Low Level	1974

B.2.3A	MIMO Channel Correlation Matrices using cross polarized antennas	1977

B.2.3A.1	Definition of MIMO Correlation Matrices using cross polarized antennas	1978

B.2.3A.2	Spatial Correlation Matrices using cross polarized antennas at eNB and UE sides	1978

B.2.3A.2.1	Spatial Correlation Matrices at eNB side	1978

B.2.3A.2.2	Spatial Correlation Matrices at UE side	1979

B.2.3A.4	Beam steering approach	1982

B.2.3B	MIMO Channel Correlation Matrices using two-dimension cross polarized antennas at eNB and cross polarized antennas at UE	1982

B.2.3B.1	Definition of MIMO Correlation Matrices using two-dimension cross polarized antennas at eNB and cross polarized antennas at UE	1983

B.2.3B.2	Spatial Correlation Matrices using two-dimension cross polarized antennas at eNB and cross polarized antennas at UE	1984

B.2.3B.2.1	Spatial Correlation Matrices at eNB side	1984

B.2.3B.2.2	Spatial Correlation Matrices at UE side	1984

B.2.3B.3	MIMO Correlation Matrices using two-dimension cross polarized antennas at eNB and cross polarized antennas at UE	1984

B.2.3B.4	Beam steering approach	1987

B.2.3B.4A	Beam steering approach with dual cluster beams	1988

B.2.4	Propagation conditions for CQI tests	1989

B.2.4.1	Propagation conditions for CQI tests with multiple CSI processes	1989

B.2.5	Void	1989

B.2.6	MBSFN Propagation Channel Profile	1989

B.2.6.1	Subcarrier spacing 15kHz or 7.5kHz	1989

B.2.6.2	Subcarrier spacing 1.25kHz	1990

B.2.6.3	Subcarrier spacing 0.37kHz	1991

B.2.6.4	Subcarrier spacing 2.5kHz	1991

B.3	High speed train scenario	1992

B.3A	HST-SFN scenario	1993

B.3B	HST-SFN scenario for 500 km/h speed	1997

B.3C	HST scenario for 500 km/h speed	1997

B.4	Beamforming Model	1997

B.4.1	Single-layer random beamforming (Antenna port 5, 7, or 8)	1997

B.4.1A	Single-layer random beamforming (Antenna port 7, 8, 11 or 13 with enhanced DMRS table configured)	1998

B.4.2	Dual-layer random beamforming (antenna ports 7 and 8)	1998

B.4.3	Generic beamforming model (antenna ports 7-14)	1999

B.4.4	Random beamforming for EPDCCH distributed transmission (Antenna port 107 and 109)	1999

B.4.5	Random beamforming for EPDCCH localized transmission (Antenna port 107, 108, 109 or 110)	2000

B.4.6	Beamforming model for CRI test	2000

B.5	Interference models for enhanced performance requirements Type-A	2001

B.5.1	Dominant interferer proportion	2002

B.5.2	Transmission mode 3 interference model	2002

B.5.3	Transmission mode 4 interference model	2002

B.5.4	Transmission mode 9 interference model	2003

B.6	Interference models for enhanced performance requirements Type-B	2003

B.6.1	Transmission mode 2 interference model	2003

B.6.2	Transmission mode 3 interference model	2003

B.6.3	Transmission mode 4 interference model	2004

B.6.4	Transmission mode 9 interference model	2004

B.6.5	CRS interference model	2005

B.6.6	Random interference model	2005

B.7	Interference models for enhanced downlink control channel performance requirements Type A and B	2006

B.7.1	PDCCH, PCFICH and PHICH interference model	2006

B.8	Burst transmission models for Frame structure type 3	2007

B.8.1	Burst transmission model for one LAA SCell	2007

B.8.2	Burst transmission model for multiple LAA SCell(s)	2007

Annex C (normative):  Downlink Physical Channels	2009

C.1	General	2009

C.2	Set-up	2009

C.3	Connection	2009

C.3.1	Measurement of Receiver Characteristics	2009

C.3.2	Measurement of Performance requirements	2010

C.3.3	Aggressor cell power allocation for Measurement of Performance Requirements when ABS is Configured	2011

C.3.4	Power Allocation for Measurement of Performance Requirements when Quasi Co-location Type B: same Cell ID	2012

C.3.5	Simplified CA testing method	2012

C.3.6	Measurement of Receiver Characteristics for Narrowband IoT	2013

Annex D (normative):  Characteristics of the interfering signal	2014

D.1	General	2014

D.2	Interference signals	2014

Annex E (normative):  Environmental conditions	2015

E.1	 General	2015

E.2	 Environmental	2015

E.2.1	Temperature	2015

E.2.2	Voltage	2015

E.2.3	Vibration	2016

Annex F (normative):  Transmit modulation	2017

F.1	Measurement Point	2017

F.2	Basic Error Vector Magnitude measurement	2017

F.3	Basic in-band emissions measurement	2018

F.4	Modified signal under test	2018

F.5	Window length	2020

F.5.1	Timing offset	2020

F.5.2	Window length	2020

F.5.3	Window length for normal CP	2020

F.5.4	Window length for Extended CP	2021

F.5.5	Window length for PRACH	2021

F.5.F	Window length for category NB1	2022

F.6	Averaged EVM	2022

F.6.F	Averaged EVM for category NB1	2023

F.7	Spectrum Flatness	2023

Annex G (informative):  Reference sensitivity level in lower SNR	2024

G.1	General	2024

G.2	Typical receiver sensitivity performance (QPSK)	2024

G.3	Reference measurement channel for REFSENSE in lower SNR	2030

Annex H (normative):  Modified MPR behavior	2033

H.1	Indication of modified MPR behavior	2033

Annex I (normative):  Supported Post Antenna Gain	2034

I.1	Declared Supported Post Antenna Gain for UE	2034

Annex J (informative):  Change history	2035